| PRR Number | 668 | PRR Title | Distribution Loss Factor Calculations |  |
| --- | --- | --- | --- | --- |
| Timeline (Normal or Urgent) | Normal | Recommended Action |  | Approval |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 13.3, Distribution Losses 13.3.1, Loss Factor Calculation |  |  |
| Proposed Effective Date |  | Upon system implementation |  |  |
| Revision Description |  | Modify the Settlement Interval Distribution Loss Factor (SILF) to better represent the non-linear relationship between the Distribution Loss Factor (DLF) and Settlement Interval ERCOT System Load (SIEL). |  |  |
| Overall Market Benefit |  | Clarification of Protocol language. |  |  |
| Overall Market Impact |  | None. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? If so, are there credit impacts? (indicate Yes or No, and if Yes, include a summary of impact) |  |  |  |  |
| Procedural History |  | - On, 05/24/06 ERCOT posted this Protocol Revision Request (PRR). - On 06/02/06, TXU Electric Delivery posted comments. - On 06/22/06, the Protocol Revisions Subcommittee (PRS) considered this PRR. - On 07/10/06, ERCOT posted the Impact Analysis (IA) - On 07/20/06, PRS assigned this PRR a priority and rank. |  |  |
| PRS Recommendation (indicate whether all segments were present for the vote, and the segment of parties that voted no or abstained) |  | On 6/22/06, PRS unanimously voted to recommend approval of PRR668 with TXU Electric Delivery’s comments. All Market Segments were present for the vote. On 07/20/06, PRS unanimously voted to assign PRR668 a rank of 32.6 and priority of 1.1 on the 2006 Project Prioritization List (PPL) and a priority of 2 (high rank) on the 2007 PPL. All Market Segments were present for the vote. |  |  |
| Summary of PRS discussion |  | On 6/22/06, PRS reviewed TXU Electric Delivery’s comments. The submitter of the PRR agreed with the comments. On 07/20/06, PRS assigned PRR668 the same priority and rank as PRR565, Calculation of Losses for Settlement associated with Project 60068_01 so that they can be implemented under the same project. The coordination of implementation should result in cost savings. |  |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 | None |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | Modifications to ERCOT Settlement System and Market Participant Shadow Settlement Systems | Minor |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | More accurate distribution loss estimates | More accurate settlements |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | None |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other | 1 | None |  |
| Comments | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Original Sponsor |  |
| --- | --- |
| Name | Eric Goff, UFE Co-Chair, on behalf of the UFE Task Force |
| Company | Constellation Energy |
| Segment | Constellation |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| TXU Electric Delivery 060206 | Proposed language changes to accurately reflect DLF determination and to eliminate references to loss study results. Additional market costs and benefits were provided. |

Proposed Protocol Language

13.3	Distribution Losses

By October 30th of each year for the next calendar year, or two (2) months prior to the posting of any update to the approved DLF codes and calculation each Distribution Service Provider (DSP), except NOIEs, shall calculate and provide ERCOT the Annual DLFs to be applied to distribution voltage level Loads in its area of certification.  ERCOT shall review and approve the DLF calculation methodology used by each DSP prior to use of the loss factors for settlement purposes.  If the DLF calculation methodology does not conform with ERCOT’s interpretation of the Protocol criteria in this subsection, ERCOT will work with the Distribution Service Provider to correct the deficiency.  Until deficiencies are resolved, the last approved DLF and the calculation methodology will be posted, and the last approved DLFs shall be used for settlement.  A DSP may only submit a change to the DLF calculation methodology annually or when a change in a DSP service area warrants an update to the approved DLF based on the DSP internal evaluation.

The TDSP shall assign a DLF code to each ESI ID.  A maximum of five (5) DLFs may be submitted for each Distribution Service Provider based upon ERCOT approved parameters, such as service voltages or number of transformations.

The following coding standards will be used to identify the Distribution Loss

Factor applicable to each ESI ID:

- T = Transmission connected Customers (no Distribution Loss Factor applied)

- A through E = TDSP defined Customer segment(s)

The Distribution Service Providers, except NOIEs, are obligated to provide DLFs to ERCOT.  ERCOT will post the DLF and calculation methodology, including any equations and constants, for each DSP.

Loss factor variables submitted by the Distribution Service Provider shall include:

(1)	The annual DLF coefficients (F1, F2, and F3) for each DLF code; and

(2)	The methodology upon which the calculation of the coefficients (F1, F2, and F3) was made.

13.3.1	Loss Factor Calculation

ERCOT shall use the DLFs submitted by the DSP to calculate the Settlement Interval DLFs.  DLFs will be calculated from the data provided by Distribution Service Providers as follows using the following equation:

SILFi = F1 * (SIELi/AAL) + F2 + F3 / (SIELi/AAL)

Where:

i =	interval

SILFi =	Settlement Interval DLF

SIELi =	Settlement Interval ERCOT System Load (forecasted or actual)

AAL =	Annual Interval Average ERCOT System Load (forecasted or actual)

F1 , F2 , F3 =	Coefficients determined by the Distribution Service Provider to allow calculation of its SILF from ERCOT System Load

ERCOT shall use the DLFs calculated for each Settlement Interval of the Operating Day for settlement purposes.

[PRR565: Replace the above sentence with the following on system implementation:] ERCOT shall use the deemed actual DLFs calculated for each Settlement Interval of the Operating Day for settlement purposes.