CRR Settlements when ERCOT is unable to execute DAM

The TPTF determined on 7/25/06 that in the event that ERCOT is unable to execute Day-Ahead Market, all CRRs shall be settled in Real-Time. Below are two options that how the CRRs could be settled.

For both options, Payments/Charges of Real-Time CRRs are uplifted based on Load Ratio Share through Real-Time Revenue Neutrality Allocation.

Option A:

Develop Real-Time CRR Settlements without considering CRR Deration:

- Description:

Option A describes a methodology for RT CRR Settlements that wouldn’t reduce CRR Owner’s payments when transmission network is over loaded due to oversold CRRs in previous CRR auctions.

- Pros: Relatively easy to implement.

- Cons: Not sticking to Day-Ahead CRR Settlements methodology described in Nodal Protocols.

- Major Protocols Impacts:

In Section 7 of Nodal Protocols, if Option A is adopted, it is necessary to develop RT CRR settlement formula for FGRs and PTP Obligations with Refund based on the general settlement formula described below. In addition, there would be slight modifications needed for the settlements of PTP Obligations, PTP Options, PTP Options with Refund in Real-Time.

- General Settlement Formula:

CRR RT Payment/Charge

= (-1) *CRR_Targeted_Payment

Where:

CRR Targed Payment = CRR Price * CRR Qty

| Type of CRR | CRR Price |
| --- | --- |
| PTP Obligations | i=14(RTSPPk,i−RTSPPj,i)4 |
| PTP Obligations with Refund |  |
| PTP Options | i=14Max(0,RTSPPk,i−RTSPPj,i)4 |
| PTP Options with Refund |  |
| FGRs | ye∈MCFRI(INFMCFRI,e∗cRTSPe,c,y)∗TLMPyyTLMPy (New Formula) |

| Type of CRR | CRR Qty |
| --- | --- |
| PTP Obligations | Outstanding CRR MW’s owned by the CRR Owner |
| PTP Options |  |
| FGRs |  |
| PTP Obligations with Refund | Min(Resource Actual Usage , Outstanding CRR MW’s owned by the CRR Owner) |
| PTP Options with Refund |  |

Option B:

Develop CRR Real-Time Settlements considering CRR Deration:

- Description:

Option B inherits the philosophy of Day-Ahead CRR Settlements in current Protocols and thus takes into consideration the situation when the transmission network is oversold in previous CRR auctions. In such case, the payment to CRR owners is reduced based on the deration factor calculated by RT feasibility test. Also CRR Hedge Value is calculated as a minimum amount guaranteed for CRR payment.

- Pros: Stick to Day-Ahead CRR Settlements methodology described in Nodal Protocols.

- Cons: Complex calculation.

- Major Protocol Impacts

In Section 7 of Nodal Protocols, if Option B is adopted, it is necessary to develop RT CRR settlement formula for PTP Obligations, PTP Obligations with Refund, PTP Options, PTP Options with Refund, and FGRs based on the general settlement formula described below.

Additionally, Option B requires RT CRR feasibility test be conducted.

- General Settlement Formula:

RT CRR Payment/Charge

= (-1) * Max ((CRR_Targeted_Payment – CRR_Derated_Amount)

, Min (CRR_Targeted_Payment , CRR_Hedge_Value))

Where:

CRR Targed Payment = CRR Price * CRR Qty

| Type of CRR | CRR Price |
| --- | --- |
| PTP Obligations | i=14(RTSPPk,i−RTSPPj,i)4 |
| PTP Obligations with Refund |  |
| PTP Options | i=14Max(0,RTSPPk,i−RTSPPj,i)4 |
| PTP Options with Refund |  |
| FGRs | ye∈MCFRI(INFMCFRI,e∗cRTSPe,c,y)∗TLMPyyTLMPy (New Formula) |

* RTSP is Real-Time Shadow Price for the SCED Interval. It is a new bill determinant.

CRR Derated Amount = CRR Deration Price * CRR Qty

| Type of CRR | CRR Deration Price |
| --- | --- |
| PTP Obligations | y(cMax(0,RTWASFj,c,y−RTWASFk,c,y)∗RTSPc,y∗RTDRFc,y∗TLMPy)yTLMPy (New Formula) |
| PTP Obligations with Refund |  |
| PTP Options |  |
| PTP Options with Refund |  |
| FGRs | ye∈MCFRI(INFMCFRI,e∗c(RTSPe,c,y∗RTDRFc,y))∗TLMPyyTLMPy (New Formula) |

* RTDRF is Real-Time Deration Factor for constraint c (MW amount by which the constraint is oversold CRR divided by total MW amount of the positive impacts on the constraint of all CRRs existing prior DAM execution) for the SCED Interval. It is a new bill determinant.

CRR Hedge Value = CRR Hedge Value Price * CRR Qty

CRR Hedge Value Price

- If the source is a Load Zone/Hub, and the sink is a Resource Node: i=14Max(0,MAXRESPRk−RTSPPj,i)4 - If the source is a Resource Node, and the sink is a Load Zone/Hub: i=14Max(0,RTSPPk,i−MINRESPRj)4 - If both the source and the sink are Resource Nodes: Max(0,MAXRESPRk−MINRESPRj)

CRR Qty is equal to:

| Type of CRR | CRR Qty |
| --- | --- |
| PTP Obligations PTP Options FGRs | Outstanding CRR MWs owned by the CRR Owner |
| PTP Obligations with Refund PTP Options with Refund | Min(Resource Actual Usage , Outstanding CRR MWs owned by the CRR Owner) |