## ERCOT Operations Report

## ROS & OWG

## July 2006

Frequency control   59.9 > F > 60.1

| Disturbance Data |  |  |
| --- | --- | --- |
| Time | Frequency |  |
| 7/2/2006 18:47 | 59.881 | Unit trip. 445 MW lost. |
| 7/9/2006 13:38 | 59.878 | Unit trip. 516 MW lost. |
| 7/13/2006 0:25 | 59.889 | Unit trip. 575 MW lost. |
| 7/14/2006 5:34 | 59.892 | Unit trip. 362 MW lost. |

Deployed Responsive (6) (Reported Above)

7/2/06 18:47

7/9/06 13:38

7/10/06 12:54

7/11/06 0:06

7/13/06 0:25

7/14/06 5:34

7/16/06 22:05

7/18/06 0:05

7/27/06 8:07

Day Ahead Capacity Purchases

| Historical Totals | July-05 | June-06 | July -06 |
| --- | --- | --- | --- |
| DFW | 76 | 84 | 76 |
| E RIO | 98 | 129 | 61 |
| W RIO | 31 | 90 | 111 |
| Laredo | 93 | 87 | 94 |
| Bryan Area | 71 | 128 | 136 |
| Corpus | 32 | 3 | 13 |
| North | n/a | 18 | 46 |
| San Antonio | n/a | 83 | 13 |
| Western | 24 | 30 | 110 |
| Austin | n/a | 71 | 32 |
| Northeast | n/a | 1 | 19 |
| South | n/a | 35 | 9 |
| Houston | 1 | 144 | 37 |
|  | 429 | 903 | 757 |

|  | RMR | OOMC | RPRS |
| --- | --- | --- | --- |
| DFW | 0 | 15 | 61 |
| E RIO | 0 | 13 | 48 |
| W RIO | 0 | 32 | 79 |
| Laredo | 93 | 0 | 1 |
| Bryan Area | 67 | 13 | 56 |
| Corpus | 0 | 3 | 10 |
| North | 0 | 1 | 45 |
| San Antonio | 0 | 0 | 13 |
| Western | 0 | 1 | 109 |
| Austin | 0 | 0 | 32 |
| Northeast | 0 | 9 | 10 |
| South | 0 | 0 | 9 |
| Houston | 0 | 0 | 37 |
|  | 160 | 87 | 510 |

OOME / Congestion Management Activity (125)

Indicates congestion affected by outages

Indicates a Remedial action plan has been developed for the element

| contingency | overload | Days |
| --- | --- | --- |
| Marion - Zorn & Clear Springs 345 kV | Seguin - Seguin West 138kV | 18 |
| Odessa 345/138kV autotransformer 2 | Odessa 345/138kV autotransformer 1 | 11 |
| Mt. Enterprise - Trinidad & Martin Lake - Stryker Creek 345 kV | Nacogdoches 345/138 kV Autotransformer | 10 |
| Roanoke Sw-Parker-Eagle Mt 345 KV | Rhome-Chisolm 138 KV | 9 |
| Double circuit Gibbons Crk-Greens Prairie and Gibbons Crk - East 138kV | Dansby-Atkins 138kV | 7 |
| Duke - SE Edinburg 138 kV (Pharr) | North Edinburg - MVEC NW Edinburg 138 kV | 6 |
| Morgan Creek 345/138kV #1 Autotransformer | Morgan Creek 345/138kV #2 Autotransformer | 6 |
| Collin 345/138 kV Autotransformer #1 | Plano Custer - Lebanon 138 kV | 4 |
| Double circuit Gibbons Crk-Greens Prairie and Gibbons Crk - East 138kV | Robertson - Watson Chapel 138kV | 4 |
| Lytton Spring-Zorn/Austrop-Zorn 345kV | Canyon to Rohr 138kV | 4 |
| Comanche Peak units 1 and 2 | Uvalde-Asphalt Mines 138 line kV | 3 |
| Comanche Peak SES - Decordova and Wolf Hollow - Rocky Creek 345kV | Concord 345/138 autotransformer | 3 |
| Garfield 345/138 kV Autotransformer #2 | Garfield 345/138 kV Autotransformer #1 | 3 |
| Morgan Creek - China Grove Switch 138 kV | Morgan Creek 138/69 kV Autotransformer #3 | 3 |
| Citgo Park-Highway 9 138kV | Nueces Bay-Morris 138kV | 2 |
| Holly - Barney Davis 138 kV | Cabaniss - Westside 138 kV | 2 |
| Holly - Barney Davis 138 kV | Airline - Cabaniss 138 kV | 2 |
| Royse Switch 345/138kv autotransformer 1 | Forney Switch - Forney West 138 kV | 3 |
| Valley 345/138kV autotransformer | Commerce Switch - Wolf City 138kV | 2 |
| Marion - Hill Country & Skyline 345 kV | Kendall - Comfort 138 kV | 1 |
| Allen Sw - Royse Sw & Monticello 345kV | Richardson East - Apollo 138kV | 1 |
| Asherton - Escondido 138kV | Uvalde - Asphalt Mines 138kV | 1 |
| Cagnon - Marbach 138kV line 1 | Cagnon - Marbach 138kV line 2 | 1 |
| Calaveras - Skyline and Walzem Road 138 kV | Westside - Five Points 138 kV | 1 |
| Clear Spring - Marion and Zorn 345 kV | Henne - Comal 138 kV | 1 |
| Comanche Peak - Wolf Hollow & Rocky Creek 345kV | Concord 345/138 autotransformer | 1 |
| Dbl Circuit MGSES - OECCS 345kV | Big Spring - Big Spring West 138 kV | 1 |
| Double Circuit Zorn-Hayes 345kV | Kendall AT1 | 1 |
| Enterprise - TRSES & SCSES - MLSES 345kV | Nacogdoches SE 345/138kV Xmfr | 1 |
| Hill Country - Marion & STP 345kV | Marion - Skyline 345kV | 1 |
| Menard 138/69kV autotransformer | Fort Mason - Mason CSW 69kV | 1 |
| Menard 138/69kV autotransformer | Ballinger - Paint Creek 69kV | 1 |
| North Edinburg - Bates 138 kV | North Edinburg - Moore Field 69 kV | 1 |
| STP - Whitepoint 345kV | Kennedy Switch - Kennedy 69kV | 1 |
| Forney Sw to East Mesquite 138kv line | Duck Creek to Forney Sw 138kV line | 1 |
| Trinidad - Stryker Creek & Mt. Enterprise 345 kV | Elkton - Athens tap 138kV | 1 |
| Watermill-Limestone 345 circuits | Jewett-Limestone 345 kV line | 1 |
| Fisher Road - Oklaunion & Bowman 345kV | Vernon - Grayback 69kV | 1 |
| Ingleside Dupont Switch - Whitepoint 138 kV | Rincon - Whitepoint 138 kV | 1 |
| Military Highway - La Palma 138 kV | La Palma - Cavazos 69 kV | 1 |
| Sargent Road - Watermill Switch 345 kV | Norwood 345/138 kV Autotransformer #1 | 1 |
| Wynn Joyce 138/69 kV Transformer | Fairdale - Walnut 69kV | 1 |

Zonal OC1’s

| CSC | Days |
| --- | --- |
| South - North | 16 |
| South - Houston | 14 |
| North - West | 6 |
| North - Houston | 2 |
| West - North | 2 |

New RAPs

- None

- New Procedures/ Forms/Operations Bulletins

| Date | Subject | Bulletin# |
| --- | --- | --- |
| 07/31/2006 | Frequency Control Desk - Relief of Non-Spin Obligation | 274 |
| 07/31/2006 | Transmission & Security Desk - Relief of Non-Spin Obligation | 273 |

Blackout/Load Shed incidences

None

Major system voltage problems

None

Voltage Instructions

None

Major Computer Problems, Enhancements & Fixes

None

Communication

None

ERCOT peak (Unofficial)

7/17   15:59   62,544 MW	 6/13	15:58	59,018	MW

Other

None

Alerts, OCN, and Emergency Notices

| 7/6/2006 | 16:30 | 19:30 | Transmission Alert | Loss of San Miguel 138/69kv Auto overloads Derby to Pearsall 69kv line. |
| --- | --- | --- | --- | --- |
| 7/17/2006 | 13:00 | 24:00 | Transmission Alert | Loss of San Miguel 138/69 kv overloads Derby - Pearsall 69 kv. |
| 7/17/2006 | 0:15 | 24:00 | OCN | Weather/ Heat Advisory for ERCOT region |
| 7/18/2006 | 14:00 | 20:00 | OCN | Insufficient balancing up bids for HE 1400 to 2000. |
| 7/18/2006 | 0:15 | 24:00 | OCN | Weather/ Heat Advisory for ERCOT region |
| 7/18/2006 | 12:00 | 22:48 | Transmission Alert | Loss of San Miguel 138/69 kV overloads Derby - Pearsall 69 kV. |
| 7/18/2006 | 14:00 | 22:16 | Transmission Alert | Loss of McCree 345/138 kV Auto overloads Brand - Ben Davis 138 kV. |
| 7/19/2006 | 0:15 | 24:00 | OCN | Weather/ Heat Advisory for ERCOT region |
| 7/19/2006 | 11:00 | 13:00 | OCN | Insufficient balancing up bids for HE 1600 to 1800. |
| 7/19/2006 | 12:30 | 23:08 | Transmission Alert | Loss of San Miguel 138/69 kV overloads Derby - Pearsall 69 kV. |
| 7/20/2006 | 2:00 | 2:30 | Transmission Alert | Adjustment Period Replacement market opened. |
| 7/20/2006 | 11:15 | 23:01 | Transmission Alert | Loss of San Miguel 138/69 kV overloads Derby - Pearsall 69 kV. |
| 7/22/2006 | 2:00 | 2:30 | Transmission Alert | Alert issued due to Adjustment Period Replacement market opened. |
| 7/22/2006 | 9:30 | 18:00 | OCN | OCN Insufficient Balancing Bids for Hours 16 and 17 |
| 7/22/2006 | 13:00 | 24:00 | Transmission Emergency | Transmission Emergency for the Laredo Area, RMR Unit Laredo #3 Forced outage. |
| 7/22/2006 | 13:00 | 24:00 | Transmission Emergency | Block Load Transfer Load some load at Laredo to CFE. |
| 7/22/2006 | 13:00 | 24:00 | Transmission Emergency | Emergency schedule from CFE over the South HVDC Tie. |
| 7/23/2006 | 0:15 | 4:00 | Transmission Emergency | Transmission Emergency for the Laredo Area, RMR Unit Laredo #3 Forced outage. |
| 7/23/2006 | 0:15 | 4:00 | Transmission Emergency | Block Load Transfer Load some load at Laredo to CFE. |
| 7/23/2006 | 0:15 | 2:15 | Transmission Emergency | Emergency schedule from CFE over the South HVDC Tie. |
| 7/23/2006 | 6:00 | 24:00 | Transmission Emergency | Transmission Emergency for the Laredo Area, RMR Unit Laredo #3 Forced outage. |
| 7/23/2006 | 6:00 | 24:00 | Transmission Emergency | Block Load Transfer Load some load at Laredo to CFE. |
| 7/23/2006 | 6:00 | 24:00 | Transmission Emergency | Emergency schedule from CFE over the South HVDC Tie. |
| 7/23/2006 | 12:00 | 17:00 | OCN | ERCOT issued an OCN due to actual temperatures exceeding the forecasted temperatures in several areas of the state. QSE's were requested to please check your resource plans and update as required. |
| 7/23/2006 | 23:00 |  | OCN | Adjustment period RPRS 23:00-23:30 for operating day 7/24/06 |
| 7/24/2006 | 0:00 | 8:10 | Transmission Emergency | Transmission Emergency for the Laredo Area, RMR Unit Laredo #3 Forced outage. |
| 7/24/2006 | 0:15 | 2:15 | Transmission Emergency | Emergency schedule from CFE over the South HVDC Tie. |
| 7/24/2006 | 0:15 | 7:30 | Transmission Emergency | Block Load Transfer Load some load at Laredo to CFE. |
| 7/25/2006 | 21:00 | 24:00 | Transmission Emergency | Emergency notification posted for the Laredo Area |
| 7/26/2006 | 0:00 | 24:00 | Transmission Emergency | Emergency notification posted for the Laredo Area- Emergency ended at midnight. |
| 7/26/2006 | 5:15 | 24:00 | Transmission Emergency | Block load Transfer ERCOT load to CFE at Laredo due to forced outage, ended at midnight. |
| 7/28/2006 | 13:47 | 24:00 | Transmission Alert | loss of Abilene South - Abilene East or Abilene East 138/69kV autotransformer overloads Abilene Elm creek - Abilene Shelton 69kV beyond first contingency limits. |
| 7/28/2006 | 14:21 | 24:00 | Transmission Alert | Loss of Duke - SE Edinburg 138kV overloads North Edinburg - MVEC (NW Edinburg) 138kV beyond first contingency limits. |
| 7/29/2006 | 0:00 | 0:20 | Transmission Alert | Transmission Alert for the loss of Abilene South - Abilene East or Abilene East 138/69kV autotransformer overloads Abilene Elm creek - Abilene Shelton 69kV beyond first contingency limits. |
| 7/29/2006 | 0:00 | 0:20 | Transmission Alert | Transmission Alert for the loss of Duke - SE Edinburg 138kV overloads North Edinburg - MVEC (NW Edinburg) 138kV beyond first contingency limits. |
| 7/31/2006 | 14:54 | 23:30 | Transmission Alert | Transmission Alert for the loss of Duke - SE Edinburg 138 kV overloads North Edinburg - MVEC (NW Edinburg) 138 kV beyond first contingency limits. |