Texas Nodal

Commercial Operations Systems

Software Requirements Specification

For Day-Ahead Market Make-Whole Settlement

Version 1.0

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 07/19/06 | 0.09 | First Draft to TPTF | Mandy Bauld |
| 08/18/06 | 0.091 | Incorporated comments from TPTF | Mandy Bauld |
| 08/23/06 | 1.0 | Approved by TPTF (with modification to add the eligibility flag for energy within the DAMGCOST calculation for Day-Ahead Make-Whole Payment and to change the related bill determinant definitions, dependencies, assumptions, and business rules.) | Mandy Bauld |

Table of Contents

1.	Day-Ahead Market Settlement	4

1.1	Proposed System Scope	4

1.2	Document Conventions	7

1.3	Document Assumptions	7

2.	Functional Requirements	8

2.1	Day-Ahead Make-Whole Payment (DAMWAMT)	8

Input Bill Determinants	10

Intermediate Bill Determinants	14

Output Bill Determinants	15

Data Dependencies	16

Assumptions	16

Business Defined Errors	20

Specific Database Needs	21

2.2	Day-Ahead Make-Whole Charge (LADAMWAMT)	21

Input Bill Determinants	22

Intermediate Bill Determinants	23

Output Bill Determinants	24

Data Dependencies	24

Assumptions	24

Business Defined Errors	26

Specific Database Needs	26

3.	Supplementary Requirements	26

3.1	Performance Requirements	26

3.2	Legal and Regulatory requirements	26

3.3	System Security Requirements	26

3.4	Back up and Recovery Requirements	27

3.5	Availability and Redundancy Requirements	27

3.6	Maintainability Requirements	27

3.7	Daylight Savings Time shall be recognized in Nodal Market settlements.	27

4.	Protocol Coverage	28

Software Requirements Specification

# Day-Ahead Market Settlement

## Proposed System Scope

ERCOT plans to implement a new Nodal market in 2009.  Sections 4, 7, and 9 of the Nodal Protocols describe the various Charge Types and Settlement Statement requirements for the Day-Ahead Market (DAM).  The Nodal Protocols will guide this project.

This document defines the requirements for DAM Make-Whole settlements.  The requirements in scope for this document include calculation of Charge Types that are

- detailed in Nodal Protocols Section 4 (006NPRR as of 06/07/2006), and

- listed as a component of the DAM Settlement Statement in Nodal Protocols Section 9 (May 5, 2006 version included in the May 2006 filing).

The following Charge Types are addressed in this document:

- Day-Ahead Make-Whole Payment (PR 4.6.2.3.1, PR 9.2.3(c))

- Day-Ahead Make-Whole Charge (PR 4.6.2.3.2, PR 9.2.3 (d))

To Be Sub-Process for Settlements and Billing

Figures 1.0 and 2.0 are provided for illustrative purposes.  Each figure illustrates the dependencies between input data and intermediate calculations for a Charge Type.  The figures should be considered in conjunction with the textual requirements.  If a contradiction exists between an illustrative figure and its associated textual requirements, the textual requirements should override the figure.

The legend below describes the levels used by the figures within this section:

Figure 1.0: Day-Ahead Make-Whole Payment Dependencies

Figure 2.0: Day-Ahead Make-Whole Charge Dependencies

## Document Conventions

- All text lifted from the Nodal Protocols is in italic font.

- All requirements are identified using FR1, FR2, etc.

- All dependencies are identified using D1, D2, etc.

- All assumptions are identified using A1, A2, etc.

- The original source system is defined for each input bill determinant.  It is important to note that the format defined for the input bill determinant is the format for which the data should exist in the Settlement System, after any interface functionality.  The bill determinant may or may not exist in the original source system in the format defined for the bill determinant.  A separate set of requirements will address data transformations and interfacing between the original source system and the Settlement System.

## Document Assumptions

- Day-Ahead Make-Whole Eligibility is documented in a separate set of requirements.

- Per the Nodal Protocols Section 4.5.3, ERCOT will communicate DAM Awards and Ancillary Service Capacity Awards and post public data (market clearing prices and ERCOT-wide quantities) to the MIS Public area no later than 1330 in the Day-Ahead.  Therefore, the aforementioned data should be available for interface process no later than 1330 in the Day-Ahead.

- All Day-Ahead Market Clearing Prices for Capacity (MCPCs) will be calculated outside of the Settlement System and therefore the SPP calculations are not detailed within this document.

- The original source system (or the relevant data archive) for the MCPCs will maintain a history for the prices.  Therefore, if an MCPC is corrected after a any settlement run (i.e. initial, final, etc.), the price that was originally used for the settlement run is still available.  Details surrounding the business process for price corrections and communications to the market are not included in this document.

- If ERCOT is unable to execute the Day-Ahead process, ERCOT may abort all or part of the Day-Ahead process and require all schedules and trades to be submitted in the Adjustment Period. (PR 4.1.2 (2))  If such an Emergency Condition occurs, Day-Ahead Make-Whole settlements may not be necessary.

- The term “channels” is used loosely within this document to represent the iterations of a settlement run (e.g. the iteration number for initial, final, true-up, and resettlement.)  The design process will define the “name” of the settlement run iterations (i.e. channels or something different) as well as how the settlement run iterations will work and be associated to the data within each settlement run.

- The charge type calculations are documented using the proposed recorder format for the bill determinants.  The following list defines the nomenclature used for the recorders (not all of these bill determinant recorder elements may apply to the charge types within this document):

<Q> = QSE

<CO> = CRR Owner

<R> = Resource

<SP> = Settlement Point

<SRSP> = Source Settlement Point

<SKSP> = Sink Settlement Point

<ST> = Start Type

<F> = Flowgate

<E> = Directional Network Element

<C> = Constraint Identifier

<M> = Market

<RUC> = RUC Process

# Functional Requirements

## Day-Ahead Make-Whole Payment (DAMWAMT)

The Day-Ahead Make-Whole Payment guarantees the QSE that the total payment received from the DAM for a DAM-committed Resource is not less than the total cost calculated based on the Startup Offer, the Minimum Energy Offer, and the Energy Offer Curve capped by the Energy Offer Curve Cap defined under Section 4.4.9.3.3, Energy Offer Curve Caps for Make-Whole Calculation Purposes. (PR 4.6.2.3 (2))

If a Generation Resource is eligible for a Day-Ahead Make-Whole payment, then Ancillary Service revenue from the hours committed in the Day-Ahead Market will be included in its Make-Whole calculation for that Resource.  (PR 4.6.2.3(3))

The Day-Ahead Make-Whole Payment compensates an eligible Generation Resource for the portion of its guaranteed operating costs that are greater than the Day-Ahead energy and ancillary service revenue for the commitment period, when that resource starts up to meet its DAM commitment.  Only non-RMR Resources that are deemed eligible (for startup and/or energy costs) can receive payment through the Day-Ahead Make Whole Payment.  A Day-Ahead Make-Whole Payment amount will be calculated, but not paid, for any eligible RMR Resource.  (PR 4.6.2.3.1(1))

| Requirement ID | FR1 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.1 (1), PR 4.6.2.3.1 (2), PR 4.6.2.3.1 (3) , PR 9.2.3 (c) |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: For each QSE, Resource, and Settlement Point with a DAMCOMMITFLAG for an Operating Day calculate the Day-Ahead Make-Whole Payment for each Operating Hour within a commitment period in the Operating Day. DAMWAMT_<Q>_<R>_<SP> h = (-1) * Max (0, DAMGCOST_<Q>_<R>_<SP> + ΣhDAEREV_<Q>_<R>_<SP>h + ΣhDAASREV_<Q>_<R>h) * DAESR_<Q>_<R>_<SP>h / (ΣhDAESR_<Q>_<R>_<SP>h) Where h represents the hours within the DAM commitment period, as determined by the DAMCOMMITFLAG. The DAMWAMT will be calculated separately for each commitment period. |  |

| Requirement ID | FR2 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.1 (3) |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate the Day-Ahead Market Guaranteed Cost by QSE, Resource, and Settlement Point, for the commitment period and Operating Day. DAMGCOST_<Q>_<R>_<SP> = (DAMWSUFLAG_<Q>_<R>_<SP> h * DASUO_<Q>_<R>_<SP>_<ST>_<M>) + Σh (DAMWENEFLAG_<Q>_<R>_<SP> h * DAMEO_<Q>_<R>_<SP>_<M>h * DALSL _<Q>_<R>_<SP>_<M>h) + Σh( DAMWENEFLAG_<Q>_<R>_<SP> h * DAAIEC_<Q>_<R>_<SP>h * (DAESR_<Q>_<R>_<SP> h - DALSL _<Q>_<R>_<SP>_<M>h)) Where h represents the hours within the DAM commitment period, as determined by the DAMCOMMITFLAG. In the DAMGCOST data cut, the value calculated for a commitment period will exist on the first hour of that commitment period. Where the market identifier <M> represents the Day-Ahead Market. |  |

| Requirement ID | FR3 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.1 (2), PR 4.6.2.3.1 (3) |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate the Day-Ahead Ancillary Service Revenue by QSE and Resource, for each Operating Hour of the Operating Day. DAASREV_<Q>_<R> = ((-1) * MCPCRU_<M> * PCRUR_<Q>_<R>_<M>) + ((-1) * MCPCRD_<M> * PCRDR_<Q>_<R>_<M>) + ((-1) * MCPCRR_<M> * PCRRR_<Q>_<R>_<M>) + ((-1) * MCPCNS_<M> * PCNSR_<Q>_<R>_<M>) Where the market identifier <M> represents the Day-Ahead Market. |  |

| Requirement ID | FR4 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.1 (3) (The parenthetical numbering in 4.6.2.3.1 needs to be corrected in an NPRR. This particular requirement should reference 4.6.2.3.1 (5) after the corrections are made.) |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate the Day-Ahead Make-Whole Payment QSE Total, for non-RMR units, by QSE and Operating Hour for the Operating Day. DAMWAMTQSETOT_<Q> = ΣSPΣR DAMWAMT_<Q>_<R>_<SP> |  |

| Requirement ID | FR5 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate the Day-Ahead Make-Whole Payment, for non-RMR units, Total by Operating Hour for the Operating Day. DAMWAMTTOT = ΣQΣSPΣR DAMWAMT_<Q>_<R>_<SP> |  |

| Requirement ID | FR6 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.1 (1) |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate a Day-Ahead Make-Whole Payment RMR Unit Offset for all RMR Units with a DAMWAMT data cut by Operating Hour for the Operating Day, using the DAMWAMT data cut. DAMWAMTRMR_<Q>_<R>_<SP> = (-1) * DAMWAMT_<Q>_<R>_<SP> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| DAMCOMMITFLAG | Misc | DAM-Committed Resource Flag per QSE and Resource by hour  Flag that indicates if an hourly interval was DAM-Committed for a QSE and Resource for the hour. A value of one (1) indicates that the interval is DAM-committed interval; a value of zero (0) indicates that the interval is not DAM-committed. | MMS |
| Interval Frequency: 1 / hour Additional Comments: For the Day-Ahead Make-Whole Payment calculation, this flag is utilized to identify the DAM-committed hours for which the calculation should occur. The DAMCOMMITFLAG will indicate the intervals in which a Resource was DAM-committed based upon an awarded Three Part Supply Offer. If an offer is cleared for multiple contiguous blocks of hours, and the cleared offers are DAM-committed, the entire block of hours is considered one commitment period. If an offer is cleared for multiple non-contiguous blocks of hours, and the cleared offers are DAM-committed, each block of hours is considered one commitment period. |  |  |  |
| DAMWSUFLAG | Misc | Day-Ahead Make-Whole Startup Eligibility Flag per QSE and Resource —The flag that indicates Day-Ahead Make-Whole Payment startup eligibility for the Resource represented by the QSE, for the hour. The value is set to zero (0) if the resource is not eligible for startup cost compensation or one (1) if the resource is eligible for startup cost compensation in the Day-Ahead Make-Whole payment, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: Only one hour within a commitment period should contain the value of one (1), if the Resource is eligible for startup cost compensation in the commitment period. See the TN.COMS.63C01.STARTUPELIGIBILITY.REQUIREMENTS.A business requirements document for details on how this flag will be created. |  |  |  |
| DAMWENEFLAG | Misc | Day-Ahead Make-Whole Energy Eligibility Flag per QSE and Resource —The flag that indicates Day-Ahead Make-Whole Payment energy eligibility for the Resource represented by the QSE, for the hour. The value is set to zero (0) if the resource is not eligible or one (1) if the resource is eligible for energy costs in the Day-Ahead Make-Whole payment, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: Every hour within the commitment period should contain a value of zero or one for the flag, and each hour should be utilized by the Day-Ahead Market Guaranteed Cost calculation. See the TN.COMS.63C01.STARTUPELIGIBILITY.REQUIREMENTS.A business requirements document for details on how this flag will be created. |  |  |  |
| DAESR | MW | Day-Ahead Energy Sale from Resource per QSE by Settlement Point per Resource by hourThe amount of energy cleared through Three-Part Supply Offers in the DAM for the Resource at the Resource Node Settlement Point represented by the QSE for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: There is an understood multiplier of 1 hour to obtain MWh from MW for the Day Ahead Market settlements. If an offer is cleared for multiple contiguous blocks of hours, and the cleared offers are DAM-committed, the entire block of hours is considered one commitment period. If an offer is cleared for multiple non-contiguous blocks of hours, and the cleared offers are DAM-committed, each block of hours is considered one commitment period. |  |  |  |
| DAEREV | $ | Day-Ahead Energy Revenue per QSE per Settlement Point per Resource by hourThe revenue received in the DAM for the Resource at the Resource Node Settlement Point represented by the QSE, based on the DAM Settlement Point Price, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This bill determinant is documented as a global bill determinant in the following requirements document: TN.COMS.63C01.DAMENERGY.REQUIREMENTS.A |  |  |  |
| DASUO | $/Start | Startup Offer per QSE per Settlement Point per ResourceThe Startup Offer included in the Three-Part Supply Offer associated with the Resource at the Resource Node Settlement Point represented by the QSE, representing an offer for all cost incurred by the Resource in starting up and reaching breaker close, for the first hour of the DAM-commitment period. | MMS |
| Interval Frequency: To be determined in design (This can be handled as interval or non-interval data) Additional Comments: This value should come from the Startup Offer captured in the DAM snapshot at 1000. If the DAM is delayed, this value should come from the Startup Offer captured at the point in time when the DAM is executed. For each Resource, only one startup cost is eligible per DAM Commitment. Therefore, the Startup Offer only applies to the first interval of the DAM commitment period. The DAMWAMT calculation spreads the Startup Offer cost across all hours of the commitment period. The <ST> startup type identifier for the DASUO data will contain one (1) for the hot Startup Offer, two (2) for an intermediate Startup Offer, or three (3) for a cold Startup Offer. The system will use the value from the STARTTYPE bill determinant that exists in the first hour of the commitment period to determine which Startup Offer (hot, intermediate, or cold) should be used in the DAMGCOST calculation. For example, if the STARTTYPE bill determinant indicates a hot startup (i.e. has a value of 1) in the first hour of the commitment period, the system will choose to use the hot Startup Offer data cut (i.e. the DASUO data cut with a value of 1 in the <ST> indicator.) If the STARTTYPE bill determinant indicates that no Startup Offer should be used because a startup did not occur (0) in the first hour of the commitment period, the system will use a zero value (0) in place of DASUO in the DAMGCOST calculation. |  |  |  |
| STARTTYPE | None | Start Type Flag – The flag that indicates the type of start (hot intermediate or cold) that a Resource incurred for the Resource, represented by the QSE for each hour for the Operating Day. The possible values include: zero (0) for none, one (1) for hot, two (2) for intermediate and three (3) for cold. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This determinant is not documented in the Nodal Protocols but is required to indicate what type of start the Resource performed. The startup type is determined using breaker status data. The STARTTYPE bill determinant will be an output from the eligibility process and will only be created for eligible Resources. See TN.COMS.63C01.STARTUPELIGIBILITY.REQUIREMENTS.A for further information on the STARTTYPE bill determinant. |  |  |  |
| DAMEO | $/MWh | Minimum-Energy Offer per QSE per Settlement Point per Resource per hour—The Minimum-Energy Offer included in the Three-Part Supply Offer associated with the Resource at the Resource Node Settlement Point represented by the QSE, for the hour. | MMS |
| Interval Frequency: To be determined in design (This can be handled as interval or non-interval data) Additional Comments: This value should come from the Minimum-Energy Offer captured in the DAM snapshot at 1000. If the DAM is delayed, this value should come from the Minimum Energy Offer captured at the point in time when the DAM is executed. |  |  |  |
| DALSL | MW | Low Sustained Limit per QSE per Settlement Point per Resource per hourThe Low Sustained Limit of the Resource at the Resource Node Settlement Point represented by the QSE, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: This is the LSL that is specified in the resource’s Current Operating Plan that was valid in the DAM 1000 snapshot. If the DAM is delayed, this value should come from the Startup Offer captured at the point in time when the DAM is executed. There is an understood multiplier of 1 hour to obtain MWh from MW for the Day Ahead Market settlements. |  |  |  |
| DAAIEC | $/MWh | Day-Ahead Average Incremental Energy Cost per QSE per Settlement Point per Resource per hour – The average incremental energy cost, calculated according to the energy offer curve capped by the generic energy price, for the output levels between they DAESR and the DALSL of the Resource at the Resource Node Settlement Point represented by the QSE, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: For additional information on Day-Ahead Average Incremental Energy Cost please refer to the TN.COMS.63C01.AIEC.REQUIREMENTS.A business requirements document. |  |  |  |
| PCRUR | MW | Procured Capacity for Reg-Up from Resource per Resource per QSE in DAM—The Reg-Up capacity quantity awarded to the QSE in the DAM for the Resource for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| MCPCRU | $/MW per hour | Market Clearing Price of Capacity for Reg-Up in DAM—The DAM Market Clearing Price of Capacity for Reg-Up for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| PCRDR | MW | Procured Capacity for Reg-Down from Resource per Resource per QSE in DAM—The Reg-Down capacity quantity awarded to the QSE in the DAM for the Resource for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| MCPCRD | $/MW per hour | Market Clearing Price of Capacity for Reg-Down in DAM—The DAM Market Clearing Price of Capacity for Reg-Down for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| PCRRR | MW | Procured Capacity for Responsive Reserve from Resource per Resource per QSE in DAM—The Responsive Reserve capacity quantity awarded to the QSE in the DAM for the Resource for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| MCPCRR | $/MW per hour | Market Clearing Price of Capacity for Responsive Reserve in DAM—The DAM Market Clearing Price of Capacity for Responsive Reserve for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| PCNSR | MW | Procured Capacity for Non-Spin from Resource per Resource per QSE in DAM—The Non-Spin capacity quantity awarded to the QSE in the DAM for the Resource for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| MCPCNS | $/MW per hour | Market Clearing Price of Capacity for Non-Spin in DAM—The DAM Market Clearing Price of Capacity for Non-Spin for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| DAMGCOST | $ | Day-Ahead Market Guaranteed Amount per QSE per Settlement Point per ResourceThe sum of the startup cost and the operating energy costs of the DAM-committed Resource at the Resource Node Settlement Point represented by the QSE, for the DAM-commitment period. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: DAMGCOST will exist in an hourly data cut. The DAMGCOST value calculated for a commitment period will exist only in the first hour of the commitment period. All other hourly intervals within the commitment period will contain a value of zero (0). |  |  |  |
| DAASREV | $ | Day-Ahead Ancillary Service Revenue per QSE per Resource by hourThe revenue received in the DAM for the Resource represented by the QSE, based on the Market Clearing Price of Capacity for each Ancillary Service in the DAM, for the hour h. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| DAMWAMT | $ | Day-Ahead Make-Whole Payment per QSE per Settlement Point per Resource per hourThe payment to the QSE to make-whole the Startup Cost and Energy Cost of the Resource committed in the DAM at the Resource Node Settlement Point for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: The amount is calculated for all resources (RMR and non-RMR.) |  |  |  |
| DAMWAMTQSETOT | $ | Day-Ahead Make-Whole Payment QSE Total per QSE - The total of the Day-Ahead Make-Whole Payments to the QSE for the DAM-committed non-RMR Generation Resources represented by this QSE for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: The QSE total amount is based upon payments calculated for non-RMR resources only. |  |  |  |
| DAMWAMTTOT | $ | Day-Ahead Make-Whole Payment Total - The total of the Day-Ahead Make-Whole Payments to all QSEs for all DAM-committed non-RMR Resources for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: The ERCOT total amount is based upon payments calculated for non-RMR resources only. |  |  |  |
| DAMWBILLAMT | $ | Day-Ahead Make-Whole Payment Bill Amount per QSE - The difference between two consecutive daily summed channels of DAMWAMT for the QSE for an Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. The BILLAMT methodology is currently under consideration with the Commercial Operations Subcommittee (COPS.) |  |  |  |
| DAMWAMTRMR | $ | Day-Ahead Make-Whole Payment RMR Unit Offset Amount per QSE per Settlement Point per RMR Unit per hour – The offset to the Day-Ahead Make-Whole Payment calculated for the RMR Unit for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: The DAMWAMTRMR data cut will offset a Day-Ahead Make-Whole Payment calculated for an eligible RMR Unit. This data cut should facilitate the determination of the total Day-Ahead Make-Whole Payment to the QSE on the Day-Ahead Market Settlement Statement. |  |  |  |

### Data Dependencies

D1)  The Day-Ahead Make-Whole Payment is dependent upon the availability of all data inputs.

D2)  The eligibility process (DAMWSUFLAG, DAMENEFLAG, and STARTTYPE) must be completed prior to attempting the Day-Ahead Make-Whole Payment.

D3)  DAAIEC must be calculated within the Settlement System prior to attempting the Day-Ahead Make-Whole Payment.

D4)  DAEREV must be calculated within the Settlement System prior to attempting the Day-Ahead Make-Whole Payment.

D5)  The Day-Ahead Make-Whole Charge is dependent upon the DAMWAMT output for RMR units.

D6)  The Day-Ahead Make-Whole Charge is dependent upon the DAMWAMTTOT output bill determinant.

### Assumptions

A1)  The Settlement System is not responsible for any validation of any component of the Three Part Supply Offer; validation of the offers will occur within the original source system.

A2) A DAMCOMMITFLAG data cut will be created for all resources that were DAM-committed in at least one hourly interval.  This data cut will be created with a zero (0) or a one (1) value in every Operating Hour.  When creating the DAMCOMMITFLAG data cut for a QSE, Resource, Settlement Point, and Operating Day the interface will substitute a 0 value for a null value.

A3)  The STARTTYPE bill determinant will be an output from the eligibility process created for Day-Ahead or RUC eligible Resources.

A4)  When creating the DAESR data cut for a QSE, Resource, Settlement Point, and Operating Day the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity (i.e. has a cleared Offer) for that Resource.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create a DAESR data cut for that Resource.

A5)  The Day-Ahead snapshot of the Startup Offer (DASUO) data will exist for each eligible Resource, for each type of startup (Hot, Intermediate, and Cold.)  A hot Startup Offer will be indicated by a 1 in the <ST> startup type indicator; an intermediate Startup Offer will be indicated by a 2 in the <ST> startup type indicator, a cold Startup Offer will be indicated by a 3 in the <ST> startup type indicator.

A6)  Section 4.4.9.2.1 (1), Startup Offer and Minimum-Energy Offer Criteria, does not include criteria for specifying the first and last hour of the Offer as does Section 4.4.9.3.1 (1), Energy Offer Curve Criteria.  It is assumed that a QSE can submit multiple Startup Offers and Minimum Energy Offers that are valid for different sets of Operating Hours and that the first and last hour of the Offer specified for the Energy Offer Curve also applies to any associated Startup Offer and Minimum-Energy Offer.  If this assumption is valid, it may be prudent to change section 4.4.9.2.1 (1) of the Protocols to be more specific and include an additional criterion for the first and last hour of the Offer.

A7)  When creating the DALSL data cut for a QSE, Resource, and Operating Day, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Resource.  If there are no intervals with a non-zero quantity for the entire Operating Day and for the DAM, then the interface will not create a DALSL data cut for that QSE, Resource, Operating Day, and DAM.

A8)  The Settlement System will calculate DAAIEC and create an hourly data cut for each Resource with a DAMCOMMITFLAG for an Operating Day.  When creating the DAAIEC data cut for a QSE, Resource, Settlement Point, and Operating Day the Settlement System will substitute a 0 value for a null value only if at least one interval within the Operating Day has a DAMCOMMITFLAG for that Resource.  If there are no intervals in the Operating Day with a DAMCOMMITFLAG for that Resource, the interface will not create a DAAIEC data cut for the Resource.

A9)  The Settlement System will create a DAMWSUFLAG hourly data cut for every QSE, Resource, Settlement Point, and Operating Day with a DAMCOMMITFLAG data cut.  The Settlement System will create a DAMWENEFLAG hourly data cut for every QSE, Resource, Settlement Point, and Operating Day with a DAMCOMMITFLAG data cut.

A10)  When creating the PCRUR data cut for a QSE, Resource, Operating Day, and DAM, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Resource.  If there are no intervals with a non-zero quantity for the entire Operating Day and for the DAM, then the interface will not create a PCRUR data cut for that QSE, Resource, Operating Day, and DAM.

A11)  When creating the PCRDR data cut for a QSE, Resource, Operating Day, and DAM, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Resource.  If there are no intervals with a non-zero quantity for the entire Operating Day and for the DAM, then the interface will not create a PCRDR data cut for that QSE, Resource, Operating Day, and DAM.

A12)  When creating the PCRRR data cut for a QSE, Resource, Operating Day, and DAM, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Resource.  If there are no intervals with a non-zero quantity for the entire Operating Day and for the DAM, then the interface will not create a PCRRR data cut for that QSE, Resource, Operating Day, and DAM.

A13)  When creating the PCNSR data cut for a QSE, Resource, Operating Day, and DAM, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Resource.  If there are no intervals with a non-zero quantity for the entire Operating Day and for the DAM, then the interface will not create a PCNSR data cut for that QSE, Resource, Operating Day, and DAM.

A14) When creating the MCPCRU data cut for the DAM for an Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given hourly interval of the MCPCRU data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of MCPCRU data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no MCPCRU data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A15) When creating the MCPCRD data cut for the DAM for an Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given hourly interval of the MCPCRD data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of MCPCRD data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no MCPCRD data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A16) When creating the MCPCRR data cut for the DAM for an Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given hourly interval of the MCPCRR data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of MCPCRR data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no MCPCRR data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A17) When creating the MCPCNS data cut for the DAM for an Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given hourly interval of the MCPCNS data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of MCPCNS data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no MCPCNS data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A18) The business requirements for the Settlement Statements will define what will be included on the Day-Ahead Settlement Statement.  The intent is that RMR units will not be paid for Make-Whole in the Day-Ahead, but QSEs will be charged in the Day-Ahead for Make-Whole amounts for both RMR and non-RMR Units.  The Day-Ahead Make-Whole Payment RMR Offset (DAMWAMTRMR) exists to facilitate generation of the Day-Ahead Settlement Statement.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR7 | PR 4.6.2.3.1 (3), PR 4.6.2.3.1 (5) | Will be completed August 31, 2006 | None of the input bill determinants should be rounded or truncated. |
| FR8 | PR 4.6.2.3.1 (3), PR 4.6.2.3.1 (5) | Will be completed August 31, 2006 | None of the intermediate bill determinants should be rounded or truncated |
| FR9 | PR 4.6.2.3.1 (3), PR 4.6.2.3.1 (5) | Will be completed August 31, 2006 | The output bill determinants should be rounded to two (2) decimal places |
| FR10 | PR 12.3 (C) | Will be completed August 31, 2006 | The following QSE specific bill determinants should be identified within the settlement system as necessary for the private data extracts: - DAMCOMMITFLAG - DAMWSUFLAG - DAMWENEFLAG - STARTTYPE - DAMGCOST - DAEREV - DAASREV - DAESR - DASUO - DAMEO - DALSL - DAAIEC - PCRUR - PCRDR - PCRRR - PCNSR - DAMWAMT - DAMWAMTQSETOT - DAMWAMTRMR (for RMR Units only) |
| FR11 | PR 12.3 (C) | Will be completed August 31, 2006 | The following ERCOT-wide bill determinants should be identified within the settlement system as necessary for the public data extracts: - MCPCRU - MCPCRD - MCPCRR - MCPCNS - DAMWAMTTOT |
| FR12 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | DRIVER: The Day-Ahead Make-Whole Payment calculation should only be initiated for a QSE, Resource, Settlement Point, and Operating Day with a DAMCOMMITFLAG data cut. |
| FR13 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | The DAMWAMTRMR data cut will be created by QSE, Resource, Settlement Point, and Operating Hour for all RMR Units with a DAMWAMT data cut for the Operating Day. |
| FR14 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | The DAMGCOST data cut created for a QSE, Resource, and Settlement Point for an Operating Day will populate the first hour of a commitment period with the DAMGCOST value calculated for the commitment period. All other hourly intervals within the commitment period will contain a value of zero (0). |
| FR15 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | When performing the DAMWAMT calculation for a QSE, Resource, and Settlement Point for a commitment period in the Operating Day, the calculation shall use the DAMGCOST value that exists on the first hour of the commitment period. |
| FR16 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | When performing the DAMWAMT calculation for a QSE, Resource, and Settlement Point for a commitment period in the Operating Day, the calculation shall use the DASUO value that is valid for the first hour of the commitment period. |
| FR17 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | When performing the DAMWAMT calculation for a QSE, Resource, and Settlement Point for a commitment period in the Operating Day, the calculation shall use the DAMEO value that is valid for the each Operating Hour within the commitment period. |
| FR18 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | The system will use the STARTTYPE bill determinant to determine which Startup Offer (i.e. none, hot, intermediate, or cold) should be used to calculate the Day-Ahead Make-Whole Payment for a QSE, Resource, Operating Day, and Operating Hour. |
| FR19 | PR 4.6.2.3.1 (5) | Will be completed August 31, 2006 | DAMWAMTQSETOT is calculated by Operating Hour for a QSE and Operating Day using the DAMWAMT payments for non-RMR units only. |
| FR20 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | DAMWAMTTOT is calculated by Operating Hour for an Operating Day using the DAMWAMT payments for non-RMR units only. |
| FR21 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If PCRUR is not available for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists with a value of 1 a default value of zero (0) should be used in place of PCRUR. No error or warning message is necessary. |
| FR22 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If PCRDR is not available for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists with a value of 1 a default value of zero (0) should be used in place of PCRUR. No error or warning message is necessary. |
| FR23 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If PCRRR is not available for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists with a value of 1 a default value of zero (0) should be used in place of PCRUR. No error or warning message is necessary. |
| FR24 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If PCNSR is not available for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists with a value of 1 a default value of zero (0) should be used in place of PCRUR. No error or warning message is necessary. |
| FR25 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | The DAMWBILLAMT should be calculated as the difference between two consecutive daily summed channels of DAMWAMT for the QSE for an Operating Day. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR26 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If DAEREV is missing for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists, a “WARN” error should occur. The system should skip processing DAMWAMT for that QSE, Resource, Settlement Point, and Operating Day and should log a warning message that indicates the missing data element, QSE, Resource, Settlement Point, and Operating Day. |
| FR27 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If DASUO is missing for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists, a “WARN” error should occur. The system should skip processing DAMWAMT for that QSE, Resource, Settlement Point, and Operating Day and should log a warning message that indicates the missing data element, QSE, Resource, Settlement Point, and Operating Day. |
| FR28 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If DAMEO is missing for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists 1, a “WARN” error should occur. The system should skip processing DAMWAMT for that QSE, Resource, Settlement Point, and Operating Day and should log a warning message that indicates the missing data element, QSE, Resource, Settlement Point, and Operating Day. |
| FR29 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If DALSL is missing for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists, a “WARN-Default” error should occur. The system should substitute a zero value for the missing DALSL value and should log a warning message that indicates the missing data element, QSE, Resource, Settlement Point, and Operating Day. |
| FR30 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If DAAIEC is missing for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists, a “WARN” error should occur. The system should skip processing DAMWAMT for that QSE, Resource, Settlement Point, and Operating Day and should log a warning message that indicates the missing data element, QSE, Resource, Settlement Point, and Operating Day. |
| FR31 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If DAESR is missing for a QSE, Resource, Settlement Point, and Operating Day for which a DAMCOMMITFLAG exists, a “WARN” error should occur. The system should skip processing DAMWAMT for that QSE, Resource, Settlement Point, and Operating Day and should log a warning message that indicates the missing data element, QSE, Resource, Settlement Point, and Operating Day. |
| FR32 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If MCPCRU is missing for that Operating Day and at least one DAM-committed Resource has a non-zero PCRUR value in that Operating Day, the system should log a WARN error message and stop processing DAMWAMT for that Operating Day. |
| FR33 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If MCPCRD is missing for that Operating Day and at least one DAM-committed Resource has a non-zero PCRDR value in that Operating Day,, the system should log a WARN error message and stop processing DAMWAMT for that QSE, Resource, and Operating Day. |
| FR34 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If MCPCRR is missing for that Operating Day and at least one DAM-committed Resource has a non-zero PCRRR value in that Operating Day, the system should log a WARN error message and stop processing DAMWAMT for that QSE, Resource, and Operating Day. |
| FR35 | PR 4.6.2.3.1 (3) | Will be completed August 31, 2006 | If MCPCNS is missing for that Operating Day and at least one DAM-committed Resource has a non-zero PCRRR value in that Operating Day, the system should log a WARN error message and stop processing DAMWAMT for that QSE, Resource, and Operating Day. |

### Specific Database Needs

None

## Day-Ahead Make-Whole Charge (LADAMWAMT)

ERCOT shall charge a Day-Ahead Make-Whole Charge to each QSE that has one or more cleared DAM Energy Bids and/or PTP Obligation Bids.  The Day-Ahead Make-Whole Charge for an hour is that QSE’s prorata share of the total amount of Make-Whole Payments for that hour. The proration must be based on the ratio of the energy amount of the QSE’s cleared DAM Energy Bids and PTP Obligation Bids to the total energy amount of all QSEs’ cleared DAM Energy Bids and PTP Obligation Bids. (PR 4.6.2.3.2)

The Day-Ahead Make-Whole Payment, for RMR and non-RMR units, is uplifted to QSE’s with DAM cleared Energy Bids and/or DAM cleared Real-Time Obligation Bids via the Day-Ahead Make Whole Charge.  The RMR units are not actually paid through the Day-Ahead Make-Whole Payment; however, the Day-Ahead Make-Whole Charge includes a component for Day-Ahead Make-Whole Payment to RMR units.  The RMR Day-Ahead Make-Whole Revenue Total component is included in the Day-Ahead Make-Whole Charge as a mechanism to collect money to be paid to the RMR resources in Real-Time settlements.

| Requirement ID | FR36 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.2, PR 9.2.3 (d) |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate the Day-Ahead Make-Whole Charge by QSE and Operating Hour for the Operating Day: LADAMWAMT_<Q> = (-1) * (DAMWAMTTOT + RMRDAMWREVTOT) * DAERS _<Q> |  |

| Requirement ID | FR37 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate the RMR Day-Ahead Make-Whole Revenue Total for the Operating Day by summing the DAMWAMT data cuts for RMR Units: RMRDAMWREVTOT = ΣQΣPΣR DAMWAMT_<Q>_<R>_<SP> |  |

| Requirement ID | FR38 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate the Day-Ahead Energy Purchase Ratio Share by QSE and Operating Hour for the Operating Day: DAERS_<Q> = DAE_<Q> / DAETOT |  |

| Requirement ID | FR39 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate the Day-Ahead Energy Total by Operating Hour for the Operating Day: DAETOT = ΣQDAE_<Q> |  |

| Requirement ID | FR40 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.2.3.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description: Calculate Day-Ahead Energy by QSE and Operating Hour for the Operating Day: DAE_<Q> = ΣSPDAEP_<Q>_<SP> + ΣSRSPΣSKSPRTOBL_<Q>_<SRSP>_<SKSP> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| DAMWAMTTOT | $ | Day-Ahead Make-Whole Payment Total - The total of the Day-Ahead Make-Whole Payments to all QSEs for all DAM-committed non-RMR Resources for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This is an output from the Day-Ahead Make-Whole Payment calculation. |  |  |  |
| DAMWAMT | $ | Day-Ahead Make-Whole Payment per QSE per Settlement Point per Resource per hourThe payment to the QSE to make-whole the Startup Cost and Energy Cost of the Resource committed in the DAM at the Resource Node Settlement Point for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: The LADAMWAMT only requires DAMWAMT calculated for RMR Units. DAMWAMT is an output from the Day-Ahead Make-Whole Payment calculation and is used by this process to calculate the RMRDAMWREVTOT. |  |  |  |
| DAEP | MW | Day-Ahead Energy Purchase per QSE per Settlement Point—The total amount of energy represented by the QSE’s cleared DAM Energy Bids at the Settlement Point for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The DAEP value should represent the sum of all of the QSE’s cleared energy purchases at the Settlement Point for the Operating Day and Operating hour. There is an understood multiplier of 1 hour to obtain MWh from MW for the Day Ahead Market settlements. |  |  |  |
| RTOBL | MW | Real-Time Obligation per QSE per pair of source and sink—The total amount of energy represented by the QSE’s cleared PTP Obligation Bids for the source and the sink, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: There is an understood multiplier of 1 hour to obtain MWh from MW for the Day Ahead Market settlements. |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RMRDAMWREVTOT | $ | RMR Day-Ahead Make-Whole Revenue Total - The total of the RMR Day-Ahead Make-Whole Revenue for all DAM-committed RMR Units for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This is an input into LADAMWAMT. |  |  |  |
| DAERS | none | Day-Ahead Energy Purchase Ratio Share per QSE - The ratio of the QSE’s total amount of energy represented by its cleared DAM Energy Bids and PTP Obligation Bids, to the total amount of energy represented by all QSEs’ cleared DAM Energy Bids and PTP Obligation Bids, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This is an input into the LADAMWAMT calculation. |  |  |  |
| DAETOT | MW | Day-Ahead Energy Total—The total amount of energy represented by all cleared DAM Energy Bids and all cleared PTP Obligation Bids for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This is an input into the DAERS calculation. |  |  |  |
| DAE | MW | Day-Ahead Energy per QSE – The QSE’s total amount of energy, represented by its cleared DAM Energy Bids and PTP Obligation Bids, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This is an input into the DAETOT and DAERS calculations. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| LADAMWAMT | $ | Day-Ahead Make-Whole ChargeThe allocated charge to the QSE to make whole all the eligible DAM-committed Resources for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| LADAMWBILLAMT | $ | Day-Ahead Make-Whole Charge Bill Amount - The difference between two consecutive daily summed channels of LADAMWAMT for the QSE for the Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. The BILLAMT methodology is currently under consideration with the Commercial Operations Subcommittee (COPS.) |  |  |  |

### Data Dependencies

D1)  The Day-Ahead Make-Whole Charge is dependent upon the availability of all data inputs.

D2)  The Day-Ahead Make-Whole Charge is dependent upon calculation of the Day-Ahead Make-Whole Payment, specifically the DAMWAMT and DAMWAMTTOT bill determinants.

### Assumptions

A1)  The Settlement System is not responsible for any validation of any component of the Energy Bids; validation of the bids will occur within the original source system.

A2)  The Settlement System is not responsible for any validation of any component of the PTP Obligation Bids; validation of the bids will occur within the original source system.

A3)  When creating the DAEP data cut for a Settlement Point, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Settlement Point.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create a DAEP data cut for that Settlement Point.

A4)  When creating the RTOBL data cut for a source and sink Settlement Point pair, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that source and sink Settlement Point pair.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create a RTOBL data cut for that source and sink Settlement Point pair.

A5)  DAM commitments cannot exist in an Operating Hour without one or more cleared Energy Bid or RT Obligation (i.e. PTP Obligation acquired in the DAM) in the same Operating Hour.  Therefore, the following scenario should not occur: one or more Day-Ahead Make-Whole Payments are calculated for an Operating Hour, but no “purchases” exist for that Operating Hour and therefore nothing exists to be used as a basis for uplift of the Day-Ahead Make-Whole Payment.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR41 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | None of the input bill determinants should be rounded or truncated. |
| FR42 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | None of the intermediate bill determinants should be rounded or truncated. |
| FR43 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | The output bill determinants should be rounded to two (2) decimal places. |
| FR44 | PR 12.3 (C) | Will be completed August 31, 2006 | The following QSE specific bill determinants should be identified within the settlement system as necessary for the private data extracts: - RTOBL - DAEP - DAE - DAERS - DAMWAMT - LADAMWAMT |
| FR45 | PR 12.3 (C) | Will be completed August 31, 2006 | The following ERCOT-wide bill determinant should be identified within the settlement system as necessary for the public data extracts: - DAMWAMTTOT - RMRDAMWREVTOT - DAETOT |
| FR46 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | DRIVER: The Day-Ahead Make-Whole Charge calculation should be performed for an Operating Day when at least one QSE has a non-zero DAEP or RTOBL value for at least one hourly interval within the Operating Day. |
| FR47 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | If a QSE has at least one DAEP data cut for an Operating Day but does not have any RTOBL data cuts for that Operating Day, the system should assume a default value of zero for RTOBL in the DAE calculation for that QSE and Operating Day. |
| FR48 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | If a QSE has at least one RTOBL data cut for an Operating Day but does not have any DAEP data cuts for that Operating Day, the system should assume a default value of zero for DAEP in the DAE calculation for that QSE and Operating Day. |
| FR49 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | The system should calculate LADAMWBILLAMT as the difference between two consecutive daily summed channels of LADAMWAMT for the QSE for the Operating Day. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR50 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | If a DAMWAMTTOT data cut does not exist for the Operating Day, the system should assume a value of zero for each hourly interval, but should log a WARN error. The error message should state the Operating Day and missing data element. |
| FR51 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | If DAETOT calculates to zero (0) for an hourly interval within an Operating Day, the system should set the DAERS value equal to zero (0) for all QSE’s for which DAE was calculated for the Operating Day and hourly interval, rather than attempt to calculate the DAERS value for each of the QSEs. This substitution will avoid a division by zero error. |
| FR52 | PR 4.6.2.3.2 | Will be completed August 31, 2006 | If the DAMWAMTTOT value is not equal to zero (0) for an Operating Hour where the DAETOT value is zero (0), the system should log a WARN error message that states the Operating Day and Operating Hour and indicates that DAMWAMTTOT was not uplifted due to a zero-value DAETOT. |

### Specific Database Needs

None

# Supplementary Requirements

## Performance Requirements

Lodestar must be able to perform its full set of Nodal settlement calculations. Performance characteristics will be determined and detailed in the conceptual and detailed design.

## Legal and Regulatory requirements

All Lodestar financial data must be saved in accordance with standard accounting principles as dictated by the ERCOT Texas Nodal Protocols.

## System Security Requirements

Lodestar will only be accessed directly by internal ERCOT personnel either through thin client UI, Citrix environment, or thick client on employee desktops. Market Participants will be able to access their statements and invoices on ERCOT private web site. ERCOT private web site access will be controlled by digital certificate authentication in accordance with ERCOT security policies.  Additional details will be provided in the Conceptual System Design.

## Back up and Recovery Requirements

All Lodestar Nodal settlement data must be backed up on a daily basis and recoverable within 24 hours.  Additional details will be provided in the Conceptual System Design

## Availability and Redundancy Requirements

Nodal Lodestar system must be available 7/24. Lodestar is considered a tier 1 system and critical to base ERCOT business processes.  Additional details will be provided in the Conceptual System Design

## Maintainability Requirements

Lodestar Nodal system will be maintained by internal ERCOT personnel. SLAs and explicit support will be detailed in the detail design document.  Additional details will be provided in the Conceptual System Design

## Daylight Savings Time shall be recognized in Nodal Market settlements.

3.7.1 Spring Daylight Savings Time Operating Day:

  - Hour Ending 0300 will be skipped, resulting in 23 hourly intervals for the day.

  - The four fifteen minute intervals within Hour Ending 0300 will be skipped, resulting in 92 fifteen minute intervals for the day.

  - The sixty one minute intervals within Hour Ending 0300 will be skipped, resulting in 1380 total one minute intervals.

3.7.2 Fall Daylight Savings Time Operating Day:

  - Hour Ending 0200 will have two sets of data, resulting in 25 hourly intervals for the day.

  - The four fifteen minute intervals within Hour Ending 0200 will have two sets of data, resulting in 100 fifteen minute intervals for the day.

  - The sixty one minute intervals within Hour Ending 0200 will have two sets of data, resulting in 1500 total one minute intervals.

# Protocol Coverage

Since one protocol may translate into many requirements, Project Managers are required to identify the coverage extent (Full/Partial) of all relevant Nodal Protocols that are mentioned in this requirements document.

| Protocol Sub-Section # (To the lowest level of granularity as possible) | Coverage by Requirements (Full/Partial) |
| --- | --- |
| 4.6.2.3.1 (1) | Partial |
| 4.6.2.3.1 (2) | Partial |
| 4.6.2.3.1 (3) | Partial |
| 4.6.2.3.2 | Partial |
| 9.2.3 (c) | Partial |
| 9.2.3 (d) | Partial |