Commercial Systems

Real-Time Emergency Operations Settlements Requirements

Version 0.09

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 08/22/2006 | 0.01 | Initial Draft | Mandy Bauld |
| 08/23/2006 | 0.02 | Includes changes from review meeting | Mandy Bauld |
| 08/30/2006 | 0.09 | Incorporated IDA Feedback | Mandy Bauld |

Table of Contents

1.	Real-Time Emergency Operations Settlement	4

1.1	Proposed System Scope	4

1.2	Document Conventions	5

1.3	Document Assumptions	5

2.	Functional Requirements	6

2.1	Payment for Emergency Power Increase Directed by ERCOT (EMREAMT)	6

Input Bill Determinants	8

Intermediate Bill Determinants	9

Output Bill Determinants	9

Data Dependencies	10

Assumptions	11

Business Defined Errors	13

Specific Database Needs	13

2.2	Charge for Emergency Power Increases (LAEMREAMT)	14

Input Bill Determinants	14

Output Bill Determinants	14

Data Dependencies	15

Assumptions	15

Business Defined Errors	16

Specific Database Needs	16

3.	Supplementary Requirements	17

3.1	Performance Requirements	17

3.2	Legal and Regulatory requirements	17

3.3	System Security Requirements	17

3.4	Back up and Recovery Requirements	17

3.5	Availability and Redundancy Requirements	17

3.6	Maintainability Requirements	17

3.7	Daylight Savings Time shall be recognized in Nodal Market settlements.	17

4.	Protocol Coverage	18

Software Requirements Specification

# Real-Time Emergency Operations Settlement

## Proposed System Scope

ERCOT plans to implement a new Nodal market in 2009.  Sections 6, 7 and 9 of the Nodal Protocols describe the various Charge Types and Settlement Statement requirements for the Real-Time Operations (RT).  The Nodal Protocols will guide this project.

This document defines the requirements for Real-Time Emergency Operations settlements.  The requirements in scope for this document include calculation of Emergency Operations-related RT settlement Charge Types that are

- detailed in Nodal Protocols Section 6 (NPRR 009 approved by TAC on 08/03/2006), and

- listed as a component of the RT Settlement Statement in Nodal Protocols Section 9 (NPRR 007 approved by the Board on 07/18/2006).

The following Charge Types are addressed in this document:

- Payment for Emergency Power Increase directed by ERCOT (PR 6.6.9.1,  PR 9.5.3 (y))

- Charge for Emergency Power Increases (PR 6.6.9.2,  PR 9.5.3 (z))

The Figures provided within this document are for illustrative purposes.  The figures should be considered in conjunction with the textual requirements.  If a contradiction exists between an illustrative figure and its associated textual requirements, the textual requirements should override the figure.

## Document Conventions

- All text lifted from the Nodal Protocols is in italic font.

- All dependencies are identified using D1, D2, etc.

- All assumptions are identified using A1, A2, etc.

- The original source system is defined for each input bill determinant.  It is important to note that the format defined for the input bill determinant is the format for which the data should exist in the Settlement System, after any interface functionality.  The bill determinant may or may not exist in the original source system in the format defined for the bill determinant.  A separate set of business requirements will address data transformations and interfacing between the original source system and the Settlement System.

## Document Assumptions

- All Real-Time Settlement Point Prices (RTSPP) will be calculated outside of the Settlement System and therefore the RTSPP calculations are not detailed within this document.

- The original source system for the Settlement Point Prices, or the relevant data archive, will maintain a history of the prices.  Therefore, if a RTSPP is corrected after a settlement run, the price that was used for the settlement run is still available.

- Any charge type that requires historical data will obtain this information via alternative means during the beginning of market open until the data is available.

- The charge type calculations are documented using the proposed recorder format for the bill determinants.  The following list defines the nomenclature used for the recorders (not all of these bill determinant recorder elements may apply to the charge types within this document):

<Q> = QSE

<CO> = CRR Owner

<R> = Resource

<SP> = Settlement Point

<BLTP> = Block Load Transfer Point

<SRSP> = Source Settlement Point

<SKSP> = Sink Settlement Point

<ST> = Start Type

<F> = Flowgate

<E> = Directional Network Element

<C> = Constraint Identifier

<M> = Market

<RUC> = RUC Process

<BLTP> = Block Load Transfer Settlement Point

# Functional Requirements

## Payment for Emergency Power Increase Directed by ERCOT (EMREAMT)

Due to Emergency Conditions, additional compensation for each Generation Resource for which ERCOT provides an Emergency Base Point may be awarded to the QSE representing the Generation Resource. If the Emergency Base Point is higher than the SCED Base Point immediately before the Emergency Condition and the Settlement Point Price at the Resource Node is lower than the Generation Resource’s Energy Offer Curve price at the Emergency Base Point, ERCOT shall pay the QSE additional compensation for the additional energy above the SCED Base Point. (PR 6.6.9)

An NPRR change is proposed to modify the Payment for Emergency Power Increase Directed by ERCOT Charge Type.  The modification to the Charge Type does not change the end result of the calculation.  The modification creates a new bill determinant, Aggregated Emergency Base Point (AEBP), to represent a sub-calculation within the Emergency Energy (EMRE) calculation.  The calculation of AEBP, a 15-minute interval value, will be performed using SCED level data.  With this new variable available as a data input into EMRE (i.e., the value that is used as the quantity for the Emergency Energy Payment), EMRE can be calculated using 15-minute data only.  Elimination of SCED level sub-calculations within a 15-minute calculation allows for better implementation of this Charge Type and is consistent with how SCED data is incorporated into other Charge Types within Real-Time settlements, such as the Base Point Deviation Charge Type.

Agree and support the proposed NPRR.

Will this calculation only be done when an Emergency base point is issued to individual resources for purposes of reliability?  Will it only be done when ERCOT issues an Emergency Notice for failure of the SCED process?  Will it be done if either of the two preceding conditions is true? Note – TXU believes that this provision is applicable whenever ERCOT issues an Emergency Base-point in accordance with the Protocols. On the other hand, will the calculation always be done and yield a non-zero result only if an emergence base point was issued?

The highlighted text below represents the proposed NPRR changes:

EMREAMT q, p, r	=	(-1) * EMREPR q, p, r * EMRE q, p, r

Where:

EMREPR q, p, r		=	Max (0, EBPWAPR q, p, r – RTSPP p)

EBPWAPR q, p, r		=	Σy(EBPPR q, p, r, y * EBP q, p, r, y * TLMP y) /

Σy(EBPq, p, r, y * TLMP y)

EMRE q, p, r	=	Max (0, Min (AEBP q, p, r * ¼ , RTMG q, p, r) – ¼ * BP q, p, r)

AEBP q, p, r	=	Σy(EBP q, p, r, y * TLMPy / 3600)

| Variable | Unit | Definition |
| --- | --- | --- |
| AEBP q, p, r | MW | Aggregated Emergency Base Point— The Generation Resource’s aggregated Emergency Base Point, for the 15-minute Settlement Interval. |

The following business requirements for the calculation of EMREAMT assume acceptance of the proposed NPRR changes:

| Requirement ID | FR 1 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.9.1 (1) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the Emergency Energy Amount for a QSE, Settlement Point, Resource, Operating Day, and 15-minute interval. EMREAMT_<Q>_<R>_<SP> = (-1) * EMREPR_<Q>_<R>_<SP>* EMRE_<Q>_<R>_<SP> |  |

| Requirement ID | FR 2 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.9.1 (1) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the Emergency Energy Price for a QSE, Settlement Point, Resource, Operating Day, and 15-minute interval. EMREPR_<Q>_<R>_<SP> = Max (0, EBPWAPR_<Q>_<R>_<SP> – RTSPP_<SP>) |  |

| Requirement ID | FR 3 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.9.1 (1) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the Emergency Energy for a QSE, Settlement Point, and Resource for the Operating Day and 15-minute interval. EMRE_<Q>_<R>_<SP> = Max (0, Min (AEBP_<Q>_<R>_<SP> , RTMG_<Q>_<R>_<SP>) – ¼ * BP_<Q>_<R>_<SP>) |  |

| Requirement ID | FR 4 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.9.1 (2) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the Emergency Energy Amount QSE Total for a QSE by Operating Day and 15-minute interval. EMREAMTQSETOT_<Q> = ΣSPΣREMREAMT_<Q>_<R>_<SP> |  |

| Requirement ID | FR 5 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.9.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the Emergency Energy Amount Total by Operating Day and 15-minute interval. EMREAMTTOT = ΣQEMREAMTQSETOT_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point Price per Settlement Point - The Real-Time Settlement Point Price at the Settlement Point, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| EBPWAPR | $/MWh | Emergency Base Point Weighted Average Price per QSE per Settlement Point per Resource—The weighted average of the energy prices corresponding with the Emergency Base Points on the Energy Offer Curve for the Resource at the Resource Node Settlement Point represented by the QSE, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: The MMS calculation of EBPWAPR needs adhere to the EBPWAPR calculation as documented in PR 6.6.9.1 (1). An MMS requirements document will address the calculation of EBPWAPR. |  |  |  |
| AEBP | MW | Aggregated Emergency Base Point— The Generation Resource’s aggregated Emergency Base Point, for the 15-minute Settlement Interval. | EMS |
| Interval Frequency: 4 / hour Additional Comments: The EMS calculation of AEBP needs adhere to the AEBP calculation as documented in the proposed NPRR changes for PR 6.6.9.1 (1). An EMS requirements document will address the calculation of AEBP. |  |  |  |
| RTMG | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource - The metered generation of the Resource at the Resource Node Settlement Point represented by the QSE in Real-Time for the 15-minute Settlement Interval. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: The RTMG value will not include any Block Load Transfer quantity. |  |  |  |
| BP | MW | Base Point per QSE per Settlement Point per Resource—The Base Point of the Resource at the Resource Node Settlement Point represented by the QSE from the SCED prior to the Emergency Condition. | EMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| EMRE | MWh | Emergency Energy per QSE per Settlement Point per Resource—The additional energy produced by the Generation Resource at the Resource Node Settlement Point represented by the QSE in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| EMREPR | $/MWh | Emergency Energy Price per QSE per Settlement Point per Resource —The compensation rate for the additional energy produced by the Generation Resource at the Resource Node Settlement Point represented by the QSE in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| EMREAMT | $ | Emergency Energy Amount per QSE per Settlement Point per Resource—The payment to the QSE as additional compensation for the additional energy produced by the Generation Resource at the Resource Node Settlement Point in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| EMREAMTQSETOT | $ | Emergency Energy Amount QSE Total per QSEThe total of the payments to the QSE as additional compensation for emergency power increases of the non-RMR Generation Resources represented by this QSE for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: An NPRR change is necessary to remove the text “non-RMR” from the EMREAMTQSETOT definition in 6.6.9.1 (2). |  |  |  |
| EMREAMTTOT | $ | Emergency Energy Amount TotalThe total of the payments to all QSEs as additional compensation for emergency power increases of the Generation Resources for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| EMREBILLAMT |  | Emergency Energy Amount per QSE - The difference between two consecutive daily summed channels of the total EMREAMT for the QSE and for the Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1) Calculation of the EMREAMT is dependent upon availability of all of the input bill determinants.

D2) Calculation of the Load-Allocated Emergency Energy Amount per QSE (LAEMREAMT) is dependent upon the calculation of EMREAMTTOT.

D3) Calculation of the RMR Adjustment Charge for a QSE is dependent upon the calculation of EMREAMT.

Figure 1.0: Payment for Emergency Power Increase Directed by ERCOT Data Dependencies

### Assumptions

A1)  The proposed NPRR changes to the EMREAMT Charge Type calculation are accepted by TPTF.

A2) 	When creating the RTSPP data cut for a given Settlement Point and Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given 15-minute interval of the RTSPP data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of RTSPP data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no RTSPP data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A3)	The original source system (EMS) will make available AEBP data, as a 15-minute weighted average value across all of the SCED intervals during the 15-minute settlement interval, for Resources that received an Emergency Base Point (EBP) instruction (verbal or otherwise) for a SCED interval within the 15-minute settlement interval.  ERCOT may issue an instruction to hold the previous interval. (PR 6.5.9.2 (3)(d)).  Any “hold” instructions should already be reflected in the AEBP data made available by EMS and appropriately reflected in the interval data cut created by the interface.

A4)  When creating the AEBP data cut for a QSE, Settlement Point, and Resource the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that QSE, Settlement Point, and Resource.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create an AEBP data cut for that QSE, Settlement Point, and Resource.

A5)  The original source system (MMS) will make available EBPWAPR data as a 15-minute weighted average value across all of the SCED intervals during the 15-minute settlement interval for Resources that received an Emergency Base Point (EBP) for a SCED interval within the 15-minute settlement interval, based upon the Resource’s EBP and the Energy Offer Curve.

A6)  When creating the EBPWAPR data cut for a QSE, Settlement Point, and Resource the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that QSE, Settlement Point, and Resource.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create an EBPWAPR data cut for that QSE, Settlement Point, and Resource.

A7)  The Data Aggregation system will create the interval data cut for RTMG.  An interval data cut will be created for all Resources (by QSE, Resource, and Settlement Point), even if it contains all zero values. The RTMG bill determinant values will be validated by Data Aggregation prior to being interface to the Settlement System.  RTMG will not include any generation related to Block Load Transfer Resource quantities.

A8)  The original source system (EMS) will make available BP data, as a 15-minute value for the Operating Day.

A9)  When creating the BP data cut for a QSE, Settlement Point, and Resource the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that QSE, Settlement Point, and Resource.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create a BP data cut for that QSE, Settlement Point, and Resource.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR6 | PR 6.6.2.1 | CSWM.P03Settle BillTransfer | DRIVER: EMREAMT should be calculated for every QSE, Settlement Point, and Resource with an AEBP data cut for the Operating Day. |
| FR7 | PR 6.6.2.1 | CSWM.P03Settle BillTransfer | None of the input bill determinants should be rounded or truncated. |
| FR8 | PR 6.6.2.1 | CSWM.P03Settle BillTransfer | All of the output bill determinants should be rounded to two (2) decimal places. |
| FR9 | PR 12.3 (C) | CSWM.P03Settle BillTransfer | The following QSE specific bill determinant should be identified within the settlement system as necessary for the private data extracts: - AEBP - BP - RTMG - EMRE - EBPWAPR - EMREPR - EMREAMT - EMREAMTQSETOT - EMREBILLAMT |
| FR10 | PR 12.3 (C) | CSWM.P03Settle BillTransfer | The following ERCOT-wide bill determinant should be identified within the settlement system as necessary for the public data extracts: - RTSPP - EMREAMTTOT |
| FR11 | PR 9.2.5 | CSWM.P03Settle BillTransfer | Calculate EMREBILLAMT for the QSE as the QSE’s Operating Day total EMREAMT for the greater settlement run less the QSE’s Operating Day total EMREAMT for the lesser settlement run. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR12 | PR 6.6.2.1 | CSWM.P03Settle BillTransfer | If BP is not available for a QSE, Settlement Point, Resource, and Operating Day for which AEBP is available, the system should stop processing for the QSE, Settlement Point, and Resource and should stop any downstream processing (i.e. LAEMREAMT) and log a WARN error message stating the QSE, Settlement Point, Resource, and Operating Day and missing bill determinant. |
| FR13 | PR 6.6.2.1 | CSWM.P03Settle BillTransfer | If EBPWAPR is not available for a QSE, Settlement Point, Resource, and Operating Day for which AEBP is available, the system should stop processing for the QSE, Settlement Point, and Resource and should stop any downstream processing (i.e. LAEMREAMT) and log a WARN error message stating the QSE, Settlement Point, Resource, and Operating Day and missing bill determinant. |
| FR14 | PR 6.6.2.1 | CSWM.P03Settle BillTransfer | If RTMG is not available for a QSE, Settlement Point, Resource, and Operating Day for which AEBP is available, the system should stop processing for the QSE, Settlement Point, and Resource and should stop any downstream processing (i.e. LAEMREAMT) and log a WARN error message stating the QSE, Settlement Point, Resource, and Operating Day and missing bill determinant. |
| FR15 | PR 6.6.2.1 | CSWM.P03Settle BillTransfer | If a RTSPP data cut is not available for a Settlement Point and Operating Day for which a driver data element exists, the system should log a “CRITICAL” error message and stop processing energy settlements for the Operating Day. This means that any calculation that depends on the RTSPP and any downstream calculation should be stopped. The error message should capture the missing data element, including the Settlement Point, and the Operating Day. |

### Specific Database Needs

None

## Charge for Emergency Power Increases (LAEMREAMT)

Each QSE shall pay a charge for emergency power increases based on its Load Ratio Share of the total additional compensation for all Generation Resources that ERCOT provides Emergency Base Points higher than the SCED Base Point prior to the Emergency Condition. (PR 6.6.9.2)

| Requirement Id | FR16 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.9.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the Charge for Emergency Power Increases by QSE and Operating Day for a 15-minute interval. LAEMREAMT q = (-1) * EMREAMTTOT * LRS q |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| LRS | None | The Load Ratio Share calculated for the QSE for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| EMREAMTTOT | $ | Emergency Energy Amount TotalThe total of the payments to all QSEs as additional compensation for emergency power increases of the Generation Resources for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| LAEMREAMT | $ | Load-Allocated Emergency Energy Amount per QSE—The QSE’s Load-Allocated amount of the total payments for all the Generation Resources with Real-Time Emergency Base Points, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| LAEMREBILLAMT | $ | Load-Allocated Emergency Energy Bill Amount — The difference between two consecutive daily summed channels of the total LAEMREAMT for the QSE and for the Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1) Calculation of the LAEMREAMT for a QSE is dependent upon availability of the data inputs.

D2) Calculation of the LAEMREAMT for a QSE is a dependent upon completion of the EMREAMT calculation process.

Figure 2.0: Charge for Emergency Power Increases Data Dependencies

### Assumptions

A1)  Data Aggregation calculates RTAML for every active QSE, and therefore LRS will be calculated and should be available for every active QSE.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR17 | PR 6.6.9.2 | CSWM.P03Settle BillTransfer | The input bill determinants should not be rounded or truncated. |
| FR18 | PR 6.6.9.2 | CSWM.P03Settle BillTransfer | All of the output bill determinants should be rounded to two (2) decimal places. |
| FR19 | PR 9.2.5 | CSWM.P03Settle BillTransfer | Calculate LAEMREBILLAMT for the QSE as the QSE’s Operating Day total LAEMREAMT for the greater channel less the QSE’s Operating Day total LAEMREAMT for the lesser channel. |
| FR20 | PR 12.3 (C) | CSWM.P03Settle BillTransfer | The following QSE specific bill determinants should be identified within the Settlement System as bill determinants that should be included in the private data extracts: - LRS - LAEMREAMT - LAEMREBILLAMT |
| FR21 | PR 12.3 (C) | CSWM.P03Settle BillTransfer | The following ERCOT-wide bill determinants should be identified within the Settlement System as bill determinants that should be included in the public data extracts: - EMREAMTTOT |
| FR22 | PR 6.6.9.2 | CSWM.P03Settle BillTransfer | DRIVER: LAEMREAMT should be calculated for all active QSEs for the Operating Day. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR23 | PR 6.6.9.2 | CSWM.P03Settle BillTransfer | If an active QSE does not have an LRS data cut for the Operating Day, the Settlement System should produce a WARN-DEFAULT error message and default the LRS values to zero. The warning message should indicate the QSE and Operating Day for which the LRS data is missing. |

### Specific Database Needs

None

# Supplementary Requirements

## Performance Requirements

Lodestar must be able to perform its full set of Nodal settlement calculations. Performance characteristics will be determined and detailed in the conceptual and detailed design.

## Legal and Regulatory requirements

All Lodestar financial data must be saved in accordance with standard accounting principles as dictated by the ERCOT Texas Nodal Protocols.

## System Security Requirements

Lodestar will only be accessed directly by internal ERCOT personnel either through thin client UI, Citrix environment, or thick client on employee desktops. Market Participants will be able to access their statements and invoices on ERCOT private web site. ERCOT private web site access will be controlled by digital certificate authentication in accordance with ERCOT security policies.  Additional details will be provided in the Conceptual System Design.

## Back up and Recovery Requirements

All Lodestar Nodal settlement data must be backed up on a daily basis and recoverable within 24 hours.  Additional details will be provided in the Conceptual System Design.

## Availability and Redundancy Requirements

Nodal Lodestar system must be available 7/24. Lodestar is considered a tier 1 system and critical to base ERCOT business processes.  Additional details will be provided in the Conceptual System Design.

## Maintainability Requirements

Lodestar Nodal system will be maintained by internal ERCOT personnel. Service Level Agreements and explicit support will be detailed in the detail design document.

## Daylight Savings Time shall be recognized in Nodal Market settlements.

Spring Daylight Savings Time Operating Day:

- Hour Ending 0300 will be skipped, resulting in 23 hourly intervals for the day.

- The four fifteen minute intervals within Hour Ending 0300 will be skipped, resulting in 92 fifteen minute intervals for the day.

- The sixty one minute intervals within Hour Ending 0300 will be skipped, resulting in 1380 total one minute intervals.

Fall Daylight Savings Time Operating Day:

- Hour Ending 0200 will have two (2) sets of data, resulting in 25 hourly intervals for the day.

- The four fifteen minute intervals within Hour Ending 0200 will have two (2) sets of data, resulting in 100 fifteen minute intervals for the day.

- The sixty one minute intervals within Hour Ending 0200 will have two (2) sets of data, resulting in 1500 total one minute intervals.

# Protocol Coverage

Since one protocol may translate into many requirements, Project Managers are required to identify the coverage extent (Full/Partial) of all relevant Nodal Protocols that are mentioned in this requirements document.

| Protocol Sub-Section # (To the lowest level of granularity as possible) | Coverage by Requirements (Full/Partial) |
| --- | --- |
| 6.6.9.1 (1) | Partial |
| 6.6.9.1 (2) | Partial |
| 6.6.9.2 | Partial |