Texas Nodal:

Congestion Revenue Rights System

Requirements Specification

Version 0.91

October 6, 2006

| Document Revisions |  |  |  |
| --- | --- | --- | --- |
| Date | Version | Description | Author(s) |
| 07/11/2006 | 0.01 | First draft | Derick Kurdy |
| 07/162006 | 0.02 | Added requirements | Nexant, Energy Solutions Division |
| 07/23/2006 | 0.03 | Added ERCOT comment | Shawna Jirasek |
| 07/23/2006 | 0.04 | Added more requirements and some editing | Nexant, Energy Solutions Division |
| 8/1/2006 | 0.05 | ERCOT Team Edit | ERCOT |
| 8/13/06 | 0.06 | Added more requirements and some editing | Nexant, Energy Solutions Division |
| 8/16/06 | 0.07 | Revised numbering to track Sept ’05 and Aug ’06 differences | Nexant, Energy Solutions Division |
| 8/18/06 | 0.08 | Accepted changes from v0.07 and provided final edits | ERCOT |
| 08/23/06 | 0.09 | Edits from ERCOT internal staff | ERCOT |
| 08/28/06 | 0.10 | Final pre-IDA edits | Grady Sanders, Bob Blackhard, Derick Kurdy, Shawna Jirasek, Beth Garza |
| 08/31/06 | 0.11 | Added comments from Shams Sadiqi, updated numbering for August ’06 protocols CRR Team Approved Version | Nexant, Energy Solutions Division |
| 09/07/06 | 0.12 | Included comments from Jun Yu (IDA) and Grady Sanders (INT) | Derick Kurdy |
| 09/12/06 | 0.13 | Adjusted Supplementary Requirements section using the same format as functional requirements | Stipe Fustar |
| 9/14/06 | 0.14 | Added additional supplemental requirements | Stipe Fustar |
| 9/14/06 | 0.15 | Adjustments based on comments from Reliant and TXU. Made it more Protocols centric. | Stipe Fustar |
| 9/25/06 | 0.16 | Team Edits to include additional TXU, Reliant and overall comments | Derick Kurdy |
| 9/27/06 | 0.17 | Further Team Edits based on review and comments | Derick Kurdy |
| 10/2/06 | 0.9 | Final edits before send to TPTF | Derick Kurdy |
| 10/6/06 | 0.91 | Edits based on TPTF and internal review | Beth Garza |

Table of Contents

1	Scope	4

2	Structure	5

3	Functional Requirements	6

3.1	General	6

3.2	Auction	22

3.3	Allocation	27

3.4	BILATERAL Market	33

4	Supplementary Requirements	36

4.1	Performance Requirements	36

4.2	Sizing Requirements	37

4.3	Legal and Regulatory requirements	39

4.4	System Integration Requirements	39

4.5	System Security Requirements	43

4.6	Back up and Recovery Requirements	44

4.7	Availability and Redundancy Requirements	44

4.8	Portability and Scalability	44

4.9	System Monitoring	45

4.10	Maintainability Requirements	45

4.11	Usability Requirements	46

4.12	Date and Time Requirements	47

4.13	Documentation	48

4.14	Training	50

4.15	ERCOT IT Standards	50

# Scope

The requirements listed in this document shall allow ERCOT to perform the following four tasks:

- Distribute Congestion Revenue Rights (CRRs) through CRR Auctions and allocate Pre-assigned Congestion Revenue Rights (PCRRs) and McCamey Flowgate Rights (MCFRIs) in a prescribed manner.

- Determine payments owed by or to the CRR Account Holders after each CRR Auction.

- Track ownership of CRRs by CRR Account Holders, including:

  - PCRRs and MCFRIs,

  - Purchase and sales in CRR auctions,

  - Bilateral trades,

  - Provide CRR ownership and transaction information to ERCOT’s Settlement System which will settle CRRs after CRR Auctions in DAM and Real Time and calculate payments due to or from CRR Account Holders.

- Provide CRR transaction information to other ERCOT systems such as Credit Monitoring, DAM, Settlements, and MIS.

# Structure

This document is titled as TN.CRR.RequirementsSpecification.v[x].doc

The Requirements sections of this document are:

- Functional Requirements

  - General

    - CRRs

    - Markets

    - Modeling

    - System

  - Auction

  - Allocation

  - Bilateral Market

- Supplementary Requirements

  - Performance Requirements

  - Sizing Requirements

  - Legal and Regulatory Requirements

  - System Integration

  - Data Exchanges

  - System Security Requirements

  - Back up and Recovery Requirements

  - Availability and Redundancy

  - Portability and Scalability

  - System Monitoring

  - Maintainability Requirements

  - Usability Requirements

  - Date and Time Requirements

  - Documentation

  - Training

  - ERCOT IT Standards

All protocol numbering refers to August 1, 2006 version except where noted.

# Functional Requirements

## General

### CRRs

| Requirement ID | FR1 |
| --- | --- |
| Short Name | Acquiring CRRs |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.1 (1) 7.1 (2) 7.1 (3) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| CRR System shall enable ERCOT to process Congestion Revenue Right (CRR) as a financial instrument that entitles the CRR Owner to be charged or to receive compensation for congestion rent.. CRRs do not represent a right to receive, or obligation to deliver, physical energy. Most CRRs are tradable in the CRR Auction, in the DAM, or bilaterally. If a CRR is designated as non-tradable, then its owner cannot offer it for sale in the auctions or trade it bilaterally. This configuration shall be used so that PTP Options with Refund and PTP Obligations with Refund are not tradable in the CRR System. The CRR System shall enable ERCOT to facilitate the following activities where the CRRs can be acquired and disposed: - CRR Auction – ERCOT shall conduct monthly and annual auctions to allow eligible CRR Account Holders to acquire CRRs. The auction also allows CRR Owners an opportunity to sell tradable CRRs that they hold. - PCRR Allocations – ERCOT shall allocate CRRs (known as Preassigned Congestion Revenue Rights or PCRRs) to eligible Municipally Owned Utilities and Electric Cooperatives. - McCamey Area Flowgate Rights Allocations – ERCOT shall allocate McCamey Area Flowgate Rights (MCFRIs), which are a type of Flowgate Right (FGR), to eligible CRR Account Holders - Bilateral Market – Any CRR Account Holder may trade PTP Options, PTP Obligations, and FGRs bilaterally. PTP Options with Refund and PTP Obligations with Refund are not tradable in the CRR System. Bilateral trading, of CRRs that are eligible to be traded, may be done privately or through ERCOT. ERCOT shall facilitate trading on the MIS Secure Area of existing CRRs between CRR Account Holders, subject to credit requirements. ERCOT shall settle CRRs with the CRR Account Holder shown on ERCOT records. Note: Although any QSE may bid for PTP Obligations in the DAM*, any instruments acquired in this method are not tracked in the CRR System The CRR System shall enable ERCOT to facilitate allocations, auctions and bilateral trades where the following CRRs types can be acquired: - Point-to-Point (PTP) Options, some of which may be PCRRs, - Point-to-Point (PTP) Obligations, some of which may be PCRRs - PTP Options with refund, all of which are PCRRs - PTP Obligations with refund, all of which are PCRRs - Flowgate Rights (FGRs), including MCFRI. 7.1 (1) 7.1 (2) 7.1 (3) |  |
| * Out of Scope from CRR System perspective |  |

| Requirement ID | FR2 |
| --- | --- |
| Short Name | Defining CRRs |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.2 7.3 (1)(5) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall enable ERCOT to allocate and auction CRRs with the following characteristics: - A quantity of MWs measured to one-tenth of a MW - A duration of one hour, auctionable and tradable only in a time of use block as defined in FR3 - An ability to be fully tradable financial instruments in specified time of use blocks except for a PTP Option with Refund and a PTP Obligation with Refund - A designated source (injection point) that is a Settlement Point and a designated sink (withdrawal point) that is a different Settlement Point, except for an FGR, which has a designated flowgate instead A CRR is defined as a PTP Option, PTP Obligation or Flowgate Right (FGR). A PTP Option or PTP Obligation is defined as a right from a Source to a Sink for a number of MWs for a certain time of use from a start date to an end date. An FGR is defined as a right for a number of MWs for a certain time of use from a start date to an end date on a flowgate. A flowgate is defined as a designated directional network element, or a bundle of directional network elements. The quantity of CRRs identified by the CRR System is limited by the transmission network capacity as reflected in the CRR Network Model used for the auction or allocation. The CRR System shall maintain a record of the quantity of CRRs owned by each CRR owner. Tradable CRRs shall be permitted to be exchanged between the CRR Owner and a CRR Account Holder from a start date to an end date in quantities as small as one Time of Use Block*** for quantities and dates not to exceed the total owned. Effectively this means that the most granular trade is a 0.1 MW time of use block for a defined effective date of a time of use block. 7.2 7.3 (1)(5) |  |
| ***This portion of the requirement is based on NPRR028 and is still pending Nodal change control approval. |  |

| Requirement ID | FR3 |
| --- | --- |
| Short Name | Defining Times of Use |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.3(6)(7) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall enable ERCOT to distribute CRRs in the following time of use blocks: - Weekday peak (5x16) time of use blocks for hours ending 0700-2200, Monday through Friday (excluding NERC holidays), - Weekend peak (2x16) time of use blocks for hours ending 0700-2200, Saturday and Sunday, and NERC holidays - Off-peak (7x8) time of use blocks for hours ending 0100-0600 and hours ending 2300-2400 Sunday through Saturday, and - 7x24 blocks (combinatorial of all of the previous three types of time of use blocks), - The CRR blocks described in the previous bullets shall be auctioned simultaneously. In the annual CRR Auctions capacity is made available for the next two years. In the monthly CRR Auctions capacity is made available for the following month. The CRR System shall allow CRR Market Operator to assign and change the time of use block associated with each hour of the year. The time of use blocks must be mutually exclusive such that they cover 24 hours without overlapping. The time of use blocks must also be collectively exhaustive to cover all hours included in the auction. CRR Account Holders shall be able to submit linked bids (but not linked offers) consisting of 3 time of use blocks to cover the 7x24 concept. 7.3 (6)(7) |  |

| Requirement ID | FR4 |
| --- | --- |
| Short Name | Identifying CRRs |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.2.1 7.5.2.1 (1)(b) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall be flexible to comply with naming conventions for CRR and across Nodal Systems by a yet to be established Task Force (proposed by the appropriate TAC subcommittee). The CRR System shall allow tracking and sorting of CRRs by any/all attributes, including the naming convention. The CRRs shall be uniquely identified by an ID. 7.2.1 7.5.2.1 (1)(b) |  |

| Requirement ID | FR5 |
| --- | --- |
| Short Name | Evaluating PTP Options and Obligations |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.3 (2)(3)(4) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall enable ERCOT to auction and allocate the PTP Options and Obligations as follows: PTP Options; - PTP Options shall be evaluated in each CRR market as the positive power flows on all directional network elements created by the injection and withdrawal at the specified source and sink points in the quantity represented by the CRR bid or offer (MW), excluding all negative flows on all directional network elements Assumption: The Settlements System shall ensure PTP Options shall never result in charges from ERCOT to the CRR Owner of record. PTP Obligations; - PTP Obligations are evaluated in each CRR market as the positive and negative power flows on all directional network elements created by the injection and withdrawal at the specified source and sink points of the quantity represented by the CRR bid or offer (MW). Assumption: The Settlements System shall ensure PTP Obligations shall result in either a payment or a charge to the CRR Owner of record. For the purposes of the CRR power flow analysis, an injection point shall be treated as a generator and a withdrawal point shall be treated as a load. Further, CRR bids represent a willingness to purchase at a ‘not to exceed amount’ and CRR offers represent a willingness to sell at a ‘minimum reservation price.’ 7.3 (2)(3)(4) |  |
| Note: The CRR System shall evaluate the simultaneous feasibility of all CRRs based on time of use blocks and monthly strips. The SFT assumptions for a particular time of use block are the same for every hour of that block for the duration of the month. However, each month may have a separate set of SFT assumptions for each time of use block. For example, the individual evaluation of hours 0100 and 0200 for the same month will be identical because they use the same input data for their SFT. Note: An NPRR is needed to remove the term ‘hourly’ from the Nodal protocol Sections 7.3(2) and 7.3(3). Note: The CRR System shall allow each time of use block to be evaluated using a different network model. |  |

| Requirement ID | FR6 |
| --- | --- |
| Short Name | Evaluating FGRs |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.3 (5) 7.3.1.2 |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| ERCOT currently has named only one flowgate which is located in the McCamey area. Moreover, ERCOT identifies the FGRs from this flowgate as MCFRIs. The CRR System shall be able to support any additional flowgates defined by the ERCOT Board in the future. The CRR System defines a flowgate as the sum of the weighted flow in a bundle of directional network elements. In the case of the McCamey flowgate, these elements originate in the McCamey Area and sink outside the McCamey Area. The boundaries of the McCamey Area shall be defined in the CRR Model. The CRR System shall evaluate FGRs (including MCFRIs) in each CRR market as the positive power flows represented by the quantity of the CRR (MW) on a flowgate 7.3 (5) 7.3.1.2 |  |
| The ‘capacity impact’ of a resource on existing the current McCamey flowgate shall be determined by adding the products of individual resource shift factor on each flowgate element multiplied by the resource capacity multiplied by the impact normalization factor. This process applies only to the allocation of MCFRIs for the existing McCamey flowgate. If the ERCOT Board approves the definition of more flowgates, the FGRs associated with those flowgates shall be made available though the auction process. 7.3 (5) 7.7.3 (1) |  |

Markets

| Requirement Id | FR7 |
| --- | --- |
| Short Name | Eligibility for PCRRs |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.1 (2)(b) 7.4.1 |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| - The CRR System shall enable ERCOT to allocate PCRRs to eligible CRR Account Holders that are Municipally Owned Utilities and Electric Cooperatives. - All three of the following requirements shall be met to receive PCRRs: - Entity shall be Non-Opt In Entities (NOIEs) - NOIE shall become CRR Account Holder - CRR Account Holder shall complete the application for allocation of PCRRs - An eligible NOIE CRR Account Holders shall either: - Own or have a long-term (greater than five years) contractual commitment that was entered into before September 1, 1999 for annual capacity and energy from specific Generation Resources; or - Have a long-term (greater than five years) allocation from the federal government for annual capacity and energy produced at a federally-owned hydroelectric Generation Resource, and that allocation was in place prior to September 1, 1999. - A Municipally Owned Utility or Electric Cooperative shall no longer receive allocated PCRRs after they opt into competition, with the exception of South Texas Electric Cooperative (STEC). STEC may be allocated PCRRs for up to three years after the date it enters into competition. 7.1 (2)(b) 7.4.1 |  |

| Requirement Id | FR8 |
| --- | --- |
| Short Name | Auction of the Network Capacity |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.1 (1) 7.5.1 (6) (7) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall enable ERCOT to auction the available network capacity of the ERCOT Transmission System not allocated as PCRRs or MCFRIs, or sold in a previous auction. The CRR Auction shall provide CRR Owners the ability to offer for sale CRRs that they hold. Each annual CRR Auction shall allow for the purchase of CRR products in multiple or one-month strips. Monthly CRR Auctions shall include products for the next month only. Note: Based on NPRR027 and NPRR028, reconfiguration of previously awarded CRRs by CRR Account Holders is limited. An NPRR is needed to better define the concept of reconfiguration in auctions in the Nodal Protocol 7.5.1 (1). - The CRR System shall enable ERCOT to offer network capacity for two years in each annual CRR Auction equal to the difference between (a) and (b): - For each month, the expected network topology for that month of the first year in the CRR Network Model scaled down to 55% for the first year and 15% for the second year; and - All outstanding CRRs that were previously awarded or allocated for the corresponding months in both years. - The CRR System shall enable ERCOT to offer network capacity for the monthly CRR Auction equal to the difference between: - The expected transmission network topology in the CRR Network Model of the month for which the CRRs are effective scaled down to 90%; and - All outstanding CRRs that were previously awarded or allocated for the month. The percentages of network capacity shall be configurable for each market. 7.5.1 (1) 7.5.1 (6) (7) |  |

| Requirement Id | FR9 |
| --- | --- |
| Short Name | General CRR Account Holder Capabilities |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.4 (1) 7.4.2* 7.8* *CRR team understanding of the intention of the protocols |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall provide a user interface to the CRR Account Holders to allow them to: - Check public and private messages sent by the CRR Market Operators - Participate in CRR markets (auctions and allocations) and obtain all data provided by the system, including: - Effective Period(s) and Submission window schedule for markets - Download all CRR related network and market data - Create, validate and manage buy bids for CRRs for auctions - Create, validate and manage sell offers for CRRs for auctions - Create, validate and manage nominations for CRRs for allocations - Upload bids and offers through a user interface - Manage bilateral trades - View and export ownership of record for CRRs. - Assumption: The CRR user interface will support uploads of XML and CSV files. This shall allow users to develop their bid/offer portfolios off-line (i.e. Excel). 7.5.4 (1) 7.4.2* 7.8* - *CRR team understanding of the intention of the protocols |  |

| Requirement Id | FR10 |
| --- | --- |
| Short Name | Information Posted Before the Market |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.3.1.1 7.7.2 (1)(2) 7.5.3.2 (1)(a)(b)(c)(d)(e)(f)(g) 7.5.3 (1) (i) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| - The CRR System shall enable ERCOT to publish the list of all flowgates available for FGRs on the MIS Public Area. If there is any change in the designation of flowgates, the CRR System shall enable ERCOT to provide notices to all CRR Account Holders. 7.3.1.1 |  |
| ERCOT shall determine the boundaries of the McCamey Area and publish maps on the MIS Secure Area indicating the Electrical Buses contained in the McCamey Area. The CRR System shall enable ERCOT to post to the MIS Secure Area the specific predefined directional network element that defines each McCamey Area flowgate. The elements that comprise new McCamey Area flowgates may be established due to changes in the transmission system. All communications about model elements shall be provided using the CIM naming standards as are employed in the ERCOT NMMS system. 7.7.2 (1)(2) - 7.5.3.2 (1)(c) |  |
| - The CRR System shall allow for configuration of the date/days on which activities are performed. - Not less than 20 days before each annual CRR Auction and not less than 10 days before each monthly CRR Auction, CRR System shall enable ERCOT to post the following to the MIS Public Area: - For the CRR Auction, number of CRRs previously awarded or allocated for each appropriate month, including the source and sink for each such CRR - For the CRR Auction, number of FGRs that have been previously awarded or allocated for each appropriate month, including the flowgate definition - Deadline for CRR Account Holders to satisfy financial requirements to participate in the auction - Assumption: Financial requirements will be identified by the Credit group. The timing of the collateral deposit will be dictated by a business process to be developed. - Specifications for the equipment and interfaces necessary to participate in the CRR Auction - Date and time by which CRR Auction Bids and CRR Auction Offers in the CRR Auction must be submitted - Bid and offer format - 7.5.3.2 (1)(a)(b)(d)(e)(f)(g) |  |
| - The CRR System shall enable ERCOT to post all CRR Network Models in CIM compliant format for each effective month of the auction on the MIS Secure Area, before each CRR Auction: - Note: There could be multiple models – one per time of use block – used as part of each CRR Auction. - For monthly auctions the model shall be posted no later than 10 Business Days before the auction. - For annual auctions the model shall be posted no later than 20 Business Days before the annual auction. - The CRR System shall allow for configuration of the date/days on which activities are performed. - 7.5.3 (1) (i) |  |

| Requirement Id | FR11 |
| --- | --- |
| Short Name | Private Information Posted After the Market |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.3.1 (1) |
| Coverage of Source | Full |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System, following each CRR Auction, shall record and make available to each CRR Account Holder on the MIS Certified Area the following information for each CRR awarded in, sold in, or allocated before, the CRR Auction to the specific CRR Account Holder: - Unique identifier of each CRR - Type of CRR (PTP Option, PTP Obligation, PTP Option with Refund, PTP Obligation with Refund, MCFRIs or other FGRs) - CRR clearing price and the pricing factor for PCRRs - The source and sink of each PTP CRR - Flowgate name and direction for each FGR - The date and time of use block for which the CRR is effective - Total MW of each PTP pair of CRR, awarded, sold or allocated, or total MW for each flowgate, awarded, sold or allocated. 7.5.3.1 (1) |  |

| Requirement Id | FR12 |
| --- | --- |
| Short Name | Public Information Posted to the MIS |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.3.1 (2) 7.5.5.2 7.7.2 (4) |
| Coverage of Source | Full |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall make available on the MIS Public Area the following information to all Market Participants once a market run (auction or allocation) has been completed: - PTP Options and PTP Options with Refund – the source and sink, and total MWs; - PTP Obligations and PTP Obligations with Refund – the source and sink and total MWs; - FGRs – the identity of each directional flowgate, and magnitude of positive flow (MW) on each directional network element represented by each flowgate.; - The identities of the CRR Account Holders that were awarded or allocated CRRs in or before the CRR Auction; - The clearing prices for each strip of CRR blocks awarded in the CRR Auction; - The identity and post contingency flow of each binding directional element based on the CRR Network Model used in the CRR Auction; and - All CRR Auction Bids and CRR Auction Offers, without identifying the name of the CRR Account Holder that submitted the bid or offer. 7.5.3.1 (2) |  |
| By the 5th business day of each month, the CRR System shall post on the MIS public area: - CRR ownership of record for each source and sink pair and each flowgate - For all CRR Account Holders, the type of CRR held by that account holder, and total MWs held by that account holder. Assumption: This posting will occur after the invoices for CRRs received in the monthly auction have been paid and prior to the 5th business day of the month for which the auction was conducted. 7.5.5.2 |  |
| ERCOT shall post on the MIS Public Area a current geographical map and an electrical one-line diagram of the boundaries of the McCamey Area. ERCOT shall revise the map and diagram as necessary to reflect any changes in transmission system configuration or new interconnections of WGRs in west Texas. 7.7.2 (4) |  |

| Requirement ID | FR15 |
| --- | --- |
| Short Name | CRR Distribution Process |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.1 (1) 7.5.5.4 (3)(d)(e) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| - Description |  |
| Each subsequent market shall be run including the results of the previous markets, including all already allocated and awarded CRRs. The CRR System shall schedule markets to run in the following order, begun by a CRR Market Operator: - PCRR PTP Allocation (Allocation Stage 1) - MCFRI Allocation (Allocation Stage 2) - Auction - 7.5.1 (1) |  |
| - At each stage of the CRR distribution process, in the event that transmission capacity has been oversold in prior CRR auctions and CRR allocations, the CRR System shall allow the CRR Market Operator to increase the transmission capacity to exactly match the amount of CRRs that have been sold or allocated on that element. This ensures initial feasibility of the previously distributed CRRs within the CRR Network Model being used. 7.5.5.4 (3) (d)(e) |  |

| Requirement Id | FR16 |
| --- | --- |
| Short Name | SFT Inputs |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.5.4 - 7.4.2 (a)* - 7.5.1 (6)* - 7.7.3 (2)* *CRR team understanding of the intention of the protocols |
| Coverage of Source | Full |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| - The CRR System shall enable ERCOT to perform the Simultaneous Feasibility Test (SFT) that confirms that the transmission system can support the allocated and awarded set of CRRs during normal system conditions, assuming that the Network Operations Model updated with Real-Time network topology is the same as that modeled (for the CRR market), while observing all base case and post-contingency operating constraints as well as non-thermal/generic constraints. - The CRR System shall use a DC power-flow model for the SFT, and model the effect of CRR Auction bids and offers on the expected system network topology during the auction term. SFT is not a system reliability test and is not intended to model actual system operating conditions. SFTs are run during the determination of the winning bids and offers for the CRR Auction. - Inputs to the SFT model shall include: - CRR bids and offers for the auction term; - All previously awarded or allocated CRRs for the study period; - Expected configuration of Transmission Facilities including scheduled outages, adjusted for oversold CRRs, as specified below; - Increased capacity of each element that has been oversold in prior CRR Auctions and CRR allocations to exactly match the amount of CRRs that have been sold or allocated on that element (this ensures the feasibility of the CRR Auction); - Thermal operating limits (including estimates for Dynamic Ratings) for transmission lines; - For the annual auction ERCOT shall use Dynamic Ratings based on a historical analysis of the maximum peak-hour temperatures for the previous 10 years; and - For the monthly auction ERCOT shall use Dynamic Ratings for the maximum peak-hour temperature forecast for the month; - Voltage and stability limits that are valid for the study period converted to thermal limits represented as non-thermal/generic constraints; - ERCOT Transmission Grid pre-and post-contingency ratings; - List of transmission element contingencies expected to be enforced by ERCOT in Real-Time Operations - Selected RAPs and SPSs 7.5.5.4 |  |
| - CRR System shall accommodate operator defined scaling factors for the network capacity and previously allocated/awarded CRRs to limit the available capacity in the market run. - 7.4.2 (a)* - 7.5.1 (6)* - 7.7.3 (2)* - *CRR team understanding of the intention of the protocols |  |

| Requirement Id | FR17 |
| --- | --- |
| Short Name | Network Model Adjustments in the CRR System |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.1 (2) 7.5.5.3 (3) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall allow the CRR Market Operator to edit the information within the markets (auctions or allocations) by modifying the following attributes: - In-service/out-of-service flag for lines and transformers (shall be used to evaluate the effect of outages and new construction) - Contingencies to include/exclude - Note: The deselection of contingencies shall be used to represent RAPs and SPSs to address Nodal Protocol 7.5.5.3 (3) - 7.5.1 (2) - 7.5.5.3 (3) |  |
| - The following attributes shall be able to be adjusted based on functionality included in the off-the-shelf system: - Non-thermal/generic constraint limits - Base-case and contingency ratings for lines and transformers (including dynamic ratings where available) Supplementary to Protocols |  |

| Requirement Id | FR18 |
| --- | --- |
| Short Name | CRR System Activities |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.3 (1)(b)(c)(d)(e)(f)(h) |
| Coverage of Source | Full |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| - The CRR System shall enable ERCOT to perform the following auction and allocation activities: - Provide calendar of CRR markets (auctions and allocations); - Initiate, direct, oversee and clear the CRR markets; - Post CRR market results; - Maintain a record of the CRRs; - Provide a mechanism to record CRR bilateral transactions; - Keep, under the ERCOT data retention policy, all information and tools necessary to reproduce CRR auctions and allocations. 7.5.3 (1)(b)(c)(d)(e)(f)(h) |  |
| - Assumption: Only qualified CRR Account Holders will be able to participate in the CRR markets. ERCOT’s registration process for qualified CRR Account Holders is outside the scope of this requirements document. Other ERCOT systems will provide the entity name and system access privileges. - Assumption: The Settlements System/process will determine CRR Auction invoice amounts by summing the CRR market transaction amounts in dollars for each CRR Account Holder - Assumption: The Settlement System / process will distribute auction revenue. |  |

| Requirement Id | FR19 |
| --- | --- |
| Short Name | Market Clearing Data Retention |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.5.3 (4) |
| Coverage of Source | Full |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| After posting the market, the CRR System and EDW shall keep, under the ERCOT data retention policy and data versioning standard, all information and tools necessary to reproduce CRR auctions and allocations: - Bids and offers for auctions and nominations for allocations - Auction participant credit limits - Previously awarded and allocated CRRs that affect the available capacity for the current market - CRR network model (including outages and new construction included in the model) - Non-thermal and generic constraints defined as the sum of the weighted branch flows (this includes constraint coming from voltage and stability limits). - List and definitions of contingencies - List and definitions of source and sinks. - Load distribution factors for Load Zone definition - Dynamic rating assumptions - RAP and SPS assumptions - 7.5.5.3 (4) |  |

### Modeling

| Requirement ID | FR15C |
| --- | --- |
| Short Name | Defining a CRR Network Model |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.3 (2) referring to 3.10.3 (1)(2)(3)(a-h)(4)(5) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| - Description |  |
| - Assumption: The CRR Network Model input into the CRR System shall be a PSS/E RAWD format file following the requirements noted below. - The CRR System shall import the CRR Network Model components from NMMS using case building capability performed by CRR Market Operators. - Names of Transmission Elements in the CRR Network Model shall be identical to those used in the Network Operations Model. - Names of Hub Buses and Electrical Buses used to describe the same device in any Hub shall be identically named in the CRR Network Model and the Network Operations Model. - Each CRR Network Model shall include: - A complete one-line diagram with all Settlement Points (indicating the Settlement Point that the Electrical Bus is a part of) and including all Hub Buses used to calculate Hub prices; - Generation Resource locations; - Transmission Elements; - Transmission impedances; - Transmission ratings; - Contingency lists; - Data inputs used in the calculation of Dynamic Ratings, and - Other selected inputs used for the CRR Network Model including non-thermal/generic constraints - Assumption: The non-thermal/generic constraints shall be applied to the CRR Network Model Base Case. Non-thermal/generic constraints shall be expressed uni-directionally. - The CRR System shall import the CRR Network Model for CRR markets a minimum of two months in advance of the effective month. - The CRR Network Model information shall be made available to Market Participants using Electric Power Research Institute (EPRI) and NERC-sponsored “Common Information Model” format and shall be provided through the MIS. - Information in the CRR Network Model shall include the effective period of the auction (monthly for monthly auctions, each month for annual auctions), Contingencies, Non-thermal/generic constraints, Source/Sinks and Flowgates to be auctioned for time of use as described in FR3. - 7.5.3 (2) referring to 3.10.3 (1)(2)(3)(a-h)(4)(5) |  |

| Requirement Id | FR20 |
| --- | --- |
| Short Name | Hubs and Load Zone Modeling |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.1 (3) referring to 3.5.2 |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall model nominations, bids and offers into the CRR market as flows based on the MW offer and defined source and sink. When the CRR System engine is run, the model must weight the Electrical Buses and Hub Buses included in a Hub or Load Zone appropriately to determine the system impacts of the CRRs. - The CRR System shall distribute injections and withdrawals to Electrical Buses in Load Zones, using the Load-weighted distribution factors for On-Peak Hours in each Load Zone from the planning cases (for the same period) for monthly CRR markets (or for the monthly models used in annual CRR markets). If monthly planning cases do not exist, ERCOT shall use the Load-weighted distribution factors for On-Peak Hours in each Load Zone from the selected seasonal planning case. - The CRR System shall distribute the injections and withdrawals for a particular Hub Settlement Point using distribution factors defined as follows: - The distribution factors for an Electrical Bus within a Hub Bus shall be a simple average for each Electrical Bus. - The distribution factors within a Hub for each defined element of the Hub shall be a simple average of the number of defined elements in the Hub. 7.5.1 (3) referring to 3.5.2 |  |

| Requirement Id | FR21 |
| --- | --- |
| Short Name | Available CRR Auction Products |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.1 (5) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall enable ERCOT to auction the following products: - In each monthly CRR Auction: one-month strips of PTP Options, PTP Obligations, and FGRs; and - In each annual CRR Auction: - PTP Options in one-month strips, any specified consecutive monthly strips, annual strips, and two-year strips for two-year terms; - PTP Obligations in one-month strips for one-month terms until the ERCOT Board approves the offering of PTP Obligations at specified source Settlement Points and sink Settlement Points for terms longer than one month - Assumption: PTP Obligations between every combination of sources and sinks shall only be available in the first month of an auction. Subsequent to Board action, if any, PTP Obligations between a selected subset sources and sinks for all months could be made available. The CRR System shall only accommodate one subset of selected sources and sinks for any given auction. - An NPRR is needed to clarify section 7.5.1 (5). - MCFRIs in one-month strips, any specified consecutive monthly strips, annual strips, and two-year strips for two-year terms. 7.5.1 (5) |  |

### System

| Requirement Id | FR22 |
| --- | --- |
| Short Name | CRR System Access |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.2 (1) 7.5.3 (4)* 7.5.4 (2)* *Covered through the assumption that credit activities will be done by the credit group |
| Coverage of Source | Full |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall only grant access to entities that are registered as valid CRR Account Holders with ERCOT. The CRR account Holders must satisfy financial assurance criteria required to participate in CRR markets. Assumption: All financial qualification processes will be completed by the Credit group. 7.5.2 (1) 7.5.3 (4)* 7.5.4 (2)* *Covered through the assumption that credit activities will be done by the credit group |  |

## Auction

| Requirement ID | FR15B |
| --- | --- |
| Short Name | Frequency of Auctions |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.1 (4) (a)(b)(d)(e) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| - Description |  |
| - The CRR System shall enable ERCOT to conduct CRR Auctions with the frequency, on the dates, and for the terms specified as follows: - PTP Options, PTP Obligations, and MCFRIs in monthly auctions for one-month terms beginning with the month prior to the Texas Nodal Market Implementation Date; - ERCOT shall conduct a monthly CRR Auction on the second Tuesday of each month immediately before the one-month term during which the CRRs being auctioned are effective. If the second Tuesday is not a Business Day, then that monthly auction must occur on the next Business Day after the second Tuesday. - A six-month trial period must be completed prior to initiation of the first annual CRR Auction. If the six-month trial period is completed prior to October 1, then CRR Options, MCFRIs and Board-approved PTP Obligations will be auctioned for the balance of the current calendar year. - After that first six-month trial period, ERCOT shall conduct an annual CRR Auction for CRR Options, MCFRIs and Board-approved PTP Obligations on the first Tuesday of each November for the two-year term that starts on the immediately following January 1. - Note: There is no requirement associated with Nodal Protocol 7.5.1 (4) (c) because there is no such protocol. A NPRR is needed to correct the numbering. - 7.5.1 (4) (a)(b)(d)(e) |  |

| Requirement Id | FR25 |
| --- | --- |
| Short Name | CRR Auction Process |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.5.3 (1) 7.5.5.1 |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall enable ERCOT to run monthly and annual single-round auctions (simultaneously optimizing across offers for all types of CRR products—obligations, options, and FGRs) with the following steps: - ERCOT shall enter into the CRR System a credit constraint for each CRR Account Holder. A CRR Account Holder’s CRR Auction credit limit is equal to the lesser of the CRR Account Holder’s Credit Limit, or, if provided, the CRR Account Holder’s self-imposed credit limit. The credit constraint is enforced during the optimization that comprises the CRR auction ensures that the following sum is less than or equal to each CRR Account Holder’s CRR Auction credit limit for that auction: Note: The protocols refer to QSEs where CRR Account Holder is the appropriate reference in section 7.5.5.3 (1). A NPRR is needed to correct this language. - All awarded CRR Auction Bids multiplied by the absolute value of the corresponding bid price; plus - All awarded CRR Auction Offers with negative offer prices multiplied by the absolute value of their corresponding offer price; plus - The additional credit requirement for all awarded PTP Obligations. Assumption: The ‘additional credit requirement’ for awarded PTP Obligations will need further clarification from the TPTF to determine the weighting and source for the Forward Mark to Market value. The simplest option for FMM determination within the CRR System is to use the bid price as the FMM value. However, this may multiply the impact of certain types of bidding. Additionally, ERCOT will need clarification regarding how to value obligations for the first (and early) markets. - The CRR System shall award CRRs in quantities truncated to the nearest tenth of a MW. - The CRR Clearing Price shall be equal to the corresponding Shadow Price for that CRR product. Note: The definition of a Shadow Price for a CRR product is the impact in the objective function of a one MW change to the CRR product. The Shadow Price for an Obligation shall be calculated as the sum of the products of the binding constraint Shadow Prices and their shift factors. The Shadow Price for an Option shall be calculated as the sum of the products of the binding constraint Shadow Prices and their shift factors, including only non-negative shift factors. The Shadow Price for an FGR shall correspond to the Shadow Price of the flowgate. CRRs of different durations are considered different products. For example, the clearing price for a 3-month strip between two points may not be equal or proportional to the clearing prices for three separate one-month strips between the same two points over the same three months. The reason for this is that the individual months are different products than the product spanning multiple months. - When a CRR Account Holder is awarded CRRs as a result of a CRR Auction, the CRRs do not become the property of the winning CRR Account Holder until the CRR Invoices have been paid in full. Note: The protocols refer to CRRs not being ‘placed in’ CRR Account Holders accounts; the CRR System will place them in the accounts upon the validation of the CRR Auction but will not ‘activate’ the CRRs until payment is received. - When a CRR Account Holder sells PTP Obligations as a result of an auction at a negative price, the CRR Account Holder is not relieved of the PTP Obligations until the CRR Invoices have been paid in full. 7.5.5.3 (1) |  |
| The CRR System shall prevent a CRR Account Holder from being awarded bids and offers that exceed the lesser of the CRR Account Holder’s self-imposed credit limit or the CRR Account Holder’s credit limit. 7.5.5.1 |  |

| Requirement Id | FR9b |
| --- | --- |
| Short Name | Creating and Validating Bids and Offers |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.2.1 7.5.2.2 7.5.2.3 7.5.2.4 |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall provide a user interface to the CRR Account Holders to allow them to participate in auction. Offers must include the following: - A CRR Auction Offer indicates a willingness to sell CRRs at the auction clearing price, if it equals or exceeds the Minimum Reservation Price. It must be submitted by a CRR Account Holder and must include the following: - The name of the CRR Account Holder; - The unique identifier for each CRR being offered Assumption: CRRs will be uniquely identified by a CRR System generated number. The CRR Name will be readable and will include the single type of CRR being offered but may not be unique. An NPRR is needed to remove ‘type’ from the requirement for a unique ID from Nodal Protocol 7.5.2.1 (1) (b). - The source Settlement Point and the sink Settlement Point or name of flowgate for the CRR or block of CRRs being offered; - The month(s) for which the CRR or block of CRRs is being offered, including block designation; - The smallest granularity that may be offered in an auction is a one-month strip for 0.1 MW of a time of use block - The longest duration that may be offered in an auction is the entire period of the auction or the amount owned, whichever is less. - The combination of two or more CRRs to create an offer is not supported. - Note: For example, in the 2012-13 annual auction, consisting of twenty-four monthly terms from January 2012 to December 2013, CRR Account Holders can submit sell offers for March 2012 and for June-July-August 2013, but not from March 15th to April 15th, as it would not fully cover the monthly terms. Additionally, they will not be able to "glue" together a 15-day CRR for the first half of a month, with another 15-day CRR for the second half of the month to create a sell offer for the full month. - The quantity of CRRs in MW, which must be the same for each hour within the block, for which the Minimum Reservation Price is effective; and - A dollars per CRR (i.e. dollars per MW per hour) for the Minimum Reservation Price. - An offer to sell an FGR must specify the name of a flowgate. - A CRR offer for a specified MW quantity of CRRs constitutes an offer to sell a quantity of CRRs equal to or less than the specified quantity. The CRR System shall not allow an offer to specify a minimum quantity of MW that the CRR Account Holder wishes to sell. 7.5.2.1 |  |
| The CRR System shall provide ability to validate CRR Auction Offers upon entry as follows: - A valid CRR Auction Offer is a CRR Auction Offer that ERCOT has determined meets the criteria above. - ERCOT shall continuously display on the MIS Certified Area information that allows any CRR Account Holder submitting a CRR Auction Offer to view its valid CRR Auction Offers. - At the point of submission, CRR System shall notify each CRR Account Holder of any of its CRR Auction Offers that are invalid. The CRR Account Holder may correct and resubmit any invalid CRR Auction Offer, if the submission window is still open. 7.5.2.2 |  |
| The CRR System shall provide a user interface to the CRR Account Holders to allow them to participate in auction according to the following bid criteria: - A CRR Auction Bid indicates a willingness to buy CRRs at the auction clearing price, if it is equal to or less than the Not-to-Exceed Price. It must be submitted by a CRR Account Holder and must include the following: - The name of the CRR Account Holder; - The single type of CRR being bid; - The source Settlement Point and the sink Settlement Point or name of flowgate for the block of CRRs being bid; - The month(s) for which the block of CRRs is being bid, including block designation; - The smallest granularity that may be bid upon in an auction is a one-month strip for an individual time of use block - The longest duration that may be bid upon in an auction is the entire period of the auction. This is subject to limitations on PTP Obligations in the annual auction outlined in FR21. - The quantity of CRRs in MW, which must be the same for each hour within the block, for which the Not-to-Exceed Price is effective; and - A dollars per CRR (i.e. dollars per MW per hour) for the Not-to-Exceed Price. - A bid to buy a PTP Option or Flowgate Right cannot specify a negative Not-to-Exceed Price. A bid to buy a PTP Obligation can specify a negative Not-to-Exceed Price. - A bid to buy an FGR must specify the name of a flowgate. - A CRR bid for a specified MW quantity of CRRs constitutes a bid to buy a quantity of CRRs equal to or less than the specified quantity. The CRR System shall not allow a bid to specify a minimum quantity of MW that the CRR Account Holder wishes to buy. 7.5.2.3 |  |
| - The CRR System shall provide ability to validate CRR Auction Bids upon entry as follows: - A valid CRR Auction Bid is a CRR Auction Bid that ERCOT has determined meets the criteria listed above, CRR Auction Bid Criteria. - ERCOT shall continuously display on the MIS Certified Area information that allows any CRR Account Holder submitting a CRR Auction Bid to view its valid CRR Auction Bids. - At the point of submission, CRR System shall notify each CRR Account Holder of any of its CRR Auction Bids that are invalid. The CRR Account Holder may correct and resubmit any invalid CRR Auction Bid, if the submission window is still open. - 7.5.2.4 |  |

| Requirement Id | FR26 |
| --- | --- |
| Short Name | CRR Auction Optimization |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.5.3 (2)(a)(b) 7.3 (7)* 7.5.1(4)(a)* 7.5.1(5)(b) *CRR team understanding of the intention of the protocols based on all listed protocols combined |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall use a linear programming auction engine model for each CRR Auction that evaluates all CRR Auction Bids and CRR Auction Offers submitted, and selects a combination of CRR Auction Bids and CRR Auction Offers that: - Makes the solution simultaneously feasible in light of the ERCOT network capability as reflected in the CRR Network Model over the auction term; and - Maximizes the objective function, which is equal to the total economic value (as expressed in the CRR Auction Bids) of the awarded CRR Auction Bids, less the total economic cost (as expressed in CRR Auction Offers) of the cleared CRR Auction Offers, while observing the credit limit constraint and following SFT constraints: - Thermal operating limits (including estimates for Dynamic Ratings as defined in FR16) for transmission lines - Voltage and stability limits that are valid for the study period converted to thermal limits modeled as non-thermal/generic constraints; - ERCOT Transmission Grid pre-and post-contingency ratings; - List of transmission element contingencies expected to be enforced by ERCOT in Real-Time Operations Note: The auction process awards CRRs with equal effective price on a pro-rata basis based on the offered/bid upon amounts. If the bid/offer price is the same as the clearing price, then a partial amount shall be awarded (between zero and the MW amount of the bid) to all bidders or sellers of the particular product with equal effective price. 7.5.5.3 (2)(a)(b) |  |
| Based on a combination of the protocol references listed below*, the following is required of the CRR System: The CRR System shall enable ERCOT to conduct an annual auction, consisting of a two-year auction, simultaneously solving all monthly periods and all time of use blocks (i.e., peak weekday, peak weekend & off-peak) and the aggregated time of use, 24-hours. ]The CRR System shall enable ERCOT to conduct a monthly auction, consisting of a single month, simultaneously solving all time of use blocks (i.e., peak weekday, peak weekend & off-peak) and the aggregated time of use, 24-hours. 7.3 (7)* 7.5.1(4)(a)* 7.5.1(5)(b)* |  |

## Allocation

| Requirement Id | FR35 |
| --- | --- |
| Short Name | PCRR Allocation Process |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.4.1 (1)* *CRR team understanding of the intention of the protocols 7.4.2 (b)(d)(e)(f)(g) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| Based on a combination of the protocol references listed below*, the following is required of the CRR System: The CRR System shall enable ERCOT to allocate PCRRs to CRR Account Holders who have requested PCRRs and been approved, on the defined effective dates of the run. The CRR System shall provide an import feature to allow the CRR Market Operator to insert the list of PCRR entitlements to the system. These entitlements shall correspond to the list of existing agreements that are eligible to receive PCRRs. Note: ERCOT shall create a list of entitlements, representing NOIE pre-1999 generation contracts and their loads. These entitlements do NOT guarantee PCRRs, but rather act an individual “caps” in the allocation process for the nominations corresponding to the entitlements. The CRR System shall also impose “group” caps to limit the capacity factor for nominations for entitlements marked with the “capacity option”. The CRR System shall then take all the valid nominations and reduce them if the initial SFT is not feasible (see below). - As part of the entitlement creation process, the CRR Market Operator shall determine the capacity available to each zone for NOIEs with loads in multiple zones. For this entitlement the CRR Market Operator shall use the aggregated monthly load data from the corresponding prior 12 months. The CRR Account Holder shall nominate, as desired, PCRRs from their list of entitlements and specify the CRR type, either PTP Option, PTP Obligation, or a combination. Once the allocation process is complete, the eligible NOIE shall not change the CRR type. The nomination shall also include a desired nomination type (refund or capacity) for non-solid-fuel and non-combined-cycle resources. The selection is binding for purchase. The CRR System shall award PCRRs truncated to one-tenth of a MW (0.1 MW). 7.4.1* 7.4.2 (b)(f)(g) |  |
| - For PCRR allocation, the CRR System shall allow non-solid-fuel and non-combined-cycle resources the option between having their PCRR entitlements modeled as capacity or refund rights on a per entitlement basis. - Refund option – The eligible NOIE may nominate up to 100% of the lesser of the net unit capacity or contractual amount for those Resources. The eligible NOIE shall refund to ERCOT any congestion revenues received above those congestion revenues flowing to the NOIE for its Output Schedule of the Resource at the PCRR source. PCRR settlement will reflect the MW value of the Output Schedule of the Resource at the PCRR source, regardless of what MW value of actual output occurred during that interval if that change in output is in response to Dispatch Instructions. The refund for any Settlement Interval is equal to the difference between the PCRR MW amount and the time-weighted average of the Output Schedules of the Resource at the PCRR source multiplied by the value of that PCRR. PCRRs allocated under the refund option are not transferable and may only be used by the NOIE to which they are allocated. - Capacity option – The eligible NOIE may nominate up to 100% of the lesser of the net unit capacity or contractual amount for those Resources. ERCOT shall allocate PCRRs in accordance with the NOIE nominations subject to the SFT. - Before the applicable CRR Auctions, the NOIE must nominate the months for which it will use its PCRRs (i.e., the NOIE may shape the PCRRs representing up to 100% of the capacity for each Resource at a capacity factor no greater than 40%. This parameter shall be configurable by the CRR Market Operator to meet market Protocols. - Assumption: The 40% capacity factor described in the protocols will be evaluated at the time of annual nominations. A NOIE won’t be able to nominate (request PCRR allocations) in an amount that exceeds a 40% capacity factor. The resulting allocation will be less (if there are network constraints) than or equal (no constraints). Monthly reevaluation will not permit capacity allocations beyond those submitted at annual nomination process. - If a Resource eligible for PCRRs is shut down due to a Force Majeure Event, then, to the extent feasible, the NOIE may reallocate its PCRRs across its PCRR-eligible facilities before the next CRR Auction. This change is effective no later than the date of the next CRR Auction, and the redesignation may be requested for each monthly auction during the Force Majeure Event. Any price difference in the reconfigured rights must be paid by (or paid to) the NOIE. Assumption: Feasibility shall be determined by deleting the PCRRs affected by the Force Majeure Event and adding the requested reallocation. If the change results in infeasibility, the capacity of the new CRR will be reduced. If it can’t be reduced enough (all the way to zero) to make the system feasible, the change will be rejected. This shall be done as an off-line study. 7.4.2(e) |  |
| - For purpose of off-line study, the CRR System shall be able to export network and market data to a .txt file. - 7.4.2(e)* - *Result of the need adjust request amount due to infeasibility |  |
| - If at the time of the annual CRR allocation, ERCOT determines that PCRR nominations are not feasible, resulting in proportionally reduced PCRR allocations, then prior to each subsequent monthly CRR Auction, ERCOT shall re-evaluate the full nomination and allocate additional PCRRs, if feasible. - Assumption: Any additional PCRRs allocated as a result of monthly true-ups will be priced based upon the clearing price in the next monthly auction. - Note: In this section of the protocols, it is made clear that if the infeasibility occurs as a result of the annual allocation of PCRRs, versus the previously awarded CRRs, the solution will be to reduce the allocation of PCRRs and ‘true-up’ as transmission capacity becomes available in subsequent monthly auctions. If in the annual or monthly auction infeasibility is discovered as a result of previously allocated or awarded CRRs, the network model will be increased as described in FR15 - 7.4.2(d) |  |

| Requirement Id | FR36 |
| --- | --- |
| Short Name | PCRR Nomination Terms |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.4.2 (a) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| A PCRR nomination is a request for one-month strips of a NOIE-specified CRR type for amounts and time of use blocks specified by the NOIE for each month of the next auction following the allocation of PCRRs. 7.4.2 (a) |  |

| Requirement Id | FR39 |
| --- | --- |
| Short Name | Allocation SFT |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.4.2 (a) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall enable the following: - Conduction of studies to evaluate whether the nominated PCRRs comply with feasibility constraints using the simultaneous feasibility test. - SFT evaluation to determine the feasible PCRR allocation amount for each month being evaluated uses 100% of that month’s expected network topology, which may result in different amounts allocated in different months. The percentage of network topology available for each market shall be configurable. - If the SFT evaluation indicates that the nominated PCRR amounts are not feasible, then the CRR System shall proportionately reduce the requested PCRRs by their Impact Ratio on violated constraints. The “Impact Ratio” is the fraction a particular PCRR’s impact relative to the impact of all PCRRs in the same direction on a violated constraint. The nominated PCRR amount, adjusted for infeasibilities in the SFT evaluation if required, shall be the quantity of PCRRs allocated. 7.4.2 (a) |  |

| Requirement Id | FR46 |
| --- | --- |
| Short Name | PCRR Pricing |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.4.2 (a) 7.4.2 (h) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| - Description |  |
| The price that a NOIE must pay for an allocated PCRR is based on the corresponding CRR clearing price in the next auction following the allocation of PCRRs (detailed below). The invoicing and payment for allocated PCRRs shall follow the same process and timeline as the invoicing and payment of CRR bids cleared in the auction immediately following each PCRR allocation process. The allocated PCRRs for a NOIE are held in escrow by ERCOT and released to the NOIE by ERCOT in phases, as described in FR15D. 7.4.2 (a) |  |
| The payment for PCRRs shall be determined according to configurable percentages of the clearing price, differentiated by generator resource type. The CRR System shall apply that percentage after the auction following the allocation completes and prior to sending the auction results to the Settlements System. August 1, 2006 version of the protocols identifies the percentages as follows: - PTP Option PCRRs: - Nuclear, coal, lignite or combined-cycle Resources: 10% of the applicable CRR Auction clearing prices; - Gas steam Resources: 15% of the applicable CRR Auction clearing prices; or - Hydro, wind, simple cycle, or other Resources not included in (A) or (B): 20% of the applicable CRR Auction clearing prices. - PTP Obligation PCRRs: - Nuclear, coal, lignite or combined-cycle Resources: 5% of the applicable CRR Auction clearing price if it is positive; 100% of the applicable CRR Auction clearing price if it is negative; - Gas steam Resources: 7.5% of the applicable CRR Auction clearing price if such price is positive; 100% of the applicable CRR Auction clearing price if it is negative; or - Hydro, wind, simple cycle, or other Resources not included in (A) or (B): 10% of the applicable CRR Auction clearing prices if it is positive; 100% of the applicable CRR Auction clearing prices if it is negative. - For a NOIE that has chosen the refund option, the allocated number of PCRRs for Resources other than solid-fuel and combined-cycle Resources are provided at no charge. 7.4.2 (h) |  |

| Requirement Id | FR38 |
| --- | --- |
| Short Name | MCFRI Allocation Process |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.1 (2)(c) 7.7.2 (3)(5) 7.7.3 (1)(2) 7.7.1 |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall provide an import feature to allow the CRR Market Operator to insert the MCFRI nominations. These nominations shall correspond to the list of Wind Generation Resources in the McCamey Area that are eligible to receive FGRs. Eligible entities in the McCamey Area must: - Have demonstrated that the WGR is in commercial operation or is expected to be in commercial operation during the period for which MCFRIs will be allocated. This determination is made at ERCOT’s sole discretion; - Have significant impact on the most limiting local operational constraint; and - Be unable to operate their facilities at full capacity simultaneously with other WGRs in the McCamey Area, when all local transmission lines are in service, without violating ERCOT reliability criteria. The MCFRI candidates shall be defined as the ‘capacity impact’ of each McCamey Area WGR eligible for MCFRIs on each McCamey Area flowgate by multiplying the maximum rated output for the WGR times its Shift Factor from the base case transmission model for the corresponding McCamey Area flowgate relative to the Load-weighted average Shift Factor of all Electrical Buses in ERCOT. 7.1 (2)(c) 7.7.2 (3)(5) 7.7.3 (1)(2) |  |
| - All commercial and committed WGRs in the McCamey Area must be included in the analysis, but MCFRIs may be allocated only to CRR Account Holders who are also QSEs representing WGRs that are in commercial operation. - Note: The protocol 7.7.3 (2) references ‘QSEs’. A NPRR is needed to reflect that MCFRIs will be deposited to CRR Account Holders, not QSEs. - 7.7.3 (2) |  |
| - The CRR System shall be capable of no longer allocating MCFRIs at a time noted by the ERCOT Board. - Note: The ERCOT Board may vote to end MCFRI allocations 30 days after the sustainable export capability from the McCamey Area reaches or exceeds 900MW. - 7.7.1 |  |

| Requirement ID | FR15D |
| --- | --- |
| Short Name | Allocation Timeline |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.4.2 (c) 7.7.3 (3) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| - Description |  |
| - The CRR System shall allow for configuration of the date/days on which activities are performed. - The below timing and percentages shall be configurable in the CRR System: - The CRR System shall enable each eligible NOIE to nominate and ERCOT to allocate PCRRs to that NOIE within the parameters of the CRR Network model as applicable to the allocation process.. The timing and percentage of release of the allocations shall be as follows: - Until the first annual CRR Auction, NOIEs must nominate PCRRs for the month before each monthly auction. Nominations must be received at ERCOT 15 Business Days prior to the commencement of the monthly auction for the one-month term which the CRRs being auctioned are effective. ERCOT shall release 90% of allocated PCRRs to the NOIE ten Business Days prior to the corresponding monthly auction and release the remaining additional 10% of allocated PCRRs to the NOIE one Business Day after the close of the corresponding monthly auction. - For the first annual CRR Auction, the NOIE must nominate PCRRs for each month of the following two years before the first annual CRR Auction. Nominations must be received at ERCOT 30 Business Days prior to the commencement of the annual auction. ERCOT shall release 15% of allocated PCRRs for all months of the second year of the annual auction and 55% of the allocated PCRRs for all months of the first year of the annual auction to the NOIE 25 Business Days prior to the annual auction. ERCOT shall allocate an additional 35% of allocated PCRRs for all months of the first year of the annual auction to the NOIE one Business Day after the close of the annual auction and allocate the remaining additional 10% of allocated PCRRs for each month to the NOIE one Business Day after the close of the corresponding monthly auction. - For all subsequent annual CRR Auctions, the NOIE must nominate PCRRs for each month of the second year before each annual CRR Auction. Nominations must be received at ERCOT 30 Business Days prior to the commencement of the annual auction. ERCOT shall release 15% of allocated PCRRs for all months of the second year of the annual auction and 40% of the allocated PCRRs for all months of the first year of the annual auction to the NOIE 25 Business Days prior to the annual auction. ERCOT shall release an additional 35% of allocated PCRRs for all months of the first year of the annual auction to the NOIE one Business Day after the close of the annual auction and release the remaining additional 10% of allocated PCRRs for each month to the NOIE one Business Day after the close of the corresponding monthly auction. 7.4.2 (c) |  |
| - The CRR System shall allocate 90% of McCamey Area flowgate capacity, adjusted for previously awarded PCRRs, MCFRIs and CRRs, 25 days prior to the annual auction and 10 business days prior to the monthly auction. This percentage shall be configurable in the CRR System. - Note: The Nodal Protocol 7.7.3 (3) references activity in the DAM. DAM activities referenced in this section are outside of the CRR System scope as envisioned in this document. - 7.7.3 (3) |  |

## BILATERAL Market

| Requirement Id | FR40 |
| --- | --- |
| Short Name | Bilateral Trading Rules |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.4.2 (i) 7.8 |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall not allow PCRRs to be bilaterally traded through ERCOT systems prior to the completion of the CRR Auction used to determine their value. Assumption: MCFRIs shall be permitted to be bilaterally traded through the ERCOT system prior to the completion of the CRR auction and may be traded immediately subsequent to their allocation because they are not tied to a clearing price. 7.4.2 (i) |  |
| The CRR System shall enable ERCOT to use the following Bilateral Trades rules: - CRR Account Holders may sell or trade PTP Options, PTP Obligations and FGRs bilaterally,. PTP Options with Refund and PTP Obligations with Refund are not tradable and the system shall not allow them to be traded. - The characteristics of the CRRs sold or traded bilaterally, excluding owner, start/end dates and MW amount, may not be modified from the original CRR for purposes of trading within the CRR system. - The CRR System shall keep a record of current and all previous owners of CRRs - A transfer of CRRs through the ERCOT CRR System is not effective until the selling CRR Account Holder reports the transaction, the buying CRR Account Holder acknowledges the transaction, and both parties meet ERCOT’s credit requirements to support the transfer. Until all of those occur, the selling CRR Account Holder is considered the CRR Owner for purposes of the Protocols, including financial responsibility. - For CRR ownership to be effective in the DAM, the CRR must be registered through the ERCOT CRR System prior to the Day-Ahead Market. - PTP Obligations acquired in DAM shall not change ownership in the ERCOT CRR System after DAM execution. - The CRR System shall have a cut-off time for bilateral trading to be recognized for a later business day. This time is determined by the time to check credit of the involved CRR Account Holders and the valuation of the CRR as well as the time to transfer information to the DAM. Assumption: The cut-off time will not be later than 0600 the day before the effective day and may be as much as 4 days prior to the effective day. 7.8 |  |

| Requirement Id | FR42 |
| --- | --- |
| Short Name | Credit Check of Bilateral Trades |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.8 (4) |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| CRR shall send proposed bilateral transactions to the Credit Monitoring group for approval. The transaction shall be put in a pending state until all credit checks are complete. CRR shall receive confirmation or denial of trade and activate or deny trade within the CRR System. Assumption: The CRR process shall check that the credit impact of a bilateral trade to both parties involved is acceptable to the ERCOT credit requirements. This shall include the impact of buying an obligation as well as selling an option or a positively valued obligation. The transaction shall be put in a pending state until all credit checks are complete. Assumption: Transactions may be in a pending state for up to 3 business days. Note: Credit check requirement for bilateral trading is an outstanding question to the Credit group regarding Credit System capability. 7.8 (4) |  |

# Supplementary Requirements

The Supplementary requirements capture the system requirements that are not readily captured in the functional use cases.

## Performance Requirements

| Requirement ID | SR1 |
| --- | --- |
| Short Name | Timing of CRR Markets |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall meet the following CRR Execution Timing Requirements: - Monthly ERCOT shall run - Allocation of PCRRs, Allocation of MCFRIs and CRR Auction - Annually ERCOT shall run - Allocation of PCRRs, Allocation of MCFRIs and CRR Auction The monthly auction optimization shall complete (end of bidding to award validation) within 40 hours. The annual auction optimization shall complete (end of bidding to award validation) within 80 hours. The bidding window shall be open and capable of accepting bids on the 2nd Tuesday of each month for the Monthly Auction. The bidding window must be open and capable of accepting bids on the 1st Tuesday of November for the Annual Auction. This timing shall be configurable within the CRR System. |  |

| Requirement ID | SR2 |
| --- | --- |
| Short Name | UI Performance Requirements |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall meet the following CRR UI Performance Requirements: The user interface response time is defined as the elapsed time from user’s request for a display until the requested display is presented complete with all requested information. Some examples of CRR System performance are presented in table below. UI Performance Matrix CRR UI Page; Response Time*. Landing Page; 1-5 sec. List of Auctions; 5-10 sec. Contingencies List; 15-30 sec. Assumption: The CRR System shall respond to users in a reasonable period of time.- Service Level Agreements of CRR components will be further elaborated during the development of the CRR System in accordance with ERCOT Enterprise Guidance. * - Includes time for backend processing |

## Sizing Requirements

| Requirement ID | SR3 |
| --- | --- |
| Short Name | CRR Calculation Engine Dimensions |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR Calculation engine shall be configured to accommodate ERCOT’s network and the CRR bid market and optimization solution latency time. The main program dimensions are given in the table below: Calculation Engine Dimensions 1 Object; Dimension. Buses; 7,500. Branches; 10,000. Contingencies; 3000. Sources and Sinks; 2000. These are NOT system boundaries; expansion of the CRR Network Model is supported but may have an effect on performance. These dimensions shall be configurable and may be modified during the course of the project as the network model data becomes available. Assumption: To meet the 40/80-hour optimization latency objective the following are suggested dimensions: Calculation Engine Dimensions 2 Object; Dimension. PTP bids and offers; 30,000. Flowgates; 40. Note: The CRR Team recognizes that to keep the total number of bids accepted below 30,000, limits would have to be invoked per Nodal Protocols section 7.5.2 (2). An additional trade-off would be to limit amount of option bids to a much smaller proportion of the total number of bids/offers. Note: As described in the Nodal Program Charter, the CRR System and process is entirely new to ERCOT. The above sizing requirements reflect the best estimate of foreseeable needs. |

| Requirement ID | SR4 |
| --- | --- |
| Short Name | CRR Database Sizing |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall meet the following CRR Database Sizing Requirements: The CRR Database subsystem shall support the following storage requirements: - CRR Transactional storage of 250 GB - CRR portion in DW and/or ODS – 1 TB* - CRR Archive – 1 TB* This is designed to cover immediate needs as well as expected growth for next 10 years. * - not CRR System requirement, applicable to EDW |  |

| Requirement ID | SR5 |
| --- | --- |
| Short Name | Simultaneous Users |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source | TBD |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall accommodate 10 MOI and 100 MUI simultaneous users without degrading performance. |  |

## Legal and Regulatory requirements

| Requirement ID | SR6 |
| --- | --- |
| Short Name | Data Retention |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| All CRR allocation, auction and bilateral trade (registered with ERCOT) data shall be stored in accordance with ERCOT corporate standards for data retention. Assumption: Data retention for the CRR System shall be 5-years, beyond 5-years will be managed by EDW. |  |

## System Integration Requirements

| Requirement ID | SR7 |
| --- | --- |
| Short Name | Integration Layer |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR application shall interact with other ERCOT systems through the integration layer (this could be using either point-to-point or Enterprise Service Bus solutions). The CRR application shall specify endpoints for these interactions to be connected into the ESB by the Integration project. Assumption: Connecting end points to the integration layer is the responsibility of Nodal Integration Project. |  |

### Data Exchange

| Requirement ID | SR24 |
| --- | --- |
| Short Name | CRR Import Data |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) |  |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall import and the following data coming from upstream systems and validate that it is acceptable to the system data requirements: - CRR Network Models including: - Transmission Elements - Transmission impedances - Transmission ratings including dynamic ratings where available - Other selected assumptions and inputs used for the CRR Network Model including non-thermal/generic constraints - Note: The non-thermal/generic constraints shall be applied to the CRR Network Model Base Case. Non-thermal/generic constraints shall be expressed uni-directionally. - Lists of Contingencies - CRR Settlements File including Sources and Sinks - Outages (this information may be embedded in the CRR Network Model) - Flowgates definitions - CRR Account Holder list - Credit Limit information for CRR Account Holders - Auction transaction payment confirmation list - Approval of bilateral transactions |  |

| Requirement ID | SR25 |
| --- | --- |
| Short Name | Data Export |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall export the following data to downstream systems: - Daily ownership of all CRRs for the next effective day - Market (Auction / Allocation) transactions - Bilateral transactions |  |

| Requirement ID | SR26 |
| --- | --- |
| Short Name | Market Transaction Data |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source | Full |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The Market Transaction (definable as the transaction list for the entire market or the transactions per individual Account Holder) shall include: - All CRRs bought in the auction - All CRRs sold in the auction - All CRRs allocated---including PCRRs and MCFRIs - For each transaction the CRR System shall include the following fields: - CRR ID - CRR owner identification - Source and sink (if PTP Option or PTP Obligation) - Flowgate (if Flowgate Right) - MW amount - Clearing price (units of $/mw/hr) - PCRR PTP pricing factor (for auctioned CRRs this is 1.0 ) - Transaction Amount (MW amount * Number of Hours * Clearing Price * PCRR pricing multiplication factor) - CRR type: Option, Obligation, or Flowgate - Time of use block - Start Date for which CRR is effective - End Date last date on which CRR is effective - With Refund, without Refund |  |

| Requirement ID | SR27 |
| --- | --- |
| Short Name | Interface Subsystem Requirements |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall meet the following Interface Subsystem Requirements: Data Exchanges with other ERCOT systems shall be facilitated by CRR Data Interface Subsystem and ERCOT Integration Layer. Fully functional CRR data interface subsystem shall provide the following: - APIs and/or service enabled interfaces - Ability to produce XML documents - Ability to manually upload files (XML and CSV documents) - CRR Input/Output staging tables* * - optional |  |

| Requirement ID | SR28 |
| --- | --- |
| Short Name | Data Exchange Frequency |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| CRR System shall interact with other Texas Nodal components with the frequencies as shown in the table below. Assumption: Mechanism for data exchanges shall be in place. Frequency of CRR Interactions System; Frequency. From Credit Management; Daily. To Credit Management; Daily (CRR ownership inventory) On-Demand (Auction results). To Day Ahead Market; Daily. From Registration System; Daily. To Settlement System; Daily. From Network Model Management System; On-Demand (expected monthly). From Settlement; Daily. To MIS; On-Demand. To EDW; Daily. |

## System Security Requirements

| Requirement ID | SR8 |
| --- | --- |
| Short Name | Security requirements |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System application level security shall comply with ERCOT Security policies and guidance as described by EA Security Architecture team. The CRR System shall be accessed by CRR Market Operators only through the Market Operator Interface in accordance with ERCOT security policies. Assumption: Access to the CRR System by CRR Account Holders shall only be through the Market User Interface via the ERCOT Certified MIS. Such access shall include the ability for CRR Account Holders to view information related to CRRs currently in their accounts, including Source, Sink, MW, Effective Date for each CRR. ERCOT Certified MIS access shall be controlled in accordance with ERCOT security policies. The CRR System shall leverage the portal security and digital certificate features for ERCOT. CRR System shall support data confidentiality and prevent unauthorized users, external processes or devices from accessing the CRR information in accordance with ERCOT security policies. Assumption: Communication between CRR client components (browser/applet) and CRR server components shall conform to ERCOT security policies and shall be secure between the client and the portal. |  |

## Back up and Recovery Requirements

| Requirement ID | SR9 |
| --- | --- |
| Short Name | Back Up and Recovery |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| All CRR System data shall be backed up daily and recoverable within 24 hours, in accordance with ERCOT IT policies. |  |

## Availability and Redundancy Requirements

| Requirement ID | SR10 |
| --- | --- |
| Short name | Availability and Redundancy |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System hardware shall be available 24/7 subject to scheduled maintenance announced via the outage notification process. CRR System hardware shall meet all availability requirements across CRR components. CRR System is not considered mission critical and therefore shall be available at least 99.9% of the time (8.76 hours of downtime per year). |  |

## Portability and Scalability

| Requirement ID | SR11 |
| --- | --- |
| Short Name | Scalability |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| CRR System shall support scalability from users, process and sizing perspective in an integrated way without changing of design and without requiring new development. |  |

## System Monitoring

| Requirement ID | SR12 |
| --- | --- |
| Short Name | System Monitoring |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| CRR shall provide means for integration with enterprise level monitoring and management solution HP Open View for proactive management and support in compliance with ERCOT IT standards. CRR System shall surface business or system significant alerts, errors and exceptions in a consistent manner (defined as needing timely human interaction). All surfaced messages shall include a concise description of the problem. Optional: Error codes and corrective actions (error handling processes) |  |

## Maintainability Requirements

| Requirement ID | SR13 |
| --- | --- |
| Short Name | Maintainability Requirements |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall meet the following Maintenance and Support Requirements: The CRR System shall be maintained by ERCOT staff and CRR System Supplier according to the CRR System maintenance and support agreement that will be finalized during the development and deployment. The list of maintenance activities to be performed by ERCOT and CRR System supplier shall be elaborated in the agreement. SLAs and explicit support will be elaborated during detailed design and development. |  |

| Requirement ID | SR14 |
| --- | --- |
| Short name | Administrative Functions |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall provide several administration functionalities such as: Single point of control to manage application functions (start / stop) Deployment support (installation instructions) Tracking of the optimization process Manual data import mechanism as a backup for data coming from other ERCOT systems Assumption: All processes and services required for CRR production shall be initiated at servers’ startup and managed by ERCOT IT staff. This includes JBOSS application server, Web Server, and database server. |  |

## Usability Requirements

| Requirement ID | SR15 |
| --- | --- |
| Short name | Usability Requirements |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR User Interface (MOI and MUI) shall comply with Texas Nodal Guideline for Effective User Experiences and Design Guide to the extent supported by the CRR off-the-shelf UI configuration capabilities (minimum or zero coding). Assumption: The Nexant CRR solution for ERCOT consists of 50% off the shelf functionality from the i-HEDGE integrated Web solution. This solution provides a pre-existing web/applet based rich user interface. Deviations from the ERCOT User Interface best practices and guidelines by the CRR System shall be discussed and resolved with the ERCOT Enterprise Architecture team in advance of CRR application modification for the ERCOT Market. |  |

| Requirement Id | SR29 |
| --- | --- |
| Short name | CRR User Messaging |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source | Partial |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall provide functionality that shall enable CRR Market Operators to issue public and private messages to CRR User. The messaging functionality shall be capable of attaching files including but not limited to the following types: - Microsoft Office - Compressed files - PDF - AutoCAD files - Image - Text, including XMLand PSS/E RAWD files At a minimum, the system must be capable of handling attachments up to 5MB in size |  |

## Date and Time Requirements

| Requirement ID | SR16 |
| --- | --- |
| Short name | Daylight Savings Time |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| - Spring Daylight Savings Time Operating Day: Hour Ending 0300 shall be skipped, resulting in 23 hourly intervals for the day. - Fall Daylight Savings Time Operating Day: Hour Ending 0200 shall be repeated, resulting in 25 hourly intervals for the day. |  |

| Requirement ID | SR17 |
| --- | --- |
| Short Name | Time Format |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall meet the following Date / Time Formats requirements: - Date / Time stored in the database shall be prevailing Central Time Zone represented as offset from UTC or ‘Z’. All timestamps will be in this predefined time zone as described in the W3C standard specified in the W3C document “XML Schema Part 2: Data Types Second Edition, Appendix D ISO 8691 Date and Time formats - All Date / Time stamp information presented to the CRR Market Operator or the CRR Market User shall be in Central prevailing time. |  |

## Documentation

### CRR Account Holder Documentation

| Requirement ID | SR18 |
| --- | --- |
| Short Name | User Guide |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 7.5.3 (3) |
| Coverage of Source | Full |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System supplier shall provide a full user guide that shall be distributed to CRR Account Holders to help them use and understand the system. [Reliant: Please note that this will comprise only a part of the guide requirement referenced in 7.5.3(3).] |  |

### Operation Documentation

| Requirement ID | SR19 |
| --- | --- |
| Short name | Delivery /Installation instructions |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| The CRR System shall meet the following Deliver to Operations Requirements: - The CRR System and subsequent updates shall be delivered to Operations in a specified directory structure in order to facilitate incorporation into the system, according to the release management processes. - An installation script or executable wrapper or precise instructions are required for deployment. The script/executable wrapper or instructions must provide install, uninstall and repair functionality. |  |

| Requirement ID | SR20 |
| --- | --- |
| Short Name | IT Operations documentation |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| CRR System shall meet the following Operations Documentation Requirements: - New releases or updates shall provide documentation to guide the installation, identify new functionality or repaired functionality and requirements for installation. - Recommended maintenance activities and schedules such as index rebuilds should be included. Application recovery steps and procedures must be provided, if applicable. |  |

| Requirement ID | SR21 |
| --- | --- |
| Short Name | General IT documentation |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| General Documentation Requirements - Release notes shall include system requirements known and/or probable issues and recommended actions to take in order to address the issues, - Design documentation: describes the purpose of the component and each of its subcomponents; how the components interface within the CRR System and with other applications - Database Descriptions: Entity Relationship Diagram describing the relationships between tables and the properties of the tables. - User Interfaces: describe the logical characteristics of each interface between the software product and the users. This may include sample screen images, GUI standards, functions that appear on every screen, error message display standards. |  |

## Training

| Requirement ID | SR22 |
| --- | --- |
| Short name | Training |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| In order to prepare all users of CRR System properly a formal training on the use of the CRR System functionality is required. The preferred training should be in the form of classroom instruction and simulated auctions. CRR vendor shall provide: - Appropriate training materials and operational procedures that clearly describe all details related to the use of CRR System including the Market Operator Interface, Market User Interface, Bilateral Trades, auction system as well as and other related components. - The training will include combination of classroom instruction for all users and trainers (‘train the trainer’ classes) along with training materials. ERCOT will lead CRR training for Market Participants. CRR solution provider will support and attend all training as necessary. Details (curriculum and schedule) of each training session will be defined in cooperation with nodal training team. |  |

## ERCOT IT Standards

| Requirement ID | SR23 |
| --- | --- |
| Short Name | IT standards |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process Maps | TBD |
| Coverage of Sub-Process | TBD |
| Description |  |
| CRR System shall comply with the following ERCOT IT standards (subject to verification): - Application Servers - JBoss 4.0.3 - Server Hardware - IBM AIX P Series - Database - Oracle 10g - Operating systems - IBM AIX 5.X - Windows 2003 on the server - Windows XP on Client - Web Browser - Internet Explorer v6 or higher |  |