Commercial Systems

Data Aggregation Requirements

V0.09

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 08/07/06 | 0.01 | Initial Draft | Calvin Opheim |
| 08/31/06 | 0.011 | Updated Draft | Calvin Opheim |
| 9/11/06 | .012 | Comments from Data Aggregation Team | Calvin Opheim |
| 10/3/06 | 0.013 | Comments from Nodal Requirements Review Meeting and other changes | Calvin Opheim |
| 10/4/06 | 0.02 | Comments from Nodal Requirements Review Meeting | Calvin Opheim |
| 10/18/06 | 0.03 | Comments from Nodal Requirements Review Meeting | Calvin Opheim |
| 10/18/06 | 0.09 | Additional changes for submittal to TPTF | Calvin Opheim |

Table of Contents

1.	Data Aggregation	3

1.1	Proposed System Scope	3

1.2	Document Conventions	3

1.3	Document Assumptions	3

2.	Functional Requirements	3

2.1	ERCOT Total Adjusted Metered Load (RTAMLTOT)	3

2.2	Real-Time Adjusted Metered Load per Settlement Point per QSE (RTAML)	3

2.3	QSE Load Ratio Share for a 15-Minute Settlement Interval (LRS)	3

2.4	QSE Load Ratio Share for an Operating Hour (HLRS)	3

2.5	Monthly Load Ratio Share per QSE (MLRS)	3

2.6	Monthly Load Ratio Share Zonal per QSE (MLRSZ)	3

2.7	Metered Energy at Bus (MEB)	3

2.8	Net Meter Real-Time Energy Total (NMRTETOT)	3

2.9	Real-Time Metered Generation per QSE per Settlement Point per Resource (RTMG)	3

2.10	Block Load Transfer Resource per QSE per Settlement Point per BLT point (BLTR)	3

2.11	Generation Resource Capacity Factor Calculation (GRCF)	3

3.	Supplementary Requirements	3

3.1	Performance Requirements	3

3.2	Legal and Regulatory requirements	3

3.3	System Security Requirements	3

3.4	Back up and Recovery Requirements	3

3.5	Availability and Redundancy Requirements	3

3.6	Maintainability Requirements	3

3.7	Daylight Savings Time shall be recognized in Nodal Market settlements.	3

4.	Protocol Coverage	3

Software Requirements Specification

# Data Aggregation

## Proposed System Scope

ERCOT plans to implement a new Nodal market in 2009.  Section 6 of the Nodal Protocols describes the Settlement Calculations for the Real-Time Energy Operations.  Included in these settlement calculations are data aggregation determinants which are a required input.  Sections 3, 4, and  6 versions dated 9/1/2006 were used to create these requirements.  A draft NPRR that was created at the 8/21/06 TPTF meeting from TXU was also used in sections 2.7 and 2.8.  Currently this NPRR has not been assigned a number.

This document defines the data aggregation determinants that are required for Settlement Calculations for the Real-Time Energy Operations.  The requirements in scope for this document include all data aggregation determinants for all real-time energy settlement calculations.

The following Data Aggregation determinants are included in real-time energy settlement and are included in the scope of this document:

- Real-Time Adjusted Metered Load per QSE per Settlement Point (RTAML) (PR 6.6.2.1, PR 6.6.2.2, PR 6.6.2.3, PR 6.6.3.2)

- Real-Time Adjusted Metered Load Total (RTAMLTOT) (PR 6.6.2.1, PR 6.6.2.2,  PR 6.6.2.3)

- Load Ratio Share per QSE (LRS) (PR 6.6.2.2, PR 6.6.5.4, PR 6.6.7.2, PR 6.6.9.2 (though it’s listed as LSR), PR 6.6.10 (though it’s listed as LSR)

- Hourly Load Ratio Share per QSE (HLRS) (PR 6.6.2.3, PR 6.6.6.5, PR 6.6.8.2)

- Real-Time Metered Generation per QSE per Settlement Point per Resource (RTMG) (PR 6.6.3.1, PR 6.6.6.1,  PR 6.6.6.2 (1), PR 6.6.6.2 (2), PR 6.6.7.1 (2.b), PR 6.6.9.1)

- Block Load Transfer Resource per QSE per Settlement Point per BLT point (BLTR) (PR 6.6.3.5)

- Generation Resource Capacity Factor calculation (GRCF) (PR 4.4.9.4.1(c), PR 4.4.9.4.1(d)) PR 3.7.1.1 (2)

## Document Conventions

- All text lifted from the Nodal Protocols is in italic font.

- All requirements are identified using FR1, FR2, etc.

- All dependencies are identified using D1, D2, etc.

- All assumptions are identified using A1, A2, etc.

- The “Original Source” column of the Input, Intermediate, and Output Bill Determinates tables (refer to Section 2, Functional Requirements) delineate the initial source of market and/or operational information required to construct each variable.  The data transformation process exists within the interface between the Settlement System and Market / Operational Systems.  A separate and distinct set of Business Requirements will address all data interfacing between the original source system and the Settlement System.

## Document Assumptions

A1)	All existing zonal Data Aggregation bill determinants will continue to be produced in the nodal system unless noted otherwise in this document.

A2)	All ESI-ids will need to be mapped to substations which are mapped to Load Zones (i.e., no change to the current process).

A3)	All EPS meters will need to be mapped to a bus.

A4)	The Settlement System should not store zero value data cuts for a Data Aggregation bill determinant, unless the zero data cut is a result of a calculation or a requirement specifies that the cut be saved.

A5)	The Data Aggregation business requirements may be subject to additional requirements for Credit Monitoring purposes.

A6)	The charge Data Aggregation bill determinant calculations are documented using the proposed format.  The following list defines the nomenclature used for the bill determinants though, not all of these bill determinant elements may apply to each bill determinant within this document):

<Q> = QSE

<CO> = CRR Owner

<R> = Resource

<SP> = Settlement Point

<SRSP> = Source Settlement Point

<SKSP> = Sink Settlement Point

<ST> = Start Type

<F> = Flowgate

<E> = Directional Network Element

<C> = Constraint Identifier

<M> = Market

<RUC> = RUC Process

<REP> = Retail Electric Provider

<PI> = Profile ID

<LC> = Loss Code

<UFE> = UFE Zone

<CM> = Congestion Management Zone

<TDSP> = Transmission Distribution Service Provider

<PGC> = Power Generation Company

<SUFE> = UFE Analysis Zone / Sub-UFE Zone

<GSC> = Generator Site Code

<GUC> = Generator Unit Code

<BLTP> = Block Load Transfer Point

<NC> = Noie Code

# Functional Requirements

## ERCOT Total Adjusted Metered Load (RTAMLTOT)

The ERCOT Total Adjusted Metered Load (RTAMLTOT) will need to be calculated for each settlement interval.  The ERCOT Total Adjusted Metered Load (RTAMLTOT) will be used in both hourly and 15-minute load ratio share calculations.

It should be noted that in the zonal market, a bill determinant named LTOTERCOT represents the same value as the nodal RTAMLTOT bill determinant.  In order to avoid having duplicate data, the LTOTERCOT bill determinant will not be created when the nodal market is implemented.

| Requirement ID | FR1 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.2.1, PR 6.6.2.2, PR 6.6.2.3 |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate ERCOT Total Adjusted Metered Load (RTAMLTOT). ERCOT total Adjusted Metered Load (excluding the DC Tie export associated with the QSEs under the “Oklaunion Exemption”) for a 15-minute Settlement Interval is calculated as follows: - RTAMLTOT =qspRTAMLQ>SP> RTAMLTOT will be calculated by summing all of the RTAML data cuts. |  |

The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RTAML | MWh | Real-Time Adjusted Metered Load per QSE per Settlement Point—The sum of the Adjusted Metered Load at the Electrical Buses that are included in Settlement Point p, represented by QSE q, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The RTAML cut will be saved in the form of RTAML_<Q>_<SP>. RTAML includes: - DC Tie Exports (RTDCEXP) except those that have been granted the Oklaunion exemption, - Block Load Transfer Loads (BLTL), and - Load Resources |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RTAMLTOT | MWh | Real-Time Adjusted Metered Load Total—The total Adjusted Metered Load in ERCOT, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The RTAMLTOT cut will be saved in the form of RTAMLTOT. |  |  |  |

### Data Dependencies

D1)	Calculation of the RTAMLTOT is dependent upon the availability of the input bill determinants.

D2)	Calculation of all RTAML load cuts must be complete prior to the calculation of the RTAMLTOT.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR2 | PR 6.6.2.1, | TBD | DRIVER: RTAML Sum all RTAML load cuts. |
| FR3 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR4 | PR 12.3 (c) | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - RTAMLTOT – public - RTAML - private |
| FR5 | Internal | TBD | The LTOTERCOT bill determinant will not be created when the nodal market is implemented. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR6 | Internal | TBD | A FATAL ERROR should be written and processing should stop for this job and all dependent jobs if no RTAML load cuts exist for each load zone for the particular operating day. The error should state “No RTAML cuts were found for load zone x”, where x is replaced by the appropriate load zone code. |

### Specific Database Needs - None

## Real-Time Adjusted Metered Load per Settlement Point per QSE (RTAML)

The Real-Time Adjusted Metered Load (RTAML) will be calculated for each settlement interval.  Real-Time Adjusted Metered Load (AML) includes retail load usage data that has been adjusted for Unaccounted for Energy (UFE), Transmission Losses, Distribution Losses, and DC Tie exports (except for the Oklaunion Exemption)(PR 2.1).

It should be noted that in the zonal market, a bill determinant named LQSECMUFE represents the same value as the RTAML with one exception.  The LQSECMUFE zonal cut includes load associated with the Oklaunion exemption.  RTAML does not include DC Tie exports under the Oklaunion exemption.  In order to avoid having duplicate data, the LQSECMUFE bill determinant will not be created when the nodal market is implemented.

| Requirement ID | FR7 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.2.1, PR 6.6.2.2, PR 6.6.2.3, PR 6.6.3.2 |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate Real-Time Adjusted Metered Load per Settlement Point per QSE (RTAML). The sum of the Adjusted Metered Load at the Electrical Buses that are included in Settlement Point p, represented by QSE q, for the 15-minute Settlement Interval. Note: RTAML will be calculated by summing all LSEGUFE cuts for Settlement Point p, represented by QSE q, for the 15-minute Settlement Interval. - RTAML_<Q>_<SP> =qcmLSEGUFEREP>Q>PI>LC>UFE>CM>TDSP> RTAML will be calculated by summing LSEGUFE data cuts for each QSE and CM Zone combination. The CM Zone attribute in the LSEGUFE data cut is equivalent to the Settlement Point attribute of the RTAML data cut. The summation will not include LSEGUFE cuts associated with DC Tie exports under the Oklaunion Exemption. |  |

The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| LSEGUFE | MWh | Retail load usage data that has been adjusted for Unaccounted for Energy (UFE), Transmission Losses, Distribution Losses, and DC Tie Exports (except for the Oklaunion Exemption). | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The LSEGUFE cut is saved in the form of LSEGUFE_<REP>_<Q>_<PI>_<LC>_<UFE>_<CM>_<TDSP>. [NOTE: ERCOT has been careful not to use determinants from the zonal market in the nodal market, which helps to avoid confusion. The LSEGUFE cut exists in the zonal market today and it seems will also exist in nodal. Is there a reason this determinant is being re-used in nodal?] |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RTAML | MWh | Real-Time Adjusted Metered Load per QSE per Settlement Point—The sum of the Adjusted Metered Load at the Electrical Buses that are included in Settlement Point p, represented by QSE q, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The RTAML cut will be saved in the form of RTAML_<Q>_<SP>. RTAML includes DC Tie Exports not under the Oklaunion exemption, Block Load Transfer Loads (BLTL), and Load Resources. |  |  |  |

### Data Dependencies

D1)	Calculation of the RTAML is dependent upon the availability of the input bill determinants.

D2)	Calculation of all GSITETOT cuts must be complete prior to the calculation of RTAML.

D3)	Calculation of all LSEGUFE load cuts must be complete prior to the calculation of the RTAML.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR8 | Internal | TBD | DRIVER: LSEGUFE. The RTAML calculation should only be attempted for QSEs that have load in the settlement system for the Operating Day |
| FR9 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR10 | PR 12.3 (c) | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - RTAML – private - LSEGUFE – private |
| FR11 | Internal | TBD | The LQSECMUFE bill determinant will not be created when the nodal market is implemented. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR12 | Internal | TBD | A FATAL ERROR should be written and processing should stop for this job and all dependent jobs if no LSEGUFE cuts exist for the particular Operating Day. The error should state “No LSEGUFE cuts were found for Operating Day mm/dd/yyyy”, where: - mm is the month with 01 representing January, 02 representing February, …, 11 representing November, and 12 representing December, - dd is the day with 01 representing the first day of the month, 02 representing the second day of the month, …, 30 representing the thirtieth day of the month, and 31 representing the thirty first day of the month, - and yyyy is the year. |

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR13 | Internal | TBD | The Settlement System needs to be able to map, and store the mapping of, each ESIID to a Load Zone. This should be accomplished by mapping an ESIID to a substation and having substations mapped to Load Zones. |

## QSE Load Ratio Share for a 15-Minute Settlement Interval (LRS)

A Load Ratio Share (LRS) will be calculated for each QSE for each Settlement Interval.

| Requirement ID | FR14 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.2.2, |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate QSE Load Ratio Share for a 15-Minute Settlement Interval (LRS). Each QSE’s Load Ratio Share for a 15-minute Settlement Interval is calculated as follows: LRS_<Q> = (ΣspRTAML_<Q>_<SP>) / RTAMLTOT LRS will be calculated by summing all RTAML data cuts for each QSE and dividing by RTAMLTOT. |  |

The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| RTAML | MWh | Real-Time Adjusted Metered Load per QSE per Settlement Point—The sum of the Adjusted Metered Load at the Electrical Buses that are included in Settlement Point p, represented by QSE q, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The RTAML cut will be saved in the form of RTAML_<Q>_<SP>. |  |  |  |
| RTAMLTOT | MWh | Real-Time Adjusted Metered Load Total—The total Adjusted Metered Load in ERCOT, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The RTAMLTOT cut will be saved in the form of RTAMLTOT. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| LRS | Misc | Load Ratio Share per QSE—The Load Ratio Share as defined in Section 2, Definitions and Acronyms, for QSE q, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The LRS cut will be saved in the form of LRS_<Q>. LRS will be stored as a decimal with the maximum number of decimal precision supported by its data type. |  |  |  |

### Data Dependencies

D1)	Calculation of the LRS is dependent upon the availability of the input bill determinants.

D2)	Calculation of all RTAML load cuts must be complete.

D3)	Calculation of the RTAMLTOT load must be complete.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR15 | Internal | TBD | DRIVERS: RTAMLTOT and RTAML. The LRS calculation should only be performed for QSEs that have load in the settlement system for the Operating Day (i.e., calculation will only be performed for QSEs that have a RTAML data cut). |
| FR16 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR17 | PR 12.3 (c) | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - LRS – private - RTAML - private - RTAMLTOT - public |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR18 | Internal | TBD | A FATAL ERROR should be written and processing should stop for this job and all dependent jobs if RTAMLTOT does not exist or if RTAMLTOT has a zero value for any 15-minute interval. If the RTAMLTOT cut does not exist, the error should state “No RTAMLTOT cut was found for Operating Day mm/dd/yyyy”. If any interval of the RTAMLTOT cut has a zero value, the error should state “RTAMLTOT cut has a zero value for Operating Day mm/dd/yyyy in interval hh:mm ” where: - mm is the month with 01 representing January, 02 representing February, …, 11 representing November, and 12 representing December, - dd is the day with 01 representing the first day of the month, 02 representing the second day of the month, …, 30 representing the thirtieth day of the month, and 31 representing the thirty first day of the month, - yyyy is the year, and - hh:mm is the hour and minute of the settlement interval with 01:mm representing the first hour, 02:mm representing the second hour, …, 23:mm representing the twenty third hour, and 24:mm representing the twenty fourth hour, and hh:15 representing the settlement interval ending fifteen minutes after hour hh, hh:30 representing the settlement interval ending 30 minutes after hour hh, hh:45 representing the settlement interval ending 45 minutes after hour hh, and hh:00 representing the settlement interval ending on hour hh. |

### Specific Database Needs - None

## QSE Load Ratio Share for an Operating Hour (HLRS)

A Hourly Load Ratio Share (HLRS) will be calculated for each QSE for each Operating Hour.

It should be noted that in the zonal market, a bill determinant named LRSTDQSE represents the same value as the HLRS with one exception.  The LRSTDQSE zonal cut has values that are rounded to 6 decimal places; while the HLRS nodal cut will have values that are not rounded.  In order to avoid having duplicate data, the LRSTDQSE bill determinant will not be created when the nodal market is implemented.

| Requirement ID | FR19 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.2.3 |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate QSE Load Ratio Share for an Operating Hour (HLRS). Each QSE’s Load Ratio Share for an Operating Hour is calculated as follows: HLRS_<Q> = (i4spRTAMLQ>SP>i ) / ( i4RTAMLTOTi ) HLRS will be calculated as follows: - Sum all RTAML data cuts for each QSE. This will result in each QSE having a 15-min data cut which represents their total RTAML. - Convert the quantities for step 1 above from 15-min data cut to an hourly data cut. This cut will represent a QSEs hourly RTAML. - Convert the RTAMLTOT 15-min data cut into an hourly data cut. - Divide each QSEs hourly RTAML values by the hourly RTAMLTOT to arrive at the HLRS. |  |

The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| RTAML | MWh | Real-Time Adjusted Metered Load per QSE per Settlement Point—The sum of the Adjusted Metered Load at the Electrical Buses that are included in Settlement Point p, represented by QSE q, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The RTAML cut will be saved in the form of RTAML_<Q>_<SP>. |  |  |  |
| RTAMLTOT | MWh | Real-Time Adjusted Metered Load Total—The total Adjusted Metered Load in ERCOT, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The RTAMLTOT cut will be saved in the form of RTAMLTOT. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| HLRS | Misc | Hourly Load Ratio Share per QSE—The Load Ratio Share as defined in Section 2, Definitions and Acronyms, for QSE q, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: The HLRS cut will be saved in the form of HLRS_<Q>. HLRS will be stored as a decimal with the maximum number of decimal precision supported by its data type. |  |  |  |

### Data Dependencies

D1)	Calculation of the HLRS is dependent upon the availability of the input bill determinants.

D2)	Calculation of all RTAML load cuts must be complete.

D3)	Calculation of the RTAMLTOT load must be complete.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR20 | Internal | TBD | DRIVER: RTAMLTOT and RTAML. The HLRS calculation should only be performed for QSEs that have load in the settlement system for the Operating Day (i.e., calculation will only be performed for QSEs that have a RTAML data cut). |
| FR21 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR22 | PR 12.3 (c) | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - HLRS – private - RTAML - private - RTAMLTOT - public |
| FR23 | Internal | TBD | The LRSTDQSE bill determinant will not be created when the nodal market is implemented. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR24 | Internal | TBD | A FATAL ERROR should be written and processing should stop for this job and all dependent jobs if RTAMLTOT does not exist or if RTAMLTOT has a zero value for any 15-minute interval. If the RTAMLTOT cut does not exist, the error should state “No RTAMLTOT cut was found for Operating Day mm/dd/yyyy”. If any interval of the RTAMLTOT cut has a zero value, the error should state “RTAMLTOT cut has a zero value for Operating Day mm/dd/yyyy in interval hh:mm ” where: - mm is the month with 01 representing January, 02 representing February, …, 11 representing November, and 12 representing December, - dd is the day with 01 representing the first day of the month, 02 representing the second day of the month, …, 30 representing the thirtieth day of the month, and 31 representing the thirty first day of the month, - yyyy is the year, and - hh:mm is the hour and minute of the settlement interval with 01:mm representing the first hour, 02:mm representing the second hour, …, 23:mm representing the twenty third hour, and 24:mm representing the twenty fourth hour, and hh:15 representing the settlement interval ending fifteen minutes after hour hh, hh:30 representing the settlement interval ending 30 minutes after hour hh, hh:45 representing the settlement interval ending 45 minutes after hour hh, and hh:00 representing the settlement interval ending on hour hh. |

### Specific Database Needs - None

## Monthly Load Ratio Share per QSE (MLRS)

The Monthly Load Ratio Share per QSE (MLRS) is calculated based on using the calendar month’s 15-minute peak load Settlement Interval.  This calculation will be performed for only initial and final settlements.  The Monthly Load Ratio Share (MLRS) is used in the CRR settlement process.

| Requirement ID | FR25 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 7.9.3.5 |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate Monthly Load Ratio Share per QSE (MLRS). Monthly Load Ratio Share per QSE—The Load Ratio Share calculated for QSE for the 15-minute monthly peak-load Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval, for the calculation of LRS for a 15-minute Settlement Interval. MLRS_<Q> = LRS_<Q> m, i Where: - q = QSE, - m = calendar month, and - i = 15-minute peak-load Settlement Interval MLRS will be calculated as follows: For Initial Settlements: - Verify that all of the previous calendar month’s initial settlements have been approved. - Determine the 15-min peak load Settlement Interval of the previous calendar month. This will be accomplished by finding the maximum value of the RTAMLTOT data cut. [Note: Earlier in the document, it said “the LTOTERCOT bill determinant will not be created when the nodal market is implemented” so this determinant cannot be used.] - Retrieve all LRS values for the 15-min peak load Settlement Interval. Save these values as MLRS_<Q> (which are based on initial settlements). For Final Settlements: - Verify that final settlements have been approved for a complete calendar month. - Determine the 15-min peak load Settlement Interval for the calendar month for which all final settlements have been approved. This will be accomplished by finding the maximum value of the RTAMLTOT data cut. - Retrieve all LRS values for the 15-min peak load Settlement Interval. Save these values as MLRS_<Q> (which are based on final settlements). |  |

The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| LRS | Misc | Load Ratio Share per QSE—The Load Ratio Share as defined in Section 2, Definitions and Acronyms, for QSE q, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The LRS cut will be saved in the form of LRS_<Q>. LRS will be stored as a decimal with the maximum number of decimal precision supported by its data type. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| MLRS | Misc | Monthly Load Ratio Share per QSE—The Load Ratio Share calculated for QSE for the 15-minute monthly peak-load Settlement Interval. | Settlement System |
| Interval Frequency: 1 / month Additional Comments: The MLRS value will be stored in the factor value table with QSE as an attribute. |  |  |  |

### Data Dependencies

D1)	Calculation of the MLRS is dependent upon the availability of the input bill determinants.

D2)	All of the previous calendar month’s initial settlements have been approved before performing the calculation of MLRS based on using initial settlement data.

D3)	All of the final settlements have been approved for a complete calendar month before performing the calculation of MLRS based on using final settlement data.

D4)	The 15-minute peak-load Settlement Interval for the calendar month will need to be identified along with the date and the time of the peak.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR26 | Internal | TBD | DRIVER: All initial settlements must be approved for the prior calendar month. |
| FR27 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR28 | Internal | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - LRS – private - MLRS – private |

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR29 | Internal | TBD | DRIVER: All final settlements must be approved for a calendar month. |
| FR30 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR31 | Internal | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - LRS – private - MLRS – private |

### Business Defined Errors – None

### Specific Database Needs - None

## Monthly Load Ratio Share Zonal per QSE (MLRSZ)

The Monthly Load Ratio Share Zonal per QSE per CMZ (MLRSZ) is calculated based on using the calendar month’s 15-minute peak load Settlement Interval.  This calculation will be performed for only initial and final settlements.  The Monthly Load Ratio Share Zonal (MLRSZ) is used in the CRR auction process.

| Requirement ID | FR32 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 7.5.7 |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate Monthly Load Ratio Share Zonal per QSE (MLRSZ). Monthly Load Ratio Share Zonal per QSE per zone—The LRS of QSE for its Load in CMZ, for the peak-Load 15-minute Settlement Interval in the calendar month. Note: The MLRSZ value will be calculated as follows: MLRSZ_<Q>_<SP> = RTAML_<Q>_<SP> m, i / RTAMLTOT m, i Where: - q = QSE, - p = Settlement Point, - m = calendar month, and - i = 15-minute peak-load Settlement Interval MLRSZ will be calculated as follows: For Initial Settlements: - Verify that all of the previous calendar month’s initial settlements have been approved. - Determine the 15-min peak load Settlement Interval of the previous calendar month. This will be accomplished by finding the maximum value of the RTAMLTOT data cut. - Retrieve all RTAML values for the 15-min peak load Settlement Interval. - Retrieve the RTAMLTOT value. - Divide each RTAML value by the RTAMLTOT value. Save these values as MLRSZ_<Q>_<SP>. For Final Settlements: - Verify that final settlements have been approved for a complete calendar month. - Determine the 15-min peak load Settlement Interval for the calendar month for which all final settlements have been approved. This will be accomplished by finding the maximum value of the RTAMLTOT data cut. - Retrieve all RTAML values for the 15-min peak load Settlement Interval. - Retrieve the RTAMLTOT value. - Divide each RTAML value by the RTAMLTOT value. Save these values as MLRSZ_<Q>_<SP>. |  |

The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| RTAML | MWh | Real-Time Adjusted Metered Load per QSE per Settlement Point—The sum of the Adjusted Metered Load at the Electrical Buses that are included in Settlement Point p, represented by QSE q, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The RTAML cut will be saved in the form of RTAML_<Q>_<SP>. |  |  |  |
| RTAMLTOT | MWh | Real-Time Adjusted Metered Load Total—The total Adjusted Metered Load in ERCOT, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The RTAMLTOT cut will be saved in the form of RTAMLTOT. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| MLRSZ | Misc | Monthly Load Ratio Share Zonal per QSE per zone—The LRS of QSE for its Load in CMZ, for the peak-Load 15-minute Settlement Interval in the month. | Settlement System |
| Interval Frequency: 1 month Additional Comments: The MLRSZ value will be stored in the factor value table with QSE and Settlement Point as attributes. |  |  |  |

### Data Dependencies

D1)	Calculation of the MLRSZ is dependent upon the availability of the input bill determinants.

D2)	All of the previous calendar month’s initial settlements have been approved before performing the calculation of MLRSZ based on using initial settlement data.

D3)	All of the final settlements have been approved for a complete calendar month before performing the calculation of MLRSZ based on using final settlement data.

D4)	The 15-minute peak-load Settlement Interval for the calendar month will need to be identified along with the date and the time of the peak.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR33 | Internal | TBD | DRIVER: All initial settlements must be approved for the prior calendar month. |
| FR34 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR35 | Internal | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - RTAML – private - RTAMLTOT - public - MLRSZ – private |

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR36 | Internal | TBD | DRIVER: All final settlements must be approved for a calendar month. |
| FR37 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR38 | Internal | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - RTAML – private - RTAMLTOT - public - MLRSZ – private |

### Business Defined Errors – None

Specific Database Needs - None

## Metered Energy at Bus (MEB)

A calculation is performed for each settlement interval for each meter associated with a Generation Site in order to determine if power is being injected or withdrawn at the metering point.  A draft NPRR that was created at the 8/21/06 TPTF meeting from TXU was also used in this section.  Currently this NPRR has not been assigned a number.  The billing determinant named MEB replaces the MEEPS bill determinant mentioned in the draft NPRR.

| Requirement ID | FR39 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.1 |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate Metered Energy at Bus (MEB). Metered Energy at busThe metered energy by the settlement meter m at Electrical Bus b for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy consumed. MEB_<GSC>_<B> = RID_<GSC>_<B> |  |

The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| RID | MWh | Resource ID for the generation site which measures energy delivered to the grid (denoted by a positive value) or energy consumed from the grid (denoted by a negative value). | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: RID will be saved in the form of RID_<GSC>_<B>. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| MEB | MWh | Metered Energy at BusThe metered energy by the settlement meter me at Electrical Bus b for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy consumed. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: MEB cut will be saved in the form of MEB_<GSC>_<B>. |  |  |  |

### Data Dependencies

D1)	Calculation of the MEB is dependent upon the availability of the input bill determinants.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR40 | PR 7.5.7 | TBD | DRIVER: Generation Sites. The MEB calculation should be performed for every active generation site in the settlement system for the Operating Day. |
| FR41 | Internal | TBD | Add generation site code and a bus as attributes to Resource Ids. |
| FR42 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR43 | Internal | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - Resource IDs – private - MEB – private |

### Business Defined Errors - None

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR44 | Internal | TBD | A new relational table will need to be added which will store Buses. Resource IDs will be assigned a Bus. |

## Net Meter Real-Time Energy Total (NMRTETOT)

A calculation is performed for each settlement interval for every Generation Site in order to determine whether the Generation Site is injecting power to the grid or withdrawing power from the grid.  The total injection measured from each meter is summed to arrive at the total gross injection to the grid on an interval by interval basis.  The total withdrawal measured from each meter is summed to arrive at the total gross withdrawal from the grid on an interval by interval basis.  The total gross injection is reduced by the total gross withdrawal to arrive at the Net Meter Real-Time Energy Total.

A draft NPRR that was created at the 8/21/06 TPTF meeting from TXU was also used in this section.  Currently this NPRR has not been assigned a number.

| Requirement ID | FR45 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.1 |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate Net Meter Real-Time Energy Total (NMRTETOT). Net Meter Real-Time Energy Total— The net sum for all settlement meters included in a net metering arrangement. A positive value indicates an injection of power to the ERCOT system and a negative value indicates a withdrawal of power from the ERCOT system. NMRTETOT_<GSC> i = 1mSMinjection - 1mSMwithdrawal ) Where: - m = total number of settlement meters at a generation site and - i = 15-minute Settlement Interval |  |

The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| SM | MWh | Settlement Meter – a meter which is included as part of a generation site that is used in a net metering arrangement. These meters are referred to as Resource Ids in the settlement system. | Settlement System |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| NMRTETOT | MWh | Net Meter Real-Time Energy Total— The net sum for all settlement meters included in a net metering arrangement. A positive value indicates an injection of power to the ERCOT system and a negative value indicates a withdrawal of power from the ERCOT system | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: NMRTETOT cut is saved in the form of NMRTETOT_<GSC>. |  |  |  |

### Data Dependencies

D1)	Calculation of the NMRETETOT is dependent upon the availability of the input bill determinants.

D2)	Successful completion of the Data Aggregation Generation Aggregation Process.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR46 | Internal | TBD | DRIVER: Generation Sites. The NMRTETOT calculation should be performed for every active generation site in the settlement system for the Operating Day. |
| FR47 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR48 | Internal | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - Resource IDs – private - NMRTETOT – private |

### Business Defined Errors - None

### Specific Database Needs - None

## Real-Time Metered Generation per QSE per Settlement Point per Resource (RTMG)

The Real-Time Metered Generation (RTMG) will be calculated for each generating unit for each settlement interval.  Real-Time Metered Generation (RTMG) includes net generation from all registered generation units.  Real-Time Metered Generation (RTMG) does not include DC Tie imports or Block Load Transfer Resources (BLTR).

It should be noted that in the zonal market, a bill determinant named GSITETOT represents the same value as the RTMG with three exceptions:

- The GSITETOT zonal cut is created for DC Tie imports,

- The GSITETOT zonal cut is created for Block Load Transfer Resources (BLTR), and

- The GSITETOT zonal cut is named using different attributes than the RTMG data cut.

The GSITETOT bill determinant will continue to be created as part of the nodal market. [NOTE: ERCOT has been careful not to use determinants from the zonal market in the nodal market, which helps to avoid confusion. The GSITETOT cut exists in the zonal market today and it seems will also exist in nodal. Is there a reason this determinant is being re-used in nodal?]

| Requirement ID | FR49 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.1, PR 6.6.6.1, PR 6.6.6.2(1), PR 6.6.6.2(2), PR 6.6.7.1(2b), PR 6.6.9.1 |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate Real-Time Metered Generation per QSE per Settlement Point per Resource (RTMG). The Real-Time energy produced by the Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. Note: RTMG will be calculated as follows: - RTMG_<Q>_<R>_<P> = GSITETOT_<Q>_<GSC>_<NC>_<PGC>_<UFE>_<CM>_<TDSP>_<SUFE>_<GUC> - RTMG will be calculated as follows: - A single GSITETOT cut is created for each generating unit with the unit being designated by the <GUC> attribute. The <GUC> attribute of the GSITETOT cut is the same as the Generation Resource <R> attribute in the RTMG cut. - The GSITETOT cut will have a Resource Node attribute added (<P>). The <P> attribute of the GSITETOT cut is the same as the Resource Node <P> attribute in the RTMG cut. |  |

- The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| GSITETOT | MWh | Total Net Generation for a Unit. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The GSITETOT cut is saved in the form of GSITETOT_<Q>_<GSC>_<NC>_<PGC>_<UFE>_<CM>_<TDSP>_<SUFE>_<GUC>_<P> |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RTMG | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource—The Real-Time energy produced by the Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: RTMG cut will be saved in the form of RTMG_<Q>_<R>_<P>. |  |  |  |

### Data Dependencies

D1)	Calculation of the RTMG is dependent on:

- availability of the input bill determinants

- successful completion of the Data Aggregation Generation Aggregation Process

D2)	The RTMG calculation must be complete prior to the calculation of RTAML.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR50 | Internal | TBD | DRIVER: GSITETOT. The RTMG calculation should only be performed for QSEs that have generation in the settlement system for the Operating Day. The RTMG calculation will only be performed for QSEs that have a GSITETOT data cut that is the result of the generation site aggregation process. GSITETOT cuts that are created as part of the DC Tie aggregation process and Block Load Transfer Resource (BLTR) aggregation process are not included in the RTMG calculation. |
| FR51 | Internal | TBD | Add Resource Node as an attribute to the GSITETOT cut. This will allow all generation units to be mapped to a Resource Node. |
| FR52 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR53 | PR 12.3 (c) | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - RTMG – private - GSITETOT – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR54 | Internal | TBD | A FATAL ERROR should be written and processing should stop for this job and all dependent jobs if no GSITETOT cuts exist for the particular trade day. The error should state “No GSITETOT cuts were found for operating day x”, where x is replaced by the appropriate operating day. |

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR55 | Internal | TBD | A new relational table will need to be added which will store Resource Nodes. Generating Units will be assigned a Resource Node. |

## Block Load Transfer Resource per QSE per Settlement Point per BLT point (BLTR)

Block Load Transfer Resource (BLTR) will be calculated for each settlement interval.

It should be noted that in the zonal market, a bill determinant named GSITETOT is created for Block Load Transfer Resources (BLTR).  GSITETOT data cuts that are associated with Block Load Transfer Resources (BLTR) will be summed to create the BLTR data cut.  The GSITETOT bill determinant will continue to be created as part of the nodal market.

| Requirement ID | FR56 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.5 |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate Block Load Transfer Resource per QSE per Settlement Point per BLT point (BLTR). The energy delivered to an ERCOT Load in Load Zone p through BLT Point bltp represented by QSE q, for the 15-minute Settlement Interval. Note: BLTR will be calculated as follows: - BLTR q, p, bltp = qrpGSITETOTq,r,p - BLTR will sum GSITETOT values which are associated with Block Load Transfer Resouces. |  |

- The above variables are defined below:

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| GSITETOT | MWh | Total Net Generation for a Unit. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The GSITETOT cut is saved in the form of GSITETOT_<Q>_<GSC>_<NC>_<PGC>_<UFE>_<CM>_<TDSP>_<SUFE>_<GUC>_<P> |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| BLTR | MWh | Block Load Transfer Resource per QSE per Settlement Point per BLT Point—The energy delivered to an ERCOT Load in Load Zone p through BLT Point bltp represented by QSE q, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The BLTR cut will be saved in the form of BLTR_<Q>_<SP>_<BLTP> |  |  |  |

### Data Dependencies

D1)	Calculation of the BLTR is dependent on

- availability of the input bill determinants

- successful completion of the Data Aggregation Generation Aggregation Process

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR57 | Internal | TBD | DRIVER: BLTR. The BLTR calculation should only be attempted for QSEs with an active Block Load Transfer Resource in the settlement system for the Operating Day. |
| FR58 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR59 | Internal | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - BLTR – private - GSITETOT – private |

### Business Defined Errors - None

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR60 | Internal | TBD | A flag will be added to the generator site history table (y/n) to indicate BLTR. |

## Generation Resource Capacity Factor Calculation (GRCF)

The previous 12 months’ capacity factor must be updated by ERCOT by the 20th day of each month using the most recent data for use in the next month. ERCOT shall post to the MIS Secure Area the capacity factor for each Resource before the start of the effective month.

| Requirement ID | FR61 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.4.9.4.1), PR 3.7.1.1 (2) (a) |
| Traceability to Sub-Process Maps | TBD |
| Description: Calculate Generation Resource Capacity Factor. - GRCF will be calculated as follows: - Sum the GSITETOT cuts for the previous 12 calendar months. This value is the total net output for the generation resource. - Multiply the capacity rating of the generation resource by the number of hours in the preceeding 12 calendar months. This value represents the continuous full power output for the generation resource. - Divide the total net output by the continuous full power output to arrive at the Generation Resource Capacity Factor. - Determine the Mitigated Offer Cap Multiplier as described in section 4.4.9.4.1, Mitigated Offer Cap. |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| GSITETOT | MWh | Total Net Generation for a Unit. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: The GSITETOT cut is saved in the form of GSITETOT_<Q>_<GSC>_<NC>_<PGC>_<UFE>_<CM>_<TDSP>_<SUFE>_<GUC>_<P> |  |  |  |
| SEASMW | MW | Seasonal MW rating reported as a Generation Resource Parameter. | Settlement System |
| Interval Frequency: 1 / season Additional Comments: There shall be a start/stop time for each season. This bill determinant is not in the Nodal Protocols. |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| GRCF | Misc | Generation Resource Capacity Factor. | Settlement System |
| Interval Frequency: 1 / month Additional Comments: The GRCF value will be stored in the factor value table with GUC as an attribute. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| MOCMULT | Misc | Mitigated Offer Cap Multiplier. | Settlement System |
| Interval Frequency: 1 / month Additional Comments: This determinant shall be made available for the MMS system. |  |  |  |

### Data Dependencies

D1)	Calculation of the GRCF is dependent on

- availability of the input bill determinants

- All initial settlements must be approved for the prior calendar month.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR62 | Internal | TBD | DRIVER: All initial settlements must be approved for the prior calendar month. |
| FR63 | Internal | TBD | No input or output bill determinants should be rounded or truncated. |
| FR64 | Internal | TBD | All input and output bill determinants should be made available to the extracts process and should be classified in the settlement system as: - GSITETOT – private - GRCF – private |

### Business Defined Errors – None

### Specific Database Needs - None

# Supplementary Requirements

## Performance Requirements

Lodestar must be able to perform its full set of Nodal settlement calculations. Performance characteristics will be determined and detailed in the conceptual and detailed design.

## Legal and Regulatory requirements

All Lodestar financial data must be saved in accordance with standard accounting principles as dictated by the ERCOT Texas Nodal Protocols.

## System Security Requirements

Lodestar will only be accessed directly by internal ERCOT personnel either through thin client UI, Citrix environment, or thick client on employee desktops. Market Participants will be able to access their statements and invoices on ERCOT private web site. ERCOT private web site access will be controlled by digital certificate authentication in accordance with ERCOT security policies.  Additional details will be provided in the Conceptual System Design.

## Back up and Recovery Requirements

All Lodestar Nodal settlement data must be backed up on a daily basis and recoverable within 24 hours.  Additional details will be provided in the Conceptual System Design.

## Availability and Redundancy Requirements

Nodal Lodestar system must be available 24/7. Lodestar is considered a tier 1 system and critical to base ERCOT business processes.  Additional details will be provided in the Conceptual System Design.

## Maintainability Requirements

Lodestar Nodal system will be maintained by internal ERCOT personnel. Service Level Agreements and explicit support will be detailed in the detail design document.

## Daylight Savings Time shall be recognized in Nodal Market settlements.

Spring Daylight Savings Time Operating Day:

- Hour Ending 0300 will be skipped, resulting in 23 hourly intervals for the day.

- The four fifteen minute intervals within Hour Ending 0300 will be skipped, resulting in 92 fifteen minute intervals for the day.

- The sixty one minute intervals within Hour Ending 0300 will be skipped, resulting in 1380 total one minute intervals.

Fall Daylight Savings Time Operating Day:

- Hour Ending 0200 will have two (2) sets of data, resulting in 25 hourly intervals for the day.

- The four fifteen minute intervals within Hour Ending 0200 will have two (2) sets of data, resulting in 100 fifteen minute intervals for the day.

- The sixty one minute intervals within Hour Ending 0200 will have two (2) sets of data, resulting in 1500 total one minute intervals.

# Protocol Coverage

Since one protocol may translate into many requirements, Project Managers are required to identify the coverage extent (Full/Partial) of all relevant Nodal Protocols that are mentioned in this requirements document.

| Protocol Sub-Section # (To the lowest level of granularity as possible) | Coverage by Requirements (Full/Partial) |
| --- | --- |
| PR 4.4.9.4.1 (c) | Partial |
| PR 4.4.9.4.1 (d) | Full |
| PR 6.6.2.1 | Full |
| PR 6.6.2.2 | Full |
| PR 6.6.2.3 | Full |
| PR 6.6.3.1 | Partial |
| PR 6.6.3.2 | Partial |
| PR 6.6.3.5 | Partial |
| PR 6.6.5.4 | Partial |
| PR 6.6.6.1 | Partial |
| PR 6.6.6.2 (1) | Partial |
| PR 6.6.6.2 (2) | Partial |
| PR 6.6.6.5 | Partial |
| PR 6.6.7.1 (2b) | Partial |
| PR 6.6.7.2 | Partial |
| PR 6.6.8.2 | Partial |
| PR 6.6.9.1 | Partial |
| PR 6.6.9.2 | Partial |
| PR 6.6.10 | Partial |