Use Case – Create NOMCR

COPS.P01ModelManageData_UseCase_CreateNOMCR_V0.9

Name: Create NOMCR for Incremental Model Update

Summary:

Create Data change file for all additions, deletions or modifications that apply to the Network, Generation or SCADA models of ERCOT. This file is attached to a Network Operations Model Change Request form that jointly constitute the NOMCR.  The creation may be done by logging in the thin client and directly entering data into NMMS using the graphic interface or sending changes in a CIM/XML file format.

Acronyms:

MC		ERCOT Model Coordinator

ERCOT	Electric Reliability Council of Texas

NOMCR	Network Operations Model Change Request (AKA: Project Files)

NMMS		Network Model Management System

CIM		Common Information Model

XML		Extensible Markup Language

IEC		International Electro-technical Committee

MIS		Market Information System

MP		Market Participant

Actor(s):

| Name | Role description |
| --- | --- |
| ERCOT Model Coordinator (the receiving entity) – Network Modeling Group | NOMCRs containing incremental changes are received from the Market Participants for Processing. ERCOT MC will enter any data that needs to be manually entered using the graphic editor |
| MP(the sending entity) | Generate NOMCR attaching the NOMCR form with the changes submitted in the form of CIM/XML, an approved Non-CIM format or directly entering the data in IMM using graphic editor. These CIM/XML Incremental files should meet the specifications of IEC 61970-552-4. These updates may include adds, deletes or changes to the Network, SCADA, or Generation models. |

Participating Systems:

| System | Services or information provided |
| --- | --- |
| The EMS systems or third party tools at the MP. If the MP does not elect to purchase a tool to generate the CIM/XML file, the file must be generated by hand (see issue 1 below). | Identify changes to the model(s) that will occur three (3) months or more from the relative date per the schedule defined in Nodal Protocols Section 3.10.1. Enter the changes directly in NMMS in one of two ways: - Log into the thin client and use the graphic editor - Create a CIM/XML incremental file(s) containing the incremental changes for each scheduled deployment date Attach the associated changes with a NOMCR form to complete the creation of a NOMCR. Create a separate NOMCR with the associated CIM/XML incremental file for each change that will be implemented on a given date. |
| The NMMS System at ERCOT | If changes are in the form of CIM/XML, the import of changed data occurs at NMMS with automatic validation of entered data changes. If the changes are entered using graphical interface saving of data changes will result in automatic validation of data entered in NMMS Send a notification of receipt within 5 Business Days. |
| Thin client for MP login | This provides a secure login to MP (NOMCR submitter), generates an NOMCR and allows entry of the changes into NMMS |

Pre-conditions:

The MP has access to an NMMS thin client to enter changes into the ERCOT system

Design Considerations:

NONE

Examples of incremental model changes are:

- Add a transformer

- Add new line or modify existing line

- Add, delete or move a load

Sample files containing the above listed changes are located on the CIM/XML e-group under the following names:

  - co_acline_mod.rdf – contains incremental changes to modify an ac line segment

  - co_acline_add.rdf – contains incremental changes to add an ac line segment

  - co_load_add.rdf – contains incremental changes to add a load

  - co_pt_add.rdf - contains incremental changes to add a transformer

  - co_load_move.rdf - contains incremental changes to move a load

  - co_load_del.rdf - contains incremental changes to delete a load

Other examples of incremental model changes are:

- Replace existing transformer with a new transformer with different ratings

- Delete an existing line

- Change rating or setting

Known assumptions, limitations, constraints, or variations that may affect this use case:

- MP  creates a NOMCR using graphic editor. MP will save changes into NMMS and complete level 1 validation.

- The MP  creates a NOMCR with a CIM/XML file, but does not use the graphic interface to get data into the NMMS.  MP will enter this data into NMMS and complete level 1 validation.

- Other data that MP fails to enter will be entered by ERCOT MC using the graphic editor in NMMS

Normal Sequence:

| Use Case Step | Description | From - To | Information Content |
| --- | --- | --- | --- |
| Step 1 | MP logs in thin client | (from) at MP (to) thin client |  |
| Step 2 | MP opens the graphic interface and selects a Substation in ERCOT NMMS system | (from) MP (to) NMMS | Graphic interface |
| Step 3 | MP makes necessary modifications – Edit/Delete/Add an equipment using the graphic interface and saves the changes in ERCOT NMMS system | (from) MP (to) NMMS | Graphic interface with modifications |
| Step 4 | NMMS performs the level 1 automatic Validation as data is saved. This includes connectivity and data range checking against the data changes saved. If the validation is not successful an error file is generated and MP makes corrections in entered data using graphic interface until there is no validation error | (from) MP (to) NMMS | Graphic interface with modifications |
| Step 5 | After successful validation is done for all required changes of that NOMCR, MP submits the NOMCR | (from) MP (to) NMMS | NOMCR with data changes already in NMMS |
| Step 6 | NMMS generates a CIM/XML file on submission of the NOMCR and creates a directory as specified to store that file. | (from) NMMS (to) a specified folder | NOMCR along with CIM/XML file stored in the specified directory |

Exceptions / Alternate Sequences:

1. MP submits a NOMCR with attached CIM/XML file

| Use Case Step | Description | From - To | Information Content |
| --- | --- | --- | --- |
| Step 1b | MP logs into thin client | (from) at MP (to) thin client |  |
| Step 2b | MP creates a NOMCR form and uses a CIM/XML file to create the data changes | (from) MP (to) NMMS | NOMCR form and Data changes in a CIM/XML file format |
| Step 3b | MP imports the CIM/XML file in NMMS | (from) MP (to) NMMS | CIM/XML file |
| Step 4b | NMMS performs the level 1 automatic Validation as CIM/XML file is imported. This includes connectivity and data range checking against the NOMCR and CIM/XML file, performs automated CIM/XML semantic and syntax checking for the NOMCR | NMMS | model data changes |
| Step 5b | After a successful validation, MP attaches the CIM/XML file with NOMCR form and NOMCR is created. MP submits this NOMCR. Please refer to Use Case COPS P01 ModelManageData_UC_ProcessNOMCR.doc If the validation is not successful MP corrects the CIM/XML file until an error free validation is successfully complete. | (from) MP (to) ERCOT MC | NOMCR with CIM/XML file attached |

Post-conditions:

ERCOT Model Coordinator has to verify all data changes are entered into IMM and are in sync.

References:

Use Cases referenced by this use case, or other documentation that clarifies the requirements or activities described.

- COPS P01 ModelManageData_UC_ProcessNOMCR.doc

The following Standards and documents are referenced by this case:

- IEC 61970-552-4, CIM/XML Model Exchange Format Rev3b 040904 Standard

- IEC 61970-501, CIM/XML RDF Schema

- ERCOT Protocol documents

- ERCOT NMMS Requirements document

Issues:

| ID | Description | Status |
| --- | --- | --- |
|  | There are a few software tools that can generate and accept a CIM/XML Incremental file. However, some MPs may need to manually generate the CIM/XML incremental input files. Some examples of these files are located on the CIM/XML e-group (http://groups.yahoo.com/group/cimxml/). The IEC 61970-552-4 standard provides the requirements should an MP wish to create their own tool. |  |
|  | Any data that MP fails to enter will be entered by ERCOT MC using the graphic editor |  |

Revision History:

| No | Date | Author | Description |
| --- | --- | --- | --- |
| 1 | 8/7/06 | M. Sengupta | Use case initial version based on current information |
| 2 | 8/10/06 | M. Sengupta | Use case edited based on further information |
| 3 | 8/25/06 | M. Sengupta | Added corrections based on Margaret’s correction, deleted steps that need to go to Process NOMCR |
| 4 | 9/5/06 | M. Sengupta | Modified and corrected based on changes done in business process of ERCOT as informed in the meeting of Aug 28-31 |
| 5 | 9/5/06 | M. Sengupta | Modified again based on telephonic discussion with Margaret |
| 6 | 9/6/06 | M. Goodrich | Review changes and added final revisions |
| 7 | 9/6/06 | M. Sengupta | Accepted final review comments of Curtis and increased version |
| 8 | 9/10/06 | M. Goodrich | Consistency Edits and prepared for ERCOT internal review. |
| 9 | 9/11/06 | M. Goodrich | Final edits from NMG |