Use Case – Create CRR Model

COWM.P05ConductCRRMarket_UC_CreateCRRModel_V0.4

Name: Create the Congestion Revenue Rights Model

Summary:

Create the Congestion Revenue Rights Model using the NMMS software and the selected NOMCRs, PMCRs and Outages as required.  SAMRs will also be selected and included in the model package.  The output is a RAWD file.  The CRR Contingency File and the CRR Settlement File should also be packaged and sent with this model if required.

Acronyms:

ERCOT	Electric Reliability Council of Texas

CRRS		Congestion Revenue Rights System

MP		Market Participant

NMMS		Network Model Management System

NOMCR	Network Operations Model Change Request (AKA: Project Files)

MC		ERCOT Model Coordinator

SAMR		Special Action Model Request

TSP		Transmission Service Provider

CU		CRRS User

Actor(s):

| Name | Role description |
| --- | --- |
| CU at ERCOT | Selects the Approved and Scheduled NOMCRs and SAMRs to be included in the CRR Model build. |
| CU at ERCOT | Select the Temperature for the Dynamic Rating processing |
| CU at ERCOT | Select the Outages to be included in the CRR Model |
| CU at ERCOT | Select the Approved PMCRs to be included in the CRR Model |

Participating Systems:

| System | Services or information provided |
| --- | --- |
| Congestion Revenue Rights System (CRRS) | Receives the Model after the Case Builder completes the CRR Model build |
| NMMS at ERCOT | The CU uses the Case Builder within the NMMS to build the CRR Model using the selected NOMCRs, SAMRS, Outages, Temperature, and PMCRs. The CRR Model is generated as a RAWD file. |

Pre-conditions:

NONE

Design Considerations:

- None

Known assumptions, limitations, constraints, or variations that may affect this use case:

- The CRR Model will be generated each month 45 days prior to the proposed auction month

- The NOMCRs, SAMRs, Outages, PMCRs and Dynamic Ratings that will be applied include those up to and including the first day of the proposed auction month.

- The CRR User will create this model using the Case Builder.

- Topology Processing must be executed prior to inserting the PMCRs into the Model.

Normal Sequence:

| Use Case Step | Description | From - To | Information Content |
| --- | --- | --- | --- |
| Step 1 | Based on inputs from the CU, the NMMS software inserts the selected NOMCRs into the Network Operations Model | (from) NMMS to (to) NMMS |  |
| Step 2 | Based on inputs from the CU, the NMMS software selects the SAMRs that will be included in the model package | (from) NMMS to (to) NMMS |  |
| Step 3 | Based on inputs from the CU, the NMMS software inserts the selected Outages into the Network Operations Model | (from) NMMS to (to) NMMS |  |
| Step 4 | Based on inputs from the CU, the NMMS software inserts the Dynamic Ratings for the equipment based on the entered temperature into the Network Operations Model | (from) NMMS to (to) NMMS |  |
| Step 5 | Based on inputs from the CU, the NMMS software inserts the selection of PMCRs into the Network Operations Model. The selected PMCRs are inserted after Topology Processing is executed. | (from) NMMS to (to) NMMS |  |
| Step 6 | NMMS software creates the Bus Branch model and creates a RAWD file which is sent to the CRRS | (from) NMMS to (to) CRRS | The data is in the RAWD format and should include all NOMCRs, PMCRs, SAMRs and Outages. |

Exceptions / Alternate Sequences:

NONE

Post-conditions:

NONE

References:

Use Cases referenced by this use case, or other documentation that clarifies the requirements or activities described.

- COWM.P05.ConductCRRMarket_UC_CreateSettlementFile

- COWM.P05.ConductCRRMarket_UC_CreateContingencyFiles

- COPS.P01ModelManageData_UseCase_ProcessSAMR

The following Standards and other documents are referenced by this case:

- ERCOT Nodal Protocols

- ERCOT NMMS Requirements

Issues:

| ID | Description | Status |
| --- | --- | --- |

Revision History:

| No | Date | Author | Description |
| --- | --- | --- | --- |
| 0 |  | J. Winkel | Initial Version |
| 1 | 8/26/06 | M. Goodrich | Reviewed and Revised |
| 2 | 9/10/06 | M. Goodrich | Added Comments from Crews and Moseley |
| 3 | 9/12/06 | M. Goodrich | Added edits from NMG |