Use Case – Create Planning Model Change Request

CPPS.P01 PlanFutureBaseCases_UC_CreatePMCR_V0.8

Name: Create PMCR – Incremental Changes

Summary:

Create data change file for all additions, deletions or modifications that apply to the Planning Models of ERCOT. This file is attached to a Planning Model Change Request form that jointly constitutes the PMCR.  The creation may be done by logging in the thin client and directly entering data into NMMS using the graphic interface, sending changes in a NON-CIM file format (may be excel spreadsheet/ one line diagram etc.) or sending changes in a  CIM/XML file format

Acronyms:

MIS 		Management Information System

NMMS  	Network Model Management System

NMG		Network Model Group

NOMCR  	Network Operating Model Change Request

OS  		Outage Scheduler

PMCR	 	Planning Model Change Request

PSS/E  		Power System Simulator/ Engineering

RAWD  	Raw Data, a PTI planning model format

SAMR		Special Action Model Request

TSP		Transmission Service Provider Planner

PC		ERCOT Planner

Actor(s):

| Name | Role description |
| --- | --- |
| TSP Planner | TSP Planner who creates the Planning Model Change Request and generates the RAWD files or IDV files. If RAWD files are generated, the TSP Planner uses comparison tools within MOD to compare two RAWD files and generate the IDV file. |
| ERCOT Planner | The ERCOT Planner may create PMCRs as well |
| NMG Staff | Using the facilities, the NMG coordinate the generation, verification and tracking of the PMCR. |

Participating Systems:

| System | Services or information provided |
| --- | --- |
| NMMS | ERCOT Network Model Management System, which is the central repository for all NOMCRs, SAMRs, and PMCRs |
| MIS Secure Area | ERCOT Management Information System Secure Area. This is a web-based system, which allows market participants to access and download ERCOT Network, Planning and other data. The MIS Secure Area includes log-on, user authentication, and the management of user access control rights. |

Pre-conditions:

- MIS Secure area contains the annual planning models

- NMMS is up and running

- The NMMS contains a copy of all approved PMCRs, SAMRs, and NOMCRs.

- The NMMS contains a copy of all candidate PMCRs

- The ERCOT user has logged-on to NMMS and has the authority to work on all data referenced in this use case.

Design Considerations:

- The NMMS is considered a black box.   Files inside NMMS only indicate that the request made by the ERCOT or TSP planner causes NMMS to make this data available to the ERCOT user for his private use in user areas (workspace).

- The user must be able to execute the NMMS functions for creation of an IDV file.

- The NMMS must allow the TSP direct access to use the functions described above.

Examples of model changes are:

- The ERCOT or TSP planner wants to create a planning model modification for a period six months in the future.

Known assumptions, limitations, constraints, or variations that may affect this use case:

NONE

Normal Sequence:

| Use Case Step | Description | From - To | Information Content |
| --- | --- | --- | --- |
| Step 1 | ERCOT or TSP Planner selects the desired planning model from the MIS website/or builds a planning model using the “Create Planning Model” use case. This is done after logging into PTC UI | ERCOT or TSP planner to MIS Secure Area | A copy of the selected Annual Planning Model is made available to the ERCOT or TSP planner for down load. |
| Step 2 | NMMS returns the desired planning Model to the ERCOT or TSP Planners computer | NMMS to ERCOT or TSP Planner | Files with the requested PMCRs |
| Step 3 | ERCOT or TSP Planner uses his own software to study candidate changes. At the end of the study, the equipment additions are described in PSS/E formats (RAWD, IDVs, Python etc) . | ERCOT or TSP Planner to NMMS | PMCR data |
| Step 4 | ERCOT or TSP Planner clicks on PTC UI – ‘CreatePMCR’ button | ERCOT or TSP Planner to NMMS | PMCR form |
| Step 5 | PTC UI brings the ‘New PMCR’ screen with generated PMCR id | NMMS | PMCR form |
| Step 6 | ERCOT or TSP Planner completes the PMCR form | ERCOT or TSP Planner to NMMS | PMCR form with associated information |
| Step 7 | ERCOT or TSP Planner uploads the associated PSS/E format files to NMMS and attaches with the PMCR and submits the PMCR | ERCOT or TSP Planner to NMMS | PMCR form and attached files in PSS/E format |
| Step 8 | NMMS stores the PMCR form and data in the NMMS tracking system. | NMMS to NMMS | PMCR form and data are stored. |
| Step 9 | The NMMS notifies the NMG staff and the ERCOT Planner of PMCR arrival. | NMMS-to NMG staff and ERCOT planner | Information about the new PMCR |

Exceptions / Alternate Sequences: Using MOD GUI

| Use Case Step | Description | From - To | Information Content |
| --- | --- | --- | --- |
| Step 1 | ERCOT or TSP Planner clicks on ‘CreatePMCR’ button on PTC UI that brings up the Create/Edit PMCR window | ERCOT or TSP Planner to NMMS | PMCR form with created id |
| Step 2 | ERCOT or TSP Planner enters the header information and Energization Date on ‘Create/Edit PMCR page | ERCOT or TSP Planner to NMMS | PMCR header info and energization Date |
| Step 3 | ERCOT or TSP Planner selects the option of ‘Approved Operational Projects’ or ‘Submitted Operational Projects’ and clicks on ‘Select Model Parameters’ | ERCOT or TSP Planner to NMMS | PMCR header info and energization Date |
| Step 4 | NMMS brings the page ‘Select Projects for Base Model’ and ERCOT or TSP Planner can see the list of Approved or Submitted NOMCRs and Submitted PMCRs. | NMMS | List of Submitted or approved NOMCRs and PMCRs |
| Step 5 | ERCOT or TSP Planner may choose to select manually only a few of the NOMCRs and PMCRs out of the entire list and after the selection is over clicks on ‘Load MOD’ button | ERCOT or TSP Planner to NMMS | NOMCRs, PMCRs |
| Step 6 | MOD UI is available to ERCOT or TSP Planner with the Base Operational Model created. | NMMS | Base Operational Model |
| Step 7 | ERCOT or TSP Planner selects the Idev files from MOD corresponding to the selected PMCRs in PTC | ERCOT or TSP Planner to NMMS | Idev files |
| Step 8 | NMMS generates the Base Planning Model | NMMS | Base Planning Model |
| Step 9 | ERCOT or TSP Planner uses MOD GUI tabular entry to enter text data for his PMCR. The user can view a one line diagram that shows the effect of the PMCR. No graphical editing is available. The PMCR may be validated at the time of entry using MOD GUI. At the end of the validation, the equipment additions are available as PSS/E files (RAWD, Idev, Python etc) . | ERCOT or TSP Planner to NMMS | PMCR data |
| Step 10 | ERCOT or TSP planner saves the Idev files created for the new PMCR in a desired folder. | ERCOT or TSP Planner to NMMS | PMCR data |
| Step 11 | ERCOT or TSP Planner accesses PTC UI. | ERCOT or TSP Planner to NMMS | PMCR form and generated files in PSS/E format |
| Step 12 | ERCOT or TSP Planner uploads PMCR files |  |  |
| Step 13 | ERCOT or TSP Planner submits the PMCR |  | PMCR form and generated files in PSS/E format |
| Step 14 | NMMS stores the PMCR form and data in the in a folder for posting and also in PTC schema | NMMS to NMMS | PMCR form and data are stored. |
| Step 15 | The NMMS notifies the NMG staff and the ERCOT Planner of PMCR arrival. | NMMS-to NMG staff and ERCOT planner | Information about the new PMCR |

Post-conditions:

- PMCR is in NMMS, ready for download and evaluation by ERCOT.

References:

Use Cases referenced by this use case, or other documentation that clarifies the requirements or activities described.

- CPPS.P01_PlanFutureBaseCases_UC_CreatePlanningModel

- CPPS.P01_PlanFutureBaseCases_UC_ProcessPMCR

The following Standards and documents are referenced by this case:

- IEC 61970-552-4, CIM XML Model Exchange Format Rev6 20050505 Standard

- IEC 61970-501, CIM RDF Schema

- ERCOT Nodal Protocols

- ERCOT NMMS Requirements

Issues:

| ID | Description | Status |
| --- | --- | --- |
|  | TSP planner must be able to access NMMS and the MIS secure area | Open |

Revision History:

| No | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | 08/04/06 | J. Waight | Initial draft for internal review |
| 1 | 08/08/06 | J. Moseley | Initial Review |
| 2 | 08/26/06 | M. Goodrich | Added Use Case Formatting, accepted changes, added revisions and clarifications and changed the title to match the naming convention. |
| 3 | 09/11/06 | M. Goodrich | Added edits from NMG |
| 4 | 10/13/06 | M. Sengupta | Added alternative path for PMCR creation where MOD GUI is used, based on ERCOT new requirement |
| 5 | 10/13/2006 | M. Sengupta | Modified based on telephonic conversation with ERCOT, to enter data only once |
| 6 | 10/20/2006 | M. Sengupta | Made further modifications to clarify the use of NMMS GUIs |
| 7 | 10/21/06 | M. Sengupta | Final Edits |