| PRR Number | EILP-PRR ‘D’ | PRR Title | Emergency Interruptible Load Service |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  |  |  |
| Requested Resolution (Normal or Urgent) |  | URGENT |  |
| Revision Description |  | This PRR changes the Protocols Sections noted above and upon approval institutes Emergency Interruptible Load Service (“EILS”)as a load interruption emergency backstop for use only during EECP Step 3. EILS would be a merit order of loads that have voluntarily offered to interrupt at an offer price and would be dispatched to interrupt in the EECP step just prior to the ISO’s dispatch order to TDSP/DSPs to shed firm load in order to preserve the system’s frequency. In the event that EILS is dispatched the marginal clearing price of the last load dispatched would set the MCPE for the entire EECP episode for both loads and generation resources. |  |
| Reason for Revision |  | This PRR is initiated as part of the efforts of the WMS task force on EILS in response to the PUCT’s call for more demand response capability in the energy-only market and ERCOT staff’s stated need for another tool to preserve system frequency during EECP events with the hope of avoiding firm load shedding. |  |
| Credit Implications (Yes or No, and summary of impact) |  | Yes, MCPEs may be set at higher prices to reflect the opportunity costs of large and small loads that typically experience very high process recovery costs and could represent the type of volatility that could cause many QSEs, that represent retailers, to exceed their posted collateral and security and could lead to significant entity business failures. Additionally, generators that are short during the applicable EECP intervals, would be subject to higher volatility that could cause QSEs to exceed their posted collateral. |  |
| Relevance to Nodal Market (Yes or No, and summary of impact) |  | It is unclear if this PRR should be applicable to the nodal market because projections of future reserve margins indicate sufficient generation capability above 12.5%. |  |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  |  |  |
| Timeline |  |  |  |
| Date Posted |  |  |  |
| Please access the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name | Floyd Trefny and Randa Stephenson (EILTF Option D) |
| E-mail Address | ftrefny@reliant.com and Randa.stephenson@txu.com |
| Company | Reliant Energy and TXU |
| Company Address |  |
| Phone Number |  |
| Fax Number |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 | Example: Key assumptions used in estimating market cost and/or benefit |  |
| --- | --- | --- | --- |
|  | 2 | Ex: Dependencies on other projects or other timing requirements |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | Example: Cost per MP to implement | Example: $10,000 each for 50 QSEs |
|  | 2 | Ex: Add’l staff required per MP | Ex: 1.5 FTE each for 6 TDSPs @ $65/hour |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Example: Reduced MP costs | Example: 2 FTE reduction for 25 CRs @ $65/hour |
|  | 2 | Ex: Enhanced MP efficiency | Ex: 2 hour savings per day for 50 generators @$65 |
|  | 3 | Ex: Reduced congestion cost | Ex: 0.5% reduction in total congestion cost |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | What to include here: Benefits that are difficult to quantify |  |
|  | 2 | What to include here: Benefits that are not certain but relatively likely |  |
|  | 3 | What to include here: Customer service impacts, cash flow impacts, transaction speed, etc. |  |
|  | 4 |  |  |
| Other | 1 | What to include here: Thoughts on ERCOT systems impacts |  |
| Comments | 2 | What to include here: Potential manual workarounds or delivery options |  |
|  | 3 | What to include here: Other comments of value to PRS, TAC and the Board of Directors |  |
|  | 4 |  |  |

Proposed Protocol Language Revision

Section 2: Definitions and Acronyms

Emergency Interruptible Load Resource

A load this is qualified and registered Resource with ERCOT to participate in Emergency Interruptible Load Service.

Emergency Interruptible Load Service

A service that is used during an EECP to reduce Load on the system as defined in Section 5.6.7, EECP Steps of these Protocols.

High System-Wide Offer Cap

The highest price than can be offered by any Market Participants as defined as the HCAP in PUCT Rule 25.504(g).

  - Acronyms

[…]

EILS	Emergency Interruptible Load Service

HCAP	High System-Wide Offer Cap

Section 4: Scheduling

4.1.1	Day Ahead Scheduling Process

| Time Period at or before: | Qualified Scheduling Entity (QSE) Submission: | ERCOT Action: |
| --- | --- | --- |
| Day Ahead |  |  |
| 0600 |  | - Publish updated transmission system information, Load Forecasts (Congestion Zone, and by total ERCOT Transmission System), Ancillary Service (AS) Plan, AS responsibility, mandatory decremental Balancing Energy Service bid percentage requirements and Transmission Loss Factors and Distribution Loss Factors. |
| 1100 | - Balanced Energy Schedule of Obligations and Resources. - Self-Arranged AS Schedule. - Remove, if necessary, EILS offers | - Validate Schedules and notify affected Qualified Scheduling Entities (QSEs) of any invalid or mismatched schedules. |
| 1115 | - Resubmit corrected schedules. | - Review Schedules: Commercial Model. - CSC Congestion. - Notify QSEs of Congested CSCs. - Post schedule impact on CSCs. - Notify QSEs of any difference in ERCOT’s zonal Load forecast and the aggregate of QSE schedules in a zone. |
| 1300 | - Update Balanced Energy Schedule of Obligations and Resources. - Update Self-Arranged AS schedule - AS bids to supply AS to ERCOT AS Market. | - Validate Schedules and notify affected QSEs of any invalid or mismatched schedules. |
| 1315 | - Resubmit corrected updated schedules. | - Validate resubmitted schedules and adjust as necessary. |
| 1330 |  | - Purchase AS through ERCOT AS Market in order to complete ERCOT’s Day Ahead AS plan. - Review Day Ahead Market Clearing Prices for Capacity (MCPC) for each Day Ahead AS market operated by ERCOT. |
| 1500 | - Submit updated AS schedule that includes Self-Arranged and AS bids selected by ERCOT. | - Review and validate AS schedules as necessary. |
| 1600 | - Submit Resource Plans with unit-specific information. - Submit Replacement Reserve Service (RPRS) Bids. | - Verify that Resource Plans are sufficient to meet the QSE schedule. If not, ERCOT will notify the QSE. |
| By 1800 |  | - Evaluate Resource Plans to determine system security. - Evaluate RPRS needed to correct CSC, and Operational Congestion – Operational Model, and capacity insufficiency. |
| By 1800 |  | - Create Merit Order of RPRS by Zone or Operational Constraint as applicable. |
| By 1800 |  | - Determine MCPC for RPRS by Zone as applicable. |
| By 1800 |  | - Close and clear Day Ahead RPRS market. |
| After 1800 | - Modify RPRS bids. |  |

    - Month Ahead Scheduling Process 
Emergency Interruptible Load Service shall not be scheduled using the Day Ahead or Adjustment Period scheduling process. Instead, EILS shall be scheduled as follows:

| Time Period at or before: | Qualified Scheduling Entity (QSE) Submission: | ERCOT Action: |
| --- | --- | --- |
| Month Ahead |  |  |
| At least 3 Business Days before the first Business Day of the Operating Month | - Submit EILS offers for the following month |  |
| At least 1 Business Days before the first Business Day of the Operating Month | - Submit to ERCOT a list of the EILS Resource ESI IDs by Operating Hour to be used in any hour of the Operating Month |  |

4.4.8	Submittal of Day Ahead Schedules and Ancillary Services Schedules

QSEs will submit Day Ahead Balanced Schedules to ERCOT before 1100 of the Day Ahead for each Settlement Interval of the Operating Day.  Obligations and Supply must be scheduled by Congestion Zone.  Although the schedules must be balanced, they need not be balanced by Congestion Zone.

The QSE’s balanced Ancillary Service schedule must specify the amount of ERCOT allocated Ancillary Service Obligation for each service type that the QSE intends to purchase from ERCOT.

Day Ahead schedules may also include Self-Arranged Ancillary Services in accordance with Section 6.2.2, provided by the QSE to meet all or a portion of its Ancillary Services Obligation.  Self-Arranged Ancillary Services may consist of groups of Resources, specific Resources, or trades with other QSEs.  After 1100, a QSE may not increase its level of Self-Arranged Ancillary Services for the Day Ahead Obligations, unless ERCOT has notified Market Participants of CSC Congestion or capacity insufficiency as described in Section 4.4.9.  If such Notice is provided, then the QSE will be permitted to update its Ancillary Service Schedule for self- arranged Ancillary Services until 1300 as detailed in Section 4.4.10.  At 1330, ERCOT will procure, on behalf of the QSEs, any Ancillary Services not self-arranged.

QSE may remove EILS offers before 1100 of the Day Ahead for each Settlement Interval of the Operating Day.

4.11 Month Ahead Scheduling Process

4.11.1 Overview

ERCOT will implement as described in this Section, the Month Ahead Scheduling Process, which will be utilized to procure Emergency Interruptible Load Service in the following Operating Month.  All ERCOT schedules, notices and other information requiring publication under this Section shall be published on the Market Information System (MIS) as set forth in Section 12 of these Protocols.

    - Month Ahead Ancillary Service Bid Submittal

QSEs may submit, by 1300 at least three (3) Business Days before the first Business Day of the Operating Month, EILS offers to ERCOT for Balancing Energy Services.  Submitted offers remain active for any single period until selected by ERCOT or removed by the QSE, as defined in the day-ahead scheduling process.

For BES offers of EILS, QSEs must provide:

(1)	The period of time for which the offer is submitted;

(3)	A dollars per megawatt hour price for the offer;

(4)	The quantity of capacity for which the offer price is effective.

4.11.3  ERCOT Validation of QSE Month Ahead Bids

ERCOT will validate that the QSE’s total MW submittal does not exceed the QSE’s amount of registered Resources for EILS.

4.11.4 QSE Resource Plans

The Month Ahead Resource Plan shall identify the specific Resources that are to provide each Month Ahead Ancillary Service.

As necessary, throughout the Operating Month the QSE shall update their Month Ahead Resource Plan to reflect any changes.  At a minimum any adjustments to the Month Ahead Resource Plan must be submitted prior to or during the Adjustment Period for the Operating Hour.

5.6.7	EECP STEPS

Step 1 — Maintain ERCOT Physical Responsive Capability (PRC) on units plus RRS MW provided from LaaR Equal to 2300 Mw.  ERCOT will:

(1)	Utilize available DC tie capability that is not already being used by the market;

(2)	 Notify the Southwest Power Pool Security Coordinator;

(3)	Issue OOM dispatch instructions to uncommitted units available within the expected timeframe of the emergency;

(4)	Notify all QSEs that system conditions may warrant a deployment of EILP loads;

[PRR543: Add the following bullet upon ERCOT Legal review of the Comision Federal de Electricidad contract:] (5) Use available BtB import capacity that is not already being used and inquire about availability of Block Load Transfers.

QSEs will:

(1)  	Notify ERCOT of any resources uncommitted but available in the timeframe of the emergency.

Step 2 — Maintain ERCOT Physical Responsive Capability (PRC) on units plus RRS MW Provided from LaaR Equal to 1750 MW.  In addition to measures associated with Step 1, ERCOT will:

(1)	Instruct TDSPs to reduce Customers load by using distribution voltage reduction measures, if deemed beneficial by the TDSP and DSPs;

(2)	Instruct QSEs to deploy all Responsive Reserve, which is supplied from Load acting as a Resource (controlled by high-set under-frequency relays); and

(3)	With the approval of the affected Non-ERCOT control area, may instruct TDSPs and DSPs to implement BLTs, which transfer load from the ERCOT control area to Non-ERCOT control areas.  Use of a BLT will be defined in the ERCOT Operating Guides.

(4)	Unless such a media appeal is already in effect, ERCOT staff shall issue an appeal through the public news media for voluntary energy conservation.

Step 3 — Emergency Interruptible Load Service (EILS) Deployment

Should frequency continue to decay after implementing Steps 1 and 2, ERCOT will instruct those QSEs who represent EILP loads  to interrupt their corresponding loads not to exceed 1000 Mw. The control signal will initiate a pre-established time-delayed equipment trip, so that the EILP load has time to deploy manually before, or to prepare for activation of, the time-delayed equipment trip. QSEs will be responsible for compliance within the required response time regardless of the pre-established time-delay.

Step 4 — Maintain system frequency at 59.8 Hz or greater

In addition to measures associated with Steps 1, 2, and 3, ERCOT will direct all TDSPs, DSPs and their agents to shed firm load, in 100 MW blocks, distributed as agreed and documented in the ERCOT Operation procedures in order to maintain a steady state system frequency of 59.8 Hz.

In addition to measures associate with Steps 1 and 2, TDSPs and DSPs will keep in mind the need to protect the safety and health of the community and the essential human needs of the citizens.  Whenever possible, TDSPs and DSPs shall not manually drop load connected to under-frequency relays during the implementation of the Emergency Electric Curtailment Plan.

5.6.7.1	Restoration of Market Operations

ERCOT shall continue EECP until sufficient bids are received and deployed by ERCOT to eliminate the conditions requiring the EECP and the EILS (if struck) have been fully recalled.  ERCOT shall recall all deployed LaaRs and RRS prior to recalling EILS.

5.6.8	Emergency Compensation

Deployment and compensation for OOMC and OOME Resources that ERCOT instructs for deployment due to a supply-related Emergency will be according to Section 6.8.2, Capacity and Energy Payments for Out of Merit Services.

There will be no compensation other than normal settlement methods to Loads in the event of Load shedding by the TDSP and/or DSP during EECP

5.8.3.1	Data Collection

ERCOT will collect all data necessary to analyze each Measurable Event.  This will include the following real-time data:

(1)	Interconnection Frequency;

(2)	Regulation Service deployed;

(3)	Responsive Reserve Service deployed;

(4)	QSE available  Responsive Reserve Service;

(5)	QSE total Generation;

(6)	QSE SCE;

(7)	QSE Bias;

(8)	QSE LaaR MW;

(9)	LaaR deployed;

(10)	QSE Responsive Reserve Service;

(11)	ERCOT Load and individual Resource(s) that contributed to the frequency deviation; and

(12)	EILS deployed.

NEW SECTION

6.1.13	Emergency Interruptible Load  Service (EILS)

As provided by ERCOT to QSEs: A service that ERCOT maintains to call upon as described in EECP Step 3  and used by ERCOT prior to firm load shed when other methods for maintaining system frequency at 59.8 Hz or greater have failed.

As provided by a QSE to ERCOT: The provision of curtailable load Dispatched from a QSE's EIL in Step of 3 EECP operations by Settlement Interval in Real Time to maintain frequency at 59.8 Hz or greater when other methods of maintaining frequency have failed.

6.2.1	Ancillary Services Provided Solely by ERCOT

ERCOT is the sole provider of system-wide Balancing Energy Service; Generation Resource unit-specific Voltage Support Service (VSS), Black Start Service, Replacement Reserve Service, Zonal OOME Service, RMR Service, OOMC, OOME, and EIL Service to QSEs.

ERCOT will arrange Resources to provide system-wide VSS, Black Start, Zonal OOME, RMR, and EIL Service from QSEs.  ERCOT will direct Resources to provide OOMC or OOME in accordance with OOMC Service and OOME Service provisions of these Protocols.

6.4.2	Determination of ERCOT Control Area Requirements

By the twentieth (20th) day of the current month, ERCOT will post a forecast of minimum Ancillary Services quantity requirements for the next calendar month.

Prior to 0600 of the Day Ahead, ERCOT will use the Day Ahead Load forecast and will develop an Ancillary Services Plan that identifies the amount of Ancillary Services necessary for each hour of the next day as specified in Section 4, Scheduling.  The amount of Ancillary Services required may vary depending on ERCOT System conditions from hour to hour.

By 0600 of the Day Ahead, ERCOT will post an ERCOT System and zonal Load forecast for the next seven (7) days, by hour.  ERCOT will notify each QSE of its allocated share of Ancillary Services for each hour for the next day, as specified in Section 4, Scheduling.  ERCOT will make available to Market Participants any ERCOT area Load forecasts used in the determination of its ERCOT System and zonal forecasts.

ERCOT will determine the total required amount of each Ancillary Service using the Operating Guides and the following:

(1)	Balancing Energy Service:  ERCOT will estimate Balancing Energy needs based on the actual Load, the difference in forecasted Loads and Loads reported in bilateral schedules, deployed Regulation Service, and forecasted Congestion.

(2)	Regulation Service:  ERCOT will use its operational judgment and experience to determine the quantity of Regulation Up Service and Regulation Down Service procured.  The quantity of Regulation Up Service may differ from the quantity of Regulation Down Service in any particular hour.

(3)	Responsive Reserve Service:  The requirement for Responsive Reserve Service is specified in the Operating Guides.  Using ERCOT-approved procedures ERCOT may increase the quantity requirement based on its judgment of reliability conditions.

(4)	Non-Spinning Reserve Service:  ERCOT will use its operational judgment and experience to determine the quantity of Non-Spinning Reserve Service procured.

(5)	Replacement Reserve Service:  Replacement Reserve Service is procured from Generation Resource units planned to be Off-line and LaaRs that are available for interruption during the period of requirement.  The QSE for a procured RPRS Resource must additionally make available to ERCOT via Balancing Energy Service bid(s) (i) for a Generation Resource, a quantity greater or equal to the High Sustainable Limit (if unavailable, the High Operating Limit) minus the Low Sustainable Limit (if unavailable, the Low Operating Limit) of the Generation Resource receiving the RPRS Dispatch Instruction in the Congestion Zone of the Generation Resource, and (ii) for a LaaR, a quantity equal to the amount of procured capacity from the LaaR in the Congestion Zone of the LaaR.  ERCOT will consider the Generation Resource capacity On-line, based on Resource Plans, in its determination of Zonal Congestion and Local Congestion requirements.  ERCOT will evaluate the need for Replacement Reserve Service necessary to correct for ERCOT total capacity insufficiency, Zonal Congestion, or Local Congestion.  ERCOT shall determine the amount of RPRS to provide sufficient capacity in appropriate locations to provide ERCOT System security as specified in the Operating Guides, given ERCOT forecasted Load conditions as posted on the Market Information System.

(6)	Voltage Support:  ERCOT in coordination with the TDSPs shall conduct studies to determine the normally desired Voltage Profile for all Voltage Support busses in the ERCOT System and shall post all Voltage Profiles on the Market Information System.  ERCOT may temporarily modify its requirements based on Current System Conditions.  ERCOT shall determine the amount of Voltage Support Service needed to provide sufficient reactive capacity in appropriate locations to provide ERCOT System security as specified in the Operating Guides.

(7)	Black Start Service: ERCOT shall periodically determine and review the location and number of Black Start Resources required, as well as any special transmission or voice communication needs required.  ERCOT and providers of this service shall meet the requirements as specified in the Operating Guides and in NERC policy.

6.5.1.1	Requirement for Operating Period Data for System Reliability and Ancillary Service Provision

Operating Period data will be used by ERCOT to monitor the reliability of the ERCOT System in Real Time, monitor compliance with Ancillary Service Obligations, perform historical analysis, and predict the short-term reliability of the ERCOT System using network analysis software.  Each TDSP, at its own expense, may obtain such Operating Period data from ERCOT or from QSEs.

(1)	A QSE representing a Generation Entity that has Generation Resources connected to a TDSP shall provide the following Real Time data to ERCOT for each individual generating unit at a Generation Resource plant location and ERCOT will make the data available to the Generation Resource’s host TDSP (at TDSP expense):

(a)	gross or net real power;

(b)	gross or net Reactive Power;

(c)	if gross quantities are provided, the plant auxiliary Load data will also be supplied;

(d)	status of switching devices in the plant switchyard not monitored by the TDSP affecting flows on the ERCOT System;

(e)	Frequency Bias of Portfolio Generation Resources under QSE operation;

(f)	any data mutually agreed by ERCOT and the QSE to adequately manage system reliability and monitor Ancillary Service Obligations;

(g)	generator breaker status;

(h)	High Operating Limit; and

(i)	Low Operating Limit.

[PRR590: Add items (j) and (k) upon system implementation:] (j) AGC status; and (k) ramp rate.

[PRR307: Revise Section 6.5.1.1(1) and 6.5.1.1(1) (c) & (g) as follows when system change implemented.] (1) A QSE representing a Generation Entity or a Competitive Retailer that has Resources connected to a TDSP shall provide the following Real Time data to ERCOT for each individual generating unit or LaaR capable of controllably reducing or increasing consumption under Dispatch control (similar to AGC) and that immediately respond proportionally to frequency changes (similar to generator governor action) at a Resource plant location and ERCOT will make the data available to the Resource’s host TDSP (at TDSP expense): (c) If gross quantities are provided, the plant auxiliary Load data will also be supplied as needed to determine the net output; (g) Resource breaker status;

[PRR590: Add paragraph (2) and renumber subsequent paragraphs upon system implementation:] (2) A QSE representing Uncontrollable Renewable Resources is exempt from the requirements of Section 6.5.1.1(1)(j) and (k).

(2)	Any QSE providing Responsive Reserve and/or Regulation must provide for communications equipment to receive ERCOT telemetered control deployments of service power.

(3)	Any QSE providing Regulation Service must provide appropriate Real Time feedback signals to report the control actions allocated to the QSEs Generation Resources.

(4)	Any QSE that represents a provider of Responsive Reserve, Non-Spinning Reserve, or Replacement Reserve using LaaR shall provide separate telemetry of the real power consumption of each LaaR providing the above Ancillary Services, the LaaR response to Dispatch Instructions for each LaaR, and the status of the breaker controlling that LaaR.  If LaaR is used as a Responsive Reserve Resource, the status of the high-set under frequency relay will also be telemetered.

[PRR307: Revise Section 6.5.1.1(3) and 6.5.1.1(4) as follows when system change implemented.] (3) Any QSE providing Regulation Service must provide appropriate Real Time feedback signals to report the control actions allocated to the QSEs Resources. (4) Any QSE that represents a provider of Responsive Reserve, Non-Spinning Reserve, or Replacement Reserve using interruptible Load as a Resource shall provide separate telemetry of the real power consumption of each interruptible Load providing the above Ancillary Services, the LaaR response to Dispatch Instructions for each LaaR, and the status of the breaker controlling that interruptible Load. If interruptible Load is used as a Responsive Reserve Resource, the status of the high-set under frequency relay will also be telemetered.

(5)	Any QSE that represents a qualified provider of Balancing Up Load (BUL) or EIL need not provide telemetry but rather shall provide an estimate in Real Time representing the real power interrupted in response to the deployment of Balancing Up Load or EIL.

(6)	Real Time data for reliability purposes must be accurate to within three percent (3%).  This telemetry may be provided from relaying accuracy instrumentation transformers.

[PRR590: Add paragraph (7) upon system implementation:] (7) A QSE representing a combined cycle plant may aggregate the AGC and ramp rate SCADA points for the individual units at a plant location into two distinct SCADA points (AGC and ramp rate) if the plant is configured to operate as such, i.e. gas turbine(s) and steam turbine(s) are controlled in aggregate from an AGC perspective.

.

NEW SECTION

6.5.11	Emergency Interruptible Load )  Service (EILS)

(1)	ERCOT will use EILS deployments to maintain system frequency at or above 59.8Hz when other mechanisms have failed.

(2)	EILS deployments may only occur during EECP Step 3.

- Emergency Interruptible Load Service may be provided by Resources which have a time delay control device installed on their system.  A QSE’s EILS Resources must be loaded and capable of reducing their load by their committed capacity compared to their baseline capacity within ten (10) minutes of the instruction by ERCOT and must be able to maintain such reduced capacity level for the entire period of the service commitment.

- Any QSE providing Emergency Interruptible Load Service must be capable of communicating actual usage data, in one minute intervals, in the event of a deployment within 48 hours of such deployment to ERCOT in an ERCOT prescribed format.

- QSEs that provide the Resources for Emergency Interruptible Load Service will ensure that Resources providing the service are able to respond in the Operating Hour for which they have been selected to provide the service.  Each EILS Resource providing EILS must meet the additional technical requirements specified in Section 6.10, Ancillary Service Qualification, Testing, and Performance Standards of these Protocols.

- EILS offers will be in one (1) MW increments only. QSEs representing EILS Resources may aggregate multiple Resources to reach the one (1) MW offer requirement.

- EILS Resources must have an installed interval data recorder (IDR) meter.

- The EILS Resource must demonstrate that it can provide the required one-minute interval data to its designated QSE within one Business Day

- QSEs representing EILs, will provide ERCOT with offers to interrupt containing MW quantity and $/MWh price pairs on a monthly basis for any hour in the next month at least three (3) Business Days before the first Business Day of the Operating Month

- (2)       EILS offers may be withdrawn by 1100 the day ahead of the Operating Day

- ERCOT shall place the offers in order from lowest price to highest price.  This shall be the EILS offer stack.

- EILS offers must indicate the qualified EILS Resource represented by each bid.

- EILS offers shall not exceed the HCAP applicable at the time of the offer.

- ERCOT shall deploy offers of EILS to restore system frequency in EECP Step3, up to 1000Mw.

- The EILS deployment will be in megawatts.  The EILS obligation shall be the power requested integrated over the interval.

- QSEs will be notified by ERCOT via the Messaging System of EILS deployments in their portfolio, and the EILS loads will have 10 minutes from that notice to respond to a level of 95% of their offered amount.

- When EILS is deployed, the MCPE of the market will be administratively set at the clearing price of the last MW deployed in the EILS offer stack and will be used in settlements for all market imbalances.

- The aggregate EILS offer stack will be published in the MIS system prior to the month for which the stack is effective.

- ERCOT shall prepare a cost-benefit analysis of the EILS program quarterly from the start of the EILS program. The IMM may review and comment, if necessary, on the effectiveness of EILS.

6.6.2	Competitive Procurement of Ancillary Service Resources by ERCOT

(1)	Except where stated to the contrary in these Protocols, ERCOT shall, to the extent the Ancillary Service Resource bids are available, use competitive procurement processes to procure sufficient Ancillary Service Resources to meet the requirements specified in these Protocols.

(2)	QSEs may submit bids to provide Regulation Down, Regulation Up, Responsive Reserves, and Non-Spinning Reserves, as part of the Scheduling Process in accordance with Section 4, Scheduling.

(3)	QSEs offering Balancing Energy Up, Balancing Energy Down, and Replacement Reserves can offer bids through the Adjustment Period, in accordance with Section 4.5, Adjustment Period Scheduling Process.

(4)	QSE’s bids to provide Ancillary Services will continue to be valid until withdrawn by the QSE prior to the market clearing or the market is cleared.  Bids may not be withdrawn during bid evaluation by ERCOT as described in Scheduling Sections 4.4 Day Ahead Scheduling Process and Section 4.5 Adjustment Period Scheduling Process or after the bid is selected.

(5)	QSEs may only submit bids from that portion of any Resource not used to provide capacity and energy to Supply the Resources in the QSE’s Balanced Schedule.

(6)	ERCOT will determine the MCPC of each interval for Regulation Up, Responsive Reserve and Non-Spinning Reserve using an optimized simultaneous selection process in order to minimize the overall cost of these Ancillary Services.

(7)	Other than as specified for Congestion Management, ERCOT shall select Replacement Reserve and Regulation Down bids based on the lowest bid for each service.

(8)	Other than as specified for Congestion Management, ERCOT shall Dispatch energy from the Balancing Energy Service bids based on price Merit Order and the requirements specified in Section 5, Dispatch Instructions.

(9)	For Congestion Management, ERCOT shall, if possible, resolve all Congestion using the Balancing Energy Service by Congestion Zone.

(10)	ERCOT shall establish, through an annual competitive procurement process, annual agreements with Resources needed to provide Black Start capability.

(11)	QSEs shall provide offers to provide EILS on a monthly basis applicable to all hours in the month.  ERCOT shall select EILS offers based on the lowest offer for the service.

(12)	Transmission congestion shall not be a consideration in the procurement  or deployment of EILS.

6.7.9	Deployment and Recall of EILS

(1)	The EILS offer stack will be created from the monthly offers for any operating hour of the month.

(2)	ERCOT may only deploy EILS during EECP Step 3.

(3)	ERCOT shall deploy all EILS offered not to exceed 1000 Mw to restore system frequency.

(4)	ERCOT will provide Notice to QSEs via the Messaging System of their awards for EILS .  QSEs will be required to respond with manual or electronic acknowledgement.

(5)	ERCOT shall notify each QSE of its instructed amount of EILS ten (10) minutes prior to the Settlement Interval in which it is to be deployed.  QSEs shall be expected to provide a power level of at least 95% of the deployment within ten (10) minutes of the instruction.

(6)	ERCOT shall recall EILS on conclusion of the EECP and return to normal operations but no sooner than 4 hours following EILS deployment.

6.10.2	General Capacity Testing Requirements

QSEs shall provide ERCOT a list identifying each Generation Resource unit that is expected to operate more than one hundred sixty eight (168) hours in a Season as a provider of energy and/or Ancillary Services.  ERCOT shall evaluate, during each Season of expected operation, the Net Dependable Capability of each unit expected to operate more than one hundred sixty eight (168) hours during that season, except for any Generation Resources used solely for energy services and whose capacity is less than ten (10) MW.  Prior to the beginning of each Season, QSEs shall identify the Generation Resources to be tested during the Season and the specific week of the test if known.  This schedule may be modified by the QSE (including retests) during the Season.  QSEs not identifying a specific week for a Generation Resource unit test must test the unit within the first one hundred sixty eight (168) hours of run time during the Season or operate with a Net Dependable Capability equal to the highest integrated hourly MWh output demonstrated during the first one hundred sixty eight (168) hours of run time.  QSEs do not have to bring units On-line or shut down solely for the purpose of the seasonal verification.  Any unit for which the QSE desires qualification to provide Ancillary Services shall have its Net Dependable Capability verified prior to providing services using the Generation Resource unit even if it fits the less than one hundred sixty eight (168) hour or small capacity exception.  The capability of hydro units operating in the synchronous condenser fast response mode to provide hydro Responsive Reserve shall be evaluated by Season.

Load acting as a Resource to provide Ancillary Services shall have its telemetry attributes verified by ERCOT annually.  In addition, once every two (2) years, any LaaR providing Responsive Reserve Service shall test the under frequency relay or the output from the solid-state switch, whichever applies, for correct operation.  However, if the Load’s performance has been verified through response to an actual event, the data from the event can be used to meet the annual telemetry verification requirement for that year and/or the biennial relay testing requirement.

In order for Emergency Interruptible Load Resources to provide Emergency Interruptible Load Service they shall have the communication and metering attributes verified by ERCOT annually through an actual load reduction.  However, if the Load’s performance has been verified through a response to an actual event, the data from the event can be used to meet the annual communication and metering verification requirements for that year.  EILS Resources must adhere to the qualifications, testing and performance standards as outlined in Section 6.11.

Specific Loads to be used for the first time as a Resource to provide Responsive Reserve, Non-Spinning Reserve or Replacement Reserve must be correctly evaluated (tripped or simulated trip, if approved by ERCOT) prior to their qualification to provide Ancillary Services.  At a time selected mutually by the Load and ERCOT, ERCOT will notify the QSE representing such Load of the need to confirm that Loads proposed to be used to provide Responsive Reserve, Non-Spinning Reserve or Replacement Reserve Services can be de-energized, or simulated, if applicable, to ERCOT’s reasonable satisfaction.  ERCOT shall develop a standard test for simulation for Load interruption required under this subsection.  Loads used to provide Responsive Reserve Service shall be qualified for correct operation by their host TDSP.

QSEs shall be responsible for qualifying any Load desiring to have the QSE represent it to provide Balancing Up Load (BUL) Service.

[PRR484: Add the following to the end of the above paragraph (“QSEs shall be responsible for qualifying…Balancing Up Load (BUL) Service. ”) upon system implementation:] Loads controlled under a qualified DLC program may be qualified as a group.

The QSE shall nominate to ERCOT, at least annually, that it is representing an amount of BUL for which it wishes to be qualified to provide.  At a time selected by ERCOT, the ERCOT operator will notify the QSE that it wants to verify the QSE’s ability to provide ERCOT with the appropriate signal simulating that it has initiated an ERCOT requested BUL reduction.  The QSE Operator will immediately simulate the initiation of the reduction and provide ERCOT with the appropriate signal representing some amount of Load to be qualified to provide BUL Resources.  Once ERCOT has verified that it has received an appropriate Load reduction signal from the QSE and has successfully completed the BUL registration process, the QSE will be qualified to provide BUL Resources.  Any changes to the BUL portfolio will require subsequent updates to the registration process.  For NOIEs representing specific Loads qualified as BULs that are located behind the NOIE Settlement Meter points, the NOIE shall provide an alternative unique descriptor of the qualified BUL Load for ERCOT’s records.

Generation Resources and Loads acting as Resources shall be evaluated at least annually by ERCOT for:

(1)	Correct operation of telemetry of the breakers controlling the Resource;

(2)	Correct mapping of QSE-provided telemetry of Ancillary Service energy to the appropriate energy Settlement Meter;

(3)	Data rate update requirements; and

(4)	Any other required telemetry attributes.

All Generation Resources and Loads acting as a Resource shall meet all requirements specified in the Operating Guides for proper response to system frequency.  ERCOT may reduce the amount a Resource may contribute toward Ancillary Services if it finds unsatisfactory performance of the Resource as defined in these Protocols and the Operating Guides.

Qualification of a Resource, including a Load acting as a Resource or a EILS Resource, shall remain valid for such Resource in the event of a change of QSE for the Resource, provided that the new QSE demonstrates to ERCOT’s reasonable satisfaction that the new QSE has adequate communications and control capability for the Resource.

[…]

6.11	Emergency Interruptible Load Service Qualification, Testing and Performance Standards

6.11.1	Introduction

QSEs providing Emergency Interruptible Load Services and EILS Resources shall meet qualification criteria and performance measures to operate satisfactorily with ERCOT.  ERCOT shall develop an Emergency Interruptible Load Service qualification and testing program.  This program will be approved by ERCOT Technical Advisory Committee and will be included in the Operating Guides.  The performance factors shall be measured as precisely and efficiently as possible.  Qualification tests allow the potential EILS Resource to demonstrate the capabilities necessary to deploy the Emergency Interruptible Load Service, performance measures, and assess the delivery of this service by an Emergency Interruptible Load Service provider.

All loads may be provisionally qualified as an EILS Resource for a period of ninety (90) days or until fully tested by ERCOT provided the following criteria are met to ERCOT’s reasonable satisfaction:

- Resource registration of EILS Resource by the QSE;

- Ability to upload one-minute interval data to the QSE in a format that meets ERCOT’s requirements; and

- Affidavit from the EILS Resource stating the Resource’s ability to receive the time-delay control signal from its QSE and demonstrate a receipt of the signal to trip the EILS Resource load committed to Emergency Interruptible Load Service.

ERCOT shall perform an EILS qualification test within a 90 day period from Registration.  Provisional qualification as described herein may be revoked by ERCOT at any time for any non-compliance with provisional qualification requirements.  During the provisional qualification period for the Emergency Interruptible Load Service Resource their maximum qualified amount to bid shall be the average of the Resource’s usage for the most recent two months, as calculated by the QSE and communicated to ERCOT.

6.11.2	General Capacity Testing Requirements

For initial qualification to provide Emergency Interruptible Load Service, a Load shall perform a full demonstration in accordance with EILS performance requirements to establish their ability to interrupt as required.  A successful initial qualification shall set the maximum quantity the EILS Resource is eligible to bid in the Month Ahead Scheduling Process. At a time determined by ERCOT, ERCOT will notify the QSE representing selected EILS Resources of the need to confirm that Loads proposed to be used to provide Emergency Interruptible Load Service can be interrupted within 10-minutes of ERCOT’s notification to ERCOT’s reasonable satisfaction.

QSEs representing EILS Resources shall have the ability to upload one minute interval meter data directly to ERCOT verified by ERCOT annually.

Once every year, the EILS Resource and its QSE shall test the time-delay trip for correct operation by verifying that the EILS Resource’s Load is tripped within the time used by the QSE.  The quantity of Load the EILS Resource sheds during a test or deployment shall set the maximum amount that EILS Resource is allowed to offer in BES until a subsequent test or event occurs.  Each EILS Resource’s designated QSE shall communicate this value to ERCOT for verification.  If the EILS Resource Load’s performance has been verified through a successful response to an actual event, this testing requirement is waived for the following year.

The QSE shall nominate to ERCOT, at least annually, that it is representing an amount of EILS Resources which it wishes to be qualified to provide.  At a time selected by ERCOT, the ERCOT operator will notify the QSE that it wants to verify the QSE’s ability to provide ERCOT with the appropriate signal simulating that it has initiated an ERCOT requested Emergency Interruptible Load deployment.  The QSE Operator will initiate the deployment and provide ERCOT with the appropriate signal representing some amount of Load to be qualified to provide EILS Resources.  Once ERCOT has verified that it has received an appropriate Load reduction signal from the QSE and has successfully completed the Emergency Interruptible Load Service registration process, the QSE will be qualified to provide EILS.  Any changes to the EILS Resource portfolio will require subsequent updates to the registration process.  For NOIEs representing specific Loads qualified as EILS Resources that are located behind the NOIE Settlement Meter points, the NOIE shall provide an alternative unique descriptor of the qualified EILS Resource for ERCOT’s records.

Qualification of an EILS Resource shall remain valid for such resource in the event of a change of QSE for the resource, provided that the new QSE demonstrates to ERCOT’s reasonable satisfaction that the new QSE has adequate communications and control capability for the Resource to provide Emergency Interruptible Load Service.

6.11.3	Ancillary Services Qualification Criteria and Portfolio Test Methods

Only QSEs that have been qualified and tested may be used to provide Emergency Interruptible Load Service.  ERCOT shall develop and operate its qualification and testing program to meet the following requirements for the Emergency Interruptible Load Service.

A QSE shall be qualified and tested to provide Emergency Interruptible Load Service prior to initial operation and every five years thereafter.

ERCOT may grant a “Provisional Qualification,” for a period not to exceed ninety (90) days, to a QSE that has performed an Emergency Interruptible Load Service qualification test in good faith but failed to qualify due to problems that, in the sole discretion of ERCOT, are determined to be non-critical for the purpose of providing Emergency Interruptible Load Service.  Notwithstanding the failure of a QSE with Provisional Qualification to meet the applicable Emergency Interruptible Load Service criteria, such QSE may provide such Emergency Interruptible Load Service to the extent permitted by the terms of the Provisional Qualification.

6.11.3.1	Emergency Interruptible Load Service Qualification Testing Criteria

(1)	Testing using EILS Resource(s)

(a)	A test for EILS shall be performed during a continuous eight (8) hour window agreed on by the QSE and ERCOT.

(b)	ERCOT shall confirm the date and time of the test with the QSE using both the primary and alternate voice circuits to validate the voice circuits.

(c)	At any time during the window, selected by ERCOT when market and reliability conditions allow, and not previously disclosed to the QSE, ERCOT shall send a VDI to the QSE requesting the Resources to be tested to provide and deploy the Emergency Interruptible Load Service.  The QSE shall acknowledge the start of the test.

(d) 	ERCOT may at anytime request EILS Resource’s designated QSE to upload to ERCOT the one-minute interval data within 48 hours of the request and in accordance with Section 6.5.1.1.

(e)	On successful demonstration of all test criteria, ERCOT shall qualify that the QSE’s Load Resources are capable of providing EILS and shall provide a copy of the certificate to the QSE designating its qualification to provide EILS from EILS Resources.

6.11.4	Emergency Interruptible Load Service Deployment Performance Measures

(1)	Following deployment of Emergency Interruptible Load Service each EILS Resource’s designated QSE shall be responsible for uploading to ERCOT within 48 hours the one-minute interval data in accordance with Section 6.5.1.1.

12.4.2	Ancillary Service -Related Information

ERCOT shall post on the MIS its Ancillary Services requirements as set forth in Section 4, Scheduling.

As set forth in Section 4, Scheduling, ERCOT shall post as specified in these Protocols, Day-Ahead Ancillary Services scheduling process hourly summaries, including:

(1)	Amount of capacity procured and the MCPCs for each Ancillary Service,

(2)	Aggregated Bid Stack for each Ancillary Service, no earlier than the day following the effective day of the bids,

(3)	Notices of system insufficiencies, and

(4)	The quantities and prices of Ancillary Services acquired under conditions of system insufficiency (OOMC),

(5)	The quantities and prices of RMR Services,

(6)	Total ERCOT actual Load.

ERCOT shall securely post information affecting the allocation of Ancillary Services to QSEs including:

(1)	Each QSE’s historical Load Ratio Share used for the Ancillary Services allocation; and

(2)	Each QSE’s allocation of each Ancillary Service based upon the information in (1) above.

(3)	As set forth in Section 6, Ancillary Services, ERCOT shall post the aggregated offer stacks for EILS no later than 48 hours prior to the start of the month for which the offers are active.

  - EILS Registration

Each EILS Resource, through its QSE, shall complete an EILS registration form developed by ERCOT.  Each EILS Resource, through its QSE, is responsible for ensuring that its EILS Registration form information is current.