| PRR Number |  | PRR Title | Emergency Interruptible Load Service (EILS) |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | Section 2: Definitions and Acronyms Section 4: Scheduling Section 5: Dispatch Section 6: Ancillary Services Section 9: Settlements Section 16: Registration |  |
| Requested Resolution (Normal or Urgent) |  | Urgent |  |
| Revision Description |  | Modify the Zonal Protocols to allow for the use of an Emergency Interruptible Load Service when EECP is declared and all available Generation and LaaR Resources have been deployed. |  |
| Reason for Revision |  | On September 15, 2006 ERCOT presented the need for another tool in the EECP toolbox to avoid firm load shed as done on April 17, 2006. This PRR attempts to provide to ERCOT the requested tool in a manner that is consistent with current market design and in a way that will minimize the costs to the market. |  |
| Credit Implications (Yes or No, and summary of impact) |  | Yes, QSEs will be required to procure or self-supply another service at a cost not yet determined. |  |
| Relevance to Nodal Market (Yes or No, and summary of impact) |  | Yes, a NPRR will need to be written to incorporate the EILSService into the Nodal market. |  |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  |  |  |
| Timeline |  |  |  |
| Date Posted |  |  |  |
| Please access the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name | EIL Task Force – Option B |
| E-mail Address |  |
| Company |  |
| Company Address |  |
| Phone Number |  |
| Fax Number |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 | Example: Key assumptions used in estimating market cost and/or benefit |  |
| --- | --- | --- | --- |
|  | 2 | Ex: Dependencies on other projects or other timing requirements |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | Example: Cost per MP to implement | Example: $10,000 each for 50 QSEs |
|  | 2 | Ex: Add’l staff required per MP | Ex: 1.5 FTE each for 6 TDSPs @ $65/hour |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Example: Reduced MP costs | Example: 2 FTE reduction for 25 CRs @ $65/hour |
|  | 2 | Ex: Enhanced MP efficiency | Ex: 2 hour savings per day for 50 generators @$65 |
|  | 3 | Ex: Reduced congestion cost | Ex: 0.5% reduction in total congestion cost |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | What to include here: Benefits that are difficult to quantify |  |
|  | 2 | What to include here: Benefits that are not certain but relatively likely |  |
|  | 3 | What to include here: Customer service impacts, cash flow impacts, transaction speed, etc. |  |
|  | 4 |  |  |
| Other | 1 | What to include here: Thoughts on ERCOT systems impacts |  |
| Comments | 2 | What to include here: Potential manual workarounds or delivery options |  |
|  | 3 | What to include here: Other comments of value to PRS, TAC and the Board of Directors |  |
|  | 4 |  |  |

Proposed Protocol Language Revision

Section 2: Definitions and Acronyms

[…]

Ancillary Services Plan

ERCOT produced plan, as described in Section 6, Ancillary Services, that identifies the types and amount of Ancillary Services necessary for each hour of the next day, next two (2) days, and/or the next month to operate the ERCOT Transmission Grid reliably in accordance with Operating Guides, and which includes the allocation of types and amounts of the Ancillary Services Obligations for each QSE.

[...]

Emergency Electric Curtailment Plan

A plan which provides an orderly, predetermined procedure for maximizing use of available Resources and, only if necessary, curtailing demand during electric system emergencies while providing for the maximum possible continuity of service and maintaining the integrity of the ERCOT System.

Emergency Interruptible Load Service

A service that is used during an EECP to reduce Load on the system after all non-EILS Resources have been deployed and prior to calling for firm load shedding.

Emergency Interruptible Load Service Business Hours (EILS Business Hours)

The hours of the day inclusive of hour ending 0900 through hour ending 2000 and excluding weekends and ERCOT holidays.

Emergency Interruptible Load Service Non-Business Hours (EILS Non-Business Hours)

Any hours not defined as an EILS Business Hour.

Emergency Notice

The fourth of four possible levels of communication issued by ERCOT in anticipation of a possible Emergency Condition detailed in Section 5.6.

[…]

Mismatched Schedule Processing Fee

The fee charged to a QSE that fails to correct a mismatched schedule in a timely manner.

Month Ahead

The calendar month period prior to the beginning of the first Operating Day in any calendar month.

Mothballed Generation Resource

A Generation Resource for which a Generation Entity has submitted a Notification of Suspension of Operations, for which ERCOT has declined to execute an RMR Agreement, and for which the Generation Entity has not announced retirement of the Generation Resource.

[…]

Operating Hour

The current clock hour.

Operating Month

The current calendar month.

Operating Period

A two-hour period comprised of the Operating Hour and the hour preceding the Operating Hour.

[…]

Resource

Facilities capable of providing electrical energy or Load capable of reducing, or increasing the need for electrical energy or providing Ancillary Services to the ERCOT System, as described in Section 6, Ancillary Services.  This includes Generation Resources, Loads acting as Resources, and Emergency Interruptible Load Service Resources.

[…]

  - Acronyms

[…]

EECP	Emergency Electric Curtailment Plan

EILS	Emergency Interruptible Load Service

EMS		Energy Management System

[…]

Section 4: Scheduling

    - Month Ahead Scheduling Process 
Emergency Interruptible Load Service shall not be scheduled using the Day Ahead or Adjustment Period scheduling process. Instead, EILS shall be scheduled as follows:

| Time Period at or before: | Qualified Scheduling Entity (QSE) Submission: | ERCOT Action: |
| --- | --- | --- |
| Month Ahead |  |  |
| At least 7 Business Days before the first Business Day of the Operating Month |  | - Publish on the Market Information System the 1000 MW of Emergency Interruptible Load Service to be procured for the following month (the “Period EILS Requirement”). ERCOT will procure separate MW quantities of Emergency Interruptible Load Service for EILS Business Hours and EILS Non-Business Hours. - Provide to each QSE its load ratio share Emergency Interruptible Load Service obligation (“EILS Obligation”) for the following month . |
| At least 5 Business Days before the first Business Day of the Operating Month | - Notify ERCOT of the quantity of EILS Obligation that will be self-arranged for the Operating Month. |  |
| At least 3 Business Days before the first Business Day of the Operating Month |  | - Conduct an auction for the EILS Market Obligation - Notify each bidding QSEs of its awarded Emergency Interruptible Load Service capacity for both the EILS Business Hours and EILS Non-Business Hours |
| At least 1 Business Days before the first Business Day of the Operating Month | - Submit to ERCOT a list of the EILS Resource ESI IDs , including for each resource (i) EILS Self-Arranged Obligation or EILS Award Obligation flag and (ii) EILS Committed Capacity by Operating Hour for each Operating Hour of the Operating Month |  |
| Operating Month |  |  |
| During the Operating Month | - As necessary, submit to ERCOT an updated list of the EILS Resource ESI IDs. |  |

4.2	Scheduling-Related Duties of ERCOT

ERCOT is responsible for administering the Day Ahead, Adjustment Period, and Month Ahead Scheduling Processes through which final schedules for energy and Ancillary Services are submitted by QSEs and a Current Operating Plan is developed for use in operating the ERCOT System in Real Time.  To fulfill its responsibilities with respect to providing information to the market necessary for QSEs to schedule energy and Ancillary Services, ERCOT shall:

(1)	Post forecasted ERCOT System conditions and Load for the next seven (7) days, by hour, by Congestion Zone.  ERCOT will also post any area Load forecasts that were used to create the system Load forecast;

(2)	Post forecasted Transmission Loss Factors and Distribution Loss Factors;

(3)	Post Load Profiles for non-IDR metered Customers;

(4)	Validate the QSEs’ energy schedules and make adjustments as necessary in accordance with the validation process;

(5)	Validate the QSEs’ Ancillary Service schedules and Self-Arranged Ancillary Service Resources;

(6)	Only accept schedules from a QSE; and

(7)	Require all Loads and Resources be represented by a QSE.

[…]

4.8	Temporary Deviations from Scheduling Procedures

If ERCOT is unable to comply with any of the deadlines in Sections 4.4, Day Ahead Scheduling Process, Section 4.5, Adjustment Period Scheduling Process, or Section 4.11, Month Ahead Scheduling Process it may temporarily deviate from those timing requirements to the extent necessary to ensure the secure operation of the ERCOT System.  Temporary measures may include varying the timing requirements as specified in Section 4.4.19, Decision to Extend Day Ahead Scheduling Process to Two Day Ahead Scheduling Process, or omitting one or more procedures in the Scheduling Process.  In such an event, ERCOT shall immediately declare an Emergency Condition and notify all QSEs of the following:

(1)	Details of the affected timing requirements and procedures;

(2)	Details of any interim requirements;

(3)	An estimate of the period for which the interim requirements will apply; and

(4)	Reasons for the temporary variation.

If, despite the variation of any time requirement or the omission of any procedure, ERCOT is unable to operate the Day Ahead Scheduling Process, ERCOT may abort the Day Ahead Scheduling Process and require all schedules to be submitted in the Adjustment Period.

If, despite the variation of any time requirement or omission of any step, ERCOT is unable to operate the Adjustment Period Scheduling Process, ERCOT may abort the Adjustment Period process and operate under its Operating Period procedures.

If ERCOT implements a Two Day Ahead Scheduling Process, that process shall be as described in Section 4.4.19, Decision to Extend Day Ahead Scheduling Process to Two Day Ahead Scheduling Process.

[…]

4.11 Month Ahead Scheduling Process

4.11.1 Overview

ERCOT will implement as described in this Section, the Month Ahead Scheduling Process, which will be utilized to procure Emergency Interruptible Load Service for the following Operating Month.  All ERCOT schedules, notices and other information requiring publication under this Section shall be published on the Market Information System (MIS) as set forth in Section 12 of these Protocols.

4.11.2 Posting of ERCOT Procurement

No later than seven Business Days before the first Business Day of the Operating Month, ERCOT will post the quantity of Emergency Interruptible Load Service (1000 MW) to be procured for the following Operating Month.

4.11.3 Notification to QSEs of Ancillary Service Obligations

ERCOT will notify QSEs by no later than seven (7) Business Days before the first Business Day of the Operating Month of the QSE’s Month Ahead Ancillary Service Obligations.  The Month Ahead Ancillary Service Obligations will be assigned to each QSE by multiplying the ERCOT Obligation by the calculated Load Ratio Shares as detailed in Section 6.3.1 of these Protocols, rounded to no fewer than six (6) decimal places, for the LSEs that a QSE represents.

4.11.4 ERCOT Month Ahead Ancillary Services Procurement Process

ERCOT will procure Emergency Interruptible Load no later than three (3) Business Days before the first Business Day of the Operating Month, in accordance with Section 6.6, Selection Methodology, to complete ERCOT’s Month Ahead Ancillary Service Plan.  ERCOT will review and then post on the MIS, the Market Clearing Prices for Capacity (MCPCs) for each Month Ahead Ancillary Service.

    - Month Ahead Ancillary Service Bid Submittal

QSEs may submit, by 1300 on the designated Month Ahead auction date, Ancillary Service bids to ERCOT for any Ancillary Service market open during the Month Ahead Period.  Submitted bids remain active for any single auction period until selected by ERCOT or the auction ends.

QSEs shall not submit bids to provide EILS at a price less than zero dollars.  In the event that ERCOT does not receive at least 100% of the quantity in bids of any Month Ahead Ancillary Service as posted in their Month Ahead Scheduling Process the Ancillary Services will be subject to a price cap.  This price cap shall not exceed XXXX ($XXXX) dollars per MW per hour for any Resource.

For Month Ahead Ancillary Service bids, QSEs must provide:

(1)	The market for which the bid is being supplied;

(2)	The period of time for which the bid is submitted (EILS Business Hours or EILS Non-Business Hours);

(3)	A dollars per megawatt price for the capacity bid;

(4)	The quantity of capacity for which the bid price is effective.

4.11.6  ERCOT Validation of QSE Month Ahead Bids

ERCOT will validate that the amount of each capacity bid is within the limits defined in Section 4.11.5, Month Ahead Ancillary Service Bid Submittal.  ERCOT will also validate that the QSE’s total MW submittal does not exceed the QSE’s amount of registered Resources for the particular Ancillary Service.

4.11.7 ERCOT Notification of Selected Ancillary Service Bids

By 1600, the day of the designated Month Ahead auction date, ERCOT will notify each QSE of the Month Ahead Ancillary Services procured from that QSE.

4.11.8 QSE Resource Plans

Each QSE that represents a Resource participating in a Month Ahead Ancillary Service will present a Month Ahead Resource Plan to ERCOT one Business Day before the first Business Day of the Operating Month.  The Month Ahead Resource Plan shall identify the specific Resources that are to provide each Month Ahead Ancillary Service.

As necessary, throughout the Operating Month the QSE shall update their Month Ahead Resource Plan to reflect any changes.  At a minimum any adjustments to the Month Ahead Resource Plan must be submitted prior to or during the Adjustment Period for the Operating Hour.

[…]

Section 5: Dispatch

5.6.7	EECP STEPS

Step 1 — Maintain ERCOT Physical Responsive Capability (PRC) on units plus RRS MW provided from LaaR Equal to 2300 Mw.  ERCOT will:

(1)	Utilize available DC tie capability that is not already being used by the market;

(2)	 Notify the Southwest Power Pool Security Coordinator;

(3)	Issue OOM dispatch instructions to uncommitted units available within the expected timeframe of the emergency; and

(

[PRR543: Add the following bullet upon ERCOT Legal review of the Comision Federal de Electricidad contract:] (5) Use available BtB import capacity that is not already being used and inquire about availability of Block Load Transfers.

QSEs will:

(1)  	Notify ERCOT of any resources uncommitted but available in the timeframe of the emergency.

Step 2 — Maintain ERCOT Physical Responsive Capability (PRC) on units plus RRS MW Provided from LaaR Equal to 1750 MW.  In addition to measures associated with Step 1, ERCOT will:

(1)	Instruct TDSPs to reduce Customers load by using distribution voltage reduction measures, if deemed beneficial by the TDSP;

(2)	Instruct QSEs to deploy all Responsive Reserve, which is supplied from Load acting as a Resource (controlled by high-set under-frequency relays); and

(3)	With the approval of the affected Non-ERCOT control area, may instruct TDSPs to implement BLTs, which transfer load from the ERCOT control area to Non-ERCOT control areas.  Use of a BLT will be defined in the ERCOT Operating Guides.

(4)	Unless such a media appeal is already in effect, ERCOT staff shall issue an appeal through the public news media for voluntary energy conservation.

Step 3 – Maintain system frequency at 59.8 Hz or greater

In addition to measures associated with Steps 1 and 2, ERCOT will deploy all Emergency Interruptible Load Service as a single block via VDI to all QSEs providing EILS. Participating loads will be deployed based on a control signal initiated by each QSE.  The control signal will initiate a pre-established time-delayed equipment trip, so that the load has time to deploy manually before, or to prepare for activation of, the time-delayed equipment trip. QSEs will be responsible for compliance within the required response time regardless of the pre-established time-delay.

Step 4 — Maintain system frequency at 59.8 Hz or greater

In addition to measures associated with Steps 1, 2, and 3, ERCOT will direct all TDSPs and their agents to shed firm load, in 100 MW blocks, distributed as agreed and documented in the ERCOT Operation procedures in order to maintain a steady state system frequency of 59.8 Hz.

In addition to measures associate with Steps 1 and 2, TDSPs will keep in mind the need to protect the safety and health of the community and the essential human needs of the citizens.  Whenever possible, TDSPs shall not manually drop load connected to under-frequency relays during the implementation of the Emergency Electric Curtailment Plan.

[…]

5.6.7.1	Restoration of Market Operations

ERCOT shall continue EECP until sufficient bids are received and deployed by ERCOT to eliminate the conditions requiring the EECP.  Emergency Interruptible Load Service Resources shall be recalled at the earlier of (i) reaching the limitations of the EILS guidelines or (ii) following initiation of the restoration of Load Acting as Resources.

[…]

Section 6: Ancillary Services

6.1	Ancillary Services Required by ERCOT

ERCOT shall be responsible for developing a daily Ancillary Services Plan with sufficient Ancillary Services (AS) to maintain the security and reliability of the ERCOT System consistent with ERCOT and North American Electric Reliability Council (NERC) standards.  The Ancillary Services required by ERCOT are described below.  ERCOT shall procure and deploy Ancillary Services on behalf of QSEs.

[…]

    - Emergency Interruptible Load Service

As provided by ERCOT to QSEs: An Ancillary Service ERCOT maintains to call upon as described in EECP Step 3 with the intention of potentially resolving the system contingency without calling upon firm load shedding.

As provided by a QSE to ERCOT:  The provision of capacity by loads capable of reducing their usage under the guidelines outlined in Section 6.5.12.

6.2	Providers of Ancillary Services

6.2.1	Ancillary Services Provided Solely by ERCOT

ERCOT is the sole provider of system-wide Balancing Energy Service; Generation Resource unit-specific Voltage Support Service (VSS), Black Start Service, Replacement Reserve Service, Zonal OOME Service, RMR Service, OOMC, and OOME Service to QSEs.

ERCOT will arrange Resources to provide system-wide VSS, Black Start, Zonal OOME, and RMR Service from QSEs.  ERCOT will direct Resources to provide OOMC or OOME in accordance with OOMC Service and OOME Service provisions of these Protocols.

6.2.2	Ancillary Services Provided in Part by ERCOT and in Part by Qualified Scheduling Entities

Each QSE may self-arrange its Obligation assigned by ERCOT for each of the following Ancillary Services:  Regulation Up, Regulation Down, Responsive Reserve, Non-Spinning Reserve, and Emergency Interruptible Load.  Any of the Ancillary Services that are not self-arranged will be procured as a service by ERCOT on behalf of the QSEs.

6.3	Responsibilities of ERCOT and Qualified Scheduling Entities

6.3.1	ERCOT Responsibilities

(1)	ERCOT, through its Ancillary Services function, shall develop the Operating Day Ancillary Services Plan for the ERCOT System, the Day Ahead Ancillary Service Obligation, and the Month Ahead Ancillary Service Obligation each of which will be assigned based on Load Ratio Share data, by LSE, aggregated to the QSE level.  Unless otherwise provided in these Protocols, a QSE’s allocation for Ancillary Service Obligation will be determined for each hour according to that LSE’s Load Ratio Share computed by ERCOT.  The LSE Ancillary Service allocation for the Day Ahead Ancillary Service Obligation shall be based on the hourly Load Ratio Share from the Initial Settlement data, for the Operating Day that is fourteen (14) days before the day in which the Obligation is being calculated, as defined in Section 9.2, Settlement Statements, for the same hour and day of the week multiplied by the quantity of the service in the Operating Day Ancillary Service Plan.

The LSE Ancillary Service allocation for the Month Ahead Ancillary Service Obligation for all hours of the Operating Month shall be based on the average of the daily Load Ratio Share from the Initial Settlement data, for the most recently available 14 days prior to the Operating Month for which the Obligation is being calculated, as defined in Section 9.2, Settlement Statements.

[PRR426: Replace Section 6.3.1, item (1) above with the following upon system implementation:] (1) ERCOT, through its Ancillary Services function, shall develop the Operating Day Ancillary Service Plan for the ERCOT System., The Day Ahead Ancillary Service Obligation, The Month Ahead Ancillary Service Obligation, except for Regulation Service, will be assigned based on Load Ratio Share data, by LSE, aggregated to the QSE level. The Day Ahead Regulation Service Obligation will be assigned based on Energy Ratio Share data, by LSE and Generation Resource, aggregated to the QSE level. Unless otherwise provided in these Protocols, a QSE’s allocation for all Ancillary Service Obligations except Regulation Service and Emergency Interruptible Load Service will be determined for each hour according to that LSE’s Load Ratio Share computation by ERCOT. For all Ancillary Services Obligations except Regulation Service and Emergency Interruptible Load Service, the LSE Ancillary Service allocation for the Day Ahead Ancillary Service Obligation shall be based on the hourly Load Ratio Share from the Initial Settlement data, for the Operating day that is fourteen (14) days before the day in which the Obligation is being calculated, as defined in Section 9.2, Settlement Statements, for the same hour and day of the week multiplied by the quantity of the service in the Operating Day AS Plan. For Emergency Interruptible Load Service, the LSE Ancillary Service allocation for the Month Ahead Ancillary Service Obligation shall be based on the average of the daily Load Ratio Share from the Initial Settlement data, for the most recently available 14 days prior to the Operating Month for which the Obligation is being calculated, as defined in Section 9.2, Settlement Statements. For Regulation Service, the LSE plus the Generation Resource Ancillary Service allocation shall be the hourly Energy Ratio Share of the Load data and the data from Uncontrollable Renewable Resources electing to utilize Renewable Production Potential for URC and OOME from the Initial Statement, as defined in Section 9.2, Settlement Statements, for the same hour and day of the week multiplied by the quantity of the service in the Operating Day Ancillary Service Plan.

(2)	ERCOT shall procure required Ancillary Services not self-arranged by QSEs.

(3)	ERCOT accepts Ancillary Service bids only from QSEs.

(4)	ERCOT shall allow the same capacity to be bid as multiple Ancillary Services types recognizing that this capacity may only be selected for one service.

(5)	ERCOT shall ensure provision of Ancillary Services to all ERCOT System Market Participants in accordance with these Protocols.

(6)	ERCOT shall not discriminate when obtaining Ancillary Services from QSEs submitting Ancillary Service bids.  ERCOT shall not discriminate between Self-Arranged Ancillary Services and ERCOT-procured Ancillary Services when dispatching Ancillary Services.

(7)	For AS that are not self-arranged, ERCOT shall procure any additional Resources ERCOT requires during the Month Ahead Scheduling Process, Day-Ahead Scheduling Process, the Adjustment Period process, or the Operating Period.

(8)	ERCOT shall procure Resources that are used to provide Reliability Must-Run Service or Black Start Service through longer-term Agreements.

(9)	Following submission of QSE self-arranged schedules, ERCOT will identify the remaining amount of Ancillary Services that must be acquired in order to complete ERCOT’s Day-Ahead Ancillary Services Plan.  Regulation Up, Regulation Down, Responsive, and Non-Spinning services will be procured by ERCOT on the timeline described in Section 4, Scheduling.

(10)	ERCOT will not profit financially from the market.  ERCOT will follow the Protocols with respect to the procurement of Ancillary Services and will not otherwise take actions regarding Ancillary Services with the intent to influence, set or control market prices.

(11)	ERCOT will provide that Market Clearing Prices are posted on the Market Information System (MIS) in a timely manner as stated in Section 12.4.1, Scheduling Information, of these Protocols.  ERCOT will monitor Market Clearing Prices for errors and will “flag” for further review questionable prices before posting, and make adjustments or notations in the posting if there are conditions that cause the price to be questionable.  ERCOT may only correct the price consistent with these Protocols.

(12)	ERCOT shall post the aggregated ERCOT AS Bid Stacks in accordance with Section 12.4.2, Ancillary Service Related Information of these Protocols.

(13)	ERCOT will, through procurement processes specified in these Protocols, procure Ancillary Services as required and charge QSEs for those Ancillary Services in accordance with these Protocols.

(14)	ERCOT will ensure ERCOT electric network reliability and adequacy and will afford the market a reasonable opportunity to supply reliability solutions.

(15)	ERCOT will not substitute one type of Ancillary Service for another.

(17)	ERCOT shall make reasonable efforts to minimize the use of OOMC, Zonal OOME, or contracted RMR Facilities.  This includes entering into MRA Agreements with Resources selected through the planning process, pursuant to Section 6.5.9.2, to provide services to meet reliability requirements at lower total expected costs than would otherwise be provided by RMR Agreements.

(18)	ERCOT will provide timely information to those Resource units providing OOMC and RMR Services as to the specific use of each unit dispatched.

6.3.2	Qualified Scheduling Entity Responsibilities

(1)	Unless contracted otherwise, and with the exception of Balancing Energy decremental bids as described in Section 4, Scheduling, of these Protocols, Resources capable of providing Ancillary Services are not required to provide those Resources or to submit bids to ERCOT, provided, however, Resources shall honor bids submitted to ERCOT for Ancillary Services under these Protocols and shall, use reasonable efforts to provide Ancillary Services in accordance with applicable emergency procedures in these Protocols and in the Operating Guides.

(2)	Ancillary Service providers shall provide and deploy, as directed by ERCOT, the Ancillary Service(s) that they have agreed to provide.

(3)	QSEs may specify Self-Arranged Ancillary Services in accordance with the Day-Ahead Scheduling as described in Section 4.4, Day Ahead Scheduling Process.

[PIP106: Current design does not provide for DLC Profiles. When DLC Profiles are implemented add this item (4) to section 6.3.2] (4) QSEs that have Direct Load Control programs as described in Section 18.7.2, Load Profiling of ESI IDs Under Direct Load Control, will notify ERCOT immediately of any deployment of the program. This applies solely to QSEs using Load Profiling for Settlement.

6.4	Standards and Determination of the Control Area Requirements for Ancillary Services

6.4.1	Standards for Determining Ancillary Services Quantities

(1)	ERCOT shall comply with the requirements for determination of Ancillary Service quantities as specified in these Protocols and the Operating Guides.

(2)	ERCOT shall, at least annually, determine with supporting data, the methodology for determining the minimum quantity requirements for each Ancillary Service needed for reliability, including the percentage of Load acting as a Resource allowed to provide Responsive Reserve Service calculated on a monthly basis.

(3)	ERCOT shall initially determine the percentage of Load acting as a Resource on a calendar month basis for the remainder of the current year allowed to provide Responsive Reserve Service within ninety (90) days after implementation of this Protocol revision.

(4)	The ERCOT Board shall review and approve ERCOT's methodology for determining the minimum Ancillary Service requirements and the monthly percentage of Load acting as a Resource allowed to provide Responsive Reserve Service.

(5)	If ERCOT determines a need for additional Ancillary Service Resources pursuant to these Protocols or the Operating Guides, after an Ancillary Services Plan for a specified day has been posted, ERCOT will inform the market by posting on the Market Information System, of ERCOT’s intent to procure additional Ancillary Service Resources in accordance with Section 4.5.8, ERCOT Notice to Procure Additional Ancillary Services.  ERCOT will post the reliability reason for the increase in service requirements.

(6)	Once specified by ERCOT for an hourly interval, Ancillary Service quantity requirements may not be decreased.

(7)	ERCOT shall instruct such that sufficient Resource capacity is on line, in appropriate locations, and available to ERCOT to meet the potential needs of the ERCOT System.

(8)	ERCOT shall include in its AS plan sufficient capacity to automatically control frequency to meet NERC standards.

(9)	ERCOT will post Engineering Studies on the MIS representing specific Ancillary Service requirements as described in Section 12, ERCOT Market Information System.

6.4.2	Determination of ERCOT Control Area Requirements

By the twentieth (20th) day of the current month, ERCOT will post a forecast of minimum Ancillary Services excluding Emergency Interruptible Load Service quantity requirements for the next calendar month.

Prior to 0600 of the Day Ahead, ERCOT will use the Day Ahead Load forecast and will develop an Ancillary Services Plan that identifies the amount of Ancillary Services necessary for each hour of the next day as specified in Section 4, Scheduling.  The amount of Ancillary Services required may vary depending on ERCOT System conditions from hour to hour.

By 0600 of the Day Ahead, ERCOT will post an ERCOT System and zonal Load forecast for the next seven (7) days, by hour.  ERCOT will notify each QSE of its allocated share of Ancillary Services for each hour for the next day, as specified in Section 4, Scheduling.  ERCOT will make available to Market Participants any ERCOT area Load forecasts used in the determination of its ERCOT System and zonal forecasts.

ERCOT will determine the total required amount of each Ancillary Service using the Operating Guides and the following:

(1)	Balancing Energy Service:  ERCOT will estimate Balancing Energy needs based on the actual Load, the difference in forecasted Loads and Loads reported in bilateral schedules, deployed Regulation Service, and forecasted Congestion.

(2)	Regulation Service:  ERCOT will use its operational judgment and experience to determine the quantity of Regulation Up Service and Regulation Down Service procured.  The quantity of Regulation Up Service may differ from the quantity of Regulation Down Service in any particular hour.

(3)	Responsive Reserve Service:  The requirement for Responsive Reserve Service is specified in the Operating Guides.  Using ERCOT-approved procedures ERCOT may increase the quantity requirement based on its judgment of reliability conditions.

(4)	Non-Spinning Reserve Service:  ERCOT will use its operational judgment and experience to determine the quantity of Non-Spinning Reserve Service procured.

(5)	Replacement Reserve Service:  Replacement Reserve Service is procured from Generation Resource units planned to be Off-line and LaaRs that are available for interruption during the period of requirement.  The QSE for a procured RPRS Resource must additionally make available to ERCOT via Balancing Energy Service bid(s) (i) for a Generation Resource, a quantity greater or equal to the High Sustainable Limit (if unavailable, the High Operating Limit) minus the Low Sustainable Limit (if unavailable, the Low Operating Limit) of the Generation Resource receiving the RPRS Dispatch Instruction in the Congestion Zone of the Generation Resource, and (ii) for a LaaR, a quantity equal to the amount of procured capacity from the LaaR in the Congestion Zone of the LaaR.  ERCOT will consider the Generation Resource capacity On-line, based on Resource Plans, in its determination of Zonal Congestion and Local Congestion requirements.  ERCOT will evaluate the need for Replacement Reserve Service necessary to correct for ERCOT total capacity insufficiency, Zonal Congestion, or Local Congestion.  ERCOT shall determine the amount of RPRS to provide sufficient capacity in appropriate locations to provide ERCOT System security as specified in the Operating Guides, given ERCOT forecasted Load conditions as posted on the Market Information System.

(6)	Voltage Support:  ERCOT in coordination with the TDSPs shall conduct studies to determine the normally desired Voltage Profile for all Voltage Support busses in the ERCOT System and shall post all Voltage Profiles on the Market Information System.  ERCOT may temporarily modify its requirements based on Current System Conditions.  ERCOT shall determine the amount of Voltage Support Service needed to provide sufficient reactive capacity in appropriate locations to provide ERCOT System security as specified in the Operating Guides.

(7)	Black Start Service: ERCOT shall periodically determine and review the location and number of Black Start Resources required, as well as any special transmission or voice communication needs required.  ERCOT and providers of this service shall meet the requirements as specified in the Operating Guides and in NERC policy.

(8)	Emergency Interruptible Load Service:  ERCOT shall procure 1000 MW of Emergency Interruptible Load Service for each hour of each category of hours (EILS Business Hours and EILS non-Business Hours) for each operating month.

6.5	Technical Requirements for Providers of Ancillary Services

Providers of Ancillary Services shall meet the general requirements specified in the subsection 6.5.1, General Technical Requirements below as well as the requirements of the specific Ancillary Service being provided, as described in Sections 6.5.2, Balancing Energy Service through 6.5.12 Emergency Interruptible Load Service.

6.5.1	General Technical Requirements

Providers of Ancillary Services shall meet the following general requirements.

6.5.1.1	Requirement for Operating Period Data for System Reliability and Ancillary Service Provision

Operating Period data will be used by ERCOT to monitor the reliability of the ERCOT System in Real Time, monitor compliance with Ancillary Service Obligations, perform historical analysis, and predict the short-term reliability of the ERCOT System using network analysis software.  Each TDSP, at its own expense, may obtain such Operating Period data from ERCOT or from QSEs.

(1)	A QSE representing a Generation Entity that has Generation Resources connected to a TDSP shall provide the following Real Time data to ERCOT for each individual generating unit at a Generation Resource plant location and ERCOT will make the data available to the Generation Resource’s host TDSP (at TDSP expense):

(a)	gross or net real power;

(b)	gross or net Reactive Power;

(c)	if gross quantities are provided, the plant auxiliary Load data will also be supplied;

(d)	status of switching devices in the plant switchyard not monitored by the TDSP affecting flows on the ERCOT System;

(e)	Frequency Bias of Portfolio Generation Resources under QSE operation;

(f)	any data mutually agreed by ERCOT and the QSE to adequately manage system reliability and monitor Ancillary Service Obligations;

(g)	generator breaker status;

(h)	High Operating Limit; and

(i)	Low Operating Limit.

[PRR590: Add items (j) and (k) upon system implementation:] (j) AGC status; and (k) ramp rate.

[PRR307: Revise Section 6.5.1.1(1) and 6.5.1.1(1) (c) & (g) as follows when system change implemented.] (1) A QSE representing a Generation Entity or a Competitive Retailer that has Resources connected to a TDSP shall provide the following Real Time data to ERCOT for each individual generating unit or LaaR capable of controllably reducing or increasing consumption under Dispatch control (similar to AGC) and that immediately respond proportionally to frequency changes (similar to generator governor action) at a Resource plant location and ERCOT will make the data available to the Resource’s host TDSP (at TDSP expense): (c) If gross quantities are provided, the plant auxiliary Load data will also be supplied as needed to determine the net output; (g) Resource breaker status;

[PRR590: Add paragraph (2) and renumber subsequent paragraphs upon system implementation:] (2) A QSE representing Uncontrollable Renewable Resources is exempt from the requirements of Section 6.5.1.1(1)(j) and (k).

(2)	Any QSE providing Responsive Reserve and/or Regulation must provide for communications equipment to receive ERCOT telemetered control deployments of service power.

(3)	Any QSE providing Regulation Service must provide appropriate Real Time feedback signals to report the control actions allocated to the QSEs Generation Resources.

(4)	Any QSE that represents a provider of Responsive Reserve, Non-Spinning Reserve, or Replacement Reserve using LaaR shall provide separate telemetry of the real power consumption of each LaaR providing the above Ancillary Services, the LaaR response to Dispatch Instructions for each LaaR, and the status of the breaker controlling that LaaR.  If LaaR is used as a Responsive Reserve Resource, the status of the high-set under frequency relay will also be telemetered.

[PRR307: Revise Section 6.5.1.1(3) and 6.5.1.1(4) as follows when system change implemented.] (3) Any QSE providing Regulation Service must provide appropriate Real Time feedback signals to report the control actions allocated to the QSEs Resources. (4) Any QSE that represents a provider of Responsive Reserve, Non-Spinning Reserve, or Replacement Reserve using interruptible Load as a Resource shall provide separate telemetry of the real power consumption of each interruptible Load providing the above Ancillary Services, the LaaR response to Dispatch Instructions for each LaaR, and the status of the breaker controlling that interruptible Load. If interruptible Load is used as a Responsive Reserve Resource, the status of the high-set under frequency relay will also be telemetered.

(5)	Any QSE that represents a qualified provider of Balancing Up Load (BUL) need not provide telemetry but rather shall provide an estimate in Real Time representing the real power interrupted in response to the deployment of Balancing Up Load.

(6)	Real Time data for reliability purposes must be accurate to within three percent (3%).  This telemetry may be provided from relaying accuracy instrumentation transformers.

[PRR590: Add paragraph (7) upon system implementation:] (7) A QSE representing a combined cycle plant may aggregate the AGC and ramp rate SCADA points for the individual units at a plant location into two distinct SCADA points (AGC and ramp rate) if the plant is configured to operate as such, i.e. gas turbine(s) and steam turbine(s) are controlled in aggregate from an AGC perspective.

- Any QSE that represents a qualified provider of Emergency Interruptible Load Service  need not provide telemetry but rather shall collect, and provide to ERCOT as necessary or upon request, accurate data representing each individual Resource’s energy consumption in one minute intervals using ERCOT approved technology and format.

6.5.1.2	Requirement for Operating Period Data for Emergency Interruptible Load Service Provision

The designated QSE of Emergency Interruptible Load Service Resources will provide to ERCOT within 48 hours of the end of an Emergency Interruptible Load Deployment one-minute interval data for the time period starting at the Settlement Interval in which ERCOT deployed the Emergency Interruptible Load and ending the end of the first full Settlement Interval following  termination of the Emergency Interruptible Load Service deployment.

[…]

    - Emergency Interruptible Load Service

- Emergency Interruptible Load Service may be provided by Resources which have a time delay control device installed on their system.  A QSE’s EILS Resources must be capable of reducing their load to a reduced capacity level equal to their baseline capacity less their committed capacity or maintaining a zero load level within ten (10) minutes of the instruction by ERCOT and must be able to maintain such reduced capacity level for the entire period of the service commitment.

- Any QSE providing Emergency Interruptible Load Service must be capable of communicating actual usage data, in one minute intervals, in the event of a deployment within 48 hours of such deployment to ERCOT in an ERCOT prescribed format.

- The minimum amount of EILS that may be offered to ERCOT is one (1) MW.

- QSEs that provide the Resources for Emergency Interruptible Load Service will ensure that Resources providing the service are able to meet their compliance obligations in the Operating Hour for which they have been selected to provide the service.  Each EILS Resource providing EILS must meet the additional technical requirements specified in Section 6.10, Ancillary Service Qualification, Testing, and Performance Standards of these Protocols.

- QSE bids for EILS will be in accordance with Section 4, Scheduling.

- For EILS, ERCOT will deploy 100% of the procured quantity, as a single block, when it is called.

- QSEs representing EILS Resources may aggregate multiple Resources to reach the one (1) MW offer requirement.

(8)	ERCOT will only deploy EILS as the last step before initiating firm load shed in accordance with Section 4, Dispatch of the Protocols.

(9) 	EILS Resources must have an installed interval data recorder (IDR) meter.

(10)	The EILS Resource must demonstrate that it can provide the required one-minute interval data to its designated QSE within one Business Day.

(11)	[The amount of EILS that ERCOT shall require the composite of all QSEs to make available in any calendar month is 1,000 MW per hour for each category of hours (EILS Business Hours and EILS non-Business Hours).]

(12)	In the event the deployment period covers both EILS Business Hours and EILS Non-Business Hours then the QSE shall deploy Resources so as to meet the QSE’s committed obligation for each hour.

(13)	The amount of monthly EILS that a QSE can self-arrange for the Operating Month is limited to its Ancillary Obligation of EILS for the Operating Month.  A QSE may bid additional EILS Resources into the ERCOT EILS market.

(14)	EILS Resources may not be deployed by ERCOT more frequently than the lesser of either (i) three (3) deployments per Operating Month or (ii) a total of 16 hours in a single Operating Month.

(15)	IMM quarterly reviews. (TXU to provide language)

6.6	Selection Methodology

6.6.1	Qualified Scheduling Entity Rights and Obligations to Self-Arrange Ancillary Service Resources

(1)	QSEs may self-arrange only Regulation Up, Regulation Down, Responsive Reserve Services, Non-Spinning Reserve Services, and Emergency Interruptible Load Services.

(2)	A QSE may self-arrange Resources by indicating the amount of each Ancillary Services that will be self-arranged in each hour of the Operating Day.  A QSE may self-arrange EILS Resources by indicating the fixed amount of Emergency Interruptible Load Services that will be self-arranged for  each category of hours (EILS Business Hour and EILS Non-Business Hour) for the Operating Month to ERCOT.  In addition, any self-arranged MW for EILS from EILS Resources may not be used during the committed period to provide other Ancillary Services.

(3)	The quantity self-arranged specified by a QSE at 1100 in the Day Ahead shall not be changed for the Day Ahead Obligation, unless ERCOT allows schedules to be updated at 1300 in accordance with Section 4.4.10, QSE Submittal of Updated Balancing Energy Schedules.

(4)	The quantity of Self-Arranged AS specified by a QSE in response to a Notice by ERCOT to obtain additional AS in the Adjustment Period cannot be greater than the allocated additional AS amount and cannot be changed once committed to ERCOT.

(5)	QSEs may schedule with ERCOT to provide Ancillary Services on another QSE’s behalf by notifying ERCOT consistent with the requirements of Section 4, Scheduling.

6.6.2	Competitive Procurement of Ancillary Service Resources by ERCOT

(1)	Except where stated to the contrary in these Protocols, ERCOT shall, to the extent the Ancillary Service Resource bids are available, use competitive procurement processes to procure sufficient Ancillary Service Resources to meet the requirements specified in these Protocols.

(2)	QSEs may submit bids to provide Regulation Down, Regulation Up, Responsive Reserves, Non-Spinning Reserves, and Emergency Interruptible Load, as part of the Scheduling Process in accordance with Section 4, Scheduling.

(3)	QSEs offering Balancing Energy Up, Balancing Energy Down, and Replacement Reserves can offer bids through the Adjustment Period, in accordance with Section 4.5, Adjustment Period Scheduling Process.

(4)	QSE’s bids to provide Ancillary Services will continue to be valid until withdrawn by the QSE prior to the market clearing or the market is cleared.  Bids may not be withdrawn during bid evaluation by ERCOT as described in Scheduling Sections 4.4 Day Ahead Scheduling Process, Section 4.11 Month Ahead Scheduling Process, and Section 4.5 Adjustment Period Scheduling Process or after the bid is selected.

(5)	QSEs may only submit bids from that portion of any Resource not used to provide capacity and energy to Supply the Resources in the QSE’s Balanced Schedule.  QSEs may only submit bids to provide Emergency Interruptible Load Services from EILS Resources that agree if committed to not use the same capacity during the committed period to provide other Ancillary Services.

(6)	ERCOT will determine the MCPC of each interval for Regulation Up, Responsive Reserve and Non-Spinning Reserve using an optimized simultaneous selection process in order to minimize the overall cost of these Ancillary Services.

(7)	Other than as specified for Congestion Management, ERCOT shall select Replacement Reserve and Regulation Down bids based on the lowest bid for each service.

(8)	Other than as specified for Congestion Management, ERCOT shall Dispatch energy from the Balancing Energy Service bids based on price Merit Order and the requirements specified in Section 5, Dispatch Instructions.

(9)	For Congestion Management, ERCOT shall, if possible, resolve all Congestion using the Balancing Energy Service by Congestion Zone.

(10)	ERCOT shall establish, through an annual competitive procurement process, annual agreements with Resources needed to provide Black Start capability.

(11)	For Emergency Interruptible Load Services, only Resources that are qualified by ERCOT and not on a probationary status may participate in the Month Ahead Scheduling Process.

[…]

6.6.8	ERCOT Month-Ahead Ancillary Service Procurement Process

(1)	ERCOT shall conduct a monthly bidding process prior to the start of each Operating Month but no later than three (3) Business Days before the first Business Day of the Operating Month for the purpose of procuring the quantities of EILS Market Obligation requested by the QSEs for all Operating Hours of the following Operating Month.

(2) 	For the Emergency Interruptible Load Service procurement process, ERCOT will select capacity bids submitted by QSEs such that the total EILS Market Obligation for the Operating Month for each category of hours (EILS Business Hours and EILS non-Business Hours) is met.  Each Emergency Interruptible Load Service bid will be arranged in the appropriate bid stack - one for EILS Business Hours and one for the EILS Non-Business Hours - such that the cost of the Emergency Interruptible Load Service is minimized.  For each of the bid stacks, if selection of the marginal capacity block will exceed ERCOT’s required Emergency Interruptible Load Service quantity, ERCOT will prorate the bids with the same price for the marginal capacity block such that only the actual amount of EILS Market Obligation capacity that is needed for each category of hours (EILS Business Hours and EILS non-Business Hours) is procured.

(3)	ERCOT will determine a fixed, monthly MCPC for all the EILS Business Hour Emergency Interruptible Load Service and a fixed monthly MCPC for all the EILS Non-Business Hour Emergency Interruptible Load Service.  ERCOT will post the required 1000 MW quantity requirement for EILS Market Obligation capacity and the monthly MCPC for both the EILS Business Hours and the EILS Non-Business Hours on the MIS.

[…]

6.8.6	Capacity Payments for Emergency Interruptible Load Service

Payments to Providers of Emergency Interruptible Load Service are described below:

6.8.6.1	Payments for Emergency Interruptible Load Service Capacity

(1)	A QSE whose bid to provide a EILS Resource to ERCOT is accepted for the EILS Business Hours or the EILS Non-Business Hours in the Month Ahead EILS auction procurement process shall be paid on an individual EILS Resource basis, multiplied by the appropriate monthly MCPC for either all the EILS Business Hours or all the EILS Non-Business Hours subject to the following adjustment:

(a)	If during the Operating Month there is an Emergency Interruptible Load Service deployment and the awarded EILS Resource fails to comply with its obligation as described in Section 6.11.4, Emergency Interruptible Load Service Deployment Performance Measures, then the total monthly MCPC payment to the QSE for that EILS Resource is reduced by the factor one (1) minus the product of (i) the difference between the EILS Baseline Capacity less EILS Reduced Operating Capacity, divided by (ii) the EILS Committed Capacity for the Operating Hour that ERCOT deployed the Emergency Interruptible Load Service.

(b) 	The payment to the QSE for each accepted EILS Resource for either the EILS Business Hours or EILS Non-Business Hours will be made once per month within twenty (20) days of the start of the following month.

6.8.6.2	Emergency Interruptible Load Service Payment to QSE

A QSE whose bid to provide Emergency Interruptible Load Service to ERCOT is accepted in ERCOT’s Emergency Interruptible Load Service procurement process shall be paid for the service per bid as follows:

PCEILBHqrim 	= 	-1 * CEILBHqrim * MCPCEILBHim

PCEILNBHqrim 	= 	-1 * CEILNBHqrim * MCPCEILNBHim

The equation below will be used to determine the Total Emergency Interruptible Load Service Payment to be allocated to each QSE as described in Section 6.9.XXX, Settlement for ERCOT Emergency Interruptible Load Service Capacity Procured in the month ahead.

PCEILi	=   ( (SUM (PCEILBHqrim)q)m  + (SUM (PCEILNBHqrim)q)m) ) x

(APCEILrdm/CCEILrdm)

Where:

i	interval being calculated

q	QSE

d	deployment by ERCOT of Emergency Interruptible Load Service

r	awarded EILS Resource

m	Procurement for market, m, of the month ahead period.

PCEILBHqrim	Emergency Interruptible Load Service for EILS Business Hours Payment per interval Procured Capacity for Emergency Interruptible Load Payments ($) per interval for that QSE for an awarded EILS Resource determined for market, m.

PCEILNBHqrim	Emergency Interruptible Load Service for EILS Non-Business Hours Payment per interval Procured Capacity for Emergency Interruptible Load Payments ($) per interval for that QSE for an awarded EILS Load Resource determined for market, m.

PCEILi	Procured Emergency Interruptible Load Capacity Costs ($) for the total market for the interval

CEILBHqrim	Awarded Emergency Interruptible Load Service Capacity for EILS Business Hours (MW) in the market, m, per interval for an awarded EILS Resource for that QSE

CEILNBHqrim	Awarded Emergency Interruptible Load Service Capacity for EILS Non-Business Hours (MW) in the market, m, per interval for an awarded EILS Resource for that QSE

MCPCEILBHi	Emergency Interruptible Load Market Clearing Price of Capacity for EILS Business Hours Cost ($/MW) per hour for market, m.

MCPCEILNBHi	Emergency Interruptible Load Market Clearing Price of Capacity for EILS Non-Business Hours Cost ($/MW) per hour for market, m.

APCEILrdm	Actual Performance Capacity of an EILS Resource for an ERCOT deployment, d, of Emergency Interruptible Load Service in any interval, i, during the month as determined by ERCOT in accordance with Section 6.11.5	Non-Compliance Actions and Performance Charges of ERCOT.  If there is no Emergency Interruptible Load Service deployment during a calendar month then APCEILrdm is deemed to equal CCEILrdm.

CCEILrdm	Awarded Committed Capacity by ERCOT to an EILS Resource for an ERCOT deployment, d, of Emergency Interruptible Load Service in any interval, i, during the month in accordance with Section 6.6.8, ERCOT Month-Ahead Emergency Interruptible Load Procurement Process.

[…]

6.9.9	Settlement for ERCOT Emergency Interruptible Load Service Procured for the Operating Month

(1)	ERCOT shall allocate to each QSE representing Load its portion of the total ERCOT requirement for Emergency Interruptible Load Service.   For settlement purposes, the QSE’s total capacity allocation for both EILS Business Hours and EILS Non-Business Hours shall be its fixed monthly EILS Obligation for all Operating Hours in the month which is determined as set forth in Section 4, Scheduling of the Protocols.

(2)	The QSE’s settlement capacity for each hour shall be the QSE’s total capacity allocation minus EILS Self-Arranged Obligation scheduled.

(3)	Each QSE’s settlement charge (or imbalance), in dollars, for Emergency Interruptible Load Service, by month, shall be equal to the sum of:

(a) 	The weighted average of the month’s MCPC for the Emergency Interruptible Load Service during EILS Business Hours and EILS Non-Business Hours for all Period Hours in the month (in dollars per megawatt), multiplied by the QSE’s EILS Market Obligation capacity for the month (in megawatts), plus

(b)	The sum of any EILS Resource performance charges determined in accordance with Section 6.11.5, Non-Compliance Actions and Performance Charges of ERCOT on an individual EILS Resource basis for each EILS Resource committed to provide Emergency Interruptible Load Service during the Operating Hour in which ERCOT deploys Emergency Interruptible Load Service, plus

(c)	The capacity payment/credit as calculated in Section 6.8.6.1, Payments for Emergency Interruptible Load Service Capacity.

6.9.9.1	Emergency Interruptible Load Service Charge

(1)	All costs of ERCOT procurement of Emergency Interruptible Load Service shall be calculated as follows:

LAEILqi 	=	EILPi  * NTOEILqi

EILPi 	=	(PCEILBHi + PCEILNBHi)  * -1 / (COBEILti – SAEILti)

NTOEILqi	=	(COBEILqi – SAEILqi)

Where:

i:	interval being calculated

LAEILqi	Emergency Interruptible Load Capacity Load Allocation Service Charge ($) per interval for that QSE

PCEILBHi	Procured Emergency Interruptible Load Capacity Costs ($) for EILS Business Hours for the total Market for the interval

PCEILNBHi	Procured Emergency Interruptible Load Capacity Costs ($) for EILS Non-Business Hours for the total Market for the interval

COBEILqi	Obligated Emergency Interruptible Load Capacity (MW) Service per interval of that QSE based on the QSE’s most recently calculated hourly Load Ratio Share

SAEILqi	Self Arranged Emergency Interruptible Load Capacity (MW) Service per interval of that QSE

EILPi	Emergency Interruptible Load Price per interval

NTOEILqi	Emergency Interruptible Load Net Obligation (MW) per QSE per interval

COBEILrti	Total Emergency Interruptible Load Obligations Capacity (MW) in that interval.

SAEILti	Total Self Arranged Interruptible Load Capacity (MW) in that interval

[…]

6.10	Ancillary Service Qualification, Testing and Performance Standards

6.10.1	Introduction

QSEs providing Ancillary Services shall meet qualification criteria and performance measures to operate satisfactorily with ERCOT.  ERCOT shall develop an Ancillary Services qualification and testing program and Real Time Monitoring Program for all suppliers of Ancillary Services that is based on the key factors needed for reliability.  These programs will be approved by ERCOT Technical Advisory Committee and will be included in the Operating Guides.  These performance factors shall be measured as precisely and efficiently as possible.  General capacity testing verifies a Generation Resources or Load acting as Resources and Net Dependable Capability.  Qualification tests allow the potential provider’s portfolio to demonstrate the minimum capabilities necessary to deploy an Ancillary Service, and performance measures assess the Real Time delivery of a service by an Ancillary Services provider.

All loads may be provisionally qualified for a period of ninety days as a Load acting as a Resource and may be eligible to participate as Resource.  Loads that have installed the appropriate equipment with verifiable testing data may be provisionally qualified as a provider of Ancillary Services once the specific requirements for the Ancillary Services specified in the Operating Guides have been completed to ERCOT’s reasonable satisfaction.

All loads may be provisionally qualified for a period of ninety (90) days to participate as a Balancing Up Load (BUL), provided the Load is metered with an IDR to ERCOT’s reasonable satisfaction.  Any Load may be qualified to participate as a BUL Resource, provided the Load meets the metering requirements (including any Sampling standards identified in Section 18, Load Profiling) established by ERCOT once the following ERCOT requirements have been met:

- ESI-ID registration of BUL by the QSE;

- Telemetered BUL response value is installed and tested between QSE and ERCOT; and

- For Dynamic Obligation Schedule (DSBUL), telemetered dynamic Obligation telemetry tested between QSE and ERCOT.

Provisional qualification as described herein may be revoked by ERCOT at any time for any non-compliance with provisional qualification requirements.

6.10.2	General Capacity Testing Requirements

QSEs shall provide ERCOT a list identifying each Generation Resource unit that is expected to operate more than one hundred sixty eight (168) hours in a Season as a provider of energy and/or Ancillary Services.  ERCOT shall evaluate, during each Season of expected operation, the Net Dependable Capability of each unit expected to operate more than one hundred sixty eight (168) hours during that season, except for any Generation Resources used solely for energy services and whose capacity is less than ten (10) MW.  Prior to the beginning of each Season, QSEs shall identify the Generation Resources to be tested during the Season and the specific week of the test if known.  This schedule may be modified by the QSE (including retests) during the Season.  QSEs not identifying a specific week for a Generation Resource unit test must test the unit within the first one hundred sixty eight (168) hours of run time during the Season or operate with a Net Dependable Capability equal to the highest integrated hourly MWh output demonstrated during the first one hundred sixty eight (168) hours of run time.  QSEs do not have to bring units On-line or shut down solely for the purpose of the seasonal verification.  Any unit for which the QSE desires qualification to provide Ancillary Services shall have its Net Dependable Capability verified prior to providing services using the Generation Resource unit even if it fits the less than one hundred sixty eight (168) hour or small capacity exception.  The capability of hydro units operating in the synchronous condenser fast response mode to provide hydro Responsive Reserve shall be evaluated by Season.

Load acting as a Resource to provide Ancillary Services shall have its telemetry attributes verified by ERCOT annually.  In addition, once every two (2) years, any LaaR providing Responsive Reserve Service shall test the under frequency relay or the output from the solid-state switch, whichever applies, for correct operation.  However, if the Load’s performance has been verified through response to an actual event, the data from the event can be used to meet the annual telemetry verification requirement for that year and/or the biennial relay testing requirement.

In order for Emergency Interruptible Load Resources to provide Emergency Interruptible Load Service they shall have the communication and metering attributes verified by ERCOT annually through a simulated load reduction.  However, if the Load’s performance has been verified through a response to an actual event, the data from the event can be used to meet the annual communication and metering verification requirements for that year.  EILS Resources must adhere to the qualifications, testing and performance standards as outlined in Section 6.11.

Specific Loads to be used for the first time as a Resource to provide Responsive Reserve, Non-Spinning Reserve or Replacement Reserve must be correctly evaluated (tripped or simulated trip, if approved by ERCOT) prior to their qualification to provide Ancillary Services.  At a time selected mutually by the Load and ERCOT, ERCOT will notify the QSE representing such Load of the need to confirm that Loads proposed to be used to provide Responsive Reserve, Non-Spinning Reserve or Replacement Reserve Services can be de-energized, or simulated, if applicable, to ERCOT’s reasonable satisfaction.  ERCOT shall develop a standard test for simulation for Load interruption required under this subsection.  Loads used to provide Responsive Reserve Service shall be qualified for correct operation by their host TDSP.

QSEs shall be responsible for qualifying any Load desiring to have the QSE represent it to provide Balancing Up Load (BUL) Service.

[PRR484: Add the following to the end of the above paragraph (“QSEs shall be responsible for qualifying…Balancing Up Load (BUL) Service. ”) upon system implementation:] Loads controlled under a qualified DLC program may be qualified as a group.

The QSE shall nominate to ERCOT, at least annually, that it is representing an amount of BUL for which it wishes to be qualified to provide.  At a time selected by ERCOT, the ERCOT operator will notify the QSE that it wants to verify the QSE’s ability to provide ERCOT with the appropriate signal simulating that it has initiated an ERCOT requested BUL reduction.  The QSE Operator will immediately simulate the initiation of the reduction and provide ERCOT with the appropriate signal representing some amount of Load to be qualified to provide BUL Resources.  Once ERCOT has verified that it has received an appropriate Load reduction signal from the QSE and has successfully completed the BUL registration process, the QSE will be qualified to provide BUL Resources.  Any changes to the BUL portfolio will require subsequent updates to the registration process.  For NOIEs representing specific Loads qualified as BULs that are located behind the NOIE Settlement Meter points, the NOIE shall provide an alternative unique descriptor of the qualified BUL Load for ERCOT’s records.

Generation Resources and Loads acting as Resources shall be evaluated at least annually by ERCOT for:

(1)	Correct operation of telemetry of the breakers controlling the Resource;

(2)	Correct mapping of QSE-provided telemetry of Ancillary Service energy to the appropriate energy Settlement Meter;

(3)	Data rate update requirements; and

(4)	Any other required telemetry attributes.

All Generation Resources and Loads acting as a Resource shall meet all requirements specified in the Operating Guides for proper response to system frequency.  ERCOT may reduce the amount a Resource may contribute toward Ancillary Services if it finds unsatisfactory performance of the Resource as defined in these Protocols and the Operating Guides.

Qualification of a Resource, including a Load acting as a Resource or a EILS Resource, shall remain valid for such Resource in the event of a change of QSE for the Resource, provided that the new QSE demonstrates to ERCOT’s reasonable satisfaction that the new QSE has adequate communications and control capability for the Resource.

[…]

6.11	Emergency Interruptible Load Service Qualification, Testing and Performance Standards

6.11.1	Introduction

QSEs providing Emergency Interruptible Load Services shall meet qualification criteria and performance measures to operate satisfactorily with ERCOT.  ERCOT shall develop an Emergency Interruptible Load Service qualification and testing program.  This program will be approved by ERCOT Technical Advisory Committee and will be included in the Operating Guides.  Qualification tests allow the potential EILS Resource to demonstrate the capabilities necessary to deploy the Emergency Interruptible Load Service, performance measures, and assess the delivery of this service by an Emergency Interruptible Load Service provider.

- All loads may be provisionally qualified as a EILS Resource for a period of ninety (90) days or until fully tested by ERCOT provided the following criteria are met to ERCOT’s reasonable satisfaction:

- Resource registration of EILS Resource by the QSE;

- Ability to upload one-minute interval data to the QSE in a format that meets ERCOT’s requirements; and

- Affidavit from the EILS Resource stating the Resource’s ability to receive the time-delay control signal from its QSE and demonstrate a receipt of the signal to trip the EILS Resource load committed to Emergency Interruptible Load Service.

ERCOT shall perform an EILS qualification test within a 90 day period from Registration.  Provisional qualification as described herein may be revoked by ERCOT at any time for any non-compliance with provisional qualification requirements.  During the provisional qualification period for the Emergency Interruptible Load Service Resource their maximum qualified amount to bid shall be the average of the Resource’s usage for the most recent two months, as calculated by the QSE and communicated to ERCOT.

6.11.2	General Capacity Testing Requirements

For initial qualification to provide Emergency Interruptible Load Service, a Load shall perform a full demonstration in accordance with EILS performance requirements to establish their ability to interrupt as required.  Loads may reapply for testing without penalty until qualified.  At a time determined by ERCOT, ERCOT will notify the QSE representing selected EILS Resources of the need to confirm that Loads proposed to be used to provide Emergency Interruptible Load Service can be interrupted within 10-minutes of ERCOT’s notification to ERCOT’s reasonable satisfaction.

QSEs representing EILS Resources shall have the ability to upload one minute interval meter data directly to ERCOT verified by ERCOT annually.

Once every year, the EILS Resource and its QSE shall test the time-delay trip for correct operation by verifying that the EILS Resource’s Load is tripped within the time used by the QSE.  Each EILS Resource’s designated QSE shall communicate this value to ERCOT for verification.  If the EILS Resource Load’s performance has been verified through a successful response to an actual event, this testing requirement is waived for the following year.

The QSE shall nominate to ERCOT, at least annually, that it is representing an amount of EILS Resources which it wishes to be qualified to provide.  At a time selected by ERCOT, the ERCOT operator will notify the QSE that it wants to verify the QSE’s ability to provide ERCOT with the appropriate signal simulating that it has initiated an ERCOT requested Emergency Interruptible Load deployment.  The QSE Operator will immediately simulate the initiation of the deployment and provide ERCOT with the appropriate signal representing some amount of Load to be qualified to provide EILS Resources.  Once ERCOT has verified that it has received an appropriate simulated Load reduction signal from the QSE and has successfully completed the Emergency Interruptible Load Service registration process, the QSE will be qualified to provide EILS.  Any changes to the EILS Resource portfolio will require subsequent updates to the registration process.  For NOIEs representing specific Loads qualified as EILS Resources that are located behind the NOIE Settlement Meter points, the NOIE shall provide an alternative unique descriptor of the qualified EILS Resource for ERCOT’s records.

Qualification of an EILS Resource shall remain valid for such resource in the event of a change of QSE for the resource, provided that the new QSE demonstrates to ERCOT’s reasonable satisfaction that the new QSE has adequate communications and control capability for the Resource to provide Emergency Interruptible Load Service.

6.11.3	Ancillary Services Qualification Criteria and Portfolio Test Methods

Only QSEs that have been qualified and tested may be used to provide Emergency Interruptible Load Service.  ERCOT shall develop and operate its qualification and testing program to meet the following requirements for the Emergency Interruptible Load Service.

A QSE shall be qualified and tested to provide Emergency Interruptible Load Service prior to initial operation and every five years thereafter.

ERCOT may grant a “Provisional Qualification,” for a period not to exceed ninety (90) days, to a QSE that has performed an Emergency Interruptible Load Service qualification test in good faith but failed to qualify due to problems that, in the sole discretion of ERCOT, are determined to be non-critical for the purpose of providing Emergency Interruptible Load Service.  Notwithstanding the failure of a QSE with Provisional Qualification to meet the applicable Emergency Interruptible Load Service criteria, such QSE may provide such Emergency Interruptible Load Service to the extent permitted by the terms of the Provisional Qualification.

6.11.3.1	Emergency Interruptible Load Service Qualification Testing Criteria

(1)	Testing using EILS Resource(s)

(a)	A test for EILS shall be performed during a continuous eight (8) hour window agreed on by the QSE and ERCOT.

(b)	ERCOT shall confirm the date and time of the test with the QSE using both the primary and alternate voice circuits to validate the voice circuits.

(c)	At any time during the window, selected by ERCOT when market and reliability conditions allow, and not previously disclosed to the QSE, ERCOT shall send a VDI to the QSE requesting the Resources to be tested to provide and deploy the Emergency Interruptible Load Service.  The QSE shall acknowledge the start of the test.

(d)	Upon ERCOT issuing a verbal instruction to the QSE to deploy the Emergency Interruptible Load Service, the QSE shall demonstrate that it has procedures in place to satisfy a satisfactory deployment of the EILS Resources as required in Step 3 of an EECP, Protocol Section 5.6.7, EECP STEPS, Step 3.  For the thirty (30) minute duration of the test, the Resource’s reduction shall be measured as clock-minute average outputs for:  (i) the clock-minute prior to the instruction being received from ERCOT; (ii) the clock-minute following receipt of instructions from ERCOT and continuing for ten (10) minutes; and (iii) for each of the subsequent nineteen (19) clock-minutes.  All measurements shall confirm the additional delivery of energy due to the deployment of Emergency Interruptible Load Service in an amount equal to at least ninety-five percent (95%) of the amount requested by ERCOT.  Satisfactory performance shall be deemed acceptable if: (i) ninety percent (90%) of each clock-minute measurement ten (10) minutes after notice through the balance of the test period is equal to at least ninety-five percent (95%) of the amount expected (utilizing the baseline from section 6.11.4) or (ii) if the Load drops to or remains at zero during the test

(e)	On successful demonstration of all test criteria, ERCOT shall qualify that the QSE is capable of providing EILS and shall provide a copy of the certificate to the QSE designating its qualification to provide EILS from EILS Resources.

6.11.4	Emergency Interruptible Load Service Deployment Performance Measures

(1)	ERCOT shall measure the performance of QSEs EILS Resources according to the following requirements.  ERCOT shall levy performance penalties against the EILS Resources represented by the QSE that perform at a less-than-satisfactory level.

(2)	Following deployment of Emergency Interruptible Load Service each EILS Resource’s designated QSE shall be responsible for uploading to ERCOT within 48 hours the one-minute interval data in accordance with Section 6.5.1.1.

(3)	An EILS Resource used by a QSE for either EILS Awarded Obligation or EILS Self-Arranged Obligation shall establish their baseline capacity calculation selection at Registration to determine their performance in the event of an ERCOT Emergency Interruptible Load Service deployment.  Each EILS Resource must choose one of the following three options for determining the applicable EILS Baseline Capacity:

(a)	The average load for the eight Settlement Intervals immediately prior to the Settlement Interval in which ERCOT issued the Emergency Interruptible Load Service instruction.

(b)	The ERCOT weather adjusted BUL baseline methodology specified in Section 6.8.1.14 to calculate AIMLDiq for an EILS Resource meter that represents a dynamic Load and AIMLSiq for an EILS Resource that represents a static Load.

[ERCOT to check effort to verify every interval of availability for committed EILS Resources in an Operating Month.  If added, penalties for not being available need to be addressed.]

(c)	The average load for all of the Settlement Intervals in the calendar month prior to the month in which the EILS Resource submits its bid.

(4)	EILS Resources must meet two performance criteria:

(a)	By the end of (10) minutes from the time ERCOT issues an Emergency Interruptible Load deployment instruction to the QSEs, each committed EILS Resource must have reduced its operating capacity by at least 95% of its bid amount or be off-line .  The EILS Reduced Operating Capacity is calculated as the average EILS Resource capacity measured for the first full one-minute interval after the 10-minute deployment period has expired.  For the EILS Resource the actual reduction will be measured as the difference between the calculated EILS Baseline Capacity and the EILS Reduced Operating Capacity.

(b)	Each committed EIL Load Resource or Aggregated Load Resource must remain at the required level of Reduced Operating Capacity or off-line following the deployment instruction and may not return to on-line status until permitted by ERCOT.

6.11.5	Non-Compliance Actions and Performance Charges of ERCOT

(1)	ERCOT may revoke the Emergency Interruptible Load Service qualifications of any QSE for failure of that QSE to adequately communicate required information to ERCOT at ERCOT’s discretion.

(2)	If an EILS Resource, that is part of a QSE’s EILS Award Obligation for the Operating Hour in which ERCOT deploys Emergency Interruptible Load Service, fails to comply with ERCOT Emergency Interruptible Load deployment instruction as determined by ERCOT in compliance with Section 6.11.4 Emergency Interruptible Load Service Deployment Performance Measures, then ERCOT will withhold from theRresource’s QSE as a performance charge a portion of the Resource’s total monthly Emergency Interruptible Load Service capacity payment calculated as one (1) minus the product of (i) the difference between the Resource’s EILS Baseline Capacity and the Resource’s EILS Reduced Operating Capacity divided by (ii) the Resource’s EILS Committed Capacity for the Operating Hour in which ERCOT deployed Emergency Interruptible Load Service.

(3)	If a EILS Resource fails (i) an annual re-qualification test, or (ii) incurs a penalty for, failure to comply as determined in compliance with Section 6.11.4, Emergency Interruptible Load Service Deployment Performance Measures then ERCOT will disqualify the EILS Resource at the end of the earliest Operating Month from providing Emergency Interruptible Load Service for a period of six months.  Following the disqualification period the disqualified EILS Resource must successfully pass a re-qualification test to be reinstated.

6.11.6   	Settlement Timeline and Dispute Process

(1)	ERCOT shall process any performance charges associated with failing to provide EILS Committed Energy for a ERCOT Emergency Interruptible Load Service deployment and assess them to the QSE representing a non-compliant EILS Resource for each ERCOT deployment of Emergency Interruptible Load Service within ten (10) Business Days after the ERCOT Compliance Department issues final determination of the compliance status of each committed EILS Resource for the Operating Hour in which the deployment took place.  ERCOT will apply debits onto Settlement Statements on a scheduled Final Settlement Statement of its choosing and in accordance with Section 9.2, Settlement Statements.

(2)	ERCOT shall process any performance charges associated with failing to comply with ERCOT’s deployment instructions as detailed in Section 6.11.4, Emergency Interruptible Load Service Deployment Measures within ten (10) Business Days of the end of the Operating Month in which the deployment occurred so they can be included in the monthly payment to the QSE for its EILS Award Obligation.

(3)	ERCOT may issue a Resettlement Statement for the performance charges and payments if required due to approved disputes or corrections.  Any such disputes must be submitted and approved thirty (30) days prior to the scheduled date for issuance of the True-Up Settlement Statement for the last day of the month in question.

(4)	Market Participants disputing their EILS Resource compliance with an ERCOT deployment of Emergency Interruptible Load Service Section 6.11.4, Emergency Interruptible Load Service Deployment Performance Measures will make disputes known to the ERCOT Compliance Department prior to the date on which ERCOT issues the performance charges for a Emergency Interruptible Load Service deployment.  After the initial performance charges are issued, Market Participants will submit disputes relating to Emergency Interruptible Load Service Deployment Performance Measures in accordance with Section 9.5, Settlement and Billing Dispute Process.

[…]

Section 9: Settlement and Billing

9.2.2	Settlement Statement Data

Settlement data used to prepare each Settlement Statement will include any available:

(1)	Aggregated estimated and actual interval and profiled consumption data for Loads represented by an LSE by specific Congestion Zones;

(2)	Aggregated estimated and actual interval generation data for generation by specific Congestion Zones;

(3)	Aggregated estimated and actual interval consumption data for UFE zones (if there are multiple UFE zones);

(4)	Resource schedules less Inter-QSE Trades;

(5)	Load schedules less Inter-QSE Trades;

(6)	Hourly and Monthly MCPCs for each type of Ancillary Service capacity;

(7)	Shadow Prices for RPRS capacity purchases;

(8)	MCPEs per Settlement Interval for Balancing Energy Services;

(9)	Total instructed energy for each Settlement Interval by Congestion Zone and by total ERCOT System;

(10)	Hourly and Monthly Obligations and any self-arranged Obligations for each Ancillary Service;

(11)	Adjustments from approved disputes or errors in data components;

(12)	Other prices and/or schedules for Market Services other than Ancillary Services;

[…]

Section 16: Registration and Qualification of Market Participants

16.5	Registration of Generation Resources, Loads Acting as a Resource, and Emergency Interruptible Load Resources

Any Generation Resource connected to the ERCOT Transmission System, any distributed generator over 1 MW, Load Acting as a Resource, or a Emergency Interruptible Load Resource must register with ERCOT.  Distributed generation is an electrical generating facility located at a customer’s point of delivery (point of common coupling) of ten (10) megawatts (MW) or less and connected at a voltage less than or equal to 60 kilovolts (kV) which may be connected in parallel operation to the utility system.

To register as a Resource, an applicant must submit a Resource application, execute a Resource Agreement and be capable of performing the functions of a Resource as described in these Protocols.

16.5.1	Responsibilities of the Resource

Each Resource shall be responsible for conducting its operations in accordance with all applicable ERCOT Protocols and guidelines.

16.5.1.1	Waiver for Federal Hydroelectric Facilities

ERCOT may grant a waiver to any federally owned hydroelectric Resource facility, within the ERCOT system, from fulfilling the requirements in Section 16.5 as they pertain to the submission of a Resource application and the execution of a Resource Agreement.  ERCOT may grant such waiver after the federally owned hydroelectric Resource facility provides ERCOT with the following:

(1)	All information necessary to meet the Resource registration requirements as provided in Section 16, Registration and Qualification of Market Participants;

(2)	The designation of a QSE for the Resource; and

(3)	A Load Serving Entity must be assigned to the ESI ID associated with any Load or net Load, if the Resource is net metered, which will be connected to the ERCOT System.  Such Load, if retail Load, shall be subject to all applicable rules and procedures, including rules concerning disconnection and POLR service, applicable to retail points of delivery.

16.5.1.2	Waiver for Block Load Transfer Resources

ERCOT may grant a waiver to any BLT Resource from fulfilling the requirements in Section 16.5, Registration of Generation Resources and Loads Acting as a Resource, as they pertain to the submission of a Resource application and the execution of a Resource Agreement.  ERCOT may grant such waiver after the BLT Resource provides ERCOT with the following:

(1)	All applicable information necessary to meet the Resource registration requirements as provided in Section 16; and

(2)	The designation of a QSE for the BLT Resource.

16.5.2	Registration Process for a Resource

To register as a Resource, an Entity must:

(1)	Designate and maintain a relationship with a QSE or QSEs as specified in the Protocols,

(2)	Demonstrate to ERCOT’s reasonable satisfaction that it is capable of performing the functions of a Resource,

(3)	Demonstrate that it is capable of complying with the requirements of all ERCOT Protocols and guidelines.

16.5.2.1	Technical and Managerial Resource Requirements

Technical and managerial resource requirements for Resources include:

(1)	Capability to comply with all policies, rules, guidelines, and procedures established by these Protocols, ERCOT or other Independent Organization, if applicable.

(2)	Capability to comply with ERCOT’s registration requirements or other Independent Organization and its system rules and contract for the purchase of power from Entities registered with or by the ERCOT or Independent Organization and capable of complying with its system rules.

16.5.2.2	Designation of a QSE

Each applicant shall designate in its Resource application the QSE that will represent the applicant for purposes of scheduling and settlement with ERCOT.  Each applicant shall acknowledge in its application that it bears sole responsibility for selecting and maintaining a commercial relationship with a QSE.  The applicant shall include in its application, a written statement from the designated QSE acknowledging that the QSE accepts responsibility for the applicant’s scheduling and settlement transactions pursuant to these Protocols.

16.5.2.3	Incomplete Resource Applications

Not more than ten (10) Business Days after receipt of each application, ERCOT shall notify the applicant in writing whether the application is complete.

If ERCOT determines that an application is not complete, ERCOT’s notification shall explain the reasons therefore and the additional information necessary to make the application complete.  The applicant shall then have five (5) Business Days from the receipt of ERCOT’s notification, or such longer period as ERCOT may allow, to provide the additional information set forth in ERCOT’s notification.  If the applicant timely responds to ERCOT’s notification, then the application shall be deemed complete on the date that ERCOT receives the applicant’s response.

If the applicant does not timely respond to ERCOT’s notification, then the application shall be rejected, and ERCOT shall retain any application fee included with the application.

16.5.2.4	Approval of the Resource Application

ERCOT shall approve or reject each application not more than ten (10) Business Days after the application has been deemed complete in accordance with this subsection.  Upon reasonable notice to the applicant, ERCOT may conduct a site visit as part of its evaluation of an application.

If ERCOT approves an application, it shall send an approval letter to the applicant, along with an agreement and any required software licensing agreements for the applicant’s signature.  The Resource shall be deemed registered when ERCOT receives back the executed agreement.

If ERCOT rejects an application, ERCOT shall send the applicant a rejection letter explaining the grounds upon which ERCOT has rejected the application.  Appropriate grounds for rejecting an application include:

(1)	Non-compliance with technical requirements; and

(2)	Non-compliance with other specific eligibility requirements set forth in this Section or in any other part of these Protocols.

Not later than ten (10) Business Days after receipt of a rejection letter, the applicant may challenge the rejection of its application utilizing the dispute resolution procedures set forth in Section 20, Alternative Dispute Resolution Procedures.  Regardless of whether or not the applicant challenges the rejection, the applicant may submit a new application and ERCOT shall process the new application in accordance with this subsection.

16.5.3	Requirements for Reporting and for Changing the Terms of a Resource Registration

In order to maintain its registration status, the Resource must keep the registration information up to date, pursuant to the following requirements:

(1)	A Resource may change its designation of QSE no more than once in any given three (3) day period.

(2)	The Resource shall include a written statement from the designated QSE acknowledging that the QSE accepts responsibility for the Resource’s scheduling and settlement transactions pursuant to these Protocols.

(3)	If a Resource’s representation by a QSE will terminate or the Resource intends to be represented by a different QSE, the Resource shall submit updated QSE representation information to ERCOT.  Within two (2) days of receiving notice, ERCOT will notify all affected parties, including the Resource’s current QSE, of the effective date of the change.

(4)	If a Resource receives notification from its QSE or from ERCOT that its current QSE intends to terminate representation, then the Resource must submit updated QSE representation information to ERCOT in accordance with this Section.  ERCOT’s systems will reflect the relationship between the Resource and its new QSE upon successful completion of all necessary testing, e.g., telemetry, SCADA and RTU (Remote Terminal Unit) points.  ERCOT will notify all affected parties (current QSE, new QSE, and affected Resource) of the effective date.

(5)	If a Resource Entity has a Switchable Resource with a requirement in the grid outside the ERCOT Region for the months of July through August (peak period), it shall report to ERCOT in writing on June 8, 2005 and then annually by April 1, the days that the identified capacity will not be available to the ERCOT System during the peak period.

Each Resource shall notify ERCOT within seven (7) Business Days of any material changes in the information included on its application, except as specified in Section 16.5.3(5).