Non-Spin Deployment Notification and Availability

Introduction:

The current Nodal protocols require ERCOT to develop a procedure, approved by TAC, to deploy Non-Spin. While analyzing the Non-Spin requirements, ERCOT and software vendors felt that some changes are suggested to improve the deployment and recall of Non-Spin. ERCOT operations, Consultants, and software vendors from AREVA and ABB, recommend two proposals: this white paper describes the business process associated with those proposals.

Nodal Recommendations:

Non-Spin Reserve Notification Process

The Nodal Protocol requires ERCOT notifications for deployment to QSEs with Resources providing Non-Spin include the MW deployment level and anticipated duration of Non-Spin deployment. The deployment of Non-Spin must always be 100% of that scheduled on an individual Resource, It is not necessary to deploy all capacity from non-spin resources at one time. An operations process will be defined to prioritize the deployment of off-line non-spin Resources..

The Generation Subsystem will send a Resource-specific Non-Spin deployment according to the Nodal protocols (including the timestamp of capacity deployment).  If Non-Spin responsibility among generators changes in hours following a Non-Spin deployment, ERCOT will send resource-specific deployment instructions for the new generators receiving Non-Spin responsibility.