| NPRR Number | 035 | NPRR Title | Nodal Protocol Clarifications Required For Net Metering Provisions |
| --- | --- | --- | --- |
| Date |  | December 5, 2006 |  |
| Submitter’s Information |  |  |  |
| Name |  | Trip Doggett on behalf of TPTF |  |
| E-mail Address |  | tdoggett@ercot.com |  |
| Company |  | ERCOT |  |
| Company Address |  | 2705 West Lake Drive, Taylor, Texas 76574 |  |
| Phone Number |  | (512)-248-6360 |  |

Comments

On 11/28/06, TPTF reviewed ERCOT comments to NPRR035 and voted to recommend approval of NPRR035 as revised by ERCOT comments and TPTF.  The vote tally was 91.7% in favor and 8.3% (IOU) in opposition with three abstentions (Coop, MOU, IOU).  All Market Segments were present for the vote.

TPTF discussed the comments submitted by LCRA, but did not take a vote on their inclusion as they were considered peripheral to the purpose of this NPRR in codifying net metering.

Revised Proposed Nodal Protocol Language

1.3.1.1		Items Considered Protected Information

Subject to the exclusions set out in Section 1.3.1.2, Items Not Considered Protected Information, “Protected Information” is information containing or revealing any of the following:

(a)	Base Points, as calculated by ERCOT;

(b)	Bids, offers, or pricing information identifiable to a specific QSE or Resource;

(c)	Status of Resources, including outages, limitations, or scheduled or metered Resource data;

(d)	Current Operating Plans;

(e)	Ancillary Service Trades, Energy Trades, Capacity Trades, and Ancillary Service Schedules identifiable to a specific QSE or Resource;

(f)	Dispatch Instructions identifiable to a specific QSE or Resource, except for RUC commitments and decommitments  as provided in Section 5.5.3, Communication of RUC Commitments and Decommitments;

(g)	Raw and Adjusted Metered Load data (demand and energy) identifiable to a specific QSE or Customer;

(h)	Settlement Statements and Invoices identifiable to a specific QSE;

(i)	Aggregated raw and Adjusted Metered Load data (demand and energy), and number of Electric Service Identifiers (ESI IDs) identifiable to a specific Load Serving Entity (LSE);

(j)	Information related to generation interconnection requests, to the extent such information is not otherwise publicly available;

(k)	Resource-specific costs, design and engineering data;

(l)	Congestion Revenue Rights (CRR) credit limits, the identity of bidders in a CRR Auction, or other bidding information identifiable to a specific CRR Account Holder;

(m)	Renewable Energy Credit (REC) account balances;

(n)	Credit limits identifiable to a specific QSE;

(o)	Any information that is designated as Protected Information in writing by Disclosing Party at the time the information is provided to Receiving Party except for information:

(i)	submitted to or collected by ERCOT under the Protocols or Other Binding Documents; or

(ii) 	provided to ERCOT in support of a Reliability Must-Run (RMR) application under Section 3.14.1, Reliability Must Run;

(p)	Any information compiled by a Market Participant on a Customer that in the normal course of a Market Participant’s business that makes possible the identification of any individual Customer by matching such information with the Customer’s name, address, account number, type of classification service, historical electricity usage, expected patterns of use, types of facilities used in providing service, individual contract terms and conditions, price, current charges, billing record, or any other information that a Customer has expressly requested not be disclosed (“Proprietary Customer Information”) unless the Customer has authorized the release for public disclosure of that information in a manner approved by the PUCT.  Information that is redacted or organized is such a way as to make it impossible to identify the Customer to whom the information relates does not constitute Proprietary Customer Information;

(q)	Any software, products of software, or other vendor information that ERCOT is required to keep confidential under its agreements;

(r)	QSE, TSP, and DSP backup plans collected by ERCOT under the Protocols or Other Binding Documents;

(s)	Direct Current (DC) Tie information provided to  a TSP or DSP under Section 4.4.4, DC Tie Schedules; and

(t)	Any Texas Standard Electronic Transaction (SET) transaction submitted by an LSE to ERCOT or received by an LSE from ERCOT.  This paragraph does not apply to ERCOT’s compliance with:

(i)	PUCT Rules on performance measure reporting;

(ii)	these Protocols or Other Binding Documents; or

(iii)	any TAC-approved reporting requirements.

(u)	Mothballed Generation Resource updates and supporting documentation submitted pursuant to Section 3.14.1.6, Mothballed Generation Resource Time to Service Updates.

(v)	For purposes of Capacity Demand Reserve Reporting, the unavailability of Switchable Generation Resources to the ERCOT System and supporting documentation submitted pursuant to Section 16.5.4(2), Maintaining and Updating Resource Entity Information, except for reporting the aggregate capacity.

(w)	Information provided by Entities under Sections 10.3.2.3, Generation Netting for ERCOT Polled Settlement Meters, and 10.3.2.4, Reporting of Net Generation Capacity.

(x)	Alternative fuel reserve capability and firm gas availability information submitted pursuant to Section 6.5.9.3.1, Operating Condition Notice, Section 6.5.9.3.2, Advisory, and Section 6.5.9.3.3, Alert; and as defined by the Operating Guides.

3.7.1	Resource Parameter Criteria

3.7.1.1	Generation Resource Parameters

(1)	General Resource Parameters submitted by a QSE must include the following for each Generation Resource represented by the QSE:

(a)	The Resource’s name;

(b)	High Reasonability Limit used to verify operator entries of HSL;

(c)	Low Reasonability Limit used to verify operator entries of LSL;

(d)	Type of Resource – steam turbine, hydro, gas turbine, combined cycle, other;

(e)	QF status, if applicable;

(f)	Normal Ramp Rate curve;

(g)	Emergency Ramp Rate curve;

(h)	Minimum On-Line time – The minimum number of consecutive hours the Resource must be On-Line before being shut down;

(i)	Minimum Off-Line time – The minimum number of consecutive hours the Resource must be Off-Line before being restarted;

(j)	Hot start time – The time, in hours, from the ERCOT notice to the Resource breaker-closing, for a Resource in its hot-temperature state;

(k)	Intermediate start time – The time interval, in hours, from the ERCOT notice to the Resource breaker-closing, for a Resource in its intermediate temperature state; and

(l)	Cold start time – The time interval, in hours, from the ERCOT notice to the Resource breaker-closing, for a Resource in its cold-temperature state.

(2)	Seasonal Resource Parameters must be submitted by a QSE and must include the following for each Generation Resource represented by the QSE:

(a)	Seasonal Net MW rating;

(b)	Conversion constants to be used to convert from Gross MW to Net MW in accordance with ERCOT Operating Guides, if applicable;

(c)	Maximum weekly starts – The maximum number of times a Resource can be started in seven consecutive days under normal operating conditions;

(d)	Maximum On-Line time – The maximum number of consecutive hours a Resource can run before it needs to be shut down;

(e)	Maximum daily starts – The maximum number of times a Resource can be started in a 24 hour period under normal operating conditions;

(f)	Maximum weekly energy – The maximum amount of energy, in MWh, a Resource can produce in seven consecutive days;

(g)	Hot-to-intermediate time – The time, in hours, after shutdown that a hot-temperature-state Resource takes to cool down to intermediate-temperature state; and

(h)	Intermediate-to-cold time – The time, in hours, after shutdown that an intermediate-temperature-state Resource takes to cool down to cold-temperature state.

6.5.5.2	Operational Data Requirements

(1)	ERCOT shall use Operating Period data to monitor and control the reliability of the ERCOT Transmission Grid and shall use it in network analysis software to predict the short-term reliability of the ERCOT Transmission Grid.  Each TSP, at its own expense, may obtain that Operating Period data from ERCOT or directly from QSEs.

(2)	A QSE representing a Generation Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time telemetry data to ERCOT for each Generation Resource.  ERCOT shall make that data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to requesting TSPs and DSPs operating within ERCOT. Such data must be provided to the requesting TSP or DSP at the requesting TSP’s or DSP’s expense, including:

(a)	Net real power (in MW) as measured by installed power metering or as calculated in accordance with ERCOT Operating Guides based on metered gross real power and conversion constants determined by the Resource Entity and provided to ERCOT as a result of section 3.7, Resource Parameters.  Net real power represents the actual generation of a Resource for all real power dispatch purposes, including use in SCED, determination of HASL, HDL, LDL and LASL, and is consistent with telemetered HSL and LSL;

(b)	Net Reactive Power (in Mvar);

(c)	Power to standby transformers serving Plant auxiliary Load;

(d)	Status of switching devices in the plant switchyard not monitored by the TSP or DSP affecting flows on the ERCOT Transmission Grid;

(e)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(f)	Generation Resource breaker and switch status;

(g)	High Sustained Limit;

(h)	High Emergency Limit, under Section 6.5.9.2, Failure of the SCED Process;

(i)	Low Emergency Limit, under Section 6.5.9.2, Failure of the SCED Process;

(j)	Low Sustained Limit;

(k)	Ancillary Service Schedule for each quantity of Reg-Up, Reg-Down, Responsive Reserve and Non-Spin; and

(l)	Reg-Up, Reg-Down and Responsive Reserve Services participation factors that represent how a QSE is deploying the Ancillary Service energy on a percentage basis to specific qualified Resource.

(3)	For each wind-powered Generation Resource the QSE shall set the HSL to the output capability of the facility based upon all available units and the current measured wind speed (HSL must be equal to or greater than the latest telemetered net real power of the wind-powered Generation Resource).

(4)	A QSE representing a Load Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time data to ERCOT for each Load Resource and ERCOT shall make the data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to the Load Resource’s host TSP or DSP at the TSP or DSP expense. The Net real power consumption, LPC and MPC shall be telemetered to ERCOT using a negative (-) sign convention:

(a)	Net real power consumption (in MW);

(b)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(c)	Load Resource breaker status;

(d)	Low Power Consumption (LPC);

(e)	Maximum Power Consumption (MPC)

(f)	Ancillary Service Schedule for each quantity of Reg-Up, Reg-Down, Responsive Reserve and Non-Spin; and

(g)	The status of the high-set under-frequency relay, if required for qualification;

(h)	For a Controllable Load Resource the Scheduled Power Consumption that represents zero Ancillary Service deployments; and

(i)	For a Controllable Load Resource, Net Reactive Power (in MVar).

(5)	A QSE with Resources used in SCED shall provide communications equipment to receive ERCOT-telemetered control deployments.

(6)	A QSE providing any Regulation Service shall provide telemetry indicating the appropriate status of Resources providing Reg-Up or Reg-Down, including status indicating whether the Resource is temporarily blocked from receiving Reg-Up and/or Reg-Down deployments from the QSE.

(7)	Real-Time data for reliability purposes must be accurate to within three percent.  This telemetry may be provided from relaying accuracy instrumentation transformers.

(8)	Each QSE shall report the current configuration of combined-cycle Resources that it represents to ERCOT.

(a)	Each configuration for a power block of combined-cycle Resources is considered as a single Resource unless multiple generators are connected to the ERCOT Transmission Grid at different voltage levels.

(b)	Each QSE shall use continuous telemetry to report changes to Combined-Cycle Configurations.  Changes must be reported by changing the Resource Status in Real-Time and in COP for that Resource representing the desired Combined-Cycle Configuration.  Each QSE shall provide ERCOT with the elements comprising each Combined-Cycle Configurations for a Resource through the Registration system, through Real-Time telemetry, and by appropriate entries in the COP.

(c)	Each QSE shall provide individual telemetered generator output (MW and Mvar) and Resource Status that indicates the Combined-Cycle Configuration to be used in SCED and RUC.

(9)	A QSE representing combined-cycle Resources shall provide ERCOT with the possible operating configurations for each power block with accompanying limits and price points.  Power augmentation methods must be made available to ERCOT as part of one or more of the configurations.  Price points for the range of the curve represented by the power augmentation method must reflect the price of the added capability.  Such power augmentation methods may include:

(a)	Combustion turbine inlet air cooling (CTIAC) methods;

(b)	Duct firing;

(c)	Other ways of temporarily increasing the output of combined-cycle Resources; and

(d)	For QFs, an LSL that represents the minimum energy available, in MW, from the Resource for economic dispatch based on the minimum stable steam delivery to the thermal host plus a justifiable reliability margin that accounts for changes in ambient conditions.

6.6.3.1	Real-Time Energy Imbalance Payment or Charge at a Resource Node

(1)	The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time SPP for the following amounts at a particular Resource Node Settlement Point:

(a) 	The energy produced by all its Generation Resource s at the Settlement Point; plus

(b) 	The amount of its Self-Schedules with sink specified at the Settlement Point; plus

(c) 	The amount of its Energy Bids cleared in the DAM at the Settlement Point; plus

(d) 	The amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(e) 	The amount of its Self-Schedules with source specified at the Settlement Point; minus

(f) 	The amount of its Energy Offers cleared in the DAM at the Settlement Point; minus

(g) 	The amount of its Energy Trades at the Settlement Point where the QSE is the seller.

(2)	The payment or charge to each QSE for Energy Imbalance Service at a Resource Node Settlement Point for a given 15-minute Settlement Interval is calculated as follows:

If the Generation Resources at the Resource Node Settlement Point p are involved with a net metering scheme

RTEIAMT q, p	=	(-1) * { Σr(NMALLOCF r * NMSAMTTOTa ) + RTSPP p * [(SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)]}

Otherwise

RTEIAMT q, p	=	(-1) * RTSPP p * {ΣrRTMG q, p, r + (SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)}

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for the Real-Time Energy Imbalance at Settlement Point p, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| RTMG q, p, r | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource—The Real-Time energy produced by the Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. |
| SSSK q, p | MW | Self-Schedule with Sink at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with sink at Settlement Point p, for the 15-minute Settlement Interval. |
| DAEP q, p | MW | Day-Ahead Energy Purchase per QSE per Settlement Point—The QSE q’s Energy Bids at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQEP q, p | MW | Real-Time QSE-to-QSE Energy Purchase per QSE per Settlement PointThe amount of MW bought by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSR q, p | MW | Self-Schedule with Source at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with source at Settlement Point p, for the 15-minute Settlement Interval. |
| DAES q, p | MW | Day-Ahead Energy Sale per QSE per Settlement Point—The QSE q’s Energy Offers at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQES q, p | MW | Real-Time QSE-to-QSE Energy Sale per QSE per Settlement PointThe amount of MW sold by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| NMSAMTTOT | $ | Net Metering Settlement Payment—The total payment to the entire facility with a net metering arrangement. |
| NMALLOCF r | none | Net Metering Allocation Factor for each resource—The net metering allocation factor for resource r that is part of a net metering arrangement. |
| q |  | A QSE. |
| p |  | A Resource Node Settlement Point. |
| r |  | A Generation Resource. |

- The total payments to a facility with a net metering arrangement, for each 15-minute Settlement Interval, shall be calculated as follows:

NMRTETOTa = ΣbΣme MEB me, b

If NMRTETOTa < 0

NMSAMTTOTa =  -1*RTSPP * NMRTETOT

Where the price RTSPP is the Real Time Settlement Point Price for the Load Zone.

Otherwise

NMSAMTTOTa	=	ΣbΣme (RTEPSPR me, b * MEB me, b)

Where the price for settlement meter is determined as follows:

For MEB me,b < 0

RTEPSPRme, b	=	Σy (RTLMP b, y * TLMP y) / Σy TLMP y

Otherwise RTEPSPR me, b =  RTSPP as determined in accordance with Section 6.6.1.1, Real Time Settlement Point Price for a Resource Node.

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| NMRTETOTa | MWh | Net Meter Real-Time Energy Total— The net sum for all settlement meters me include in net metering arrangement a. A positive value indicates an injection of power to the ERCOT system and a negative value indicates an withdrawal of power from the ERCOT system. |
| NMSAMTTOT | $ | Net Metering Settlement Payment—The total payment to the entire facility with a net metering arrangement. |
| RTEPSPR me, b | $/MWh | Real-Time Price for the Energy Metered for each EPS meter at busThe Real-Time Price for the settlement meter me at Electrical Bus b, for the 15-minute Settlement Interval. |
| MEB me, b | MWh | Metered Energy at busThe metered energy by the settlement meter me at Electrical Bus b for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy consumed. |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per intervalThe Real-Time LMP at Electrical Bus b, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the SCED interval y. |
| me | none | A settlement meter for the facility with net metering. The summation is over all the settlement meters for the facility. |
| R | none | A Generation Resource that is located at the facility with net metering. The summation is over all the Generation Resources at the facility. |
| Y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| B | none | An Electrical Bus. |
| P | none | A Resource Node Settlement Point. |
| A | none | A Net Metering Arrangement. |
| Q | none | A QSE. |

(4)	The Net Metering Allocation Factor for each resource within a net metering arrangement for the 15-minute Settlement Interval is calculated as follows:

NMALLOCFr,a =  NPTELEMr,a / ∑r  NPTELEMr,a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| NMALLOCF r | None | Net Metering Allocation Factor for per resource—The net metering allocation factor for resource r that is part of a net metering arrangement. |
| NPTELEM r | MWh | Net Real Power Provided via Telemetry—The net real power provided via telemetry per resource within the net metering arrangement, integrated for the 15-minute Settlement Interval. |
| A | None | A Net Metering Arrangement |
| R | None | A Generation Resource that is located at the facility with net metering. The summation is over all the Generation Resources at the facility. |

(5)	The total net payments and charges to each QSE for Energy Imbalance Service at all Resource Node Settlement Points for the 15-minute Settlement Interval is calculated as follows:

RTEIAMTQSETOT q	=	ΣpRTEIAMT q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTEIAMTQSETOT q | $ | Real-Time Energy Imbalance Amount QSE Total per QSEThe total net payments and charges to QSE q for Real-Time Energy Imbalance at all Resource Node Settlement Points for the 15-minute Settlement Interval. |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for the Real-Time Energy Imbalance at Settlement Point p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |

10.3.2.3	Generation Netting for EPS Meters

(1)	At Generation Resource Facilities, generation and associated Load must be metered at their points of interconnection to the ERCOT Transmission Grid.  IDR meters must be used to determine generator output or Load usage.  In the intervals where the generation output exceeds the Load, the net must be settled as generation.  In the intervals where the Load exceeds the generation output, the net must be settled as Load and carry any applicable Load shared charges.

(2)	For settlement purposes, generation netting is not allowed except under one of the following conditions:

(a)	Single point of interconnection with delivered and received metering data channels;

(b)	Multiple points of interconnection where the Loads and generator output are electrically connected to a common switchyard, as defined below.  In addition, there must be sufficient generator capacity to serve all plant Loads for netting to occur;

(c)	A Qualifying Facility (QF) with point(s) of interconnection where the QF is selling to the QF’s thermal host(s) may net the Load meters of the thermal host with its generation meters when the Load and generation are electrically connected to a common switchyard.  In instances in which Load is served by new on-site generation through a common switchyard, the TSP or DSP may install monitoring equipment necessary for measuring Load to determine stranded cost charges, if any are applicable, as determined under Public Utility Regulatory Act (PURA) and applicable PUCT rules.  If the PUCT requires other Load served by onsite generators to pay the system benefit fund charges, then, in instances in which Load is served by generation through a common switchyard, the TSP or DSP may install metering equipment solely for purposes of the TSP or DSP’s calculation of system benefit fund charges, as provided by PURA, if any is applicable.  For purposes of this Section, new on-site generation has the meaning as contained in Public Utility Regulatory Act, Tex. Util. Code Ann.  § 39.252 and § 39.262(k) (Vernon 1998 & Supp. 2005) (PURA); or

(d)	For Generation Resources and/or Load with flow-through on a private, contiguous transmission system (not included in a TSP or DSP rate base) and in a configuration existing as of October 1, 2000, the meters at the interconnections with the ERCOT Transmission Grid may be netted for the purpose of determining Generation Resources or Load.  For Settlement purposes, when the net is a Load, the metered interconnection points must be assigned to the same Load Zone and UFE zone.

(e)	ERCOT shall maintain descriptions of the metering facilities of all Common Switchyards that contain multiple points of interconnection of Loads (ESIDs) and generation meters (EPS). The description is limited to identifying the Entities within a Common Switchyard and a simplified diagram showing the metering configuration of all SCADA and settlement metering points.

(f)	All Load(s) included in the netting arrangement for an EPS Metering Facility shall only be electrically connected to the ERCOT grid through the EPS Metering point(s) for such Facility.  Such Loads shall not be electrically connected to the ERCOT Grid through electrical connections that are not metered by the EPS metering point(s) for the Facility.

(3)	For purposes of this Section, a common switchyard is defined as an electric substation facility where the point of interconnection for Load and Generation Resources are located at the same facility but where the interconnection points are physically not greater than 400 yards apart.  The physical connections of the Load to its point of interconnection and the Generation Resource to its point of interconnection cannot be Facilities that have be en placed in a TSP’s or DSP’s rate base.