| NPRR Number |  | NPRR Title | Section 7: Settlements Clean-up NPRR |
| --- | --- | --- | --- |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  | Section 7 Congestion Revenue Rights |  |
| Revision Description |  |  |  |
| Reason for Revision |  |  |  |
| Reason for Revision (from Transition Plan Task Force (TPTF) Charter Scope) |  | (1) Revisions resulting from Commission orders; (2) Clarifications of Protocol language that do not change the intent or technical specifications of the Protocols; (3) Correction of technical errors or processes that are found to not be technically feasible; (4) Revisions to the Protocols necessary to implement the results of the value engineering analysis or to otherwise avoid severe cost impacts; or (5) Other (describe): |  |
| TPTF Review (Yes or No, and summary of conclusion) |  |  |  |
| Credit Implications (Yes or No, and summary of impact) |  |  |  |
| Timeline |  |  |  |
| Date Posted |  |  |  |
| Please access the ERCOT website for current timeline information. |  |  |  |

| Sponsor |  |
| --- | --- |
| Name |  |
| E-mail Address |  |
| Company |  |
| Company Address |  |
| Phone Number |  |
| Fax Number |  |

ERCOT/Market Segment Impacts and Benefits

Instructions:  To allow for comprehensive NPRR consideration, please fill out each block below completely, even if your response is “none,” “not known,” or “not applicable.”  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the NPRR.

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Maintain consistency between business requirements and protocols. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other | 1 |  |  |
| Comments | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

Proposed Nodal Protocol Language Revision

7.5.5.3	Auction Process

(1)	The auction must be a single-round, simultaneous auction for selling the CRRs available for all auction products, with the following steps:

(a)	ERCOT shall enter into the CRR Auction engine model a credit constraint for each Counter-Party.  A Counter-Party’s CRR Auction credit limit is equal to the lesser of the credit limit as determined in Section 16.11.4.6.1, Credit Requirements for CRR Auction Participation, or, if provided, the Counter-Party’s self-imposed CRR Auction credit limit.  The credit constraint for each Counter-Party ensures that the following sum is less than or equal to its CRR Auction credit limit:

(i)	all awarded CRR Auction Bids multiplied by the absolute value of the corresponding bid price; plus

(ii)	all awarded CRR Auction Offers with negative offer prices multiplied by the absolute value of their corresponding offer price; plus

(iii)	the additional credit requirement for all awarded PTP Obligations.

(b)	ERCOT shall award CRRs in quantities truncated to the nearest tenth MW (0.1 MW).

(c)	The CRR Clearing Price is equal to the corresponding Shadow Price for that CRR product.

(d)	When a CRR Account Holder is awarded CRRs as a result of a CRR Auction, the CRRs do not become the property of the winning CRR Account Holder, and the CRRs may not be placed in their CRR accounts, until the CRR Invoices have been paid in full.

(e)	When a CRR Account Holder sells PTP Obligations as a result of an auction at a negative price, the CRR Account Holder is not relieved of the PTP Obligations until the CRR Invoices have been paid in full.

(2)	ERCOT shall use a linear programming auction engine model for each CRR Auction that evaluates all CRR Auction Bids and CRR Auction Offers submitted, and selects a combination of CRR Auction Bids and CRR Auction Offers that:

(a)	Makes the solution simultaneously feasible within the limits of the ERCOT network capability over the auction term; and

(b)	Maximizes the objective function, which is equal to the total economic value (as expressed in the CRR Auction Bids) of the awarded CRR Auction Bids, less the total economic cost (as expressed in CRR Auction Offers) of the awarded CRR Auction Offers, while observing all applicable constraints.

(3)	The CRR Network Model must, to the extent practicable, reflect the continuous and post-contingency system operating limits and operational procedures (i.e., Special Protection Systems and Remedial Action Plans) in the Network Operations Model used by ERCOT during Real-Time Operations, as discussed below in Section 7.5.5.4, Simultaneous Feasibility Test.

(4)	Once a CRR Auction is complete, ERCOT shall archive and keep the CRR Auction system and all models used to finalize the CRR Auction results under ERCOT’s data retention policy as that policy applies to data that may be needed to resolve requests for billing adjustments under applicable billing adjustment procedures.

7.5.7	Method for Distributing CRR Auction Revenues

(1)	ERCOT shall determine, for each month, the CRR Monthly Revenues (CMR). The CMR is the sum of:

(a)	Monthly CRR revenue for that calendar month; and

(b)	PCRR revenues.

(2)	For the first three years after the TNT Market Implementation Date, ERCOT shall credit the net CRR Auction revenue (including PCRR revenue) produced from CRRs cleared in each CRR Auction that source from a Settlement Point located within a 2003 ERCOT Congestion Management Zone (CMZ) and sink at a Settlement Point located within the same 2003 ERCOT CMZ to QSEs in the 2003 ERCOT CMZ on a zonal Load Ratio Share basis.  All other net CRR Auction revenues must be allocated to QSEs on an ERCOT-wide Load Ratio Share basis.  For these allocation purposes, any NOIE Load Zone is considered to be located entirely within the 2003 ERCOT CMZ that represented the largest Load for that NOIE or group of NOIEs in 2003.  Before the end of the three-year period described above, the ERCOT Board shall consider whether to extend the policy or ratify some other alternative.

(3)	For Initial distribution of CRR Monthly Revenues, revenues shall be paid to each QSE based on that QSE’s Load Ratio Share in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the calendar month.

(4)	ERCOT shall true up the distribution of CRR Monthly Revenues based on that QSE’s Load Ratio Share in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the calendar month.

(5)	The net CRR Auction Revenue produced from CRRs cleared and paid for in each CRR Auction that source from a Settlement Point within a 2003 ERCOT CMZ and sink at a Settlement Point located within the same 2003 ERCOT CMZ shall be distributed on a zonal Load Ratio Share basis.  The portion of the net monthly CRR Auction Revenue to be distributed to each QSE with load in that zone for a given month is calculated as follows:

LACMRZAMT z, q=	(-1) * Σa(CRRZREV z, a + PCRRZREV z, a) * MLRSZ z, q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LACMRZAMT z, q | $ | Load-Allocated CRR Monthly Revenue Zonal Amount per zone per QSE—The payment to QSE q of the revenues resulted from the CRRs that source and sink in CMZ z, for the month. |
| CRRZREV z, a | $ | CRR Zonal Revenue per zone per CRR Auction—The revenue resulted from the CRRs that source and sink in CMZ z, cleared through CRR Auction Offers and CRR Auction Bids in CRR Auction a, for the month. |
| PCRRZREV z, a | $ | PCRR Zonal Revenue per zone per CRR Auction—The revenue resulted from the PCRRs that source and sink in CMZ z, pertaining to CRR Auction a, for the month. |
| MLRSZ q, z | none | Monthly Load Ratio Share Zonal per QSE per zone—The LRS of QSE q for its Load in CMZ z, for the peak-Load 15-minute Settlement Interval in the calendar month. |
| q | none | A QSE. |
| z | none | A 2003 ERCOT CMZ. |
| a | none | A CRR Auction. |

(6)	The net CRR Auction Revenue produced from CRRs cleared and paid for in each CRR Auction that do not source from a Settlement Point within a 2003 ERCOT CMZ and sink at a Settlement Point located within the same 2003 ERCOT CMZ shall be distributed on an ERCOT-wide LRS basis.  The portion of the net monthly CRR Auction Revenue Amount (from CRRs with paths that cross the 2003 ERCOT CMZ boundaries) to be distributed for a given month is calculated as follows:

LACMRNZAMT q	=	(-1) * Σa(CRRNZREV a + PCRRNZREV a) * MLRS q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LACMRNZAMT q | $ | Load-Allocated CRR Monthly Revenue Non-Zonal Amount per QSE—The payment to QSE q of the revenues resulted from the CRRs that source and sink in different CMZs, for the month. |
| CRRNZREV a | $ | CRR Zonal Revenue per CRR Auction—The revenue resulted from the CRRs that source and sink in different CMZs, cleared through CRR Auction Offers and CRR Auction Bids in CRR Auction a, for the month. |
| PCRRNZREV a | $ | PCRR Zonal Revenue per CRR Auction—The revenue resulted from the PCRRs that source and sink in different CMZs, pertaining to CRR Auction a, for the month. |
| MLRS q | none | Monthly Load Ratio Share per QSE—The LRS calculated for QSE q for the peak-Load 15-minute Settlement Interval in the calendar month. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |
| a | none | A CRR Auction. |

7.9	CRR Settlements

7.9.1	Day-Ahead CRR Payments and Charges

7.9.1.1	Payments and Charges for PTP Obligations Settled in DAM

(1)	Except as specified otherwise in paragraph (2) below, ERCOT shall pay or charge the owner of each PTP Obligation based on the difference in the Day-Ahead Settlement Point Price between the sink Settlement Point and the source Settlement Point.

(2)	For PTP Obligations that have a positive value and source or sink at a Resource Node, the PTP Obligation payment may be reduced due to directional network elements that are oversold in previous CRR auctions.

(3)	The payment or charge to each CRR Owner for a given Operating Hour of PTP Obligations with each pair of source and sink Settlement Points settled in the DAM is calculated as follows:

If the PTP Obligation has a non-positive value or both source and sink at a Load Zone or Hub, i.e., (DAOBLPR (j, k)  0) OR (j is a Load Zone or Hub AND k is also a Load Zone or Hub), then

DAOBLAMT o, (j, k)	=	(-1) * DAOBLTP o, (j, k)

If the PTP Obligation has a positive value and either source or sink is a Resource Node, then

DAOBLAMT o, (j, k)	=	(-1) * Max ((DAOBLTPo, (j, k) – DAOBLDAo, (j, k)), Min (DAOBLTP o, (j, k), DAOBLHV o, (j, k)))

Where:

The target payment:

DAOBLTP o, (j, k)	=	DAOBLPR (j, k) * DAOBL o, (j, k)

The price based on the difference of the Settlement Point Prices:

DAOBLPR (j, k)	=	DASPP k - DASPP j

The derated amount:

DAOBLDA o, (j, k)	=	OBLDRPR (j, k) * DAOBL o, (j, k)

The price used to calculate the derated amount:

OBLDRPR (j, k)	=	Σc(Max (0, DAWASF j, c – DAWASF k, c) * DASP c * DRFc)

The hedge value:

DAOBLHV o, (j, k)	=	DAOBLHVPR (j, k) * DAOBL o, (j, k)

The price of the hedge value:

If the source, j, is a Load Zone or Hub and the sink, k, is a Resource Node,

DAOBLHVPR (j, k)	=	Max (0, MAXRESPR k – DASPP j)

If the source, j, is a Resource Node and the sink, k, is a Load Zone or Hub,

DAOBLHVPR (j, k)	=	Max (0, DASPP k – MINRESPR j)

If the source, j, is a Resource Node and the sink, k, is also a Resource Node,

DAOBLHVPR (j, k)	=	Max (0, MAXRESPR k – MINRESPR j)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOBLAMT o, (j, k) | $ | Day-Ahead Obligation Amount per CRR Owner per source and sink pair—The payment or charge to CRR Owner o for the PTP Obligations with the source j and the sink k settled in the DAM, for the hour. |
| DAOBLTP o, (j, k) | $ | Day-Ahead Obligation Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Obligations with the source j and the sink k settled in the DAM, for the hour. |
| DAOBLHV o, (j, k) | $ | Day-Ahead Obligation Hedge Value per CRR Owner per source and sink pair—The hedge value of CRR Owner o’s PTP Obligations with the source j and the sink k settled in the DAM, for the hour. |
| DAOBLDA o, (j, k) | $ | Day-Ahead Obligation Derated Amount per CRR Owner per source and sink pair—The derated amount of CRR Owner o’s PTP Obligations with the source j and the sink k settled in the DAM, for the hour. |
| DAOBLPR (j, k) | $/MW per hour | Day-Ahead Obligation Price per source and sink pair—The DAM price of a PTP Obligation with the source j and the sink k, for the hour. |
| DASPP j | $/MWh | Day-Ahead Settlement Point Price at source—The DAM Settlement Point Price at the source Settlement Point j, for the hour. |
| DASPP k | $/MWh | Day-Ahead Settlement Point Price at sink—The DAM Settlement Point Price at the sink Settlement Point k, for the hour. |
| OBLDRPR (j, k) | $/MW per hour | Obligation Deration Price per source and sink pair—The deration price of a PTP Obligation with the source j and the sink k, for the hour. |
| DASP c | $/MW per hour | Day-Ahead Shadow Price per constraint—The DAM Shadow Price of the constraint c for the hour. |
| DRF c | none | Deration Factor per constraint—The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASF j, c | none | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and the directional network element for constraint c, in the hour. |
| DAWASF k, c | None | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and the directional network element for constraint c, in the hour. |
| DAOBLHVPR (j, k) | $/MWh | Day-Ahead Obligation Hedge Value Price per source and sink pair—The Day-Ahead hedge price of a PTP Obligation with the source j and the sink k, for the hour. |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for the Resources located at the source Settlement Point j. |
| MAXRESPR k | $/MWh | Max Resource Price for sink—The highest Maximum Resource Price for the Resources located at the sink Settlement Point k. |
| DAOBL o, (j, k) | MW | Day-Ahead Obligation per CRR Owner per source and sink pair—The number of CRR Owner o’s PTP Obligations with the source j and the sink k settled in the DAM for the hour. |
| O | none | A CRR Owner. |
| J | none | A source Settlement Point. |
| K | none | A sink Settlement Point. |
| C | none | A constraint associated with a directional network element for the hour. |

(4)	The net total payment or charge to each CRR Owner for the Operating Hour of all its PTP Obligations settled in the DAM is calculated as follows:

DAOBLAMTOTOT o	=	DAOBLCROTOT o + DAOBLCHOTOT o

Where:

DAOBLCROTOT o	=	ΣjΣkMin (0, DAOBLAMT o, (j, k))

DAOBLCHOTOT o	=	ΣjΣkMax (0, DAOBLAMT o, (j, k))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOBLAMTOTOT o | $ | Day-Ahead Obligation Amount Owner Total per CRR Owner—The net total payment or charge to CRR Owner o for all its PTP Obligations settled in the DAM, for the hour. |
| DAOBLCROTOT o | $ | Day-Ahead Obligation Credit Owner Total per CRR Owner—The total payment to CRR Owner o for its PTP Obligations settled in the DAM, for the hour. |
| DAOBLCHOTOT o | $ | Day-Ahead Obligation Charge Owner Total per CRR Owner—The total charge to CRR Owner o for its PTP Obligations settled in the DAM, for the hour. |
| DAOBLAMT o, (j, k) | $ | Day-Ahead Obligation Amount per CRR Owner per pair of source and sink—The payment or charge to CRR Owner o for its PTP Obligations with the source j and the sink k settled in the DAM, for the hour. |
| O | none | A CRR Owner. |
| J | none | A source Settlement Point. |
| K | none | A sink Settlement Point. |

7.9.1.2	Payments for PTP Options Settled in DAM

(1)	Except as specified otherwise in paragraph (2) below, ERCOT shall pay the owner of a PTP Option the difference in the Day-Ahead Settlement Point Price between the sink Settlement Point and the source Settlement Point, if positive.

(2)	For PTP Options that source or sink at a Resource Node, the PTP Option payment may be reduced due to transmission elements that are oversold in previous CRR auctions.

(3)	The payment to each CRR Owner for a given Operating Hour of PTP Options with each pair of source and sink Settlement Points settled in the DAM is calculated as follows:

If the source, j, is a Load Zone or Hub and sink, k, is also a Load Zone or Hub, then

DAOPTAMT o, (j, k)	=	(-1) * DAOPTTP o, (j, k)

If either the source, j, or sink, k, is a Resource Node, then

DAOPTAMT o, (j, k)	=	(-1) * Max ((DAOPTTP o, (j, k) – DAOPTDA o, (j, k)), Min (DAOPTTP o, (j, k), DAOPTHV o, (j, k)))

Where:

The target payment:

DAOPTTP o, (j, k)	=	DAOPTPR (j, k) * DAOPT o, (j, k)

The price based on the difference of the Settlement Point Prices:

DAOPTPR o, (j, k)	=	Max (0, DASPP k – DASPP j)

The derated amount:

DAOPTDA o, (j, k)	=	OPTDRPR (j, k) * DAOPT o, (j, k)

The price used to calculate the derated amount:

OPTDRPR (j, k)	=	Σc(Max (0, DAWASF j, c – DAWASF k, c) * DASP c * DRFc)

The hedge value:

DAOPTHV o, (j, k)	=	DAOPTHVPR (j, k) * DAOPT o, (j, k)

The price of the hedge value:

If the source, j, is a Load Zone or Hub and the sink, k, is a Resource Node,

DAOPTHVPR (j, k)	=	Max (0, MAXRESPR k – DASPP j)

If the source, j, is a Resource Node and the sink, k, is a Load Zone or Hub,

DAOPTHVPR (j, k)	=	Max (0, DASPP k – MINRESPR j)

If the source, j, is a Resource Node and the sink, k, is also a Resource Node,

DAOPTHVPR (j, k)	=	Max (0, MAXRESPR k – MINRESPR j)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTAMT o, (j, k) | $ | Day-Ahead Option Amount per CRR Owner per source and sink pair The payment to CRR Owner o for the PTP Options with the source j and the sink k settled in the DAM, for the hour. |
| DAOPTTP o, (j, k) | $ | Day-Ahead Option Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Options with the source j and the sink k settled in the DAM, for the hour. |
| DAOPTHV o, (j, k) | $ | Day-Ahead Option Hedge Value per CRR Owner per source and sink pair—The hedge value of CRR Owner o’s PTP Options with the source j and the sink k settled in the DAM, for the hour. |
| DAOPTDA o, (j, k) | $ | Day-Ahead Option Derated Amount per CRR Owner per source and sink pair—The derated amount of CRR Owner o’s PTP Options with the source j and the sink k settled in the DAM, for the hour. |
| DAOPTPR (j, k) | $/MW per hour | Day-Ahead Option Price per source and sink pair The DAM price of a PTP Option with the source j and the sink k, for the hour. |
| DASPP j | $/MWh | Day-Ahead Settlement Point Price at source The DAM SPP at the source Settlement Point j, for the hour. |
| DASPP k | $/MWh | Day-Ahead Settlement Point Price at sink The DAM SPP at the sink Settlement Point k, for the hour. |
| OPTDRPR (j, k) | $/MW per hour | Option Deration Price per source and sink pair—The deration price of a PTP Option with the source j and the sink k, for the hour. |
| DASP c | $/MW per hour | Day-Ahead Shadow Price per constraint—The DAM Shadow Price of the constraint c for the hour. |
| DRF c | none | Deration Factor per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASF j, c | none | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and the directional network element for constraint c, in the hour. |
| DAWASF k, c | none | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and the directional network element for constraint c, in the hour. |
| DAOPTHVPR (j, k) | $/MWh | Day-Ahead Option Hedge Value Price per source and sink pair—The Day-Ahead hedge price of a PTP Option with the source j and the sink k, for the hour. |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for Resources located at the source Settlement Point j. |
| MAXRESPR k | $/MWh | Max Resource Price for sink—The highest Maximum Resource Price for Resources located at the sink Settlement Point k. |
| DAOPT o, (j, k) | MW | Day-Ahead Option per CRR Owner per source and sink pairThe number of CRR Owner o’s PTP Options with the source j and the sink k settled in the DAM for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| c | none | A constraint associated with a directional network element for the hour. |

(4)	The total payment to each CRR Owner for the Operating Hour of all its PTP Options settled in the DAM is calculated as follows:

DAOPTAMTOTOT o	=	ΣjΣkDAOPTAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTAMTOTOT o | $ | Day-Ahead Option Amount Owner Total per CRR Owner—The total payment to CRR Owner o for all its PTP Options settled in the DAM, for the hour. |
| DAOPTAMT o, (j, k) | $ | Day-Ahead Option Amount per CRR Owner per pair of source and sink—The payment to CRR Owner o for its PTP Options with the source j and the sink k settled in the DAM, for the hour. |
| O | none | A CRR Owner. |
| J | none | A source Settlement Point. |
| K | none | A sink Settlement Point. |

(5)	For informational purposes, the following calculation of PTP Option value shall be posted on the MIS Public Area:

DAOPTPRINFO (j, k)	=	Σc(DASP c * Max (0, (DAWASF j, c – DAWASF k, c)))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTPRINFO (j, k) | $/MW per hour | Day-Ahead Option Informational Price per pair of source and sink—The Informational DAM price of the PTP Options with the source Settlement Point j and the sink Settlement Point k, for the hour. |
| DAWASF j, c |  | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and for the constrained directional network element for constraint c, in the hour. |
| DAWASF k, c | none | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and for the constrained directional network element for constraint c, in the hour. |
| DASP c | $/MW per hour | Day-Ahead Shadow Price per constraint—The DAM Shadow Price for the constraint c for the hour. |
| C | none | A constraint associated with a directional network element for the hour. |

7.9.1.3	Minimum and Maximum Resource Prices

(1)	The following prices specified in paragraphs (2) and (3) below are used in CRR hedge value calculation for CRRs settled in the DAM and PTP Options settled in Real-Time.

(2)	Minimum Resource Prices of source Settlement Points are:

MINRESPR j	 =	 Min ( MINRESRPR j, r ) r

Where:

Minimum Resource Prices for Resources located at source Settlement Points (MINRESRPR j, r) are:

(a)	Nuclear = -$20.00/MWh;

(b)	Hydro = -$20.00/MWh;

(c)	Coal and Lignite = $0.00/MWh;

(d)	Combined Cycle greater than 90 MW = FIP * 5 MMBtu/MWh;

(e)	Combined Cycle less than or equal to 90 MW = FIP * 6 MMBtu/MWh;

(f)	Gas -Steam Supercritical Boiler = FIP * 6.5 MMBtu/MWh;

(g)	Gas Steam Reheat Boiler = FIP * 7.5 MMBtu/MWh;

(h)	Gas Steam Non-reheat or boiler without air-preheater = FIP * 10.5 MMBtu/MWh;

(i)	Simple Cycle greater than 90 MW = FIP * 10 MMBtu/MWh;

(j)	Simple Cycle less than or equal to 90 MW = FIP * 11 MMBtu/MWh;

(k)	Diesel = FIP * 12 MMBtu/MWh;

(l)	Wind = -$35/MWh;

(m)	RMR Resource = RMR contract price Energy Offer Curve at LSL; and

(n)	Other Renewable = -$10.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for the Resources located at the source Settlement Point j. |
| MINRESRPR j | $/MWh | Minimum Resource Price for Resource—The Minimum Resource Price for the Resources located at the source Settlement Point j. |
| r | none | A Generation Resource located at the source Settlement Point j. |
| j | none | A source Settlement Point. |

(3)	Maximum Resource Prices of sink Settlement Points are:

MAXRESPR k	 =	 Max ( MAXRESRPR k, r ) r

Where:

Maximum Resource Prices for Resources located at sink Settlement Points (MAXRESRPR k, r )are:

(a)	Nuclear = $15.00/MWh;

(b)	Hydro = $10.00/MWh;

(c)	Coal and Lignite = $18.00/MWh;

(d)	Combined Cycle greater than 90 MW = FIP * 9 MMBtu/MWh;

(e)	Combined Cycle less than or equal to 90 MW = FIP * 10 MMBtu/MWh;

(f)	Gas -Steam Supercritical Boiler = FIP * 10.5 MMBtu/MWh;

(g)	Gas Steam Reheat Boiler = FIP * 11.5 MMBtu/MWh;

(h)	Gas Steam Non-reheat or boiler without air-preheater = FIP * 14.5 MMBtu/MWh;

(i)	Simple Cycle greater than 90 MW = FIP * 14 MMBtu/MWh;

(j)	Simple Cycle less than or equal to 90 MW = FIP * 15 MMBtu/MWh;

(k)	Diesel = FIP * 16 MMBtu/MWh;

(l)	Wind = $0/MWh;

(m)	RMR Resource = RMR contract price Energy Offer Curve at HSL; and

(n)	Other Renewable = $0.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| MAXRESPR k | $/MWh | Maximum Resource Price for source—The highest Maximum Resource Price for the Resources located at the sink Settlement Point k. |
| MAXRESRPR k | $/MWh | Maximum Resource Price for Resource—The Maximum Resource Price for the Resources located at the sink Settlement Point k. |
| r | none | A Generation Resource located at the sink Settlement Point k. |
| k | none | A sink Settlement Point. |

7.9.1.4	Payments for FGRs Settled in DAM

(1)	If an FGR is competitive, i.e., all directional network elements associated with the FGR are Competitive Constraints, ERCOT shall pay the owner of the FGR an amount equal to the sum of the Shadow Price of the hour for each directional network element associated with the FGR for each contingency (including the null contingency or base case) normalized to the impact of the principal network element of the FGR (the normal rating of which is used to determine the total MW amount for the flowgate).  The payment to each CRR Owner for its FGRs determined by the principle network element of each flowgate for a given hour is calculated as follows:

DAFGRAMT o, f	=	(-1) * DAFGRTP o, f

Where:

DAFGRTP o, f	=	DAFGRPR f * DAFGR o, f

DAFGRPR f	=	Σe∈f(INF f, e * ΣcDASP e, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAFGRAMT o, f | $ | Day-Ahead FGR Amount per CRR Owner per flowgateThe payment to CRR Owner o of the flowgate f settled in DAM, for the hour. |
| DAFGRTP o, f | $ | Day-Ahead FGR Target Payment per CRR Owner per flowgate—The target payment for CRR Owner o’s flowgate f settled in the DAM, for the hour. |
| DAFGRPR f | $/MW per hour | Day-Ahead FGR Price per flowgateThe DAM price of the flowgate f for the hour. |
| DASP e, c | $/MW per hour | Day-Ahead Shadow Price per element per constraintThe DAM Shadow Price on the directional network element e, for constraint c, for the hour. |
| INF f, e | none | Impact Normalization Factor per element per flowgateThe parameter that reflects the normalized impact on the directional network element e relative to the impact on the principal network element of flowgate f. |
| DAFGR o, f | MW | Day-Ahead FGR per CRR Owner per flowgateThe CRR Owner o’s total number of FGRs determined by the principle element of flowgate f settled in the DAM for the hour. |
| O | none | A CRR Owner. |
| F | none | A flowgate. |
| E | none | A directional network element. |
| C | none | A constraint. |
| ef | none | The directional network element e belongs to the flowgate f. |

(2)	If an FGR is non-competitive, i.e., one or more directional network elements associated with the FGR are Non-Competitive Constraints, the FGR payment may be reduced due to transmission elements that are oversold in previous CRR auctions.  The payment for MCFRI, when it is not designated as a Competitive Constraint, is calculated in paragraph (3); the payment for any other FGR, when it is non-competitive, will be specified upon introduction of the FGR.

(3)	The payment to each CRR Owner for its MCFRI for a given hour, when MCFRI is not designated as a Competitive Constraint, is calculated as follows:

DAFGRAMT o, MCFRI	=	(-1) * Max ((DAFGRTP o, MCFRI –         DAFGRDA o, MCFRI), Min (DAFGRTP o, MCFRI, DAFGRHV o, MCFRI))

Where:

The target payment:

DAFGRTP o, MCFRI	=	DAFGRPR MCFRI * DAFGR o, MCFRI

DAFGRPR MCFRI	=Σe∈MCFRI(INF MCFRI, e * ΣcDASP e, c)

The derated amount:

DAFGRDA o, MCFRI	=	FGRDRPR MCFRI * DAFGR o, MCFRI

FGRDRPR MCFRI	=	Σe∈MCFRI(INF MCFRI, e * Σc(DASP e, c * DRF e, c))

The hedge value:

DAFGRHV o ,MCFRI	=	DAFGRHVPR MCFRI * DAFGR o, MCFRI

DAFGRHVPR MCFRI	=	Max (0, (DAWALBEP – MINRESPR j, MCFRI - Σe∉MCFRIΣc (DASP e, c * (DASFMCWGRS e, c - DAWASFLB e, c))) / (DASFMCWGRS e=MCFRI principle element, c(Base Case - DAWASFLB e=MCFRI principle element, cBase Case))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAFGRAMT o, f | $ | Day-Ahead FGR Amount per CRR Owner per flowgateThe payment to CRR Owner o of the FGRs associated with flowgate f settled in DAM, for the hour. |
| DAFGRTP o, f | $ | Day-Ahead FGR Target Payment per CRR Owner per flowgate—The target payment for CRR Owner o’s flowgate f settled in the DAM, for the hour. |
| DAFGRHV o, f | $ | Day-Ahead FGR Hedge Value per CRR Owner per flowgate—The hedge value of CRR Owner o’s flowgate f settled in the DAM, for the hour. |
| DAFGRDA o, f | $ | Day-Ahead FGR Derated Amount per CRR Owner per flowgate—The derated amount of CRR Owner o’s flowgate f settled in the DAM, for the hour. |
| DAFGRPR f | $/MW per hour | Day-Ahead FGR Price per flowgateThe DAM price of the flowgate f for the hour. |
| FGRDRPR f | $/MW per hour | FGR Deration Price per flowgate—The deration price of the flowgate f for the hour. |
| INF f, e | none | Impact Normalization Factor per element per flowgateThe parameter that reflects the normalized impact on the directional network element e relative to the impact on the principal network element of flowgate f. |
| DASP e, c | $/MW per hour | Day-Ahead Shadow Price per element per constraintThe DAM Shadow Price on the directional network element e, for constraint c, for the hour. |
| DAFGR o, f | MW | Day-Ahead FGR per CRR Owner per flowgateThe CRR Owner o’s total number of FGRs determined by the principle element of the flowgate f settled in the DAM for the hour. |
| DRF e, c | none | Deration Factor per element per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASFLB e, c | none | Day-Ahead Weighted Average Shift Factor of Load Buses per element per constraint—The Day-Ahead weighted average Shift Factor for all load buses on the directional network element e, for constraint c, in the hour. |
| DASFMCWGRS e, c | none | Day-Ahead Shift Factor of McCamey WGRs per element per constraint—The Day-Ahead McCamey Area WGR maximum rated output weighted Shift Factor on the directional network element e, for constraint c, in the hour. |
| DAFGRHVPR f | $/MWh | Day-Ahead FGR Hedge Value Price per flowgate—The Day-Ahead hedge price of the flowgate f, for the hour. |
| MINRESPR j, MCFRI | $/MWh | Minimum Resource Price—The lowest Minimum Resource Price for the Resources located at the source j of MCFRI. |
| DAWALBEP | $/MWh | Day-Ahead Weighted Average Load Bus Energy Price—The weighted average DAM energy price of all load buses for the hour. |
| O | none | A CRR Owner. |
| F | none | A flowgate; in this application f = MCFRI. |
| E | none | A directional network element, including principal element. |
| C | none | A constraint. |
| j | none | A source Settlement Point. |
| eMCFRI | none | The directional network element e belongs to MCFRI. |
| eMCFRI | none | The directional network element e doesn’t belong to MCFRI. |
| cBase Case | none | The constraint c is under the Base Case. |

(4)	The total of the payments to each CRR Owner for the Operating Hour of all its FGRs settled in the DAM is calculated as follows:

DAFGRAMTOTOT o	=	ΣfDAFGRAMT o, f

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAFGRAMTOTOT o | $ | Day-Ahead FGR Amount Owner Total per CRR Owner—The total payment to CRR Owner o of all its FGRs settled in the DAM, for the hour. |
| DAFGRAMT o, f | $ | Day-Ahead FGR Amount per CRR Owner per flowgate—The payment to CRR Owner o of the FGRs associated with flowgate f settled in DAM, for the hour. |
| O | none | A CRR Owner. |
| F | none | A flowgate. |

7.9.1.5	Payments and Charges for PTP Obligations with Refund Settled in DAM

(1)	Except as specified otherwise in paragraph (2) below, ERCOT shall pay the owner of a PTP Obligation with Refund the difference in the Day-Ahead Settlement Point Prices between the sink Settlement Point and the source Settlement Point, subject to a charge for refund, when the price difference is positive, as described in the item (e) (i) of Section 7.4.2, PCRR Allocation Terms and Conditions.

(2)	The payment of PTP Obligations with Refund may be further reduced due to transmission elements that are oversold in previous CRR auctions.

(3)	The payment or charge to each CRR Owner for a given Operating Hour of PTP Obligations with Refund with each pair of source and sink Settlement Points settled in the DAM is calculated as follows:

If the PTP Obligation with Refund has a non-positive value, i.e., (DAOBLPR (j, k)  0), then

DAOBLRAMT o, (j, k)	=	(-1) * DAOBLRTP o, (j, k)

If the PTP Obligation with Refund has a positive value, i.e., (DAOBLPR (j, k) > 0), then

DAOBLRAMT o, (j, k)	=	(-1) * Max (DAOBLRTP o, (j, k) - DAOBLRDA, Min (DAOBLRTP, DAOBLRHV))

Where:

The target payment:

DAOBLRTP o, (j, k)	=	DAOBLPR (j, k) * Min (DAOBLR o, (j, k), OBLRACT o, (j, k))

DAOBLPR (j, k)	=	DASPP k - DASPP j

OBLRACT o, (j, k)	=	Σy(Σr(OBLROF o, r, (j, k) * RESACT r, (j, k), y) * TLMP y) / (ΣyTLMP y) * OBLRF o, (j, k)

If (OS r, y exists)

RESACT r, (j, k), y	=	OS r, y

Otherwise

If (EBP r, y exists)

RESACT r, (j, k), y	=	EBP r, y

Otherwise

RESACT r, (j, k), y	=	BP r, y

The derated amount:

DAOBLRDA o, (j, k)	=	OBLDRPR (j, k) * Min (DAOBLR o, (j, k),  OBLRACT o, (j, k))

OBLDRPR (j, k)	=	Σc(Max (0, DAWASF j, c – DAWASF k, c) *     DASP c * DRF c)

The hedge value:

DAOBLRHV o, (j, k)	=	DAOBLHVPR (j, k) * Min (DAOBLR o, (j, k), OBLRACT o, (j, k))

If the source, j, is a Load Zone or Hub and the sink, k, is a Resource Node,

DAOBLHVPR (j, k)	=	Max (0, MAXRESPR k – DASPP j)

If the source, j, is a Resource Node and the sink, k, is a Load Zone or Hub,

DAOBLHVPR (j, k)	=	Max (0, DASPP k – MINRESPR j)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOBLRAMT o, (j, k) | $ | Day-Ahead Obligation with Refund Amount per CRR Owner per pair of source and sinkThe payment to CRR Owner o for the PTP Obligation with Refund with the source j and the sink k, settled in the DAM, for the hour. |
| DAOBLRTP o, (j, k) | $ | Day-Ahead Obligation with Refund Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Obligations with Refund, with the source j and the sink k, settled in the DAM, for the hour. |
| DAOBLRHV o, (j, k) | $ | Day-Ahead Obligation with Refund Hedge Value per CRR Owner per source and sink pair—The hedge value of CRR Owner o’s PTP Obligations with Refund, with the source j and the sink k, settled in the DAM, for the hour. |
| DAOBLRDA o, (j, k) | $ | Day-Ahead Obligation with Refund Derated Amount per CRR Owner per source and sink pair—The derated amount of CRR Owner o’s PTP Obligations with Refund, with the source j and the sink k, settled in the DAM, for the hour. |
| DAOBLPR (j, k) | $/MW per hour | Day-Ahead Obligation PriceThe DAM price of a PTP Obligation with the source j and the sink k, for the hour. |
| DASPP j | $/MWh | Day-Ahead Settlement Point Price at sourceThe DAM Settlement Point Price at the source Settlement Point j for the hour. |
| DASPP k | $/MWh | Day-Ahead Settlement Point Price at sinkThe DAM Settlement Point Price at the sink Settlement Point k for the hour. |
| OBLDRPR (j, k) | $/MW per hour | Obligation Deration Price per source and sink pair—The deration price of a PTP Obligation with the source j and the sink k, for the hour. |
| DASP c | $/MW per hour | Day-Ahead Shadow Price per constraint—The DAM Shadow Price of the constraint c for the hour. |
| DRF c | none | Deration Factor per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASF j, c | none | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and the directional network element for constraint c, in the hour. |
| DAWASF k, c | none | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and the directional network element for constraint c, in the hour. |
| DAOBLHVPR (j, k) | $/MWh | Day-Ahead Obligation Hedge Value Price per source and sink pair—The Day-Ahead hedge price of a PTP Obligation with the source j and the sink k, for the hour. |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for Resources located at the source Settlement Point j. |
| MAXRESPR k | $/MWh | Max Resource Price for sink—The highest Maximum Resource Price for Resources located at the sink Settlement Point k. |
| DAOBLR o, (j, k) | MW | Day-Ahead Obligation with Refund per CRR Owner per pair of source and sink The number of CRR Owner o’s PTP Obligations with Refund with the source j and the sink k settled in DAM for the hour. |
| OBLRACT o, (j, k) | MW | Obligation with Refund Actual usage per CRR Owner per pair of source and sink—CRR Owner o’s actual usage for the PTP Obligations with Refund with the source j and the sink k, for the hour. |
| RESACT r, (j, k), y | MW | Resource Actual per resource associated with pair of source and sink per interval—The output of Resource r associated with the PTP Obligations with Refund with the source j and the sink k, for the SCED interval y. |
| OBLROF o, r, (j, k) | none | Obligation with Refund Ownership Factor per CRR Owner per resource associated with pair of source and sink—The factor showing the percentage usage of Resource r for CRR Owner o’s PTP Obligations with Refund with the source j and the sink k. Its value is 1, if only one CRR Owner has acquired PCRRs under the refund provision using this Resource r. |
| OS r, y | MW | Output Schedule per resource per SCED interval—The Output Schedule for Resource r for the SCED interval y. |
| EBP r, y | MW | Emergency Base Point per resource per SCED interval—The Emergency Base Point of Resource r for the SCED interval y. |
| BP r, y | MW | Base Point per resource per SCED interval—The Base Point of Resource r for the SCED interval y. |
| OBLRF o, (j, k) | none | Obligation with Refund Factor associated with pair of source and sink per CRR Owner—The ratio of CRR Owner o’s capacity allocated to the PTP Obligations with Refund with the source j and sink k to the same CRR Owner’s total capacity nominated for all the PCRRs under the refund provision with the same source j. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| o | none | A CRR Owner. |
| y | none | A SCED interval in the hour. |
| r | none | A Resource. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| c | none | A constraint associated with a directional network element for the hour. |

(4)	The net total payment or charge to each CRR Owner for the Operating Hour of all its PTP Obligations with Refund settled in the DAM is calculated as follows:

DAOBLRAMTOTOT o	=	DAOBLRCROTOT o + DAOBLRCHOTOT o

Where:

DAOBLRCROTOT o	=	ΣjΣkMin (0, DAOBLRAMT o, (j, k))

DAOBLRCHOTOT o	=	ΣjΣkMax (0, DAOBLRAMT o, (j, k))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOBLRAMTOTOT o | $ | Day-Ahead Obligation with Refund Amount Owner Total per CRR Owner—The net total payment or charge to CRR Owner o for all its PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRCROTOT o | $ | Day-Ahead Obligation with Refund Credit Owner Total per CRR Owner—The total payment to CRR Owner o for its PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRCHOTOT o | $ | Day-Ahead Obligation with Refund Charge Owner Total per CRR Owner—The total charge to CRR Owner o for its PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRAMT o, (j, k) | $ | Day-Ahead Obligation with Refund Amount per CRR Owner per pair of source and sink—The payment or charge to CRR Owner o for the PTP Obligations with Refund with the source j and the sink k settled in the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

7.9.1.6	Payments for PTP Options with Refund Settled in DAM

(1)	Except as specified otherwise in paragraph (2) below, ERCOT shall pay the owner of a PTP Option with Refund the difference in the DAM Settlement Point Prices between the sink Settlement Point and the source Settlement Point, if positive, subject to a charge for refund, as described in the item (e) (i) of Section 7.4.2, PCRR Allocation Terms and Conditions.

(2)	The payment of PTP Options with Refund may be further reduced due to transmission elements that are oversold in previous CRR auctions.

(3)	The payment to each CRR Owner for a given Operating Hour of its PTP Options with Refund with each pair of source and sink Settlement Points settled in the DAM is calculated as follows:

DAOPTRAMT o, (j, k)	=	(-1) * Max ((DAOPTRTP o, (j, k) –       DAOPTRDA o, (j, k)), Min (DAOPTRTP o, (j, k), DAOPTRHV o, (j, k)))

Where:

The target payment:

DAOPTRTP o, (j, k)	=	DAOPTPR (j, k) * Min (DAOPTR o, (j, k), OPTRACT o, (j, k)  * DAOPTR o, (j, k) /        (DAOPTR o, (j, k) + RTOPTR o, (j, k)))

DAOPTPR (j, k)	=	Max (0, DASPP k – DASPP j)

OPTRACT o, (j, k)	=	Σy(Σr(OPTROF o, r, (j, k) * RESACT r, (j, k), y) *   TLMP y) / (ΣyTLMPy) * OPTRF o, (j, k)

If (OS r, y exists)

RESACT r, (j, k), y	=	OS r, y

Otherwise

If (EBP r, y exists)

RESACT r, (j, k), y	=	EBP r, y

Otherwise

RESACT r, (j, k), y	=	BP r, y

The derated amount:

DAOPTRDA o, (j, k)	=	OPTDRPR (j, k) * Min (DAOPTR o, (j, k),    OPTRACT o, (j, k)  * DAOPTR o, (j, k) /        (DAOPTR o, (j, k) + RTOPTR o, (j, k)))

OPTDRPR (j, k)	=	Σc(Max (0, DAWASF j, c – DAWASF k, c) * DASP c * DRF c)

The hedge value:

DAOPTRHV o, (j, k)	=	DAOPTHVPR (j, k) * Min (DAOPTR o, (j, k), OPTRACT o, (j, k) * DAOPTR o, (j, k) /         (DAOPTR o, (j, k) + RTOPTR o, (j, k)))

DAOPTHVPR (j, k)	=	Max (0, DASPP k – MINRESPR j)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTRAMT o, (j, k) | $ | Day-Ahead Option with Refund Amount per CRR Owner per pair of source and sinkThe payment to CRR Owner o for its PTP Options with Refund with the source j and the sink k, settled in the DAM, for the hour. |
| DAOPTRTP o, (j, k) | $ | Day-Ahead Option with Refund Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in the DAM, for the hour. |
| DAOPTRHV o, (j, k) | $ | Day-Ahead Option with Refund Hedge Value per CRR Owner per source and sink pair—The hedge value of CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in the DAM, for the hour. |
| DAOPTRDA o, (j, k) | $ | Day-Ahead Option with Refund Derated Amount per CRR Owner per source and sink pair—The derated amount of CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in the DAM, for the hour. |
| DAOPTPR (j, k) | $/MW per hour | Day-Ahead Option Price per pair of source and sinkThe DAM price of the PTP Option with the source j and the sink k, for the hour. |
| DASPP j | $/MWh | Day-Ahead Settlement Point Price at sourceThe DAM Settlement Point Price at the source Settlement Point j, for the hour. |
| DASPP k | $/MWh | Day-Ahead Settlement Pont Price at sinkThe DAM Settlement Point Price at the sink Settlement Point k, for the hour. |
| DAOPTR o, (j, k) | MW | Day-Ahead Option with Refund per CRR Owner per pair of source and sinkThe number of CRR Owner o’s PTP Options with Refund with the source j and the sink k, settled in DAM, for the hour. |
| RTOPTR o, (j, k) | MW | Real-Time Option with Refund per CRR Owner per pair of source and sink—The number of CRR Owner o’s PTP Options with Refund with the source j and the sink k settled in Real-Time, for the hour. |
| OPTRACT o, (j, k) | MW | Option with Refund Actual usage per CRR Owner per pair of source and sink—CRR Owner o’s actual usage for the PTP Options with Refund with the source j and the sink k, for the hour. |
| RESACT r, (j, k), y | MW | Resource Actual per resource associated with pair of source and sink per interval—The output of Resource r associated with the PTP Options with Refund with the source j and the sink k, for the SCED interval y. |
| OPTROF o, r, (j, k) | none | Option with Refund Ownership Factor per CRR Owner per resource associated with pair of source and sink—The factor showing the percentage usage of Resource r for CRR Owner o’s PTP Options with Refund with the source j and the sink k. Its value is 1, if only one CRR Owner has acquired PCRRs under the refund provision using this Resource r. |
| OS r, y | MW | Output Schedule per resource per SCED interval—The Output Schedule for Resource r for the SCED interval y. |
| EBP r, y | MW | Emergency Base Point per resource per SCED interval—The Emergency Base Point of Resource r for the SCED interval y. |
| BP r, y | MW | Base Point per resource per SCED interval—The Base Point of Resource r for the SCED interval y. |
| OPTRF o, (j, k) | none | Option with Refund Factor associated with pair of source and sink per CRR Owner—The ratio of CRR Owner o’s capacity allocated to the PTP Options with Refund with the source j and sink k to the same CRR Owner’s total capacity nominated PCRRs under the refund provision with the same source j. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| OPTDRPR (j, k) | $/MW per hour | Option Deration Price per source and sink pair—The deration price of a PTP Option with the source j and the sink k, for the hour. |
| DASP c | $/MW per hour | Day-Ahead Shadow Price per constraint—The DAM Shadow Price of the constraint c for the hour. |
| DRF c | none | Deration Factor per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASF j, c | none | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and the directional network element for constraint c, in the hour. |
| DAWASF k, c | none | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and the directional network element for constraint c, in the hour. |
| DAOPTHVPR (j, k) | $/MWh | Day-Ahead Option Hedge Value Price per pair of source and sink—The Day-Ahead hedge price of a PTP Option with the source j and the sink k, for the hour. |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for Resources located at the source Settlement Point j. |
| o | none | A CRR Owner. |
| y | none | A SCED interval in the hour. |
| r | none | A Resource. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| c | none | A constraint associated with a directional network element for the hour. |

(4)	The total payment to each NOIE CRR Owner for the Operating Hour of all its PTP Options with Refund settled in the DAM is calculated as follows:

DAOPTRAMTOTOT o	=	ΣjΣkDAOPTRAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTRAMTOTOT o | $ | Day-Ahead Option with Refund Amount Owner Total per CRR Owner—The total payment to NOIE CRR Owner o for all its PTP Options with Refund settled in the DAM, for the hour. |
| DAOPTRAMT o, (j, k) | $ | Day-Ahead Option with Refund Amount per CRR Owner per pair of source and sink—The payment to NOIE CRR Owner o for the PTP Options with Refund with the source j and the sink k settled in the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

7.9.2	Real-Time CRR Payments and Charges

7.9.2.1	Payments and Charges for PTP Obligations Settled in Real-Time

(1)	ERCOT shall pay or charge the QSE of each PTP Obligation acquired in the DAM the difference in Real-Time Settlement Point Prices between the sink Settlement Point and the source Settlement Point.  The payment or charge to each QSE for a given Operating Hour of its cleared PTP Obligations with each pair of source and sink Settlement Points is calculated as follows:

RTOBLAMT q, (j, k)	=	(-1) * RTOBLPR (j, k) * RTOBL q, (j, k)

Where:

RTOBLPR (j, k)	=	Σ4i=1(RTSPP k, i – RTSPP j, i) / 4

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTOBLAMT q, (j, k) | $ | Real-Time Obligation Amount per QSE per pair of source and sink—The payment or charge to QSE q for its PTP Obligations with the source j and the sink k settled in Real-Time, for the hour. |
| RTOBLPR (j, k) | $/MW per hour | Real-Time Obligation Price—The Real-Time price of the PTP Obligation, for the hour. |
| RTSPP j, i | $/MWh | Real-Time Settlement Point Price at source per interval—The Real-Time Settlement Point Price at the source j for the 15-minute Settlement Interval i. |
| RTSPP k, i | $/MWh | Real-Time Settlement Point Price at sink per interval—The Real-Time Settlement Point Price at the sink k for the 15-minute Settlement Interval i. |
| RTOBL q, (j, k) | MW | Real-Time Obligation per QSE per pair of source and sink—The number of QSE q’s PTP Obligations for the source j and the sink k settled in Real-Time for the hour. |
| q | none | A QSE. |
| i | none | A 15-minute Settlement Interval in the Operating Hour. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(2)	The net total payment or charge to each QSE for the Operating Hour of all its PTP Obligations settled in Real-Time is calculated as follows:

RTOBLAMTQSETOT q	=	ΣjΣkRTOBLAMT q, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTOBLAMTQSETOT q | $ | Real-Time Obligation Amount QSE Total per QSE—The net total payment or charge to QSE q of all its PTP Obligations settled in Real-Time, for the hour. |
| RTOBLAMT q, (j, k) | $ | Real-Time Obligation Amount per QSE per pair of source and sink—The payment or charge to QSE q for the PTP Obligations with the source j and the sink k settled in Real-Time, for the hour. |
| q | none | A QSE. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

7.9.2.2	Payments for PTP Options Settled in Real-Time

(1)	Except as specified in paragraph (2) below, ERCOT shall pay the NOIE that owns a PTP Option that was declared before DAM execution by the NOIE to be settled in Real-Time and not cleared in the DAM, the positive difference in Real-Time Settlement Point Prices between the sink and the source.

(2)	For PTP Options that source or sink at a Resource Node, the PTP Option payment may be reduced due to transmission elements that are oversold in previous CRR auctions.

(3)	The payment to each NOIE CRR Owner for a given Operating Hour of the PTP Options with each pair of source and sink Settlement Points settled in Real-Time is calculated as follows:

If the source, j, is a Load Zone or Hub and the sink, k, is also a Load Zone or Hub, then

RTOPTAMT o, (j, k)	=	(-1) * RTOPTTP o, (j, k)

If either the source, j, or the sink, k, is a Resource Node, then

RTOPTAMT o, (j, k)	=	(-1) * Max ((RTOPTTP o, (j, k) – RTOPTDA o, (j, k)), Min (RTOPTTP o, (j, k), RTOPTHV o, (j, k)))

Where:

The target payment:

RTOPTTP o, (j, k)	=	RTOPTPR (j, k) * RTOPT o, (j, k)

RTOPTPR (j, k)	=	Σ4i=1Max (0, RTSPP k, i – RTSPP j, i) / 4

The derated amount:

RTOPTDA o, (j, k)	=	OPTDRPR (j, k) * RTOPT o, (j, k)

OPTDRPR (j, k)	=	Σc(Max (0, DAWASF j, c – DAWASF k, c) * DASP c * DRF c)

The hedge value:

RTOPTHV o, (j, k)	=	RTOPTHVPR (j, k) * RTOPT o, (j, k)

If the source, j, is a Load Zone or Hub and the sink, k, is a Resource Node,

RTOPTHVPR (j, k)	=	Σ4i=1Max (0, MAXRESPR k – RTSPP j, i) / 4

If the source, j, is a Resource Node and the sink, k, is a Load Zone or Hub,

RTOPTHVPR (j, k)	=	Σ4i=1Max (0, RTSPP k, i – MINRESPR j) / 4

If the source, j, is a Resource Node and the sink, k, is also a Resource Node,

RTOPTHVPR (j, k)	=	Max (0, MAXRESPR k – MINRESPR j)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTOPTAMT o, (j, k) | $ | Real-Time Option Amount per CRR Owner per source and sink pair —The payment to NOIE CRR Owner o of PTP Options with the source j and the sink k settled in Real-Time, for the hour. |
| RTOPTTP o, (j, k) | $ | Real-Time Option Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Options with the source j and the sink k settled in Real-Time, for the hour. |
| RTOPTHV o, (j, k) | $ | Real-Time Option Hedge Value per CRR Owner per source and sink pair—The hedge value of CRR Owner o’s PTP Options with the source j and the sink k settled in Real-Time, for the hour. |
| RTOPTDA o, (j, k) | $ | Real-Time Option Derated Amount per CRR Owner per source and sink pair—The derated amount of CRR Owner o’s PTP Options with the source j and the sink k settled in Real-Time, for the hour. |
| RTOPTPR (j, k) | $/MW per hour | Real-Time Option Price per source and sink pair —The Real-Time price of a PTP Option with the source j and the sink k for the hour. |
| RTSPP j, i | $/MWh | Real-Time Settlement Point Price at source per interval—The Real-Time Settlement Point Price at the source Settlement Point j, for the 15-minute Settlement Interval i. |
| RTSPP k, i | $/MWh | Real-Time Settlement Point Price at sink per interval—The Real-Time Settlement Point Price at the sink Settlement Point k, for the 15-minute Settlement Interval i. |
| OPTDRPR (j, k) | $/MW per hour | Option Deration Price per source and sink pair—The deration price of a PTP Option with the source j and the sink k, for the hour. |
| DASP c | $/MW per hour | Day-Ahead Shadow Price per constraint—The DAM Shadow Price of the constraint c for the hour. |
| DRF c | none | Deration Factor per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASF j, c | none | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and the constrained directional network element for constraint c, in the hour. |
| DAWASF k, c | none | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and the constrained directional network element for constraint c, in the hour. |
| RTOPTHVPR (j, k) | $/MWh | Real-Time Option Hedge Value Price per source and sink pair—The Day-Ahead hedge price of a PTP Option with the source j and the sink k, for the hour. |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for Resources located at the source Settlement Point j. |
| MAXRESPR k | $/MWh | Max Resource Price for sink—The highest Maximum Resource Price for Resources located at the sink Settlement Point k. |
| RTOPT o, (j, k) | MW | Real-Time Option per CRR Owner per pair of source and sink—The number of NOIE CRR Owner o’s PTP Options with the source j and the sink k settled in Real-Time for the hour. |
| o | none | A CRR Owner. |
| i | none | A 15-minute Settlement Interval in the Operating Hour. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| c | none | A DAM constraint associated with a directional network element for the hour. |

(4)	The total payment to each NOIE CRR Owner for the Operating Hour of all its PTP Options settled in Real-Time is calculated as follows:

RTOPTAMTOTOT o	=	ΣjΣkRTOPTAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTOPTAMTOTOT o | $ | Real-Time Option Amount Owner Total per CRR Owner—The total payment to NOIE CRR Owner o for all its PTP Options settled in Real-Time, for the hour. |
| RTOPTAMT o, (j, k) | $ | Real-Time Option Amount per CRR Owner per pair of source and sink—The payment to NOIE CRR Owner o for its PTP Options with the source j and the sink k settled in Real-Time, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(5)	For informational purposes, the following calculation of PTP Option value shall be posted on the MIS Public Area:

RTOPTPRINFO (j, k)	=	Σc[Σy(RTSP c, y * Max (0, RTWASF j, c, y – RTWASF k, c, y) * TLMP y) / (ΣyTLMP y)]

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTOPTPRINFO (j, k) | $/MW per hour | Real-Time Option Price per pair of source and sink—The Real-Time price of the PTP Options with the source Settlement Point j and the sink Settlement Point k, for the hour. |
| RTWASF j, c, y | none | Real-Time Weighted Average Shift Factor at source per constraint per SCED interval—The Real-Time Shift Factor for the source Settlement Point and for the constrained directional network element for constraint c, in the SECD interval y. |
| RTWASF k, c, y | none | Real-Time Weighted Average Shift Factor at sink per constraint per SCED interval—The Real-Time Shift Factor for the sink Settlement Point and for the constrained directional network element for constraint c, in the SCED interval y. |
| RTSP c, y | $/MW per hour | Real-Time Shadow Price per constraint per SCED interval—The Real-Time Shadow Price for the constraint c in the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| c | none | A constraint associated with a directional network element for the hour |
| y | none | A SCED interval in the hour. |

7.9.2.3	Payments for NOIE PTP Options with Refund Settled in Real-Time

(1)	Except as specified in paragraph (2) below, ERCOT shall pay the NOIE that owns a PTP Option with Refund that was allocated to that NOIE as a PCRR and that was, declared before DAM execution by the NOIE to be settled in Real-Time but not cleared in the DAM, for the MW quantity up to the pro-rata actual usage based on the positive difference in Real-Time Settlement Point Price between the sink and the source.

(2)	The payment of PTP Options with Refund may be further reduced due to transmission elements that are oversold in previous CRR auctions.

(3)	The payment to each NOIE CRR Owner for a given Operating Hour of the PTP Options with Refund with each pair of source and sink Settlement Points settled in Real-Time is calculated as follows:

RTOPTRAMT o, (j, k)	=	(-1) * Max ((RTOPTRTP o, (j, k) – RTOPTRDA o, (j, k)), Min (RTOPTRTP o, (j, k), RTOPTRHV o, (j, k)))

Where:

The target payment:

RTOPTRTP o, (j, k)	=	RTOPTPR (j, k) * Min (RTOPTR o, (j, k),       (OPTRACT o, (j, k) * RTOPTR o, (j, k) /               (RTOPTR o, (j, k) + DAOPTR o, (j, k))))

RTOPTPR (j, k)	=	Σ4i=1Max (0, RTSPP k, i – RTSPP j, i) / 4

OPTRACT o, (j, k)	=	Σy(Σr(OPTROF o, r, (j, k) * RESACT r, (j, k), y) * TLMP y) / (ΣyTLMPy) * OPTRF o, (j, k)

If (OS r, y exists)

RESACT r, (j, k), y	=	OS r, y

Otherwise

If (EBP r, y exists)

RESACT r, (j, k), y	=	EBP r, y

Otherwise

RESACT r, (j, k), y	=	BP r, y

The derated amount:

RTOPTRDA o, (j, k)	=	OPTDRPR (j, k) * Min (RTOPTR o, (j, k),         (OPTRACT o, (j, k) * RTOPTR o, (j, k) /               (RTOPTR o, (j, k) + DAOPTR o, (j, k))))

OPTDRPR (j, k)	=	Σc(Max (0, DAWASF j, c – DAWASF k, c) * DASP c * DRF c)

The hedge value:

RTOPTRHV o, (j, k)	=	RTOPTHVPR (j, k) * Min (RTOPTR o, (j, k),   (OPTRACT o, (j, k) * RTOPTR o, (j, k) /               (RTOPTR o, (j, k) + DAOPTR o, (j, k))))

RTOPTHVPR (j, k)	=	Max (0, RTSPP k – MINRESPR j)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTOPTRAMT o, (j, k) | $ | Real-Time Option with Refund Amount per CRR Owner per pair of source and sink—The payment to CRR Owner o of the PTP Options with Refund with the source j and the sink k, settled in Real-Time, for the hour. |
| RTOPTRTP o, (j, k) | $ | Real-Time Option with Refund Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in Real-Time, for the hour. |
| RTOPTRHV o, (j, k) | $ | Real-Time Option with Refund Hedge Value per CRR Owner per source and sink pair—The hedge value of CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in Real-Time, for the hour. |
| RTOPTRDA o, (j, k) | $ | Real-Time Option with Refund Derated Amount per CRR Owner per source and sink pair—The derated amount of CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in Real-Time, for the hour. |
| RTOPTPR (j, k) | $/MW per hour | Real-Time Option Price per pair of source and sink—The Real-Time price of the PTP Options with the source j and the sink k, for the hour. |
| RTSPP j, i | $/MWh | Real-Time Settlement Point Price at source per interval—The Real-Time Settlement Point Price at the source j for the 15-minute Settlement Interval i. |
| RTSPP k, i | $/MWh | Real-Time Settlement Point Price at sink per interval—The Real-Time Settlement Point Price at the sink k for the 15-minute Settlement Interval i. |
| OPTRACT o, (j, k) | MW | Option with Refund Actual usage per CRR Owwner per pair of source and sink—CRR Owner o’s actual usage for the PTP Options with Refund with the source j and the sink k, for the hour. |
| RESACT r, (j, k), y | MW | Resource Actual per resource associated with pair of source and sink per interval—The output of Resource r recognized for the CRR Owner’s PTP Options with Refund with the source j and the sink k, for the SCED interval y. |
| OPTROF o, r, (j, k) | none | Option with Refund Ownership Factor per CRR Owner per resource associated with pair of source and sink—The factor showing the percentage usage of Resource r for CRR Owner o’s PTP Options with Refund with the source j and the sink k. Its value is 1, if only one CRR Owner uses this Resource for PCRRs under the refund provision. |
| OS r, y | MW | Output Schedule per resource per SCED interval—The Output Schedule for Resource r for the SCED interval y. |
| EBP r, y | MW | Emergency Base Point per resource per SCED interval—The Emergency Base Point of Resource r for the SCED interval y. |
| BP r, y | MW | Base Point per resource per SCED interval—The Base Point of Resource r for the SCED interval y. |
| OPTRF o, (j, k) | none | Option with Refund Factor associated with pair of source and sink per CRR Owner—The ratio of CRR Owner o’s capacity allocated to the PTP Options with Refund with the source j and sink k to the same CRR Owner’s total capacity nominated for all the PCRRs under the refund provision with the same source j. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| RTOPTR (j, k) | MW | Real-Time Option with Refund per pair of source and sink—The number of the CRR Owner’s PTP Options with Refund with the source j and the sink k, settled in Real-Time, for the hour. |
| DAOPTR o, (j, k) | MW | Day-Ahead Option with Refund per CRR Owner per pair of source and sink—The number of CRR Owner o’s PTP Options with Refund settled in the DAM and sourced and sunk at the same Settlement Points as the PTP Option with Refund settled in Real-Time, for the hour. |
| OPTDRPR (j, k) | $/MW per hour | Option Deration Price per source and sink pair—The deration price of a PTP Option with the source j and the sink k, for the hour. |
| DASP c | $/MW per hour | Day-Ahead Shadow Price per constraint—The DAM Shadow Price of the constraint c for the hour. |
| DRF c | none | Deration Factor per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASF j, c | none | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and the directional network element for constraint c, in the hour. |
| DAWASF k, c | none | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and the directional network element for constraint c, in the hour. |
| RTOPTHVPR (j, k) | $/MWh | Real-Time Option Hedge Value Price per source and sink pair—The Real-Time hedge price of a PTP Option with the source j and the sink k, for the hour. |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for Resources located at the source Settlement Point j. |
| o | none | A CRR Owner. |
| r | none | A Resource. |
| y | none | A SCED interval in the hour. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| c | none | A constraint associated with a directional network element for the hour. |

(4)	The total payment to each NOIE CRR Owner for the Operating Hour of all its PTP Options with Refund settled in Real-Time is calculated as follows:

RTOPTRAMTOTOT o	=	ΣjΣkRTOPTRAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTOPTRAMTOTOT o | $ | Real-Time Option with Refund Amount Owner Total per CRR Owner—The total payment to NOIE CRR Owner o for all its PTP Options with Refund settled in Real-Time, for the hour. |
| RTOPTRAMT o, (j, k) | $ | Real-Time Option with Refund Amount per CRR Owner per pair of source and sink—The payment to NOIE CRR Owner o for the PTP Options with Refund with the source j and the sink k settled in Real-Time, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

7.9.3	CRR Balancing Account

7.9.3.1	DAM Congestion Rent

(1)	The DAM Congestion Rent is calculated as the sum of the following payments and charges:

(a)	The total of payments to all QSEs for cleared DAM energy offers (this does not include any revenue calculated for an RMR Unit, even though its Three-Part Supply Offer was cleared in the DAM), whether through Three-Part Supply Offers or through DAM Energy-Only Offer Curves, calculated under Section 4.6.2.1., Day-Ahead Energy Payment;

(b)	The total of revenue for all RMR Units as calculated below;

(c)	The total of charges to all QSEs for cleared DAM Energy Bids, calculated under Section 4.6.2.2	, Day-Ahead Energy Charge; and

(d)	The total of charges or payments to all QSEs for PTP Obligation Bids cleared in the DAM, calculated under Section 4.6.3, Settlement for PTP Obligations Bought in DAM.

(2)	The DAM Congestion Rent for a given Operating Hour is calculated as follows:

DACONGRENT	=	DAESAMTTOT + RMRDAEREVTOT + DAEPAMTTOT + DARTOBLAMTTOT

Where:

DAESAMTTOT	=	ΣqDAESAMTQSETOT q

DAEPAMTTOT	=	ΣqDAEPAMTQSETOT q

DARTOBLAMTTOT	=	ΣqDARTOBLAMTQSETOT q

RMRDAEREVTOT	=	ΣqΣpΣrDAEREV q, p, r

DAEREV q, p, r	=	(-1) * DASPP p * DAESR q, p, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DACONGRENT | $ | Day-Ahead Congestion RentThe Congestion Rent collected in the DAM for the hour. |
| DAESAMTTOT | $ | Day-Ahead Energy Sale Amount TotalThe total payment to all QSEs for cleared DAM energy offers, whether through Three-Part Supply Offers or through DAM Energy-Only Offer Curves for the hour. |
| RMRDAEREVTOT | $ | RMR Day-Ahead Energy Revenue Total(The total of the RMR Day-Ahead Energy Revenue for all RMR Units for the hour. See Section 6.6.6, Reliability Must-Run Settlement. |
| DAEPAMTTOT | $ | Day-Ahead Energy Purchase Amount TotalThe total charge to all QSEs for cleared DAM Energy Bids for the hour. |
| DARTOBLAMTTOT | $ | Day-Ahead Real-Time Obligation Amount TotalThe net total charge or payment to all QSEs for cleared PTP Obligation Bids in the DAM for the hour. |
| DAESAMTQSETOT q | $ | Day-Ahead Energy Sale Amount QSE Total per QSEThe total payment to QSE q for cleared DAM energy offers, whether through Three-Part Supply Offers or through DAM Energy-Only Offer Curves, for the hour. See Section 4.6.2.1, Day-Ahead Energy Payment, item (2). |
| DAEREV q, p, r | $ | Day-Ahead Energy Revenue per QSE by Settlement Point per unit(The revenue received in the DAM for RMR Unit r at Resource Node p represented by QSE q, based on the DAM Settlement Point Price, for the hour. |
| DASPP p | $/MWh | Day-Ahead Settlement Point Price by Settlement Point(The DAM Settlement Point Price at Resource Node p for the hour. |
| DAESR q, p, r | MW | Day-Ahead Energy Sale from Resource per QSE by Settlement Point per unit(The amount of energy cleared through Three-Part Supply Offers in the DAM and/or DAM Energy-Only Offer Curves for RMR Unit r at Resource Node p represented by QSE q for the hour. |
| DAEPAMTQSETOT q | $ | Day-Ahead Energy Purchase Amount QSE Total per QSEThe total charge to QSE q for cleared DAM Energy Bids for the hour. See Section 4.6.2.2, Day-Ahead Energy Charge, item (2). |
| DARTOBLAMTQSETOT q | $ | Day-Ahead Real-Time Obligation Amount QSE Total per QSE The total charge or payment to QSE q for PTP Obligation Bids cleared in the DAM for the hour. See Section 4.6.3, Settlement for PTP Obligations Bought in DAM, item (2). |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | An RMR Unit. |

7.9.3.2	Credit to CRR Balancing Account

If the Day-Ahead Congestion Rent is greater than the total payment to all CRR Owners for the CRRs settled in the DAM for any Operating Hour, a credit is put into the CRR Balancing Account for that Operating Hour.  The credit to the CRR Balancing Account for a given Operating Hour is calculated as follows:

CRRBACR	=	Max (0, (DACONGRENT + DACRRCRTOT + DACRRCHTOT))

Where:

DACRRCRTOT	=	DAOBLCRTOT + DAOBLRCRTOT + DAOPTAMTTOT + DAOPTRAMTTOT + DAFGRAMTTOT

DACRRCHTOT	=	DAOBLCHTOT + DAOBLRCHTOT

DAOBLCRTOT	=	ΣoDAOBLCROTOT o

DAOBLCHTOT	=	ΣoDAOBLCHOTOT o

DAOBLRCRTOT	=	ΣoDAOBLRCROTOT o

DAOBLRCHTOT	=	ΣoDAOBLRCHOTOT o

DAOPTAMTTOT	=	ΣoDAOPTAMTOTOT o

DAOPTRAMTTOT	=	ΣoDAOPTRAMTOTOT o

DAFGRAMTTOT	=	ΣoDAFGRAMTOTOT o

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| CRRBACR | $ | CRR Balancing Account Credit—The credit to the CRR Balancing Account for the hour. |
| DACONGRENT | $ | Day-Ahead Congestion Rent—The Congestion Rent collected in the DAM for the hour. See 7.9.3.1. |
| DACRRCRTOT | $ | Day-Ahead CRR Credit Total—The total payment to all CRR Owners of all CRRs settled in the DAM for the hour. |
| DACRRCHTOT | $ | Day-Ahead CRR Charge Total—The total charge to all CRR Owners of all CRRs settled in the DAM for the hour. |
| DAOBLCRTOT | $ | Day-Ahead Obligation Credit Total—The total payment of all PTP Obligations settled in the DAM, for the hour. |
| DAOBLCHTOT | $ | Day-Ahead Obligation Charge Total—The total charge of all PTP Obligations settled in the DAM, for the hour. |
| DAOBLRCRTOT | $ | Day-Ahead Obligation with Refund Credit Total—The total payment of all PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRCHTOT | $ | Day-Ahead Obligation with Refund Charge Total—The total charge of all PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOPTAMTTOT | $ | Day-Ahead Option Amount Total—The total payment of all PTP Options settled in the DAM, for the hour. |
| DAOPTRAMTTOT | $ | Day-Ahead Option with Refund Amount Total—The total payment of all PTP Options with Refund settled in the DAM, for the hour. |
| DAFGRAMTTOT | $ | Day-Ahead FGR Amount Total—The total payment of all FGRs settled in the DAM, for the hour. |
| DAOBLCROTOT o | $ | Day-Ahead Obligation Credit Owner Total per owner—The total payment to CRR Owner o of PTP Obligations settled in the DAM, for the hour. See Section 7.9.1.1, Payments and Charges for PTP Obligations Settled in DAM. |
| DAOBLCHOTOT o | $ | Day-Ahead Obligation Charge Owner Total per owner—The total charge to CRR Owner o of PTP Obligations settled in the DAM, for the hour. See Section 7.9.1.1 Payments and Charges for PTP Obligations Settled in DAM. |
| DAOBLRCROTOT o | $ | Day-Ahead Obligation with Refund Credit Owner Total per owner—The total payment to the CRR Owner o of PTP Obligations with Refund settled in the DAM, for the hour. See Section 7.9.1.5, Payments and Charges for PTP Obligations with Refund Settled in DAM. |
| DAOBLRCHOTOT o | $ | Day-Ahead Obligation with Refund Charge Owner Total per owner—The total charge to CRR Owner o of PTP Obligations with Refund settled in the DAM, for the hour. See Section 7.9.1.5, Payments and Charges for PTP Obligations with Refund Settled in DAM. |
| DAOPTAMTOTOT o | $ | Day-Ahead Option Amount Owner Total per owner—The total payment to the CRR Owner o of PTP Options settled in the DAM, for the hour. See Section 7.9.1.2, Payments for PTP Options Settled in DAM. |
| DAOPTRAMTOTOT o | $ | Day-Ahead Option with Refund Amount Owner Total per owner—The total payment to the CRR Owner o of PTP Options with Refund settled in the DAM, for the hour. See Section 7.9.1.6, Payments for PTP Options with Refund Settled in DAM. |
| DAFGRAMTOTOT o | $ | Day-Ahead FGR Amount Owner Total per owner—The total payment to the CRR Owner o of FGRs settled in the DAM, for the hour. See Section 7.9.1.4, Payments for FGRs Settled in DAM. |
| o | none | A CRR Owner. |

7.9.3.3	Shortfall Charges to CRR Owners

(1)	For each Operating Hour, if the Day-Ahead Congestion Rent is less than the total payment to all CRR Owners for the CRRs settled in the DAM, a charge will be made to each CRR Owner for any of its CRRs settled in the DAM or Real-Time that have positive settlement prices, except for CRRs bought in the DAM.

(2)	The charge to each CRR Owner for its CRRs settled in the DAM for a given Operating Hour is calculated as follows:

DACRRSAMT o	=	DACRRSAMTTOT * CRRCRRSDA o

Where:

DACRRSAMTTOT	=	(-1) * Min (0, DACONGRENT + DACRRCRTOT + DACRRCHTOT)

CRRCRRSDA o	=	(DAOBLCROTOT o + DAOBLRCROTOT o + DAOPTAMTOTOT o + DAOPTRAMTOTOT o + DAFGRAMTOTOT o) / (DACRRCRTOT + RTOPTAMTTOT + RTOPTRAMTTOT)

RTOPTAMTTOT	=	ΣoRTOPTAMTOTOT o

RTOPTRAMTTOT	=	ΣoRTOPTRAMTOTOT o

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DACRRSAMT o | $ | Day-Ahead CRR Shortfall Amount per owner—The shortfall charge to CRR Owner o for its CRRs settled in the DAM, due to deration, for the hour. |
| DACRRSAMTTOT | $ | Day-Ahead CRR Shortfall Amount Total—The shortfall charge to all CRR Owners for their CRRs settled in the DAM, due to deration, for the hour. |
| DACONGRENT | $ | Day-Ahead Congestion Rent—The Congestion Rent collected in the DAM for the hour. See 7.9.3.1. |
| DACRRCRTOT | $ | Day-Ahead CRR Credit Total—The total payment to all CRR Owners of all the CRRs settled in the DAM, for the hour. See 7.9.3.3. |
| DACRRCHTOT | $ | Day-Ahead CRR Charge Total—The total charge to all CRR Owners of all the CRRs settled in the DAM, for the hour. See 7.9.3.3. |
| CRRCRRSDA o | none | CRR Credit Ratio Share Day-Ahead per owner—The ratio of the total payments to CRR Owner o of its CRRs settled in the DAM to the total payments to all CRR Owners of all CRRS, for the hour. |
| DAOBLCROTOT o | $ | Day-Ahead Obligation Credit Owner Total per owner—The total payment to CRR Owner o of PTP Obligations settled in the DAM, for the hour. See Section 7.9.1.1, Payments and Charges for PTP Obligations Settled in DAM. |
| DAOBLRCROTOT o | $ | Day-Ahead Obligation with Refund Credit Owner Total per owner—The total payment to CRR Owner o of PTP Obligations with Refund settled in the DAM, for the hour. See Section 7.9.1.5, Payments and Charges for PTP Obligations with Refund Settled in DAM. |
| DAOPTAMTOTOT o | $ | Day-Ahead Option Amount Owner Total per owner—The total payment to CRR Owner o of PTP Options settled in the DAM, for the hour. See Section 7.9.1.2, Payments PTP Options Settled in DAM. |
| DAOPTRAMTOTOT o | $ | Day-Ahead Option with Refund Amount Owner Total per owner—The total payment to CRR Owner o of PTP Options with Refund settled in the DAM, for the hour. See Section 7.9.1.6, Payments for PTP Options with Refund Settled in DAM |
| DAFGRAMTOTOT o | $ | Day-Ahead FGR Amount Owner Total per owner—The total payment to CRR Owner o of FGRs settled in the DAM, for the hour. See Section 7.9.1.4, Payments for FGRs Settled in DAM. |
| RTOPTAMTTOT | $ | Real-Time Option Amount Total—The total of payments to all CRR Owners of all PTP Options settled in Real-Time for the hour. |
| RTOPTRAMTTOT | $ | Real-Time Option with Refund Amount Total—The total of payments to all CRR Owners of all PTP Options with Refund settled in Real-Time for the hour. |
| RTOPTAMTOTOT o | $ | Real-Time Option Amount Owner Total per owner—The total payment to CRR Owner o of all its PTP Options settled in Real-Time for the hour. See Section 7.9.2.2, Payments for PTP Options Settled in Real-Time. |
| RTOPTRAMTOTOT o | $ | Real-Time Option with Refund Amount Owner Total per owner—The total payment to CRR Owner o of all its PTP Options with Refund settled in Real-Time for the hour. See Section 7.9.2.3, Payments for NOIE PTP Options with Refund Settled in Real-Time. |
| o | none | A CRR Owner. |

(3)	The charge to each CRR Owner for its CRRs settled in Real-Time for a given Operating Hour is calculated as follows:

RTCRRSAMT o	=	DACRRSAMTTOT * CRRCRRSRT o

Where:

CRRCRRSRT o	=	(RTOPTAMTOTOT o + RTOPTRAMTOTOT o) / (DACRRCRTOT + RTOPTAMTTOT + RTOPTRAMTTOT)

RTOPTAMTTOT	=	ΣoRTOPTAMTOTOT o

RTOPTRAMTTOT	=	ΣoRTOPTRAMTOTOT o

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTCRRSAMT o | $ | Real-Time CRR Shortfall Amount per owner—The shortfall charge to CRR Owner o for its CRRs settled in Real-Time, due to deration, for the hour. |
| DACRRSAMTTOT | $ | Day-Ahead CRR Shortfall Amount Total—The shortfall charge to all CRR Owners for their CRRs settled in the DAM, due to deration, for the hour. |
| CRRCRRSRT o | none | CRR Credit Ratio Share Real-Time per owner—The ratio of the total payments to CRR Owner o of its CRRs settled in Real-Time to the total payments to all CRR Owners of all CRRS, for the hour. |
| RTOPTAMTTOT | $ | Real-Time Option Amount Total—The total of payments to all CRR Owners of all PTP Options settled in Real-Time for the hour. |
| RTOPTRAMTTOT | $ | Real-Time Option with Refund Amount Total—The total of payments to all CRR Owners of all PTP Options with Refund settled in Real-Time for the hour. |
| RTOPTAMTOTOT o | $ | Real-Time Option Amount Owner Total per owner—The total payment to CRR Owner o of all its PTP Options settled in Real-Time for the hour. See Section 7.9.2.2, Payments for PTP Options Settled in Real-Time. |
| RTOPTRAMTOTOT o | $ | Real-Time Option with Refund Amount Owner Total per owner—The total payment to CRR Owner o of all its PTP Options with Refund settled in Real-Time for the hour. See Section 7.9.2.3, Payments for NOIE PTP Options with Refund Settled in Real-Time. |
| o | none | A CRR Owner. |

7.9.3.4	Monthly Refunds to Short-Paid CRR Owners

On a monthly basis, a refund may be paid to the CRR Owners that have a shortfall charge for any Operating Hour in a month.   The refund to each CRR Owner for a given month is calculated as follows:

CRRRAMT o	=	(-1) * Min (CRRBACRTOT, CRRSAMTTOT) * CRRSAMTRS o

Where:

CRRBACRTOT	=	ΣhCRRBACR h

If (CRRSAMTTOT = 0)

CRRSAMTRS o	=	0

Otherwise

CRRSAMTRS o	=	CRRSAMTOTOT o / CRRSAMTTOT

CRRSAMTTOT	=	ΣoCRRSAMTOTOT o

CRRSAMTOTOT o	=	Σh(DACRRSAMT o, h + RTCRRSAMT o, h)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| CRRRAMT o | $ | CRR Refund Amount per owner—The refund to the short-paid CRR Owner o for the month. |
| CRRBACRTOT | $ | CRR Balancing Account Credit Total—The total of credits accumulated in the CRR Balancing Account for all Operating Hours in the month. |
| CRRSAMTTOT | $ | CRR Shortfall Amount Total—The total of shortfall charges to all CRR Owners for all Operating Hours in the month. |
| CRRSAMTRS o | none | CRR Shortfall Amount Ratio Share per owner—The ratio of the CRR Owner o’s total shortfall-charge to the total of all the CRR Owners’ shortfall charges, for the month. |
| CRRSAMTOTOT o | $ | CRR Shortfall Amount Owner Total per owner—The total of shortfall charges to CRR Owner o for all Operating Hours in the month. |
| DACRRSAMT o, h | $ | Day-Ahead CRR Shortfall Amount per owner per hour—The shortfall charge to CRR Owner o for its CRRs settled in the DAM for the hour h. |
| RTCRRSAMT o, h | $ | Real-Time CRR Shortfall Amount per owner per hour—The shortfall charge to CRR Owner o for its CRRs settled in Real-Time for the hour h. |
| CRRBACR h | $ | CRR Balancing Account Credit per hour—The credit to the CRR Balancing Account for the hour h. |
| h | none | An Operating Hour in the month. |
| o | none | A CRR Owner. |

7.9.3.5	CRR Balancing Account Closure

(1)	At the end of each month, the surplus in the CRR Balancing Account, if any, is paid to the QSEs representing LSEs based on a monthly Load Ratio Share.  The monthly Load Ratio Share is the 15-minute Load Ratio Share calculated for the peak-load Settlement Interval during the month.

(2)	The credit to each QSE representing LSEs for a given month is calculated as follows:

LACRRAMT q	=	(-1) * (CRRBACRTOT + CRRRAMTTOT) * MLRS q

Where:

CRRRAMTTOT	=	ΣoCRRRAMT o

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LACRRAMT q | $ | Load-Allocated CRR Amount per QSE—The allocated surplus in the CRR Balancing Account at the end of the month to QSE q, based on Load Ratio Share for the month. |
| CRRBACRTOT | $ | CRR Balancing Account Credit Total—The total credit accumulated in the CRR Balancing Account during the month. See its calculation in Section 7.9.3.4, Monthly Refunds to Short-Paid CRR Owners. |
| CRRRAMTTOT | $ | CRR Refund Amount Total—The total refund to all the previously short-paid CRR Owners at the end of the month. |
| CRRRAMT o | $ | CRR Refund Amount per owner—The refund credited to the CRR Owner o at the end of the month. |
| MLRS q | none | Monthly Load Ratio Share per QSE—The Load Ratio Share calculated for QSE q for the 15-minute monthly peak-load Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval, for the calculation of LRS for a 15-minute Settlement Interval. |
| q | none | A QSE. |
| o | none | A CRR Owner. |