Texas Nodal

Market Management Systems (MMS)

Requirements Specification for Constraint Competitiveness Test (CCT)

Version 1.0.0

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 08/14/06 | 1.0 | Initial Draft, for delivery to prospective vendor | Mark Patterson |
| 9/1/2006 | 1.1 | Updated for a detailed description of the functional requirements | Hedayat Etemadi |
| 9/14/2006 | 1.2 | Updated Use Cases, Revised based on the ERCOT review on 09/14/06 | Hedayat Etemadi |
| 9/20/2006 | 1.3 | Updated for clarification to the list of competitive constraints to post as well as definition of “import” and “export” side of a constraint. Updated to reflect ERCOT recommended changes to the user cases | Mark Patterson |
| 9/25/2006 | 1.4 | Updated on how to set import/export terminal of a constraint | Hedayat Etemadi |
| 10/25/2006 | 0.9.0 | Updated tp reflect IDA comments Add Sections 1.7 & 1.8 | Mark Patterson implemented by Hedayat Etemadi |
| 11/01/2006 | 0.9.1 | Updated Section 1 per Customer Requirements | Mary Nelson |
| 11-02-2006 | 0.9.2 | Updated formatting | Hedayat Etemadi |
| 12-08-2006 | 0.9.3 | Revised to incorporate TPTF comments | Shu Tao |
| 01-22-2007 | 0.9.4 | Revised to incorporate TPTF comments | Hedaya Etemadi |
| 01-25-2007 | 1.0.0 | No content changes. New version number. | Hedaya Etemadi |

Table of Contents

1.	Introduction	5

1.1	Purpose	5

1.2	Objective	5

1.3	Traceability	5

1.4	Market Management System Requirement Documents	5

2.	Constraint Competitiveness Test (CCT)	6

2.1	CCT Function	8

2.2	Annual CCT	11

2.3	Monthly CCT	13

2.4	Daily CCT	15

2.5	CCT Inputs and Outputs	17

2.6	Assumptions and Dependencies	18

3.	Functional Requirements	18

3.1	Business Sub-Process 1: Input Data Interface	18

3.2	Business Sub-Process 2: Posting of Competitive Constraints to MIS Secure Area	18

3.3	Business Sub-Process 3: CCT Function	20

3.4	Business Sub-Process 4: Annual CCT	20

3.5	Business Sub-Process 5: Monthly CCT	21

3.6	Business Sub-Process 6: Daily CCT	22

4.	Use Cases	23

4.1	Actor Catalog	23

4.2	Use Case Catalog	23

4.3	Protocol Coverage	24

4.4	Sub-Process Coverage	24

5.	Appendix A - Clarification Notes on the CCT Test Procedures for Texas’s Nodal Market	24

5.1	Nodal Protocol Description of Constraint Competitiveness Test (Protocol reference: Section 3.19.1)	Error! Bookmark not defined.

5.2	Test Procedure 1 – Determine Element Competitiveness Index (ECI)	Error! Bookmark not defined.

5.3	Test Procedure 2 – Determine whether a Pivotal Player exists	Error! Bookmark not defined.

5.4	Algorithmic description of the CCT Procedure	Error! Bookmark not defined.

5.5	Definitions	Error! Bookmark not defined.

5.6	Test Procedure 1 – Determine ECI on the import terminal of the constraint	Error! Bookmark not defined.

5.7	Test Procedure 2 - Determine whether a Pivotal Player exists	Error! Bookmark not defined.

5.8	Test Procedure 1 – Determine ECI on the export terminal of the constraint	Error! Bookmark not defined.

List of Figures

Figure 2-1 – Real Time Market Overview Diagram	7

Figure 2-2– CCT Function Overview Diagram	9

Figure 2-3 – CCT Function Overview Diagram (continued)	10

Figure 2-4 – Annual CCT Overview Diagram	12

Figure 2-5 – Monthly CCT Overview Diagram	14

Figure 2-6 – Daily CCT Overview Diagram	16

Requirements Specification

(This Requirements Document is Subordinate To and Compliant with the Texas Nodal Protocols effective May, 2006.)

# Introduction

The primary authority directing the operational characteristics and behavior of the Market Management System is the Texas Nodal Protocols. This requirements document set is provided as a supplement to assist in the implementation of this system which includes all processes, tools (hardware and software), and operations and constraints necessary for full compliance to the Texas Nodal Protocols. The Texas Nodal Protocols with this set of requirements documents completes the System “Design To” definition for implementation, test, and operation of a fully compliant Market Management System.

This set of requirements documents consists of five (5) documents as listed in the Market Management Systems Requirement Documents section which follows.

## Purpose

The requirements for the document Texas Nodal Market Management Systems (MMS) Requirement Specification for Constraint Competitiveness Test (CCT) and its associated processes are described in the Texas Nodal Protocols. This supplemental specification describes the external behavior of this specific sub-system. The Texas Nodal Protocols (Protocols) and this document together with all applied vendor provided products’ functional specifications, describe both functional and nonfunctional requirements, design constraints, and other factors, in order to provide a complete and comprehensive description of the operational performance to be delivered. All statements or requirements specified in this document are subordinate to the Protocols.

## Objective

The objective of this set of documents is to provide a clear, concise and unambiguous set of requirements together with the Texas Nodal Protocols which provides the complete required technical description of this Texas Nodal Sub-System to allow the developer/implementer to deliver a fully operational, compliant and robust Texas Nodal System.

## Traceability

All requirements are traceable to the Nodal Protocols, and/or regulations such as NERC and FERC.

## Market Management System Requirement Documents

  - Texas Nodal Market Management Systems (MMS) Requirement Specification For Day-Ahead Market (DAM) and Supplemental Ancillary Service Market (SASM)

  - Texas Nodal Market Management Systems (MMS) Requirement Specification For RUC

  - Texas Nodal Market Management Systems (MMS) Requirement Specification For SCED and Real-Time MMS Processes

  - Texas Nodal Market Management Systems (MMS) Requirement Specification For Constraint Competitiveness Test (CCT)

  - Texas Nodal Market Management Systems (MMS) Requirement Specification For Overall MMS System and Other Processes

# Constraint Competitiveness Test (CCT)

Constraint Competitiveness Tests (CCT) are to be performed on each “Competitive Constraint” to determine if the constraint is “Competitive” or not. These constraints are the contingency/limiting Transmission Element pairs that represent the Commercially Significant Constraints (CSCs) and Closely Related Elements (CREs) and will be defined immediately prior to the Texas Nodal Market Implementation date.

Two test procedures are employed in the CCT:

- In Test Procedure 1 the effective capacity available to resolve the constraint on both the import and export side is first determined. The available effective capacity is then used to compute the Element Competitiveness Index (ECI), an index similar to the Herfindahl-Hirschman Index on the import and export side.

- In Test Procedure 2, analysis is conducted to check for the existence of any pivotal player. A constraint is determined to be competitive or non-competitive depending on whether the computed ECIs meet the Protocol defined ECI threshold on the import and export side as well as whether there is a pivotal player. The CCT test is to be performed annually, monthly, and daily.

Resource energy offers may be mitigated in real-time Security Constrained Economic Dispatch process due to the existence of a non-competitive constraint. The Security Constrained Economic Dispatch (SCED) process uses a two-step methodology that applies mitigation prospectively to resolve Non-Competitive Constraints for the current Operating Hour. The first step is to execute the SCED process to determine Reference LMPs.

In this step, ERCOT executes SCED using the full Network Operations Model while only observing limits of Competitive Constraints. In the second step, each energy offer curve is capped at the greater of the reference LMP at the resource node or the appropriate mitigated offer cap; or bounded at the lesser of the reference LMP or the appropriate mitigated offer floor. The SCED process is executed to produce Base Points, Shadow Prices, and LMPs, subject to security constraints (including Competitive and Non-Competitive Constraints) and other Resource constraints.

In DAM and RUC process, any contingency defined with SPS/RAP is treated in the following manner: If SPS/RAP triggering condition is met, then the associated constraint is removed as it is assumed that the SPS/RAP activation will secure the contingency. In other words, constraint violations for such contingencies exist only when the SPS/RAP is not activated. Therefore, all contingencies are modeled without the SPS/RAP activation in the CCT process, thereby consistent with the DAM and RUC processes.

Figure 2-1 presents an overview diagram of Real-Time Market. In this diagram, the CCT process in the box labeled as Competitiveness Test for Constraints supplies competitive/non-competitive constraint input data for the SCED process. The results of the daily CCT process are applied in the SCED process for real-time market.

The following section describes CCT sub processes.

Figure 2-1 – Real Time Market Overview Diagram

## CCT Function

CCT function is the main sub-process in CCT process. CCT function sub process is used by annual CCT, monthly CCT and daily CCT sub process to perform the following calculations:

- Calculate Element Competitive Index (ECI) on both import and export side of each constraint in the CSCs and CREs list to determine whether sufficient competition exist in resolving the constraint. The CSCs and CREs list shall include information regarding the direction of each constraint. The constraint’s direction information shall be used to determine import/export terminal of the constraint, i.e., the from-bus is designated as the “export terminal” and the to-bus the “import terminal”.

- Determine whether a pivotal player exists in resolving the constraint.

An overview diagram of the CCT function is shown in Figure 2-2– CCT Function Overview Diagram.

Figure 2-2– CCT Function Overview Diagram

Figure 2-3 – CCT Function Overview Diagram (continued)

## Annual CCT

Annual CCT sub-process shall identify competitive and non-competitive constraints for each constraint in the CSCs and CREs list for every month of the year. A constraint is designated as a competitive constraint for a year if it is a competitive constraint for every month of the year. The network model used for annual CCT is based on the monthly peak case used to auction the on-peak CRRS. The monthly peak case must include planned transmission and generation outages for the month. An overview diagram of annual CCT is shown in Figure 2-4.

Figure 2-4 – Annual CCT Overview Diagram

## Monthly CCT

Monthly CCT sub-process shall update competitive and non-competitive constraint status for a particular month as determined by the annual CCT sub-process. That is, a constraint designated as a competitive constraint in the annual CCT could change its status to a non-competitive constraint and vise versa in a monthly CCT sub-process. The network model used for monthly CCT is based on the monthly peak case used to auction the on-peak CRRs. Monthly CCT criteria to designate a constraint as a competitive or a non-competitive constrains are different from the annual CCT criteria. An overview diagram of the monthly CCT is shown in Figure 2-5.

Figure 2-5 – Monthly CCT Overview Diagram

## Daily CCT

Daily CCT sub-process shall update the competitive and non-competitive constraint status for the particular day as determined by the monthly CCT sub-process. That is, a constraint designated as a competitive constraint in the monthly CCT could change its status to a non-competitive constraint and vise versa in the daily CCT sub-process. The network model used for daily CCT is based on the peak hour case for the particular day. Daily CCT criteria to designate a constraint as a competitive or a non-competitive are the same as those used in the monthly CCT sub-process. An overview diagram of the daily CCT is shown in Figure 2-6.

Figure 2-6 – Daily CCT Overview Diagram

## CCT Inputs and Outputs

This section provides a summary of inputs and outputs to CCT process. Table 2-1 lists all the required CCT input data.

The “Source” column of the input table identifies the source of the data. The “Destination” column of the output table identifies the destination target where the data generated by the CCT resides.

Table 2-1 – CCT Inputs

| Source | Data |
| --- | --- |
| ERCOT | - Network model data in PSS/E format - See PSS/E data format for all network model data - For annual and monthly CCT sub-processes, monthly peak case used to auction the On-Peak CRRs shall be provided. For Daily CCT, peak hour case of the particular day shall be provided. - The monthly peak case and the peak hour case must include planned transmission and generation outages. - List of Commercially Significant Constraints (CSCs) and Closely Related Elements (CREs). - Contingency/Limiting Transmission Element pair Note: The CSCs and CREs list includes information regarding the direction of each constraint. - Contingency definition - Resource available capacity, resource type and resource entity - Resource available capacity Note: “Available Capacity” is defined as the High Sustained Limit (HSL) of a Generation resource, including switchable generation that is not on a planned outage (expect wind-powered generation), expected On-Peak wind generation output, and full import capability of the DC tie lines. - Resource type - Minimum energy amounts of Coal and lignite generations - Resource entity (ownership information) - Mapping resource type to PSS/E generator data |

The table below lists outputs from CCT to various systems.

Table 2-2 CCT Outputs

| Destination | Data |
| --- | --- |
| Real Time Market (RTM) data storage | List of competitive and non-competitive constraints |

## Assumptions and Dependencies

The functionality and requirements specified in this document are based on the following assumptions:

- CCT process does not maintain any network model data in its database. All the network model data is provided to the CCT process in a PSS/E format.

- The monthly peak case and the peak hour case include planned transmission and generation outages.

- Figures are intended to be only a general guide and not controlling language, and any conflict between figures and another section of the requirements documents is controlled by the other section in the requirements document.

- All tunable parameters described in this requirement document that are changed must be posted and available for Market Participants. The changes to these tunable parameters must be tightly controlled and only allowable with the concurrence of Market Participants except in case of operational emergency.

# Functional Requirements

## Business Sub-Process 1: Input Data Interface

| Requirement ID | FR1 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 3.19 (1) |
| Traceability to Sub-Process Maps | SP1; Input data interface |
| Description : The input data interface sub-process shall retrieve and store all the input data in Table 2-1 – CCT Inputs. The sub-process shall provide the most recent input data to other CCT sub-processes (Annual CCT, Monthly CCT and Daily CCT). The sub-process shall provide a Graphical User Interface (GUI) to ERCOT market operator for all input data. |  |

## Business Sub-Process 2: Posting of Competitive Constraints to MIS Secure Area

| Requirement ID | FR2 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 3.19 (5) |
| Traceability to Sub-Process Maps | SP2; Posting of competitive constraints to MIS secure area |
| Description : The sub-process shall post the Board approved list of the “Competitive Constraints” to the MIS Secure Area for at least 5 Business Days before any change takes effect. This is the list of contingency/limiting Transmission Elements pairs that represent the Commercially Significant Constraints (CSCs) and Closely Related Elements (CREs). |  |

## Business Sub-Process 3: CCT Function

| Requirement ID | FR3 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 3.19(3)(b),3.19.1(2),3.19.1(3) |
| Traceability to Sub-Process Maps | SP3; CCT Function |
| Description : The CCT function sub-process shall provide all components and steps described in Figure 2-2– CCT Function Overview Diagram. The input data to CCT function sub-process shall be provided as described in Table 2-1 – CCT Inputs. The sub-process shall provide all outputs as described in Table 2-2 CCT Outputs. |  |

## Business Sub-Process 4: Annual CCT

| Requirement ID | FR4 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 3.19(6),3.19.1(1) |
| Traceability to Sub-Process Maps | SP4; Annual CCT |
| Description : The annual CCT sub-process shall provide all components and steps described in Figure 2-4 – Annual CCT Overview Diagram. The input data to annual CCT sub-process shall be provided as described in Table 2-1 – CCT Inputs. The sub-process shall provide all outputs as described in Table 2-2 CCT Outputs. The network model used for annual CCT is based on the monthly peak case used to auction the on-peak CRRs. A default value of one third (which will be multiplied by the highest resource shift factor) is used as Shift Factor Thresholds in annual CCT sub-process to: - Determine the effective capacity needed on import side to resolve constraint - Determine the effective capacity needed on export side to resolve the constraint ERCOT market operator shall be able to modify the default setting for Shift Factor Thresholds. The following set of default values shall be used as ECI Thresholds in annual CCT sub-process: - 2000 on the import side and 2500 on the export side ERCOT market operator shall be able to modify the default setting for ECI. The annual CCT process shall store only the most recent results produced by the sub-process. |  |

## Business Sub-Process 5: Monthly CCT

| Requirement ID | FR5 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 3.19.2(1) |
| Traceability to Sub-Process Maps | SP5; Monthly CCT |
| Description : Prior to the monthly CRR auction the monthly peak case , used to auction the monthly on-peak CRRs, and the most current Managed Capacity information shall be used to perform a monthly CCT for each competitive and non-competitive constraint as determined by the annual CCT sub-process for the particular month. The monthly CCT process shall have the ability to change the treatment of a competitive constraint to a non-competitive constraint and vice versa for the particular month if the constraint meets the following conditions: - A Competitive Constraint shall be treated as a non-competitive constraint if the ECI is greater than 2,500 on the import side or the ECI is greater than 3,000 on the export side or there is a pivotal player in resolving the constraint. The ECI shall be determined as described in Figure 2-3 – CCT Function Overview Diagram (continued). The existence of a pivotal player shall be determined as described in Figure 2-2– CCT Function Overview Diagram. - A non-competitive constraint shall be treated as a Competitive Constraint if the ECI is less than 1,500 on the import side and the ECI is less than 2,000 on the export side and there is no pivotal player in resolving the constraint. The ECI shall be determined as described in Figure 2-3 – CCT Function Overview Diagram (continued). The existence of a pivotal player shall be determined as described in Figure 2-2– CCT Function Overview Diagram. The monthly CCT sub-process shall provide all components and steps described in Figure 2-5 – Monthly CCT Overview Diagram. The input data to monthly CCT sub-process shall be provided as described in Table 2-1 – CCT Inputs. The sub-process shall provide all outputs as described in Table 2-2 CCT Outputs. A default value of 1/3 (which will be multiplied by the highest resource shift factor) is used as Shift Factor Thresholds in monthly CCT sub-process to: - Determine effective capacity needed on import side to resolve constraint - Determine effective capacity needed on export side to resolve the constraint ERCOT market operator shall be able to modify the Shift Factor Thresholds. ERCOT market operator shall be able to modify the ECI Thresholds. The monthly CCT sub-process shall use the most recent results produced by annual CCT sub-process. |  |

## Business Sub-Process 6: Daily CCT

| Requirement ID | FR6 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | 3.19.3(1),3.19.3(2) |
| Traceability to Sub-Process Maps | SP6; Daily CCT |
| Description : In the day-ahead of the operation day (a timeline to be defined by ERCOT), the DAM network model for the peak hour, and the most current Managed Capacity information shall be used to perform a daily CCT for each competitive and non-competitive constraint resulting from monthly CCT sub-process for the particular day. The daily CCT sub-process shall have the ability to change the treatment of a competitive constraint to a non-competitive constraint and vice versa for the particular day if the constraint meets the following conditions: - A Competitive Constraint shall be treated as a non-competitive constraint if the ECI is greater than 2,500 on the import side or the ECI is greater than 3,000 on the export side or there is a pivotal player in resolving the constraint. The ECI shall be determined as described in Figure 2-3 – CCT Function Overview Diagram (continued). The existence of a pivotal player shall be determined as described in Figure 2-2– CCT Function Overview Diagram. - A non-competitive constraint shall be treated as a Competitive Constraint if the ECI is less than 1,500 on the import side and the ECI is less than 2,000 on the export side and there is no pivotal player in resolving the constraint. The ECI shall be determined as described in Figure 2-3 – CCT Function Overview Diagram (continued). The existence of a pivotal player shall be determined as described in Figure 2-2– CCT Function Overview Diagram. The daily CCT sub-process shall provide all components and steps described in Figure 2-6 – Daily CCT Overview Diagram. The input data to monthly CCT sub-process shall be provided as described in Table 2-1 – CCT Inputs. The sub-process shall provide all outputs as described in Table 2-2 CCT Outputs. A default value of 1/3 (which will be multiplied by the highest resource shift factor) is used as Shift Factor Thresholds in monthly CCT sub-process to: - Determine effective capacity needed on import side to resolve constraint - Determine effective capacity needed on export side to resolve the constraint ERCOT market operator shall be able to modify the Shift Factor Thresholds. ERCOT market operator shall be able to modify the ECI Thresholds. The daily CCT sub-process shall use the most recent results produced by annual CCT sub-process. |  |

# Use Cases

Use cases start when the actor does something; an actor always initiates use cases. The use case describes what the actor does and what the system does in response. It is phrased in the form of a dialog between the actor and the system. The use case describes what happens inside the system, but not how or why.

## Actor Catalog

| # | Actor |  | Description |  |
| --- | --- | --- | --- | --- |
| 1 | MMS |  | Input Data Interface |  |
| 2 | ERCOT |  | Posting of competitive constraints to MIS secure area |  |
| 3 | MMS |  | CCT Function |  |
| 4 | ERCOT Market Operator |  | Annual CCT |  |
| 5 |  | ERCOT Market Operator |  | Monthly CCT |
| 6 |  | ERCOT Market Operator |  | Daily CCT |

## Use Case Catalog

| Use Case ID | Name | Use Case Description |
| --- | --- | --- |
| CCT-UC1 | Input Data Interface | This use case describes the sub process of retrieving input data and storing in CCT data storage. |
| CCT-UC2 | Posting of competitive constraints to MIS secure area | This use case describes the sub process of posting the Board approved list of competitive constraints to MIS secure area. |
| CCT-UC3 | CCT function | This use case describes the sub process to: - Calculate Element Competitive Index (ECI) on both import and export side of each constraint in the CSCs and CREs list to determine whether sufficient competition exist in resolving the constraint. - Determine whether a pivotal player exists in resolving the constraint. |
| CCT-UC4 | Annual CCT | This use case describes the sub process to identify constraints in CSCs and CREs list as a competitive or non-competitive constraint for a year. |
| CCT-UC5 | Monthly CCT | This use case describes the sub process to update competitive and non-competitive constraints as determined by annual CCT for a particular month. |
| CCT-UC6 | Daily CCT | This use case describes the process to update competitive and non-competitive constraints as determined by monthly CCT for a particular day. |

## Protocol Coverage

| Protocol Sub-Section # (To the lowest level of granularity as possible) | Description | Coverage by Requirements (Full/Partial) |
| --- | --- | --- |
| 3.19 | Constraint Competitive Tests | Partial(FR1)(FR2)(FR3) |
| 3.19.1(1) | Annual Competitiveness Test | Full(FR3)(FR4) |
| 3.19.1(2) | Test Procedure 1 | Full(FR3) |
| 3.19.1(3) | Test Procedure 2 | Full(FR3) |
| 3.19.2 | Monthly Competitiveness Test | Partial(FR3)(FR5) |
| 3.19.3 | Daily Competitiveness Test | Partial(FR3)(FR6) |

## Sub-Process Coverage

| Sub-Process ID | Sub-Process Name | Coverage by Requirements (Full/Partial) |
| --- | --- | --- |
| SP1 | Input data interface | Full |
| SP2 | Posting competitive constraints to MIS secure area | Full |
| SP3 | CCT Function | Full |
| SP4 | Annual CCT | Full |
| SP5 | Monthly CCT | Full |
| SP6 | Daily CCT | Full |

# Appendix A - Clarification Notes on the CCT Test Procedures for Texas’s Nodal Market

During the TPTF review of the “Market Management Systems (MMS) Requirements Specification for Constraint Competitiveness Test (CCT)”, it was recommended that additional clarification be provided on the procedures and algorithms that will be used in the annual/monthly/daily CCT processes. This clarification note is in response to the TPTF recommendation.

This clarification note is organized as follows. The CCT procedures as defined in the Nodal Protocol (May 2006 version) are first described. Based on the Nodal Protocol defined procedures, an algorithmic description follows to provide further clarification of ERCOT proposed implementation approach to the CCT.

## Nodal Protocol Description of Constraint Competitiveness Test (Protocol reference: Section 3.19.1)

According to Texas Nodal Protocol Section 3.19.1, Constraint Competitiveness Test (CCT) is required to be performed on an annual, monthly, and daily basis. Although the CCT main function in all three types of CCT is same, but the network model and resource available capacities used for the three types of test could be different.

The following two test procedures are required to be performed in all three types of CCT:

Test Procedure 1 - Determine Element Competitiveness Index (ECI)

Test Procedure 2 - Determine whether a Pivotal Player exist

## Test Procedure 1 – Determine Element Competitiveness Index (ECI)

Test Procedure 1 consists of the following steps:

- Determine the effective capacity available to resolve the constraint on the import side.

  - Determine shift factors of all Electrical Buses relative to the import terminal of the constraint as the reference Electrical Bus.

  - Determine the effective Load on the export side by multiplying all Loads at Electrical Buses by the corresponding Electrical Bus shift factors identified in step 1.a. above.

  - Determine the effective capacity needed to meet Load and to supply power over the constraint on the export side by:

- Multiplying all Available Capacity at Electrical Buses by the corresponding shift factor from step 1a. above;

- Stacking the effective capacity in decreasing shift factor order; and then

- Selecting the sufficient effective capacity from the stack to meet the effective Load plus the flow limit on the constraint. These Resources are flagged as XUSED and they shall not be considered in determining effective Available Capacity to resolve the constraint on the import side.

  - Determine the absolute value of shift factors of all Electrical Buses relative to the export terminal of the constraint as the reference Electrical Bus; and

  - Determine the effective capacity available to resolve the constraint on the import side. The effective capacity is determined by summing, over all the resources that are not flagged as XUSED in the above step 1.c. and shift factors greater than one-third of the highest resource shift factor, the products of (A) Available Capacity for that Resource and (B) the shift factor of that Resource from the above step 1.d.

- Determine the effective capacity available to resolve the constraint on the export side.

  - Determine the absolute value of shift factors of all Electrical Buses relative to the export terminal of the constraint as the reference Electrical Bus.

  - Determine the effective Load on the import side by multiplying all Loads at Electrical Buses by the corresponding Electrical Bus shift factors from the above step 2.a.

  - Determine the effective capacity needed to meet Load less imported power over the constraint on the import side by:

- Multiplying all Available Capacity at Electrical Buses by the corresponding shift factor from the above step 2.a.;

- Stacking the effective capacity in decreasing shift factor order; and then

- Selecting the sufficient effective capacity from the stack to meet the effective Load minus the flow limit on the constraint. These Resources are flagged as IUSED and they are not considered in determining effective capacity available to resolve the constraint on the export side.

  - Determine the shift factors of all Electrical Buses relative to the import terminal of the constraint as the reference Electrical Bus.

  - Determine the effective capacity available to resolve the constraint on the export side. The effective capacity is determined by summing, over all the resources that are not flagged as ISUED and have shift factors greater than one-third of the highest resource shift factor, the products (A) the Available Capacity for that Resource and (B) the shift factor of that Resource from the above step 2.d.

- Determine the ECI’s on the import and the export side of the constraint.

  - Determine the total Managed Capacity by each Entity and its Affiliates on the import and export side that is available to resolve the constraint on the export and the import side respectively. Managed Capacity for an Entity is a Resource or portion of a Resource for which the Entity or its Affiliates has the decision-making authority over how the Resource or portion of the Resource is offered or scheduled (e.g., Output Schedules), either by virtue of ownership, agreement or otherwise. Each QSE shall submit annually a list of which Entity has that decision-making authority for each Resource or portion of a Resource the QSE represents. In addition, each QSE shall notify ERCOT of any known changes in that list no later than 1800 in the day prior to the date that the change takes effect. Each Resource Entity shall provide its QSE with the information necessary to comply with the foregoing requirements in a timely manner.

  - Determine the percentage of Managed Capacity by each Entity and its Affiliates on the import and export side respectively.

  - The ECI on the import side is equal to the sum of the square of the percentages of Managed Capacity by each Entity and its Affiliates on the import side.

  - The ECI on the export side is equal to the sum of the square of the percentages of Managed Capacity by each Entity and its Affiliates on the export side.

## Test Procedure 2 – Determine whether a Pivotal Player exists

If the constraint satisfies the test for sufficient competition as evaluated in terms of the ECI thresholds, then determine if there is a pivotal player in resolving the constraint in the manner described below:

If the constraint cannot be resolved by eliminating all Available Capacity on the import side, except Nuclear capacity and Minimum-energy amounts of Coal and Lignite capacity that is Managed Capacity by any one Entity and its Affiliates during peak Load conditions, then a pivotal player exists. A constraint satisfies the Test Procedure 2 if no Entity is a pivotal player.

## Algorithmic description of the CCT Procedure

The CCT procedure is described below from an algorithmic perspective to provide additional clarification.

## Definitions

- Resource available capacity

A resource’s available capacity is defined as the High Sustained Limit (HSL) of a Generation Resource, including switchable generation that is not on a planned outage for the CCT test month or day (except wind powered generation), expected on-peak wind generation output, and full import capability of the DC Tie lines. Resources that are on a planned outage, to be mothballed, or to be retired for the CCT test month or day are considered unavailable.

Available resources for the CCT are represented with set Resources and indexed by r. A resource’s available capacity is denoted withHSLr, the high sustained limit for resource r.

- Network model for the CCT

For Annual CCT: For each of the months corresponding to the annual CCT, the network model used for each month is based on the monthly peak case used to auction the on-peak CRRs in the annual CRR auction.

For Monthly CCT: The monthly peak case network used to auction the monthly on-peak CRRs and the most current Managed Capacity information shall be used to perform a monthly CCT for each competitive and non-competitive constraint as determined by the annual CCT sub-process for the particular month.

For Daily CCT: The DAM network model for the peak hour and the most current Managed Capacity information shall be used to perform a daily CCT for each competitive and non-competitive constraint resulting from monthly CCT sub-process for the particular day

- Power flow case for the CCT

With the power flow case determined for the CCT, electrical node loads are represented with the symboldi, representing load at electrical node i.

The total load, D, for the power flow case is:D=idi.

- Constraint shift factors

With the selected network model for each CCT test period (month or day), shift factors for each constraint are calculated using the DC power flow model. The shift factor for constraint c at electrical node i is denoted withSFc,i. Similarly, the shift factor for constraint c for resource r is denoted withSFc,r.

For constraint c, its export and import terminal nodes are denoted withf(c) andt(c). Its limit is denoted withLc.

## Test Procedure 1 – Determine ECI on the import terminal of the constraint

The ECI on the import side for constraint c is calculated following the steps below:

- Determine shift factors relative to the import terminal.

SFc,iImp−Ref		←		SFc,i−SFc,t(c)

SFc,rImp−Ref		←		SFc,r−SFc,t(c)

- Determine the effective load on the export side.

EffLodc,f(c)           ←            iSFImp−REfc,i×di

- Determine resources’ effective capacities.

EffCapc,r	        ←		SFImp−Refc,r×HSLr

- Stack the effective capacity (EffCapc,r) in decreasing shift factor order (SFImp−Refc,r). The re-ordered resource set is indexed with r.

- Select resources from the re-ordered resource stack to meet load. Assume that the top rf resources in the stack meet the following condition:

r=1rf−1HSLr×SFImp−Refc,rEffLodc,f(c)+Lc

And

r=1rfHSLr×SFImp−Refc,rEffLodc,f(c)+Lc

The top rf resources satisfying the above condition are denoted with set XUSED.

- Determine the resources not belonging to set XUSED.

Resources=Resources−XUSED, which is indexed by r.

- Determine absolute value of shift factors relative to the export terminal.

SFExp−Refc,i		←		|SFc,i−SFc,f(c)|

SFExp−Refc,r		←		|SFc,r−SFc,f(c)|

- Determine the effective capacity available to resolve the constraint on the import side.

Assume the maximum absolute value of the shift factors (SFExp−Refc,r)for all resources r is SFcIM−Max.

    - The effective capacity available to resolve the constraint on the import side by each Entity (who manages or has control over the resources) is calculated as follows:

SFExp−Refc,r≥13SFExp−RefcIM−Maxr∈en

where en is indexed over set Entities including all the entities.

- Compute ECI on the import side.

  - Total effective capacity on the import side:

TotalImportSideEffCap=enEffCapByEntityc,en

- The percentage of Managed Capacity by each Entity:

EffCapPercentgeByEntityc,en=EffCapByEntityc,enTotalImportSideEffCap×100

- ECI on the import side:

ECIcIM=enEffCapPercentgeByEntityc,en2

## Test Procedure 2 - Determine whether a Pivotal Player exists

The following steps (per Shams’ presentation) describe how to determine whether a Pivotal Player exists to resolve a constraint.

- At step h) of the “Test Procedure 1 – Determine ECI on the import terminal of the constraint” select the largest (EffCapByEntity), excluding Nuclear capacity and minimum energy amounts from coal and ignite capacity, from the stack.

- Re-order all other resources, excluding Nuclear capacity and minimum energy amounts from coal and ignite capacity and the largest (EffCapByEntity), in the whole system in decreasing shift factor order (SFExp−Refc,r). The re-ordered resource set is indexed with r.

- Calculate the total capacity of Nuclear capacity and minimum energy amounts from coal and ignite apacity:

n=1NHSLn=GBase

Where n is indexed over set of all Nuclear capacity and minimum energy amounts from coal and ignite capacity in the whole system.

- Select from the re-ordered stack resulted in step b) above enough capacity to meet load D.

r=1rf−1HSLr+GBase=D1 And D1+Gc,r=D

Gc,r = Partial capacity of the last capacity selected from the stack.

- Evaluate the impact on the constraint.

Impact=r=1rf−1(SFImp−Refc,r×HSLr)+SFImp−Refc,r× EMBED Equation.3  +n=1n=N(SFImp−Refc,r×HSLn) −i=1n(SFImp−Refc,i×di)

- If  Impact≤Lc,  no pivotal player exists.

## Test Procedure 1 – Determine ECI on the export terminal of the constraint

The ECI on the export side for constraint c is calculated following the steps below:

- Determine absolute value of shift factors relative to the export terminal.

SFExp−Refc,i		←		|SFc,i−SFc,f(c)|

SFExp−Refc,r		←		|SFc,r−SFc,f(c)|

- Determine the effective load on the import side.

EffLodc,t(c)           ←            iSFExp−Refc,i×di

- Determine resources’ effective capacities.

EffCapc,r	←		SFExp−Refc,r×HSLr

- Stack the effective capacity (EffCapc,r) in decreasing absolute shift factor order (SFExp−Refc,r). The re-ordered resource set is indexed with r.

- Select resources from the re-ordered resource stack to meet load. Assume that, for the top rf resources in the stack, the following condition is true:

r=1rf−1HSLr×SFExp−Refc,rEffLodc,t(c)−Lc

And

r=1rfHSLr×SFExp−Refc,rEffLodc,t(c)−Lc

The top rf resources satisfying the above condition are denoted with set IUSED.

- Determine the resources not belonging to set IUSED.

Resources=Resources−IUSED, which is indexed by r.

- Determine shift factors relative to the import terminal.

SFImp−Refc,i		←		SFc,i−SFc,t(c)

SFImp−Refc,r		←		SFc,r−SFc,t(c)

- Determine the effective capacity available to resolve the constraint on the export side.

Assume the maximum value of the shift factors (SFImp−Refc,r) for all resources ris SFcEX−Max.

- The effective capacity available to resolve the constraint on the export side by each Entity (who manages or has control over the resources) is calculated as follows:

SFImp−Refc,r≥13SFcEX−Maxr∈en

where en is indexed over set Entities including all the entities.

- Compute ECI on the export side.

  - Total effective capacity on the export side:

TotalExportSideEffCap=enEffCapByEntityc,en

- The percentage of Managed Capacity by each Entity:

EffCapPercentgeByEntityc,en=EffCapByEntityc,enTotalExportSideEffCap×100

- ECI on the export side:

ECIcEX=enEffCapPercentgeByEntityc,en2