| NPRR Number | 057 | NPRR Title | Posting Requirements Pursuant PUC Subst. R. 25.505 |
| --- | --- | --- | --- |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 3.2, Analysis of Long-Term Resource Adequacy 3.2.1, Calculation of Aggregate Generating Capacity 3.2.2, Long-Term Demand Forecasts 3.2.3, System Adequacy Report 3.2.4, Statement of Opportunities (New) 3.12, Load Forecasting 3.12.1, Mid-Term Load Forecast 3.12.2, Long-Term Load Forecast 3.13, Renewable Production Potential Forecasts 4.2.3, Posting Forecasted ERCOT System Conditions |  |
| Revision Description |  | This NPRR implements posting requirements outlined in new P.U.C. Subst. R. 25.505, Resource Adequacy in the Electric Reliability of Texas Power Region. |  |
| Reason for Revision |  | Compliance with PUCT Order in Project No. 31972, Rulemaking Concerning Resource Adequacy and Market Power in the Electric Reliability Council of Texas Power Region. This NPRR reflects only a first set of changes to implement the rule provisions adopted under P.U.C. Project No. 31972. Additional NPRRs will be proposed to implement other rule provisions resulting from P.U.C. Project No. 31972. |  |
| Reason for Revision (from Transition Plan Task Force (TPTF) Charter Scope) |  | (1) Revisions resulting from Commission orders; (2) Clarifications of Protocol language that do not change the intent or technical specifications of the Protocols; (3) Correction of technical errors or processes that are found to not be technically feasible; (4) Revisions to the Protocols necessary to implement the results of the value engineering analysis or to otherwise avoid severe cost impacts; or (5) Other (describe): |  |
| TPTF Review (Yes or No, and summary of conclusion) |  | Yes. On 4/03/07, TPTF voted to approve the draft NPRR for Posting Requirement Changes. There were three abstentions from the Consumer and Independent Generator Market Segments. All Market Segments were present for the vote. |  |
| Credit Implications (Yes or No, and summary of impact) |  | No. |  |
| Date Posted |  | April 9, 2007 |  |

| Sponsor |  |
| --- | --- |
| Name | Trip Doggett on behalf of TPTF |
| E-mail Address | tdoggett@ercot.com |
| Company | ERCOT |
| Company Address | 2705 West Lake Dr., Taylor, TX 76574 |
| Phone Number | (512) 248-6360 |
| Fax Number | (512) 248-3992 |

ERCOT/Market Segment Impacts and Benefits

| Assumptions | 1 | None |  |
| --- | --- | --- | --- |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | None |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | None |  |
| Additional Qualitative Information | 1 | Compliance with PUCT Order in Project No. 31972, Rulemaking Concerning Resource Adequacy and Market Power in the Electric Reliability Council of Texas Power Region |  |
| Other Comments | 1 |  |  |

Proposed Nodal Protocol Language Revision

3.2 	Analysis of Resource Adequacy

3.2.1	Calculation of Aggregate Resource Capacity

(1)	ERCOT shall use Outages in the Outage Scheduler and the Resource Status from the Current Operating Plan to calculate the aggregate capacity from Generating Resources and Load Resources projected to be available in the ERCOT Region and in Forecast Zones in ERCOT.  “Forecast Zones” have the same boundaries as the 2003 ERCOT Congestion Management Zones. Each Resource will be mapped to a Forecast Zone during the registration process.

(2)	On a rolling 36-month basis, ERCOT shall calculate the aggregate weekly Generating Resource capacity and Load Resource capacity in the ERCOT Region and the Forecast Zones projected to be available during the ERCOT Region peak Load hour of each week for the following 36 months, starting with the second week.

(3)	On a rolling hourly basis, ERCOT shall calculate the aggregate hourly Generating Resource capacity and Load Resource capacity in the ERCOT Region and Forecast Zones projected to be available during each hour for the following seven days.

(4)	Projections of Generating Resource capacity from WGRs shall be consistent with that capacity forecasted in Section 3.13,  Renewable Production Potential Forecasts.

3.2.2	Demand Forecasts

(1)	ERCOT shall develop and publish weekly on the MIS Secure Area peak Demand forecasts by Forecast Zone for each week, starting with the second week for the next 36 months using the 36-Month Load Forecast as described in Section 3.12, Load Forecasting.  During the development of this forecast, ERCOT may consult with QSEs, TSPs, and other Market Participants that may have knowledge of potential Load growth.

(2)	ERCOT may, at its discretion, publish on the MIS additional peak Demand analyses for periods beyond 36 months.

(3)	ERCOT shall develop and publish hourly on the MIS Secure Area peak Demand forecasts by Forecast Zone for each hour of the next seven days using the seven-Day Load Forecast as described in Section 3.12.

(4)	For purposes of Demand forecasting, ERCOT may choose to use the same forecast as that used for the Load forecast.

3.2.3	System Adequacy Reports

ERCOT shall publish system adequacy reports to assess the adequacy of Resources and Transmission Facilities to meet the projected Demand.  ERCOT shall provide reports on a system-wide basis and by Forecast Zone

(1)	ERCOT shall generate and post a “Medium-Term System Adequacy Report” on the MIS Secure Area.  ERCOT shall update the report daily using the latest aggregate Generating Resource capacity and Load Resource capacity. The data will be provided for each week, starting with the second week, of a rolling 36-month period. The Medium-Term System Adequacy Report will provide:

(a)	Generating Resource capacity at the time of forecasted weekly peak Demand;

(b)	Load Resource capacity at the time of the forecasted weekly peak Demand;

(c)	Weekly peak forecast Demand described in Section 3.2.2, Demand Forecasting;

(d)	Calculated system reserve, highlighting any deficiency hours that excludes Load Resource capacity;

(e)	Calculated system reserve, highlighting any deficiency hours that includes Load Resource capacity shown as a reduction in forecast Demand;

(f)	Ancillary Service requirements; and

(g)	Transmission constraints that have a high probability of being binding in SCED or DAM.

(2)	ERCOT shall generate and post a “Short-Term System Adequacy Report” on the MIS Secure Area.  ERCOT shall update this report hourly following updates to the seven-Day Load Forecast and on detection of a change to Resource Status that changes the availability of a Resource.  The Short-Term System Adequacy Report will provide:

(a)	For Generation Resources, the available on-line Resource capacity for each hour, using the Current Operating Plan for the first seven days;

(b)	For Load Resources, the available capacity for each hour using the COP;

(b)	Forecast Demand for each hour described in Section 3.2.2 Demand Forecasting;

(c)	Ancillary Service requirements for the Operating Day and subsequent days; and

(d)	Transmission constraints that have a high probability of being binding in SCED or DAM.

3.2.4	Statement of Opportunities

(1)	ERCOT shall annually publish a “Statement of Opportunities” report that provides a projection of the capability of existing and planned Generation Resources, Load Resources, and transmission facilities to reliably meet ERCOT’s projected needs.  A Statement of Opportunities report published in even-numbered years shall use a ten-year study horizon and be published by December 31 of those years.  A Statement of Opportunity report published in odd-numbered years shall use a five-year study horizon and be published on or around October 1 of those years.  ERCOT shall prescribe reporting requirements for generation entities and TSPs to report to ERCOT their plans for adding new facilities, upgrading existing facilities, and mothballing or retiring existing facilities. ERCOT also shall prescribe reporting requirements for Load entities to report to ERCOT their plans for adding new Load Resources or retiring existing Load Resources.

(2)	Prior to prescribing new reporting requirements for the development of the Statement of Opportunities, ERCOT shall use information already being provided by Market Participants if doing so is cost-effective.

3.12	Load Forecasting

ERCOT shall produce and use Load forecasts to serve operations and planning objectives.

(a)	ERCOT shall update and post hourly on the MIS Secure Area a “seven-Day Load Forecast” that generates forecasted hourly Load over the next 168 hours for each of the Weather Zones and for each of the Forecast Zones.

(b)	ERCOT shall update and post daily on the MIS Secure Area a “36-Month Load Forecast” that provides a daily minimum and maximum forecast for the next 36-months for each of the Weather Zones and for each of the Forecast Zones.

3.12.1	Seven-Day Load Forecast

(1)	ERCOT shall use the seven-Day Load Forecast to predict hourly Loads for the next 168 hours based on current weather forecast parameters within each Weather Zone.  Preparation for Day-Ahead Operations requires an accurate forecast of the Loads for which generation capacity must be secured.  The seven-Day Load Forecast must have a “self-training” mode that allows ERCOT to review historic Load data and provide the ability to retrain the seven-Day Load Forecast algorithm.

(2)	The inputs for the seven-Day Load Forecast are as follows:

(a)	Hourly forecasted weather parameters for the weather stations within the Weather Zones, which are updated at least once per hour; and

(b)	Training information based on historic hourly integrated Weather Zone Loads.

(3)	ERCOT shall review the forecast suggested by seven-Day Load Forecast and shall use its judgment, if necessary, to modify the result prior to implementation in the Ancillary Service capacity Monitor, DRUC, HRUC, and Resource adequacy reporting.

3.12.2	Thirty-six-Month Load Forecast

(1)	ERCOT shall use the 36-Month Load Forecast to predict daily minimum and maximum Loads for the next 36 months.  An accurate 36-Month Load Forecast is required to perform Outage Coordination, Resource adequacy reporting and other Operations analysis for the three years ahead.  The 36-Month Load Forecast must have a “self-training” mode that allows ERCOT to review historic Load data and provides the ability to retrain the  36-Month Load Forecast algorithm.

(2)	The inputs for the 36-Month Load Forecast are as follows:

(a)	Daily actual minimum and maximum hourly integrated Loads by Weather Zone; and

(b)	Training based on historic daily maximum and minimum hourly integrated Weather Zone Loads.

(3)	ERCOT shall review the forecast suggested by the 36-Month Load Forecast and shall use its judgment if necessary to modify the result before implementation and posting on the MIS Secure Area.

3.13	Renewable Production Potential Forecasts

(1)	ERCOT shall produce forecasts of Renewable Production Potential (RPP) for Wind-powered Generation Resources (WGR) to be used as an input into the Day-Ahead Reliability Unit Commitment (DRUC) and Hour-Ahead Reliability Unit Commitment (HRUC).  ERCOT shall produce the forecasts using information provided by WGR Entities, meteorological information, and SCADA.  WGR Entities shall install telemetry at their WGRs and transmit the ERCOT-specified site-specific meteorological information to ERCOT.  WGR Entities shall also provide detailed equipment status at the WGR facility as specified by ERCOT to support the RPP forecast.  ERCOT shall provide forecasts for each WGR to the QSEs representing WGRs.  QSEs shall use the ERCOT-provided forecasts for WGRs throughout the Day-Ahead and Operating Day for applicable markets and RUCs.  Similar requirements for solar power and run-of-the-river hydro must be developed as needed.

(2)	WGR Entities shall provide ERCOT and their respective QSEs with Long-Term Wind Power Forecast (LTWPF) profiles for each WGR having an aggregated rating larger than 10 MW at its point of interconnection with the transmission system.  The profiles must forecast the daily generation shape by hourly production of wind power Renewable Production Potential and the WGR Entities shall provide the profiles to ERCOT for each month on a rolling 36 month basis.

(3)	ERCOT shall develop cost-effective tools or services to forecast energy production from IRRs with technical assistance from QSEs scheduling Renewable Resources.  ERCOT shall use its best efforts to develop accurate and unbiased forecasts, as limited by the availability of relevant explanatory data.  ERCOT shall post on the MIS Secure Area objective criteria and thresholds for unbiased, accurate forecasts within five Business Days of change.

4.2.3	Posting Forecasted ERCOT System Conditions

No later than 0600 in the Day-Ahead, ERCOT shall post on the MIS Secure Area, and make available for download, the following information for the Operating Day:

(a)	The Network Operations Model topology that includes known transmission line and other Transmission Facilities Outages in the Common Information Model format for the minimum Load hour and the peak Load hour;

(b)	Weather assumptions used by ERCOT to forecast ERCOT System conditions and used in the Dynamic Rating Processor;

(c)	Any weather-related changes to the transmission contingency list;

(d)	ERCOT System, Weather Zone, and Load Zone Load forecasts for the next seven days, by hour, and a message on update indicating any changes to the forecasts by means of the Messaging System;

(e)	Load forecast distribution factors from which Market Participants can calculate Load at the Electrical Bus level by hour for the next seven days;

(f)	Load Profiles for non-IDR metered Customers;

(g)	Distribution Loss Factors and forecasted  ERCOT-wide Transmission Loss Factors, as described in Section 13.3, Distribution Losses and in Section 13.2, Transmission Losses,  for each Settlement Interval of the Operating Day;

(h)	A current list of all Settlement Points that may be used for market processes and transactions;

(i)	A mapping of Settlement Points to Electrical Buses in the Network Operations Model; and

(j)	A list of Transmission constraints that have a high probability of binding in the SCED or DAM.