Standard Interfaces Document

Texas Nodal <Project Name>

External Interfaces Specification

Version 0.94

DRAFT

02-May-07

Document Revisions

| Date | Version | Description | Author(s) |
| --- | --- | --- | --- |
| 12/19/2006 | 0.10 | First draft | Scott Neumann |
| 12/20/2006 | 0.11 | Revised security section | Nemat Sarnevesht |
| 12/21/2006 | 0.13 | Augmentation to descriptions of bidding interfaces. | Scott Neumann |
| 12/22/2006 | 0.14 | Added sections 2.4-2.10 to discuss other aspects of web services strategy. | Scott Neumann |
| 12/27/2006 | 0.15 | Miscellaneous corrections and clarifications. | Scott Neumann |
| 12/28/2006 | 0.16 | Miscellaneous changes to reflect internal review | Scott Neumann |
| 01/10/2007 | 0.17 | Miscellaneous changes to reflect internal review. Provided additional bid types. | Scott Neumann |
| 1/19/2007 | 0.18 | Changes to reflect additional interfaces | Scott Neumann |
| 1/22/2007 | 0.19 | Correction to XML example to reflect schema changes. | Scott Neumann |
| 2/9/2007 | 0.20 | Updates to reflect initial TPTF comments, addition of Bid IDs, addition of more interfaces and resolution of issues. | Scott Neumann |
| 2/16/2007 | 0.21 | Defined additional interfaces. Clarifications on data types. | Scott Neumann |
| 2/23/2007 | 0.22 | Defined additional interfaces. Changed transactionId to mrid. Added description of required elements for bidding. Revised model interfaces. | Scott Neumann |
| 2/27/2007 | 0.23 | Revisions to award structures. Defined additional interfaces. | Scott Neumann |
| 3/9/2007 | 0.24 | Changed mrid to mRID. Added text to section 2.3.4 to clarify use of transaction IDs. Corrected the BidSet figure in section 3.3. Added clarifications on the use of bid sets to section 3.3. | Scott Neumann |
| 3/16/2007 | 0.25 | Revised to reflect comments from market participants. Added tables to describe required fields for each bid. Changed tradeDate to tradingDate. Added curveStyle to PriceCurve to be used as an indicator of the type of curve. Added section on error handling. | Scott Neumann |
| 3/22/2007 | 0.26 | Added more interface definitions to section 4. Augmented description of CIM usage, particularily with respect to CIM curves and schedules. Eliminated functionally redundant interfaces from section 4. Modified interfaces in section 4 to leverage IrregularIntervalSchedules instead of RegularIntervalSchedules where appropriate. Added XML examples. | Scott Neumann |
| 3/23/2007 | 0.27 | Edits required for public release. | Scott Neumann |
| 3/26/2007 | 0.28 | Correction to notification list in section 5.3.1 | Scott Neumann |
| 3/27/2007 | 0.29 | Minor corrections to some examples. Corrected figure in section 5.3.6. | Scott Neumann |
| 3/28/2007 | 0.90 | Revisions to reflect decisions of TPTF API SubGroup meeting: - Redefined timestamps in section 2.3.2 - Removed units from curves and schedules - Miscellaneous corrections to XML examples - Added state diagram for bids to section 3.1 | Scott Neumann |
| 3/30/2007 | 0.91 | Corrected omission of two curves for incremental/decremental offers | Scott Neumann |
| 4/19/2007 | 0.92 | Revisions to address April 13 comments from market participants. Other corrections and updates: - General: Corrected verbs within document to be consistent with WSDL , specifically ‘change’ is used instead of ‘update’ - 7.2.2: Corrected noun to be OutageSchedule - Added Report interface to section 9, which is a WS wrapper for PR50007 - Extended description of the base64 encoding and compression in section 2.1.3. Added example in new appendix G. | Scott Neumann |
|  | 0.93 | - Added captions for tables and figures. - Added request option to section 4.3.2 - Corrected payload definition for section 7.2.1 - Corrected timestamps that still incorrectly used Zulu time - Clarified use of common header and payload in section 5.3. - Corrected sequence diagram for Outages in section 7. - Corrected valid intervals for OutputSchedule, DCTieSchedules and SelfSchedules - Revised ThreePartOffer to reflect change in MMS definition to allow an hourly FIP/FOP - Clarified data requirements in message ‘sub-structures’ on diagrams in section 3.1. - Changed section 3.3.8 to use HSL, LSL, HEL and LEL | Scott Neumann |
| 5/2/2007 | 0.94 | - Changed some section headings in section 3 to be consistent with the protocol terminology. - Changed AS types to be consistent with protocols (e.g. Reg-Up, Reg-Down, …) - Replaces term ‘bid’ with ‘bid/offer/trade/schedule’ where appropriate - Added notification for Startup/Shutdown Instructions - Added notification for AS Obligations - Defined TmSchedule in section 2.3.1 - Global change to use TmSchedule type for XML definitions - Changes BidPriceCurve to EnergyOfferCurve in 3.3.1 - Added combined cycle to 3.3.8, 3.3.13 and generally anywhere resource is specified - Changed BidPriceCurve to PriceCurve - Changes CapacitySchedule to ASSchedule in 3.3.5 - Made more complex ‘Error’ structure for bids/offers/trades/schedules - Return ERCOT current time in section 8.2.2 - Added notification for rejection of bids/offers/trades - Appendix H extended to identify open issues | Scott Neumann |

Document Approvals

| Date | Approved By | Approval Documented In (select) |
| --- | --- | --- |
|  | Michael Stanley Enterprise Architecture Manager | ___ Approval email on file ___ Signature |

The following table identifies section by section approvals. This recognizes that the document will evolve and need to be approved in phases during the course of the project.

| Date | Version | Sections Approved | Approved By |
| --- | --- | --- | --- |

Table of Contents

List of Figures	v

1.	Introduction	4

1.1.	Purpose	4

1.2.	Scope	4

1.3.	Definitions, Acronyms, and Abbreviations	5

1.4.	References	7

1.5.	Overview	7

1.6.	Program-level Standards	7

2.	Services Organization	9

2.1.	Common Message Structure	11

2.1.1.	Message Header Structure	11

2.1.2.	Request Message Structures	13

2.1.3.	Payload Structures	14

2.1.4.	Response Message Structures	16

2.2.	Common Security Implementation	16

2.2.1.	Secure the Transport layer	17

2.2.2.	Secure SOAP messages	18

2.3.	Modeling and Conventions	18

2.3.1.	Use of the IEC CIM	19

2.3.2.	Representation of Time	24

2.3.3.	Market Products	24

2.3.4.	Management and Use of Transaction IDs (mRIDs)	25

2.3.5.	Other Conventions	27

2.4.	Delivery Approach	28

2.5.	Technical Interoperability	28

2.6.	Service Level Agreements	29

2.7.	Auditing, Monitoring and Management	30

2.8.	Versioning	30

2.9.	Governance	31

2.10.	Web Service Configuration Standards	31

3.	Market Transaction Service	32

3.1.	Interfaces Provided	32

3.2.	Interfaces Required	35

3.3.	Message Specifications	36

3.3.1.	Three Part Supply Offer	38

3.3.2.	Self Arranged Ancillary Services	44

3.3.3.	Incremental and Decremental Offer Curves	47

3.3.4.	Ancillary Service Offer	51

3.3.5.	Ancillary Service Trade	53

3.3.6.	Capacity Trade	55

3.3.7.	Congestion Revenue Rights (CRR) Offer	57

3.3.8.	Current Operating Plan (COP)	61

3.3.9.	DC Tie Schedule	66

3.3.10.	DAM Energy Bid	70

3.3.11.	DAM Energy Only Offer	71

3.3.12.	Energy Trade	73

3.3.13.	Output Schedule	76

3.3.14.	PTP Obligation Bid	78

3.3.15.	Self Schedule	81

3.4.	Error Handling	82

3.5.	Usage and XML Examples	84

3.5.1.	Bid Set Submission	84

3.5.2.	Querying Bids, Offers, Trades and Schedules	88

3.5.3.	Updating Bids, Offers, Trades and Schedules	89

3.5.4.	Canceling Bids, Offers, Trades and Schedules	91

4.	Market Information	93

4.1.	Interfaces Provided	93

4.2.	Interfaces Required	94

4.3.	Message Specifications	94

4.3.1.	Get AwardSet	95

4.3.2.	Forecasted Load	97

4.3.3.	Real-Time System Load	98

4.3.4.	Market Totals	100

4.3.5.	Market LMPs and SPPs	102

4.3.6.	Market MCPCs	105

4.3.7.	Binding Constraints	107

4.3.8.	Active Contingencies in SCED	109

4.3.9.	Ancillary Service Schedule Obligations	111

4.3.10.	Dynamic Ratings	113

4.3.11.	Voltage Profiles	114

4.3.12.	Total Regulation	116

4.3.13.	Load Ratio Share	117

4.3.14.	Competitive Constraints	119

4.3.15.	Load Distribution Factors	120

4.3.16.	Shift Factors	121

4.3.17.	Customer Load Profile	122

4.3.18.	Aggregated Ancillary Service Offer Curves	124

4.3.19.	Ancillary Service System Plan	126

4.3.20.	Unit Availability	128

4.3.21.	Startup and Shutdown Instructions	130

4.3.22.	Total Ancillary Service Offers	131

4.3.23.	Total DAM Energy	133

4.3.24.	Derated CRRs	135

4.3.25.	Proxy Curves	136

4.3.26.	Mitigated Curves	137

5.	Notifications	139

5.1.	Interfaces Provided	139

5.2.	Interfaces Required	140

5.3.	Message Specifications	141

5.3.1.	Notices and Alerts	141

5.3.2.	Bid Set Acceptance	143

5.3.3.	Bid Set Errors	144

5.3.4.	Pending Trade	146

5.3.5.	Energy Offer Awards	146

5.3.6.	DAM Energy Only Offer Awards	147

5.3.7.	DAM Energy Bid Award	148

5.3.8.	Ancillary Service Awards	149

5.3.9.	CRR Awards	150

5.3.10.	PTP Obligation Awards	151

5.3.11.	Ancillary Service Obligations	152

5.3.12.	Outage Notifications	153

5.3.13.	Startup/Shutdown Instructions	154

6.	Acknowledgement of Alerts	156

6.1.	Interfaces Provided	156

6.2.	Interfaces Required	156

6.3.	Message Specifications	157

7.	Outage Scheduling Interfaces	158

7.1.	Interfaces Provided	158

7.2.	Message Specifications	158

7.2.1.	Outage Creation	158

7.2.2.	Outage Schedule Query	161

7.2.3.	Outage Cancellation	162

8.	Utility Interfaces	163

8.1.	Interfaces Provided	163

8.2.	Message Specifications	163

8.2.1.	Get mRID	163

8.2.2.	System Status	164

9.	Reports	165

9.1.	Interfaces Provided	165

9.2.	Message Specifications	165

9.2.1.	Get Reports	165

Appendix A: WS-Notifications	167

Appendix B: WSDL for Market Requests	173

Appendix C: XML Schemas for Message and Payload Definitions	176

Appendix E: Annotated SOAP Message	204

Appendix F: XSL Examples	208

Appendix G: Payload Compression Example	211

Appendix H: Issues	212

# List of Figures

Figure 1 - Web Service Interfaces	10

Figure 2 - Message Header Structure	12

Figure 3 - Request Message Structure	13

Figure 4 - Message Request Parameters	14

Figure 5 - Message Payload Container Structure	15

Figure 6 - Response Message Structure	16

Figure 7 - CIM Curves and Schedules	20

Figure 8 - Time Schedule Structure	21

Figure 9 - Required Keys for Market Products	25

Figure 10 - mRID Structures	27

Figure 11 - Example BidSet Sequence Diagram	33

Figure 12 - State Diagram for Bids, Trades, Offers and Schedules	35

Figure 13 -  Message: create BidSet	36

Figure 14 - Message: reply BidSet	36

Figure 15 - BidSet Header Structure	37

Figure 16 – Bid/Offer/Trade/Schedule Common Elements	38

Figure 17 – ThreePartOffer Structure	39

Figure 18 - PriceCurve Structure	40

Figure 19 - Fip/Fop, StartupCost and MinimumEnergy Structures	41

Figure 20 - ThreePartOffer Requirements	43

Figure 21 - SelfArrangedAS Structure	45

Figure 22 - CapacitySchedule Structure	46

Figure 23 - SelfArrangedAS Requirements	47

Figure 24 - IncDecOffer Structure	48

Figure 25 - IncDecOffer Requirements	49

Figure 26 - ASOffer Structure	52

Figure 27 - ASOffer Requirements	53

Figure 28 - ASTrade Structure	54

Figure 29 - ASTrade Requirements	55

Figure 30 - CapacityTrade Structure	56

Figure 31 - CapacityTrade Requirements	57

Figure 32 - CRR Structure	58

Figure 33 - MinimumReservationPrice Structure	58

Figure 34 - NOIEPeakLoadForecast Structure	59

Figure 35 - CRR Requirements	60

Figure 36 - COP Structure	62

Figure 37 - Limits Structure	63

Figure 38 - ASCapacity Structure	63

Figure 39 - COP Requirements	64

Figure 40 - DCTieSchedule Structure	67

Figure 41 - EnergySchedule Structure	68

Figure 42 - NERCTags Structure	68

Figure 43 - DCTieSchedule Requirements	69

Figure 44 - EnergyBid Structure	70

Figure 45 - EnergyBid Requirements	71

Figure 46 – EnergyOnlyOffer Structure	72

Figure 47 - EnergyOnlyOffer Requirements	73

Figure 48 - EnergyTrade Structure	74

Figure 49 - EnergySchedule Structure	75

Figure 50 - EnergyTrade Requirements	76

Figure 51 - OutputSchedule Structure	77

Figure 52 - OutputSchedule Requirements	78

Figure 53 - PTPObligation Requirements	79

Figure 54 - MaximumPrice Sub-Structure	79

Figure 55 - PTPObligation Requirements	80

Figure 56 - SelfSchedule Structure	81

Figure 57 - SelfStructure Requirements	82

Figure 58 - Error Handling	83

Figure 59 - create BidSet Request Parameters	84

Figure 60 - reply BidSet Parameters	84

Figure 61 - get BidSet Parameters	88

Figure 62 - reply BidSet Parameters	89

Figure 63 - change BidSet Parameters	90

Figure 64 - reply BidSet Parameters	90

Figure 65 - cancel BidSet Parameters	91

Figure 66 - reply BidSet Parameters	91

Figure 67 - Market Information Request Sequence Diagram	93

Figure 68 - Parameters for 'get' Requests	94

Figure 69 - Parameters for 'reply' Messages	94

Figure 70 - Get AwardSet Parameters	95

Figure 71 - Get AwardSet Response	95

Figure 72 - AwardSet Container Structure	96

Figure 73 - Generalized Award Structure	96

Figure 74 - LoadForecast Request Parameters	97

Figure 75 - LoadForecast Response Parameters	98

Figure 76 - Load Forecast Structure	98

Figure 77 - SystemLoad Request Parameters	99

Figure 78 - SystemLoad Response Parameters	99

Figure 79 - SystemLoad Structure	100

Figure 80 - Market Totals Request Parameters	101

Figure 81 - MarketTotals Response Parameters	101

Figure 82 - MarketTotals Structure	102

Figure 83 - LMP and SPP Request Parameters	103

Figure 84 - LMP and SPP Response Parameters	103

Figure 85 LMP Container Structure	104

Figure 86 - SPP Container Structure	105

Figure 87 - MCPC Container Structure	107

Figure 88 - BindingConstraint Container Structure	109

Figure 89 - SCEDViolatedConstraints Container Structure	111

Figure 90 - ASObligations Container Structure	113

Figure 91 - DynamicRatings Container Structure	114

Figure 92 - VoltageProfiles Container Structure	115

Figure 93 - TotalRegulation Structure	117

Figure 94 - LoadRatioShares Container Structure	118

Figure 95 - CompetitiveConstraints Container Structure	120

Figure 96 - LoadDistributionFactors Container Structure	121

Figure 97 - ShiftFactors Container Structure	122

Figure 98 - LoadProfile Container Structure	124

Figure 99 - AggregatedASOfferCurve Schedule	126

Figure 100 - ASSystemPlan Container Structure	127

Figure 101 - ASSchedule Structure	128

Figure 102 - UnitAvailabilities Container Structure	129

Figure 103 - StartupShutdownInstructions Container Structure	131

Figure 104 - ASOffers Structure	133

Figure 105 - TotalEnergy Structure	135

Figure 106 - DeratedCRRs Container Structure	136

Figure 107 - ProxyCurves Container Structure	137

Figure 108 - MitigatedCurves Container Structure	138

Figure 109 - Notification Sequence Diagram	140

Figure 110 - WS-Notitifications Message Structure	141

Figure 111 - Notification Structure	142

Figure 112 - Notice and Alert Types	143

Figure 113 - UnconfirmedTrades Container	146

Figure 114 - AwardedEnergyOffer Structure	147

Figure 115 - AwardedEnergyOnlyOffer Structure	148

Figure 116 - AwardedEnergyBid Structure	149

Figure 117 - AwardedAS Structure	150

Figure 118 - AwardedCRR Structure	151

Figure 119 - AwardedPTPObligation Structure	152

Figure 120 – Ancillary Service Obligation Structure	153

Figure 121 - Outage Structure	154

Figure 122 - StartupShutdownInstructions Structure	155

Figure 123 - Alert Acknowledgement Request	156

Figure 124 - Alert Acknowledgement Response	156

Figure 125 - Outage Scheduling Interface Sequence Diagram	158

Figure 126 - Outage Creation Request	159

Figure 127 - Outage Creation Response	159

Figure 128 - Outage Structure	161

Figure 129 - OutageSchedule Query Request	161

Figure 130 - OutageSchedule Query Response	161

Figure 131 - OutageSchedule Structure	161

Figure 132 - Outage Cancel Request	162

Figure 133 - Outage Cancel Response	162

Figure 134 - Get mRID Request	163

Figure 135 - Get mRID Response	164

Figure 136 - SystemStatus Request	164

Figure 137 - SystemStatus Response	164

Figure 138 - Reports Request	165

Figure 139 - Reports Response	166

Figure 140 - Reports Container	166

# Introduction

This document describes the message structure machine to machine interfaces for Market Participant applications that need to interact with ERCOT Nodal Market systems. The intended audience of this document are developers that will be integrating Market Participant applications to the ERCOT Nodal systems through the use of the interfaces described within this specification.

Where sections 1 and 2 of this document apply to all interfaces, sections 3 and beyond describe specific groupings of interfaces. The appendices provide XML Schemas, WSDLs and additional examples.

The interfaces and related interactions described by this document define the externally-visible (black box view) perspective of the services provided by this project. It is the intent of this specification and interface architecture to shield Market Participants from the details of systems integration internal to ERCOT.

Information contained within this document is subject to change as the design of the interfaces and designs of supporting systems are finalized.

## Purpose

The interfaces described by this document are intended to be used by Market Participants for machine to machine integration. This document is intended to provide all the details of the message structures and technical interoperability standards required for the machine to machine interface.

## Scope

The scope of this document is to describe web services provided for integration by Market Participants from the perspective of external integration. This document has program level scope as related to web services that would be used by Market Participants for machine to machine interaction with nodal applications as detailed in an agreed list of interfaces to be managed by the Nodal project. The intent of this design is to leverage the integration layer (IL) to expose web services needed for external integration by Market Participants.

The following are specifically outside the scope of this document:

- The details of integration from the IL to specific applications (e.g. MMS, EMS) is outside the scope of this specification and will be described in subsequent design documents

- Inter-Control Center Communications Protocol (ICCP) communications

- Interactions with application User Interfaces (UI)

- Access of information through MIS

This document is not intended as a replacement for the current services provided by the ERCOT used to access reports. The ‘get Reports’ interface described in section 9.2.1 provides a wrapper for this interface.

The specific list of interfaces to be included within the scope of this specification is currently in a draft under review.

## Definitions, Acronyms, and Abbreviations

| Term/Acronym | Definition |
| --- | --- |
| ADJ | Adjustment |
| AS | Ancillary Services, includes spinning reserve, non-spinning reserve, responsive reserve and regulation |
| award | An award identifies the acceptance of a bid or offer. |
| BidSet | A BidSet is a container for a collection of bids, offers, trades and schedules |
| CIM | Common Information Model, an IEC standard |
| CRR | Congestion Revenue Rights, a system implemented at ERCOT by Nexant that is responsible for CRR auctions |
| CSV | A file format that uses values separated by commas |
| DAM | Day Ahead Market |
| DRUC | Day-Ahead Reliability Unit Commitment |
| EMS | Energy Management System, a system implemented at ERCOT by Areva |
| FIP | Fuel Index Price |
| FOP | Fuel Oil Price |
| FTP | File Transfer Protocol |
| GUID | Globally Unique Identifier. GUIDs are usually implemented as random 128 bit numbers that are usually represented as a string of hexadecimal digits. |
| HASL | High ancillary services limit |
| HDL | High dispatch limit |
| HEL | High emergency limit |
| HRUC | Hour-Ahead Reliability Unit Commitment |
| HSL | High sustained limit |
| IEC | International Electrotechnical Commission |
| LASL | Low ancillary services limit |
| LDL | Low dispatch limit |
| LEL | Low emergency limit |
| LMP | Location marginal price |
| LSL | Low sustained limit |
| MCPC | Market clearing price for capacity |
| MIS | Market Information System, an umbrella for the various interfaces provided to Market Participants by ERCOT |
| MMS | Market Management System, a system implemented at ERCOT by ABB |
| MP | Market Participant |
| MW | Megawatt, a measure of power |
| MWh | Megawatt hour, a measure of energy |
| NMMS | Network Model Management System, a system implemented at ERCOT by Siemens |
| NOMCR | Network operating model change request |
| Non-Spin | Non-spinning reserve |
| OASIS | Organization for the Advancement of Structured Information Systems |
| Operating Date | Synonymous with Trade Date |
| PMCR | Planning Model Change Request |
| POC | Proof Of Concept |
| QSE | Qualified Scheduling Entity |
| RDF | Resource Descriptor format. An XML format used by NMMS for model exchanges using IEC 61970-501. |
| Reg-Down | Regulation down |
| Reg-Up | Regulation up |
| RRS | Responsive reserve service |
| RTM | Real-Time Market |
| SAMR | Special Action Model Request |
| sink | Sink settlement point |
| SOAP | Simple Object Access Protocol |
| SoSA | System of Systems Architecture |
| source | Within a bid, offer, schedule, trade or award this refers to the source settlement point. Within a message header, this refers to the ID of the market participant. |
| SP | Settlement Point |
| SPP | Settlement Point Price |
| TP | Transmission Provider |
| Trading Date | Synonymous with Operating Date. Represented in XML using the ‘tradingDate’ tag. |
| UTC | Universal Coordinated Time, an international standard |
| WS | Web Services. There are many web service standards that are commonly prefixed by ‘WS’. |
| WSDL | Web Services Definition Language |
| XML | eXtensible Markup Language |
| XSD | XML Schema, used to define the structure of XML documents |
| XSL | XML Stylesheet Language |
| Z | Zulu, an indicator for the use of GMT or UTC time |

## References

| Artifact | Definition |
| --- | --- |
| External Interfaces Conceptual Design | Conceptual design for external interfaces using web services |
| External Interfaces Security Design Specification | Detailed security design for external interfaces. This is a companion document to the External Interfaces Conceptual design. |
| OASIS WS-Notifications | OASIS Web Services Base Notification standard |
| OASIS WS-Security | OASIS Web Services Security |

Additional references to related standards are described in section 1.6.

## Overview

This document focuses on the external interface design and related interface definitions from all perspectives except for security, which is described in detail in a companion document. The interfaces are to be provided using web services, where a rationale is provided in subsequent sections. The web services defined by this document will support a wide variety of machine to machine information exchanges.

## Program-level Standards

In general, the design described by this document will leverage web services and related security standards as defined by the World-Wide Web Consortium (W3C) and OASIS. Program-level standards include those related to security, as well as basic web service standards including:

- XML

- XML Schema

- XPath

- XSL

- SOAP

- Web Services

- WSDL

- RDF

These are described in the companion security design document. W3C standards can be freely accessed from http://www.w3.org.

Another key program standard is the IEC Common Information Model (CIM), as defined by IEC 61970-301. This is used to define models used by ERCOT, which are exchanged using IEC 61970-501. It will also be leveraged by this design for the definition of messages used for interfaces. There is also a standard for message structures defined by IEC 61968-1. There standards can be purchased from the IEC web site at http://www.iec.ch. Materials related to IEC standards, including the CIM model it self can be freely obtained from the UCA International Users Group sharepoint at http://sharepoint.ucausers.group.org/CIM.

There are several key Internet Engineering Task Force references. These include:

- Internet Engineering Task Force RFC 2828: Internet Security Glossary

- Internet Engineering Task Force RFC 2119: Key words to indicate RFC requirement levels

- Internet Engineering Task Force RFC 2246: Transport Layer Security (TLS)

- Internet Engineering Task Force RFC 3275: XML Digital Signature and Processing

IETF documents can be freely obtained from http://www.ietf.org.

The OASIS WS-BaseNotifications standard is used to define the mechanism for issuance of asynchronous notification messages to Market Participants. Given the application of WS-Notifications for Nodal, only a subset of the capabilities of WS-Notifications are required. OASIS standards being leveraged include:

- Web Services Notifications (WS-Notifications)

- Web Services Security (WS-Security)

- Web Services Security X.509 Token Profile

- Web Services Security Username Token Profile

OASIS standards can be freely obtained from http://www.oasis-open.org.

The definition of timestamps is specified by ISO-8601, with the exception that timestamps of 24:00:00 are not used for compatibility reasons. There are some implementations of timestamps within software products that do not correctly handle timestamps of 24:00:00.

ISO standards can be purchased online from a variety of sources including ANSI, at http://www.ansi.org. Descriptions of ISO-8601 can be freely obtained from other sources including wikipedia

# Services Organization

The services described by this document are defined using a combination of Web Services Definition Language (WSDL) and XML Schema. The WSDLs are organized as follows:

- One or more WSDLs defined by ERCOT, defining operations related to synchronous request/reply web service messages

- WSDL defined by OASIS for WS-Notifications to provide support for asynchronous messaging, using web services

In both of the above cases, one or more XML Schemas (XSD) are used to define the structure of message payloads.

Example WSDL and XSD are provided in the appendices. It is anticipated that these would be key design artifacts for developers. The following diagram provides an overview of the interfaces described by this document.

Figure 1 - Web Service Interfaces

## Common Message Structure

Unless otherwise specified, all messages use a common message envelope, where a predefined structure is used for requests and another structure is used for responses. This structure is based upon the IEC 61968-1 standard. Messages are constructed with several sections, including:

- Header: required for all messages, using a common structure for all service interfaces

- Request: optional, defining parameters needed to qualify request messages

- Reply: Used for response messages to indicate success, failure and error details

- Payload: optional, used to convey message information as a consequence of the ‘verb’ and ‘noun’ in the message Header. The payload structure provides options for payload compression.

### Message Header Structure

Common to both the request and response messages is a header structure. The header has several required fields that must be populated, these include:

- Verb, to identify a specific action to be taken. There are an enumerated set of valid verbs, where commonly used values include ‘get’, ‘create’, ‘change’, ‘cancel’, ‘close’ and ‘reply’.

- Noun: to identify the subject of the action and/or the type of the payload (e.g. BidSet, Notification) if a payload is provided.

- Source: identifying the source of the message, which should be the ID of the Market Participant or ERCOT (typically for reply messages)

- Revision: To indicate the revision of the message definition. This should be ‘1’ by default.

- Nonce: A unique number that would not be repeated by the Market Participant within the period of at least a day. This could be a sequence number, large random number or a GUID. This is defined by WS-Security. A combination of this number and the timestamp make the message unique for a given time period.

- Created: A timestamp to indicate when the message was created. This value and the Nonce are used to protect against replay attacks. This is defined by WS-Security.

The following diagram describes the header structure used for request and response messages.

Figure 2 - Message Header Structure

There are several optional fields that may be populated. In the above diagram, the optional items are represented using dashed borders. If the MessageID is populated on a request, it will be returned on the reply. The Comment field is never used for any processing-related logic. The UserID may be used to indicate the person responsible for initiating a transaction, and will be logged as appropriate, but verification is the responsibility of the Source system.

### Request Message Structures

The following diagram describes the structure of a request message that would be used in conjunction with a WSDL operation.

Figure 3 - Request Message Structure

The RequestMessage can also optionally contain a package with parameters relevant to the request, called Request. It is likely that different or variant Request packages may be defined to be used in conjunction with messages for a specific web service operation. In those cases, the corresponding WSDL and XSD would identify the optional parameters. The description of the interface (in subsequent sections of this document) would identify the usage of those parameters. The following is an example RequestType used in the definition of a Request package that defines some common parameters used for requests, however it is important to note that these are typically application specific. These parameters are most commonly used in conjunction with ‘get’ requests as qualifiers.

Figure 4 - Message Request Parameters

One key use of the RequestType is to avoid the placement of application specific request parameters in the header or within payload definitions. Also, where a set of requests that were supported by a specific web service operation had significantly different requirements for information in the RequestType, it could justify the use of RequestType variants, where each variant will be used for the definition of messages for the specific web service operation.

### Payload Structures

There are some requests where a Payload must be provided, as would be the case for a message with a verb of ‘create’ or ‘change’. Payloads are typically XML documents that conform to a defined XML schema. However, there are exceptions to this rule. Some XML payloads may not have useful XML schemas, as in the case of RDF files or dynamic query results, as well as non-XML formats such as CSV and PDF. There may also be cases where a large payload must be compressed, in the event that it would become very large and otherwise consume significant network bandwidth.  In order to accommodate a variety of payload format options the following payload structure is used.

Figure 5 - Message Payload Container Structure

In the previous diagram, any type of XML document may be included, using the XML ‘any’ structure. While this provides options for loose-coupling, specific complex types defined by XML schemas (XSDs) can be used as well. The WSDL in the appendix provides an example of this case.

Payloads can also be supplied as XML encoded strings using the ‘Document’ tag, although this method is less preferred than used of the XML ‘any’.

There are also some cases where a zipped, base64 encoded string is necessary, and would be passed using the ‘Compressed’ tag. The Gnu Zip compression shall be used in order to provide compatibility within both Java and Microsoft .Net implementations. A Java example is provided in Appendix G. Specific examples of the usage of payload compression would be where:

- An XML payload, conforming to a recognized XML schema exceeds a predefined size (e.g. 1MB). This would be very common for large Market Participant sets of bids, offers, trades or schedules

- A payload has a non-XML format, such as PDF, Excel spreadsheet, CSV file or binary image

- A payload is XML, but has no XML schema and exceeds a predefined size, as would be the case of a dynamic query that would return an XML result set

When a payload is compressed and base64 encoded, it is stored within the Payload/Compressed message element as a string.

The format tag can be used to identify specific data formats, such as XML, RDF, PDF, DOC, CSV, etc. This is especially useful if the payload is compressed. The use of this tag is optional, and would typically only be used when the payload is stored using the Payload/Compressed message element.

The above options provide an alternative to the use of SOAP attachments. SOAP attachments are more difficult to secure since the SOAP envelope signature signs the SOAP body but does not sign the attachment. This also requires that the payload is processed separately from the rest of the SOAP message (e.g. the message is parsed to extract the payload, and then the payload is parsed and processed). However, we believe this implementation approach is less complex than using SOAP attachments.

### Response Message Structures

The following diagram describes the structure of a response message that would be used in conjunction with a WSDL operation, as a response to the request message.

Figure 6 - Response Message Structure

The ReplyCode would be set to OK to indicate that the request was successful, otherwise it would be set to ERROR, and one or more Error elements would be provided to describe the error(s). There may also be more specific error information provided within the payload, as in the case of bids within a BidSet container.

If the MessageID was set in the Header for the RequestMessage, the value will be returned in the Header of the ResponseMessage.

## Common Security Implementation

This section will provide an overview of security from the perspective of implementation requirements for Market Participants. Market Participants MUST take two basic steps in securing their Web Services Interaction with ERCOT:

- Secure the Transport layer

- Secure SOAP messages

### Secure the Transport layer

The transport layer is secured by deploying Secure Socket Layer (SSL) and Transport Layer Security (TLS) following these steps:

- Obtain client side certificate (these Certificates are issued by Verisign under the ERCOT brand).

- Implement mutual authentication (explained below).

- Ensure minimum SSL/TLS security settings

Note that TLS is an enhanced specification based on SSL. References to SSL refer to both SSL and TLS.

SSL is a standard mechanism for Web services that is available on virtually all application servers. This widely used, mature technology, which secures the communication channel between client and server, will satisfy all of ERCOT’s use cases for secure Web Service communications. Since it works at the transport layer, SSL covers all information passed in the channel as part of a message exchange between a client and a server, including attachments. Authentication is an important aspect of establishing an HTTPS connection.  Many platforms support the following authentication mechanisms for Web Services using HTTPS:

- The server authenticates itself to clients with SSL and makes its certificate available.

- The client uses basic authentication over an SSL channel.

- Mutual authentication with SSL, using the server certificate as well as the client certificate, so that both parties can authenticate to each other.

With Web Services, the interaction use case is usually machine to machine; that is, it is an interaction between two application components with no human involvement. Machine-to-machine interactions have a different trust model from typical website interactions. In a machine-to-machine interaction, trust must be established proactively, since there can be no real-time interaction with a user about whether to trust a certificate. Ordinarily, when a user interacts with a website via a browser and the browser does not have the certificate for the site, the user is prompted about whether to trust the certificate. The user can accept or reject the certificate at that moment. With Web Services, the individuals involved in the deployment of the Web Service interaction must distribute and exchange the server certificate, and the client certificate ( for mutual authentication), prior to the interaction occurrence.

The combination of the two settings—CONFIDENTIAL for transport guarantee and CLIENT-CERT for auth-method—enables mutual authentication. When set to these values, the containers for the client and the target service both provide digital certificates sufficient to authenticate each other. (These digital certificates contain client-specific identifying information.)

### Secure SOAP messages

Besides creating a secure communication channel between a client and a Web Service, ERCOT Web Service message exchanges require that security information be embedded within the SOAP message itself. This is often the case when a message needs to be processed by several intermediary nodes before it reaches the target service or when a message must be passed among several services to be processed.

Message-level security is very useful in XML document-centric applications, since different sections of the XML document may have different security requirements or be intended for different users.

SOAP messages’ signing is done through the following:

- Obtain application/system signing certificate.  (These certificates are issued by Verisign under the ERCOT brand).

- Sign all SOAP messages, using Web Services Security Standards and its X.509 Certificate Token Profile (see J2EE and .NET code snippets in Appendix D)

- Message headers MUST include a timestamp and a nonce

- Validate all SOAP messages have:

  - Signature

  - Certificates

  - Revocation status of certificates

  - Use of timestamp and nonce (to prevent replay attacks)

Appendix D provides examples for the generation of signatures. Appendix E provides an annotated example of a SOAP message.

## Modeling and Conventions

There are several conventions that are used for definitions, data items and information models.  Note that additional values and conventions will be defined as market requirements are finalized.

### Use of the IEC CIM

Where possible the IEC CIM should be leveraged. The ERCOT Nodal project and related systems use the CIM as a key standard. One example of this is for the submission and management of models through the NMMS system. Examples of leveraging the CIM for external web services include:

- The use of data structures defined by the IEC CIM where appropriate in payload definitions

- CIM naming conventions are used wherever possible, e.g. ClassName, propertyName

- The properties ‘startTime’ and ‘endTime’ are typically used to identify time intervals, as they are also used within many CIM classes. Instead of using combinations of start date, start hour and potentially an interval number (e.g. to represent 15 minute intervals), absolute times should be specified.

- The property ‘mRID’ is used to uniquely identify objects such as bids, offers, trades, awards and schedules

- A trade or operating date is always derived from the startTime of a bid, offer, trade or schedule.

Where interfaces have references to resources, QSEs, settlement points and electrical buses, the  ‘name’ property is used for the reference, as opposed to ‘mrid’. This is to maximize legibility and to provide for consistency with underlying market applications. Consequentially, simplified XML is used, where the use of the ‘name’ property is implied as opposed to explicit use of a <name> tag.

The following class diagram describes the structure of CIM Curves, IrregularIntervalSchedules and RegularIntervalSchedules. These classes are CIM building blocks, where there are many other classes in the CIM that inherit from these three basic types of curves and schedules (e.g. PriceCurve).

Figure 7 - CIM Curves and Schedules

For use within ERCOT Nodal, customizations and specializations of CIM curves and schedules are defined. One example is that units (e.g. xUnit, y1Unit, value1Unit) are not used.

For Curves:

- A curveStyle can be specified to identify the type of curve (e.g. CURVE, FIXED, VARIABLE) for interpretation of the curve by an application:

- A commonly used curve is a PriceCurve, where the ‘xvalue’ is used for MW and the ‘y1value’ is used for $/MWh. Not all cusrve styles are allowed for each product type. The interpretation of curveStyle is as follows:

  - FIXED: only one point is provided, where the intent is that the bid or offer is for all or nothing (i.e. either ‘xvalue’ or 0)

  - VARIABLE: only one point is provided, but the intent is that any value in the range 0 to ‘Xvalue’ can be taken

  - CURVE: Up to 11 points (to define up to 10 segments) can be provided

- Curves are often used where values are NOT plotted against time (e.g. MW vs. $/MWh)

The following is an example of a PriceCurve, that is derived from a CIM Curve:

<PriceCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>60</xvalue>

<y1value>40.00</y1value>

</CurveData>

<CurveData>

<xvalue>80</xvalue>

<y1value>45.00</y1value>

</CurveData>

<CurveData>

<xvalue>90</xvalue>

<y1value>47.00</y1value>

</CurveData>

</PriceCurve>

In order to better meet the needs of ERCOT Market Participants for interfaces related to bidding, an alternative to an IrregularIntervalSchedule is defined. This is called a ‘TmSchedule’. A TmSchedule has one or more ‘TmPoints’, where one or more values can be specified at each point in time. This is illustrated by the following diagram:

Figure 8 - Time Schedule Structure

The following is an XML example of an EnergySchedule, which is based upon a TmSchedule:

<msg:EnergySchedule>

<msg:startTime>2007-10-17T00:00:00-05:00</msg:startTime>

<msg:endTime>2007-10-17T24:00:00-06:00</msg:endTime>

<msg:TmPoint>

<msg:time>2007-10-17T00:00:00-05:00</msg:time>

<msg:value1>120</msg:value1>

</msg:TmPoint>

<msg:TmPoint>

<msg:time>2007-10-17T10:00:00-06:00</msg:time>

<msg:value1>130</msg:value1>

</msg:TmPoint>

<msg:TmPoint>

<msg:time>2007-10-17T16:00:00-06:00</msg:time>

<msg:value1>115</msg:value1>

</msg:TmPoint>

</msg:EnergySchedule>

It is important to note that a TmPoint identifies the start of an interval. The following diagram provides a graphic depiction of the XML example.

The optional ‘ending’ tag can be used to identify the end time for a specific TmPoint. This allows a schedule to have gaps. This is demonstrated by the following example.

<msg:EnergySchedule>

<msg:startTime>2007-10-17T00:00:00-05:00</msg:startTime>

<msg:endTime>2007-10-17T24:00:00-06:00</msg:endTime>

<msg:TmPoint>

<msg:time>2007-10-17T00:00:00-05:00</msg:time>

<msg:ending>2007-10-17T05:00:00-05:00</msg:ending>

<msg:value1>120</msg:value1>

</msg:TmPoint>

<msg:TmPoint>

<msg:time>2007-10-17T10:00:00-06:00</msg:time>

<msg:value1>130</msg:value1>

</msg:TmPoint>

<msg:TmPoint>

<msg:time>2007-10-17T16:00:00-06:00</msg:time>

<msg:value1>115</msg:value1>

</msg:TmPoint>

</msg:EnergySchedule>

The following diagram provides a graphic depiction of the previous XML example.

### Representation of Time

The ISO 8601 standard is used to define the representations of time values that are conveyed through interfaces. This avoids issues related to time zones and daylight savings time changes.

- For timestamps in messages published by ERCOT would use Texas prevailing time, using the following format: 2007-03-27T14:00:00-05:00 (as time changes from CDT to CST, the -05:00 would change to -06:00)

- Timestamps in messages sent to ERCOT by market participants could use any ISO 8601 compliant timestamp

It is extremely important to note that the use of ISO 8601 timestamps within message definitions for the external interfaces defined by this document in no way constrains other representations of time that may include:

- User interfaces, where local time or market hours may be used as desired

- Reports, where reports would be generated using an appropriate local time

- Internal integration, where an application may internally require some other time structure

In some cases it may be desirable to convert between a timestamp and a local time or a market hour. This can be readily accomplished using software functions, XSL and/or XPath expressions. Examples of time conversion using XSL are provided in Appendix F. Most development environments have a wide variety of APIs that can also be used for necessary time conversions.

In cases where there are a sequence of startTime and endTime intervals provided (as in the cases of ResourceStatus or MinimumEnergy structures as examples), overlapping time intervals are not allowed and will be flagged as an error. Typically in these cases the endTime for one interval should identically match the startTime for the next interval.

### Market Products

The bidding interfaces described in this document use a number of different market products, as well as several types of schedules. The following table identifies each product or schedule type, along with the set of message fields (aside from the market participant ID of the submitting QSE, operating date and operating hour) that are used to distinctly identify a bid, trade, offer or schedule. In effect the primary key for each product type is formed by the QSE ID, operating date, operating hour and items described in the following table.

| Product or Schedule Type | resource | asType | sp | tradeId | bidId | crrId | source | sink | buyer | seller | expiration | NERC tag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASOffer | X | X |  |  |  |  |  |  |  |  | opt |  |
| ASTrade |  | X |  | opt |  |  |  |  | X | X |  |  |
| CapacityTrade |  |  |  | opt |  |  |  |  | X | X |  |  |
| COP | X |  |  |  |  |  |  |  |  |  |  |  |
| CRR |  |  |  |  |  | X | X | X |  |  |  |  |
| DCTieSchedule |  |  | X |  |  |  |  |  | X | X |  | X |
| EnergyBid |  |  | X |  | X |  |  |  |  |  | opt |  |
| EnergyOnlyOffer |  |  | X |  | X |  |  |  |  |  | opt |  |
| EnergyTrade |  |  | X | opt |  |  |  |  | X | X |  |  |
| IncDecOffer | X |  |  |  |  |  |  |  |  |  | opt |  |
| OutputSchedule | X |  |  |  |  |  |  |  |  |  |  |  |
| PTPObligation |  |  |  |  | X |  | X | X |  |  |  |  |
| SelfArrangedAS |  | X |  |  |  |  |  |  |  |  |  |  |
| SelfSchedule |  |  |  |  |  |  | X | X |  |  |  |  |
| ThreePartOffer | X |  |  |  |  |  |  |  |  |  | opt |  |

Figure 9 - Required Keys for Market Products

The items marked with an ‘X’ are required key fields needed for submission. The use of ‘tradeId’s and ‘expiration’ times are optional. The operating date for each bid, trade, offer or schedule is derived from the startTime provided by the bid, offer, trade or schedule.

The XML schema provided to describe product types has all fields optional. This is because the schemas can be used for create, get and cancel operations. A bid, offer, trade or schedule can be referenced for a ‘get’ or ‘cancel’ request by specifying the transaction ID (mRID) of the desired bids, offers, trades or schedules (using the mRID that was returned by the reply to the submission request).

In each section related to market products, a table is provided that describes data requirements. Fields that are used as ‘keys’ to identify a bid/offer/trade/schedule are identified by ‘K’. Required fields are identified by ‘Y’, option fields are identified by ‘N’. There are some exception cases where identified key fields are identified as not being required (e.g. expiration), and are notated as ‘K, N’.

### Management and Use of Transaction IDs (mRIDs)

In order to uniquely identify a bid, offer or trade, a transaction ID is allocated by ERCOT. The name of the tag used for the transaction ID is ‘mRID’, as this conforms with the IEC CIM. This transaction ID is returned immediately in response to the submission of a bid set, where each bid, offer or trade within the bid set is given its own transaction ID. If a set of N valid unique bids are submitted using a bid set, there will be N transaction IDs supplied, each corresponding to the order of the bid/offer/trade/schedule on the input bid set. This transaction ID (mRID) may be used for the following purposes:

- To query the status of a specific transaction

- To cancel a specific transaction

- To relate an award to a bid

It is important to note that an update or change to a transaction does not use an mRID. To update a transaction it is simply resubmitted, where it will overwrite the previous submission of the bid. The resulting mRID for both the original submission and the revised submission will be the same.

A transaction ID (mRID) is a string that takes the following form:

<QSE ID>.<date>.<key string>

Within this, the following substrings are defined:

- QSE ID: ID of the market participant

- Date in format YYYYMMDD

- Key string, a derived string that includes required key fields for each market product

Examples of transaction ID (mRID) are as follows:

TXU.20081112.COP.UnitABC

REL.20081112.TPO.UnitXYZ.

When referring to a specific hour of a bid, the hour may be appended to the transaction ID, resulting in a string of the following form:

<QSE ID>.<date>.<key string>.<hour>

Examples of transaction ID as would be used for hourly awards are as follows:

REL.20081112.TPO.UnitXYZ..1

REL.20081112.TPO.UnitXYZ..2

REL.20081112.TPO.UnitXYZ..2R   (example for change to CST)

REL.20081112.TPO.UnitXYZ..3

REL.20081112.TPO.UnitXYZ..4

On a day with 25 hours, the mRID the represents the ‘repeated’ hour is suffixed with an ‘R’, e.g. ‘REL.20081112.TPO.UnitXYZ..2R’. As can be seen in the previous examples, there may be ‘null’ fields within the mRID string where an optional mRID field (e.g. expiration) is not used, resulting in consecutive ‘.’s.

Within the definitions of bids, trades, offers and awards, the transaction ID is conveyed using the ‘mRID’ property. This is for consistency with the IEC CIM. The following table describes the initial scheme to be used for each product type.

| Product or Schedule Type | Key string composition |
| --- | --- |
| ASOffer | ASO.resource.asType.expiration |
| ASTrade | AST.asType.tradeId.buyer.seller |
| CapacityTrade | CT.tradeId.buyer.seller |
| COP | COP.resource |
| CRR | CRR.crrId.source.sink |
| DCTieSchedule | DCT.sp.buyer.seller.NERCtag |
| EnergyBid | EB.sp.bidId.expiration |
| EnergyOnlyOffer | EOO. sp.bidId.expiration |
| EnergyTrade | ET.sp.tradeId.buyer.seller |
| IncDecOffer | IDO.resource.expiration |
| OutputSchedule | OS.resource |
| PTPObligation | PTP.bidid.source.sink |
| SelfArrangedAS | SAA.asType |
| SelfSchedule | SS.source.sink |
| ThreePartOffer | TPO.resource.expiration |

Figure 10 - mRID Structures

An important consideration has to do with the use of the interfaces described in this document in conjunction with the submission of bids by a user using a web-based user interface. The web-based user interface for bidding has no notion of the transaction ID itself, just the individual values used to construct the key string.

Another important note is that mRID values are should not be used for purposes of non-repudiation. It is the intent of the signature of the SOAP message, as referenced in section 2.2.2 to provide mechanisms that can be leveraged for non-repudiation.

Note: The structure of the transaction ID (mRID) is subject to change in the future. The ‘get mRID’ interface described in section 8.2.1 provides a mechanism to generate an mRID given a set of ‘key’ values.

### Other Conventions

The following are other conventions that must be followed by this specification:

- Within XML definitions, tags should be namespace qualified. For example, a tag of <tag> should be prefixed by a specific namespace reference, e.g. <ns:tag>. This will help to eliminate ambiguity. (Note that many examples in this document are not namespace qualified for brevity and to aid legibility)

- Units for power quantities are in megawatts (MW).

- Units for capacity quantities are in megawatts (MW).

- Units for energy quantities are in megawatt-hours (MWh).

- Units for energy prices are in $/MWh.

- Valid market types include: DAM, RTM, DRUC, HRUC, SCED, SASM, CRR and ADJ. Note that these are not currently required by bidding interfaces

- Trading dates are specified using YYYY-MM-DD, which indicates the operating day

- Valid ancillary service types offered as products include: Reg-Up, Reg-Down, Non-Spin,  RRS

- Settlement point names are the same as the name of the associated electrical bus, if defined at a bus level. They may also be defined at other levels (e.g. hubs, load zones).

## Delivery Approach

In advance of a Nodal go live and in accordance with agreed schedule and dependencies, ERCOT will provide the following:

- Interface specifications for web services

- Design artifacts, including XML schemas and WSDLs

- Source code examples for web service clients

- A sand box environment for testing the interactions between the Maket Particpants and ERCOT. This environment will eventually be used for qualification of QSEs.

The interface specifications, artifacts and implementation of the sand box environment will be staged. An iterative implementation approach will be used, where feedback from each stage will be used to plan subsequent stages.

## Technical Interoperability

There are several strategies that are being employed in order to achieve technical interoperability of MPs with ERCOT. These include:

  - Use of open standards

  - API subgroup allows input from Market Participants on interoperability issues

  - The early deployment of a sandbox environment that enables ERCOT to work with the Market Participants to insure interoperability issues are addressed prior to market trials

  - Providing sample Java and .Net web service client code

  - Deployment of interfaces via the sandbox environment for early testing by Market Participants, so that feedback can be provided to ERCOT

Open standards are a key part of the strategy to achieve technical interoperability. Standards of particular interest include:

- W3C standards

- OASIS WS-* standards

- IEC Common Information Model and related standards (e.g. IEC 61968-1)

It is very important that the implementation of Web Service interfaces not be dependent upon any proprietary, third party products. Another key requirement is that implementation of web service clients must be possible using both Java and .Net development tools.

More details on technical interoperability will be provided as a consequence of existing web services provided by ERCOT, detailed design and experience with the Sand Box environment.

## Service Level Agreements

Different categories of Web Services will have different service level agreements (SLAs). The SLAs for some Web Services are directly impacted by the variability in the amount of data that can be transferred (e.g. large bid sets).

The response time periods specified for each interface covered by an SLA typically will vary to some degree, based upon factors such as network and system loading. Consequentially, each SLA will be stated in a manner such that each SLA will be honored Z% of the time.

The following are example SLA specifications for categories of Web Services. However, more specific SLAs will be identified for specific interfaces as required.

Web Services for DAM interactions:

- Must be over X% available for a given trading day

- Validation of bid/trade/offer/schedule sets will be processed within M1 minutes after the bid set is initially submitted or changed

- Inquiries about a specific bid set will be processed within S1 seconds, with an additional S2 seconds per bid

Web Services for RTM interactions:

- Must be over X% available for a given trading day

- Validation of bid sets will be processed within M1 minutes after the bid set is initially submitted or changed

- Bid set inquiries will be processed within S1 seconds, with an additional S2 seconds per bid

Web services for obtaining real-time market information and providing acknowledgments and confirmations:

  - Must be over X% available

  - Requests involving less than 10KB data should be processed within S3 seconds, unless otherwise specified for a specific interface

Web services for other than real time transactions and information requests:

  - Must be over Y% available

  - Requests involving less than 10KB data should be processed within S4 seconds, unless otherwise specified for a specific interface

Notification interfaces, where the SLA obligations are also on the Market Participant as well as ERCOT:

  - ERCOT will post notifications to a Market Participant within S5 seconds of the time of internal posting

  - Notification service interface provided by Market Participant should be minimally Y% available. Any ‘downtime’ or periods of inaccessibility will directly impact the timeliness of notifications (e.g. validation of bid sets, alerts, etc.) from ERCOT.

Note: These values and additional SLAs will be finalized once the vendor requirements are complete.

## Auditing, Monitoring and Management

ERCOT will perform auditing, monitoring and management for Web Services described by this specification using common services. Internal auditing by ERCOT will be used to track and insure that SLAs are met by ERCOT. All Web Service requests will be logged by ERCOT in order to permit calculations related to SLAs. The signatures supplied on SOAP messages will be recorded with transactions as a means of non-repudiation of each transaction.

It is important to recognize that ERCOT is not responsible for the monitoring and management of Market Participant software and network connectivity. Therefore ERCOT cannot guarantee that notification interfaces provided by Market Participants are accessible as needed for timely delivery of notifications.

## Versioning

It is important to recognize that new versions of interfaces may be provided over time, largely as a consequence of:

- Staging of initial implementation

- New requirements

- Upgrades to vendor products

Wherever possible, interfaces will be evolved through augmentation, where a newer version of an interface is compatible with a previous version of an interface. However, this will not always be possible. New versions of interfaces will be manifested by:

- Changes to WSDLs

- Changes to XML Schemas

- Changes to software implementations

New versions will be deployed within a Sand Box environment for a testing/trial period. WSDL and XML schemas namespaces will include a date reference. Messages will use the Header/Revision field to identify a specific revision - this will enable ERCOT to process multiple versions of an interface where appropriate.

A detailed versioning strategy will be developed and presented for approval to the API Subgroup and TPTF.

## Governance

The web service interfaces will be critical to the operations of both ERCOT and Market Participants. The Web Services will evolve for many reasons, especially as the needs of the market evolve. Governance policies and processes will need to be defined for the Web Service lifecycle that provide strict guidelines related to:

- Design

- Implementation

- Testing

- Deployment

- Management

A comprehensive governance strategy will need to be developed and implemented by ERCOT with input from the API Subgroup and TPTF.

## Web Service Configuration Standards

ERCOT will configure its web servers with specific parameters for use of Web Services by Market Participants (e.g. for security). Market Participants will also need to set up Web Services to handle notifications from ERCOT. ERCOT will define specific configuration details and parameters to be used by Market Participants.

Detailed web service configuration standards will be provided to the API Subgroup and TPTF and will be refined through detailed design and experience with the Nodal Sand Box environment.

# Market Transaction Service

The purpose of the MarketTransaction Service is to support interfaces required for market transactions. This section describes the use of web services by Market Participants as required for bidding processes that involve the submission, change and cancellation of bids, trades, offers and schedules for specific markets.

On a given trading day, each Market Participant provides information (using a BidSet) to ERCOT that will be used at the close of the market to determine awards and obligations and to provide schedules needed for market operations. Up until the close of the market, a Market Participant may create, update or cancel bids, trades, offers and schedules. When submitted, ERCOT will validate the submission, reporting format or submission errors to Market Participants using notification messages.

Note: The interfaces described in this section are subject to change pending final MMS design.

## Interfaces Provided

The interfaces provide the means to create (i.e. submit), get (i.e. query) and cancel (i.e. withdraw) bids/trades/offers/schedules on a given trading date. A single container class ‘BidSet’ is used to hold a set of bids/trades/offers/schedules within the Payload section of the message, where each of the bids, trades, schedules or offers may be of a different type.

The following diagram shows an example message sequence, using the ‘verb’ and ‘noun’ convention. Where this section focuses on the requests made by Market Participant systems to the ERCOT Nodal Web Services, the sequence diagram also includes notification messages sent from ERCOT to Market Participant Notification services (as described in section 5).

Figure 11 - Example BidSet Sequence Diagram

The message sequence example shown involves the following steps:

- Market participant sends a RequestMessage for ‘create BidSet’ with an initial BidSet to ERCOT for a specific market

- In response to step 1, ERCOT performs a simple syntax scan and typically sends a ResponseMessage with ReplyCode=OK. In the response payload, each bid/offer/trade/schedule will identify a ‘SUBMITTED’ status and an mRID value. (An alternative example could result in a reply of ‘ERRORS’ if the syntax check failed, in which case steps 3 and 4 would not occur)

- ERCOT validates the bids within the BidSet. This could take several minutes.

- A notification message (using verb=changed) is sent to the notification interface provided by the Market Participant. The status of the bids within the BidSet will indicate whether the bid/offer/trade/schedule was ACCEPTED or had ERRORS. This message will not include the complete BidSet.

- In the event of ERRORS, the Market Participant may make a request to get the current bids within the BidSet using a RequestMessage with verb=get. The request may be for individual bids using the mRID values, or for the entire set of bids within the BidSet using the TradeDate.

- In response to step 5, any errors will be identified for each specific bid, as returned in a ResponseMessage using verb=reply. The errors will be listed along with each mRID, where the errors are provided by MMS.

- A Market Participant may choose to cancel one or more bids using a RequestMessage with verb=cancel. The mRID is used for cancellation of a bid. (See section 2.3.4)

- In response to step 7, a ResponseMessage is sent with verb=’reply’

- The Market Participant may resubmit some bids (e.g. to correct errors), to change bids (e.g. change prices) and/or submit new bids, using a RequestMessage with verb=change (note that BidSets are a special case where the verbs ‘create’ and ‘change’ can be used interchangeably).

- In response to step 9, a ResponseMessage is sent with verb=’reply’.

- The newly submitted bids within the BidSet (i.e. the set of bids consequential to steps 7 and 9) is validated.

- As in step 4, a notification message is sent to the Market Participant to indicate whether or not the BidSet was validated and accepted. The notification message uses verb=updated.

- After the close of the market, awards and obligations are determined.

- Market participants are notified of specific awards and/or obligations. The notification message uses verb=’created’, noun=’AwardSet’.

The following diagram describes the potential state transitions of each bid/offer/trade/schedule within a BidSet:

Figure 12 - State Diagram for Bids, Trades, Offers and Schedules

## Interfaces Required

The following table describes the parameters used in the request message (RequestMessage) for market transactions, noting that each transaction has a request and a response message. The verbs create, update, get and cancel are respectively used to submit (create or change), query and cancel bid sets.

| Message Element | Value |
| --- | --- |
| Header/Verb | create/get/change/cancel |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload | BidSet (used for create, update and cancel request messages, and may optionally be used for get requests to identify specific bids of interest) |

Figure 13 -  Message: create BidSet

The corresponding response messages (ResponseMessage) would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error message if the ReplyCode is ERROR |
| Payload | BidSet, where the status of each bid/offer/trade/schedule within the BidSet may be ACCEPTED, SUBMITTED or ERRORS. Specific errors may be identified for each bid. |

Figure 14 - Message: reply BidSet

In the cases of payloads that would otherwise exceed 1 megabyte, the payloads should be zipped, base64 encoded and stored within the ‘Payload/Compressed’ tag.

For the purposes of BidSets, the verbs create and update can be used interchangeably.

## Message Specifications

A BidSet is the payload type used for the submission, query and cancellation of bids and offers, and serves as a container for the different types of bids, offers, trades and scheduled that are submitted by a QSE for a given trading day. A BidSet identifies the trading date and Market Participant with a sequence of any number of bids, trades, offers and schedules.

The header of a BidSet is shown in the following diagram:

Figure 15 - BidSet Header Structure

The submission (using ’create’ verb and ‘BidSet’ noun in the message header) of a BidSet by a Market Participant will have the effect of either creating new bids (or offers, schedules, trades, etc.) for a given trading date, or overwriting existing bids that were previously submitted for a given BidSet for a given trading day.

Additionally, bids, offers, trades and schedules may be submitted in partial batches, where more than one bid set is submitted for a given trading day. When this is done, new bids are aggregated with previously submitted bids and updates to previously submitted bids overwrite the previous bid. The following sequence describes the aggregation of bid sets:

- A first bid set is submitted by a market participant for a trading day, with bids 1, 2, 3, 4.

- A second bid set is submitted by the market participant for the day with bids 5, 6, 7, 8.

- The market participant ‘get’s the bid set for the day, the reply returns the details of bids 1-8.

- The market participant submits a cancel for bid 1.

- A third bid set is submitted by the market participant for the day, where there are updates to bids 5 and 7, as well as a new bid 9

- The market participant ‘get’s the bid set for the day, the reply returns the details of bids 2-9.

In order to cancel a bid, a BidSet must be sent using the ‘cancel’ verb, with the specific bids to be cancelled identified within the BidSet. Cancel can only be used to cancel specific bids, not a whole BidSet for a given market.

In order to query a BidSet, A BidSet is sent using the ‘get’ verb, where the desired bids for the specific BidSet are identified. There are then two options:

- If no bids are identified within the BidSet, all bids for the particular trading date will be returned in the BidSet in the response message.

- If specific bids are identified within the BidSet, only the details of the specified bids will be returned in the BidSet in the response message.

When a BidSet is returned by a ‘get’ request for a given trade date, the status value of each in the BidSet is populated. Values could include:

  - SUBMITTED (to indicate submission, but no further processing has been completed)

  - ACCEPTED (to indicate successful validation and acceptance of the bid)

  - ERRORS (to indicate that there are one or more errors for the bid)

The following sub sections describe the structure of specific bid, offer, trade and schedule types. Typically each bid/offer/trade/schedule will have a set of properties that along with the type of bid/offer/trade/schedule makes it unique (see section 2.3.3). Typically these would include:

- Bid, offer, schedule or trade type

- Resource

- Ancillary Service type

When submitting a bid/offer/schedule/trade using create or update, all properties for the bid must be specified. When performing a get or cancel request, only those parameters that uniquely identify the bid/offer/trade/schedule must be specified. The following diagram shows information commonly maintained for each type of bid/offer/trade/schedule. The status for a given bid/offer/trade/schedule may be SUBMITTED, ACCEPTED, ERRORS or CANCELED. If the status is ERRORS, there may be one or more error stings identified.

Figure 16 – Bid/Offer/Trade/Schedule Common Elements

The mRID is not supplied for the initial submission of a bid, but must be supplied for cancellations to previously submitted bids. The marketType is currently optional and is reserved for future use.

### Three Part Supply Offer

The following diagram defines the structure of a Three Part Supply Offer that could be included within a BidSet. This is one of the more complex structures as it involves startup costs, minimum generation costs and price curves. The PriceCurve is defined by CIM directly as a subclass of CurveSchedule.

Figure 17 – ThreePartOffer Structure

The error tag is used to return one or more errors that may be the consequence of the failure of business or syntax validation rules for each type of bid/offer/trade/schedule. The EnergyOfferCurve element is based upon a PriceCurve type. Within PriceCurve, the values of ‘xvalue’ are used to identify the quantity in MW, and the values of ‘y1value’ are used to define the price in $/MWh. The ‘curveStyle’ serves as the ‘indicator’, to identify whether the data is to be treated as FIXED, VARIABLE or a CURVE. The number of points in each PriceCurve is limited to 11. Where more than one PriceCurve is supplied for a bid/offer/trade, the start and end times must not overlap. The incExcFlag is to indicate whether the offer is linked to an AS offer or not. The reason code is typically used for adjustments. The structure of a PriceCurve type is detailed in the following diagram:

Figure 18 - PriceCurve Structure

The following diagrams describe the structures used for FIP/FOP, startup costs and minimum generation costs within the ThreePartOffer message. Note that if more than one StartupCost or MinimumEnergy is provided, there must not be overlapping start and end times. That is to say that more than one StartupCost or more than one MinimumEnergy can be specified within the day, provided that the time intervals for each instance are distinct.

Figure 19 - Fip/Fop, StartupCost and MinimumEnergy Structures

On submission, the following items are required for a ThreePartOffer. Fields that are used as ‘keys’ to identify a bid/offer/trade/schedule (see section 2.3.3) are identified by ‘K’. Required fields are identified by ‘Y’, option fields are identified by ‘N’. There are some exception cases where identified key fields are identified as not being required (e.g. expirationTime), and are notated as ‘K, N’.

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| resource | K | string | Resource | Valid resource name |
| combinedCycle | N | string | Combined cycle to which resource is associated | NULL or Valid plant name if unit is part of a combined cycle |
| expirationTime | K, N | datetime | Time of offer expiration | Valid time before trade date |
| FipFop/startTime | Y | datetime | Start time | Valid hour boundary within the trade date |
| FipFop/endTime | Y | datetime | End time | Valid hour boundary within the trade date |
| FipFop/fipPercent | Y | decimal | Fuel index price percent | >= 0, <= 100 |
| FipFop/fopPercent | Y | decimal | Fuel oil price percent | >= 0, <= 100 |
| StartupCost/startTime | N | datetime | Start time for startup costs | Valid hour boundary within the trade date |
| StartupCost/endTime | N | datetime | End time for startup costs | Valid hour boundary within the trade date |
| StartupCost/hot | N | decimal | Hot startup cost | >= 0 |
| StartupCost/intermediate | N | decimal | Intermediate startup cost | >= 0 |
| StartupCost/cold | N | decimal | Cold startup cost | >= 0 |
| MinimumEnergy/startTime | N | datetime | Start time for minimum energy costs | Valid hour boundary within the trade date |
| MinimumEnergy/endTime | N | datetime | End time for energy costs | Valid hour boundary within the trade date |
| MinimumEnergy/cost | N | decimal | Minimum energy cost | >= 0 |
| EnergyOfferCurve/startTime | N | datetime | Start time for curve | Valid hour boundary within the trade date |
| EnergyOfferCurve/endTime | N | datetime | End time for curve | Valid hour boundary within the trade date |
| EnergyOfferCurve/curveStyle | N | string | Used as an indicator to describe the type of ‘curve’ | CURVE |
| EnergyOfferCurve/CurveData/xvalue | N | float | Megawatts | Quantity in MW |
| EnergyOfferCurve/CurveData/y1value | N | float | $/MWh | Price in $/MWh |
| EnergyOfferCurve/incExcFlag | N | string | Identifies whether or not an AS offer is linked to this offer. | INC or EXC. Default should be INC, where it is linked to an ASOffer |
| EnergyOfferCurve/reason | N | string | Reason for adjustment | Text description |

Figure 20 - ThreePartOffer Requirements

There should minimally be a StartupCost, MinimumEnergy or PriceCurve specified for a Three Part Offer. The following is an XML example of a Three Part Offer:

<ThreePartOffer xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<expirationTime>2007-12-31T04:00:00-06:00</expirationTime>

<resource>AcmeUnit1</resource>

<combinedCycle>AcmeCC</combinedCycle>

<FipFop>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<fipPercent>50</fipPercent>

<fopPercent>50</fopPercent>

</FipFop>

<StartupCost>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<hot>333.33</hot>

<intermediate>444.44</intermediate>

<cold>666.66</cold>

</StartupCost>

<MinimumEnergy>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<cost>1001.11</cost>

</MinimumEnergy>

<EnergyOfferCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>60</xvalue>

<y1value>40.00</y1value>

</CurveData>

<CurveData>

<xvalue>80</xvalue>

<y1value>45.00</y1value>

</CurveData>

<CurveData>

<xvalue>90</xvalue>

<y1value>47.00</y1value>

</CurveData>

<incExcFlag>INC</incExcFlag>

</EnergyOfferCurve>

<EnergyOfferCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>60</xvalue>

<y1value>42.00</y1value>

</CurveData>

<CurveData>

<xvalue>80</xvalue>

<y1value>55.00</y1value>

</CurveData>

<CurveData>

<xvalue>90</xvalue>

<y1value>57.00</y1value>

</CurveData>

<incExcFlag>INC</incExcFlag>

</EnergyOfferCurve>

</ThreePartOffer>

### Self Arranged Ancillary Services

Self Arranged Ancillary Services describe a schedule of ancillary services that a QSE has arranged. The following diagram describes the structure of a schedule for self arranged ancillary services.

Figure 21 - SelfArrangedAS Structure

Within the SelfArrangedAS is a CapacitySchedule. CapacitySchedules are defined using TmSchedules types, as shown in the following diagram. The values of ‘value1’ within the CapacitySchedule are used to identify megawatts.

Figure 22 - CapacitySchedule Structure

On submission, the following items are required for a SelfArrangedAS:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| asType | K | string | Ancillary service type | Valid AS type code |
| CapacitySchedule/startTime | Y | datetime | Start time for capacity schedule | Valid hour boundary |
| CapacitySchedule/endTime | N | dateTime | End time for capacity schedule | Valid hour boundary |
| CapacitySchedule/ TmPoint/time | Y | integer | Absolute time for start of interval | >= 0 |
| CapacitySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 23 - SelfArrangedAS Requirements

The following XML is an example of a self arranged AS, showing the use of a CapacitySchedule:

<SelfArrangedAS xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<asType>NON-SPIN</asType>

<CapacitySchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>30</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>39</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T04:00:00-06:00</time>

<value1>30</value1>

</TmPoint>

</CapacitySchedule>

</SelfArrangedAS>

### Incremental and Decremental Offer Curves

The following diagram describes the structure of  Incremental and Decremental Offer Curves. This shows the details of the FipFop and PriceCurve structures that are contained within.

Note - This structure is subject to change.

Figure 24 - IncDecOffer Structure

Both the IncrementalCurve and DecrementalCurve use the PriceCurve structure as described previously in this document. On submission, the following items are required for an IncDecOffer:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| resource | K | string | Resource | Valid resource name |
| combinedCycle | N | string | Combined cycle to which resource is associated | Valid plant name if unit is part of a combined cycle |
| expirationTime | K, N | datetime | Offer expiration time | Valid time before or during the trade date |
| FipFop/fipPercent | N | decimal | Fuel index price percent | >= 0, <= 100 |
| FipFop/fopPercent | N | decimal | Fuel oil price percent | >= 0, <= 100 |
| IncrementalCurve/startTime | Y | datetime | Start time for curve | Valid hour boundary |
| IncrementalCurve/endTime | Y | datetime | End time for curve | Valid hour boundary |
| IncrementalCurve/curveStyle | Y | string | Type of curve | CURVE |
| IncrementalCurve/CurveData/xvalue | Y | float | Megawatts | Quantity in MW |
| IncrementalCurve/CurveData/y1value | Y | float | $/MWh | Incremental price in $/MWh, must be greater than corresponding decremental curve value |
| DecrementalCurve/startTime | Y | datetime | Start time for curve | Valid hour boundary |
| DecrementalCurve/endTime | Y | datetime | End time for curve | Valid hour boundary |
| DecrementalCurve/curveStyle | Y | string | Type of curve | CURVE |
| DecrementalCurve/CurveData/xvalue | Y | float | Megawatts | Quantity in MW |
| DecrementalCurve/CurveData/y1value | Y | float | $/MWh | Decremental price in $/MWh, must be less than corresponding incremental curve value |

Figure 25 - IncDecOffer Requirements

The following is an XML example for an IncDecOffer:

<IncDecOffer xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<expirationTime>2007-12-31T23:00:00-06:00</expirationTime>

<resource>ACMEUnit2</resource>

<combinedCycle>ACMECC</combinedCycle>

<FipFop>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<fipPercent>20</fipPercent>

<fopPercent>70</fopPercent>

</FipFop>

<IncrementalCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>35.50</y1value>

</CurveData>

<CurveData>

<xvalue>40</xvalue>

<y1value>45.50</y1value>

</CurveData>

<CurveData>

<xvalue>60</xvalue>

<y1value>55.50</y1value>

</CurveData>

</IncrementalCurve>

<IncrementalCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>37.50</y1value>

</CurveData>

<CurveData>

<xvalue>40</xvalue>

<y1value>47.50</y1value>

</CurveData>

<CurveData>

<xvalue>60</xvalue>

<y1value>57.50</y1value>

</CurveData>

</IncrementalCurve>

<DecrementalCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>25.50</y1value>

</CurveData>

<CurveData>

<xvalue>40</xvalue>

<y1value>25.50</y1value>

</CurveData>

<CurveData>

<xvalue>60</xvalue>

<y1value>25.50</y1value>

</CurveData>

</DecrementalCurve>

<DecrementalCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>27.50</y1value>

</CurveData>

<CurveData>

<xvalue>40</xvalue>

<y1value>27.50</y1value>

</CurveData>

<CurveData>

<xvalue>60</xvalue>

<y1value>27.50</y1value>

</CurveData>

</DecrementalCurve>

</IncDecOffer>

### Ancillary Service Offer

An Ancillary Service Offer is used to offer ancillary services from a specified resource. The following diagram describes the structure of an Ancillary Service Offer (ASOffer):

Note: This structure is subject to change.

Figure 26 - ASOffer Structure

The PriceCurve structure is identical to the structure described previously in this document. On submission, the following items are required for an ASOffer:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| resource | K | string | Resource | Valid resource name |
| combinedCycle | N | string | Combined cycle | Valid combined cycle ID |
| asType | K | string | Ancillary service type | Valid AS type code |
| expirationTime | K, N | datetime | Time of offer expiration | Valid time before trade date |
| PriceCurve/startTime | Y | datetime | Start time for curve | Valid hour boundary |
| PriceCurve/endTime | Y | datetime | End time for curve | Valid hour boundary |
| PriceCurve/curveStyle | Y | string | Used as an indicator to describe the type of ‘curve’ | FIXED or VARIABLE |
| PriceCurve/CurveData/xvalue | Y | float | Megawatts | Quantity in MW |
| PriceCurve/CurveData/y1value | Y | float | $/MWh | Price in $/MWh |

Figure 27 - ASOffer Requirements

The following is an XML example for an ASOffer:

<ASOffer xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<expirationTime>2007-12-31T22:00:00-06:00</expirationTime>

<resource>AcmeUnit1</resource>

<asType>Reg-Up</asType>

<PriceCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>FIXED</curveStyle>

<CurveData>

<xvalue>60</xvalue>

<y1value>20.00</y1value>

</CurveData>

</PriceCurve>

<PriceCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>FIXED</curveStyle>

<CurveData>

<xvalue>80</xvalue>

<y1value>23.00</y1value>

</CurveData>

</PriceCurve>

</ASOffer>

### Ancillary Service Trade

An Ancillary Service Trade is used to describe a scheduled trade of ancillary services between a buyer and a seller. The following diagram describes the structure of an Ancillary Service Trade (ASTrade).

Figure 28 - ASTrade Structure

The ASSchedule structure is based upon a TmSchedule as described previously in this document. The submission of an ASTrade (or generally any trade) requires a matching trade to be submitted by the counterparty befor the close of the market. On submission, the following items are required for an ASTrade:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| asType | K | string | Ancillary service type | Valid AS type code |
| tradeId | K, N | string | Trade ID | Must be either not specified or be a unique for combination of buyer, seller and sp on trade date |
| buyer | K | string | Trade buyer | Valid QSE |
| seller | K | string | Trade seller | Valid QSE |
| expirationTime | K, N | datetime | Time of offer expiration | Valid time before trade date |
| ASSchedule/startTime | Y | datetime | Start time for AS schedule | Valid hour boundary |
| ASSchedule/endTime | N | dateTime | End time for AS schedule | Valid hour boundary |
| ASSchedule/ TmPoint/time | Y | integer | Absolute time for beginning of interval | >= 0 |
| ASSchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 29 - ASTrade Requirements

The following is an XML example for an ASTrade:

<ASTrade xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<buyer>Acme</buyer>

<seller>Cogswell</seller>

<tradeID>1</tradeID>

<asType>Reg-Up</asType>

<ASSchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>70</value1>

</TmPoint>

</ASSchedule>

</ASTrade>

### Capacity Trade

The following diagram describes the structure of a Capacity Trade:

Figure 30 - CapacityTrade Structure

The CapacitySchedule structure is identical to the structure described previously in this document. The submission of a trade requires a matching trade to be submitted by the counterparty. The following table describes the required fields:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| tradeId | K, N | string | Trade ID | Must be either not specified or be a unique for combination of buyer, seller and sp on trade date |
| buyer | K | string | Trade buyer | Valid QSE |
| seller | K | string | Trade seller | Valid QSE |
| CapacitySchedule/startTime | Y | datetime | Start time for capacity schedule | Valid hour boundary |
| CapacitySchedule/endTime | N | dateTime | End time for capacity schedule | Valid hour boundary |
| CapacitySchedule/ TmPoint/time | Y | integer | Absolute time for beginning of interval | >= 0 |
| CapacitySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 31 - CapacityTrade Requirements

The following is an XML example for a CapacityTrade:

<CapacityTrade xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<buyer>ACME</buyer>

<seller>XYZ</seller>

<tradeID>3</tradeID>

<CapacitySchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>70</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>90</value1>

</TmPoint>

</CapacitySchedule>

</CapacityTrade>

### Congestion Revenue Rights (CRR) Offer

The following diagram describes the structure of a Congestion Revenue Rights (CRR) Offer:

Figure 32 - CRR Structure

The following structure is used for minimum reservation prices. Note than if more than on MinimumReservationPrice is provided, the start end end times must not overlap.

Figure 33 - MinimumReservationPrice Structure

The following structure is used for the peak load forecast for NOIEs:

Figure 34 - NOIEPeakLoadForecast Structure

The CapacitySchedule structure is identical to the structure described previously in this document. On submission, the following table describes items are required for a CRR. In addition, the PeakLoadForecast must be specified for NOIEs.

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for offer | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for offer | Valid end hour boundary for trade date |
| crrId | K | string | CRR ID | ID of CRR account holder |
| source | K | string | Source settlement point | Valid settlement point name |
| sink | K | string | Sink settlement point | Valid settlement point name |
| CapacitySchedule/startTime | Y | datetime | Start time for capacity schedule | Valid hour boundary |
| CapacitySchedule/endTime | N | dateTime | End time for capacity schedule | Valid hour boundary |
| CapacitySchedule/ TmPoint/time | Y | integer | Absolute start time for interval | >= 0 |
| CapacitySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |
| MnimumReservationPrice/ startTime | Y | datetime | Start time for minimum reservation price | Valid hour boundary |
| MnimumReservationPrice/ endTime | N | datetime | End time for minimum reservation price | Valid hour boundary |
| MnimumReservationPrice/ price | Y | decimal | Minimum price in $/MWh | >= 0.00 |
| NOIEPeakLoadForecast/startTime | If NOIE | datetime | Start time for peak load forecast | Valid hour boundary |
| NOIEPeakLoadForecast/endTime | N | dateTime | End time for peak load forecast | Valid hour boundary |
| NOIEPeakLoadForecast/ TmPoint/time | If NOIE | integer | Absolute start time for intgerval | >= 0 |
| NOIEPeakLoadForecast/ TmPoint/value1 | If NOIE | float | Forecasted megawatts | >= 0 |

Figure 35 - CRR Requirements

The following is an XML example for a CRR:

<CRR xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<crrId>345</crrId>

<source>SP3536</source>

<sink>SP3333</sink>

<CapacitySchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>50</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>60</value1>

</TmPoint>

</CapacitySchedule>

<MinimumReservationPrice>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<price>33.00</price>

</MinimumReservationPrice>

<NOIEPeakLoadForecast>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>1200</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>1500</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>1600</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>1700</value1>

</TmPoint>

</NOIEPeakLoadForecast>

</CRR>

### Current Operating Plan (COP)

The following diagram describes the structure of a Current Operating Plan (COP), where the COP provides a schedule for a resource:

Figure 36 - COP Structure

The structures for Limits and ASCapacity are shown in the following diagrams. If more than one ResourceStatus, Limits or ASCapacity block is provided, start and end times must not overlap.

Figure 37 - Limits Structure

Figure 38 - ASCapacity Structure

COP submissions are never cancelled, as a COP must be submitted. A resubmission of a COP overwrites the previous submission. On submission, the following items are required for a COP:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for COP | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for COP | Valid end hour boundary for trade date |
| resource | K | string | Resource | Valid resource name |
| conbinedCycle | N | string | Combined cycle | Valid combined cycle ID |
| ResourceStatus/startTime | Y | datetime | Start time for status | Valid hour boundary |
| ResourceStatus /endTime | Y | datetime | End time for status | Valid hour boundary |
| ResourceStatus /status | Y | string | status | Valid status code |
| Limits/startTime | Y | datetime | Start time for limits | Valid hour boundary |
| Limits/endTime | Y | datetime | End time for limits | Valid hour boundary |
| Limits/hsl | Y | integer | High sustained limit in MW | >=0 |
| Limits/lsl | Y | integer | Low sustained limit in MW | >=0 |
| Limits/hel | Y | integer | High emergency limit in MW | >=0 |
| Limits/lel | Y | integer | Low sustained limit in MW | >=0 |
| ASCapacity/startTime | Y | datetime | Start time for AS capacity schedule | Valid hour boundary |
| ASCapacity/endTime | Y | dateTime | End time for AS capacity schedule | Valid hour boundary |
| ASCapacity/regUp | Y | integer | Regulation up MW | >= 0 |
| ASCapacity/regDown | Y | integer | Regulation down MW | >= 0 |
| ASCapacity/rrs | Y | integer | Responsive reserve MW | >= 0 |
| ASCapacity/nonSpin | Y | integer | Non-spinning MW | >= 0 |

Figure 39 - COP Requirements

The following is an XML example for a COP:

<COP xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<resource>ACMEUnit2</resource>

<ResourceStatus>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06 :00</endTime>

<operatingMode>ON</operatingMode>

</ResourceStatus>

<ResourceStatus>

<startTime>2008-01-01T10:00:00-06 :00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<operatingMode>OFF</operatingMode>

</ResourceStatus>

<Limits>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<hsl>100</hsl>

<lsl>20</lsl>

<hel>110</hel>

<lel>10</lel>

</Limits>

<Limits>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<hsl>0</hsl>

<lsl>0</lsl>

<hel>0</hel>

<lel>0</lel>

</Limits>

<ASCapacity>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<regUp>20</regUp>

<regDown>20</regDown>

<rrs>40</rrs>

<nonSpin>80</nonSpin>

</ASCapacity>

<ASCapacity>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<regUp>0</regUp>

<regDown>0</regDown>

<rrs>0</rrs>

<nonSpin>0</nonSpin>

</ASCapacity>

</COP>

The terms maxEmergency, maximumEconomic, minEmergency and minimumEcononic are used to in order to be consistent with the IEC CIM.

Currently defined resource operatingModes include:

- ONRUC – On-Line and the hour is a RUC-Committed Interval

- ONREG – On-Line Resource with Energy Offer Curve providing Regulation Service

- ON – On-Line Resource with Energy Offer Curve

- ONDSR – On-Line Dynamically Scheduled Resource

- ONOS – On-Line Resource with Output Schedule

- ONOSREG – On-Line Resource with Output Schedule providing Regulation Service

- ONDSRREG – On-Line Dynamically Scheduled Resource providing Regulation Service

- ONTEST – On-Line Test with Output Schedule

- ONEMR – On-Line EMR (available for commitment or dispatch only for ERCOT-declared Emergency Conditions; the QSE may appropriately set LSL and HSL to reflect operating limits)

- ONRR – On-Line as a synchronous condenser (hydro) providing Responsive Reserve but unavailable for dispatch by SCED and available for commitment by RUC

- OUT – Off-Line and unavailable

- OFFNS – Off -Line but reserved for Non-Spin

- OFF – Off-Line but available for commitment by DAM and RUC

- EMR – Available for commitment only for ERCOT-declared Emergency Condition events; the QSE may appropriately set LSL and HSL to reflect operating limits

- ONRGL – Available for dispatch of Regulation Service

- ONRRCLR – Available for dispatch of Responsive Reserve Service as a Controllable Load Resource

- ONRL – Available for dispatch of Responsive Reserve Service or Non-Spin, excluding Controllable Load Resources

- OUTL – Not available

### DC Tie Schedule

The following diagram describes the structure of a DC Tie Schedule:

Figure 40 - DCTieSchedule Structure

The following diagrams describes the structure of the EnergySchedule and the NERCTags. The EnergySchedule is based upon a TmSchedule type.

Figure 41 - EnergySchedule Structure

Figure 42 - NERCTags Structure

On submission, the following items are required for a DCTieSchedule:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid 15 minute interval boundary for trade date |
| endTime | K | datetime | End time for bid | Valid 15 minute interval boundary for trade date |
| buyer | K | string | Trade buyer | Valid QSE |
| seller | K | string | Trade seller | Valid QSE |
| sp | K | string | Settlement point | Valid settlement point name |
| NERCTags/startTime | Y | datetime | Start time for tag | Valid time string |
| NERCTages/endTime | Y | datetime | End time for tag | Valid time string |
| NERCTags/tag | Y | string | NERC tag string | Valid NERC tag string |
| EnergySchedule/startTime | Y | datetime | Start time for capacity schedule | Valid 15 minute interval boundary |
| EnergySchedule/endTime | N | dateTime | End time for capacity schedule | Valid 15 minute interval boundary |
| EnergySchedule/ TmPoint/time | Y | integer | Absolute time for start of interval | >= 0 |
| EnergySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 43 - DCTieSchedule Requirements

The following is an XML example for a DCTieSchedule:

<DCTieSchedule xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<buyer>ACME</buyer>

<seller>XYZ</seller>

<sp>DC56</sp>

<EnergySchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>500</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T04:00:00-06:00</time>

<value1>400</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T10:00:00-06:00</time>

<value1>520</value1>

</TmPoint>

</EnergySchedule>

<NERCTags>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<tag>NERC Tag String</tag>

</NERCTags>

</DCTieSchedule>

### DAM Energy Bid

The following diagram describes the structure of a DAM Energy Bid:

Figure 44 - EnergyBid Structure

The PriceCurve structure is identical to the structure described previously in this document. On submission, the following items are required for an EnergyBid:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| sp | K | string | Settlement point | Valid settlement point name |
| bidId | K | string | Bid ID | MP supplied bid ID |
| PriceCurve/startTime | Y | datetime | Start time for curve | Valid hour boundary |
| PriceCurve/endTime | Y | datetime | End time for curve | Valid hour boundary |
| PriceCurve/curveStyle | Y | string | Used as an indicator to describe the type of ‘curve’ | FIXED, VARIABLE or CURVE |
| PriceCurve/CurveData/xvalue | Y | float | Megawatts | Quantity in MW |
| PriceCurve/CurveData/y1value | Y | float | Price in $/MWh | $/MWh |

Figure 45 - EnergyBid Requirements

The following is an XML example for an EnergyBid:

<EnergyBid xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<expirationTime>2007-12-31T23:00:00-06:00</expirationTime>

<sp>ACME4</sp>

<bidID>2</bidID>

<PriceCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>FIXED</curveStyle>

<CurveData>

<xvalue>40</xvalue>

<y1value>43.54</y1value>

</CurveData>

</PriceCurve>

<PriceCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>FIXED</curveStyle>

<CurveData>

<xvalue>50</xvalue>

<y1value>63.54</y1value>

</CurveData>

</PriceCurve>

</EnergyBid>

### DAM Energy Only Offer

The following diagram describes the structure of a DAM Energy Only Offer:

Figure 46 – EnergyOnlyOffer Structure

The EnergyOfferCurve structure uses the PriceCurve type structure described previously in this document. On submission, the following items are required for an EnergyOnlyOffer:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| sp | K | string | Settlement point | Valid settlement point name |
| bidId | K | string | Bid ID | QSE supplied bid ID |
| expirationTime | K, N | datetime | Time of offer expiration | Valid time before trade date |
| EnergyOfferCurve/startTime | Y | datetime | Start time for curve | Valid hour boundary |
| EnergyOfferCurve/endTime | Y | datetime | End time for curve | Valid hour boundary |
| EnergyOfferCurve/curveStyle | Y | string | Used as an indicator to describe the type of ‘curve’ | FIXED, VARIABLE or CURVE |
| EnergyOfferCurve/CurveData/xvalue | Y | float | Quantity in megawatts | MW |
| EnergyOfferCurve/CurveData/y1value | Y | float | Price in $/MWh | $/MWh |

Figure 47 - EnergyOnlyOffer Requirements

The following is an XML example for an EnergyOnlyOffer:

<EnergyOnlyOffer xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<expirationTime>2007-12-31T22:00:00-06:00</expirationTime>

<sp>ACME3</sp>

<bidID>1</bidID>

<EnergyOfferCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>VARIABLE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>43.00</y1value>

</CurveData>

</EnergyOfferCurve>

<EnergyOfferCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>VARIABLE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>53.00</y1value>

</CurveData>

</EnergyOfferCurve>

</EnergyOnlyOffer>

### Energy Trade

An EnergyTrade describes a scheduled trade of energy between a buyer and a seller at a specified settlement point. The following diagram describes the structure of an Energy Trade:

Figure 48 - EnergyTrade Structure

Within an EnergyTrade is an EnergySchedule. EnergySchedules are defined using the TmSchedule type. This is shown in the following diagram:

Figure 49 - EnergySchedule Structure

In order for a trade to be accepted, both the buyer and seller must submit matching trades, as identified using the buyer, seller and tradeId tags. On submission, the following items are required for an EnergyTrade:

- startTime

- endTime

- buyer (QSE ID)

- seller (QSE ID)

- sp (settlement point name)

- EnergySchedule

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| sp | K | string | Settlement point | Valid settlement point name |
| tradeId | K, N | string | MP supplied trade ID | Not currently used, reserved for future use |
| buyer | K | string | Trade buyer | Valid QSE |
| seller | K | string | Trade seller | Valid QSE |
| EnergySchedule/startTime | Y | datetime | Start time for capacity schedule | Valid hour boundary |
| EnergySchedule/endTime | N | dateTime | End time for capacity schedule | Valid hour boundary |
| EnergySchedule/ TmPoint/time | Y | integer | Absolute time for start of interval | >= 0 |
| EnergySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 50 - EnergyTrade Requirements

The following is an XML example for an EnergyTrade:

<EnergyTrade xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<buyer>ACME</buyer>

<seller>Cogswell</seller>

<tradeID>4</tradeID>

<sp>ACME23</sp>

<EnergySchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>200</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>220</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>260</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>100</value1>

</TmPoint>

</EnergySchedule>

</EnergyTrade>

### Output Schedule

An OutputSchedule describes the scheduled output for a resource at 5 minute intervals. The following diagram describes the structure of an Output Schedule:

Figure 51 - OutputSchedule Structure

The EnergySchedule structure is identical to the structure described previously in this document. On submission, the following items are required for an OutputSchedule:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for schedule | Valid 5 minute interval boundary for trade date |
| endTime | K | datetime | End time for schedule | Valid 5 minute interval boundary for trade date |
| resource | K | string | Resource | Valid resource name |
| combinedCycle | N | string | Combined cycle | Valid combined cycle ID |
| EnergySchedule/startTime | Y | datetime | Start time for energy schedule | Valid 5 minute boundary |
| EnergySchedule/endTime | N | dateTime | End time for energy schedule | Valid 5 minute boundary |
| EnergySchedule/ TmPoint/time | Y | integer | Absolute time for start of interval | >= 0 |
| EnergySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 52 - OutputSchedule Requirements

The following is an XML example for an OutputSchedule:

<OutputSchedule xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<resource>ResourceXYZ</resource>

<EnergySchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>200</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>220</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>260</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>100</value1>

</TmPoint>

</EnergySchedule>

</OutputSchedule>

### PTP Obligation Bid

The following diagram describes the structure of a PTP Obligation bid:

Figure 53 - PTPObligation Requirements

The CapacitySchedule structure is identical to the structure described previously in this document. The following diagram describes the structure for maximum prices. If more than one MaximumPrice block is provided, start and end times must not overlap.

Figure 54 - MaximumPrice Sub-Structure

On submission, the following items are required for a PTPObligation:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| source | K | string | Source settlement point | Valid settlement point name |
| sink | K | string | Sink settlement point | Valid settlement point name |
| bidId | K | string | MP assigned bid ID |  |
| MaximumPrice/startTime | Y | datetime | Start time for minimum generation costs | Valid hour boundary |
| MaximumPrice/endTime | Y | datetime | End time for generation costs | Valid hour boundary |
| MaximumPrice/cost | Y | decimal | Minimum generation cost | >= 0 |
| CapacitySchedule/startTime | Y | datetime | Start time for capacity schedule | Valid hour boundary |
| CapacitySchedule/endTime | N | dateTime | End time for capacity schedule | Valid hour boundary |
| CapacitySchedule/ TmPoint/time | Y | integer | Absolute start time for beginning of interval | >= 0 |
| CapacitySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 55 - PTPObligation Requirements

The following is an XML example for a PTPObligation:

<PTPObligation xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<source>XYZ3</source>

<sink>ACME34</sink>

<bidId>1</bidId>

<CapacitySchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>30</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>35</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>20</value1>

</TmPoint>

</CapacitySchedule>

<MaximumPrice>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<price>333.5</price>

</MaximumPrice>

<MaximumPrice>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<price>373.5</price>

</MaximumPrice>

</PTPObligation>

### Self Schedule

The following diagram describes the structure of a Self Schedule:

Figure 56 - SelfSchedule Structure

The EnergySchedule structure is defined using a TmSchedule type. On submission, the following items are required for a SelfSchedule:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid 15 minute boundary for trade date |
| endTime | K | datetime | End time for bid | Valid 15 minute boundary for trade date |
| source | K | string | Source settlement point | Valid settlement point name |
| sink | K | string | Sink settlement point | Valid settlement point name |
| EnergySchedule/startTime | Y | datetime | Start time for capacity schedule | Valid 15 minute boundary |
| EnergySchedule/endTime | N | dateTime | End time for capacity schedule | Valid 15 minute boundary |
| EnergySchedule/ TmPoint/time | Y | integer | Absolute time for start of interval | Date time |
| EnergySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 57 - SelfStructure Requirements

The following is an XML example for a SelfSchedule:

<SelfSchedule xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<source>SP456</source>

<sink>ACME23</sink>

<EnergySchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>35</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>50</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

</EnergySchedule>

</SelfSchedule>

## Error Handling

The purpose of this section is to describe the handling of errors related to submissions, updates, queries and cancellations. This is especially important, as BidSets can be partially accepted, where some bids, offers, trades or schedules within a BidSet can be accepted, and others rejected.

| Condition | Action | Error Reported |
| --- | --- | --- |
| Bad parameters in message header | Request is rejected | ERROR: INVALID REQUEST |
| Invalid Source or UserId | Request is rejected | ERROR: NOT AUTHORIZED |
| Payload can not be interpretted | Request is rejected | ERROR: BAD PAYLOAD |
| Parameters within BidSet header are invalid, e.g. bad trading date | No bids or updates are accepted | ERROR: BAD BIDSET |
| Request for a get, cancel or update identifies an unknown transaction (i.e. bid, offer, trade, schedule) | The request is processed only for the known transactions. | WARNING: UNKNOWN ID: …… |
| Some transactions within a BidSet are rejected | Valid transactions are accepted as submitted, invalid transactions are rejected and marked with errors. | BidSet on reply identifies errors for those transactions that were rejected on a transaction by transaction basis. |

Figure 58 - Error Handling

It is also important to note that the submission of a BidSet involves multiple steps:

- BidSet is syntax scanned, where only basic validity checks are performed. The reply identifies where basic validity checks failed. Transactions within the BidSet that pass basic validation checks are marked as ‘SUBMITTED’ and a transaction ID (mRID) value is returned.

- A more thorough validity check is performed asynchronously by the Market Management System (MMS)

- After acceptance or rejection by MMS, a notification is issued that identified whether the transaction was ‘ACCEPTED’ or had error.

The following structure is used to report errors, where the severity, area of the error or the specific hour can be identified.

Note: the Error structure is subject to change.

## Usage and XML Examples

The purpose of this section is to provide usage details with XML examples for the submission (create), query (get), update and cancellation (cancel) of bids, offers, trades and schedules.

### Bid Set Submission

When submitting a set of bids, offers, trades and/or schedules, the following structures are used within the message:

| Message Element | Value |
| --- | --- |
| Header/Verb | create |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload | BidSet |

Figure 59 - create BidSet Request Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | BidSet, returning transaction IDs (mRIDs) for individual bids, offers, trades or schedules and or error messages. |

Figure 60 - reply BidSet Parameters

The following is an example message for a request message for the submission of a BidSet by a Market Participant. The following example uses a SelfArrangedAS.

<msg:Message xsi:noNamespaceSchemaLocation="Message.xsd"

xmlns:wsse="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd"

xmlns:wsu="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd"

xmlns:msg="http://www.ercot.com/schema"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<msg:Header>

<msg:Verb>create</msg:Verb>

<msg:Noun>BidSet</msg:Noun>

<msg:ReplayDetection>

<wsse:Nonce wsu:Id="" EncodingType="">3264874657467444949</wsse:Nonce>

<wsu:Created wsu:Id="">2006-12-09 15:35:57Z</wsu:Created>

</msg:ReplayDetection>

<msg:Revision>001</msg:Revision>

<msg:Source>TXU</msg:Source>

<msg:UserID>Nemat</msg:UserID>

<msg:MessageID>231232466</msg:MessageID>

<msg:Comment>Example message</msg:Comment>

</msg:Header>

<msg:Payload>

<msg:BidSet>

<msg:tradingDate>2007-01-04</msg:tradingDate>

<msg:SelfArrangedAS>

<msg:startTime>2007-12-17T00:00:00-06:00</msg:startTime>

<msg:endTime>2007-12-18T00:00:00-06:00</msg:endTime>

<asType>Non-Spin</asType>

<msg:CapacitySchedule>

<msg:startTime>2007-12-17T00:00:00-06:00</msg:startTime>

<msg:TmPoint>

<msg:time>2007-12-17T00:00:00-06:00</msg:time>

<msg:value1>120</msg:value1>

</msg:TmPoint>

<msg:TmPoint>

<msg:time>2007-12-17T02:00:00-06:00</msg:time>

<msg:value1>130</msg:value1>

</msg:TmPoint>

<msg:TmPoint>

<msg:time>2007-12-17T06:00:00-06:00</msg:time>

<msg:value1>115</msg:value1>

</msg:TmPoint>

</msg:CapacitySchedule>

</msg:SelfArrangedAS>

… additional bids inserted here …

</msg:BidSet>

</msg:Payload>

</msg:Message>

The following is an example response message, where the submission request was successful:

<msg:ResponseMessage xsi:schemaLocation="http://www.ercot.com/schema Message.xsd"

xmlns:wsse="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd"

xmlns:wsu="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd"

xmlns:msg="http://www.ercot.com/schema" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<msg:Header>

<msg:Verb>reply</msg:Verb>

<msg:Noun>BidSet</msg:Noun>

<msg:ReplayDetection>

<wsse:Nonce wsu:Id="" EncodingType="">74646464</wsse:Nonce>

<wsu:Created wsu:Id="">2006-12-09T15:36:03-06:00</wsu:Created>

</msg:ReplayDetection>

<msg:Revision>001</msg:Revision>

<msg:Source>ERCOT</msg:Source>

<msg:UserID>MMS</msg:UserID>

<msg:MessageID>231232466</msg:MessageID>

</msg:Header>

<msg:Reply>

<msg:ReplyCode>OK</msg:ReplyCode>

</msg:Reply>

<msg:Payload>

<msg:BidSet>

<msg:tradingDate>2007-01-04</msg:tradingDate>

<msg:SelfArrangedAS>

<msg:mRID>TXU.20070104.XXXXX</msg:mRID>

<msg:status>SUBMITTED</msg:status>

<msg:/SelfArrangedAS>

<msg:/BidSet>

<msg:/Payload>

</msg:ResponseMessage>

The following is an example response where a submission was unsuccessful, as a consequence of invalid parameters on the BidSet:

<msg:ResponseMessage xsi:schemaLocation="http://www.ercot.com/schema Message.xsd"

xmlns:wsse="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd"

xmlns:wsu="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd"

xmlns:msg="http://www.ercot.com/schema" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<msg:Header>

<msg:Verb>reply</msg:Verb>

<msg:Noun>BidSet</msg:Noun>

<msg:ReplayDetection>

<wsse:Nonce wsu:Id="" EncodingType="">74646464</wsse:Nonce>

<wsu:Created wsu:Id="">2006-12-09T15:36:03-06:00</wsu:Created>

</msg:ReplayDetection>

<msg:Revision>001</msg:Revision>

<msg:Source>ERCOT</msg:Source>

<msg:UserID>MMS</msg:UserID>

<msg:MessageID>231232466</msg:MessageID>

</msg:Header>

<msg:Reply>

<msg:ReplyCode>ERROR</msg:ReplyCode>

<msg:Error>Bad trading date</msg:Error>

</msg:Reply>

</msg:ResponseMessage>

The following is an example response where a submission was unsuccessful, as a consequence of syntax failures within a specific bid/offer/trade/schedule within the BidSet. It is important to note that only the status and errors (if any exist) are returned for each bid.

<msg:ResponseMessage xsi:schemaLocation="http://www.ercot.com/schema Message.xsd"

xmlns:wsse="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd"

xmlns:wsu="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd"

xmlns:msg="http://www.ercot.com/schema" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<msg:Header>

<msg:Verb>reply</msg:Verb>

<msg:Noun>BidSet</msg:Noun>

<msg:ReplayDetection>

<wsse:Nonce wsu:Id="" EncodingType="">74646464</wsse:Nonce>

<wsu:Created wsu:Id="">2006-12-09T15:36:03-06:00</wsu:Created>

</msg:ReplayDetection>

<msg:Revision>001</msg:Revision>

<msg:Source>ERCOT</msg:Source>

<msg:UserID>MMS</msg:UserID>

<msg:MessageID>231232466</msg:MessageID>

</msg:Header>

<msg:Reply>

<msg:ReplyCode>ERROR</msg:ReplyCode>

<msg:Error>Bid syntax errors</msg:Error>

</msg:Reply>

<msg:Payload>

<msg:BidSet>

<msg:tradingDate>2007-01-04</msg:tradingDate>

<msg:XYZ>

<msg:status>ERROR</msg:status>

<msg:error>Unknown bid type XYZ</msg:error>

</msg:XYZ>

<msg:ThreePartOffer>

<msg:status>ERROR</msg:status>

<msg:error>Bad schema</msg:error>

</msg:ThreePartOffer>

<msg:COP>

<msg:mRID>TXU.20070104.YYYYYYYY</msg:mRID>

<msg:status>SUBMITTED</msg:status>

</msg:COP>

... sequence of bids with status and  errors (if any)  appears here ...

</msg:BidSet>

</msg:Payload>

</msg:ResponseMessage>

The more detailed validations errors would typically be available after a period of minutes, where a notification would be issued to identify that errors were found.

An important note is that the order of the bids, offers, trades and schedules within the reply BidSet is identical to the order that was provided in the BidSet for the submission. This is important so that the mRID in the response can be associated with the correct bid, offer, trade or schedule from the submission.

### Querying Bids, Offers, Trades and Schedules

A query request uses the ‘get’ verb on the message header is used to query specific bids, offers, trades and schedules. There are two options for querying a set of bids, offers, trades or schedules:

- The tranaction ID (mRID) for each bid, offer, trade or schedule of interest is identified using the message Request/ID parameter.

- Only the TradingDate is supplied, where all bids, offers, schedules and trades for that trading date will be returned.

In all cases the verb is ‘get’, the noun is ‘BidSet’ and the Request/tradingDate is specified.

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | Optional: one or more mRID values |
| Request/TradingDate | Trading date of interest |

Figure 61 - get BidSet Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | BidSet, returning details for desired bids, offers, trades and schedules |

Figure 62 - reply BidSet Parameters

The following is an example response payload, where the requested trade was not validated:

<BidSet xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<tradingDate>2008-01-01</tradingDate>

<ASTrade xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ERROR</status>

<error><severity>ERROR</severity><text>Invalid AS Type</text></error>

<error><severity>ERROR</severity><text>Invalid seller</text></error>

<error><severity>ERROR</severity><hour>CapacitySchedule</area><hour>20</hour><text>Invalid capacity: -400</text></error>

<buyer>Acme</buyer>

<seller>CogsweLL</seller>

<tradeID>1</tradeID>

<asType>Reg-Up</asType>

<ASSchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T20:00:00-06:00</time>

<value1>-400</value1>

</TmPoint>

</ASSchedule>

</ASTrade>

</BidSet>

### Updating Bids, Offers, Trades and Schedules

A request to update specific bids, offers, trades and schedules is identically the same as a submission request. If the bid, offer, schedule or trade was previously submitted it will be updated. If it did not previously exist, it will be created.

For the purpose of bidding, the ‘create’ and ‘change’ verbs are used interchangeably.

| Message Element | Value |
| --- | --- |
| Header/Verb | change |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload | BidSet |

Figure 63 - change BidSet Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | BidSet, returning transaction IDs (mRIDs) for individual bids, offers, trades or schedules and or error messages. |

Figure 64 - reply BidSet Parameters

The following is an example request payload, where a previously submitted ASTrade will be updated. There may be more than one bid, offer, trade or schedule updated at a time. New submissions may also be mixed with updates.

<BidSet xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<tradingDate>2008-01-01</tradingDate>

<ASTrade xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<buyer>Acme</buyer>

<seller>Cogswell</seller>

<tradeID>1</tradeID>

<asType>Reg-Up</asType>

<ASSchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T20:00:00-06:00</time>

<value1>70</value1>

</TmPoint>

</ASSchedule>

</ASTrade>

</BidSet>

### Canceling Bids, Offers, Trades and Schedules

A cancellation request uses the ‘cancel’ verb on the message header in order to cancel one or more query specific bids, offers, trades and schedules. It is important to note that a COP submission can not be cancelled, as it is required and can otherwise only be updated.

| Message Element | Value |
| --- | --- |
| Header/Verb | cancel |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | Optional: one or more mRID values |

Figure 65 - cancel BidSet Parameters

When canceling a bid, it is also possible to cancel individual hours of a bid. This would be done by suffixing the mRID of the bid to be canceled with the specific hour. If the hour is not provided, all hours will be canceled.

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload/BidSet | BidSet identifying bids cancelled, identifying mRID and status=CANCELED |

Figure 66 - reply BidSet Parameters

Note: Some details of this interface are dependent upon finalization of the MMS design to determine if the cancel will be confirmed synchronously or asynchronously.

The following is an example BidSet that would be provided in the response message:

<BidSet xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<tradingDate>2008-01-01</tradingDate>

<ASOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>CANCELED</status>

</ASOffer>

<ASTrade>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>CANCELED</status>

</ASTrade>

<CapacityTrade>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>CANCELED</status>

</CapacityTrade>

<ThreePartOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>CANCELED</status>

</ThreePartOffer>

</BidSet>

# Market Information

This service is used to request specific types of market-related information. These are read-only interfaces, as opposed to transactional interfaces.

Note: The interfaces described in this section are subject to change based upon final EMS and/or MMS designs.

## Interfaces Provided

Specific interfaces using specific combinations of verbs and nouns (i.e. payload types) are defined to permit a market participant to programmatically access market information. The verb to be used for requests would in all cases be ‘get’. The noun would identify the type of information being requested. Each request could use a message ‘Request’ package to specify one or more parameters that would qualify the request.

The processing sequence is shown in the following sequence diagram.

Figure 67 - Market Information Request Sequence Diagram

## Interfaces Required

The messages for market information requests would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | Name of payload type |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/? | Optional: Other request parameters may be specified as needed |
| Payload | Message payload data with type defined by Noun |

Figure 68 - Parameters for 'get' Requests

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Defined payload type name |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | Defined payload type |

Figure 69 - Parameters for 'reply' Messages

In the cases of payloads that would otherwise exceed 1 megabyte, the payloads would be zipped, base64 encoded and stored within the ‘Payload/Compressed’ tag.

## Message Specifications

Specific payload definitions are not currently defined. These will be defined in future revisions.

### Get AwardSet

The Get AwardSet interface provides the means for a market participant to obtain awards. The following parameters are specified in the RequestMessage:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | AwardSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/TradingDate | Trading date |

Figure 70 - Get AwardSet Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | AwardSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/LoadForecast | AwardSet |

Figure 71 - Get AwardSet Response

An AwardSet is returned in the payload of the ResponseMessage using the following structure:

Figure 72 - AwardSet Container Structure

Within an AwardSet, there are a variety of different Award types. The following diagram describes the basic structure for each type of Award:

Figure 73 - Generalized Award Structure

The specific structured for each type of Award are described in section 5.3.

### Forecasted Load

The purpose of this interface is to provide the means to obtain forecasted load values from ERCOT. Depending upon the time interval specified, the forecasts may be short-term (ST), medium-term (MT) or long term (LT). The input parameters to this request include:

- startTime

- endTime

- zone, which could be a weather zone of a load zone. If either ‘system’ or omitted, the system load forecast is returned.

- Desired forecast type (ST, MT or LT)

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LoadForecast |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |
| Request/Zone | Desired weather or load zone. Default is for system load. |
| Request/Option | Desired forecast type: ST, MT or LT |

Figure 74 - LoadForecast Request Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LoadForecast |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/LoadForecast | Load forecast |

Figure 75 - LoadForecast Response Parameters

The following diagram describes the structure of a load forecast, which is based upon a CIM IrregularIntervalSchedule. The values of ‘value1’ indicate forecasted megawatts. In the case of long-term (LT) forecasts, the value of ‘value2’ will be used to indicate the minimum load for a given day.This will be used to return the data for the specified zone (or system is zone was not specified).

Figure 76 - Load Forecast Structure

### Real-Time System Load

The purpose of this interface is to provide the means to obtain real-time system load from ERCOT. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | SystemLoad |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Optional: Start time of interest |
| Request/EndTime | Optional: End time of interest |

Figure 77 - SystemLoad Request Parameters

If no startTime is specified, the loads for the current day will be returned. The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | SystemLoad |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/SystemLoad | System loads for period of interest |

Figure 78 - SystemLoad Response Parameters

The structure of SystemLoad is described by the following diagram. This used a CIM RegularIntervalSchedule.

Figure 79 - SystemLoad Structure

### Market Totals

The purpose of this interface is to provide a query for market totals. Market totals supported by this interface include:

- Total energy bought in DAM

- Total energy sold in DAM

- Total ancillary service offers received (for each type of ancillary service)

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | MarketTotals |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/option | One of: - AncillaryServiceOffers - EnergyBoughtInDAM - EnergySoldInDAM |
| Request/asType | Ancillary services type (optional, used for option AncillaryServiceOffers) |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

Figure 80 - Market Totals Request Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | MarketTotals |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/MarketTotals | Market totals of interest |

Figure 81 - MarketTotals Response Parameters

The Totals structure is based upon a CIM RegularIntervalSchedule, where the values of ‘value1’ are total megawatt values.

Figure 82 - MarketTotals Structure

### Market LMPs and SPPs

The purpose of this interface is to provide a query for market LMPs and SPPs. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LMPs (or SPPs) |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |
| Request/MarketType | DAM or RTM |
| Request/ID | Optional: bus name for LMP request or settlement point name for SPP request |

Figure 83 - LMP and SPP Request Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LMPs (or SPPs) |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ | LMPs (or SPPs) |

Figure 84 - LMP and SPP Response Parameters

The structure of an LMPs/SPPs payload are shown by the following diagrams, where each LMP/SPP uses a TmSchedule type for a specified bus or SettlementPoint.

Figure 85 LMP Container Structure

Figure 86 - SPP Container Structure

In both cases the data structure is based upon a CIM IrregularIntervalSchedule, where values of ‘value1’ are in $/MWh and values of ‘time’ are seconds from the startTime. The last point on the schedule is valid until the endTime.

### Market MCPCs

The purpose of this interface is to provide a query for MCPCs, for either the DAM or ADJ market.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | MCPCs |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/MarketType | DAM or RTM |
| Request/StartTime | Start time |
| Request/EndTime | End time |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | MCPCs |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/MCPCs | MCPCs |

The following diagram describes the structure of MCPCs:

Figure 87 - MCPC Container Structure

Within each MCPCSchedule, an asType is specified and the values of ‘value1’ convey individual MCPC values.

### Binding Constraints

This section describes interfaces used to retrieve binding constraints. There are several options for retrieval of binding constraints:

- Binding constraints related to DRUC

- Binding constraints related to HRUC

- DAM/RTM shadow prices for binding constraints

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | BindingConstraints |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/Option | DAM/RTM/DRUC/HRUC |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BindingConstraints |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/BindingConstraints | BindingConstraints |

The following diagram describes the structures that are used for the retrieval of binding constraints and shadow prices:

Figure 88 - BindingConstraint Container Structure

The following elements are used to report binding constraints:

- Name of the constraint

- Value that exceeded limit

- Limit defined for constraint

- Maximum shadow price

- Delivery time

- Settlement point names (sp)

- Shift factors

- RUC type (DRUC, HRUC, DAM, RTM)

### Active Contingencies in SCED

This section describes interfaces used to retrieve active contingencies in SCED. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | SCEDViolatedConstraints |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/marketType | RTM or ADJ |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | SCEDViolatedConstraints |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/SCEDViolatedConstraints | SCEDViolatedConstraints |

The structure of SCED violated constraints are described by the following diagram:

Figure 89 - SCEDViolatedConstraints Container Structure

The following elements are used to report violated constraints:

- Name of the constraint

- Value that exceeded limit

- Defined limit for constraint

- Timestamp of violation

- Settlement point names (sp)

- Shift factors

### Ancillary Service Schedule Obligations

The purpose of this interface is to provide ancillary service schedule obligations by service type for the requesting QSE. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | ASObligations |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ASType | Optional: AS type |
| Request/OperatingDate | OperatingDate |

If the AS type is not specified, all AS obligations will be returned. The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | ASObligations |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ASObligations | ASObligations |

The structure of the ASObligations payload is described by the following diagram, using a CIM RegularIntervalSchedule. The values of ‘value1’ would represent the MW obligation and the values of ‘value2’ would represent a positive or negative difference from COP.

Figure 90 - ASObligations Container Structure

### Dynamic Ratings

This section describes interfaces used to retrieve current thermal ratings for conducting equipment. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | DynamicRatings |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | DynamicRatings |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ DynamicRatings | DynamicRatings |

The payload structure is described by the following diagram:

Figure 91 - DynamicRatings Container Structure

The ‘equipment’ tag identifies the name of the transmission element. The types of ratings that are currently defined include:

- Normal (in MVA)

- 2-hour (in MVA)

- 15-min (in MVA)

### Voltage Profiles

This section describes interfaces used to retrieve voltage profiles for electrical buses. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | VoltageProfiles |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | VoltageProfiles |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/VoltageProfiles | VoltageProfiles |

The payload structure is described by the following diagram:

Figure 92 - VoltageProfiles Container Structure

### Total Regulation

This section describes interfaces used to retrieve the daily total reg up and reg down for a given time period. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | TotalRegulation |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | TotalRegulation |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/TotalRegulation | Schedule of total regulation. Value1 = reg up MW, value2 = reg down MW. |

The payload structure is described by the following diagram, using a CIM IrregularIntervalSchedule.

Figure 93 - TotalRegulation Structure

The values of ‘value1’ are the regulation up in megawatts, and the values of ‘value2’ are the regulation down megawatts.

### Load Ratio Share

This section describes interfaces used to retrieve the load ratio share for each QSE. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LoadRatioShares |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LoadRatioShares |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/LoadRatioShares | LoadRatioShares |

The LoadRatioShares payload structure is described by the following diagram:

Figure 94 - LoadRatioShares Container Structure

The values of ‘value1’ would be used to identify the proportion assigned to each QSE for each hour.

### Competitive Constraints

This section describes interfaces used to retrieve competitive constraints. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | CompetitiveConstraints |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | Optional: End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | CompetitiveConstraints |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ CompetitiveConstraints | List of competitive constraints |

The payload structure is described by the following diagram:

Figure 95 - CompetitiveConstraints Container Structure

The following elements are used to report competitive constraints:

- Name of the constraint

- Limit: name of limit

- Value: value that exceeded limit

- Status of constraint

- Timestamp of constraint

- Flag: DAILY, MONTHLY or YEARLY

### Load Distribution Factors

This section describes interfaces used to retrieve load distribution factors. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LoadDistributionFactors |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | ‘SYSTEM’ or ID of weather zone or load group for which the factors for ‘children’ are desired |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LoadDistributionFactors |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ LoadDistributionFactors | LoadDistributionFactors |

The payload structure is described by the following diagram:

Figure 96 - LoadDistributionFactors Container Structure

Each load distribution factor identifies:

- Name of the zone, group or load

- Level, to identify ZONE, LOADGROUP or LOAD

- Factor, as a proportion

### Shift Factors

This section describes interfaces used to retrieve real-time shift factors. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | ShiftFactors |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | ShiftFactors |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ShiftFactors | ShiftFactors |

The payload structure is described by the following diagram, where there is one shift factor for each resource and branch pair:

Figure 97 - ShiftFactors Container Structure

For each shift factor the following elements are provided:

- Name of resource

- Name of branch (equipment)

- Shift factor

### Customer Load Profile

This section describes interfaces used to retrieve the load profile for non-IDR metered customers. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LoadProfile |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | ID of customer(s) |
| Request/StartTime | Start time |
| Request/EndTime | End time |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LoadProfile |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |
| Payload/LoadProfile | LoadProfile |

The payload structure is described by the following diagram:

Figure 98 - LoadProfile Container Structure

The values of ‘value1’ are used to convey the KWh consumption for each time interval.

TBD: The specific key needed to reference a customer load profile needs to be identified.

### Aggregated Ancillary Service Offer Curves

This section describes interfaces used to retrieve aggregated ancillary service offer curves. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | AggregatedASOfferCurves |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time |
| Request/EndTime | End time |
| Request/asType | Optional: AS type (if not specified, all are returned) |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | AggregatedASOfferCurves |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ AggregatedASOfferCurves | AggregatedASOfferCurves |

The payload structure is described by the following diagrams, where there is one curve for each AS type per hour.

Figure 99 - AggregatedASOfferCurve Schedule

For each point on the AggregatedASOfferCurve, the xvalue identifies a megawatt level, the the y1value indicates a $/hour value.

### Ancillary Service System Plan

This section describes interfaces used to retrieve the ancillary service system plan. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | ASSystemPlan |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time |
| Request/EndTime | End time |
| Request/asType | Optional: AS type (if not specified, all are returned) |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | ASSystemPlan |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ASSystemPlan | ASSystemPlan |

The payload structure is described by the following diagram:

Figure 100 - ASSystemPlan Container Structure

Figure 101 - ASSchedule Structure

The values of ‘value1’ identify the total scheduled MW capacity for the given AS type.

### Unit Availability

This section describes interfaces used to retrieve the availability for units. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | UnitAvailabilities |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time |
| Request/EndTime | End time |
| Request/option | Optional: RMR or BlackStart, default is all |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | UnitAvailabilities |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ UnitAvailabilities | UnitAvailabilities |

The payload structure is described by the following diagram:

Figure 102 - UnitAvailabilities Container Structure

For each UnitAvailability element, the following are provided:

- Name of resource

- Contract type: RMR or BlackStart

- Start time

- End time (if known)

- Available (true/false)

- Available megawatts

### Startup and Shutdown Instructions

This section describes interfaces used to retrieve unit startup and shutdown instructions. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | StartupShutdownInstructions |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | StartupShutdownInstructions |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ StartupShutdownInstructions | StartupShutdownInstructions |

The payload structure is described by the following diagram:

Figure 103 - StartupShutdownInstructions Container Structure

For each startup or shutdown instruction, the following elements are provided:

- RUC type (DRUC, HRUC, DAM)

- Delivery date

- Resource name

- Cleared commit

- Cleared decommit

- Reason code for decommit

### Total Ancillary Service Offers

This section describes interfaces used to retrieve total ancillary services offers. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | TotalASOffers |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | TotalASOffers |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ TotalASOffers | TotalASOffers |

The payload structure is described by the following diagrams:

Figure 104 - ASOffers Structure

The values of ‘value1’ identify the total MW offers for the given AS type. There is no consideration given to linked offers.

### Total DAM Energy

This section describes interfaces used to retrieve the total amount of energy bought or sold in the Day Ahead Market. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | TotalEnergy |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time |
| Request/EndTime | End time |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | TotalEnergy |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ TotalEnergy | IrregularIntervalSchedule, where value1=total energy bought and value2=total energy sold |

The payload structure is described by the following diagram:

Figure 105 - TotalEnergy Structure

The values of ‘value1’ indicate the total energy bought and the the values of ‘value2’ indicate the total energy sold.

### Derated CRRs

This section describes interfaces used to retrieve derated CRRs. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | DeratedCRRs |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/OperatingDate | Date of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | DeratedCRRs |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/DeratedCRRs | DeratedCRRs |

The payload structure is described by the following diagram:

Figure 106 - DeratedCRRs Container Structure

### Proxy Curves

This section describes interfaces used to retrieve proxy curves. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | ProxyCurves |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/OperatingDate | Date of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | ProxyCurves |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ProxyCurves | ProxyCurves |

The payload structure is described by the following diagram:

Figure 107 - ProxyCurves Container Structure

The detailed structure of the PriceCurve is described in section 3.3.1.

### Mitigated Curves

This section describes interfaces used to retrieve mitigated curves. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | MitigatedCurves |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/OperatingDate | Date of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | MitigatedCurves |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/MitigatedCurves | MitigatedCurves |

The payload structure is described by the following diagram:

Figure 108 - MitigatedCurves Container Structure

The detailed structure of the PriceCurve is described in section 3.3.1.

# Notifications

A key aspect of the design for external interfaces for Market Participants is the use of the OASIS WS-Notifications standard. Each Market Participant using the external interface would be required to provide an interface for the receipt of notification messages, compliant with the WS-Notifications standard.

## Interfaces Provided

The interfaces provided are described in the WS-Notifications specification. For the purposes of use by Market Participants, there are only two interfaces of interest:

- Notify

- Get Messages (currently it is proposed that this interface not be provided for reasons of performance, in order to avoid periodic polling)

These interfaces are used as a means to asynchronously provide information to Market Participants. Specific examples of this information include:

- Notices

- Alerts

- BidSet acceptance or error reporting

- Awards

- Obligations

- Startup/Shutdown Instructions

The following sequence diagram describes the typical/potential set of information exchanges related to Notifications. The actual flows would be different based on the type of notification.

Figure 109 - Notification Sequence Diagram

Note: This section may be augmented in future versions of this specification.

## Interfaces Required

The contents of the any structure would be wrapped using the ResponseMessage structure defined in section 2.1.4, as this would allow necessary security information to be passed in a uniform manner. The following table describes the contents of the header.

| Message Element | Value |
| --- | --- |
| Header/Verb | created/updated |
| Header/Noun | Type of payload |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | Payload |

In the cases of payloads that would otherwise exceed 1 megabyte, the payloads would be zipped, base64 encoded and stored within the ‘Payload/Compressed’ tag.

## Message Specifications

The WS-Notifications specification identifies a single message structure for notification messages. The payload that is to be delivered by the notification message is represented by the ‘any’ element in the following message structure, loosely coupling the definition of the payload from the definition of the container message structure defined by WS-Notifications thereby allowing WS-Notifications to be used with any XML data structure.

Figure 110 - WS-Notitifications Message Structure

It is important to note that the payloads for NotificationMessages are ALWAYS conveyed using the common message structure defined in section 2.1, with a header and a payload, serving as the ‘any’ sub-structure in the NotificationMessage structure.

### Notices and Alerts

The specific payload for an Alert or Notice is described in the following diagram. This could be used for Notices and Alerts. Where there may be many different types of Notifications, the primary difference between a Notice and an Alert is that there is an expectation that an Alert would be acknowledged.

Figure 111 - Notification Structure

Specific types of notices and alerts are described by the following table:

| Type | Alert? | Description | Protocol Ref | Dest | Data |
| --- | --- | --- | --- | --- | --- |
| SCEDFailure | N | Notice of SCED failure | 6.5.9.2 | All QSEs | Time interval |
| NoCurve | N | Notice that an RTM energy curve or output schedule was not submitted for resource |  | QSE | Resource, time interval |
| NeedAdditionalAS | N | Notice of need for additional AS resources | 3.16 | All QSEs | Needed MW, AS type, time interval |
| Alert | Y | Generic alert | 6.5.9.C.C |  |  |
| Resettlement | N | Notice of resettlement for DAM/RTM | 9.2.6, 9.5.7 | All MPs | Trade Date |
| RTMTrueUp | N | Notice of True-Up settlement timeline changes for RTM | 9.5.9 | All MPs | Trade date, Time of true-up |

Figure 112 - Notice and Alert Types

### Bid Set Acceptance

The purpose of this notification is to indicate the acceptance of a BidSet to a QSE. This is necessary as BidSets are validated some time after the initial submission.

A more detailed description of this process is described in section 3. The following is an example payload where all bids have been accepted.

<BidSet xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<tradingDate>2008-01-01</tradingDate>

<ASOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</ASOffer>

<ASTrade>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</ASTrade>

<CapacityTrade>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</CapacityTrade>

<COP>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</COP>

<CRR>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</CRR>

<DCTieSchedule>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</DCTieSchedule>

<EnergyBid>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</EnergyBid>

<EnergyOnlyOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</EnergyOnlyOffer>

<EnergyTrade>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</EnergyTrade>

<IncDecOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</IncDecOffer>

<OutputSchedule>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</OutputSchedule>

<PTPObligation>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</PTPObligation>

<SelfArrangedAS>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</SelfArrangedAS>

<SelfSchedule>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</SelfSchedule>

<ThreePartOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</ThreePartOffer>

</BidSet>

### Bid Set Errors

The purpose of this message is to indicate that one or more bids in a BidSet had errors. Those bids in a BidSet without errors are otherwise accepted. Error may also indicate the rejection of bids or offers for reasons such as the failure to receive credit approval.

Examples of errors are described in section 3.4. The following is an XML example where some bids were not valid and therefore rejected.

<BidSet xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<tradingDate>2008-01-01</tradingDate>

<ASOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</ASOffer>

<ASTrade>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ERROR</status>

<error><severity>ERROR</severity><text>Invalid AS Type</text></error>

</ASTrade>

<CapacityTrade>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ERROR</status>

<error><severity>ERROR</severity><hour>1</hour><text>Error message as provided by MMS</text></error>

<error><severity>ERROR</severity><hour>2</hour><text>Error message as provided by MMS</text></error>

</CapacityTrade>

<COP>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</COP>

<CRR>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</CRR>

<DCTieSchedule>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ERROR</status>

<error><severity>ERROR</severity><hour>1</hour><text>Error message as provided by MMS</text></error>

<error><severity>ERROR</severity><hour>2</hour><text>Error message as provided by MMS</text></error>

</DCTieSchedule>

<EnergyBid>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</EnergyBid>

<EnergyOnlyOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</EnergyOnlyOffer>

<EnergyTrade>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</EnergyTrade>

<IncDecOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</IncDecOffer>

<OutputSchedule>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</OutputSchedule>

<PTPObligation>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</PTPObligation>

<SelfArrangedAS>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</SelfArrangedAS>

<SelfSchedule>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</SelfSchedule>

<ThreePartOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</ThreePartOffer>

</BidSet>

### Pending Trade

A notification is issued periodically to a QSE where they have been identified as the counter party in a trade, but have not yet entered a matching energy, capacity or ancillary services trade. If a matching trade is not entered by the close of the market, the trade is rejected.

The following structure is used for conveying unconfirmed trades:

Figure 113 - UnconfirmedTrades Container

The details of the structures for ASTrades, CapacityTrades and EnergyTrades are described in sections 3.3.5, 3.3.6 and 3.3.12, respectively. The following is an XML of an unconfirmed trades payload:

<?xml version="1.0" encoding="UTF-8"?>

<UnconfirmedTrades xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<ASTrade>

<mRID>ACME.20080101.YYYYYYYY</mRID>

</ASTrade>

<CapacityTrade>

<mRID>XYZ.20080101.YYYYYYYY</mRID>

</CapacityTrade>

<EnergyTrade>

<mRID>ACME.20080101.YYYYYYYY</mRID>

</EnergyTrade>

</UnconfirmedTrades>

The mRID used in the message is the mRID for the trade that was submitted by the counterparty.

### Energy Offer Awards

The purpose of this message is to identify Energy Offer Awards to a QSE. The structure of this message is shown by the following diagram.

Figure 114 - AwardedEnergyOffer Structure

The following is an XML example:

<AwardedEnergyOffer xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String</mRID>

<marketType>DAM</marketType>

<resource>ACMEUnit1</resource>

<dispatchMWh>110</dispatchMWh>

<spp>22.78</spp>

<lmp>23.28</lmp>

</AwardedEnergyOffer>

### DAM Energy Only Offer Awards

The purpose of this message is to identify DAM Energy Offer Awards to a QSE. The structure of this message is shown by the following diagram.

Figure 115 - AwardedEnergyOnlyOffer Structure

The following is an XML example:

<AwardedEnergyOnlyOffer xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String</mRID>

<marketType>DAM</marketType>

<sp>SP4</sp>

<awardedMWh>50</awardedMWh>

<spp>23.62</spp>

</AwardedEnergyOnlyOffer>

### DAM Energy Bid Award

The purpose of this message is to identify DAM Energy Bid Awards to a QSE. The structure of this message is shown by the following diagram.

Figure 116 - AwardedEnergyBid Structure

The following is an XML example:

<AwardedEnergyBid xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String</mRID>

<marketType>DAM</marketType>

<sp>SP3</sp>

<awardedMWh>70</awardedMWh>

<spp>23.65</spp>

</AwardedEnergyBid>

### Ancillary Service Awards

The purpose of this message is to identify Ancillary Services Awards to a QSE. The structure of this message is shown by the following diagram.

Figure 117 - AwardedAS Structure

The following is an XML example:

<AwardedAS xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String.1</mRID>

<marketType>DAM</marketType>

<resource>ACMEUnit3</resource>

<asType>REG-UP</asType>

<dispatchMW>100</dispatchMW>

<selfSchedMW>80</selfSchedMW>

<mcpc>13.55</mcpc>

</AwardedAS>

### CRR Awards

The purpose of this message is to identify CRR Awards to a QSE. The structure of this message is shown by the following diagram.

Figure 118 - AwardedCRR Structure

The following is an XML example:

<AwardedCRR xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String.1</mRID>

<awardedMW>100</awardedMW>

<source>ABC</source>

<sink>XYZ</sink>

<price>12.56</price>

<crrId>11243</crrId>

</AwardedCRR>

### PTP Obligation Awards

The purpose of this message is to identify PTP Obligation Awards to a QSE. The structure of this message is shown by the following diagram.

Figure 119 - AwardedPTPObligation Structure

The following is an XML example:

<AwardedPTPObligation xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String</mRID>

<marketType>String</marketType>

<awardedMW>0</awardedMW>

<source>SP23</source>

<sink>SP4</sink>

<price>23.76</price>

<bidId>1</bidId>

</AwardedPTPObligation>

### Ancillary Service Obligations

The purpose of this notification message is to identify Ancillary Service Obligations to a QSE. The structure of this message is shown by the following diagram.

Figure 120 – Ancillary Service Obligation Structure

### Outage Notifications

The purpose of this notification is to notify market participants of created, updated or canceled outages. The following structure is used to convey outage notifications.

Figure 121 - Outage Structure

### Startup/Shutdown Instructions

The purpose of this notification is to notify market participants of startup and shutdown instructions. The following structure is used to convey these notifications.

Figure 122 - StartupShutdownInstructions Structure

# Acknowledgement of Alerts

The purpose of this service is to provide the means for a Market Participant to acknowledge Alert messages. Alert messages are sent as notification messages, as described in section 5.

## Interfaces Provided

There is a single operation provided to acknowledge Alert messages.

## Interfaces Required

The messages to acknowledge alerts would use the following message fields, where the ‘close’ verb is used to signify acknowledgement of the Alert.:

| Message Element | Value |
| --- | --- |
| Header/Verb | close |
| Header/Noun | Alert |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | ID of notification |
| Payload | No payload required |

Figure 123 - Alert Acknowledgement Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Alert |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload | None |

Figure 124 - Alert Acknowledgement Response

## Message Specifications

There is no payload used for either request or response messages in conjunction with the acknowledgement of alerts. The ID of the Alert is specified in the Request package within the message.

# Outage Scheduling Interfaces

The purpose of this section is to describe interfaces related to the creation, query and cancellation of outages. These interfaces would be used by QSEs and TPs.

Note: The interfaces in this section may change as a consequence of the MMS design effort.

## Interfaces Provided

Interfaces are provided for the creation, query and cancellation of outages by QSEs and TPs. The interfaces use a basic request reply.

Figure 125 - Outage Scheduling Interface Sequence Diagram

Outages can be submitted by TP for transmission elements, or by QSEs for resources.

There are also notifications to MPs when outages are created or updated. These are described in section 5.y.on elements, or by QSEs for resources. be returned.ustomers.t participant for a given tr

## Message Specifications

### Outage Creation

The request message for outage creation or update would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | create or update |
| Header/Noun | Outage |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload/Outage | Outage |

Figure 126 - Outage Creation Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Outage |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |

Figure 127 - Outage Creation Response

The structure of an Outage record is described by the following diagram:

Figure 128 - Outage Structure

### Outage Schedule Query

The request message for querying outages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | OutageSchedule |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Optional: start time of interest |
| Request/EndTime | Optional: end time of interest |
| Request/ID | Optional: mRID of outage of interest |

Figure 129 - OutageSchedule Query Request

If startTime and endTime are not specified, only active outages will be returned.

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | OutageSchedule |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/OutageSchedule | Outage schedule |

Figure 130 - OutageSchedule Query Response

The structure of the OutageSchedule payload is described by the following diagram. This leverages the structure of an Outage defined previously.

Figure 131 - OutageSchedule Structure

### Outage Cancellation

The request message for outage creation would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | cancel |
| Header/Noun | Outage |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | mRID of outage to be canceled |

Figure 132 - Outage Cancel Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Outage |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |

Figure 133 - Outage Cancel Response

# Utility Interfaces

The purpose of this section is to describe interfaces that provide utility services that are supplemental to the interfaces described in previous sections.

## Interfaces Provided

The interfaces provided include the following:

- The means to status check the ERCOT web services

- The means to generate an mRID from a set of key values

Additional utility interfaces may be defined in the future as needed.

## Message Specifications

### Get mRID

The purpose of this interface is to provide the means to obtain an mRID value that corresponds to a given set of key values for a market product.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | mRID |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload/BidSet | A BidSet that provides the ‘key’ information required for a specific type of market product as needed to construct the desired mRID |

Figure 134 - Get mRID Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | mRID |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Reply/ID | mRID string |

Figure 135 - Get mRID Response

### System Status

The purpose of this interface is to provide the means to verify that a connection can be established with the ERCOT web services through a simple status check. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | SystemStatus |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |

Figure 136 - SystemStatus Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | SystemStatus |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Reply/Timestamp | Identifies current ERCOT local time |

Figure 137 - SystemStatus Response

# Reports

The purpose of this section is to describe interfaces that provide access to reports. This is a web service repackaging of the MID REST Service (PR 500007) that is currently used by Zonal.

## Interfaces Provided

The interfaces provided include the following:

- The means to request the URLs to a set of reports for a given operating date using web services

The actual download of the documents themselves would continue to be done using the URL that is returned for each report.

## Message Specifications

### Get Reports

The purpose of this interface is to provide the means to obtain a list of URLs for reports related to a specific operating date. The URLs can then be used for downloading the reports.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | Reports |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/operatingDate | The operating date of interest |

Figure 138 - Reports Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Reports |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/Reports | List of Report structures, where each Report identifies a number of data items including the URL where the actual report may be downloaded |

Figure 139 - Reports Response

The following structure is used within the payload to convey a list of reports:

Figure 140 - Reports Container

# Appendix A: WS-Notifications

The OASIS WS-Notifications specification can be obtained from http://www.oasis-open.org. The key portions of WS-Notifications that are relevant to this specification are provided here for convenience. The following is the subset of the XML Schema for WS-Notifications that is relevant to this design.

<?xml version="1.0" encoding="UTF-8"?>

<!--

OASIS takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights. Information on OASIS's procedures with respect to rights in OASIS specifications can be found at the OASIS website. Copies of claims of rights made available for publication and any assurances of licenses to be made available, or the result of an attempt made to obtain a general license or permission for the use of such proprietary rights by implementors or users of this specification, can be obtained from the OASIS Executive Director.

OASIS invites any interested party to bring to its attention any copyrights, patents or patent applications, or other proprietary rights which may cover technology that may be required to implement this specification. Please address the information to the OASIS Executive Director.

Copyright (C) OASIS Open (2004-2006). All Rights Reserved.

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works. However, this document itself may not be modified in any way, such as by removing the copyright notice or references to OASIS, except as needed for the purpose of developing OASIS specifications, in which case the procedures for copyrights defined in the OASIS Intellectual Property Rights document must be followed, or as required to translate it into languages other than English.

The limited permissions granted above are perpetual and will not be revoked by OASIS or its successors or assigns.

This document and the information contained herein is provided on an "AS IS" basis and OASIS DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

-->

<xsd:schema

targetNamespace="http://docs.oasis-open.org/wsn/b-2"

xmlns:wsnt="http://docs.oasis-open.org/wsn/b-2"

xmlns:wsa="http://www.w3.org/2005/08/addressing"

xmlns:wsrf-bf="http://docs.oasis-open.org/wsrf/bf-2"

xmlns:wstop="http://docs.oasis-open.org/wsn/t-1"

xmlns:xsd="http://www.w3.org/2001/XMLSchema"

elementFormDefault="qualified"  attributeFormDefault="unqualified">

<!-- ======================== Imports  ============================ -->

<xsd:import namespace="http://www.w3.org/2005/08/addressing"

schemaLocation="http://www.w3.org/2005/08/addressing/ws-addr.xsd"

/>

<xsd:import namespace="http://docs.oasis-open.org/wsrf/bf-2"

schemaLocation="http://docs.oasis-open.org/wsrf/bf-2.xsd"

/>

<xsd:import namespace="http://docs.oasis-open.org/wsn/t-1"

schemaLocation="http://docs.oasis-open.org/wsn/t-1.xsd"

/>

<!-- ================= Notification Metadata  ===================== -->

<xsd:element name="SubscriptionReference"

type="wsa:EndpointReferenceType" />

<xsd:element name="Topic"

type="wsnt:TopicExpressionType" />

<xsd:element name="ProducerReference"

type="wsa:EndpointReferenceType" />

<!-- ================== Message Helper Types  ===================== -->

<xsd:complexType name="NotificationMessageHolderType" >

<xsd:sequence>

<xsd:element ref="wsnt:SubscriptionReference"

minOccurs="0" maxOccurs="1" />

<xsd:element ref="wsnt:Topic"

minOccurs="0" maxOccurs="1" />

<xsd:element ref="wsnt:ProducerReference"

minOccurs="0" maxOccurs="1" />

<xsd:element name="Message">

<xsd:complexType>

<xsd:sequence>

<xsd:any namespace="##any" processContents="lax"

minOccurs="1" maxOccurs="1"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

</xsd:sequence>

</xsd:complexType>

<xsd:element name="NotificationMessage"

type="wsnt:NotificationMessageHolderType"/>

<!-- ========== Message Types for NotificationConsumer  =========== -->

<xsd:element name="Notify" >

<xsd:complexType>

<xsd:sequence>

<xsd:element ref="wsnt:NotificationMessage"

minOccurs="1" maxOccurs="unbounded" />

<xsd:any namespace="##other" processContents="lax"

minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<!-- ======== Message Types for PullPoint  ======================== -->

<xsd:element name="GetMessages">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="MaximumNumber"

type="xsd:nonNegativeInteger"

minOccurs="0"/>

<xsd:any namespace="##other" processContents="lax"

minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

<xsd:anyAttribute/>

</xsd:complexType>

</xsd:element>

<xsd:element name="GetMessagesResponse">

<xsd:complexType>

<xsd:sequence>

<xsd:element ref="wsnt:NotificationMessage"

minOccurs="0" maxOccurs="unbounded" />

<xsd:any namespace="##other" processContents="lax"

minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

<xsd:anyAttribute/>

</xsd:complexType>

</xsd:element>

</xsd:schema>

The following WSDL represents the portion of WS-Notifications that is used for the receipt of Notification messages.

<?xml version="1.0" encoding="utf-8"?>

<!--

OASIS takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights. Information on OASIS's procedures with respect to rights in OASIS specifications can be found at the OASIS website. Copies of claims of rights made available for publication and any assurances of licenses to be made available, or the result of an attempt made to obtain a general license or permission for the use of such proprietary rights by implementors or users of this specification, can be obtained from the OASIS Executive Director.

OASIS invites any interested party to bring to its attention any copyrights, patents or patent applications, or other proprietary rights which may cover technology that may be required to implement this specification. Please address the information to the OASIS Executive Director.

Copyright (C) OASIS Open (2004-2006). All Rights Reserved.

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works. However, this document itself may not be modified in any way, such as by removing the copyright notice or references to OASIS, except as needed for the purpose of developing OASIS specifications, in which case the procedures for copyrights defined in the OASIS Intellectual Property Rights document must be followed, or as required to translate it into languages other than English.

The limited permissions granted above are perpetual and will not be revoked by OASIS or its successors or assigns.

This document and the information contained herein is provided on an "AS IS" basis and OASIS DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

-->

<wsdl:definitions name="WS-BaseNotification"

targetNamespace="http://docs.oasis-open.org/wsn/bw-2"

xmlns:wsntw="http://docs.oasis-open.org/wsn/bw-2"

xmlns:wsnt="http://docs.oasis-open.org/wsn/b-2"

xmlns:wsa="http://www.w3.org/2005/08/addressing"

xmlns:wsrf-rw="http://docs.oasis-open.org/wsrf/rw-2"

xmlns:xsd="http://www.w3.org/2001/XMLSchema"

xmlns:wsdl="http://schemas.xmlsoap.org/wsdl/">

<!-- ========================== Imports =========================== -->

<wsdl:import

namespace="http://docs.oasis-open.org/wsrf/rw-2"

location="http://docs.oasis-open.org/wsrf/rw-2.wsdl"/>

<!-- ===================== Types Definitions ====================== -->

<wsdl:types>

<xsd:schema>

<xsd:import

namespace="http://docs.oasis-open.org/wsn/b-2"

schemaLocation="http://docs.oasis-open.org/wsn/b-2.xsd"/>

</xsd:schema>

</wsdl:types>

<!-- ================ NotificationConsumer::Notify ================

Notify(

NotificationMessage

(SubscriptionReference, TopicExpression, ProducerReference,

Message)*

returns: n/a (one way)

-->

<wsdl:message name="Notify">

<wsdl:part name="Notify" element="wsnt:Notify"/>

</wsdl:message>

<!-- ========== PullPoint::GetMessages ===========

GetMessages(MaximumNumber)

returns: NotificationMessage list

-->

<wsdl:message name="GetMessagesRequest">

<wsdl:part name="GetMessagesRequest"

element="wsnt:GetMessages"/>

</wsdl:message>

<wsdl:message name="GetMessagesResponse">

<wsdl:part name="GetMessagesResponse"

element="wsnt:GetMessagesResponse"/>

</wsdl:message>

<wsdl:message name="UnableToGetMessagesFault">

<wsdl:part name="UnableToGetMessagesFault"

element="wsnt:UnableToGetMessagesFault"/>

</wsdl:message>

<!-- =================== PortType Definitions ===================== -->

<!-- ========= NotificationConsumer PortType Definition =========== -->

<wsdl:portType name="NotificationConsumer">

<wsdl:operation name="Notify">

<wsdl:input message="wsntw:Notify" />

</wsdl:operation>

</wsdl:portType>

<!-- ========== PullPoint PortType Definition ===================== -->

<wsdl:portType name="PullPoint">

<wsdl:operation name="GetMessages">

<wsdl:input  name="GetMessagesRequest"

message="wsntw:GetMessagesRequest" />

<wsdl:output name="GetMessagesResponse"

message="wsntw:GetMessagesResponse" />

<wsdl:fault  name="ResourceUnknownFault"

message="wsrf-rw:ResourceUnknownFault" />

<wsdl:fault  name="UnableToGetMessagesFault"

message="wsntw:UnableToGetMessagesFault" />

</wsdl:operation>

</wsdl:definitions>

# Appendix B: WSDL for Market Requests

This WSDL uses a set of operations for servicing all market requests, related to bidding information requests and alert acknowledgements.

<?xml version="1.0" encoding="UTF-8"?>

<wsdl:definitions xmlns:http="http://schemas.xmlsoap.org/wsdl/http/" xmlns:mime="http://schemas.xmlsoap.org/wsdl/mime/" xmlns:ns1="http://schemas.xmlsoap.org/soap/encoding/" xmlns:soap="http://schemas.xmlsoap.org/wsdl/soap/" xmlns:wsdl="http://schemas.xmlsoap.org/wsdl/" xmlns:xsd="http://www.w3.org/2001/XMLSchema" xmlns:ns="http://www.ercot.com/wsdl/nodal/2006-12" name="Nodal" targetNamespace="http://www.ercot.com/wsdl/nodal/2006-12">

<wsdl:import namespace="http://www.ercot.com/wsdl/nodal/2006-12" location="../../CommonXSD/Message.xsd"/>

<wsdl:types>

<xsd:schema targetNamespace="http://www.ercot.com/wsdl/nodal/2006-12">

<xsd:include schemaLocation="../../CommonXSD/Message.xsd"/>

</xsd:schema>

</wsdl:types>

<wsdl:message name="Request">

<wsdl:part element="ns:RequestMessage" name="Message"/>

</wsdl:message>

<wsdl:message name="Response">

<wsdl:part element="ns:ResponseMessage" name="Message"/>

</wsdl:message>

<wsdl:message name="FaultMessage">

<wsdl:part name="part1" element="ns:FaultMessage"/>

</wsdl:message>

<wsdl:portType name="Nodal">

<wsdl:operation name="MarketTransactions">

<wsdl:input message="ns:Request"/>

<wsdl:output message="ns:Response"/>

<wsdl:fault name="fault1" message="ns:FaultMessage"/>

</wsdl:operation>

<wsdl:operation name="MarketInfo">

<wsdl:input message="ns:Request"/>

<wsdl:output message="ns:Response"/>

<wsdl:fault name="fault1" message="ns:FaultMessage"/>

</wsdl:operation>

<wsdl:operation name="Alerts">

<wsdl:input message="ns:Request"/>

<wsdl:output message="ns:Response"/>

<wsdl:fault name="fault1" message="ns:FaultMessage"/>

</wsdl:operation>

<wsdl:operation name="Reports">

<wsdl:input message="ns:Request"/>

<wsdl:output message="ns:Response"/>

<wsdl:fault name="fault1" message="ns:FaultMessage"/>

</wsdl:operation>

<wsdl:operation name="Utilities">

<wsdl:input message="ns:Request"/>

<wsdl:output message="ns:Response"/>

<wsdl:fault name="fault1" message="ns:FaultMessage"/>

</wsdl:operation>

<wsdl:operation name="OutageScheduling">

<wsdl:input message="ns:Request"/>

<wsdl:output message="ns:Response"/>

<wsdl:fault name="fault1" message="ns:FaultMessage"/>

</wsdl:operation>

</wsdl:portType>

<wsdl:binding name="NodalSOAP" type="ns:Nodal">

<soap:binding style="document" transport="http://schemas.xmlsoap.org/soap/http"/>

<wsdl:operation name="MarketTransactions">

<soap:operation soapAction="http://www.ercot.com/Nodal/MarketTransactions"/>

<wsdl:input>

<soap:body use="literal"/>

</wsdl:input>

<wsdl:output>

<soap:body use="literal"/>

</wsdl:output>

</wsdl:operation>

<wsdl:operation name="MarketInfo">

<soap:operation soapAction="http://www.ercot.com/Nodal/MarketInfo"/>

<wsdl:input>

<soap:body use="literal"/>

</wsdl:input>

<wsdl:output>

<soap:body use="literal"/>

</wsdl:output>

</wsdl:operation>

<wsdl:operation name="Alerts">

<soap:operation soapAction="http://www.ercot.com/Nodal/Alerts"/>

<wsdl:input>

<soap:body use="literal"/>

</wsdl:input>

<wsdl:output>

<soap:body use="literal"/>

</wsdl:output>

</wsdl:operation>

<wsdl:operation name="Reports">

<soap:operation soapAction="http://www.ercot.com/Nodal/Reports"/>

<wsdl:input>

<soap:body use="literal"/>

</wsdl:input>

<wsdl:output>

<soap:body use="literal"/>

</wsdl:output>

</wsdl:operation>

<wsdl:operation name="Utilities">

<soap:operation soapAction="http://www.ercot.com/Nodal/Utilities"/>

<wsdl:input>

<soap:body use="literal"/>

</wsdl:input>

<wsdl:output>

<soap:body use="literal"/>

</wsdl:output>

</wsdl:operation>

<wsdl:operation name="OutageScheduling">

<soap:operation soapAction="http://www.ercot.com/Nodal/OutageScheduling"/>

<wsdl:input>

<soap:body use="literal"/>

</wsdl:input>

<wsdl:output>

<soap:body use="literal"/>

</wsdl:output>

</wsdl:operation>

</wsdl:binding>

<wsdl:service name="Nodal">

<wsdl:port binding="ns:NodalSOAP" name="NodalSOAP">

<soap:address location="http://www.ercot.com/Nodal/"/>

</wsdl:port>

</wsdl:service>

</wsdl:definitions>

# Appendix C: XML Schemas for Message and Payload Definitions

The following XML schema is used to define Message structures for request and response messages for the WSDL defined in Appendix B.

<?xml version="1.0" encoding="UTF-8"?>

<!-- edited with XMLSpy v2007 sp2 (http://www.altova.com) by Scott Neumann (UISOL) -->

<xsd:schema xmlns="http://www.ercot.com/wsdl/nodal/2006-12" xmlns:xsd="http://www.w3.org/2001/XMLSchema" xmlns:wsse="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd" xmlns:wsu="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd" targetNamespace="http://www.ercot.com/wsdl/nodal/2006-12" elementFormDefault="qualified" attributeFormDefault="unqualified">

<xsd:import namespace="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd" schemaLocation="WSS200401wssecurity-secext-10.xsd"/>

<xsd:import namespace="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd" schemaLocation="WSS200401wssecurity-utility-10.xsd"/>

<xsd:element name="Request">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="MarketType" minOccurs="0">

<xsd:simpleType>

<xsd:restriction base="xsd:string">

<xsd:enumeration value="DAM"/>

<xsd:enumeration value="ADJ"/>

<xsd:enumeration value="RTM"/>

</xsd:restriction>

</xsd:simpleType>

</xsd:element>

<xsd:element name="OperatingDate" type="xsd:dateTime" minOccurs="0"/>

<xsd:element name="TradingDate" type="xsd:date" minOccurs="0"/>

<xsd:element name="StartTime" type="xsd:dateTime" minOccurs="0"/>

<xsd:element name="EndTime" type="xsd:dateTime" minOccurs="0"/>

<xsd:element name="Zone" type="xsd:string" minOccurs="0"/>

<xsd:element name="ASType" type="xsd:string" minOccurs="0"/>

<xsd:element name="Option" type="xsd:string" minOccurs="0"/>

<xsd:element name="ID" type="xsd:string" minOccurs="0" maxOccurs="unbounded"/>

<xsd:any namespace="##other" minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="Reply">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="ReplyCode" type="xsd:string"/>

<xsd:element name="Error" type="xsd:string" minOccurs="0" maxOccurs="unbounded"/>

<xsd:element name="Timestamp" type="xsd:dateTime" minOccurs="0"/>

<xsd:element name="ID" type="xsd:string" minOccurs="0"/>

<xsd:any namespace="##other" minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="Payload">

<xsd:complexType>

<xsd:sequence>

<xsd:choice>

<xsd:any namespace="##other" minOccurs="0" maxOccurs="unbounded"/>

<xsd:element name="Document" type="xsd:string" minOccurs="0" maxOccurs="unbounded"/>

<xsd:element name="Compressed" type="xsd:string" minOccurs="0"/>

</xsd:choice>

<xsd:element name="format" type="xsd:string" minOccurs="0"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="ReplayDetection">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="Nonce" type="wsse:EncodedString"/>

<xsd:element name="Created" type="wsu:AttributedDateTime"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="Header">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="Verb" default="get">

<xsd:simpleType>

<xsd:restriction base="xsd:string">

<xsd:enumeration value="cancel"/>

<xsd:enumeration value="canceled"/>

<xsd:enumeration value="change"/>

<xsd:enumeration value="changed"/>

<xsd:enumeration value="create"/>

<xsd:enumeration value="created"/>

<xsd:enumeration value="close"/>

<xsd:enumeration value="closed"/>

<xsd:enumeration value="delete"/>

<xsd:enumeration value="deleted"/>

<xsd:enumeration value="get"/>

<xsd:enumeration value="reply"/>

<xsd:enumeration value="submit"/>

<xsd:enumeration value="update"/>

<xsd:enumeration value="updated"/>

</xsd:restriction>

</xsd:simpleType>

</xsd:element>

<xsd:element name="Noun" type="xsd:string"/>

<xsd:element ref="ReplayDetection"/>

<xsd:element name="Revision" type="xsd:string" default="001"/>

<xsd:element name="Source" type="xsd:string"/>

<xsd:element name="UserID" type="xsd:string" minOccurs="0"/>

<xsd:element name="MessageID" type="xsd:string" minOccurs="0"/>

<xsd:element name="Comment" type="xsd:string" minOccurs="0"/>

<xsd:any namespace="##other" minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="Message">

<xsd:complexType>

<xsd:sequence>

<xsd:element ref="Header"/>

<xsd:choice>

<xsd:element ref="Request" minOccurs="0"/>

<xsd:element ref="Reply" minOccurs="0"/>

</xsd:choice>

<xsd:element ref="Payload" minOccurs="0"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="RequestMessage">

<xsd:complexType>

<xsd:sequence>

<xsd:element ref="Header"/>

<xsd:element ref="Request" minOccurs="0"/>

<xsd:element ref="Payload" minOccurs="0"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="ResponseMessage">

<xsd:complexType>

<xsd:sequence>

<xsd:element ref="Header"/>

<xsd:element ref="Reply" minOccurs="0"/>

<xsd:element ref="Payload" minOccurs="0"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="FaultMessage">

<xsd:complexType>

<xsd:sequence>

<xsd:element ref="Reply"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

</xsd:schema>

The following XML schema provides examples for the definition of structures for payload definitions. The focus of the examples provided here are for bids, offers and ERCOT notifications. The examples provide here should be extended with valid enumerations and defaults in some cases. This XML schema is referenced by the preceding WSDL definition.

<?xml version="1.0" encoding="UTF-8"?>

<!-- edited with XMLSpy v2007 sp2 (http://www.altova.com) by Scott Neumann (UISOL) -->

<xs:schema xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/wsdl/nodal/2006-12/mms" elementFormDefault="qualified" attributeFormDefault="unqualified">

<!--Simple Type Definitions-->

<xs:simpleType name="URL">

<xs:annotation>

<xs:documentation>URL</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:anyURI"/>

</xs:simpleType>

<xs:simpleType name="SettlementPoint">

<xs:annotation>

<xs:documentation>Settlement point, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="ElectricalBus">

<xs:annotation>

<xs:documentation>Electrical bus, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="QSE">

<xs:annotation>

<xs:documentation>QSE type, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="ASType">

<xs:annotation>

<xs:documentation>Ancillary services type</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="Reg-Up"/>

<xs:enumeration value="Reg-Down"/>

<xs:enumeration value="Non-Spin"/>

<xs:enumeration value="RRS"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="RUCType">

<xs:annotation>

<xs:documentation>RUC type</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="DRUC"/>

<xs:enumeration value="HRUC"/>

<xs:enumeration value="DAM"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="OperatingMode">

<xs:annotation>

<xs:documentation>Operating mode type</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="ONRUC"/>

<xs:enumeration value="ON"/>

<xs:enumeration value="ONREG"/>

<xs:enumeration value="ONTEST"/>

<xs:enumeration value="ONOS"/>

<xs:enumeration value="ONOSREG"/>

<xs:enumeration value="ONDSRREG"/>

<xs:enumeration value="ONDSR"/>

<xs:enumeration value="OFF"/>

<xs:enumeration value="OFFNS"/>

<xs:enumeration value="ONEMR"/>

<xs:enumeration value="ONRR"/>

<xs:enumeration value="OUT"/>

<xs:enumeration value="EMR"/>

<xs:enumeration value="ONRGL"/>

<xs:enumeration value="ONRRCLR"/>

<xs:enumeration value="ONRL"/>

<xs:enumeration value="OTL"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Resource">

<xs:annotation>

<xs:documentation>Resource, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="CombinedCycle">

<xs:annotation>

<xs:documentation>Combined cycle configuration</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="Equipment">

<xs:annotation>

<xs:documentation>Equipment, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="EnergyConsumer">

<xs:annotation>

<xs:documentation>EnergyConsumer</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="DayType">

<xs:annotation>

<xs:documentation>Day type</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<!--CIM type definitions-->

<xs:complexType name="IdentifiedObject">

<xs:annotation>

<xs:documentation>CIM IdentifiedObject superclass</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="name"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PowerSystemResource">

<xs:annotation>

<xs:documentation>CIM PSR superclass</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="IdentifiedObject"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="ConductingEquipment">

<xs:annotation>

<xs:documentation>CIM Conducting Equipment</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="PowerSystemResource"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="IntervalSchedule">

<xs:annotation>

<xs:documentation>IntervalSchedule is used to schedule values for give time intervals</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="operatingDate" type="xs:date">

<xs:annotation>

<xs:documentation>Operating date</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="IntervalData" type="IntervalData" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="IntervalData">

<xs:annotation>

<xs:documentation>IntervalData defines a set of values for one or more intervals</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startInterval" type="xs:time">

<xs:annotation>

<xs:documentation>Starting interval, identified by end time in format HH:MM</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="endInterval" type="xs:time">

<xs:annotation>

<xs:documentation>Ending interval, identified by end time in format HH:MM</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value1" type="xs:integer">

<xs:annotation>

<xs:documentation>Data value 1</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value2" type="xs:integer" minOccurs="0">

<xs:annotation>

<xs:documentation>Data value 2</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

<xs:attribute name="repeatedHour" type="xs:boolean" default="false">

<xs:annotation>

<xs:documentation>Flag to indicate change from CDT->CST</xs:documentation>

</xs:annotation>

</xs:attribute>

</xs:complexType>

<xs:complexType name="TmSchedule">

<xs:annotation>

<xs:documentation>Time schedule, with a set of values ordered by absolute time within the startTime and endTime</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="BasicIntervalSchedule">

<xs:sequence>

<xs:element name="TmPoint" type="TmPoint" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="TmPoint">

<xs:annotation>

<xs:documentation>Timepoints for TmSchedule</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="time" type="xs:dateTime">

<xs:annotation>

<xs:documentation>Absolute time start time for the data values</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ending" type="xs:dateTime" minOccurs="0">

<xs:annotation>

<xs:documentation>Ending time for values, if not  beginning of next TmPoint or end of TmSchedule</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value1" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>Data value 1</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value2" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>Data value 2</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value3" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>Data value 3</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="BasicIntervalSchedule">

<xs:annotation>

<xs:documentation>CIM Basic Interval Schedule</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Curve">

<xs:annotation>

<xs:documentation>CIM Curve</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="IdentifiedObject">

<xs:sequence>

<xs:element name="curveStyle" type="xs:string" minOccurs="0"/>

<xs:element name="CurveData" type="CurveData" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="CurveData">

<xs:annotation>

<xs:documentation>CIm Curve Data</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="xvalue" type="xs:float" minOccurs="0"/>

<xs:element name="y1value" type="xs:float" minOccurs="0"/>

<xs:element name="y2value" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Document">

<xs:annotation>

<xs:documentation>CIM Document superclass</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="mRID" type="xs:string" minOccurs="0"/>

<xs:element name="docType" type="xs:string"/>

<xs:element name="lastModified" type="xs:dateTime"/>

<xs:element name="comments" type="xs:string" minOccurs="0"/>

<xs:element name="docStatus" type="xs:string" minOccurs="0"/>

<xs:element name="docTitle" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PNode">

<xs:annotation>

<xs:documentation>CIM Pricing Node</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="name" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="LoadForecast">

<xs:annotation>

<xs:documentation>Forecasted load: value1Unit is MW</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="RegulatingCondEq">

<xs:annotation>

<xs:documentation>CIM Regulating conducting equipment</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="ConductingEquipment"/>

</xs:complexContent>

</xs:complexType>

<!--Types for bidding-->

<xs:complexType name="MarketRequest">

<xs:annotation>

<xs:documentation>Market common request parameters</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="tradingDate" type="xs:date"/>

<xs:element name="status" type="xs:string" minOccurs="0"/>

<xs:element name="mode" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PriceCurve">

<xs:annotation>

<xs:documentation>Relationship between a price in $/hour (Y-axis) and a MW value (X-axis). </xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime">

<xs:annotation>

<xs:documentation>Start time to identify when curve is used</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="endTime" type="xs:dateTime">

<xs:annotation>

<xs:documentation>End time to indicate when curve is no longer used</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="curveStyle" minOccurs="0">

<xs:annotation>

<xs:documentation>Style of curve: FIXED, VARIABLE, CURVE</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="FIXED"/>

<xs:enumeration value="VARIABLE"/>

<xs:enumeration value="CURVE"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="CurveData" maxOccurs="11">

<xs:complexType>

<xs:annotation>

<xs:documentation>The point data values that define a curve</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="xvalue" type="xs:float">

<xs:annotation>

<xs:documentation>The data value of the X-axis variable,  depending on the X-axis units</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="y1value" type="xs:float">

<xs:annotation>

<xs:documentation>The data value of the  first Y-axis variable, depending on the Y-axis units</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="incExcFlag" minOccurs="0">

<xs:annotation>

<xs:documentation>Inclusive/Exclusive flag for three part offers to identify whether ancillary services are included or excluded</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="INC"/>

<xs:enumeration value="EXC"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="reason" minOccurs="0">

<xs:annotation>

<xs:documentation>Reason for adjustment</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="OUT"/>

<xs:enumeration value="FUEL"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="BidSet">

<xs:annotation>

<xs:documentation>CIM container class for bids and offers</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="MarketRequest">

<xs:sequence>

<xs:element name="ASOffer" type="ASOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ASTrade" type="ASTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="CapacityTrade" type="CapacityTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="COP" type="COP" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="CRR" type="CRR" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="DCTieSchedule" type="DCTieSchedule" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="EnergyBid" type="EnergyBid" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="EnergyOnlyOffer" type="EnergyOnlyOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="EnergyTrade" type="EnergyTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="IncDecOffer" type="IncDecOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="OutputSchedule" type="OutputSchedule" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="PTPObligation" type="PTPObligation" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="SelfArrangedAS" type="SelfArrangedAS" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="SelfSchedule" type="SelfSchedule" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ThreePartOffer" type="ThreePartOffer" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="FipFop">

<xs:annotation>

<xs:documentation>FIP FOP percentages</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="fipPercent" type="xs:float"/>

<xs:element name="fopPercent" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="StartupCost">

<xs:annotation>

<xs:documentation>Resource startup costs</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="hot" type="xs:float" minOccurs="0"/>

<xs:element name="intermediate" type="xs:float" minOccurs="0"/>

<xs:element name="cold" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MinimumEnergy">

<xs:annotation>

<xs:documentation>Resource minimum generation costs</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="cost" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MaximumPrice">

<xs:annotation>

<xs:documentation>Maximum prices</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="price" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MinimumPrice">

<xs:annotation>

<xs:documentation>Minimum prices</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="price" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ResourceStatus">

<xs:annotation>

<xs:documentation>Resource Status </xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="operatingMode" type="OperatingMode"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Limits">

<xs:annotation>

<xs:documentation>Resource Limits</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="hsl" type="xs:int"/>

<xs:element name="lsl" type="xs:int"/>

<xs:element name="hel" type="xs:int"/>

<xs:element name="lel" type="xs:int"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ASCapacity">

<xs:annotation>

<xs:documentation>Ancillary Services Capacity</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="regUp" type="xs:int" minOccurs="0"/>

<xs:element name="regDown" type="xs:int" minOccurs="0"/>

<xs:element name="rrs" type="xs:int" minOccurs="0"/>

<xs:element name="nonSpin" type="xs:int" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="NERCTags">

<xs:annotation>

<xs:documentation>NERC Tags</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="tag" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="CapacitySchedule">

<xs:annotation>

<xs:documentation>Capacity Schedule: value1Unit is MW</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="PriceSchedule">

<xs:annotation>

<xs:documentation>Price Schedules: value1Unit is $/MW</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="EnergySchedule">

<xs:annotation>

<xs:documentation>MW Schedules: value1Unit is MW</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="Error">

<xs:annotation>

<xs:documentation>Error structure</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="text" type="xs:string"/>

<xs:element name="severity" minOccurs="0">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="ERROR"/>

<xs:enumeration value="WARNING"/>

<xs:enumeration value="INFORMATIVE"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="area" type="xs:string" minOccurs="0"/>

<xs:element name="hour" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<!--Parent classes for bidding-->

<xs:complexType name="Bid">

<xs:annotation>

<xs:documentation>CIM Bid class</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="endTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="mRID" type="xs:string" minOccurs="0"/>

<xs:element name="marketType" type="xs:string" minOccurs="0"/>

<xs:element name="status" type="xs:string" minOccurs="0"/>

<xs:element name="error" type="Error" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Trade">

<xs:annotation>

<xs:documentation>Abstract super class for trades</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="buyer" type="QSE" minOccurs="0"/>

<xs:element name="seller" type="QSE" minOccurs="0"/>

<xs:element name="tradeID" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="Offer">

<xs:annotation>

<xs:documentation>Abstrract super class for offers</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="expirationTime" type="xs:dateTime" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<!--Market products-->

<xs:complexType name="ThreePartOffer">

<xs:annotation>

<xs:documentation>Three Part Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Offer">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="combinedCycle" type="CombinedCycle" minOccurs="0"/>

<xs:element name="FipFop" type="FipFop" minOccurs="0" maxOccurs="25"/>

<xs:element name="StartupCost" type="StartupCost" minOccurs="0" maxOccurs="25"/>

<xs:element name="MinimumEnergy" type="MinimumEnergy" minOccurs="0" maxOccurs="25"/>

<xs:element name="EnergyOfferCurve" type="PriceCurve" minOccurs="0" maxOccurs="25"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="COP">

<xs:annotation>

<xs:documentation>Current Operating Plan (should consider adaptation of CIM GenUnitOpSchedule)</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="combinedCycle" type="xs:string" minOccurs="0"/>

<xs:element name="ResourceStatus" type="ResourceStatus" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Limits" type="Limits" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ASCapacity" type="ASCapacity" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="IncDecOffer">

<xs:annotation>

<xs:documentation>Incremental Decremental Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Offer">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="combinedCycle" type="CombinedCycle" minOccurs="0"/>

<xs:element name="FipFop" type="FipFop" minOccurs="0"/>

<xs:element name="IncrementalCurve" type="PriceCurve" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="DecrementalCurve" type="PriceCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="EnergyBid">

<xs:annotation>

<xs:documentation>Energy Bid</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="expirationTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="sp" type="SettlementPoint" minOccurs="0"/>

<xs:element name="bidID" type="xs:string" minOccurs="0"/>

<xs:element name="PriceCurve" type="PriceCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="EnergyOnlyOffer">

<xs:annotation>

<xs:documentation>Energy Only Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Offer">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint" minOccurs="0"/>

<xs:element name="bidID" type="xs:string" minOccurs="0"/>

<xs:element name="EnergyOfferCurve" type="PriceCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="EnergyTrade">

<xs:annotation>

<xs:documentation>Energy Trade</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Trade">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint" minOccurs="0"/>

<xs:element name="EnergySchedule" type="EnergySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="CapacityTrade">

<xs:annotation>

<xs:documentation>Capacity Trade</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Trade">

<xs:sequence>

<xs:element name="CapacitySchedule" type="CapacitySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="ASTrade">

<xs:annotation>

<xs:documentation>Ancillary Services Trade</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Trade">

<xs:sequence>

<xs:element name="asType" type="ASType" minOccurs="0"/>

<xs:element name="ASSchedule" type="TmSchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="ASOffer">

<xs:annotation>

<xs:documentation>Ancillary Services Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Offer">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="combinedCycle" type="xs:string" minOccurs="0"/>

<xs:element name="asType" type="ASType" minOccurs="0"/>

<xs:element name="PriceCurve" type="PriceCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="SelfArrangedAS">

<xs:annotation>

<xs:documentation>Self-Arranged Ancillary Services</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="asType" type="ASType" minOccurs="0"/>

<xs:element name="CapacitySchedule" type="CapacitySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="CRR">

<xs:annotation>

<xs:documentation>Congestion Revenue Rights</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="crrId" minOccurs="0"/>

<xs:element name="source" type="SettlementPoint" minOccurs="0"/>

<xs:element name="sink" type="SettlementPoint" minOccurs="0"/>

<xs:element name="CapacitySchedule" type="CapacitySchedule" minOccurs="0"/>

<xs:element name="MinimumReservationPrice" type="MinimumPrice" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="NOIEPeakLoadForecast" type="LoadForecast" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="PTPObligation">

<xs:annotation>

<xs:documentation>PTP Obligation</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="source" type="SettlementPoint" minOccurs="0"/>

<xs:element name="sink" type="SettlementPoint" minOccurs="0"/>

<xs:element name="bidId" type="xs:string" minOccurs="0"/>

<xs:element name="CapacityShedule" type="TmSchedule" minOccurs="0"/>

<xs:element name="MaximumPrice" type="MaximumPrice" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<!--Schedules-->

<xs:complexType name="Schedule">

<xs:annotation>

<xs:documentation>CIM Schedule class</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="endTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="mRID" type="xs:string" minOccurs="0"/>

<xs:element name="marketType" type="xs:string" minOccurs="0"/>

<xs:element name="status" type="xs:string" minOccurs="0"/>

<xs:element name="error" type="Error" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="OutputSchedule">

<xs:annotation>

<xs:documentation>Output Schedule</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="EnergySchedule" type="EnergySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="DCTieSchedule">

<xs:annotation>

<xs:documentation>DC Tie Schedule</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="buyer" type="xs:string" minOccurs="0"/>

<xs:element name="seller" type="xs:string" minOccurs="0"/>

<xs:element name="sp" type="SettlementPoint" minOccurs="0"/>

<xs:element name="EnergySchedule" type="EnergySchedule" minOccurs="0"/>

<xs:element name="NERCTags" type="NERCTags" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="SelfSchedule">

<xs:annotation>

<xs:documentation>Self schedules</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="source" type="SettlementPoint" minOccurs="0"/>

<xs:element name="sink" type="SettlementPoint" minOccurs="0"/>

<xs:element name="EnergySchedule" type="EnergySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<!--Notifications-->

<xs:complexType name="Notification">

<xs:annotation>

<xs:documentation> Notitication class</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="notificationID" type="xs:string"/>

<xs:element name="notificationType" type="xs:string"/>

<xs:element name="priority" type="xs:int"/>

<xs:element name="source" type="xs:string"/>

<xs:element name="issued" type="xs:dateTime"/>

<xs:element name="expiration" type="xs:dateTime" minOccurs="0"/>

<xs:element name="summary" type="xs:string"/>

<xs:element name="details" type="xs:string" minOccurs="0"/>

<xs:element name="objectID" type="xs:string" minOccurs="0"/>

<xs:element name="referenceURL" type="xs:string" minOccurs="0"/>

<xs:any namespace="##other" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="UnconfirmedTrades">

<xs:annotation>

<xs:documentation>Unconfirmed trades</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="ASTrade" type="ASTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="CapacityTrade" type="CapacityTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="EnergyTrade" type="EnergyTrade" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<!--Awards-->

<xs:complexType name="AwardSet">

<xs:annotation>

<xs:documentation>Container for awards</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="tradingDate" type="xs:date"/>

<xs:element name="marketType" type="xs:string" minOccurs="0"/>

<xs:sequence>

<xs:element name="AwardedAS" type="AwardedAS" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedCRR" type="AwardedCRR" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedEnergyBid" type="AwardedEnergyBid" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedEnergyOffer" type="AwardedEnergyOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedEnergyOnlyOffer" type="AwardedEnergyOnlyOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedPTPObligation" type="AwardedPTPObligation" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Award">

<xs:annotation>

<xs:documentation>Award superclass</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="qse" type="QSE"/>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="tradingDate" type="xs:date"/>

<xs:element name="mRID" type="xs:string"/>

<xs:element name="marketType" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="AwardedAS">

<xs:annotation>

<xs:documentation>Ancillary Service Award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="asType" type="ASType"/>

<xs:element name="dispatchMW" type="xs:int" minOccurs="0"/>

<xs:element name="selfSchedMW" type="xs:int" minOccurs="0"/>

<xs:element name="mcpc" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedEnergyOffer">

<xs:annotation>

<xs:documentation>EnergyAward Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="dispatchMWh" type="xs:int"/>

<xs:element name="spp" type="xs:float" minOccurs="0"/>

<xs:element name="lmp" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedCRR">

<xs:annotation>

<xs:documentation>CRR award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="awardedMW" type="xs:int"/>

<xs:element name="source" type="SettlementPoint"/>

<xs:element name="sink" type="SettlementPoint"/>

<xs:element name="price" type="xs:float"/>

<xs:element name="crrId" type="xs:string"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedEnergyBid">

<xs:annotation>

<xs:documentation>Energy Bid Award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint"/>

<xs:element name="awardedMWh" type="xs:int"/>

<xs:element name="spp" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedPTPObligation">

<xs:annotation>

<xs:documentation>PTP Obligation award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="awardedMW" type="xs:int"/>

<xs:element name="source" type="SettlementPoint"/>

<xs:element name="sink" type="SettlementPoint"/>

<xs:element name="price" type="xs:float"/>

<xs:element name="bidId" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedEnergyOnlyOffer">

<xs:annotation>

<xs:documentation>Energy Only Offer Award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint"/>

<xs:element name="awardedMWh" type="xs:int"/>

<xs:element name="spp" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="ASObligation">

<xs:annotation>

<xs:documentation>Ancillary Service Obligation</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule">

<xs:sequence>

<xs:element name="asType" type="ASType"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<!--Market Information-->

<xs:complexType name="VoltageProfile">

<xs:complexContent>

<xs:extension base="TmSchedule">

<xs:sequence>

<xs:element name="bus" type="ElectricalBus"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="LMP">

<xs:annotation>

<xs:documentation>Locational Marginal Price</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="PriceSchedule">

<xs:sequence>

<xs:element name="bus" type="ElectricalBus"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="SPP">

<xs:annotation>

<xs:documentation>Settlement Point Price</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="PriceSchedule">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="MCPCSchedule">

<xs:complexContent>

<xs:extension base="PriceSchedule">

<xs:sequence>

<xs:element name="asType" type="ASType"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="LoadRatioShare">

<xs:complexContent>

<xs:extension base="TmSchedule">

<xs:sequence>

<xs:element name="qse" type="QSE"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="Constraint">

<xs:complexContent>

<xs:extension base="IdentifiedObject">

<xs:sequence>

<xs:element name="value">

<xs:annotation>

<xs:documentation>Value that triggered constraint</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="limit">

<xs:annotation>

<xs:documentation>Limit defined for constraint</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="maxShadowPrice" minOccurs="0">

<xs:annotation>

<xs:documentation>Maximum shadow price</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="SettlementPointFactor">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint"/>

<xs:element name="shiftFactor" type="xs:decimal"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="SCEDViolatedConstraint">

<xs:complexContent>

<xs:extension base="Constraint">

<xs:sequence>

<xs:element name="timestamp" type="xs:dateTime"/>

<xs:element name="SF" type="SettlementPointFactor" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="BindingConstraint">

<xs:complexContent>

<xs:extension base="Constraint">

<xs:sequence>

<xs:element name="deliveryTime" type="xs:dateTime"/>

<xs:element name="SF" type="SettlementPointFactor" maxOccurs="unbounded"/>

<xs:element name="rucType" type="RUCType" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="CompetitiveConstraint">

<xs:complexContent>

<xs:extension base="IdentifiedObject">

<xs:sequence>

<xs:element name="limit" type="xs:float"/>

<xs:element name="value" type="xs:float"/>

<xs:element name="status" type="xs:string"/>

<xs:element name="timestamp" type="xs:dateTime"/>

<xs:element name="flag">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="DAILY"/>

<xs:enumeration value="MONTHLY"/>

<xs:enumeration value="YEARLY"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="LoadDistributionFactor">

<xs:sequence>

<xs:element name="name"/>

<xs:element name="level">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="ZONE"/>

<xs:enumeration value="LOADGROUP"/>

<xs:enumeration value="LOAD"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="factor" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ShiftFactor">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="equipment" type="Equipment"/>

<xs:element name="factor" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="AggregatedASOfferCurve">

<xs:sequence>

<xs:element name="asType" type="ASType"/>

<xs:element name="PriceCurve" type="PriceCurve"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ASSystemPlan">

<xs:sequence>

<xs:element name="startTime"/>

<xs:element name="endTime"/>

<xs:element name="asType"/>

<xs:element name="quantity"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ASSchedule">

<xs:sequence>

<xs:element name="asType" type="ASType"/>

<xs:element name="Schedule" type="TmSchedule"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="StartupShutdown">

<xs:annotation>

<xs:documentation>Unit startup and shitdown instructions</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="rucType" type="RUCType"/>

<xs:element name="deliveryDate" type="xs:dateTime"/>

<xs:element name="resource" type="Resource"/>

<xs:element name="clearedCommit" type="xs:boolean"/>

<xs:element name="clearedDecommit" type="xs:boolean"/>

<xs:element name="decommitReasonCode" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="UnitAvailability">

<xs:annotation>

<xs:documentation>Unit availability schedule</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="contractType" minOccurs="0">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="RMR"/>

<xs:enumeration value="BlackStart"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="available" type="xs:boolean"/>

<xs:element name="availableMW" type="xs:integer" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="DeratedCRR">

<xs:annotation>

<xs:documentation>Derated CRRs</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="crrHolder" type="xs:string"/>

<xs:element name="crrId" type="xs:string"/>

<xs:element name="source" type="SettlementPoint"/>

<xs:element name="sink" type="SettlementPoint"/>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="inputQuantity" type="xs:integer"/>

<xs:element name="deratedQuantity" type="xs:integer"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="DynamicRating">

<xs:annotation>

<xs:documentation>Dynamic rating</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="equipment" type="Equipment"/>

<xs:element name="rating" type="Rating" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Rating">

<xs:annotation>

<xs:documentation>Rating</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="ratingType">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="Normal"/>

<xs:enumeration value="2-hour"/>

<xs:enumeration value="15-min"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="ratingValue"/>

<xs:element name="units" minOccurs="0">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="MVA"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="NonConformLoadSchedule">

<xs:annotation>

<xs:documentation>Non conforming load schedule</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule">

<xs:sequence>

<xs:element name="EnergyConsumer" type="EnergyConsumer"/>

<xs:element name="DayType" type="DayType" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="VoltageLimits">

<xs:sequence>

<xs:element name="bus" type="ElectricalBus"/>

<xs:element name="minimumKV" type="xs:float"/>

<xs:element name="maximumKV" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="TapLimits">

<xs:sequence>

<xs:element name="equipment" type="Equipment"/>

<xs:element name="minimumStep" type="xs:integer"/>

<xs:element name="maximumStep" type="xs:integer"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ProxyCurve">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="PriceCurve" type="PriceCurve"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MitigatedCurve">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="PriceCurve" type="PriceCurve"/>

</xs:sequence>

</xs:complexType>

<!--Model Management-->

<xs:complexType name="Project">

<xs:annotation>

<xs:documentation>Project parent class</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Document">

<xs:sequence>

<xs:element name="participant" type="xs:string"/>

<xs:element name="projectName" type="xs:string"/>

<xs:element name="projectOwner" type="xs:string"/>

<xs:element name="projectType" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="compressedDataFile" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="energizationDate" type="xs:dateTime" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="NOMCR">

<xs:annotation>

<xs:documentation>Network Operating Model Change Request</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Project">

<xs:sequence>

<xs:element name="originalNOMCR" minOccurs="0"/>

<xs:element name="dependentNOMCR" minOccurs="0"/>

<xs:element name="dependentSAMR" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="SAMR">

<xs:complexContent>

<xs:extension base="Project">

<xs:sequence>

<xs:element name="associatedNOMCR" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="PMCR">

<xs:annotation>

<xs:documentation>Planning Model Change Request</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Project"/>

</xs:complexContent>

</xs:complexType>

<!--Outage Scheduling-->

<xs:complexType name="Outage">

<xs:annotation>

<xs:documentation>Scheduled Outage</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="mRID" minOccurs="0"/>

<xs:element name="outageType"/>

<xs:element name="requestId"/>

<xs:element name="version"/>

<xs:element name="participant"/>

<xs:element name="resource"/>

<xs:element name="resourceType"/>

<xs:element name="primaryContact"/>

<xs:element name="secondaryContact"/>

<xs:element name="status" minOccurs="0"/>

<xs:element name="description"/>

<xs:element name="cause"/>

<xs:element name="workDescription"/>

<xs:element name="stationVoltage" minOccurs="0"/>

<xs:element name="plannedStart"/>

<xs:element name="plannedEnd"/>

<xs:element name="earliestStart" minOccurs="0"/>

<xs:element name="actualStart" type="xs:dateTime" minOccurs="0"/>

<xs:element name="actualEnd" type="xs:dateTime" minOccurs="0"/>

<xs:element name="latestEnd" minOccurs="0"/>

<xs:element name="mwCurtailed" minOccurs="0"/>

<xs:element name="remedialAction" minOccurs="0"/>

<xs:element name="NOMCR" minOccurs="0"/>

<xs:element name="dependentTransmissionId" minOccurs="0"/>

<xs:element name="terminals" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<!--Report Access-->

<xs:complexType name="Report">

<xs:sequence>

<xs:element name="operatingDate" type="xs:date">

<xs:annotation>

<xs:documentation>Operating date for extracts</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="accessGroup">

<xs:annotation>

<xs:documentation>Access group name, e.g. 'Public'</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="reportGroup">

<xs:annotation>

<xs:documentation>Group name for reports, e.g.  'Market Information Extract'</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="fileName">

<xs:annotation>

<xs:documentation>File name of report</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="created" type="xs:dateTime">

<xs:annotation>

<xs:documentation>When report file was created</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="size">

<xs:annotation>

<xs:documentation>Size of file n KB</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="format">

<xs:annotation>

<xs:documentation>File format, e.g. PDF, zip, txt</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="URL" type="URL">

<xs:annotation>

<xs:documentation>URL for file download</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<!--Elements used for payload definitions using ComplexTypes-->

<xs:element name="ASOffer" type="ASOffer"/>

<xs:element name="ThreePartOffer" type="ThreePartOffer"/>

<xs:element name="SelfSchedule" type="SelfSchedule"/>

<xs:element name="BidSet" type="BidSet"/>

<xs:element name="Notification" type="Notification"/>

<xs:element name="SelfArrangedAS" type="SelfArrangedAS"/>

<xs:element name="ASCapacity" type="ASCapacity"/>

<xs:element name="CRR" type="CRR"/>

<xs:element name="CapacityTrade" type="CapacityTrade"/>

<xs:element name="ASTrade" type="ASTrade"/>

<xs:element name="PTPObligation" type="PTPObligation"/>

<xs:element name="DCTieSchedule" type="DCTieSchedule"/>

<xs:element name="COP" type="COP"/>

<xs:element name="IncDecOffer" type="IncDecOffer"/>

<xs:element name="EnergyBid" type="EnergyBid"/>

<xs:element name="EnergyOnlyOffer" type="EnergyOnlyOffer"/>

<xs:element name="EnergyTrade" type="EnergyTrade"/>

<xs:element name="AwardedAS" type="AwardedAS"/>

<xs:element name="AwardedEnergyOffer" type="AwardedEnergyOffer"/>

<xs:element name="AwardedCRR" type="AwardedCRR"/>

<xs:element name="AwardedPTPObligation" type="AwardedPTPObligation"/>

<xs:element name="AwardedEnergyBid" type="AwardedEnergyBid"/>

<xs:element name="AwardedEnergyOnlyOffer" type="AwardedEnergyOnlyOffer"/>

<xs:element name="LoadForecast" type="LoadForecast"/>

<xs:element name="SystemLoad" type="TmSchedule"/>

<xs:element name="OutputSchedule" type="OutputSchedule"/>

<xs:element name="MarketTotals" type="TmSchedule"/>

<xs:element name="SPPs">

<xs:annotation>

<xs:documentation>Settlement Point Prices</xs:documentation>

</xs:annotation>

<xs:complexType>

<xs:sequence>

<xs:element name="SPP" type="SPP" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="LMPs">

<xs:annotation>

<xs:documentation>Locational Marginal Prices</xs:documentation>

</xs:annotation>

<xs:complexType>

<xs:sequence>

<xs:element name="LMP" type="LMP" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="MCPCs">

<xs:complexType>

<xs:sequence>

<xs:element name="MCPCSchedule" type="MCPCSchedule" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="BindingConstraints">

<xs:complexType>

<xs:sequence>

<xs:element name="BindingConstraint" type="BindingConstraint" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="SCEDViolatedConstraints">

<xs:complexType>

<xs:sequence>

<xs:element name="SCEDViolatedConstraint" type="SCEDViolatedConstraint" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="ASObligations">

<xs:complexType>

<xs:sequence>

<xs:element name="ASObligation" type="ASObligation" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="UnconfirmedTrades" type="UnconfirmedTrades"/>

<xs:element name="NOMCR" type="NOMCR"/>

<xs:element name="SAMR" type="SAMR"/>

<xs:element name="PMCR" type="PMCR"/>

<xs:element name="ProjectStatus">

<xs:complexType>

<xs:sequence>

<xs:element name="Project" type="Document" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="OutageSchedule">

<xs:complexType>

<xs:sequence>

<xs:element name="Outage" type="Outage" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="TotalRegulation" type="TmSchedule"/>

<xs:element name="LoadRatioShares">

<xs:complexType>

<xs:sequence>

<xs:element name="LoadRatioShare" type="LoadRatioShare" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="TotalEnergy" type="TmSchedule"/>

<xs:element name="Outage" type="Outage"/>

<xs:element name="CompetitiveConstraints">

<xs:complexType>

<xs:sequence>

<xs:element name="CompetitiveConstraint" type="CompetitiveConstraint" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="ShiftFactors">

<xs:complexType>

<xs:sequence>

<xs:element name="ShiftFactor" type="ShiftFactor" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="AggregatedASOfferCurves">

<xs:complexType>

<xs:sequence>

<xs:element name="AggregatedASOfferCurve" type="AggregatedASOfferCurve" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="TotalASOffers">

<xs:complexType>

<xs:sequence>

<xs:element name="ASOffers" type="ASSchedule" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="ASSystemPlan">

<xs:complexType>

<xs:sequence>

<xs:element name="ASSchedule" type="ASSchedule" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="StartupShutdownInstructions">

<xs:complexType>

<xs:sequence>

<xs:element name="StartupShutdown" type="StartupShutdown" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="UnitAvailabilities">

<xs:complexType>

<xs:sequence>

<xs:element name="UnitAvailability" type="UnitAvailability" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="DynamicRatings">

<xs:complexType>

<xs:sequence>

<xs:element name="DynamicRating" type="DynamicRating" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="LoadProfiles">

<xs:complexType>

<xs:sequence>

<xs:element name="LoadProfile" type="NonConformLoadSchedule" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="LoadDistributionFactors">

<xs:complexType>

<xs:sequence>

<xs:element name="LoadDistributionFactor" type="LoadDistributionFactor" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="VoltageProfiles">

<xs:complexType>

<xs:sequence>

<xs:element name="VoltageLimits" type="VoltageLimits" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="TapLimits" type="TapLimits" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="DeratedCRRs">

<xs:complexType>

<xs:sequence>

<xs:element name="DeratedCRR" type="DeratedCRR" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="ProxyCurves">

<xs:complexType>

<xs:sequence>

<xs:element name="ProxyCurve" type="ProxyCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="MitigatedCurves">

<xs:complexType>

<xs:sequence>

<xs:element name="MitigatedCurve" type="MitigatedCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="Reports">

<xs:complexType>

<xs:sequence>

<xs:element name="Report" type="Report" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:schema>

Appendix D: Sample Code for Signing a SOAP Message

The purpose of this appendix is to provide examples for signing SOAP messages.

J2EE

Java Cryptography Architecture Example

import java.security.*;

...

// generate new RSA keypair

KeyPairGenerator k =

KeyPairGenerator.getInstance(“RSA”);

KeyPair kp = k.generateKeyPair();

// sign data

Signature s =

Signature.getInstance("SHA1withRSA");

s.initSign(kp.getPrivate());

byte[] signature = s.sign(data);

.NET

public static string ClientBase64KeyID = "ODytWwSUPj9/uGbXZTAdEhhzxLE=";

public void CallWebService()

{

PDCRegistrationProxy proxy = new PDCRegistrationProxy();

SoapContext requestContext = proxy.RequestSoapContext;

X509SecurityToken token = GetSigningToken();        //private–keystore

SignupRequest request = new SignupRequest();

request.Name = "Utility Integration Solutions";

request.Address = ERCOT, Taylor, TX";

requestContext.Security.Tokens.Add(token);

requestContext.Security.Elements.Add(new Signature(token));

SignupResponse response = proxy.SignupForPDC(request);

….

# Appendix E: Annotated SOAP Message

An annotated example of a SOAP message follows, which includes the required security elements. The example is divided into sections and marked by a number, e.g. 1.  Following the SOAP message are explanatory notes describing each numbered section. In order to improve readability, some attributes that have URI values have been shortened.

The example does not include any namespace declarations.  The table below lists the namespaces used in this example.

| Prefix | Description | Namespace |
| --- | --- | --- |
| SOAP-ENV | SOAP Envelop | http://schemas.xmlsoap.org/soap/envelope/ |
| ds | XML Digital Signature | http://www.w3.org/2000/09/xmldsig# |
| wsse | Web Service Security Extensions | http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd |
| wsu | Web Services Security Utility | http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd |
| xsd | XML Schema | http://www.w3.org/2001/XMLSchema |
| msg | An private namespace for specifying elements and attributes that are specific to ERCOT Web Services | TBD |

1

<SOAP-ENV:Envelope ">

<SOAP-ENV:Header ">

2

<wsse:Security SOAP-ENV:mustUnderstand="1">

<wsse:BinarySecurityToken

EncodingType="…#Base64Binary"

ValueType="…#X509v3"

wsu:Id="CertId-1776694">

MIIDDDCCAfSgAwIBAgIQM6YEf7FVYx/tZyEXgVComTANBgkqhkiG9w0 DAVPQVNJUzEeMBwGA1UEAwwVT0FTSVMgSW50ZXJvcCBUZXN0IENBMB4DTE4MDMxOTIzNTk1OVowQjEOMAwGA1UECgwFT0FTSVMxIDAeBgNVBAsVGVzdCBDZXJ0MQ4wDAYDVQQDDAVBbGljZTCBnzANBgkqhkiG9w0BAQE9By1VYo0aHrkKCNT4DkIgPL/SgahbeKdGhrbu3K2XG7arfD9tqIBIKMyvq+mUnMpNcKnLXLOjkTmMCqDYbbkehJlXPnaWLzve+mW0pJdPxtf3r sZKT8DN5Kyz+EZsCAwEAAaOBkzCBkDAJBgNVHRMEAjAAMDMGA1UdHwQaW50ZXJvcC5iYnRlc3QubmV0L2NybC9jYS5jcmwwDgYDVR0PAQH/BAQ4l0TUHZ1QV3V2QtlLNDm+PoxiDAfBgNVHSMEGDAWgBTAnSj8wes1oR3 JTw==

</wsse:BinarySecurityToken>

3

<ds:Signature>

<ds:SignedInfo>

<ds:CanonicalizationMethod

Algorithm="http://www.w3.org/2001/10/xml-exc-c14n#" />

<ds:SignatureMethod

Algorithm="http://www.w3.org/2000/09/xmldsig#rsa-sha1" />

4

<ds:Reference URI="#id-1464350">

<ds:Transforms>

<ds:Transform

Algorithm="http://www.w3.org/2001/10/xml-exc-c14n#" />

</ds:Transforms>

<ds:DigestMethod

Algorithm="http://www.w3.org/2000/09/xmldsig#sha1" />

<ds:DigestValue">

1JmC1C0FrlPB42xfFKolgaCew5k=

</ds:DigestValue>

</ds:Reference>

5

<ds:Reference URI="#STRId-13498124">

<ds:Transforms>

<ds:Transform

Algorithm="…#STR-Transform" />

<wsse:TransformationParameters">

<ds:CanonicalizationMethod

Algorithm="...xml-exc-c14n#"/>

</wsse:TransformationParameters>

</ds:Transform>

</ds:Transforms>

<ds:DigestMethod

Algorithm="http://www.w3.org/2000/09/xmldsig#sha1" />

<ds:DigestValue>

sEaCJjrObpDVfM8zuabwQdBGKbY=

</ds:DigestValue>

</ds:Reference>

6

</ds:SignedInfo>

<ds:SignatureValue">

H1b7jH2bHpbrzJXkFS0msdUYycDMH4n6m4oTRtbo4Yk35/JzGcuwUYZ3

AwPcnqmcP5ROshjJparaPGuvQhbFR7zCxet2aoawJFWgG8jIeuDZDE8y6n+kbBzxadF2tGN8/nH6IlKg0+onD09i81rPHDAa 2kstCclX2NDet1Rnmfs=

</ds:SignatureValue>

7

<ds:KeyInfo>

<wsse:SecurityTokenReference wsu:Id="STRId-13498124">

<wsse:Reference URI="#CertId-1776694" ValueType="…#X509v3" />

</wsse:SecurityTokenReference>

</ds:KeyInfo>

</ds:Signature>

</wsse:Security>

</SOAP-ENV:Header>

8

<SOAP-ENV:Body wsu:Id="id-1464350" >

<msg:Message>

<msg:Header>

<msg:Verb>verb</msg:Verb>

<msg:Noun>noun</msg:Noun>

<msg:ReplayDetection>

<wsu:Created>

2006-11-29T20:05:55.022Z

</wsu:Created>

<wsse:Nonce EncodingType="…#Base64Binary">

75753793-50c2-455b-a9b3-123cb26474e7

</wsse:Nonce>

</msg:ReplayDetection>

<msg:Revision>1</msg:Revision>

<msg:Source>market participant ID</msg:Source>

</msg:Header>

.

.

.

</msg:Message>

</SOAP-ENV:Body>

</SOAP-ENV:Envelope>

1

This section starts the SOAP envelope and the SOAP header

2

This section starts the Web Services Security Extensions, which includes security tokens and the digital signature. The first element in this section is the signer’s X.509 certificate, which is encoded in Base 64 binary. Note the identification of this element (CertId-1776694). The ID is later used in section 7 to reference this certificate. Note that there is only one certificate in this message. The message verifier must ensure that the certificate chains to a trusted root.

3

This section starts the digital signature block of the SOAP message. The signature is computed using the SHA-1 hash algorithm with RSA encryption.

4

This section designates the first of two objects that are signed. This one points to the entire message body (#id-1464350), which is specified in section 8 . The hashing algorithm is SHA-1.

5

This section designates the second of two objects that are signed. This one points to the reference to the certificate (#STRId-13498124), which is specified in section 7 .

6

This section specifies the value of the signature. That is, the SHA-1 hash of references to sections 7 and 8  and the encryption of this hash using the signer’s private key.

7

This section designates a reference to the signer’s certificate. In this case, the certificate is embedded in this SOAP message, and is referenced via the ID #CertId-1776694. This ID instructs the message verifier to get the certificate from section 2of this SOAP message.

8

This section starts the SOAP message body. It is designated using ID id-1464350, which is referenced as a signed element in section 2. Note that the message body includes an element called ReplayDetection, which consists of a timestamp indicating when the message was signed and a unique number (the nonce). These two elements help detect and prevent replay attacks. The rest of the message body (i.e., the business transaction) is not shown.

# Appendix F: XSL Examples

The purpose of this appendix is to provide XSL examples related to the processing and transformation of thre XML described in this specification. One example of XSL usage would be to convert UTC time into separate date and hour values based on the local time zone. For example, given the following XML:

<startTime>2007-12-17T06:00:00Z</startTime>

An XSL template can be used to generate tags that are desired by a consumer of the message such as the following where the hour is an hour ending in local time:

<startTime>2007-12-17T06:00:00Z</startTime>

<startDate>2007-12-17</startDate>

<startHour>1</startHour>

The following XSL example is implemented using custom functions within TIBCO BusinessWorks.

<?xml version='1.0'?>

<xsl:stylesheet version="1.0"

xmlns:xsl="http://www.w3.org/1999/XSL/Transform"

xmlns:msg="http://www.uisol.com"

xmlns:tib="http://www.tibco.com/bw/xslt/custom-functions"

exclude-result-prefixes="msg tib">

<xsl:template match="/">

<xsl:apply-templates/>

</xsl:template>

<xsl:template match="*">

<xsl:choose>

<xsl:when test="name() = 'msg:startTime'">

<xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

<xsl:variable name="vStartTime"><xsl:value-of select="tib:translate-timezone(text(), tib:get-timezone-from-dateTime(current-dateTime()))"/></xsl:variable>

<msg:startDate>

<xsl:value-of select="substring($vStartTime,1,10)"/>

</msg:startDate>

<msg:startTime>

<xsl:choose>

<xsl:when test="substring($vStartTime,12,2) = '00'">24</xsl:when>

<xsl:otherwise><xsl:value-of select="format-number(substring($vStartTime,12,2),'#0')"/></xsl:otherwise>

</xsl:choose>

</msg:startTime>

</xsl:when>

<xsl:when test="name() = 'msg:endTime'">

<xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

<xsl:variable name="vEndTime"><xsl:value-of select="tib:translate-timezone(text(), tib:get-timezone-from-dateTime(current-dateTime()))"/></xsl:variable>

<msg:endDate><xsl:value-of select="substring($vEndTime,1,10)"/></msg:endDate>

<msg:endTime>

<xsl:choose>

<xsl:when test="substring($vEndTime,12,2) = '00'">24</xsl:when>

<xsl:otherwise><xsl:value-of select="format-number(substring($vEndTime,12,2),'#0')"/></xsl:otherwise>

</xsl:choose>

</msg:endTime>

</xsl:when>

<xsl:otherwise>

<xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

</xsl:otherwise>

</xsl:choose>

</xsl:template>

</xsl:stylesheet>

The following XSL example is implemented using Xalan:

<?xml version='1.0'?>

<xsl:stylesheet version="1.0"

xmlns:xsl="http://www.w3.org/1999/XSL/Transform"

xmlns:msg="http://www.uisol.com"

xmlns:lxslt="http://xml.apache.org/xslt"

xmlns:mk="ext1"

extension-element-prefixes="mk"

exclude-result-prefixes="msg lxslt">

<lxslt:component prefix="mk" functions="ConvertDate">

<lxslt:script lang="javascript">

function ConvertDate(inYear,inMonth,inDay,inTime,inType){

var strDate = inMonth + "/" + inDay + "/" + inYear + " " + inTime + " UTC";

var GMTDate = new Date(strDate);

strDate = inType == "Date" ? GMTDate.getFullYear() + "-" + GMTDate.getMonth() + "-" + GMTDate.getDate() : GMTDate.getHours();

return strDate.toString();

}

</lxslt:script>

</lxslt:component>

<xsl:template match="/">

<xsl:apply-templates/>

</xsl:template>

<xsl:template match="*">

<xsl:choose>

<xsl:when test="(name() = 'msg:startTime' or name() = 'msg:endTime')”>

<xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

<xsl:variable name="vYear"><xsl:value-of select="substring(text(),1,4)"/></xsl:variable>

<xsl:variable name="vMonth"><xsl:value-of select="substring(text(),6,2)"/></xsl:variable>

<xsl:variable name="vDay"><xsl:value-of select="substring(text(),9,2)"/></xsl:variable>

<xsl:variable name="vTime"><xsl:value-of select="substring(text(),12,8)"/></xsl:variable>

<xsl:choose>

<xsl:when test="name() = 'msg:startTime'">

<msg:startDate>

<xsl:value-of select="mk:ConvertDate(string($vYear),string($vMonth),string($vDay),string($vTime),'Date')"/>

</msg:startDate>

<msg:startTime>

<xsl:value-of select="mk:ConvertDate(string($vYear),string($vMonth),string($vDay),string($vTime),'Time')"/>

</msg:startTime>

</xsl:when>

<xsl:when test="name() = 'msg:endTime'">

<msg:endDate>

<xsl:value-of select="mk:ConvertDate(string($vYear),string($vMonth),string($vDay),string($vTime),'Date')"/>

</msg:endDate>

<msg:endTime>

<xsl:value-of select="mk:ConvertDate(string($vYear),string($vMonth),string($vDay),string($vTime),'Time')"/>

</msg:endTime>

</xsl:when>

</xsl:choose>

</xsl:when>

<xsl:otherwise>

<xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

</xsl:otherwise>

</xsl:choose>

</xsl:template>

</xsl:stylesheet>

# Appendix G: Payload Compression Example

The purpose of this section is to provide an example of the code required to compress and encode a payload, where the payload is then passed as the contents of the Payload/Compressed element. The following is a Java example that leverages commonly used classes for compression and base64 encoding.

package BusinessService.TS.WebServiceClient.CompressionClient;

import java.util.*;

import java.io.*;

import java.io.File;

import java.io.FileInputStream;

import java.io.FileOutputStream;

import java.io.InputStream;

import java.io.IOException;

import java.util.zip.*;

import org.apache.soap.encoding.soapenc.Base64;

public class CompressionClientCompressandEncode{

protected byte[] input = null;

protected String base64GzipInput = "";

public byte[] getinput() {

return input;

}

public void setinput(byte[] val) {

input = val;

}

public String getbase64GzipInput() {

return base64GzipInput;

}

public void setbase64GzipInput(String val) {

base64GzipInput = val;

}

public CompressionClientCompressandEncode() {

}

public void invoke() throws Exception {

In  : byte[] input

Out : String base64GzipInput

ByteArrayOutputStream bas = new ByteArrayOutputStream();

GZIPOutputStream bis = new GZIPOutputStream(bas);

bis.write(input);

bis.close();

base64GzipInput = Base64.encode(bas.toByteArray());

}

}

# Appendix H: Issues

The purpose of this section is to capture issues and resolutions.

| Issue ID | Date | Issues | Resolution |
| --- | --- | --- | --- |
| 1 | 1/15/2007 | Bid IDs. Need to augment interfaces to provide a unique bid ID that can be used for the update and cancellation of bids as opposed to specifying a set of primary key values that differ for each bid type | Added mRID definition. |
| 2 | 1/15/2007 | Need to finalize scheme for acceptance and rejection of trades | Duplicate trades must be entered. Periodic notification for unmatched trades. |
| 3 | 1/17/2007 | Currently EnergySchedules and CapacitySchedules use CIM IrregularTimeSchedules. Could decide to use a more compact/custom structure | Decided to stay consistent with CIM. |
| 4 | 1/17/2007 | References to resources (i.e. resource names) could be more CIM-compliant using RegisteredResource.name | Reference is by name only. Use simplest XML. |
| 5 | 1/17/2007 | References to sources, sinks and sp could be more CIM-compliant by using Pnode.name. Related to issue 4. | Reference is by name only. Use simplest XML. |
| 6 | 1/18/2007 | May want to add QSE ID to each award message | Done. |
| 7 | 1/23/2007 | Need to clarify the use of market types for bidding interfaces for consistency with Nodal protocols. | Market types are not used. Retain as an optional field for potential future use. |
| 8 | 1/23/2007 | SOAP faults will be added to the WSDL in the future. | Added to SandBox. |
| 9 | 2/11/2007 | Need to define indicator for ASOffer and mcpc for AwardedAS. Define lmp and spp for AwardedEnergyOffer. | Use curveStyle |
| 10 | 2/11/2007 | Need to define enumerations for AS types and revise section 2.3.5 and add enumerations to XML schema. We would then update examples. | Reg-Up, Reg-Down, Non-Spin, RRS |
| 11 | 2/20/2007 | Need to define resource status enumerations and add to XML Schema | Done. |
| 12 | 3/2/2007 | Need to decide whether or not expiration is used for EnergyOnlyBid or EnergyOnlyOffer | Current view is both. |
| 13 | 3/2/2007 | Need to identify the specific identifier to be used to retrieve load profiles for non-IDR customers. |  |
| 14 | 3/2/2007 | Need to add notifications to MPs for the create, update or cancel of Outages | Section 5.3.11 |
| 15 | 3/14/2007 | Need to define SLAs |  |
| 16 | 3/14/2007 | In section 3.3.4, is indicator needed? | Indicator is now handled by curveStyle |
| 17 | 3/14/2007 | Consider segmentation of Ercot.xsd into more XSDs, e.g. ErcotCommon.xsd, ErcotBid.xsd, ErcotNotification.xsd, ErcotInfo.xsd |  |
| 18 | 3/21/2007 | Need to define convention for resource names, as unit names are not unique unless concatenated with the station name |  |
| 19 | 3/23/2007 | Depending upon usage (i.e. more than one record per bid), it may be more efficient to redefine complex types for StartupCost, MinimumEnergy, MaximumPrice, MinimumReservationPrice, Limits, ResourceStatus or ASCapacity as an IrregularIntervalSchedule. |  |
| 20 | 3/23/2007 | XML examples should be use correct namespaces and qualifications. |  |
| 21 | 3/27/2007 | After revision 1.0 and finalization of the MMS representation of market intervals, the bidding interfaces will be revised as appropriate to use ‘market time’, where market hours and intervals will be identified. TPTF will be consulted on the final design. | Decided to use TmSchedule structure |
| 22 | 3/27/2007 | Need to identify the means to notify market participants when documents in the repository have been updated or created. |  |
| 23 | 3/27/2007 | MPs need to provide input with respect to additional filtering capabilities needed for interfaces. This would apply primarily to those interfaces in section 4. This input would be used to extend the interface specifications. |  |
| 24 | 3/27/2007 | Note that ASOffer may change pending MMS design |  |
| 25 | 3/27/2007 | IncDecOffer should be modified to allow two curves. | Defined two curves, need final definition to be consistent with MMS design |
| 26 | 3/30/2007 | Need to add non-spin deployment |  |
| 27 | 4/6/2007 | Corrected verb to be ‘change’ instead of ‘update’ to be consistent with enumerations within WSDL | done |
| 28 | 4/19/2007 | Need to add labels for figures and tables throughout document | done |
| 29 | 4/24/2007 | Need to add notification message for startup and shutdowns | done |
| 30 | 5/2/2007 | Need to revise IncDecOffer interface so that it is consistent with final MMS design |  |
| 31 | 5/2/2007 | Would like to have PENDING status prior to credit approval, after approval it would be ACCEPTED. This is useful when transactions are submitted 7 days in advance |  |
| 32 | 5/2/2007 | Need to use DUNS numbers for trades. Need to verify with MMS |  |
| 33 | 5/2/2007 | Need web service for DC tie flows |  |
| 34 | 5/2/2007 | Need to explore use of QSE supplied external IDs that are managed by MMS |  |
| 35 | 5/2/2007 | Need notification message for AS Obligations |  |
| 36 | 5/2/2007 | Need notification for bid rejection |  |
| 37 | 5/2/2007 | Upon request by MPs, added combinedCycle wherever resource was identified (e.g. COP). Need to verify this will be supported within MMS |  |