For partial private use networks; this modeling would be to:

- Include these partial self serve networks in the ERCOT internal area.

- Receive both the gross generation from each unit, and the net generation to ERCOT from the private network units.

- Install a load at each of these private area network busses, and model the self serve load at these busses; but do NOT include this load in ERCOT load accounting, load forecast, or load distribution.

Example:

A location registers as a private use network.

It is required to register ALL generators at the location, and the full capacity of each generator.

During market operations, it is permitted to offer only the portion of the generators which is not serving its self-serve load.

On a particular day, a participant has 700 MW of capacity, and around 500 MW of self-serve load.  It chooses to split this load equally between the two units; This QSE bid this unit into the ancillary service market, and sold 20 MW of regulation  service on unit 1. No other AS obligation exists

Registration

Example registered: Location A

2 generators:

Reasonability limit on load – 800 MW

Current Seasonal Net Dependable Capability Test (NDC):  Unit 1 = 500 MW, Unit 2 = 200 MW

URL data

COP submittal:

| Unit 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hour | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| HSL | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 |
| LSL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| HEL | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 | 143 |
| LEL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Status | onreg | onreg | onreg | onreg | onreg | onreg | onreg | onreg | onreg | onreg | onreg | onreg | onreg |
| Reg-up cap | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Reg-down_cap | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Responsive Reserve | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Nonspin | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Unit 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hour | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| HSL | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 |
| LSL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| HEL | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 | 57 |
| LEL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Status | on | on | on | on | on | on | on | on | on | on | on | on | on |
| Reg-up cap | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Reg-down_cap | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Responsive Reserve | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Nonspin | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Offer Curve |  |  |  |  |
| --- | --- | --- | --- | --- |
| Selling QSE | QSE J |  |  |  |
| Resource | Unit1 |  |  |  |
| Price | 80 | 100 | 140 | 150 |
| Quantity | 0 | 50 | 100 | 143 |
| 1st hour offer | 1 |  |  |  |
| Last hour offer | 24 |  |  |  |
| expatriation time and date of offer | 24:00 01/01/20010 |  |  |  |
| Ancillary service offer | Regulation: 50 |  |  |  |
| Inclusive or exclusive designation | Exclusive |  |  |  |
| % FIP | 100 |  |  |  |
| % FOP | 0 |  |  |  |
| Selling QSE | QSE J |  |  |  |
| Resource | Unit2 |  |  |  |
| Price | 80 | 100 | 140 | 150 |
| Quantity | 0 | 30 | 50 | 57 |
| 1st hour offer | 1 |  |  |  |
| Last hour offer | 24 |  |  |  |
| expiration time and date of offer | 24:00 01/01/20010 |  |  |  |
| Ancillary service offer | Regulation: 50 |  |  |  |
| Inclusive or exclusive designation | Exclusive |  |  |  |
| % FIP | 100 |  |  |  |
| % FOP | 0 |  |  |  |

In real time: ERCOT is receiving telemetry as follows:

Unit 1 net real power = 80 MW

Unit 1 net reactive power = 20 MW

Unit 1 gross power = 458 MW

Unit 1 gross reactive power = 25

Power to standby xfr serving plant aux load = 20

HSL=143

LSL=0

HEL=143

LEL=0

Reg-Up responsibility =0

Reg-down responsibility=0

Responsive Reserve responsibility=20

Non-spin responsibility =0

Reg up Participation factor=100

Reg down participation factor=100

Responsive Reserve Schedule = 20

Non-spin schedule = 0

Unit 2 net real power = 20 MW

Unit 2 net reactive power = 5 MW

Unit 2 gross power = 163 MW

Unit 2 gross reactive power = 25

Power to standby xfr serving plant aux load = 20

HSL=57

LSL=0

HEL=57

LEL=0

Reg-Up responsibility =0

Reg-down responsibility =0

Responsive Reserve responsibility =0

Non-spin responsibility =0

Reg up Participation factor=100

Reg down participation factor=100

Responsive Reserve Schedule = 0

Non-spin schedule = 0

State Estimator interaction and Network Security Analysis interaction

ERCOT’s real-time network (state estimator) is modeling the following

Line α                           Line β

Only the net generation telemetries will be included in load forecast or ERCOT load calculations. (i.e. the load is modeled as EXTERNAL to ERCOT)

The units available reactive is calculated as follows:

Unit 1: At generation gross=458; calculated reactive CURL reactive capability= +324,-285

Unit 2: At generation gross = 163, calculated CURL reactive capability =108.5, -88.5

For reserve calculations the reserve on unit 1 = 143 net HSL – 80 net gen = 63 MW

Reserve on unit 2 = 57 net HSL- 20 net gen = 37 MW

For real-time Responsive Reserve service calculations; the reserve on either unit will be limited to 20% of the Net Dependable Capability or:

Responsive Reserve Unit 1 = Minimum(0.2 * Seasonal Net Dependable Capability, reserve)

= Minimum (0.2 * 500, 63)

= Minimum (100,63)

= 63 MW

Responsive Reserve Unit 2 = Minimum (0.2 * Seasonal Net Dependable Capability, reserve)

= Minimum (0.2 * 200, 37)

= Minimum (40,37)

= 37 MW

For real-time TCM (contingency analysis); loss of the unit 1 results in a station net injection (load) of:

Net = ∑Gross Load - ∑generation remaining

= Calculated Load (from state estimator above) – Gross MW Unit 2

= 521 – 163 = 358 MW (net load or post-contingency station injection)

Loss of unit 2 results in a net station injection (load) of :

Net = ∑Gross Load - ∑generation remaining

= Calculated Load (from state estimator) – Gross MW Unit 1

= 641 – 458 = 63 MW

Reliability Unit Commitment Interaction

For RUC it will be necessary to create non-ERCOT adapted load models (i.e. do not go into ERCOT total load forecast, but are used in RUC models. RUC models the following for hours 1-14 assuming that it found the units economical to dispatch for 80 net and 20 net for each hour.

Line α			Line β

The question is asked, if these units are scheduled in an offline status, can they be committed by RUC?  Yes, if the online units have a sum of HSL greater then zero, then the self-committed units are required to provide a net injection to ERCOT; and additional unit commitment behind the fence will result in additional energy available to ERCOT.

For example, If this location had 400 MW of peak load for a given day, and self commits unit 1 to serve that load, it should submit a 3 part supply offer to ERCOT for Unit 2 with a Startup Offer, Minimum-Energy Offer, and Energy Offer curve.  In this scenario Unit 1 would be offered as On with an HSL of 100 MW; and Unit 2 would be offered to RUC with an HSL of 200 MW, and a status of OFF, or EMR; allowing ERCOT to commit this unit in RUC.    If no three part offer were received at ERCOT, ERCOT would generate

For DAM market interaction:

When calculating load distribution for DAM, the load distribution would typically see no load distribution factors for this bus, because there is normally no net load.  Thus, a load bid into the zone containing this bus would get no contribution assigned to this bus.

Generation Subsystem (GSS-LFC) interaction

GSS sees only the telemetered Net generation from unit 1 and unit 2.  When frequency is low, it will see regulation room available on unit 1, and will deploy regulation to that unit (via a portfolio deployment to the host QSE).  GSS will also calculate the HDL and HASL as shown below.

Unit 1 SURAMP = Min(NORMRAMP-(RUSTELEM/5), [EMRRAMP-(rrstelem/10)])

= Min(10-20/5,10-(0/10)) = 6

Unit 1 LASL = LSLTELEM+ RDSTELEM = 0 + 0 = 0

Unit 1 HASL = Max(LASL, (HSLTELEM-(RRSTELEM+RUSTELEM+NSRSTELEM)))

= Max(0, 143-(0+20+0)) = 123

Unit 1 HDL = Minimum(Powertelem+(suramp*5),HASL) = min(80+30,123) = 110 MW

Unit 1 LDL = Max(Powertelem – (SDRAMP*5, LASL) = 50 MW

Similarly for Unit 2

Unit 2 SURAMP = 10

Unit 2 LASL = 0

Unit 2 HASL = 57

Unit 2 HDL = 57

Unit 2 LDL = 0

SCED interaction – real time

SCED receives only the current net generation, HASL, NDL, and shift factors for constraints affected by this generator.  In this example, if I assume that there is a transmission limit on line α of 200 MW, we would expect that GSS passes to SCED:

Unit 1 Powertelem = 80

Unit 1 HASL = 123

Unit 1 LASL = 0

Unit 1 HDL = 110

Unit 1 LDL = 50

Unit 2 Powertelem = 20

Unit 2 HASL = 53

Unit 2 LASL = 0

Unit 2 HDL = 53

Unit 2 LDL = 0

If approved by the operator in TCM, a constraint will be passed to SCED as follows:

Constraint:  Unit 1 Basepoint * 1 (sf) + Unit 2 Basepoint * 1 (sf) < 200MW

Outage Scheduling and Outage Evaluation

If an outage is requested within the 1 week timeframe, the modeling in outage evaluation is exactly as described in the RUC modeling above.

If an outage is requested over 1 week ahead, then there is no COP data to calculate the expected load behind the interface.  For outage evaluation and studies a load schedule will be kept and adapted from the results of the real-time analysis for each private use load.  This schedule will populate the private use network loads for studies and outage evaluation over 1 week in the future.  This load will not be subtracted from the load forecast load apportioned to all non-private use buses.