Commercial Systems

Software Requirements SpecificationReliability Unit Commitment Settlements Requirements

v1.1

GP&L Comments on pages 28,35,42

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 08/22/06 | 1.0 | Approved by TPTF | Keely Hilton |
| 04/17/07 | 1.1 | Synchronized to RUC CSD v1.0 and RUC Use Cases v1.0. NPRR revisions incorporated, including: NPRR041, NPRR042 and NPRR050. | Keely Hilton |

Table of Contents

1.	Reliability Unit Commitment Settlement	5

1.1	Proposed System Scope	5

1.2	Document Conventions	6

1.3	Document Assumptions	6

2.	Functional Requirements	7

2.1	Startup Price (SUPR)	7

2.2	Minimum-Energy Price (MEPR)	12

2.3	RUC Guarantee (RUCG)	17

2.4	RUC Minimum-Energy Revenue (RUCMEREV)	22

2.5	Revenue Less Costs Above LSL During RUC-Committed Hours (RUCEXRR)	25

2.6	Revenue Less Costs During QSE Clawback Intervals (RUCEXRQC)	30

2.7	Number of continuous RUC Committed Hours (RUCHR)	35

2.8	Number of Continuous RUC De-committed Hours (NCDCHR)	37

2.9	RUC Make-Whole Payment (RUCMWAMT)	38

2.10	RUC Clawback Charge (RUCCBAMT)	45

2.11	RUC Decommitment Payment (RUCDCAMT)	53

2.12	RUC Capacity-Short Charge (RUCCSAMT)	59

2.13	RUC Make-Whole Uplift Charge (LARUCAMT)	81

2.14	RUC Clawback Payment (LARUCCBAMT)	84

2.15	RUC Decommitment Charge (LARUCDCAMT)	87

3.	Supplementary Requirements	90

3.1	Performance Requirements	90

3.2	Legal and Regulatory requirements	90

3.3	System Security Requirements	90

3.4	Back up and Recovery Requirements	90

3.5	Availability and Redundancy Requirements	90

3.6	Maintainability Requirements	90

3.7	Daylight Savings Time shall be recognized in Nodal Market settlements.	91

4.	Protocol Coverage	91

Software Requirements Specification

# Reliability Unit Commitment Settlement

## Proposed System Scope

ERCOT plans to implement a new Nodal market in 2009.  Sections 4, 5, 6 and 9 of the Nodal Protocols describe the various Charge Type requirements and Settlement Statement requirements for the settlement of Reliability Unit Commitment (RUC).  The Nodal Protocols will guide this project.

This document defines the requirements for Reliability Unit Commitment (RUC) settlements.  The requirements in scope for this document include the calculation of RUC settlement Charge Types and their components which are derived from Section 4, Section 5, Section 6 and Section 9 of the February 2007 Nodal Protocols.

- Also in scope for this document are revisions to the Nodal Protocols, included in:

- NPRR041, Corrections to Section 4, Day-Ahead Operations

- NPRR042, Corrections to Section 5, Transmission Security Analysis and Reliability Unit Commitment

- NPRR050, Clarifications to HSL values for WGRs and WCR values to be used in the RUC Capacity-Short calculation

The following Settlement Charge Types are considered RUC settlement and are included in the scope of this document:

- RUC Make-Whole Payment (PR 5.7.1)

- RUC Clawback Charge (PR 5.7.2)

- RUC Decommitment Payment (PR 5.7.3)

- RUC Capacity-Short Charge (PR 5.7.4.1)

- RUC Make-Whole Uplift Charge (PR 5.7.4.2)

- RUC Clawback Payment (PR 5.7.5)

- RUC Decommitment Charge (PR 5.7.6)

The following global bill determinants for RUC settlement are also included in scope for this document.  Global bill determinants for RUC settlement are bill determinants that are used by multiple RUC Charge Types.  The RUC settlement global bill determinants include:

- Startup Price (SUPR) (PR 5.7.1.1, PR 5.7.3)

- Minimum-Energy Price (MEPR) (PR 5.7.1.1, PR 5.7.1.4, PR 5.7.3)

- RUC Guarantee (RUCG) (PR 5.7.1.1)

- RUC Minimum-Energy Revenue (RUCMEREV) (PR 5.7.1.2)

- Revenue Less Cost Above LSL During RUC-Committed Hours (RUCEXRR) (PR 5.7.1.3)

- Revenue Less Cost During QSE Clawback Intervals (RUCEXRQC) (PR 5.7.1.4)

## Document Conventions

- All text lifted from the Nodal Protocols is in italic font.

- All requirements are identified using FR1, FR2, etc.

- All dependencies are identified using D1, D2, etc.

- All assumptions are identified using A1, A2, etc.

- The “Original Source” column of the Input, Intermediate, and Output Bill Determinates tables (refer to Section 2, Functional Requirements) delineate the initial source of market and/or operational information required to construct each variable.  The data transformation process exists within the interface between the Settlement System and Market / Operational Systems.  A separate and distinct set of Business Requirements will address all data interfacing between the original source system and the Settlement System.

## Document Assumptions

A1)	Startup Eligibility, including determination of QSE Clawback intervals, is documented in a separate set of requirements.  See TN.COMS.63C01.ELIGIBILITY.REQUIREMENTS.A for further information.

A2)	Verifiable Costs, including startup and minimum-energy, is documented in a separate set of requirements.  See TN.COMS.63C01.VERIFIABLECOSTS.REQUIREMENTS.A for further information.

A3)	All Settlement Point Prices will be calculated outside of the Settlement System and therefore the SPP calculations are not detailed within this document.

A4)	The Settlement System shall not store zero value data cuts for a charge type, unless the zero data cut is a valid result of a calculation or a requirement specifies that the cut be saved.

A5)	Any charge type that requires historical data will obtain this information via alternative means during the beginning of market open until the data is available.

A6)	The charge type calculations are documented using the proposed recorder format for the bill determinants.  The following list defines the nomenclature used for the recorders (not all of these bill determinant recorder elements apply to the charge types within this document):

<Q> = QSE

<CO> = CRR Owner

<R> = Resource

<SP> = Settlement Point

< BLTP> = Block Load Transfer Point

<SRSP> = Source Settlement Point

<SKSP> = Sink Settlement Point

<ST> = Start Type

<F> = Flowgate

<E> = Directional Network Element

<C> = Constraint Identifier

<M> = Market

<RUC> = RUC Process

<OPi> = Ordered Pair Number

# Functional Requirements

## Startup Price (SUPR)

The SUPR is the Startup Offer (SUO) component of the validated Three-Part Supply Offer for a Resource, unless a validated Three-Part Supply Offer has not been submitted.

If a validated Three-Part Supply Offer has not been submitted for a Resource, then the SUPR is the ERCOT-approved verifiable startup cost (VERISU) for that Resource.

If ERCOT has not approved verifiable startup costs, then the SUPR is the Resource Category Generic Startup Cap (RCGSC) for the Resource Category assigned to the Resource.

| Requirement ID | FR1 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.1.1 PR 5.7.3 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the SUPR by QSE and Resource, for each Start Type, for each Resource with a RUC-Commitment or RUC-Decommitment, for each hour within the Operating Day. If, SUO for the Resource exists in the settlement system for the Operating Day, SUPR_<Q>_<R>_<SP>_<ST> = SUO_<Q>_<R>_<SP>_<ST> Else, If VERISU for the Resource is available in the settlement system for the Operating Day, SUPR_<Q>_<R>_<SP>_<ST> = VERISU_<Q>_<R>_<SP>_<ST> Else, SUPR_<Q>_<R>_<SP>_<ST> = RCGSC (resource category for the Resource) |  |

| Requirement ID | FR2 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.4.9.2.3(1) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: For each Resource Category the Resource Category Generic Startup Cap (RCGSC) is determined by the following O&M costs: Resource Category; RCGSC Calculation. Nuclear; $7,200. Coal and Lignite; $7,200. Hydro; $7,200. Renewable; $7,200. Combined Cycle > 90 MW with 5+ hours offline; $6,810. Combined Cycle > 90 MW with less than 5 hours offline; $5,310. Combined Cycle <= 90 MW with 5+ hours offline; $6,810. Combined Cycle <= 90 MW with less than 5 hours offline; $5,310. Gas Steam Supercritical Boiler; $4,800. Gas Steam Reheat Boiler; $3,000. Gas Steam Non-Reheat or Boiler without air-preheater; $2,310. Simple Cycle > 90 MW; $5,000. Simple Cycle <= 90 MW; $2,300. Diesel; $1. RMR Resource; (Not Applicable). |

### Input Bill Determinants

| Variable | Unit |  | Definition | Original Source |
| --- | --- | --- | --- | --- |
| RUCHR | Misc |  | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |
| NCDCHR | Misc | Number of Continuous RUC-Decommitted Hours— The number of continuous RUC-Decommitment hours, for the Resource within an Operating Day. |  | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |
| SUO | $/Start |  | Startup Offer per QSE per Resource per Settlement Point—The Startup Offer included in the Three-Part Supply Offer associated with Resource r at Resource Node p represented by QSE q, representing an offer for all costs incurred by the Resource in starting up and reaching breaker close. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The settlement system will receive an offer, from each QSE/Resource combination that submitted a Three-Part Supply Offer, for each type of start: Hot, Intermediate and Cold. The SUO value used in this calculation is the value that is valid at the end of the adjustment period for each hour. |  |  |  |  |
| VERISU | $/Start |  | Verifiable Startup Costs—The verifiable startup costs of a Resource for a start, submitted by the QSE for the Resource, according to Section 5.6.1, Verifiable Costs and Section 5.6.1.1, Verifiable Startup Costs, for the Start Type for the Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: The VERISU bill determinant will be an output from the Verifiable Costs process and will only be created for Resources that have a RUC-Commitment or RUC-Decommitment for the Operating Day and with ERCOT approved Verifiable Costs. There will be a VERISU, for each QSE/Resource combination with a RUC-Commitment or RUC-Decommitment, for each type of start: Hot, Intermediate and Cold. See TN.COMS.63C01.VERIFIABLECOSTS.REQUIREMENTS.A for further information on the VERISU bill determinant. |  |  |  |  |
| RCGSC | $/Start |  | Resource Category Generic Startup Cost—The Resource Category Generic Startup Cost for the category of the Resource, according to Section 4.4.8.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. | Settlement System |
| Interval Frequency: Scalar value, with effective Start and Stop dates Additional Comments: None |  |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| SUPR | $/Start | Startup Price per start—The settlement price for the start. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: There shall be a SUPR, for each QSE/Resource combination with a RUC-Commitment or RUC-Decommitment, for each type of start: Hot, Intermediate and Cold. |  |  |  |

### Data Dependencies

D1)	Calculation of the SUPR is dependent on:

- availability of the input bill determinants

- successful calculation of RUCHR

- successful calculation of NCDCHR

- successful calculation of VERISU

D2)	Calculation of SUPR must be complete prior to the calculation of the RUC Guarantee (RUCG) and the RUC Decommitment Payment (RUCDCAMT).

### Assumptions

A1)	Validation of all components of the Three-Part Supply Offer will occur within the original source system.

A2)	Startup Offer (SUO) data will exist for each Resource with a valid Three-Part Supply Offer, for each type of startup (Hot, Intermediate, and Cold.)  A hot Startup Offer will be indicated by a 1 in the <ST> startup type indicator; an intermediate Startup Offer will be indicated by a 2 in the <ST> startup type indicator, and a cold Startup Offer will be indicated by a 3 in the <ST> startup type indicator.

A3)	Protocol 4.4.9.2.1 (1), Startup and Minimum-Energy Offer Criteria, does not include criteria for specifying the first and last hour of the Offer as does Protocol 4.4.9.3.1 (1), Energy Offer Curve Criteria.  It is assumed that a QSE can submit multiple Startup and Minimum-Energy Offers that are valid for different sets of Operating Hours and that the first and last hour of the Offer specified for the Energy Offer Curve also applies to any associated Startup and Minimum-Energy Offer in a Three-Part Supply Offer.  If this assumption is valid, it may be prudent to change Protocol 4.4.9.2.1 (1) of the Protocols to be more specific and include an additional criterion for the first and last hour of the Offer.NPRR041

A4)	Settlements requires instruction on how to settle Combined Cycle resources based on Combined Cycle configurations.  More information is needed.  Until more information available, when no offer is submitted and no verifiable exists, the generic value for Combined Cycle Resources will be determined using the existing table in Section 4.4.9.2.3 (1) for calculation of Startup Price (SUPR).

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR3 | PR 5.7.1.1 PR 5.7.3 | CSWM.P03SettleBillTransfer | DRIVER: The SUPR calculation shall only be attempted for QSE/Resource combinations with a RUC-Commitment or RUC-Decommitment in the settlement system for the Operating Day. |
| FR4 | PR 5.7.1.1 PR 5.7.3 | CSWM.P03SettleBillTransfer | No input or output bill determinants shall be rounded or truncated. |
| FR5 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCHR – private - NCDCHR - private - SUO – private - VERISU – private - RCGSC - public - SUPR – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR6 | PR 5.7.1.1 PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating SUPR, if no RUC-Commitments or RUC-Decommitments exist in the Settlement System for the Operating Day, the Settlement System should process as if no RUC-Commitments or RUC-Decommitments were issued and exit the process. |
| FR7 | PR 5.7.1.1 PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating SUPR, if SUO does not exist in the Settlement System for the hour, for the Operating Day, and for the QSE/Resource combination, the Settlement System shall default to the QSE/Resource VERISU. (This applies to a missing SUO and also an existing SUO with a null hour.) |
| FR8 | PR 5.7.1.1 PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating SUPR, if VERISU does not exist in the Settlement System for the Operating Day and for the QSE/Resource combination, the Settlement System shall default to the appropriate RCGSC for the Resource. |
| FR9 | PR 5.7.1.1 PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating SUPR, if the factor of RCGSC does not exist in the Settlement System for the Resource Category, the Settlement System shall default to zero and provide a WARN-DEFAULT error message indicating that RCGSC for Resource Category <Resource Category> was not available for calculation. |

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR10 | PR 5.7.1.1 PR 5.7.3 | CSWM.P03SettleBillTransfer | The Settlement System shall map, and store the mapping of, each Resource to a Resource Category (RCGSC) for use in the calculation SUPR. Each value for RCGSC shall be a variable and stored with a start and stop time. |

## Minimum-Energy Price (MEPR)

The MEPR is the Minimum-Energy Offer (MEO) component of the validated Three-Part Supply Offer for a Resource, unless a validated Three-Part Supply Offer has not been submitted.

If a validated Three-Part Supply Offer has not been submitted for a Resource, then the MEPR is the ERCOT-approved verifiable minimum-energy cost (VERIME) for that Resource.

If ERCOT has not approved verifiable minimum-energy costs, then the MEPR is the Resource Category Generic Minimum-Energy Cap (RCGMEC) for the Resource Category assigned to the Resource.

| Requirement ID | FR11 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.1.1 PR 5.7.1.4 PR 5.7.3 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the MEPR by QSE and Resource, for each Resource with a RUC-Commitment or RUC-Decommitment, for each hour within the Operating Day. If, MEO for the Resource exists in the settlement system for the Operating Day, MEPR_<Q>_<R>_<SP> = MEO_<Q>_<R>_<SP> Else, If VERIME for the Resource is available in the settlement system for the Operating Day, MEPR_<Q>_<R>_<SP> = VERIME_<Q>_<R>_<SP> Else, MEPR_<Q>_<R>_<SP> = RCGMEC (resource category for the Resource) |  |

| Requirement ID | FR12 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.4.9.2.3(2)PR 4.4.9.2.3(3) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: The Resource Category Minimum-Energy Generic Cap (RCGMEC) is the cost, per MWh of energy, for a Resource to producing energy up to and including the Resource’s LSL after breaker close according to the following: Resource Category; RCGMEC Calculation. Hydro; $10.00/MWh. Coal and Lignite; $18.00/MWh. Combined Cycle > 90 MW; 10.0 MMBtu/MWh * ((%FOP * FOP) + (%FIP * FIP)), as specified in the Minimum-Energy Offer;. Combined Cycle <= 90 MW; 10.0 MMBtu/MWh * ((%FOP * FOP) + (%FIP * FIP)), as specified in the Minimum-Energy Offer;. Gas Steam Supercritical Boiler; 16.5 MMBtu/MWh * ((%FOP * FOP) + (%FIP * FIP)), as specified in the Minimum-Energy Offer;. Gas Steam Reheat Boiler; 17.0 MMBtu/MWh * ((%FOP * FOP) + (%FIP * FIP)), as specified in the Minimum-Energy Offer;. Gas Steam Non-Reheat or Boiler without air-preheater; 19.0 MMBtu/MWh * ((%FOP * FOP) + (%FIP * FIP)), as specified in the Minimum-Energy Offer;. Simple Cycle > 90 MW; 15.0 MMBtu/MWh * ((%FOP * FOP) + (%FIP * FIP)), as specified in the Minimum-Energy Offer;. Simple Cycle <= 90 MW; 15.0 MMBtu/MWh * ((%FOP * FOP) + (%FIP * FIP)), as specified in the Minimum-Energy Offer;. Diesel; 16.0 MMBtu/MWh * FOP. RMR Resource; RMR contract estimated fuel cost using it’s contract I/O curve at it’s LSL * FIP. Nuclear; $0. Renewable; $0. Settlements will only use the cap if no offer is submitted therefore Settlements shall use 100% of the minimum of FIP or FOP. |

### Input Bill Determinants

| Variable |  | Unit |  |  | Definition | Original Source |
| --- | --- | --- | --- | --- | --- | --- |
| RUCHR | Misc |  |  | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. |  | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |  |  |
| NCDCHR | Misc |  | Number of Continuous RUC-Decommitted Hours— The number of continuous RUC-Decommitment hours, for the Resource within an Operating Day. |  |  | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |  |  |
| MEO |  | $/MWh |  |  | Minimum-Energy Offer per QSE per Resource per Settlement Point—The Minimum-Energy Offer included in the Three-Part Supply Offer associated with Resource r at Resource Node p represented by QSE q. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The MEO value used in this calculation is the value that is valid at the end of the adjustment period for each hour. |  |  |  |  |  |  |
| VERIME |  | $/MWh |  |  | Verifiable Minimum-Energy Costs—The verifiable minimum-energy costs of a Resource for a operating at LSL submitted by the QSE for the Resource, according to Section 5.6.1, Verifiable Costs and Section 5.6.1.2, Verifiable Minimum-energy Costs, for the Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: The VERIME bill determinant will be an output from the Verifiable Costs process and will only be created for Resources that have a RUC-Commitment or RUC-Decommitment for the Operating Day and with ERCOT approved Verifiable Costs. See TN.COMS.63C01.VERIFIABLECOSTS.REQUIREMENTS.A for further information on the VERIME bill determinant. |  |  |  |  |  |  |
| RCGMEC |  | $/MWh |  |  | Resource Category Generic Minimum-Energy Cost—The Resource Category Generic Minimum Energy Cost cap for the category of the Resource, according to Section 4.4.8.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. | Settlement System |
| Interval Frequency: Scalar value, with effective Start and Stop dates Additional Comments: None |  |  |  |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| MEPR | $/MWh | Minimum-Energy Price—The settlement price for minimum energy for the Settlement Interval i. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Data Dependencies

D1)	Calculation of the MEPR is dependent on:

- availability of the input bill determinants

- successful calculation of RUCHR

- successful calculation of NCDCHR

- successful calculation of VERIME

D2)	The MEPR calculation must be complete prior to the calculation of the RUC Guarantee (RUCG), Energy Revenue Less Cost During QSE Clawback Intervals (RUCEXRQC) and the Payment When ERCOT Decommits a QSE-Committed Resource (RUCDCAMT).

### Assumptions

A1)	Validation of all components of the Three-Part Supply Offer will occur within the original source system.

A2)	Protocol 4.4.9.2.1 (1), Startup and Minimum-Energy Offer Criteria, does not include criteria for specifying the first and last hour of the Offer as does Protocol 4.4.9.3.1 (1), Energy Offer Curve Criteria.  It is assumed that a QSE can submit multiple Startup and Minimum-Energy Offers that are valid for different sets of Operating Hours and that the first and last hour of the Offer specified for the Energy Offer Curve also applies to any associated Startup and Minimum-Energy Offer in a Three-Part Supply Offer.  If this assumption is valid, it may be prudent to change Protocol 4.4.9.2.1 (1) of the Protocols to be more specific and include an additional criterion for the first and last hour of the Offer.NPRR041

A3)	Settlements requires instruction on how to settle Combined Cycle resources based on Combined Cycle configurations.  More information is needed.  Until more information available, when no offer is submitted and no verifiable exists, the generic value for Combined Cycle Resources will be determined using the existing table in Section 4.4.9.2.3 (2) for calculation of Minimum-Energy Price (MEPR).

### .Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR13 | PR 5.7.1.1 PR 5.7.1.4 PR 5.7.3 | CSWM.P03SettleBillTransfer | DRIVER: The MEPR calculation shall only be attempted for QSE/Resource combinations with a RUC-Commitment or RUC-Decommitment in the settlement system for the Operating Day. |
| FR14 | PR 5.7.1.1 PR 5.7.1.4 PR 5.7.3 | CSWM.P03SettleBillTransfer | No input or output bill determinants shall be rounded or truncated. |
| FR15 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCHR – private - NCDCHR - private - MEO – private - VERIME – private - RCGMEC - public - MEPR – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR16 | PR 5.7.1.1 PR 5.7.1.4 PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating MEPR, if no RUC-Commitments or RUC-Decommitments exist in the Settlement System for the Operating Day, the Settlement System should process as if no RUC-Commitments or RUC-Decommitments were issued and exit the process. |
| FR17 | PR 5.7.1.1 PR 5.7.1.4 PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating MEPR, if MEO does not exist in the Settlement System for the hour, for the Operating Day, and for the QSE/Resource combination, the Settlement System shall default to the QSE/Resource VERIME. (This applies to a missing MEO and also an existing MEO with a null hour.) |
| FR18 | PR 5.7.1.1 PR 5.7.1.4 PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating MEPR, if VERIME does not exist in the Settlement System for the Operating Day and for the QSE/Resource combination, the Settlement System shall default to the appropriate RCGMEC for the Resource. |
| FR19 | PR 5.7.1.1 PR 5.7.1.4 PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating MEPR, if the factor of RCGMEC does not exist in the Settlement System for the Resource Category, the Settlement System shall default to zero and provide a WARN-DEFAULT error message indicating that RCGMEC for Resource Category <Resource Category> was not available for calculation. |

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR20 | PR 5.7.1.1 PR 5.7.1.4 PR 5.7.3 | CSWM.P03SettleBillTransfer | The Settlement System shall map, and store the mapping of, each Resource to a Resource Category (RCGMEC) for use in the calculation MEPR. Each value for RCGMEC shall be a variable and stored with a start and stop time. |

## RUC Guarantee (RUCG)

The RUC Guarantee insures that a resource will recover its verifiable (or generic) startup and minimum energy costs that result from its operation in a RUC-Committed settlement interval.  Eligibility for startup cost recovery is contingent on the resource’s sequenced output breaker closure to open such that each pair of these events constitutes a startup.  To the extent that the resources is characterized in its resource parameters to have cold, intermediate and hot start verifiable costs, each start-up can have one of these unique verifiable costs.  Minimum energy is guaranteed in RUC-Committed settlement intervals during which the RUC-Committed resource operates for any portion of the interval.  The RUCG is a daily value that is the result of the entire Operating Day’s RUC-Commitments for the Resource.

| Requirement ID | FR21 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.1.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCG by QSE and Resource, for each Resource with a RUC-Commitment, for all eligible starts and settlement intervals within all RUC-Commitments for the Operating Day. RUCG_<Q>_<R>_<SP> = Σs(SUPR_<Q>_<R>_<SP>_<ST> * RUCSUFLAG_<Q>_<R>_<SP>) + Σi(MEPR_<Q>_<R>_<SP> * Min ((LSL_<Q>_<R>_<SP> * (¼)), RTMG_<Q>_<R>_<SP>) Note: Startup portion of the RUCG calculation For each contiguous RUC-Commitment; If startup is eligible (RUCSUFLAG = 1 in the first hour of the contiguous RUC-Commitment), determine the type of start (hot, intermediate, cold) based on the STARTTYPE bill determinant. STARTTYPES: Not Eligible = 0 Hot = 1 Intermediate = 2 Cold = 3 If STARTTYPE = 0 for a contiguous RUC-Commitment, the Settlement System shall use a default value of zero (0) in place of SUPR when calculating RUCG. Multiply the SUPR for the type of start indicated on the STARTTYPE bill determinant and the RUCSUFLAG (which will be one (1)). Then sum all eligible starts for the Operating Day. There can be more than one (1) contiguous RUC-Commitment within an Operating Day however, when the commitments are contiguous, “only one Startup Cost is eligible per block of contiguous RUC-Committed hours.” PR 5.6.2(1). Therefore, it is possible to have more than one (1) eligible startup for an Operating Day. |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| RUCHR | Misc | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| SUPR | $/Start | Startup Price per start—The settlement price for the start s. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: There shall be a SUPR, for each QSE/Resource combination with a RUC-Commitment or RUC-Decommitment, for each type of start: Hot, Intermediate and Cold. |  |  |  |
| RUCSUFLAG | Misc | RUC Startup Flag—The flag that indicates whether or not the start s is eligible for RUC Make-Whole Payment. Its value is one if eligible; otherwise, zero. See Section 5.6.2, RUC Startup Cost Eligibility and Section 5.6.3, Forced Outage of a RUC-Committed Resource, for more information on startup eligibility. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: The RUCSUFLAG bill determinant will be an output from the eligibility process and will only be created for Resources with RUC-Commitments. See TN.COMS.63C01.ELIGIBILITY.REQUIREMENTS.A for further information on the RUCSUFLAG bill determinant. |  |  |  |
| STARTTYPE | Misc | Start Type Flag – The flag that indicates the type of start (hot intermediate or cold) that a Resource incurred for Resource r, represented by QSE q for each hour for the Operating Day. The value is one (1) for hot, two (2) for intermediate, three (3) for cold and zero (0) for not eligible. | MMS |
| Interval Frequency: 1 / hour Additional Comments: This determinant is not documented in the Nodal Protocols but is required to calculate an accurate startup payment for a Resource. |  |  |  |
| MEPR | $/MWh | Minimum-Energy Price—The settlement price for minimum energy for the Settlement Interval i. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RTMG | MWh | Real-Time Metered Generation—The Resource’s metered generation for the Settlement Interval i. | Settlement System through the Data Aggregation Process |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| LSL | MW | Low Sustained Limit—The limit established by the QSE, continuously updated in Real-Time, that describes the minimum sustained energy production capability of the Resource for the hour that includes the Settlement Interval i. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCG | $ | RUC Guarantee—The sum of the Resource’s Startup Costs and Minimum-Energy Costs during all RUC-Committed Hours, for the Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: None |  |  |  |

### Data Dependencies

D1)	Calculation of the RUC Guarantee (RUCG) is dependent on:

- availability of the input bill determinants

- successful calculation of RUCHR

- successful calculation of RTMG

- successful calculation of SUPR

- successful calculation of MEPR

- successful calculation of RUCSUFLAG

- successful calculation of STARTTYPE

D2)	The RUCG calculation must be complete prior to the calculation of the RUC Make-Whole Payment (RUCMWAMT) and the RUC Clawback Charge (RUCCBAMT).

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR22 | PR 5.7.1.1 | CSWM.P03SettleBillTransfer | DRIVER: The RUCG calculation shall only be attempted for QSE/Resource combinations with a RUC-Commitment in the settlement system for the Operating Day. |
| FR23 | PR 5.7.1.1 | CSWM.P03SettleBillTransfer | No input or output bill determinants shall be rounded or truncated. |
| FR24 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCHR – private - SUPR – private - MEPR – private - RUCSUFLAG – private - STARTTYPE – private - RTMG – private - LSL – private - RUCG – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR25 | PR 5.7.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCG, if no RUC-Commitments exist in the Settlement System for the Operating Day, the Settlement System should process as if no RUC-Commitments were issued and exit the process. |
| FR26 | PR 5.7.1.1 | Will be completed August 31, 2006 | While calculating RUCG, if SUPR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination and Start Type, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that SUPR for QSE <Q>, Resource <R> and Start Type <ST> was not available for calculation. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR27 | PR 5.7.1.1 | Will be completed August 31, 2006 | While calculating RUCG, if MEPR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that MEPR for QSE <Q> and Resource <R> was not available for calculation. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR28 | PR 5.7.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCG, if RUCSUFLAG does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all hours of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCSUFLAG for QSE <Q> and Resource <R> was not available for calculation. |
| FR29 | PR 5.7.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCG, if STARTTYPE does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all hours of the Operating Day and provide a WARN-DEFAULT error message indicating that STARTTYPE for QSE <Q> and Resource <R> was not available for calculation. |
| FR30 | PR 5.7.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCG, if RTMG does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTMG for QSE <Q> and Resource <R> was not available for calculation. |
| FR31 | PR 5.7.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCG, if LSL does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all hours of the Operating Day. |

### Specific Database Needs

None

## RUC Minimum-Energy Revenue (RUCMEREV)

The RUC Minimum-Energy Revenue (RUCMEREV) represents the revenue, earned in Real-Time, for the Resource’s generation up to LSL during all RUC-Committed Hours of the Operating Day.

| Requirement ID | FR32 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.1.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCMEREV by QSE and Resource, for each 15-minute Settlement Interval within each RUC-Committed hour of the Operating Day. RUCMEREV_<Q>_<R>_<SP> = Σi(RTSPP_<SP> * Min (RTMG_<Q>_<R>_<SP>, (LSL_<Q>_<R>_<SP> * (¼)))) |  |

### Input Bill Determinants

| Variable | Unit | Definition |  | Source |
| --- | --- | --- | --- | --- |
| RUCHR | Misc | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. |  | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |
| RTSPP | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Settlement Point for the Settlement Interval i. | MMS |  |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |
| RTMG | MWh | Real-Time Metered Generation—The Resource’s metered generation for the Settlement Interval i. | Settlement System through the Data Aggregation Process |  |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |
| LSL | MW | Low Sustained Limit—The limit established by the QSE, continuously updated in Real-Time, that describes the minimum sustained energy production capability of the Resource for the hour that includes the Settlement Interval i. | MMS |  |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCMEREV | $ | RUC Minimum-Energy Revenue—The sum of the energy revenues for the Resource’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Data Dependencies

D1)	Calculation of the RUCMEREV is dependent on:

- availability of the input bill determinants

- successful calculation of RUCHR

- successful calculation of RTMG

D2)	The RUCMEREV calculation must be complete prior to the calculation of the RUC Make-Whole Payment (RUCMWAMT) and the RUC Clawback Charge (RUCCBAMT).

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR33 | PR 5.7.1.2 | CSWM.P03SettleBillTransfer | DRIVER: The RUCMEREV calculation shall only be attempted for QSE/Resource combinations with a RUC-Commitment in the settlement system for the Operating Day. |
| FR34 | PR 5.7.1.2 | CSWM.P03SettleBillTransfer | No input or output bill determinants shall be rounded or truncated. |
| FR35 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCHR – private - RTSPP – public - RTMG – private - LSL – private - RUCMEREV – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR36 | PR 5.7.1.2 | CSWM.P03SettleBillTransfer | While calculating RUCMEREV, if no RUC-Commitments exist in the Settlement System for the Operating Day, the Settlement System should process as if no RUC-Commitments were issued and exit the process. |
| FR37 | PR 5.7.1.2 | CSWM.P03SettleBillTransfer | While calculating RUCMEREV, if RTMG does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTMG for QSE <Q> and Resource <R> was not available for calculation. |
| FR38 | PR 5.7.1.2 | CSWM.P03SettleBillTransfer | While calculating RUCMEREV, if LSL does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all hours of the Operating Day. |
| FR39 | PR 5.7.1.2 | CSWM.P03SettleBillTransfer | While calculating RUCMEREV, if RTSPP does not exist in the Settlement System for the Operating Day for the Settlement Point, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTSPP for Settlement Point <SP> was not available for calculation. |

### Specific Database Needs

None

## Revenue Less Costs Above LSL During RUC-Committed Hours (RUCEXRR)

The Revenue Less Cost Above LSL During RUC-Committed Hours (RUCEXRR) represents the total revenue for the Resource operating above its LSL less the cost during all RUC-Committed Hours of the Operating Day.

| Requirement ID | FR40 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.1.3, PR 6.6.7.1, PR 6.6.9.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCEXRR by QSE and Resource, for each 15-minute Settlement Interval within each RUC-Committed hour of the Operating Day. RUCEXRR_<Q>_<R>_<SP> = Max {0, Σi[RTSPP_<SP> * Max (0, RTMG_<Q>_<R>_<SP> – (LSL_<Q>_<R>_<SP> * (¼))) + (-1) * (VSSVARAMT_<Q>_<R>_<SP> + VSSEAMT_<Q>_<R>_<SP>) + (-1) * EMREAMT_<Q>_<R>_<SP> – RTAIEC_<Q>_<R>_<SP> * Max (0, RTMG_<Q>_<R>_<SP> – (LSL_<Q>_<R>_<SP> * (¼)))]} |  |

### Input Bill Determinants

| Variable | Unit |  | Definition |  | Source |
| --- | --- | --- | --- | --- | --- |
| RUCHR | Misc |  | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. |  | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |  |
| RTSPP |  | $/MWh |  | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Settlement Point for the Settlement Interval i. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |
| RTAIEC | $/MWh |  | Real-Time Average Incremental Energy CostThe average incremental energy cost, calculated using the Energy Offer Curve capped by the Energy Offer Curve Cap, for the Resource’s generation above the LSL for the Settlement Interval i. See Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC). |  | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.AIEC.REQUIREMENTS.A for further information on the RTAIEC bill determinant. |  |  |  |  |  |
| RTMG |  | MWh |  | Real-Time Metered Generation—The Resource’s metered generation for the Settlement Interval i. | Settlement System through the Data Aggregation Process |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |
| LSL |  | MW |  | Low Sustained Limit—The limit established by the QSE, continuously updated in Real-Time, that describes the minimum sustained energy production capability of the Resource for the hour that includes the Settlement Interval i. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |  |
| VSSVARAMT | $ |  | Voltage Support Service var Amount by interval—The payment to the QSE for the VSS provided by Generation Resource for the 15-minute Settlement Interval. See Section 6.6.7.1, Voltage Support Service Payments. |  | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.RTVOLTAGE.REQUIREMENTS.A for further information on the VSSVARAMT bill determinant. |  |  |  |  |  |
| VSSEAMT | $ |  | Voltage Support Service Energy Amount by interval—The lost opportunity payment to the QSE for ERCOT-directed VSS from the Generation Resource for the 15-minute Settlement Interval. See Section 6.6.7.1, Voltage Support Service Payments. |  | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.RTVOLTAGE.REQUIREMENTS.A for further information on the VSSEAMT bill determinant. |  |  |  |  |  |
| EMREAMT | $ |  | Emergency Energy Amount by interval—The payment to the QSE as additional compensation for the additional energy produced by the Generation Resource in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. See Section 6.6.9.1, Payment for Emergency Power Increase directed by ERCOT. |  | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.RTEMEROPS.REQUIREMENTS.A for further information on the EMREAMT bill determinant. |  |  |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCEXRR | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total energy revenue for the Resource’s generation operating above LSL, less the cost of that generation, during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.5, Energy Revenue Less Cost Above LSL During RUC-Committed Hours. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Data Dependencies

D1)	Calculation of the RUCEXRR is dependent on:

- availability of the input bill determinants

- successful calculation of RUCHR

- successful calculation of RTMG

- successful calculation of RTAIEC

- successful calculation of VSSVARAMT

- successful calculation of VSSEAMT

- successful calculation of EMREAMT

D2)	The RUCEXRR calculation must be complete prior to the calculation of the RUC Make-Whole Payment (RUCMWAMT) and the RUC Clawback Charge (RUCCBAMT).

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR41 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | DRIVER: The RUCEXRR calculation shall only be attempted for QSE/Resource combinations with a RUC-Commitment in the settlement system for the Operating Day. |
| FR42 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | No input or output bill determinants shall be rounded or truncated. |
| FR43 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCHR – private - RTSPP – public - RTAIEC - private - RTMG – private - LSL – private - VSSVARAMT – private - VSSEAMT – private - EMREAMT – private - RUCEXRR – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR44 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | While calculating RUCEXRR, if no RUC-Commitments exist in the Settlement System for the Operating Day, the Settlement System should process as if no RUC-Commitments were issued and exit the process. |
| FR45 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | While calculating RUCEXRR, if RTMG does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTMG for QSE <Q> and Resource <R> was not available for calculation. |
| FR46 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | While calculating RUCEXRR, if LSL does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all hours of the Operating Day. |
| FR47 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | While calculating RUCEXRR, if RTAIEC does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTAIEC for QSE <Q> and Resource <R> was not available for calculation. |
| FR48 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | While calculating RUCEXRR, if VSSVARAMT does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR49 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | While calculating RUCEXRR, if VSSEAMT does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR50 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | While calculating RUCEXRR, if EMREAMT does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR51 | PR 5.7.1.3 | CSWM.P03SettleBillTransfer | While calculating RUCEXRR, if RTSPP does not exist in the Settlement System for the Operating Day for the Settlement Point, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTSPP for Settlement Point <SP> was not available for calculation. |

### Specific Database Needs

None

## Revenue Less Costs During QSE Clawback Intervals (RUCEXRQC)

The Revenue Less Cost During QSE Clawback Intervals (RUCEXRQC) represents the total revenue for the Resource less the cost during all QSE Clawback Intervals of the Operating Day.

A QSE Clawback Interval is:  Any QSE-Committed Interval that is part of a contiguous block that includes at least one RUC-Committed Hour unless it is:

(1)	QSE-committed before the first RUC instruction for any RUC-Committed Hour in that contiguous block; or

(2)	Part of a contiguous block of a QSE-Committed Intervals, at least one of which was committed by the QSE before the RUC instruction described in paragraph (1) above.  PR 2.1

A QSE-Committed Interval is:  A Settlement Interval for which the QSE for a Resource has committed the Resource without a RUC instruction to commit it.  PR 2.1

QSE Clawback Intervals will be determined during the Eligibility process.

See TN.COMS.63C01.ELIGIBILITY.REQUIREMENTS.A for further information.

| Requirement ID | FR52 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.1.4, PR 6.6.7.1, PR 6.6.9.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCEXRQC by QSE and Resource, for each 15-minute Settlement Interval determined to be a QSE Clawback interval (QCLAW_<Q>_<R>_<SP> = 1) of the Operating Day. RUCEXRQC_<Q>_<R>_<SP> = Max {0, Σi[(RTSPP_<SP> * RTMG_<Q>_<R>_<SP>) + (-1) * (VSSVARAMT_<Q>_<R>_<SP> + VSSEAMT_<Q>_<R>_<SP>) + (-1) * EMREAMT_<Q>_<R>_<SP> – [MEPR_<Q>_<R>_<SP> * Min (RTMG_<Q>_<R>_<SP>, (LSL_<Q>_<R>_<SP> * (¼)))] – [RTAIEC_<Q>_<R>_<SP> * Max (0, RTMG_<Q>_<R>_<SP> – (LSL_<Q>_<R>_<SP> * (¼)))]]} |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |  |
| --- | --- | --- | --- | --- |
| RUCHR | Misc | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. | Settlement System |  |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |
| QCLAW | Misc | QSE Clawback Interval Flag—The flag that indicates, for the resource, whether or not the settlement interval is a QSE Clawback interval. If the settlement interval is a QSE Clawback interval, its value is one; otherwise, zero. | Settlement System |  |
| Interval Frequency: 4 / hour Additional Comments: This determinant is not documented in the Nodal Protocols but is required to determine QSE Clawback intervals for a Resource. The QCLAW bill determinant will be an output from the Eligibility process and will only be created for Resources with RUC commitments. See TN.COMS.63C01.ELIGIBILITY.REQUIREMENTS.A for further information on the QCLAW bill determinant. |  |  |  |  |
| RTSPP | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Settlement Point for the Settlement Interval i. | MMS |  |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |
| RTAIEC | $/MWh | Real-Time Average Incremental Energy CostThe average incremental energy cost, calculated using the Energy Offer Curve capped by the Energy Offer Curve Cap, for the Resource’s generation above the LSL for the Settlement Interval i. See Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC). | Settlement System |  |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.AIEC.REQUIREMENTS.A for further information on the RTAIEC bill determinant. |  |  |  |  |
| MEPR | $/MWh | Minimum-Energy Price—The settlement price for minimum energy for the Settlement Interval i. | Settlement System |  |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |
| RTMG | MWh | Real-Time Metered Generation—The Resource’s metered generation for the Settlement Interval i. | Settlement System through the Data Aggregation Process |  |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |
| LSL | MW | Low Sustained Limit—The limit established by the QSE, continuously updated in Real-Time, that describes the minimum sustained energy production capability of the Resource for the hour that includes the Settlement Interval i. | MMS |  |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |
| VSSVARAMT | $ | Voltage Support Service var Amount by interval—The payment to the QSE for the VSS provided by Generation Resource for the 15-minute Settlement Interval. See Section 6.6.7.1, Voltage Support Service Payments. |  | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.RTVOLTAGE.REQUIREMENTS.A for further information on the VSSVARAMT bill determinant. |  |  |  |  |
| VSSEAMT | $ | Voltage Support Service Energy Amount by interval—The lost opportunity payment to the QSE for ERCOT-directed VSS from the Generation Resource for the 15-minute Settlement Interval. See Section 6.6.7.1, Voltage Support Service Payments. |  | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.RTVOLTAGE.REQUIREMENTS.A for further information on the VSSEAMT bill determinant. |  |  |  |  |
| EMREAMT | $ | Emergency Energy Amount by interval—The payment to the QSE as additional compensation for the additional energy produced by the Generation Resource in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. See Section 6.6.9.1, Payment for Emergency Power Increase directed by ERCOT. |  | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.RTEMEROPS.REQUIREMENTS.A for further information on the EMREAMT bill determinant. |  |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCEXRQC | $ | Energy Revenue Less Cost Duringfrom QSE-Clawback Intervals—The sum of the total energy revenue for the Resource’s generation less the cost of the generationprofits for energy during all QSE-Clawback Intervals, for the Operating Day. See Section 5.7.1.6, Energy Revenue Less Cost During QSE Clawback Intervals. | Settlement System |
| Interval Frequency: 4 / hour GP&L Comments: spelling or spacing on these words Additional Comments: None |  |  |  |

### Data Dependencies

D1)	Calculation of the RUCEXRQC is dependent on:

- availability of the input bill determinants

- successful calculation of RUCHR

- successful calculation of RTMG

- successful calculation of QCLAW

- successful calculation of RTAIEC

- successful calculation of MEPR

- successful calculation of VSSVARAMT

- successful calculation of VSSEAMT

- successful calculation of EMREAMT

D2)	The RUCEXRQC calculation must be complete prior to the calculation of the RUC Make-Whole Payment (RUCMWAMT) and the RUC Clawback Charge (RUCCBAMT).

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR53 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | DRIVER: The RUCEXRQC calculation shall only be attempted for QSE/Resource combinations with a RUC-Commitment in the settlement system for the Operating Day. |
| FR54 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | No input or output bill determinants shall be rounded or truncated. |
| FR55 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCHR – private - QCLAW – private - RTSPP – public - RTAIEC – private - MEPR – private - RTMG – private - LSL – private - VSSVARAMT – private - VSSEAMT – private - EMREAMT – private - RUCEXRQC – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR56 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | While calculating RUCEXRQC, if no RUC-Commitments exist in the Settlement System for the Operating Day, the Settlement System should process as if no RUC-Commitments were issued and exit the process. |
| FR57 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | While calculating RUCEXRQC, if QCLAW does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that QCLAW for QSE <Q> and Resource <R> was not available for calculation. |
| FR58 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | While calculating RUCEXRQC, if RTMG does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTMG for QSE <Q> and Resource <R> was not available for calculation. |
| FR59 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | While calculating RUCEXRQC, if LSL does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR60 | PR 5.7.1.4 | Will be completed August 31, 2006 | While calculating RUCEXRQC, if MEPR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that MEPR for QSE <Q> and Resource <R> was not available for calculation. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR61 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | While calculating RUCEXRQC, if RTAIEC does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTAIEC for QSE <Q> and Resource <R> was not available for calculation. |
| FR62 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | While calculating RUCEXRQC, if VSSVARAMT does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR63 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | While calculating RUCEXRQC, if VSSEAMT does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR64 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | While calculating RUCEXRQC, if EMREAMT does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR65 | PR 5.7.1.4 | CSWM.P03SettleBillTransfer | While calculating RUCEXRQC, if RTSPP does not exist in the Settlement System for the Operating Day for the Settlement Point, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTSPP for Settlement Point <SP> was not available for calculation. |

### Specific Database Needs

None

## Number of continuous RUC Committed Hours (RUCHR)

The Number of RUC-Committed Hours (RUCHR) represents the total number of hours a Resource is RUC-Committed for an Operating Day.

| Requirement ID | FR66 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.1, PR 5.7.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCHR by QSE and Resource, for each hour of the Operating Day. ΣhΣruc If Resource is RUC-Committed for the hour, RUCHR_<Q>_<R>_<SP> = 1 Else, RUCHR_<Q>_<R>_<SP> = 0 |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| RUC | Misc | RUC—The RUC-Commitment, issued by a RUC process, indicating the hours in which the Resource was committed by that RUC process. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCHR | Misc | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Data Dependencies

D1)	Calculation of the RUCHR is dependent on:

- availability of the input bill determinants

D2)	The RUCHR calculation must be complete prior to the calculation of the RUC Make-Whole Payment (RUCMWAMT) and the RUC Clawback Charge (RUCCBAMT).

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR67 | PR 5.7.1 PR 5.7.2 | CSWM.P03SettleBillTransfer | DRIVER: The RUCHR calculation shall only be attempted for QSE/Resource combinations with a RUC-Commitment (RUC_<Q>_<R>_<SP>_<RUC>) in the settlement system for the Operating Day. |
| FR68 | PR 5.7.1 PR 5.7.2 | CSWM.P03SettleBillTransfer | No input or output bill determinants shall be rounded or truncated. |
| FR69 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUC – private - RUCHR – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR70 | PR 5.7.1 PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUCHR, if there are no RUC existing in the Settlement System for the Operating Day, the Settlement System shall process as if no RUC-Commitments were issued and exit the process. |

### Specific Database Needs

None

## Number of Continuous RUC De-committed Hours (NCDCHR)

The Number of Continuous RUC-Decommitted hours (NCDCHR) represents the total number of hours a Resource is RUC-Decommitted for an Operating Day.

| Requirement ID | FR71 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.3 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the NCDCHR by QSE and Resource by setting NCDCHR, for each hour of the RUC-Decommitment period, equal to the sum of all continuous RUC-Decommitted hours of the RUC-Decommitment. For the hour within a RUC-Decommitment; NCDCHR_<Q>_<R>_<SP> = ΣhΣruc (RUCD_<Q>_<R>_<SP>_<RUC>) |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| RUCD | Misc | RUCD—The RUC-Decommitment, issued by a RUC process, indicating the hours in which the Resource was decommitted. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| NCDCHR | Misc | Number of Continuous De-committed Hours— The number of continuous decommitment hours within an Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Data Dependencies

D1)	Calculation of the NCDCHR is dependent on:

- availability of the input bill determinants

D2)	The NCDCHR calculation must be complete prior to the calculation of the Startup Price (SUPR), Minimum-Energy Price (MEPR) and RUC Decommitment Payment (RUCDCAMT).

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR72 | PR 5.7.3 | CSWM.P03SettleBillTransfer | DRIVER: The NCDCHR calculation shall only be attempted for QSE/Resource combinations with a RUC-Decommitment (RUCD_<Q>_<R>_<SP>_<RUC>) in the settlement system for the Operating Day. |
| FR73 | PR 5.7.3 | CSWM.P03SettleBillTransfer | No input or output bill determinants shall be rounded or truncated. |
| FR74 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCD – private - NCDCHR – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR75 | PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating NCDCHR, if there are no RUCD existing in the Settlement System for the Operating Day, the Settlement System shall process as if no RUC-Decommitments were issued and exit the process. |

### Specific Database Needs

None

## RUC Make-Whole Payment (RUCMWAMT)

ERCOT shall pay each QSE, whose Resource is selected by RUC, the dollar amount required to make the Resource whole (RUCG) after taking in to account all revenues (RUCMEREV, RUCEXRR, RUCEXRQC) earned by the Resource for the Operating Day.  The RUC Make-Whole Payment is a calculation of daily values spread evenly across all RUC-Committed hours of the Operating Day.

If the revenues (RUCMEREV, RUCEXRR, RUCEXRQC) earned by the Resource for the Operating Day is greater than the dollar amount required to make the Resource whole (RUCG) then no RUC Make-Whole Payment is due to the QSE for the Resource.

| Requirement ID | FR76 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCMWAMT by QSE and Resource, with a RUC Commitment, for each RUC-committed hour of the Operating Day. RUCMWAMT_<Q>_<R>_<SP> = (-1) * MAX(0, RUCG_<Q>_<R>_<SP> - RUCMEREV_<Q>_<R>_<SP> -RUCEXRR_<Q>_<R>_<SP> - RUCEXRQC_<Q>_<R>_<SP>) / RUCHR_<Q>_<R>_<SP> Note: RUCMEREV, RUCEXRR and RUCEXRQC are calculated and stored on a 15-minute interval basis. When calculating RUCMWAMT, the sum of all intervals for the Operating Day shall be used. |  |

| Requirement ID | FR204 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 9.5.3 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCMWAMTQSETOT by QSE, by summing all RUCMWAMT payments received by each Resource represented by the QSE, for each hour of the Operating Day. RUCMWAMTQSETOT_<Q> = Σr (RUCMWAMT_<Q>_<R>_<SP>) |  |

| Requirement ID | FR77 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCMWAMTRUCTOT by RUC Process, for each RUC-committed hour of the Operating Day. RUCMWAMTRUCTOT_<RUC> = ΣqΣrΣspRUCMWAMT_<Q>_<R>_<SP> Each RUCMWAMTTUCTOT must sum all RUCMWAMT for all QSE/Resource combinations that were RUC-Committed in the same hour by the same RUC Process, for each hour of the Operating Day. There is a possibility of having more than one RUCMWAMTRUCTOT for the same hour per Operating Day but for different RUC processes, depending on the number of RUC Processes that committed Resources. The RUC_<Q>_<R>_<SP>_<RUC> bill determinants will reflect RUC committed hours and the RUC process that committed them. |  |

| Requirement ID | FR78 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCMWAMTTOT for each hour of the Operating Day. RUCMWAMTTOT = ΣrucRUCMWAMTRUCTOT_<RUC> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RUC | Misc | RUC—The RUC-Commitment, issued by a RUC process, indicating the hours in which the Resource was committed by that RUC process. | MMS |
| Interval Frequency: 1/ hour Additional Comments: RUC shall be used in the calculation of RUCMWAMTRUCTOT to identify the RUC Process ID. |  |  |  |
| RUCHR | Misc | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RUCG | $ | RUC Guarantee—The sum of the Resource’s Startup Costs and Minimum-Energy Costs during all RUC-Committed Hours, for the Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: None |  |  |  |
| RUCMEREV | $ | RUC Minimum-Energy Revenue—The sum of the energy revenues for the Resource’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: RUCMEREV is calculated and stored on a 15-minute interval basis. When calculating RUCMWAMT, the sum of all intervals for the Operating Day shall be used. |  |  |  |
| RUCEXRR | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total energy revenue for the Resource’s generation operating above the its LSL less the cost of the generation during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.5, Energy Revenue Less Cost Above LSL During RUC-Committed Hours. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: RUCEXRR is calculated and stored on a 15-minute interval basis. When calculating RUCMWAMT, the sum of all intervals for the Operating Day shall be used. |  |  |  |
| RUCEXRQC | $ | Energy Revenue Less Cost Duringfrom QSE-Clawback Intervals—The sum of the total energy revenue for the Resource’s generation less the cost of the generationprofits for energy during all QSE-Clawback Intervals, for the Operating Day. See Section 5.7.1.6, Energy Revenue Less Cost During QSE Clawback Intervals. GP&L Comments: spelling or spacing on these words | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: RUCEXRQC is calculated and stored on a 15-minute interval basis. When calculating RUCMWAMT, the sum of all intervals for the Operating Day shall be used. |  |  |  |

### Output Bill Determinants

| Variable | Unit |  | Definition | Destination |
| --- | --- | --- | --- | --- |
| RUCMWAMT | $ |  | RUC Make-Whole Payment—The RUC Make-Whole Payment to the QSE for a Resource, for each RUC-Committed Hour of the Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |
| RUCMWAMTQSETOT | $ | RUC Make-Whole Payment Amount QSE Total  The QSE total for RUC Make-Whole payments for the hour. |  | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements |  |  |  |  |
| RUCMWBILLAMT | $ | RUC Make-Whole Payment Bill Amount  The difference between two consecutive daily summed settlement runs of RUCMWAMTQSETOT, for the QSE, for an Operating Day. |  | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |  |
| RUCMWAMTRUCTOT | $ |  | RUC Make-Whole Amount Total per RUC Process—The sum of RUC Make-Whole Payments for a particular RUC process, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This is the total sum of all RUC Make-Whole payments (RUCMWAMT) for each hour for a particular RUC process and is used in the RUC Capacity-Short Charge (RUCCSAMT). For example, if in the Day-Ahead RUC, a unit is asked to be on-line for hour 12 and through this calculation is paid $1,000 for that hour but later another unit, in an Hour-Ahead RUC, is asked to be on-line for hour 12 and is paid $2,000 for that hour then we need to differentiate between the first RUC instance of hour 12 for $1,000 and the second RUC instance of hour 12 for $2,000. The RUCMWAMTRUCTOT must indicate the RUC process it represents. |  |  |  |  |
| RUCMWAMTTOT | $ |  | RUC Make-Whole Amount Total—The sum of RUC Make-Whole Payments for all QSEs and RUC processes, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: |  |  |  |  |

### Data Dependencies

D1)	Calculation of the RUCMWAMT is dependent on:

- availability of the input bill determinants

- successful calculation of RUCHR

- successful calculation of RUCG

- successful calculation of RUCMEREV

- successful calculation of RUCEXRR

- successful calculation of RUCEXRQC

D2)	Calculation of the RUCMWAMTRUCTOT is dependent on the successful calculation of RUCMWAMT.

D3)	Calculation of the RUCMWAMTTOT is dependent on the successful calculation of RUCMWAMTRUCTOT.

D4)	The RUCMWAMTRUCTOT calculation must be complete prior to the calculation of the RUC Capacity-Short Charge (RUCCSAMT).

D5)	The RUCMWAMTTOT calculation must be complete prior to the calculation of the RUC Make-Whole Uplift Charge (LARUCAMT).

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR79 | PR 5.7.1 | CSWM.P03SettleBillTransfer | DRIVER: The RUCMWAMT calculation shall only be attempted for QSE/Resource combinations with a RUC-Commitment in the settlement system for the Operating Day. |
| FR80 | PR 5.7.4.1 | CSWM.P03SettleBillTransfer | DRIVER: For each RUC Process, the RUCMWAMTRUCTOT calculation shall only be attempted if there is at least one (1) RUC Make-Whole Payment calculated whose payment amount is attributable to the RUC Process for the Operating Day. |
| FR81 | PR 5.7.4.1 | CSWM.P03SettleBillTransfer | The RUCMWAMTRUCTOT bill determinant must be identified with a RUC Process identifier. |
| FR82 | PR 5.7.4.2 | CSWM.P03SettleBillTransfer | DRIVER: The RUCMWAMTTOT calculation shall only be attempted for an Operating Day when at least one <1> RUCMWAMTRUCTOT is calculated for the Operating Day. |
| FR205 | PR 9.5.5 (2) PR 9.5.6 (6) PR9.5.8 (3) | CSWM.P03SettleBillTransfer | The RUCMWBILLAMT value shall be calculated as the difference between the total daily sum of RUCMWAMTQSETOT for current Settlement Run and the total daily sum of RUCMWAMTQSETOT for the most previous Settlement Run. |
| FR83 | PR 5.7.1 | CSWM.P03SettleBillTransfer | No input bill determinants shall be rounded or truncated. |
| FR84 | PR 9.5.3 | CSWM.P03SettleBillTransfer | All output bill determinants shall be rounded to two (2) decimal places. |
| FR85 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUC – private - RUCHR – private - RUCG – private - RUCMEREV – private - RUCEXRR – private - RUCEXRQC – private - RUCMWAMT – private - RUCMWAMTQSETOT – private - RUCMWBILLAMT – private - RUCMWAMTRUCTOT – public - RUCMWAMTTOT – public |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR86 | PR 5.7.1 | CSWM.P03SettleBillTransfer | While calculating RUCMWAMT, if there are no RUCHR existing in the Settlement System for the Operating Day, the Settlement System shall process as if no RUC-Commitments were issued and exit the process. |
| FR87 | PR 5.7.1 | Will be completed August 31, 2006 | While calculating RUCMWAMT, if RUCHR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “RUCHR for QSE <Q> and Resource <R> was not available for calculation of RUCMWAMT.” See FR86 for RUC Make-Whole Amount driver error handling. |
| FR88 | PR 5.7.1 | CSWM.P03SettleBillTransfer | While calculating RUCMWAMT, if RUCG does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCG for QSE <Q> and Resource <R> was not available for calculation. |
| FR89 | PR 5.7.1 | CSWM.P03SettleBillTransfer | While calculating RUCMWAMT, if RUCMEREV does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCMEREV for QSE <Q> and Resource <R> was not available for calculation. |
| FR90 | PR 5.7.1 | CSWM.P03SettleBillTransfer | While calculating RUCMWAMT, if RUCEXRR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCEXRR for QSE <Q> and Resource <R> was not available for calculation. |
| FR91 | PR 5.7.1 | CSWM.P03SettleBillTransfer | While calculating RUCMWAMT, if RUCEXRQC does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCEXRQC for QSE <Q> and Resource <R> was not available for calculation. |
| FR92 | PR 5.7.4.1 | Will be completed August 31, 2006 | While calculating RUCMWAMTRUCTOT and for each RUC Process, if RUCMWAMT does not exist in the Settlement System for the Operating Day for the RUC Process, the Settlement System shall process as if there were no RUC Make-Whole Payments for that RUC Process and exit the calculation. The calculation driver method eliminates the need for this requirement. See FR80. |
| FR93 | PR 5.7.4.2 | Will be completed August 31, 2006 | While calculating RUCMWAMTTOT, if there are no RUCMWAMTRUCTOT in the Settlement System for the Operating Day, the Settlement System shall create a RUCMWAMTTOT data cut with zeros for all hours of the Operating Day. There will be no RUCMWAMTRUCTOT data cuts for any RUC Process for the Operating Day for this condition to occur. The calculation driver method eliminates the need for this requirement. See FR82. |

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR94 | PR 5.7.4.1 | CSWM.P03SettleBillTransfer | The settlement system shall visibly identify and track individual RUC Processes within or for the bill determinant of RUCMWAMTRUCTOT. |

## RUC Clawback Charge (RUCCBAMT)

ERCOT shall charge each QSE, whose Resource is selected by RUC, the dollar amount earned through revenues (RUCMEREV, RUCEXRR, RUCEXRQC) that is greater than the amount required to make the Resource whole (RUCG) for the Operating Day.  The amount charged to the QSE for the Resource will depend on whether the QSE submitted a valid Three-Part Supply Offer for the Resource.

If a valid Three-Part Supply Offer was submitted into the DAM:

- For RUC-Committed Hours the Clawback percentage is 50% (Factor = 0.5) unless EECP is in effect, then the Clawback percentage is 0% (Factor = 0.0)

- For QSE-Clawback Intervals the Clawback percentage is 0% (Factor = 0.0)

If a valid Three-Part Supply Offer was not submitted into the DAM:

  - For RUC-Committed Hours the Clawback percentage is 100% (Factor = 1.0) unless EECP is in effect, then the Clawback percentage is 50% (Factor = 0.5)

  - For QSE-Clawback Intervals the Clawback percentage is 50% (Factor = 0.5)

If the revenues (RUCMEREV, RUCEXRR, RUCEXRQC) earned by the Resource for the Operating Day are less than the dollar amount required to make the Resource whole (RUCG) then no charge is made the QSE for the Resource.  If RUC-Committed Resource was not made whole by revenue earned and gets a RUC Make-Whole Payment it shall not get a RUC Clawback Charge.  If a Resource that received a RUC Commitment does not get a RUC Make-Whole Payment, it is eligible for a RUC Clawback Charge.

The RUC Clawback Charge is a calculation of daily values divided evenly across all RUC-Committed hours of the Operating Day.

| Requirement ID | FR95 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate RUC Clawback factors (RUCCBFR for RUC Committed hours and RUCCBFC for Clawback hours) by QSE and Resource, with a RUC Commitment, for each Operating Day. If QSE submits a valid Three-Part Supply Offer into the DAM (VTPSOFLAG = 1) for the Resource, Then, RUCCBFR_<Q>_<R>_<SP> = 0.5 for the Operating Day RUCCBFC_<Q>_<R>_<SP> = 0.0 for the Operating Day Otherwise, RUCCBFR_<Q>_<R>_<SP> = 1.0 for the Operating Day RUCCBFC_<Q>_<R>_<SP> = 0.5 for each hour of the Operating Day If EECP is in effect for any hour of the Operating Day where the Resource is RUC-Committed (EECP = 1) and if QSE submits a valid Three-Part Supply Offer into the DAM (VTPSOFLAG = 1) for the Resource, Then, RUCCBFR_<Q>_<R>_<SP> = 0.0 for the Operating Day Otherwise, RUCCBFR_<Q>_<R>_<SP> = 0.5 for the Operating Day The Clawback factor for Clawback intervals (RUCCBFC) does not change due to an EECP implementation. |  |

| Requirement ID | FR96 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCCBAMT by QSE and Resource, with a RUC Commitment, for each hour of the Operating Day. If (RUCMEREV_<Q>_<R>_<SP> + RUCEXRR_<Q>_<R>_<SP> - RUCG_<Q>_<R>_<SP>) > 0, Then, RUCCBAMT_<Q>_<R>_<SP> = [(RUCMEREV_<Q>_<R>_<SP> + RUCEXRR_<Q>_<R>_<SP> - RUCG_<Q>_<R>_<SP>) * RUCCBFR_<Q>_<R>_<SP> + RUCEXRQC_<Q>_<R>_<SP> * RUCCBFC_<Q>_<R_<SP>] / RUCHR_<Q>_<R>_<SP> Otherwise, RUCCBAMT_<Q>_<R>_<SP> = [Max (0, RUCMEREV_<Q>_<R>_<SP> + RUCEXRR_<Q>_<R>_<SP> + RUCEXRQC_<Q>_<R>_<SP> - RUCG_<Q>_<R>_<SP>) * RUCCBFC_<Q>_<R>_<SP>] / RUCHR_<Q>_<R>_<SP> RUCMEREV, RUCEXRR and RUCEXRQC are calculated and stored on a 15-minute interval basis. When calculating RUCCBAMT, the sum of all intervals for the Operating Day shall be used. |  |

| Requirement ID | FR206 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 9.5.3 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCCBAMTQSETOT by QSE, by summing all RUCCBAMT charges received by each Resource represented by the QSE, for each hour of the Operating Day. RUCCBAMTQSETOT_<Q> = Σr (RUCCBAMT_<Q>_<R>_<SP>) |  |

| Requirement ID | FR97 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.5 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCCBAMTTOT for each hour of the Operating Day. RUCCBAMTTOT = ΣqΣrΣspRUCCBAMT_<Q>_<R>_<SP> |  |

### Input Bill Determinants

| Variable | Unit |  | Definition | Original Source |
| --- | --- | --- | --- | --- |
| RUCHR | Misc |  | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |  |
| EECP | Misc |  | Emergency Electric Curtailment Plan Flag—The flag that indicates, for each hour of the Operating Day, when an Emergency Electric Curtailment Plan is in effect. The value will be one (1) if EECP is in effect for any portion of the hour and zero (0) if EECP is not in effect for any portion of the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: This determinant is not documented in the Nodal Protocols but is required to determine RUC Clawback factors for the calculation of RUCCBAMT. |  |  |  |  |
| VTPSOFLAG | Misc |  | Valid Three-Part Supply Offer Flag—The flag that indicates, for each Operating Day, when valid Three-Part Supply Offer has been submitted for consideration in the DAM. The value will be one (1) if a Three-Part Supply Offer is submitted for consideration in the DAM and zero (0) otherwise. | MMS |
| Interval Frequency: 1 / day Additional Comments: This determinant is not documented in the Nodal Protocols but is required to determine RUC Clawback factors for the calculation of RUCCBAMT. |  |  |  |  |
| RUCG | $ |  | RUC Guarantee—The sum of the Resource’s Startup Costs and Minimum-Energy Costs during all RUC-Committed Hours, for the Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: None |  |  |  |  |
| RUCMEREV | $ |  | RUC Minimum-Energy Revenue—The sum of the energy revenues for the Resource’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: RUCMEREV is calculated and stored on a 15-minute interval basis. When calculating RUCCBAMT, the sum of all intervals for the Operating Day shall be used. |  |  |  |  |
| RUCEXRR | $ |  | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total energy revenue for the Resource’s generation operating above the its LSL less the cost of the generation during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.5, Energy Revenue Less Cost Above LSL During RUC-Committed Hours. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: RUCEXRR is calculated and stored on a 15-minute interval basis. When calculating RUCCBAMT, the sum of all intervals for the Operating Day shall be used. |  |  |  |  |
| RUCEXRQC | $ |  | Energy Revenue Less Cost Duringfrom QSE-Clawback Intervals—The sum of the total energy revenue for the Resource’s generation less the cost of the generationprofits for energy during all QSE-Clawback Intervals, for the Operating Day. See Section 5.7.1.6, Energy Revenue Less Cost During QSE Clawback Intervals. | Settlement System |
| Interval Frequency: 4 / hour GP&L Comments: spelling or spacing on these words Additional Comments: RUCEXRQC is calculated and stored on a 15-minute interval basis. When calculating RUCCBAMT, the sum of all intervals for the Operating Day shall be used. |  |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCCBFR | Misc | RUC Claw-Back Factor for RUC-Committed Hours—The Resource’s Claw-Back Factor for RUC-Committed Hours for the Operating Day, which is 50% if a Three-Part Supply Offer was submitted and 100% otherwise. During EECP conditions the Resource’s clawback factor for RUC-Committed Hours for the Operating Day is 0% if a Three-Part Supply Offer was submitted and 50% otherwise. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: None |  |  |  |
| RUCCBFC | Misc | RUC Claw-Back Factor for QSE Clawback intervals— The Resource’s clawback factor for QSE Clawback Intervals for the Operating Day, which is 0% if a Three-Part Supply Offer was submitted and 50% otherwise. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCCBAMT | $ | RUC Clawback Charge––The RUC Clawback Charge to a QSE for a Resource as described in Section 5.7.2, RUC Clawback Charge, for each RUC-Committed Hour of the Operating Day for that Resource. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RUCCBAMTQSETOT | $ | RUC Clawback Charge Amount QSE Total  The QSE total for RUC Clawback Charges for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |
| RUCCBBILLAMT | $ | RUC Clawback Charge Bill Amount  The difference between two consecutive daily summed settlement runs of RUCCBAMTQSETOT for the QSE, for an Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |
| RUCCBAMTTOT | $ | RUC Clawback Amount Total—The sum of RUC Clawback Charges for all QSEs, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Data Dependencies

D1)	Calculation of the RUCCBFR and RUCCBFC are dependent on:

  - availability of the input bill determinants

  - availability of factor value data (within settlement system)

D2)	Calculation of the RUCCBAMT is dependent on:

  - availability of the input bill determinants

  - successful calculation of RUCHR

  - successful calculation of RUCG

  - successful calculation of RUCMEREV

  - successful calculation of RUCEXRR

  - successful calculation of RUCEXRQC

  - successful calculation of RUCCBFR

  - successful calculation of RUCCBFC

D3)	Calculation of the RUCCBAMTTOT is dependent on the successful calculation of RUCCBAMT.

D4)	The RUCCBAMTTOT calculation must be complete prior to the calculation of the RUC Clawback Payment (LARUCCBAMT).

### Assumptions

A1)	The valid Three-Part Supply Offer submitted for consideration in the DAM for the Operating Day will be used to determine Clawback factors.

A2)	If EECP is in effect for any hour that a Resource is RUC-Committed within the Operating Day, the Clawback factor for RUC hours (RUCCBFR) will default to the appropriate value for EECP for all RUC-Committed hours of the Operating Day for that Resource.

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR98 | PR 5.7.2 | CSWM.P03SettleBillTransfer | DRIVER: The RUCCBAMT calculation shall only be attempted for QSE/Resource combinations with a RUC-Commitment in the settlement system for the Operating Day. |
| FR99 | PR 5.7.5 | CSWM.P03SettleBillTransfer | DRIVER: The RUCCBAMTTOT calculation shall only be attempted for an Operating Day when the sum of all RUC Clawback Charges is not equal to zero. |
| FR100 | PR 5.7.2 | CSWM.P03SettleBillTransfer | The Clawback factor value options for RUC-Committed Hours of 0.5 for offer submitted and 1.0 for offer not submitted, used to calculate each QSE/Resource RUCCBFR are required to be stored as variables with start and stop times. |
| FR101 | PR 5.7.2 | CSWM.P03SettleBillTransfer | The Clawback factor value options for RUC-Committed Hours when EECP is in effect of 0.0 for offer submitted and 0.5 for offer not submitted, used to calculate each QSE/Resource RUCCBFR are required to be stored as variables with start and stop times. |
| FR102 | PR 5.7.2 | CSWM.P03SettleBillTransfer | The Clawback factor value options for QSE Clawback Intervals of 0.0 for offer submitted and 0.5 for offer not submitted, used to calculate each QSE/Resource RUCCBFC are required to be stored as variables with start and stop times. |
| FR207 | PR 9.5.5 (2) PR 9.5.6 (6) PR9.5.8 (3) | CSWM.P03SettleBillTransfer | The RUCCBBILLAMT value shall be calculated as the difference between the total daily sum of RUCCBAMTQSETOT for current Settlement Run and the total daily sum of RUCCBAMTQSETOT for the most previous Settlement Run. |
| FR103 | PR 5.7.2 | CSWM.P03SettleBillTransfer | No input or intermediate bill determinants shall not be rounded or truncated. |
| FR104 | PR 9.5.3 | CSWM.P03SettleBillTransfer | All output bill determinants shall be rounded to two (2) decimal places. |
| FR105 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and shall be classified in the settlement system as: - RUCHR – private - VTPSOFLAG – private - EECP – public - RUCCBFR – private - RUCCBFC – private - RUCG – private - RUCMEREV – private - RUCEXRR – private - RUCEXRQC – private - RUCCBAMT – private - RUCCBAMTQSETOT – private - RUCCBBILLAMT – private - RUCCBAMTTOT – public |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR106 | PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUC Clawback Factors (RUCCBFR and RUCCBFC), if there are no RUCHR existing in the Settlement System for the Operating Day, the Settlement System shall process as if no RUC-Commitments were issued and exist the process. |
| FR107 | PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUCCBFR, if Clawback Factor Values for RUC-Committed Hours do not exist in the Settlement System for the Operating Day, the Settlement System shall process as if no offer was submitted, default to <1.0> for the Operating Day and provide a WARN-DEFAULT error message indicating that Clawback Factors for RUC-Committed Hours were not available for calculation. |
| FR108 | PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUCCBFR, if Clawback Factor Values for RUC-Committed Hours when EECP is in effect do not exist in the Settlement System for the Operating Day, the Settlement System shall process as if no EECP condition exists, calculate RUCCBFR based on VTPSOFLAG status and provide a WARN-DEFAULT error message indicating that Clawback Factors for RUC-Committed Hours when EECP is in effect were not available for calculation. |
| FR216 | PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUCCBFC, if Clawback Factor Values for QSE Clawback intervals do not exist in the Settlement System for the Operating Day, the Settlement System shall process as if no offer was submitted, default to <0.5> for the Operating Day and provide a WARN-DEFAULT error message indicating that Clawback Factors for QSE Clawback Intervals were not available for calculation. |
| FR109 | PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUCCBAMT, if there are no RUCHR existing in the Settlement System for the Operating Day, the Settlement System shall process as if no RUC-Commitments were issued and exit the process. |
| FR110 | PR 5.7.2 | Will be completed August 31, 2006 | While calculating RUCCBAMT, if RUCHR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “RUCHR for QSE <Q> and Resource <R> was not available for calculation of RUCCBAMT.” See FR109 for RUC Clawback Amount driver error handling. |
| FR111 | PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUCCBAMT, if RUCG does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCG for QSE <Q> and Resource <R> was not available for calculation. |
| FR112 | PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUCCBAMT, if RUCMEREV does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCMEREV for QSE <Q> and Resource <R> was not available for calculation. |
| FR113 | PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUCCBAMT, if RUCEXRR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCEXRR for QSE <Q> and Resource <R> was not available for calculation. |
| FR114 | PR 5.7.2 | CSWM.P03SettleBillTransfer | While calculating RUCCBAMT, if RUCEXRQC does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCEXRQC for QSE <Q> and Resource <R> was not available for calculation. |
| FR115 | PR 5.7.5 | Will be completed August 31, 2006 | While calculating RUCCBAMTTOT, if there are no RUCCBAMT in the Settlement System for the Operating Day, the Settlement System shall create a RUCCBAMTTOT data cut with zeros for all hours of the Operating Day. There will be no RUCCBAMT data cuts for any QSE/Resource combination for the Operating Day for this condition to occur. The calculation driver method eliminates the need for this requirement. See FR99. |

### Specific Database Needs

None

## RUC Decommitment Payment (RUCDCAMT)

If ERCOT decommits a QSE committed Resource that is not scheduled to shutdown within the Operating Day, then ERCOT shall pay the affected QSE an amount as calculated below for the hours of decommitment.  The hours used in the calculation are the hours beginning with the first decommitted hour until the later of:

(a)	The hour ERCOT determines that the Resource may again be at LSL; and

(b)	The end of the last hour of the Operating Day. PR 5.7.3 (2)

If ERCOT decommits a QSE-committed Resource during the RUC process earlier than its scheduled shutdown within the Operating Day, then no compensation is due to the affected QSE from ERCOT. PR 5.7.3 (1)

The RUC Decommitment Payment is calculated using per start, hourly and 15-minute data divided evenly across all RUC-Decommitted hours of the Operating Day.

| Requirement ID | FR116 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.3 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCDCAMT, for each eligible QSE/Resource, with a RUC-Decommitment, for each hour of the Operating Day. RUCDCAMT_<Q>_<R>_<SP> = (-1) * Max (0, (SUPR_<Q>_<R>_<SP>_<ST> - Σi(Max (0, MEPR_<Q>_<R>_<SP> - RTSPP_<SP>) * (LSL_<Q>_<R>_<SP> * (¼))))) / NCDCHR_<Q>_<R>_<SP> The Eligibility process will provide a Startup Flag (SUFLAG) and RUCDSTARTTYPE for use in the RUCDCAMT calculation. The SUFLAG will indicate if the start is attributable to the RUC-Decommitment and the RUCDSTARTTYPE will indicate the type of start (hot, intermediate or cold) that the QSE/Resource shall be paid See TN.COMS.63C01.ELIGIBILITY.REQUIREMENTS.A for further information. |  |

| Requirement ID | FR208 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 9.5.3 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCDCAMTQSETOT by QSE, by summing all RUCDCAMT payments received by each Resource represented by the QSE, for each hour of the Operating Day. RUCDCAMTQSETOT_<Q> = Σr (RUCDCAMT_<Q>_<R>_<SP>) |  |

| Requirement ID | FR117 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.6 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCDCAMTTOT for each hour of the Operating Day. RUCDCAMTTOT = ΣqΣrRUCDCAMT_<Q>_<R> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| SUFLAG | Misc | Startup Flag – The flag that indicates who is responsible paying the startup costs, for a start. If DAM is responsible for the start the value will be <1>, if a RUC-Commitment is responsible for the start the value will be <2>, and if a RUC-Decommitment is responsible for the start the value will be <3>. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: |  |  |  |
| RUCDSTARTTYPE | Misc | RUC-Decomitment Start Type — The type of start the RUC-Decommitted Resource is eligible to be paid. A value of <1> indicates the start type is hot. A value of <2> indicates the start type is intermediate. A value of <3> indicates the start type is cold. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: |  |  |  |
| NCDCHR | Misc | Number of Continuous De-committed Hours— The number of continuous decommitment hours within an Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: |  |  |  |
| SUPR | $/Start | Startup Price per start—The settlement price for the start for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: There shall be a SUPR, for each QSE/Resource combination with a RUC-Commitment, for each type of start: Hot, Intermediate and Cold. |  |  |  |
| MEPR | $/MWh | Minimum-Energy Price—The settlement price for minimum energy for the Settlement Interval i. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| LSL | MW | Low Sustained Limit—The limit established by the QSE, continuously updated in Real-Time, that describes the minimum sustained energy production capability of the Resource for the hour that includes the Settlement Interval i. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RTSPP | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Settlement Point for the Settlement Interval i. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCDCAMT | $ | RUC De-commitment Payment Amount—The payment to the QSE for the Resource that was de-committed by ERCOT but that was not scheduled to shut down in the Operating Day, for each decommited hour of the Operating Day. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RUCDCAMTQSETOT | $ | RUC De-commitment Payment Amount QSE Total  The QSE total for RUC De-commitment payments for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |
| RUCDCBILLAMT | $ | RUC De-commitment Payment Bill Amount  The difference between two consecutive daily summed settlement runs of RUCDCAMTQSETOT for the QSE, for an Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |
| RUCDCAMTTOT | $ | RUC Decommitment Payment Total—The sum of RUC Decommitment Payments to all QSEs, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Data Dependencies

D1)	Calculation of the RUCDCAMT is dependent on:

- availability of the input bill determinants

- successful calculation of NCDCHR

- successful calculation of SUPR

- successful calculation of MEPR

D2)	Calculation of the RUCDCAMTTOT is dependent on the successful calculation of RUCDCAMT.

D3)	The RUCDCAMTTOT calculation must be complete prior to the calculation of the RUC Decommitment Charge (LARUCDCAMT).

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR118 | PR 5.7.3 | CSWM.P03SettleBillTransfer | DRIVER: The RUCDCAMT calculation shall only be attempted for QSE/Resource combinations with a RUC-Decommitment in the settlement system for the Operating Day. |
| FR119 | PR 5.7.6 | CSWM.P03SettleBillTransfer | DRIVER: The RUCDCAMTTOT calculation shall only be attempted for an Operating Day when the sum of all RUC Decommitment Payments is not equal to zero. |
| FR209 | PR 9.5.5 (2) PR 9.5.6 (6) PR9.5.8 (3) | CSWM.P03SettleBillTransfer | The RUCDCBILLAMT value shall be calculated as the difference between the total daily sum of RUCDCAMTQSETOT for current Settlement Run and the total daily sum of RUCDCAMTQSETOT for the most previous Settlement Run. |
| FR120 | PR 5.7.3 | CSWM.P03SettleBillTransfer | No input bill determinants shall not be rounded or truncated. |
| FR121 | PR 9.5.3 | CSWM.P03SettleBillTransfer | All output bill determinants shall be rounded to two (2) decimal places. |
| FR122 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and shall be classified in the settlement system as: - NCDCHR – private - SUFLAG – private - RUCDSTARTTYPE – private - SUPR – private - MEPR – private - RTSPP – public - LSL – private - RUCDCAMT – private - RUCDCAMTQSETOT – private - RUCDCBILLAMT – private - RUCDCAMTOT – public |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR123 | PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating RUCDCAMT, if there are no NCDCHR existing in the Settlement System for the Operating Day, the Settlement System shall process as if no RUC-Decommitments were issued and exit the process. |
| FR124 | PR 5.7.3 | Will be completed August 31, 2006 | While calculating RUCDCAMT, if NCDCHR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “NCDCHR for QSE <Q> and Resource <R> was not available for calculation of RUCDCAMT.” See FR123 for RUC Decommitment Amount driver error handling. |
| FR210 | PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating RUCDCAMT, if SUFLAG does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall calculate RUCDCAMT for the QSE/Resource with a zero for each hour of the Operating Day and provide an WARN error message indicating that SUFLAG for QSE <Q> and Resource <R> was not available for calculation. |
| FR211 | PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating RUCDCAMT, if RUCDSTARTTYPE does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall calculate RUCDCAMT for the QSE/Resource with a zero for each hour of the Operating Day and provide an WARN error message indicating that RUCDSTARTTYPE for QSE <Q> and Resource <R> was not available for calculation. |
| FR125 | PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating RUCDCAMT, if SUPR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination and Start Type, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that SUPR for QSE <Q>, Resource <R> and Start Type <ST> was not available for calculation. |
| FR126 | PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating RUCDCAMT, if MEPR does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that MEPR for QSE <Q> and Resource <R> was not available for calculation. |
| FR127 | PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating RUCDCAMT, if LSL does not exist in the Settlement System for the Operating Day for the QSE/Resource combination, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR128 | PR 5.7.3 | CSWM.P03SettleBillTransfer | While calculating RUCDCAMT, if RTSPP does not exist in the Settlement System for the Operating Day and Settlement Point, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RTSPP for Settlement Point <SP> was not available for calculation. |
| FR129 | PR 5.7.6 | Will be completed August 31, 2006 | While calculating RUCDCAMTTOT, if there is no RUCDCAMT in the Settlement System for the Operating Day, the Settlement System shall create a RUCDCAMTTOT data cut with zeros for all hours of the Operating Day. There will be no RUCDCAMT data cuts for any QSE/Resource combination for the Operating Day for this condition to occur. The calculation driver method eliminates the need for this requirement. See FR119. |

### Specific Database Needs

None

## RUC Capacity-Short Charge (RUCCSAMT)

For the RUC Capacity-Short Charge (RUCCSAMT):

ERCOT shall charge each QSE an amount, subject to a cap, if they are found to be short in capacity.  This calculation will be performed for each QSE for each RUC Process in which a RUCMWAMTTUCTOT exists for the Operating Day.

The dollar amount charged to each QSE, due to capacity shortfalls for a particular RUC Process, is the QSE’s shortfall ratio share (RUCSFRS) multiplied by the total Make-Whole Payments to all QSEs for that RUC Process (RUCMWAMTRUCTOT), subject to a cap. The cap is two (2) multiplied by the total RUC Make-Whole Payments for all QSEs (RUCMWAMTRUCTOT) multiplied by that QSE’s capacity shortfall for that RUC process (RUCSF), divided by the total capacity of all RUC-Committed Resources for the RUC process.

The RUC Capacity-Short Charge is calculated by QSE for each RUC Process for each 15-minute settlement interval.

For the RUC Shortfall (RUCSF) and RUC Shortfall Ratio Share (RUCSFRS):

ERCOT shall calculate for each QSE, a RUC Shortfall MW amount and a RUC Shortfall Ratio Share using a comparison of two separate sets of data, one showing each QSEs capacity when the RUC Process was executed compared to their Real-Time Adjusted Metered Load and one showing each QSEs capacity at the end of the adjustment period compared to their Real-Time Adjusted Metered Load.

The RUC Shortfall and RUC Shortfall Ratio Share will reflect a capacity credit for any capacity they are found short and charged for the same interval of the same Operating Day for previous RUC Processes (See RUCCAPCREDIT below).

The RUC Shortfall and RUC Shortfall Ratio Share are calculated by QSE for each RUC Process for each 15-minute settlement interval.

For the RUC Capacity Credit (RUCCAPCREDIT):

A QSE that is charged for a capacity shortfall in one RUC process gets a capacity credit equal to the minimum of the QSE’s RUC shortfall (MW) or the total RUC capacity purchased multiplied by the QSE’s shortfall ratio share.  The capacity credit is to be used in future RUC processes for the same 15-minute Settlement Interval. (PR 5.7.4.1.2)

ERCOT shall calculate a capacity credit for each QSE to account for any capacity shortfall that has been assigned to them in the same interval in the same Operating Day, in a previous RUC Process.  The RUCCAPCREDIT will be calculated after the RUC Shortfall, RUC Shortfall Ratio Share and RUC Capacity-Short Charge calculations have completed for each RUC Process of the Operating Day. The credit will be carried forward in the calculation of RUC Shortfall amounts (RUCSF) in subsequent RUC processes for the same Operating Day.  The QSE must have been previously charged for a shortfall for the capacity shortfall to be included in the RUC Capacity Credit.

The RUC Capacity Credit is calculated by QSE for each RUC Process for each 15-minute settlement interval.

| Requirement ID | FR130 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.1.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCCAPADJ and RUCSFADJ by QSE and RUC Process, for each 15-minute settlement interval of the Operating Day. RUCSFADJ_<Q>_<RUC> = Max (0, ((ΣspRTAML_<Q>_<SP>) *4) – RUCCAPADJ_<Q>_<RUC>)) Where, RUCCAPADJ_<Q>_<RUC> = ΣrHASLADJ_<Q>_<R>_<SP> + (RUCCPADJ_<Q> – RUCCSADJ_<Q>) + (ΣspDAEP_<Q>_<SP> – ΣspDAES_<Q>_<SP>) + (ΣspRTQQEPADJ_<Q>_<SP> – ΣspRTQQESADJ_<Q>_<SP>) RUCCAPADJ and RUCSFADJ must be calculated for each RUC Process due to HASLADJ adjustments for RUC-Decommitments given within 2 hours before the Operating Hour (See FR141) and Forced Outages occurring in the 2 hours before the Settlement Interval (See FR213). RTAML, RUCCPADJ, RUCCSADJ, DAEP, DAES, RTQQEPADJ and RTQQESADJ will be Adjustment Period values when calculating for each RUC Processes. |  |

| Requirement ID | FR131 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.1.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCCAPSNAP and RUCSFSNAP by QSE and RUC Process, for each 15-minute settlement interval of the Operating Day. RUCSFSNAP_<Q>_<RUC> = Max (0, ((ΣspRTAML_<Q>_<SP>) *4) – RUCCAPSNAP_<Q>_<RUC>)) Where, RUCCAPSNAP_<Q>_<RUC> = ΣrHASLSNAP_<Q>_<R>_<SP>_<RUC> + (RUCCPSNAP_<Q>_<RUC> – RUCCSSNAP_<Q>_<RUC>) + (ΣspDAEP_<Q>_<SP> – ΣspDAES_<Q>_<SP>) + (ΣspRTQQEPSNAP_<Q>_<SP>_<RUC> – ΣspRTQQESSNAP_<Q>_<SP>_<RUC>) RUCCAPSNAP and RUCSFSNAP must be calculated for each RUC Process, using snapshot data corresponding to the RUC Process. RTAML, DAEP and DAES will be Adjustment Period values, not snapshot values. |  |

| Requirement ID | FR132 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.1.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCSF by QSE and RUC Process, for each 15-minute settlement interval of the Operating Day. RUCSF_<Q>_<RUC> = Max (0, Max (RUCSFSNAP_<Q>_<RUC>, RUCSFADJ_<Q>) – Σz is prior to rucRUCCAPCREDIT_<Q>z) RUCSF must be calculated for each RUC Process. The RUCCAPCREDIT used for the QSE must be the sum of all “applicable” RUCCAPCREDITs, for that QSE, from all previous RUC Processes calculated for the Operating Day. “Applicable” means that the previous shortfall calculated for the QSE, and used in a previous calculation of RUCCAPCREDIT, must have resulted in a RUC Capacity-Short Charge for the QSE. |  |

| Requirement ID | FR133 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.1.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCSFRS by QSE and RUC Process, for each 15-minute settlement interval of the Operating Day. RUCSFRS_<Q>_<RUC> = RUCSF_<Q>_<RUC> / RUCSFTOT_<RUC> Where: RUCSFTOT_<RUC> = ΣqRUCSF_<Q>_<RUC> RUCSFTOT and RUCSFRS must be calculated for each RUC Process. |  |

| Requirement ID | FR134 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCCSAMT by QSE and RUC Process, for each 15-minute settlement interval of the Operating Day. RUCCSAMT_<Q>_<RUC> = (-1) * Max [(RUCSFRS_<Q>_<RUC> * RUCMWAMTRUCTOT_<RUC>), (2 * RUCSF_<Q>_<RUC> * RUCMWAMTRUCTOT_<RUC> / RUCCAPTOT_<RUC>)] / 4 Where: RUCCAPTOT_<RUC> = ΣqΣrRUCHSL_<Q>_>R>_<SP>_<RUC> RUCCAPTOT and RUCCSAMT must be calculated for each RUC Process. |  |

| Requirement ID | FR212 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 9.5.3 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCCSAMTQSETOT by QSE, by summing all RUCCSAMT charges for the QSE for each RUC Process, for each hour of the Operating Day. RUCCSAMTQSETOT_<Q> = Σruc (RUCCSAMT_<Q>_<RUC>) |  |

| Requirement ID | FR135 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCCSAMTTOT by RUC Process, for each 15-minute settlement interval of the Operating Day. RUCCSAMTTOT = ΣqΣrucRUCCBAMT_<Q>_<RUC> RUCCSAMTTOT shall be calculated one (1) time for the Operating Day. RUCCSAMTTOT is not calculated for each RUC Process, but is the sum across all RUC Processes for the Operating Day. |  |

| Requirement ID | FR136 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.1.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RUCCAPCREDIT by QSE and RUC Process, for each 15-minute settlement interval of the Operating Day. RUCCAPCREDIT_<Q>_<RUC> = Min [RUCSF_<Q>_<RUC>, (RUCCAPTOT_<RUC>* RUCSFRS_<Q>_<RUC>)] RUCCAPCREDIT must be calculated for each RUC Process. The value calculated will be used in the RUCSF calculation for all future RUC Processes for the Operating Day, if the shortfall for the current RUC Process results in a RUC Capacity-Short Charge for the QSE. |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RTAML | MWh | Real-Time Adjusted Metered Load—The QSE q’s Adjusted Metered Load at the Settlement Point p for the 15-minute Settlement Interval. | ERCOT Settlement System through the Data Aggregation Process |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| TOFO | Misc | Time of Forced Outage — The time in which a forced outage occurs, for a Resource. | MMS |
| Interval Frequency: Not Applicable (Timestamp) Additional Comments: This determinant is not documented in the Nodal Protocols but is required to determine when a forced outage has occurred so that the QSE’s HASLADJ can be adjusted to the value included in the HASLSNAP, per PR 5.7.4.1.1 (9) definition for HASLADJ. |  |  |  |
| RUCHSL | MW | High Sustained Limit— A High Sustainable Limit of a Generation Resource as defined in Section 2, Definitions, for the hour that includes the Settlement Interval i, at the time the RUC Process committed the Resource. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| DAEP | MW | Day-Ahead Energy Purchase—The QSE q’s energy purchased in the DAM at the Settlement Point p for the hour that includes the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| DAES | MW | Day-Ahead Energy Sale—The QSE q’s energy sold in the DAM at the Settlement Point p for the hour that includes the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| HASLSNAP | MW | High Ancillary Services Limit at Snapshot—The High Ancillary Services Limit of the Resource r represented by the QSE q, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RUCCPSNAP | MW | RUC Capacity Purchase at Snapshot—The QSE q’s Capacity purchase, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RUCCSSNAP | MW | RUC Capacity Sale at Snapshot—The QSE q’s capacity sale, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RTQQEPSNAP | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval, in the COP and Trades Snapshot. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTQQESSNAP | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval, in the COP and Trades Snapshot. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| HASLADJ | MW | High Ancillary Services Limit at Adjustment Period—The High Ancillary Services Limit of the Resource r represented by the QSE q, according to the adjustment period snapshot, for the hour that includes the 15-minute Settlement Interval. If the HASL for a Resource was credited to the QSE during the RUC Snapshot but the Resource experiences a Forced Outage within two hours before the start of the Settlement Interval, then the HASL for that Resource is also credited to the QSE in the HASLADJ. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RUCCPADJ | MW | RUC Capacity Purchase at Adjustment Period—The QSE q’s capacity purchase, according to the Adjustment Period COP and Trades Snapshot for the hour that includes the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RUCCSADJ | MW | RUC Capacity Sale at Adjustment Period—The QSE q’s capacity sale, according to the Adjustment Period COP and Trades Snapshot for the hour that includes the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RTQQEPADJ | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTQQESADJ | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RUCCAPCREDIT | MW | RUC Capacity Credit by QSE—The capacity credit resulting from capacity paid through the RUC Capacity-Short Charge for the 15-minute Settlement Interval. | Settlement System – calculated for previous RUC Processes for the Operating Day. |
| Interval Frequency: 4 / hour Additional Comments: The RUCCAPCREDIT that is calculated using data from a particular RUC Process will not be used in the RUCSF calculation for that particular RUC Process. This RUCCAPCREDIT will be used in the RUCSF calculation of future RUC Processes for the Operating Day. |  |  |  |
| RUCMWAMTRUCTOT | $ | RUC Make-Whole Amount Total per RUC Process—The sum of RUC Make-Whole Payments for a particular RUC process, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants – Level 1 (See Data Dependencies)

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCCAPSNAP | MW | RUC Capacity Snapshot at time of RUC—The amount of the QSE’s calculated capacity in the COP and Trades Snapshot for a 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RUCCAPADJ | MW | RUC Capacity Snapshot during Adjustment Period—The amount of the QSE’s calculated capacity in the RUC according to the COP and Trades Snapshot at the end of the Adjustment Period for a 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RUCCAPTOT | MW | RUC Capacity Total—The sum of the HSLs of all RUC-committed Resources for a particular RUC process, for the hour that includes the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants – Level 2 (See Data Dependencies)

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCSFSNAP | MW | RUC Shortfall at Snapshot—The QSE q’s capacity shortfall according to the snapshot for the RUC process for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RUCSFADJ | MW | RUC Shortfall at Adjustment Period—The QSE q’s adjustment period capacity shortfall for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants – Level 3 (See Data Dependencies)

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCSF | MW | RUC Shortfall—The QSE’s capacity shortfall for a particular RUC process for the 15-minute Settlement Interval. See formula in Section 5.7.4.1.1, Capacity Shortfall Ratio Share. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants – Level 4 (See Data Dependencies)

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCSFTOT | MW | RUC Shortfall Total—The sum of all QSEs’ capacity shortfalls, for a RUC process, for a 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants – Level 5 (See Data Dependencies)

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RUCSFRS | Misc | RUC Shortfall Ratio Share—The ratio of the QSE’s capacity shortfall to the sum of all QSEs’ capacity shortfalls for a particular RUC process, for the 15-minute Settlement Interval. See Section 5.7.4.1.1, Capacity Shortfall Ratio Share. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |  |
| --- | --- | --- | --- | --- |
| RUCCAPCREDIT | MW | RUC Capacity Credit by QSE—The capacity credit resulting from capacity paid through the RUC Capacity-Short Charge for the 15-minute Settlement Interval. | Settlement System |  |
| Interval Frequency: 4 / hour Additional Comments: The RUCCAPCREDIT that is calculated using data from a particular RUC Process will not be used in the RUCSF calculation for that particular RUC Process. This RUCCAPCREDIT will be used in the RUCSF calculation of future RUC Processes for the Operating Day. |  |  |  |  |
| RUCCSAMT | $ | RUC Capacity-Short Amount—The charge to a QSE, due to capacity shortfall for a particular RUC process, for the 15-minute Settlement Interval. | Settlement System |  |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |
| RUCCSAMTQSETOT | $ | RUC Capacity-Short Charge Amount QSE Total  The QSE total for RUC Capacity-Short Charges, across all RUC Processes, for the 15-minute settlement interval. |  | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |  |
| RUCCSBILLAMT | $ | RUC Capacity-Short Charge Bill Amount  The difference between two consecutive daily summed settlement runs of RUCCSAMTQSETOT for the QSE, for an Operating Day. | Settlement System |  |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |  |
| RUCCSAMTTOT | $ | RUC Capacity-Short Amount Total—The sum of RUC Capacity-Short Charges for all QSEs and RUC processes for the 15-minute Settlement Interval. | Settlement System |  |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |

### Data Dependencies

RUC Capacity-Short Calculations

Level 1 Calculation Dependencies:

D1)	Calculation of the RUC Capacity at Snapshot (RUCCAPSNAP) is dependent on:

- availability of the input bill determinants (successful import from MMS)

D2)	Calculation of the RUC Capacity at Adjustment Period (RUCCAPADJ) is dependent on:

- availability of the input bill determinants (successful import from MMS)

D3)	Calculation of the RUC Capacity Total (RUCCAPTOT) is dependent on:

- availability of the input bill determinants (successful import from MMS)

D4)	The RUC Capacity at Snapshot (RUCCAPSNAP) calculation must be complete prior to the calculation of the RUC Shortfall at Snapshot (RUCSFSNAP).

D5)	The RUC Capacity at Adjustment Period (RUCCAPADJ) calculation must be complete prior to the calculation of the RUC Shortfall at Adjustment Period (RUCSFADJ).

D6)	The RUC Capacity Total (RUCCAPTOT) calculation must be complete prior to the calculation:

- RUC Capacity-Short Charge (RUCCSAMT)

- RUC Capacity Credit (RUCCAPCREDIT)

Level 1 Calculations

Level 2 Calculation Dependencies:

D7)	Calculation of the RUC Shortfall at Snapshot (RUCSFSNAP) is dependent on:

- successful calculation on RTAML

- successful calculation of RUC Capacity at Snapshot (RUCCAPSNAP)

D8)	Calculation of the RUC Shortfall at Adjustment Period (RUCSFADJ) is dependent on:

- successful calculation on RTAML

- successful calculation of RUC Capacity at Adjustment Period (RUCCAPADJ)

D9)	The RUC Shortfall at Snapshot (RUCSFSNAP) calculation must be complete prior to the calculation of the RUC Shortfall (RUCSF).

D10)	The RUC Shortfall at Adjustment Period (RUCSFADJ) calculation must be complete prior to the calculation of the RUC Shortfall (RUCSF).

Level 2 Calculations

Level 3 Calculation Dependencies:

D11)	Calculation of the RUC Shortfall (RUCSF) is dependent on:

- successful calculation of RUC Capacity Credit (RUCCAPCREDIT) from previous RUC Processes

- successful calculation of RUC Shortfall at Snapshot (RUCSFSNAP)

- successful calculation of RUC Shortfall at Adjustment Period (RUCSFADJ)

D12)	The RUC Shortfall (RUCSF) calculation must be complete prior to the calculation:

- RUC Capacity Credit for the Current RUC Process (RUCCAPCREDIT)

- RUC Shortfall Total (RUCSFTOT)

- RUC Shortfall Ratio Share (RUCSFRS)

- RUC Capacity-Short Charge (RUCCSAMT)

Level 3 Calculations

Level 4 Calculation Dependencies:

D13)	Calculation of the RUC Shortfall Total RUCSFTOT is dependent on:

- successful calculation of RUC Shortfall (RUCSF)

D14)	The RUC Shortfall Total (RUCSFTOT) calculation must be complete prior to the calculation of the RUC Shortfall Ratio Share (RUCSFRS).

Level 4 Calculations

Level 5 Calculation Dependencies:

D15)	Calculation of the RUC Shortfall Ratio Share (RUCSFRS) is dependent on:

- successful calculation of RUC Shortfall (RUCSF)

- successful calculation of RUC Shortfall Total (RUCSFTOT)

D16)	The RUC Shortfall Ratio Share (RUCSFRS) calculation must be complete prior to the calculation:

- RUC Capacity Credit for the Current RUC Process (RUCCAPCREDIT)

- RUC Capacity-Short Charge (RUCCSAMT)

Level 5 Calculations

Level 6 and 7 Calculation Dependencies:

D17)	Calculation of the RUC Capacity-Short Charge (RUCCSAMT) is dependent on:

- successful calculation of Shortfall Ratio Share (RUCSFRS)

- successful calculation of RUC Shortfall (RUCSF)

- successful calculation of RUC Capacity Total (RUCCAPTOT)

- successful calculation of RUC Make-Whole Payment RUC Total (RUCMWAMTRUCTOT)

D18)	Calculation of the RUC Capacity-Short Charge Total (RUCCSAMTTOT) is dependent on:

- successful calculation of  RUC Capacity-Short Charge (RUCSAMT)

D19)	Calculation of the RUC Capacity Credit (RUCCAPCREDIT) is dependent on:

- successful calculation of  RUC Shortfall (RUCSF)

- successful calculation of Shortfall Ratio Share (RUCSFRS)

- successful calculation of RUC Capacity Total (RUCCAPTOT)

D20)	The RUC Capacity-Short Charge (RUCCSAMT) calculation must be complete prior to the calculation of RUC Capacity-Short Charge Total (RUCCSAMTTOT).

D21)	The RUC Capacity-Short Charge Total (RUCCSAMTTOT) calculation must be complete prior to the calculation of the RUC Make-Whole Uplift Charge (LARUCAMT).

D22)	The RUCCAPCREDIT calculation for the current RUC Process must be complete prior to the calculation of the RUC Shortfall (RUCSF) for the next RUC Process.

Level 6 and 7 Calculations

### Assumptions

A1)	The interface from MMS to the Settlement System will adjust the HASLSNAP and HASLADJ for each WGR to the value represented in the COP and Trades Snapshot prior to the RUC Process, which shall be equal to the WGRs WGRPP. (PR 5.7.4.1.1(1))

PR 4.2.2(7) satisfies:

- PR 5.7.4.1.1(1) which states:  In calculating the amount short for each QSE, the QSE must be given a capacity credit for its WGRs based on the WGRPP of the Resource by setting the HASLSNAP and HASLADJ variables used below equal to the WGRPP, and

- PR 5.7.4 (2) which states:  To determine whether a QSE is capacity-short, the WGR Production Potential, as described in Section 4.2.2, Wind-Powered Generation Resource Production Potential, for a WGR used in the corresponding RUC is considered the available capacity of the WGR when determining responsibility for the corresponding RUC charges, regardless of the Real-Time output of the WGR.NPRR050

A2)	The interface from MMS to the Settlement System will adjust the HASLSNAP and HASLADJ for non-wind Resources that were given notice of decommitment within the two hours before the Operating Hour as a result of the RUC process by setting the HASLSNAP and HASLADJ variable equal to the HASLSNAP value for the Resource immediately before the decommitment instruction was given. (PR 5.7.4.1.1(2))FR141

### Business Rules

| Req. ID |  | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- | --- |
| FR137 |  | PR 5.7.4.1 PR 5.7.4.1.1 PR 5.7.4.1.2 | CSWM.P03SettleBillTransfer | DRIVER: For each RUC Process, all calculations included in the RUC Capacity-Short Charge (RUCCSAMT), except RUCCSAMTTOT and RUCCAPCREDIT, shall only be attempted when RUCMWAMTRUCTOT_<RUC> exists in the settlement system for the Operating Day for that RUC Process. |
| FR138 |  | PR 5.7.4.2 | CSWM.P03SettleBillTransfer | DRIVER: The RUCCSAMTTOT calculation shall only be attempted for an Operating Day when the sum of all RUC Capacity-Short Charges is not equal to zero. |
| FR139 |  | PR 5.7.4.2 | CSWM.P03SettleBillTransfer | DRIVER: The calculation of RUCCAPCREDIT shall be calculated for a QSE if both, RUCMWAMTRUCTOT for the RUC Process or RUCCSAMT not equal to zero for the QSE and RUC Process, exist in the Settlement System for the Operating Day. |
| FR140 |  | PR 5.7.4(2) PR 5.7.4.1.1(1) | Will be completed August 31, 2006 | The HASLSNAP and HASLADJ for each Wind-Powered Generation Resource (WGR) shall be equal to the WGRPP. Assumption A1 eliminates the need for this requirement in the Settlement System. |
| FR141 |  | PR 5.7.4.1.1(2) | CSWM.P03SettleBillTransfer | The HASLADJ for each non-wind Resource that is given notice of decommitment within the two hours before the Operating Hour, as a result of the RUC process, must be equal to the HASLSNAP value for the Resource immediately before the decommitment instruction was given. |
| FR213 |  | PR 5.7.4.1.1(9) | CSWM.P03SettleBillTransfer | If the HASL for a Resource was credited to the QSE in the HASLSNAP, but the Resource experiences a Forced Outage within two hours before the start of the Settlement Interval, the HASL for that Resource is also credited to the QSE in the HASLADJ. |
| FR142 |  | PR 5.7.4.1 | CSWM.P03SettleBillTransfer | The RUCCSAMT factor of <2> shall be stored as a variable with start and stop time. |
| FR214 | PR 9.5.5 (2) PR 9.5.6 (6) PR9.5.8 (3) |  | CSWM.P03SettleBillTransfer | The RUCCSBILLAMT value shall be calculated as the difference between the total daily sum of RUCCSAMTQSETOT for current Settlement Run and the total daily sum of RUCCSAMTQSETOT for the most previous Settlement Run. |
| FR143 |  | PR 5.7.4.1 PR 5.7.4.1.1 PR 5.7.4.1.2 | CSWM.P03SettleBillTransfer | No input bill determinants shall be rounded or truncated. |
| FR144 |  | PR 9.5.3 | CSWM.P03SettleBillTransfer | No intermediate bill determinants shall be rounded or truncated. |
| FR145 |  | PR 9.5.3 | CSWM.P03SettleBillTransfer | Output bill determinants that shall be rounded to two (2) decimal places are: - RUCCSAMT - RUCCSAMTQSETOT - RUCCSBILLAMT - RUCCSAMTTOT Output bill determinants that shall not be rounded or truncated are: - RUCCAPCREDIT |
| FR146 |  | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RTAML – private - TOFO – private - RUCD – private - RUCHSL – private - DAEP – private - DAES – private - HASLSNAP – private - RUCCPSNAP – private - RUCCSSNAP – private - RTQQEPSNAP – private - RTQQESSNAP – private - HASLADJ – private - RUCCPADJ – private - RUCCSADJ – private - RTQQEPADJ – private - RTQQESADJ – private - RUCCAPCREDIT – private - RUCMWAMTRUCTOT – public - RUCCAPSNAP – private - RUCCAPADJ – private - RUCCAPTOT – public - RUCSFSNAP – private - RUCSFADJ – private - RUCSF – private - RUCSFTOT – public - RUCSFRS – private - RUCCSAMT – private - RUCCSAMTQSETOT – private - RUCCSBILLAMT – private - RUCCSAMTTOT – public |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR147 | PR 5.7.4.1 PR 5.7.4.1.1 PR 5.7.4.1.2 | CSWM.P03SettleBillTransfer | While calculating RUCCAPSNAP, RUCCAPADJ, RUCSFSNAP, RUCSFADJ, RUCSF, RUCSFTOT, RUCSFRS, RUCCAPTOT, RUCCSAMT and RUCCAPCREDIT, if RUCMWAMTRUCTOT does not exist in the Settlement System for the Operating Day and for the RUC Process, the Settlement System shall process as if no RUC Make-Whole Payments, for the RUC Process, were calculated and exit the calculation for the RUC Process. |
| FR148 | PR 5.7.4.1.2 | CSWM.P03SettleBillTransfer | While calculating RUCCAPCREDIT, if RUCCSAMT (for the RUC Process and QSE) contains zeros for each interval of the Operating Day or does not exist in the Settlement System for the Operating Day, the Settlement System should exit the calculation for the QSE. |
| FR149 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPADJ, if no HASLADJ exists in the Settlement System for the Operating Day for the QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR150 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPADJ, if no RUCCPADJ exists in the Settlement System for the Operating Day for the QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR151 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPADJ, if no RUCCSADJ exists in the Settlement System for the Operating Day for the QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR152 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPADJ, if no DAEP exists in the Settlement System for the Operating Day for the QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR153 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPADJ, if no DAES exists in the Settlement System for the Operating Day for the QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR154 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPADJ, if no RTQQEPADJ exists in the Settlement System for the Operating Day for the QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR155 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPADJ, if no RTQQESADJ exists in the Settlement System for the Operating Day for the QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR156 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPSNAP, if no HASLSNAP exists in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR157 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPSNAP, if no RUCCPSNAP exists in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR158 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPSNAP, if no RUCCSSNAP exists in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR159 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPSNAP, if no DAEP exists in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR160 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPSNAP, if no DAES exists in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR161 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPSNAP, if no RTQQEPSNAP exists in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR162 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPSNAP, if no RTQQESSNAP exists in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR163 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCSFADJ, if RTAML does not exist in the Settlement System for the Operating Day for the QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR164 | PR 5.7.4.1.1 | Will be completed August 31, 2006 | While calculating RUCSFADJ, if RUCCAPADJ does not exist in the Settlement System for the Operating Day for the QSE, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “While calculating RUCSFADJ for RUC Process <RUC>, RUCCAPADJ for QSE <Q> was not available for calculation.” Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR165 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCSFSNAP, if RTAML does not exist in the Settlement System for the Operating Day for the QSE, the Settlement System shall default to zero for all intervals of the Operating Day. |
| FR166 | PR 5.7.4.1.1 | Will be completed August 31, 2006 | While calculating RUCSFSNAP, if RUCCAPSNAP does not exist in the Settlement System for the Operating Day for the QSE, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “While calculating RUCSFSNAP for RUC Process <RUC>, RUCCAPSNAP for QSE <Q> was not available for calculation.” Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR167 | PR 5.7.4.1.1 | Will be completed August 31, 2006 | While calculating RUCSF, if RUCSFSNAP does not exist in the Settlement System for the Operating Day, for the RUC Process and for the QSE, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “While calculating RUCSF for RUC Process <RUC>, RUCSFSNAP for QSE <Q> was not available for calculation.” Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR168 | PR 5.7.4.1.1 | Will be completed August 31, 2006 | While calculating RUCSF, if RUCSFADJ does not exist in the Settlement System for the Operating Day for the QSE, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “While calculating RUCSF for RUC Process <RUC>, RUCSFADJ for QSE <Q> was not available for calculation.” Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR169 | PR 5.7.4.1.1 | CSWM.P03SettleBillTransfer | While calculating RUCSF, if there are no RUCCAPCREDIT in the Settlement System for the Operating Day and for the QSE, for all previously calculated RUC Processes, the Settlement System shall default to zero for all intervals of the Operating Day. There will be no RUCCAPCREDIT data cuts for the QSE, for any previously calculated RUC Process for the Operating Day, for this condition to occur. |
| FR170 | PR 5.7.4.1.1 | Will be completed August 31, 2006 | While calculating RUCSFTOT, if there are no RUCSF in the Settlement System for the Operating Day for the RUC Process, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “While calculating RUCSFTOT for RUC Process <RUC>, no RUCSF were available for calculation.” There should be no RUCSF data cuts for the Operating Day for the RUC Process for this condition to occur. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR171 | PR 5.7.4.1.1 | Will be completed August 31, 2006 | While calculating RUCSFRS, if RUCSF does not exist in the Settlement System for the Operating Day, for the RUC Process and for the QSE, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “While calculating RUCSFRS for RUC Process <RUC>, RUCSF for QSE <?> was not available for calculation.” Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR172 | PR 5.7.4.1.1 | Will be completed August 31, 2006 | While calculating RUCSFRS, if RUCSFTOT does not exist in the Settlement System for the Operating Day, then the Settlement System should default to zero for all intervals of the Operating Day and should provide a WARN-DEFAULT error message indicating the failure as “While calculating RUCSFRS for RUC Process <RUC>, RUCSFTOT was not available for calculation.” Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR173 | PR 5.7.4.1 | CSWM.P03SettleBillTransfer | While calculating RUCCAPTOT, if no RUCHSL exist in the Settlement System for the Operating Day for the RUC Process, the Settlement System shall use a default value of zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that no RUCHSL, for RUC Process <RUC>, was available for calculation. |
| FR174 | PR 5.7.4.1 | Will be completed August 31, 2006 | While calculating RUCCSAMT, if RUCSFRS does not exist in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCSFRS was unavailable for QSE <Q> and RUC Process <RUC>. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR175 | PR 5.7.4.1 | Will be completed August 31, 2006 | While calculating RUCCSAMT, if RUCSF does not exist in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCSF was unavailable for QSE <Q> and RUC Process <RUC>. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR176 | PR 5.7.4.1 | Will be completed August 31, 2006 | While calculating RUCCSAMT, if RUCCAPTOT does not exist in the Settlement System for the Operating Day for the RUC Process, the Settlement System should default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCCAPTOT was unavailable for RUC Process <RUC>. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR177 | PR 5.7.4.2 | Will be completed August 31, 2006 | While calculating RUCCSAMTTOT, if there are no RUCCSAMT in the Settlement System for the Operating Day, the Settlement System shall create a RUCCSAMTTOT data cut with zeros for all intervals of the Operating Day. There will be no RUCCSAMT data cuts, for any QSE for the Operating Day, for this condition to occur. The calculation driver method eliminates the need for this requirement. See FR138. |
| FR178 | PR 5.7.4.1.2 | Will be completed August 31, 2006 | While calculating RUCCAPCREDIT, if RUCSF does not exist in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCSF for QSE <Q> was unavailable for calculation. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR179 | PR 5.7.4.1.2 | Will be completed August 31, 2006 | While calculating RUCCAPCREDIT, if RUCSFRS does not exist in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCSFRS for QSE <Q> was unavailable for calculation. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |
| FR180 | PR 5.7.4.1.2 | Will be completed August 31, 2006 | While calculating RUCCAPCREDIT, if RUCCAPTOT does not exist in the Settlement System for the Operating Day, RUC Process and QSE, the Settlement System shall default to zero for all intervals of the Operating Day and provide a WARN-DEFAULT error message indicating that RUCCAPTOT for QSE <Q> was unavailable for calculation. Based on planned functionality and system configuration behind functionality, this requirement is superfluous and functionally un-testable. See RUC detailed design functional decomposition for more information. |

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR181 | PR 5.7.4.1 PR 5.7.4.1.1 PR 5.7.4.1.2 | CSWM.P03SettleBillTransfer | The settlement system shall visibly identify and track individual RUC Processes within or for various bill determinants used in the calculations of RUCCSAMT. |

## RUC Make-Whole Uplift Charge (LARUCAMT)

If the revenues from the RUC Capacity-Short Charge are not enough to cover all RUC Make-Whole Payments for a 15-minute Settlement Interval, then the difference will be uplifted to all QSEs on a Load Ratio Share basis, as a RUC Make-Whole Uplift Charge.

| Requirement ID | FR182 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.4.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the LARUCAMT by QSE for each 15-minute settlement interval of the Operating Day. LARUCAMT_<Q> = (-1) * (RUCMWAMTTOT / 4 + RUCCSAMTTOT) * LRS_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| LRS | Misc | Load Ratio Share—The ratio of Adjusted Metered Load to the total ERCOT Adjusted Metered Load for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. | Settlement System through the Data Aggregation process |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.RTENERGY.REQUIREMENTS.A for further information on the LRS bill determinant. |  |  |  |
| RUCMWAMTTOT | $ | RUC Make-Whole Amount Total—The sum of RUC Make-Whole Payments for all QSEs and RUC processes, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| RUCCSAMTTOT | $ | RUC Capacity-Short Amount Total—The sum of RUC Capacity-Short Charges for all QSEs and RUC processes for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| LARUCAMT | $ | RUC Make-Whole Uplift Charge—The amount owed from the QSE based on Load Ratio Share, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| LARUCBILLAMT | $ | RUC Make-Whole Uplift Charge Bill Amount  The difference between two consecutive daily summed settlement runs of LARUCAMT for the QSE, for an Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements |  |  |  |

### Data Dependencies

D1)	Calculation of the RUC Make-Whole Uplift Charge (LARUCAMT) is dependent on:

- successful calculation of 15-minute Load Ratio Share (LRS)

- successful calculation of RUC Make-Whole Amount Total (RUCMWAMTTOT)

- successful calculation of RUC Capacity-Short Amount Total (RUCCSAMTTOT)

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR183 | PR 5.7.4.2 | CSWM.P03SettleBillTransfer | DRIVER: The LARUCAMT calculation should only be attempted for the Operating Day when there is a RUCMWAMTTOT for the Operating Day. |
| FR218 | PR 5.7.4.2 | CSWM.P03SettleBillTransfer | DRIVER: The LARUCAMT calculation should only be attempted for the QSE when an LRS exists for the QSE for the Operating Day. |
| FR215 | PR 9.5.5 (2) PR 9.5.6 (6) PR9.5.8 (3) | CSWM.P03SettleBillTransfer | The LARUCBILLAMT value shall be calculated as the difference between the total daily sum of LARUCAMT for current Settlement Run and the total daily sum of LARUCAMT for the most previous Settlement Run. |
| FR184 | PR 5.7.4.2 | CSWM.P03SettleBillTransfer | No input bill determinants shall be rounded or truncated. |
| FR185 | PR 9.5.3 | CSWM.P03SettleBillTransfer | All output bill determinants shall be rounded to two (2) decimal places. |
| FR186 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCMWAMTTOT – public - LRS – private - RUCCSAMTTOT – public - LARUCAMT – private - LARUCBILLAMT – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR187 | PR 5.7.4.2 | CSWM.P03SettleBillTransfer | While calculating LARUCAMT, if RUCMWAMTTOT does not exist in the Settlement System for the Operating Day, the Settlement System shall process as if no RUC Make-Whole payments were calculated for the Operating Day and exit the calculation. |
| FR188 | PR 5.7.4.2 | CSWM.P03SettleBillTransfer | While calculating LARUCAMT, if LRS does not exist in the Settlement System for the Operating Day for the QSE, the Settlement System shall process as if the QSE does not have Load and exit the calculation for the QSE. |
| FR189 | PR 5.7.4.2 | CSWM.P03SettleBillTransfer | While calculating LARUCAMT, if RUCCSAMTTOT does not exist in the Settlement System for the Operating Day, the Settlement System shall default to zero for all intervals of the Operating Day. |

### Specific Database Needs

None

## RUC Clawback Payment (LARUCCBAMT)

ERCOT shall pay the revenues from all RUC Clawback Charges in a 15-minute Settlement Interval to all QSEs, on a Load Ratio Share basis, as the RUC Clawback Payment. (PR 5.7.5)

| Requirement ID | FR190 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.5 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the LARUCCBAMT by QSE for each 15-minute settlement interval of the Operating Day. LARUCCBAMT_<Q> = (-1) * [(RUCCBAMTTOT / 4) * LRS_<Q>] |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| LRS | Misc | Load Ratio Share—The ratio of Adjusted Metered Load to the total ERCOT Adjusted Metered Load for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. | Settlement System through the Data Aggregation process |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.RTENERGY.REQUIREMENTS.A for further information on the LRS bill determinant. |  |  |  |
| RUCCBAMTTOT | $ | RUC Clawback Amount Total—The sum of RUC Clawback Charges for all QSEs, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| LARUCCBAMT | $ | RUC Clawback Payment—The RUC Make-Whole Clawback Payment to a QSE to uplift RUC Make-Whole Clawback Charges received, for a 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| LARUCCBBILLAMT | $ | RUC Clawback Payment Bill Amount  The difference between two consecutive daily summed settlement runs of LARUCCBAMT for the QSE, for an Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements |  |  |  |

### Data Dependencies

D1)	Calculation of the RUC Clawback Payment (LARUCCBAMT) is dependent on:

- successful calculation of 15-minute Load Ratio Share (LRS)

- successful calculation of RUC Clawback Amount Total (RUCCBAMTTOT)

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR191 | PR 5.7.5 | CSWM.P03SettleBillTransfer | DRIVER: The LARUCCBAMT calculation should only be attempted for the Operating Day when there is a RUCCBAMTTOT for the Operating Day. |
| FR217 | PR 9.5.5 (2) PR 9.5.6 (6) PR9.5.8 (3) | CSWM.P03SettleBillTransfer | The LARUCCBBILLAMT value shall be calculated as the difference between the total daily sum of LARUCCBAMT for current Settlement Run and the total daily sum of LARUCCBAMT for the most previous Settlement Run. |
| FR192 | PR 5.7.5 | CSWM.P03SettleBillTransfer | No input bill determinants shall not be rounded or truncated. |
| FR193 | PR 9.5.3 | CSWM.P03SettleBillTransfer | All output bill determinants shall be rounded to two (2) decimal places. |
| FR194 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCCBAMTTOT – public - LRS – private - LARUCCBAMT – private - LARUCCBBILLAMT – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR195 | PR 5.7.5 | CSWM.P03SettleBillTransfer | While calculating LARUCCBAMT, if RUCCBAMTTOT does not exist in the Settlement System for the Operating Day, the Settlement System shall process as if no RUC Clawback charges were calculated for the Operating Day and exit the calculation. |
| FR196 | PR 5.7.5 | CSWM.P03SettleBillTransfer | While calculating LARUCCBAMT, if LRS does not exist in the Settlement System for the Operating Day for the QSE, the Settlement System shall process as if the QSE does not have Load and exit the calculation for the QSE. |

### Specific Database Needs

None

## RUC Decommitment Charge (LARUCDCAMT)

ERCOT shall charge each QSE a RUC Decommitment Charge, on a Load Ratio Share basis, all revenues paid as a result of RUC Decommitment Payments. (PR 5.7.6)

| Requirement ID | FR197 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 5.7.6 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the LARUCDCAMT by QSE for each 15-minute settlement interval of the Operating Day. LARUCDCAMT_<Q> = (-1) * [(RUCDCAMTTOT / 4) * LRS_<Q>] |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| LRS | Misc | Load Ratio Share—The ratio of Adjusted Metered Load to the total ERCOT Adjusted Metered Load for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. | Settlement System through the Data Aggregation process |
| Interval Frequency: 4 / hour Additional Comments: See TN.COMS.63C01.RTENERGY.REQUIREMENTS.A for further information on the LRS bill determinant. |  |  |  |
| RUCDCAMTTOT | $ | RUC Decommitment Payment Total—The sum of RUC Decommitment Payments to all QSEs for the hour that includes the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| LARUCDCAMT | $ | RUC Decommitment Charge—The RUC Decommitment Charge to a QSE, for a 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| LARUCDCBILLAMT | $ | RUC Clawback Payment Bill Amount  The difference between two consecutive daily summed settlement runs of LARUCCBAMT for the QSE, for an Operating Day. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements |  |  |  |

### Data Dependencies

D1)	Calculation of the RUC Decommitment Charge (LARUCDCAMT) is dependent on:

- successful calculation of 15-minute Load Ratio Share (LRS)

- successful calculation of RUC Decommitment Payment Total (RUCDCAMTTOT)

### Assumptions

None

### Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR198 | PR 5.7.6 | CSWM.P03SettleBillTransfer | DRIVER: The LARUCDCAMT calculation shall only be attempted for the Operating Day when there is a RUCDCAMTTOT for the Operating Day. |
| FR219 | PR 9.5.5 (2) PR 9.5.6 (6) PR9.5.8 (3) | CSWM.P03SettleBillTransfer | The LARUCDCBILLAMT value shall be calculated as the difference between the total daily sum of LARUCDCAMT for current Settlement Run and the total daily sum of LARUCDCAMT for the most previous Settlement Run. |
| FR199 | PR 5.7.6 | CSWM.P03SettleBillTransfer | No input bill determinants shall not be rounded or truncated. |
| FR200 | PR 9.5.3 | CSWM.P03SettleBillTransfer | All output bill determinants shall be rounded to two (2) decimal places. |
| FR201 | PR 12.3 (c) | CSWM.P03SettleBillTransfer | All input and output bill determinants shall be made available to the extracts process and be classified in the settlement system as: - RUCDCAMTTOT – public - LRS – private - LARUCDCAMT – private - LARUCDCBILLAMT – private |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR202 | PR 5.7.6 | CSWM.P03SettleBillTransfer | While calculating LARUCDCAMT, if RUCDCAMTTOT does not exist in the Settlement System for the Operating Day, the Settlement System shall process as if no RUC-Decommitment payments were calculated for the Operating Day and exit the calculation. |
| FR203 | PR 5.7.6 | CSWM.P03SettleBillTransfer | While calculating LARUCDCAMT, if LRS does not exist in the Settlement System for the Operating Day for the QSE, the Settlement System shall process as if the QSE does not have Load and exit the calculation for the QSE. |

### Specific Database Needs

None

# Supplementary Requirements

## Performance Requirements

Lodestar must be able to perform its full set of Nodal settlement calculations. Performance characteristics will be determined and detailed in the conceptual and detailed design.

## Legal and Regulatory requirements

All Lodestar financial data must be saved in accordance with standard accounting principles as dictated by the ERCOT Texas Nodal Protocols.

## System Security Requirements

Lodestar will only be accessed directly by internal ERCOT personnel either through thin client UI, Citrix environment, or thick client on employee desktops. Market Participants will be able to access their statements and invoices on ERCOT private web site. ERCOT private web site access will be controlled by digital certificate authentication in accordance with ERCOT security policies.  Additional details will be provided in the Conceptual System Design.

## Back up and Recovery Requirements

All Lodestar Nodal settlement data must be backed up on a daily basis and recoverable within 24 hours.  Additional details will be provided in the Conceptual System Design.

## Availability and Redundancy Requirements

Nodal Lodestar system must be available 7/24. Lodestar is considered a tier 1 system and critical to base ERCOT business processes.  Additional details will be provided in the Conceptual System Design.

## Maintainability Requirements

Lodestar Nodal system will be maintained by internal ERCOT personnel. Service Level Agreements and explicit support will be detailed in the detail design document.

## Daylight Savings Time shall be recognized in Nodal Market settlements.

Spring Daylight Savings Time Operating Day:

- Hour Ending 0300 will be skipped, resulting in 23 hourly intervals for the day.

- The four fifteen minute intervals within Hour Ending 0300 will be skipped, resulting in 92 fifteen minute intervals for the day.

- The sixty one minute intervals within Hour Ending 0300 will be skipped, resulting in 1380 total one minute intervals.

Fall Daylight Savings Time Operating Day:

- Hour Ending 0200 will have two (2) sets of data, resulting in 25 hourly intervals for the day.

- The four fifteen minute intervals within Hour Ending 0200 will have two (2) sets of data, resulting in 100 fifteen minute intervals for the day.

- The sixty one minute intervals within Hour Ending 0200 will have two (2) sets of data, resulting in 1500 total one minute intervals.

# Protocol Coverage

| Protocol Sub-Section # (To the lowest level of granularity as possible) | Coverage by Requirements (Full/Partial) |
| --- | --- |
| 4.4.9.2.3 (1) | Partial |
| 4.4.9.2.3 (2) | Partial |
| 4.4.9.2.3 (3) | Partial |
| 5.7.1 | Partial |
| 5.7.1.1 | Partial |
| 5.7.1.2 | Partial |
| 5.7.1.3 | Partial |
| 5.7.1.4 | Partial |
| 5.7.2 | Partial |
| 5.7.3 | Partial |
| 5.7.4.1 | Partial |
| 5.7.4.1.1 | Partial |
| 5.7.4.1.2 | Partial |
| 5.7.4.2 | Partial |
| 5.7.5 | Partial |
| 5.7.6 | Partial |
| 6.6.7.1 | Partial |
| 6.6.9.1 | Partial |
| 9.5.3 | Partial |
| 12.3 (c) | Partial |