Texas Nodal EDW

Performance Monitoring & Compliance

Software Requirements Specification

Including Protocol Section 8

Version 0.03

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 4/24/2006 | 0.01 | Draft version based upon John Adam’s review of entire set of protocols. | Chandra Ramireddy |
| 5/11/2007 | 0.02 | Revised introduction, updated total list of requirements to reflect latest round of requirements analysis completed by John Adams | Scott Privette |
| 5/18/2007 | 0.03 | Completed input from business managers around report names and protocol references for each requirement line. | Scott Privette |

Table of Contents

1.	Introduction	4

1.1	Scope	4

1.2	Document Conventions	5

1.3	Assumptions and Dependencies	5

1.4	Definitions	5

2.	Structure	7

3.	Functional Requirements	8

3.1	Requirements from Nodal Protocols	8

3.2	Requirements from Other Binding Documents	39

4.	Supplementary Requirements	40

4.1	Performance Requirements	40

4.2	Confidentiality Requirements	41

4.3	Legal and Regulatory requirements	41

4.4	System and Communication Requirements	41

4.5	System Security Requirements	41

4.6	Back up and Recovery Requirements	41

4.7	Availability and Redundancy Requirements	41

4.8	Maintainability Requirements	41

4.9	Training and Documentation Requirements	41

4.10	Usability Requirements	41

5.	Protocol Coverage	41

6.	Sub-Process Coverage	42

4

4

4

4

4

4

4

4

4

4

4

4

4

4

4

4

4

4

4

Software Requirements Specification

# Introduction

## Scope

The TNMIP EDW Project is designed to provide or transition the enterprise data solution foundation and functional components to:

(a)	Replicate and archive the ERCOT data sets identified in the Nodal Protocols;

(b)	Ensure data extraction and reporting is provided to the nodal market in a manner consistent with the intent of the Nodal Protocols;

(c)	Ensure ERCOT operational and market support entities have access to current and historical data sets provided to the nodal market and query capability to resolve market inquires; and

(d)	Ensure ERCOT data archival systems have in place row-level security measures for constructing data access procedures consistent with protocol section 1.3.1, Restrictions on Protected Information;

The TNMIP EDW Project will provide the specific data services and products that are identified within the protocols that assist in the function of reporting ERCOT performance and compliance.  These data services and products will contain information on various aspects of ERCOT’s performance in the following activities:

(a)	Coordinating the wholesale electric market transactions;;

(b)	System-wide transmission planning; and;

(c)	Network reliability functions in the ERCOT Region;

The specific list of data services and products will include include the following ERCOT activities:

### 1.) Transmissions Control;

### 2.) Resource Control;

### 3.) Load Forecasting;

### 4.) System Operating Constraints;

### 5.) Settlement Stability;

### 6.) Performance in implementing network model updates;

### 7.) Network Operations Model validation;

### 8.) Back-up control plan;

### 9.) Written Black-Start plan;

### 10.) SAS 70 audit results; and

### 11.) Computer and communication systems Real-Time availability and systems security.

## Document Conventions

- All text lifted from the Nodal Protocols or PUCT Rulings is in italic font.

- All dependencies are identified using D1, D2, etc.

- All assumptions are identified using A1, A2, etc.

## Assumptions and Dependencies

### A1) The Data Archive as stated in the Protocols is a data system which archives on-line operational data for 4 years and data in this system will be maintained and accessible for an additional 3 years.

### A2) The Data Warehouse as stated in the protocols is a data system which retains on-line operational data used for reporting for 4 years.

### A3) Compliance requirements are a mixture of business processes and IT deliverables; the EDW requirements will only be covering the data services and products that support business processes relating to compliance with the Nodal Protocols.

A4) The compliance requirements that result in EDW deliverables will utilize ERCOT business SMEs for detailed definitions of the data services and products necessary for reporting compliance.

### D1) ERCOT data and process owners, commonly referred to as business owners, will identify the appropriate source system of record for all required and included data to be delivered by the EDW Project data services and products.

### D2) Nodal project teams will be responsible for delivery of the ERCOT source systems and data sets therein required to define and construct data products in support of compliance requirements.

### D3) Nodal project teams will be responsible for delivery of the ERCOT business processes in support of compliance requirements.

### D4) The presentation functions of the MIS Public Area will be provided by the Nodal MIS project.

## Definitions

Definitions listed below provide clarity to the language used in Functional and Supplemental Requirements.

| Business Intelligence Products | A business intelligence product delivers a concise set of highly integrated data elements to support detailed business analysis functions as trending, statistics, simulations, operational projections, historical analysis, and gap analysis. |
| --- | --- |
| Data Archive/ Data Archival | The function of replicating data off of the ERCOT source systems to a data store that provides the function of access to historical data. The previous ERCOT database for this function was also named “Data Archive” and in certain places in the protocols appears as a reference to a specific system. For the purpose of these requirements traceability to the protocols the term is used as the generic function without reference to a specific database that performs the function. |
| Data Attributes/ Attributes | Entities that define properties of data elements. |
| Data Element | A basic unit of information built on standard structures having a unique meaning and distinct units or values. |
| Data Integrity | The accuracy, correctness, and validity of data individually and as a whole. |
| Data Repository | Any system which has as it primary function that of retaining data from other systems, and typically serves as a distribution point for data products. Also see “Data Archive” |
| Data Services | Data services is the high-level grouping of IT functions including data replication, data archiving, data extraction, real-time and historic reporting, and generation of business intelligence products. |
| Data Warehouse | The function of transforming data and storing such data for the expressed purpose of reporting. The previous ERCOT database for this function was also named “Data Warehouse” and in certain places in the protocols appears as a reference to a specific system. For the purpose of these requirements the term is used as the generic function without reference to a specific database that performs the function. |
| ERCOT data storage systems | This phrase is used throughout these requirements and addresses the systems used to replicate, store, and report required data to WEMM and PUCT as defined in FR1 |
| Functional Requirements | Functional requirements specify specific behaviors of a system. This should be contrasted with non-functional requirements which specify overall characteristics. |
| Latency | Latency is the time delay between the moment a request for data is initiated, and the moment when the requested data is returned. |
| Performance | How fast a business operation performs under defined circumstances. |
| Reliability | The ability in percent of operational time that a device or system performs a required function under stated conditions. |
| Report, Reporting | The creation and distribution of distinct data sets, bound by specific business rules to assist a ERCOT business function, PUCT function, WEMM function, or Market data request. |
| Role | The named set of features defined over a collection of entities (e.g., people, systems, organizations) participating in a particular context. |
| SLA | Service Level Agreement (SLA) – the part of a service contract in which a certain level of service is agreed between service provider and service recipient. |
| Supplementary Requirements | Also known as non-functional requirements. Supplementary requirements impose constraints and specify overall characteristics. Examples of supplementary requirements include performance, reliability, availability, security, legal and reusability, etc, |
| Supportability | The level to which the system enables the technical support personnel to troubleshoot or perform root cause analysis in pursuit of solving a problem with a product/system. |
| Usability | The ease with which people can employ a particular tool or system in order to achieve a particular goal. In user-interface design usability usually refers to the degree in which the design utilizes intuitively constructed symbols, icons, and layouts. |

# Structure

This document is titled as TN.EDW.ComplianceRequirementsSpecification.v[x].doc

The Requirements sections of this document are:

- Functional Requirements

  - Nodal Protocol Requirements

  - Other Binding Document

- Supplemental Requirements

All protocol references are from the current draft Nodal Protocols as of April 30, 2007.

# Functional Requirements

## Requirements from Nodal Protocols

The following line item requirements were derived from an ERCOT review of the entire Nodal Protocol set (Jan. 2007).  The review process was conducted by ERCOT business ndicate managers froerational areas.  The results of this input were reduced to remove those compliance items that are satisfied by an ERCOT business process entirely without any type of reporting.  The items that were finally included in this document should reflect requirement items that are not completely covered by the requirements sets of other Nodal Project Teams.

| FR | Protocol Name | Protocol Description | How ERCOT assures | Reporting method | Report name | Report description |
| --- | --- | --- | --- | --- | --- | --- |
| FR 14 | NP.Section 11.1.11 | Whenever ERCOT makes an edit to data previously provided to the TDSP, ERCOT shall provide the revised data to the TDSP by noon of the tenth (10th) Business Day after the edit is made. | ERCOT Business Process | Manually Initiated Report | 2 reports NOMCR edited data Telemetry manual over-rides | 2 reports NOMCR edited data Telemetry manual over-rides |
| FR 15 | NP.Section 6.4.8.2.3 (2) | As soon as practicable, but no later than the time specified in Section 6.4.8.2, Supplemental Ancillary Services Market, ERCOT shall post on the MIS Public Area the hourly: (a) SASM MCPC for each type of Ancillary Service for each hour; (b) Total Ancillary Service procured in MW by Ancillary Service type for each hour; and (c) Aggregated Ancillary Service Offer Curve for each Ancillary Service for each hour. | ERCOT business process | Automated Report | 2 reports NOMCR edited data Telemetry manual over-rides | 2 reports NOMCR edited data Telemetry manual over-rides |
| FR 16 | NP.Section 3.9.2(4) | If the QSE does not correct the deficiency within one hour after receiving the notice from ERCOT, then ERCOT shall follow the procedures outlined in Section 6.4.8.1, Evaluation and Maintenance of Ancillary Service Capacity Sufficiency. | Business Process | Manually Initiated Report | Ancillary Service Insufficiency | Dates/time AS obligation and AS actual for a time period |
| FR 17 | NP.Section 3.14.2 (4) | ERCOT shall schedule random testing or simulation, or both, to verify Black Start Service is operable according to the ERCOT System restoration plan. | Business Process & Monitored | Automated Report | Ancillary Service Insufficiency | Dates/time AS obligation and AS actual for a time period |
| FR 18 | NP.Section 4.4.7.1 (5) (e) | If a QSE does not self-arrange all of its Ancillary Service Obligation, ERCOT shall procure the remaining amount of the Ancillary Service Obligation for the QSE. | System | Automated Report | Ancillary Service Procurement | Report on ancillary service procured, amount, and on behalf of which participant |
| FR 19 | NP.Section 3.1.4.5 (4) | When ERCOT approves a Maintenance Outage, ERCOT shall coordinate timing of the appropriate course of action under these Protocols. | Business Process & Monitored | N/A | Ancillary Service Procurement | Report on ancillary service procured, amount, and on behalf of which participant |
| FR 20 | NP.Section 3.1.5.1 (1) | ERCOT shall evaluate requests under Section 3.1.5.11, Evaluation of Transmission Facilities Planned Outage or Maintenance Outage Requests. | ERCOT business process | Manually Initiated Report | Ancillary Service Procurement | Report on ancillary service procured, amount, and on behalf of which participant |
| FR 21 | NP.Section 3.16 (5) | ERCOT shall post engineering studies on the MIS Secure Area representing specific Ancillary Service requirement on an annual basis. | ERCOT Business Process | Manual Process | AS engineering Studies | AS engineering Studies |
| FR 22 | NP.Section 22 Attachment J Section 8. Operation - B | ERCOT shall timely report to Participant the status of the Synchronous Condenser Unit with respect to Operational Limitations. | ERCOT business process | N/A | AS engineering Studies | AS engineering Studies |
| FR 23 | NP.Section 4.2.1.2(2) | By 0600 of the Day-Ahead, ERCOT shall notify each QSE of its Ancillary Service Obligation for each service and for each hour of the Operating Day. | ERCOT Business Process | Automated Report | AS Obligations | AS Obligations |
| FR 24 | NP.Section 4.2.1.2(3) | By 0600 of the Day-Ahead, ERCOT shall post on the MIS Certified Area each QSE’s Load Ratio Share used for the Ancillary Service Obligation calculation. | ERCOT Business Process | Automated Report | AS Obligations | AS Obligations |
| FR 25 | NP.Section 4.2.4 | ERCOT shall provide each QSE with the information necessary to pre-validate its data for DAM, including publishing validation rules for offers, bids and trades and posting any software documentation and code that is not Protected Information to the MIS Se | system | Automated Report | Bidding rules and requirements | Information on rules for offers, trade postings, and software documentation |
| FR 26 | NP.Section 13.1.1 | On the day following the Operating Day, ERCOT shall also calculate DLFs for each Settlement Interval using actual ERCOT System Load for that Settlement Interval and post the resulting deemed actual DLFs to the settlement system and the MIS. | System | Automated Report | Calculated distribution Loss Factors | Calculated distribution loss factors |
| FR 27 | NP.Section 5.5.2(3) | ERCOT shall, within one day after making any changes to the RUC-recommended commitments, post to the MIS Secure Area any changes that ERCOT made to the RUC-recommended commitments with an explanation of the changes. | Business Process - System | Automated Report | Changes to RUC recommended commitments | Report on difference between RUC recommendation & actual commitments with explination |
| FR 28 | NP.Section 3.1.7.1 (2) | Timelines for Response by ERCOT on Reliability Resource Outages ERCOT shall approve or reject no later than: No less than 30 days - 15 days before the start of the proposed Outage Greater than 45 days - 30 days before the start of the proposed Outage | ERCOT business process | N/A | Changes to RUC recommended commitments | Report on difference between RUC recommendation & actual commitments with explination |
| FR 29 | NP.Section 3.16 (5) | ERCOT shall notify the QSE representing an RMR Unit for any unit that is being committed in the DAM or the DRUC at the same time that the DAM and DRUC participants are notified of the results of that respective process. | ERCOT Business Process | Manually Initiated Report | committed resource | committed resource |
| FR 30 | NP.Section 5.5.2(3) | ERCOT shall notify each QSE which of its Resources have been committed as a result of the RUC process. | ERCOT Business Process | Automated Report | committed resource | committed resource |
| FR 31 | NP.Section 3.10.7.4(4) | Each TSP and QSE shall use fully redundant data communication links (ICCP or RTU) between its control center systems and ERCOT systems such that any single element of the communication system can fail and: | System | Automated Report | Communications availability | report on link availability |
| FR 32 | NP.Section 3.19 (5) | ERCOT shall post the Competitive Constraints to the MIS Secure Area at least five Business Days before any change takes effect. | ERCOT Business Process | Manually Initiated Report | Competitive Constraints | Date of posting and implementation and list of constraints |
| FR 33 | NP.Section 22 Attachment J Section 8. Operation C(1) | ERCOT shall notify Participant, through its QSE, of the hours and levels of generation, if any, that the Synchronous Condenser Unit is to operate. | ERCOT business process | N/A | Competitive Constraints | Date of posting and implementation and list of constraints |
| FR 34 | NP.Section 3.19 (5) | ERCOT shall post any Competitive Constraints that have been suspended and the duration of the suspension as soon as practicable to the MIS Secure Area. | ERCOT Business Process | Manually Initiated Report | Competitive Constraints suspended | Date of suspension and list of constraints |
| FR 35 | NP.Section 3.1.1 (1) | ERCOT shall coordinate and use reasonable efforts, consistent with Good Utility Practice, to accept, approve or reject all Outage schedules for maintenance, repair, and construction of both Transmission Facilities and Resources within the ERCOT System. | ERCOT business process | Automated Report | Competitive Constraints suspended | Date of suspension and list of constraints |
| FR 36 | NP.Section 3.10(2) | By providing Base Points and pricing signals by Electrical Bus to Market Participants, the Market Participants’ responses result in power flows on all Transmission Elements that ERCOT must monitor and, if necessary for reliability reasons, manage within ratings provided by the TSP and limits assigned by ERCOT. | ERCOT Business Process | Automated Report | congestion | congestion details |
| FR 37 | NP.Section 5.5.1 (11) | On continued de-selection of contingencies, ERCOT shall prepare an analysis to determine the cause of the error. | ERCOT business process | N/A | congestion | congestion details |
| FR 38 | NP.Section 4.2.1.1 (1) | ERCOT shall analyze the expected Load conditions for the Operating Day and develop an Ancillary Service Plan that identifies the Ancillary Service MW necessary for each hour of the Operating Day. | ERCOT business process | N/A | congestion | congestion details |
| FR 39 | NP.Section 3.10.8.4(3) | ERCOT shall identify additional Transmission Elements that have a high probability of providing significant added economic efficiency to the ERCOT market through Dynamic Rating and request such Dynamic Ratings from the associated TSP. | ERCOT Business Process | Manually Initiated Report | congestion | congestion details |
| FR 40 | NP.Section 3.10.8.4(3) | ERCOT shall post semi-annually the list of the Transmission Elements and identify if the TSP has agreed to provide the rating on the MIS Secure Area. | ERCOT Business Process | Manually Initiated Report | congestion | congestion details |
| FR 41 | NP.Section 6.3.3 | ERCOT may temporarily deviate from the Real-Time deadlines but only to the extent necessary to ensure the secure operation of the ERCOT System. Temporary measures may include varying the timing requirements as specified below or omitting one or more procedures in the Real-Time Sequence. In such an event, ERCOT shall immediately declare an Emergency Condition and notify all QSEs of the following: (a) Details of the affected timing requirements and procedures; (b) Details of any interim requirements; (c) An estimate of the period for which the interim requirements apply; and (d) Reasons for the temporary variation. | ERCOT business process | N/A | congestion | congestion details |
| FR 42 | NP.Section 6.5.1.2 (5) | ERCOT shall post shift schedules on the MIS Secure Area. | ERCOT business process | N/A | congestion | congestion details |
| FR 43 | NP.Section 3.9(3) | ERCOT shall monitor the accuracy of each QSE’s COP as outlined in Section 8, Performance Monitoring and Compliance. | System | Automated Report | COP accuracy | Cop accuracy by day, month, year |
| FR 44 | NP.Section 6.4.8.1.2 (1) | The HRUC process must honor the HASL and LASL for each Resource for each hour of the RUC Study Period unless by doing so a transmission constraint exists where energy from the Resource is needed to resolve the constraint that cannot be resolved by any other means or the energy output from the Resource must be decreased such that the Resource is unable to provide the Ancillary Service capacity allocated to that Resource in the COP. | Monitored | Automated Report | COP accuracy | Cop accuracy by day, month, year |
| FR 45 | NP.Section 4.5.1 (1) | At 1000 in the Day-Ahead, ERCOT shall start the DAM clearing process. | Business Process - System | Automated Report | DAM execution summary | Time of DAM execution/ completion time/ completion status |
| FR 46 | NP.Section 3.1.6.1 | ERCOT shall acknowledge each request for approval of a Resource Planned Outage schedule within two Business Hours of the receipt of the request. | ERCOT business process | Manually Initiated Report | DAM execution summary | Time of DAM execution/ completion time/ completion status |
| FR 47 | NP.Section 4.5.2 (1) (c) | If the additional Ancillary Service Offers are still insufficient to supply the Ancillary Service required in the Day-Ahead Ancillary Service Plan then ERCOT shall run the DAM by reducing the Ancillary Service Plan quantities only for purposes of the DAM by the amount of insufficiency. | ERCOT Business Process | Manually Initiated Report | DAM results | Report DAM results summary by DATE |
| FR 48 | NP.Section 4.5.2 (1) (d) | When ERCOT must reduce the Ancillary Service Plan for purposes of the DAM due to insufficient Ancillary Service Offers, ERCOT shall preserve the Ancillary Service Plan in the DAM in the following order of priority:(i) Reg-Up; (ii) Reg-Down; (iii) RRS; and (iv) Non-Spin. | System | Manually Initiated Report | DAM results | Report DAM results summary by DATE |
| FR 49 | NP.Section 4.5.2 (1) (d) | Then ERCOT must reduce the Ancillary Service Plan for purposes of the DAM due to insufficient Ancillary Service Offers, ERCOT shall preserve the Ancillary Service Plan in the DAM in the following order of priority: (i) Reg-Up; (ii) Reg-Down; (iii) RRS; and (iv) Non-Spin. | System | Manually Initiated Report | DAM results | Report DAM results summary by DATE |
| FR 50 | NP.Section 3.1.6.6 | ERCOT must accept, but ERCOT may discuss reliability and scheduling impacts to minimize hazard/cost to ERCOT System in an attempt to accomplish minimum overall impact. | ERCOT business process | Manually Initiated Report | DAM results | Report DAM results summary by DATE |
| FR 51 | NP.Section 3.1.6.8 (4) | ERCOT must give as much notice as practicable to the TSP. | ERCOT business process | Manually Initiated Report | DAM results | Report DAM results summary by DATE |
| FR 52 | NP.Section 3.1.6.9 (1) | Within two hours of receiving such request, ERCOT shall either: | ERCOT business process | N/A | DAM results | Report DAM results summary by DATE |
| FR 53 | NP.Section 4.5.3 (3) | ERCOT shall monitor Day-Ahead MCPCs and Day-Ahead hourly LMPs for errors and shall “flag” for further review questionable prices before posting and make notations in the posting if there are conditions that cause the price to be questionable. | Business Process - System | Manually Initiated Report | DAM results flagged | Report on DAM results flagged for questionable pricess |
| FR 54 | NP.Section 3.1.7.1 (1) | ERCOT shall approve or reject each request in accordance with the following table: | ERCOT business process | N/A | DAM results flagged | Report on DAM results flagged for questionable pricess |
| FR 55 | NP.Section 9.17.2(1) | By the seventh Business Day of each month, ERCOT shall provide the requesting TSP or DSP data pertaining to transactions over the DC Ties for the immediately preceding month. | System | Automated Report | DC tie Transactions | Report on DC tie transactions for previous month |
| FR 56 | NP.Section 9.17.2(2) | ERCOT shall maintain and provide the requesting TSP or DSP data pertaining to transactions over the DC Ties for the period from June 2001 to the present. | ERCOT Business Process | Manually Initiated Report | DC tie Transactions | Report on DC tie transactions for previous month |
| FR 57 | NP.Section 8.1.1 (a) | For all frequency deviations exceeding 0.1 Hz, ERCOT shall use recorded two-second scan rate values of real power output for each Resource to evaluate governor response or response to Dispatch Instructions. | ERCOT Business Process | Automated Report | disturbance performance | governor response evaluation |
| FR 58 | NP.Section 8.1.1 (a) | ERCOT shall use the recorded MW data beginning one minute before the start of the frequency excursion event or Dispatch Instruction until 20 minutes after the start of the frequency excursion event or Dispatch Instruction. | ERCOT Business Process | Automated Report | disturbance performance | governor response evaluation |
| FR 59 | NP.Section 3.10.7.4.2 (6) | ERCOT shall implement a study mode version of the State Estimator with special tools designed for troubleshooting and tuning purposes that can be used independently of any other ERCOT process that is dependant on the real time State Estimator. | ERCOT business process | N/A | DSR scada failure report | Start & end time of DSR scada failures |
| FR 60 | NP.Section 6.4.2.5(3)(b) | If the DSR Load signal fails for more than 10 consecutive hours, ERCOT shall suspend the QSE’s ability to use Dynamically Scheduled Resources until the signal is reliably restored (as determined by ERCOT). | ERCOT Business Process | Automated Report | DSR suspensions | DSR suspensions |
| FR 61 | NP.Section 3.10.7.4.2 (6) | ERCOT must be guided in this by TAC-approved accuracy standards. | ERCOT business process | N/A | DSR suspensions | DSR suspensions |
| FR 62 | NP.Section 3.10.7.1.5(3) | ERCOT shall provide copies of the switching plan to the Market Participants via the MIS Secure Area. | ERCOT Business Process | Manually Initiated Report | equipment status | logic for determining states of reactive devices |
| FR 63 | NP.Section 21.8.5 | Upon posting of a SCR, ERCOT shall conduct an initial SCR Impact Analysis (SIA) based on the submitted language. | ERCOT business process | Manually Initiated Report | equipment status | logic for determining states of reactive devices |
| FR 64 | NP.Section 5.5.2 (6) | ERCOT shall create Three-Part Supply Offers for all Resources that did not submit a Three-Part Supply Offer but are specified as available but Off-Line, excluding Resources with a Resource Status of EMR, in a QSE’s COP. | System | Automated Report | ERCOT created 3-part offers | Report on all ERCOT created 3-part supply offers |
| FR 65 | NP.Section 5.5.3(2) | For such Resources, ERCOT shall use in the RUC process 150% of any approved verifiable startup cost and verifiable minimum-energy cost or if verifiable costs have not been approved, the applicable Resource Category Generic Startup Offer Cost and the applicable Resource Category Generic Minimum-Energy Offer Cost as described specified in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, registered with ERCOT. | System | Manually Initiated Report | ERCOT created 3-part offers | Report on all ERCOT created 3-part supply offers |
| FR 66 | NP.Section 3.1.7.2 | ERCOT shall approve or reject the proposed change within 15 days of receiving the change request form. | ERCOT business process | N/A | ERCOT created 3-part offers | Report on all ERCOT created 3-part supply offers |
| FR 67 | NP.Section 3.2.1 | ERCOT shall use Planned Outages submitted by Resource Entities under Section 3.1, Outage Coordination, to calculate the aggregate capacity from Resources projected to be available in the ERCOT Region. | ERCOT business process | N/A | ERCOT created 3-part offers | Report on all ERCOT created 3-part supply offers |
| FR 68 | NP.Section | To determine the projected energy output level of each Resource and to project potential congestion patterns for each hour of the RUC, ERCOT shall calculate proxy Energy Offer Curves based on the Mitigated Offer Caps for the type of Resource as specif | System | N/A | ERCOT created proxy Energy Offer Curves | Report on Proxy offer curves created for RUC by Day? |
| FR 69 | NP.Section 3.1.7.1 (3) | ERCOT shall not be deemed to have approved the Outage request associated with the Planned Outage until ERCOT notifies the Single Point of Contact of its approval. | ERCOT business process | N/A | ERCOT created proxy Energy Offer Curves | Report on Proxy offer curves created for RUC by Day? |
| FR 70 | NP.Section 3.10.4 (1) | ERCOT shall post all documents relating to the State Estimator Performance Standard on the MIS Secure Area. | ERCOT business process | Manually Initiated Report | ERCOT directed unit commitments | Commitments, & identification of RUC recommendatio or identification as not RUC |
| FR 71 | NP.Section 6.5.7.6.2.3(6) | For non-DSRs with Output Schedules providing Non-Spin, on deployment of Non-Spin, ERCOT shall adjust the Resource Output Schedule for the remainder of the Operating Period to reflect the amount of deployment. | System | Automated Report | ERCOT's adjustment of output schedules | All output schedules ERCOT adjusted and why |
| FR 72 | NP.Section 3.10(4) | For each model, ERCOT shall post ratings and the ambient temperatures used to calculate the ratings on the MIS Secure Area when the model is published. | system | Automated Report | facility ratings | facility ratings |
| FR 73 | NP.Section 3.10(4) | ERCOT must use system ratings consistent with the ratings expected to be used during Real-Time for the system condition being modeled, including Dynamic Ratings using expected temperatures for those system conditions. | system | Automated Report | facility ratings | facility ratings |
| FR 74 | NP.Section ERCOT Protocols Section 22 Attachment A 8.A | The specific times for Planned Outages of the Black Start Resource must be approved by ERCOT. | ERCOT business process | Manually Initiated Report | facility ratings | facility ratings |
| FR 75 | NP.Section 4.2.1.1 (1) | ERCOT must post the Ancillary Service Plan to the MIS Public Area by 0600 of the Day-Ahead. | ERCOT business process | N/A | facility ratings | facility ratings |
| FR 76 | NP.Section 3.1.4.5(1) | In the event of a Forced Outage, after the affected equipment is removed from service, the Resource Entity or QSE, as appropriate, or TSP must notify ERCOT as soon as practicable of its action by: | ERCOT Business Process | Manually Initiated Report | FOD to COP time | Time between FOD outage detection and change to COP by QSE |
| FR 77 | NP.Section 3.1.4.5(3) | For Maintenance Outages, the Resource Entity or QSE, as appropriate, or TSP shall notify ERCOT of any Resource or Transmission Facilities Maintenance Outage according to the Maintenance Outage Levels by updating the Current Operating Plan and Outage Scheduler. | ERCOT Business Process | Manually Initiated Report | FOD to COP time | Time between FOD outage detection and change to COP by QSE |
| FR 78 | NP.Section 13.1.1 | ERCOT shall forecast the DLFs expressed as a percent of Load for each Settlement Interval of the Operating Day. | Business Process - System | Automated Report | Forecast Distribution Loss Factors | Distribution Loss factors posted by day |
| FR 79 | NP.Section 13.2.1 | At 0600 of the Day Ahead period, ERCOT shall forecast a Transmission Loss Factor for each Settlement Interval of the Operating Day and post the forecasted Transmission Loss Factors which correspond to the Operating Day forecast. | Business Process - System | Automated Report | Forecast Transmission Loss Factors | Forecast Transmission loss factors posted by day |
| FR 80 | NP.Section 8.1.2.2 (1) | ERCOT shall evaluate during each season of expected operation the Net Dependable Capability of each Resource expected to operate more than 168 hours during that season, except for Generation Resources used solely for energy services and whose capacity is less than ten MW. | Monitored | Automated Report | Forecast Transmission Loss Factors | Forecast Transmission loss factors posted by day |
| FR 81 | NP.Section 6.5.7.2(1) | ERCOT shall calculate the HASL, LASL, SCED Up Ramp Rate (SURAMP), SCED Down Ramp Rate (SDRAMP), HDL and LDL within 4 seconds after a change of the Resource-specific attributes provided as part of the QSE’s SCADA telemetry under Section 6.5.5.2, Operat | System | Automated Report | Generator Parameter Report | 1 minute average HSL, HASL, HDL, LDL, LASL, SURAMP, SDRAMP for a given time period |
| FR 82 | NP.Section 5.5.1 (6) | ERCOT shall include the reason for removal of any contingency as soon as practicable but not later than one hour after removal. | ERCOT business process | Manually Initiated Report | Generator Parameter Report | 1 minute average HSL, HASL, HDL, LDL, LASL, SURAMP, SDRAMP for a given time period |
| FR 83 | NP.Section 3.7.1.1(1)(h) | Minimum On-Line time – The minimum number of consecutive hours the Resource must be On-Line before being shut down; | System | Manually Initiated Report | Generator Parameters | Report on generator parameters used by the system by date range/Unit - selectable |
| FR 84 | NP.Section Section 1: Overview 1.1(2)(b)(ii) | Minimum Off-Line time – The minimum number of consecutive hours the Resource must be Off-Line before being restarted; | System | Manually Initiated Report | Generator Parameters | Report on generator parameters used by the system by date range/Unit - selectable |
| FR 85 | NP.Section 3.10.4(5) | Each TSP shall provide reasonably accurate information at the time of the original submission. | ERCOT Business Process | Manually Initiated Report | Incident Report | Report on incident reports in a time period |
| FR 86 | NP.Section 3.10.7.1.5(2) | Each Market Participant shall provide ERCOT with the following information, subject to the naming conventions in Section 3.10.7.1, Modeling of Transmission Elements and Parameters:(a) Equipment owner(s); (b) Equipment operator(s); (c) The Transmission Element name; (d) The substation name; (e) Voltage or time switched on; (f) Voltage or time switched off; (g) Associated switching device name; (h) Connectivity; (i) Nominal voltage and associated capacitance or reactance; and (j) Other data necessary to model Transmission Element(s). | ERCOT Business Process | Manually Initiated Report | Incident Report | Report on incident reports in a time period |
| FR 87 | NP.Section 3.10.7.1.5(1) | Each Market Participant shall provide ERCOT with complete information on each device’s capabilities and normal switching schema. | ERCOT Business Process | Manually Initiated Report | Incident Report | Report on incident reports in a time period |
| FR 88 | NP.Section 17.3.3 (2) | ERCOT shall report to the PUCT and WEMM any failure by a Market Participant to provide accurate and complete information in the manner and time requested under these Protocols, and that failure may be treated as grounds for action against the Market Participant. | ERCOT Business Process | Manually Initiated Report | Incident Report | Report on incident reports in a time period |
| FR 89 | NP.Section 4.5.1 (5) | ERCOT shall determine the appropriate Load distributions to allocate offers, bids, and source and sink of CRRs at a Load Zone across the Electrical Buses that are modeled with Load in that Load Zone. | system | Automated Report | Load distributuion | Report of load distributions on a historic day |
| FR 90 | NP.Section 3.1.6.4 (4) | ERCOT shall accept this change and, in the event that a Transmission Facilities Outage was scheduled concurrently with the affected Resource(s) Outage, ERCOT shall coordinate between the TSP and the Resource Entity to schedule a time mutually agreeable to both parties for the Resource to be On-Line. | ERCOT business process | N/A | Load distributuion | Report of load distributions on a historic day |
| FR 91 | NP.Section 4.5.1 (5) | If ERCOT decides, in its sole discretion, to change this distribution for reasons such as anticipated weather events or holidays, ERCOT shall select a State Estimator distribution from a proxy day reasonably reflecting the anticipated distribution in the Operating Day. | ERCOT Business Process | Automated Report | Manual over-rides of load distribution | Report of manual overrides load distribution |
| FR 92 | NP.Section 3.1.6.4 (4) | If mutual agreement cannot be reached, then ERCOT shall decide, considering expected impact on system security, future Outage plans, and participants. | ERCOT business process | N/A | Manual over-rides of load distribution | Report of manual overrides load distribution |
| FR 93 | NP.Section 6.5.9.5.1(4) | ERCOT shall post the registration details of all registerd BLTs to the MIS Secure Area. | System | Automated Report | Market Postings |  |
| FR 94 | NP.Section 1.3.3(1)© | One hundred eighty days after the Operating Day, ERCOT shall make the following information available on the MIS Secure Area in a standard reporting format: | System | Manually Initiated Report | Market Postings |  |
| FR 95 | NP.Section 4.4.6.2 (2) | ERCOT shall continuously display on the MIS Certified Area information that allows any QSE submitting a PTP Obligation Bid to view its valid PTP Obligation Bid. | System | Manually Initiated Report | Market Postings |  |
| FR 96 | NP.Section 4.4.9.4.1 (d) | ERCOT shall post to the MIS Secure Area the capacity factor for each Resource before the start of the effective month. | System | Manually Initiated Report | Market Postings |  |
| FR 97 | NP.Section 4.5.3 (2) | As soon as practicable, but no later than 1330, ERCOT shall post on the MIS Public Area the hourly:{Abbreviated: MCPC, LMPs, shadow prices, Quanity of AS offers received, energy bought in DAM for each hour, each settlement point,energy sole in DAM each settlement poiint, Aggrageted offer curve for each AS} | System | Manually Initiated Report | Market Postings |  |
| FR 98 | NP.Section 3.1.3.2 (2) | ERCOT shall report statistics monthly on how Resource Planned Outages compare with actual Resource Outages, and post those statistics to the MIS Secure Area. | Business Process & Monitored | Automated Report | Market Postings |  |
| FR 99 | NP.Section 3.1.5.13 | ERCOT shall post on the MIS Secure Area: (a) All proposed Transmission Facilities Outages that have not yet been approved or rejected within one hour of receipt by ERCOT; and (b) Any approved, accepted or rejected Transmission Facilities Outage within one hour of approval, acceptance or rejection of the Outage | ERCOT business process | Automated Report | Transmission Outage |  |
| FR 100 | NP.Section Section 4. Representations, Warranties, and Covenants- Section 9- Payment(E)(5) | A “Misconduct Event” means any hour or hours during which Participant is requested to, but does not, synchronize the Unit to the ERCOT Transmission Grid during any hour in which the Unit is shown Available in the Availability Plan. (2) Each day that a Misconduct Event continues after Participant receives written notice from ERCOT of the Misconduct Event is a separate Misconduct Event. Misconduct Event is measured on a daily basis. (5) ERCOT shall inform Participant in writing of its determination if a Misconduct Event is unexcused. | ERCOT Business Process | Manually Initiated Report | Misconduct Events | Report on all misconduct events, date, time, description, notification date,etc |
| FR 101 | NP.Section 8.5.2 (2) | ERCOT shall determine the C Point for each event. | Business Process & Monitored | Automated Report | Misconduct Events | Report on all misconduct events, date, time, description, notification date,etc |
| FR 102 | NP.Section 10.3.2.1.3 | For any interval when ERCOT has not received a Real-Time signal for any one of the virtual generating units, ERCOT shall use the last valid percentage ratio for a completed interval. | System | Manually Initiated Report | Missing virtual unit scada report | Date/time of invalid telemetry, last value, last valid percentage ration, and calculated MWH value |
| FR 103 | NP.Section 3.10.4(7) | ERCOT shall provide model information through the use of the Electric Power Research Institute (EPRI) and NERC-sponsored Common Information Model (CIM) and Web-based XML communications. | ERCOT business process | Automated Report | Missing virtual unit scada report | Date/time of invalid telemetry, last value, last valid percentage ration, and calculated MWH value |
| FR 104 | NP.Section 8.1.1 (a) | For a Generation Resource that commences commercial operation after January 1, 2004, ERCOT shall construct an incremental mitigated offer cap curve (Section 6.5.7.3, Security Constrained Economic Dispatch) such that each point on the Mitigated Offer C | System | Manually Initiated Report | Mitigated offer cap curves | Report on mitigated offer cap curves applied by unit by date |
| FR 105 | NP.Section 3.1.5.11 (1) | ERCOT shall reject Outage requests that do not meet the submittal timeline specified in Section 3.1.5.12, Submittal Timeline for Transmission Facility Outage Requests. | ERCOT business process | Manually Initiated Report | Mitigated offer cap curves | Report on mitigated offer cap curves applied by unit by date |
| FR 106 | NP.Section 6.4.2.4(2) | ERCOT shall notify the QSE submitting an Output Schedule by the Messaging System if the schedule was rejected or was considered invalid for any reason. | system | Automated Report | MMS details | MMS details |
| FR 107 | NP.Section 3.7.2 | ERCOT shall verify that all Resource Parameters submitted by a QSE comply with the applicable criteria in Section 3.7.1, Resource Parameter Criteria. | system | Automated Report | MMS flag |  |
| FR 108 | NP.Section 3.18 (5) (d) | The amount of RRS provided from a Load Resource must be less than or equal to the LPC minus the MPC. | Monitored | Automated Report | MMS flag |  |
| FR 109 | NP.Section 3.3.2.2 | ERCOT shall maintain a record of all work approved in accordance with Section 3.3, Management of Changes to ERCOT Transmission Grid, and shall publish, and update monthly, information on the MIS Secure Area regarding each new Transmission Element to be in | system | Automated Report | model changes | model changes |
| FR 110 | NP.Section 3.10(8) | ERCOT shall notify each NOMCR requestor when the requested change is processed and implemented in accordance with Section 3.10.1, Time Line for Network Operations Model Change Requests, ERCOT shall also provide the submitting TSP a link to a network model | system | Automated Report | Network Model |  |
| FR 111 | NP.Section 3.10(8) | ERCOT shall post all NOMCRs on the MIS Secure Area within five Business Days following receipt of the NOMCR, consistent with CEII standards. | system | Automated Report | model changes | change tracking |
| FR 112 | NP.Section 3.10(10) | ERCOT shall post any changes to an NOMCR on the MIS Secure Area within three Business Days of accepting corrections. | system | Automated Report | model changes | change tracking |
| FR 113 | NP.Section 3.10(11) | Following ERCOT approval, ERCOT shall publish a summary of the NOMCR on the MIS Secure Area. | system | Automated Report | model changes | change tracking |
| FR 114 | NP.Section 3.10.1(1) | ERCOT shall perform periodic updates to the ERCOT Network Operations Model. | system | Automated Report | model changes | change tracking |
| FR 115 | NP.Section 3.10.2(3) | ERCOT shall make available to TSPs and/or DSPs and all appropriate Market Participants, consistent with applicable policies regarding release of Critical Energy Infrastructure Information (CEII), the transmission model used in transmission planning. | system | Automated Report | Transmission Model | change tracking |
| FR 116 | NP.Section 3.10.2(4) | ERCOT shall post such information and the model on the MIS Secure Area six (6) months prior to the annual and two-year CRR auctions. | system | Automated Report | model changes | change tracking |
| FR 117 | NP.Section 3.10.4(7) | ERCOT shall provide model information through the use of the Electric Power Research Institute (EPRI) and NERC-sponsored Common Information Model (CIM) and Web-based XML communications. | system | Automated Report | model changes | change tracking |
| FR 118 | NP.Section 3.10.4(5) | ERCOT shall provide notice via electronic means to all Market Participants and post the notice on the MIS Secure Area detailing the changed model information and the reason for the interim update within two Business Days following the report to PUCT staff | ERCOT Business Process | Manually Initiated Report | model changes | temporary configuration chage |
| FR 119 | NP.Section 3.10.7.1(2) | In addition to the Network Operations Model releases described in Section 3.10.1, Time Line for Network Operations Model Change Requests, ERCOT shall provide all names and parameters of all Transmission Elements to Market Participants posted on MIS Secure | system | Automated Report | model changes | change tracking |
| FR 120 | NP.Section 3.10.7.1.4(5) | ERCOT shall post to the MIS Secure Area information regarding all transformers represented in the Network Operations Model. | ERCOT Business Process | Automated Report | model changes | change tracking |
| FR 121 | NP.Section 8.1.2.2.3 (3) | Each QSE shall ensure that each Resource is able to meet the Resource’s obligations to provide the Resource’s Ancillary Service Schedule. | ERCOT business process |  | model changes | model changes |
| FR 122 | NP.Section 3.14.1.11 (2) (b) | Thirty days after receipt of the preliminary Eligible Costs budget, ERCOT shall notify the RMR Unit owner of its selections under the alternatives provided in the preliminary budget. | ERCOT business process | N/A | model changes | change tracking |
| FR 123 | NP.Section 3.14.1.11 (2) (b) | The RMR Unit owner and ERCOT shall set the Target Availability consistent with the options presented to and selected by during the budgeting process. | ERCOT business process | N/A | model changes | change tracking |
| FR 124 | NP.Section 9.2.5(2) | ERCOT shall issue a DAM Resettlement Statement for a given DAM due to error in data other than prices when the total of all significant errors in data other than prices results in an impact greater than two percent of the total payments due to ERCOT for the DAM, excluding bilateral transactions. ERCOT shall issue DAM Resettlement Statements as soon as possible to correct the errors. ERCOT shall review this percentage on an annual basis. | ERCOT business process | Manually Initiated Report | model changes | change tracking |
| FR 125 | NP.Section 9.18(3) | Within 60 days after ERCOT approves a profile segment, ERCOT staff shall evaluate the costs submitted and shall disallow any costs not meeting these criteria. The remaining costs must comprise the total reimbursable cost. Within the same 60-day period, ERCOT shall post a report on the MIS Public Area summarizing the allowed expenses by paragraphs (1) and (2) above. | ERCOT business process | Manually Initiated Report | model changes | change tracking |
| FR 126 | NP.Section Section 6. ERCOT Obligations- B | If ERCOT receives any notice similar to that described in Section 5.B from any Market Participant, ERCOT shall provide notice of same to Participant. | ERCOT business process | Manually Initiated Report | model changes | change tracking |
| FR 127 | NP.Section 21.3 | ERCOT shall consult with the chair of the PRS to coordinate and establish the meeting schedule for the PRS or other assigned subcommittee. | ERCOT business process | N/A | model changes | change tracking |
| FR 128 | NP.Section 21.4.1 | ERCOT shall provide an electronic return receipt response to the submitter upon receipt of the PRR. | ERCOT business process | Automated Report | model changes | change tracking |
| FR 129 | NP.Section 21.4.3 | Within three (3) Business Days after PRS takes action to approve, approve with modifications, or reject the PRR, ERCOT shall issue a report (“PRS Recommendation Report”) to TAC reflecting the PRS action and post the same to the MIS. | ERCOT business process | Manually Initiated Report | PRS Recommendation Report |  |
| FR 130 | NP.Section 21.4.4 | Within three (3) Business Days of receipt of comments related to the PRS Recommendation Report, ERCOT shall post such comments to the MIS. | ERCOT business process | Manually Initiated Report | PRS Recommendation Report |  |
| FR 131 | NP.Section 21.4.5 | Upon receipt of a PRR submitted by any Entity other than ERCOT, ERCOT shall perform an initial evaluation of the impact on ERCOT and include the evaluation in ERCOT’s comments. | ERCOT business process | Manually Initiated Report | PRS Recommendation Report | change tracking |
| FR 132 | NP.Section 21.4.6 | If PRS revises its recommendation, ERCOT shall update the PIA and issue the updated PIA at least three (3) Business Days prior to the regularly scheduled TAC meeting. | ERCOT business process | Manually Initiated Report | PRS Recommendation Report | change tracking |
| FR 133 | NP.Section 21.4.8 | ERCOT shall issue the updated PIA (if any) to the ERCOT Board and post it on the MIS within three (3) Business Days of issuance. | ERCOT business process | Manually Initiated Report | model changes | temporary configuration chage |
| FR 134 | NP.Section 21.4.11 | If the PUCT or other Governmental Authority rules on the PRR, ERCOT shall post the ruling on the MIS. | ERCOT business process | Manually Initiated Report | Rulings |  |
| FR 135 | NP.Section 21.5 | If approved, ERCOT shall submit an Urgent PRS Recommendation Report to the TAC within three (3) Business Days after PRS takes action. | ERCOT business process | Manually Initiated Report | PRS Recommendation Report |  |
| FR 136 | NP.Section 21.8.2 | ERCOT shall post a notice of the submitter’s withdrawal of a SCR on the MIS within one (1) Business Day of the submitter’s notice to ERCOT. | ERCOT business process | Manually Initiated Report | model changes | change tracking |
| FR 137 | NP.Section 6.5.7.1.11 (3) | When ERCOT identifies a binding constraint on a repeated basis ERCOT shall have procedures established to contact the appropriate TSP and validate the accuracy of the Network Operations Model according to Section 3.10.4(5) ERCOT Responsibilities. | ERCOT Business Process | Manually Initiated Report | Monthly Operations Report | Monthly Operations Report |
| FR 138 | NP.Section 3.10(1) | ERCOT shall use the physical characteristics, ratings, and operational limits of all Transmission Elements of the ERCOT Transmission Grid and other information from the TSPs to specify limits within which the transmission network is defined in the net | system | Automated Report | network model | network model |
| FR 139 | NP.Section 3.10(6) | ERCOT shall use a Network Operations Model Change Request (NOMCR) process to control all information entering the Network Operations Model. | ERCOT Business Process | Automated Report | network model | network model |
| FR 140 | NP.Section 8.1.2.2.6 (2) (a) (viii) | The Resource Entity for the Black Start Resource shall provide ERCOT with data to verify these settings. | ERCOT business process | Manually Initiated Report | network model | network model |
| FR 141 | NP.Section 4.2.1.1 (3) | ERCOT shall determine the total required amount of each Ancillary Service under Section 3.16, Standards for Determining Ancillary Service Quantities, or use its operational judgment and experience to change the daily quantity of each required Ancillary Service. | ERCOT business process | N/A | network model | network model |
| FR 142 | NP.Section 4.2.3 | No later than 0600 in the Day-Ahead, ERCOT shall post on the MIS Secure Area, and make available for download, the following information for the Operating Day: | system | Automated Report | network model/system conditions | network model/system conditions |
| FR 143 | NP.Section 5.3(4) | ERCOT shall provide each QSE with the information necessary to pre-validate their data for DRUC and HRUC including: | system | Automated Report | network model/system conditions | network model/system conditions |
| FR 144 | NP.Section 6.5.7.6.2.3(6) | For Dynamically Scheduled Resources (DSRs) providing Non-Spin, on deployment of Non-Spin, the DSR’s QSE shall adjust its Resource Output Schedule to reflect the amount of deployment. | NA | Automated Report | Non-Spin deployment schedule adjustment time | report on individual event time from deployment to schedule adjustment on all non-spin providers |
| FR 145 | NP.Section 5.5.1 (6) | ERCOT shall post to the MIS Secure Area the standard contingency list, including identification of changes from previous versions before being used in the Security Sequence. | system | Automated Report | Operations Contingency list | Operations Contingency list |
| FR 146 | NP.Section 3.1.5.1(1) | A TSP shall request a Planned Outage or Maintenance Outage for any Transmission Element in the Network Operations Model that requires the Transmission Element to be removed from its normal service. | ERCOT Business Process | Automated Report | Outage Report | By QSE outages requested, % by type, etc |
| FR 147 | NP.Section 3.1.6.8(2) | The QSE must give as much notice as practicable to ERCOT. | System | Automated Report | Outage request lead time | Report by outage type of average hours prior to start time that request was submitted. |
| FR 148 | NP.Section 3.5.1(3) | If any Electrical Bus is disconnected from the Network Operations Model or the CRR Network Model through operations changes in transmission topology temporarily, ERCOT shall provide notice to all Market Participants as soon as practicable and exclude | system | Automated Report | outage schedular | equipment status change |
| FR 149 | NP.Section 3.10.7.4.2 (3) | On monitored non-Load substations, each TSP shall install, at the direction of ERCOT, sufficient telemetry such that there is an “N-1 Redundancy.” | ERCOT business process | Manually Initiated Report | outage schedular | equipment status change |
| FR 150 | NP.Section 3.1.3.1 (1) | Each TSP shall provide to ERCOT a Planned Outage or Maintenance Outage plan in an ERCOT-provided format for the next 12 months updated monthly. | ERCOT Business Process | Automated Report | outage schedular |  |
| FR 151 | NP.Section 3.1.5.3 (4) | Any such suggested amendments accepted by the TSP must be processed by ERCOT as a Planned Outage or Maintenance Outage of Transmission Facilities request under this Section. | ERCOT Business Process | Automated Report | outage schedular |  |
| FR 152 | NP.Section 3.10(3) | ERCOT shall post all equipment ratings on the MIS Secure Area on the day prior to the ratings becoming effective including the identity of the Transmission Element, old rating and new rating, effective date, and a text reason supplied by the appropriate T | system | Automated Report | rating changes | rating changes |
| FR 153 | NP.Section 8.1.2.2.6 (2) (d)(ii) | To pass the test, (a) the potential Black Start Resource must start the next start unit (as determined by ERCOT), or start the next start unit’s largest required motor and satisfied the next start unit’s minimum startup Load requirements; or (b) the Resource Entity shall demonstrate to the satisfaction of ERCOT staff through simulation studies conducted by the Resource Entity or a qualified third party, that the potential Black Start Resource is capable of starting the next start unit’s largest required motor while meeting the next start unit’s minimum startup Load requirements. | ERCOT business process | Manually Initiated Report | rating changes | rating changes |
| FR 154 | NP.Section 3.10(3) | ERCOT shall post all methodologies on the MIS Secure Area seven days following a change in methodology. | ERCOT Business Process | Manually Initiated Report | rating methodology changes | rating methodology changes |
| FR 155 | NP.Section 8.1.2.2.6 (2) (d)(iv) | If a physical test is performed, the Black Start Resource must remain stable (in both voltage and frequency) and controlling voltage for 30 minutes. | ERCOT business process | Manually Initiated Report | rating methodology changes | rating methodology changes |
| FR 156 | NP.Section 3.10.8(4) | ERCOT shall use the most limiting rating and report the circumstance to the Wholesale Electric Market Monitor. | system | Automated Report | ratings | default rating |
| FR 157 | NP.Section 6.5.7.1.7(1) | ERCOT shall post summaries of dynamically adjusted Transmission Element limits on the MIS Secure Area in a form that allows Market Participants to directly upload Real-Time data into the Common Information Model. | system | Automated Report | ratings | ratings |
| FR 158 | NP.Section 6.5.7.1.7(2) | On a monthly basis, ERCOT shall provide a summary report for each dynamically rated Transmission Element specifying the average change in Normal Rating in MVA that is gained on the element through use of a Dynamic Rating rather than the Normal Rating. | system | Automated Report | ratings | ratings |
| FR 159 | NP.Section 6.5.7.1.7(2) | ERCOT shall post this report to the MIS Secure Area. | system | Automated Report | ratings | ratings |
| FR 160 | NP.Section 21.11.3.10(4) | If the PUCT or other Governmental Authority rules on the NPRR, ERCOT shall post the ruling on the MIS and shall implement the effect, if any, of the ruling on the Nodal Protocols. | ERCOT business process | Manually Initiated Report | ratings | default rating |
| FR 161 | NP.Section 6.5.7.6.2.2 (13) | To the extent that ERCOT deploys a Load Resource that has chosen a block deployment option, ERCOT shall either deploy the entire offer or, if only partial deployment is possible, skip the offer by the Load Resource with the block deployment option and proceed to deploy the next available Resource. | ERCOT business process | N/A | ratings | ratings |
| FR 162 | NP.Section 6.5.7.6.2.3 (1) | ERCOT shall deploy Non-Spinning Reserve Service using SCED for On-Line Generation Resources with Energy Offer Curves and by Operator Dispatch Instruction for Resources with Output Schedules, Off-Line Generation Resources and Load Resources. | ERCOT business process | N/A | ratings | ratings |
| FR 163 | NP.Section 6.5.7.6.2.3 (2) | Once Generation Resources providing Non-Spin are deployed and On-Line, ERCOT shall use SCED to determine the amount of energy to be dispatched from those Resources. | ERCOT business process | N/A | ratings | ratings |
| FR 164 | NP.Section 6.5.7.6.2.1(4)(d) | ERCOT shall integrate the control signal sent to providers of Reg-Up and shall calculate the amount of energy deployed by Reg-Up in each Settlement Interval. | System | Manually Initiated Report | Regulation Report | 5 minute average or 15 minute or 1 hour average of reg-up & reg-down deployed by unit or QSE |
| FR 165 | NP.Section 6.5.7.6.2.1(4)(e) | ERCOT shall integrate the control signal sent to providers of Reg-Down and shall calculate the amount of energy deployed by Reg-Down in each Settlement Interval. | System | Manually Initiated Report | Regulation Report | 6 minute average or 15 minute or 1 hour average of reg-up & reg-down deployed by unit or QSE |
| FR 166 | NP.Section 6.4.8.1 (1) | ERCOT shall evaluate Ancillary Service requirements and capacity sufficiency using evaluation tools including the Ancillary Services Capacity Monitor, described in Section 6.5.7.5, Ancillary Services Capacity Monitor, throughout the Adjustment Period and Operating Period. | ERCOT business process | N/A | Regulation Report | 5 minute average or 15 minute or 1 hour average of reg-up & reg-down deployed by unit or QSE |
| FR 167 | NP.Section 6.5.7.6.1 (6) | If all Reg-Up capacity has been deployed, ERCOT shall use the LFC system to deploy Responsive Reserve on Generation Resources and Controllable Load Resources. | ERCOT business process | N/A | Regulation Report | 6 minute average or 15 minute or 1 hour average of reg-up & reg-down deployed by unit or QSE |
| FR 168 | NP.Section 4.4.10 (a) | If the criteria are not met, ERCOT shall reject all of the QSE’s bids and offers, and shall notify the QSE through the MIS Certified Area as soon as practicable. | System | Manually Initiated Report | Rejected Bids | All rejected DAM bids or offers and the reason and if they were accepted on resubmittal |
| FR 169 | NP.Section 4.4.10 (b) | ERCOT shall accept the QSE’s resubmitted valid bids and offers that meet the criteria if such resubmission is prior to 1000. | System | Manually Initiated Report | Rejected bids | All rejected DAM bids or offers and the reason and if they were accepted on resubmittal |
| FR 170 | NP.Section 3.1.6 (2) (b) | ERCOT shall accept Planned Outage proposals, except that ERCOT shall reject an Outage proposal if it will impair ERCOT’s ability to meet applicable reliability standards and other solutions cannot be exercised. | ERCOT business process | N/A | Rejected Bids | All rejected DAM bids or offers and the reason and if they were accepted on resubmittal |
| FR 171 | NP.Section 3.1.6 (2) © | ERCOT shall accept Forced and Maintenance Outage plans from a Qualifying Facility (QF) that result from the outage of the QF's thermal host facility. | ERCOT business process | N/A | Rejected bids | All rejected DAM bids or offers and the reason and if they were accepted on resubmittal |
| FR 172 | NP.Section 3.10(7) | ERCOT shall reject any NOMCRs that are not CIM compliant. | system | Automated Report | rejected CIM | rejected CIM |
| FR 173 | NP.Section 3.14 | ERCOT shall procure Resources that are used to provide Reliability Must-Run Service or Black Start Service through Agreements. | ERCOT business process | N/A | rejected CIM | rejected CIM |
| FR 174 | NP.Section 3.10(10) | ERCOT shall notify a requestor of any deficiencies in its NOMCR. | ERCOT Business Process | Automated Report | rejected NOMCR | rejected NOMCR |
| FR 175 | NP.Section 16.11.6.2.7 | ERCOT shall take one or more of the following actions: | ERCOT business process | Manually Initiated Report | rejected NOMCR | rejected NOMCR |
| FR 176 | NP.Section 3.7.2 | If a Resource Parameter is determined to be invalid, then ERCOT shall reject it and provide written notice to the QSE of the reason for the rejection. | ERCOT Business Process | Manually Initiated Report | rejected resource | resource w/invalid/incomplete data |
| FR 177 | NP.Section 4.5.1 (4) (c) (iii) (B) | The sum of the awarded Ancillary Service capacities for each Resource must be within the Resource limits specified in COP and Section 3.18, Resource Limits in Providing Ancillary Service, and the Resource parameters as described in Section 3.7, Resource Parameters. | Monitored | Automated Report | rejected resource | resource w/invalid/incomplete data |
| FR 178 | NP.Section 8.1.2.2 (2) | Before the start of each season, a QSE shall provide ERCOT a list identifying each Controllable Load Resource that is expected to operate in a season as a provider of Ancillary Service. | Business Process - System | Automated Report | Reported controllable load expected to provide AS | Reported controllable load expected to provide AS |
| FR 179 | NP.Section 8.1.2.2 (1) | Before the start of each season, a QSE shall provide ERCOT a list identifying each Generation Resource that is expected to operate more than 168 hours in a season as a provider of energy or Ancillary Service. | Business Process - System | Automated Report | Reported units expected to run over 168 hours | Reported units expected to run over 168 hours |
| FR 180 | NP.Section 8.1.2.3.2(1) | ERCOT shall continuously monitor the capacity of each Resource to provide Responsive Reserve. | System | Manually Initiated Report | Responsive Reserve availability | report on responsive reserve quanitity available by hour by resource for a selected time period |
| FR 181 | NP.Section 8.1.2.3.3(1) | ERCOT shall continuously monitor the capacity of each Resource to provide Non-Spinning Reserve. | System | Manually Initiated Report | Responsive Reserve availability | report on responsive reserve quanitity available by hour by resource for a selected time period |
| FR 182 | NP.Section 6.5.9.1 (2) | ERCOT shall, to the fullest extent practicable, utilize the Day-Ahead process, the Adjustment Period process, and the Real-Time process before ordering Resources to specific output levels with Emergency Base Point instructions. | ERCOT business process | N/A | Responsive Reserve availability | report on responsive reserve quanitity available by hour by resource for a selected time period |
| FR 183 | NP.Section 6.5.9.1 (3) | ERCOT shall report any NERC or FERC penalties assessed for violating those constraints to Market Participants and the PUCT. | Incident report => PUCT | Manually Initiated Report | Responsive Reserve availability | report on responsive reserve quanitity available by hour by resource for a selected time period |
| FR 184 | NP.Section 4.4.8 (a) | By 1000 in the Day-Ahead, ERCOT shall submit, in ERCOT’s sole discretion, Three-Part Supply Offers based on RMR Agreement rates and any other relevant information as provided under contract on behalf of RMR Units for any RMR Units to be considered, in the DAM, DRUC, or HRUC. | ERCOT Business Process | Automated Report | RMR offers | For a time period, the date/time RMR units were offered, the date/time the offer was valid, and the offer data |
| FR 185 | NP.Section 3.1.5.3 (2) | For Outages scheduled at least three days before the scheduled start date of the proposed Outage, ERCOT shall make reasonable attempts to accommodate unusual circumstances that support TSP requests for approval earlier than required by the schedule above. | Business Process & Monitored | Manually Initiated Report | RMR offers | For a time period, the date/time RMR units were offered, the date/time the offer was valid, and the offer data |
| FR 186 | NP.Section 3.14.1(1)(i) | ERCOT shall post on the MIS Secure Area all information relative to the use of RMR Units including energy deployed monthly. | System | Automated Report | RMR usage | Report on RMR energy delivered by day and sum for month: either for all units, or by unit |
| FR 187 | NP.Section 8.5.1.2 (3) | When the governor of a Generation Resource is blocked while the Resource is operating, the QSE shall promptly inform ERCOT. | ERCOT business process | Automated Report | RMR usage | Report on RMR energy delivered by day and sum for month: either for all units, or by unit |
| FR 188 | NP.Section 3.9 (2) | ERCOT shall use the information provided in the COP to calculate the High Ancillary Service Limit (HASL) and Low Ancillary Service Limit (LASL) for each Resource for the RUC processes. | System | Manually Initiated Report | RUC Input Data | Report on date/time and input data to RUC run including unit MW, HASL, LASL, HDL, LDL, (constraints) Generation to be dispatched, Constraints (i.e. sufficient input to duplicate results) |
| FR 189 | NP.Section 6.4.6.2 (1) | The Resource must be shown as available for HRUC commitment. | Monitored | Automated Report | RUC Input Data | Report on date/time and input data to RUC run including unit MW, HASL, LASL, HDL, LDL, (constraints) Generation to be dispatched, Constraints (i.e. suffieient input to duplicate results) |
| FR 190 | NP.Section 5.5.2(5) | ERCOT shall use the RUC process to evaluate the need to commit Resources for which a QSE has submitted Three-Part Supply Offers and other available Off-Line Resources in addition to Resources that are planned to be On-Line during the RUC Study Period. | ERCOT Business Process | Automated Report | RUC runs and recommendations | Report on date/time of RUC completions, status, and recommendations |
| FR 191 | NP.Section 3.1.7.1 (3) | ERCOT shall transmit approvals electronically. | ERCOT business process | N/A | RUC runs and recommendations | Report on date/time of RUC completions, status, and recommendations |
| FR 192 | NP.Section 4.5.2 (2) | ERCOT shall procure the difference in capacity between the Day-Ahead Ancillary Service Plan and the DAM-reduced Ancillary Service Plan amounts using the DRUC from Resources that are qualified to provide the needed Ancillary Service. | System | Automated Report | RUC runs and recommendations | Report on date/time of RUC completions, status, and recommendations |
| FR 193 | NP.Section 3.1.6.9 (1) (b) | If mutual agreement is not reached, then ERCOT shall decide on the appropriate time, after considering expected impacts on system security, future Outage plans, and participants and issue a Verbal Dispatch Instruction to the Resource’s QSE to stay Off-Line. | ERCOT business process | N/A | RUC runs and recommendations | Report on date/time of RUC completions, status, and recommendations |
| FR 194 | NP.Section 6.4.8.2.3(3) | ERCOT shall monitor SASM MCPCs for errors and shall “flag” for further review questionable prices before posting and make notations in the posting if there are conditions that cause the prices to be questionable. | ERCOT Business Process | Automated Report | SASM flagged questionable prices | Report for a time period lists each SASM date/time and the flagged prices |
| FR 195 | NP.Section 3.14.2 (7) | ERCOT shall periodically determine and review the location and number of Black Start Resources required, as well as any special transmission or voice communication needs required. | ERCOT business process | Manually Initiated Report | SASM flagged questionable prices | Report for a time period lists each SASM date/time and the flagged prices |
| FR 196 | NP.Section 6.5.7.3(5)© | ERCOT shall provide the summary to Market Participants on the MIS Secure Area and to the WEMM. | System | Automated Report | SCED constraints | Report on transmission constraints used in SCED and manual over-rides |
| FR 197 | NP.Section 5.5.1 (7) | As part of the Network Security Analysis, for each hour of the RUC Study Period, ERCOT shall analyze all selected contingencies and perform the following: | ERCOT business process | N/A | SCED constraints | Report on transmission constraints used in SCED and manual over-rides |
| FR 198 | NP.Section 3.10.4(2) | During Real-Time, ERCOT shall calculate LMPs and take remedial actions to ensure that actual flow on a given Transmission Element is less than the Normal Rating and any calculated flow due to a contingency is less than the applicable Emergency Rating | System | Automated Report | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 199 | NP.Section 5.5.1(5) | ERCOT shall evaluate the need for Resource-specific deployments during Real-Time operations for management of congestion consistent with the Operating Guides. | Business Process - System | Automated Report | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 200 | NP.Section 3.15.1 (7) | If actual conditions indicate probable non-compliance of TSPs and DSPs with the requirements to provide voltage support, ERCOT shall require power factor measurements at the time of its choice while providing sufficient notice to perform the measurements. | ERCOT business process | N/A | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 201 | NP.Section 3.1.7.1 (1) | ERCOT shall approve or reject no later than: | ERCOT business process | N/A | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 202 | NP.Section 6.5.7.1.8 | Once transmission line and transformer Dynamic Ratings are retrieved, ERCOT shall compare the actual flow and state estimated flow calculation of MVA to the effective Transmission Element limit and, if an out-of-limit condition exists, ERCOT shall produce an overload notification. | System | Automated Report | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 203 | NP.Section 6.5.7.1.10(1) | Using the input provided by the State Estimator, ERCOT shall use the Network Security Analysis (NSA) processor to perform analysis of all contingencies remaining in the active list. | System | Automated Report | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 204 | NP.Section 6.5.7.1.10(1) | ERCOT shall use the NSA processor to verify Electrical Bus voltage limits to be within a percentage tolerance as outlined in the ERCOT Operating Guides. | System | Automated Report | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 205 | NP.Section 6.5.7.8 (1) | ERCOT shall issue all Resource Dispatch Instructions to the QSE that represents the affected Resource. | ERCOT business process | N/A | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 206 | NP.Section 6.5.7.8 (3) | If ERCOT believes that a Resource has inadequately responded to a Dispatch Instruction, ERCOT shall notify the QSE representing the Resource as soon as practicable. | ERCOT business process | N/A | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 207 | NP.Section 6.5.7.8 (8) | ERCOT shall record and file all electronic Dispatch Instructions and acknowledgements as soon as practicable after the issuance of the Dispatch Instruction. | ERCOT business process | N/A | Security violations | Report on pre and post-congingency security violations without RAPS or DRAPS for a given period |
| FR 208 | NP.Section 10.3.2.1.1 | ERCOT shall integrate the signals and provide a MWh value for each 15-minute interval for each virtual generator unit. | System | Manually Initiated Report | Split meter report | Integrated MWH values by virtual unit by interval |
| FR 209 | NP.Section 10.3.2.1.4 | For split-metered generating units, ERCOT shall provide for settlement the net MWh value for each 15-minute interval. | System | Manually Initiated Report | Split meter report | Integrated MWH values by virtual unit by interval |
| FR 210 | NP.Section 10.3.2.1.4 | ERCOT shall use a standard “integration” mechanism to perform this function. | System | Manually Initiated Report | Split meter report | Integrated MWH values by virtual unit by interval |
| FR 211 | NP.Section 3.9.2 (3) | If a Resource is designated in the COP to provide Ancillary Service, then ERCOT shall verify that the COP complies with Section 3.16, Standards for Determining Ancillary Service Quantities. | ERCOT business process | N/A | Split meter report | Integrated MWH values by virtual unit by interval |
| FR 212 | NP.Section 3.10.7.4.1 (1) | In making the determination to request installation of additional telemetry from a breaker or switch, ERCOT shall consider the economic implications of inaccurate representation of Model Loads in LMP results versus the cost to remedy. | ERCOT business process | N/A | Split meter report | Integrated MWH values by virtual unit by interval |
| FR 213 | NP.Section 3.10.7.4.2 (3) | In making the determination to request additional telemetry, ERCOT shall consider the economic implications of inaccurate representation of Load Models in LMP results versus the cost to remedy. | ERCOT business process | N/A | Split meter report | Integrated MWH values by virtual unit by interval |
| FR 214 | NP.Section 10.3.2.1 | When the generator unit is registered with ERCOT, the owners of the unit shall submit all required ERCOT Facility registration documentation and an ERCOT-approved splitting agreement executed by an authorized representative from each owning Entity. Such agreement shall contain a defined and fixed ownership percentage as among the owning Entities. ERCOT shall establish this generator as a “split,” essentially establishing a virtual generator meter. | ERCOT Business Process | Manually Initiated Report | Split meter report | Integrated MWH values by virtual unit by interval |
| FR 215 | NP.Section 3.10.7.3(2) | ERCOT shall post all SPSs and RAPs under consideration on the MIS Secure Area within five Business Days of receipt by ERCOT. | ERCOT Business Process | Manually Initiated Report | sps/rap | description of approved RAP & SPS |
| FR 216 | NP.Section 3.10.7.3(3) | ERCOT shall post on the MIS Secure Area all approved SPSs and RAPs at least two Business Days before implementation, identifying the date of implementation. | ERCOT Business Process | Manually Initiated Report | sps/rap | description of approved RAP & SPS |
| FR 217 | NP.Section 21.8.7 | If TAC recommends approval of a SCR, ERCOT shall prepare a TAC Recommendation Report, issue the report to the ERCOT Board and post the report on the MIS within three (3) Business Days of the TAC recommendation concerning the SCR. | ERCOT business process | Manually Initiated Report | sps/rap | description of approved RAP & SPS |
| FR 218 | NP.Section 21.8.8 | ERCOT shall issue the updated SIA (if any) to the ERCOT Board and post it on the MIS within three (3) Business Days of issuance. | ERCOT business process | Manually Initiated Report | sps/rap | description of approved RAP & SPS |
| FR 219 | NP.Section 3.10.7.4.2(5) | Within 30 days, the TSP or QSE shall take the actions necessary to correct the failure or provide ERCOT with a detailed plan with a projected timeframe to correct the failure. | ERCOT Business Process | Manually Initiated Report | Status of outstanding telemetry problems | Date problem reported to QSE/TSP; Description of problem, Date of each participant response, current status |
| FR 220 | NP.Section 6.5.9.3.3 (5) | If ERCOT issues an Alert because insufficient Ancillary Service Offers were received in the DAM, and if the Alert does not result in sufficient offer and the DAM is executed with insufficient offers, then ERCOT shall acquire the insufficient amount of Ancillary Services in the DRUC and shall issue Dispatch Instructions to the QSEs for Resources that were RUC-Committed to provide Ancillary Services, informing them of the requirement that the Resources be prepared to provide those Ancillary Services. | ERCOT business process | Manually Initiated Report | Status of outstanding telemetry problems | Date problem reported to QSE/TSP; Description of problem, Date of each participant response, current status |
| FR 221 | NP.Section 4.4.4(4) | As soon as practicable, ERCOT shall notify each QSE through the Messaging System of any of its DC Tie Schedules that are invalid DC Tie Schedules. | ERCOT Business Process | Manually Initiated Report | tag adjustments | tag adjustments |
| FR 222 | NP.Section 6.5.7.1.10(2) | When the NSA processor notifies ERCOT of a security violation, ERCOT shall immediately initiate the process described in Section 6.5.7.1.11, Transmission Constraint Management. | Business Process - System | Automated Report | TCM results | Report on TCM identified real-time violations |
| FR 223 | NP.Section 6.5.7.9 (3) | ERCOT shall resolve the conflict by issuing another Dispatch Instruction. | ERCOT business process | N/A | TCM results | Report on TCM identified real-time violations |
| FR 224 | NP.Section 3.10.7.4.1(2) | ERCOT shall measure TSP or QSE performance in providing accurate data that do not include ambiguous changes in state and shall report the performance metrics on the MIS Secure Area on a monthly basis. | System | Automated Report | Telemetry standard report | Report on telemetry standard performance |
| FR 225 | NP.Section 8.1.2.2.3 (8) | ERCOT shall confirm the date and time of the test with the QSE using both the primary and alternate voice circuits to validate the voice circuits. | Monitored | Manually Initiated Report | Telemetry standard report | Report on telemetry standard performance |
| FR 226 | NP.Section 3.10.7.4(3) | The telemetry provided to ERCOT by each TSP must be updated at a 10 second or less scan rate and be provided to ERCOT at the same rate. | Business Process - System | Manually Initiated Report | Telemetry with suspect scan rate | Report of analog telemetry whos analog value has not changed within over 10 seconds across a threshold period. |
| FR 227 | NP.Section 6.5.9.4.2 | ERCOT shall continue EECP until sufficient offers are received and deployed by ERCOT to eliminate the conditions requiring the EECP and normal SCED operations are restored. | ERCOT business process | N/A | Telemetry with suspect scan rate | Report of analog telemetry whos analog value has not changed within over 10 seconds across a threshold period. |
| FR 228 | NP.Section 3.10.4(5) | If an interim update is implemented, ERCOT shall report changes to the PUCT staff and the WEMM. | ERCOT Business Process | Manually Initiated Report | temporary configuration chage | temporary configuration chage |
| FR 229 | NP.Section 21.4.8 | For PRRs not designated Urgent, ERCOT shall review the TAC Recommendation Report and update the PIA as soon as practicable, but no later than thirty (30) days after the TAC Recommendation Report is issued, unless a longer period is warranted due to the complexity of the changes proposed by TAC. | ERCOT business process | Manually Initiated Report | temporary configuration chage | temporary configuration chage |
| FR 230 | NP.Section 13.1.1 | ERCOT shall forecast the ERCOT-wide Transmission Loss Factors expressed as a percent of Load for each settlement interval of the Operating Day. | Business Process - System | Automated Report | Transmission Loss Factors | Transmission loss factors posted by day |
| FR 231 | NP.Section 6.5.5.2 (2) | ERCOT shall make that data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to requesting TSPs and DSPs operating within ERCOT. | ERCOT Business Process | Manually Initiated Report | TSP/DSP requested data | Generation data requested by TSP/DSP |
| FR 232 | NP.Section 4.2.2 (2) | From the STWPF, ERCOT shall produce and update WGRPP forecasts each hour for each WGR to be used as input into each RUC process as per Section 5, Transmission Security Analysis and Reliability Unit Commitment. | ERCOT business process | Automated Report | TSP/DSP requested data | Generation data requested by TSP/DSP |
| FR 233 | NP.Section 4.2.2 (3) | ERCOT shall produce the WGRPP forecasts using the STWPF information provided by WGR owners including WGR availability, meteorological information, and SCADA. | ERCOT business process | Automated Report | TSP/DSP requested data | Generation data requested by TSP/DSP |
| FR 234 | NP.Section 8.1(2) | Each QSE shall meet, and shall cause each Resource that it represents to meet, performance measures as described in this Subsection and in the Operating Guides. | ERCOT Business Process | Automated Report | Unit performance | Report on units performance in following energy + regulation + Responsive; can select responsive reserve providers only |
| FR 235 | NP.Section 8.5.2.2 | ERCOT shall collect all data necessary to analyze each Measurable Event. | Business Process & Monitored | Automated Report | Unit performance | Report on units performance in following energy + regulation + Responsive; can select responsive reserve providers only |
| FR 236 | NP.Section 8.1.2.4.2(1) | When deploying any Responsive Reserve Service, the QSE shall control its Resources to operate to the Resource’s Base Point or Scheduled Power Consumption Snapshot at the time of the Dispatch Instruction plus the instructed Responsive Reserve Service power | ERCOT Business Process | Automated Report | Unit performance | Report on units performance in following energy + regulation + Responsive; can select responsive reserve providers only |
| FR 237 | NP.Section 8.1.2.4.4 | When deploying any Regulation, Responsive Reserve, or Non-Spinning Reserve Service, the QSE shall control its Resources to operate to the sum of the final Base Points plus any deployed Ancillary Service. | ERCOT Business Process | Automated Report | Unit performance | Report on units performance in following energy + regulation + Responsive; can select responsive reserve providers only |
| FR 238 | NP.Section 6.5.9.3.3 (7) | Corrective actions identified by ERCOT must be communicated through Dispatch Instructions to all TSPs, DSPs and QSEs required to implement the corrective action. | ERCOT business process | Manually Initiated Report | Unit performance | Report on units performance in following energy + regulation + Responsive; can select responsive reserve providers only |
| FR 239 | NP.Section 6.5.9.3.4 (5) | If the Emergency Condition is the result of a transmission problem that puts the ERCOT System in an unreliable condition, then ERCOT shall act immediately to return the ERCOT System to a reliable condition, including instructing Resources to change output and instructing TSPs or DSPs to drop Load. | ERCOT business process | N/A | Unit performance | Report on units performance in following energy + regulation + Responsive; can select responsive reserve providers only |
| FR 240 | NP.Section 4.4.3(3) | As soon as practicable, ERCOT shall notify the QSE through the Messaging System of any of its Self-Schedules that are invalid Self-Schedules. | system | Automated Report | Validated Self Schedules | Validated Self Schedules |
| FR 241 | NP.Section 4.4.3.2(2) | ERCOT shall continuously validate Self-Schedules and continuously display on the MIS Secure Area information that allows the QSE named in a Self-Schedule to view validated Self-Schedules. | system | Automated Report | Validated Self Schedules | Validated Self Schedules |
| FR 242 | NP.Section 3.1.6.9(1)(a) | Approve the request unless, as a result of complying with the request, ERCOT cannot maintain system reliability or security with the Resource injection. In such a case, ERCOT shall issue a Verbal Dispatch Instruction to the Resource’s QSE to stay Off-Line; or | ERCOT Business Process | Manually Initiated Report | Verbal Dispatch Instructions | Report on VDIs issued over a given time period |
| FR 243 | NP.Section 8.1.2.2.1 (2) | A QSE must be qualified and tested to provide Ancillary Service prior to initial operation and every five years thereafter. | ERCOT business process | Manually Initiated Report | Verbal Dispatch Instructions | Report on VDIs issued over a given time period |
| FR 244 | NP.Section 3.15.2(d) | Except under Force Majeure conditions, a TSP must maintain transmission system voltage within two percent of the scheduled voltage. | Business Process - System | Automated Report | Voltage outside profile | Buses with voltage outside the voltage setpoint +/- 2% |
| FR 245 | NP.Section 17.6 | ERCOT shall evaluate its system operation and market performance to identify potential areas for improvements. | ERCOT business process | N/A | Voltage outside profile | Buses with voltage outside the voltage setpoint +/- 2% |

|  | 14 |
| --- | --- |
| Description (1) ERCOT shall develop and maintain a detailed data catalog which defines the data that ERCOT collects. ERes using business inpu(3) ERCOT shall follow the existing ERCOT standard practices for data protection. a versioning system that can be used to validate the current document status. |  |

## Requirements from Other Binding Documents

No other requirements from binding documents.

# Supplementary Requirements

| Requirement ID | SR7 |
| --- | --- |
| Requirement Name | Data Access Security |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process |  |
| Sub-Process Element Coverage |  |
| Description - ERCOT shall comply with security architectural guidelines for the Nodal Market to maintain appropriate access privileges and protections for data collected under the scope of this protocol. - ERCOT shall ensure that all data requests to and resultant data sets from the Data Archive and Data Warehouse are routed through the security layer. Nodal Reference: High-Level Nodal Architecture |  |

| Requirement ID | SR8 |
| --- | --- |
| Requirement Name | Data Access Performance Requirement |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | Supplementary to Protocols |
| Coverage of Source |  |
| Traceability to Sub-Process |  |
| Sub-Process Element Coverage |  |
| Description ERCOT shall create a set of SLAs to set expectations around : - the timeliness of ERCOTs response to data requirements changes - the latency associated with availability of replicated data - the performance of extract and report requests Protocol Reference: 17.3, 17.3.1 |  |

## Performance Requirements

No performance requirements exist in the protocols or supplementary documents.  Performance characteristics will be defined by PUCT, ERCOT Business, the WEMM function, or Market Participant agreement and detailed in the detailed requirements documents for each data service, product, or groups of products.

## Confidentiality Requirements

All data services delivered by the Nodal EDW Project using the EIS data systems will follow the restrictions for protected information as set forth in the NP Section 1.3.

## Legal and Regulatory requirements

ERCOT shall conform to collection, handling, verification, and retention guidelines outlined in subsection (e) of P.U.C Subst. R. 25.362, Electric Reliability Council of Texas (ERCOT) governance.

## System and Communication Requirements

None.

## System Security Requirements

Protocols requests concerning the system security requirements are specified in requirements SR17.

## Back up and Recovery Requirements

No back up and recovery requirements exist in the protocols or supplementary documents.  Back up and recovery characteristics will be defined by PUCT, ERCOT Business, the WEMM function, or Market Participant agreement and detailed in the detailed requirements documents for each data service, product, or groups of products.

## Availability and Redundancy Requirements

No Redundancy requirements exist in the protocols or supplementary documents.  Redundancy characteristics will be defined in the EDW conceptual design in accordance with ERCOT standard disaster recovery plans.

## Maintainability Requirements

None

## Training and Documentation Requirements

Training and documentation related to compliance deliverables identified by these protocols will be developed according to existing ERCOT processes and standards and delivered with the specific Data Services.

## Usability Requirements

No usability requirements exist in the protocols or supplementary documents.  Usability characteristics will be defined by PUCT, ERCOT Business, the WEMM function, or Market Participant agreement and detailed in the detailed requirements documents for each data service, product, or groups of products.

# Protocol Coverage

| Protocol Sub-Section # (To the lowest level of granularity as possible) | Description | Coverage by Requirements (Full/Partial) |
| --- | --- | --- |

# Sub-Process Coverage

| Sub-Process ID | Sub-Process Name | Coverage by Requirements (Full/Partial) |
| --- | --- | --- |
| n/a |  |  |