IDA045

LMP Accuracy Standards

- Causes of LMP Errors

There is unlimited number of causes and variety of circumstances that are causing incorrect LMP.

  - Input Data Errors

    - Transmission network model errors

    - Transmission outage and de-rating errors

    - Hourly offers are incorrect

    - Load data errors

    - Over/under mitigation errors

  - Data Model Errors

- Mapping inconsistencies across systems/applications

  - Software Processing Errors

    - Missing LMPs, i.e. LMPs are not posted at all

    - Incorrect LMPs are posted, i.e. time intervals are incorrect

    - Singularities and degeneracy of optimization model (all units at limits while transmission line is congested, high slopes on offer price curves, redundant constraints, model degeneracy …)

    - LMPs inconsistent with optimality conditions due numerical ill-conditions and instabilities

- LMP Checking Rules

  - LMP Value Checking

    - Time stamps of LMPs must be between 4 and 6 minutes apart to identify missing time intervals

    - LMPs outside some range (e.g. below 0 and above 100) should be checked

    - LMP changes between two successive time intervals should be within some range (+- 50)

    - LMPs at last and the first time interval of an hour should be closed (cross hour LMP checking)

  - MCP Checking

    - MCP outside min/max offer price range should be checked

    - MCP changes between two successive time intervals should be within some range (+- 50)

    - MCP at last and the first time interval of an hour should be closed (cross hour LMP checking)

  - Transmission Congestion

(It is assumed that transmission constraint infeasibility is filtered out from LMPS – what is not case in current design)

    - If transmission constraint is not binding then shadow cost must be zero

    - If transmission constraint is binding then shadow cost must be non-zero

    - If all shift factors have the same sign then shadow costs should be checked

    - If all resources are at the limits at one side of the line then shadow costs should be checked

    - The optimal line power flows must be closed to operational line power flows (telemetry)

  - Optimality Conditions

    - If resource is dispatched at maximum then LMP must be higher than offer price at maximum

    - If resource is dispatched at minimum then LMP must be lower than offer price at maximum

    - If resource is dispatched within minimum and maximum then LMP must be equal to offered price at dispatched MW point

- Methods for LMP Correction

    - Set LMPs to the last good values

    - Set LMPs to average of previous and the next time interval

    - Set LMPs to As Offer Prices

    - Fix data errors and re-run software

- LMP Correction Process

- Provide input data and output result validation within market software

- Analyze save cases and reserve prices usually for all hour (without consideration of mitigation errors)

- Analyze LMPs and perform LMP corrections

- Post corrected LMPs for settlement

- Settlement to do bill corrections