| NPRR Number | 069 | NPRR Title | Changes to SURAMP |
| --- | --- | --- | --- |
| Date |  | June 21, 2007 |  |
| Submitter’s Information |  |  |  |
| Name |  | Trip Doggett on behalf of Transition Plan Task Force (TPTF) |  |
| E-mail Address |  | tdoggett@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | (512) 248-6360 |  |
| Cell Number |  | (512) 626-0275 |  |
| Market Segment |  | n/a |  |

| Overall Market Benefit |  |
| --- | --- |
| Overall Market Impact |  |
| Consumer Impact |  |

ERCOT/Market Segment Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 |  |  |
| Additional Qualitative Information | 1 |  |  |
| Other Comments | 1 |  |  |

Comments

On 6/21/07, TPTF voted to recommend approval of NPRR069 with the modifications listed below.  There was one abstention from the Investor Owned Utility Market Segment.  The Independent Generator and Independent Retail Electric Provider Market Segments were not present for the vote.

| Revision Description | This NPRR revises the Security Constrained Economic Dispatch (SCED) Up Ramp Rate (SURAMP) formulation. |
| --- | --- |

Proposed Nodal Protocol Language Revision

6.5.7.2	Resource Limit Calculator

(1)	ERCOT shall calculate the HASL, LASL, SCED Up Ramp Rate (SURAMP), SCED Down Ramp Rate (SDRAMP), HDL and LDL within 4 seconds after a change of the Resource-specific attributes provided as part of the QSE’s SCADA telemetry under Section 6.5.5.2, Operational Data Requirements.  The formulas described below define which Resource-specific attributes must be used to calculate each Resource limit.  The Resource limits are used as inputs into both the SCED process and the Ancillary Service Capacity Monitor as described in Section 6.5.7.6, Load Frequency Control.  These Resource limits help ensure that the deployments produced by the SCED and LFC processes will respect the commitment of a Resource to provide Ancillary Services as well as individual Resource physical limitations.

(2)	The figure below illustrates how the Resource Limit Calculator determines the Resource limits for both Generation and Load Resources:

LSL

HSL

Time

HSL

-

LSL

-

0

LASL

-

HASL

-

Generation

Increase

Generation

Decrease

Ancillary

services

provided: Reg

Down

Ancillary services

provided: Reg Up,

Responsive, Non

-

Spin

(as applicable)

Current

Telemetry

HDL

LDL

Ramp

Rate

5 Minutes

HSL=

-

LPC

LSL=

-

MPC

LASL

-

HASL

-

Consumption

Decrease

Consumption

Increase

Current load

Telemetry

Ancillary services

provided: Reg Down

Ancillary services

provided: Reg Up,

Responsive

, Non-Spin

Generation

5

-

30

Minutes

Load Resources

Sign Convention for Load Resources

Normal Load

fluctuation

Quantity

Offer Curve Generation

LSL

HSL

-

-

0

-

-

Generation

Increase

Generation

Decrease

-

Spin

Ramp

Rate

5 Minutes

-

-

-

-

Consumption

Decrease

Consumption

Increase

5

-

30

-

-

0

-

-

Generation

Increase

Generation

Decrease

-

Spin

Ramp

Rate

5 Minutes

-

-

-

-

Consumption

Decrease

Consumption

Increase

5

-

30

Price

(3)	High Ancillary Service Limit (HASL) is calculated as follows:

HASL	=	Max (LASL, (HSLTELEM – (RRSTELEM + RUSTELEM + NSRSTELEM)))

| Variable | Description |
| --- | --- |
| HASL | High Ancillary Service Limit |
| HSLTELEM | High Sustained Limit provided via telemetry – per Section 6.5.5.2 Operational Data Requirements |
| LASL | Low Ancillary Service Limit |
| RRSTELEM | Responsive Reserve designation provided by telemetry |
| RUSTELEM | Reg-Up designation provided by telemetry |
| NSRSTELEM | Non-Spin designation provided via telemetry; ERCOT shall set the NSRSTELEM to adjust for deployed Non-Spin on On-Line Generation Resources |

(4)	Low Ancillary Service Limit (LASL) is calculated as follows:

LASL	= 	LSLTELEM + RDSTELEM

| Variable | Description |
| --- | --- |
| LASL | Low Ancillary Service Limit |
| LSLTELEM | Low Sustained Limit provided via telemetry |
| RDSTELEM | Reg-Down designation provided by telemetry |

(5)	For each Generation Resource, the SURAMP is calculated as follows:

SURAMP	= 	 [RAMPRATE– (RUSTELEM / 5)]

| Variable | Description |
| --- | --- |
| SURAMP | SCED up ramp rate |
| RAMPRATE | = Normal Ramp Rate when RRS is not deployed or when the subject resource is not providing RRS. = Emergency Ramp Rate for Resources deploying RRS. |
| RUSTELEM | Reg-Up designation provided by telemetry |

(6)	For each Generation Resource, the SDRAMP is calculated as follows:

SDRAMP	= 	NORMRAMP – (RDSTELEM / 5)

| Variable | Description |
| --- | --- |
| SDRAMP | SCED down ramp rate |
| NORMRAMP | Normal Ramp Rate |
| RDSTELEM | Reg-Down designation by Resource provided via telemetry |

(7)	For Generation Resources, HDL is calculated as follows:

HDL	= 	Min (POWERTELEM + (SURAMP * 5), HASL)

| Variable | Description |
| --- | --- |
| HDL | High Dispatch Limit |
| POWERTELEM | Gross or net real power provided via telemetry |
| SURAMP | SCED Up Ramp Rate – |
| HASL | High Ancillary Service Limit – definition provided in Section 2, Definitions and Acronyms. |

(8)	For Generation Resources, LDL is calculated as follows:

LDL	= 	Max (POWERTELEM - (SDRAMP * 5), LASL)

| Variable | Description |
| --- | --- |
| LDL | Low Dispatch Limit |
| POWERTELEM | Gross or net real power provided via telemetry |
| SDRAMP | SCED Down Ramp Rate – |
| LASL | Low Ancillary Service Limit – definition provided in Section 2, Definitions and Acronyms. |