Verification of Resource Status, Energy Offer Curves, Output Schedules,

and Inc/Dec offers for DSRs

The current MMS implementation has COP as the basis for Output Schedule (OS)/ Energy Offer Curve (EOC) / Incremental and Decremental Energy Offer Curve (Inc/Dec) validation, i.e. the current implementation rejects OS/EOC/Inc/Dec submission if it is found inconsistent with COP data or if COP data is not available.

Market Participants were concerned about the complications that might arise due the cross validation between COP and real time operations. Based on the comments received on the Market Submission White Paper, the rejection of submission based on COP will be changed to warning messages and some additional warning messages will be added to indicate inconsistencies in telemetered Resource Status.

If the telemetered Resource Status is inconsistent with the submission of EOC, OS or Inc/Dec, then the EOC, OS, or Inc/Dec submission is used and a warning message is send to the QSE. i.e. EOC will be used by SCED if it is available even if the telemetered Resource Status is ONOS, ONOSREG, ONDSRREG or ONDSR.  The QSE is expected to correct the Resource Status to be consistent with the submission of EOC, OS or Inc/Dec.  If the Resource Status is correct and the submission is somehow incorrect, then the QSE must use the EOC, OS or Inc/Dec submission until the next valid opportunity to correct for any hours remaining in the adjustment period.

The new proposed changes are listed below

- OS for non-DSR

  - Issue warning message (changed from rejection to warning for submission and end of Adjustment Period)

    - If MW difference between 2 consecutive 5min intervals is greater than 10* SCED Up ramp rate for schedules showing an increase from the prior period, or 10*SCED Down ramp rate for schedules showing a decrease from the prior period

    - If LSL>MW or HSL<MW

    - if OS is not entered for all of the remaining five-minute intervals for which COP status is online

    - if no COP is available

    - if COP status is not either ONTEST, ONOS, ONOSREG, ONDSRREG or ONDSR for the same hour(s) of the OS

- EOC

  - Issue warning message (changed from rejection to warning for submission / added warning for 2hr ahead notification and end of Adjustment Period)

    - If COP with ONTEST, ONOS, ONOSREG, ONDSRREG or ONDSR status exists for that hour for that Resource.

    - if no COP is available

- Inc/Dec

  - Issue warning message (changed from rejection to warning for submission / added warning for 2hr ahead notification and end of Adjustment Period)

    - If COP status for that hour for that Resource is not ONDSR REG or ONDSR.

    - if no COP is available

  - Reject (added for submission)

    - If the Resource’s QSE is not qualified to have a DSR Load

- Add requirement to send “inconsistent Resource Status” warning message in Real Time

  - if the telemetered Resource Status is ONTEST, ONOS, ONOSREG, ONDSRREG or ONDSR and EOC exists for that hour for that Resource.

  - if the telemetered Resource Status is not ONDSR REG or ONDSR and Inc/Dec exists for that hour for that Resource

  - if the telemetered Resource Status is not ONTEST, ONOS, ONOSREG, ONDSRREG or ONDSR and OS exists for that hour for that Resource.