| NPRR Number | 059 | NPRR Title | Reconfiguring the Annual CRR Auction |
| --- | --- | --- | --- |
| Date |  | 06/26/07 |  |
| Submitter’s Information |  |  |  |
| Name |  | Vanessa Spells on behalf of Credit WG |  |
| E-mail Address |  | vspells@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512 225 7029 |  |

| Overall Market Benefit | This NPRR will redefine the annual CRR auction so that it will be solved in a reasonable amount of time with available software and hardware. |
| --- | --- |
| Overall Market Impact | Not known. |
| Consumer Impact | Not known. |

| ERCOT/Market Segment Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 | The process and technology for running six concurrent optimizations is feasible. |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | 24 month CRR products will no longer be available. Multiple bids and awards will be required. |  |
|  | 2 | 24 Hour CRR products will no longer be available. Multiple bids and awards will be required. |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Feasible Annual CRR Auction |  |
| Additional Qualitative Information | 1 | None. |  |
| Other | 1 | None. |  |
| Comments | 2 |  |  |

Comments

ERCOT credit staff and the Credit Working Group (Credit WG) have reviewed NPRR 059 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. However, the group believes there may eventually be a need to break down credit limits for each separate optimization for each Time of Use block.

Revised Proposed Nodal Protocol Language

7.3	Types of Congestion Revenue Rights to Be Auctioned

(1)	ERCOT shall auction the following types of CRRs:

(a)	PTP Options;

(b)	PTP Obligations; and

(c) 	FGRs that are offered by CRR Account Holders.

(2)	PTP Options are evaluated hourly in each CRR Auction as the positive power flows on all directional network elements created by the injection and withdrawal at the specified source and sink points in the quantity represented by the CRR bid or offer (MW), excluding all negative flows on all directional network elements.

(3)	PTP Obligations are evaluated hourly in each CRR Auction as the positive and negative power flows on all directional network elements created by the injection and withdrawal at the specified source and sink points of the quantity represented by the CRR bid or offer (MW).

(4)	PTP Options can only result in payments from ERCOT to the CRR Owner of record.  A PTP Obligation may result in either a payment or a charge to the CRR Owner of record.

(5)	FGRs are evaluated in each CRR Auction as the positive power flows represented by the quantity of the CRR bid or offer (MW) on a flowgate, (i.e., predefined directional network element or a predefined bundle of directional network elements).The flowgates on which FGRs are offered by ERCOT are specified in Section 7.3.1.2, Defined Flowgates.

(6)	CRRs must be auctioned in the following Time Of Use (TOU) blocks (having the same MW amount for each hour within the block):

(a)	5x16 blocks for hours ending 0700-2200, Monday through Friday (excluding NERC holidays), in one-month strips;

(b)	2x16 blocks for hours ending 0700-2200, Saturday and Sunday, and NERC holidays in one-month strips;

(c)	7x8 blocks for hours ending 0100-0600 and hours ending 2300-2400 Sunday through Saturday, in one-month strips; and

(7)	The CRR blocks described in paragraph (6) above must be auctioned simultaneously in the annual CRR Auctions, in which capacity is made available for the next two years.

(8)	CRR Auction Bids and PCRR nominations must specify a TOU block.

7.5	CRR Auctions

7.5.1	Nature and Timing

(1)	The CRR Auction auctions the available network capacity of the ERCOT Transmission System not allocated as described in Section 7.4, Allocation of Preassigned Congestion Revenue Rights and in Section 7.7.3, Allocation of McCamey Flowgate Rights (MCFRIs), or sold in a previous auction.  The CRR Auction also allows CRR Owners an opportunity to offer for sale CRRs that they hold.  Each annual and monthly CRR Auction allows for the purchase of CRR products as described in Section 7.3, Types of Congestion Revenue Rights to Be Auctioned, paragraph (6) in one-month strips and allows for the reconfiguration of all CRR blocks that were previously awarded.  Monthly CRR Auctions will include products for the next month only.

(2)	The CRR Network Model must be based on, but is not the same as, the Network Operations Model.  The CRR Network Model must, to the extent practicable, include the same topology, contingencies, and operating procedures as used in the Network Operations Model as reasonably expected to be in place for each month.  The expected network topology used in the CRR Network Model for any month must include all outages from the Outage Scheduler and identified by ERCOT staff as expected to have a significant impact upon transfer capability during the month.  These outages included in the CRR Network Model shall be posted on the MIS Secure Area consistent with model posting requirements by ERCOT with accompanying cause and duration information, as indicated in the Outage Scheduler. Transmission system upgrades and changes must be accounted for in the CRR Network Model for CRR Auctions held after the month in which the element is placed into service.

(a)	ERCOT shall use Dynamic Ratings in the CRR Network Model as required under Section 3.10.8, Dynamic Ratings.

(b)	The CRR Network Model must use the peak Load conditions of the month being modeled.

(c)	ERCOT’s criteria for determining if an Outage should be in the CRR Network Model shall be in accordance with these Protocols and described in the ERCOT Operating Guides.

(3)	ERCOT shall model bids and offers into the CRR Auction as flows based on the MW offer and defined source and sink.  When the Simultaneous Feasibility Test (SFT) is run, the model must weight the Electrical Buses and Hub Buses included in a Hub or Load Zone appropriately to determine the system impacts of the CRRs.

(a)	To distribute injections and withdrawals to buses within a Hub, ERCOT shall use distribution factors specified in Section 3.5.2, Hub Definitions.

(b)	To distribute injections and withdrawals to Electrical Buses in Load Zones, ERCOT shall use the Load-weighted distribution factors for On-Peak Hours in each Load Zone from the planning cases (for the same period) for monthly CRR Auctions (or for the monthly models used in an annual CRR Auction).  If monthly planning cases do not exist, ERCOT shall use the Load-weighted distribution factors for On-Peak Hours in each Load Zone from the appropriate seasonal planning case.

(4)	ERCOT shall conduct CRR Auctions with the frequency, on the dates, and for the terms specified as follows:

(a)	PTP Options, PTP Obligations, and MCFRIs in monthly auctions for one-month terms beginning with the month prior to the Texas Nodal Market Implementation Date;

(b)	ERCOT shall conduct a monthly CRR Auction during the month preceding  the month during which the CRRs being auctioned are effective. ERCOT shall publish a calendar of relevant auction dates each year for the following year’s activities.

(d)	Six monthly CRR Auctions must be completed prior to initiation of the first annual CRR Auction.  If six monthly CRR Auctions are completed prior to October 1, then CRR Options, MCFRIs and Board-approved PTP Obligations will be auctioned for the balance of the current calendar year.

(e)	After the completion of at least six monthly CRR Auctions ERCOT shall conduct an annual CRR Auction for CRR Options, MCFRIs and Board-approved PTP Obligations commencing  during October for the two-year period that starts on the immediately following January 1.

(5)	ERCOT shall auction the following products:

(a)	In each monthly CRR Auction:  one-month strips of PTP Options, PTP Obligations, and MCFRIs; and

(b)	In each annual CRR Auction:

(i)	PTP Options in one-month strips, any specified consecutive monthly strips within the same calendar year, and annual strips;

(ii)	PTP Obligations in one-month strips for one-month terms until the ERCOT Board approves the offering of PTP Obligations for specified source Settlement Points and sink Settlement Points for terms longer than one month; and

(iii)	MCFRIs in one-month strips, any specified consecutive monthly strips within the same calendar year, and annual strips.

(6)	ERCOT shall offer network capacity for two years in each annual CRR Auction equal to the difference between (a) and (b) :

(a)	For each month, the expected network topology for that month of the first year in the CRR Network Model scaled down to 55% for the first year and 15% for the second year; and

(b)	All outstanding CRRs that were previously awarded or allocated for the corresponding months in both years.

(7)	ERCOT shall offer network capacity for the monthly CRR Auction equal to the difference between:

(a)	The expected transmission network topology in the CRR Network Model of the month for which the CRRs are effective scaled down to 90%; and

(b)	All outstanding CRRs that were previously awarded or allocated for the month.