Texas Nodal

Commercial Operations Systems

Real-Time Energy Settlements Requirements

Version 1.11

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 08/15/2006 | 0.09 | Initial draft to TPTF | Mandy Bauld John S.H. Adams |
| 09/08/2006 | 0.10 | Incorporated TPTF Comments | Mandy Bauld John S.H. Adams |
| 09/12/2006 | 1.0 | TPTF Approved | Mandy Bauld John S.H. Adams |
| 03/29/2007 | 1.01 | Updated for NPRR044, NPRR035 and findings in Use Case preparation | John Bieltz |
| 04/24/2007 | 1.02 | Corrected section 2.9.2 language which had switched Over and Under scenarios listed incorrectly. | Nile Scott |
| 05/24/2007 | 1.1 | Reviewed at TPTF | John Bieltz |
| 05/29/2007 | 1.11 | Updated for changes made to the scope section and updated some Protocol references for other NPRRs. | John Bieltz |

Table of Contents

1.	Real-Time Operations Energy Settlement	5

1.1	Proposed System Scope	5

1.2	Document Conventions	6

1.3	Document Assumptions	6

2.	Functional Requirements	7

2.1	ERCOT Total Real-Time Adjusted Metered Load (RTAMLTOT)	7

Assumptions	7

2.2	QSE Load Ratio Share for a 15-Minute Settlement Interval (LRS)	7

Assumptions	8

2.3	QSE Load Ratio Share for an Operating Hour (HLRS)	8

Assumptions	9

2.4	Real-Time Energy Imbalance Payment or Charge (RTEIAMT)	9

Input Bill Determinants	15

Output Bill Determinants	18

Data Dependencies	18

Assumptions	21

Business Defined Errors	24

Specific Database Needs	25

2.5	Real-Time Energy Payment for DC Tie Import (RTDCIMPAMT)	26

Input Bill Determinants	26

Output Bill Determinants	27

Data Dependencies	28

Assumptions	29

Business Defined Errors	30

Specific Database Needs	30

2.6	Real-Time Payment for a Block Load Transfer Point (BLTRAMT)	31

Input Bill Determinants	32

Output Bill Determinants	32

Data Dependencies	33

Assumptions	33

Business Defined Errors	35

Specific Database Needs	35

2.7	Real-Time Energy Charge for DC Tie Export Represented by the QSE Under the Oklaunion Exemption (RTDCEXPAMT)	35

Input Bill Determinants	36

Output Bill Determinants	36

Data Dependencies	37

Assumptions	38

Business Defined Errors	40

Specific Database Needs	40

2.8	Real-Time Congestion Payment or Charge for Self-Schedules (RTCCAMT)	40

Input Bill Determinants	41

Output Bill Determinants	41

Data Dependencies	42

Assumptions	43

Business Defined Errors	44

Specific Database Needs	45

2.9	Base Point Deviation Charge (BPDAMT)	45

Input Bill Determinants	48

Output Bill Determinants	50

Data Dependencies	50

Assumptions	50

Business Defined Errors	52

Specific Database Needs	52

2.10	Base Point Deviation Payment (LABPDAMT)	52

Input Bill Determinants	53

Intermediate Bill Determinants	53

Output Bill Determinants	54

Data Dependencies	54

Assumptions	54

Business Defined Errors	55

Specific Database Needs	55

2.11	Real-Time Revenue Neutrality Allocation (LARTRNAMT)	55

Input Bill Determinants	56

Output Bill Determinants	59

Data Dependencies	59

Assumptions	60

Business Defined Errors	63

Specific Database Needs	63

2.12	ERCOT System Administration Charge (ESACAMT)	64

Input Bill Determinants	64

Output Bill Determinants	64

Data Dependencies	65

Assumptions	65

Business Defined Errors	66

Specific Database Needs	66

3.	Supplementary Requirements	67

3.1	Performance Requirements	67

3.2	Legal and Regulatory requirements	67

3.3	System Security Requirements	67

3.4	Back up and Recovery Requirements	67

3.5	Availability and Redundancy Requirements	67

3.6	Maintainability Requirements	67

3.7	Daylight Savings Time shall be recognized in Nodal Market settlements.	67

4.	Protocol Coverage	68

Software Requirements Specification

# Real-Time Operations Energy Settlement

## Proposed System Scope

ERCOT plans to implement a new Nodal market in 2009.  Sections 6 and 9 of the Nodal Protocols describe the various Charge Types and Settlement Statement requirements for the Real-Time Operations (RT).  The Nodal Protocols will guide this project.

This document defines the requirements for Real-Time Operations energy settlements.  The requirements in scope for this document include calculation of energy-related RT settlement Charge Types that are

- detailed in Nodal Protocols Section 6 (approved by the Board on 12/28/2006), and

- listed as a component of the RTM Settlement Statement in Nodal Protocols Section 9 (approved by the Board on 12/28/2006).

- included in NPRR033 Settlements of CRRs When DAM Does Not Execute (approved by the Board on 12/14/2006, 033NPRR-06 BOD Action Report 121206)

- included in NPRR035 Nodal Protocol Clarifications Required for Net Metering Provisions (035NPRR-10 PRS Recommendation Report 022207 dated 02/22/2007 and updated by 035NPRR-09 ERCOT Staff Comments 050907 dated 05/09/2007)

- included in NPRR044 Corrections to Section 6, Adjustment Period and Real-Time Operations (044NPRR-02 PRS Recommendation Report 032207 dated 03/26/2007)

- included in NPRR052 Settlement for Non-Modeled Generators (052NPRR-02 PRS Recommendation Report 032207 dated 03/27/07)

The following Charge Types are addressed in this document:

- Real-Time Energy Imbalance Payment or Charge at a Resource Node, Load Zone, or Hub (NP §6.6.3.1, NP §6.6.3.2, NP §6.6.3.3, NP §9.5.3 (h))

- Real-Time Energy Payment for DC-Tie Import (NP §6.6.3.4, NP §9.5.3 (i))

- Real-Time Payment for a Block Load Transfer Point (NP §6.6.3.5,  NP §9.5.3 (j))

- Real-Time Energy Charge for DC Tie Export represented by the QSE under Oklaunion Exemption (NP §6.6.3.6,  NP §9.5.3 (k))

- Real-Time Congestion Payment or Charge for Self-Schedules (NP §6.6.4,  NP §9.5.3 (l))

- Base Point Deviation Charge for Over, Under, and Intermittent Renewable Resource (IRR) Generation ( NP §6.6.5.1.1, NP §6.6.5.1.2, NP §6.6.5.2 and  NPRR007 §9.5.3(m) )

- Base Point Deviation Payment ( NP §6.6.5.4 and  NPRR007 §9.5.3 (n) )

- Real-Time Revenue Neutrality Allocation (NP §6.6.10,  NP §9.5.3 (aa))

- ERCOT System Administration Charge (NP §9.10.1,  NP §9.5.3 (rr))

This document also defines requirements for the calculation of the following global bill determinants:

- Real-Time Adjusted Metered Load Total (NP §6.6.2.1)

- Load Ratio Share (NP §6.6.2.2)

- Hourly Load Ratio Share (NP § 6.6.2.3)

The Figures provided within this document are for illustrative purposes.  The figures should be considered in conjunction with the textual requirements.  If a contradiction exists between an illustrative figure and its associated textual requirements, the textual requirements should override the figure.

## Document Conventions

- All text lifted from the Nodal Protocols is in italic font.

- All dependencies are identified using D1, D2, etc.

- All assumptions are identified using A1, A2, etc.

- The original source system is defined for each input bill determinant.  It is important to note that the format defined for the input bill determinant is the format for which the data should exist in the Settlement System, after any interface functionality.  The bill determinant may or may not exist in the original source system in the format defined for the bill determinant.  A separate set of business requirements will address data transformations and interfacing between the original source system and the Settlement System.

## Document Assumptions

- All Real-Time Settlement Point Prices (RTSPP) will be calculated outside of the Settlement System and therefore the RTSPP calculations are not detailed within this document.

- The original source system for the Settlement Point Prices, or the relevant data archive, will maintain a history of the prices.  Therefore, if a RTSPP is corrected after a settlement run, the price that was used for the settlement run is still available.

- Any charge type that requires historical data will obtain this information via alternative means, which will be defined in the Detail Design phase, during the beginning of market open until the data is available.

- Modifications to the RT Energy Settlements to accommodate net metering are not included within this business requirement document at this time.  When the necessary net metering settlement modifications are approved by TPTF they will be incorporated into this requirements document via an addendum.

- The Settlement System will have the information to know if a Settlement Point is a Resource Node, Load Zone, Hub, or DC Tie.

- The requirements for modification to the Load Allocated Real-Time Revenue Neutrality Amount Charge Type due to DAM not executing are not included in this business requirements document.  Once determined, these requirements will be incorporated into this requirements document via an addendum.

- The charge type calculations are documented using the proposed recorder format for the bill determinants.  The following list defines the nomenclature used for the recorders (not all of these bill determinant recorder elements may apply to the charge types within this document):

<Q> = QSE

<CO> = CRR Owner

<R> = Resource

<SP> = Settlement Point

< BLTP> = Block Load Transfer Point

<SRSP> = Source Settlement Point

<SKSP> = Sink Settlement Point

<ST> = Start Type

<F> = Flowgate

<E> = Directional Network Element

<C> = Constraint Identifier

<M> = Market

<RUC> = RUC Process

<NMC> = Net Meter Agreement

<ME>  = Settlement Meter at a Net Metering Facility

<B>  = An Electrical Bus

# Functional Requirements

## ERCOT Total Real-Time Adjusted Metered Load (RTAMLTOT)

ERCOT total Adjusted Metered Load represents the total Adjusted Metered Load across ERCOT, as measured in Real-Time, excluding DC Tie exports associated with QSEs under the “Oklaunion Exemption.”  (PR 6.6.2.1) The RTAMLTOT value is calculated ERCOT-wide by Operating Day and 15-Minute Interval.

The calculation of RTAMLTOT will be performed in the Data Aggregation system and will be passed to the Settlement System.  Detailed requirements for this calculation are included in the Data Aggregation requirements document.  Data Aggregation will calculate the RTAMLTOT per Protocols, using the following calculation:

RTAMLTOT	=	ΣQΣSPRTAML_<Q>_<SP>

The RTAMLTOT is not required by the Settlement System in order to perform any settlement calculations; it is required by the Settlement System for the purpose of validation and inclusion in the data extracts.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR1 | PR 6.6.2.1 | CSWM. P03SettleBill Transfer | Data Aggregation calculates RTAMLTOT for each Operating Day and makes it available to the Settlement System prior to initiation of the Real-Time Settlements process, in the format of a 15-minute interval data cut for each Operating Day. |
| FR2 | PR 6.6.2.1 | CSWM. P03SettleBill Transfer | The RTAMLTOT bill determinant provided to the Settlement System will retain as many decimal places as supported by the Settlement System, to be determined in design. |
| FR3 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following ERCOT-wide bill determinant should be identified within the Settlement System as necessary for the public data extracts: - RTAMLTOT |

### Assumptions

A1)  The Data Aggregation system will create the interval data cut for RTAMLTOT; the Settlement System interface will not have to create this bill determinant interval data cut.

A2)  The Data Aggregation system will not provide an RTAMLTOT data cut with null interval values.

A3) Any Block Load Transfer Load quantity within a Load Zone Settlement Point will already be reflected in the RTAML value for that Settlement Point, and therefore will be included in the RTAMLTOT value.

A4)  The RTAML data used by the source system to calculate RTAMLTOT will exclude DC Tie exports associated with the QSEs under the “Oklaunion Exemption” received.

## QSE Load Ratio Share for a 15-Minute Settlement Interval (LRS)

Load Ratio Share is the ratio of an Entity’s Adjusted Metered Load to total ERCOT Adjusted Metered Load for an interval.  (PR 2.1)  For the purpose of calculating LRS, the Entity is a QSE and the interval is a 15-minute settlement interval.

The calculation of LRS will be performed in the Data Aggregation system and will be passed to the Settlement System.  Detailed requirements for this calculation are included in the Data Aggregation requirements document.  Data Aggregation will calculate the LRS per Protocols, using the following calculation:

LRS_<Q>		=	(ΣSPRTAML_<Q>_<SP>) / RTAMLTOT

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR4 | PR 6.6.2.2 | CSWM. P03SettleBill Transfer | Data Aggregation calculates LRS for each Operating Day and QSE with RTAML data and makes it available to the Settlement System prior to initiation of the Real-Time Settlements process, in the format of a 15-minute interval data cut for each QSE and Operating Day. |
| FR5 | PR 6.6.2.2 | CSWM. P03SettleBill Transfer | The LRS bill determinant provided to the Settlement System will retain as many decimal places as supported by the Settlement System, to be determined in design. |
| FR6 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following ERCOT-wide bill determinant should be identified within the Settlement System as necessary for the private data extracts: - LRS |

Data Dependencies

D1)  The Base Point Deviation Payment (LABPDAMT) is dependent upon the calculation of LRS.

D2)  The Voltage Support Charge (LAVSSAMT) is dependent upon the calculation of LRS.

D3)  The Charge for Emergency Power Increases (LAEMREAMT) is dependent upon the calculation of LRS.

D4)  The Real-Time Revenue Neutrality Allocation (LARTRNAMT) is dependent upon the calculation of LRS.

### Assumptions

A1)  The Data Aggregation system will create the interval data cut for LRS; the Settlement System interface will not have to create this bill determinant interval data cut.

A2)  The Data Aggregation system will not provide an LRS data cut with null interval values.

A3)  Data Aggregation calculates RTAML for every active QSE, and therefore LRS will be calculated for every active QSE.

## QSE Load Ratio Share for an Operating Hour (HLRS)

Load Ratio Share is the ratio of an Entity’s Adjusted Metered Load to total ERCOT Adjusted Metered Load for an interval.  (PR 2.1)  For the purpose of calculating HLRS, the Entity is a QSE and the interval is an hourly settlement interval.  In order to obtain an hourly LRS value, the input bill determinants are summed across the 15-minute Settlement Intervals within an Operating Hour.

The calculation of HLRS will be performed in the Data Aggregation system and will be passed to the Settlement System.  Detailed requirements for this calculation are included in the Data Aggregation requirements document.  Data Aggregation will calculate the HLRS per Protocols, using the following calculation:

HLRS_<Q>	=	(Σ4i=1ΣSPRTAML_<Q>_<SP>i) / Σ4i=1RTAMLTOT i

Where,

i = 15-minute Settlement Interval in the Operating Hour

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR7 | PR 6.6.2.3 | CSWM. P03SettleBill Transfer | Data Aggregation calculates HLRS for each Operating Day and QSE with RTAML data and makes it available to the Settlement System prior to initiation of the Real-Time Settlements process, in the format of an hourly interval data cut for each QSE and Operating Day. |
| FR8 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The LRS bill determinant provided to the Settlement System will retain as many decimal places as supported by the Settlement System, to be determined in design. |
| FR9 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following ERCOT-wide bill determinant should be identified within the Settlement System as necessary for the private data extracts: - HLRS |

Data Dependencies

D1)  The RMR Service Charge (LARMRAMT) is dependent upon the calculation of HLRS.

D2)  The Black Start Capacity Charge (LABSSAMT) is dependent upon the calculation of HLRS.

D3)  The Adjustments to Cost Allocations for Ancillary Services Procurement (RUONET, RDONET, RRONET, and NSONET) is dependent upon the calculation of HLRS.

### Assumptions

A1)  The Data Aggregation system will create the interval data cut for HLRS; the Settlement System interface will not have to create this bill determinant interval data cut.

A2)  The Data Aggregation system will not provide an HLRS data cut with null interval values.

A3)  Data Aggregation calculates RTAML for every active QSE, and therefore HLRS will be calculated for every active QSE.

## Real-Time Energy Imbalance Payment or Charge (RTEIAMT)

The ‘Real-Time Energy Imbalance Payment or Charge’ Charge Type should be calculated for Resource Node, Load Zone, and Hub Settlement Points.  The QSE is charged or paid at the Real-Time Settlement Point Price based upon the difference between the injection and withdrawal energy quantities at the Settlement Point.  Assuming a positive Settlement Point Price, excess energy injections at the Settlement Point will result in a payment to the QSE, while excess energy withdrawals will result in a charge to the QSE.  The Charge Type calculation varies slightly for each type of Settlement Point, but the Charge Type will be reflected as one item on the RTM Settlement Statement.

RTEIAMT for a Resource Node Settlement Point (PR 6.6.3.1 (1)):

(1) The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time SPP for the following amounts at a particular Resource Node Settlement Point:

(a)	The energy produced by all its Generation Resources at the Settlement Point; plus

(b)	The amount of its Self-Schedules with sink specified at the Settlement Point; plus

(c)	The amount of its Energy Bids cleared in the DAM at the Settlement Point; plus

(d)	The amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(e)	The amount of its Self-Schedules with source specified at the Settlement Point; minus

(f)	The amount of its Energy Offers cleared in the DAM at the Settlement Point; minus

(g)	The amount of its Energy Trades at the Settlement Point where the QSE is the seller

(2) The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time SPP for the following amounts at a particular Resource Node Settlement Point if the Generation Resources at the Resource Node Settlement Point p are involved with a net metering scheme:

Assuming a positive RTSPP:

Figure 4.1: Real-Time Energy Imbalance at a Resource Node with a Net Metering Agreement

| Requirement Id | FR 10 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.1 (2), PR 6.6.3.1 (3), PR 6.6.3.1 (4) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the Real-Time Energy Imbalance Payment or Charge by QSE and a Resource Node Settlement Point for each 15-Minute Settlement Interval in the Operating Day. RTEIAMT_<Q>_<SP> = (-1)* ((ΣR (GSPLITPER_<R>) * NMSAMTTOT) + (RTSPP_<SP> * { (SSSK_<Q>_<SP>* ¼) + (DAEP_<Q>_<SP> * ¼) + (RTQQEP_<Q>_<SP> * ¼) - (SSSR_<Q>_<SP> * ¼) – (DAES_<Q>_<SP> * ¼) – (RTQQES_<Q>_<SP> * ¼)})) a) If NMRTETOT_<NMC> > 0 Then: NMSAMTTOT_<NMC> = ΣbΣme(RTRMPR_<NMC>_<B> * MEB_<NMC>_<ME>) Else: The meter result will already be included in the Load Zone Imbalance calculation. b) GSPLITPER_<R> = GSSPLITSCA_<R>_<NMC> / ΣRGSSPLITSCA_<R>_<NMC> |  |

RTEIAMT for a Load Zone Settlement Point (PR 6.6.3.2 (1)):

The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time SPP for the following amounts at a particular Load Zone Settlement Point:

(a) 	The amount of its Self-Schedules with sink specified at the Settlement Point; plus

(b) 	The amount of its Energy Bids cleared in the DAM at the Settlement Point; plus

(c) 	The amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(d) 	The amount of its Self-Schedules with source specified at the Settlement Point; minus

(e) 	The amount of its Energy Offers cleared in the DAM at the Settlement Point; minus

(f) 	The amount of its Energy Trades at the Settlement Point where the QSE is the seller; minus

(g) 	The Adjusted Metered Load at the Settlement Point, plus

(h)          The aggregated generation of its Non-Modeled Generators in the Load Zone.

Assuming a positive RTSPP:

Figure 5.0: real Time Energy Imbalance at a Load Zone

| Requirement Id | FR 11 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.2 (2) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the Real-Time Energy Imbalance Payment or Charge by QSE and a Load Zone Settlement Point for each 15-Minute Settlement Interval in the Operating Day. RTEIAMT_<Q>_<SP> = (-1) * RTSPP_<SP> * {(SSSK_<Q>_<SP>* ¼) + (DAEP_<Q>_<SP> * ¼) + (RTQQEP_<Q>_<SP> * ¼) – (SSSR_<Q>_<SP> * ¼) – (DAES_<Q>_<SP> * ¼) – (RTQQES_<Q>_<SP> * ¼) – RTAML_<Q>_<SP> + RTMGNM_<Q>_<SP>} |  |

RTEIAMT for a Hub Settlement Point (PR 6.6.3.3 (1)):

The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time SPP for the following amounts at a particular Hub Settlement Point:

(a) 	The amount of its Self-Schedules with sink specified at the Settlement Point; plus

(b) 	The amount of its Energy Bids cleared in the DAM at the Settlement Point; plus

(c) 	The amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(d) 	The amount of its Self-Schedules with source specified at the Settlement Point; minus

(e) 	The amount of its Energy Offers cleared in the DAM at the Settlement Point; minus

(f) 	The amount of its Energy Trades at the Settlement Point where the QSE is the seller.

Assuming a positive RTSPP:

Figure 6.0: Real-Time Energy Imbalance at a Hub

| Requirement Id | FR 12 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.3 (2) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the Real-Time Energy Imbalance Payment or Charge by QSE and a Hub Settlement Point for each 15-Minute Settlement Interval in the Operating Day. RTEIAMT_<Q>_<SP> = (-1) * RTSPP_<SP> * {(SSSK_<Q>_<SP>* ¼) + (DAEP_<Q>_<SP> * ¼) + (RTQQEP_<Q>_<SP> * ¼) – (SSSR_<Q>_<SP> * ¼) – (DAES_<Q>_<SP> * ¼) – (RTQQES_<Q>_<SP> * ¼) } |  |

| Requirement Id | FR 13 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.1 (3), PR 6.6.3.2 (3), PR 6.6.3.3 (3) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total of RTEIAMT across all Settlement Points for each QSE and for each 15-Minute Interval in the Operating Day. RTEIAMTQSETOT_<Q> = ΣSPRTEIAMT_<Q>_<SP> |  |

| Requirement Id | FR 14 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.10 (3) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total of RTEIAMT across all QSEs and all Settlement Points for each 15-Minute Interval in the Operating Day. RTEIAMTTOT = ΣQ RTEIAMTQSETOT_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point Price per Settlement Point - The Real-Time Settlement Point Price at the Settlement Point, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTMG | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource - The Real-Time energy produced by the Generation Resource represented by the QSE at the Resource Node, for the 15-minute Settlement Interval. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: The RTMG value should be used when calculating RTEIAMT for a Resource Node Settlement Point. The RTMG value will not include any Block Load Transfer quantity. |  |  |  |
| RTAML | MWh | Real-Time Adjusted Metered Load per QSE per Settlement Point—The sum of the Adjusted Metered Load at the Electrical Buses that are included in the Settlement Point represented by the QSE for the 15-minute Settlement Interval. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: The RTAML value should be used when calculating RTEIAMT for a Load Zone Settlement Point. Any BLT load quantity within a Load Zone will already be included in the RTAML for that Load Zone. |  |  |  |
| SSSK | MW | Self-Schedule with Sink at Settlement Point per QSE per Settlement Point – The QSE’s Self-Schedule with sink at the Settlement Point, for the 15-Minute Settlement Interval | MMS |
| Interval Frequency: 4 / hour Additional Comments: Comments: Per PR 4.4.3.1, Self-Schedules will be reported as a MW value for each 15-minute interval. The SSSK value should contain the value as it exists at the end of the Adjustment Period for the interval. This represents the total for all of the QSE’s Self-Schedules with this Settlement Point as the Sink. |  |  |  |
| DAEP | MW | Day-Ahead Energy Purchase per QSE per Settlement Point – The QSE’s Energy Bids at the Settlement Point cleared in the DAM, for the Hour that includes the 15-Minute Settlement Interval | MMS |
| Interval Frequency: 1 / hour Additional Comments: The value used in this RT Settlement should be the same value that was used for Day-Ahead settlement. See TN.COMS.63C01.DAMENERGY.REQUIREMENTS.A for further details on this bill determinant. |  |  |  |
| RTQQEP | MW | Real-Time QSE-to-QSE Energy Purchase per QSE per Settlement point – the amount of MW bought by the QSE through Energy Trades at the Settlement Point, for the 15-Minute Settlement Interval | MMS |
| Interval Frequency: 4 / hour Additional Comments: Per PR 4.4.2.1, Energy Trades will be reported as a MW value for each 15-minute interval. The RTQQEP value should contain the value as it exists by 1430 in the day following the Operating Day. (PR 4.4.2 (3)) |  |  |  |
| SSSR | MW | Self-Schedule with Source at Settlement Point per QSE per Settlement Point – The QSE’s Self-Schedule with source at the Settlement Point, for the 15-Minute Settlement Interval | MMS |
| Interval Frequency: 4 / hour Additional Comments: Comments: Per PR 4.4.3.1, Self-Schedules will be reported as a MW value for each 15-minute interval. The SSSR value should contain the value as it exists at the end of the Adjustment Period for the interval. This represents the total for all of the QSE’s Self-Schedules with this Settlement Point as the Source. |  |  |  |
| DAES | MW | Day-Ahead Energy Sale per QSE per Settlement Point – The QSE’s Energy Offers at the Settlement Point cleared in the DAM, for the Hour that includes the 15-Minute Settlement Interval | MMS |
| Interval Frequency: 1 / hour Additional Comments: The value used in this RT Settlement should be the same value that was used for Day-Ahead Settlement. See TN.COMS.63C01.DAMENERGY.REQUIREMENTS.A for further details on this bill determinant. |  |  |  |
| RTQQES | MW | Real-Time QSE-to-QSE Energy Sales per QSE per Settlement point – the amount of MW Sold by the QSE through Energy Trades at the Settlement Point, for the 15-Minute Settlement Interval | MMS |
| Interval Frequency: 4 / hour Additional Comments: Per PR 4.4.2.1, Energy Trades will be reported as a MW value for each 15-minute interval. The RTQQES value should contain the value as it exists by 1430 in the day following the Operating Day. (PR 4.4.2 (3)) |  |  |  |
| NMRTETOT | MWh | Net Meter Real-Time Energy Total— The net sum for all settlement meters me include in net metering arrangement a. A positive value indicates an injection of power to the ERCOT system and a negative value indicates an withdrawal of power from the ERCOT system. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: None. |  |  |  |
| RTRMPR | $/MWh | Real-Time Price for the Energy Metered for each resource meter at busThe Real-Time Price for the settlement meter at Electrical Bus b, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None. |  |  |  |
| MEB | MWh | Metered Energy at busThe metered energy by the settlement meter me for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy consumed. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: None. |  |  |  |
| GSSPLITSCA | MWh | Generation Resource SCADA Net Real Power Provided via Telemetry—The net real power provided via telemetry per resource within the net metering arrangement, integrated for the 15-minute Settlement Interval. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: None. |  |  |  |
| RTMGNM | MWh | Real-Time Metered Generation from Non-Modeled generators per QSE per Settlement Point—The total Real-Time energy produced by Non-Modeled Generators represented by QSE q in Load Zone Settlement Point p, for the 15-minute Settlement Interval. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: None. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RTEIAMT | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to the QSE for the Real-Time Energy Imbalance at the Settlement Point, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTEIBILLAMT | $ | Real-Time Energy Imbalance Bill Amount per QSE - The difference between two consecutive daily summed settlement runs of the total RTEIAMT for the QSE and for the Operating Day in the Real Time market. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |
| RTEIAMTQSETOT | $ | Real-Time Energy Imbalance Amount QSE Total per QSE— The total net payments and charges to the QSE for Real-Time Energy Imbalance at all Settlement Points for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTEIAMTTOT | $ | Real-Time Energy Imbalance Amount Total The Total net payments and charges for Real-Time Energy Imbalance at all Settlement Points (Resource, Load Zone, or Hub) for the 15-Minute Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| NMSAMTTOT | $ | Net Metering Settlement Payment—The total payment to the entire facility with a net metering arrangement. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| GSPLITPER | None | Generation Resource SCADA Splitting Percentage—The generation allocation percentage for resource r that is part of a net metering arrangement. GSPLITPER is calculated by taking the SCADA values (GSSPLITSCA) for a particular Generation Resource r that is part of a net metering configuration and dividing by the sum of all SCADA values for all resources that are included in the net metering configuration for each interval. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Data Dependencies

D1) Calculation of the RTEIAMT is dependent upon availability of the input bill determinants.

D2) Calculation of RTEIAMTQSETOT for a QSE is dependent upon calculation of RTEIAMT for the QSE.

D3) Calculation of RTEIAMTTOT is dependent upon calculation of RTEIAMTQSETOT for all appropriate QSEs.

D4) Calculation of the RT Revenue Neutrality Allocation is dependent upon the calculation of RTEIAMTTOT.

D5) An Energy Trade may be submitted for any Settlement Interval within an Operating Day before 1430 of the following day. (PR 4.4.2 (3))  Initiation of the Real-Time Energy Imbalance calculation process has a dependency on data that will be available after 1430 of the day after the Operating Day.

Figure 7.0: Real-Time Energy Imbalance Data Dependencies

### Assumptions

A1) The DAES and DAEP data will be available in the Settlement System prior to Real-Time settlement because it will have been used for Day-Ahead settlement of the same Operating Day.

A2)	The Data Aggregation system will create the interval data cut for RTMG.  An interval data cut will be created for all Resources (by QSE, Resource, and Settlement Point), even if it contains all zero values. The RTMG bill determinant values will be validated by Data Aggregation prior to being interface to the Settlement System.  RTMG will not include any generation related to Block Load Transfer Resource quantities.

A3) 	Data Aggregation calculates RTAML for every active QSE, and therefore LRS will be calculated for every active QSE.The RTAML bill determinant values will be validated by Data Aggregation prior to being interfaced to the Settlement System.  Any Block Load Transfer Load quantity within a Load Zone will already be reflected in the RTAML value for that Load Zone Settlement Point.

A4)	The RTQQES and RTQQEP values used by the interface to create the RTQQES and RTQQEP bill determinant data cuts will be validated by the original source system and as submitted by 1430 in the day following the Operating Day. (PR 4.4.2 (3))  MMS will not accept this information after 1430 of the day following the Operating Day.

A5)  The SSSK and SSSR values used by the interface to create the SSSK and SSSR bill determinant data cuts will be validated by the original source system.

A6)  When creating the RTSPP data cut for a given Settlement Point and Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given 15-minute interval of the RTSPP data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of RTSPP data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no RTSPP data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A7)  When creating the SSSK data cut for a Settlement Point, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Settlement Point.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create an SSSK data cut for that Settlement Point.

A8)  When creating the SSSR data cut for a Settlement Point, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Settlement Point.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create an SSSR data cut for that Settlement Point.

A9)  When creating the DAEP data cut for a Settlement Point, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Settlement Point.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create a DAEP data cut for that Settlement Point.

A10)  When creating the DAES data cut for a Settlement Point, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Settlement Point.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create an DAES data cut for that Settlement Point.

A11)  When creating the RTQQEP data cut for a Settlement Point, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Settlement Point.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create an RTQQEP data cut for that Settlement Point.

A12)  When creating the RTQQES data cut for a Settlement Point, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Settlement Point.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create an RTQQES data cut for that Settlement Point.

A13) When creating the RTMGNM data cut for a Settlement Point, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Settlement Point.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create an RTMGNM data cut for that Settlement Point.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR15 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | None of the input bill determinants should be rounded or truncated. |
| FR16 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | All of the output bill determinants should be rounded to two (2) decimal places. |
| FR17 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following QSE specific bill determinants should be identified within the Settlement System as bill determinants that should be included in the private data extracts: - RTMG - RTAML - SSSK - SSSR - DAEP - DAES - RTQQEP - RTQQES - RTEIAMT - RTEIBILLAMT - RTEIAMTQSETOT - NPTELEM - NMSAMTTOT - MEB - GSPLITPER |
| FR18 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following ERCOT-wide bill determinant should be identified within the Settlement System as a bill determinant that should be included in the public data extracts: - RTSPP - RTEIAMTTOT |
| FR19 | PR 6.6.3.1 (2), PR 6.6.3.2 (2), PR 6.6.3.3 (2) | CSWM. P03SettleBill Transfer | DRIVER: RTEIAMT should be calculated for a QSE and Settlement Point that has at least one of the following data cuts available for an Operating Day: - RTMG - RTAML - SSSK - SSSR - DAEP - DAES - RTQQEP - RTQQES - RTMGNM |
| FR20 | PR 9.5.6 (6), PR 9.5.8 (3), PR 9.5.5 (2) | CSWM. P03SettleBill Transfer | Calculate RTEIBILLAMT for the QSE as the QSE’s Operating Day total RTEIAMT for the latter settlement run less the QSE’s Operating Day total RTEIAMT for the most previous settlement run. |
| FR21 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | If an RTMG data cut is not available for the QSE, Resource Node Settlement Point, and Operating Day for which another driver data element exists, the system should use a default value of zero (0) for each interval and should create a warning error message stating the QSE, Resource Node, Settlement Point, and Operating Day and the missing data element. |
| FR22 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | If an RTAML data cut is not available for the QSE, Load Zone Settlement Point, and Operating Day for which another driver data element exists, the system should use a default value of zero (0) for each interval and should create a warning error message stating the QSE, Resource Node, Settlement Point, and Operating Day and the missing data element. |
| FR23 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | If an SSSK data cut is not available for the QSE, Settlement Point, and Operating Day for which a driver data element exists, the system should use a default value of zero (0) for each interval. No error message is necessary. |
| FR24 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | If a DAEP data cut is not available for the QSE, Settlement Point, and Operating Day for which another driver data element exists, the system should use a default value of zero (0) for each interval. No error message is necessary. |
| FR25 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | If a RTQQEP data cut is not available for the QSE, Settlement Point, and Operating Day for which another driver data element exists, the system should use a default value of zero (0) for each interval. No error message is necessary. |
| FR26 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | If a SSSR data cut is not available for the QSE, Settlement Point, and Operating Day for which another driver data element exists, the system should use a default value of zero (0) for each interval. No error message is necessary. |
| FR27 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | If a DAES data cut is not available for the QSE, Settlement Point, and Operating Day for which another driver data element exists, the system should use a default value of zero (0) for each interval. No error message is necessary. |
| FR28 | PR 6.6.3.1 (2) | CSWM. P03SettleBill Transfer | If a RTQQES data cut is not available for the QSE, Settlement Point, and Operating Day for which a driver data element exists, the system should use a default value of zero (0) for each interval. No error message is necessary. |
| FR29 | PR 6.6.3.1 (3), PR 6.6.3.2 (3), PR 6.6.3.3 (3) | CSWM. P03SettleBill Transfer | RTEIAMTQSETOT should be calculated for a QSE that has at least one RTEIAMT data cut available for an Operating Day. If RTEIAMT data is not available for a QSE, then an RTEIAMTQSETOT data cut should not be created for the QSE. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR30 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | RTEIAMTTOT should be calculated for every Operating Day. If RTEIAMT data is not available for the Operating Day, the RTEIAMTTOT data should be created with default values of zero and the system should produce a “WARN-DEFAULT” error message indicating that no RTEIAMT is available for the Operating Day. |
| FR31 | PR 6.6.3.1 (2), PR 6.6.3.2 (2), PR 6.6.3.3 (2) | CSWM. P03SettleBill Transfer | If a RTSPP data cut is not available for a Settlement Point and Operating Day for which a driver data element exists, the system should log a “CRITICAL” error message and stop processing energy settlements for the Operating Day. This means that any calculation that depends on the RTSPP and any downstream calculation should be stopped. The error message should capture the missing data element, including the Settlement Point, and the Operating Day. |
| FR143 | PR 6.6.3.1 (2), (3) | CSWM. P03SettleBill Transfer | If a NMSAMTTOT data cut is not available for a Settlement Point and Operating Day for which a driver data element exists, the system should log a “CRITICAL” error message and stop processing energy settlements for the Operating Day. This means that any calculation that depends on the NMSAMTTOT and any downstream calculation should be stopped. The error message should capture the missing data element, including the Settlement Point, and the Operating Day. |
| FR144 | PR 6.6.3.1 (2), (4) | CSWM. P03SettleBill Transfer | If a GSPLITPER data cut is not available for a Settlement Point and Operating Day for which a driver data element exists, the system should log a “CRITICAL” error message and stop processing energy settlements for the Operating Day. This means that any calculation that depends on the GSPLITPER and any downstream calculation should be stopped. The error message should capture the missing data element, including the Settlement Point, and the Operating Day. |
| FR145 | PR 6.6.3.2 (2) | CSWM. P03SettleBill Transfer | If an RTMGNM data cut is not available for the QSE, Settlement Point, and Operating Day for which a driver data element exists, the system should use a default value of zero (0) for each interval. No error message is necessary. |

### Specific Database Needs

None

## Real-Time Energy Payment for DC Tie Import (RTDCIMPAMT)

The payment to each QSE for energy imported into the ERCOT System through each DC Tie is calculated based on the Real-Time Settlement Point Price at the DC Tie Settlement Point.  (PR 6.6.3.4 (1)) The DC Tie import value is the aggregation of DC Tie Schedules submitted by the QSE at that DC Tie Settlement Point.

| Requirement Id | FR32 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.4 (1) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RTDCIMPAMT for a QSE, DC Tie Settlement Point, Operating Day, and 15-minute interval. RTDCIMPAMT_<Q>_<SP> = (-1) * RTSPP_<SP> * (RTDCIMP_<Q>_<SP> * ¼) |  |

| Requirement Id | FR33 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.4 (2) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total payment to a QSE for energy imported into the ERCOT System through all DC Tie Settlement Points, for the Operating Day and 15-minute interval. RTDCIMPAMTQSETOT_<Q> = ΣSPRTDCIMPAMT_<Q>_<SP> |  |

| Requirement Id | FR 34 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.10 (3) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total payment to of the RTDCIMPAMT payments for the Operating Day and 15-minute interval. RTDCIMPAMTTOT = ΣQ RTDCIMPAMTQSETOT_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point Price per Settlement Point - The Real-Time Settlement Point Price at the Settlement Point, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTDCIMP | MW | Real-Time DC Import per QSE per Settlement Point—The aggregated DC Tie Schedule submitted by the QSE as an importer into the ERCOT System through the DC Tie Settlement Point, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: RTDCIMP represents the aggregation, by 15-minute interval, of all DC Tie imports, as scheduled, at that DC Tie Settlement Point unless the schedule changes during the Operating Period. ERCOT shall use the changed schedule if there is one. The original source system will contain all versions of the schedule including the final schedule. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RTDCIMPAMT | $ | Real-Time DC Import Amount per QSE per Settlement Point —The payment to the QSE for DC Tie import through the DC Tie, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTDCIMPAMT-QSETOT | $ | Real-Time DC Import Amount QSE Total per QSEThe total of the payments to the QSE for energy imported into the ERCOT System through DC Ties for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTDCIMPAMTTOT | $ | Real-Time DC Import Amount Total—The summation of payments for DC Tie import, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTDCIMP-BILLAMT | $ | Real-Time DC Import Bill Amount per QSE  The difference between two consecutive daily summed settlement runs of the total RTDCIMPAMT for the QSE and for the Operating Day in the Real Time market. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1) Calculation of the RTDCIMPAMT is dependent upon availability of the input bill determinants.

D2) The calculation of RTDCIMPAMTQSETOT is dependent upon the calculation of RTDCIMPAMT for a QSE.

D3) The calculation of RTDCIMPAMTTOT is dependent upon the calculation of RTDCIMPAMTQSETOT for all appropriate QSEs.

D4) The calculation of LARTRNAMT is dependent upon the calculation of RTDCIMPAMTTOT.

Figure 8.0: Real-Time Energy Payment for DC Tie Import Data Dependencies

### Assumptions

A1)  When creating the RTSPP data cut for a given Settlement Point and Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given 15-minute interval of the RTSPP data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of RTSPP data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no RTSPP data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A2)  MMS will have the appropriate values for RTDCIMP.  If the DC Tie Schedule flows as planned, then ERCOT shall use schedules as the deemed meter readings for Real-Time settlement.  If the interconnected non-ERCOT Control Area schedule changes during the Operating Period, then ERCOT shall use the changed interconnected non-ERCOT Control Area schedule as the deemed meter reading for Real-Time settlement. (PR 4.4.4.2 (2))   If the DC Tie Schedule for import flows as planned, then the actual data from MMS will be the original schedule.  If the schedule changed during the Operating Period, then the value from MMS will be the altered schedule.

A3)  The settlement system interface will create the RTDCIMP interval data cut using the RTDCIMP data from MMS.  When creating the interval data cut, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for the QSE and Settlement Point.

A4)  A DC Tie Schedule must be transaction-specific, i.e., one schedule per transaction per DC Tie, rather than aggregate (net) schedules per DC Tie. (PR 4.4.4 (1))  When the interface creates the RTDCIMP data cut it will obtain the aggregate of all the individual import transactions for the QSE, DC Tie, and Operating Day and create a 15-minute interval data cut.

A5) The interface will only create and provide an RTDCIMP interval data cut for DC Ties with import transactions in the original source system.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR35 | PR 6.6.3.4 | CSWM. P03SettleBill Transfer | The Real-Time Payment for DC Import Charge Type calculation is only performed for DC Tie Settlement Points. |
| FR36 | PR 6.6.3.4 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. |
| FR37 | PR 6.6.3.4 | CSWM. P03SettleBill Transfer | All of the output bill determinants should be rounded to two (2) decimal places. |
| FR38 | PR 9.5.6 (6), PR 9.5.8 (3), PR 9.5.5 (2) | CSWM. P03SettleBill Transfer | Calculate RTDCIMPBILLAMT for the QSE as the QSE’s Operating Day total RTDCIMPAMT for the latter settlement run less the QSE’s Operating Day total RTDCIMPAMT for the most previous settlement run. |
| FR39 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following QSE specific bill determinants should be identified within the Settlement System as bill determinants that should be included in the private data extracts: - RTDCIMP - RTDCIMPAMT - RTDCIMPAMTQSETOT - RTDCIMPBILLAMT |
| FR40 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following ERCOT-wide bill determinants should be identified within the Settlement System as bill determinants that should be included in the public data extracts: - RTSPP - RTDCIMPAMTTOT |
| FR41 | PR 6.6.3.4 | CSWM. P03SettleBill Transfer | DRIVER: RTDCIMPAMT should be calculated for a QSE and DC Tie Settlement Point that has an RTDCIMP data cut available for a Settlement Point and Operating Day. |
| FR42 | PR 6.6.3.4 | CSWM. P03SettleBill Transfer | RTDCIMPAMTQSETOT should be calculated for a QSE that has at least one RTDCIMPAMT data cut available for the Operating Day. |
| FR43 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | RTDCIMPAMTTOT should be calculated for every Operating Day. |
| FR44 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTDCIMPAMTQSETOT data is not available for the Operating Day, the RTDCIMPAMTTOT data cut should be created with default values of zero. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR45 | PR 6.6.3.4 (1) | CSWM. P03SettleBill Transfer | If a RTSPP data cut is not available for a Settlement Point and Operating Day for which a driver data element exists, the system should log a “CRITICAL” error message and stop processing energy settlements for the Operating Day. This means that any calculation that depends on the RTSPP and any downstream calculation should be stopped. The error message should capture the missing data element, including the Settlement Point, and the Operating Day. |

### Specific Database Needs

None

## Real-Time Payment for a Block Load Transfer Point (BLTRAMT)

The payment for the energy delivered to an ERCOT Load through a BLT Point is made to the QSE for that BLT Point for each 15-minute Settlement Interval, based on the Real-Time Settlement Point Price for the Load Zone Settlement Point where the ERCOT Load normally locates.  (PR 6.6.3.5 (1))

When load that is normally served by ERCOT is instead picked up as load by a non-ERCOT Control Area via a Block Load Transfer, ERCOT will pay the QSE that represents the non-ERCOT Control Area and provides the generation.  The Block Load Transfer quantity (BLTR) will be associated to a Block Load Transfer Point (BLTP).   A BLTP is not a type of ERCOT defined Settlement Points (i.e. it is not a type of Resource Node, Load Zone, Hub, or DC Tie Settlement Point); it is a unique point that will be defined upon the occurrence of the Block Load Transfer event.  However, the BLTP will map to the defined ERCOT Load Zone Settlement Point where the load normally locates.  The payment is calculated using the Load Zone Settlement Point Price for the Load Zone Settlement Point associated to the BLTP.

| Requirement Id | FR46 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.5 (1) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the BLTRAMT for a QSE, Load Zone Settlement Point, and BLT Settlement Point for an Operating Day, and 15-minute Settlement Interval. BLTRAMT _<Q>_<BLTP> _<SP>= (-1) * RTSPP_<SP> * BLTR_<Q>_<BLTP>_<SP> Where <SP> is the Load Zone Settlement Point where the load normally locates and the BLTP has been configured within the Settlement System. |  |

| Requirement Id | FR47 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.5 (2) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total of the payments to each QSE for energy delivered to ERCOT Loads through BLT Points for an Operating Day and 15-minute Settlement Interval. BLTRAMTQSETOT_<Q> = ΣSP ΣBLTPBLTRAMT _<Q>_<BLTP>_<SP> Where <SP> is the Load Zone Settlement Point where the load normally locates and the BLTP has been configured within the Settlement System. |  |

| Requirement Id | FR48 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.10 (3) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total of the payments for energy delivered into the ERCOT System through BLT Points for the Operating Day and 15-minute Settlement Interval. BLTRAMTTOT = ΣQ BLTRAMTQSETOT_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point Price per Settlement Point - The Real-Time Settlement Point Price at the Settlement Point, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| BLTR | MWh | Block Load Transfer Resource per QSE per Settlement Point per BLT Point—The energy delivered to an ERCOT Load in the Load Zone Settlement Point through the BLT Point represented by the QSE, for the 15-minute Settlement Interval. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: This is the validated BLT Resource quantity that exists at the end of all Adjustment Periods (i.e. at the end of the Operating Day) for the QSE and Operating Day. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| BLTRAMT | $ | Block Load Transfer Resource Amount per QSE per Settlement Point per BLT Point—The payment to the QSE for the BLT Resource that delivers energy to the Load Zone through the BLT Point, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| BLTRAMTQSETOT | $ | Block Load Transfer Resource Amount QSE Total per QSEThe total of the payments to the QSE for energy delivered into the ERCOT System through BLT Points for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| BLTRAMTTOT | $ | Block Load Transfer Resource Amount TotalThe total of the payments for energy delivered into the ERCOT System through BLT Points for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| BLTRBILLAMT | $ | Block Load Transfer Resource Bill Amount per QSE - The difference between two consecutive daily summed settlement runs of the total BLTRAMT for the QSE and for the Operating Day in the Real Time market. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1) Calculation of the BLTRAMT is dependent upon availability of the input bill determinants.

D2) The calculation of BLTRAMTQSETOT is dependent upon the calculation of BLTRAMT for a QSE.

D3) The calculation of BLTRAMTTOT is dependent upon the calculation of BLTRAMTQSETOT for all appropriate QSEs.

D4) Calculation of BLTRAMT is dependent upon successful configuration of the Block Load Transfer Point within the Settlement System.

Figure 9.0: Real-Time Payment for a Block Load Transfer Point

### Assumptions

A1) When creating the RTSPP data cut for a given Settlement Point and Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given 15-minute interval of the RTSPP data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of RTSPP data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no RTSPP data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A2)  The Data Aggregation system will create the BLTR data cut that includes the BLTP and the associated Load Zone Settlement Point as part of the naming convention; the Settlement System interface will not have to create the data cut.  The interval data cut will not contain any null values.

A3)  When a Block Load Transfer event occurs, the BLTP will be defined and will be configured in the Settlement System prior to running the RT Settlements process. Configuration of the Block Load Transfer Point includes, but is not limited to, the name of the BLTP and a mapping to the associated Load Zone Settlement Point.

A4)  Data Aggregation will only create a BLTR data cut for Settlements if there is at least one non-zero BLTR value for the Operating Day.

A5)  The TDSP meter data for a Block Load Transfer may not be available until after the initial settlement run for the Operating Day.  Therefore, BLTRAMT may not be settled on the initial settlement run for an Operating Day.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR49 | PR 6.6.3.5 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. |
| FR50 | PR 6.6.3.5 | CSWM. P03SettleBill Transfer | All of the output bill determinants should be rounded to two (2) decimal places. |
| FR51 | PR 9.5.6 (6), PR 9.5.8 (3), PR 9.5.5 (2) | CSWM. P03SettleBill Transfer | Calculate BLTRBILLAMT for the QSE as the QSE’s Operating Day total BLTRAMT for the latter settlement run less the QSE’s Operating Day total BLTRAMT for the most previous settlement run. |
| FR52 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following QSE specific bill determinants should be identified within the Settlement System as bill determinants that should be included in the private data extracts: - BLTR - BLTRAMT - BLTRAMTQSETOT - BLTRBILLAMT |
| FR53 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following ERCOT-wide bill determinants should be identified within the Settlement System as bill determinants that should be included in the public data extracts: - RTSPP - BLTRAMTTOT |
| FR54 | PR 6.6.3.5 | CSWM. P03SettleBill Transfer | DRIVER: BLTRAMT should be calculated for a QSE, Settlement Point, and BLT Point that has a BLTR data cut available for the Settlement Point and BLT Point for the Operating Day. |
| FR55 | PR 6.6.3.5 | CSWM. P03SettleBill Transfer | BLTRAMTQSETOT should be calculated for a QSE that has at least one BLTRAMT data cut available for the Settlement Point and BLT Point for the Operating Day. |
| FR56 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | BLTRAMTTOT should be calculated for every Operating Day. |
| FR57 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If BLTRAMTQSETOT data is not available for the Operating Day, the BLTRAMTTOT data cut should be created with default values of zero. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR58 | PR 6.6.3.5 (1) | CSWM. P03SettleBill Transfer | If a RTSPP data cut is not available for a Settlement Point and Operating Day for which a driver data element exists, the system should log a “CRITICAL” error message and stop processing energy settlements for the Operating Day. This means that any calculation that depends on the RTSPP and any downstream calculation should be stopped. The error message should capture the missing data element, including the Settlement Point, and the Operating Day. |

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR59 | PR 6.6.3.5 (1) | CSWM. P03SettleBill Transfer | The Settlement System must be able to store and maintain Block Load Transfer Points and the relationship between each BLTP and a Load Zone Settlement Point. |

## Real-Time Energy Charge for DC Tie Export Represented by the QSE Under the Oklaunion Exemption (RTDCEXPAMT)

The charge to a QSE that is exporting energy from the ERCOT System under the “Oklaunion Exemption” through a DC Tie associated with the exemption is calculated based on the Real-Time Settlement Point Price at the DC Tie Settlement Point. (PR 6.6.3.6 (1)) The DC Tie export value used for settlements (RTDCEXP) is the aggregation of the individual DC Tie Schedules submitted by the QSE at that DC Tie Settlement Point.

| Requirement Id | FR60 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.6 (1) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate RTDCEXPAMT for a QSE, DC Tie Settlement Point, Operating Day, and 15-minute interval. RTDCEXPAMT_<Q>_<SP> = RTSPP_<SP> * (RTDCEXP_<Q>_<SP> * ¼) |  |

| Requirement Id | FR61 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.3.6 (2) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total of the RTDCEXPAMT charges to the QSE for the Operating Day and 15-minute interval. RTDCEXPAMTQSETOT_<Q> = ΣSPRTDCEXPAMT_<Q>_<SP> |  |

| Requirement Id | FR62 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.10 (3) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total of all RTDCEXPAMT charges to the market for the Operating Day and 15-minute interval. RTDCEXPAMTTOT = ΣQ RTDCEXPAMTQSETOT_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point Price per Settlement Point - The Real-Time Settlement Point Price at the Settlement Point, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTDCEXP | MW | Real-Time DC Export per QSE per Settlement Point—The aggregated DC Tie Schedule through the DC Tie Settlement Point submitted by the QSE that is under the “Oklaunion Exemption” as an exporter from the ERCOT area, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: This is the validated DC Tie Schedule for exports that exists at the end of all Adjustment Periods (i.e. at the end of the Operating Day) for the QSE and Operating Day. Additional Comments: RTDCEXP represents the aggregation, by 15-minute interval, of all DC Tie imports, as scheduled, at that DC Tie Settlement Point, unless the schedule changes during the Operating Period. ERCOT shall use the changed schedule if there is one. The original source system will contain the all versions of the schedule including the final schedule. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RTDCEXPAMT | $ | Real-Time DC Export Amount per QSE per Settlement Point—The charge to the QSE for the DC Tie exports through the DC Tie Settlement Point, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTDCEXPAMT-QSETOT | $ | Real-Time DC Export Amount QSE Total per QSEThe total of the charges to the QSE for energy exported from the ERCOT System through DC Ties for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTDCEXPAMTTOT | $ | Real-Time DC Export Amount Total—The summation of charges to all QSEs that are under the “Oklaunion Exemption” for DC Tie export, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTDCEXPBILL-AMT | $ | Real-Time DC Export Bill Amount per QSE - The difference between two consecutive daily summed settlement runs of the total RTDCAMT for the QSE and for the Operating Day in the Real Time market. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1) Calculation of the RTDCEXPAMT is dependent upon availability of the input bill determinants.

D2) The calculation of RTDCEXPAMTQSETOT is dependent upon the calculation of RTDCEXPAMT for a QSE.

D3) The calculation of RTDCEXPAMTTOT is dependent upon the calculation of RTDCEXPAMT for all QSEs.

Figure 10.0: Real-Time Energy Charge for DC Tie Export Represented

by the QSE Under the Oklaunion Exemption

### Assumptions

A1) When creating the RTSPP data cut for a given Settlement Point and Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given 15-minute interval of the RTSPP data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of RTSPP data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no RTSPP data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A2)  MMS will have the appropriate value for RTDCEXP.  If the DC Tie Schedule flows as planned, then ERCOT shall use schedules as the deemed meter readings for Real-Time settlement.  If the interconnected non-ERCOT Control Area schedule changes during the Operating Period, then ERCOT shall use the changed interconnected non-ERCOT Control Area schedule as the deemed meter reading for Real-Time settlement. (PR 4.4.4.2 (2))   If the DC Tie Schedule for export flows as planned, then the actual from MMS will be the original schedule.  If the schedule changed during the Operating Period, then the value from MMS will be the altered schedule.

A3)  The settlement system interface will create the RTDCEXP interval data cut using the RTDCEXP data from MMS.  When creating the interval data cut, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for the QSE and Settlement Point

A4)  A DC Tie Schedule must be transaction-specific, i.e., one schedule per transaction per DC Tie, rather than aggregate (net) schedules per DC Tie. (PR 4.4.4 (1))  When the interface system creates the RTDCEXP data cut it will aggregate all of the individual import transactions for the QSE, DC Tie, and Operating Day by 15-minute interval.

A5) The interface will only create and provide an RTDCEXP interval data cut for DC Ties with export transactions in the original source system.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR63 | PR 6.6.3.4 | CSWM. P03SettleBill Transfer | The Real-Time Energy Charge for DC Export Represented by the QSE Under the Oklaunion Exemption Charge Type calculation is only performed for DC Tie Settlement Points. |
| FR64 | PR 6.6.3.6 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. |
| FR65 | PR 6.6.3.6 | CSWM. P03SettleBill Transfer | All of the output bill determinants should be rounded to two (2) decimal places. |
| FR66 | PR 9.5.6 (6), PR 9.5.8 (3), PR 9.5.5 (2) | CSWM. P03SettleBill Transfer | Calculate RTDCEXPBILLAMT for the QSE as the QSE’s Operating Day total RTDCEXPAMT for the latter settlement run less the QSE’s Operating Day total RTDCEXPAMT for the most previous settlement run. |
| FR67 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following QSE specific bill determinants should be identified within the Settlement System as bill determinants that should be included in the private data extracts: - RTDCEXP - RTDCEXPAMT - RTDCEXPAMTQSETOT - RTDCEXPBILLAMT |
| FR68 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following ERCOT-wide bill determinants should be identified within the Settlement System as bill determinants that should be included in the public data extracts: - RTSPP - RTDCEXPAMTTOT |
| FR69 | PR 6.6.3.6 | CSWM. P03SettleBill Transfer | DRIVER: RTDCEXPAMT should be calculated for a QSE and Settlement Point that has an RTDCEXP data cut available for that Settlement Point, for the Operating Day. |
| FR70 | PR 6.6.3.6 | CSWM. P03SettleBill Transfer | RTDCEXPAMTQSETOT should be calculated for a QSE that has at least one RTDCEXPAMT data cut available for the Operating Day. |
| FR71 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | RTDCEXPAMTTOT should be calculated for every Operating Day. |
| FR72 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTDCEXPAMT data is not available for the Operating Day, the RTDCEXPAMTTOT data cut should be created with default values of zero. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR73 | PR 6.6.3.5 (1) | CSWM. P03SettleBill Transfer | If a RTSPP data cut is not available for a Settlement Point and Operating Day for which a driver data element exists, the system should log a “CRITICAL” error message and stop processing energy settlements for the Operating Day. This means that any calculation that depends on the RTSPP and any downstream calculation should be stopped. The error message should capture the missing data element, including the Settlement Point, and the Operating Day. |
| FR74 | PR 4.4.4.3 (3) | CSWM. P03SettleBill Transfer | If the sum of the RTDCEXP across all QSEs for a given Settlement Point is greater than the RTMG for the Oklaunion Resource then a warn error message should be created. |

### Specific Database Needs

None

## Real-Time Congestion Payment or Charge for Self-Schedules (RTCCAMT)

The congestion payment or charge to each QSE submitting a Self-Schedule calculated based on the difference in Real-Time SPPs at the specified sink and the source of the Self-Schedule multiplied by the amount of the Self-Schedule.  (PR 6.6.4(1))

| Requirement Id | FR75 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.4 (1) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate RTCCAMT by QSE, Source and Sink Settlement Point pair, Operating Day, and 15-minute interval, for all QSEs with a Self-Schedule for the Source and Sink Settlement Point pair. RTCCAMT_<Q>_<SRSP>_<SKSP> = (RTSPP_<SKSP> - RTSPP_<SRSP>) * (SSQ_<Q>_<SRSP>_<SKSP> * ¼) |  |

| Requirement Id | FR76 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.4 (2) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total RTCCAMT for a QSE, Operating Day, and 15-minute interval. RTCCAMTQSETOT _<Q> = ΣSRSPΣSKSPRTCCAMT_<Q>_<SRSP>_<SKSP> |  |

| Requirement Id | FR77 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.10 (3) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the total RTCCAMT to the market for the Operating Day and 15-minute interval. RTCCAMTTOT = ΣQ RTCCAMTQSETOT _<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RTSPP (Sink) | $/MWh | Real-Time Settlement Point Price per Settlement Point - The Real-Time Settlement Point Price at the sink Settlement Point, for the 15-minute Settlement Interval, for the self-schedule. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTSPP (Source) | $/MWh | Real-Time Settlement Point Price per Settlement Point - The Real-Time Settlement Point Price at the source Settlement Point, for the 15-minute Settlement Interval, for the self-schedule. | MMS |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| SSQ | MW | Self-Schedule Quantity per Self-Schedule—The QSE’s Self Schedule MW quantity for the Self-Schedule, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: This is the validated Self-Schedule at the end of all Adjustment Periods (i.e. at the end of the Operating Day) for the QSE and Operating Day. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| RTCCAMT | $ | Real-Time Congestion Cost Amount per QSE per Self-Schedule—The congestion charge to the QSE for its Self-Schedule, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTCCAMTQSETOT | $ | Real-Time Congestion Cost Amount QSE Total per QSEThe total net congestion payments and charges to the QSE for its Self-Schedules for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTCCAMTTOT | $ | Real-Time Energy Congestion Cost Amount TotalThe total net congestion payments and charges for all of the Self-Schedules for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| RTCCBILLAMT | $ | Real-Time Congestion Cost Amount per QSE - The difference between two consecutive daily summed settlement runs of the total RTCCAMT for the QSE and for the Operating Day in the Real Time market. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1) Calculation of the RTCCAMT is dependent upon availability of the input bill determinants.

D2) The calculation of RTCCAMTQSETOT for a QSE is dependent upon the calculation of RTCCAMT for all Self-Schedules for that QSE.

D3) The calculation of RTCCAMTTOT is dependent upon the calculation of RTCCAMT for all QSEs.

D4) The Real-Time Revenue Neutrality Allocation Charge Type (LARTRNAMT) is dependent upon calculation of RTCCAMTTOT.

Figure 11.0: Real-Time Congestion Payment or Charge for Self-Schedules

### Assumptions

A1)  When creating the SSQ data cut for a QSE and source and sink Settlement Point pair, the interface will substitute a 0 value for a null value only if at least one interval within the Operating Day has a non-zero quantity for that Settlement Point pair.  If there are no intervals with a non-zero quantity for the entire Operating Day, then the interface will not create an SSQ data cut for that Settlement Point pair and Operating Day.

A2)  When creating the SSQ data cut for a QSE and source and sink Settlement Point pair, the interface will use the Self-Schedule values that exist in MMS at the end of the Adjustment Period for each interval of the Operating Day.  The interface will aggregate all SSQ data available for the same QSE, interval, and source and sink Settlement Point pair.

A3) When creating the RTSPP data cut for a given Settlement Point and Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given 15-minute interval of the RTSPP data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of RTSPP data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no RTSPP data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A4)  A Self Schedule contains a unique combination of the name of the QSE, the source Settlement Point and a sink Settlement Point. Therefore, ERCOT will identify the Self-schedule by each unique source / sink point combinations for the calculation of this Charge Type.

Business Rules

| Req. ID |  | Protocol(s) |  | Sub-Process Map |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| FR78 |  | PR 6.6.3.6 |  | CSWM. P03SettleBill Transfer |  | The input bill determinants should not be rounded or truncated. |
| FR79 |  | PR 6.6.3.6 |  | CSWM. P03SettleBill Transfer |  | All of the output bill determinants should be rounded to two (2) decimal places. |
| FR80 | PR 9.5.6 (6), PR 9.5.8 (3), PR 9.5.5 (2) |  | CSWM. P03SettleBill Transfer |  | Calculate RTCCBILLAMT for the QSE as the QSE’s Operating Day total RTCCAMT for the latter settlement run less the QSE’s Operating Day total RTCCAMT for the most previous settlement run. |  |
| FR81 |  | PR 12.3 (C) |  | CSWM. P03SettleBill Transfer |  | The following QSE specific bill determinants should be identified within the Settlement System as bill determinants that should be included in the private data extracts: - SSQ - RTCCAMT - RTCCAMTQSETOT - RTCCBILLAMT |
| FR82 |  | PR 12.3 (C) |  | CSWM. P03SettleBill Transfer |  | The following ERCOT-wide bill determinants should be identified within the Settlement System as bill determinants that should be included in the public data extracts: - RTSPP - RTCCAMTTOT |
| FR83 |  | PR 6.6.3.6 |  | CSWM. P03SettleBill Transfer |  | DRIVER: RTCCAMT should be calculated for a QSE and Settlement Point that has an SSQ data cut available for that Settlement Point, for the Operating Day. |
| FR84 |  | PR 6.6.3.6 |  | CSWM. P03SettleBill Transfer |  | RTCCAMTQSETOT should be calculated for a QSE that has at least one RTCCAMT data cut available for the Operating Day. |
| FR85 |  | PR 6.6.10 (3) |  | CSWM. P03SettleBill Transfer |  | RTCCAMTTOT should be calculated for every Operating Day. |
| FR86 |  | PR 6.6.10 (2) |  | CSWM. P03SettleBill Transfer |  | If RTCCAMT data is not available for the Operating Day, the RTCCAMTTOT data cut should be created with default values of zero. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR87 | PR 6.6.3.5 (1) | CSWM. P03SettleBill Transfer | If a RTSPP data cut is not available for a Settlement Point and Operating Day for which a driver data element exists, the system should log a “CRITICAL” error message and stop processing energy settlements for the Operating Day. This means that any calculation that depends on the RTSPP and any downstream calculation should be stopped. The error message should capture the missing data element, including the Settlement Point, and the Operating Day. |

### Specific Database Needs

None

## Base Point Deviation Charge (BPDAMT)

To Be Sub-Process for Settlements and Billing

The following charts illustrate the dependencies between the various Base Point Deviation Settlement calculations.  Disclaimer:  the diagram is meant to provide contextual information only, to the extent that the information in the Functional Requirements conflict with the diagram, the functional requirements shall override the diagram.

Figure 12.0: Base Point Deviation Settlement Data Dependencies

Calculation Process for Base Point Deviation Charge

| Requirement ID | FR88 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.5 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description The ERCOT Settlement system shall determine the resource type from the resource name within the AABP bill determinant. If the resource type is an Intermittent Renewable Resource (IRR) Generation Resource, then the ERCOT Settlement system shall perform the Calculation Process for IRR Generation Resources; otherwise the ERCOT Settlement system shall perform the Calculation Process for Generation Resources, Excluding IRRs. If AABP_<Q>_<R>_<SP> resource type = IRR Then Perform Calculation Process for IRR Generation Resources Otherwise Perform Calculation Process for Generation Resources, excluding IRR |  |

2.9.1	Calculation Process for IRR Generation Resources

(1)	ERCOT shall charge a QSE for an IRR a Base-Point deviation charge if the IRR metered generation is more than 10% above its Base Point and if the Base Point is 2 MW or more below the IRR’s HSL. The deviation charge may be refunded if the IRR shows, to ERCOT’s satisfaction, that the IRR was taking the necessary control actions to produce at levels equal to or less than the Base Point but was unable to comply solely due to increasing renewable energy input.  The IRR must always take the necessary control actions, in its capability, to comply with Base Point Dispatch Instructions if the Base Point is 2 MW or more below the IRR’s HSL as soon as practicable.

(2)	The charge to each QSE for non-excused over-generation of each IRR at each Resource Node Settlement Point, if the Real-Time metered generation is greater than the upper tolerance during a 15-minute Settlement Interval, is calculated as follows: (6.6.5.2)

Charge is calculated by QSE, Settlement Point, and Resource for each 15-Minute Interval of the Operating Day.

| Requirement ID | FR89 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.5.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description For IRR Generation Resources: If AABP_<Q>_<R>_<SP> > (HSL_<Q>_<R>_<SP> – QIRR) Then BPDAMT_<Q>_<R>_<SP> = 0 Otherwise BPDAMT_<Q>_<R>_<SP> = Max (0, RTSPP_<SP>) * Max (0, TWTG_<Q>_<R>_<SP> – ¼ * AABP_<Q>_<R>_<SP> * (1 + KIRR)) |  |

2.9.2 	Calculation Process for Generation Resources, excluding IRR

| Requirement ID | FR90 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.5.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description For all Generation Resources, except Intermittent Renewable Resources, the ERCOT Settlement system shall determine whether or not the Generation Resource over-generated or under-generated, on an interval by interval basis, as compared to the resource’s Base Point. If AABP_<Q>_<R>_<SP> > TWTG_<Q>_<R>_<SP> Then Perform Calculation Process for Generation Resources, excluding IRRs, for Under Generation. Otherwise Perform Calculation Process for Generation Resources, excluding IRRs, for Over Generation |  |

2.9.2.1	Calculation Process for Generation Resources, excluding IRRs, for Over Generation

ERCOT shall charge a QSE for a Generation Resource for over generation that exceeds the following tolerance.  The tolerance is the greater of:

(a)	5% of the average of the Base Points in the Settlement Interval adjusted for any Ancillary Services deployments; or

(b)	5 MW for metered generation above the average of the Base Points in the Settlement Interval adjusted for any Ancillary Services deployments.

The charge to each QSE for over-generation of each Generation Resource at each Resource Node Settlement Point, if the Real-Time metered generation is greater than the upper tolerance during a given 15-minute Settlement Interval, is calculated as follows: (PR 6.6.5.1.1)

Charge is calculated by QSE, Settlement Point, and Resource for each 15-Minute Interval of the Operating Day.

| Requirement ID | FR91 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.5.1.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description BPDAMT_<Q>_<R>_<SP> = Max (0, RTSPP_<SP>) * Max [0, (TWTG_<Q>_<R>_<SP> – ¼ * Max (((1 + K1) * AABP_<Q>_<R>_<SP>),(AABP_<Q>_<R>_<SP> + Q1)))] |  |

2.9.2.2	Calculation Process for Generation Resources, excluding IRRs, for Under Generation

(1)	ERCOT shall charge a QSE for a Generation Resource for under generation if the metered generation is below the lesser of:

(a)	95% of the average of the Base Points in the Settlement Interval adjusted for any Ancillary Service deployments; or

(b)	The average of the Base Points in the Settlement Interval adjusted for any Ancillary Service deployments minus 5 MW.

(2)	The charge to each QSE for under-generation of each Generation Resource at each Resource Node Settlement Point for a given 15-minute Settlement Interval is calculated as follows: (PR 6.6.5.1.2)

Charge is calculated by QSE, Settlement Point, and Resource for each 15-Minute Interval of the Operating Day.

| Requirement ID | FR92 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.5.1.2 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description BPDAMT_<Q>_<R>_<SP> = Max (0, RTSPP_<SP>) * Min (1, KP) * Max [0, (Min (((1 – K2) * ¼ (AABP_<Q>_<R>_SP>)) , ¼ (AABP_<Q>_<R>_<SP> – Q2)) – TWTG_<Q>_<R>_<SP>)] |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| AABP | MW | Adjusted Aggregated Base Point per QSE per Settlement Point per Resource—The aggregated Base Point adjusted for Ancillary Service deployments, of Generation Resource R represented by QSE Q at Resource Node SP, for the 15-minute Settlement Interval. | EMS |
| Interval Frequency: 4 / hour Additional Comments: The AABP calculation process needs to adhere to §6.4.2.2, §6.6.5.1(2), and §6.6.5.1(3). For example (but not limited to), the AABP should equal zero for a 15-minute settlement interval where the Resource’s BP deviation is in a direction that contributes to frequency correction that resolves an ERCOT System frequency deviation and the ERCOT System frequency deviation is greater than ±0.05 Hz at any point during the 15-minute settlement interval. |  |  |  |
| TWTG | MWh | Time-Weighted Telemetered Generation per QSE per Settlement Point per Resource—The telemetered generation of Generation Resource R represented by QSE Q at Resource Node SP, for the 15-minute Settlement Interval. | EMS |
| Interval Frequency: 4 / hour Additional Comments: The TWTG calculation process needs to adhere to §6.4.2.2, §6.6.5.1(2), and §6.6.5.1(3). For example (but not limited to), the TWTG should equal zero for a 15-minute settlement interval where the Resource’s BP deviation is in a direction that contributes to frequency correction that resolves an ERCOT System frequency deviation and the ERCOT System frequency deviation is greater than ±0.05 Hz at any point during the 15-minute settlement interval. |  |  |  |
| RTSPP | $ / MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point SP, for the 15-minute Settlement Interval. | MMS |
| Interval Frequency: 4 / hour Additional Comments: This value will need to have additional effective dating functionality to allow for future system / value changes. |  |  |  |
| K1 | Scalar | The percentage tolerance for over-generation, 5%. | Settlement System |
| Interval Frequency: Constant Additional Comments: This value will need to have additional effective dating functionality to allow for future system / value changes. |  |  |  |
| K2 | Scalar | The percentage tolerance for under-generation, 5%. | Settlement System |
| Interval Frequency: Factor Value -- Constant Additional Comments: This value will need to have additional effective dating functionality to allow for future system / value changes. |  |  |  |
| KIRR | Scalar | The percentage tolerance for over-generation of an IRR, 10%. | Settlement System |
| Interval Frequency: Constant Additional Comments: This value will need to have additional effective dating functionality to allow for future system / value changes. |  |  |  |
| Q1 | MW | The MW tolerance for over-generation, 5 MW. | Settlement System |
| Interval Frequency: Constant Additional Comments: This value will need to have additional effective dating functionality to allow for future system / value changes. |  |  |  |
| Q2 | MW | The MW tolerance for under-generation, 5 MW. | Settlement System |
| Interval Frequency: Constant Additional Comments: This value will need to have additional effective dating functionality to allow for future system / value changes. |  |  |  |
| QIRR | MW | The threshold to test the adjusted aggregated Base Point against the HSL for an IRR, 2 MW. | Settlement System |
| Interval Frequency: Constant Additional Comments: This value will need to have additional effective dating functionality to allow for future system / value changes. |  |  |  |
| KP | Scalar | The coefficient applied to the Settlement Point Price for under-generation charge, 1.0. | Settlement System |
| Interval Frequency: Constant Additional Comments: This value will need to have additional effective dating functionality to allow for future system / value changes. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| BPDAMT | $ | Base Point Deviation Charge per QSE per Settlement Point per Resource—The charge to QSE Q for Generation Resource R at Resource Node SP, for its deviation from Base Point, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| BPDBILLAMT | $ | Base Point Deviation Charge per QSE per Settlement Point per Resource— The difference between two consecutive daily summed settlement runs of BPDAMT for QSE Q in the Real-Time Market. | Settlement System |
| Interval Frequency: 1 / Day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1)  Calculation of the BPDAMT depends upon the availability of the RTSPP, AABP, and TWTG bill determinants.

### Assumptions

A1) 	When creating the RTSPP data cut for a given Settlement Point and Operating Day, the interface will include logic to evaluate and handle null values.  If the interface encounters a null value for any given 15-minute interval of the RTSPP data cut for a Settlement Point and Operating Day, the interface should initiate a “CRITICAL” error.  Essentially, this “CRITICAL” error should be raised when at least one interval of RTSPP data for a Settlement Point and Operating Day has a null value but other intervals have values, or when no RTSPP data exists for an entire Operating Day for a Settlement Point that is valid for that Operating Day.

A2)	The Original Source system should only make available, or appropriately flag, AABP data, as a 15-minute weighted average value across all of the SCED intervals during the 15-minute settlement interval, for Resources that are required to pay a Base Point Deviation, as defined by the ERCOT Nodal Protocols.  Essentially, the Original Source system should not propagate AABP data for any RMR Units, Dynamically Scheduled Resources (except as described in §6.4.2.2 of the Nodal Protocols,) Qualifying Facilities that do not submit an Energy Offer Curve for the Settlement Interval, or Generation Resources which meet the specified exclusions laid out in §6.6.5 (e.g. violation of Resource Parameters), §6.6.5.1(2), and §6.6.5.1(3) of the Nodal Protocols.  The Interface shall appropriately interpolate data from the original source system in a fashion to create an AABP interval data cut for an entire day, substituting zeros for the intervals that have null values.  As such, the Settlement System will settle based upon any AABP data created, or flagged appropriately, by the original source system.

A3)	The original source system should only make available, or appropriately flag, TWTG data, as a 15-minute weighted average value across all of the SCED intervals during the 15-minute settlement interval, for Resources that are required to pay a Base Point Deviation, as defined by the ERCOT Nodal Protocols. Essentially, the Original Source system should not propagate TWTG data for any RMR Units, Dynamically Scheduled Resources (except as described in §6.4.2.2 of the Nodal Protocols,) Qualifying Facilities that do not submit an Energy Offer Curve for the Settlement Interval, or Generation Resources which meet the specified exclusions laid out in §6.6.5 (e.g. violation of Resource Parameters), §6.6.5.1(2), and §6.6.5.1(3) of the Nodal Protocols.  The Interface shall appropriately interpolate data from the original source system in a fashion to create a TWTG interval data cut for an entire day, substituting zeros for the intervals that have null values.  As such, the Settlement System will settle based upon any TWTG data created, or flagged appropriately, by the original source system.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR93 | (PR 6.6.5.1.1) (PR 6.6.5.1.2) | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: RTSPP TWTG AABP K1 Q1 K2 Q2 KIRR QIRR KP |
| FR94 | (PR 6.6.5.1.1) (PR 6.6.5.1.2) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: BPDAMT |
| FR95 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that ought to be included in the public data extracts. RTSPP K1 Q1 K2 Q2 KIRR QIRR KP All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. AABP TWTG BPDAMT |
| FR96 | (PR 6.6.5.1.1) (PR 6.6.5.1.2) | CSWM. P03SettleBill Transfer | If an AABP data cut is not available for the QSE, Settlement Point, Resource and Operating Day for which a TWTG data cut for the same QSE, Settlement Point, Resource, and Operating Day exists, the system should use a default value of zero (0) for each interval. No error message is necessary. |
| FR97 | (PR 6.6.5.1.1) (PR 6.6.5.1.2) | CSWM. P03SettleBill Transfer | If an TWTG data cut is not available for the QSE, Settlement Point, Resource and Operating Day for which a AABP data cut for the same QSE, Settlement Point, Resource, and Operating Day exists, the system should use a default value of zero (0) for each interval. No error message is necessary. |
| FR98 | (PR 6.6.5.1.1) (PR 6.6.5.1.2) | CSWM. P03SettleBill Transfer | DRIVER: The BPDAMT calculation should only be attempted for QSEs for which there is a resource with a AABP and TWTG data cuts in the settlement system. |
| FR99 | PR 9.5.6 (6), PR 9.5.8 (3), PR 9.5.5 (2) | CSWM. P03SettleBill Transfer | The BPDBILLAMT value shall be calculated as the entire day’s sum of the BPDAMT (aggregated across Settlement Points and Resources for a QSE) values for the latter settlement run less the entire day’s sum of the BPDAMT (aggregated across Settlement Points and Resources for a QSE) values for the most previous settlement run. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR100 | (PR 6.6.5.1.1) (PR 6.6.5.1.2) | CSWM. P03SettleBill Transfer | If a RTSPP data cut is not available for a Settlement Point and Operating Day for which a driver data element exists, the system should log a “CRITICAL” error message and stop processing energy settlements for the Operating Day. This means that any calculation that depends on the RTSPP and any downstream calculation should be stopped. The error message should capture the missing data element, including the Settlement Point, and the Operating Day. |

### Specific Database Needs

- The Settlement System will need a structure sufficient enough to allow for effective dating and value changes for the Constants and Coefficients listed above.

- The Settlement System will need to maintain a structure that allows the System to lookup the Resource Type based upon the Resource Name.

## Base Point Deviation Payment (LABPDAMT)

ERCOT shall pay the Base-Point deviation charges collected from the QSEs representing Generation Resources to the QSEs representing Load based on Load Ratio Share. The payment to each QSE for a given 15-minute Settlement Interval is calculated as follows: (6.6.5.4)

Payment is calculated by QSE, Settlement Point, and Resource for each 15-Minute Interval of the Operating Day.

| Requirement ID | FR101 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.5.4 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description LABPDAMT_<Q> = (-1) * BPDAMTTOT * LRS_<Q> |  |

| Requirement ID | FR102 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.5.4 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description BPDAMTTOT = ΣQBPDAMTQSETOT_<Q> |  |

| Requirement ID | FR103 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.5.4 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description BPDAMTQSETOT_<Q> = ΣSPΣRBPDAMT_<Q>_<R>_<SP> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| BPDAMT | $ | Base Point Deviation Charge per QSE per Settlement Point per Resource—The charge to QSE Q for Generation Resource R at Resource Node SP, for its deviation from Base Point, for the 15-minute Settlement Interval. | Settlement System See FR88, FR90, FR91 |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| LRS | None | The Load Ratio Share calculated for QSE Q for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. | Data Aggregation See Section 2.2 |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| BPDAMTQSETOT | $ | Base-Point Deviation Amount QSE Total per QSE—The total of Base-Point Deviation Charges to QSE Q for all Generation Resources represented by this QSE, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| BPDAMTTOT | $ | Base-Point Deviation Amount Total—The total of Base-Point Deviation Charges to all QSEs for all Generation Resources, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| LABPDAMT | $ | Load-Allocated Base-Point Deviation Amount per QSE—QSE Q’s share of the total charge for all the Generation Resource’s Base Point deviation, based on Load Ratio Share, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| LABPDBILLAMT | $ | Load-Allocated Base-Point Deviation Amount per QSE — The difference between two consecutive daily summed settlement runs of LABPDAMT for QSE Q for the Operating Day in the Real Time market. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1)  Calculation of the LABPDAMT depends upon the completion of the BPDAMT calculation process and the availability of the LRS bill determinant.

### Assumptions

None

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR104 | (PR 6.6.5.4) | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: BPDAMT LRS BPDAMTQSETOT BPDAMTTOT |
| FR105 | (PR 6.6.5.4) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: LABPDAMT |
| FR106 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that ought to be included in the public data extracts. BPDAMTTOT All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. LRS BPDAMT BPDAMTQSETOT LABPDAMT |
| FR107 | (PR 6.6.5.4) | CSWM. P03SettleBill Transfer | DRIVER: The LABPDAMT calculation should be calculated for all active QSEs. |
| FR108 | PR 9.5.5 (2),PR 9.5.6 (6), PR 9.5.8 (3) | CSWM. P03SettleBill Transfer | The LABPDBILLAMT value shall be calculated as the entire day’s sum of the LABPDAMT values for the latter settlement run less the entire day’s sum of the BPDAMT values for the most previous settlement run. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR109 | (PR 6.6.5.4) | CSWM. P03SettleBill Transfer | If an active QSE does not have a LRS data cut for the Operating Day, then the Settlement System should LABPDAMT to zero for each interval of the Operating Day. Additionally the Settlement System should issue a Warn / Default error with the QSE and Operating Day included in the error message. |

### Specific Database Needs

None

## Real-Time Revenue Neutrality Allocation (LARTRNAMT)

ERCOT must be revenue-neutral in each Settlement Interval. Each QSE receives an allocated share, on a Load Ratio Share basis, of the net amount of:

(a)	Real-Time Energy Imbalance payments or charges under Section 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at  a Resource Node;

(b)	Real-Time Energy Imbalance payments or charges under Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone;

(c)	Real-Time Energy Imbalance payments or charges under Section 6.6.3.3, Real-Time Energy Imbalance Payment or Charges at a HUB;

(d)	Real-Time energy payments under Section 6.6.3.4, Real-Time Energy Payment for DC Tie Import;

(e)	Real-Time energy payments under Section 6.6.3.5, Payment for a Block Load Transfer Point;

(f)	Real-Time energy charge under Section 6.6.3.6, Charge for an Export Represented by a QSE under the Oklaunion Exemption;

(g)	Real-Time congestion payments or charges under Section 6.6.4, Real-Time Congestion Payment or Charge for Self Schedules;

(h)	Real-Time value of Day-Ahead energy sale from RMR Units under Section 6.6.6.5, RMR Service Charge;

(i)	Real-Time payments or charges to the CRR Owners under Section 7.9.2, Real-Time CRR Payments and Charges

| Requirement Id | FR110 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 6.6.10 (2) PR 6.6.10 (3) |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the RT Revenue Neutrality Allocation by QSE for each 15-minute interval in the Operating Day if the Day Ahead Market executes. NODAMFLAG = 0. LARTRNAMT_<Q> = (-1) * (RTEIAMTTOT + BLTRAMTTOT + RTDCIMPAMTTOT + RTDCEXPAMTTOT + RTCCAMTTOT + RMRDAESRTVTOT + RTOBLAMTTOT / 4 + RTOPTAMTTOT / 4 + RTOPTRAMTTOT /4) * LRS q If the Day Ahead Market does not execute use (NODAMFLAG = 1): LARTRNAMT_<Q> = (-1) * (RTEIAMTTOT + BLTRAMTTOT + RTDCIMPAMTTOT + RTDCEXPAMTTOT + RTCCAMTTOT + RMRDAESRTVTOT + NDRTOBLAMTTOT / 4 + NDRTOPTAMTTOT / 4 + NDRTOPTRAMTTOT /4+NDRTOBLRAMTTOT/4+NDRTFGRAMTTOT/4) * LRS q |  |

### Input Bill Determinants

|  | Variable |  | Unit | Definition |  | Original Source |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RTEIAMTTOT |  | $ | Real-Time Energy Imbalance Amount Total The Total net payments and charges for Real-Time Energy Imbalance at all Settlement Points (Resource, Load Zone, or Hub) for the 15-Minute Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | BLTRAMTTOT |  | $ | Block Load Transfer Resource Amount TotalThe total of the payments for energy delivered into the ERCOT System through BLT Points for the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | RTDCIMPAMTTOT |  | $ | Real-Time DC Import Amount Total—The summation of payments for DC Tie import, for the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | RTDCEXPAMTTOT |  | $ | Real-Time DC Export Amount Total—The summation of charges to all QSEs that are under the “Oklaunion Exemption” for DC Tie export, for the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | RTCCAMTTOT |  | $ | Real-Time Energy Congestion Cost Amount TotalThe total net congestion payments and charges for all of the Self-Schedules for the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | RMRDAESRTVTOT |  | $ | RMR Day-Ahead Energy Sale Real-Time Value Total—The total of the Real-Time value of the Day-Ahead energy sales from all RMR Units, for the 15-minute Settlement Interval. See Section 6.6.6, Reliability Must Run Settlement. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: For more information on this bill determinant see the TN.COMS.63C01.RTRMR.REQUIREMENTS.A business requirements document. |  |  |  |  |  |  |  |
|  | RTOBLAMTTOT |  | $ | Real-Time Obligation Amount Total—The sum of all payments and charges for PTP Obligations settled in Real-Time, for the hour that includes the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 1 / hour Additional Comments: For more information on this bill determinant see the TN.COMS.63C01.RTCRR.REQUIREMENTS.A business requirements document. |  |  |  |  |  |  |  |
|  | RTOPTAMTTOT |  | $ | Real-Time Option Amount Total—The sum of all payments for PTP Options settled in Real-Time, for the hour that includes the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 1 / hour Additional Comments: For more information on this bill determinant see the TN.COMS.63C01.RTCRR.REQUIREMENTS.A business requirements document. |  |  |  |  |  |  |  |
|  | RTOPTRAMTTOT |  | $ | Real-Time Option with Refund Amount Total—The sum of all payments for PTP Options with Refund settled in Real-Time, for the hour that includes the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 1 / hour Additional Comments: For more information on this bill determinant see the TN.COMS.63C01.RTCRR.REQUIREMENTS.A business requirements document. |  |  |  |  |  |  |  |
|  | NDRTOBLAMTTOT |  | $ | No DAM Real-Time Obligation Amount Total—The sum of all payments and charges for PTP Obligations settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | NDRTOPTAMTTOT |  | $ | No DAM Real-Time Option Amount Total—The sum of all payments for PTP Options settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | NDRTOPTRAMTTOT |  | $ | No DAM Real-Time Option with Refund Amount Total—The sum of all payments for PTP Options with Refund settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | NDRTFGRAMTTOT |  | $ | No DAM Real-Time FGR Amount Total— The sum of all payments for FGRs settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | NDRTOBLRAMTTOT |  | $ | No DAM Real-Time Obligation with Refund Amount Total— The sum of all payments for PTP Obligations with Refund settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  | Settlement System |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
|  | LRS |  |  | Load Ratio Share per QSE—The Load Ratio Share as defined in Section 2, Definitions and Acronyms, for the QSE, for the 15-minute Settlement Interval. |  | Data Aggregation |  |  |
|  | Interval Frequency: 4 / hour Additional Comments: None |  |  |  |  |  |  |  |
| NODAMFLAG |  | None |  |  | No Day Ahead Market Flag - a flag to indicate when ERCOT was unable to execute the Day-Ahead Market for an Operating Day. A value of one (1) indicates that DAM was not executed for the Operating Day; a value of zero (0) indicates that the DAM was executed for the Operating Day. |  | Interface, based upon MMS data |  |
| Interval Frequency: Daily Additional Comments: The interface will create this bill determinant by evaluating DASPP, DAES, and DAEP data for an Operating Day. If the interface finds no DASPP data, no awarded Energy Offers (DAES), and no awarded Energy Bids (DAEP) for the Operating Day then it will set NODAMFLAG = 1for the Operating Day. If any of the data is found, it will set the NODAMFLAG = 0. |  |  |  |  |  |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| LARTRNAMT | $ | Load-Allocated Real-Time Revenue Neutrality Amount per QSE — The QSE’s share of the total Real-Time revenue neutrality amount, for the 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| LARTRNBILL-AMT | $ | Load-Allocated Real-Time Revenue Neutrality Bill Amount per QSE - The difference between two consecutive daily summed settlement runs of the total LARTRNAMT for the QSE and for the Operating Day in the Real Time market. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of RTEIAMTTOT.

D2) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of BLTRAMTTOT.

D3) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of RTDCIMPAMTTOT.

D4) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of RTDCEXPAMTTOT.

D5) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of RTCCAMTTOT.

D6) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of RMRDAESRTVTOT.

D7) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of RTOBLAMTTOT.

D8) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of RTOPTAMTTOT.

D9) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of RTOPTRAMTTOT.

D10) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of NDRTOBLAMTTOT.

D11) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of NDRTOPTAMTTOT.

D12) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of NDRTOPTRAMTTOT.

D13) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of NDOBLRAMTTOT.

D14) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of NDRTFGRAMTTOT.

D15) Calculation of the LARTRNAMT for a QSE is dependent upon calculation of LRS.

Figure 13.0: Load-Allocated Real-Time Revenue Neutrality Amount Data Dependencies

### Assumptions

A1)  Data Aggregation calculates RTAML for every active QSE, and therefore LRS will be calculated and should be available for every active QSE.

A2) When determining whether the Day Ahead Market executed or not, NODAMFLAG will be populated with -0- when the Day Ahead Market executes.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR111 | PR 6.6.3.1 (3), PR 6.6.3.2 (3), PR 6.6.3.3 (3) | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. |
| FR112 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | All of the output bill determinants should be rounded to two (2) decimal places. |
| FR113 | PR 9.5.6 (6), PR 9.5.8 (3), PR 9.5.5 (2) | CSWM. P03SettleBill Transfer | Calculate LARTRNBILLAMT for the QSE as the QSE’s Operating Day total LARTRNAMT for the latter settlement run less the QSE’s Operating Day total LARTRNAMT for the most previous settlement run. |
| FR114 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following QSE specific bill determinants should be identified within the Settlement System as bill determinants that should be included in the private data extracts: - LRS - LARTRNAMT - LARTRNBILLAMT |
| FR115 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following ERCOT-wide bill determinants should be identified within the Settlement System as bill determinants that should be included in the public data extracts: - RTEIAMTTOT - RTBLTRAMTTOT - RTDCIMPAMTTOT - RTDCEXPAMTTOT - RTCCAMTTOT - RMRDAESRTVTOT - RTOBLAMTTOT - RTOPTAMTTOT - RTOPTRAMTTOT - NDRTOBLAMTTOT - NDRTOPTAMTTOT - NDRTOPTRAMTTOT - NDRTOBLRAMTTOT - NDRTFGRAMTTOT |
| FR116 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | DRIVER: LARTNRAMT should be calculated for every active QSE for the Operating Day. |
| FR117 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTEIAMTTOT data is not available for the Operating Day, the RTEIAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR118 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTBLTRAMTTOT data is not available for the Operating Day, the RTBLTRAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR119 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTDCIMPAMTTOT data is not available for the Operating Day, the RTDCIMPAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR120 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTDCEXPAMTTOT data is not available for the Operating Day, the RTDCEXPAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR121 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTCCAMTTOT data is not available for the Operating Day, the RTCCAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR122 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RMRDAESRTVTOT data is not available for the Operating Day, the RMRDAESRTVTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR123 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTOBLAMTTOT data is not available for the Operating Day, the RTOBLAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR124 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTOPTAMTTOT data is not available for the Operating Day, the RTOPTAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR125 | PR 6.6.10 (2) | CSWM. P03SettleBill Transfer | If RTOPTRAMTTOT data is not available for the Operating Day, the RTOPTRAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR137 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | If DAM did not run and NDRTOBLAMTTOT data is not available for the Operating Day, the NDRTOBLAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR138 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | If DAM did not run and NDRTOPTAMTTOT data is not available for the Operating Day, the NDRTOPTAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR139 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | If DAM did not run and NDRTOPTRAMTTOT data is not available for the Operating Day, the NDRTOPTRAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR140 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | If DAM did not run and NDRTFGRAMTTOT data is not available for the Operating Day, the NDRTFGRAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR141 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | If DAM did not run and NDRTOBLRAMTTOT data is not available for the Operating Day, the NDRTOBLRAMTTOT data should be created with default values of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |
| FR142 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | If NODAMFLAG data is not available for the Operating Day, the NODAMFLAG data should be created with default value of zero and the system should log a warning message that indicates the Operating Day and the missing data element. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR126 | PR 6.6.3.1 (3), PR 6.6.3.2 (3), PR 6.6.3.3 (3) | CSWM. P03SettleBill Transfer | If an active QSE does not have an LRS data cut for the Operating Day, the Settlement System should produce a WARN-DEFDAULT error message and default the LRS values to zero. The warning message should indicate the QSE and Operating Day for which the LRS data is missing. |
| FR136 | PR 6.6.10 (3) | CSWM. P03SettleBill Transfer | If DAM does not execute for the Operating Day, the settlement system should produce a warn error message. The warning message should indicate the Operating Day for which the DAM did not execute. |

### Specific Database Needs

None

## ERCOT System Administration Charge (ESACAMT)

Each QSE shall pay an ERCOT System administration charge to administer the RTM market.  The ERCOT System administration charge is for each 15-minute Settlement Interval for each QSE. (PR 9.16.1)

| Requirement Id | FR127 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 9.16.1 |
| Traceability to Sub-Process Maps | CSWM.P03SettleBillTransfer |
| Description: Calculate the ERCOT System Administration Charge by QSE and Operating Day for a 15-minute interval. ESACAMT_<Q> = LAFF * (ΣSPRTAML_<Q>_<SP>) (NPRR change required: Modify 9.16.1 so that the equation sums the RTAML across Settlement Points before applying the Load Administration Fee Factor. |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| RTAML | MWh | Real-Time Adjusted Metered Load—The QSE Adjusted Metered Load at the Settlement Point for the 15-minute Settlement Interval. | Data Aggregation |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| LAFF | $/MWh | Load Administration Fee Factor—The ERCOT System administration fee rate in dollars per MWh. | Settlement System |
| Interval Frequency: Constant that is valid for a range of Operating Days Additional Comments: This value should be stored within the Settlement System with effective dates. The LAFF value is applicable to all QSEs for the Operating Day range specified by the effective dates. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| ESACAMT | $ | ERCOT System Administration Charge—The ERCOT System administration charge for each QSE per 15-minute Settlement Interval. | Settlement System |
| Interval Frequency: 4 / hour Additional Comments: None |  |  |  |
| ESACBILLAMT | $ | ERCOT System Administration Charge Bill Amount— The difference between two consecutive daily summed settlement runs of the total ESACAMT for the QSE and for the Operating Day in the Real Time market. | Settlement System |
| Interval Frequency: 1 / day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1) Calculation of the ESACAMT for a QSE is dependent upon availability of the data inputs.

Figure 14.0: ERCOT System Administration Charge Data Dependencies

### Assumptions

A1) The ERCOT Board shall determine, subject to PUCT approval, the administrative fees, as described in this Section 9.10, Administrative Fees, and ERCOT shall post them on the MIS Public Area within two Business Days following PUCT approval.  (PR 9.16)  Once PUCT approved and published, the LAFF value will be available for entry into the Settlement System.  (NPRR change required: Correct the reference in PR 9.16 to make reference to 9.16 instead of 9.10.)

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR128 | PR 9.16.1 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. |
| FR129 | PR 9.16.1 | CSWM. P03SettleBill Transfer | All of the output bill determinants should be rounded to two (2) decimal places. |
| FR130 | PR 9.5.6 (6), PR 9.5.8 (3), PR 9.5.5 (2) | CSWM. P03SettleBill Transfer | Calculate ESACBILLAMT for the QSE as the QSE’s Operating Day total ESACAMT for the latter settlement run less the QSE’s Operating Day total ESACAMT for the most previous settlement run. |
| FR131 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following QSE specific bill determinants should be identified within the Settlement System as bill determinants that should be included in the private data extracts: - RTAML - ESACAMT - ESACBILLAMT |
| FR132 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following ERCOT-wide bill determinants should be identified within the Settlement System as bill determinants that should be included in the public data extracts: - LAFF |
| FR133 | PR 9.16.1 | CSWM. P03SettleBill Transfer | DRIVER: ESACAMT should be calculated for a QSE that has an RTAML data cut available for any Settlement Point, for the Operating Day. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR134 | PR 9.16.1 | CSWM. P03SettleBill Transfer | If LAFF is not available for an Operating Day, the system should stop processing ESACAMT and should log an error indicating the missing data element and the Operating Day. |

### Specific Database Needs

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR135 | PR 9.16.1 | CSWM. P03SettleBill Transfer | The Settlement System should have the ability to store and maintain the LAFF value for defined ranges of Operating Days. |

# Supplementary Requirements

## Performance Requirements

Lodestar must be able to perform its full set of Nodal settlement calculations. Performance characteristics will be determined and detailed in the conceptual and detailed design.

## Legal and Regulatory requirements

All Lodestar financial data must be saved in accordance with standard accounting principles as dictated by the ERCOT Texas Nodal Protocols.

## System Security Requirements

Lodestar will only be accessed directly by internal ERCOT personnel either through thin client UI, Citrix environment, or thick client on employee desktops. Market Participants will be able to access their statements and invoices on ERCOT private web site. ERCOT private web site access will be controlled by digital certificate authentication in accordance with ERCOT security policies.  Additional details will be provided in the Conceptual System Design.

## Back up and Recovery Requirements

All Lodestar Nodal settlement data must be backed up on a daily basis and recoverable within 24 hours.  Additional details will be provided in the Conceptual System Design.

## Availability and Redundancy Requirements

Nodal Lodestar system must be available 7/24. Lodestar is considered a tier 1 system and critical to base ERCOT business processes.  Additional details will be provided in the Conceptual System Design.

## Maintainability Requirements

Lodestar Nodal system will be maintained by internal ERCOT personnel. Service Level Agreements and explicit support will be detailed in the detail design document.

## Daylight Savings Time shall be recognized in Nodal Market settlements.

Spring Daylight Savings Time Operating Day:

- Hour Ending 0300 will be skipped, resulting in 23 hourly intervals for the day.

- The four fifteen minute intervals within Hour Ending 0300 will be skipped, resulting in 92 fifteen minute intervals for the day.

- The sixty one minute intervals within Hour Ending 0300 will be skipped, resulting in 1380 total one minute intervals.

Fall Daylight Savings Time Operating Day:

- Hour Ending 0200 will have two (2) sets of data, resulting in 25 hourly intervals for the day.

- The four fifteen minute intervals within Hour Ending 0200 will have two (2) sets of data, resulting in 100 fifteen minute intervals for the day.

- The sixty one minute intervals within Hour Ending 0200 will have two (2) sets of data, resulting in 1500 total one minute intervals.

# Protocol Coverage

Since one protocol may translate into many requirements, Project Managers are required to identify the coverage extent (Full/Partial) of all relevant Nodal Protocols that are mentioned in this requirements document.

| Protocol Sub-Section # (To the lowest level of granularity as possible) | Coverage by Requirements (Full/Partial) |
| --- | --- |
| 6.6.2.1 | Partial |
| 6.6.2.2 | Partial |
| 6.6.2.3 | Partial |
| 6.6.3.1 (1), (2), and (3) | Partial |
| 6.6.3.2 (1), (2), and (3) | Partial |
| 6.6.3.3 (1), (2), and (3) | Partial |
| 6.6.3.4 (1) and (2) | Partial |
| 6.6.3.5 (1) and (2) | Partial |
| 6.6.3.6 (1) and (2) | Partial |
| 6.4 (1) and (2) | Partial |
| 6.6.5 | Partial |
| 6.6.5.1 | Partial |
| 6.6.5.1.1 | Partial |
| 6.6.5.1.2 | Partial |
| 6.6.5.2 | Partial |
| 6.6.5.3 | Partial |
| 6.6.5.4 | Partial |
| 6.6.10 (1) and (2) | Partial |
| 9.5.5 (2) | Partial |
| 9.5.6 (6) | Partial |
| 9.5.8 (3) | Partial |
| 9.16.1 | Partial |