| NPRR Number | 044 | NPRR Title | Corrections to Section 6, Adjustment Period and Real-Time Operations |
| --- | --- | --- | --- |
| Timeline | Normal | Recommended Action | Approval |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone 6.6.5.4, Base Point Deviation Payment 6.6.9.1, Payment for Emergency Power Increase directed by ERCOT 6.6.9.2, Charge for Emergency Power Increases 6.6.10, Real-Time Revenue Neutrality Allocation |  |
| Proposed Effective Date |  | Upon Implementation of the Texas Nodal Market |  |
| Priority and Rank Assigned |  | N/A |  |
| Revision Description |  | TPTF determined the need for these Nodal Protocol language revisions during its review of the business requirements associated with these Nodal Protocol sections. |  |
| Overall Market Benefit |  | These Nodal Protocol language revisions maintain consistency between business requirements and Nodal Protocols. |  |
| Overall Market Impact |  | None. |  |
| Consumer Impact |  | Unknown. |  |
| Credit Impacts |  | ERCOT credit staff and the Credit Work Group (Credit WG) have reviewed NPRR044 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| Procedural History |  | - NPRR044 was posted on 2/22/07. - PRS considered this NPRR on 3/22/07. - On 6/8/07, ERCOT posted a Baseline1 Group 7 (B1G7) IA for this NPRR. - On 6/12/07, PRS reviewed the B1G7 IA. - On 6/28/07, TAC considered NPRR044. |  |
| PRS Recommendation |  | On 3/22/07, PRS unanimously voted to recommend approval of NPRR044 as submitted. All Market Segments were present for the vote. On 6/12/07, PRS voted to endorse the B1G7 IA for NPRR044 as amended by PRS and forward the document to TAC. The motion passed with one abstention from the Investor Owned Utility (IOU) Market Segment. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 3/22/07, PRS proposed revisions to the B1G11 IA that were acceptable to ERCOT Staff On 6/12/07, there was no discussion. |  |
| TAC Recommendation |  | On 6/28/07, TAC voted to recommend approval of NPRR044 and endorse the B1G7 IA as recommended by PRS. The motion passed with five abstentions from the Independent Power Marketer (1) and Consumer (4) Market Segments. All Market Segments were present for the vote. |  |
| Summary of TAC Discussion. |  | On 6/28/07, TAC noted that NPRR044 is a new NPRR in Baseline 1. |  |

ERCOT/Market Segment Impacts and Benefits

| Assumptions | 1 | n/a |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | none |  |
|  | 2 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Maintain consistency between business requirements and protocols. |  |
|  | 2 |  |  |
| Additional Qualitative Information | 1 | n/a |  |
|  | 2 |  |  |
| Other | 1 |  |  |
| Comments | 2 |  |  |

| Original Sponsor |  |
| --- | --- |
| Name | Trip Doggett on behalf of TPTF |
| E-mail Address | tdoggett@ercot.com |
| Company | ERCOT |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| None |  |

Proposed Nodal Protocol Language Revision

# 6.6.3.2	Real-Time Energy Imbalance Payment or Charge at a Load Zone

(1)	The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time SPP for the following amounts at a particular Load Zone Settlement Point:

(a) 	The amount of its Self-Schedules with sink specified at the Settlement Point; plus

(b) 	The amount of its Energy Bids cleared in the DAM at the Settlement Point; plus

(c) 	The amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(d) 	The amount of its Self-Schedules with source specified at the Settlement Point; minus

(e) 	The amount of its Energy Offers cleared in the DAM at the Settlement Point; minus

(f) 	The amount of its Energy Trades at the Settlement Point where the QSE is the seller; minus

(g) 	Its Adjusted Metered Load at the Settlement Point.

(2)	The payment or charge to each QSE for Energy Imbalance Service at a Load Zone for a given 15-minute Settlement Interval is calculated as follows:

RTEIAMT q, p	=	(-1) * RTSPP p * { (SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼) – RTAML q, p }

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for the Real-Time Energy Imbalance at Settlement Point p, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| RTAML q, p | MWh | Real-Time Adjusted Metered Load per QSE per Settlement Point—The sum of the Adjusted Metered Load at the Electrical Buses that are included in Settlement Point p represented by QSE q for the 15-minute Settlement Interval. |
| SSSK q, p | MW | Self-Schedule with Sink at Settlement Point per QSE per Settlement Point—The QSE q’s Self Schedule with sink at Settlement Point p, for the 15-minute Settlement Interval. |
| DAEP q, p | MW | Day-Ahead Energy Purchase per QSE per Settlement Point—The QSE q’s Energy Bids at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQEP q, p | MW | Real-Time QSE-to-QSE Energy Purchase per QSE per Settlement PointThe amount of MW bought by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSR q, p | MW | Self-Schedule with Source at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with source at Settlement Point p, for the 15-minute Settlement Interval. |
| DAES q, p | MW | Day-Ahead Energy Sale per QSE per Settlement Point—The QSE q’s Energy Offers at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQES q, p | MW | Real-Time QSE-to-QSE Energy Sale per QSE per Settlement PointThe amount of MW sold by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| q |  | A QSE. |
| p |  | A Load Zone Settlement Point. |

(3)	The total net payments and charges to each QSE for Energy Imbalance Service at all Load Zones for the 15-minute Settlement Interval is calculated as follows:

RTEIAMTQSETOT q	=	ΣpRTEIAMT q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTEIAMTQSETOT q | $ | Real-Time Energy Imbalance Amount QSE Total per QSEThe total net payments and charges to QSE q for Real-Time Energy Imbalance at all Load Zone Settlement Points for the 15-minute Settlement Interval. |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The charge to QSE q for the Real-Time Energy Imbalance at Settlement Point p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Load Zone Settlement Point. |

# 6.6.5.4	Base Point Deviation Payment

ERCOT shall pay the Base-Point deviation charges collected from the QSEs representing Generation Resources to the QSEs representing Load based on Load Ratio Share. The payment to each QSE for a given 15-minute Settlement Interval is calculated as follows:

LABPDAMT q	=	(-1) * BPDAMTTOT * LRS q

Where:

BPDAMTTOT	=	ΣqBPDAMTQSETOT q

BPDAMTQSETOT q	=	ΣpΣrBPDAMT q, p, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LABPDAMT q | $ | Load-Allocated Base-Point Deviation Amount per QSE—QSE q’s share of the total charge for all the Generation Resource’s Base Point deviation, based on Load Ratio Share, for the 15-minute Settlement Interval. |
| BPDAMTTOT | $ | Base-Point Deviation Amount Total—The total of Base-Point Deviation Charges to all QSEs for all Generation Resources, for the 15-minute Settlement Interval. |
| BPDAMTQSETOT q | $ | Base-Point Deviation Amount QSE Total per QSE—The total of Base-Point Deviation Charges to QSE q for all Generation Resources represented by this QSE, for the 15-minute Settlement Interval. |
| BPDAMT q, p, r | $ | Base Point Deviation Charge per QSE per Settlement Point per Resource—The charge to QSE q for Generation Resource r at Resource Node p, for its deviation from Base Point, for the 15-minute Settlement Interval. |
| LRS q | none | The Load Ratio Share calculated for QSE q for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |

6.6.9.1	Payment for Emergency Power Increase Directed by ERCOT

(1)	If the Emergency Base Point issued to a Generation Resource is higher than the SCED Base Point immediately before the Emergency Condition, then ERCOT shall pay the QSE an additional compensation for the Resource at its Resource Node Settlement Point.  The payment for a given 15-minute Settlement Interval is calculated as follows:

EMREAMT q, p, r	=	(-1) * EMREPR q, p, r * EMREq, p, r

Where:

EMREPR q, p, 	r	=	Max (0, EBPWAPR q, p, r – RTSPP p)

EBPWAPR q, p, r	=	Σy(EBPPR q, p, r, y * EBP q, p, r, y * TLMP y) /

Σy(EBPq, p, r, y * TLMP y)

EMRE q, p, r		=	Max (0, Min (AEBP q, p, r * ¼Σy, RTMG q, p, r) – ¼ * BP q, p, r)

AEBP q, p, r		=	Σy(EBP q, p, r, y * TLMPy / 3600)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EMREAMT q, p, r | $ | Emergency Energy Amount per QSE per Settlement Point per Resource—The payment to QSE q as additional compensation for the additional energy produced by Generation Resource r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| EMREPR q, p, r | $/MWh | Emergency Energy Price per QSE per Settlement Point per Resource—The compensation rate for the additional energy produced by Generation Resource r at Resource Node p represented by QSE q in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| EMRE q, p, r | MWh | Emergency Energy per QSE per Settlement Point per Resource—The additional energy produced by Generation Resource r at Resource Node p represented by QSE q in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| EBPWAPR q, p, r | $/MWh | Emergency Base Point Weighted Average Price per QSE per Settlement Point per Resource—The weighted average of the energy prices corresponding with the Emergency Base Points on the Energy Offer Curve for Resource r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval. |
| BP q, p, r | MW | Base Point per QSE per Settlement Point per Resource—The Base Point of Resource r at Resource Node p represented by QSE q from the SCED prior to the Emergency Condition. |
| AEBP q, p, r | MW | Aggregated Emergency Base Point— The Generation Resource’s aggregated Emergency Base Point, for the 15-minute Settlement Interval. |
| EBP q, p, r, y | MW | Emergency Base Point per QSE per Settlement Point per Resource by interval—The Emergency Base Point of Resource r at Resource Node p represented by QSE q for the Emergency Base Point interval or SCED interval y. If a Base Point instead of an Emergency Base Point is effective during the interval y, its value equals the Base Point. |
| EBPPR q, p, r, y | $/MWh | Emergency Base Point Price per QSE per Settlement Point per Resource by interval—The Real-Time energy offer price corresponding with the Emergency Base Point of Resource r at Resource Node p represented by QSE q, for the Emergency Base Point interval or SCED interval y. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| RTMG q, p, r | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource—The metered generation of Resource r at Resource Node p represented by QSE q in Real-Time for the 15-minute Settlement Interval. |
| TLMP y | second | Duration of Emergency Base Point interval or SCED interval per interval—The duration of the portion of the Emergency Base Point interval or SCED interval y within the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |
| y | none | An Emergency Base Point interval or SCED interval that overlaps the 15-minute Settlement Interval. |

(2)	The total additional compensation to each QSE for emergency power increases of Generation Resources for the 15-minute Settlement Interval is calculated as follows:

EMREAMTQSETOT q	=	ΣpΣrEMREAMT q, p, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EMREAMTQSETOT q | $ | Emergency Energy Amount QSE Total per QSEThe total of the payments to QSE q as additional compensation for emergency power increases of the non-RMR Generation Resources represented by this QSE for the 15-minute Settlement Interval. |
| EMREAMT q, p, r | $ | Emergency Energy Amount per QSE per Settlement Point per Resource—The payment to QSE q as additional compensation for the additional energy produced by Generation Resource r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| q | None | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |

6.6.9.2	Charge for Emergency Power Increases

Each QSE shall pay a charge for emergency power increases based on its Load Ratio Share of the total additional compensation for all Generation Resources that ERCOT provides Emergency Base Points higher than the SCED Base Point prior to the Emergency Condition. The charge to each QSE for a given 15-minute Settlement Interval is calculated as follows:

LAEMREAMT q	=	(-1) * EMREAMTTOT * LRS q

Where:

EMREAMTTOT	=	ΣqEMREAMTQSETOT q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LAEMREAMT q | $ | Load-Allocated Emergency Energy Amount per QSE—The QSE q’s Load-Allocated amount of the total payments for all the Generation Resources with Real-Time Emergency Base Points, for the 15-minute Settlement Interval. |
| EMREAMTTOT | $ | Emergency Energy Amount TotalThe total of the payments to all QSEs as additional compensation for emergency power increases of the Generation Resources for the 15-minute Settlement Interval. |
| EMREAMTQSETOT q | $ | Emergency Energy Amount QSE Total per QSEThe total of the payments to QSE q as additional compensation for emergency power increases of the Generation Resources represented by this QSE for the 15-minute Settlement Interval. |
| LRS q | none | The Load Ratio Share calculated for QSE q for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |

6.6.10	Real-Time Revenue Neutrality Allocation

- (1)	ERCOT must be revenue-neutral in each Settlement Interval. Each QSE receives an allocated share, on a Load Ratio Share basis, of the net amount of:

(a)	Real-Time Energy Imbalance payments or charges under Section 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at  a Resource Node;

(b)	Real-Time Energy Imbalance payments or charges under Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone;

(c)	Real-Time Energy Imbalance payments or charges under Section 6.6.3.3, Real-Time Energy Imbalance Payment or Charges at a HUB;

(d)	Real-Time energy payments under Section 6.6.3.4, Real-Time Energy Payment for DC Tie Import;

(e)	Real-Time energy payments under Section 6.6.3.5, Payment for a Block Load Transfer Point;

(f)	Real-Time energy charge under Section 6.6.3.6, Charge for an Export Represented by a QSE under the Oklaunion Exemption;

(g)	Real-Time congestion payments or charges under Section 6.6.4, Real-Time Congestion Payment or Charge for Self Schedules;

(h)	Real-Time value of Day-Ahead energy sale from RMR Units under Section 6.6.6.5, RMR Service Charge;

(i)	Real-Time payments or charges to the CRR Owners under Section 7.9.2,  Real-Time CRR Payments and Charges; and

- (2)	The Real-Time Revenue Neutrality Allocation for each QSE for a given 15-minute Settlement Interval is calculated as follows:

LARTRNAMT q	=	(-1) * (RTEIAMTTOT + BLTRAMTTOT + RTDCIMPAMTTOT + RTDCEXPAMTTOT + RTCCAMTTOT + RMRDAESRTVTOT + RTOBLAMTTOT / 4 + RTOPTAMTTOT / 4 + RTOPTRAMTTOT /4) * LRS q

(3)	In the event that ERCOT is unable to execute the DAM, the Real-Time Revenue Neutrality Allocation for each QSE for a given 15-minute Settlement Interval is calculated as follows:

LARTRNAMT q	=	(-1) * (RTEIAMTTOT + BLTRAMTTOT + RTDCIMPAMTTOT + RTDCEXPAMTTOT + RTCCAMTTOT + RMRDAESRTVTOT + NDRTOBLAMTTOT / 4 + NDRTOPTAMTTOT / 4 + NDRTOPTRAMTTOT /4 + NDRTFGRAMTTOT /4 + NDRTOBLRAMTTOT /4) * LRS q

Where:

Total Real-Time Energy Imbalance payment (or charge) at Settlement Point (or Hub)

RTEIAMTTOT	=	ΣqRTEIAMTQSETOT q

- Total Real-Time payment for Block Load Transfer Resources

BLTRAMTTOT	=	ΣqBLTRAMTQSETOT q

- Total Real-Time payment for DC Tie Imports

RTDCIMPAMTTOT	=	ΣqRTDCIMPAMTQSETOT q

Total Real-Time charge for DC Tie Exports (under “Oklaunion Exemption”)

RTDCEXPAMTTOT	=	ΣqRTDCEXPAMTQSETOT q

Total Real-Time Congestion Payment or Charge for Self Schedules

RTCCAMTTOT	=	ΣqRTCCAMTQSETOT q

- Total Real-Time payment or charge for PTP Obligations

RTOBLAMTTOT	=	ΣqΣo RTOBLAMTQSETOT q

- Total Real-Time payment for PTP Options

RTOPTAMTTOT	=	ΣoRTOPTAMTOTOT o

- Total Real-Time payment for PTP Options with Refund

RTOPTRAMTTOT	=	ΣoRTOPTRAMTOTOT o

Total Real-Time payment or charge for PTP Obligations when ERCOT is unable to execute the DAM

NDRTOBLAMTTOT	=	Σo NDRTOBLAMTOTOT o

Total Real-Time payment for PTP Options when ERCOT is unable to execute the DAM

NDRTOPTAMTTOT	=	ΣoNDRTOPTAMTOTOT o

Total Real-Time payment for PTP Options with Refund when ERCOT is unable to execute the DAM

NDRTOPTRAMTTOT	=	ΣoNDRTOPTRAMTOTOT o

Total Real-Time payment for FGRs when ERCOT is unable to execute the DAM

NDRTFGRAMTTOT	=	Σo NDRTFGRAMTOTOT o

Total Real-Time payment or charge for PTP Obligations with Refund when ERCOT is unable to execute the DAM

NDRTOBLRAMTTOT	=	Σo NDRTOBLRAMTOTOT o

The above variables are defined as follows:

| Variable |  |  | Unit |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| LARTRNAMT q |  |  | $ |  | Load-Allocated Real-Time Revenue Neutrality Amount per QSE—The QSE q’s share of the total Real-Time revenue neutrality amount, for the 15-minute Settlement Interval. |  |
| RTEIAMTTOT |  |  | $ |  | Real-Time Energy Imbalance Amount Total The Total net payments and charges for Real-Time Energy Imbalance at all Settlement Points (Resource, Load Zone, or Hub) for the 15-Minute Interval. |  |
| BLTRAMTTOT |  |  | $ |  | Block Load Transfer Resource Amount TotalThe total of the payments for energy delivered into the ERCOT System through BLT Points for the 15-minute Settlement Interval. |  |
| RTDCIMPAMTTOT |  |  | $ |  | Real-Time DC Import Amount Total—The summation of payments for DC Tie import, for the 15-minute Settlement Interval. |  |
| RTDCEXPAMTTOT |  |  | $ |  | Real-Time DC Export Amount Total—The summation of charges to all QSE that are under the “Oklaunion Exemption” for DC Tie export, for the 15-minute Settlement Interval. |  |
| RTCCAMTTOT |  |  | $ |  | Real-Time Energy Congestion Cost Amount TotalThe total net congestion payments and charges for all of the Self-Schedules for the 15-minute Settlement Interval. |  |
| RMRDAESRTVTOT |  |  | $ |  | RMR Day-Ahead Energy Sale Real-Time Value Total—The total of the Real-Time value of the Day-Ahead energy sales from all RMR Units, for the 15-minute Settlement Interval. See Section 6.6.6, Reliability Must Run Settlement. |  |
| RTOBLAMTTOT |  |  | $ |  | Real-Time Obligation Amount Total—The sum of all payments and charges for PTP Obligations settled in Real-Time, for the hour that includes the 15-minute Settlement Interval. |  |
| RTOPTAMTTOT |  |  | $ |  | Real-Time Option Amount Total—The sum of all payments for PTP Options settled in Real-Time, for the hour that includes the 15-minute Settlement Interval. |  |
| RTOPTRAMTTOT |  |  | $ |  | Real-Time Option with Refund Amount Total—The sum of all payments for PTP Options with Refund settled in Real-Time, for the hour that includes the 15-minute Settlement Interval. |  |
| NDRTOBLAMTTOT |  |  | $ |  | No DAM Real-Time Obligation Amount Total—The sum of all payments and charges for PTP Obligations settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  |
| NDRTOPTAMTTOT |  |  | $ |  | No DAM Real-Time Option Amount Total—The sum of all payments for PTP Options settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  |
| NDRTOPTRAMTTOT |  |  | $ |  | No DAM Real-Time Option with Refund Amount Total—The sum of all payments for PTP Options with Refund settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  |
| NDRTFGRAMTTOT |  |  | $ |  | No DAM Real-Time FGR Amount Total— The sum of all payments for FGRs settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  |
| NDRTOBLRAMTTOT |  |  | $ |  | No DAM Real-Time Obligation with Refund Amount Total— The sum of all payments for PTP Obligations with Refund settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour that includes the 15-minute Settlement Interval. |  |
| RTEIAMTQSETOT q |  |  | $ |  | Real-Time Energy Imbalance Amount QSE Total per QSEThe total net payments and charges to QSE q for Real-Time Energy Imbalance at all Resource Node Settlement Points for the 15-minute Settlement Interval. |  |
| RTCCAMTQSETOT q |  |  | $ |  | Real-Time Congestion Cost Amount QSE Total per QSEThe total net congestion payments and charges to QSE q for its Self-Schedules for the 15-minute Settlement Interval. |  |
| BLTRAMTQSETOT q |  |  | $ |  | Block Load Transfer Resource Amount QSE Total per QSEThe total of the payments to QSE q for energy delivered into the ERCOT System through BLT Points for the 15-minute Settlement Interval. |  |
| RTDCIMPAMTQSETOT q |  |  | $ |  | Real-Time DC Import Amount QSE Total per QSEThe total of the payments to QSE q for energy imported into the ERCOT System through DC Ties for the 15-minute Settlement Interval. |  |
| RTDCEXPAMTQSETOT q |  |  | $ |  | Real-Time DC Export Amount QSE Total per QSEThe total of the charges to QSE q for energy exported from the ERCOT System through DC Ties for the 15-minute Settlement Interval. |  |
| RTOBLAMTQSETOT q |  |  | $ |  | Real-Time Obligation Amount QSE Total per QSE—The net total payment or charge to QSE q of all its PTP Obligations settled in Real-Time, for the hour that includes the 15-minute Settlement Interval. See Section 7.9.2.1, Payments and Charges for PTP Obligations Settled in Real-Time, paragraph (2). |  |
| RTOPTAMTOTOT o |  |  | $ |  | Real-Time Option Amount Owner Total per owner—The total payment for all the PTP Options, held by the CRR Owner o and settled in Real-Time, for the hour that includes the 15-minute Settlement Interval. See Section 7.9.2.2, Payments for PTP Options Settled in Real-Time, paragraph (2). |  |
| RTOPTRAMTOTOT o |  |  | $ |  | Real-Time Option with Refund Amount Owner Total per owner —The payment for the PTP Options with Refund, held by the CRR Owner o and settled in Real-Time, for the hour that includes the 15-minute Settlement Interval. See Section 7.9.2.3, Payments for NOIE PTP Options with Refund Settled in Real-Time, paragraph (2). |  |
| NDRTOBLAMTOTOT o |  |  | $ |  | No DAM Real-Time Obligation Amount Owner Total per CRR Owner—The net total payment or charge to CRR Owner o of all its PTP Obligations settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |  |
| NDRTOPTAMTOTOT o |  |  | $ |  | No DAM Real-Time Option Amount Owner Total per CRR Owner—The total payment to CRR Owner o for all its PTP Options settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |  |
| NDRTOPTRAMTOTOT o |  |  | $ |  | No DAM Real-Time Option with Refund Amount Owner Total per CRR Owner—The total payment to NOIE CRR Owner o for all its PTP Options with Refund settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |  |
| NDRTFGRAMTOTOT o |  |  | $ |  | No DAM Real-Time FGR Amount Owner Total per CRR Owner—The total payment to CRR Owner o of all its FGRs settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |  |
| NDRTOBLRAMTOTOT o |  |  | $ |  | No DAM Real-Time Obligation with Refund Amount Owner Total per CRR Owner—The net total payment or charge to CRR Owner o for all its PTP Obligations with Refund settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour. |  |
| LRS q |  |  | none |  | The Load Ratio Share calculated for QSE q for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |  |
| q |  |  | none |  | A QSE. |  |
| o |  |  | none |  | A CRR Owner. |  |