| NPRR Number | 069 | PRR Title | Changes to SURAMP |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Recommended Action |  | Approval |
| Nodal Protocol Section(s) Requiring Revision |  | 6.5.7.2, Resource Limit Calculator |  |  |
| Proposed Effective Date |  | Upon Texas Nodal Market implementation |  |  |
| Priority and Rank Assigned |  | N/A |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) revises the Security Constrained Economic Dispatch (SCED) Up Ramp Rate (SURAMP) formulation. |  |  |
| Overall Market Benefit |  | This NPRR will allow efficient use of Resources’ Ramp Rates in Real-Time dispatch and avoids unnecessarily increasing dispatch costs. |  |  |
| Overall Market Impact |  | Minor changes to requirements. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts |  | ERCOT credit staff and the Credit Work Group (Credit WG) have reviewed NPRR069 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - NPRR069 was posted on 6/15/07. - On 6/21/07, ERCOT Staff presented the Impact Analysis (IA) for NPRR069. - On 6/21/07, PRS considered this NPRR. - On 6/28/07, TAC considered this NPRR. |  |  |
| PRS Recommendation |  | On 6/21/07, PRS voted to recommend approval of NPRR069 as amended by Transition Plan Task Force (TPTF) comments, endorse the IA and forward documents to TAC. The motion passed with one abstention from the (IOU) Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 6/21/07, the TPTF submitted comments with proposed revisions to the “Revision Description” and protocol language. |  |  |
| TAC Recommendation |  | On 6/28/07, TAC voted to recommend approval of NPRR069 and endorse the Baseline 2 IA as recommended by PRS. The motion passed unanimously with all Market Segments present for the vote. |  |  |
| Summary of TAC Discussion |  | On 06/28/07, TAC inquired whether the cost estimates are preliminary. ERCOT Staff explained that these are final estimates that may be on the high side and may be reduced as implementation strategies are refined. ERCOT Staff emphasized that delays in the approval process or piece-meal approval of cost estimates will make negotiations with the vendors difficult, may create risk to the schedule, and may cause cost increases. |  |  |

ERCOT/Market Segment Impacts and Benefits

| Assumptions | 1 | n/a |  |
| --- | --- | --- | --- |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | None |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Correcting a technical error |  |
| Additional Qualitative Information | 1 | None |  |
| Other Comments | 1 | None |  |

| Original Sponsor |  |
| --- | --- |
| Name | Jeyant Tamby |
| Company | ERCOT |
| Segment | n/a |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| TPTF 062107 | Proposed revised language. |

Proposed Protocol Language Revision

6.5.7.2	Resource Limit Calculator

(1)	ERCOT shall calculate the HASL, LASL, SCED Up Ramp Rate (SURAMP), SCED Down Ramp Rate (SDRAMP), HDL and LDL within 4 seconds after a change of the Resource-specific attributes provided as part of the QSE’s SCADA telemetry under Section 6.5.5.2, Operational Data Requirements.  The formulas described below define which Resource-specific attributes must be used to calculate each Resource limit.  The Resource limits are used as inputs into both the SCED process and the Ancillary Service Capacity Monitor as described in Section 6.5.7.6, Load Frequency Control.  These Resource limits help ensure that the deployments produced by the SCED and LFC processes will respect the commitment of a Resource to provide Ancillary Services as well as individual Resource physical limitations.

(2)	The figure below illustrates how the Resource Limit Calculator determines the Resource limits for both Generation and Load Resources:

LSL

HSL

Time

HSL

-

LSL

-

0

LASL

-

HASL

-

Generation

Increase

Generation

Decrease

Ancillary

services

provided: Reg

Down

Ancillary services

provided: Reg Up,

Responsive, Non

-

Spin

(as applicable)

Current

Telemetry

HDL

LDL

Ramp

Rate

5 Minutes

HSL=

-

LPC

LSL=

-

MPC

LASL

-

HASL

-

Consumption

Decrease

Consumption

Increase

Current load

Telemetry

Ancillary services

provided: Reg Down

Ancillary services

provided: Reg Up,

Responsive

, Non-Spin

Generation

5

-

30

Minutes

Load Resources

Sign Convention for Load Resources

Normal Load

fluctuation

Quantity

Offer Curve Generation

LSL

HSL

-

-

0

-

-

Generation

Increase

Generation

Decrease

-

Spin

Ramp

Rate

5 Minutes

-

-

-

-

Consumption

Decrease

Consumption

Increase

5

-

30

-

-

0

-

-

Generation

Increase

Generation

Decrease

-

Spin

Ramp

Rate

5 Minutes

-

-

-

-

Consumption

Decrease

Consumption

Increase

5

-

30

Price

(3)	High Ancillary Service Limit (HASL) is calculated as follows:

HASL	=	Max (LASL, (HSLTELEM – (RRSTELEM + RUSTELEM + NSRSTELEM)))

| Variable | Description |
| --- | --- |
| HASL | High Ancillary Service Limit |
| HSLTELEM | High Sustained Limit provided via telemetry – per Section 6.5.5.2 Operational Data Requirements |
| LASL | Low Ancillary Service Limit |
| RRSTELEM | Responsive Reserve designation provided by telemetry |
| RUSTELEM | Reg-Up designation provided by telemetry |
| NSRSTELEM | Non-Spin designation provided via telemetry; ERCOT shall set the NSRSTELEM to adjust for deployed Non-Spin on On-Line Generation Resources |

(4)	Low Ancillary Service Limit (LASL) is calculated as follows:

LASL	= 	LSLTELEM + RDSTELEM

| Variable | Description |
| --- | --- |
| LASL | Low Ancillary Service Limit |
| LSLTELEM | Low Sustained Limit provided via telemetry |
| RDSTELEM | Reg-Down designation provided by telemetry |

(5)	For each Generation Resource, the SURAMP is calculated as follows:

SURAMP	= 	 [RAMPRATE– (RUSTELEM / 5)]

| Variable | Description |
| --- | --- |
| SURAMP | SCED up ramp rate |
| RAMPRATE | = Normal Ramp Rate when RRS is not deployed or when the subject resource is not providing RRS. = Emergency Ramp Rate for Resources deploying RRS. |
| RUSTELEM | Reg-Up designation provided by telemetry |

(6)	For each Generation Resource, the SDRAMP is calculated as follows:

SDRAMP	= 	NORMRAMP – (RDSTELEM / 5)

| Variable | Description |
| --- | --- |
| SDRAMP | SCED down ramp rate |
| NORMRAMP | Normal Ramp Rate |
| RDSTELEM | Reg-Down designation by Resource provided via telemetry |

(7)	For Generation Resources, HDL is calculated as follows:

HDL	= 	Min (POWERTELEM + (SURAMP * 5), HASL)

| Variable | Description |
| --- | --- |
| HDL | High Dispatch Limit |
| POWERTELEM | Gross or net real power provided via telemetry |
| SURAMP | SCED Up Ramp Rate – |
| HASL | High Ancillary Service Limit – definition provided in Section 2, Definitions and Acronyms. |

(8)	For Generation Resources, LDL is calculated as follows:

LDL	= 	Max (POWERTELEM - (SDRAMP * 5), LASL)

| Variable | Description |
| --- | --- |
| LDL | Low Dispatch Limit |
| POWERTELEM | Gross or net real power provided via telemetry |
| SDRAMP | SCED Down Ramp Rate – |
| LASL | Low Ancillary Service Limit – definition provided in Section 2, Definitions and Acronyms. |