| PRR Number | 730 | PRR Title | Balancing Energy Price Adjustment for Deployment of Replacement Reserve Service |
| --- | --- | --- | --- |
| Date Posted |  | June 19, 2007 |  |
| Protocol Section(s) Requiring Revision (include Section No. and Title) |  | 6.8.1.12, Payments for Balancing Energy Provided from Ancillary Services During the Operating Period 6.9.5.1, Balancing Energy Clearing Price |  |
| Requested Resolution (Normal or Urgent) |  | Normal |  |
| Revision Description |  | This PRR adjusts the Market Clearing Price for Energy (MCPE) when Replacement Reserve Service (RPRS) is deployed. This PRR is intended to be similar to PRR 650, Balancing Energy Price Adjustment Due to Non-Spinning Reserve Service Energy Deployment, which removes non-spin deployments from the bid stack before calculating the MCPE. |  |
| Reason for Revision |  | The balancing energy market prices are being depressed when ERCOT procures RPRS. This PRR would address this problem by removing the RPRS deployments when calculating the MCPE. One of the sources of revenue for new or existing resources is the real-time energy market that is operated by ERCOT. Each time ERCOT procures RPRS, the MCPE is depressed, which is an unintended consequence of ERCOT’s actions and was clearly not expected when the PUCT created an energy-only market. |  |
| Overall Market Benefit |  | Will send an accurate price signal to the market when ERCOT procures RPRS. |  |
| Overall Market Impact |  | May increase the MCPE. |  |
| Consumer Impact |  | Will benefit consumers by providing accurate price signals for customers that are willing to interrupt their load. In addition, entities interested in investing in new generation in ERCOT, will see a more accurate price signal. |  |
| Credit Implications (Yes or No, and summary of impact) |  | This PRR should not impact credit monitoring by ERCOT. |  |
| Relevance to Nodal Market (Yes or No, and summary of impact) |  | No |  |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  | Not applicable |  |

| Sponsor |  |
| --- | --- |
| Name | Jeff Brown |
| E-mail Address | Jeff.A.Brown@Shell.com |
| Company | Coral Power, L.L.C. |
| Company Address | 909 Fannin Street, Houston, TX 77010 |
| Phone Number | 713-767-5317 |
| Cell Number | 713-582-3750 |
| Market Segment | Independent Power Marketer |

ERCOT/Market Segment Impacts and Benefits

| Assumptions | 1 | ERCOT’s use of RPRS is depressing the Market Clearing Price for Energy. |  |
| --- | --- | --- | --- |
|  | 2 | The PUCT did not anticipate that ERCOT would depress price signals through RPRS when they adopted an energy-only market design. |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | May increase the MCPE |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Provides accurate price signals |  |
| Additional Qualitative Information | 1 |  |  |
| Other Comments | 1 | Should not impact ERCOT’s systems or staff if this PRR is implemented similar to PRR 650. |  |

Proposed Protocol Language Revision

6.8.1.12	Payments for Balancing Energy Provided from Ancillary Services During the Operating Period

All Balancing Energy deployed by a Dispatch Instruction and delivered by a generating Resource from Balancing Energy service, Responsive Reserve Service, Regulation Reserve Service, or Non-Spinning Reserve Service shall be settled as Balancing Energy at the MCPE of the Congestion Zone of the Resource providing the energy according to Section 6.8.1.13, Resource Imbalance, except when Non-Spinning Reserve Service is deployed in accordance with Section 6.7.4, Deployment of Non-Spinning Reserve Service, or Replacement Reserve Service is deployed in accordance with Section 6.7.5, Deployment of Replacement Reserve Service, and there are no binding CSC constraints for that interval.  In that case, the MCPE shall be the MCPE that would have occurred if the deployments of Non-Spinning Reserve Service or Replacement Reserve Service did not occur.  All Balancing Energy deployed by a Dispatch Instruction and delivered by a Load acting as Balancing Energy service, Responsive Reserve Service, Regulation Reserve Service, or Non-Spinning Reserve Service shall be settled as Balancing Energy at the MCPE of the Congestion Zone of the Resource providing the energy according to Section 6.9.5.2, Settlement for Balancing Energy for Load Imbalance, except when Non-Spinning Reserve Service is deployed in accordance with Section 6.7.4, Deployment of Non-Spinning Reserve Service, or Replacement Reserve Service is deployed in accordance with Section 6.7.5, Deployment of Replacement Reserve Service, and there are no binding CSC constraints for that interval.  In that case, the MCPE shall be the MCPE that would have occurred if the deployments of Non-Spinning Reserve Service or Replacement Reserve Service did not occur.  Settlement for any energy provided without an authorized Dispatch Instruction will be settled according to Section 6.8.1.15, Payments for Balancing Energy Provided from Uninstructed Deviation.

Deployed Balancing Energy Up on qualified Balancing Up Loads will be paid a capacity payment for the first Settlement Interval that it is deployed equal to the MCPC of Non-Spinning Reserve Service for the hour in which the deployment occurs.  A continuous deployment of Balancing Energy Up on qualified Loads for over sixty (60) minutes will be paid a capacity payment for each subsequent Settlement Interval which it is deployed equal to the MCPC of Non-Spinning Reserve Service for that hour divided by four (4).

6.9.5.1	Balancing Energy Clearing Price

(1)	A Market Clearing Price for Energy (MCPE) will be calculated for each Settlement Interval by Congestion Zone as a product of the mathematical optimization model.  The MCPE will be used to pay or charge each QSE, for each Settlement Interval for Balancing Energy Service.

(2)	In the event that Non-Spinning Reserve Service (NSRS) is deployed for an interval in accordance with Section 6.7.4, Deployment of Non-Spinning Reserve Service or Replacement Reserve Service is deployed in accordance with Section 6.7.5, Deployment of Replacement Reserve Service, ERCOT shall evaluate if there were binding CSC constraints during the original market clearing of Balancing Energy Services, or if removal of the NSRS or RPRS deployments would create binding CSC constraints.  If no binding CSC constraints exist in either evaluation, ERCOT shall adjust the MCPE by disregarding the deployments of NSRS or RPRS and procuring the Balancing Energy service that would have been necessary. Afterwards, a MCPE will be calculated for each Settlement Interval by Congestion Zone as a product of the mathematical optimization model.  The MCPE will be used to pay or charge each QSE, for each Settlement Interval for Balancing Energy Service.