| NPRR Number | 029 | NPRR Title | Network Model Testing Clarification |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Approved Baseline 1 Group 11 Impact Analysis |
| Nodal Protocol Sections Requiring Revision |  | 3.10.1, Time Line for Network Operations Model Change Requests 3.10.4, ERCOT Responsibilities |  |  |
| Effective Date |  | Upon Texas Nodal Market implementation. |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Revision Description |  | This revision divides the model testing into Real-Time testing and simulation testing. |  |  |
| Overall Market Benefit |  | Clarification of testing processes and requirements. |  |  |
| Overall Market Impact |  | None. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts: Has the Credit Workgroup reviewed the PRR? |  | No. |  |  |
| Procedural History |  | - On 9/15/06, NPRR029 and its Impact Analysis (IA) were posted. - On 9/21/06, PRS considered this NPRR. - On 10/6/06, TAC considered this NPRR. - On 10/17/06, the ERCOT Board of Directors (Board) considered this NPRR. - On 6/8/07, Baseline 1, Group 11 (B1G11) IA was posted. - On 6/12/07, PRS reviewed the B1G11 IA. - On 6/28/07, TAC considered the B1G11 IA. - On 7/17/07, the Board considered the B1G11 IA. |  |  |
| PRS Recommendation |  | On 9/21/06, PRS unanimously voted to recommend approval of NPRR029 as submitted. All Market Segments were present for the vote. On 06/12/07, PRS unanimously voted to forward the B1G11 IA as modified by PRS to TAC for approval. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 9/21/06, there was no discussion. On 6/12/07, PRS proposed revisions to the B1G11 IA that were acceptable to ERCOT Staff. |  |  |
| TAC Recommendation |  | On 10/6/06, TAC voted unanimously to recommend approval of NPRR029 as recommended by PRS. All Market Segments were present for the vote. On 6/28/07, TAC voted to recommend approval of the B1G11 IA as recommended by PRS. There were four (4) abstentions from the Consumer Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 10/6/06, there was no discussion. On 6/28/07, there was no discussion. |  |  |
| Board Action |  | On 10/17/06, the Board approved NPRR029 as recommended by TAC. On 7/17/07, the Board approved the B1G11 IA as recommended by TAC. |  |  |

| ERCOT/Market Segment Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 | None |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | None |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | None | None |
| Additional Qualitative Information | 1 | This NPRR clarifies testing processes and requirements. |  |
| Other | 1 | None |  |

| Sponsor |  |
| --- | --- |
| Name | John Moseley on behalf of ERCOT Staff |
| Company | ERCOT |
| Market Segment | N/A |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| None |  |

Proposed Nodal Protocol Language Revision

3.10.1		Time Line for Network Operations Model Change Requests

(1)	ERCOT shall perform periodic updates to the ERCOT Network Operations Model.  Market Participants may provide Network Operations Model updates to ERCOT to implement planned transmission and Resource construction one year before the required submittal date below.  TSPs and Resource Entities must timely submit Network Operations Model changes pursuant to the schedule in this Section to be included in the updates.

(2)	For a facility addition, revision, or deletion to be included in any Network Operations Model update, all technical modeling information must be submitted to ERCOT pursuant to the ERCOT NOMCR process.

(3)	TSPs and Resource Entities shall submit Network Operations Model updates at least three months prior to the physical equipment change.  ERCOT shall update the Network Operations Model according to the following table:

| Deadline to Submit Information to ERCOT Note 1 | Model Complete and Available for Test Note 2 | Updated Network Operations Model Testing Complete Note 3 | Update Network Operations Model Production Environment | Target Physical Equipment In-Service Date Note 4 |
| --- | --- | --- | --- | --- |
| Jan 1 | Feb 15 | March 15 | April 1 | Month of April |
| Feb 1 | March 15 | April 15 | May 1 | Month of May |
| March 1 | April 15 | May 15 | June 1 | Months of June–August SUMMER MODEL |
| June 1 | July 15 | August 15 | September 1 | Month of September |
| July 1 | August 15 | September 15 | October 1 | Month of October |
| August 1 | September 15 | October 15 | November 1 | Month of November |
| September 1 | October 15 | November 15 | December 1 | Month of December |
| October 1 | November 15 | December 15 | January 1 | Month of January (the next year) |
| November 1 | December 15 | January 15 | February 1 | Month of February (the next year) |
| December 1 | January 15 | February 15 | March 1 | Month of March (the next year) |

Notes:

1.	Transmission and Resource data submissions complete per ERCOT Network Operations Model Change Request process for inclusion in next update period.

2.	Network Operations Model data changes and preliminary fidelity test complete by using the Network Operations Model test facility described in Section 3.10.4.3.  The test version of the Network Operations Model will be available for market review and further testing by Market Participants.

3.	Testing of the Network Operations Model by Market Participants is complete and ERCOT begins the Energy Management System testing prior to placing the new model into the production environment.

4.	Updates include changes starting at this date and ending within the same month.

(4) 	ERCOT shall only approve energization requests when the Transmission Element is satisfactorily modeled in the Network Operations Model.

(5)	In order to allow for construction schedules, TSPs may use pseudo-breakers and switches to designate future facilities configurations such that the Network Operations Model topology may be correctly implemented as construction of new facilities is accomplished.

3.10.4	ERCOT Responsibilities

(1)	ERCOT shall design, install, operate, and maintain its systems and establish applicable related processes to meet the TAC-approved State Estimator (SE) performance standard for Transmission Elements that under typical system conditions potentially affect the calculation of LMPs as described in Section 3.10.7.4, Telemetry Criteria, and Section 3.10.9, State Estimator Performance Standard.  ERCOT shall post all documents relating to the State Estimator Performance Standard on the MIS Secure Area.

(2)	During Real-Time, ERCOT shall calculate LMPs and take remedial actions to ensure that actual flow on a given Transmission Element is less than the Normal Rating and any calculated flow due to a contingency is less than the applicable Emergency Rating and 15-Minute Rating.

(3) 	ERCOT shall install Network Operations Model test facilities that will accommodate execution of a test Real Time Sequence and preliminary test LMP calculator to demonstrate the correct operation of new Network Operations Models prior to releasing the model to Market Participants for detail testing and verification.  The Network Operations Model test facilities support power flow and contingency analyses to test the data set representation of a proposed transmission model update and simulates LMP calculations using typical test data.

(4)	ERCOT shall install Energy Management System test and simulation facilities that accommodate execution of the State Estimator and LMP calculator, respectively.  These facilities will be used to conduct tests prior to placing a new model into ERCOT’s production environment to verify the new model’s accuracy.  The Energy Management System test facilities allow a potential model to  be tested before replacing the current production environment model.  The Energy Management System test and simulation facilities must perform Real Time Security Analysis to test a proposed transmission model before replacing the current production environment model.  The Energy Management System State Estimation test facilities must have Real-Time ICCP and SCADA links to test the state estimation function using actual Real-Time conditions.  The Energy Management System LMP Test Facilities must accept data uploads from the production environment providing QSE Resource offers, and SCADA telemetry.  If the production data are unavailable, ERCOT may employ a data simulation tool or process to develop test data sets for the LMP Test Facilities.    ERCOT shall acquire model comparison software that will show all differences between the next production model and production environment model and shall post this information on the MIS Secure Area within one week following test completion.  This comparison shall indicate differences in device parameters, missing or new devices, and status changes.

(4)	When implementing Transmission Element changes, ERCOT shall correct errors uncovered during testing that are due to submission of inaccurate information.  Each TSP shall provide reasonably accurate information at the time of the original submission.  ERCOT may update the model on an interim basis, outside of the timeline described in Section 3.10.1, Time Line for Network Operations Model Change Requests, for the correction of temporary configuration changes in a system restoration situation, such as after a storm, or correction of impedances and ratings.  Interim updates to the Network Operations Model caused by unintentional inconsistencies of the model with the physical transmission grid may be made.  If an interim update is implemented, ERCOT shall report changes to the PUCT staff and the WEMM.  ERCOT shall provide notice via electronic means to all Market Participants and post the notice on the MIS Secure Area detailing the changed model information and the reason for the interim update within two Business Days following the report to PUCT staff and WEMM.

(5)	When ERCOT identifies active or binding transmission constraints on a repeated basis, ERCOT shall contact the appropriate TSP to:

(a)	Verify that ratings of Transmission Facilities in the Network Operations Model and in the Updated Network Model causing the event are current and correctly represented;

(b)	Verify, when the TSP’s analysis results differ from those of ERCOT, that the configuration of the Transmission Grid in the Network Operations Model and in the Updated Network Model matches that in use by the TSP.  To recognize operational time constraints, that verification must focus on Transmission Elements believed to have affected the event; and

(c)	Mutually identify with the TSP any additional operational intervention or system monitoring that could be implemented to manage recurring congestion due to a recurring cause.

(6)	ERCOT shall make available to TSPs and other Market Participants, consistent with applicable policies regarding release of CEII, the full transmission model used to manage the reliability of the transmission system as well as proposed models to be implemented at a future date.  ERCOT shall provide model information through the use of the Electric Power Research Institute (EPRI) and NERC-sponsored Common Information Model (CIM) and Web-based XML communications.