| NPRR Number | 064 | NPRR Title | Modeling of Generic Transmission Constraints |
| --- | --- | --- | --- |
| Timeline | Normal | Action | Approved |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms and Abbreviations 3.10, Network Operations Modeling and Telemetry 3.10.7.5, Modeling of Generic Transmission Limits (new) |  |
| Effective Date |  | Upon Texas Nodal Market implementation. |  |
| Priority and Rank Assigned |  | N/A. |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) provides for the definition of and posting of Generic Transmission Limits (GTLs) in Network Operations Modeling requirements. |  |
| Overall Market Benefit |  | Generic constraints are typically more limiting than a Transmission Element’s thermal limit and can have significant impact on Locational Marginal Prices (LMPs) when they constrain a solution. This NPRR provides for the modeling of these constraints and provides notice to the Market Participants (MPs) through postings to the Market Information System (MIS). |  |
| Overall Market Impact |  | Not known. |  |
| Consumer Impact |  | Not known. |  |
| Credit Impacts |  | ERCOT credit staff and the Credit Work Group (Credit WG) have reviewed NPRR064 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| Procedural History |  | - On 5/29/07, NPRR064 was posted. - On 6/01/07, ERCOT comments were posted. - On 6/21/07, ERCOT staff presented the Impact Analysis (IA) fro NPRR064. - On 6/21/07, PRS considered this NPRR, ERCOT comments and the IA. - On 06/28/07, TAC considered this NPRR. - On 7/17/07, the ERCOT Board of Directors (Board) considered this NPRR. |  |
| PRS Recommendation |  | On 6/21/07, PRS unanimously voted to recommend approval of NPRR064 as modified by ERCOT comments, to endorse the IA and to forward the documents to TAC. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 6/21/07, ERCOT staff reviewed the NPRR and explained ERCOT comments. |  |
| TAC Recommendation |  | On 06/28/07, TAC unanimously voted to recommend approval of NPRR064 and its IA as recommended by PRS. All Market Segments were present for the vote. |  |
| Summary of TAC Discussion |  | On 06/28/07, a concern was raised regarding the vagueness of the IA that it had only been posted for a few days. It was asked what would be the impact of deferring action until the next meeting. ERCOT explained that deferring action will increase costs and cause schedule delays. ERCOT explained that no additional staff is required to implement this NPRR beyond what is already contemplated to be required for nodal go live. |  |
| Board Action |  | On 7/17/07, the Board approved NPRR064 as recommended by TAC. |  |

| ERCOT/Market Segment Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 | None. |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | Not known. |  |
|  | 2 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Better modeling. |  |
|  | 2 | Notices through MIS postings. |  |
| Additional Qualitative Information | 1 | None. |  |
| Other | 1 | None. |  |
| Comments | 2 |  |  |

| Original Sponsor |  |
| --- | --- |
| Name | Trip Doggett on behalf of TPTF |
| E-mail Address | tdoggett@ercot.com |
| Company | ERCOT |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| ERCOT060107 | Proposed changes to the timing of posting requirements for GTLs. |

Revised Proposed Nodal Protocol Language

## DEFINITIONS

Generic Transmission Limit (GTL)

A transmission flow limit more constraining than a Transmission Element’s Normal Limit established to constrain flow between geographic areas of the ERCOT Transmission System that is used to enforce stability and voltage constraints that can not be modeled directly in ERCOT’s transmission security analysis applications.

## ACRONYMS AND ABBREVIATIONS

GTL		Generic Transmission Limit

3.10	Network Operations Modeling and Telemetry

(1)	ERCOT shall use the physical characteristics, ratings, and operational limits of all Transmission Elements of the ERCOT Transmission Grid and other information from the TSPs to specify limits within which the transmission network is defined in the network models made available to Market Participants on the MIS Secure Area and used to operate the ERCOT Transmission Grid as updated.

(2)	Because the ERCOT market requires accurate modeling of Transmission Elements in order to send reasonably accurate Base Points and pricing signals to Market Participants, ERCOT shall manage the Network Operations Model.  By providing Base Points and pricing signals by Electrical Bus to Market Participants, the Market Participants’ responses result in power flows on all Transmission Elements that ERCOT must monitor and, if necessary for reliability reasons, manage within ratings provided by the TSP and limits assigned by ERCOT including Generic Transmission Limits (GTLs) as may be defined in Section 3.10.7.x, Modeling of Generic Transmission Limits.

(3)	TSPs shall provide ERCOT with equipment ratings and update the ratings as required by ERCOT.  ERCOT shall post all equipment ratings on the MIS Secure Area no later than the day prior to the ratings becoming effective including the identity of the Transmission Element, old rating and new rating, effective date, and a text reason supplied by the appropriate TSP(s) for the rating change. ERCOT may request TSPs to provide detailed information on the methodology, including data for determination of each requested rating.  ERCOT may review and comment on the methodology.  ERCOT shall post all methodologies on the MIS Secure Area within seven days following a change in methodology.

(4)	ERCOT must use system ratings consistent with the ratings expected to be used during Real-Time for the system condition being modeled, including Dynamic Ratings using expected temperatures for those system conditions. For each model, ERCOT shall post ratings and the ambient temperatures used to calculate the ratings on the MIS Secure Area when the model is published.

(5)	ERCOT shall use consistent information within and between the various models used by ERCOT Operations, ERCOT Planning, and other workgroups in a manner that yields consistent results.  For operational and planning models that are intended to represent the same system state the results should be consistent and the naming should be identical.  An independent audit must be performed at least annually to confirm that consistent information is used in all ERCOT Operations models.

(6)	ERCOT shall use a Network Operations Model Change Request (NOMCR) process to control all information entering the Network Operations Model.  In order to allow for construction schedules, each NOMCR must be packaged as a single package describing any incremental changes and referencing any prerequisite NOMCRs, using an industry standard data exchange format. A package must contain a series of instructions that define the changes that need to be made to implement a network model modification.  ERCOT shall verify each package for completeness and accuracy prior to the period it is to be implemented.

(7)	ERCOT shall use an automated process to manage the CIM-compliant packages loaded into the Network Operations Model as each construction phase is completed.    ERCOT shall reject any NOMCRs that are not CIM compliant.  Each CIM compliant NOMCR must also be associated with commands to update the graphical displays associated with the network model modification.  During the testing phase, each NOMCR must be tested for proper sequencing and its effects on downstream applications.

(8)	ERCOT shall track each request received from TSPs and Resources via the NOMCR process, through implementation and final testing of the change.  ERCOT shall notify each NOMCR requestor when the requested change is processed and implemented in accordance with Section 3.10.1, Time Line for Network Operations Model Change Requests, ERCOT shall also provide the submitting TSP a link to a network model containing the change for verifying the implementation of the NOMCR and associated one-line displays. ERCOT shall post all NOMCRs on the MIS Secure Area within five Business Days following receipt of the NOMCR, consistent with CEII standards.  When posting a NOMCR, each change must be posted using the CIM data exchange format showing incremental changes to the last posted ERCOT Network Operations Model, to facilitate TSPs and other Market Participants in updating their internal network models to reflect changes made at ERCOT.  For each NOMCR, ERCOT shall post on the MIS Secure Area current status on the in-service date, including any prerequisite NOMCRs provided by the requestor.

(9)	ERCOT shall update the Network Operations Model under this Section and coordinate it with the Planning Models for consistency to the extent applicable.

(10)	Any requestor of an NOMCR must receive approval from ERCOT of an NOMCR before connecting of any associated equipment to the ERCOT Transmission Grid.  ERCOT shall notify a requestor of any deficiencies in its NOMCR.    ERCOT shall accept corrections to the NOMCR if the requestor has corrected any deficiencies by the required submittal date.  ERCOT shall post any changes to an NOMCR on the MIS Secure Area within three Business Days of accepting corrections.

(11)	On receipt of the information set forth in Section 3.10.7, ERCOT System Modeling Requirements, ERCOT shall review the information and notify the requestor of any required modifications.  ERCOT may, at its discretion, require changes or more details regarding the work plan for the NOMCR.  The requestor shall notify ERCOT and any other affected Entities as soon as practicable of any requested changes to the work plan defined in the NOMCR.  The requestor shall consult with other Entities likely to be affected and shall revise the work plan, following any necessary or appropriate discussions with ERCOT and other affected Entities.  ERCOT shall approve or reject the request, including any revisions made by the requestor, within 15 days of receipt of the complete request and any revisions.  Following ERCOT approval, ERCOT shall publish a summary of the NOMCR on the MIS Secure Area.

3.10.7.5	Modeling of Generic Transmission Limits

(1)	For the sole purpose of creating transmission flow constraints between areas of the ERCOT Transmission Grid for systems unable to recognize system stability limits and voltage limits on Electrical Buses, ERCOT may create GTLs for use in reliability and market analysis. ERCOT shall not use GTLs for constraints in ERCOT systems capable of being directly modeled as security actions required to enforce stability and voltage limits.

(2)	Except as provided in (3) below, ERCOT shall post all GTLs on the MIS Secure Area no later than the day prior to the GTL becoming effective in any ERCOT application. Posting of GTLs shall include the identity of all constrained transmission flows, old limits, if applicable, and new flow limits, effective date, and an explanation for each flow limit and for the change, if applicable. ERCOT shall provide detailed information on the methodology, including data and studies used for determination of each GTL, on the MIS Secure Area.  Market Participants may review and comment on ERCOT’s methodology.  Within seven days following receipt of any comments, ERCOT shall post the comments with the subject GTL.

(3)	If an unexpected change to ERCOT System conditions requires a new GTL to manage ERCOT System reliability not posted pursuant to (2) above, ERCOT will declare an Alert and post on the MIS Secure Area the new GTL, including the detail information described above. ERCOT shall include an explanation regarding why it did not post the limit change on the previous day.