| NPRR Number | 050 | NPRR Title | Clarifications for HSL Values for WGRs and WGR Values to be Used in the RUC Capacity Short Calculation |
| --- | --- | --- | --- |
| Timeline | Normal | Action | Approved |
| Nodal Protocol Sections Requiring Revision |  | 4.2.2, Wind-Powered Generation Resource Production Potential 5.7.4.1.1, Capacity Shortfall Ratio Share |  |
| Proposed Effective Date |  | Upon implementation of Texas Nodal Market. |  |
| Priority and Rank Assigned |  | Not Applicable. |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) proposes the following clarifications: - Eliminate item (6) of Section 4.2.2because of inconsistency with Protocol language in item (7) of Section 3.9.1, Current Operating Plan (COP) Criteria (“A QSE representing a Wind-Powered Generation Resource (WGR) must enter an High Sustainable Limit (HSL) value that is less than or equal to the amount for that Resource from the most recent Wind-Powered Generation Resource Production Potential provided by ERCOT”); - Eliminate item (7) of Section 4.2.2 because the settlement information is described in Section 5.7.4.1, Capacity Shortfall Ratio Share; and - Revise Section 5.7.4.1.1 to clarify how to do the settlements. |  |
| Overall Market Benefit |  | Not known. |  |
| Overall Market Impact |  | Not known. |  |
| Consumer Impact |  | Not known. |  |
| Credit Impacts |  | ERCOT credit staff and the Credit Work Group (CWG) have reviewed NPRR050 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |
| Procedural History |  | - On 03/12/07, NPRR050 was posted. - On 03/22/07, PRS considered this NPRR. - On 06/05/07, the Baseline 1, Group 1 (B1G1) IA was posted. - On 06/12/07, the PRS reviewed this NPRR and its B1G1 IA. - On 06/28/07, TAC considered this NPRR and its B1G1 IA. - On 7/17/07, the ERCOT Board of Directors (Board) considered this NPRR and its B1G1 IA. |  |
| PRS Action |  | On 03/22/07, PRS unanimously voted to table NPRR050 until the next PRS meeting. All Market Segments were present for the vote. On 06/12/07, PRS unanimously voted to recommend approval of NPRR050 and to forward it and its B1G1 IA to TAC for approval. All Market Segments were present for the vote. |  |
| Summary of PRS Discussion |  | On 03/22/07, there was no discussion. On 06/12/07, PRS noted that the B1G1 NPRR requirements are already included in the nodal program and they have no cost or schedule impact. |  |
| TAC Recommendation |  | On 06/28/07, TAC voted to recommend approval of NPRR050 and the B1G1 IA as recommended by PRS. There were five (5) abstentions (4 – Consumer Market Segment; 1 – Independent Power Marketer (IPM) Market Segment). All Market Segments were present for the vote. |  |
| Summary of TAC Discussion |  | On 6/28/07, there was no discussion. |  |
| Board Action |  | On 7/17/07, the Board approved NPRR050 as recommended by TAC. |  |

| ERCOT/Market Segment Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 | None |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | none |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | n/a |  |
| Additional Qualitative Information | 1 | n/a |  |
| Other | 1 | None. |  |
| Comments | 2 |  |  |

| Name | Trip Doggett on behalf of TPTF |
| --- | --- |
| E-mail Address | tdoggett@ercot.com |
| Company | ERCOT |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| None |  |

Proposed Nodal Protocol Language Revision

4.2.2	Wind-Powered Generation Resource Production Potential

(1)	ERCOT shall produce and update hourly a Short-Term Wind Power Forecast (STWPF) that provides a rolling 48-hour hourly forecast of wind production potential for each Wind-Powered Generation Resource (WGR). Each Generation Entity that owns a WGR shall install and telemeter to ERCOT the site-specific meteorological information that ERCOT determines is necessary to produce the STWPF forecasts. ERCOT shall establish procedures specifying the accuracy requirements of WGR meteorological information telemetry.

(2)	The WGR Production Potential (WGRPP) is an hourly 80% confidence level forecast of energy production for each WGR. From the STWPF, ERCOT shall produce and update WGRPP forecasts each hour for each WGR to be used as input into each RUC process as per Section 5, Transmission Security Analysis and Reliability Unit Commitment.

(3)	ERCOT shall produce the WGRPP forecasts using the STWPF information provided by WGR owners including WGR availability, meteorological information, and SCADA.

(4)	Each hour, ERCOT shall provide, through the Messaging System, the WGRPP forecasts for each WGR to the QSE that represents that WGR and shall post each WGRPP forecast on the MIS Certified Area.

(5)	Each hour, ERCOT shall post the aggregated WGRPP forecast of all WGRs on the MIS Secure Area.

5.7.4.1.1	Capacity Shortfall Ratio Share

(1)	In calculating the amount short for each QSE, the QSE must be given a capacity credit for its WGRs based on the HSL values entered into the COP by the QSE just prior to the RUC execution.  For WGRs, ERCOT shall use for settlement purposes the COP and Trades Snapshot prior to the RUC regardless of Real-Time capacity or actual generation.  Therefore, the HASLSNAP and HASLADJ variables used below shall be equal to the HSL values entered into the QSE’s COP submitted prior to the RUC for WGRs.

(2)	In calculating the amount short for each QSE, the QSE must be given a capacity credit for non-wind Resources that were given notice of decommitment within the two hours before the Operating Hour as a result of the RUC process by setting the HASLSNAP and HASLADJ variables used below equal to the HASLSNAP value for the Resource immediately before the decommitment instruction was given.

(3)	The capacity shortfall ratio share of a specific QSE for a particular RUC process is calculated, for a 15-minute Settlement Interval, as follows:

RUCSFRSruc,i,q	=	RUCSFruc,i,q / RUCSFTOTruc,i

Where:

RUCSFTOTruc,i	=	ΣqRUCSFruc,i,q

(4)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval is:

RUCSFruc,i,q	=	Max (0, Max (RUCSFSNAPq,i, RUCSFADJq,i) – Σz is prior to rucRUCCAPCREDITq,i,z)

(5)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at the snapshot, is:

RUCSFSNAPq,i	=	Max (0, ((ΣpRTAMLq,p,i * 4) –  RUCCAPSNAPq,i ))

(6)	The amount of capacity that a QSE had according to the RUC snapshot for a 15-minute Settlement Interval.

RUCCAPSNAPq,i =	ΣrHASLSNAPq,r,h + (RUCCPSNAPq,h –RUCCSSNAPq,h) + (ΣpDAEPq,p,h –ΣpDAESq,p,h) + (ΣpRTQQEPSNAPq,p,i – ΣpRTQQESSNAPq,p,i)

(7)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at Real Time, is:

RUCSFADJq,i	=	Max (0, ((ΣpRTAMLq,p,i) *4)  – RUCCAPADJq,i))

(8)	The amount of capacity that a QSE had in Real Time for a 15-minute Settlement Interval.

RUCCAPADJq,i =	ΣrHASLADJq,r,h + (RUCCPADJq,h – RUCCSADJq,h) + (ΣpDAEPq,p,h – ΣpDAESq,p,h) + (ΣpRTQQEPADJq,p,i– ΣpRTQQESADJq,p,i)

(9)	The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCSFRSruc,i,q | none | RUC Shortfall Ratio Share—The ratio of the QSE’s capacity shortfall to the sum of all QSEs’ capacity shortfalls, for the RUC process, for the 15-minute Settlement Interval. |
| RUCSFruc,i,q | MW | RUC Shortfall—The QSE q’s capacity shortfall for the RUC process for the 15-minute Settlement Interval. |
| RUCSFTOTruc,i | MW | RUC Shortfall Total-The sum of all QSEs’ capacity shortfalls, for a RUC process, for a 15-minute Settlement Interval. |
| RUCSFSNAPq,i | MW | RUC Shortfall at Snapshot—The QSE q’s capacity shortfall according to the snapshot for the RUC process for the 15-minute Settlement Interval. |
| RUCSFSADJq,i | MW | RUC Shortfall at Adjustment Period—The QSE q’s adjustment period capacity shortfall for the 15-minute Settlement Interval. |
| RUCCAPCREDITq,i,z | MW | RUC Capacity Credit by QSE—The capacity credit resulting from capacity paid through the RUC Capacity-Short Charge for the 15-minute Settlement Interval. |
| RTAMLq,p,i | MWh | Real-Time Adjusted Metered Load—The QSE q’s Adjusted Metered Load at the Settlement Point p for the 15-minute Settlement Interval. |
| RUCCAPSNAPq,i | MW | RUC Capacity Snapshot at time of RUC -The amount of the QSE’s calculated capacity in the COP and Trades Snapshot for a 15-minute Settlement Interval. |
| HASLSNAPq,r,h | MW | High Ancillary Services Limit at Snapshot—The High Ancillary Services Limit of the Resource r represented by the QSE q, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RUCCPSNAPq,h | MW | RUC Capacity Purchase at Snapshot—The QSE q’s Capacity purchase, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RUCCSSNAPq,h | MW | RUC Capacity Sale at Snapshot—The QSE q’s capacity sale, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RUCCAPADJq,i | MW | RUC Capacity Snapshot during Adjustment Period-The amount of the QSE’s calculated capacity in the RUC according to the COP and Trades Snapshot at the end of the Adjustment Period for a 15-minute Settlement Interval |
| HASLADJq,r,h | MW | High Ancillary Services Limit at Adjustment Period - The High Ancillary Services Limit of the Resource r represented by the QSE q, according to the adjustment period snapshot, for the hour that includes the 15-minute Settlement Interval. If the HASL for a Resource was credited to the QSE during the RUC Snapshot but the Resource experiences a Forced Outage within two hours before the start of the Settlement Interval, then the HASL for that Resource is also credited to the QSE in the HASLADJ. |
| RUCCPADJq,h | MW | RUC Capacity Purchase at Adjustment Period—The QSE q’s capacity purchase, according to the Adjustment Period COP and Trades Snapshot for the hour that includes the 15-minute Settlement Interval. |
| RUCCSADJq,h | MW | RUC Capacity Sale at Adjustment Period—The QSE q’s capacity sale, according to the Adjustment Period COP and Trades Snapshot for the hour that includes the 15-minute Settlement Interval. |
| DAEPq,p,h | MW | Day-Ahead Energy Purchase—The QSE q’s energy purchased in the DAM at the Settlement Point p for the hour that includes the 15-minute Settlement Interval. |
| DAESq,p,h | MW | Day-Ahead Energy Sale—The QSE q’s energy sold in the DAM at the Settlement Point p for the hour that includes the 15-minute Settlement Interval. |
| RTQQEPSNAPq,p,i | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval, in the COP and Trades Snapshot. |
| RTQQESSNAPq,p,i | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval, in the COP and Trades Snapshot. |
| RTQQEPADJq,p,i | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. |
| RTQQESADJq,p,i | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. |
| Q | none | A QSE. |
| P | none | A Settlement Point. |
| R | none | A Generation Resource that is QSE-committed or RUC-decommitted (subject to paragraphs 1 and 2 above) for the Settlement Interval. |
| Z | none | A previous RUC process for the Operating Day. |
| I | none | A 15-minute Settlement Interval. |
| h | none | The hour that includes the Settlement Interval i. |
| ruc | none | The RUC process for which this Capacity Shortfall Ratio Share is calculated. |