Texas Nodal:

EDS 3

SCED Phase 1 Test Scripts

Version 0.03

Tuesday, July 10, 2007

Revision History

| Document Revisions |  |  |  |
| --- | --- | --- | --- |
| Date | Version | Description | Author(s) |
| 7/10 | 0.01 | Initial Draft document created for EDS 3 SCED Planning Workshop w/ Market Participants. This version is a working draft for planning discussion with Participants. | IRT |
| 7/16 | 0.02 | Web service specific test cases; Get System status, Get mRID, Cancel Offer XMl, Query Offer XML | IRT |
| 7/30 | 0.03 | Update Variable List to reflect XML Update XML to sandbox compliant format | IRT |

## Phase 1: Test Scripts

## Test Script overview

The test scripts define step by step procedures and data requirements for preparing and submitting the submission items during EDS 3 SCED phase 1. This document has been created as a tool to help Market Participants prepare for the phase 1 activities.  These test scripts support both the User Interface and External Web Services testing. The User Interface steps provided in this document are further defined with screen shots in the Market Management System User Guide available on the ERCOT Nodal Readiness Center - EDS 3 Documents. This document supports the following scenarios defined in the EDS 3 SCED Market Participant Handbook.

MMS User Interface / External Web Services

Current Operating Plan - COP Display / COP

- Verify each QSE can Submit, Update and Query a COP using COP UI and External Web Services

- Receive valid Transaction Identifier / mRID for both Valid and Invalid transactions

- Reject COP with corresponding error message(s) if at least one of the following conditions is not met:

  - Resource name must be a valid registered resource name

  - Resource Status must be a valid status code (Currently there are no validations for specific load or generation statuses. The system only validates for an acceptable Status)

  - HSL must be less than or equal to High Reasonability Limit

  - LSL must be greater than or equal to Low Reasonability Limit

  - HEL must be greater than or equal to HSL

  - LEL must be less than or equal to LSL

  - HSL must be greater than or equal to the sum of (Reg Up Responsibility + Responsive Reserve Responsibility + Non-Spin Responsibility +LSL)  * Note: Need to validate calculation and if Regulation Down Responsibility is required for this validation.

  - COP is submitted for an invalid time period (e.g. in the past)

- Issue corresponding warning(s) if at least one of the following conditions is not met:

  - A Resource with a Resource Status of ONTEST must have a corresponding valid Output Schedule

  - For combined-cycle Resources, the COP data items must be submitted for each registered configuration

Three-Part Energy Offer Display / Three Part Supply Offer

- Submit, Update, Query, Cancel

- Cancel w/ valid Output Schedule

- Submit w/ specific Expiration Date/Time

- Validate overwriting behavior of Energy Offer Curve

- Reject if FIP% and FOP% incorrect (resource type dependent)

- Receive valid Transaction Identifier / mRID

- Rejection due to invalid Energy Offer Curve (must be monotonically non-decreasing and all prices within applicable caps/floors, last MW point must be >= 1 MW, no more than 10 price/quantity pairs, )

- Review all results

Output Schedule Display / Output Schedule

- Submit, Update, Query (DSR/Non-DSR)

- Rejection due to valid Energy Offer Curve

- Receive valid Transaction Identifier / mRID

- Rejection due to no COP (This feature may change as indicated earlier in the document)

- Rejection when MW level is not between LSL & HSL of COP (This feature may change as indicated earlier in the document)

- Rejection due to violation of SCED Up/Down ramp rates calculated based on COP data.

- Market window

- Review results

Incremental, Decremental Offer Curve Display  /  Inc/Dec Energy Offer

- Submit, Update, Query, Cancel (DSR)

- Receive Transaction identifier / mRID

- Rejection if first pq quantity is not 0 MW

- Must be monotonically increasing for Inc and monotonically decreasing for DEC

- Market window

- Review results

- Test Data / Variable List

Throughout this document you will find tables similar to the one listed below. These tables identify the specific testing data that should be prepared by QSEs prior to engaging in EDS activities. Prior preparation of these data elements will help ensure that the ERCOT team can provide the most effective support during scheduled testing support calls.

| [ResourceName] | The name of the resource used for the test execution. |
| --- | --- |
| [ResourceStatus] | Resource planned operating status |
| [HSL] | High sustainable limit |
| [LSL] | Low sustainable limit |
| [HEL] | High emergency limit |
| [LEL] | Low emergency limit |
| [Mw1…Mwn ] | Mw break points of a curve |
| … | … |
| … | … |
| … | … |

## Current Operating Plan (COP)

## COP Data Elements

In order for the Market Participants to submit a COP the following data elements must be defined. These data elements must be in alignment with the registration data within the MMS system for validation.

| [QSE_ID] | QSE name | QSE id (String) |
| --- | --- | --- |
| [ResourceName] | Resource Name | Valid resource Name for QSE |
| [OperatingMode] | Res planned operation status | Status of resource |
| [CCConfiguration] | Combined cycle mode | The mode that the Combined Cycle will submit offer against |
| [TradingDate] | Trading day for operation | date of the offer schedule (format: YYYY-MM-DD) |
| startTime & endTime are used in 4 parts of the offer (which can be different for each part) for: the total COP, the resourceStatus, the Limits, the AS Capacity |  |  |
| [startTime] | Combing of date & hour (hour has to be put into GMT format) | [TradingDay]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [endTime] | Combing of date & hour (hour has to be put into GMT format) | [TradingDay+1]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [HSL] | High sustainable limit | HSL must be less than or equal to High Reasonability Limit HSL must be greater than or equal to the sum of (Reg Up Responsibility + Responsive Reserve Responsibility + Non-Spin Responsibility +LSL) |
| [LSL] | Low sustainable limit | LSL must be greater than or equal to Low Reasonability Limit |
| [HEL] | High emergency limit | HEL must be greater than or equal to HSL |
| [LEL] | Low emergency limit | LEL must be less than or equal to LSL |
| [RegUp] | Regulation up value | Sum of all AS + LSL should be less than or equal to HSL |
| [RegDown] | Regulation down value | Sum of all AS + LSL should be less than or equal to HSL |
| [RRS] | Responsive reserve value | Sum of all AS + LSL should be less than or equal to HSL |
| [NSpin] | Non Spin value | Sum of all AS + LSL should be less than or equal to HSL |
| [COPmRID] | COP mRID | [QSE_ID].[TradingDate].COP refer to section 7.1.2.1 |

## COP Submit - User Interface

Submitting a valid COP: if the variables are a valid representation of the physical resource the COP will be successfully accepted, refer to Market Management System User Guide on details for submitting a COP

|  | # | Test Step | Data |
| --- | --- | --- | --- |
|  | 1 | Log into the Market Management System |  |
|  | 2 | Use the Main Menu drop down | Trading |
|  | 3 | Use the Sub Menu drop down | “COP” |
|  | 4 | Use the Trade Date drop down | [TradingDate] |
|  | 5 | Select Resource Name from drop down | [ResourceName] |
|  | 6 | Expand the Resource Status tab |  |
|  | 7 | Select Start HR from drop down | Hour from [startTime] |
|  | 8 | Select End Hr from drop down | Hour from [endTime] |
|  | 9 | Select Resource Status from drop down | [OperatingMode] |
|  | 10 | Select Repeated HR from drop down | “Exclude” |
|  | 11 | Expand the Limits tab |  |
|  | 12 | Select Start HR from drop down and the end hour to and | Hour from [startTime] |
|  | 13 | Select End HR from drop down | Hour from [endTime] |
|  | 14 | Enter HSL, LSL, HEL, LEL | [HSL], [LSL], [HEL], [LEL] |
|  | 15 | Expand the AS capacity tab |  |
|  | 15 | Enter Reg-Up, Reg-Down, RRS, NSpin | [RegUp], [RegDown], [RRS], [Nspin] |

## COP Submit - External Webservice XML

Submit XML through the webservices using the same variables used in section 3.1.

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>create</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>0</UserID>

<MessageID>MessageID1</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews">

<tradingDate>[TradingDate]</tradingDate>

<COP>

<startTime>[startTime]</startTime>

<endTime>[endTime]</endTime>

<marketType>DAM</marketType>

<status> </status>

<error>

<text>NO ERRORS </text>

</error>

<resource>[ResourceName]</resource>

<combinedCycle>[CCConfiguration]</combinedCycle>

<ResourceStatus>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<operatingMode>[OperatingMode]</operatingMode>

</ResourceStatus>

<Limits>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<hsl>[HSL]</hsl>

<lsl>[LSL]</lsl>

<hel>[HEL]</hel>

<lel>[LEL]</lel>

</Limits>

<ASCapacity>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<regUp>[regUp]</regUp>

<regDown>[regDown]</regDown>

<rrs>[RRS]</rrs>

<nonSpin>[Nspin]</nonSpin>

</ASCapacity>

</COP>

</BidSet>

</Payload>

</RequestMessage>

## COP Submit – Scenario Variations

    - Submit failure due to HSL limit

Using the data elements from section 3.1

1.  Change the [HSL] with a value greater than high reasonability limit

2.  Submit the COP

3.  Verify that after submitting the COP, the COP will be rejected.

    - Submit failure due to LSL limit

Using the data elements from section 3.1

- Change the [LSL] with a value less than low reasonability limit

- Submit the COP

- Verify that after submitting the COP, the COP will be rejected.

    - Submit failure due to HEL limit

Using the data elements from section 3.1

- Change the [HEL] with a value less than [HSL]

- Submit the COP

- Verify that after submitting the COP, the COP will be rejected.

    - Submit failure due to LEL limit

Using the data elements from section 3.1

- Change the [LEL] with a value greater than [LSL]

- Submit the COP

- Verify that after submitting the COP, the COP will be rejected.

    - Submit failure due to AS limits

Using the data elements from section 3.1

- Update the [RegUp], [RegDown], [RRS], and [NSpin], such that the sum of all is greater than [HSL]

- Submit the COP

- Verify that after submitting the COP, the COP will be rejected.

    - Submit failure due to invalid time interval

Using the data elements from section 3.1

- Update [TradingDate] such that it’s for a closed trading date;

- Submit the COP

- Verify that after submitting the COP, the COP will be rejected.

    - Submit failure due to missing output schedule

Using the data elements from section 3.1

- Update the [OperatingMode] such that it’s “ONTEST”

- Submit the COP

- Verify that after submitting the COP, the COP will be rejected.

    - Submit failure due to invalid CC configuration

Using the data elements from section 3.1

- Select a Combined Cycle resource

- Submit resource limits not associated with the combined cycle resource ([ResourceName]-[CCConfiguration]),

- Submit the COP

- Verify that after submitting the COP, the COP will be rejected.

## COP Query & Update – User Interface

Using the data elements from section 3.1, query for the successfully submitted COP, and update the COP AS values to a valid variable and resubmit. Verify the COP will be successfully accepted.

|  | # | Test Step | Data |
| --- | --- | --- | --- |
|  | 1 | Use the Main Menu drop down | “Trading” |
|  | 2 | Use the Sub Menu drop down | “COP” |
|  | 3 | Use the Trade Date drop down | [TradingDate] |
|  | 4 | Select Resource Name from drop down | [ResourceName] |
|  | 5 | Select Action | “Query” |
|  | 6 | Click the query button |  |
|  |  | Update Steps |  |
|  | 1 | Expand the AS Capacity tab |  |
|  | 2 | Update Reg-Up, Reg-Down value with & the Reg Down value with | [RegUp] +1, [RegDown] - 1 |
|  | 3 | Click Submit button (make sure the action is submit) |  |

  - COP Query - External Webservice XML

Query XML through the webservices using the same variables used in section 3.1. (please refer to section 7.1.2.1 for retrieving mRID)

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>get</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-7-25T00:00:00.000-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>0</UserID>

<MessageID>getMessage</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Request>

<MarketType>DAM</MarketType>

<OperatingDate>[StartTime]</OperatingDate>

<TradingDate>[TradingDate]</TradingDate>

<StartTime>[startTime]</StartTime>

<EndTime>[endTime]</EndTime>

<Zone>anyType</Zone>

<ASType>anyType</ASType>

<Option>anyType</Option>

<ID>[COPmRID]</ID>

</Request>

</RequestMessage>

  - COP Update – Scenario Variations

    - Update Rejection – User Interface

Using the data elements from section 3.1

- Query for the successfully submitted COP

- Update the COP AS values, such that the sum of the variables is greater than [HSL]

- Submit the COP

- Verify the COP will be rejected

    - Update Rejection Variation – External Webservice XML

- Submit a COP using the data elements from section 3.1

- Verify the COP is accepted successfully

- Update the submit XML for a COP with “change” as the verb

- Update the AS values such that the sum of the variables is greater than the [HSL]

- Submit the COP

- Verify the COP will be rejected.

## Three part energy offer

Prior to submitting a Three Part Energy Offer, verify that there is a valid COP for the trading date (please refer section 3 for submitting a valid COP)

  - TPO Data Elements

In order for the Market Participants to submit a three part supply offer the following data elements must be defined. These data elements must be in alignment with the registration data within the MMS system for validation.  As well as the COP submitted as part of section 3.1

| [QSE_ID] | QSE name | QSE id (String) |
| --- | --- | --- |
| [ResourceName] | Resource Name | Valid resource Name for QSE |
| [CCConfiguration] | Combined cycle mode | The mode that the Combined Cycle will submit offer against |
| [TradingDate] | Trading day for operation | date of the offer schedule (format: YYYY-MM-DD |
| startTime & endTime are used in 5 parts of the offer (which can be different for each part) for: the total 3PO, the FIP/FOP, the startup Cost, the minimum load cost, the energy offer curve |  |  |
| [startTime] | Combing of date & hour (hour has to be put into GMT format) | [TradingDate]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [endTime] | Combing of date & hour (hour has to be put into GMT format) | [TradingDate+1]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [expirationTime] | Time of offer expiration | format: MM/DD/YYYY HH24:MI:SS -06:00 |
| [FIP] | FIP percent amount | FIP+FOP = 100, Percentage (integer) |
| [FOP] | FOP percent amount | FIP+FOP = 100, Percentage (integer) |
| [Mw1…Mwn ] | Mw break points of a curve | (MW1 has to equal LSL of COP) (MWN has to equal HSL of COP) (MW1 > MW2 > MW3 > ... > MWN) |
| [PR1…PRn ] | Price break points | (PR1 > PR2 > PR3 > ... > PRN) |
| [N] | Amount of energy break points | Integer from 1-10 |
| [MinLoadCost] | miniumum load cost | Integer |
| [hot] | cost of starting a hot resource | Integer |
| [intermediate] | cost of starting an intemediate resource | Integer |
| [cold] | cost of starting a cold resource | Integer |
| [TPOmRID] | Three Part Offer mRID | [QSE_ID].[TradingDate].TPO refer to section 7.1.2.2 |

  - TPO Submit - User Interface

|  | # | Test Step | Data |
| --- | --- | --- | --- |
|  | 1 | Log into the Market Management System |  |
|  | 2 | Use the Main Menu drop down | “Trading” |
|  | 3 | Use the Sub Menu drop down | “Three part energy supply offers” |
|  | 4 | Use the Trade Date drop down | [TradingDate] |
|  | 5 | Select Resource Name from drop down | [ResourceName] |
|  | 6 | Expand the energy offer curves FIP/FOP tab |  |
|  | 7 | Select Start HR from drop down | Hour from [startTime] |
|  | 8 | Select End Hr from drop down | Hour from [endTime] |
|  | 9 | Select Repeated HR from drop down | “Exclude” |
|  | 10 | Enter FIP % | [FIP] |
|  | 11 | Enter FOP % | [FOP] |
|  | 12 | Expand the energy Offer curves tab |  |
|  | 13 | Select Start HR from drop down | Hour from [startTime] |
|  | 14 | Select End HR from drop down | Hour from [endTime] |
|  | 15 | Select Repeated HR from drop down | “Exclude” |
|  | 16 | Select Inc Excl from drop down | “Inclusive” |
|  | 17 | Click the Add Step button for number of hours | [N] -1 |
|  | 18 | Enter each price segment and MW segment | [PR1,2,…, N] [MW1,2,… N]] |
|  | 19 | Select the submit option, then click the submit button |  |

  - TPO Submit - External Webservice XML

Submit XML through the webservices using the same variables used in section 4.1.

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<!--?xml version="1.0" encoding="UTF-8"?-->

<RequestMessage xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>create</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>0</UserID>

<MessageID>MessageID1</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>[TradingDate]</tradingDate>

<ThreePartOffer>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<marketType>DAM</marketType>

<status> </status>

<error>

<text> </text>

</error>

<expirationTime>[expirationTime]</expirationTime>

<resource>[ResourceName]</resource>

<combinedCycle>[CCConfiguration]</combinedCycle>

<FipFop>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<fipPercent>[FIP]</fipPercent>

<fopPercent>[FOP]</fopPercent>

</FipFop>

<StartupCost>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<hot>[hot]</hot>

<intermediate>[intermediate]</intermediate>

<cold>[cold]</cold>

</StartupCost>

<MinimumEnergy>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<cost>[MinLoadCost]</cost>

</MinimumEnergy>

<EnergyOfferCurve>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<CurveData>

<xvalue>[MW1]</xvalue>

<y1value>[PR1]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW2]</xvalue>

<y1value>[PR2]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW3]</xvalue>

<y1value>[PR3]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW4]</xvalue>

<y1value>[PR4]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW5]</xvalue>

<y1value>[PR5]</y1value>

</CurveData>

</EnergyOfferCurve>

</ThreePartOffer>

</BidSet>

</Payload>

</RequestMessage>

  - TPO Submit – Scenario Variation

## Submit failure due to FIP/FOP

Using the data elements from section 4.1

- Submit a valid COP (refer to section 3.1 on submitting a valid COP)

- Set the [FIP]/[FOP] such that they are not valid

- Submit the TPO

- Verify that after submitting the Three Part Offer, it will be rejected.

## Submit failure due to Non-Monotonically increasing

Using the data elements from section 4.1

- Submit a valid COP (refer to section 3.1 on submitting a valid COP)

- Set the [PR1] such that [PR1] is greater than [PR2];

- Submit the TPO

- Verify that after submitting the Three Part Offer, it will be rejected.

  - TPO Query & Update – User Interface

Using the data elements from section 4.1, query for the successfully submitted Three Part Offer, and update the Three Part Offer MW price of the first segment, [PR1], to a different but still valid price and resubmit. Verify the Three part Offer will be successfully accepted.

|  | # | Test Step | Data |
| --- | --- | --- | --- |
|  | 1 | Use the Main Menu drop down | “Trading” |
|  | 2 | Use the Sub Menu drop down | “Three part energy supply offers” |
|  | 3 | Use the Trade Date drop down | [TradingDate] |
|  | 4 | Select Resource Name from drop down | [ResourceName] |
|  | 5 | Select the query option, the click query |  |
|  |  | Update Steps |  |
|  | 1 | Expand the energy Offer curve tab |  |
|  | 2 | Update a mid price segment and MW segment | [PR X] [MW X] |
|  | 3 | Click Submit button (make sure the action is submit) |  |

## TPO  Query - External Webservice XML

Query XML through the webservices using the same variables used in section 4.1. (please refer to section 7.1.2.2 for retrieving mRID)

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>get</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-07-25T00:00:00.000-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>Jamil  Daouk</UserID>

<MessageID>getMessage</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Request>

<MarketType>DAM</MarketType>

<TradingDate>[TradingDate]</TradingDate>

<StartTime>[StartTime]</StartTime>

<EndTime>[endTime]</EndTime>

<Zone>anyType</Zone>

<ASType>anyType</ASType>

<Option>anyType</Option>

<ID>[TPOmRID]</ID>

</Request>

</RequestMessage>

## TPO Update – Scenario Variations

## Update Rejection Variation – User Interface

Using the data elements from section 4.1

- Query for the successfully submitted Three Part Offer,

- Update the Three Part Offer MW price of the first segment, [PR1], such that it’s greater than [PR2],

- Submit the TPO.

- Verify that the Three Part Offer will be rejected.

## Update Rejection Variation – External Webservice XML

- Update the submit XML for a TPO with “change” as the verb

- Update the Three Part Offer Mw price of the first segment, [PR1], such that it’s greater than [PR2]

- Submit the TPO

- Verify that the Three Part Offer will be rejected

  - TPO Cancel – User Interface

Using the data elements from section 4.1, query for the successfully submitted Three Part Offer, and cancel the Three Part Offer.  Verify the Three part Offer will be successfully cancelled

|  | # | Test Step | Data |
| --- | --- | --- | --- |
|  | 1 | Use the Main Menu drop down | “Trading” |
|  | 2 | Use the Sub Menu drop down | “Three part energy supply offers” |
|  | 3 | Use the Trade Date drop down | [TradingDate] |
|  | 4 | Select Resource Name from drop down | [ResourceName] |
|  | 5 | Select the query option, the click query |  |
|  | 6 | Select the cancel option, then click the cancel button |  |

  - TPO Cancel – External Web Service XML

Cancel XML through the webservices using the same variables used in section 4.1. (please refer to section 7.1.2.2 for retrieving mRID)

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>cancel</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-07-25T00:00:00.000-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>Jamil  Daouk</UserID>

<MessageID>getMessage</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Request>

<MarketType>DAM</MarketType>

<TradingDate>[TradingDate]</TradingDate>

<StartTime>[StartTime]</StartTime>

<EndTime>[endTime]</EndTime>

<Zone>anyType</Zone>

<ASType>anyType</ASType>

<Option>anyType</Option>

<ID>[TPOmRID]</ID>

</Request>

</RequestMessage>

- Output Schedule

Prior to submitting an Output Schedule, verify that there is a valid COP for the trading date (please refer section 3 for submitting a valid COP)

  - Output Schedule Data Elements

In order for the Market Participants to submit an output schedule the following data elements must be defined. These data elements must be in alignment with the registration data within the MMS system for validation.  As well as the COP submitted as part of section 3.1

| [QSE_ID] | QSE ID | String |
| --- | --- | --- |
| [ResourceName] | Resource Name | Valid resource Name for QSE |
| [CCConfiguration] | Combined cycle mode | The mode that the Combined Cycle will submit offer against |
| [TradingDate] | Trading day for operation | Date (MM/DD/YYYY) |
| [expirationTime] | Combing of date & hour (hour has to be put into GMT format) | Date when Offer expires |
| startTime & endTime are used in 4 parts of the offer (which can be different for each part) for: output schedule energy schedule, the minimum load cost, the energy offer curve |  |  |
| [startTime] | Combing of date & hour (hour has to be put into GMT format) | [TradingDate]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [endTime] | Combing of date & hour (hour has to be put into GMT format) | [TradingDate+1]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [time] | Combing of date & hour (hour has to be put into GMT format) in 5 minute intervals | [TradingDate]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [ending] | Combing of date & hour (hour has to be put into GMT format) in 5 minute intervals | [TradingDate+1]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [MwLevel] | Output schedule Mw level | Integer |
| [OSmRID] | Output Schedule mRID | [QSE_ID].[TradingDate].OS refer to section 7.1.2.3 |

  - OS Submit - User Interface

|  | # | Test Steps | Data |
| --- | --- | --- | --- |
|  | 1 | Use the Main Menu drop down | “Trading” |
|  | 2 | Use the Sub Menu drop down | “Output Schedules” |
|  | 3 | Use the Trade Date drop down | [TradingDate] |
|  | 4 | Select Resource Name from drop down | [ResourceName] |
|  | 5 | Cancel EOC | Unchecked |
|  | 6 | Expand the Mw Levels tab |  |
|  | 7 | Select Start HR from drop down | Hour from [startTime] |
|  |  | Select Start Int from drop down | Minute from [time] |
|  | 8 | Select End Hr from drop down | Hour from [endTime] |
|  |  | Select End Int from drop down | Minute from [ending] |
|  | 9 | Select Repeated HR from drop down | “Exclude” |
|  | 11 | Enter Mw Level | [MwLevel] |
|  | 12 | Select the submit option, then click the submit button |  |

  - OS Submit - External Webservice XML

Submit XML through the webservices using the same variables used in section 5.1

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<!--?xml version="1.0" encoding="UTF-8"?-->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>create</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>0</UserID>

<MessageID>MessageID1</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>[TradingDate]</tradingDate>

<status>string</status>

<mode>string</mode>

<OutputSchedule>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<mRID>String</mRID>

<externalId>String</externalId>

<marketType>String</marketType>

<status> </status>

<error>

<text> </text>

</error>

<resource>[ResourceName]</resource>

<combinedCycle>[CCConfiguration]</combinedCycle>

<EnergySchedule>

<startTime>[endTime]</startTime>

<endTime>[endTime]</endTime>

<TmPoint>

<time>[time]</time>

<ending>[ending]</ending>

<value1>[MWLevel]</value1>

</TmPoint>

</EnergySchedule>

</OutputSchedule>

</BidSet>

</Payload>

</RequestMessage>

  - OS Submit – Scenario Variation

    - Submit failure due valid Cost Curve

Using the data elements from section 5.1

- Submit a valid COP (refer to section 3.1 on submitting a valid COP)

- Submit a valid TPO with an energy offer curve (refer to section 4.1 on submitting a valid three part offer)

- Submit a valid output schedule for the resource;

- Submit the Output Schedule

- Verify that when the Output Schedule gets submitted it’s rejected.

    - Submit failure due to MW level outside limits

Using the data elements from section 5.1,

- submit a valid COP (refer to section 3.1 on submitting a valid COP)

- change the [MWLevel] such that it’s greater than [HSL] or lower than [LSL].

- submit the OS

- Verify that when the Output Schedule gets submitted it’s rejected.

    - Submit failure due to Ramp Rate

Using the data elements from section 5.1,

- submit a valid COP (refer to section 3.1 on submitting a valid COP)

- Change the [MWLevel], such that the difference between two interval [MWLevel] is greater than the resource’s ramping capabilities.

- submit the OS

- Verify that when the Output Schedule gets submitted it’s rejected.

  - OS Query & Update – User Interface

Using the data elements from section 5.1, query for the successfully submitted Output Schedule and update the [MWLevel]. Verify the Output Schedule will be successfully accepted.

|  | # | Test Step | Data |
| --- | --- | --- | --- |
|  | 1 | Use the Main Menu drop down | “Trading” |
|  | 2 | Use the Sub Menu drop down | “Output Schedule” |
|  | 3 | Use the Trade Date drop down | [TradingDate] |
|  | 4 | Select Resource Name from drop down | [ResourceName] |
|  | 5 | Select the query option, the click query |  |
|  |  | Update Steps |  |
|  | 1 | Expand the MW Level tab |  |

  - OS Query – External Webservice XML

Query XML through the webservices using the same variables used in section 5.1. (please refer to section 7.1.2.3 for retrieving mRID)

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>get</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-7-25T00:00:00.000-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>Jamil Daouk</UserID>

<MessageID>getMessage</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Request>

<MarketType>DAM</MarketType>

<TradingDate>[TradingDate]</TradingDate>

<StartTime>[StartTime]</StartTime>

<EndTime>[endTime]</EndTime>

<Zone>anyType</Zone>

<ASType>anyType</ASType>

<Option>anyType</Option>

<ID>[OSmRID]</ID>

</Request>

</RequestMessage>

## OS Update – Scenario Variations

    - Update Rejection Variation – User Interface

Using the data elements from section 5.1,

- query for a successfully submitted Output Schedule,

- update the [MWLevel] such that it’s greater than [HSL].

- submit the OS

- Verify that the Output Schedule will be rejected.

    - Update Rejection Variation – External Webservice XML

- Update the submit XML for an Output Schedule with “change” as the verb

- Update the [MWLevel] such that it’s greater than [HSL].

- submit the OS

- Verify that the Output schedule will be rejected

  - OS Cancel – User Interface

Using the date elements from section 5.1, query for the successfully submitted Output Schedule, and cancel the Output Schedule.  Verify the Output Schedule will be successfully cancelled.

|  | # | Test Steps | Data |
| --- | --- | --- | --- |
|  | 1 | Use the Main Menu drop down | “Trading” |
|  | 2 | Use the Sub Menu drop down | “Output Schedules” |
|  | 3 | Use the Trade Date drop down | [TradingDate] |
|  | 4 | Select Resource Name from drop down | [ResourceName] |
|  | 5 | Select the query option, the click query |  |
|  | 6 | Select the cancel option, then click the cancel button |  |

  - OS cancel – External Webservice XML

Cancel XML through the webservices using the same variables used in section 5.1. (please refer to section 7.1.2.3 for retrieving mRID)

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>cancel</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-7-25T00:00:00.000-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>Jamil Daouk</UserID>

<MessageID>getMessage</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Request>

<MarketType>DAM</MarketType>

<TradingDate>[TradingDate]</TradingDate>

<StartTime>[StartTime]</StartTime>

<EndTime>[endTime]</EndTime>

<Zone>anyType</Zone>

<ASType>anyType</ASType>

<Option>anyType</Option>

<ID>[OSmRID]</ID>

</Request>

</RequestMessage>

- Incremental/Decremental

Prior to submitting an Output Schedule, verify that there is a valid COP for the trading date (please refer section 3 for submitting a valid COP), also verify that there is a valid output schedule for the trading date (please refer to section 5 for submitting a valid OS).

  - Inc/Dec Data Elements

In order for the Market Participants to submit an Inc/Dec offer curve, the following data elements must be defined. These data elements must be in alignment with the registration data within the MMS system for validation.  As well as the COP submitted as part of section 3.1 & the output schedule submitted in section 5.1

| [ResourceName] | Resource Name | Valid resource Name for QSE |
| --- | --- | --- |
| [CCConfiguration] | Combined cycle mode | The mode that the Combined Cycle will submit offer against |
| [TradingDate] | Trading day for operation | Date (MM/DD/YYYY) |
| startTime & endTime are used in 2 parts of the offer (which can be different for each part) for: incremental curve, decremental curve |  |  |
| [startTime] | Combing of date & hour (hour has to be put into GMT format) | [TradingDate]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [endTime] | Combing of date & hour (hour has to be put into GMT format) | [TradingDate+1]+[startHR](GMT) (format: MM/DD/YYYY HH24:MI:SS -06:00) |
| [FIP] | FIP percent amount | FIP+FOP = 100, Percentage (integer) |
| [FOP] | FOP percent amount | FIP+FOP = 100, Percentage (integer) |
| [MW1…MWn ] | Mw break points of a curve | (MW1 has to equal LSL of COP) (MWN has to equal HSL of COP) (MW1 < MW2 < MW3 < ... < MWN) |
| [PR1…PRn ] | Price break points | (PR1 < PR2 < PR3 < ... < PRN) |
| [dMW1…dMWn ] | Mw break points of a curve | (MW1 has to equal LSL of COP) (MWN has to equal HSL of COP) (MW1 < MW2 < MW3 < ... , MWN) |
| [dPR1…dPRn ] | Price break points | (dPR1 > dPR2 > dPR3 > ... > dPRN) |
| [N] | Amount of energy break points | Integer from 1-10 |
| [IncDecmRID] | Inc/Dec mRID | [QSE_ID].[TradingDate].IDO refer to section 7.1.2.4 |

  - Inc/Dec Submit - User Interface

|  | # | Test Steps | Data |
| --- | --- | --- | --- |
|  | 1 | Use the Main Menu drop down | “Trading” |
|  | 2 | Use the Sub Menu drop down | “Incremental/Decremental” |
|  | 3 | Use the Trade Date drop down | [TradingDate] |
|  | 4 | Select Resource Name from drop down | [ResourceName] |
|  | 5 | Expand the Energy Offer Curve FIP/FOP tab |  |
|  | 6 | Select Start HR from drop down | Hour from [startTime] |
|  | 7 | Select End Hr from drop down | Hour from [endTime] |
|  | 8 | Select Repeated HR from drop down | “Exclude” |
|  | 9 | Write in the FIP % | [FIP] |
|  | 10 | Write in the FOP % | [FOP] |
|  | 11 | Expand the Energy Offer Curve tab |  |
|  | 12 | Select Start HR from drop down | Hour from [startTime] |
|  | 13 | Select End Hr from drop down | Hour from [endTime] |
|  | 14 | Select Repeated HR from drop down | “Exclude” |
|  | 15 | Click the Add Step button for number of hours | [N] -1 |
|  | 16 | Enter each price segment and MW segment | [PR1,2,…, N] [MW1,2,… N]] |
|  | 17 | Select the submit option, then click the submit button |  |

  - Inc/Dec Submit – External Webservice XML

Submit XML through the webservices using the same variables used in section 6.1

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<!--?xml version="1.0" encoding="UTF-8"?-->

<RequestMessage xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>create</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>5e680f6e02a1c89314a99bbc01794bb3</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>0</UserID>

<MessageID>5e680f6e02a1c89314a99bbc01794bb3</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews">

<tradingDate>[TradingDate]</tradingDate>

<status>String</status>

<mode>String</mode>

<IncDecOffer>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<mRID>String</mRID>

<externalId>String</externalId>

<marketType>ERCOT</marketType>

<status> </status>

<error>

<text> </text>

</error>

<resource>[ResourceName]</resource>

<combinedCycle>[CCConfiguration]</combinedCycle>

<FipFop>

<startTime>[startTime]</startTime>

<endTime>[endTime]</endTime>

<fipPercent>[FIP]</fipPercent>

<fopPercent>[FOP]</fopPercent>

</FipFop>

<IncrementalCurve>

<startTime>[startTime]</startTime>

<endTime>[endTime]</endTime>

<curveStyle>FIXED</curveStyle>

<CurveData>

<xvalue>[MW1]</xvalue>

<y1value>[PR1]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW2]</xvalue>

<y1value>[PR2]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW3]</xvalue>

<y1value>[PR3]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW4]</xvalue>

<y1value>[PR4]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW5]</xvalue>

<y1value>[PR5]</y1value>

</CurveData>

<incExcFlag>INC</incExcFlag>

<reason>OUT</reason>

</IncrementalCurve>

</IncDecOffer>

</BidSet>

</Payload>

</RequestMessage>

The following is an example of a decremental offer, using the DecrementalCurve:

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<!--?xml version="1.0" encoding="UTF-8"?-->

<RequestMessage xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>create</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>5e680f6e02a1c89314a99bbc01794bb3</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>0</UserID>

<MessageID>5e680f6e02a1c89314a99bbc01794bb3</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews">

<tradingDate>[TradingDate]</tradingDate>

<status>String</status>

<mode>String</mode>

<IncDecOffer>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<mRID>String</mRID>

<externalId>String</externalId>

<marketType>ERCOT</marketType>

<status> </status>

<error>

<text> </text>

</error>

<resource>[ResourceName]</resource>

<combinedCycle>[CCConfiguration]</combinedCycle>

<FipFop>

<startTime>[startTime]</startTime>

<endTime>[endTime]</endTime>

<fipPercent>[FIP]</fipPercent>

<fopPercent>[FOP]</fopPercent>

</FipFop>

<DecrementalCurve>

<startTime>[startTime]</startTime>

<endTime>[endTime]</endTime>

<curveStyle>FIXED</curveStyle>

<CurveData>

<xvalue>[dMW1]</xvalue>

<y1value>[dPR1]</y1value>

</CurveData>

<CurveData>

<xvalue>[dMW2]</xvalue>

<y1value>[dPR2]</y1value>

</CurveData>

<CurveData>

<xvalue>[dMW3]</xvalue>

<y1value>[dPR3]</y1value>

</CurveData>

<CurveData>

<xvalue>[dMW4]</xvalue>

<y1value>[dPR4]</y1value>

</CurveData>

<CurveData>

<xvalue>[dMW5]</xvalue>

<y1value>[dPR5]</y1value>

</CurveData>

<incExcFlag>INC</incExcFlag>

<reason>OUT</reason>

</DecrementalCurve>

</IncDecOffer>

</BidSet>

</Payload>

</RequestMessage>

  - Inc/Dec Submit – Scenario Variation

    - Submit failure due to first MW breakpoint

Using the same data elements from section 6.1

- Submit a valid COP (refer to section 3.1 on submitting a valid COP)

- Change the [MW1] such that it’s not 0.

- Submit the Inc/Dec offer curve

- Verify that when the Incremental/Decremental curve gets submitted it’s rejected.

    - Submit failure due to non monotonically increasing

Using the same data elements from section 6.1

- Submit a valid COP (refer to section 3.1 on submitting a valid COP)

- Change the [PR1] such that it’s greater than [PR2].

- submit the Inc/Dec offer curve

- Verify that when the Incremental/Decremental curve gets submitted it’s rejected.

  - Inc/Dec Query & Update – User Interface

Using the data elements from section 6.1, query for the successfully submitted Incremental/Decremental curve, and update the [MW3] to a valid Mw amount. Verify the Incremental/Decremental curve will be successfully accepted.

|  | # | Test Step | Data |
| --- | --- | --- | --- |
|  | 1 | Use the Main Menu drop down | “Trading” |
|  | 2 | Use the Sub Menu drop down | “Incremental/Decremental” |
|  | 3 | Use the Trade Date drop down | [TradingDate] |
|  | 4 | Select Resource Name from drop down | [ResourceName] |
|  | 5 | Select the query option, the click query |  |
|  |  | Update Steps |  |
|  | 1 | Expand the Energy Offer Curve tab |  |
|  | 2 | Update the MW3 break point | [MW3] |
|  | 3 | Click Submit button (make sure the action is submit) |  |

  - Inc/Dec Query – External Webservice XML

Query XML through the webservices using the same variables used in section 6.1. (please refer to section 7.1.2.4 for retrieving mRID)

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>get</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-7-25T00:00:00.000-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>Jamil  Daouk</UserID>

<MessageID>getMessage</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Request>

<MarketType>DAM</MarketType>

<TradingDate>[TradingDate]</TradingDate>

<StartTime>[StartTime]</StartTime>

<EndTime>[endTime]</EndTime>

<Zone>anyType</Zone>

<ASType>anyType</ASType>

<Option>anyType</Option>

<ID>[IncDecmRID]</ID>

</Request>

</RequestMessage>

  - Inc/Dec Query – Scenario Variation

    - Update Rejection Variation – User Interface

Using the data elements from section 6.1

- Query for the successfully submitted Incremental/Decremental curve

- Update [MW1] to be a number greater than 0.

- Submit the Inc/Dec offer curve

- Verify that when the Incremental/Decremental curve gets submitted it’s rejected.

    - Update Rejection Variation – External Webservice XML

- Update the submit XML for an Inc/Dec offer with “change” as the verb

- update [MW1] to be a number greater than 0

- submit the Inc/Dec offer curve

- verify that when the Incremental/Decremental curve gets submitted it’s rejected.

  - Inc/Dec Cancel – User Interface

Using the data elements from section 6.1, query for the successfully submitted Incremental/Decremental curve, and cancel the Incremental/Decremental curve. Verify the Incremental/Decremental curve will be cancelled.

|  | # | Test Steps | Data |
| --- | --- | --- | --- |
|  | 1 | Use the Main Menu drop down | “Trading” |
|  | 2 | Use the Sub Menu drop down | “Incremental/Decremental” |
|  | 3 | Use the Trade Date drop down | [TradingDate] |
|  | 4 | Select Resource Name from drop down | [ResourceName] |
|  | 5 | Select the query option, the click query |  |
|  | 6 | Select the cancel option, then click the cancel button |  |

  - Inc/Dec Cancel – External Webservice XML

Cancel XML through the webservices using the same variables used in section 6.1. (please refer to section 7.1.2.4 for retrieving mRID)

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>cancel</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-7-25T00:00:00.000-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>Jamil  Daouk</UserID>

<MessageID>getMessage</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Request>

<MarketType>DAM</MarketType>

<TradingDate>[TradingDate]</TradingDate>

<StartTime>[StartTime]</StartTime>

<EndTime>[endTime]</EndTime>

<Zone>anyType</Zone>

<ASType>anyType</ASType>

<Option>anyType</Option>

<ID>[IncDecmRID]</ID>

</Request>

</RequestMessage>

- Additional Web Services

  - Get – External WebService XML

In order for the Market Participants to retrieve information from the MMS system, the following data elements must be defined. These data elements must be in alignment with the registration data within the MMS system for validation.  As well as the offers submitted as part of section 3.1, 4.1, 5.1, and/or 6.1

    - System Status

Submit XML through the webservices to retrieve system status information

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>get</Verb>

<Noun>SystemStatus</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>EDS</UserID>

</Header>

</RequestMessage>

    - Get MRID

      - Get MRID for COP

Submit XML through the webservices; using the same variables defined in section 3.1

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>get</Verb>

<Noun>mRID</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>EDS</UserID>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews">

<COP>

<resource>[ResourceName]</resource>

</COP>

</BidSet>

</Payload>

</RequestMessage>

      - Get MRID for TPO

Submit XML through the webservices; using the same variables defined in section 4.1

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>get</Verb>

<Noun>mRID</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>EDS</UserID>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews">

<ThreePartOffer>

<resource>[ResourceName]</resource>

</ThreePartOffer>

</BidSet>

</Payload>

</RequestMessage>

      - Get MRID for OS

Submit XML through the webservices; using the same variables defined in section 5.1

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>get</Verb>

<Noun>mRID</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>EDS</UserID>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<!--Sample XML file generated by XMLSpy v2007 sp2 (http://www.altova.com)-->

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews">

<OutputSchedule>

<resource>[ResrouceName]</resource>

</OutputSchedule>

</BidSet>

</Payload>

</RequestMessage>

      - Get MRID for INC/DEC Offer Curve

Submit XML through the webservices; using the same variables defined in section 6.1

<!-- edited with XMLSpy v2007 rel. 3 (http://www.altova.com) by Jamil Daouk (jd1) -->

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>get</Verb>

<Noun>mRID</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>[QSE_ID]</Source>

<UserID>EDS</UserID>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews">

<IncDecOffer>

<resource>[ResourceName]</resource>

<IncrementalCurve>

<startTime>[StartTime]</startTime>

<endTime>[endTime]</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>[MW1]</xvalue>

<y1value>[PR1]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW2]</xvalue>

<y1value>[PR2]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW3]</xvalue>

<y1value>[PR3]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW4]</xvalue>

<y1value>[PR4]</y1value>

</CurveData>

<CurveData>

<xvalue>[MW5]</xvalue>

<y1value>[PR5]</y1value>

</CurveData>

</IncrementalCurve>

</IncDecOffer>

</BidSet>

</Payload>

</RequestMessage>

  - ERCOT Asynchronous Services

    - Bid Set Acceptance

Bid set acceptance is a notification provided by ERCOT to a QSE indicating the acceptance of a BidSet.  The information found in the Bid Set Acceptance is relative to the variables  submitted in section 3.1, 4.1, 5.1, & 6.1.

Example of Bid Set Acceptance

<BidSet xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<tradingDate>2008-01-01</tradingDate>

<COP>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</COP>

<IncDecOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</IncDecOffer>

<OutputSchedule>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</OutputSchedule>

<ThreePartOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</ThreePartOffer>

</BidSet>

    - Bid Set Errors

Bid Set Errors is a notification provided by ERCOT to a QSE indicating that the BidSet has errors.  The information found in Bid Set Error is relative to the variables submitted in section 3.4, 4.4, 5.4, & 6.4.

Example of Bid Set Error

<BidSet xmlns="http://www.ercot.com/wsdl/nodal/2006-12/mms">

<tradingDate>2008-01-01</tradingDate>

<COP>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</COP>

<IncDecOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</IncDecOffer>

<OutputSchedule>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</OutputSchedule>

<ThreePartOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

</ThreePartOffer>

</BidSet>