ERCOT Operations Report

June 2007 ROS & OWG

Frequency Control

| Time | Freq(Hz) | Duration |  |
| --- | --- | --- | --- |
| 6/1 01:07:24 | 60.118 | 3 m 16 s | Out of Down Regulation 01:07 - 01:27. Two QSEs with significant SCE. One QSE unable to deliver DRS service due to Category 2 deployments |
| 6/2 00:55:38 | 59.891 | 6 s | 460 MW resource lost |
| 6/3 19:39:30 | 59.893 | 4 s | 500 MW resource lost |
| 6/8 05:59:14 | 60.109 | 1 m 36 s | 550 MW ERCOT SCE |
| 6/16 00:00:40 | 59.893 | 4 s | -1100 MW ERCOT SCE Multiple QSE negative during 00:00 ramp |
| 6/16 10:01:00 | 59.820 | 1 m 46 s | 780 MW resource lost |
| 06/25 13:57:26 | 59.894 | 4 s | 2 units totaling 530 MW lost |

Deployed Responsive (Reported Above)

| ID | Time of deployed | Stop Time | Duration | Max | Details |
| --- | --- | --- | --- | --- | --- |
| 1 | 6/1 0:01:00 | 6/1 0:07:36 | 0:06:36 | 330.73 | QSE SCE |
| 2 | 6/2 0:55:38 | 6/2 1:01:22 | 0:05:44 | 401.96 | Unit Trip |
| 3 | 6/3 19:39:30 | 6/3 19:55:16 | 0:15:46 | 391.77 | Unit Trip |
| 4 | 6/7 19:48:56 | 6/7 19:54:00 | 0:05:04 | 310.37 | Unit Trip |
| 5 | 6/15 23:59:58 | 6/16 0:10:14 | 0:10:16 | 391.77 | QSE SCE |
| 6 | 6/16 10:01:00 | 6/16 10:17:00 | 0:16:00 | 763.21 | Unit Trip |
| 7 | 6/25 13:57:28 | 6/25 14:08:54 | 0:11:26 | 386.69 | Unit Trip |

Day Ahead OOMC and RMR Capacity Purchases

| Details | RMR | OOMC | RPRS | Total |
| --- | --- | --- | --- | --- |
| Austin | 0 | 20 | 45 | 65 |
| Bryan Area | 0 | 0 | 2 | 2 |
| Corpus | 0 | 2 | 0 | 2 |
| DFW | 0 | 3 | 0 | 3 |
| E RIO | 0 | 0 | 0 | 0 |
| Houston | 0 | 7 | 1 | 8 |
| Laredo | 90 | 0 | 0 | 90 |
| North | 0 | 5 | 7 | 12 |
| Northeast | 0 | 0 | 0 | 0 |
| San Antonio | 0 | 5 | 6 | 11 |
| South | 0 | 0 | 2 | 2 |
| W RIO | 0 | 0 | 0 | 0 |
| Western | 0 | 1 | 1 | 2 |
|  | 90 | 43 | 64 | 197 |

| Historical Totals | Jun-06 | May-07 | Jun-07 |
| --- | --- | --- | --- |
| Austin | 71 | 48 | 65 |
| Bryan Area | 128 | 3 | 2 |
| Corpus | 3 | 1 | 2 |
| DFW | 84 | 4 | 3 |
| E RIO | 129 | 4 | 0 |
| Houston | 144 | 2 | 8 |
| Laredo | 87 | 77 | 90 |
| North | 18 | 8 | 12 |
| Northeast | 1 | 0 | 0 |
| San Antonio | 83 | 10 | 11 |
| South | 35 | 4 | 2 |
| W RIO | 90 | 20 | 0 |
| Western | 30 | 2 | 2 |
| Total | 903 | 183 | 197 |

http://www.ercot.com/mktinfo/services/rprsdr/index.html

OOME and Congestion Management Activity

Congestion affected by outages

Remedial action plan has been developed for the element

| Contingency | Overloaded Element | Days |
| --- | --- | --- |
| Temple Pecan Creek - Trading House & Temple Switch - Lake Creek SES 345 kV | Lake Creek SES - Temple Pecan Creek 138 kV | 14 |
| Enterprise - Trinidad & Martin Lake - Stryker Creek 345 kV | Nacogdoches 345/138 kV Auto 1 | 8 |
| Austrop 345/138 kV Auto 2 | Austrop 345/138 kV Auto 1 | 6 |
| Enterprise - Trinidad & martin lake SES - Stryker Creek 345 kV | Nacogdoches Southeast - Nocogdoches South 138 kV | 5 |
| Gibbons Creek - Twin Oak Switch345 kV | Jewett - Watson Chapel 138 kV | 4 |
| Cagnon Auto | Medical Center - Hamilton Wolfe | 4 |
| Centerville Switch - Forney Switch 345 kV | Lake Hubbard - Duck Creek Tap | 4 |
| Garfield - Onion & Hicross 138 kV | Lytton Spring Tr | 4 |
| Hill Country - Helotes 138 kV | Medical Center - Hamilton Wolfe | 3 |
| Gibbons Creek - Greens Prairie & Bryan East | College Station - Greens Prairie | 3 |
| Centerville Switch - Forney Switch 345 kV | Forney Switch - Duck Creek Tap | 3 |
| P H Robinson - Southshore 138 kV | Dickinson - League City 138 kV | 3 |
| Marion - Zorn & Clear Springs 345 kV | Seguin - Seguin West 138 kV | 3 |
| Allen Switch - Royse Switch & Monticello | Centerville Road Switch - Garland Rd | 2 |
| Trinidad SES - Stryker Creek SES & Mount Enterprise 345 kV | Athens North Switch - Athens Tap 138 kV | 2 |
| Bell County - Temple Switch 138 kV | Poage - Temple Taylor Valley 69 kV | 2 |
| Big Brown - Jewett 345 kV | Big Brown - Jewett 345 kV | 1 |
| Carrollton Northwest - Plano Tennyson & Collin | Collin SES - Frisco A 138 kV | 1 |
| China Grove - Morgan Creek 138 kV | Morgan Creek 138/69 kV Auto 3 | 1 |
| Graham - Parker & Benbrook 345 kV | Willow Creek - Parker 345 kV | 1 |
| Lake Creek - Jewett 138 kV | Navarro - Richland 69 kV | 1 |
| Martin Lake - Shamburger 345 kV | Tyler Southeast - Tyler South 138 kV | 1 |
| Menard - Gillespie LCRA 138 kV | Menard 138/69 kV auto | 1 |
| Menard 138/69 kV Auto | Eola - Paint Rock | 1 |
| Midland East - Odessa EHV Switch 345 kV | Odessa - Odessa EHV 138 kV | 1 |
| Midland East - Odessa EHV Switch 345 kV | Odessa Southwest - Moss Switch | 1 |
| Odessa EHV Switch 345/138 kV auto 2 | Odessa EHV Switch 345/138 kV auto 1 | 1 |
| San Angelo - Menard 138 kV | Ballinger - Paint Rock 69 kV | 1 |
| Skyline - Calaveras & Walzem 138 kV | Five Points - West Side 138 kV | 1 |
| South Lane - Lane City 138 kV | New Gult - Texas Gulf Sulphur 138 kV | 1 |
| Temple Switch - Bell County 138 kV | Poage - Temple Taylor Valley 69 kV | 1 |
| Temple Switch - Bell County 138 kV | Seaton - Poage 69 kV | 1 |
| Wynn Joyce 138/69 kV Transformer | Apollo - Naaman 69 kV | 1 |
| Trinidad - Martin Lake & Stryker Creek 345 kV | Nacogdoches 345/138 kV Auto 1 | 1 |
| Trinidad - Stryker Creek & Enterprise 345 kV | Stryker Creek - Stryker Cherokee POI | 1 |
| Sandow - Taylor 138 kV | Elgin - Taylor 138 kV | 1 |
| STP - Dow 345 kV | STP - White Point 345 kV | 1 |
| Gibbons Creek - Twin Oak Switch345 kV | Robertson - Watson Chapel 138 kV | 1 |
| Zorn - Lytton Springs & Austrop 345 kV | Canyon LCRA - Rohr 138 kV | 1 |

Zonal OC1

| CSC | Days |
| --- | --- |
| South - Houston | 15 |
| North - Houston | 11 |
| North - West | 4 |

New RAP

None

New Procedures/Forms/Operations Bulletins

| 06/29/2007 | Transmission & Security Desk Relief of Non-Spin Obligation | 336 |
| --- | --- | --- |
| 06/20/2007 | Day Ahead Desk V3, Rev31 | 333 |
| 06/09/2007 | Shift Supervisor, V3 Rev 39 | 335 |
| 06/01/2007 | Transmission & Security Desk, V3 Rev 84 | 334 |
| 06/01/2007 | Shift Supervisor Desk V3 Rev38 | 332 |

## Blackout/Load Shed Incidences

None

EECP

6/9 11:24 -12:10   Implemented EECP step 1.

6/18 15:09 –18:15   Implemented EECP step 1.

Major System Voltage Problems

None

Major Computer Problems, Enhancements & Fixes

6/1	00:00	OC0 shift factors were not populated into the MMS for June.

6/11 11:09 – 11:23  AGC application failed.

6/18  18:00  RPRS market failed to clear.

6/25  18:00  RPRS market failed to clear.

6/27  18:00  RPRS market failed to clear.

## Communication

6/28 Held interval 14:15.

## ERCOT peak (Unofficial)

6/19	17:00   56,350

Yearly MTLF Error Comparison

| Month | MAPE for MTLF | MAPE for MTLF | MAPE for MTLF |
| --- | --- | --- | --- |
|  | run at 16:00 | run at 16:00 | run at 16:00 |
|  | Day Ahead (2005) | Day Ahead (2006) | Day Ahead (2007) |
| January | 5.02 | 4.79 | 4.55 |
| February | 3.47 | 4.81 | 3.75 |
| March | 3.41 | 4.95 | 2.7 |
| April | 3.37 | 5.59 | 3.002 |
| May | 4.21 | 3.59 | 3.22 |
| June | 3.91 | 2.65 | 4.46 |
| Average Annual MAPE | 4.56 | 3.79 | 3.95 |

OCN, Advisory, Alerts & Emergency Notices

Capacity

| Date | Start | End | Details |
| --- | --- | --- | --- |
| 6/1/07 | 15:22 | 18:10 | Advisory issued for ARR below 3000 MW. |
| 6/2/07 | 14:01 | 19:00 | Advisory issued for ARR below 3000 MW. |
| 6/8/07 | 15:11 | 19:08 | Advisory issued ARR below 3000 MW. |
| 6/9/07 | 9:34 | 20:30 | Advisory issued ARR below 3000 MW. |
| 6/9/07 | 11:11 | 20:00 | Alert issued ARR below 2500 MW. |
| 6/9/07 | 11:24 | 12:10 | Declared EECP Step 1 due to ARR below 2300 MW. |
| 6/10/07 | 13:47 | 20:00 | Advisory issued ARR below 3000 MW. |
| 6/12/07 | 14:35 | 17:45 | Advisory issued ARR below 3000 MW. |
| 6/13/07 | 15:25 | 18:15 | Advisory issued ARR below 3000 MW. |
| 6/14/07 | 0:30 | 1:45 | Advisory issued ARR below 3000 MW. |
| 6/14/07 | 15:45 | 17:00 | Advisory issued ARR below 3000 MW. |
| 6/15/07 | 12:00 | 14:30 | OCN issued for shortage of UBES bids for HE15 through HE18. |
| 6/15/07 | 16:40 | 19:15 | Advisory issued ARR below 3000 MW. |
| 6/16/07 | 2:00 | 3:20 | Advisory issued ARR below 3000 MW. |
| 6/17/07 | 0:01 | 2:05 | Advisory issued ARR below 3000 MW. |
| 6/18/07 | 10:35 | 22:20 | Advisory issued ARR below 3000 MW. |
| 6/18/07 | 12:10 | 20:10 | Alert issued ARR below 2500 MW. |
| 6/18/07 | 15:09 | 18:15 | Declared EECP Step 1, ARR below 2300 MW. |
| 6/19/07 | 10:57 | 18:58 | Advisory issued ARR below 3000 MW. |
| 6/19/07 | 14:13 | 18:58 | Alert issued ARR below 2500 MW. |
| 6/22/07 | 14:00 | 19:19 | Advisory issued ARR below 3000 MW. |
| 6/23/07 | 10:55 | 17:53 | Advisory issued ARR below 3000 MW. |
| 6/24/07 | 12:46 | 19:46 | Advisory issued ARR below 3000 MW. |
| 6/24/07 | 14:21 | 19:46 | Alert issued ARR below 2500 MW. |
| 6/25/07 | 11:21 | 18:36 | Advisory issued ARR below 3000 MW. |
| 6/26/07 | 11:10 | 19:20 | Advisory issued ARR below 3000 MW. |
| 6/26/07 | 13:30 | 18:20 | Alert issued ARR below 2500 MW. |
| 6/27/07 | 12:55 | 21:05 | Advisory issued ARR below 3000 MW. |
| 6/27/07 | 15:45 | 18:10 | Alert issued ARR below 2500 MW. |
| 6/28/07 | 0:05 | 2:00 | Advisory issued ARR below 3000 MW. |
| 6/29/07 | 12:08 | 20:00 | Advisory issued ARR below 3000 MW. |
| 6/29/07 | 13:24 | 20:00 | Alert issued ARR below 2500 MW. |
| 6/29/07 | 23:15 | 24:00 | Advisory issued ARR below 3000 MW. |
| 6/30/07 | 0:15 | 6:03 | Advisory issued ARR below 3000 MW. |

Transmission

| Date | Start | End | Details |
| --- | --- | --- | --- |
| 6/15/07 | 16:30 | 20:00 | Alert for loss of Dilley Sw- San Miguel loads Derby - Pearsall |

Weather

| Date | Start | End | Details |
| --- | --- | --- | --- |
| 6/27/07 | 7:00 | 24:00 | OCN issued due to wet weather across Central Texas. |
| 6/28/07 | 0:15 | 24:00 | OCN issued due to wet weather across Central Texas. |
| 6/29/07 | 0:15 | 14:34 | OCN issued due to wet weather across Central Texas. |