ERCOT Nodal Protocols

Section 4: Day-Ahead Operations

August 1, 2007

(Effective Upon Texas Nodal Market Implementation)

4	Day-Ahead Operations	4-1

4.1	Introduction	4-1

4.1.1	Day-Ahead Timeline Summary	4-1

4.1.2	Day-Ahead Process and Timing Deviations	4-2

4.2	ERCOT Activities in the Day-Ahead	4-2

4.2.1	Ancillary Service Plan and Ancillary Service Obligation	4-2

4.2.1.1	Ancillary Service Plan	4-2

4.2.1.2	Ancillary Service Obligation Assignment and Notice	4-3

4.2.2	Wind-Powered Generation Resource Production Potential	4-3

4.2.3	Posting Forecasted ERCOT System Conditions	4-4

4.2.4	ERCOT Notice of Validation Rules for the Day-Ahead	4-5

4.3	QSE Activities and Responsibilities in the Day-Ahead	4-5

4.4	Inputs into DAM and Other Trades	4-5

4.4.1	Capacity Trades	4-5

4.4.1.1	Capacity Trade Criteria	4-6

4.4.1.2	Capacity Trade Validation	4-6

4.4.2	Energy Trades	4-7

4.4.2.1	Energy Trade Criteria	4-7

4.4.2.2	Energy Trade Validation	4-7

4.4.3	Self-Schedules	4-8

4.4.3.1	Self-Schedule Criteria	4-8

4.4.3.2	Self-Schedule Validation	4-9

4.4.4	DC Tie Schedules	4-9

4.4.4.1	DC Tie Schedule Criteria	4-10

4.4.4.2	DC Tie Schedule Validation	4-11

4.4.4.3	Oklaunion Exemption	4-11

4.4.5	CRR Offers	4-12

4.4.5.1	CRR Offer Criteria	4-12

4.4.5.2	CRR Offer Validation	4-13

4.4.6	PTP Obligation Bids	4-13

4.4.6.1	PTP Obligation Bid Criteria	4-13

4.4.6.2	PTP Obligation Bid Validation	4-14

4.4.7	Ancillary Service Supplied and Traded	4-14

4.4.7.1	Self-Arranged Ancillary Service Quantities	4-14

4.4.7.2	Ancillary Service Offers	4-15

4.4.7.2.1	Ancillary Service Offer Criteria	4-16

4.4.7.2.2	Ancillary Service Offer Validation	4-17

4.4.7.3	Ancillary Service Trades	4-17

4.4.7.3.1	Ancillary Service Trade Criteria	4-18

4.4.7.3.2	Ancillary Service Trade Validation	4-18

4.4.7.4	Ancillary Service Supply Responsibility	4-19

4.4.8	RMR Offers	4-20

4.4.9	Energy Offers and Bids	4-20

4.4.9.1	Three-Part Supply Offers	4-20

4.4.9.2	Startup Offer and Minimum-Energy Offer	4-21

4.4.9.2.1	Startup Offer and Minimum-Energy Offer Criteria	4-21

4.4.9.2.2	Startup Offer and Minimum-Energy Offer Validation	4-22

4.4.9.2.3	Startup Offer and Minimum-Energy Offer Generic Caps	4-22

4.4.9.2.4	Verifiable Startup Offer and Minimum-Energy Offer Caps	4-24

4.4.9.3	Energy Offer Curve	4-24

4.4.9.3.1	Energy Offer Curve Criteria	4-25

4.4.9.3.2	Energy Offer Curve Validation	4-26

4.4.9.3.3	Energy Offer Curve Caps for Make-Whole Calculation Purposes	4-26

4.4.9.4	Mitigated Offer Cap and Mitigated Offer Floor	4-27

4.4.9.4.1	Mitigated Offer Cap	4-27

4.4.9.4.2	Mitigated Offer Floor	4-28

4.4.9.5	DAM Energy-Only Offer Curves	4-28

4.4.9.5.1	DAM Energy-Only Offer Curve Criteria	4-29

4.4.9.5.2	DAM Energy-Only Offer Validation	4-29

4.4.9.6	DAM Energy Bids	4-30

4.4.9.6.1	DAM Energy Bid Criteria	4-30

4.4.9.6.2	DAM Energy Bid Validation	4-30

4.4.10	Credit Requirement for DAM Bids and Offers	4-31

4.4.11	System-Wide Offer Caps	4-32

4.4.11.1	Scarcity Pricing Mechanism	4-32

4.5	DAM Execution and Results	4-33

4.5.1	DAM Clearing Process	4-33

4.5.2	Ancillary Service Insufficiency	4-36

4.5.3	Communicating DAM Results	4-37

4.6	DAM Settlement	4-38

4.6.1	Day-Ahead Settlement Point Prices	4-38

4.6.1.1	Day-Ahead Settlement Point Prices for Resource Nodes	4-38

4.6.1.2	Day-Ahead Settlement Point Prices for Load Zones	4-39

4.6.1.3	Day-Ahead Settlement Point Prices for Hubs	4-39

4.6.2	Day-Ahead Energy and Make-Whole Settlement	4-39

4.6.2.1	Day-Ahead Energy Payment	4-39

4.6.2.2	Day-Ahead Energy Charge	4-40

4.6.2.3	Day-Ahead Make-Whole Settlements	4-41

4.6.2.3.1	Day-Ahead Make-Whole Payment	4-42

4.6.2.3.2	Day-Ahead Make-Whole Charge	4-46

4.6.3	Settlement for PTP Obligations Bought in DAM	4-48

4.6.4	Settlement of Ancillary Services Procured in the DAM	4-49

4.6.4.1	Payments for Ancillary Services Procured in the DAM	4-49

4.6.4.1.1	Regulation Up Service Payment	4-49

4.6.4.1.2	Regulation Down Service Payment	4-50

4.6.4.1.3	Responsive Reserve Service Payment	4-50

4.6.4.1.4	Non-Spinning Reserve Service Payment	4-51

4.6.4.2	Charges for Ancillary Services Procurement in the DAM	4-52

4.6.4.2.1	Regulation Up Service Charge	4-52

4.6.4.2.2	Regulation Down Service Charge	4-53

4.6.4.2.3	Responsive Reserve Service Charge	4-54

4.6.4.2.4	Non-Spinning Reserve Service Charge	4-55

4.6.5	Calculation of “Average Incremental Energy Cost” (AIEC)	4-56

# Day-Ahead Operations

- 4.1	Introduction

(1)	The Day-Ahead Market (DAM) is a daily, co-optimized market in the Day-Ahead for Ancillary Service capacity, certain Congestion Revenue Rights, and forward financial energy transactions.

(2)	Participation in the DAM is voluntary, except for Reliability Must Run (RMR) Units, the participation of which is governed by their respective RMR Agreements and Section 4.4.8, RMR Offers.

(3)	DAM energy settlements use DAM Settlement Point Prices that are calculated for Resource Nodes, Load Zones, and Hubs for a one-hour Settlement Interval using the LMPs from DAM.  In contrast, the Real-Time energy settlements use Real-Time Settlement Point Prices that are calculated for Resource Nodes, Load Zones, and Hubs for a 15-minute Settlement Interval.

4.1.1	Day-Ahead Timeline Summary

The figure below shows the major activities that occur in the Day-Ahead:

4.1.2	Day-Ahead Process and Timing Deviations

(1)	ERCOT may temporarily deviate from the timing of its obligations in this Section but only to the extent necessary to ensure the secure operation of the ERCOT System.  In that event, ERCOT shall immediately issue an Alert and notify all QSEs of the following:

(a)	Details of the affected timing and procedures;

(b)	Details of any interim requirements;

(c)	An estimate of the period for which the interim requirements apply; and

(d)	Reasons for the temporary variation.

(2)	If, despite the varying timing or omitting any procedure, ERCOT is unable to execute the Day-Ahead process, ERCOT may abort all or part of the Day-Ahead process and require all schedules and trades to be submitted in the Adjustment Period.  In that event, ERCOT shall declare an Emergency Condition and notify all QSEs of the following:

(a)	Details of the affected timing and procedures;

(b)	Details of any interim requirements;

(c)	An estimate of the period for which the interim requirements apply; and

(d)	Reasons for the temporary variation.

(3)	If, despite varying timing or omitting steps, ERCOT is unable to operate the Adjustment Period process, then ERCOT may abort the Adjustment Period process and operate under its Operating Period procedures.

- 4.2	ERCOT Activities in the Day-Ahead

4.2.1	Ancillary Service Plan and Ancillary Service Obligation

4.2.1.1	Ancillary Service Plan

(1)	ERCOT shall analyze the expected Load conditions for the Operating Day and develop an Ancillary Service Plan that identifies the Ancillary Service MW necessary for each hour of the Operating Day.  The MW of each Ancillary Service required may vary from hour to hour depending on ERCOT System conditions.  ERCOT must post the Ancillary Service Plan to the MIS Public Area by 0600 of the Day-Ahead.

(2)	If ERCOT determines that an Emergency Condition may exist that would adversely affect ERCOT System reliability, it may change the percentage of Load Resources that are allowed to provide Responsive Reserve Service (RRS) from the monthly amounts determined previously, as described in Section 3.16, Standards for Determining Ancillary Service Quantities, and must post any change in the percentage to the MIS Public Area by 0600 of the Day-Ahead.

(3)	ERCOT shall determine the total required amount of each Ancillary Service under Section 3.16, or use its operational judgment and experience to change the daily quantity of each required Ancillary Service.

(4)	ERCOT shall include in the Ancillary Service Plan enough capacity to automatically control frequency with the intent to meet NERC standards.

(5)	ERCOT shall notify the QSE representing an RMR Unit for any unit that is being committed in the DAM or the DRUC at the same time that the DAM and DRUC participants are notified of the results of that respective process.

(6)	Once specified by ERCOT for an hour and published on the MIS Public Area, Ancillary Service quantity requirements for an Operating Day may not be decreased.

4.2.1.2	Ancillary Service Obligation Assignment and Notice

(1)	ERCOT shall assign part of the Ancillary Service Plan quantity, by service, by hour, to each LSE based on Load Ratio Share and shall then aggregate those quantities, by service, by hour to the QSE level.  The resulting Ancillary Service quantity for each QSE, by service, by hour, is called its Ancillary Service Obligation. ERCOT shall base the LSE Ancillary Service allocation on the hourly Load Ratio Share from the real time market data used for Initial Settlement for the same hour and day of the week, for the most recent day for which Initial Settlement Statements are available, multiplied by the quantity of that service required in the Day-Ahead Ancillary Service Plan. The Ancillary Service Obligation defined shall be adjusted based on the most current real time settlement and resettlement data for the Operating Day for which the Ancillary Service was procured.

(2)	By 0600 of the Day-Ahead, ERCOT shall notify each QSE of its Ancillary Service Obligation for each service and for each hour of the Operating Day.

(3)	By 0600 of the Day-Ahead, ERCOT shall post on the MIS Certified Area each QSE’s Load Ratio Share used for the Ancillary Service Obligation calculation.

4.2.2	Wind-Powered Generation Resource Production Potential

(1)	ERCOT shall produce and update hourly a Short-Term Wind Power Forecast (STWPF) that provides a rolling 48-hour hourly forecast of wind production potential for each Wind-Powered Generation Resource (WGR). Each Generation Entity that owns a WGR shall install and telemeter to ERCOT the site-specific meteorological information that ERCOT determines is necessary to produce the STWPF forecasts. ERCOT shall establish procedures specifying the accuracy requirements of WGR meteorological information telemetry.

(2)	The WGR Production Potential (WGRPP) is an hourly 80% confidence level forecast of energy production for each WGR. From the STWPF, ERCOT shall produce and update WGRPP forecasts each hour for each WGR to be used as input into each RUC process as per Section 5, Transmission Security Analysis and Reliability Unit Commitment.

(3)	ERCOT shall produce the WGRPP forecasts using the STWPF information provided by WGR owners including WGR availability, meteorological information, and SCADA.

(4)	Each hour, ERCOT shall provide, through the Messaging System, the WGRPP forecasts for each WGR to the QSE that represents that WGR and shall post each WGRPP forecast on the MIS Certified Area.

(5)	Each hour, ERCOT shall post the aggregated WGRPP forecast of all WGRs on the MIS Secure Area.

4.2.3	Posting Forecasted ERCOT System Conditions

No later than 0600 in the Day-Ahead, ERCOT shall post on the MIS Secure Area, and make available for download, the following information for the Operating Day:

(a)	The Network Operations Model topology that includes known transmission line and other Transmission Facilities Outages in the Common Information Model format for the minimum Load hour and the peak Load hour;

(b)	Weather assumptions used by ERCOT to forecast ERCOT System conditions and used in the Dynamic Rating Processor;

(c)	Any weather-related changes to the transmission contingency list;

(d)	ERCOT System, Weather Zone, and Load Zone Load forecasts for the next seven days, by hour, and a message on update indicating any changes to the forecasts by means of the Messaging System;

(e)	Load forecast distribution factors from which Market Participants can calculate Load at the Electrical Bus level by hour for the next seven days;

(f)	Load Profiles for non-IDR metered Customers;

(g)	Distribution Loss Factors and forecasted  ERCOT-wide Transmission Loss Factors, as described in Section 13.3, Distribution Losses and in Section 13.2, Transmission Losses,  for each Settlement Interval of the Operating Day;

(h)	A current list of all Settlement Points that may be used for market processes and transactions;

(i)	A mapping of Settlement Points to Electrical Buses in the Network Operations Model; and

(j)	A list of transmission constraints that have a high probability of binding in the Security-Constrained Economic Dispatch (SCED) or DAM.

4.2.4	ERCOT Notice of Validation Rules for the Day-Ahead

ERCOT shall provide each QSE with the information necessary to pre-validate its data for DAM, including publishing validation rules for offers, bids and trades and posting any software documentation and code that is not Protected Information to the MIS Secure Area within five Business Days after ERCOT receives it.

- 4.3	QSE Activities and Responsibilities in the Day-Ahead

(1)	During the Day-Ahead, a QSE:

(a)	Must submit its COP and update its COP as required in Section 3.9, Current Operating Plan (COP);

(b)	May submit Three-Part Supply Offers, DAM Energy-Only Offers, DAM Energy Bids, Energy Trades, Self-Schedules, Capacity Trades, DC Tie Schedules, Ancillary Service Offers, Ancillary Service Trades, Self-Arranged Ancillary Service Quantities, PTP Obligation Bids, and CRR Offers as specified in this Section; and

(2)	By 0600 in the Day-Ahead, each QSE representing RMR Units, or Black Start Resources shall submit information to ERCOT indicating availability of RMR Units, and Black Start Resources for the Operating Day, and any other information that ERCOT may need to evaluate use of the units as set forth in the applicable Agreements and this Section.

- 4.4	Inputs into DAM and Other Trades

4.4.1	Capacity Trades

(1)	A Capacity Trade is the information for a QSE-to-QSE transaction that transfers financial responsibility for capacity between a buyer and a seller.

(2)	A Capacity Trade for hours in the Operating Day that is reported to ERCOT before 1430 in the Day-Ahead creates:

(i)	A capacity supply in the DRUC process for the buyer; and

(ii)	A capacity obligation in the DRUC process for the seller.

(3)	A Capacity Trade submitted at or after 1430 in the Day-Ahead for the Operating Day creates a capacity supply or obligation in any HRUC processes executed after the Capacity Trade is reported to ERCOT.  Capacity Trades submitted after the DRUC snapshot are considered in the Adjustment Period snapshot.

(4)	As soon as practicable, ERCOT shall notify each QSE through the Messaging System of any of its Capacity Trades that are invalid Capacity Trades.  The QSE may correct and resubmit any invalid Capacity Trade within the appropriate market timeline.

4.4.1.1	Capacity Trade Criteria

(1)	A Capacity Trade must be submitted by a QSE and must include the following:

(a)	The buying QSE;

(b)	The selling QSE;

(c)	The quantity in MW; and

(d)	The first hour and last hour of the trade.

(2)	A Capacity Trade must be confirmed by both the buyer and seller to be considered valid.

4.4.1.2	Capacity Trade Validation

(1)	A validated Capacity Trade is a Capacity Trade that ERCOT has determined meets the criteria listed in Section 4.4.1.1, Capacity Trade Criteria.  Only one confirmed Capacity Trade is allowed for the same buying and selling QSEs for each hour.

(2)	When a Capacity Trade is reported to ERCOT, ERCOT shall notify both the buying and selling QSEs by using the Messaging System, if available, and on the MIS Certified Area.

(3)	ERCOT shall continuously validate Capacity Trades and continuously display on the MIS Certified Area information that allows any QSE named in a Capacity Trade to view confirmed and unconfirmed Capacity Trades.

(4)	The QSE that first reports the Capacity Trade to ERCOT is deemed to have confirmed the Capacity Trade unless it subsequently affirmatively rejects it.  The QSE that first reports a Capacity Trade may reject, edit, or delete a Trade that its counterpart has not confirmed.  The counterpart is deemed to have confirmed the Capacity Trade when it submits to ERCOT an identical Capacity Trade.  After both the buyer and seller have confirmed a Capacity Trade, either party may reject it at any time, but the rejection is effective only for any ERCOT settlement process for which the deadline for reporting Capacity Trades has not yet passed.

4.4.2	Energy Trades

(1)	An Energy Trade is the information for a QSE-to-QSE transaction that transfers financial responsibility for energy at a Settlement Point between a buyer and a seller.

(2)	An Energy Trade for hours in the Operating Day that is reported to ERCOT before 1430 in the Day-Ahead creates a capacity supply or obligation in the DRUC process.  Energy Trades submitted after 1430 in the Day-Ahead for the Operating Day create a capacity supply or obligation in any HRUC processes executed after the Energy Trade is reported to ERCOT.  Energy Trades submitted after the DRUC snapshot are considered in the Adjustment Period.

(3)	An Energy Trade may be submitted for any Settlement Interval within an Operating Day before 1430 of the following day.

(4)	As soon as practicable, ERCOT shall notify each QSE through the Messaging System of any of its Energy Trades that are invalid Energy Trades.  The QSE may correct and resubmit any invalid Energy Trade within the appropriate market timeline.

4.4.2.1	Energy Trade Criteria

(1)	Each Energy Trade must be reported by a QSE and must include the following information:

(a)	The buying QSE;

(b)	The selling QSE;

(c)	The quantity of MW for each 15-minute Settlement Interval of the trade;

(d)	The first and last 15-minute Settlement Intervals of the trade; and

(e)	The Settlement Point of the trade.

(2)	An Energy Trade must be confirmed by both the buyer and seller to be considered valid.

4.4.2.2	Energy Trade Validation

(1)	A validated Energy Trade is an Energy Trade that ERCOT has determined meets the criteria listed in Section 4.4.2.1, Energy Trade Criteria.  Only one confirmed Energy Trade is allowed for the same buying and selling QSEs at the same Settlement Point for each 15-minute Settlement Interval.

(2)	When an Energy Trade is reported to ERCOT, ERCOT shall notify both the buying and selling QSEs by using the Messaging System if available and the MIS Certified Area.

(3)	ERCOT shall continuously validate Energy Trades and continuously display on the MIS Certified Area information that allows any QSE named in an Energy Trade to view confirmed and unconfirmed Energy Trades.

(4)	The QSE that first reports the Energy Trade to ERCOT is considered to have confirmed the Energy Trade unless it subsequently affirmatively rejects it.  The QSE that first reports an Energy Trade may reject, edit, or delete an Energy Trade that its counterpart has not confirmed.  The counterpart is deemed to have confirmed the Energy Trade when it submits an identical Energy Trade.  After both the buyer and seller have confirmed an Energy Trade, either party may reject it at any time, but the rejection is effective only for any ERCOT process for which the deadline for reporting Energy Trades has not yet passed.

4.4.3	Self-Schedules

(1)	A Self-Schedule is the information that a QSE submits for Real-Time Settlement that specifies the amount of the QSE’s energy supply at a specified source Settlement Point to be used to meet the QSE’s energy obligation at a specified sink Settlement Point .

(2)	A Self-Schedule may be submitted for any Settlement Interval before the end of the Adjustment Period for that Settlement Interval.

(3)	As soon as practicable, ERCOT shall notify the QSE through the Messaging System of any of its Self-Schedules that are invalid Self-Schedules.  The QSE may correct and resubmit any invalid Self-Schedule within the appropriate market timeline.

4.4.3.1	Self-Schedule Criteria

(1)	Each Self-Schedule must be reported by a QSE and must include the following information:

(a)	The name of the QSE;

(b)	The quantity of MW for each 15-minute Settlement Interval of the schedule;

(c)	The first and last 15-minute Settlement Intervals of the schedule; and

(d)	The source Settlement Point of the schedule;

(e)	The sink Settlement Point of the schedule.

4.4.3.2	Self-Schedule Validation

(1)	A validated Self-Schedule is a Self-Schedule that ERCOT has determined meets the criteria listed in Section 4.4.3.1, Self-Schedule Criteria.

(2)	ERCOT shall continuously validate Self-Schedules and continuously display on the MIS Secure Area information that allows the QSE named in a Self-Schedule to view validated Self-Schedules.

4.4.4	DC Tie Schedules

(1)	A DC Tie Schedule is the information for a physical transaction between a buyer and a seller, one of which is in ERCOT and the other of which is in a Non-ERCOT Control Area, for energy at a Settlement Point that is a DC Tie. A DC Tie Schedule must be implemented under these Protocols, any applicable NERC scheduling protocols, any applicable NERC operating policies, and any applicable operating agreements between ERCOT and Mexico.  A DC Tie Schedule must be transaction-specific, i.e., one schedule per transaction per DC Tie, rather than aggregate (net) schedules per DC Tie.

(2)	Each QSE shall follow all NERC policies for tagging of Control Area interchange transactions.  Only transactions across ERCOT interconnections to SPP, WSCC, Mexico, or other areas must be tagged by the QSE as prescribed in the NERC tagging guidelines.

(3)	A DC Tie Schedule for hours in the Operating Day that is reported to ERCOT before 1430 in the Day-Ahead creates a capacity supply or for the equivalent Resource or an obligation for the equivalent load of the DC Tie in the DRUC process.  DC Tie Schedules submitted after 1430 in the Day-Ahead for the Operating Day create a capacity supply or obligation in any applicable HRUC processes executed after the DC Tie Schedule is reported to ERCOT.  DC Tie Schedules submitted after the RUC snapshot are considered in the Adjustment Period snapshot in accordance with the market timeline.

(4)	As soon as practicable, ERCOT shall notify each QSE through the Messaging System of any of its DC Tie Schedules that are invalid DC Tie Schedules.  The QSE may correct and resubmit any invalid DC Tie Schedules within the appropriate market timeline.

(5)	A QSE that is an importer into ERCOT through a DC Tie in a Settlement Interval under a DC Tie Schedule must be treated as a Resource at that DC Tie Settlement Point for that Settlement Interval.

(6)	A QSE that is an exporter from ERCOT through a DC Tie in a Settlement Interval under a DC Tie Schedule must be treated as a Load at the DC Tie Settlement Point for that Settlement Interval and is responsible for allocated Transmission Losses, UFE System Administration Fee, and any other applicable ERCOT fees.  This applies to all exports across the DC Ties except those that qualify for the Oklaunion Exemption.

(7)	ERCOT shall confirm each valid DC Tie Schedule with the applicable interconnected non-ERCOT Control Area and shall coordinate the approval process for the NERC tags for the ERCOT Control Area.

(8)	Using the DC Tie Schedule information submitted by QSEs, ERCOT shall update and maintain a Current Operating Plan for each DC Tie for which the aggregated DC Tie Schedules for that tie show a net export out of ERCOT for the applicable interval.  When the net energy schedule for a DC Tie indicates an export, ERCOT shall treat the DC Tie as an Off-Line Resource and set the HSL and LSL for that DC Tie Resource to zero.  ERCOT shall monitor the associated Resource Status telemetry during the Operating Period. When the net energy schedule for a DC Tie shows a net import, the Resource HSL, HASL and LSL must be set appropriately, considering the resulting net import and any Ancillary Service Schedules for the DC Tie Resource.

(9)	A QSE submitting a DC Tie Schedule shall:

(a)	Secure and maintain a NERC tag service to submit NERC tags and monitor NERC tag status according to NERC requirements;

(b)	Submit NERC tags for all proposed transactions; and

(c)	Implement backup procedures in case of NERC tag service failure.

4.4.4.1	DC Tie Schedule Criteria

(1)	Each DC Tie Schedule must be submitted by a QSE and must include the following information:

(a)	The QSE or non-ERCOT Control Area buying the energy;

(b)	The QSE or non-ERCOT Control Area selling the energy;

(c)	For each DC Tie Schedule, the DC Tie Settlement Point;

(d)	The quantity in MW for each 15-minute Settlement Interval of the schedule;

(e)	The first and last 15-minute Settlement Intervals of the schedule; and

(f)	The NERC tag information, which must conform to the standards set forth in NERC Policy 3 and associated appendixes.

(2)	A DC Tie Schedule must be intended to match what the submitting QSE reasonably expects the DC Tie Schedule to be in Real-Time.

(3)	A DC Tie Schedule must be confirmed by the non-ERCOT Control Area to be considered valid.

4.4.4.2	DC Tie Schedule Validation

(1)	A validated DC Tie Schedule is a DC Tie Schedule that ERCOT has determined:

(a)	Meets the criteria listed in Section 4.4.4.1, DC Tie Schedule Criteria;

(b)	Is matched—in quantity, time period, DC Tie Settlement Point, and other NERC tag information—by a schedule submitted by a non-ERCOT Control Area; and

(c)	For the NERC tag:

(i)	All Control Areas and transmission service providers with approval rights approve the NERC tag (active approval); or

(ii)	No Entity with approval rights over the NERC tag has denied it, and the approval time window has ended (passive approval).

(2)	Any changes in the interconnected non-ERCOT Control Area schedules due to a de-rating of the DC Tie or other change within the NERC or Mexico’s scheduling protocols must be communicated to ERCOT by the DC Tie Operator or designated reliability authority for the interconnected non-ERCOT Control Area.  For any interconnected non-ERCOT Control Area schedules revised during the Operating Period, the DC Tie Operator shall communicate to ERCOT the integrated schedule for the Settlement Intervals.  If the DC Tie Schedule flows as planned, then ERCOT shall use schedules as the deemed meter readings for Real-Time settlement.  If the interconnected non-ERCOT Control Area schedule changes during the Operating Period, then ERCOT shall use the changed interconnected non-ERCOT Control Area schedule as the deemed meter reading for Real-Time settlement.

4.4.4.3	Oklaunion Exemption

(1)	The export schedules from the Public Service Company of Oklahoma, the Oklahoma Municipal Power Authority, and the AEP Texas North Company for their share of the Oklaunion Resource over the North DC Tie are not treated as Load connected at transmission voltage, are not subject to any of the fees described in Section 4.4.4, DC Tie Schedules, and are limited to the actual net output of the Oklaunion Resource (“Oklaunion Exemption”).  ERCOT shall record DC Tie Schedules that qualify for the Oklaunion Exemption to support the billing of applicable TSP tariffs.

(2)	A QSE requesting the Oklaunion Exemption shall:

(a)	Apply to ERCOT for the exemption;

(b)	Set up  a separate QSE (or sub-QSE) solely to schedule DC Tie exports under the exemption; and

(c)	Secure the Resources for a DC Tie Schedule by a DC Tie Schedule from each QSE representing part or all the Oklaunion Resource.

(3)	ERCOT shall verify for each Settlement Interval that the sum of the “exempted” exports under the Oklaunion Exemption is not more than the total output from the Oklaunion Resource.

4.4.5	CRR Offers

(1)	A CRR Offer is the information for an offer by a CRR Account Holder to sell CRRs that it owns in the DAM.

(2)	All CRRs held by CRR Account Holders are settled based on applicable DAM settlement prices, except for PTP Options and PTP Options with Refund that have been declared by a NOIE before DAM execution to be settled in Real-Time and are still held by that NOIE in Real-Time.

(3)	PTP Options and PTP Options with Refund that are declared by NOIEs for Real-Time settlement may specify an offer price (Minimum Reservation Price) in the DAM.  If no Minimum Reservation Price is specified, ERCOT shall assign a default value of $2,000 per MW per hour, as an offer in the DAM.  If such an offer clears in the DAM, it is settled as part of the DAM and is not carried to Real-Time.

4.4.5.1	CRR Offer Criteria

(1)	A CRR Offer must include the following:

(a)	The name of the CRR Account Holder that owns the CRRs being offered;

(b)	The unique identifier for each CRR being offered, which includes the single type of CRR being offered;

(c)	The source Settlement Point and the sink Settlement Point for the CRR or block of CRRs being offered;

(d)	The first hour and the last hour for which the CRR or block of CRRs is being offered;

(e)	The quantity of CRRs in MW for which the Minimum Reservation Price is effective;

(f)	A dollars per MW per hour for the Minimum Reservation Price; and

(g)	For PTP Options that a NOIE has designated for Real-Time settlement, the NOIE peak Load forecast for the Operating Day.

(2)	The CRR Account Holder for whom the CRR Offer is being submitted must be shown as the owner in the ERCOT CRR registration system of the CRRs being offered.

(3)	If the CRR Offer is for more than one CRR (which is 1 MW for one hour), the CRR Offer must have the following characteristics:

(a)	All CRRs must be of the same type;

(b)	All CRRs must have the same source and sink Settlement Points, and

(c)	A block CRR Offer must have the same number of CRRs offered in each hour; and

(d) 	A block CRR Offer must have contiguous hours for the CRRs offered.

(4)	For each NOIE that designated PTP Options or PTP Options with Refund for Real-Time settlement, the designation of such CRRs to be settled in Real-Time may not exceed 110% of that NOIE’s peak Load forecast.

4.4.5.2	CRR Offer Validation

(1)	A validated CRR Offer is a CRR Offer that ERCOT has determined meets the criteria listed in Section 4.4.5.1, CRR Offer Criteria.

(2)	ERCOT shall continuously display on the MIS Certified Area information that allows any QSE submitting a CRR Offer to view its valid CRR Offers.

(3)	As soon as practicable, ERCOT shall notify each CRR Account Holder through the Messaging System of any of its CRR Offers that are invalid.  The CRR Account Holder may correct and resubmit any invalid CRR Offer within the appropriate market timeline.

4.4.6	PTP Obligation Bids

(1)	A PTP Obligation Bid is a bid that specifies the source and sink, a range of hours, and a maximum price that the bidder is willing to pay (“Not-to-Exceed Price”).

(2)	PTP Obligations that are bought in the DAM must be settled based on the applicable Real-Time Settlement Point Prices.

4.4.6.1	PTP Obligation Bid Criteria

(1)	A PTP Obligation Bid must be submitted by a QSE and must include the following:

(a)	The name of the QSE submitting the PTP Obligation Bid;

(b)	The source Settlement Point and the sink Settlement Point for the PTP Obligation or block of PTP Obligations being bid;

(c)	The first hour and the last hour for which the PTP Obligation or block of PTP Obligations is being bid;

(d)	The quantity of PTP Obligations in MW for which the Not-to-Exceed Price is effective; and

(e)	A dollars per MW per hour for the Not-to-Exceed Price.

(2)	If the PTP Obligation Bid is for more than one PTP Obligation (which is 1 MW for one hour), the block bid must:

(a) 	Include the same number of PTP Obligations in each hour of the block;

(b) 	Be for PTP Obligations that have the same source and sink Settlement Points; and

(c) 	Be for contiguous hours.

4.4.6.2	PTP Obligation Bid Validation

(1)	A validated PTP Obligation Bid is a bid that ERCOT has determined meets the criteria listed in Section 4.4.6.1, PTP Obligation Bid Criteria.

(2)	ERCOT shall continuously display on the MIS Certified Area information that allows any QSE submitting a PTP Obligation Bid to view its valid PTP Obligation Bid.

(3)	As soon as practicable, ERCOT shall notify each QSE through the Messaging System of any of its PTP Obligation Bids that are invalid.  The QSE may correct and resubmit any invalid PTP Obligation Bid within the appropriate market timeline.

4.4.7	Ancillary Service Supplied and Traded

4.4.7.1	Self-Arranged Ancillary Service Quantities

(1)	A QSE may self-arrange all or a portion thereof, but not to exceed, the Ancillary Service Obligation allocated to it by ERCOT.  If a QSE elects to self-arrange Ancillary Service capacity, then ERCOT shall not pay the QSE for the Self-Arranged Ancillary Service Quantities for the portion that meets its Ancillary Service Obligation.

(2)	The QSE must indicate before 1000 in the Day-Ahead the Self-Arranged Ancillary Service Quantities, by service, so ERCOT can determine how much Ancillary Service capacity, by service, needs to be obtained through the DAM.

(3)	At or after 1000 in the Day-Ahead, a QSE may not change its Self-Arranged Ancillary Service Quantities unless ERCOT opens a Supplemental Ancillary Service Market.

(4)	Before 1430 in the Day-Ahead, all Self-Arranged Ancillary Service Quantities must be represented by physical capacity, either by Generation Resources or Load Resources, or backed by Ancillary Service Trades.

(5)	When a QSE chooses to self-arrange all or a portion of its Ancillary Service Obligations, it commits to the following conditions:

(a)	The QSE may self-arrange Regulation Up Service (Reg-Up), Regulation Down Service (Reg-Down), Responsive Reserve Service (RRS), and Non-Spin;

(b)	The QSE may provide all or part of its Self-Arranged Ancillary Service Quantity from one or more Resources it represents;

(c)	The QSE may provide all or a part of its Self-Arranged Ancillary Service Quantity through an Ancillary Service Trade;

(d)	The additional Self-Arranged Ancillary Service Quantity specified by the QSE in response to a Supplemental Ancillary Service Market notice by ERCOT to obtain additional Ancillary Services in the Adjustment Period cannot be more than the additional Ancillary Service amount allocated by ERCOT to that QSE, as stated in the SASM notice, and cannot be changed once committed to ERCOT; and

(e)	If a QSE does not self-arrange all of its Ancillary Service Obligation, ERCOT shall procure the remaining amount of the Ancillary Service Obligation for the QSE.

4.4.7.2	Ancillary Service Offers

(1)	By 1000 in the Day-Ahead, a QSE may submit Generation Resource-specific Ancillary Service Offers to ERCOT for the DAM and may offer the same Generation Resource capacity for any or all of the Ancillary Service products simultaneously with any Energy Offer Curves from that Generation Resource in the DAM. A QSE may also submit Ancillary Service Offers in a Supplemental Ancillary Service Market (SASM). Offers of more than one Ancillary Service product from one Generation Resource may be inclusive or exclusive of each other and of any Energy Offer Curves, as specified according to a procedure developed by ERCOT.

(2)	By 1000 in the Day-Ahead, a QSE may submit Load Resource-specific Ancillary Service Offers for Regulation Service, Non-Spinning Reserve Service and Responsive Reserve Service to ERCOT and may offer the same Load Resource capacity for any or all of those Ancillary Service products simultaneously. Offers of more than one Ancillary Service product from one Load Resource may be inclusive or exclusive of each other, as specified according to a procedure developed by ERCOT.

(3)	Ancillary Service Offers remain active for the offered period until either:

(a)	Selected by ERCOT;

(b)	Automatically inactivated by the software at the offer expiration time specified by the QSE when the offer is submitted; or

(c)	Withdrawn by the QSE, but a withdrawal is not effective if the deadline for submitting offers has already passed.

(4)	A Load Resource that is not a Controllable Load Resource may specify whether its Ancillary Service Offer for Responsive Reserve Service may only be procured by ERCOT as a block.

4.4.7.2.1	Ancillary Service Offer Criteria

(1)	Each Ancillary Service Offer must be submitted by a QSE and must include the following information:

(a)	The selling QSE;

(b)	The Resource represented by the QSE from which the offer would be supplied;

(c)	The quantity in MW and Ancillary Service type from that Resource for this specific offer and the specific quantity in MW and Ancillary Service type of any other Ancillary Service offered from this same capacity;

(d)	An Ancillary Service Offer linked to a Three-Part Supply Offer from a Resource designated to be Off-Line for the offer period in its COP may only be struck if the Three-Part Supply Offer is struck.  The total capacity struck must be within limits as defined in item (4)(c)(iii) of Section 4.5.1, DAM Clearing Process.

(e)	An Ancillary Service Offer linked to other Ancillary Service offers or an Energy Offer Curve from a Resource designated to be On-Line for the offer period in its COP may only be struck if the total capacity struck is within limits as defined in item (4)(c)(iii) of Section 4.5.1.

(f)	The first and last hour of the offer;

(g)	A fixed quantity block, or variable quantity block indicator for the offer;

(i)	If a fixed quantity block, not to exceed 150 MW, which may only be offered by a Load Resource, the single price (in $/MW) and single quantity (in MW) for all hours offered in that block;

(ii)	If a variable quantity block, which may be offered by a Generation Resource or a Load Resource, the single price (in $/MW) and single “up to” quantity (in MW) contingent on the purchase of all hours offered in that block; and

(h)	The expiration time and date of the offer.

(2)	A valid Ancillary Service Offer in the DAM must be received before 1000 for the effective DAM.  A valid Ancillary Service Offer in a SASM must be received before the applicable deadline for that SASM.

(3)	No Ancillary Service Offer price may exceed the System-Wide Offer Cap (in $/MW).

(4)	The minimum amount per Resource for each Ancillary Service product that may be offered is one MW.

(5)	A Resource may offer more than one Ancillary Service.

(6)	A Load Resource that is qualified to perform as a Controllable Load Resource may not offer to provide Ancillary Services as a Controllable Load Resource and a Load Resource controlled by high-set under-frequency relay simultaneously behind a common breaker.

4.4.7.2.2	Ancillary Service Offer Validation

(1)	A valid Ancillary Service Offer is one that ERCOT has determined meets the criteria listed in Section 4.4.7.2.1, Ancillary Service Offer Criteria.

(2)	ERCOT shall continuously validate Ancillary Service Offers and continuously display on the MIS Certified Area information that allows any QSE named in an Ancillary Service Offer to view its confirmed Ancillary Service Offers.

(3)	ERCOT shall notify the QSE submitting an Ancillary Service Offer if the offer was rejected or was considered invalid for any reason.  The QSE may then resubmit the offer within the appropriate market timeline.

4.4.7.3	Ancillary Service Trades

(1)	An Ancillary Service Trade is the information for a QSE-to-QSE transaction that transfers an obligation to provide Ancillary Service capacity between a buyer and a seller.

(2)	An Ancillary Service Trade that is reported to ERCOT by 1430 in the Day-Ahead changes the Ancillary Service Supply Responsibility of the buyer and seller in the DRUC process. An Ancillary Service Trade that is reported to ERCOT after 1430 in the Day-Ahead changes the Ancillary Service Supply Responsibility of the buyer and seller in any applicable HRUC process, the deadline for which is after the trade is submitted.

(3)	As soon as practicable, ERCOT shall notify each QSE through the Messaging System of any of its Ancillary Service Trades that are invalid Ancillary Service Trades.  The QSE may correct and resubmit any invalid Ancillary Service Trade, but the reporting time of the trade is determined by when the validated Ancillary Service Trade was submitted and not when the original invalid Ancillary Service Trade was submitted.

4.4.7.3.1	Ancillary Service Trade Criteria

(1)	Each Ancillary Service Trade must be reported by a QSE and must include the following information:

(a)	The buying QSE;

(b)	The selling QSE;

(c)	The type of Ancillary Service;

(d)	The quantity in MW; and

(e)	The first and last hours of the trade.

(2)	An Ancillary Service Trade must be confirmed by both the buyer and seller to be considered valid and to be used in an ERCOT process.

4.4.7.3.2	Ancillary Service Trade Validation

(1)	A valid Ancillary Service Trade is an Ancillary Service Trade that ERCOT has determined meets the criteria listed in Section 4.4.7.3.1, Ancillary Service Trade Criteria.  Only one confirmed Ancillary Service Trade is allowed for the same buying and selling QSEs for each type of Ancillary Service for each hour.

(2)	When an Ancillary Service Trade is reported to ERCOT, ERCOT shall notify both the buying and selling QSEs by using the Messaging System if available and the MIS Certified Area.

(3)	ERCOT shall continuously validate Ancillary Service Trades and continuously display on the MIS Certified Area information that allows any QSE named in an Ancillary Service Trade to view its confirmed and unconfirmed Ancillary Service Trades.

(4)	The QSE that first reports the Ancillary Service Trade to ERCOT is deemed to have confirmed the Ancillary Service Trade unless it subsequently affirmatively rejects it.  The QSE that first reports an Ancillary Service Trade may reject, edit, or delete an Ancillary Service Trade that its counterpart has not confirmed.  The counterpart is deemed to have confirmed the Ancillary Service Trade when it submits an identical Ancillary Service Trade.  After both the buyer and seller have confirmed an Ancillary Service Trade, either party may reject it at any time, but the rejection is effective only for any ERCOT process for which the deadline for reporting Ancillary Service Trades has not yet passed.

4.4.7.4	Ancillary Service Supply Responsibility

(1)	A QSE’s Ancillary Service Supply Responsibility is the net amount of Ancillary Service capacity that the QSE is obligated to deliver to ERCOT, by hour and service type, from Resources represented by the QSE. The Ancillary Service Supply Responsibility is the difference in MW, by hour and service type, between the amounts specified in (a) and (b) defined as follows:

(a)	The sum of:

(i)	the QSE’s Self-Arranged Ancillary Service Quantity;

(ii)	the total (in MW) of Ancillary Service Trades for which the QSE is the seller; plus

(iii)	Awards to the QSE of Ancillary Service Offers in the DAM; and

(b)	The total Ancillary Service Trades for which the QSE is the buyer.

(2)	A QSE may only use a Resource to provide its Ancillary Service during non-RUC-Committed Intervals.

(3)	By 1430 in the Day-Ahead, the QSE must notify ERCOT, in the QSE’s COP, which Resources represented by the QSE will provide the Ancillary Service capacity necessary to meet the QSE’s Ancillary Service Supply Responsibility, specified by Resource, hour, and service type. The DAM Ancillary Service awards are Resource-specific; the QSE must include those DAM awards in its COP, and the QSE may not change that Resource-specific DAM award information until after 1600 under the conditions set out in Section 3.9, Current Operating Plan (COP).

(4)	Section 6.4.8.1.3, Replacement of Ancillary Service Due to Failure to Provide, specifies what happens if the QSE fails on its Ancillary Service Supply Responsibility.

4.4.8	RMR Offers

ERCOT shall decide, in its sole discretion, when to make an RMR Unit available for commitment in DRUC, HRUC, or DAM, considering relevant factors such as whether it is likely to be needed in Real-Time for reliability reasons, whether SCED will solve operating constraints, contractual constraints on the Resource, and any other adverse effects on the RMR Unit that may occur as the result of the dispatch of the RMR Resource.

(a)	By 1000 in the Day-Ahead, ERCOT shall submit, in ERCOT’s sole discretion, Three-Part Supply Offers based on RMR Agreement rates and any other relevant information as provided under contract on behalf of RMR Units for any RMR Units to be considered, in the DAM, DRUC, or HRUC.

(b)	ERCOT may submit Energy Offer Curves based on RMR Agreement rates and any other relevant information as provided under contract on behalf of RMR Units committed in the DAM, DRUC, or HRUC.

4.4.9	Energy Offers and Bids

4.4.9.1	Three-Part Supply Offers

(1)	A Three-Part Supply Offer consists of a Startup Offer, a Minimum-Energy Offer, and an Energy Offer Curve.  ERCOT must validate each Startup Offer, Minimum-Energy Offer, and Energy Offer Curve before it can be used in any ERCOT process.

(2)	The DAM uses all three parts of the Three-Part Supply Offer and also uses Energy Offer Curves submitted without a Startup Offer and without a Minimum-Energy Offer.  The RUC only uses the Startup Offer and the Minimum-Energy Offer components for determining RUC commitments, but the Energy Offer Curve may be used in settlement to claw back some or all of a RUC-committed Resource’s energy payments.  The Energy Offer Curve may also be used by SCED in Real-Time Operations.

(3)	A QSE may submit an Energy Offer Curve without also submitting a Startup Offer and a Minimum-Energy Offer for the DAM and during the Adjustment Period, but only Three-Part Supply Offers are used in the RUC process.  A QSE that submits an Energy Offer Curve without also submitting a Startup Offer and a Minimum-Energy Offer is considered not to be offering the Resource into the RUC, but that does not prevent the Resource from being committed in the RUC process like any other Resource that does not submit an offer in the RUC.

(4)	For any hours in which the Resource is not RUC-committed, ERCOT shall consider all Three-Part Supply Offers in the RUC process until:

(a) 	The QSE withdraws the offer; or

(b) 	The offer expires by its terms.

4.4.9.2	Startup Offer and Minimum-Energy Offer

The Startup Offer component represents all costs incurred by a Generation Resource in starting up and reaching breaker close, as indicated by a telemetered Resource status of On-Line. The Minimum-Energy Offer component represents a proxy for the costs incurred by a Resource in producing energy up to and including the Resource’s LSL after breaker close, as indicated by a telemetered Resource status of On-Line.

4.4.9.2.1	Startup Offer and Minimum-Energy Offer Criteria

(1)	Each Startup Offer and Minimum-Energy Offer must be reported by a QSE and must include the following information:

(a)	The selling QSE;

(b)	The Resource represented by the QSE from which the offer would be supplied;

(c)	The Resource’s hot, intermediate, and cold Startup Offer in dollars;

(d)	The Resource’s Minimum-Energy Offer in dollars per MWh;

(e)	The first and last hour of the Startup and Minimum-Energy Offers

(f)	The expiration time and date of the offer;

(g)	Percentage of FIP to the extent that the startup and minimum energy will be supplied by gas to determine the offer cap; and

(h)	Percentage of FOP to the extent that the startup and minimum energy will be supplied by oil to determine the offer cap.

(2)	Valid Startup Offers and Minimum-Energy Offers (which must be part of a Three-Part Supply Offer) must be received before 1000 for the effective DAM and DRUC.

(3)	A QSE may update and submit a Three-Part Supply Offer for a Resource during the Adjustment Period for any hours in which the Resource is not RUC-committed before the offer is updated or submitted.

(4)	The Resource’s Startup Offer must be equal to or less than the Resource Category Generic Startup Cost for that type of Resource listed in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, unless ERCOT has approved verifiable Resource-specific startup costs for that Resource, under Section 4.4.9.2.4, Verifiable Startup Offer and Minimum-Energy Offer Caps, in which case the Resource’s Startup Offer must be equal to or less than those approved verifiable Resource-specific startup costs.

(5)	The Resource’s Minimum-Energy Offer must be equal to or less than the Resource Category Generic Minimum-Energy Cost for that type of Resource listed in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, unless ERCOT has approved verifiable Resource-specific minimum-energy costs for that Resource, under Section 4.4.9.2.4, Verifiable Startup Offer and Minimum-Energy Offer Caps, in which case the Resource’s Minimum-Energy Offer must be equal to or less than those approved verifiable Resource-specific minimum-energy costs.

4.4.9.2.2	Startup Offer and Minimum-Energy Offer Validation

(1)	A valid Startup Offer and Minimum-Energy Offer is an offer that ERCOT has determined meets the criteria listed in Section 4.4.9.2.1, Startup Offer and Minimum-Energy Offer Criteria, and that are part of a Three-Part Supply Offer for which the Energy Offer Curve has also been validated.

(2)	ERCOT shall continuously display on the MIS Certified Area information that allows any QSE submitting a Startup Offer and Minimum-Energy Offer to view its valid Startup Offers and Minimum-Energy Offers.

(3)	ERCOT shall notify the QSE submitting a Startup Offer and Minimum-Energy Offer (which must be part of a Three-Part Supply Offer) if the offer was rejected or was considered invalid for any reason.  The QSE may then resubmit the offer within the appropriate market timeline.

(4)	Where a Split Generation Resource has submitted a Startup Offer and Minimum-Energy Offer, ERCOT shall validate the offers in accordance with Section 3.8, Special Considerations for Split Generation Meters.

4.4.9.2.3	Startup Offer and Minimum-Energy Offer Generic Caps

(1)	The Resource Category Startup Offer Generic Cap, by applicable Resource category, is determined by the following O&M costs by Resource category:

| Resource Category | O&M Costs ($) |
| --- | --- |
| Nuclear, Coal, Lignite, Hydro, Renewable | 7,200 |
| Combined Cycle greater than 90 MW with 5+ HRS off line | 6,810 |
| Combined Cycle greater than 90 MW with less than 5 HRS off line | 5,310 |
| Combined Cycle less than or equal to 90 MW with 5+ HRS off line | 6,810 |
| Combined Cycle less than or equal to 90 MW with less than 5 HRS off line | 5,310 |
| Gas steam supercritical boiler | 4,800 |
| Gas steam reheat boiler | 3,000 |
| Gas steam non-reheat or boiler w/o air-preheater | 2,310 |
| Simple cycle greater than 90 MW | 5,000 |
| Simple cycle less than or equal to 90 MW | 2,300 |
| Diesel | 1 |
| RMR Resource | Not Applicable |

(2)	The Resource Category Minimum-Energy Generic Cap is the cost per MWh of energy for a Resource in producing energy up to and including the Resource’s LSL after breaker close, as indicated by a telemetered Resource status of On-Line, according to the following:

(a)	Hydro = $10.00/MWh;

- (b)	Coal and lignite = $18.00/MWh;

- (c)	Combined cycle greater than 90 MW = 10 MMBtu/MWh * FIP or FOP, as specified in Minimum-Energy Offer;

- (d)	Combined cycle less than or equal to 90 MW = 10 MMBtu/MWh * FIP or FOP, as specified in Minimum-Energy Offer;

- (e)	Gas steam supercritical boiler = 16.5 MMBtu/MWh * FIP or FOP, as specified in Minimum-Energy Offer;

- (f)	Gas steam reheat boiler = 17.0 MMBtu/MWh * FIP or FOP, as specified in Minimum-Energy Offer;

- (g)	Gas steam non-reheat or boiler without air-preheater = 19.0 MMBtu/MWh * FIP or FOP, as specified in Minimum-Energy Offer;

- (h)	Simple cycle greater than 90 MW = 15.0 MMBtu/MWh * FIP or FOP, as specified in Minimum-Energy Offer;

- (i)	Simple cycle less than or equal to 90 MW = 15.0 MMBtu/MWh * FIP or FOP, as specified in Minimum-Energy Offer;

- (j)	Diesel = 16.0 MMBtu/MWh * FOP;

- (k)	RMR Resource = RMR contract estimated fuel cost using its contract I/O curve at its LSL times FIP;

- (l)	Nuclear = Not Applicable; and

- (m)	Renewable = $0.

- (3)	The FIP and FOP used to calculate the Resource Category Minimum-Energy Generic Cap shall be the FIP or FOP for the Operating Day.  In the event the Resource Category Minimum-Energy Generic Cap must be calculated before the FIP or FOP is available for the particular Operating Day, the FIP and FOP for the most recent preceding Operating Day shall be used.  Once the FIP and FOP are available for a particular Operating Day, those values shall be used in the calculations.  If the percentage fuel mix is not specified for Resource categories having the option to specify the fuel mix, then the minimum of FIP or FOP shall be used.

(4)	Items (2)(c) and (2)(d) are determined by capacity of largest simple-cycle combustion turbine in the train.

4.4.9.2.4	Verifiable Startup Offer and Minimum-Energy Offer Caps

Once verifiable Resource-specific startup costs and minimum-energy costs are established and approved by ERCOT in accordance with Section 5.6.1, Verifiable Costs, then they are used in place of generic costs as described in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps. A QSE may file verifiable unit-specific costs for a Resource at any time, but it is not required to file those costs only because of a DAM commitment. The most recent approved verifiable costs on file must be used going forward.

4.4.9.3	Energy Offer Curve

(1)	The “Energy Offer Curve” represents the QSE’s willingness to sell energy at or above a certain price and at a certain quantity in the DAM or its willingness to be dispatched by SCED in Real-Time Operations.

(2)	A QSE may submit Resource-specific Energy Offer Curves to ERCOT.

(3)	Energy Offer Curves remain active for the offered period until either:

(a)	Selected by ERCOT; or

(b)	Automatically inactivated by the software at the offer expiration time selected by the QSE.

(4)	For any hour that is not a RUC-Committed Interval or a DAM-Committed Interval for a Resource, the QSE for that Resource may submit or change Energy Offer Curves in the Adjustment Period and a QSE may withdraw an Energy Offer Curve if:

(a)	An Output Schedule is submitted for all intervals for which an Energy Offer Curve is withdrawn, or

(b) 	The Resource is forced Off-Line and notifies ERCOT of the Forced Outage by changing the Resource Status appropriately and updating its COP.

(5)	For any hour that is a RUC-Committed Interval or a DAM-Committed Interval for a Resource, a QSE for that Resource may not change an Energy Offer Curve, except as specified in (a) and (b) below:

(a)	A QSE may change the Energy Offer Curve if the Resource is required, due to external fuel curtailments, to change fuel type or source during the Adjustment Period.  ERCOT shall develop reasonable procedures for QSEs to report and document such fuel curtailments.

(b)	A QSE may change the Energy Offer Curve if the Resource suffers a partial Forced Outage by truncating the Energy Offer Curve at the Resource’s HSL as modified by the partial Forced Outage.

(6)	If a valid Energy Offer Curve or an Output Schedule does not exist for a Resource that has a status of On-Line at the end of the Adjustment Period, then ERCOT shall notify the QSE and set the Output Schedule equal to the then current telemetered output of the Resource until an Output Schedule or Energy Offer Curve is submitted in a subsequent Adjustment Period.

4.4.9.3.1	Energy Offer Curve Criteria

(1)	Each Energy Offer Curve must be reported by a QSE and must include the following information:

(a)	The selling QSE;

(b)	The Resource represented by the QSE from which the offer would be supplied;

(c)	A monotonically increasing offer curve for both price (in $/MWh) and quantity (in MW) with no more than ten price/quantity pairs;

(d)	The first and last hour of the Offer;

(e)	The expiration time and date of the offer;

(f)	List of Ancillary Service Offers from the same Resource;

(g)	Inclusive or exclusive designation relative to other DAM offers; and

(h)	Percentage of FIP and FOP for generation above LSL.

(2)	An Energy Offer Curve must be within the range of -$250.00 per MWh and the System-Wide Offer Cap in dollars per MWh.  The software systems must be able to provide ERCOT with the ability to enter Resource-specific Energy Offer Curve floors and caps.

(3)	The minimum amount per Resource for each Energy Offer Curve that may be offered is one MW.

4.4.9.3.2	Energy Offer Curve Validation

(1)	A valid Energy Offer Curve is an offer curve that ERCOT has determined meets the criteria listed in Section 4.4.9.3.1, Energy Offer Curve Criteria, and the Energy Offer Curve that is part of a Three-Part Supply Offer for which the Startup Offer and Minimum-Energy Offer has also been validated.

(2)	ERCOT shall notify the QSE submitting an Energy Offer Curve by the Messaging System if the offer was rejected or was considered invalid for any reason.  The QSE may then resubmit the offer within the appropriate market timeline.

(3)	ERCOT shall continuously validate Energy Offer Curves and continuously display on the MIS Certified Area information that allows any QSE to view its valid Energy Offer Curves.

4.4.9.3.3	Energy Offer Curve Caps for Make-Whole Calculation Purposes

(1)	The following Energy Offer Curve Caps must be used for the purpose of Make-Whole Settlements:

- (a)	Nuclear = $15.00/MWh;

- (b)	Coal and Lignite = $18.00/MWh;

- (c)	Combined Cycle greater than 90 MW = FIP * 9 MMBtu/MWh;

- (d)	Combined Cycle less than or equal to 90 MW = FIP * 10 MMBtu/MWh;

- (e)	Gas -Steam Supercritical Boiler = FIP * 10.5 MMBtu/MWh;

- (f)	Gas Steam Reheat Boiler = FIP * 11.5 MMBtu/MWh;

- (g)	Gas Steam Non-reheat or boiler without air-preheater = FIP * 14.5	MMBtu/MWh;

- (h)	Simple Cycle greater than 90 MW = FIP * 14 MMBtu/MWh;

- (i)	Simple Cycle less than or equal to 90 MW = FIP * 15 MMBtu/MWh;

- (j)	Diesel = FIP * 16 MMBtu/MWh;

- (k)	Hydro = $10.00/MWh;

- (l)	Other Renewable = $0/MWh; and

- (m)	RMR Resource = RMR contract price Energy Offer Curve.

(2)	Items in (d) and (e) are determined by capacity of largest simple-cycle combustion turbine in the train selected.

4.4.9.4	Mitigated Offer Cap and Mitigated Offer Floor

4.4.9.4.1	Mitigated Offer Cap

Energy Offer Curves may be subject to mitigation in Real-Time Operations under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Cap.  The “Mitigated Offer Cap” is:

(a)	For a Generation Resource that commences commercial operation after January 1, 2004, ERCOT shall construct an incremental mitigated offer cap curve (Section 6.5.7.3) such that each point on the Mitigated Offer Cap curve (Cap vs. output level) is the greater of:

(i)	14.5 MMBtu/MWh times Fuel Index Price (FIP) or Fuel Oil Price (FOP), as specified in the Energy Offer Curve; or

(ii)	the Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by the FIP or FOP specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in (c).

(b)	For all other Generation Resources, each point on the Mitigated Offer Cap curve (Cap vs. output level) is the greater of:

(i)	10.5 MMBtu/MWh times FIP or FOP, as specified in the Energy Offer Curve; or

(ii)	the Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by the FIP or FOP specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in (c).

(c)	The multipliers for Section 4.4.9.4.1, Mitigated Offer Cap, paragraphs (a)(ii) and (b)(ii) are as follows:

(i)	1.10 for Resources running at a ≥ 50% capacity factor for the previous 12 months;

(ii)	1.15 for Resources running at a ≥ 30 and < 50% capacity factor for the previous 12 months;

(iii)	1.20 for Resources running at a ≥ 20 and < 30% capacity factor for the previous 12 months;

(iv)	1.25 for Resources running at a ≥ 10 and < 20% capacity factor for the previous 12 months;

(v)	1.30 for Resources running at a ≥ 5 and < 10% capacity factor for the previous 12 months;

(vi)	1.40 for Resources running at a ≥ 1 and < 5% capacity factor for the previous 12 months; and

(vii)	1.50 for Resources running at a less than 1% capacity factor for the previous 12 months.

(d)	The previous 12 months’ capacity factor must be updated by ERCOT by the 20th day of each month using the most recent data for use in the next month. ERCOT shall post to the MIS Secure Area the capacity factor for each Resource before the start of the effective month.

(e)	The process for developing the mitigate offer cap in (a) and (b) above must be described by ERCOT in a procedure approved by the appropriate TAC Subcommittee, and posted to the MIS Secure Area within one Business Day after initial approval, and after each approved change.

4.4.9.4.2	Mitigated Offer Floor

Energy Offer Curves may be subject to mitigation in Real-Time Market under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Floor.  The “Mitigated Offer Floor” is:

| Resource Category | Mitigated Offer Floor |
| --- | --- |
| Nuclear and Hydro | -$250/MWh |
| Coal and Lignite | -$20/MWh |
| Combined Cycle | 1 MMBtu/MWh * FIP |
| Gas/Oil Steam and Combustion Turbine | 6 MMBtu/MWh * FIP or FOP, as specified in the Energy Offer Curve |
| QF | -$ 50/MWh |
| Wind | -$100/MWh |
| Other Renewables | -$ 50/MWh |

4.4.9.5	DAM Energy-Only Offer Curves

(1)	A QSE must submit any DAM Energy-Only Offer Curves by 1000 for the effective DAM.

(2)	The DAM Energy-Only Offer Curve represents the QSE’s willingness to sell energy at or above a certain price and at a certain quantity at a specific Settlement Point in the DAM. A DAM Energy-Only Offer Curve may be offered only in the DAM.

(3)	DAM Energy-Only Offer Curves are not Resource-specific.

4.4.9.5.1	DAM Energy-Only Offer Curve Criteria

(1)	Each DAM Energy-Only Offer Curve must be reported by a QSE and must include the following information:

(a)	The selling QSE;

(b)	The Settlement Point;

(c)	The fixed quantity block, variable quantity block, or curve indicator for the offer;

(i)	If a fixed quantity block, the single price (in $/MWh) and single quantity (in MW) for all hours offered in that block;

(ii)	If a variable quantity block, the single price (in $/MWh) and single “up to” quantity (in MW) contingent on the purchase of all hours offered in that block; and

(iii)	If a curve, a monotonically increasing energy offer curve for both price (in $/MWh) and quantity (in MW) with no more than ten price/quantity pairs;

(d)	The first and last hour of the offer; and

(e)	The expiration time and date of the offer.

(2)	A DAM Energy-Only Offer Curve must be within the range of -$250.00 per MWh and the System-Wide Offer Cap in dollars per MWh.

(3)	The minimum amount for each DAM Energy-Only Offer Curve that may be offered is one MW.

4.4.9.5.2	DAM Energy-Only Offer Validation

(1)	A valid DAM Energy-Only Offer Curve is an offer that ERCOT has determined meets the criteria listed in Section 4.4.9.5.1, DAM Energy-Only Offer Curve Criteria.

(2)	ERCOT shall notify the QSE submitting a DAM Energy-Only Offer Curve by the Messaging System if the offer was rejected or was considered invalid for any reason.  The QSE may then resubmit the offer within the appropriate market timeline.

(3)	ERCOT shall continuously validate DAM Energy-Only Offers and continuously display on the MIS Certified Area information that allows any QSE to view its valid DAM Energy-Only Offers.

4.4.9.6	DAM Energy Bids

(1)	A QSE must submit any DAM Energy Bids by 1000 for the effective DAM.

(2)	A DAM Energy Bid represents the QSE’s willingness to buy energy at or below a certain price and at a certain quantity at a specific Settlement Point in the DAM. A DAM Energy Bid may be made only in the DAM.

4.4.9.6.1	DAM Energy Bid Criteria

(1)	Each DAM Energy Bid must be reported by a QSE and must include the following information:

(a)	The buying QSE;

(b)	The Settlement Point;

(c)	Fixed quantity block, variable quantity block, or curve indicator for the bid;

(i)	If a fixed quantity block, the single price (in $/MWh) and single quantity (in MW) for all hours bid in that block;

(ii)	If a variable quantity block, the single price (in $/MWh) and single “up to” quantity (in MW) contingent on the purchase of all hours bid in that block; and

(iii)	If a curve, a monotonically decreasing energy bid curve for both price (in $/MWh) and quantity (in MW) with no more than 10 price/quantity pairs.

(d)	The first and last hour of the bid; and

(e)	The expiration time and date of the bid.

(2)	The minimum amount for each DAM Energy Bid that may be bid is one MW.

4.4.9.6.2	DAM Energy Bid Validation

(1)	A valid DAM Energy Bid is a bid that ERCOT has determined meets the criteria listed in Section 4.4.9.6.1, DAM Energy Bid Criteria.

(2)	ERCOT shall notify the QSE submitting a DAM Energy Bid by the Messaging System if the bid was rejected or was considered invalid for any reason.  The QSE may then resubmit the bid within the appropriate market timeline.

(3)	ERCOT shall continuously validate DAM Energy Bids and continuously display on the MIS Certified Area information that allows any QSE to view its valid DAM Energy Bids.

4.4.10	Credit Requirement for DAM Bids and Offers

(1)	Each QSE’s ability to bid and offer in the DAM is subject to credit exposure from the QSE’s bids and offers being within the credit limit for DAM participation established for the entire Counter-Party of which the QSE is part, as specified in item (1) of Section 16.11.4.6.2, Credit Requirements for DAM Participation, and taking into account the credit exposure of accepted DAM bid and offers of the Counter-Party’s other QSEs.

(2)	DAM bids and offers of all QSEs of the Counter-Party are accepted in the order submitted while ensuring that the credit exposure from accepted bids and offers do not exceed the Counter-Party’s credit limit for DAM participation.

(3)  	ERCOT shall reject the QSE’s individual bids and offers whose credit exposure, as calculated in item (6) below, exceeds the Counter-Party’s credit limit for DAM participation as described in items (1) and (2) above,  and shall notify the QSE through the MIS Certified Area as soon as practicable.

(4) 	The QSE may revise and resubmit  such rejected bids and offers described in item (3) above, provided that the resubmitted bids and offers are valid and within the Counter-Party’s credit limit for DAM participation adjusted for all accepted DAM bids and offers of the Counter-Party’s QSE’s limit and that such resubmission occurs prior to 1000 of the Operating Day.

(5)	DAM shall use the Counter-Party’s credit limit for DAM participation provided on the most recent Business Day and adjusted for accepted bids and offers for markets cleared, until a new credit limit for DAM participation is available.

(6)	ERCOT shall calculate credit exposure for bids and offers in the DAM as follows:

(a)	For each DAM Energy Bid, the quantity of the bid multiplied by the bid price.

(b)	For each DAM Energy Offer, the product of the quantity of the offer times the 95th percentile of the hourly difference of Real-Time Settlement Point Price and Day-Ahead Settlement Point Price over the previous 30 days for the hour.

(c)	For DAM Energy Bids and Offers at the same Settlement Point for the same hour ERCOT shall calculate the credit exposure as the maximum of the credit exposure for the DAM Energy Bid as calculated in item (a) or the credit exposure for the DAM Energy Offer as calculated in item (b) above.

(d)	For PTP Obligation Bids, the sum of the quantity of bid multiplied by the bid price, if positive, plus 95th percentile of the hourly positive price difference between the source Real-Time Settlement Point Price minus the sink Real-Time Settlement Point Price over the previous 30 days for the hour.

(e)	For Ancillary Services not self-arranged, the product of the quantity of Ancillary Service not self-arranged times the 95th percentile of the hourly MCPC for that Ancillary Service over the previous 30 days for that hour.

4.4.11	System-Wide Offer Caps

(1)	The System-Wide Offer Cap (SWCAP) is as follows:

(a)	The low system-wide offer cap (LCAP) is set on a daily basis at the higher of:

(i)	$500 per MWh for energy and $500 per MW per hour for Ancillary Services; or

(ii)	Fifty times the Fuel Index Price (FIP) of the previous Operating Day, expressed in dollars per MWh for energy and dollars per MW per hour for Ancillary Services.

(b)	The high system-wide offer cap (HCAP) is $2,250 per MWh for energy and $2,250 per MW per hour for Ancillary Services.

(c)	Beginning two months after nodal implementation, the HCAP shall be $3,000 per MWh for energy and $3,000 per MW per hour for Ancillary Services.

(d)	At the beginning of each annual resource adequacy cycle, the SWCAP shall be set equal to the HCAP and maintained at this level as long as the peaker net margin (PNM) during an annual resource adequacy cycle is less than or equal to $175,000 per MW.  During an annual resource adequacy cycle, the SWCAP shall be as set forth above in items (b) and (c) above, unless the PNM has exceeded $175,000 per MW by the date specified.  If the PNM exceeds $175,000 per MW during an annual resource adequacy cycle, on the next Operating Day, the SWCAP shall be reset to the LCAP for the remainder of that annual resource adequacy cycle.

(2)	Any offers that exceed the current SWCAP shall be rejected by ERCOT.

4.4.11.1	Scarcity Pricing Mechanism

(1)	ERCOT shall operate the scarcity pricing mechanism (SPM) as follows:

(a)	The SPM operates on an annual resource adequacy cycle, starting on January 1 and ending on December 31 of each year.

(b)	For each day of the annual resource adequacy cycle, the peaking operating cost (POC) shall be ten times the FIP for the previous Operating Day.  The POC is calculated in dollars per megawatt-hour (MWh).

(c)	For the purpose of this Section, the Real-Time energy price (RTEP) shall be measured as the ERCOT Hub Average 345 kV Hub price.

(d)	For the current annual resource adequacy cycle, the PNM shall be calculated in dollars per megawatt (MW) on a cumulative basis for all past intervals in the annual resource adequacy cycle as follows:

∑((RTEP – POC) * (.25)) for each settlement interval where (RTEP – POC) >0

(e)	By the end of the next Business Day following the applicable Operating Day, ERCOT shall post the updated value of the PNM and the current SWCAP on the MIS Public Area.

- 4.5	DAM Execution and Results

4.5.1	DAM Clearing Process

(1)	At 1000 in the Day-Ahead, ERCOT shall start the DAM clearing process.

(2)	Prior to execution of the DAM, ERCOT shall complete a Day-Ahead Simultaneous Feasibility Test.  This test uses the Day-Ahead Updated Network Model topology and evaluates all CRRs for feasibility to determine hourly oversold quantities.

(3)	The purpose of the DAM is to economically and simultaneously clear offers and bids described in Section 4.4, Inputs into DAM and Other Trades.

(4)	The DAM uses a multi-hour mixed integer programming algorithm to maximize bid-based revenues minus the offer-based costs over the Operating Day, subject to security and other constraints, and ERCOT Ancillary Service procurement requirements.

- The bid-based revenues include revenues from DAM Energy Bids and PTP Obligation Bids.

(b)	The offer-based costs include costs from the Startup Offer, Minimum Energy Offer, and Energy Offer Curve of any Resource that submitted a Three-Part Supply Offer, DAM Energy-Only Offers, CRR Offers, and Ancillary Service Offers.

(c)	Security constraints specified to prevent DAM solutions that would overload the elements of the ERCOT Transmission Grid include the following:

(i)	Transmission constraints – Transfer limits on energy flows through the ERCOT Transmission Grid, e.g., thermal or stability limits. These limits must be satisfied by the intact network and for certain specified contingencies.

These constraints may represent:

(A)	Thermal constraints – protect transmission facilities against thermal overload.

(B)	Generic constraints – protect the ERCOT Transmission Grid against transient instability, dynamic stability or voltage collapse.

(C)	Power flow constraints – the energy balance at required Electrical Buses in the ERCOT Transmission Grid must be maintained.

(ii)	Resource constraints – the physical and security limits on Resources that submit Three-Part Supply Offers:

(A)	Resource output constraints – the LSL and HSL of each Resource, and

(B)	Resource operational constraints – includes minimum run time, minimum down time, and configuration constraints.

(iii)	Other constraints –

(A)	Linked offers –the DAM may not select any one part of that Resource capacity to provide more than one Ancillary Service or to provide both energy and an Ancillary Service in the same Operating Hour. The DAM may, however, select part of that Resource capacity to provide one Ancillary Service and another part of that capacity to provide a different Ancillary Service or energy in the same Operating Hour.

(B)	The sum of the awarded Ancillary Service capacities for each Resource must be within the Resource limits specified in COP and Section 3.18, Resource Limits in Providing Ancillary Service, and the Resource parameters as described in Section 3.7, Resource Parameters.

(C)	Block Ancillary Service Offers for a Load Resource– blocks will not be cleared unless the entire quantity block can be awarded.

(D)	Block CRR Offers and PTP Obligation Bids- blocks will not be cleared unless the entire time block can be awarded.

(d)	Ancillary Service needs for each Ancillary Service include the needs specified in the Ancillary Service Plan that are not part of the Self-Arranged Ancillary Service Quantity and that must be met from available DAM Ancillary Service Offers while co-optimizing with DAM Energy Offers. ERCOT may not buy more of one Ancillary Service in place of the quantity of a different service. See Section 4.5.2, Ancillary Service Insufficiency, for what happens if insufficient Ancillary Service Offers are received in the DAM.

(5)	ERCOT shall determine the appropriate Load distributions to allocate offers, bids, and source and sink of CRRs at a Load Zone across the Electrical Buses that are modeled with Load in that Load Zone. The default distribution is the State Estimator hourly distribution for the seven days before the Operating Day.  If ERCOT decides, in its sole discretion, to change this distribution for reasons such as anticipated weather events or holidays, ERCOT shall select a State Estimator distribution from a proxy day reasonably reflecting the anticipated distribution in the Operating Day.  ERCOT may also modify this distribution to account for predicted differences in network topology between the proxy day and Operating Day.  ERCOT shall develop a methodology, subject to TAC approval to describe the modification of the proxy day bus-load distribution for this purpose.

(6)	ERCOT shall allocate offers, bids, and source and sink of CRRs at a Hub using the distribution factors specified in the definition of that Hub in Section 3.5.2, Hub Definitions.

(7)	A Resource that has a Three-Part Supply Offer cleared in the DAM may be eligible for make whole payment of the Startup Offer and Minimum Energy Offer submitted by the QSE representing the Resource under Section 4.6, DAM Settlement.

(8)	The directional network element flows for PTP Options declared for settlement in Real-Time must be properly accounted for in determining available transmission network capacity in the DAM.  In the event the available transmission capability in the DAM cannot accommodate all PTP Options declared for settlement in Real-Time, any PTP Option declared for settlement in Real-Time that impacts overloaded directional network elements must be appropriately derated for DAM modeling purposes only, in proportion to that impact.  The derated MW of PTP Options declared for settlement in Real-Time will be settled in the DAM if their Minimum Reservation Prices are less than or equal to the DAM prices for corresponding PTP Options. Otherwise, the derated MW will be settled in Real-Time.

(9)	The DAM settlement is based on hourly MW awards and on Day-Ahead hourly Settlement Point Prices.  All PTP Options settled in the DAM are settled based on the Day-Ahead Settlement Point Prices.

(10)	The Day-Ahead Market Clearing Price for Capacity (MCPC) for each hour for each Ancillary Service is the Shadow Price for that Ancillary Service for the hour as determined by the DAM algorithm.

(11)	If the Day-Ahead MCPC cannot be calculated by ERCOT, the Day-Ahead MCPC for the particular Ancillary Service is equal to the Day-Ahead MCPC for that Ancillary Service in the same Settlement Interval of the preceding Operating Day.

(12)	If the Day-Ahead Settlement Point Prices cannot be calculated by ERCOT, all CRRs shall be settled based on Real-Time Prices.  Settlements for all CRRs shall be reflected on the RT Settlement Statement.

4.5.2	Ancillary Service Insufficiency

(1)	ERCOT shall determine if there is an insufficiency in Ancillary Service Offers before executing the DAM. If ERCOT receives insufficient Ancillary Service Offers in the DAM to procure one or more required Ancillary Service such that the Ancillary Service Plan is deficient and system security and reliability is threatened:

(a)	ERCOT shall declare an Ancillary Service insufficiency and issue an Alert under Section 6.5.9.3.3, Alert.

(b)	ERCOT shall request additional Ancillary Service Offers.

(i)	A QSE may resubmit an offer for an Ancillary Service that it submitted before the Alert for the same Ancillary Service, but the resubmitted offer must meet the following criteria to be considered a valid offer:

(A)	The offer quantity may not be less than the offer quantity submitted before the Alert, unless the  portion of the offer not resubmitted was priced higher than the portion of the offer that is being resubmitted; and

(B)	For the amount of the offer quantity that is not more than the offer quantity submitted before the Alert, the offer must be priced equal to or less than the price of the offer submitted before the Alert.

(ii)	For any amount of the offer that is greater in quantity than the QSE’s offer that was not submitted before the Alert, the incremental amount of the offer may be submitted at a price subject to the offer cap.

(c)	ERCOT shall not begin executing the DAM sooner than 30 minutes after issuing the Alert.  If the additional Ancillary Service Offers are still insufficient to supply the Ancillary Service required in the Day-Ahead Ancillary Service Plan then ERCOT shall run the DAM by reducing the Ancillary Service Plan quantities only for purposes of the DAM by the amount of insufficiency.

(d)	When ERCOT must reduce the Ancillary Service Plan for purposes of the DAM due to insufficient Ancillary Service Offers, ERCOT shall preserve the Ancillary Service Plan in the DAM in the following order of priority:

(i)	Reg-Up;

(ii)	Reg-Down;

(iii)	RRS; and

(iv)	Non-Spin.

(2)	ERCOT shall procure the difference in capacity between the Day-Ahead Ancillary Service Plan and the DAM-reduced Ancillary Service Plan amounts using the DRUC from Resources that are qualified to provide the needed Ancillary Service.

4.5.3	Communicating DAM Results

(1)	As soon as practicable, but no later than 1330 in the Day-Ahead, ERCOT shall notify the parties to each cleared DAM transaction (e.g., the buyer and the seller) of the results of the DAM as follows:

(a)	Awarded Ancillary Service Offers, specifying Resource, MW, Ancillary Service Type, and price, for each hour of the awarded offer;

(b)	Awarded energy offers from Three-Part Supply Offers and from DAM Energy-Only Offers, specifying Resource (except for DAM Energy-Only Offers), MWh, Settlement Point, and Settlement Point Price, for each hour of the awarded offer;

(c)	Awarded DAM Energy Bids, specifying MWh, Settlement Point, and Settlement Point Price for each hour of the awarded bid;

(d)	Awarded CRR Offers (PTP Options and PTP Options with Refund), specifying CRR identifier(s), number of CRRs in MW, source and sink Settlement Points, and price, for each Settlement Interval of the awarded offer; and

(e)	Awarded PTP Obligation Bids, number of PTP Obligations in MW, source and sink Settlement Points, and price for each Settlement Interval of the awarded bid.

(2)	As soon as practicable, but no later than 1330, ERCOT shall post on the MIS Public Area the hourly:

(a)	Day-Ahead MCPC for each type of Ancillary Service for each hour of the Operating Day;(b)	Day-Ahead Settlement Point Prices for each Settlement Point for each hour of the Operating Day;

(c)	Day-Ahead hourly LMPs for each Electrical Bus for each hour of the Operating Day;

(d)	Shadow Prices for every binding constraint for each hour of the Operating Day;

(e)	Quantity of total Ancillary Service Offers received in the DAM, in MW by Ancillary Service type for each hour of the Operating Day;

(f)	Total quantity of energy (in MWh) bought in DAM at each Settlement Point for each hour of the Operating Day;

(g) 	Total quantity of energy (in MWh) sold in the DAM at each Settlement Point for each hour of the Operating Day; and

(h)	Aggregated Ancillary Service Offer Curve of all Ancillary Service Offers for each type of Ancillary Service for each hour of the Operating Day.

(3)	ERCOT shall monitor Day-Ahead MCPCs and Day-Ahead hourly LMPs for errors and shall “flag” for further review questionable prices before posting and make notations in the posting if there are conditions that cause the price to be questionable.

(4)	All DAM LMPs, MCPCs, and Settlement Point Prices are final at 1000 of the next Business Day after the Operating Day.  After DAM LMPs, MCPCs, and Settlement Point Prices are final, they cannot be changed unless the Board finds that the DAM LMPs, MCPCs, or Settlement Point Prices are significantly affected by a software or data error.

- 4.6	DAM Settlement

4.6.1	Day-Ahead Settlement Point Prices

4.6.1.1	Day-Ahead Settlement Point Prices for Resource Nodes

The Day-Ahead Settlement Point Price (DASPP) for a Resource Node Settlement Point for an hour is the Locational Marginal Price at that Resource Node for that hour as calculated in the DAM process.

4.6.1.2	Day-Ahead Settlement Point Prices for Load Zones

The DASPP for a Load Zone Settlement Point for an hour is calculated as follows:

DASPP	=	(DADF b * DALMP b)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP | $/MWh | Day-Ahead Settlement Point PriceThe DAM SPP at the Settlement Point for the hour. |
| DALMP b | $/MWh | Day-Ahead Locational Marginal Price per busThe DAM LMP at Electrical Bus b for the hour. |
| DADF b | none | Day-Ahead Distribution Factor per busThe Load distribution factor, as described in Section 4.5.1, DAM Clearing Process, for Electrical Bus b in the Load Zone for the hour. |
| b | none | An Electrical Bus that is assigned to the Load Zone. |

4.6.1.3	Day-Ahead Settlement Point Prices for Hubs

The DASPP for a Settlement Point at a Hub is determined according to the methodology included in the definition of that Hub in Section 3.5, Hubs.

4.6.2	Day-Ahead Energy and Make-Whole Settlement

4.6.2.1	Day-Ahead Energy Payment

(1)	The Day-Ahead Energy Payment is made for all cleared offers (excluding offers submitted for the RMR Units) to sell energy in the DAM, whether through Three-Part Supply Offers or DAM Energy-Only Offer Curves.  The payment to each QSE for each Settlement Point for a given hour of the Operating Day is calculated as follows:

DAESAMT q, p	=	(-1) * DASPP p * DAES q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAESAMT q, p | $ | Day-Ahead Energy Sale Amount per QSE per Settlement PointThe payment to QSE q for the cleared energy offers at Settlement Point p for the hour. |
| DASPP p | $/MWh | Day-Ahead Settlement Point Price per Settlement PointThe DAM SPP at Settlement Point p for the hour. |
| DAES q, p | MW | Day-Ahead Energy Sale per QSE per Settlement PointThe total amount of energy represented by QSE q’s cleared Three-Part Supply Offers in the DAM and cleared DAM Energy-Only Offer Curves at Settlement Point p, excluding the offers submitted for RMR Units at the same Settlement Point, for the hour. |
| q | none | A QSE. |
| p | none | A Settlement Point. |

(2)	The total of the Day-Ahead Energy Payments to each QSE for the hour is calculated as follows:

DAESAMTQSETOT q	=	ΣpDAESAMT q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAESAMTQSETOT q | $ | Day-Ahead Energy Sale Amount QSE Total per QSEThe total of the payments to QSE q for its cleared energy offers at all Settlement Points for the hour. |
| DAESAMT q, p | $ | Day-Ahead Energy Sale Amount per QSE per Settlement PointThe payment to QSE q for the cleared energy offers at Settlement Point p for the hour. |
| q | none | A QSE. |
| p | none | A Settlement Point. |

4.6.2.2	Day-Ahead Energy Charge

(1)	The Day-Ahead Energy Charge is made for all cleared DAM Energy Bids.  This charge to each QSE for each Settlement Point for a given hour of the Operating Day is calculated as follows:

DAEPAMT q, p 	=	DASPP p * DAEP q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAEPAMT q, p | $ | Day-Ahead Energy Charge per QSE per Settlement PointThe charge to QSE q for all its cleared DAM Energy Bids at Settlement Point p for the hour. |
| DASPP p | $/MWh | Day-Ahead Settlement Point Price per Settlement PointThe DAM SPP at Settlement Point p for the hour. |
| DAEP q, p | MW | Day-Ahead Energy Purchase per QSE per Settlement PointThe total amount of energy represented by QSE q’s cleared DAM Energy Bids at Settlement Point p for the hour. |
| q | none | A QSE. |
| p | none | A Settlement Point. |

(2)	The total of the Day-Ahead Energy Charges to each QSE for the hour is calculated as follows:

DAEPAMTQSETOT q	=	ΣpDAEPAMT q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAEPAMTQSETOT q | $ | Day-Ahead Energy Purchase Amount QSE Total per QSEThe total of the charges to QSE q for its cleared DAM Energy Bids at all Settlement Points for the hour. |
| DAEPAMT q, p | $ | Day-Ahead Energy Purchase Amount per QSE per Settlement PointThe charge to QSE q for its cleared DAM Energy Bids at Settlement Point p for the hour. |
| q | none | A QSE. |
| p | none | A Settlement Point. |

4.6.2.3	Day-Ahead Make-Whole Settlements

(1)	A QSE that has a Three-Part Supply Offer cleared in the DAM is eligible for a Day-Ahead Make-Whole Payment startup cost compensation, if, for the Resource associated with the offer:

(a)	The generator’s breakers were open, as indicated by a telemetered Resource status of Off-Line, for at least five minutes during the Adjustment Period for the beginning of the DAM commitment;

(b)	The generator’s breakers were closed, as indicated by a telemetered Resource status of On-Line, for at least one minute during the DAM commitment period; and

(c)	The breaker open-close sequence, as indicated by the On-Line/Off-Line sequence from the telemetered Resource status, for which the QSE is eligible for startup cost compensation in the DAM or RUC for the previous Operating Day does not qualify in meeting the criteria in items (a) and (b) above.

(d)	The breaker open-close sequence for which the QSE is eligible for startup cost compensation in an earlier DAM commitment period within the same Operating Day does not qualify in meeting the criteria in items (a) and (b) above.

(2)	A QSE that has a Three-Part Supply Offer cleared in the DAM is eligible for Day-Ahead Make-Whole Payment energy cost compensation in a DAM-committed Operating Hour, if, for the Resource associated with the offer the generator’s breakers were closed for at least one minute during the DAM-committed Operating Hour.

(3)	The Day-Ahead Make-Whole Payment guarantees the QSE that the total payment received from the DAM for a DAM-committed Resource is not less than the total cost calculated based on the Startup Offer, the Minimum Energy Offer, and the Energy Offer Curve capped by the Energy Offer Curve Cap defined under Section 4.4.9.3.3, Energy Offer Curve Caps for Make-Whole Calculation Purposes.

(4)	If a Generation Resource is eligible for startup or energy cost compensation in the Day-Ahead Make-Whole payment, then Ancillary Service revenue from the hours committed in the Day-Ahead Market will be included in its Make-Whole calculation for that Resource.

4.6.2.3.1	Day-Ahead Make-Whole Payment

(1)	ERCOT shall pay the QSE a Day-Ahead Make-Whole Payment for an eligible Resource, except that the Day-Ahead Make-Whole RMR Revenue amount is calculated but not paid for any RMR Unit, for each Operating Hour in a DAM-commitment period.

(2)	Any Ancillary Service Offer cleared for the same Operating Hour, QSE, and Generation Resource as a Three-Part Supply Offer cleared in the DAM shall be included in the calculation of the Day-Ahead Make-Whole Payment.

(3)	The Day-Ahead Make-Whole Payment to each QSE for each DAM-committed Generation Resource (excluding RMR units) is calculated as follows:

DAMWAMT q, p, r, h	=	(-1) * Max (0, DAMGCOST q, p, r + DAEREV q, p, r, h

+ DAASREV q, r, h) * DAESR q, p, r, h / (DAESR q, p, r, h)

Where:

DAMGCOST q, p, r	=	SUO q, p, r + (MEO q, p, r, h * LSL q, p, r, h)

+ (DAAIEC q, p, r, h * (DAESR q, p, r, h – LSL q, p, r, h))

DAEREV q, p, r, h	=	(-1) * DASPP p, h * DAESR q, p, r, h

DAASREV q, r, h =	((-1) * MCPCRUDAM, h * PCRUR r, q, DAM, h) +

((-1) * MCPCRDDAM, h  * PCRDR r, q,DAM, h) +

((-1) * MCPCRRDAM, h  * PCRRR r, q,DAM, h) +

((-1) * MCPCNSDAM, h  * PCNSR r, q,DAM, h)

(4)	The Day-Ahead Make-Whole RMR Revenue for each QSE for each DAM-committed RMR Unit is calculated as follows:

DAMWRMRREV q, p,r, h  = (-1) * Max (0, DAMGCOST q, p, r +DAEREV q, p, r, h

+ DAASREV q, r, h) * DAESR q, p, r, h / (DAESR q, p, r, h)

Where:

DAMGCOST q, p, r	=	SUO q, p, r + (MEO q, p, r, h * LSL q, p, r, h)

+ (DAAIEC q, p, r, h * (DAESR q, p, r, h – LSL q, p, r, h))

DAEREV q, p, r, h	=	(-1) * DASPP p, h * DAESR q, p, r, h

DAASREV q, r, h =	((-1) * MCPCRUDAM, h * PCRUR r, q, DAM, h) +

((-1) * MCPCRDDAM, h  * PCRDR r, q,DAM, h) +

((-1) * MCPCRRDAM, h  * PCRRR r, q,DAM, h) +

((-1) * MCPCNSDAM, h  * PCNSR r, q,DAM, h)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAMWAMT q, p, r, h | $ | Day-Ahead Make-Whole Payment per QSE per Settlement Point per Resource per hour  The payment to QSE q to make-whole the Startup Cost and Energy Cost of Resource r committed in the DAM at Resource Node p for the hour h. |
| DAMWRMRREV q, p, r, h | $ | Day-Ahead Make-Whole RMR Revenue per QSE per Settlement Point, per RMR Resource, per hour  The revenue calculated but not paid to QSE q to make-whole the Startup Cost and Energy Cost of the RMR Resource r committed in the DAM at Resource Node p for the hour h. |
| DAMGCOST q, p, r | $ | Day-Ahead Market Guaranteed Amount per QSE per Settlement Point per Resource The sum of the startup cost and the operating energy costs of the DAM-committed Resource r at Resource Node p represented by QSE q, for the DAM-commitment period. |
| DAEREV q, p, r, h | $ | Day-Ahead Energy Revenue per QSE per Settlement Point per Resource by hour  The revenue received in the DAM for Resource r at Resource Node p represented by QSE q, based on the DAM Settlement Point Price, for the hour h. |
| DAASREV q, r, h | $ | Day-Ahead Ancillary Service Revenue per QSE per Resource by hour The revenue received in the DAM for Resource r represented by QSE q, based on the Market Clearing Price of Capacity for each Ancillary Service in the DAM, for the hour h. |
| DASPP p, h | $/MWh | Day-Ahead Settlement Point Price by Settlement Point by hour The DAM Settlement Point Price at Resource Node p for the hour h. |
| DAESR q, p, r, h | MW | Day-Ahead Energy Sale from Resource per QSE by Settlement Point per Resource by hour  The amount of energy cleared through Three-Part Supply Offers in the DAM for Resource r at Resource Node p represented by QSE q for the hour h. |
| PCRUR r, q, DAM | MW | Procured Capacity for Reg-Up from Resource per Resource per QSE in DAM—The Reg-Up capacity quantity awarded to QSE q in the DAM for Resource r for the hour. |
| MCPCRU DAM | $/MW per hour | Market Clearing Price of Capacity for Reg-Up in DAM—The DAM Market Clearing Price of Capacity for Reg-Up for the hour. |
| PCRDR r, q, DAM | MW | Procured Capacity for Reg-Down from Resource per Resource per QSE in DAM—The Reg-Down capacity quantity awarded to QSE q in the DAM for Resource r for the hour. |
| MCPCRD DAM | $/MW per hour | Market Clearing Price of Capacity for Reg-Down in DAM—The DAM Market Clearing Price of Capacity for Reg-Down for the hour. |
| PCRRR r, q, DAM | MW | Procured Capacity for Responsive Reserve from Resource per Resource per QSE in DAM—The Responsive Reserve capacity quantity awarded to QSE q in the DAM for Resource r for the hour. |
| MCPCRR DAM | $/MW per hour | Market Clearing Price of Capacity for Responsive Reserve in DAM—The DAM Market Clearing Price of Capacity for Responsive Reserve for the hour. |
| PCNSR r, q, DAM | MW | Procured Capacity for Non-Spin from Resource per Resource per QSE in DAM—The Non-Spin capacity quantity awarded to QSE q in the DAM for Resource r for the hour. |
| MCPCNS DAM | $/MW per hour | Market Clearing Price of Capacity for Non-Spin in DAM—The DAM Market Clearing Price of Capacity for Non-Spin for the hour. |
| SUO q, p, r | $/start | Startup Offer per QSE per Settlement Point per Resource (The Startup Offer included in the Three-Part Supply Offer associated with Resource r at Resource Node p represented by QSE q, for the first hour of the DAM-commitment period.) |
| MEO q, p, r, h | $/MWh | Minimum-Energy Offer per QSE per Settlement Point per Resource per hour—The Minimum-Energy Offer included in the Three-Part Supply Offer associated with Resource r at Resource Node p represented by QSE q, for the hour h. |
| LSL q, p, r, h | MW | Low Sustained Limit per QSE per Settlement Point per Resource per hourThe Low Sustained Limit of Resource r at Resource Node p represented by QSE q, for the hour h. |
| DAAIEC q, p, r h | $/MWh | Day-Ahead Average Incremental Energy Cost per QSE per Settlement Point per Resource per hourThe average incremental energy cost, calculated according to the energy offer curve capped by the generic energy price, for the output levels between the DAESR and the LSL of Resource r at Resource Node p represented by QSE q, for the hour h. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A DAM-committed Generation Resource. |
| h | none | An hour in the DAM-commitment period. |

(5)	The calculation of the Day-Ahead Average Incremental Energy Cost for each Resource for each hour is illustrated with the picture below, where Pcap is the Energy Offer Curve Cap.  The method to calculate such cost is described in Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC).

$/

MWh

DASPP

P cap

P3

P2

P1

Q (P1) 	Q (P2)	   Q (P3)	Q (P cap)	        Q cleared	MW

[LSL]				     [DAESR]

Energy Offer Curve

The area under the capped Energy Offer Curve equals (DAAIEC * (DAESR – LSL))

(6)	The total of the Day-Ahead Make-Whole Payments to each QSE for non-RMR Generation Resources for a given hour is calculated as follows:

DAMWAMTQSETOT q	=	ΣpΣrDAMWAMT q, p, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAMWAMTQSETOT q | $ | Day-Ahead Make-Whole Payment QSE Total per QSEThe total of the Day-Ahead Make-Whole Payments to QSE q for the DAM-committed non-RMR Generation Resources represented by this QSE for the hour. |
| DAMWAMT q, p, r | $ | Day-Ahead Make-Whole Payment per QSE per Settlement Point per ResourceThe payment to QSE q to make-whole the Startup Cost and Energy Cost of Resource r committed in the DAM at Resource Node p for the hour. |
| q | none | A QSE. |
| p | none | A Settlement Point. |
| r | none | A DAM-committed non-RMR Generation Resource. |

Q (P1) 	Q (P2)	   Q (P3)	Q (P cap)	        Q cleared	MW

[LSL]				     [DAESR]

Energy Offer Curve

The area under the capped Energy Offer Curve equals (DAAIEC * (DAESR – LSL))

(7)	The total of the Day-Ahead Make-Whole RMR Revenue for each QSE for RMR Units for a given hour is calculated as follows:

DAMWRMRREVQSETOT q	=	ΣpΣrDAMWRMRREV q, p, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAMWRMRREVQSETOT q | $ | Day-Ahead Make-Whole RMR Revenue QSE Total per QSEThe total of the Day-Ahead Make-Whole Revenue calculated for QSE q for DAM-committed RMR Units represented by this QSE for the hour. |
| DAMWRMRREV q, p, r, | $ | Day-Ahead Make-Whole RMR Revenue per QSE per Settlement Point, per RMR Resource, per hour ( The revenue calculated but not paid to QSE q to make-whole the Startup Cost and Energy Cost of the RMR Resource r committed in the DAM at Resource Node p for the hour. |
| q | none | A QSE. |
| p | none | A Settlement Point. |
| r | none | A DAM-committed RMR Unit. |

4.6.2.3.2	Day-Ahead Make-Whole Charge

ERCOT shall charge a Day-Ahead Make-Whole Charge to each QSE that has one or more cleared DAM Energy Bids and/or PTP Obligation Bids.  The Day-Ahead Make-Whole Charge for an hour is that QSE’s prorata share of the total amount of Day-Ahead Make-Whole Payments and Day-Ahead Make-Whole RMR Revenue for that hour. The proration must be based on the ratio of the energy amount of the QSE’s cleared DAM Energy Bids and PTP Obligation Bids to the total energy amount of all QSEs’ cleared DAM Energy Bids and PTP Obligation Bids.  The Day-Ahead Make-Whole Charge to each QSE for a given hour is calculated as follows:

LADAMWAMT q =	(-1) * (DAMWAMTTOT + RMRDAMWREVTOT) * DAERS q

Where:

Day-Ahead Make-Whole Payment Total

DAMWAMTTOT	=	ΣqDAMWAMTQSETOT q

RMR Day-Ahead Make-Whole Revenue Total

RMRDAMWREVTOT	=	ΣqDAMWRMRREVQSETOT q

Day-Ahead Energy Purchase Ratio Share per QSE

DAERS q	=	DAE q / DAETOT

DAETOT	=	ΣqDAE q

DAE q	=	ΣpDAEP q, p + ΣjΣkRTOBL q, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LADAMWAMT q | $ | Day-Ahead Make-Whole ChargeThe allocated charge to QSE q to make whole all the eligible DAM-committed Resources for the hour. |
| DAMWAMTTOT | $ | Day-Ahead Make-Whole Payment TotalThe total of the Day-Ahead Make-Whole Payments to all QSEs for all DAM-committed non-RMR Resources for the hour. |
| DAMWAMTQSETOT q | $ | Day-Ahead Make-Whole Payment QSE Total per QSEThe total of the Day-Ahead Make-Whole Payments to QSE q for the DAM-committed non-RMR Generation Resources represented by this QSE for the hour. |
| RMRDAMWREVTOT | $ | RMR Day-Ahead Make-Whole Revenue TotalThe total of the RMR Day-Ahead Make-Whole Revenue for all DAM-committed RMR Units for the hour. |
| DAMWRMRREVQSETOT q | $ | Day-Ahead Make-Whole RMR Revenue QSE Total per QSEThe total of the Day-Ahead Make-Whole Revenue calculated for QSE q for DAM-committed RMR Units represented by this QSE for the hour. |
| DAERS q | none | Day-Ahead Energy Purchase Ratio Share per QSE The ratio of QSE q’s total amount of energy represented by its cleared DAM Energy Bids and PTP Obligation Bids, to the total amount of energy represented by all QSEs’ cleared DAM Energy Bids and PTP Obligation Bids, for the hour. |
| DAETOT | MW | Day-Ahead Energy Total—The total amount of energy represented by all cleared DAM Energy Bids and all cleared PTP Obligation Bids for the hour. |
| DAE q | MW | Day-Ahead Energy per QSE—QSE q’s total amount of energy, represented by its cleared DAM Energy Bids and PTP Obligation Bids, for the hour. |
| DAEP q, p | MW | Day-Ahead Energy Purchase per QSE per Settlement Point—The total amount of energy represented by QSE q’s cleared DAM Energy Bids at the Settlement Point p for the hour. |
| RTOBL q, (j, k) | MW | Real-Time Obligation per QSE per pair of source and sink—The total amount of energy represented by QSE q’s cleared PTP Obligation Bids with the source j and the sink k, for the hour. |
| q | none | A QSE. |
| r | none | An RMR Unit. |
| p | none | A Settlement Point. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

4.6.3	Settlement for PTP Obligations Bought in DAM

(1)	ERCOT shall pay or charge a QSE for a cleared PTP Obligation Bid the difference in the DAM Settlement Point Prices between the sink Settlement Point and the source Settlement Point.  The charge or payment to each QSE for a given Operating Hour of its cleared PTP Obligation Bids with each pair of source and sink Settlement Points is calculated as follows:

DARTOBLAMT q, (j, k)	=	DAOBLPR (j, k) * RTOBL q, (j, k)

Where:

DAOBLPR (j, k)	=	DASPP k – DASPP j

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DARTOBLAMT q, (j, k) | $ | Day-Ahead Real-Time Obligation Amount per QSE per pair of source and sinkThe charge or payment to QSE q for a PTP Obligation Bid cleared in the DAM with the source j and the sink k, for the hour. |
| DAOBLPR (j, k) | $/MW per hour | Day-Ahead Obligation Price per pair of source and sinkThe DAM clearing price of a PTP Obligation Bid with the source j and the sink k, for the hour. |
| DASPP j | $/MWh | Day-Ahead Settlement Point Price at sourceThe DAM Settlement Point Price at the source Settlement Point j for the hour. |
| DASPP k | $/MWh | Day-Ahead Settlement Point Price at sinkThe DAM Settlement Point Price at the sink Settlement Point k for the hour. |
| RTOBL q, (j, k) | MW | Real-Time Obligation per QSE per pair of source and sinkThe total MW of the QSE’s PTP Obligation Bids cleared in the DAM for the source j and the sink k for the hour. |
| q | none | A QSE. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(2)	The net total charge or payment to the QSE for the hour of all its cleared PTP Obligation Bids is calculated as follows:

DARTOBLAMTQSETOT q		=	ΣjΣkDARTOBLAMT q, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DARTOBLAMTQSETOTq | $ | Day-Ahead Real-Time Obligation Amount QSE Total per QSE - The net total charge or payment to QSE q for all its PTP Obligation Bids cleared in the DAM for the hour. |
| DARTOBLAMT q, (j, k) | $ | Day-Ahead Real-Time Obligation Amount per QSE per pair of source and sink - The charge or payment to QSE q for a PTP Obligation Bids cleared in the DAM with the source j and the sink k, for the hour. |
| q | none | A QSE. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

4.6.4	Settlement of Ancillary Services Procured in the DAM

ERCOT shall pay each QSE providing Ancillary Services procured in the DAM the amount of Ancillary Service Capacity in MW procured from the QSE multiplied by the MCPC for the Ancillary Service provided, expressed in $/MW.  Each QSE shall pay for its share of each Ancillary Service procured by ERCOT on behalf of the QSE through the DAM.

4.6.4.1	Payments for Ancillary Services Procured in the DAM

4.6.4.1.1	Regulation Up Service Payment

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Reg-Up to ERCOT were cleared in the DAM, for each hour as follows:

PCRUAMT q, DAM	=	(-1) * MCPCRU DAM * PCRU q, DAM

Where:

PCRU q, DAM	=	ΣrPCRUR r, q, DAM

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRUAMT q, DAM | $ | Procured Capacity for Reg-Up Amount per QSE in DAM—The DAM Reg-Up payment for QSE q for the hour. |
| PCRU q, DAM | MW | Procured Capacity for Reg-Up per QSE in DAM—The total Reg-Up Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by this QSE for the hour. |
| PCRUR r, q, DAM | MW | Procured Capacity for Reg-Up from Resource per Resource per QSE in DAM—The Reg-Up capacity quantity awarded to QSE q in the DAM for Resource r for the hour. |
| MCPCRU DAM | $/MW per hour | Market Clearing Price of Capacity for Reg-Up in DAM—The DAM Market Clearing Price of Capacity for Reg-Up for the hour. |
| r | none | A Resource. |
| q | none | A QSE. |

4.6.4.1.2	Regulation Down Service Payment

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Reg-Down to ERCOT were cleared in the DAM, for each hour as follows:

PCRDAMT q, DAM	=	(-1) * MCPCRD DAM * PCRD q, DAM

Where:

PCRD q, DAM	=	ΣrPCRDR r, q, DAM

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRDAMT q, DAM | $ | Procured Capacity for Reg-Down Amount per QSE in DAM—The DAM Reg-Down payment for QSE q for the hour. |
| PCRD q, DAM | MW | Procured Capacity for Reg-Down per QSE in DAM—The total Reg-Down Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by this QSE for the hour. |
| PCRDR r, q, DAM | MW | Procured Capacity for Reg-Down from Resource per Resource per QSE in DAM—The Reg-Down capacity quantity awarded to QSE q in the DAM for Resource r for the hour. |
| MCPCRD DAM | $/MW per hour | Market Clearing Price of Capacity for Reg-Down in DAM—The DAM Market Clearing Price of Capacity for Reg-Down for the hour. |
| r | none | A Resource. |
| q | none | A QSE. |

4.6.4.1.3	Responsive Reserve Service Payment

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Responsive Reserve to ERCOT were cleared in the DAM, for each hour as follows:

PCRRAMT q, DAM	=	(-1) * MCPCRR DAM * PCRR q, DAM

Where:

PCRR q, DAM	=	ΣrPCRRR r, q, DAM

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRRAMT q, DAM | $ | Procured Capacity for Responsive Reserve Amount per QSE in DAM—The DAM Responsive Reserve payment for QSE q for the hour. |
| PCRR q, DAM | MW | Procured Capacity for Responsive Reserve per QSE in DAM—The total Responsive Reserve Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by this QSE for the hour. |
| PCRRR r, q, DAM | MW | Procured Capacity for Responsive Reserve from Resource per Resource per QSE in DAM—The Responsive Reserve capacity quantity awarded to QSE q in the DAM for Resource r for the hour. |
| MCPCRR DAM | $/MW per hour | Market Clearing Price of Capacity for Responsive Reserve in DAM—The DAM Market Clearing Price of Capacity for Responsive Reserve for the hour. |
| r | none | A Resource. |
| q | none | A QSE. |

4.6.4.1.4	Non-Spinning Reserve Service Payment

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Non-Spin to ERCOT were cleared in the DAM, for each hour as follows:

PCNSAMT q, DAM	=	(-1) * MCPCNS DAM * PCNS q, DAM

Where:

PCNS q, DAM	=	ΣrPCNSR r, q, DAM

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCNSAMT q, DAM | $ | Procured Capacity for Non-Spin Amount per QSE in DAM—The DAM Non-Spin payment for QSE q for the hour. |
| PCNS q, DAM | MW | Procured Capacity for Non-Spin per QSE in DAM—The total Non-Spin Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by this QSE for the hour. |
| PCNSR r, q, DAM | MW | Procured Capacity for Non-Spin from Resource per Resource per QSE in DAM—The Non-Spin capacity quantity awarded to QSE q in the DAM for Resource r for the hour. |
| MCPCNS DAM | $/MW per hour | Market Clearing Price of Capacity for Non-Spin in DAM—The DAM Market Clearing Price of Capacity for Non-Spin for the hour. |
| r | none | A Resource. |
| q | none | A QSE. |

4.6.4.2	Charges for Ancillary Services Procurement in the DAM

4.6.4.2.1	Regulation Up Service Charge

Each QSE shall pay to ERCOT a Reg-Up Service charge for each hour as follows:

DARUAMT q	=	DARUPR * DARUQ q

Where:

DARUPR	= 	(-1) * PCRUAMTTOT DAM / DARUQTOT

PCRUAMTTOT DAM	=	ΣqPCRUAMT q, DAM

DARUQTOT	=	ΣqDARUQ q

DARUQ q	=	DARUONET q – RUSQ q, DAM

DARUONET q	=	DARUO q + DARUCS q – DARUCP q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DARUAMT q | $ | Day-Ahead Reg-Up Amount per QSE—QSE q’s share of the DAM cost for Reg-Up, for the hour. |
| DARUPR | $/MW per hour | Day-Ahead Reg-Up Price—The Day-Ahead Reg-Up price for the hour. |
| DARUQ q | MW | Day-Ahead Reg-Up Quantity per QSE—The portion of QSE q’s net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resources capacity, for the hour. |
| PCRUAMTTOT DAM | $ | Procured Capacity for Reg-Up Amount Total in DAM—The total of the DAM Reg-Up payments for all QSEs for the hour. |
| PCRUAMT q, DAM | $ | Procured Capacity for Reg-Up Amount per QSE in DAM—The DAM Reg-Up payment for QSE q for the hour. |
| DARUQTOT | MW | Day-Ahead Reg-Up Quantity Total—The sum of every QSE’s portion of its net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resource, for the hour. |
| DARUONET q | MW | Day-Ahead Reg-Up Obligation Net per QSE—The net Day-Ahead Ancillary Service obligation of QSE q, for the hour. |
| DARUO q | MW | Day-Ahead Reg-Up Obligation per QSE—The Reg-Up capacity obligation for QSE q for the DAM for the hour. |
| DARUCS q | MW | Reg-Up Capacity Sale per QSE—The total Reg-Up capacity shown in Ancillary Service Trades with QSE q as a seller for the DAM, for the hour. |
| DARUCP q | MW | Reg-Up Capacity Purchase per QSE—The total Reg-Up capacity shown in Ancillary Service Trades with QSE q as a buyer for the DAM, for the hour. |
| RUSQ q, DAM | MW | Reg-Up Supplied Quantity per QSE in DAM—The capacity for Reg-Up to be supplied with Resources represented by QSE q to meet its Ancillary Service Obligation and/or its Ancillary Service trades, for the DAM, for the hour. |

4.6.4.2.2	Regulation Down Service Charge

Each QSE shall pay to ERCOT a Reg-Down Service charge for each hour as follows:

DARDAMT q	=	DARDPR * DARDQ q

Where:

DARDPR	= 	(-1) * PCRDAMTTOT DAM / DARDQTOT

PCRDAMTTOT DAM	=	ΣqPCRDAMT q, DAM

DARDQTOT	=	ΣqDARDQ q

DARDQ q	=	DARDONET q – RDSQ q, DAM

DARDONET q	=	DARDO q + DARDCS q – DARDCP q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DARDAMT q | $ | Day-Ahead Reg-Down Amount per QSE—QSE q’s share of the DAM cost for Reg-Down, for the hour. |
| DARDPR | $/MW per hour | Day-Ahead Reg-Down Price—The Day-Ahead Reg-Down price for the hour. |
| DARDQ q | MW | Day-Ahead Reg-Down Quantity per QSE—The portion of QSE q’s net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resources capacity, for the hour. |
| PCRDAMTTOT DAM | $ | Procured Capacity for Reg-Down Amount Total in DAM—The total of the DAM Reg-Down payments for all QSEs for the hour. |
| PCRDAMT q, DAM | $ | Procured Capacity for Reg-Down Amount per QSE in DAM—The DAM Reg-Down payment for QSE q for the hour. |
| DARDQTOT | MW | Day-Ahead Reg-Down Quantity Total—The sum of every QSE’s portion of its net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resource, for the hour. |
| DARDONET q | MW | Day-Ahead Reg-Down Obligation Net per QSE—The net Day-Ahead Ancillary Service obligation of QSE q, for the hour. |
| DARDO q | MW | Day-Ahead Reg-Down Obligation per QSE—The Reg-Down capacity obligation for QSE q for the DAM for the hour. |
| DARDCS q | MW | Reg-Down Capacity Sale per QSE—The total Reg-Down capacity shown in Ancillary Service Trades with QSE q as a seller for the DAM, for the hour. |
| DARDCP q | MW | Reg-Down Capacity Purchase per QSE—The total Reg-Down capacity shown in Ancillary Service Trades with QSE q as a buyer for the DAM, for the hour. |
| RDSQ q, DAM | MW | Reg-Down Supplied Quantity per QSE in DAM—The capacity for Reg-Down to be supplied with Resources represented by QSE q to meet its Ancillary Service Obligation and/or its Ancillary Service trades, for the DAM, for the hour. |

4.6.4.2.3	Responsive Reserve Service Charge

Each QSE shall pay to ERCOT a Responsive Reserve Service charge for each hour as follows:

DARRAMT q	=	DARRPR * DARRQ q

Where:

DARRPR	= 	(-1) * PCRRAMTTOT DAM / DARRQTOT

PCRRAMTTOT DAM	=	ΣqPCRRAMT q, DAM

DARRQTOT	=	ΣqDARRQ q

DARRQ q	=	DARRONET q – RRSQ q, DAM

DARRONET q	=	DARRO q + DARRCS q – DARRCP q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DARRAMT q | $ | Day-Ahead Responsive Reserve Amount per QSE—QSE q’s share of the DAM cost for Responsive Reserve, for the hour. |
| DARRPR | $/MW per hour | Day-Ahead Responsive Reserve Price—The Day-Ahead Responsive Reserve price for the hour. |
| DARRQ q | MW | Day-Ahead Responsive Reserve Quantity per QSE—The portion of QSE q’s net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resources capacity, for the hour. |
| PCRRAMTTOT DAM | $ | Procured Capacity for Responsive Reserve Amount Total in DAM—The total of the DAM Responsive Reserve payments for all QSEs for the hour. |
| PCRRAMT q, DAM | $ | Procured Capacity for Responsive Reserve Amount per QSE for DAM—The DAM Responsive Reserve payment for QSE q for the hour. |
| DARRQTOT | MW | Day-Ahead Responsive Reserve Quantity Total—The sum of every QSE’s portion of its net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resource, for the hour. |
| DARRONET q | MW | Day-Ahead Responsive Reserve Obligation Net per QSE—The net Day-Ahead Ancillary Service obligation of QSE q, for the hour. |
| DARRO q | MW | Day-Ahead Responsive Reserve Obligation per QSE—The Responsive Reserve capacity obligation for QSE q for the DAM for the hour. |
| DARRCS q | MW | Responsive Reserve Capacity Sale per QSE—The total Responsive Reserve capacity shown in Ancillary Service Trades with QSE q as a seller for the DAM, for the hour. |
| DARRCP q | MW | Responsive Reserve Capacity Purchase per QSE—The total Responsive Reserve capacity shown in Ancillary Service Trades with QSE q as a buyer for the DAM, for the hour. |
| RRSQ q, DAM | MW | Responsive Reserve Supplied Quantity per QSE in DAM—The capacity for Responsive Reserve to be supplied with Resources represented by QSE q to meet its Ancillary Service Obligation and/or its Ancillary Service trades, for the DAM, for the hour. |

4.6.4.2.4	Non-Spinning Reserve Service Charge

Each QSE shall pay to ERCOT a Non-Spin Service charge for each hour as follows:

DANSAMT q	=	DANSPR * DANSQ q

Where:

DANSPR	= 	(-1) * PCNSAMTTOT DAM / DANSQTOT

PCNSAMTTOT DAM	=	ΣqPCNSAMT q, DAM

DANSQTOT	=	ΣqDANSQ q

DANSQ q	=	DANSONET q – NSSQ q, DAM

DANSONET q	=	DANSO q + DANSCS q – DANSCP q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DANSAMT q | $ | Day-Ahead Non-Spin Amount per QSE—QSE q’s share of the DAM cost for Non-Spin, for the hour. |
| DANSPR | $/MW per hour | Day-Ahead Non-Spin Price—The Day-Ahead Non-Spin price for the hour. |
| DANSQ q | MW | Day-Ahead Non-Spin Quantity per QSE—The portion of QSE q’s net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resources capacity, for the hour. |
| PCNSAMTTOT DAM | $ | Procured Capacity for Non-Spin Amount Total in DAM—The total of the DAM Non-Spin payments for all QSEs for the hour. |
| PCNSAMT q, DAM | $ | Procured Capacity for Non-Spin Amount per QSE in DAM—The DAM Non-Spin payment for QSE q for the hour. |
| DANSQTOT | MW | Day-Ahead Non-Spin Quantity Total—The sum of every QSE’s portion of its net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resource, for the hour. |
| DANSONET q | MW | Day-Ahead Non-Spin Obligation Net per QSE—The net Day-Ahead Ancillary Service obligation of QSE q, for the hour. |
| DANSO q | MW | Day-Ahead Non-Spin Obligation per QSE—The Non-Spin capacity obligation for QSE q for the DAM for the hour. |
| DANSCS q | MW | Non-Spin Capacity Sale per QSE—The total Non-Spin capacity shown in Ancillary Service Trades with QSE q as a seller for the DAM, for the hour. |
| DANSCP q | MW | Non-Spin Capacity Purchase per QSE—The total Non-Spin capacity shown in Ancillary Service Trades with QSE q as a buyer for the DAM, for the hour. |
| NSSQ q, DAM | MW | Non-Spin Supplied Quantity per QSE in DAM—The capacity for Non-Spin to be supplied with Resources represented by QSE q to meet its Ancillary Service Obligation and/or its Ancillary Service trades, for the DAM, for the hour. |

4.6.5	Calculation of “Average Incremental Energy Cost” (AIEC)

The methodology of AIEC calculation is presented below.  AIEC is used to account for the additional cost for a Generation Resource to produce energy above its LSL.  This cost calculation methodology is used for the calculation of DAAIEC, RTAIEC, RTVSSAIEC, and RTHSLAIEC variables.  The DAAIEC and RTAIEC are subject to the Energy Offer Curve Cap, while the RTVSSAIEC and RTHSLAIEC are not subject to price caps.

- Energy Offer Curve

| Index (i) | MW | $/MWh |
| --- | --- | --- |
| 1 | Q1 | P1 |
| 2 | Q2 | P2 |
| N (N10) | QN | PN |

Variables DAAIEC and RTAIEC should calculate the associated price caps as specified in steps II through IV, the calculation process for Variables RTVSSAIEC and RTHSLAIEC should skip steps II through IV and continue with step V.

- MW quantity corresponding with Energy Offer Curve Cap, P ($/MWh), where Pi<P≤Pi+1 (i=1,2,⋯,N−1)

Q ($/MWh), where Q=Qi+Qi+1−QiPi+1−Pi(P−Pi)

- Energy Offer Curve capped with the Energy Offer Curve Cap;

- When

| Index (j) | MW | $/MWh |
| --- | --- | --- |
| 1 | Q1 | P1 |
| ⋮ | ⋮ | ⋮ |
| i | Qi | Pi |
| i+1 | Q | P |
| i+2 | QN | P |

- When :

| Index (j) | MW | $/MWh |
| --- | --- | --- |
| 1 | Q1 | P1 |
| N | QN | PN |

- Cleared offer on the capped Energy Offer Curve

- When :

Q (MW), where Qj<Q≤Qj+1 (j=1,⋯,i,i+1)

- When :

Q (MW), where Qj<Q≤Qj+1 (j=1,⋯,N−1)

- Incremental energy price corresponding with cleared offer, on the capped Energy Offer Curve:

P ($/MWh), where P=Pj+Pj+1−PjQj+1−Qj(Q−Qj)

- Average incremental energy cost corresponding with (Q-Q1>0), on the capped Energy Offer Curve:

AIEC=P1+P2, for Q1<Q≤Q2k=1j−1Pk+Pk+12(Qk+1−Qk)+Pj+P2(Q−Qj)/(Q−Q1), for Q>Q2

If the Energy Offer Curve Cap is less than the lowest price of the energy offer curve, the AIEC is the Energy Offer Curve Cap.  If the Energy Offer Curve Cap is greater than the highest price of the energy offer curve, then Qdoes not need to be calculated.