| NPRR Number | TBD | NPRR Title | Settlement Clarifications to RUC Capacity Shortfall Ratio Share Formula. |
| --- | --- | --- | --- |
| Date Posted |  | Sent to TPTF August 24, 2007 |  |
| Nodal Protocol Section(s) Requiring Revision (Include Section No. and Title) |  | 5.7.1.1, RUC Guarantee 5.7.1.2, RUC Minimum-Energy Revenue 5.7.1.3, Revenue Less Cost Above LSL During RUC-Committed Hours 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals 5.7.3, Payment When ERCOT Decommits a QSE -Committed Resource 5.7.4.1.1, Capacity Shortfall Ratio Share |  |
| Revision Description |  | This NPRR : - Clarifies the section that deals with the Capacity Shortfall Ratio Share to include DC Tie imports and exports. - Modifies the definition of the variable HASLADJ (High Ancillary Service Limit) used in calculating the RUC Capacity Shortfall Ratio Share in Section 5.7.4. - Modifies the definition of the LSL (Low Sustained Limit) used in RUC Section 5. |  |
| Reason for Revision |  | The current Protocol language in Section 5.7.4.1.1 does not include DC Tie imports and exports when calculating the QSE’s Capacity at each RUC process. The inclusion of DC Tie activity is necessary to correctly reflect each QSE’s Capacity at each RUC process as specified in Section 4.4.4 paragraph (3). Also, the current definition of ‘HASLADJ’ in Section 5.7.4 includes specific instructions that need to be pulled out to stand alone. In addition, the definition of LSL (Low Sustained Limit) in Section 5 needs clarification that the LSL value used will come from the COP. |  |
| Overall Market Benefit |  | Updates Protocols so they better and more clearly reflect the intent of the Market Participants. |  |
| Overall Market Impact |  | Minimizes confusion and disputes. |  |
| Consumer Impact |  | None. |  |
| Credit Implications (Yes or No, and summary of impact) |  | None |  |
| Reason for Revision (from Transition Plan Task Force (TPTF) Charter Scope) |  | (1) Revisions resulting from Commission orders; (2) Clarifications of Protocol language that do not change the intent or technical specifications of the Protocols; (3) Correction of technical errors or processes that are found to not be technically feasible; (4) Revisions to the Protocols necessary to implement the results of the value engineering analysis or to otherwise avoid severe cost impacts; or (5) Other (describe): |  |
| TPTF Review (Yes or No, and summary of conclusion) |  | No |  |

| Sponsor |  |
| --- | --- |
| Name | Kenneth Ragsdale and Pamela Shaw |
| E-mail Address | kragsdale@ercot.com |
| Company | ERCOT |
| Company Address | 2705 West Lake Dr., Taylor, TX 76574 |
| Phone Number | (512) 248-3917 |
| Cell Number | (512) 750-3505 |
| Market Segment | N/A |

ERCOT/Market Segment Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Maintain consistency between business requirements and protocols. |  |
| Additional Qualitative Information | 1 |  |  |
| Other Comments | 1 |  |  |

Proposed Nodal Protocol Language Revision

5.7.1.1	RUC Guarantee

(1)	If a validated Three-Part Supply Offer has been submitted for a Resource for the RUC, then the RUC Guarantee for that Resource is based on the Startup Offer and Minimum-Energy Offer in that validated Three-Part Supply Offer.  If a validated Three-Part Supply Offer has not been submitted for a Resource for the RUC and ERCOT has not yet approved verifiable unit-specific costs for the Resource, then the RUC Guarantee for a Resource is based on the Resource Category Startup Generic Cap and the Resource Category Minimum-Energy Generic Cap.  If a validated Three-Part Supply Offer has not been submitted for a Resource for the RUC and ERCOT has approved verifiable unit-specific costs for the Resource, then the RUC Guarantee for a Resource is based on the most recent ERCOT-approved verifiable unit-specific costs for that Resource.  The RUC Guarantee Minimum-Energy Costs are prorated according to the actual generation when the Resource’s average output during a 15-minute Settlement Interval is below the corresponding LSL.

(2)	The RUC Guarantee is calculated as follows:

RUCGq,r,d	=	 Σs(SUPRq,r,s * RUCSUFLAGq,r,s) + Σi(MEPRq,r,i * Min ((LSLq,r,i * (¼)), RTMGq,r,i))

If the QSE submitted a validated Three-Part Supply Offer,

Then, 		SUPRq,r,s	=	SUOq,r,s

MEPRq,r,i	=	MEOq,r,i

Otherwise, 	SUPRq,r,s 	= 	SUCAPq,r,s

MEPRq,r,i 	= 	MECAPq,r,i

If ERCOT has approved verifiable startup and minimum-energy costs for the Resource,

Then, 		SUCAPq,r,s	=	verifiable startup costsq,r,s

MECAPq,r,i	=	verifiable minimum-energy costsq,r,i

Otherwise, 	SUCAPq,r,s 	= 	RCGSCs

MECAPq,r,i	= 	RCGMECi

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCGq,r,d | $ | RUC Guarantee—The sum of the Resource’s eligible Startup Costs and Minimum-Energy Costs during all RUC-Committed Hours, for the Operating Day. |
| SUPRq,r,s | $/Start | Startup Price per start—The settlement price for the start s. |
| SUOq,r,s | $/Start | Startup Offer per start—Represents an offer for all costs incurred by a Generation Resource in starting up and reaching breaker close, as indicated by a telemetered Resource status of On-Line. |
| SUCAPq,r,s | $/Start | Startup Cap—The amount used as startup costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGSC unless ERCOT has approved verifiable unit-specific startup costs for that Resource, in which case the startup cap is the verifiable unit-specific startup cost. See Section 5.6.1, Verifiable Costs for more information on verifiable costs. |
| RCGSCs | $/Start | Resource Category Generic Startup Cost—The Resource Category Generic Startup Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| RUCSUFLAGq,r,s | none | RUC Startup Flag—The flag that indicates whether or not the start s is eligible for RUC Make-Whole Payment. Its value is one if eligible; otherwise, zero. See Section 5.6.2, RUC Startup Cost Eligibility and Section 5.6.3, Forced Outage of RUC-Committed Resource, for more information on startup eligibility. |
| MEPRq,r,i | $/MWh | Minimum-Energy Price—The settlement price for minimum energy for the Settlement Interval i. |
| MEOq,r,i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by a Resource in producing energy at the Resource’s LSL for the Settlement Interval i. |
| MECAPq,r,i | $/MWh | Minimum Energy Cap— The amount used for minimum-energy costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGMEC unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1, Verifiable Costs for more information on verifiable costs. |
| RCGMECi | $/MWh | Resource Category Generic Minimum-Energy Cost—The Resource Category Generic Minimum Energy Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| RTMGq,r,i | MWh | Real-Time Metered Generation—The Resource’s metered generation for the Settlement Interval i. |
| LSLq,r,i | MW | Low Sustained Limit—The Low Sustainable Limit of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| i | none | A 15-minute Settlement Interval within the hour that includes a RUC-commitment. |
| s | none | A start that is eligible to have its costs included in the RUC Guarantee. |

5.7.1.2	RUC Minimum-Energy Revenue

The energy revenue for the Resource’s generation up to LSL during all RUC-Committed Hours of the Operating Day is calculated as follows:

RUCMEREVq,r,d	=	Σi(RTSPPp,i * Min (RTMGq,r,i, (LSLq,r,i * (¼))))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCMEREVq,r,d | $ | RUC Minimum-Energy Revenue—The sum of the energy revenues for the Resource’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. |
| RTSPPp,i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource Node for the Settlement Interval i. |
| RTMGq,r,i | MWh | Real-Time Metered Generation—The Resource’s metered generation for the Settlement Interval i. |
| LSLq,r,i | MW | Low Sustained Limit— The Low Sustainable Limit of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval within the hour that includes a RUC-commitment. |

5.7.1.3	Revenue Less Cost Above LSL During RUC-Committed Hours

The total revenue for the Resource operating above its LSL less the cost based on the Resource’s Energy Offer Curve capped by the energy offer curve cap (as described in Section 4.4.9.3, Energy Offer Curve and in Section 4.4.9.3.3, Energy Offer Curve Caps for Make-Whole Calculation Purposes) or proxy Energy Offer Curve described in Section 6.5.7.3, Security Constrained Economic Dispatch, as applicable, during all RUC-Committed Hours of the Operating Day is calculated as follows:

RUCEXRRq,r,d   =   Max {0, Σi[RTSPPp,i * Max (0, RTMGq,r,i – (LSLq,r,i * (¼)))

+ (-1) * (VSSVARAMTq,r,i + VSSEAMTq,r,i)

+ (-1) * EMREAMTq,r,i
– RTAIECq,r,i * Max (0, RTMGq,r,i – (LSLq,r,i * (¼)))]}

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCEXRRq,r,d | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for the Resource operating above its LSL less the cost during all RUC-Committed Hours, for the Operating Day. |
| RTSPPp,i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource’s Settlement Point for the Settlement Interval i. |
| RTAIECq,r,i | $/MWh | Real-Time Average Incremental Energy CostThe average incremental energy cost, calculated using the Energy Offer Curve capped by the Energy Offer Curve Cap, for the Resource’s generation above the LSL for the Settlement Interval i. See Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC). |
| RTMGq,r,i | MWh | Real-Time Metered Generation—The Resource’s metered generation for the Settlement Interval i. |
| LSLq,r,i | MW | Low Sustained Limit— The Low Sustainable Limit of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. |
| VSSVARAMTq,r,i | $ | Voltage Support Service var Amount by interval—The payment to the QSE for the VSS provided by Generation Resource for the 15-minute Settlement Interval i. See Section 6.6.7.1, Voltage Support Service Payments. |
| VSSEAMTq,r,i | $ | Voltage Support Service Energy Amount by interval—The lost opportunity payment to the QSE for ERCOT-directed VSS from the Generation Resource for the 15-minute Settlement Interval i. See Section 6.6.7.1, Voltage Support Service Payments. |
| EMREAMTq,r,i | $ | Emergency Energy Amount by interval—The payment to the QSE as additional compensation for the additional energy produced by the Generation Resource in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval i. See Section 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval within the hour that includes a RUC instruction. |

5.7.1.4	Revenue Less Cost During QSE Clawback Intervals

The total revenue for the Resource less the cost based on the Resource’s Energy Offer Curve capped by the energy offer curve cap (as described in Section 4.4.9.3, Energy Offer Curve and in Section 4.4.9.3.3, Energy Offer Curve Caps for Make-Whole Calculation Purposes) or proxy Energy Offer Curve described in Section 6.5.7.3, Security Constrained Economic Dispatch, as applicable, during all QSE Clawback Intervals of the Operating Day is calculated as follows:

RUCEXRQCq,r,d	=	Max {0, Σi[(RTSPPp,i * RTMGq,r,i )
+ (-1) * (VSSVARAMTq,r,i + VSSEAMTq,r,i)
+ (-1) * EMREAMTq,r,i
– [MEPRq,r,i * Min (RTMGq,r,i, (LSLq,r,i * (¼)))] 
– [RTAIECq,r,i * Max (0, RTMGq,r,i – (LSLq,r,i * (¼)))]]}

If the QSE submitted a validated Three-Part Supply Offer,

Then, 		MEPRq,r,i	=	MEOq,r,i

Otherwise, 	MEPRq,r,i 	= 	MECAPq,r,i

If QSE verifiable minimum-energy costs for the Resource are on file,

Then,		MECAPq,r,i	=	verifiable minimum-energy costsq,r,i

Otherwise, 	MECAPq,r,i	= 	RCGMECi

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCEXRQCq,r,d | $ | Revenue Less Cost During QSE-Clawback Intervals—The sum of the total revenue for the Resource less the cost during all QSE-Clawback Intervals for the Operating Day. |
| RTSPPp,i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource’s Settlement Point for the Settlement Interval i. |
| MEPRq,r,i | $/MWh | Minimum-Energy Price—The settlement price for minimum energy for the Settlement Interval i. |
| MEOq,r,i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by a Resource in producing energy at the Resource’s LSL for the Settlement Interval i. |
| MECAPq,r,i | $/MWh | Minimum Energy Cap— The amount used for minimum-energy costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGMEC unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1, Verifiable Costs for more information on verifiable costs. |
| RCGMECi | $/MWh | Resource Category Generic Minimum-Energy Cost—The Resource Category Generic Minimum-Energy Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| RTAIECq,r,i | $/MWh | Real-Time Average Incremental Energy CostThe average incremental energy cost, calculated using the Energy Offer Curve capped by the Energy Offer Curve Cap, for the Resource’s generation above the LSL for the Settlement Interval i. See Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC). |
| RTMGq,r,i | MWh | Real-Time Metered Generation—The Resource’s metered generation for the Settlement Interval i. |
| LSLq,r,i | MW | Low Sustained Limit— The Low Sustainable Limit of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. |
| VSSVARAMTq,r,i | $ | Voltage Support Service var Amount by interval—The payment to the QSE for the VSS provided by Generation Resource for the 15-minute Settlement Interval i. See Section 6.6.7.1, Voltage Support Service Payments. |
| VSSEAMTq,r,i | $ | Voltage Support Service Energy Amount by interval—The lost opportunity payment to the QSE for ERCOT-directed VSS from the Generation Resource for the 15-minute Settlement Interval i. See Section 6.6.7.1, Voltage Support Service Payments. |
| EMREAMTq,r,i | $ | Emergency Energy Amount by interval—The payment to the QSE as additional compensation for the additional energy produced by the Generation Resource in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval i. See Section 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval within the hour that is identified as a QSE-Clawback Interval. |

5.7.3	Payment When ERCOT Decommits a QSE -Committed Resource

(1)	If ERCOT decommits a QSE-committed Resource during the RUC process earlier than its scheduled shutdown within the Operating Day, then no compensation is due to the affected QSE from ERCOT.

(2)	If ERCOT decommits a QSE committed Resource that is not scheduled to shutdown within the Operating Day, then ERCOT shall pay the affected QSE an amount as calculated below for the hours of decommitment.  The number of continuous decommitted hours used in the calculation are the hours beginning with the first decommitted hour until the earlier of:

(a)	The hour ERCOT determines that the Resource may again be at LSL; and

(b)	The end of the last hour of the Operating Day.

(3)	If ERCOT decommits a QSE-committed Resource not scheduled to shutdown within the Operating Day, and the decommitment period spans more than one Operating Day, the RUC Decommitment Payment shall be calculated and paid in the Operating Day in which the RUC Decommitment originated.  The number of continuous decommitted hours used in the calculation are the hours beginning with the first decommitted hour until the end of the last hour of the Operating Day in which the RUC Decommitment originated.

(4)	The payment for a RUC decommitment instruction for a Resource, including RMR units, is calculated for each hour as follows:

RUCDCAMTq,r,h	=	(-1) * Max (0, (SUPRq,r,s - Σi(Max (0, MEPRq,r,i - RTSPPp,i) * (LSLq,r,i * (¼))))) / NCDCHRq,r,h

Where:

If the QSE submitted a validated Three-Part Supply Offer,

Then, 		SUPRq,r,s		=	SUOq,r,s

MEPRq,r,i	=	MEOq,r,i

Otherwise, 	SUPRq,r,s 	= 	SUCAPq,r,s

MEPRq,r,i 	= 	MECAPq,r,i

If QSE verifiable startup and minimum-energy costs for the Resource are on file,

Then, 		SUCAPq,r,s	=	verifiable startup costsq,r,s

MECAPq,r,i	=	verifiable minimum-energy costsq,r,i

Otherwise, 	SUCAPq,r,s 	= 	RCGSCs

MECAPq,r,i	= 	RCGMECi

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCDCAMTq,r,h | $ | RUC De-commitment Payment Amount—The payment to the QSE for the Resource that was de-committed by ERCOT but that was not scheduled to shut down in the Operating Day, for each decommited hour of the Operating Day. |
| SUPRq,r,s | $/Start | Startup Price per start—The settlement price for the start s. |
| SUOq,r,s | $/Start | Startup Offer per start—Represents an offer for all costs incurred by a Generation Resource in starting up and reaching breaker close, as indicated by a telemetered Resource status of On-Line. |
| SUCAPq,r,s | $/Start | Startup Cap—The amount used as startup costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGSC unless ERCOT has approved verifiable unit-specific startup costs for that Resource, in which case the startup cap is the verifiable unit-specific startup cost. See Section 5.6.1, Verifiable Costs for more information on verifiable costs. |
| RCGSCs | $/Start | Resource Category Generic Startup Cost—The Generic Startup Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| MEPRq,r,I | $/MWh | Minimum-Energy Price—The settlement price for minimum energy for the Settlement Interval i. |
| MEOq,r,i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by a Resource in producing energy at the Resource’s LSL for the Settlement Interval i. |
| MECAPq,r,i | $/MWh | Minimum Energy Cap— The amount used for minimum-energy costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGMEC unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1, Verifiable Costs, for more information on verifiable costs. |
| RCGMECi | $/MWh | Resource Category Generic Minimum Energy Cost—The Generic Minimum Energy Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| LSLq,r,i | MW | Low Sustained Limit— The Low Sustainable Limit of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. |
| RTSPPp,i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource’s Settlement Point for the Settlement Interval i. |
| NCDCHRq,r,h | none | Number of Continuous De-committed Hours— The number of continuous decommitment hours within an Operating Day. |
| q | none | A QSE. |
| r | none | A RUC-decommitted Generation Resource. |
| h | none | An hour in the RUC Decommitment period. |
| p | none | A Resource Node Settlement Point. |
| s | none | A Start. |
| i | none | A 15-minute Settlement Interval within the hour that includes an ERCOT De-commitment. |

5.7.4.1.1	Capacity Shortfall Ratio Share

(1)	In calculating the amount short for each QSE, the QSE must be given a capacity credit for its WGRs based on the HSL values entered into the COP by the QSE just prior to the RUC execution.  For WGRs, ERCOT shall use for settlement purposes the COP and Trades Snapshot prior to the RUC regardless of Real-Time capacity or actual generation.  Therefore, the HASLSNAP and HASLADJ variables used below shall be equal to the HSL values entered into the QSE’s COP submitted prior to the RUC for WGRs.

(2)	In calculating the amount short for each QSE, the QSE must be given a capacity credit for non-wind Resources that were given notice of decommitment within the two hours before the Operating Hour as a result of the RUC process by setting the HASLSNAP and HASLADJ variables used below equal to the HASLSNAP value for the Resource immediately before the decommitment instruction was given.

(3)	In calculating the amount short for each QSE, if the HASL for a Resource was credited to the QSE during the RUC Snapshot but the Resource experiences a Forced Outage within two hours before the start of the Settlement Interval, then the HASL for that Resource is also credited to the QSE in the HASLADJ.

(4)	In calculating the amount short for each QSE, if the DCIMP was credited to the QSE during the RUC Snapshot but the entire DC Tie experiences a Forced Outage within two hours before the start of the Settlement Interval, then the DCIMP  is also credited to the QSE in the DCIMPADJ.

(5)	The capacity shortfall ratio share of a specific QSE for a particular RUC process is calculated, for a 15-minute Settlement Interval, as follows:

RUCSFRSruc,i,q	=	RUCSFruc,i,q / RUCSFTOTruc,i

Where:

RUCSFTOTruc,i	=	ΣqRUCSFruc,i,q

(4)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval is:

RUCSFruc,i,q	=	Max (0, Max (RUCSFSNAPq,i, RUCSFADJq,i) – Σz is prior to rucRUCCAPCREDITq,i,z)

(5)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at the snapshot, is:

RUCSFSNAPq,i	=	Max (0, ((ΣpRTAMLq,p,i * 4)  + Σp RTDCEXP q, p, i –  RUCCAPSNAPq,i ))

(6)	The amount of capacity that a QSE had according to the RUC snapshot for a 15-minute Settlement Interval.

RUCCAPSNAPq,i =	ΣrHASLSNAPq,r,h + (RUCCPSNAPq,h –RUCCSSNAPq,h) + (ΣpDAEPq,p,h –ΣpDAESq,p,h) + (ΣpRTQQEPSNAPq,p,i – ΣpRTQQESSNAPq,p,i) +  Σp DCIMPSNAP q, p, i

(7)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at Real Time, is:

RUCSFADJq,i	=	Max (0, ((ΣpRTAMLq,p,i) *4) + Σp RTDCEXP q, p, i – RUCCAPADJq,i)

(8)	The amount of capacity that a QSE had in Real Time for a 15-minute Settlement Interval.

RUCCAPADJq,i =	ΣrHASLADJq,r,h + (RUCCPADJq,h – RUCCSADJq,h) + (ΣpDAEPq,p,h – ΣpDAESq,p,h) + (ΣpRTQQEPADJq,p,i– ΣpRTQQESADJq,p,i) + Σp DCIMPADJ q, p, i

(9)	The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCSFRSruc,i,q | none | RUC Shortfall Ratio Share—The ratio of the QSE’s capacity shortfall to the sum of all QSEs’ capacity shortfalls, for the RUC process, for the 15-minute Settlement Interval. |
| RUCSFruc,i,q | MW | RUC Shortfall—The QSE q’s capacity shortfall for the RUC process for the 15-minute Settlement Interval. |
| RUCSFTOTruc,i | MW | RUC Shortfall Total-The sum of all QSEs’ capacity shortfalls, for a RUC process, for a 15-minute Settlement Interval. |
| RUCSFSNAPq,i | MW | RUC Shortfall at Snapshot—The QSE q’s capacity shortfall according to the snapshot for the RUC process for the 15-minute Settlement Interval. |
| RUCSFADJq,i | MW | RUC Shortfall at Adjustment Period—The QSE q’s adjustment period capacity shortfall for the 15-minute Settlement Interval. |
| RUCCAPCREDITq,i,z | MW | RUC Capacity Credit by QSE—The capacity credit resulting from capacity paid through the RUC Capacity-Short Charge for the 15-minute Settlement Interval. |
| RTAMLq,p,i | MWh | Real-Time Adjusted Metered Load—The QSE q’s Adjusted Metered Load at the Settlement Point p for the 15-minute Settlement Interval. |
| RUCCAPSNAPq,i | MW | RUC Capacity Snapshot at time of RUC -The amount of the QSE’s calculated capacity in the COP and Trades Snapshot for a 15-minute Settlement Interval. |
| HASLSNAPq,r,h | MW | High Ancillary Services Limit at Snapshot—The High Ancillary Services Limit of the Resource r represented by the QSE q, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RTDCEXP q, p | MW | Real-Time DC Export per QSE per Settlement Point—The aggregated DC Tie Schedule through DC Tie p submitted by QSE q that is under the “Oklaunion Exemption” as an exporter from the ERCOT area, for the 15-minute Settlement Interval. |
| DCIMPADJ q, p | MW | DC Import per QSE per Settlement Point—The approved aggregated DC Tie Schedule submitted by QSE q as an importer into the ERCOT System through DC Tie p according to the adjustment period Snapshot, for the 15-minute Settlement Interval. |
| DCIMPSNAP q, p | MW | DC Import per QSE per Settlement Point—The approved aggregated DC Tie Schedule submitted by QSE q as an importer into the ERCOT System through DC Tie p, according Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RUCCPSNAPq,h | MW | RUC Capacity Purchase at Snapshot—The QSE q’s Capacity purchase, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RUCCSSNAPq,h | MW | RUC Capacity Sale at Snapshot—The QSE q’s capacity sale, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RUCCAPADJq,i | MW | RUC Capacity Snapshot during Adjustment Period-The amount of the QSE’s calculated capacity in the RUC according to the COP and Trades Snapshot at the end of the Adjustment Period for a 15-minute Settlement Interval |
| HASLADJq,r,h | MW | High Ancillary Services Limit at Adjustment Period - The High Ancillary Services Limit of the Resource r represented by the QSE q, according to the adjustment period snapshot, for the hour that includes the 15-minute Settlement Interval. |
| RUCCPADJq,h | MW | RUC Capacity Purchase at Adjustment Period—The QSE q’s capacity purchase, according to the Adjustment Period COP and Trades Snapshot for the hour that includes the 15-minute Settlement Interval. |
| RUCCSADJq,h | MW | RUC Capacity Sale at Adjustment Period—The QSE q’s capacity sale, according to the Adjustment Period COP and Trades Snapshot for the hour that includes the 15-minute Settlement Interval. |
| DAEPq,p,h | MW | Day-Ahead Energy Purchase—The QSE q’s energy purchased in the DAM at the Settlement Point p for the hour that includes the 15-minute Settlement Interval. |
| DAESq,p,h | MW | Day-Ahead Energy Sale—The QSE q’s energy sold in the DAM at the Settlement Point p for the hour that includes the 15-minute Settlement Interval. |
| RTQQEPSNAPq,p,i | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval, in the COP and Trades Snapshot. |
| RTQQESSNAPq,p,i | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval, in the COP and Trades Snapshot. |
| RTQQEPADJq,p,i | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. |
| RTQQESADJq,p,i | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. |
| q | none | A QSE. |
| p | none | A Settlement Point. |
| r | none | A Generation Resource that is QSE-committed or RUC-decommitted (subject to paragraphs 1 and 2 above) for the Settlement Interval. |
| z | none | A previous RUC process for the Operating Day. |
| i | none | A 15-minute Settlement Interval. |
| h | none | The hour that includes the Settlement Interval i. |
| ruc | none | The RUC process for which this Capacity Shortfall Ratio Share is calculated. |

LCRA submits comments....

1.  RTDCEXP should be defined as relating to any DC tie, not just the Oklaunion tie.

2.  The term DCIMP is not defined.