ERCOT Power Operations Bulletin #349

ERCOT has revised the Day Ahead Desk Procedure Manual.

The specific changes are shown below.

A copy of the procedure is available at:

- http://www.ercot.com/mktrules/guides/procedures/index.html

| ## 2.12 Initiate ERCOT Day Ahead Replacement Capacity Market |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | - Before 0600, on the Day Ahead Market Management Display, CONFIRM a Replacement Market was created for tomorrow for hours 0100 to 2400. - If NOT, then NOTIFY EMMS Production or the Helpdesk and manually create the RPRS market for tomorrow, OTHERWISE CONTINUE. |
| NOTE: | Ensure OOMC for Voltage Support (if applicable) are complete prior to Step 2. This includes units for voltage support from the Outage Notes. |
| NOTE: | QSEs that plan to provide Non-Spin from off-line GTs that can start in 30 minutes or less, should have their Non-Spin flag checked in order to identify the off-line units being used for Non-Spin.. RPRS study will ignore “On-line and Available” units and units with lead time longer than 30 minutes that have their Non-Spin flags checked. RPRS study will consider the off-line GTs as on-line if their Non-Spin flag is checked. The Non-Spin obligation will be added to the Replacement System Capacity Obligation as well. |
| NOTE: | If it is determined that deficiencies exist with the input data into RPRS (i.e. Incorrect or Invalid Resource Plan, Invalid Load Forecast, etc), and these deficiencies may potentially affect the RPRS solution, additional actions may need to be taken. If RPRS completes it may result in excessive procurement of Step 2 capacity. |
| 2 | Make sure that the “Execute W/L” checkbox is checked. If it is not, then click on it to check the box. |
| 3 | After the Resource Plan is validated, RUN Steps 1 and 2 of Replacement Analysis by clicking the “EXECUTE” button. - If a QSE notifies ERCOT of a Resource Plan problem after the Study has begun, THEN: - Consult with the Shift Supervisor to determine whether or not to stop RPRS. - If the decision is to let RPRS complete on its own, THEN do so and go to Step 4 of this procedure. Otherwise, CONTINUE. - If the decision is to stop RPRS, THEN - STOP RPRS, and wait for the Resource Plan to be updated by the QSE. - Re-run RPRS once the Resource Plan is updated. |
| NOTE: | Resource de-selection and re-selection is done on the Procurement Summary Page. |
| 4 | Review Analysis Results - Do NOT deselect any RMR units selected by the RPRS study. Use the study selections to provide an Updated Final RMR Delivery Plan to the appropriate QSE. - Refer to Outage Coordinator Notes for unit requirements due to outages and compare with RPRS results. - If STEP 1 (local congestion)of the RPRS study has selected a steam turbine (ST) from a combined cycle train which requires a combustion turbine (CT) from the same train to be online, it is acceptable to OOMC that CT if it was not already expected to be online or procured. - If Step 2 (zonal congestion and capacity) of the RPRS study has selected a steam turbine from a combined cycle train which requires a combustion turbine from the same train to be online, the market participant must bring on the necessary CT on their own without an OOMC, because resources are usually procured by bids in Step 2. - If Step 1 or 2 selected only combined cycle CT(s), do NOT OOMC its steam units. - Investigate discrepancies, if any. - Deselect additional units for each hour if you know they are not needed. - If deselecting a unit, do NOT replace a de-selection with an OOMC unless you are substituting with another unit for a justifiable reason. |
| 5 | Execute RPRS Step 3 (Pricing). |

| ## 2.13 Procure Replacement Reserve Services and Manage Capacity Shortfalls |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | If it is determined that weather, outage or other conditions will require capacity from resources with lead times too long to be considered by RPRS, then additional capacity should be OOMCd via Verbal Dispatch Instruction. The choice of resources to OOMC should be made considering the causative conditions, available resources, lead times, congestion management and other operational concerns. |
| 1 | By 1800 for a Day Ahead market, - Review the solution of the Replacement Capacity analysis for reasonableness. If capacity was procured in step 2 then spot check Market Clearing Prices - If reasonable, APPROVE, and task is complete. OTHERWISE, CONTINUE - If not reasonable: - Verify whether or not the Resource Plan has been corrected by the QSE. - If the Resource Plan has been corrected, THEN re-run RPRS. - If the Resource Plan has not been corrected, THEN contact the QSE and determine how long it will take for the Resource Plan to be updated. Once the Resource Plan is updated, THEN re-run RPRS. - If the result is reasonable, THEN initiate Step 3 (Pricing) and APPROVE the study and go on to STEP 2. Otherwise, CONTINUE - Discuss with Operations Support. If the consensus is that the results are not reasonable, then, either make additional changes and APPROVE OR click UNAPPROVE. - If it is determined in any of the steps above that a valid RPRS result will not be achieved when Step 1 and 2 of RPRS completes, UNAPPROVE the study. - If the RPRS study completes in a “Failed” state, click on the “Execute” button, and re-run RPRS, if time permits. If time does not permit, click on the “UnApprove,” button. Immediately contact EMMS Production Support On-Call staff, and have them determine, and fix, the problem that caused the RPRS study to fail. - If not completed by 1800, issue an Emergency Notice that this time requirement will not be met. |
| 2 | Following any procurement or non-approval of replacement capacity, determine the OOM/RMR units to be manually dispatched if the market cannot resolve a capacity shortage. - IDENTIFY any expected capacity shortages for any hour(s) of the operating day. - IF a capacity shortage for a given hour is insignificant (i.e., less than 100 MW), RECOMMEND to the Shift Supervisor that it be observed during the adjustment period for possible correction. - IF the shortage is significant (i.e., greater than 100 MW), confer with the Transmission & Security desk operator about constraints and consider OOM for enough capacity in any zone to make up the shortage for the hours needed. Minimize starts in the OOMC process. - IF a QSE refuses to commit capacity selected in step 2 of the RPRS study, OOMC equivalent MW capacity elsewhere. Log the event & notify the shift supervisor. |