Texas Nodal Market Implementation:

Monitoring Programs for QSEs, TSPs and ERCOT

(To be incorporated into Nodal Operating Guides Section 2 when approved by TAC)

v0.04

Background	3

Objectives	3

1	QSE/Resource Monitoring Program	4

1.1  Net dependable real power capability testing	4

1.2  Reactive testing	4

1.3  Real-Time data	4

1.4  Compliance with Verbal Dispatch Instructions	5

1.5   Supplying and validating data for generator models	5

1.6   Resource Outage Scheduling and Reporting	6

1.7   Generation under-frequency relay coordination	7

1.8   Backup control for Resource Energy Deployment	7

1.9   Qualified staffing requirement	7

1.10  Automatic Voltage Regulator (AVR) and Power System Stabilizer (PSS)	8

1.11  Current Operating Plan metrics, for QSEs	8

1.12  Testing and performance of governor for Generation Resources	9

2	TSP Monitoring Program	9

2.1  Rating Methodology Reporting Program	10

2.2  Real-time data Monitor	10

2.3  Outage Scheduling and coordination monitor	11

2.5  Compliance with Verbal Dispatch instructions monitoring	12

2.6  Voltage and Reactive Control Performance monitoring	12

3 	ERCOT Monitoring Program	12

3.1   Transmission Control	12

3.2   Resource Control	14

3.3   Load Forecasting	16

3.4   System Operating Constraints	17

3.5   Network Operations Model update implementation performance	18

3.6   Network Operations Model validation	18

3.7   Back-up control plan	18

3.8   ERCOT Black-Start Plan	19

3.9   Computer and communication systems Real-Time availability and systems security	19

3.10  Voltage and Reactive Control Performance monitoring	19

4	Ancillary Services Monitoring Program	20

4.1  Regulation Control Performance	20

4.2  Hydro responsive testing, for QSEs	20

4.3  Black Start Service requirements, for TSPs, QSEs and Generation Resources	20

4.4  Resource-specific Responsive Reserve performance for QSEs and  Resources	20

4.5  Voltage and Reactive support performance for QSEs and Generation Resources	21

4.6  Constant Frequency Control	21

4.7  Resource-specific Non-Spinning Reserve	21

- Objectives

This document will identify the performance and market monitoring reports ERCOT operations will be producing and what data is needed to produce these reports.  The context of this document comes from the ERCOT Nodal Protocols, Section 8: Performance Monitoring and Compliance.

# 1	QSE/Resource Monitoring Program

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

- ERCOT shall develop a TAC-approved QSE/Resource monitoring Program to be included in the Operating Guides. The metrics developed by ERCOT and approved by TAC must include the provision of this Subsection.

- 1.1	Net dependable real power capability testing

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(a) Net dependable real power capability testing for QSEs and Resources

ERCOT shall produce reports annually which present the test results provided by the QSE.  The QSE shall provide these results to ERCOT upon completion of Net Dependable Capability testing.

- 1.2	Reactive testing

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(b) Reactive testing, for Generation Resources

The QSE shall provide Unit name, QSE, Date, time, tested generation capability, reported time and CURL and URL information. ERCOT shall produce reports annually which describe when the last reactive capability test took place, what the MW and MVAR outputs were during the test period, how long the output was held, and a minimum of 4 MW/MVAR pairs describing the CURL of the unit.  This data shall be provided by the QSE to ERCOT when reactive testing indicates a change in unit capability and at a minimum of once every 2 years.

- 1.3 	Real-Time data

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

- (c) Real-Time data for QSEs:

- (i) 	Telemetry standards;

- (ii) 	Communication system;

(iii)	Operational Data Requirements required under Section 6.5.5.2, Operational Data Requirements.

ERCOT shall produce reports describing real time performance of QSE’s.  These reports shall include measurements of telemetry availability and comparison with the performance metrics established annually by the TAC.

(1)	Telemetry Standards

(a)	ERCOT shall produce reports as required describing compliance with the protocols and the TAC approved Telemetry Standards for QSEs.  Included in these reports shall be:

(i)	Quarterly telemetry availability statistics; and

(ii)	Critically important telemetry availability statistics.

(2) 	Communication Systems

ERCOT shall produce reports describing the percentage availability of each participants ICCP data link to ERCOT.

(3) 	Operational Data Requirements

ERCOT shall produce reports for the Operational Data Requirements as listed in the Protocols.

(a) 	ERCOT shall produce reports describing Resource Telemetry availability, generator status, and Generator Net energy and capability availability.

(b)	ERCOT shall produce reports for DSR monitoring.  DSR Load signal failures greater than 5 minutes will be recorded.  Reports shall be generated describing the monthly availability performance.

- 1.4    Compliance with Verbal Dispatch Instructions

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

- (f)	 Compliance with Dispatch Instructions, for QSEs;

ERCOT shall produce reports describing QSE and Resource Verbal dispatch instruction performance.

- 1.5   Supplying and validating data for generator models, aka CIM Performance Monitor

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

- (i)	 Supplying and validating data for generator models, as requested, for Generation Resources;

ERCOT shall record when the Resource Asset Registration Form (RARF) was received, and when dynamic response information is received at ERCOT.  ERCOT shall create reports to present this data which identifies when updates were requested by ERCOT and when such was received from the respondent. This report will be posted to the secure area of MIS.

CIM Performance Monitor

(1)	ERCOT shall prepare monthly reports summarizing the performance of the CIM modeling process.  Included in these reports shall be:

(a)	Number of NOMCRs submitted by QSE;

(b)	Number of invalid NOMCRs rejected by ERCOT and returned to the QSE;

(c)	Number of resource changes proposed for effect during the month period; and

(d) 	Number of interim reports submitted to the PUCT accumulated by QSE.

(2)	ERCOT shall post reports showing individual QSE performance on the MIS Secure area.

- 1.6  Resource Outage Scheduling and Reporting

(1)	       Outage Reporting

(a)	Outage requests submitted or updated in the ERCOT Outage Scheduler greater than 11 months in advance.

(b)	Outage requests submitted or updated in the ERCOT Outage Scheduler 30 days or more in advance of the desired outage date;

(c)	Outage requests submitted or updated in the ERCOT Outage Scheduler 8 days or more in advance and less than 30 days in advance of the desired outage date; and

(d) 	Outage requests submitted or updated in the ERCOT Outage Scheduler less than 8 days in advance of the desired outage date.

(e)	Outage requests submitted less than 3 days in advance.

(f)   	Outages requested, approved, and canceled by outage type;

(g)	Outages completed early;

(h) 	Outages which extended past their initially requested time frame; and

(i)	Outages requiring forced extension.

- 1.7  Generation under-frequency relay coordination

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(m)	Generation under-frequency relay coordination as specified in the Operating Guides for Generation Resources

ERCOT shall survey generation resources that trip during frequency deviations between 59.8 and 59.4 Hz.  ERCOT shall produce a report listing units which went offline while frequency was in this range without an approved outage.

- 1.8  Backup control for Resource Energy Deployment

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

- (n)	The backup control for Resource energy deployment due to loss of communication with ERCOT, to be tested by ERCOT randomly at least once a year for QSEs with Resources

- ERCOT shall produce a report identifying the date of backup control testing and the success or failure of that test.  Results shall be provided by the QSE to ERCOT at the completion of all tests.  Backup control tests should be conducted at least annually.

- 1.9  Qualified staffing requirement

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(p)	Twenty-four hours per day, seven days per week qualified staffing requirement, as described in the Operating Guides, for QSEs

A QSE shall maintain a 24x7 scheduling center with qualified personnel with the authority to commit and bind the QSE.  ERCOT shall report to the TRE, instances of suspected non-performance. .

- 1.10  Automatic Voltage Regulator (AVR) and Power System Stabilizer (PSS)

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(q)	Automatic Voltage Regulator (AVR) and Power System Stabilizer (PSS) requirements, for QSEs and Generation Resources

ERCOT shall record QSE provided AVR and PSS test reports which will include:

(a) 	The minimum and maximum excitation limiters;

(b)	Volts/hertz settings;

(c)	Gain and time constants;

(d)	Type of voltage regulator control function;

(e)	Date tested;

(f)	Voltage regulator control setting;

(g)	PSS tuning parameters; and

(h)	PSS control setting.

ERCOT shall produce a report that will identify resources that have not submitted test reports within the last 60 months.

- 1.11  Current Operating Plan metrics, for QSEs

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(t)	Current Operating Plan metrics, for QSEs

(1) 	ERCOT shall report the number of times that telemetered limits significantly disagree with the COP limits as captured in a snapshot 5 minutes before the hour begins.

(2) 	ERCOT shall report when resources have not submitted a COP for the next 7 Operating Days.

(3) 	ERCOT shall report when the capacity of Resources in the Operating Day COP is not sufficient to supply AS requirements.

(4) 	ERCOT shall report when an Energy Offer Curve or Output Schedule is not provided for a Generation Resource that is scheduled on-line in COP for the next hour.

- 1.12  Testing and performance of governor for Generation Resources

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(u)	Testing and performance of governor under the Operating Guides, for Generation Resources

ERCOT shall monitor governor response performance for governor frequency and report governor response performance against the criteria specified in the operating guides.

# 2	TSP Monitoring Program

Reference:  Protocol Section 8.3,TSP Performance Monitoring and Compliance

(1)	ERCOT shall develop a TAC-approved TSP monitoring Program to be included in the Operating Guides for TSPs.

- 2.1  Rating Methodology Reporting Program

Reference:  Protocol Section 8.3,TSP Performance Monitoring and Compliance

(1)(a)  Transmission Element rating calculations

TSPs shall submit their equipment rating methodology to ERCOT in the first quarter of each year or provide notice that the rating methodology has not changed since the previous year.  ERCOT shall keep records describing the rating methodologies of each TSP in ERCOT. ERCOT will prepare an annual report summarizing these rating methodologies and submit this information to the TRE.  The TRE will post this information in accordance with NERC rules.

- 2.2  Real-time data Monitor

Reference:  Protocol Section 8.3,TSP Performance Monitoring and Compliance

(1)(b) Real-Time data:

(i)	Meeting telemetry standards, including the installation of new measurement equipment and the accuracy of measurements

(ii)	Communications system availability

ERCOT shall produce reports describing real time performance of TSPs.  These reports shall include measurements of telemetry availability and comparison with the performance metrics established annually by the TAC.

(1)	Telemetry Standards

ERCOT shall produce reports as required describing compliance with the Protocols and the TAC approved Telemetry Standards for TSPs.

- 2.3  Outage Scheduling and Reporting

Reference:  Protocol Section 8.3,TSP Performance Monitoring and Compliance

(1)(c)  Outage scheduling and coordination; TSP Outage planning and scheduling statistics must have less weight the further out these statistics are from the Planned Outage date

(1)	       Outage Reporting

(a)	Outage requests submitted or updated in the ERCOT Outage Scheduler greater than 11 months in advance.

(b)	Outage requests submitted or updated in the ERCOT Outage Scheduler 30 days or more in advance of the desired outage date;

(c)	Outage requests submitted or updated in the ERCOT Outage Scheduler 8 days or more in advance and less than 30 days in advance of the desired outage date; and

(d) 	Outage requests submitted or updated in the ERCOT Outage Scheduler less than 8 days in advance of the desired outage date.

(e)	Outage requests submitted less than 3 days in advance.

(f)   	Outages requested, approved, and canceled by outage type;

(g)	Outages completed early;

(h) 	Outages which extended past their initially requested time frame; and

(i)	Outages requiring forced extension.

2.4  CIM Performance Monitor

Reference:  Protocol Section 8.3,TSP Performance Monitoring and Compliance

(1)(d)  Compliance with model update requirements, including provision of network data in CIM compatible format and consistency with the Transmission Element naming convention developed in accordance under Section 3, Management Activities for the ERCOT System.

(1)	ERCOT shall prepare monthly reports summarizing the performance of the CIM modeling process.  Included in these reports shall be:

(a)	Number of NOMCRs submitted during the month by the TSP;

(b)	Number of invalid NOMCRs rejected by ERCOT and returned to the TSP;

(c)	Number of transmission changes proposed for effect during the month period; and

(d) 	Number of interim reports submitted to the PUCT accumulated by TSP.

(2)	ERCOT shall post reports showing individual TSP performance on the MIS Secure area.

- 2.5  Compliance with Verbal Dispatch instructions monitoring

Reference:  Protocol Section 8.3,TSP Performance Monitoring and Compliance

- (1)(h)  Compliance with Dispatch Instructions

ERCOT shall produce reports describing TSP Verbal dispatch instruction performance.

# 3 	ERCOT Monitoring Program

Reference:  Protocol Section 82,ERCOT Performance Monitoring and Compliance

(1)	TAC, or a subcommittee designated by TAC, shall continually review the WEMM’s assessments of ERCOT’s operations and ERCOT’s performance in controlling the ERCOT control Area according to requirements and criteria established by the Operating Guides and NERC policy and standards operating of Control Areas (see additional 3.* sections)

## 3.1  Transmission Control

Reference:  Protocol Section 8.2,ERCOT Performance Monitoring and Compliance (In Part)

(2)(a)  Transmission control:

(i)	Transmission system availability objectives

(iii)	NERC transmission metrics (e.g., monitoring and managing rated paths)

(iv)	Other transmission monitoring and control metrics

(v)	Metrics describing how to minimize uplift to markets caused by transmission operations

(vi)	Metrics describing performance of the State Estimator.

ERCOT shall prepare a report describing availability of every transmission line and autotransformers operated at voltages greater then 60 kV.

(a) 		ERCOT shall prepare a monthly report on the following:

(i)	Transmission equipment ratings that are exceeded;

(ii)		Violations of system operating limits [power transfer (Generic Constraint)] for over ten minutes; and

(iii)		Violations of power transfer (Generic Constraint) operating limits following a contingency for over thirty minutes.

(3)		Other transmission monitoring and control metrics

(a)		ERCOT shall prepare reports describing:

(i)	The number of forced outages by transmission provider by month;

(ii)	Any overload detected during the month, the time duration of the loading above normal operating limit, the (average) operating limit exceeded, if the element was dynamically rated, and the average percentage loading.

(b)		ERCOT shall post on the MIS Secure Area its ability to process TSP outage requests in accordance with the following table:

| Amount of time between the request for approval of the proposed Outage and the scheduled start date of the proposed Outage: | ERCOT shall approve or reject no later than: |
| --- | --- |
| Three days | 1800 hours, two days before the start of the proposed Outage |
| Between three and eight days | 1800 hours, three days before the start of the proposed Outage |
| Between nine days and 45 days | Four days before the start of the proposed Outage |
| Between 46 and 90 days | Forty days before the start of the proposed Outage |
| Greater than 90 days | Seventy-five days before the start of the proposed Outage |

(c)		ERCOT shall post on the MIS Secure Area it’s ability to send TSP’s Rejection Notices that includes:

(1)	Specific concerns causing the rejection

(2)	Possible remedies or transmission schedule revisions

(3)	A copy of the ERCOT Case Study for review by the TSP

(d)		ERCOT shall post on the MIS Secure Area, by TSP, the number of Withdrawals of Approved Outages.

(4)		Minimizing uplift to markets caused by transmission operations

(a)		ERCOT shall report monthly on:

(i)	RUC charges uplifted;

(ii)	Voltage support charges uplifted; and

(iii)	Uplift charges due to verbal dispatch instructions (instructed deviation from base point) on a monthly basis.

(5)		State Estimator Performance Metrics

ERCOT shall report State Estimator performance in accordance with the Protocols and the TAC approved standard and post such on the MIS Secure Site.

## 3.2  Resource Control

Reference:  Protocol Section 8.2,ERCOT Performance Monitoring and Compliance (In Part)

(2)(b) Resource control:

(i)	Regulation control metrics

(a)	NERC control performance

(b)	Average sum of Reg-Up and Reg-Down energy near zero

(c)		Total amount of Reg-Up energy deployed and the total amount of Reg-Down energy deployed in each Settlement Interval.

(ii)	Metrics for Reserve monitoring as described in Section 8.1, QSE/Resource Performance Monitoring and Compliance

(iii)	Metrics describing RUC commitments and deployments

(iv)	Metrics describing the performance of Dynamically Scheduled Resources

(v)	Metrics describing conflicting instructions to Generation Resources from interval to interval

(vi)	NERC generation control metrics for the ERCOT Control Area (e.g., CPS, and DCS or their successors)

(vii)	Metrics describing the overall Resource response to frequency deviations in the ERCOT Region

(1) 		Resource Control Metrics:

- Average sum of Reg-Up and Reg-Down energy

ERCOT shall report for each month the difference between the 15-minute average monthly sum of Reg-Up energy deployed and the 15-minute average monthly sum of Reg-Down energy deployed for each 15-minute interval of the day.

(b)		Total Reg–Up and Reg-Down per interval

ERCOT shall develop a monthly report detailing the total amount of Reg-Upenergy deployed in the settlement interval and the total amount of Reg-Down energy deployed for each settlement  interval of the day.

(2)	Reserve Monitoring

ERCOT shall prepare reports describing amounts of time that it operated without sufficient operating reserves as defined in the Protocols.

(3)		RUC commitments and deployments

For each month, ERCOT shall report, Generation Resources committed in the RUC process, including Resource name, intervals deployed, process in which the Resource was committed, and any deviation from Resources committed and actual Resource performance.

(4)		DSR Performance

ERCOT shall produce reports for DSR Load signal failures of more than five minutes.

(5)		Conflicting Instructions to Generation Resources for interval to interval

ERCOT shall record instances of dispatch instructions to Resources not providing Regulation Service in which there is an inflection in the instruction for 4  consecutive SCED intervals for validation and review.  Inflection is defined as a reversal in ramp direction between consecutive intervals.

(6)		ERCOT Wide Frequency Response to measurable frequency events

ERCOT shall report monthly the resulting system performance for each measurable frequency event.

## 3.3  Load Forecasting

Reference:  Protocol Section 8.2,ERCOT Performance Monitoring and Compliance (In Part)

2(c)  Load forecasting;

(i)	The accuracy of each day’s load forecast posted at 0600 in the Day-Ahead of the Operating Day as compared with the actual ERCOT load for each hour of the Operating Day; and

(ii)	Accuracy of the Load forecast used for Day-Ahead RUC compared to the actual ERCOT Load for each hour of the Operating Day.

(iii)	The accuracy of the Load forecast for the following items compared to the average of the State Estimator Load at each Electrical Bus for each hour:

(a)	Hourly Load forecast used in the Day-Ahead RUC by Load Zone;

(b)	Hourly Load forecast used in the Day-Ahead RUC by Weather Zone;

(c)	Hourly Load forecast used in the Hourly RUC by Load Zone;

(d)	Hourly load forecast used in the Hourly RUC by Weather Zone;

(e)	the accuracy of the Load forecast used in the Day-Ahead RUC for the largest MW and MVA differences between the hourly Bus Load Forecast and the Real-Time Load at each Electrical Bus, by Load Zone; and

(f)	The accuracy of the Load forecast used in the Day-Ahead RUC for the largest MW and MVA differences between the hourly Bus Load Forecast and the Real-Time Load at each Electrical Bus, by Weather Zone.

ERCOT shall report the Mean Absolute Percent Error (MAPE) each month for the following:

(a) 		The accuracy of each day’s System Load forecast posted at 0600 in the Day-Ahead of the Operating Day as compared with the actual average ERCOT Load for each hour of the Operating Day;

(b)		Accuracy of the System Load forecast used for Day-Ahead RUC compared to the actual average ERCOT Load for each hour of the Operating Day; and

(c) 		The accuracy of the Load forecast for the following items compared to the average of the State Estimated Load at each Electrical Bus with a modeled Load for each hour:

(i)		Hourly Load forecast used in the Day-Ahead RUC by Load Zone;

(ii)		Hourly Load forecast used in the Day-Ahead RUC by Weather Zone;

(iii)	Hourly Load forecast used in the Hourly RUC by Load Zone;

(iv)	Hourly Load forecast used in the Hourly RUC by Weather Zone;

(v)		The accuracy of the Load forecast used in the Day-Ahead RUC for the largest MW and MVA differences between the hourly Bus Load Forecast and the Real-Time Load at each Electrical Bus, by Load Zone; and

(vi)	The accuracy of the Load forecast used in the Day-Ahead RUC for the largest MW and MVA differences between the hourly Bus Load Forecast and the Real-Time Load at each Electrical Bus, by Weather Zone.

## 3.4   System Operating Constraints

Reference:  Protocol Section 8.2,ERCOT Performance Monitoring and Compliance (In Part)

(2)(d)  System Operating Constraints:

(i)	Comparison of system operating limits identified as constraining limits in the Day-ahead Market to system operating limits identified as constraining limits in the Real-Time Market;

(ii)	Comparison of system operating limits identified as constraining limits in the Hourly RUC to system operating limits identified as constraining limits in the Real-Time Market;

(iv)	Comparison of system operating limits identified as constraining limits in the Day-ahead RUC to the level the corresponding system parameter was operated in the Real-Time Market;

(v)	Comparison of system operating limits identified as constraining limits in the    Hour-Ahead Market to the level the corresponding system parameter was  operated in the Real-Time Market;

ERCOT shall report the following comparisons with respect to Real-Time operations as they occur:

(a) 	Three consecutive days of a particular congestion constraint (constraints passed to SCED from NSA) in Real-Time not identified in the Day Ahead market;

(b)  	Two consecutive hours of a particular congestion constraint in Real-Time not identified in the HRUC process; and

(c)	Two consecutive days of a particular congestion constraint in Real-Time not identified in the DRUC process;

## 3.5   Network Operations Model update implementation performance

Reference:  Protocol Section 8.2,ERCOT Performance Monitoring and Compliance (In Part)

(2)(f)  Performance in implementing network model updates

ERCOT shall report monthly on the Network Operations Model update implementation performance.  The report shall include the total number of Network Operations Model Change Requests (NOMCR) submitted, approved, rejected, withdrawn and any other status contained in the Protocols or defined by ERCOT.  The report shall also include the number of Interim Updates and reasons provided (previously included in Protocol required PUC reports) for the Interim Updates provided by the Market Participants.   The timeline defined in the Protocols shall be used in the performance evaluation.  The report should itemize any non-Generation Resource Transmission Element whose limits were changed outside of the NOMCR process.

## 3.6	Network Operations Model validation

Reference:  Protocol Section 8.2,ERCOT Performance Monitoring and Compliance (In Part)

(2)(g)		Network Operations Model validation, by comparison to other appropriate models or other methods;

ERCOT shall report Network Operations Model problems and subsequent update requests identified after the model update submittal deadline described in protocol or during the test periods defined in the Protocols section 3.10.1.  The report shall include the number of updates submitted, the number accepted as submitted, and the number returned for correction.

## 3.7	Back-up control plan

Reference:  Protocol Section 8.2,ERCOT Performance Monitoring and Compliance (In Part)

(2)(h)		Back-up control plan

ERCOT shall report by the end of the 1st quarter of each year:

(a)	QSEs/TSPs which have approved back-up procedures on file with ERCOT and the date that they were last updated; and

(b)	The status of ERCOT’s back-up plan and the status of back-up training for QSEs/TSPs.

ERCOT shall confirm a staffing plan is part of these procedures.

## 3.8	ERCOT Black-Start Plan

Reference:  Protocol Section 8.2,ERCOT Performance Monitoring and Compliance (In Part)

(2)(i)		Written Black-Start plan

ERCOT shall produce an annual Black Start Plan.

## 3.9	Computer and communication systems Real-Time availability and systems security

Reference:  Protocol Section 8.2,ERCOT Performance Monitoring and Compliance (In Part)

(2)(k)		Computer and communication systems Real-Time availability and systems security

ERCOT shall report monthly the performance of its computer and communications systems as compared to its standards.

- 3.10  Voltage and Reactive Control Performance monitoring

ERCOT shall monitor voltage violations, N-1 voltage violations, reactive reserves, and import margins to voltage constrained areas not resolved within 15 minutes.  ERCOT shall produce reports for each TSP summarizing:

(a)	Number of real-time voltage violations and metrics describing duration of such violations;

(b)	Number of N-1 voltage based security violations and metrics describing duration of such violations;

(c)	Metrics describing reactive reserve in voltage constrained areas; and

(d)	Metrics describing N-1 import limitations in voltage constrained areas.

# 4	Ancillary Services Monitoring Program

Reference:  Protocol Section 8.1.2,QSE Ancillary Service Performance Standards (In Part)

… ERCOT shall develop a TAC-approved Ancillary Service monitoring program to evaluate the performance of QSEs and Resources providing Ancillary Services

- 4.1  Regulation Control Performance

Reference:  Protocol Section 8.1,QSE/Resource Performance Monitoring and Compliance (In Part)

(3)     ERCOT shall monitor the following categories of performance and compliance:

(e)	Regulation control performance, for QSEs and as applicable, Resource-specific performance

ERCOT shall produce monthly and yearly reports of QSE and Resource specific regulation performance metrics as required in the Protocols.

- 4.2  Hydro responsive testing, for QSEs

Reference:  Protocol Section 8.1,QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(g)	Hydro responsive testing, for QSE’s

ERCOT shall produce reports of Hydro responsive tests and verify results submitted by QSE for qualification.

- 4.3 Black Start Service requirements, for TSPs, QSEs and Generation Resources

Reference:  Protocol Section 8.1,QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(h)	Black Start Service requirements, for QSEs and Generation Resources

ERCOT shall produce reports containing record(s) of if and when a black start procedure was last received at ERCOT including the QSE or TSP name and date procedure received.  The QSEs participating in black-start service and TSPs shall provide an updated black start procedure to ERCOT  by the end of the 1st quarter and when the procedure has changed.

- 4.4  Resource-specific Responsive Reserve performance for QSEs and  Resources

Reference:  Protocol Section 8.1,QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(k)	Resource-specific Responsive Reserve performance for QSEs and Resources

ERCOT shall produce reports detailing Resource specific Responsive Reserve performance, for both capacity deployment and energy delivered as required in the Protocols.

## 4.5 Voltage and Reactive support performance for QSEs and Generation Resources

Reference:  Protocol Section 8.1,QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

(l)	Voltage and Reactive support performance for QSEs and Generation Resources

(1)	ERCOT shall produce reports of generators operating continuously outside a 0.98 power factor either leading or lagging for over 4 consecutive hours.

(2)	ERCOT shall report each voltage set point instruction issued to generating units including the date, the time, and the instruction.

(3)	ERCOT shall report when an On-Line generator’s AVR is not in service.

(4)	ERCOT shall report Verbal Dispatch instructions regarding voltage instructions on a monthly basis.

## 4.6 Constant Frequency Control

Reference:  Protocol Section 8.1.2.2.2,Regulation service (In Part)

(2)	A QSE shall demonstrate to ERCOT that they have the ability to switch control to constant frequency operation as specified in the Operating Guides using telemetry at the QSE’s control center.

ERCOT shall survey QSEs qualified to provide Regulation Services to attest that the QSEs have the capability to operate in constant frequency control mode.

- 4.7  Resource-specific Non-Spinning Reserve

Reference:  Protocol Section 8.1, QSE/Resource Performance Monitoring and Compliance (In Part)

(3)	ERCOT shall monitor the following categories of performance and compliance:

- (o)	Resource-specific Non-Spinning Reserve (Non-Spin) performance, for QSEs and Resources

Non-Spin Resource specific summary reports describing Non-Spin deployed by Resource, and the Resource response after 30 minutes shall be produced by ERCOT.  Each Resource shall meet at all times the Non-Spin energy deployment criteria as described in the Protocols.