COMS

Registration Requirements

(B2) v3.0

Revision History

| Date | Version | Description | Author(s) |
| --- | --- | --- | --- |
| 11/1/2006 | 0.01 | Initial Draft | Jamison Roof |
| 11/7/2006 | 0.02 | Created Business Process/Functional Requirements | Art Deller |
| 11/8/2006 | 0.03 | Minor changes | Jamison Roof |
| 11/10/2006 | 0.04 | Incorporated suggested modifications to proposed Scope | Jamison Roof |
| 11/14/2006 | 0.05 | Added requirements for existing functionality | Jamison Roof |
| 11/15/2006 | 0.06 | Revised and refine requirements, added document references, and updated MP registration diagram. | Art Deller, Jamison Roof |
| 11/17/2006 | 0.07 | Accepted all changes and made a few format and grammatical changes. | Art Deller |
| 11/18/2006 | 0.08 | Performed minor formatting, updated Activity Map diagram, included document references | Jamison Roof |
| 11/21/2006 | 0.09 | Clarified scope, | Ted Hailu |
| 11/22/2006 | 0.091 | Released to TPTF | Art Deller |
| 1/23/2007 | 1.0 | V1.0 posted | Art Deller |
| 3/27/2007 | 1.1 | Removed functional requirements for Resource Parameters. Requirements moved to MMS. Re-numbered FRxx to FR57. | Art Deller |
| 5/17/2007 | 1.1 | Responded to comments | Art Deller |
| 9/18/07 | 2.01 | Revised to conform to Baseline 2 requirements | Art Deller |
| 10/4/2007 | 3.0 | TPTF Approval | Raj Chudgar |

Table of Contents

1	Registration	3

1.1	Proposed System Scope	3

1.1.1	New Functionality In Scope	3

1.1.2	Existing Functionality In Scope	4

1.1.3	Out of Scope - Covered in other Requirements Documents	4

1.2	Document Conventions	6

1.3	Document Assumptions	6

2	Functional Requirements	7

2.1	New Functionality	8

2.1.1	Assumptions	8

2.1.2	Requirements	9

2.2	Existing Functionality	11

2.2.1	Assumptions	11

2.2.2	Entity Application and Agreements Requirements	11

2.2.3	Resource Registration Requirements	13

2.2.4	Service Filing Requirements	15

2.2.5	Notice of Change of Information Requirements	15

2.2.6	EPS Metering Requirements	16

2.2.7	NOIE Identification and Meter Point Registration Requirements	16

2.2.8	QSE Acknowledgement Requirements	16

3	Supplementary Requirements	17

3.1	Performance Requirements	17

3.2	Legal and Regulatory requirements	17

3.3	System Security Requirements	17

3.4	Back up and Recovery Requirements	17

3.5	Availability and Redundancy Requirements	17

3.6	Maintainability Requirements	17

4	Protocol Coverage	17

# Registration

## Proposed System Scope

ERCOT plans to implement a Nodal market by 2009.  The Nodal Protocols describe the various requirements for the registration and maintenance of registration information for Market Participant entities and their assets. This document defines the requirements for Registration of Market Participants in the ERCOT systems as defined in the August 2007 version of the ERCOT Nodal Protocols. For the purposes of this document, Registration is the process of receiving, recording and maintaining data on Market Participant entities and their assets needed for the initiation and ongoing operation of transactions in the ERCOT market. The processes of transmittal of registration data within and throughout the ERCOT sub-systems are outside the scope of these requirements. In addition, this document will cover Generation and Load Resource Asset Registration functionality as described in Sections 3 and 6 of the Nodal Protocols. Market Participant Registration is largely addressed in Section 16 (“Registration and Qualification of Market Participants”). However, additional registration requirements are found in sections 3, 4, 5, 6, 7, 8, 9, and 10 and are detailed in this specification.

The Registration System will provide registration services for:

- Qualified Scheduling Entities (QSEs)

- Load Serving Entities (LSEs)

- Resource Entities

- Resources

- Generation Resources

- Load Resources

- Congestion Revenue Right Account Holders (CRR Account Holders)

- Transmission and Distribution Service Providers (TDSPs)

- Municipally Owned Utilities (MOUs)

- Electric Cooperatives

- Renewable Energy Credit Account Holders

### New Functionality In Scope

The Registration System will require new functionality to meet the Nodal Protocols.  Requirements for this new functionality are detailed in this specification.  Changes to the current Registration System that are required per the Nodal Protocols include:

- Addition of CRR account holder registration

- Capturing relationships of CRR Account Holders to other Market participant types

- Capturing qualification status of QSEs and CRR Account Holders

- Capturing ancillary service qualification of Resources

- Capturing relationships between QSEs and Sub-QSEs

- Modifications to Resource Registration

- Resource Parameter

- Resource Verifiable Costs

### Existing Functionality In Scope

Much of the functionality required by the Registration System can be provided by existing systems. Requirements for this existing functionality are specified without going into great detail. Existing functionality includes:

- EPS Metering documentation and processing

- LSE Applications and Agreements

- QSE Applications and Agreements

- QSE Acknowledgements

- QSE Credit Applications

- QSE Service Filings

- Resource Entity Applications and Agreements

- TDSP Applications and Agreements

- NOIE (Non-Opt-In Entity) Identification and Meter Point Registration Forms

- Notices of Change of Information

### Out of Scope - Covered in other Requirements Documents

Some new and existing functionality that may be considered part of the Registration System will not be included in this specification. Customer Registration functionality, as described in Protocol Section 15, Customer Registration, is associated with the operation of the ERCOT retail market, and is beyond the scope of this document. The following functionality is related to the Registration System, and is addressed in other requirement specifications:

- Network Operations Model Change Requests (NOMCR), as specified in NMMS_REQUIREMENTS

- Outage Scheduler data, as specified in TN.EMS.61C01.OUTAGEEVALUATION

- Resource Parameters, as specified in MMS requirements documents.

LEVEL3 SUB-PROCESS MAP FOR REGISTRATION

Brief Description:  The following sub-process flow is part of the Capability “Provide Wholesale Market Services.” It is a high level process view of registering and validating a Market Participant Applicant where “I” represents an input, “P” represents a process, and “1” is representative of either entity or asset data:

## Document Conventions

All text cited from the Nodal Protocols is in italic font.

All requirements are identified using FR1, FR2, etc.

All dependencies are identified using D1, D2, etc.

All assumptions are identified using A1, A2, etc.

All referenced documents are designated as such by the use of capital letters.

The term “Registration System” is used throughout this document to represent any ERCOT internal application that is used to capture and store data that is required by any registration process.  It does not refer only to Siebel.

## Document Assumptions

A1)	The term “CRR” will be used to describe the “Congestion Revenue Right”.

A2)	The term “CRR Account Holder” will be used to describe the Congestion Revenue Rights Account Holder.

A3)	The term “CRR Auction” will be used to indicate an auction hosted by ERCOT that will allow eligible CRR Account Holders to buy and sell CRRs.

A4)	The term “QSE” will be used to describe the Qualified Scheduling Entity.

A5)	The term “NOIE” will be used to describe the Non-Opt-In Entity.

A6)	The term “CRR Owner” will be used to describe the CRR Account Holder that owns one or more CRRs.

A7)	The term “WGR” will be used to describe “Wind Generation Resource”.

A8)	The term “MCFRI” will be used to describe “McCamey Flowgate Right”.

A9)	The term “USA” will be used to describe “User Security Administrator”.

A10) 	The term “MP” will be used to describe “Market Participant”.

# Functional Requirements

ACTIVITY MAP 1: Market Participant Registration Process

Market Participants must register with ERCOT, and complete the necessary and standard application forms and agreements posted on the MIS web portal. Upon receipt of this information (after it has been appropriately reviewed and approved by ERCOT Legal and ERCOT Credit Monitoring), Market Services will enter the Market Participant Account information into the Registration System. Upon successful completion of all Registration, agreements, and applicable qualification testing requirements, the MP will be issued a Production Digital Certificate.

## New Functionality

### Assumptions

A1) Verifiable costs, as specified in TN.COMS.63C01.VERIFIABLECOSTS.REQUIREMENTS.A will be submitted through the use of the Service Request process.

### Requirements

The ERCOT Registration System will provide the following new functionality:

| Req. # | Protocol Reference(s) | Protocol Coverage | Requirement |
| --- | --- | --- | --- |
| FR1 | 3.7, 16.2.3.3, 16.3.4, 16.5.4, 16.8.3.1, | Full | The Registration System must enable the timely update of information from each Market Participant provided to ERCOT in the application process. ERCOT will give confirmation of timely processing of registration information and inform the MPs when a registration change will become effective. Information collected will include, where appropriate: - Addresses - E-mail and Phone contact information - Officers, Directors, Authorized representatives, Credit Contacts, User Security Administrators. - List of Affiliates - Other information deemed necessary and included on Market Participant Registration documents. |
| FR2 | 5.6.1(2) | Full | The Registration System must be able to capture and store verifiable cost information submitted by each QSE per TN.COMS.63C01.VERIFIABLECOSTS.REQUIREMENTS.A, including; 1) Actual fuel cost to operate the unit at LSL 2) Variable operation and maintenance expenses 3) Nodal implementation surcharges to operate the unit at LSL 4) Seasonal heat rate, if applicable |
| FR3 | 6.5.5.2(8)(b) | Full | The Registration System must capture the list of possible Combined-Cycle Configurations for each Combined Cycle Resource. |
| FR4 | 7.5.3 (1)(a), 16.8 | Full | The Registration System must capture and track the qualification and registration of eligible CRR Account Holders. |
| FR5 | 3.13, 7.7.2 (3), 7.7.2 (5) | Partial | The Registration System must be able to distinguish Wind Generation Resources and identify them as eligible to receive MCFRIs if they are located inside the McCamey area. |
| FR6 | 8.1.2.1(1) | Partial | The Registration System must maintain the Ancillary Service qualification status of QSEs. |
| FR7 | 8.1.2.2 | Partial | The Registration System must maintain the Ancillary Service qualification status of Resources. |
| FR8 | 8.1.2.2 | Partial | The Registration System must be able to accept changes to Load Resource registration information, including changes to its capability to provide Ancillary Service. |
| FR9 | 8.1.2.2 | Partial | For entities representing specific Load Resources, the Registration System must be able to maintain and enforce a unique descriptor of the qualified Load Resource. |
| FR10 | 3.14.1, 8.1.2.2.2, 8.1.2.2.3, 8.1.2.2.4,8.1.2.2.5, 8.1.2.2.6 | Partial | The Registration System must be able to capture and track resource qualification, including the Resource’s capability to provide: 1) Regulation Service, 2) Responsive Reserve Service, 3) Non-Spinning Reserve, 4) Reactive power capacity, 5) Black Start Service capacity , and 6) Reliability Must Run Service. |
| FR11 | 16.2.1 (4) | Full | The Registration System must maintain relationships between QSEs and their Subordinate QSEs. |
| FR12 | 16.4 | Full | The Registration System must be able to distinguish between entities that are TSP, DSP and both TSP and DSP. |
| FR13 | 16.11 |  | The Registration System must maintain optional relationships between CRR Account Holders and any other Registered Market Participant. |
| FR14 | 16.11.6.1.6(1) | Full | The Registration System must maintain the qualification status of entities in the Registration System. The available status identifiers include: 1) Active 2) Pending 3) Provisional 4) Disqualified 5) Not Qualified |
| FR15 | 16.11.6.1.6(1) | Full | The Registration System must be able to designate a Counter-Party’s Rights as Suspended or Terminated. |
| FR16 | 16.11.6.1.6 | Partial | The Registration System must allow for the removal of an entity’s rights to conduct activities under the Protocols and will not prevent the entity from meeting its obligations for all costs and expenses incurred. |
| FR57 | 16.2.4 | Full | The Registration System will produce a list of current Qualified Scheduling Entities on a user definable schedule. |

## Existing Functionality

The following requirements apply to Registration System functionality that will be provided by existing systems. They cover entity applications, Resource registrations, and service filings.

### Assumptions

A1) Transmission Service Providers (TSP) and Distribution Service Providers (DSP) will continue to be called Transmission and/or Distribution Service Providers (TDSP) in the ERCOT system, with the Registration System able to distinguish between the different types.

A2) Any QSEs, LSEs, TDSPs, or Resource Entities that are in ERCOT Registration System when the nodal market is opened will not need to re-register.

A3) While Resources and their relationships to Resource entities will be carried over from the current Registration System, additional registration data such as Resource Parameters and verifiable cost information will be collected from applicable Resources.

A4) RMR verifiable costs are not covered in this document since this information is considered transactional. Entity Application and Agreements Requirements

| Req. # | Protocol Reference(s) | Protocol Coverage | Requirement |
| --- | --- | --- | --- |
| FR17 | 3.1.4.1 | Partial | The Registration System must hold a Single Point of Contact for each TSP or Resource Entity to submit outage plans and to coordinate Resource outages with primary and alternate means of communication. |
| FR18 | 4.4.4.3(2) | Full | The Registration System must accept submission of applications from QSEs requesting an Oklaunion exemption. |
| FR19 | 9.3, 9.6, 9.8, 9.10 | Partial | Remittance information must be accepted and maintained in order to remit payment to or draw payment from a Market Participant. The remittance information includes: 1) Account Number, 2) Bank Name, and 3) Electronic Transfer Instructions |
| FR20 | 16.2.1(1)(b), 16.3, 16.5, 16.8.3(a) | Full | Executed Market Participant Agreements must be maintained. |
| FR21 | 16.2.1 (1)(d), 16.2.3.1(1) (a), 16.8.3 (b), 16.11 | Full | The financial qualification status of QSEs and CRR Account Holders must be maintained. |
| FR22 | 16.2.1(1) (h), 16.8.3(b), 16.8.1(1), | Full | The financial information of QSEs and CRR Account Holders must be maintained. |
| FR23 | 16.2.1(6) | Full | The Registration System must treat sub-QSEs as individual QSEs. |
| FR24 | 16.2.3.1(1)(b), 16.3.1.1(2), 16.3.3(2), 16.5.1.1(1), 16.5.3, 16.6(3), | Full | Current and historical relationship information between Market Participants and between Market Participants and their assets must be maintained. Designation of a backup QSE by an LSE will also be maintained. |
| FR25 | 16.12 | Partial | The Registration System must record and maintain registration information on each Market Participant’s designated USA, and its optionally designated backup USA. |
| FR26 | 16.2.3.2, 16.3.4, 16.5.4, 16.8.3.1 | Full | The Registration System must provide the ability for registered Market Participants to update registration information provided to ERCOT. |
| FR27 | 16.1(3) | Full | The necessary Applications and Service Filing forms must be available on the MIS Public site. |
| FR28 | 16.2.1, 16.3, 16.4, 16.5, 16.6,16.7, 16.8.1, 16.9, 16.10 | Full | Market Participant Registration information provided during the Application and Service Filing Process must be maintained. |
| FR29 | 16.2.6.2 | Full | The Registration System must be able to record the qualification status of a QSE as an Emergency QSE or Vitual QSE. |
| FR30 | 16.3.5 | Full | The Registration System must be able to designate LSEs as located outside the ERCOT region. |
| FR31 | 16.5.1.2 | Full | The Registration System must be able to designate a Resource Entity as a hydroelectric All-Inclusive Resource and maintain the status of their waiver. |
| FR32 | 16.5.1.3 | Full | The Registration System must be able to designate a Resource Entity for block Load Transfer and maintain the status of the waiver. |
| FR33 | 16.2.2.1, 16.2.2.2(1), 16.2.2.2(2), 16.2.2.3(1), 16.2.2.3(2), 16.2.2.3(4), 16.3.2.1, 16.3.2.2(1), 16.3.2.2(2), 16.3.2.3(1), 16.3.2.3(2), 16.5.2.1, 16.5.2.2(1), 16.5.2.2(2), 16.5.2.3(1), 16.5.2.3(2), 16.5.2.3(3), 16.8.2.1, 16.8.2.2(1), 16.8.2.3(1), 16.8.2.3(2), 16.8.2.3(4) | Full | Entity applications for QSEs, LSEs, Resource Entities, and CRR Account Holders must be processed in the following times: 1) 3 business days to provide a written receipt, 2) 10 business days to provide a notification of a complete application, 3) 10 business days after an application is deemed complete to reject the application with a written explanation, and 4) 10 business days to provide any required agreements if the application is not rejected. |

### Resource Registration Requirements

| Req. # | Protocol Reference(s) | Protocol Coverage | Requirement |
| --- | --- | --- | --- |
| FR34 | 3.6(2) | Full | The Registration System must capture Load Resources’ registration and qualification information. |
| FR35 | 3.10.7.2(1) | Partial | The Registration System must be able to capture information necessary to describe each Generation Resource and Load Resource connected to the transmission system. |
| FR36 | 3.7.1.1(1) | Partial | The Registration System must be able to capture Resource Parameters submitted by each QSE for each Generation Resource. Generation Resource Parameters submitted by each QSE must be captured and stored, including: 1) Resource Name, 2) High Reasonability Limit, 3) Low Reasonability Limit, 4) Type of Resource, 5) QF Status, |
| FR37 | 3.7.1.1(2), 8.1.2.2(1) | Partial | The Registration System must be able to capture Seasonal Resource Parameters submitted by each QSE including: 1) Seasonal MW (Net Dependable Capability), |
| FR38 | 3.7.1.2 | Partial | The Registration System must be able to capture Resource Parameters submitted by each QSE for each Load Resource, including: 1) Resource Name, |
| FR39 | 3.13 | Partial | The Registration System must be able to capture Resource parameters from new and existing WGRs needed for the RPP forecasting process including: 1) Resource Name (facility), |
| FR40 | 3.14.1.1 | Partial | For Generation Resources, notices of suspension of operations must be captured and tracked. |
| FR41 | 3.14.1.6 | Partial | For every Generation Entity that owns or controls a Mothballed Generation Resource or an RMR Unit the Registration System must capture and store for every unit the estimated lead time for return to service and the generation capacity. |
| FR42 | 5.6.1 | Partial | The Registration System must capture and store the submission of unit-specific verifiable cost and must track the date of submission. |
| FR43 | 6.5.9.5.1(1) | Partial | BLTs must be registered as Resources and assigned a Resource ID. The Registration System must verify that applicable asset registration forms have been completed and metering design documentation has been provided. |
| FR44 | 6.5.9.5.1(2) | Partial | The designation of the LSE representing a BLT and its QSE affiliation must be captured. |
| FR45 | 3.8(1), 10.3.2.1 | Partial | For each Generation Resource splitting output into virtual generating units, the Registration System must: 1) designate this generator as “split,” 2) be able to capture all required ERCOT Facility registration documentation, 3) capture a splitting agreement containing a defined and fixed ownership percentage among the owning Entities, and 3) record a percentage allocation of the Resource (used to determine the capacity available at each virtual generator unit). |
| FR46 | 16.9, 16.10 | Full | Signed Standard Form Agreements must be maintained for entities providing RMR Service, Black Start Service and any other service that may be required by ERCOT. |

### Service Filing Requirements

| Req. # | Protocol Reference(s) | Protocol Coverage | Requirement |
| --- | --- | --- | --- |
| FR47 | 4.4.4.3(2) | Full | For QSEs that have requested an Oklaunion Exemption, the Registration System must be able capture and maintain separate QSEs (or sub-QSEs). |
| FR48 | 16.2.3.1 | Full | Service Filings for QSEs must be processed in the following times: 1) 3 business days to provide a written notice that the file has been received and 2) 10 business days to provide a notification that operation may commence or that the filing is incomplete. |
| FR49 | 16.2.3.1(1)(c) | Full | The Registration System must maintain the date by which the QSE proposes to start QSE activities. |

### Notice of Change of Information Requirements

| Req. # | Protocol Reference(s) | Protocol Coverage | Requirement |
| --- | --- | --- | --- |
| FR50 | 16.2.1(3), 16.2.3.3, 16.2.3.4, 16.3.3(1), 16.3.4, 16.5.4(1), 16.5.4(2), 16.8.1(2), 16.8.3.1 | Full | The Registration System must allow the update of registration information through a Notice of Change of Information submitted by the appropriate Market Participant. |
| FR51 | 16.2.3.4 | Full | In the event that a QSE provides written notice of intent to terminate representation of an LSE or Resource, the Registration System must be able to capture this change and provide appropriate notification to the affected parties. |
| FR52 | 16.5.4 (2) | Partial | The Registration System must maintain a record for each Resource Entity of the days that the identified capacity will not be available from a Generation Resource during the Peak Period. |

### EPS Metering Requirements

| Req. # | Protocol Reference(s) | Protocol Coverage | Requirement |
| --- | --- | --- | --- |
| FR53 | 10.4.2 | Partial | The Registration System must accept the following from TDSPs: 1) EPS Design Proposals, 2) EPS MDAS Configuration, 3) TDSP Cutover information for EPS Metering Points |
| FR54 | 16.5 | Full | The Registration System must maintain the date by which the Resource Entity proposes to start activities. |

### NOIE Identification and Meter Point Registration Requirements

| Req. # | Protocol Reference(s) | Protocol Coverage | Requirement |
| --- | --- | --- | --- |
| FR55 | 16.6(4) | Full | Each NOIE’s wholesale point of delivery must be assigned an ESI ID. The ESI ID must be assigned to an LSE. |

### QSE Acknowledgement Requirements

| Req. # | Protocol Reference(s) | Protocol Coverage | Requirement |
| --- | --- | --- | --- |
| FR56 | 16.3.3(2), 16.3.3(3), 16.5.3(2), 16.5.3(3) | Full | The Registration System must allow the update of QSE representation through a QSE acknowledgement form submitted by the registering entity. |

# Supplementary Requirements

## Performance Requirements

The Registration System is a High Availability system, essential to the Market and Commercial Operations of ERCOT. The Registration System must be able to sustain acceptable application performance.  Performance characteristics will be determined and detailed in the conceptual and detailed design.

## Legal and Regulatory requirements

All registration data must be saved in accordance with standard principles as dictated by the ERCOT Records Retention policy.

## System Security Requirements

The Registration System will only be accessed directly by internal ERCOT personnel.  The Registration System contains information considered Protected Information under the ERCOT Protocols and will be afforded the appropriate security measures.  Additional details will be provided in the Conceptual System Design.

## Back up and Recovery Requirements

All ERCOT IT Systems containing Registration data must have the data backed up on a daily basis and recoverable within 24 hours.

## Availability and Redundancy Requirements

The Registration System is critical to ERCOT business processes. It must be available 7/24.

## Maintainability Requirements

The ERCOT IT Systems containing Registration data (Siebel, Lodestar, MMS, NMMS, CRR and CMM) must be maintainable by ERCOT personnel, and must conform to ERCOT IT Operations standards. Protocol Coverage

| Protocol Sub-Section | Coverage by Requirements (Full/Partial) |
| --- | --- |
| 3.13 | Partial |
| 3.7 | Partial |
| 9.1 | Full |
| 9.3 | Partial |
| 9.6 | Partial |
| 9.8 | Partial |
| 10.3.2.1 | Partial |
| 10.4.2 | Partial |
| 16.1 | Partial |
| 16.11 | Partial |
| 16.12 | Partial |
| 16.3 | Full |
| 16.4 | Full |
| 16.5 | Full |
| 16.6 | Full |
| 16.7 | Full |
| 16.8 | Full |
| 16.9 | Full |
| 16.1(3) | Full |
| 16.11.6.1.6 | Full |
| 16.11.6.1.6(1) | Full |
| 16.2.1 | Full |
| 16.2.1(1) | Full |
| 16.2.1(1)(b) | Full |
| 16.2.1(1)(d) | Full |
| 16.2.1(1)(h) | Full |
| 16.2.1(3) | Full |
| 16.2.1(6) | Full |
| 16.2.2.1 | Full |
| 16.2.2.2(1) | Full |
| 16.2.2.2(2) | Full |
| 16.2.2.3(1) | Full |
| 16.2.2.3(2) | Full |
| 16.2.2.3(4) | Full |
| 16.2.3 | Full |
| 16.2.3.1 | Full |
| 16.2.3.1(1)(a) | Full |
| 16.2.3.1(1)(b) | Full |
| 16.2.3.1(2)(c) | Full |
| 16.2.3.1(2)(c) | Full |
| 16.2.3.2 | Full |
| 16.2.3.3 | Full |
| 16.2.3.4 | Full |
| 16.2.4 | Full |
| 16.2.6.3 | Full |
| 16.3.2.1 | Full |
| 16.3.2.2(1) | Full |
| 16.3.2.2(2) | Full |
| 16.3.2.3(1) | Full |
| 16.3.2.3(2) | Full |
| 16.3.3(1) | Full |
| 16.3.3(2) | Full |
| 16.3.3(2) | Full |
| 16.3.3(3) | Full |
| 16.3.4 | Full |
| 16.3.5 | Full |
| 16.5.1.1(1) | Full |
| 16.5.1.2 | Full |
| 16.5.1.3 | Full |
| 16.5.2.1 | Full |
| 16.5.2.2(1) | Full |
| 16.5.2.2(2) | Full |
| 16.5.2.3(1) | Full |
| 16.5.2.3(2) | Full |
| 16.5.2.3(3) | Full |
| 16.5.3 | Full |
| 16.5.3(2) | Full |
| 16.5.3(3) | Full |
| 16.5.4 | Full |
| 16.5.4(1) | Full |
| 16.5.4(2) | Partial |
| 16.5.4(2) | Full |
| 16.6(3) | Full |
| 16.6(4) | Full |
| 16.8.1 | Full |
| 16.8.1(1) | Full |
| 16.8.1(2) | Full |
| 16.8.2.1 | Full |
| 16.8.2.2(1) | Full |
| 16.8.2.3(1) | Full |
| 16.8.2.3(2) | Full |
| 16.8.2.3(4) | Full |
| 16.8.3(a) | Full |
| 16.8.3(b) | Full |
| 16.8.3.1 | Full |
| 3.1.4.1 | Partial |
| 3.10.7.2(1) | Partial |
| 3.14.1.1 | Partial |
| 3.14.1.12 | Partial |
| 3.14.1.6 | Partial |
| 3.6(2) | Full |
| 3.7.1.1(1) | Partial |
| 3.7.1.1(2) | Partial |
| 3.7.1.2 | Partial |
| 3.8(1) | Partial |
| 4.4.4.3(2) | Full |
| 4.4.4.3(2) | Full |
| 4.4.4.3(2) | Full |
| 5.6.1 | Partial |
| 5.6.1(2) | Partial |
| 6.5.5.2(8)(b) | Partial |
| 6.5.9.5.1(1) | Partial |
| 6.5.9.5.1(2) | Partial |
| 7.5.3 | Partial |
| 7.7.2 | Partial |
| 8.1.2.1 | Partial |
| 8.1.2.1(1) | Partial |
| 8.1.2.1(7) | Partial |
| 8.1.2.2(1) | Partial |
| 8.1.2.2(5) | Partial |
| 8.1.2.2.2 | Partial |
| 8.1.2.2.3 | Partial |
| 8.1.2.2.4 | Partial |
| 8.1.2.2.5 | Partial |
| 8.1.2.2.6 | Partial |