| PRR Number |  | PRR Title | EDS 3 LFC Test Settlement PRR - DRAFT |
| --- | --- | --- | --- |
| Date Posted |  |  |  |
| Protocol Section(s) Requiring Revision (Include Section No. and Title) |  | 6.8.2.3 Energy Payments, 7.4.3.1 Balancing Energy Up Service from a Specific Resource, 7.4.3.1 Balancing Energy Down Service from a Specific Resource |  |
| Requested Resolution (Normal or Urgent, and justification for Urgent status) |  |  |  |
| Revision Description |  | Modifies OOME and LBE payment formulas for EDS 3 LFC Phase 3 Test. |  |
| Reason for Revision |  | EDS 3 LFC Phase 3 Test is the full system test for SCED. This PRR will detail how ERCOT will settle the market during this test period. |  |
| Overall Market Benefit |  |  |  |
| Overall Market Impact |  |  |  |
| Consumer Impact |  |  |  |
| Credit Implications (Yes or No, and summary of impact) |  |  |  |
| Relevance to Nodal Market (Yes or No, and summary of impact) |  | Yes – EDS 3 LFC Phase 3 Test driven by nodal implementation |  |
| Nodal Protocol Section(s) Requiring Revision (Include Section No. and Title, and submit NPRR if applicable) |  | None |  |

Quantitative Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration and development of the Cost Benefit Analysis (CBA), please fill out each block below completely and provide as much detailed information as possible.  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Bill Barnes, Matt Tozer |
| E-mail Address | bbarnes@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6850 |
| Cell Number |  |
| Market Segment |  |

| Market Rules Staff Contact |  |
| --- | --- |
| Name |  |
| E-Mail Address |  |
| Phone Number |  |

Proposed Protocol Language Revision

6.8.2.3	Energy Payments

(1)	Whenever a Generation Resource is called for OOME Up or Zonal OOME Up, the QSE will receive any Resource Imbalance payment it is entitled to receive as specified in Section 6.8.1.13, Resource Imbalance.  In addition, an OOME payment adjustment will be made such that the resulting total payment to the Resource(s) is equal to the higher of either: (a) Resource Category Generic Fuel Cost (RCGFC) for the Resource(s); or the (b) MCPE for the Congestion Zone.

(2)	The calculation for Generation Resources OOME Up or Zonal OOME Up is as follows:

PEOOMUPiq 	=	SUM (PEOOMUPiuq)uq+ SUM (PEOOMUPivq)vq

Where:

PEOOMUPiuq	=	-1 * EOOMUPiuq * Max [(RCGFCc - MCPEiz), 0]

EOOMUPiuq	=	Max (0, Min ((MRiuq - OLiuq), IOOMUPiuq))

PEOOMUPivq	=	-1 * EOOMUPivq * Max [(RCGFCc - MCPEiz), 0]

EOOMUPivq	=	Max (0, Min ((MRivq – OLivq), NETUEQivq)) * OOMAGRivq

The equation below will be used to determine the Total OOM Energy Payments to be allocated to each QSE in Section 6.9.7.2, OOM Energy Charge.

PEOOMUPi 	=	SUM (PEOOMUPiu)  + SUM (PEOOMUPiv)

NETUEQivq	=	Max(0,((NETOOMUEQivq + NETLBEUQivq) – (NETOOMDEQivq + NETLBEDQivq))

NETOOMUEQivq 	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq 	=	Max(0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq	=	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

OOMAGRivq	=	[SUM(IOOMUPqui)u + SUM(IOOMDNqui)u] / [SUM(LBEUQqui)u + SUM(LBEDQqui)u + 
SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

c	Resource category

i	interval

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

q	QSE

MCPEiz	Market Clearing Price of Energy for that interval of the zone in which unit resides

PEOOMUPiuq	Out of Merit Energy Up Payment ($) for that interval per unit per QSE

PEOOMUPivq	Out of Merit Energy Up Payment ($) for that interval per Aggregated Unit per QSE

PEOOMUPiq	Out of Merit Energy Up Payment ($) for that interval per QSE

PEOOMUPi	Summation of Out of Merit Energy Up Payment ($) per interval for all QSEs in the market

EOOMUPiuq	OOM Up quantity deployed for that unit per interval per QSE

EOOMUPivq	OOM Up quantity deployed for that Aggregated Unit per interval per QSE

IOOMUPiuq	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMDNiuq	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRiuq	Meter Reading for that unit for that interval per QSE

MRivq	Meter Reading for that Aggregated Unit for that interval per QSE.

OLiuq	Resource Plan output level submitted by the QSE for the specific unit given the OOM instruction for that interval.

OLivq	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the specific unit given the OOM instruction for that interval.

RCGFCc	Resource Category Generic Fuel Cost for a specific category.

NETOOMUEQivq The net of all unit-specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivq	The net of all unit-specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq The net of all unit-specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq The net of all unit-specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value.

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

NETUEQiv	The overall net direction per the instructions given to the Aggregated Unit results in an up direction

OOMAGRiv 	The percentage of the aggregated instructions to the Aggregated Unit that represents the OOM Energy market.

- The Nodal Market Implementation requires Early Delivery System testing activities.  During the Early Delivery System 3 LFC Phase 3 test, the formula in section 6.8.2.3 (2)  will be modified in the following way:

IOOMUPiuq	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

IOOMDNiuq	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

LBEUQqui	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

LBEDQqui	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| IOOMUPiuq | MWh | OOM Energy Up Instructions—The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an OOME Up deployment. |
| IOOMDNiuq | MWh | OOM Energy Down Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an OOME Down deployment. |
| LBEUQqui | MWh | Local Balancing Energy Up Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with a Local Balancing Energy Up deployment. |
| LBEDQqui | MWh | Local Balancing Energy Down Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an Local Balancing Energy Down deployment. |
| BP y | MW | Base Point by interval—The Base Point generated from SCED test system for the Generation Resource for the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| y | none | A SCED interval in the Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| i | none | 15-minute Settlement Interval |
| u | none | Individual generation unit (including units within an Aggregated Unit) |
| q | none | QSE |

    - When a combined cycle plant receives SCED base points at the combined cycle configuration level, ERCOT will use telemetry based splitting percentages to allocate the OOME or Local Balancing Energy Instruction, as calculated in section (a) above, to each individual unit that makes up the configuration for each 15-minute settlement interval.

    - If after receiving the Initial Statement for the Operating Day on which there was EDS 3 LFC Phase 3 testing, a QSE does not agree with the updated Instructed Output level or payment for OOME or Local Balancing Energy, a QSE may submit a settlement dispute in accordance with the dispute process outlined in Section 9.5, Settlement and Billing Dispute Process.  In addition to the standard information required on the dispute form on the ERCOT Portal, the dispute should clearly indicate what is being disputed.  Verification of these claims must be submitted to ERCOT by the QSE or the Resource to allow resolution by the end of the dispute process for Settlement True-Up as defined in Section 9.2.6, True-Up Statement.  Upon verification of the data provided, ERCOT shall grant such disputes…add language for specific settlement intervals involved in the test

(3)	If ERCOT sends a Resource-specific Dispatch Instruction for OOME Up or Zonal OOME Up Service and the payment for OOME Service does not cover all costs of providing the Service, then that QSE may submit verifiable, additional costs directly attributable to the OOME Dispatch Instruction, which exceed the payment for OOME Service calculated pursuant to Section 6.8.2.3(2).  Verifiable costs are subject to the approved documentation requirements in Section 6.8.2.3(4)(b).  These costs may include the cost of exceeding swing gas contract limits, additional gas Demand costs set by the fuel supply or transportation contracts, nodal implementation surcharge, and any additional costs to purchase emissions credits or other costs incurred due to environmental regulations.  Verification of these costs must be submitted to ERCOT by the QSE or the Resource to allow resolution by the end of the dispute process for Settlement True-Up as defined in Section 9.2.6, True-Up Statement.

(4)	If a QSE requests cost-based recovery, the entire payment will be based solely on verifiable costs.  QSEs requesting such cost recovery shall perform the following:

(a)	After receiving the Initial Settlement Statement for the subject Operating Day, submit a settlement dispute in accordance with the dispute process outlined in Section 9.5, Settlement and Billing Dispute Process.  In addition to the standard information required on the dispute form on the ERCOT Portal, the dispute should clearly indicate:

(i)	The Dispatch Instruction received from ERCOT to provide the OOME Services;

(ii)	The payment received for providing the OOME Service;

(iii)	The actual cost of providing the OOME Service;

(iv)	A reference to the documentation to be provided in writing as indicated in these Protocols; and

(b)	Provide written documentation to allow ERCOT to verify the claimed amounts.  All documentation submitted to ERCOT for verification pursuant to this Section 6.8.2.3(4)(b) shall be considered Protected Information in accordance with Section 1.3.1.1, Items Considered Protected Information.

ERCOT shall not make payments for verifiable cost outside the defined documentation requirements until after the QSE has followed the steps outlined in Section 6.8.2.3(4)(b) and the ERCOT Board has approved the documentation requirements.

Fuel costs, including transportation and storage costs, will require supporting documentation of sufficient detail to allow for the verification of the cost of fuel consumed at the deployed Resource.  Documentation may include contracts, invoices or other documents.  For gas fired Resources, such documentation will not be required if the requested incremental fuel cost is less than one hundred ten percent (110%) of the Fuel Index Price.

Defined Documentation Requirements

(i)	Operational Fuel Cost for providing the OOME Service shall be determined by multiplying the change in instructed energy (MWh) due to the OOME instruction by the associated Marginal Heat Rate (MMBtu/MWh) and the associated fuel cost ($/MMBtu) of the instructed Resource for the intervals that the Resource was procured to provide the OOME Service.

(ii)	The Marginal Heat Rate shall be determined by subtracting the total amount of fuel burn of the Resource (in MMBtu/hr) based on the planned output shown in the Resource Plan from the total amount of fuel burn (in MMBtu/hr) based on the lesser of the instructed level of the Resource or the actual Resource output, dividing the result by the net change in output level from the Resource Plan to the lesser of the instructed level of the Resource or the actual output of the unit (in MW).  The amount of fuel consumed (in MMBtu) to provide OOME Service shall be calculated from the input/output curve coefficients or linear interpolation of test data points for the Resource from the most recently conducted heat rate tests filed with ERCOT.  Test data shall be provided in sufficient detail to allow for the validation of the heat rate curve provided.

(iii)	ERCOT nodal implementation surcharge for providing OOME Service, calculated by multiplying the change in instructed energy (MWh) due to the OOME instruction by the NODSF ($/MWh).

(c)	Compensation for types of cost, whose documentation requirements are not covered in Sections 6.8.2.3(4)(b)(i) through 6.8.2.3(4)(b)(iii) for the deployed Resource, will be denied pending possible review by the ERCOT Board of Directors.  The requesting QSE may request approval of the documentation requirements by the Board, and if requested will be considered by the Board at its next regularly scheduled meeting for which proper notice may be posted following ERCOT’s receipt of the request.  The requesting party may request that this review be conducted at an Executive Session of the ERCOT Board of Directors.  Requests must be presented in person by a representative of the company submitting the request and must also include language suitable to be included in the Protocols to define the documentation requirements for future requests of a similar nature.  Subsequent to the approval of such costs, the requesting company shall submit a Protocol Revision Request, in accordance with Section 21, Process for Protocol Revision, incorporating the necessary documentation standards provided to the ERCOT Board.  Once approved by the ERCOT Board, ERCOT will process the request for payments as described in Section 9.2.5, Resettlement Statement.

(d)	Submit a signature sheet signed by the authorized representative of the Resource Entity certifying the costs submitted are directly attributable to the OOME deployment and which satisfies the documentation standards in Section 6.8.2.3(4)(b).

(5)	Whenever a Generation Resource is called on for OOME Down or Zonal OOME Down, the QSE will be paid an energy payment equal to the total amount of energy to be reduced by the Resource, multiplied by the greater of: (a) the difference between (i) the MCPE of the Congestion Zone and (ii) the RCFGC for that Resource or (b) $0.00.  The calculation for OOME Down is as follows:

PEOOMDNiuqz	=	-1 * EOOMDNiu  * Max (0,(MCPEiz-RCGFCc))

EOOMDNiu 	= 	Max (0, Min((OLuiq - MRiu ),IOOMDNiuq))

PEOOMDNivqz	=	-1 * EOOMDNiv  * Max (0,(MCPEiz-RCGFCc))

EOOMDNiv 	=	Max (0,Min((OLivq - MRivq ), NETDEQivq)) * OOMAGRivq

PEOOMDNiq	=	SUM (PEOOMDNiu)q + SUM (PEOOMDNiv)q

PEOOMDNiz	=	SUM (PEOOMDNiu)z + SUM (PEOOMDNiv)z

The equation below will be used to determine the Total OOM Down or Zonal OOM Down Payments to be allocated to each QSE in Section 6.9.7.2, OOM Energy Charge.

PEOOMDNi = SUM (PEOOMDNiq)q

NETDEQivq	=	Max [0, ((NETOOMDEQivq + NETLBEDQivq) –
(NETOOMUEQivq + NETLBEUQivq))]

NETOOMUEQivq 	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq 	=	Max(0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq	=	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDEQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

OOMAGRivq	=	[SUM(IOOMUPqui)u + SUM(IOOMDNqui)u] / 
[SUM(LBEUQqui)u + SUM(LBEDQqui)u + 
SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

c	Resource category

i	interval

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

q	QSE

PEOOMDNiuqz	Out of Merit Energy Down Payment ($) for that interval per unit for the QSE

PEOOMDNivqz	Out of Merit Energy Down Payment ($) for that interval per Aggregated Unit per QSE

PEOOMDNiq	Total Out of Merit Energy Down Payment in the interval for the QSE

PEOOMDNiz	Out of Merit Energy Down Payment ($) per interval per zone

PEOOMDNiv	Out of Merit Energy Down Payment ($) per interval per zone to Aggregated Units

EOOMDNiu 	OOM Energy quantity instructed down for that unit per interval

EOOMDNiv 	OOM Energy quantity instructed down for that Aggregated  Unit per interval

MCPEiz	Market Clearing Price for Energy per interval for that zone

IOOMUPiuq	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMDNiuq	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

MRiu	Meter reading per unit per interval

MRiv	Meter reading per Aggregated Unit per interval

OLiuq	Resource Plan submitted by the QSE for the specific unit given the OOM instruction for that interval.

OLivq	Resource Plan submitted by the QSE for the Aggregated Unit containing the individual unit(s) given the OOM instruction for that interval.

RCGFCc	Resource Category Generic Fuel Cost for a specific category as defined in Section 6.8.2

PEOOMDNi	Total OOM Down Energy Payments

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

NETOOMUEQivq The net of all unit-specific OOM instructions (Up minus Down) that result in a positive value.

NETLBEUQivq	The net of all unit-specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq The net of all unit-specific OOM instructions (Down minus Up) that result in a positive value.

NETLBEDQivq The net of all unit-specific Local Balancing Energy Instructions (Down minus Up) that result in a positive value.

NETDEQiv	The overall net direction per the instructions given to the Aggregated Unit results in a down direction

OOMAGRiv 	The percentage of the aggregated instructions to the Aggregated Unit that represents the OOM Energy market.

- The Nodal Market Implementation requires Early Delivery System testing activities.  During the Early Delivery System 3 LFC Phase 3 test, the formula in section 6.8.2.3 (5)  will be modified in the following way:

IOOMUPiuq	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

IOOMDNiuq	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

LBEUQqui	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

LBEDQqui	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| IOOMUPiuq | MWh | OOM Energy Up Instructions—The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an OOME Up deployment. |
| IOOMDNiuq | MWh | OOM Energy Down Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an OOME Down deployment. |
| LBEUQqui | MWh | Local Balancing Energy Up Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an Local Balancing Energy Up deployment. |
| LBEDQqui | MWh | Local Balancing Energy Down Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an Local Balancing Energy Down deployment. |
| BP y | MW | Base Point by interval—The Base Point generated from SCED test system for the Generation Resource for the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| y | none | A SCED interval in the Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| i | none | 15-minute Settlement Interval |
| u | none | Individual generation unit (including units within an Aggregated Unit) |
| q | none | QSE |

    - If a combined cycle plant receives SCED base points at the combined cycle configuration level, ERCOT will use telemetry based splitting percentages to allocate the OOME or Local Balancing Energy Instruction, as calculated in section (a) above, to each individual unit that makes up the configuration for each 15-minute settlement interval.

    - If after receiving the Intial Statement for the Operating Day on which there was LFC testing, a QSE does not agree with the updated Instructed Output level or payment for OOME or Local Balancing Energy, a QSE may submit a settlement dispute in accordance with the dispute process outlined in Section 9.5, Settlement and Billing Dispute Process.  In addition to the standard information required on the dispute form on the ERCOT Portal, the dispute should clearly indicate what is being disputed.  Verification of these claims must be submitted to ERCOT by the QSE or the Resource to allow resolution by the end of the dispute process for Settlement True-Up as defined in Section 9.2.6, True-Up Statement.  Upon verification of the data provided, ERCOT shall grant such disputes.

(6)	If a wind Resource is called upon to provide OOME Down Service and the normal payment for OOME Service is insufficient to cover actual verifiable costs of providing the service, then that Resource may file a claim with ERCOT to be paid the lower of:

(a)	All verifiable costs that are directly attributable to the OOME service; or

(b)	MaxCap * CCF * Curtail% * Hrs *CRP

Where:

MaxCap:	Registered Maximum Capacity of the Resource (MW)

CCF:	Capacity Conversion Factor as defined at 30%

Curtail%:	Set at 15% for the period July 1, 2002 through June 30, 2003; 10% for the period July 1, 2003 through June 30, 2004; and 5% from July 1, 2004 and thereafter.

Hrs:	Total number of hours in the month

CRP:	Price paid for curtailment, $27/MWH

The verifiable costs of providing the service shall be provided to ERCOT in writing from the QSE, certified by an authorized representative of the Resource and within a timeframe to allow resolution by the end of the dispute process for Settlement True-Up.  Verifiable costs under section (a) above shall be based on the actual verifiable MW production that the unit could have produced based on actual operating and maintenance conditions and actual wind speeds.  Verifiable costs must be established in order for a claim to be valid even if paid under method (b) above.

A wind Resource filing a claim under this Section 6.8.2.3(6) must provide actual production and actual wind speed data for the entire month of the claim.  ERCOT will review this data for all hours of the month.  In the event ERCOT determines that it paid the Resource for any OOME down service in any hour of the month that the actual wind speed data show could not have actually been provided, then ERCOT shall deduct any such excess payment from the payment otherwise payable under (a) or (b) above.  A wind Resource filing a claim under Section 6.8.2.3(6) shall also provide an affidavit indicating that the wind Resource, or its Affiliate(s), has tax liability sufficient to fully utilize any lost credits claimed to offset taxes for the year in which the credit would have been received.

This provision will expire at the earlier of:

(i)	December 31, 2006;

(ii)	When ERCOT implements direct assignment of Local Congestion costs in accordance with the PUCT’s Order on Rehearing in Docket 23220; or

(iii)	Within one (1) month after accumulated verifiable costs paid under this Section 6.8.2.3(5) reach ten million ($10,000,000) dollars.

(7)	Whenever a Load acting as a Resource is called for OOME Up, the QSE will receive an OOME payment of the instructed amount of OOME Up provided by the Load acting as a Resource multiplied by a price equal to item (a) minus item (b), as defined below:

(a)	The lower of the:

(i)	Sum of the bid premium submitted by the specific Load acting as a Resource and the MCPE for the Congestion Zone; or

(ii)	The OOME Up Price.

(b)	The MCPE for the Congestion Zone.

OOME Up Price is determined by multiplying the Fuel Index by the appropriate Heat Rate.  The Heat Rate will be set at 18 MMBtu / MWh.

The FIP shall be the midpoint price expressed in $ / MMBtu, published in Gas Daily, in the Daily Price Survey, under the heading “East-Houston-Katy, Houston Ship Channel” for the day of OOME deployment.  The FIP for Saturdays, Sundays, holidays, and other days for which there is no FIP published in Gas Daily, shall be the next published FIP after the day of OOME deployment.  In the event that the FIP is not published for more than two (2) days, the previous day published FIP will be used for Initial Settlement and the next day published FIP will be used for true up of the Final Statement.

The calculation for OOME Up for Load acting as a Resource is as follows:

PEOOMUPiq 	=	SUM (PEOOMUPiuq)uq

Where:

PEOOMUPiuq	=	-1 * EOOMUPiuq * [Max [Min ((FI * HR), (BPiuq + MCPEiz)), MCPEiz] - MCPEiz]

EOOMUPiuq	=	Max (0, Min (OLiuq - LAARTOT), IOOMUPiu))

The equation below will be used to determine the Total OOM Energy Payments to be allocated to each QSE in Section 6.9.7.2, OOM Energy Charge.

PEOOMUPi 	= SUM(PEOOMUPiu)

Where:

i		interval

q		QSE

u		unit

z		zone

FI		Fuel Index ($/Btu)

HR	The standard Heat Rate for OOM of a Load Acting as a Resource is 18 MMBtu

EOOMUPiuq	OOM Up quantity deployed for that unit per interval per QSE

IOOMUPiu	The difference between the Resource Plan and the OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

BPiuq	Actual Bid Premium Up bid price submitted by the specific Load acting as a Resource

LAARTOT	Actual ESI ID usage

MCPEiz	Market Clearing Price of Energy for that interval of the zone in which unit resides

OLiuq	Resource Plan output level submitted by the QSE for the specific Resource given the OOM instruction for that interval

PEOOMUPi	Summation of Out of Merit Energy Up Payment ($) per interval for all QSEs in the market

PEOOMUPiq	Out of Merit Energy Up Payment ($) for that interval per QSE

PEOOMUPiuq	Out of Merit Energy Up Payment ($) for that interval per unit per QSE

If a Load acting as a Resource is called to provide OOME Up service and the payment for OOME service is insufficient to cover all costs of providing the Service, then that Load acting as a Resource will be paid, in addition to the energy payment, all verifiable costs in excess of the OOME payment that are directly attributable to the OOME Service.  These costs may include the cost of lost raw material, product rendered unusable, idled labor, clean up cost associated with the deployment required to restart production. In addition incremental production cost such as labor, energy and material, as well as any incremental energy cost associated with running backup generation may also be included. Idled labor will only be allowed if the OOME instruction is longer than three (3) hours in duration. Such payment will not exceed one thousand dollars ($1000) per MWh for the duration of the deployment plus an additional restart period not to exceed three (3) hours.  Verification of these costs must be submitted to ERCOT to allow resolution by the end of the dispute process for Settlement True-Up as defined in Section 9.2.4, Final Statements, True-Up Statements.

Compensation for types of cost, whose documentation requirements are not outlined in this Section 6.8.2.3(7), must be reviewed and approved by the ERCOT Board of Directors during an Executive Session of their next regularly scheduled meeting.  Requests must be presented in person by a representative of the company submitting the request and must also include language suitable to be included in the Protocols to define the documentation requirements for future requests of a similar nature.  Subsequent to the approval of such costs, the requesting company shall submit a Protocol Revision Request, in accordance with Section 21, Process for Protocol Revision, incorporating the necessary documentation standards provided to the ERCOT Board. Once approved by the ERCOT Board, ERCOT will process payments for such cost on a Resettlement Statement as described in Section 9.2.5, Resettlement Statement.

7.4.3.1	Balancing Energy Up from a Specific Resource

(1)	In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, whenever a specific Resource with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Up, is deployed incrementally to solve Local Congestion, the QSE will be paid the difference between the MCPE of the Congestion Zone in which the specific Resource is located and the incremental bid premium specified.  Aggregated Unit rules as described in Section 6.8.2, Capacity and Energy Payments for Out of Merit Service, apply to Aggregated Units approved by ERCOT.

The calculation of the payment for Balancing Energy Up from a specific Resource is as follows:

LPCRSUquiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (MRqiuz -OLqiuz, IOLqiuz - OLqiuz)))

LPCRSUqviz	=	 -1 * Σ ((PMqivz – MCPEiz )* Max (0, Min (MRqivz -OLqivz, NETUEQivq)) * LBEAGRivq)

For gas-fired units:

FBPMqiuz = (BPMqiuz  / FIPd-1) * FIPd

PMqiuz	= Max (FBPMqiuz, MCPEiz )

PMqivz	= Min(PMqiuz)

For all other Resource categories:

PMqiuz	= Max (BPMqiuz, MCPEiz )

PMqivz	= Min(PMqiuz)

NETUEQivq	=	Max [0, ((NETOOMUEQivq + NETLBEUQivq) –(NETOOMDEQivq + NETLBEDQivq))]

NETOOMUEQivq	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq	=	Max (0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq	=	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

LBEAGRivq 	=	[(SUM(LBEUQqui)u + SUM(LBEDQqui)u)] /
[SUM(LBEUQqui)u + SUM(LBEDQqui)u +
SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

d	Operating Day of interval i

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

BPMqiuz	Incremental bid premium for that specific unit per interval

FBPMqiuz	Incremental bid premium, adjusted for fuel price, for that specific unit per interval

FIPd	Fuel Index Price for Operating Day of interval i

FIPd-1	Fuel Index Price used to calculate Resource category bid limits effective for Operating Day of interval i

IOLqiuz	The Instructed output level for the unit given the unit-specific instruction

IOOMDNqui	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMUPqui	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

LPCRSUquiz	Payment to QSE for providing Balancing Energy Up from Specific unit to solve Local Congestion per QSE per interval per zone

LPCRSUqviz	Payment to QSE for providing Balancing Energy Up from Aggregated Units to solve Local Congestion per QSE per interval per zone

LBEAGRivq	The percentage of the aggregated instructions to the Aggregated Unit that represents the OOM Energy market

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

MCPEiz	Market Clearing Price for Energy per interval per zone

MRqiuz	Meter reading per unit per interval

MRqivz	Meter reading per Aggregated Unit per interval

NETLBEDQivq	The net of all unit specific Local Balancing Energy Instructions

(Down minus Up) that result in a positive value

NETLBEUQivq	The net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value

NETOOMDEQivq	The net of all unit specific OOM instructions (Down minus Up) that result in a positive value

NETOOMUEQivq	The net of all unit specific OOM instructions (Up minus Down) that result in a positive value

NETUEQivq	The overall net direction per the instructions given to the Aggregated Unit results in an up direction

OLqiuz	Resource Plan output level submitted by the QSE for the specific unit given the unit-specific instruction for that interval

OLqivz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval

PMqiuz	Incremental Premium Specified for that specific unit per interval

PMqivz	Incremental Premium Specified for that specific Aggregated Unit per interval

- The Nodal Market Implementation requires Early Delivery System testing activities.  During the Early Delivery System 3 LFC Phase 3 test, the formula in section 7.4.3.1 (1)  will be modified in the following way:

IOOMUPiuq	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

IOOMDNiuq	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

LBEUQqui	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

LBEDQqui	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| IOOMUPiuq | MWh | OOM Energy Up Instructions—The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an OOME Up deployment. |
| IOOMDNiuq | MWh | OOM Energy Down Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an OOME Down deployment. |
| LBEUQqui | MWh | Local Balancing Energy Up Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an Local Balancing Energy Up deployment. |
| LBEDQqui | MWh | Local Balancing Energy Down Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an Local Balancing Energy Down deployment. |
| BP y | MW | Base Point by interval—The Base Point generated from SCED test system for the Generation Resource for the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| y | none | A SCED interval in the Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| i | none | 15-minute Settlement Interval |
| u | none | Individual generation unit (including units within an Aggregated Unit) |
| q | none | QSE |

    - If a combined cycle plant receives SCED base points at the combined cycle configuration level, ERCOT will use telemetry based splitting percentages to allocate the OOME or Local Balancing Energy Instruction, as calculated in section (a) above, to each individual unit that makes up the configuration for each 15-minute settlement interval.

    - If after receiving the Intial Statement for the Operating Day on which there was LFC testing, a QSE does not agree with the updated Instructed Output level or payment for OOME or Local Balancing Energy, a QSE may submit a settlement dispute in accordance with the dispute process outlined in Section 9.5, Settlement and Billing Dispute Process.  In addition to the standard information required on the dispute form on the ERCOT Portal, the dispute should clearly indicate what is being disputed.  Verification of these claims must be submitted to ERCOT by the QSE or the Resource to allow resolution by the end of the dispute process for Settlement True-Up as defined in Section 9.2.6, True-Up Statement.  Upon verification of the data provided, ERCOT shall grant such disputes.

(2)	Whenever a LaaR with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Up, is deployed to solve Local Congestion, the QSE will be paid the incremental premium specified as defined below:

LPCLAARquiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (OLqiuz -LAARTOTqiuz, OLqiuz - IOLqiuz)))

PMqiuz	= Max (FBPMqiuz, MCPEiz )

FBPMqiuz	= (BPMqiuz / FIPd-1) * FIPd

Where:

i	interval

q	QSE

u	individual LAAR unit

z	zone

BPMqiuz	Incremental bid premium for that specific LaaR unit per interval

FBPMqiuz	Incremental bid premium, adjusted for fuel price, for that specific unit per interval

FIPd	Fuel Index Price for Operating Day of interval i

FIPd-1	Fuel Index Price used to calculate Resource category bid limits effective for Operating Day of interval i

IOLqiuz	The Instructed output level for the LaaR unit given the unit-specific instruction

LAARTOTqiuz	Actual ESI ID usage per unit per interval

LPCLAARquiz	Payment to QSE for providing Balancing Energy Up from specific LaaR unit to solve Local Congestion per QSE per interval per zone

MCPEiz	Market Clearing Price for Energy per interval per zone

OLqiuz	Resource Plan output level submitted by the QSE for the specific LaaR unit given the unit-specific instruction for that interval

PMqiuz	Incremental Premium specified for that specific LaaR unit per interval

7.4.3.2	Balancing Energy Down from a Specific Resource

In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, whenever a specific Generation Resource with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Down, is deployed decrementally to solve Local Congestion, the QSE will be paid the difference between the MCPE of the Congestion Zone in which the specific Generation Resource is located, and the decremental bid premium specified.

The calculation of the payment for Balancing Energy Down from a specific Resource is as follows:

LPCRSDquiz 	= 	-1 * Σ(MAX(0,(MCPEiz - PMqiuz )) * Max(0, Min(OLqiuz -MRqiuz, OLqiuz - IOLqiuz)))u

LPCRSDqviz 	=	-1 * Σ(MAX(0,(MCPEiz - PMqivz )) * Max(0, Min(OLqivz -MRqivz  NETDEQivq)))v * LBEAGRivq

For gas-fired units:

FBPMqiuz	= (BPMqiuz  / FIPd-1) * FIPd

PMqiuz	= FBPMqiuz

PMqivz	= Max(PMqiuz)

For all other Resource categories:

PMqiuz	= BPMqiuz

PMqivz	= Max(PMqiuz)

NETDEQivq	=	Max [0, ((NETOOMDEQivq + NETLBEDQivq) – (NETOOMUEQivq + NETLBEUQivq))]

NETOOMUEQivq	=	Max [0, (SUM (IOOMUPqui)u – SUM(IOOMDNqui)u)]

NETLBEUQivq 	=	Max [0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u)]

NETOOMDEQivq	=	Max [0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u)]

NETLBEDQivq 	=	Max[0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u)]

LBEAGRivq	=	[SUM(LBEUQqui)u + SUM(LBEDQqui)u] /
[SUM(LBEUQqui)u + SUM(LBEDQqui)u +
SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

d	Operating Day of interval i

i	interval

q	QSE

u	individual generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

BPMqiuz	Decremental bid premium specified for that specific unit per interval

FBPMqiuz	Decremental bid premium, adjusted for fuel price, for that specific unit per interval

FIPd	Fuel Index Price for Operating Day of interval i

FIPd-1	Fuel Index Price used to calculate Resource category bid limits effective for Operating Day of interval i

IOLqiuz	The instructed output level for the unit given a unit-specific instruction

IOOMDNqui	OOM Energy Down instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMUPqui	OOM Energy Up instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

LBEAGRivq	The percentage of the aggregated instructions to the Aggregate Unit that represents the OOM Energy market.

LBEDQqui	Local Balancing Energy Down instructions for that interval per specific unit

LBEUQqui	Local Balancing Energy Up instructions for that interval per specific unit

LPCRSDquiz	Payment to QSE for providing Balancing Energy Down from Specific unit to solve Local Congestion per QSE per interval per zone

LPCRSDqviz	Payment to QSE for providing Balancing Energy Down from Aggregated Units to solve Local Congestion per QSE per interval per zone

MCPEiz	Market Clearing Price of Energy per interval per zone

MRqiuz	Meter reading per unit per interval.

MRqivz	Meter reading per Aggregated Unit per interval.

NETDEQivq	The overall net direction per the instructions given to the Aggregated Unit results in a down direction

NETLBEDQivq	The net of all unit specific Local Balancing Energy instructions (Down minus Up) that result in a positive value.

NETLBEUQivq	The net of all unit specific Local Balancing Energy instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq	The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETOOMUEQivq	The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

OLqiuz	Resource Plan output level submitted by the QSE for the unit given the unit-specific instruction for that interval

OLqivz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval

PMqiuz	Decremental Premium for that specific unit per interval

PMqivz	Decremental Premium for that specific Aggregated Unit per interval

- The Nodal Market Implementation requires Early Delivery System testing activities.  During the Early Delivery System 3 LFC Phase 3 test, the formula in section 7.4.3.2 will be modified in the following way:

IOOMUPiuq	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

IOOMDNiuq	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

LBEUQqui	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

LBEDQqui	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| IOOMUPiuq | MWh | OOM Energy Up Instructions—The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an OOME Up deployment. |
| IOOMDNiuq | MWh | OOM Energy Down Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an OOME Down deployment. |
| LBEUQqui | MWh | Local Balancing Energy Up Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an Local Balancing Energy Up deployment. |
| LBEDQqui | MWh | Local Balancing Energy Down Instructions —The Generation Resource’s aggregated Base Point, for the 15-minute Settlement Interval that corresponds with an Local Balancing Energy Down deployment. |
| BP y | MW | Base Point by interval—The Base Point generated from SCED test system for the Generation Resource for the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| y | none | A SCED interval in the Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred. |
| i | none | 15-minute Settlement Interval |
| u | none | Individual generation unit (including units within an Aggregated Unit) |
| q | none | QSE |

    - If a combined cycle plant receives SCED base points at the combined cycle configuration level, ERCOT will use telemetry based splitting percentages to allocate the OOME or Local Balancing Energy Instruction, as calculated in section (a) above, to each individual unit that makes up the configuration for each 15-minute settlement interval.

    - If after receiving the Intial Statement for the Operating Day on which there was LFC testing, a QSE does not agree with the updated Instructed Output level or payment for OOME or Local Balancing Energy, a QSE may submit a settlement dispute in accordance with the dispute process outlined in Section 9.5, Settlement and Billing Dispute Process.  In addition to the standard information required on the dispute form on the ERCOT Portal, the dispute should clearly indicate what is being disputed.  Verification of these claims must be submitted to ERCOT by the QSE or the Resource to allow resolution by the end of the dispute process for Settlement True-Up as defined in Section 9.2.6, True-Up Statement.  Upon verification of the data provided, ERCOT shall grant such disputes.