COMS

DAM Ancillary Service Settlements

Requirements

(B2)  v3.0

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 05/1/06 | 0.1 | Initial Draft | John Adams |
| 06/1/06 | 0.2 | Final Draft for ERCOT CS Team Review. This draft is based off of the 9-23-06 protocols. | John Adams |
| 6/21/06 | 0.9 | TPTF Review Process | John Adams |
| 7/21/06 | 0.91 | TPTF Comments Included | John Adams |
| 7/26/06 | 1.0 | TPTF Approved | John Adams |
| 3/29/07 | 1.01 | Updated for changes found while processing Use Cases | John Bieltz |
| 07/27/07 | 2.0 | Updated for Baseline 1, Version 2.0. | April Vega |
| 08/27/07 | 2.01 | Removed FR58, FR77, FR96, FR115 | April Vega |
| 10/03/07 | 2.02 | Responses to comments | April Vega |
| 10/09/07 | 3.0 | Approved by TPTF | April Vega |

Table of Contents

1.	Day-Ahead Market Ancillary Service Settlement	4

1.1	Proposed System Scope	4

1.2	Document Conventions	7

1.3	Document Assumptions	7

1.4	Global Input Calculation Requirements	7

2.	Functional Requirements	8

2.1	Regulation Up Service Payment (PCRUAMT)	8

2.2	Regulation Down Service Payment (PCRDAMT)	11

2.3	Responsive Reserve Service Payment (PCRRAMT)	13

2.4	Non-Spin Service Payment (PCNSAMT)	16

2.5	Regulation Up Service Charge (DARUAMT)	19

2.6	Regulation Down Service Charge (DARDAMT)	24

2.7	Responsive Reserve Service Charge (DARRAMT)	29

2.8	Non-Spin Service Charge (DANSAMT)	34

3.	Supplementary Requirements	38

3.1	Performance Requirements	39

3.2	Legal and Regulatory requirements	39

3.3	System Security Requirements	39

3.4	Back up and Recovery Requirements	39

3.5	Availability and Redundancy Requirements	39

3.6	Maintainability Requirements	39

3.7	Daylight Savings Time shall be recognized in Nodal Market settlements.	39

3.7.2	Fall Daylight Savings Time Operating Day:	39

4.	Protocol Coverage	40

Software Requirements Specification

# Day-Ahead Market Ancillary Service Settlement

## Proposed System Scope

ERCOT plans to implement a new Nodal market in 2009.  Sections 4, 9, and 12 of the Nodal Protocols describe the various Charge Types and Settlement Statement requirements for the Day-Ahead Market (DAM).  The Nodal Protocols will guide this project.

This document defines the requirements for settlement of the Ancillary Service components of the Day-Ahead Market (DAM) as identified in Sections 4 of the Protocols.

The following Settlement Charge Types are included within this Business Requirements Document:

- Regulation Up Service Payment

- Regulation Down Service Payment

- Responsive Reserve Service Payment

- Non-Spinning Reserve Service Payment

- Regulation Up Service Charge

- Regulation Down Service Charge

- Responsive Reserve Service Charge

- Non-Spinning Reserve Service Charge

To Be Sub-Process for Settlements and Billing

Following Chart provides context around the Day-Ahead Market Settlements process

The following chart illustrates the dependencies between the various DAM Ancillary Service Settlement calculations.

## Document Conventions

- All text lifted from the Nodal Protocols is in italic font.

- All dependencies are identified using D1, D2, etc.

- All assumptions are identified using A1, A2, etc.

- The “Original Source” column of the Input, Intermediate, and Output Bill Determinates tables (refer to Section 2, Functional Requirements) delineate the initial source of market and/or operational information required to construct each variable.  The data transformation process exists within the interface between the Settlement System and Market / Operational Systems.  A separate and distinct set of Business Requirements will address all data interfacing between the original source system and the Settlement System.

## Document Assumptions

1. Per the Nodal Protocols Section 4.5.3, ERCOT will communicate DAM Ancillary Service Capacity Awards and post public data (market clearing prices and ERCOT-wide quantities) to the MIS Public area no later than 1330 in the Day-Ahead.  Therefore, the aforementioned data should be available for interface process no later than 1330 in the Day-Ahead.

2. All Day-Ahead Market Clearing Prices for Ancillary Service Capacity (MCPCxx) will be calculated outside of the Settlement System and therefore the market clearing price calculations are not detailed within this document.

3. The original source system for the MCPCxxs will maintain a history for the prices.  Therefore, if a MCPCxx is corrected after a settlement run, the price that was used for the initial settlement run is still available.

4. If ERCOT is unable to execute the Day-Ahead process, ERCOT may abort all or part of the Day-Ahead process and require all schedules and trades to be submitted in the Adjustment Period. (PR 4.1.2 (2))  If such an Emergency Condition occurs, Day-Ahead Ancillary Service settlements may not be necessary.

5.	The charge type calculations are documented using the proposed recorder format for the bill determinants.  The following list defines the nomenclature used for the recorders:

<Q> = QSE

<CO> = CRR Owner

<R> = Resource

<SP> = Settlement Point

<SRSP> = Source Settlement Point

<SKSP> = Sink Settlement Point

<ST> = Start Type

<F> = Flowgate

<E> = Directional Network Element

<C> = Constraint Identifier

<M> = Market

<RUC> = RUC Process

## Global Input Calculation Requirements

None Applicable

# Functional Requirements

## Regulation Up Service Payment (PCRUAMT)

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Reg-Up to ERCOT were cleared in the DAM. (PR 4.6.4.1.1)  Payment is calculated by QSE for each hour of the Operating Day within the Day Ahead Market.

These particular bill determinants, PCRUAMT and PCRU, will need to be calculated and recorded with a market identifier.  This is necessary to maintain market Integrity.  As a result, it will be necessary to propagate the market identifier from the original input bill determinant to the output bill determinant.

| Requirement ID | FR1 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.1.1 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCRUAMT_<Q>_<M> = (-1) * MCPCRU_<M>* PCRU_<Q>_<M> |  |

| Requirement ID | FR2 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.1.1 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCRU_<Q>_<M> = ΣRPCRUR_<Q>_<R>_<M> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| PCRUR | MW | Procured Capacity for Reg-Up from Resource per Resource per QSE in DAM—The Reg-Up capacity quantity awarded to QSE Q in the DAM for Resource R for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: PCRUR data cuts will be created for Load Resources, as well as Generation Resources, whose Regulation Up offers were accepted in the DAM. |  |  |  |
| MCPCRU | $/MW per hour | Market Clearing Price of Capacity for Reg-Up in DAM—The DAM Market Clearing Price of Capacity for Reg-Up for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| PCRU | MW | Procured Capacity for Reg-Up per QSE in DAM—The total Reg-Up Service capacity quantity awarded to QSE Q in the DAM for all the Resources represented by this QSE for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the PCRUAMT calculation. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| PCRUAMT | $ | Procured Capacity for Reg-Up Amount per QSE in DAM—The DAM Reg-Up payment for QSE Q for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| PCRUBILLAMT | $ | Procured Capacity for Reg-Up Bill Amount per QSE in DAM—The difference between two consecutive daily summed channels of PCRUAMT for QSE Q in the Day Ahead Ancillary Service Market. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / Day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1)  The Regulation Up Service Payment is dependent upon the availability of the PCRUR and MCPCRU quantity cut.

### Assumptions

A1)  If the Day-Ahead MCPC cannot be calculated by ERCOT, the Day-Ahead MCPC for the particular Ancillary Service is equal to the Day-Ahead MCPC for that Ancillary Service in the same Settlement Interval of the preceding Operating Day. (PR 4.5.1 (11))  The Settlement System will continue to receive the MCPCRU data cut from the original source system with the substitute values; thereby relying on the original source system to make the determination of the calculability of the MCPC and the associated, appropriate, substitute value.

A2)	The Interface Project should define what errors should occur in the event that the MCPCRU is not available from the original source system.  Distinctly, a major alert should occur and Settlements should not begin any Day-Ahead Market Ancillary Service Regulation Up Settlements without the MCPCRU available.

A3)	The original source system should only create a PCRUR data cut for a resource that was actually awarded Regulation Up Capacity in the DAM for at least one hour of the operating day.  PCRUR data cuts should not be created with all zero values.

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR3 | PR 4.6.4.1.1 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: PCRUR, and MCPCRU. |
| FR4 | PR 4.6.4.1.1 | CSWM. P03SettleBill Transfer | The following intermediate bill determinant should not be rounded or truncated: PCRU. |
| FR5 | PR 9.4.1 (2) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: PCRUAMT and PCRUBILLAMT. |
| FR6 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that should be included in the public data extracts. MCPCRU All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. PCRUR PCRU PCRUAMT |
| FR7 | PR 4.6.4.1.1 | CSWM. P03SettleBill Transfer | All of the input, intermediate, and output bill determinants defined above should be identified with a unique market identifier. |
| FR8 | PR 9.2.5 (3) | CSWM. P03SettleBill Transfer | The PCRUBILLAMT value shall be calculated as the entire day’s sum of the PCRUAMT values for the current channel less the entire day’s sum of the PCRUAMT values for the most recent previous channel. |
| FR9 | PR 4.6.4.1.1 | CSWM. P03SettleBill Transfer | DRIVER: The PCRUAMT calculation should only be attempted for QSEs for which there is a resource with a PCRUR data cut, for the Operating Day, in the settlement system. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR10 | PR 4.6.4.1.1 | CSWM. P03SettleBill Transfer | MCPCRU is a critical bill determinant. The Settlement System should include this bill determinant in its check for critical data, prior to beginning the settlements process. If MCPCRU does not exist for a given Operating Day, or a given operating hour interval, the system should abend. The error message should capture the missing data element, Operating Day, and Operating Hour. |

### Specific Database Needs

None

## Regulation Down Service Payment (PCRDAMT)

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Reg-Down to ERCOT were cleared in the DAM. (PR 4.6.4.1.2) Payment is calculated by QSE for each hour of the Operating Day within the Day Ahead Market.

These particular bill determinants, PCRDAMT and PCRD, will need to be calculated and recorded with a market identifier.  This is necessary to maintain market Integrity.  As a result, it will be necessary to propagate the market identifier from the original input bill determinant to the output bill determinant.

| Requirement Id | FR 11 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.1.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCRDAMT_<Q>_<M> = (-1) * MCPCRD_<M> * PCRD_<Q>_<M> |  |

| Requirement Id | FR 12 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.1.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCRD_<Q>_<M> = ΣRPCRDR_<Q>_<R>_<M> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| PCRDR | MW | Procured Capacity for Reg-Down from Resource per Resource per QSE in DAM—The Reg-Down capacity quantity awarded to QSE Q in the DAM for Resource R for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: PCRDR data cuts will be created for Load Resources, as well as Generation Resources, whose Regulation Up offers were accepted in the DAM. |  |  |  |
| MCPCRD | $/MW per hour | Market Clearing Price of Capacity for Reg-Down in DAM—The DAM Market Clearing Price of Capacity for Reg-Down for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| PCRD | MW | Procured Capacity for Reg-Down per QSE in DAM—The total Reg-Down Service capacity quantity awarded to QSE Q in the DAM for all the Resources represented by this QSE for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the PCRDAMT calculation. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| PCRDAMT | $ | Procured Capacity for Reg-Down Amount per QSE in DAM—The DAM Reg-Down payment for QSE Q for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| PCRDBILLAMT | $ | Procured Capacity for Reg-Down Bill Amount per QSE in DAM— The difference between two consecutive daily summed channels of PCRDAMT for QSE Q in the Day Ahead Ancillary Service Market. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / Day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

Data Dependencies

D1)  The Regulation Down Service Payment is dependent upon the availability of the PCRDR quantity cut.

Assumptions

A1)  If the Day-Ahead MCPC cannot be calculated by ERCOT, the Day-Ahead MCPC for the particular Ancillary Service is equal to the Day-Ahead MCPC for that Ancillary Service in the same Settlement Interval of the preceding Operating Day. (PR 4.5.1 (11))  The Settlement System will continue to receive the MCPCRD data cut from the original source system with the substitute values; thereby relying on the original source system to make the determination of the calculability of the MCPC and the associated, appropriate, substitute value.

A2)	The Interface Project should define what errors should occur in the event that the MCPCRD is not available from the original source system.  Distinctly, a major alert should occur and Settlements should not begin any Day-Ahead Market Ancillary Service Regulation Down Settlements without the MCPCRD available.

A3)  The original source system should only create a PCRDR data cut for a resource that was actually awarded Regulation Down Capacity in the DAM for at least one hour of the operating day.  PCRDR data cuts should not be created with all zero values

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR13 | PR 4.6.4.1.2 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: PCRDR, and MCPCRD. |
| FR14 | PR 4.6.4.1.2 | CSWM. P03SettleBill Transfer | The following intermediate bill determinant should not be rounded or truncated: PCRD. |
| FR15 | PR 9.4.1 (2) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: PCRDAMT and PCRDBILLAMT. |
| FR16 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that should be included in the public data extracts. MCPCRD All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. PCRDR PCRD PCRDAMT |
| FR17 | PR 4.6.4.1.2 | CSWM. P03SettleBill Transfer | All of the input, intermediate, and output bill determinants defined above should be identified with a unique market identifier. |
| FR18 | PR 9.2.5 (3) | CSWM. P03SettleBill Transfer | The PCRDBILLAMT value shall be calculated as the entire day’s sum of the PCRDAMT values for the current channel less the entire day’s sum of the PCRDAMT values for the most recent previous channel. |
| FR19 | PR 4.6.4.1.2 | CSWM. P03SettleBill Transfer | DRIVER: The PCRDAMT calculation should only be attempted for QSEs for which there is a resource with a PCRDR data cut exists, for the Operating Day, in the settlement system. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR20 | PR 4.6.4.1.2 | CSWM. P03SettleBill Transfer | MCPCRD is a critical bill determinant. The Settlement System should include this bill determinant in its check for critical data, prior to beginning the settlements process. If MCPCRD does not exist for a given Operating Day, or a given operating hour interval, the system should abend. The error message should capture the missing data element, Operating Day, and Operating Hour. |

### Specific Database Needs

None

## Responsive Reserve Service Payment (PCRRAMT)

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Responsive Reserve to ERCOT were cleared in the DAM. (PR 4.6.4.1.3)  Payment is calculated by QSE for each hour of the Operating Day within the Day Ahead Market.

These particular bill determinants, PCRRAMT and PCRR, will need to be calculated and recorded with a market identifier.  This is necessary to maintain market Integrity.  As a result, it will be necessary to propagate the market identifier from the original input bill determinant to the output bill determinant.

| Requirement ID | FR21 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.1.3 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCRRAMT_<Q>_<M> = (-1) * MCPCRR_<M>* PCRR_<Q>_<M> |  |

| Requirement ID | FR22 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.1.3 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCRR_<Q>_<M> = ΣRPCRRR_<Q>_<R>_<M> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Original Source |
| --- | --- | --- | --- |
| PCRRR | MW | Procured Capacity for Responsive Reserve from Resource per Resource per QSE in DAM—The Responsive Reserve capacity quantity awarded to QSE Q in the DAM for Resource R for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: PCRRR data cuts will be created for Load Resources, as well as Generation Resources, whose Responsive Reserve offers were accepted in the DAM. |  |  |  |
| MCPCRR | $/MW per hour | Market Clearing Price of Capacity for Responsive Reserve in DAM—The DAM Market Clearing Price of Capacity for Responsive Reserve for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| PCRR | MW | Procured Capacity for Responsive Reserve per QSE in DAM—The total Responsive Reserve Service capacity quantity awarded to QSE Q in the DAM for all the Resources represented by this QSE for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the PCRRAMT calculation. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| PCRRAMT | $ | Procured Capacity for Responsive Reserve Amount per QSE in DAM—The DAM Responsive Reserve payment for QSE Q for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| PCRRBILLAMT | $ | Procured Capacity for Responsive Reserve Bill Amount per QSE in DAM— The difference between two consecutive daily summed channels of PCRRAMT for QSE Q in the Day Ahead Ancillary Service Market. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / Day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1)  The Responsive Reserve Service Payment is dependent upon the availability of the PCRRR quantity cut.

### Assumptions

A1)  If the Day-Ahead MCPC cannot be calculated by ERCOT, the Day-Ahead MCPC for the particular Ancillary Service is equal to the Day-Ahead MCPC for that Ancillary Service in the same Settlement Interval of the preceding Operating Day. (PR 4.5.1 (11))  The Settlement System will continue to receive the MCPCRR data cut from the original source system with the substitute values; thereby relying on the original source system to make the determination of the calculability of the MCPC and the associated, appropriate, substitute value.

A2)	The Interface Project should define what errors should occur in the event that the MCPCRR is not available from the original source system.  Distinctly, a major alert should occur and Settlements should not begin any Day-Ahead Market Ancillary Service Responsive Reserve Settlements without the MCPCRR available.

A3)	The original source system should only create a PCRRR data cut for a resource that was actually awarded Responsive Reserve Capacity in the DAM for at least one hour of the operating day.  PCRRR data cuts should not be created with all zero values

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR23 | PR 4.6.4.1.3 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: PCRRR, and MCPCRR. |
| FR24 | PR 4.6.4.1.3 | CSWM. P03SettleBill Transfer | The following intermediate bill determinant should not be rounded or truncated: PCRR. |
| FR25 | PR 9.4.1 (2) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: PCRRAMT and PCRRBILLAMT. |
| FR26 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that should be included in the public data extracts. MCPCRR All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. PCRRR PCRR PCRRAMT |
| FR27 | PR 4.6.4.1.3 | CSWM. P03SettleBill Transfer | All of the input, intermediate, and output bill determinants defined above should be identified with a unique market identifier. |
| FR28 | PR 9.2.5 (3) | CSWM. P03SettleBill Transfer | The PCRRBILLAMT value shall be calculated as the entire day’s sum of the PCRRAMT values for the current channel less the entire day’s sum of the PCRRAMT values for the most recent previous channel. |
| FR29 | PR 4.6.4.1.3 | CSWM. P03SettleBill Transfer | DRIVER: The PCRRAMT calculation should only be attempted for QSEs for which there is a resource with a PCRRR data cut exists, for the Operating Day, in the settlement system. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR30 | PR 4.6.4.1.3 | CSWM. P03SettleBill Transfer | MCPCRR is a critical bill determinant. The Settlement System should include this bill determinant in its check for critical data, prior to beginning the settlements process. If MCPCRR does not exist for a given Operating Day, or a given operating hour interval, the system should abend. The error message should capture the missing data element, Operating Day, and Operating Hour. |

### Specific Database Needs

None

## Non-Spin Service Payment (PCNSAMT)

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Non-Spin to ERCOT were cleared in the DAM. (PR 4.6.4.1.4)  Payment is calculated by QSE for each hour of the Operating Day within the Day Ahead Market.

These particular bill determinants, PCNSAMT and PCNS, will need to be calculated and recorded with a market identifier.  This is necessary to maintain market Integrity.  As a result, it will be necessary to propagate the market identifier from the original input bill determinant to the output bill determinant.

| Requirement ID | FR31 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.1.4 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCNSAMT_<Q>_<M> = (-1) * MCPCNS_<M>* PCNS_<Q>_<M> |  |

| Requirement ID | FR32 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.1.4 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCNS_<Q>_<M> = ΣRPCNSR_<Q>_<R>_<M> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| PCNSR | MW | Procured Capacity for Non-Spin from Resource per Resource per QSE in DAM—The Non-Spin capacity quantity awarded to QSE Q in the DAM for Resource R for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: PCNSR data cuts will be created for Load Resources, as well as Generation Resources, whose Non-Spin offers were accepted in the DAM. |  |  |  |
| MCPCNS | $/MW per hour | Market Clearing Price of Capacity for Non-Spin in DAM—The DAM Market Clearing Price of Capacity for Non-Spin for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| PCNS | MW | Procured Capacity for Non-Spin per QSE in DAM—The total Non-Spin Service capacity quantity awarded to QSE Q in the DAM for all the Resources represented by this QSE for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the PCNSAMT calculation. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| PCNSAMT | $ | Procured Capacity for Non-Spin Amount per QSE in DAM—The DAM payment for QSE Q for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| PCNSBILLAMT | $ | Procured Capacity for Non-Spin Bill Amount per QSE in DAM— The difference between two consecutive daily summed channels of PCNSAMT for QSE Q in the Day Ahead Ancillary Service Market. This bill determinant needs to be identified with a DAM Market | Settlement System |
| Interval Frequency: 1 / Day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1)  The Non-Spin Service Payment is dependent upon the availability of the PCNSR quantity cut.

### Assumptions

A1)  If the Day-Ahead MCPC cannot be calculated by ERCOT, the Day-Ahead MCPC for the particular Ancillary Service is equal to the Day-Ahead MCPC for that Ancillary Service in the same Settlement Interval of the preceding Operating Day. (PR 4.5.1 (11))  The Settlement System will continue to receive the MCPCNS data cut from the original source system with the substitute values; thereby relying on the original source system to make the determination of the calculability of the MCPC and the associated, appropriate, substitute value.

A2)	The Interface Project should define what errors should occur in the event that the MCPCNS is not available from the original source system.  Distinctly, a major alert should occur and Settlements should not begin any Day-Ahead Market Ancillary Service Non-Spin Settlements without the MCPCNS available.

A3)	The original source system should only create a PCNSR data cut for a resource that was actually awarded Non-Spin Capacity in the DAM for at least one hour of the operating day.  PCNSR data cuts should not be created with all zero values

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR33 | PR 4.6.4.1.4 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: PCNSR, and MCPCNS. |
| FR34 | PR 4.6.4.1.4 | CSWM. P03SettleBill Transfer | following intermediate bill determinant should not be rounded or truncated: PCNS. |
| FR35 | PR 9.4.1 (2) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: PCNSAMT and PCNSBILLAMT. |
| FR36 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that should be included in the public data extracts. MCPCNS All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. PCNSR PCNS PCNSAMT |
| FR37 | PR 4.6.4.1.4 | CSWM. P03SettleBill Transfer | All of the input, intermediate, and output bill determinants defined above should be identified with a unique market identifier. |
| FR38 | PR 9.2.5 (3) | CSWM. P03SettleBill Transfer | The PCNSBILLAMT value shall be calculated as the entire day’s sum of the PCNSAMT values for the current channel less the entire day’s sum of the PCNSAMT values for the most recent previous channel. |
| FR39 | PR 4.6.4.1.4 | CSWM. P03SettleBill Transfer | DRIVER: The PCNSAMT calculation should only be attempted for QSEs for which there is a resource with a PCNSR data cut exists, for the Operating Day, in the settlement system. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR40 | PR 4.6.4.1.4 | CSWM. P03SettleBill Transfer | MCPCNS is a critical bill determinant. The Settlement System should include this bill determinant in its check for critical data, prior to beginning the settlements process. If MCPCNS does not exist for a given Operating Day, or a given operating hour interval, the system should abend. The error message should capture the missing data element, Operating Day, and Operating Hour. |

### Specific Database Needs

None

## Regulation Up Service Charge (DARUAMT)

The Regulation Up Service Charge is assessed to a QSE whose net Regulation Up obligation is in excess of its Regulation Up self-arranged quantity.  The QSE’s net obligation is the result of netting the QSE’s self-arranged amounts against its Regulation Up obligation.  The obligation is based upon the QSE’s Load Ratio Share.  ERCOT determines each QSE’s Ancillary Service obligation for DAM settlement based on Load ratio Share values from the most recent Initial settlement data for the same hour and day of the week as the Operating Day in question.  The QSE is charged for its net obligation at a dollar per MW price that is calculated using the total Regulation Up payment ($) to the market and the total of the excess Regulation Up obligation (MW) for the market.

PR 4.4.7.1 (1) indicates that a QSE cannot self-arrange Regulation Up Service in excess of its Regulation Up obligation.  The QSE’s obligation may also be reduced after the Operating Day, in effect making the self arranged quantity greater than the obligation.  However, this will not occur until after the settlement process for the Operating Day obtains new load values and therefore this is not an issue for the DA settlements process.

Payment is calculated by QSE for each hour of the Operating Day within the Day Ahead Market.

| Requirement ID | FR41 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.1 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARUAMT_<Q> = DARUPR * DARUQ_<Q> |  |

| Requirement ID | FR42 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.1 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARUPR = (-1) * PCRUAMTTOT_<M> / DARUQTOT |  |

| Requirement ID | FR43 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.1 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCRUAMTTOT_<M> = ΣQPCRUAMT_<Q>_<M> |  |

| Requirement ID | FR44 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.1 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARUQTOT = ΣQDARUQ_<Q> |  |

| Requirement ID | FR45 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.1 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARUQ_<Q> = DARUONET_<Q> – RUSQ_<Q>_<M> |  |

| Requirement ID | FR46 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.1 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARUONET_<Q> = DARUO_<Q> + DARUCS_<Q> – DARUCP_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| PCRUAMT | $ | Procured Capacity for Reg-Up Amount per QSE in DAM—The DAM Reg-Up payment for QSE Q for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: For further information on this bill determinant, see Section 2.1 |  |  |  |
| DARUO | MW | Day-Ahead Reg-Up Obligation per QSE—The Reg-Up capacity obligation for QSE Q for the DAM for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Obligation data received from the original source system shall be based upon the Load Ratio Share calculated for each of the QSE’s LSEs. ERCOT shall base the LSE Ancillary Service allocation on the hourly Load Ratio Share for the same hour and day of the week, for the most recent day for which Initial Settlement Statements are available. (PR 4.2.1.2 (1)) The obligation value is available to the QSE by 0600 in the Day-Ahead. |  |  |  |
| DARUCS | MW | Reg-Up Capacity Sale per QSE—The total Reg-Up capacity shown in Ancillary Service Trades with QSE Q as a seller for the DAM, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Reg-Up sale value is available from the MMS after the close of the DAM. |  |  |  |
| DARUCP | MW | Reg-Up Capacity Purchase per QSE—The total Reg-Up capacity shown in Ancillary Service Trades with QSE Q as a buyer for the DAM, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Reg-Up purchase value is available from the MMS after the close of the DAM. |  |  |  |
| RUSQ | MW | Reg-Up Supplied Quantity per QSE in DAM—The capacity for Reg-Up to be supplied with Resources represented by QSE Q to meet its Ancillary Service Obligation and/or its Ancillary Service trades, for the DAM, for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Reg-Up supplied quantity is available from the MMS after the close of the DAM. |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| DARUPR | $/MW per hour | Day-Ahead Reg-Up Price—The Day-Ahead Reg-Up price for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments This value is the result of an intermediate calculation and is an input into the DARUAMT calculation. |  |  |  |
| PCRUAMTTOT | $ | Procured Capacity for Reg-Up Amount Total in DAM—The total of the DAM Reg-Up payments for all QSEs for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARUPR calculation. |  |  |  |
| DARUQTOT | MW | Day-Ahead Reg-Up Quantity Total—The sum of every QSE’s portion of its net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resource, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARUPR calculation. |  |  |  |
| DARUQ | MW | Day-Ahead Reg-Up Quantity per QSE—The portion of QSE’s Q net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resources capacity, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARUQTOT calculation. |  |  |  |
| DARUONET | MW | Day-Ahead Reg-Up Obligation Net per QSE—The net Day-Ahead Ancillary Service obligation of QSE Q, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARUQ calculation. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| DARUAMT | $ | Day-Ahead Reg-Up Amount per QSE—QSE’s Q share of the DAM cost for Reg-Up, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| DARUBILLAMT | $ | Day-Ahead Reg-Up Bill Amount per QSE in DAM— The difference between two consecutive daily summed channels of DARUAMT for QSE Q in the Day Ahead Ancillary Service Market. | Settlement System |
| Interval Frequency: 1 / Day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1)  Calculation of DARUAMT is dependent upon the availability of the input bill determinants.

D2)  The settlement process must calculate the Regulation Up Service Payments (PCRUAMT for all QSEs) before attempting to calculate the DARUPR Bill Determinant.

### Assumptions

A1)  The Operations system must prevent a QSE from specifying a self-arranged quantity greater than its obligation, as assigned by ERCOT, per PR 4.4.7.1 (1).

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR47 | PR 4.6.4.2.1 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: PCRUAMT, DARUO, DARUCS, DARUCP, and RUSQ. |
| FR48 | PR 9.4.1 (2) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: DARUAMT and DARUBILLAMT. |
| FR49 | PR 4.6.4.2.1 | CSWM. P03SettleBill Transfer | The following intermediate bill determinants should not be rounded or truncated: DARUPR, PCRUAMTTOT, DARUQTOT, DARUQ, and DARUONET. |
| FR50 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that should be included in the public data extracts. DARUQTOT PCRUAMTTOT DARUPR All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. DARUCP DARUCS DARUO DARUONET RUSQ DARUQ PCRUAMT DARUAMT |
| FR51 | PR 4.6.4.2.1 | CSWM. P03SettleBill Transfer | The following input and intermediate bill determinants should be identified with a unique Market Identifier: PCRUAMTTOT, PCRUAMT, and RUSQ. |
| FR52 | PR 9.2.5 (3) | CSWM. P03SettleBill Transfer | The DARUBILLAMT value shall be calculated as the entire day’s sum of the DARUAMT values for the current channel less the entire day’s sum of the DARUAMT values for the most recent previous channel. |
| FR53 | PR 4.6.4.2.1 | CSWM. P03SettleBill Transfer | DRIVER: The DARUAMT calculation should only be attempted for QSEs for which there exists at least one of the following cuts: DARUO, DARUCS, DARUCP, or RUSQ – for the Operating Day, in the settlement system. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR54 | PR 4.6.4.2.1 | CSWM. P03SettleBill Transfer | If DARUO does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR55 | PR 4.6.4.2.1 | CSWM. P03SettleBill Transfer | If DARUCS does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR56 | PR 4.6.4.2.1 | CSWM. P03SettleBill Transfer | If DARUCP does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR57 | PR 4.6.4.2.1 | CSWM. P03SettleBill Transfer | If RUSQ does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR58 | PR 4.6.4.2.1 | Will be completed August 31 | If PCRUAMT does not exist for any QSE, assume a default value of zero. No error message is necessary. |
| FR59 | PR 4.6.4.2.1 | CSWM. P03SettleBill Transfer | If the DARUQTOT cut equals zero for an interval then the DARUPR cut should automatically set to zero for that same interval to prevent a division by zero error. |

### Specific Database Needs

None

## Regulation Down Service Charge (DARDAMT)

The Regulation Down Service Charge is assessed to a QSE whose net Regulation Down obligation is in excess of its Regulation Down self-arranged quantity.  The QSE’s net obligation is the result of netting the QSE’s self-arranged amounts against its Regulation Down obligation.  The obligation is based upon the QSE’s Load Ratio Share.  ERCOT determines each QSE’s Ancillary Service obligation for DAM settlement based on Load ratio Share values from the most recent Initial settlement data for the same hour and day of the week as the Operating Day in question.  The QSE is charged for its net obligation at a dollar per MW price that is calculated using the total Regulation Down payment ($) to the market and the total of the excess Regulation Down obligation (MW) for the market.

PR 4.4.7.1 (1) indicates that a QSE cannot self-arrange Regulation Down Service in excess of its Regulation Down obligation.  The QSE’s obligation may also be reduced after the Operating Day, in effect making the self arranged quantity greater than the obligation.  However, this will not occur until after the settlement process for the Operating Day obtains new load values and therefore this is not an issue for the DA settlements process.

Payment is calculated by QSE for each hour of the Operating Day within the Day Ahead Market.

| Requirement ID | FR60 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARDAMT_<Q> = DARDPR * DARDQ_<Q> |  |

| Requirement ID | FR61 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARDPR = (-1) * PCRDAMTTOT_<M> / DARDQTOT |  |

| Requirement ID | FR62 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCRDAMTTOT_<M> = ΣQPCRDAMT_<Q>_<M> |  |

| Requirement ID | FR63 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARDQTOT = ΣQDARDQ_<Q> |  |

| Requirement ID | FR64 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARDQ_<Q> = DARDONET_<Q> – RDSQ_<Q>_<M> |  |

| Requirement ID | FR65 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.2 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARDONET_<Q> = DARDO_<Q> + DARDCS_<Q> – DARDCP_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| PCRDAMT | $ | Procured Capacity for Reg-Down Amount per QSE in DAM—The DAM Reg-Down payment for QSE Q for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: For further information on this bill determinant, see Section 2.2 |  |  |  |
| DARDO | MW | Day-Ahead Reg-Down Obligation per QSE—The Reg-Down capacity obligation for QSE Q for the DAM for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Obligation data received from the original source system shall be based upon the Load Ratio Share calculated for each of the QSE’s LSEs. ERCOT shall base the LSE Ancillary Service allocation on the hourly Load Ratio Share for the same hour and day of the week, for the most recent day for which Initial Settlement Statements are available. (PR 4.2.1.2 (1)) The obligation value is available to the QSE by 0600 in the Day-Ahead. |  |  |  |
| DARDCS | MW | Reg-Down Capacity Sale per QSE—The total Reg-Down capacity shown in Ancillary Service Trades with QSE Q as a seller for the DAM, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Reg-Down sale value is available from the MMS after the close of the DAM. |  |  |  |
| DARDCP | MW | Reg-Down Capacity Purchase per QSE—The total Reg-Down capacity shown in Ancillary Service Trades with QSE Q as a buyer for the DAM, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Reg-Down purchase value is available from the MMS after the close of the DAM. |  |  |  |
| RDSQ | MW | Reg-Down Supplied Quantity per QSE in DAM—The capacity for Reg-Down to be supplied with Resources represented by QSE Q to meet its Ancillary Service Obligation and/or its Ancillary Service trades, for the DAM, for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Reg-Down supplied quantity is available from the MMS after the close of the DAM. |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| DARDPR | $/MW per hour | Day-Ahead Reg-Down Price—The Day-Ahead Reg-Down price for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARDAMT calculation. |  |  |  |
| PCRDAMTTOT | $ | Procured Capacity for Reg-Down Amount Total in DAM—The total of the DAM Reg-Down payments for all QSEs for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARDPR calculation. |  |  |  |
| DARDQTOT | MW | Day-Ahead Reg-Down Quantity Total—The sum of every QSE’s portion of its net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resource, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARDPR calculation. |  |  |  |
| DARDQ | MW | Day-Ahead Reg-Down Quantity per QSE—The portion of QSE’s Q net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resources capacity, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARDQTOT calculation. |  |  |  |
| DARDONET | MW | Day-Ahead Reg-Down Obligation Net per QSE—The net Day-Ahead Ancillary Service obligation of QSE Q, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARDQ calculation. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| DARDAMT | $ | Day-Ahead Reg-Down Amount per QSE—QSE’s Q share of the DAM cost for Reg-Down, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| DARDBILLAMT | $ | Day-Ahead Reg-Down Bill Amount per QSE in DAM— The difference between two consecutive daily summed channels of DARDAMT for QSE Q in the Day Ahead Ancillary Service Market. | Settlement System |
| Interval Frequency: 1 / Day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1)  Calculation of DARDAMT is dependent upon the availability of the input bill determinants.

D2)  The settlement process must calculate the Regulation Down Service Payments (PCRDAMT for all QSEs) before attempting to calculate the DARDPR Bill Determinant.

### Assumptions

A1)  The Operations system must prevent a QSE from specifying a self-arranged quantity greater than its obligation, as assigned by ERCOT, per PR 4.4.7.1 (1).

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR66 | PR 4.6.4.2.2 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: PCRDAMT, DARDO, DARDCS, DARDCP, and RDSQ. |
| FR67 | PR 9.4.1 (2) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: DARDAMT and DARDBILLAMT. |
| FR68 | PR 4.6.4.2.2 | CSWM. P03SettleBill Transfer | The following intermediate bill determinants should not be rounded or truncated: DARDPR, PCRDAMTTOT, DARDQTOT, DARDQ, and DARDONET. |
| FR69 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that should be included in the public data extracts. DARDQTOT PCRDAMTTOT DARDPR All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. DARDCP DARDCS DARDO DARDONET RDSQ DARDQ PCRDAMT DARDAMT |
| FR70 | PR 4.6.4.2.2 | CSWM. P03SettleBill Transfer | The following input and intermediate bill determinants should be identified with a unique Market Identifier: PCRDAMTTOT, PCRDAMT, and RDSQ. |
| FR71 | PR 9.2.5 (3) | CSWM. P03SettleBill Transfer | The DARDBILLAMT value shall be calculated as the entire day’s sum of the DARDAMT values for the current channel less the entire day’s sum of the DARDAMT values for the most recent previous channel. |
| FR72 | PR 4.6.4.2.2 | CSWM. P03SettleBill Transfer | DRIVER: The DARDAMT calculation should only be attempted for QSEs for which there exists at least one of the following cuts: DARDO, DARDCS, DARDCP, or RDSQ – for the Operating Day, in the settlement system. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR73 | PR 4.6.4.2.2 | CSWM. P03SettleBill Transfer | If DARDO does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR74 | PR 4.6.4.2.2 | CSWM. P03SettleBill Transfer | If DARDCS does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR75 | PR 4.6.4.2.2 | CSWM. P03SettleBill Transfer | If DARDCP does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR76 | PR 4.6.4.2.2 | CSWM. P03SettleBill Transfer | If RDSQ does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR77 | PR 4.6.4.2.2 | Will be completed August 31 | If PCRDAMT does not exist for any QSE, assume a default value of zero. No error message is necessary. |
| FR78 | PR 4.6.4.2.2 | CSWM. P03SettleBill Transfer | If the DARDQTOT cut equals zero for an interval then the DARDPR cut should automatically set to zero for that same interval to prevent a division by zero error. |

### Specific Database Needs

None

## Responsive Reserve Service Charge (DARRAMT)

The Responsive Reserve Service Charge is assessed to a QSE whose net Responsive Reserve obligation is in excess of its Responsive Reserve self-arranged quantity.  The QSE’s net obligation is the result of netting the QSE’s self-arranged amounts against its Responsive Reserve obligation.  The obligation is based upon the QSE’s Load Ratio Share.  ERCOT determines each QSE’s Ancillary Service obligation for DAM settlement based on Load ratio Share values from the most recent Initial settlement data for the same hour and day of the week as the Operating Day in question.  The QSE is charged for its net obligation at a dollar per MW price that is calculated using the total Responsive Reserve payment ($) to the market and the total of the excess Responsive Reserve obligation (MW) for the market.

PR 4.4.7.1 (1) indicates that a QSE cannot self-arrange Responsive Reserve Service in excess of its Responsive Reserve obligation.  The QSE’s obligation may also be reduced after the Operating Day, in effect making the self arranged quantity greater than the obligation.  However, this will not occur until after the settlement process for the Operating Day obtains new load values and therefore this is not an issue for the DA settlements process.

Payment is calculated by QSE for each hour of the Operating Day within the Day Ahead Market.

| Requirement ID | FR79 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.3 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARRAMT_<Q> = DARRPR * DARRQ_<Q> |  |

| Requirement ID | FR80 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.3 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARRPR = (-1) * PCRRAMTTOT_<M> / DARRQTOT |  |

| Requirement ID | FR81 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.3 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCRRAMTTOT_<M> = ΣQPCRRAMT_<Q>_<M> |  |

| Requirement ID | FR82 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.3 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARRQTOT = ΣQDARRQ_<Q> |  |

| Requirement ID | FR83 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.3 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARRQ_<Q> = DARRONET_<Q> – RRSQ_<Q>_<M> |  |

| Requirement ID | FR84 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.3 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DARRONET_<Q> = DARRO_<Q> + DARRCS_<Q> – DARRCP_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| PCRRAMT | $ | Procured Capacity for Responsive Reserve Amount per QSE in DAM—The DAM Responsive Reserve payment for QSE Q for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: For further information on this bill determinant, see Section 2.3 |  |  |  |
| DARRO | MW | Day-Ahead Responsive Reserve Obligation per QSE—The Responsive Reserve capacity obligation for QSE Q for the DAM for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Obligation data received from the original source system shall be based upon the Load Ratio Share calculated for each of the QSE’s LSEs. ERCOT shall base the LSE Ancillary Service allocation on the hourly Load Ratio Share for the same hour and day of the week, for the most recent day for which Initial Settlement Statements are available. (PR 4.2.1.2 (1)) The obligation value is available to the QSE by 0600 in the Day-Ahead. |  |  |  |
| DARRCS | MW | Responsive Reserve Capacity Sale per QSE—The total Responsive Reserve capacity shown in Ancillary Service Trades with QSE Q as a seller for the DAM, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Responsive Reserve sale value is available from the MMS after the close of the DAM. |  |  |  |
| DARRCP | MW | Responsive Reserve Capacity Purchase per QSE—The total Responsive Reserve capacity shown in Ancillary Service Trades with QSE Q as a buyer for the DAM, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Responsive Reserve purchase value is available from the MMS after the close of the DAM. |  |  |  |
| RRSQ | MW | Responsive Reserve Supplied Quantity per QSE in DAM—The capacity for Responsive Reserve to be supplied with Resources represented by QSE Q to meet its Ancillary Service Obligation and/or its Ancillary Service trades, for the DAM, for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Responsive Reserve supplied quantity is available from the MMS after the close of the DAM. |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| DARRPR | $/MW per hour | Day-Ahead Responsive Reserve Price—The Day-Ahead Responsive Reserve price for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARRAMT calculation. |  |  |  |
| PCRRAMTTOT | $ | Procured Capacity for Responsive Reserve Amount Total in DAM—The total of the DAM Responsive Reserve payments for all QSEs for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARRPR calculation. |  |  |  |
| DARRQTOT | MW | Day-Ahead Responsive Reserve Quantity Total—The sum of every QSE’s portion of its net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resource, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARRPR calculation. |  |  |  |
| DARRQ | MW | Day-Ahead Responsive Reserve Quantity per QSE—The portion of QSE’s Q net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resources capacity, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARRQTOT calculation. |  |  |  |
| DARRONET | MW | Day-Ahead Responsive Reserve Obligation Net per QSE—The net Day-Ahead Ancillary Service obligation of QSE Q for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DARRQ calculation. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| DARRAMT | $ | Day-Ahead Responsive Reserve Amount per QSE—QSE’s Q share of the DAM cost for Responsive Reserve, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| DARRBILLAMT | $ | Day-Ahead Responsive Reserve Bill Amount per QSE in DAM— The difference between two consecutive daily summed channels of DARRAMT for QSE Q in the Day Ahead Ancillary Service Market. | Settlement System |
| Interval Frequency: 1 / Day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. |  |  |  |

### Data Dependencies

D1)  Calculation of DARRAMT is dependent upon the availability of the input bill determinants.

D2)  The settlement process must calculate the Responsive Reserve Service Payments (PCRRAMT for all QSEs) before attempting to calculate the DARRPR Bill Determinant.

### Assumptions

A1)  The Operations system must prevent a QSE from specifying a self-arranged quantity greater than its obligation, as assigned by ERCOT, per PR 4.4.7.1 (1).

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR85 | PR 4.6.4.2.3 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: PCRRAMT, DARRO, DARRCS, DARRCP, and RRSQ. |
| FR86 | PR 9.4.1 (2) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: DARRAMT and DARRBILLAMT. |
| FR87 | PR 4.6.4.2.3 | CSWM. P03SettleBill Transfer | The following intermediate bill determinants should not be rounded or truncated: DARRPR, PCRRAMTTOT, DARRQTOT, DARRQ, and DARRONET. |
| FR88 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that should be included in the public data extracts. DARRQTOT PCRRAMTTOT DARRPR All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. DARRCP DARRCS DARRO DARRONET RRSQ DARRQ PCRRAMT DARRAMT |
| FR89 | PR 4.6.4.2.3 | CSWM. P03SettleBill Transfer | The following input and intermediate bill determinants should be identified with a unique Market Identifier: PCRRAMTTOT, PCRRAMT, and RRSQ. |
| FR90 | PR 9.2.5 (3) | CSWM. P03SettleBill Transfer | The DARRBILLAMT value shall be calculated as the entire day’s sum of the DARRAMT values for the current channel less the entire day’s sum of the DARRAMT values for the most recent previous channel. |
| FR91 | PR 4.6.4.2.3 | CSWM. P03SettleBill Transfer | DRIVER: The DARRAMT calculation should only be attempted for QSEs for which there exists at least one of the following cuts: DARRO, DARRCS, DARRCP, or RRSQ – for the Operating Day, in the settlement system. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR92 | PR 4.6.4.2.3 | CSWM. P03SettleBill Transfer | If DARRO does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR93 | PR 4.6.4.2.3 | CSWM. P03SettleBill Transfer | If DARRCS does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR94 | PR 4.6.4.2.3 | CSWM. P03SettleBill Transfer | If DARRCP does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR95 | PR 4.6.4.2.3 | CSWM. P03SettleBill Transfer | If RRSQ does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR96 | PR 4.6.4.2.3 | Will be completed August 31 | If PCRRAMT does not exist for any QSE, assume a default value of zero. No error message is necessary. |
| FR97 | PR 4.6.4.2.3 | CSWM. P03SettleBill Transfer | If the DARRQTOT cut equals zero for an interval then the DARRPR cut should automatically set to zero for that same interval to prevent a division by zero error. |

### Specific Database Needs

None

## Non-Spin Service Charge (DANSAMT)

The Non-Spin Service Charge is assessed to a QSE whose net Non-Spin obligation is in excess of its Non-Spin self-arranged quantity.  The QSE’s net obligation is the result of netting the QSE’s self-arranged amounts against its Non-Spin obligation.  The obligation is based upon the QSE’s Load Ratio Share.  ERCOT determines each QSE’s Ancillary Service obligation for DAM settlement based on Load ratio Share values from the most recent Initial settlement data for the same hour and day of the week as the Operating Day in question.  The QSE is charged for its net obligation at a dollar per MW price that is calculated using the total Non-Spin payment ($) to the market and the total of the excess Non-Spin obligation (MW) for the market.

PR 4.4.7.1 (1) indicates that a QSE cannot self-arrange Non-Spin Service in excess of its Non-Spin obligation.  The QSE’s obligation may also be reduced after the Operating Day, in effect making the self arranged quantity greater than the obligation.  However, this will not occur until after the settlement process for the Operating Day obtains new load values and therefore this is not an issue for the DA settlements process.

Payment is calculated by QSE for each hour of the Operating Day within the Day Ahead Market.

| Requirement ID | FR98 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.4 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DANSAMT_<Q> = DANSPR * DANSQ_<Q> |  |

| Requirement ID | FR99 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.4 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DANSPR = (-1) * PCNSAMTTOT_<M> / DANSQTOT |  |

| Requirement ID | FR100 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.4 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description PCNSAMTTOT_<M> = ΣQPCNSAMT_<Q>_<M> |  |

| Requirement ID | FR101 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.4 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DANSQTOT = ΣQDANSQ_<Q> |  |

| Requirement ID | FR102 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.4 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DANSQ_<Q> = DANSONET_<Q> – NSSQ_<Q>_<M> |  |

| Requirement ID | FR103 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) | PR 4.6.4.2.4 |
| Traceability to Sub-Process Maps | Will be completed August 31, 2006 |
| Description DANSONET_<Q> = DANSO_<Q> + DANSCS_<Q> – DANSCP_<Q> |  |

### Input Bill Determinants

| Variable | Unit | Definition | Source |
| --- | --- | --- | --- |
| PCNSAMT | $ | Procured Capacity for Non-Spin Amount per QSE in DAM—The DAM Non-Spin payment for QSE Q for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: For further information on this bill determinant, see Section 2.4 |  |  |  |
| DANSO | MW | Day-Ahead Non-Spin Obligation per QSE—The Non-Spin capacity obligation for QSE Q for the DAM for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Obligation data received from the original source system shall be based upon the Load Ratio Share calculated for each of the QSE’s LSEs. ERCOT shall base the LSE Ancillary Service allocation on the hourly Load Ratio Share for the same hour and day of the week, for the most recent day for which Initial Settlement Statements are available. (PR 4.2.1.2 (1)) The obligation value is available to the QSE by 0600 in the Day-Ahead. |  |  |  |
| DANSCS | MW | Non-Spin Capacity Sale per QSE—The total Non-Spin capacity shown in Ancillary Service Trades with QSE Q as a seller for the DAM, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Non-Spin sale value is available from the MMS after the close of the DAM. |  |  |  |
| DANSCP | MW | Non-Spin Capacity Purchase per QSE—The total Non-Spin capacity shown in Ancillary Service Trades with QSE Q as a buyer for the DAM, for the hour. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Non-Spin purchase value is available from the MMS after the close of the DAM. |  |  |  |
| NSSQ | MW | Non-Spin Supplied Quantity per QSE in DAM—The capacity for Non-Spin to be supplied with Resources represented by QSE Q to meet its Ancillary Service Obligation and/or its Ancillary Service trades, for the DAM, for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | MMS |
| Interval Frequency: 1 / hour Additional Comments: The Non-Spin supplied quantity is available from the MMS after the close of the DAM. |  |  |  |

### Intermediate Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| DANSPR | $/MW per hour | Day-Ahead Non-Spin Price—The Day-Ahead Non-Spin price for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DANSAMT calculation. |  |  |  |
| PCNSAMTTOT | $ | Procured Capacity for Non-Spin Amount Total in DAM—The total of the DAM Non-Spin payments for all QSEs for the hour. This bill determinant needs to be identified with a DAM Market Identifier to ensure proper classification. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DANSPR calculation. |  |  |  |
| DANSQTOT | MW | Day-Ahead Non-Spin Quantity Total—The sum of every QSE’s portion of its net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resource, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DANSPR calculation. |  |  |  |
| DANSQ | MW | Day-Ahead Non-Spin Quantity per QSE—The portion of QSE’s Q net Day-Ahead Ancillary Service obligation that is not self-supplied with its Resources capacity, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DANSQTOT calculation. |  |  |  |
| DANSONET | MW | Day-Ahead Non-Spin Obligation Net per QSE—The net Day-Ahead Ancillary Service obligation of QSE Q, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: This value is the result of an intermediate calculation and is an input into the DANSQ calculation. |  |  |  |

### Output Bill Determinants

| Variable | Unit | Definition | Destination |
| --- | --- | --- | --- |
| DANSAMT | $ | Day-Ahead Non-Spin Amount per QSE—QSE’s Q share of the DAM cost for Non-Spin, for the hour. | Settlement System |
| Interval Frequency: 1 / hour Additional Comments: None |  |  |  |
| DANSBILLAMT | $ | Day-Ahead Non-Spin Bill Amount per QSE in DAM— The difference between two consecutive daily summed channels of DARDAMT for QSE Q in the Day Ahead Ancillary Service Market. | Settlement System |
| Interval Frequency: 1 / Day Additional Comments: This bill determinant is not specifically documented in the Nodal Protocols but is required as a result of Section 9 Statements and Invoices Protocol requirements. v |  |  |  |

### Data Dependencies

D1)  Calculation of DANSAMT is dependent upon the availability of the input bill determinants.

D2)  The settlement process must calculate the Non-Spin Service Payments (PCNSAMT for all QSEs) before attempting to calculate the DANSPR Bill Determinant.

### Assumptions

A1)  The Operations system must prevent a QSE from specifying a self-arranged quantity greater than its obligation, as assigned by ERCOT, per PR 4.4.7.1 (1).

Business Rules

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR104 | PR 4.6.4.2.4 | CSWM. P03SettleBill Transfer | The input bill determinants should not be rounded or truncated. This includes the following bill determinants: PCNSAMT, DANSO, DANSCS, DANSCP, and NSSQ. |
| FR105 | PR 9.4.1 (2) | CSWM. P03SettleBill Transfer | The following output bill determinants should be rounded to two (2) decimal places: DANSAMT and DANSBILLAMT. |
| FR106 | PR 4.6.4.2.4 | CSWM. P03SettleBill Transfer | The following intermediate bill determinants should not be rounded or truncated: DANSPR, PCNSAMTTOT, DANSQTOT, DANSQ, and DANSONET. |
| FR107 | PR 12.3 (C) | CSWM. P03SettleBill Transfer | The following bill determinants shall be identified within the Settlement System as a bill determinant that should be included in the public data extracts. DANSQTOT PCNSAMTTOT DANSPR All of the other input, intermediate, and output bill determinants defined below should be identified within the Settlement System as a bill determinant that should be included in the private data extracts. DANSCP DANSCS DANSO DANSONET NSSQ DANSQ PCNSAMT DANSAMT |
| FR108 | PR 4.6.4.2.4 | CSWM. P03SettleBill Transfer | The following input and intermediate bill determinants should be identified with a unique Market Identifier: PCNSAMTTOT, PCNSAMT, and NSSQ. |
| FR109 | PR 9.2.5 (3) | CSWM. P03SettleBill Transfer | The DANSBILLAMT value shall be calculated as the entire day’s sum of the DANSAMT values for the current channel less the entire day’s sum of the DANSAMT values for the most recent previous channel. |
| FR110 | PR 4.6.4.2.4 | CSWM. P03SettleBill Transfer | DRIVER: The DANSAMT calculation should only be attempted for QSEs for which there exists at least one of the following cuts: DANSO, DANSCS, DANSCP, or NSSQ – for the Operating Day, in the settlement system. |

### Business Defined Errors

| Req. ID | Protocol(s) | Sub-Process Map | Description |
| --- | --- | --- | --- |
| FR111 | PR 4.6.4.2.4 | CSWM. P03SettleBill Transfer | If DANSO does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR112 | PR 4.6.4.2.4 | CSWM. P03SettleBill Transfer | If DANSCS does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR113 | PR 4.6.4.2.4 | CSWM. P03SettleBill Transfer | If DANSCP does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR114 | PR 4.6.4.2.4 | CSWM. P03SettleBill Transfer | If NSSQ does not exist for the QSE, assume a default value of zero. No error message is necessary. |
| FR115 | PR 4.6.4.2.4 | Will be completed August 31 | If PCNSAMT does not exist for any QSE, assume a default value of zero. No error message is necessary. |
| FR116 | PR 4.6.4.2.4 | CSWM. P03SettleBill Transfer | If the DANSQTOT cut equals zero for an interval then the DANSPR cut should automatically set to zero for that same interval to prevent a division by zero error. |

### Specific Database Needs

None

# Supplementary Requirements

## Performance Requirements

Lodestar must be able to perform its full set of Nodal settlement calculations. Performance characteristics will be determined and detailed in the conceptual and detailed design.

## Legal and Regulatory requirements

All Lodestar financial data must be saved in accordance with standard accounting principles as dictated by the ERCOT Texas Nodal Protocols.

## System Security Requirements

Lodestar will only be accessed directly by internal ERCOT personnel either through thin client UI, Citrix environment, or thick client on employee desktops. Market Participants will be able to access their statements and invoices on ERCOT private web site. ERCOT private web site access will be controlled by digital certificate authentication in accordance with ERCOT security policies.  Additional details will be provided in the Conceptual System Design.

## Back up and Recovery Requirements

All Lodestar Nodal settlement data must be backed up on a daily basis and recoverable within 24 hours.  Additional details will be provided in the Conceptual System Design.

## Availability and Redundancy Requirements

Nodal Lodestar system must be available 7/24. Lodestar is considered a tier 1 system and critical to base ERCOT business processes.  Additional details will be provided in the Conceptual System Design.

## Maintainability Requirements

Lodestar Nodal system will be maintained by internal ERCOT personnel. Service Level Agreements and explicit support will be detailed in the detail design document.

## Daylight Savings Time shall be recognized in Nodal Market settlements.

## Spring Daylight Savings Time Operating Day:

### Spring Daylight Savings Time Operating Day:

### Hour Ending 0300 will be skipped, resulting in 23 hourly intervals for the day.

### The four fifteen minute intervals within Hour Ending 0300 will be skipped, resulting in 92 fifteen minute intervals for the day.

### The sixty one minute intervals within Hour Ending 0300 will be skipped, resulting in 1380 total one minute intervals.

## Fall Daylight Savings Time Operating Day:

### Hour Ending 0200 will be repeated, resulting in 25 hourly intervals for the day.

### The four fifteen minute intervals within Hour Ending 0200 will be repeated, resulting in 100 fifteen minute intervals for the day.

### The sixty one minute intervals within Hour Ending 0200 will be repeated, resulting in 1500 total one minute intervals.

# Protocol Coverage

| Protocol Sub-Section # (To the lowest level of granularity as possible) | Coverage by Requirements (Full/Partial) |
| --- | --- |
| 4.6.4.1.1 | Full |
| 4.6.4.1.2 | Full |
| 4.6.4.1.3 | Full |
| 4.6.4.1.4 | Full |
| 4.6.4.2.1 | Full |
| 4.6.4.2.2 | Full |
| 4.6.4.2.3 | Full |
| 4.6.4.2.4 | Full |
| 9.2.5 | Partial |
| 9.4.1(2) | Partial |
| 12.3(C) | Partial |