| NPRR Number | 076 | NPRR Title | Synchronization of Emergency Electric Curtailment Plan (EECP) Event Realignment |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Approved |
| Date of Decision |  | December 11, 2007 |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, DEFINITIONS 2.2, ACRONYMS AND ABBREVIATIONS 6.5.7.5, Ancillary Services Capacity Monitor 6.5.9.4, Emergency Electric Curtailment Plan 6.5.9.4.1, General Procedures Prior to EECP Operations (NEW) 6.5.9.4.1, EECP Steps |  |  |
| Proposed Effective Date |  | Upon Texas Nodal Market implementation. |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) synchronizes the Nodal Protocols with PRR682, EECP Event Realignment, which was approved by the ERCOT Board of Directors (Board) on November 14, 2006. |  |  |
| Overall Market Benefit |  | Earlier communication of reliability concerns by ERCOT to Market Participants (MPs) and concerned Entities. |  |  |
| Overall Market Impact |  | None. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts |  | ERCOT credit staff and the Credit Work Group (Credit WG) have reviewed NPRR076 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 7/02/07, NPRR076 was posted. - On 7/03/07, the Impact Analysis (IA) was posted. - On 7/10/07, TXU comments were posted. - On 7/16/07, Luminant comments were posted. - On 7/19/07, PRS considered this NPRR. - On 8/31/07, the Demand Side Working Group (DSWG) posted comments. - On 9/11/07, ERCOT Staff posted comments. - On 9/20/07, PRS again considered NPRR076. - On 10/18/07, PRS considered NPRR076. - On 11/14/07, a revised IA was posted. - On 11/15/07, PRS considered the Recommendation Report and revised IA. - On 11/29/07, TAC considered NPRR076. - On 12/11/07, the ERCOT Board of Directors (Board) considered NPRR076 |  |  |
| PRS Decision |  | On 7/19/07, PRS unanimously voted to remand NPRR076 to WMS and direct that the Demand Side Working Group (DSWG) further review the Load Resource selection process in Section 6.5.9.4.1, EECP Steps. PRS asked that WMS provide feedback at the August 23rd PRS meeting. On 9/20/07, PRS unanimously voted to table further consideration of NPRR076 until the October PRS meeting to allow for additional comments. All Market Segments were present for the vote. On 10/18/07, PRS voted to recommend approval of NPRR076 as revised by DSWG. The motion passed by roll-call vote with two opposing votes from the Independent Power Marketer (IPM) Market Segment and five abstentions from the Municipally Owned Utility (MOU)(2), Investor Owned Utility (IOU)(1), Independent Generator (IG)(1), and Independent Retail Electric Provider (IREP)(1) Market Segments. All Market Segments were present for the vote. On 11/15/07, PRS voted to forward the Recommendation Report and IA to TAC. There was one opposing vote from the IPM Market Segment and two abstentions from the MOU Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 7/19/07, ERCOT raised the issue that language allowing ERCOT to create a procedure to deploy Load Resources during EECP Step 2 was removed in the original NPRR076. It was stated that this issue was discussed at the 07/19/07 WMS meeting and it was recommended that DSWG further vet the Load Resource selection process. On 9/20/07, participants expressed concern over the use of a Reserve Discount Factor (RDF) in calculating the ERCOT-wide Physical Response Capability (PRC). Participants opined that the PRC would be an improper legacy carry-over from the zonal market, where it was originally intended to be a short-term solution. Participants also discussed ERCOT Staff comments and the proper use of the XML and verbal dispatch instructions. On 10/18/07, Participants discussed the comments submitted by the DSWG to address the notification procedures. Participants questioned whether PRS should approve an NPRR that inserts a Reserve Discount Factor (RDF) when the Reliability Operations Subcommittee (ROS) is reviewing the discount factor. ERCOT Staff noted that no party had submitted formal comments advocating removal of the RDF; that PRS has no data to support removing the discount factor; and that ERCOT needs the RDF to maintain the functionality and the flexibility needed to move forward and meet nodal implementation deadlines. Some participants agreed that reality shows that the full amount of registered capacity frequently does not come on-line. Therefore, the value assigned to the discount factor may not be correct at this time; however, there is a real need for this factor. On 11/15/07, the increase in the estimated cost of NPRR076 was discussed. ERCOT Staff clarified that due to the additional requirements of NPRR076, mainly in the Market Management System (MMS), the cost of this NPRR076 is estimated at $250,000 - $500,000. |  |  |
| TAC Decision |  | On 11/29/07, TAC voted to recommend approval of NPRR076 and to instruct ERCOT to design the Nodal system in such a way to allow the RDF to be reduced to zero. The motion passed with four opposing votes from the IPM(1), Consumer(1), and IG(2) Market Segments and four abstentions from the IPM(1), IG(2), and MOU(1) Market Segments. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 11/29/07, Market Participants raised concerns regarding the RDF and argued that it should not carry into the nodal market. ERCOT Staff responded that they were currently working on designing the Nodal system so that the RDF could be reduced to zero. ERCOT Staff noted that NPRR076 also provides direction on how to dispatch Load Resources during an EECP event and that approval was needed for timely development of systems. . |  |  |
| Board Action |  | On 12/11/07, the Board approved NPRR076 as recommended by TAC. |  |  |

| ERCOT/Market Segment Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 | Reliability requirements that are applicable in the ERCOT Zonal Market are equally applicable in the ERCOT Nodal Market. |  |
|  | 2 | PRR682 provisions are to be ported across to the ERCOT Nodal Market consistent with the structure, processes and systems that are described in the ERCOT Nodal Protocols and the implementing ERCOT Business Requirements and Conceptual Design Documentation. |  |
|  |  | Impact Area | Monetary Impact |
| Market Cost | 1 | Addition of the calculation for operator and MIS display of the Physical Responsive Capability by the Ancillary Service Capacity Monitor described in ERCOT Nodal Protocol Section 6.5.7.5 | To be determined. |
|  | 2 | Addition of the requirement that ERCOT develop criteria and a process for assigning Load Resources, controlled by high-set under-frequency relays, to one of two groups of 50% of the Load Resource capacity following the DRUC. | To be determined. |
|  | 3 | Posting of the Load Resource group assignments from 2 above on the MIS Certified Area and development of dispatch provisions for the actual deployment. | To be determined. |
|  | 4 |  |  |
|  |  | Impact Area | Monetary Impact |
| Market Benefit | 1 | Timely calculation of the PRC consist with telemetered parameters available to the Ancillary Service Capacity Monitor | To be determined. |
|  | 2 | Items 2 an 3 above allow ERCOT operations the ability to deploy Load Resources consistent with system needs. | To be determined. |
| Additional Qualitative Information | 1 | n/a |  |
| Other | 1 |  |  |
| Comments | 2 |  |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| TXU 071007 | Proposed language to increase the array of communications channels available to ERCOT Staff to advise MPs and concerned Entities of system reliability concerns; and to involve effective communication of EECP triggers. |
| Luminant 071607 | Proposed removing the definition and references to Total Responsive Capability (TRC). |
| DSWG 083107 | Recommendation as to how the two groups of Load Resources should be deployed in Step 2. |
| ERCOT 091107 | Offered clarifying language regarding the proper XML format. |
| DSWG 101607 | Offered clarifying language regarding the timing of the XML and verbal dispatch. |

| Original Sponsor |  |
| --- | --- |
| Name | Trip Doggett on behalf of TPTF |
| E-mail Address | tdoggett@ercot.com |
| Company | ERCOT |

Proposed Nodal Protocol Language Revision

## DEFINITIONS

Physical Responsive Capability (PRC)

A representation of the total amount of system wide online capability that has a high probability of being able to quickly respond to system disturbances.

Reserve Discount Factor (RDF)

A representation of the average amount of system wide capability that, for whatever reason, is historically undeliverable during periods of high system demand.  The RDF will be verified by ERCOT and then approved by the ROS.

## ACRONYMS AND ABBREVIATIONS

PRC		Physical Responsive Capability

RDF		Reserve Discount Factor

6.5.7.5	Ancillary Services Capacity Monitor

(1)	ERCOT shall calculate the following every 10 seconds and provide Real-Time summaries to ERCOT operators and all Market Participants using the MIS Secure Area and ICCP, giving updates of calculations every 10 seconds, which show the Real-Time total system amount of:

(a)	Responsive Reserve Capacity from Generation Resources;

(b)	Responsive Reserve Capacity from Load Resources excluding Controllable Load Resources;

(c)	Responsive Reserve Capacity from Controllable Load Resources;

(d)	Non-Spinning Reserve available from On-Line Generation Resources with Energy Offer Curves;

(e)	Non-Spinning Reserve available from undeployed Load Resources;

(f)	Non-Spinning Reserve available from Off-Line Generation Resources;

(g)	Non-Spinning Reserve available from Resources with Output Schedules;

(h)	Undeployed Reg-Up and undeployed Reg-Down;

(i)	Available capacity with Energy Offer Curves in the ERCOT System that can be used to increase Base Points in SCED;

(j)	Available capacity with Energy Offer Curves in the ERCOT System that can be used to decrease Base Points in SCED;

(k)	Available capacity without Energy Offer Curves in the ERCOT System that can be used to increase Base Points in SCED;

(l)	Available capacity without Energy Offer Curves in the ERCOT System that can be used to decrease Base Points in SCED; and.

(m)	The ERCOT-wide Physical Responsive Capability (PRC) calculated as follows:

generationresourcei=onlineonlinegenerationAllresources

PRC1 =			Min(Max((RDF*HSL – Actual Net Telemetered Output)i , 0.0) , 0.2*RDF*HSLi)

generationresourcei=onlineonlinegenerationAllresources

PRC2 =	((Hydro-synchronous condenser output)i as qualified by Operating Guide Section 2.5.2.3, Types of Responsive Reserve))

PRC =	PRC1 + PRC2

Where:

PRC1 = Generation On-line greater than 0 MW;

PRC2 = Hydro-synchronous condenser output in MW;

PRC = Physical Responsive Capability in MW;

RDF = The currently approved Reserve Discount Factor.

(2)	Each QSE shall operate Resources providing Ancillary Service capacity to meet its obligations. If a QSE experiences temporary conditions where its total obligation for providing Ancillary Service can not be met on the QSE's Resources, then the QSE may add additional capability from other Resources that it represents.  It adds that capability by changing the Resource Status and updating the Ancillary Service Schedules of the affected Resources and notifying ERCOT under Section 6.4.8.1, Evaluation and Maintenance of Ancillary Service Capacity Sufficiency.  If the QSE is unable to meet its total obligations to provide committed Ancillary Services capacity, the QSE shall notify ERCOT immediately of the expected duration of the QSE’s inability to meet its obligations.  ERCOT shall determine whether replacement Ancillary Services will be procured on behalf of the affected QSE according to Section 6.4.8.1, Evaluation and Maintenance of Ancillary Service Capacity Sufficiency.

6.5.9.4	Emergency Electric Curtailment Plan

(1)	At times it may be necessary to reduce ERCOT System Demand because of a temporary decrease in available electricity supply.  To provide orderly, predetermined procedures for curtailing Demand during such emergencies, ERCOT shall initiate and coordinate the implementation of the Emergency Electric Curtailment Plan (EECP) following the steps set forth below in Section 6.5.9.4.2, EECP Steps.

(2)	The goal of the EECP is to provide for maximum possible continuity of service while maintaining the integrity of the ERCOT System to reduce the chance of cascading Outages.

(3)	ERCOT’s operating procedures must meet the following goals:

(a)	Use of market processes to the fullest extent practicable without jeopardizing the reliability of the ERCOT System;

(b)	Use of Responsive Reserve Services and other Ancillary Services to the extent permitted by ERCOT System conditions;

(c)	Maximum use of ERCOT System capability;

(d)	Maintenance of station service for nuclear-powered Generation Resources;

(e)	Securing startup power for Generation Resources;

(f)	Operation of Generation Resources during loss of communication with ERCOT; and

(g)	Restoration of service to Loads in the manner defined in the Operating Guides.

(4)	ERCOT is responsible for coordinating with QSEs, TSPs, and DSPs to monitor ERCOT System conditions, initiating the EECP steps, notifying all QSEs, and coordinating the implementation of the EECP steps while maintaining transmission security limits.

(5)	ERCOT, at management’s discretion, may at any time issue an appeal through the public news media for voluntary energy conservation.

(6)	During the EECP, ERCOT has the authority to obtain energy from non-ERCOT Control Areas using the DC Ties or by using Block Load Transfers (BLTs) to move load to non-ERCOT Control Areas.  ERCOT maintains the authority to curtail energy schedules flowing into or out of the ERCOT System across the DC Ties in accordance with NERC scheduling guidelines.

(7)	Some of the EECP steps are not applicable if transmission security violations exist. There may be insufficient time to implement all EECP steps in sequence, however, to the extent practicable, ERCOT shall use Ancillary Services that QSEs have made available in the market to maintain or restore reliability.

(8)	ERCOT may immediately implement Step 3 of the EECP any time the steady-state system frequency is below 59.8 Hz and shall immediately implement Step 3 any time the steady-state frequency is below 59.5 Hz.

(9)	Percentages for Step 3 Load shedding will be based on the previous year’s TSP peak Loads, as reported to ERCOT, and must be reviewed by ERCOT and modified annually as required.

6.5.9.4.1	General Procedures Prior to EECP Operations

- (1)	Prior to declaring EECP Step 1 detailed in Section 6.5.9.4.2, EECP Steps, ERCOT may perform the following operations consistent with Good Utility Practice:

- Provide Dispatch Instructions to QSEs for specific Resources to operate at an Emergency Base Point to maximize Resource deployment so as to increase Responsive Reserve levels on other Resources;

- Commit available Resources as necessary that can respond in the timeframe of the emergency.  Such commitments will be settled using the HRUC process;

(c)	Start RMR units available in the time frame of the emergency. RMR units should be loaded to full capability;

(d)	Issue Dispatch Instructions to QSEs to suspend any ongoing ERCOT required generating unit or Resource performance testing;

(e)	Utilize available resources providing Non-Spin Reserve Services as required; and

(f)	ERCOT shall use the PRC to determine the appropriate Emergency Notification and EECP Steps.

(2)	In addition, ERCOT may issue an appeal through the public news media for voluntary load reduction if authorized by the ERCOT Chief Executive Officer or its designee based on an evaluation of existing and anticipated system conditions.

6.5.9.4.2	EECP Steps

(1)	Step 1 -

Maintain a sum total of 2,300 MW  that results from adding 1) the amount of ERCOT Physical Responsive Capability (PRC) MW (refer to Section 6.5.7.5) and 2) the amount of RRS MW which is supplied from Load Resources.

(a)	ERCOT will:

(i)	Notify the Southwest Power Pool Security Coordinator;

(ii)	Initiate manual HRUC Dispatch Instructions to Generation Resources available and off-line that can perform within the expected timeframe of the emergency;

- Use available DC Tie import capacity not already being used and inquire about availability of BLTs;

(b)	QSEs will notify ERCOT of any Resources uncommitted but available in the timeframe of the emergency.

(2)	Step 2 -

Maintain a sum total of 1,750 MW that results from adding 1) the amount of ERCOT Physical Responsive Capability (PRC) MW (refer to Section 6.5.7.5) and 2) the amount of RRS MW which is supplied from Load Resources..

(a)	In addition to the measures associated with Step 1, ERCOT will:

(i)	Instruct TSPs and DSPs to reduce Customers load by using distribution voltage reduction measures, if deemed beneficial by the TSP or DSP;

(ii)	Instruct QSEs to deploy Responsive Reserve that is supplied from Load Resources (controlled by high-set under-frequency relays) in accordance with the following:

- Instruct QSEs to deploy half of the Responsive Reserve that is supplied from Load Resources (controlled by high-set under-frequency relays) by instructing the QSE representing the specific Load Resource to interrupt Group 1 Load Resources providing Responsive Reserve.  QSEs shall deploy Load Resources according to the group designation and will be given some discretion to deploy additional Load Resources from Group 2 if Load Resource operational considerations require such.  ERCOT shall issue notification of the deployment via XML message.  ERCOT shall follow this XML notification with a Hotline VDI, which shall initiate the ten-minute deployment period.  .

- At the discretion of the ERCOT Operator, instruct QSEs to deploy the remaining Responsive Reserve that is supplied from Load Resources (controlled by high-set under-frequency relays) by instructing the QSE representing the specific Load Resource to interrupt Group 2 Load Resources providing Responsive Reserve. ERCOT shall issue notification of the deployment via XML message.  ERCOT shall follow this XML notification with a Hotline VDI, which shall initiate the ten-minute deployment period..

- The ERCOT Operator may deploy both of the groups of Load Resources providing Responsive Reserves at the same time.  ERCOT shall issue notification of the deployment via XML message.  ERCOT shall follow this XML notification with a Hotline VDI, which shall initiate the ten-minute deployment period; and

- ERCOT shall post a list of Load Resources on the MIS Certified Area immediately following the DRUC for each QSE with a Load Resource obligation which may be deployed to interrupt under paragraph (1), Group 1 and paragraph (2), Group 2.   ERCOT shall develop a process for determining which individual Load Resources to place in Group 1 and which to place in Group 2.  ERCOT Procedures shall select Group 1 and Group 2 based on a random sampling of individual Load Resources.  At ERCOT’s discretion, ERCOT may deploy all Load Resources at any given time during EECP Step 2.

(iii)	With the approval of the affected Non-ERCOT control area, may instruct TSPs or DSPs to implement BLTs, which transfer load from the ERCOT control area to Non-ERCOT control areas.  Use of a BLT will be defined in the ERCOT Operating Guides.

(iv)	Unless such a media appeal is already in effect, ERCOT staff shall issue an appeal through the public news media for voluntary energy conservation.

(b)

Confidentiality requirements regarding transmission operations and system capacity information will be lifted, as needed to restore reliability.

(3)	Step 3 -

Maintain system frequency at 59.8 Hz or greater.

(a)	In addition to measures associated with Steps 1 and 2, ERCOT will direct all TSPs and  DSPs, or their agents to shed firm load, in 100 MW blocks, as documented in the ERCOT Operating Guides in order to maintain a steady state system frequency of 59.8 Hz.

(b)	In addition to measures associated with Steps 1 and 2, TSPs and DSPs will keep in mind the need to protect the safety and health of the community and the essential human needs of the citizens.  Whenever possible, TSPs and DSPs shall not manually drop load connected to under-frequency relays during the implementation of the Emergency Electric Curtailment Plan.