| PRR Number | 742 | PRR Title | Balancing Energy Price Adjustment Due to OOMC, OOME and RRS Deployments During Alert and Emergency Notice Conditions |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Rejected |
| Date of Decision |  | December 13, 2007 |  |  |
| Protocol Section Requiring Revision |  | 6.8.1.12, Payment for Balancing Energy Provided from Ancillary Services During the Operating Period |  |  |
| Proposed Effective Date |  | TBD. |  |  |
| Priority & Rank Assigned |  | Not applicable. |  |  |
| Revision Description |  | This Protocol Revision Request (PRR) is an alternative to PRR 739. It adjusts the Market Clearing Price of Energy (MCPE) when the deployment of Out of Merit Capacity (OOMC), or Out of Merit Energy (OOME) or Responsive Reserve Service (RRS) occurs during an Alert or Emergency Notice condition. This adjustment is similar to that applied to Non-Spin Reserve Service as described in PRR 650, Balancing Energy Price Adjustment Due to Non-Spinning Reserve Service Energy Deployment, which was approved by the ERCOT Board on August 15, 2007. However, in this PRR, these adjustments are effective only during intervals in which an ERCOT-declared Alert or Emergency Notice condition exists. Although ex-post pricing is not a perfect solution, it does allow the market to establish prices during emergency conditions rather than an administratively-set price. |  |  |
| Overall Market Benefit |  | This proposal should result in accurate price signals for OOMC, OOME, and RRS deployed during Alert and/or Emergency Notice conditions. |  |  |
| Consumer Impact |  | This proposal may increase the MCPE (after re-settlement), however this should be somewhat offset by reduction in OOME and OOMC uplift charges during Alert or Emergency Notice conditions. Also, the proposed price adjustment may stimulate participation by interruptible Loads, reducing the likelihood for firm-Load shedding. |  |  |
| Credit Implications |  | Modifications to MCPE calculation may affect prices paid to or by Market Participants. |  |  |
| Procedural History |  | - On 10/25/07, PRR742 was posted. - On 10/31/07, the motion to grant PRR742 Urgent Status failed via email vote. - On 11/5/07, Exelon comments were posted. - On 12/06/07, ERCOT Staff comments were posted. - On 12/13/07, PRS considered PRR742. |  |  |
| PRS Decision |  | On 12/13/07, PRS voted to reject PRR742 with one abstention from the Independent Power Marketer (IPM) Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 12/13/07, participants stated that there have been significant market changes since the inception of PRR742 and that those changes should be allowed to work before making additional changes to the market; therefore, the PRR should be rejected without prejudice. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 | Utilize the same manual process contemplated by PRR 650 to remove OOMC, OOME, RRS deployments during Alert or Emergency Notice conditions, manually re-run SPD and post the resulting prices. |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | MCPE | Unknown |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Reliability | Unknown |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | Improves price signals during periods of capacity shortage which would likely lead to increased reliability due to additional resources (generation and demand) entering the market. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Henry Durrwachter |
| E-mail Address | hdurrwachter@luminant.com |
| Company | Luminant Energy Company LLC |
| Phone Number | 214-875-9285 |
| Cell Number | 214-437-4622 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| Exelon 110507 | Opposition to PRR742. |
| ERCOT 120507 | Clarification of Market Clearing Price for Energy (MCPE) and comments on the implementation of the PRR742. |

Proposed Protocol Language Revision

6.8.1.12	Payments for Balancing Energy Provided from Ancillary Services During the Operating Period

All Balancing Energy deployed by a Dispatch Instruction and delivered by a generating Resource from Balancing Energy service, Responsive Reserve Service, Regulation Reserve Service, or Non-Spinning Reserve Service shall be settled as Balancing Energy at the MCPE of the Congestion Zone of the Resource providing the energy according to Section 6.8.1.13, Resource Imbalance, except when Non-Spinning Reserve Service is deployed in accordance with Section 6.7.4, Deployment of Non-Spinning Reserve Service and there are no binding CSC constraints for that interval.  In that case, the MCPE shall be the MCPE that would have occurred if the deployments of Non-Spinning Reserve Service did not occur.  Likewise, during intervals in which an Alert (as defined by Section 5.6.5, Alert) or an Emergency Notice (as defined in Section 5.6.6, Emergency Notice) has been issued by ERCOT, and OOMC, OOME or Responsive Reserve Service deployments (excluding any OOMC deployed Day-Ahead) occur and there are no binding CSC constraints for that interval, the MCPE for those intervals shall be the MCPE that would have occurred if the deployments of OOMC, OOME or Responsive Reserve Service did not occur.  All Balancing Energy deployed by a Dispatch Instruction and delivered by a Load acting as Balancing Energy service, Responsive Reserve Service, Regulation Reserve Service, or Non-Spinning Reserve Service shall be settled as Balancing Energy at the MCPE of the Congestion Zone of the Resource providing the energy according to Section 6.9.5.2, Settlement for Balancing Energy for Load Imbalance, except when OOMC or OOME or Responsive Reserve Service or Non-Spinning Reserve Service is deployed in accordance with Section 5.6.5 or 5.6.6 or Section 6.7.4, Deployment of Non-Spinning Reserve Service and there are no binding CSC constraints for that interval.  In  those cases, the MCPE shall be the MCPE that would have occurred if the deployments of OOMC or OOME or Responsive Reserve Service or Non-Spinning Reserve Service did not occur.  Settlement for any energy provided without an authorized Dispatch Instruction will be settled according to Section 6.8.1.15, Payments for Balancing Energy Provided from Uninstructed Deviation.

Deployed Balancing Energy Up on qualified Balancing Up Loads will be paid a capacity payment for the first Settlement Interval that it is deployed equal to the MCPC of Non-Spinning Reserve Service for the hour in which the deployment occurs.  A continuous deployment of Balancing Energy Up on qualified Loads for over sixty (60) minutes will be paid a capacity payment for each subsequent Settlement Interval which it is deployed equal to the MCPC of Non-Spinning Reserve Service for that hour divided by four (4).