| NPRR Number | 070 | NPRR Title | Changes to DAM Clearing Start Time Allocation of McCamey Flowgate Rights in Day-Ahead Market |
| --- | --- | --- | --- |
| Date |  | June 21, 2007 |  |
| Submitter’s Information |  |  |  |
| Name |  | Trip Doggett on behalf of TPTF |  |
| E-mail Address |  | tdoggett@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | (512) 248-6360 |  |
| Cell Number |  | (512) 626-0275 |  |
| Market Segment |  | n/a |  |

Comments

TPTF voted to approve NPRR070 with the modifications listed in these comments on 6/21/07.  There were no abstentions.   All Market Segments except for the Independent Retail Electric Provider Market Segments were present.

Section 4.5.1 was removed from the revision request because no changes are desired - TPTF is not proposing that it be deleted.

| NPRR Number | 070 | NPRR Title |  | Allocation of McCamey Flowgate Rights in Day-Ahead Market |
| --- | --- | --- | --- | --- |
| Nodal Protocol Section(s) Requiring Revision (include Section No. and Title) |  | Section 7.7.3, Allocation of McCamey Flowgate Rights (MCFRIs) |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) incorporates the TPTF discussions for Allocation of McCamey Flowgate Rights (MCFRIs) in Day-Ahead. |  |  |
| Reason for Revision |  | The proposed revision coordinates the timeline of allocation of MCFRIs with Simultaneous Feasibility Test in Day-Ahead. Value engineering to avoid additional offline study tool. |  |  |
| Overall Market Benefit |  |  | Synchronizes the network model used for MCFRI allocation with that used in the DAM. |  |

Proposed Nodal Protocol Language Revision

7.7.3	Allocation of McCamey Flowgate Rights (MCFRIs)

(1)	ERCOT shall determine the “Capacity Impact” of each McCamey Area WGR eligible for MCFRIs on each McCamey Area flowgate by multiplying the maximum rated output for the WGR times its Shift Factor from the base case transmission model for the corresponding McCamey Area flowgate relative to the Load-weighted average Shift Factor of all Electrical Buses in ERCOT.

(2)	At each CRR Auction, ERCOT shall allocate 90% of the limit for each McCamey Area flowgate adjusted for impact of allocated PCRRs for the corresponding auction as MCFRIs to each McCamey Area WGR in proportion to its Capacity Impact as a percentage of the sum of all Capacity Impacts for McCamey Area WGRs for the corresponding flowgate.  All commercial and committed WGRs in the McCamey Area must be included in the analysis, but MCFRIs may be allocated only to QSEs representing WGRs that are in commercial operation.  The determination of available McCamey Area flowgate capacity must account for reduced flowgate element capacities resulting from CRRs already sold or allocated.

(3)	ERCOT shall allocate 90% of McCamey Area flowgate capacity to be auctioned in any particular auction and the DAM, adjusted for PCRR impacts and MCFRIs previously allocated, as MCFRIs to each McCamey Area WGR as described in (2) above 25 days prior to an annual auction, ten Business Days prior to a monthly auction, and no later than one hour prior to the DAM.