EIP 
MIS External Interfaces Specification

v0.36

January 31, 2008

Document Revisions

| Date | Version | Description | Author(s) |
| --- | --- | --- | --- |
| 2/26/2007 | 0.23 | Initial draft based upon External Interfaces Specification | Scott Neumann |
| 3/19/2007 | .24 | Added the synchronous get notification section. | Nemat Sarnevesht |
| 3/21/2007 | .25 | Finalized the synchronous get notification section. Added sequence diagram and XML examples | Nemat Sarnevesht |
| 3/28/2007 | .26 | added 6 new web services in support of MIS dashboard: Weather Data, List of zones, hubs, buses, ACE, Frequency, Responsive Reserve Capacity, non-spinning Reserve | Nemat Sarnevesht |
| 4/02/2007 | .27 | Added xsd diagrams to web service interfaces Corrected a few input variables. Factored in a few observations from Mehdi A. | Nemat Sarnevesht |
| 4/02/2007 | .28 | Updated section 2.0 from the External Interface document published by Scott | Nemat Sarnevesht |
| 4/05/2007 | .29 | Updated section 3 per Scott Neumann comments – changes naming, etc. | Nemat Sarnevesht |
| 4/09/2007 | .30 | Added web service for WGRRP Added DynamicRating option to WeatherData | Nemat Sarnevesht |
| 4/10/2007 | .30 | Added web service Dynamic rating deviation Average monthly Dynamic deviation Total ERCOT Load Total ERCOT Generation Total ERCOT Generation Operating Reserve | Nemat Sarnevesht |
| 4/16/2007 | .31 | Incorporated Scott N. comments/suggestions | Nemat Sarnevesht |
| 4/16/2007 | .32 | Dropped a few sections and referred back to Ext Int. Specification | Nemat Sarnevesht |
| 5/3/2007 | .33 | Adopted tm_schedule type. Corrected many XML examples as a result | Nemat Sarnevesht |
| 7/20/2007 | .34 | Added MID-MIR ReportList service | Manoj Aduru |
| 1/4/08 | .35 | Updated namespaces | Stephen Kerr |
| 2/4/08 | .36 | - Removed Role, Category, and SortBy element from the Request structure, keeping it consistent with EIS 1.08 - Confirmed with MIS project team to handle these attributes after the query within their portal - Added a table of valid AS Types for all AS transactions to section 2.3.3 | Nemat Sarnevesht |

Document Approvals

| Date | Approved By | Approval Documented In (select) |
| --- | --- | --- |
|  | <Name> <Role> | ___ Approval email on file ___ Signature |

Table of Contents

1.	Introduction	4

1.1.	Purpose	4

1.2.	Scope	4

1.3.	Definitions, Acronyms, and Abbreviations	4

1.4.	References	6

1.5.	Overview	7

1.6.	Program-level Standards	7

2.	Services Organization	8

2.1.	Common Message Structure	8

2.1.1.	Message Header Structure	8

2.1.2.	Request Message Structures	10

2.1.3.	Payload Structures	11

2.1.4.	Response Message Structures	12

2.2.	Common Security Implementation	13

2.2.1.	Secure the Transport layer	14

2.2.2.	Secure SOAP messages	15

2.3.	Modeling and Conventions	15

2.3.1.	Use of the IEC CIM	15

2.3.2.	Representation of Time	15

2.3.3.	Other Conventions	16

2.4.	Delivery Approach	16

2.5.	Technical Interoperability	17

2.6.	Versioning	17

3.	Nodal Information Requests	18

3.1.	Interfaces Provided	18

3.2.	Interfaces Required	18

3.3.	Message Specifications	19

3.3.1.	Weather Data	19

3.3.2.	List of Zones, Hubs, or Buses	22

3.3.3.	System Control Error	24

3.3.4.	Frequency	25

3.3.5.	Responsive Reserve Capacity	27

3.3.6.	Non-Spinning Reserve	30

3.3.7.	Undeployed Reg-Up and Reg-Down	32

3.3.8.	Available Capacity	33

3.3.9.	Wind-Powered Generation Resource Production Potential	35

3.3.10.	Short-Term Wind Power Forecast	37

3.3.11.	Dynamic ratings deviation adjusted for the current temperature	39

3.3.12.	Average monthly Dynamic deviation	41

3.3.13.	Total ERCOT Load	43

3.3.14.	Total ERCOT Generation	44

3.3.15.	Total ERCOT Generation Operating Reserve	46

3.3.16.	MID-MIR Get Report List	48

3.3.17.	RRS Deployment Instruction for non-controllable load resources	53

4.	Notifications	54

4.1.	Interfaces Provided	54

4.2.	Interfaces Required	56

4.3.	XML Data Structures	58

4.4.	Notifications XML Examples	60

4.5.	Message Specifications	61

Appendix A: Issues	62

# Introduction

This document describes interfaces for use by the ERCOT MIS portal. The intended audience of this document is developers that will be integrating MIS portal to the ERCOT Nodal systems through the use of the interfaces described within this specification. This interface specification describes interfaces that are supplemental to the ERCOT External Interfaces Specification.

The interfaces and related interactions described by this document define the externally-visible (black box view) perspective of the services provided by this project. It is the intent of this specification and interface architecture to shield the MIS portal from the details of systems integration internal to ERCOT.

## Purpose

The interfaces described by this document are intended to be used by the MIS portal but will also be exposed externally.

## Scope

The scope of this document is to describe web services provided for integration by the MIS portal. This document has program level scope as related to web services that would be used by the MIS portal for interaction with nodal applications as detailed in an agreed list of interfaces to be managed by the Nodal project. The intent of this design is to leverage the integration layer (IL) to expose web services needed for external integration by Market Participants.

The following are specifically outside the scope of this document:

- The details of integration from the IL to specific applications (e.g. MMS, EMS) are outside the scope of this specification and will be described in subsequent design documents.

- Inter Control Canter Communications Protocol (ICCP) communications

- Interactions with User Interfaces (UI)

This document is not intended as a replacement for the web services provided by PR50024.

  - Definitions, Acronyms, and Abbreviations

| Term/Acronym | Definition |
| --- | --- |
| ADJ | Adjustment |
| AGC | Automatic Generation Control |
| AS | Ancillary Services, includes spinning reserve, non-spinning reserve, responsive reserve and regulation |
| bid | The term bid in this document is used for bids, offers and trades |
| BidSet | A BidSet is a container for a collection of bids, offers, trades and schedules |
| CIM | Common Information Model, an IEC standard |
| CRR | Congestion Revenue Rights, a system implemented at ERCOT by Nexant that is responsible for CRR auctions |
| CSV | A file format that uses values separated by commas |
| DAM | Day Ahead Market |
| DRUC | Day-Ahead Unit Commitment |
| EMS | Energy Management System, a system implemented at ERCOT by Areva |
| FIP | Fuel Index Price |
| FOP | Fuel Oil Price |
| FTP | File Transfer Protocol |
| HASL | High ancillary services limit |
| HDL | High dispatch limit |
| HEL | High emergency limit |
| HRUC | Hour-ahead unit commitment |
| HSL | High sustained limit |
| IEC | International Electro technical Commission |
| LASL | Low ancillary services limit |
| LDL | Low dispatch limit |
| LEL | Low emergency limit |
| LMP | Location marginal price |
| LSL | Low sustained limit |
| MCPC | Market clearing price for capacity |
| MIS | Market Information System, an umbrella for the various interfaces provided to Market Participants by ERCOT |
| MMS | Market Management System, a system implemented at ERCOT by ABB |
| MP | Market Participant |
| MW | Megawatt, a measure of power |
| MWh | Megawatt hour, a measure of energy |
| Non-Spin | Non-spinning reserve service |
| NSRS | Non-spinning reserve service |
| OASIS | Organization for the Advancement of Structured Information Systems |
| Operating Date | Synonymous with Trade Date |
| POC | Proof Of Concept |
| QSE | Qualified Scheduling Entity |
| Reg-Down | Regulation down |
| Reg-Up | Regulation up |
| RRC | Responsive Reserve Capacity |
| RRS | Responsive reserve |
| RTM | Real-Time Market |
| SCE | System Control Error |
| SCED | Security constrained economic dispatch |
| sink | Sink settlement point |
| SOAP | Simple Object Access Protocol |
| SoSA | System of Systems Architecture |
| source | Within a bid or award this refers to the source settlement point. Within a message header, this refers to the ID of the market participant. |
| SP | Settlement Point |
| SPP | Settlement Point Price |
| STWPF | Short-Term Wind Power Forecast |
| TP | Transmission Provider |
| Trade Date | Synonymous with Operating Date |
| WGR | Wind Generation Resource |
| WGRPP | Wind Powered Generation Resource Production Potential |
| WS | Web Services. There are many web service standards that are commonly prefixed by ‘WS’. |
| WSDL | Web Services Definition Language |
| XML | eXtensible Markup Language |
| XSD | XML Schema, used to define the structure of XML documents |
| Z | Zulu, an indicator for the use of GMT or UTC time |

## References

| Artifact | Definition |
| --- | --- |
| External Interfaces Specification |  |
| External Interfaces Design | Design for external interfaces using web services |
| External Interfaces Security Design Specification | Detailed security design for external interfaces. This is a companion document to the External Interfaces Conceptual design. |
| OASIS WS-Notifications | OASIS Web Services Base Notification standard |
| OASIS WS-Security | OASIS Web Services Security |

## Overview

This document focuses on the external interface design and related interface definitions from all perspectives except for security, which is described in detail in a companion document. The interfaces are to be provided using web services, where a rationale is provided in subsequent sections. The web services defined by this document will support a wide variety of machine to machine information exchanges.

## Program-level Standards

In general, this design described by this document will leverage web services and related security standards as defined by the World-Wide Web Consortium (W3C) and OASIS. Program-level standards include those related to security. These are described in the companion security design document.

Another key program standard is the IEC Common Information Model (CIM), as defined by IEC 61970-301. This is used to define models used by ERCOT. It will also be leveraged by this design for the definition of messages used for interfaces. There is also a standard for message structures defined by IEC 61968-1.

The OASIS WS-Base Notifications standard is used to define the mechanism for issuance of asynchronous notification messages to Market Participants. Given the application of WS-Notifications for Nodal, only a subset of the capabilities of WS-Notifications is required.

The definition of timestamps is specified by ISO-8601, with the exception that timestamps of 24:00:00 are not used for compatibility reasons.

# Services Organization

The services described by this document are defined using a combination of Web Services Definition Language (WSDL) and XML Schema. The WSDLs are organized as follows:

- One or more WSDLs defined by ERCOT, defining operations related to synchronous request/reply web service messages

- WSDL defined by OASIS for WS-Notifications to provide support for asynchronous messaging, using web services

In both of the above cases, one or more XML Schemas (XSD) is used to define the structure of message payloads.

Example WSDL and XSD are provided in the ERCOT External Interface Specification. It is anticipated that these would be key design artifacts for developers.

## Common Message Structure

Unless otherwise specified, all messages use a common message envelope, where a predefined structure is used for requests and another structure is used for responses. This structure is based upon the IEC 61968-1 standard. Messages are constructed with several sections, including:

- Header: required for all messages, using a common structure for all service interfaces

- Request: optional, defining parameters needed to qualify request messages

- Reply: Used for response messages to indicate success, failure and error details

- Payload: optional, used to convey message information as a consequence of the ‘verb’ and ‘noun’ in the message Header. The payload structure provides options for payload compression.

### Message Header Structure

Common to both the request and response messages is a header structure. The header has several required fields that must be populated, these include:

- Verb, to identify a specific action to be taken. There are an enumerated set of valid verbs, where commonly used values include ‘get’, ‘create’, ‘update’, ‘cancel’, ‘close’ and ‘reply’.

- Noun: to identify the subject of the action and/or the type of the payload (e.g. BidSet, Notification) if a payload is provided.

- Source: identifying the source of the message, which should be the ID of the Market Participant or ERCOT (typically for reply messages)

- Revision: To indicate the revision of the message definition. This should be ‘1’ by default.

- Nonce: A unique number that would not be repeated by the Market Participant within the period of at least a day. This could be a sequence number, large random number or a GUID. This is defined by WS-Security. A combination of this number and the timestamp make the message unique for a given time period.

- Created: A timestamp to indicate when the message was created. This value and the Nonce are used to protect against replay attacks. This is defined by WS-Security.

The following diagram describes the header structure used for request and response messages.

There are several optional fields that may be populated. In the above diagram, the optional items are represented using dashed borders. If the MessageID is populated on a request, it will be returned on the reply. The Comment field is never used for any processing-related logic. The UserID may be used to indicate the person responsible for initiating a transaction, and will be logged as appropriate, but verification is the responsibility of the Source system.

### Request Message Structures

The following diagram describes the structure of a request message that would be used in conjunction with a WSDL operation.

The RequestMessage can also optionally contain a package with parameters relevant to the request, called Request. It is likely that different or variant Request packages may be defined to be used in conjunction with messages for a specific web service operation. In those cases, the corresponding WSDL and XSD would identify the optional parameters. The description of the interface (in subsequent sections of this document) would identify the usage of those parameters. The following is an example RequestType used in the definition of a Request package that defines some common parameters used for requests, however it is important to note that these are typically application specific. These parameters are most commonly used in conjunction with ‘get’ requests as qualifiers.

One key use of the RequestType is to avoid the placement of application specific request parameters in the header or within payload definitions. Also, where a set of requests that were supported by a specific web service operation had significantly different requirements for information in the RequestType, it could justify the use of RequestType variants, were each variant was used for the definition of messages for the specific web service operation.

### Payload Structures

There are some requests where a Payload must be provided, as would be the case for a message with a verb of ‘create’ or ‘update’. Payloads are typically XML documents that conform to a defined XML schema. However, there are exceptions to this rule. Some XML payloads may not have useful XML schemas, as in the case of RDF files or dynamic query results, as well as non-XML formats such as CSV and PDF. There may also be cases where a large payload must be compressed, in the event that it would become very large and otherwise consume significant network bandwidth.  In order to accommodate a variety of payload format options the following payload structure is used.

In the previous diagram, any type of XML document may be included, using the XML ‘any’ structure. While this provides options for loose-coupling, specific complex types defined by XML schemas (XSDs) can be used as well. The WSDL in the External Interface Specification Version provides an example of this case.

Payloads can also be supplied as XML encoded strings using the ‘Document’ tag, although this method is less preferred than used of the XML ‘any’.

There are also some cases where a zipped, base64 encoded string is necessary, and would be passed using the ‘Compressed’ tag. The Gnu Zip compression shall be used in order to provide compatibility within both Java and Microsoft .Net implementations. Specific examples of the usage of payload compression would be where:

- An XML payload, conforming to a recognized XML schema exceeds a predefined size (e.g. 1MB). This would be very common for large Market Participant sets of bids.

- A payload has a non-XML format, such as PDF, Excel spreadsheet, CSV file or binary image

- A payload is XML, but has no XML schema and exceeds a predefined size, as would be the case of a dynamic query that would return an XML result set

The format tag can be used to identify specific data formats, such as XML, RDF, PDF, DOC, CSV, etc. This is especially useful if the payload is compressed.

The above options provide an alternative to the use of SOAP attachments. SOAP attachments are more difficult to secure since the SOAP envelope signature signs the SOAP body but does not sign the attachment. This also requires that the payload is processed separately from the rest of the SOAP message (e.g. the message is parsed to extract the payload, and then the payload is parsed and processed). However, we believe this implementation approach is less complex than using SOAP attachments.

### Response Message Structures

The following diagram describes the structure of a response message that would be used in conjunction with a WSDL operation, as a response to the request message.

The ReplyCode would be set to OK to indicate that the request was successful, otherwise it would be set to ERROR, and one or more Error elements would be provided to describe the error(s). There may also be more specific error information provided within the payload, as in the case of bids within a BidSet container.

If the MessageID was set in the Header for the RequestMessage, the value will be returned in the Header of the ResponseMessage.

## Common Security Implementation

This section will provide an overview of security from the perspective of implementation requirements for Market Participants. Market Participants MUST take two basic steps in securing their Web Services Interaction with ERCOT:

- Secure the Transport layer

- Secure SOAP messages

### Secure the Transport layer

The transport layer is secured by deploying Secure Socket Layer (SSL) and Transport Layer Security (TLS) following these steps:

- Obtain client side certificate (these Certificates are issued by Verisign under the ERCOT brand).

- Implement mutual authentication (explained below).

- Ensure minimum SSL/TLS security settings

Note that TLS is an enhanced specification based on SSL. References to SSL refer to both SSL and TLS.

SSL is a standard mechanism for Web services that is available on virtually all application servers. This widely used, mature technology, which secures the communication channel between client and server, will satisfy all of ERCOT’s use cases for secure Web Service communications. Since it works at the transport layer, SSL covers all information passed in the channel as part of a message exchange between a client and a server, including attachments. Authentication is an important aspect of establishing an HTTPS connection.  Many platforms support the following authentication mechanisms for Web Services using HTTPS:

- The server authenticates itself to clients with SSL and makes its certificate available.

- The client uses basic authentication over an SSL channel.

- Mutual authentication with SSL, using the server certificate as well as the client certificate, so that both parties can authenticate to each other.

With Web Services, the interaction use case is usually machine to machine; that is, it is an interaction between two application components with no human involvement. Machine-to-machine interactions have a different trust model from typical website interactions. In a machine-to-machine interaction, trust must be established proactively, since there can be no real-time interaction with a user about whether to trust a certificate. Ordinarily, when a user interacts with a website via a browser and the browser does not have the certificate for the site, the user is prompted about whether to trust the certificate. The user can accept or reject the certificate at that moment. With Web Services, the individuals involved in the deployment of the Web Service interaction must distribute and exchange the server certificate, and the client certificate ( for mutual authentication), prior to the interaction occurrence.

The combination of the two settings—CONFIDENTIAL for transport guarantee and CLIENT-CERT for auth-method—enables mutual authentication. When set to these values, the containers for the client and the target service both provide digital certificates sufficient to authenticate each other. (These digital certificates contain client-specific identifying information.)

### Secure SOAP messages

Besides creating a secure communication channel between a client and a Web Service, ERCOT Web Service message exchanges require that security information be embedded within the SOAP message itself. This is often the case when a message needs to be processed by several intermediary nodes before it reaches the target service or when a message must be passed among several services to be processed.

Message-level security is very useful in XML document-centric applications, since different sections of the XML document may have different security requirements or be intended for different users.

SOAP messages’ signing is done through the following:

- Obtain application/system signing certificate.  (These certificates are issued by Verisign under the ERCOT brand).

- Sign all SOAP messages, using Web Services Security Standards and its X.509 Certificate Token Profile (see J2EE and .NET code snippets in External Interface Specification Version 0.92 )

- Message headers MUST include a timestamp and a nonce

- Validate all SOAP messages have:

  - Signature

  - Certificates

  - Revocation status of certificates

  - Use of timestamp and nonce (to prevent replay attacks)

External Interface Specification Version 0.92 Appendix D provides examples for the generation of signatures. External Interface Specification Version 0.92 Appendix E provides an annotated example of a SOAP message.

## Modeling and Conventions

There are several conventions that are used for definitions, data items and information models.

### Use of the IEC CIM

Please refer to ERCOT External Interface Specification for a complete analysis and description of representation of time in the specified interfaces.

### Representation of Time

Please refer to ERCOT External Interface Specification for a complete analysis and description of representation of time in the specified interfaces.

### Other Conventions

The following are other conventions that must be followed by this specification:

- Within XML definitions, tags should be namespace qualified. For example, a tag of <tag> should be prefixed by a specific namespace reference, e.g. <ns:tag>. This will help to eliminate ambiguity. (Note that many examples in this document are not namespace qualified for brevity and to aid legibility)

- Units for power quantities are in megawatts (MW).

- Units for capacity quantities are in megawatts (MW).

- Units for energy quantities are in megawatt-hours (MWh).

- Units for energy prices are in $/MWh.

- Valid market types include: DAM, RTM, DRUC, HRUC, SCED, SASM, CRR and ADJ. Note that these are not currently required by bidding interfaces

- Trading dates are specified using YYYY-MM-DD, which indicates the operating day

- Valid ancillary service types offered as products include:

| ASType | Supported in |
| --- | --- |
| Off-Non-Spin | ASOffer, |
| Reg-Down | ASOffer,SelfArrangedAS,ASTrade |
| REGUP-RRS-ONNS | ASOffer, |
| Reg-Up | SelfArrangedAS,ASTrade |
| RRS | SelfArrangedAS,ASTrade |
| RRSLD | SelfArrangedAS |
| Non-Spin | SelfArrangedAS,ASTrade |

- Settlement point names are the same as the name of the associated electrical bus, if defined at a bus level. They may also be defined at other levels (e.g. hubs, load zones).

- QSE short names (as opposed to DUNS numbers) are used to identify the QSE as well as ‘buyer’ and ‘seller’ for trades.

## Delivery Approach

In advance of a Nodal go live and in accordance with agreed schedule and dependencies, ERCOT will provide the following:

- Interface specifications for web services

- Design artifacts, including XML schemas and WSDLs

- Source code examples for web service clients

- A sand box environment for testing the interactions between the MIS portal and Integration layer. This environment will eventually be used for qualification of QSEs.

The interface specifications, artifacts and implementation of the sand box environment will be staged. An iterative implementation approach will be used, where feedback from each stage will be used to plan subsequent stages.

## Technical Interoperability

Please refer to ERCOT External Interface Specification for a complete analysis and description of technical interoperability in the specified interfaces.

## Versioning

It is important to recognize that new versions of interfaces may be provided over time, largely as a consequence of:

- Staging of initial implementation

- New requirements

- Upgrades to vendor products

Wherever possible, interfaces will be evolved through augmentation, where a newer version of an interface is compatible with a previous version of an interface. However, this will not always be possible. New versions of interfaces will be manifested by:

- Changes to WSDLs

- Changes to XML Schemas

- Changes to software implementations

New versions will be deployed within a Sand Box environment for a testing/trial period. WSDL and XML schemas namespaces will include a date reference. Messages will use the Header/Revision field to identify a specific revision - this will enable ERCOT to process multiple versions of an interface where appropriate.

# Nodal Information Requests

This service is used to request specific types of nodal information via synchronous requests.

## Interfaces Provided

Specific interfaces using specific combinations of verbs and nouns (i.e. payload types) are defined to permit a market participant to programmatically access market information. The verb to be used for requests would in all cases be ‘get’. The noun would identify the type of information being requested. Each request could use a message ‘Request’ package to specify one or more parameters that would qualify the request.

The processing sequence is shown in the following sequence diagram.

## Interfaces Required

The messages for market information requests would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | Name of payload type |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/? | Optional: Other request parameters may be specified as needed |
| Payload | Message payload data with type defined by Noun |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Defined payload type name |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | Defined payload type |

In the cases of payloads that would otherwise exceed 1 megabyte, the payloads would be zipped, base64 encoded and stored within the ‘Payload/Compressed’ tag.

## Message Specifications

Specific payload definitions are not currently defined. These will be defined in future revisions.

### Weather Data

The purpose of this interface is to provide a query for fetching the Actual, historical, and forecast weather related information from EMS system.   EMS gets its weather feeds directly from national weather service. This Web services is used to populate Weather (by City or Zone) on the MIS portal screens.  A request option of “DynamicRating” will return the weather data which was used to perform the last Dynamic Rating calculations.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | WeatherInfo |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/startTime | Optional: Start time of interest |
| Request/endTime | Optional: End time of interest |
| Request/Option | Zone (city) of interest Or “DynamicRating” |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | WeatherInfo |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ | Weather Information |

The structure of a reply for weather payload is shown by the following diagrams.

The following is an XML example for a response for forecast weather related information from EMS system

<WeatherInfo xmlns=" http://www.ercot.com/schema/2007-06/nodal/ews">

<area>North Austin</area>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<temperature>72</temperature>

<dewPoint>34</dewPoint>

<relativeHumidity>80.7</relativeHumidity>

<heatIndex>22</heatIndex>

<windChill>69</windChil>

<windDirection>NW<windDirection>

<windSpeed>30<windSpeed>

<temperatureWetBulb>65<temperatureWetBulb>

<cloudCover>30<cloudCover>

<sunshineMinutes>2345<sunshineMinutes>

</ WeatherInfo>

### List of Zones, Hubs, or Buses

The purpose of this interface is to provide a query for fetching a list of Zones, Hubs, or Buses.  This Web services is used to populate a List of Values on the MIS portal to further select the market LMPs and SPPs (via another appropriate Web Service).

An optional Request ID, holding the Hub Name can be specified to get a list of all the buses in the specified Hub, similarly an optional Request ID, holding the Zone Name can be specified to get a list of all the hubs within the specified zone.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | Buses/Hubs/Zones |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/ID | Optional: Hub name for list of buses within a Hub or zone name for list of Hubs within a zone request |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Buses/Hubs/Zones |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ | Buses/Hubs/Zones |

The structure of a request for list Buses, Hubs, and Zones payload is shown by the following diagrams.

The following is an XML example for a list buses:

<Buses xmlns=" http://www.ercot.com/schema/2007-06/nodal/ews">

<Buses>Taylor220KV ABCDE01</Buses>

<Buses>Taylor220KV ABCDE02</Buses>

<Buses>Taylor220KV ABCDE03</Buses>

<Buses>Taylor220KV ABCDE04</Buses>

<Buses>Taylor220KV ABCDE05</Buses>

<Buses>Taylor220KV ABCDE06</Buses>

<Buses>Taylor220KV ABCDE07</Buses>

</Buses>

The following is an XML example for a list Hubs:

<Hubs xmlns="httphttp://www.ercot.com/schema/2007-06/nodal/ews">

<Hubs>SouthEast 750KV ABCDE01</Hubs>

<Hubs>SouthEast 750KV ABCDE02</Hubs>

<Hubs>SouthEast 750KV ABCDE03</Hubs>

<Hubs>SouthEast 750KV ABCDE04</Hubs>

<Hubs>SouthEast 750KV ABCDE05</Hubs>

<Hubs>SouthEast 750KV ABCDE06</Hubs>

<Hubs>SouthEast 750KV ABCDE07</Hubs>

</Hubs>

The following is an XML example for a list Zones:

<Zones xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<Zones>TXU North 500KV 01</Zones>

<Zones>TXU North 500KV 02</Zones>

<Zones>TXU North 500KV 03</Zones>

<Zones>TXU North 500KV 04</Zones>

<Zones>TXU North 500KV 05</Zones>

<Zones>TXU North 500KV 06</Zones>

<Zones>TXU North 500KV 07</Zones>

</Zones>

### System Control Error

The purpose of this interface is to provide a query for fetching AGC System Control Error (SCE) reading.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | SCE |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/startTime | Optional: Start time of interest |
| Request/endTime | Optional: End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | SCE |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ | SCE reading for period of interest |

The structure of a request for ACE payload is shown by the following diagrams.

The following is an XML example for SCE:

<SCE xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>200</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>100</value1>

</TmPoint>

</SCE>

### Frequency

The purpose of this interface is to provide a query for fetching real time frequency measurement.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | Frequency |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/startTime | Optional: Start time of interest |
| Request/endTime | Optional: End time of interest |
| Request/Option | Optional: Thresholds to receive the frequency thresholds as well |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Frequency |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |
| Payload/ | Frequency along with the frequency thresholds (if requested). |

The structure of a request for frequency payload is shown by the following diagrams.

The following is an XML example for frequency:

<Frequency xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>60.02</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>60.03</value1>

</TmPoint>

</Frequency>

### Responsive Reserve Capacity

The purpose of this interface is to provide a query for fetching Responsive Reserve Capacity (RRC) for:

- Generation Resources

- Controllable Load Resources

- Non-Controllable Load Resources

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | responsiveReserveCapacity |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/ID | Optional: QSE ID If blank, Total is returned |
| Request/Option | Required Qualifier: generationResources controllableLoadResources nonControllableLoadResources |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | responsiveReserveCapacity |
| Header/Source | ERCOT |
| Request/startTime | Optional: Start time of interest |
| Request/endTime | Optional: End time of interest |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |
| Payload/ | Responsive Reserve Capacity for specific Request Option submitted |

The structure of a request for RRC payload is shown by the following diagrams.

The following is an XML example for an Responsive Reserve Capacity (RRC):

<responsiveReserveCapacity xmlns="httphttp://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>1230</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>1240</value1>

</TmPoint>

</responsiveReserveCapacity >

### Non-Spinning Reserve

The purpose of this interface is to provide a query for fetching Non-Spinning Reserve (NSR) for:

- On-line Generation Resources

- Off-line Generation Resources

- Undeployed Load Resource

- Output Scheduled Resource

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | nonSpinningReserve |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/startTime | Optional: Start time of interest |
| Request/endTime | Optional: End time of interest |
| Request/ID | Optional: QSE ID |
| Request/Option | Required Qualifier: onlineGenerationResources offlineGenerationResources undeployedLoadResource outputScheduledResource |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | nonSpinningReserve |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |
| Payload/ | Non-Spinning Reserve for Request Option submitted |

The structure of a request for RRC payload is shown by the following diagrams.

The following is an XML example for Non-Spinning Reserve:

<nonSpinningReserve xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>675</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>755</value1>

</TmPoint>

</nonSpinningReserve>

### Undeployed Reg-Up and Reg-Down

The purpose of this interface is to provide a query for fetching Undeployed Regulation Up and Undeployed Regulation Down.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | undeployedRegulation |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/startTime | Optional: Start time of interest |
| Request/endTime | Optional: End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | undeployedRegulation |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |
| Payload/ | Value1 = Undeployed Reg-Up and Value2= Undeployed Reg-Down |

The structure of a request for Undeployed Reg payload is shown by the following diagrams.

The following is an XML example for available Capacity:

<undeployedRegulation xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>2675</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>2685</value1>

</TmPoint>

</undeployedRegulation>

### Available Capacity

The purpose of this interface is to provide a query for fetching Available capacity with or without Energy Offer Curves in the ERCOT System that can be used to increase or decrease Base Points in SCED.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | availableCapacity |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/startTime | Optional: Start time of interest |
| Request/endTime | Optional: End time of interest |
| Request/ID | Optional: QSE ID |
| Request/Option | Required Qualifier: withIncreasing withDecreasing withoutIncreasing withoutDecreasing |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | availableCapacity |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |
| Payload/ | Available capacity with or without Energy Offer Curves in the ERCOT System that can be used to increase or decrease Base Points in SCED. (based on Option submitted) |

The structure of a request for available Capacity payload is shown by the following diagrams.

The following is an XML example for available Capacity:

<availableCapacity xmlns="httphttp://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>2050</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>2080</value1>

</TmPoint>

</availableCapacity>

### Wind-Powered Generation Resource Production Potential

On the MIS Portal, each hour, ERCOT provides information about the wind powered generation resource production potential (WGRPP) forecasts for each wind generation resource (WGR) to the QSE that represents that WGR.  Additionally aggregated WGRPP forecast of all WGRs are also displayed on the MIS portal Secure Area.

The purpose of this interface is to provide a query for fetching WGRPP for each WGR as well as aggregated WGRPP forecast for all WGRs.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | WGRPP |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/startTime | Optional: Start time of interest |
| Request/endTime | Optional: End time of interest |
| Request/ID | Optional: WGR identity If No WGR is provided, Total WGRPP for all WGRs is returned. |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | WGRPP |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |
| Payload/ | WGRPP reading for period of interest and for WGR specified or aggregated WGRPP reading. |

The structure of a request for WGRPP is shown by the following diagrams.

The following is an XML example for SCE:

< WGRPP xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>78</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>89</value1>

</TmPoint>

</ WGRPP >

### Short-Term Wind Power Forecast

On the MIS Portal, each hour, ERCOT provides information about the Short-Term Wind Power Forecast (STWPF) that provides a rolling 48-hour hourly forecast of wind production potential for each Wind-Powered Generation Resource (WGR).

The purpose of this interface is to provide a query for fetching STWPF for each WGR and aggregated STWPF forecast for all WGRs.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | STWPF |
| Header/Source | MIS |
| Header/UserID | Optional: ID of user |
| Request/startTime | Optional: Start time of interest |
| Request/endTime | Optional: End time of interest |
| Request/ID | Optional: WGR identity If No WGR provided then Total STWPF for all WGRs is returned |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | STWPF |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |
| Payload/ | STWPF reading for period of interest and for WGR specified or aggregated. |

The structure of a request for STWPF is shown by the following diagrams.

The following is an XML example for STWPF:

< STWPF xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>50</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>100</value1>

</TmPoint>

</ STWPF >

### Dynamic ratings deviation adjusted for the current temperature

Protocol (3.10.8.3): Each Operating Hour, ERCOT shall post on the MIS Secure Area updated Dynamic Ratings adjusted for the current temperature.

This section describes interfaces used to retrieve Dynamic Ratings Deviation Data for conducting equipment for the current operating hour. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | DynamicRatingsDeviation |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | DynamicRatingsDeviation |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ DynamicRatings | DynamicRatingsDeviation |

The payload structure is described by the following diagram:

The following is an XML example for Dynamic Ratings Deviation:

< DynamicRatingsDeviation xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<DynamicRatingDeviation>

<equipment>Taylor 500KV Xformer 1234</equipment>

<rating>

<ratingType>Normal</ratingType>

<ratingValue>15</ratingValue>

<units>KV</units>

</rating>

</DynamicRatingDeviation>

<DynamicRatingDeviation>

<equipment>Taylor 500KV Xformer 1234</equipment>

<rating>

<ratingType>Emergency</ratingType>

<ratingValue>25</ratingValue>

<units>KV</units>

</rating>

</DynamicRatingDeviation>

</ DynamicRatingsDeviation >

### Average monthly Dynamic deviation

This section describes interfaces used to retrieve Average Monthly Dynamic Ratings Deviation Data for conducting equipment. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | AvgMonthlyDynamicDeviation |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/OperatingDate | Month of interest Month of the Provided OperatingDate |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | AvgMonthlyDynamicDeviation |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ AvgMonthlyDynamicDeviation | AvgMonthlyDynamicDeviation For the month requested |

The payload structure is described by the following diagram:

The following is an XML example for Average Monthly Dynamic Ratings:

< AvgMonthlyDynamicDeviation xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<equipment>Taylor 500KV Xformer 1234</equipment>

<rating>

<ratingType>Normal</ratingType>

<ratingValue>15</ratingValue>

<units>KV</units>

</rating>

</ AvgMonthlyDynamicDeviation >

### Total ERCOT Load

This section describes interfaces used to retrieve the total amount of ERCOT load. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | TotalLoad |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/startTime | Start time |
| Request/endTime | End time |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | TotalLoad |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ TotalLoad | IrregularIntervalSchedule, where value1=total ERCOT Load |

The payload structure is described by the following diagram:

The following is an XML example for total amount of ERCOT load:

< TotalLoad xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>15000</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>16000</value1>

</TmPoint>

</ TotalLoad >

### Total ERCOT Generation

This section describes interfaces used to retrieve the total amount of ERCOT electric generation. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | TotalGeneration |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/startTime | Start time |
| Request/endTime | End time |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | TotalGeneration |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ TotalGeneration | IrregularIntervalSchedule, where value1=total Generation Load |

The payload structure is described by the following diagram:

The following is an XML example for total amount of ERCOT electric generation:

< TotalGeneration xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>16000</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>17000</value1>

</TmPoint>

</ TotalGeneration >

### Total ERCOT Generation Operating Reserve

This section describes interfaces used to retrieve the total amount of ERCOT electric generation operating reserve. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Get |
| Header/Noun | TotalGenOpReserve |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/startTime | Start time |
| Request/endTime | End time |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Reply |
| Header/Noun | TotalGenOpReserve |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ TotalGenOpReserve | IrregularIntervalSchedule, where value1=total Generation Operating reserve |

The payload structure is described by the following diagram:

The following is an XML example for total amount of ERCOT electric generation operating reserve:

< TotalGenOpReserve xmlns="http://www.ercot.com/schema/2007-06/nodal/ews">

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T07:00:00.12:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>16700</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>17800</value1>

</TmPoint>

</ TotalGenOpReserve >

### MID-MIR Get Report List

The purpose of this interface is to get the available Report list from Report server, MID-MIR, based on registration date and report type.  The downloading of specific reports are not included as part of this interface details.

This interface conforms to CMS.xsd schema for request/response messages and would use the following fields:

| Message Element | Value |
| --- | --- |
| Header/Context | Environment name |
| Header/Verb | get |
| Header/Noun | ReportList |
| Header/Source | MIS |
| Header/UserID/EmployeeId | EmployeeId |
| Header/UserID/Duns | Duns Number |
| Header/payload/ ReportListServiceRequest/day | List of Reports from date (day) |
| Header/payload/ ReportListServiceRequest/month | List of Reports from date (month) |
| Header/payload/ ReportListServiceRequest/year | List of Reports from date (year) |
| Header/payload/ ReportListServiceRequest /folder_id | Folder Id in MID |
| Header/payload/ ReportListServiceRequest /mimic_duns | Mimic Duns |
| Header/payload/ ReportListServiceRequest /Folder_name | Type of the report or “ALL” (Folder_Name) |
| Header/payload/ ReportListServiceRequest/Folder_depth | Folder depth in MID |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Context | Environment name |
| Header/Verb | reply |
| Header/Noun | ReportList |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ ReportListServiceResponse | Response Payload is list of report names |

The payload structure is described by the following diagram:

The following is an XML example get List of Reports Request:

<?xml version = "1.0" encoding = "UTF-8"?>

<il:Message>

<il:Header>

<il:Context>PRODUCTION</il:Context>

<il:Verb>get</il:Verb>

<il:Noun>ReportList</il:Noun>

<il:Timestamp>2007-05-31T13:20:00-05:00</il:Timestamp>

<il:Revision>1.0</il:Revision>

<il:User>

<il:EmployeeID></il:EmployeeID>

<il:Duns></il:Duns>

</il:User>

</il:Header>

<il:Payload>

<ns1:ReportListServiceRequest xmlns:ns1 = "http://www.ercot.com/schema/2005/GetReportListImpl">

<ns1:folder_id>10001685</ns1:folder_id>

<ns1:mimic_duns>0000000081791134</ns1:mimic_duns>

</ns1:ReportListServiceRequest>

</il:Payload>

</il:Message>

The following is an XML example get List of Reports Response message:

<?xml version = "1.0" encoding = "UTF-8"?>

<il:Message>

<il:ReportListServiceResponse xmlns:SOAP-ENV = "http://schemas.xmlsoap.org/soap/envelope/" xmlns:ns0 = "http://www.ercot.com/2005/schema">

<il:Header>

<il:Context>PRODUCTION</il:Context>

<il:Verb>get</il:Verb>

<il:Noun>Report List</il:Noun>

<il:Timestamp>2007-05-31T13:20:00-05:00</il:Timestamp>

<il:Revision>1.0</il:Revision>

<il:User>

<il:EmployeeID></il:EmployeeID>

<il:Duns></il:Duns>

</il:User>

</il:Header>

<il:Payload>

<OperationResponse/>

<ReportResponse>

<TotalReports>1</TotalReports>

<Groups id = "0000000081791134" name = "ACN Energy">

<Groups id = "10001411" name = "Public">

<Groups id = "10001685" name = "ASBIDSTACKS">

<Reports id = "14539171" name = "ext.00000207.0000000000000000.20060601.140120.ASBIDSTACKS.zip" type = "207">

<CreatedDT>2006-06-01T14:01:20</CreatedDT>

<Size>16469</Size>

<Format>zip</Format>

<CstDateTime>20060601.140120</CstDateTime>

<StyleSheet/>

<SecurityClassification>MPPUBLIC</SecurityClassification>

</Reports>

</Groups>

<Groups id = "10001694" name = "ASDEPLOYMENTS"/>

<Groups id = "10001703" name = "Actual Transmission Loss Factor"/>

<Groups id = "10001721" name = "Backcasted Profiles Extract"/>

<Groups id = "10001730" name = "Dayahead Report"/>

<Groups id = "10001766" name = "Forecast Data Extract"/>

<Groups id = "10001775" name = "Forecasted Profiles"/>

<Groups id = "10001784" name = "Forecasted Trans loss Factor"/>

<Groups id = "10001802" name = "IDR Protocol Compliance"/>

<Groups id = "10001865" name = "Load Estimation Counts"/>

<Groups id = "10001874" name = "Load Estimation Volume"/>

<Groups id = "10001883" name = "Market Information Extract"/>

<Groups id = "10001892" name = "Market Shadow Prices Extract"/>

<Groups id = "10001928" name = "TDSP Distribution Loss Factors - Forecasted"/>

<Groups id = "10001964" name = "Unaccounted For Energy"/>

<Groups id = "10001973" name = "WS and NWS IDR Proxy Dates"/>

</Groups>

<Groups id = "10001459" name = "ESIID Extract"/>

<Groups id = "10001522" name = "Load Estimation Counts"/>

<Groups id = "10001531" name = "Load Estimation Volume"/>

<Groups id = "10001540" name = "Load Extract"/>

</Groups>

<MimicDuns>0000000081791134</MimicDuns>

</ReportResponse>

</ns0:ReportListServiceResponse>

</il:Payload>

</il:Message>

### RRS Deployment Instruction for non-controllable load resources

TBD

# Notifications

A key aspect of the design for external interfaces for Market Participants is the use of the OASIS WS-Notifications standard. Each Market Participant using the external interface would be required to provide an interface for the receipt of notification messages, compliant with the WS-Notifications standard.

## Interfaces Provided

The interfaces provided are described in the WS-Notifications specification. For the purposes of use by Market Participants, there are only two interfaces of interest:

- Asynchronous Notify message

- Synchronous Get Messages

Asynchronous Interfaces are used as a means to asynchronously provide information to Market Participants. Specific examples of this information include:

- Notices

- Alerts

- Bidset acceptance or error reporting

- Awards

- Obligations

Synchronous interfaces are used as a means to synchronously (retrieval) provide information to Market Participants upon request. Specific examples of this information include:

- Retrieve the list of all the notifications for a given user.

- .

- Get notifications based on the date intervals.

- Retrieve details on a notification for a given notification id.

The following sequence diagram describes the typical/potential set of information exchanges related to Notifications. The actual flows would be different based on the type of notification.

The following sequence diagram describes the typical retrieval of Notifications.

## Interfaces Required

The contents of the any structure would be wrapped using the RequestMessage structure defined in section 2.1.2, as this would allow necessary security information to be passed in a uniform manner. The following table describes the contents of the SOAP message.

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | Notification |
| Header/Replay Detection | Nonce + Created time |
| Header/Revision | Message Revision |
| Header/Source | Market Participant ID |
| Header/UserID | User ID |
| Header/MessageID | Message ID |
| Request/StartTime | From Date |
| Request/EndTime | To Date |
| Request/ID - Optional | Notification ID |

The contents of the any structure would be wrapped using the ResponseMessage structure defined in section 2.1.4, as this would allow necessary security information to be passed in a uniform manner. The following table describes the contents of the SOAP message.

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Notification |
| Header/Replay Detection | Nonce + Created time |
| Header/Revision | Message Revision |
| Header/Source | ERCOT |
| Header/MessageID | Message ID |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | May be any number of error messages |
| Reply/ID | Reply ID |
| Payload/notificationID | Notification ID |
| Payload/ notificationType | Notification Type |
| Payload/priority | Notification Priority |
| Payload/source | Notification Source |
| Payload/issued | Notification Issued Date/Time |
| Payload/expiration | Notification Expiration Date/Time |
| Payload/details | Notification detail text |
| Payload/objectID | Notification Object ID |
| Payload/referenceURL | Notification Reference URL |

In the cases of payloads that would otherwise exceed 1 megabyte, the payloads would be zipped, base64 encoded and stored within the ‘Payload/Compressed’ tag.

## Message Specifications

The WS-Notifications specification identifies a single message structure for notification messages. The payload that is to be delivered by the notification message is represented by the ‘any’ element in the following message structure, loosely coupling the definition of the payload from the definition of the container message structure defined by WS-Notifications thereby allowing WS-Notifications to be used with any XML data structure. For ERCOT usem the SubscriptionReference, Topic and ProducerReference are not used.

It is important to note that the payloads for NotificationMessages are ALWAYS conveyed using the common message structure (e.g. Message, RequestMessage or ResponseMessage) as defined in section 2.1 of External Interface Specification serving as the ‘any’ sub-structure in the NotificationMessage structure.

ERCOT uses a variant of WS-Notifications, where an ‘Acknowledge’ message is sent to acknowledge the receipt of a notification. The WSDL and XSD provided by ERCOT are also simplified to eliminate definitions for interfaces and structures not used by ERCOT. These are described in the appendix.

ERCOT will allow a Market Participant to specify up to two URLs. ERCOT will send notifications to port 443 of the specified URL using an HTTPS connection. ERCOT will sign the SOAP message, where the signature can be verified using the ERCOT public key. This will verify that the notification was sent by ERCOT and was not altered. The ‘Acknowledge’ message does not need to be signed.

## Notifications XML Examples

The following XML is an example of a Request Message for notifications:

<RequestMessage>

<header>

<verb>get</verb>

<noun>Notification</noun>

<ReplayDetection>

<Nonce>809b2bf3a747277e29bfafb03c4cea0d</ns0:Nonce>

<Created>2007-03-21T08:06:49-05:00</ns0:Created>

<ReplayDetection>

<Revision>001</Revision>

<Source>TXU</Source>

<MessageID>8cf5f140e08cc892a7fadbd301f2ae72</MessageID>

</header>

<request>

<StartTime>2007-03-21T08:06:49-05:00</StartTime>

<EndTime>2007-03-21T08:06:49-05:00</EndTime>

<ID>845598452435</ID>

</request>

</RequestMessage>

The following XML is an example of a Response Message for notifications:

<ResponseMessage>

<header>

<verb>reply</verb>

<noun>Notification</noun>

<ReplayDetection>

<Nonce>809b2bf3a747277e29bfafb03c4cea0d</ns0:Nonce>

<Created>2007-03-21T08:06:49-05:00</ns0:Created>

<ReplayDetection>

<Revision>001</Revision>

<Source>ERCOT</Source>

<MessageID>8cf5f140e08cc892a7fadbd301f2ae72</MessageID>

</header>

<reply>

<ReplyCode>OK</ReplyCode>

<Error/>

<ID>0462eff719eca048a78ad78191147dd4</ID>

<tradingDate>2007-01-10</tradingDate>

<participant>606252393</participant>

<userId>Zoro</userId>

</reply>

<payload>

<Notification>

<notificationID>8765487654876</notificationID>

<notificationType>Alert</notificationType>

<priority>High</priority>

<source>MMS</source>

<issued>2007-12-17T00:00:00Z</issued>

<expiration>2008-12-17T00:00:00Z</expiration>

<summary>Bid Accepted</summary>

<details>The reuqested BidSet submiteed on 2007-12-17T00:00:00Z is accepted.</details>

<referenceURL>http://nodal.ercot.com/messages/8768768/</referenceURL>

</Notification>

</payload>

</ResponseMessage>

- Appendix A: Issues

The purpose of this section is to capture issues and resolutions.

| Issue ID | Date | Issues | Resolution |
| --- | --- | --- | --- |
| 1 |  |  |  |