| PRR Number |  | PRR Title | Distributed Renewable Generation Modifications |
| --- | --- | --- | --- |
| Date Posted |  |  |  |
| Protocol Section(s) Requiring Revision (Include Section No. and Title) |  | 11, Data Acquisition and Aggregation 11.4.4.2, Adjustment for Energy Exports of PV Generation Behind the Meter (new) 11.4.4.3, Adjustment for Energy Exports of Non-PV Renewable Generation Behind the Meter (new) 18, Load Profiling 18.2, Methodology 18.2.2, Load Profiles for Non-Interval Metered Loads Without Distributed Renewable Generation 18.2.3, Load Profiles for Non-Interval Metered Loads With Distributed Renewable Generation (new) 18.2.9, Adjustments and Changes to Load Profile Development |  |
| Requested Resolution (Normal or Urgent, and justification for Urgent status) |  | Normal |  |
| Revision Description |  | These changes allow for load profiling and data aggregation methodology to better represent output of distributed renewable generation < 50 kW. |  |
| Reason for Revision |  | To comply with HB 3693. |  |
| Overall Market Benefit |  |  |  |
| Overall Market Impact |  |  |  |
| Consumer Impact |  |  |  |
| Credit Implications (Yes or No, and summary of impact) |  |  |  |
| Relevance to Nodal Market (Yes or No, and summary of impact) |  |  |  |
| Nodal Protocol Section(s) Requiring Revision (Include Section No. and Title, and submit NPRR if applicable) |  |  |  |

Quantitative Impacts and Benefits

Instructions:  To allow for comprehensive PRR consideration and development of the Cost Benefit Analysis (CBA), please fill out each block below completely and provide as much detailed information as possible.  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 | e.g.: Key assumptions used in estimating market cost and/or benefit |  |
| --- | --- | --- | --- |
|  | 2 | Dependencies on other projects or other timing requirements |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | e.g.: Cost per MP to implement | e.g.: $10,000 each for 50 QSEs |
|  | 2 | Add’l staff required per MP | 1.5 FTE each for 6 TDSPs @ $65/hour |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | e.g.: Reduced MP costs | e.g.: 2 FTE reduction for 25 CRs @ $65/hour |
|  | 2 | Enhanced MP efficiency | 2 hour savings per day for 50 generators @$65 |
|  | 3 | Reduced congestion cost | 0.5% reduction in total congestion cost |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | e.g.: Benefits that are difficult to quantify |  |
|  | 2 | Benefits that are not certain but relatively likely |  |
|  | 3 | Customer service impacts, cash flow impacts, transaction speed, etc. |  |
|  | 4 |  |  |
| Other Comments | 1 | e.g.: Thoughts on ERCOT systems impacts |  |
|  | 2 | Potential manual workarounds or delivery options |  |
|  | 3 | Other comments of value to PRS, TAC and the Board of Directors |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Ernie Podraza on behalf of the Profiling Working Group (PWG) |
| E-mail Address | Ernest.Podraza@directenergy.com |
| Company | Direct Energy |
| Phone Number | 713-877-3517 |
| Cell Number |  |
| Market Segment | Independent Retail Electric Providers |

| Market Rules Staff Contact |  |
| --- | --- |
| Name |  |
| E-Mail Address |  |
| Phone Number |  |

Proposed Protocol Language Revision

Instructions:  Guidelines and helpful hints may be found at the following link: http://www.ercot.com/mktrules/protocols/index.html.

Please remember the following:

- Use the most current version of the Protocol language, which is available on the ERCOT website.

- Show original Protocol baseline language in black.

- Present the entire titled (sub) section as the baseline – not just the paragraphs subject to revision.

- Make all revisions in redlined format (using “Track Changes”).  Be sure to change the user name to the appropriate individual or company name.  Do NOT show revisions by changing font color or font strikethrough.

- Use highlighting when inserting non-Protocol language comments (like questions) into Protocol language.

- Be sure proposed changes are reflected in both boxed and unboxed sections, if appropriate.

Insert proposed Protocol language here.

11.4.4.2	Adjustment for Energy Exports from PV Generation Behind the Meter

ESI ID locations that have photovoltaic (PV) renewable distributed generation behind the meter and have a meter that measures energy exported to the grid in a separate register shall have their profiled non-interval data adjusted to account for the energy exports.  Only ESI IDs that have been assigned a PV profile segment as specified in Load Profiling Guide Appendix D, Profile Decision Tree, shall be eligible for this adjustment.

Intervals in the period from 11 am to 3 pm shall be reduced by the following amount.

PVadjusti  =   KWHout / (READDAYS * 16)

Where:

PVadjusti	Adjustment for PV generation site energy export for interval i

KWHout	Actual (measured) exported kWh

READDAYS	Number of days in meter read period

11.4.4.3	Adjustment for Energy Exports from Non-PV Renewable Generation Behind the Meter

ESI ID locations that have renewable distributed generation other than photovoltaic behind the meter and have a meter that measures energy exported to the grid in a separate register shall have their profiled non-interval data adjusted to account for the energy exports.  Only ESI IDs that have been assigned a non-PV renewable distributed generation profile segment as specified in Load Profiling Guide Appendix D, Profile Decision Tree, shall be eligible for this adjustment.

All intervals in the meter read period shall be reduced by the following amount.

RENadjusti  =   KWHout / READINTS

Where:

RENadjusti	Adjustment for non-PV renewable generation site energy export for interval i

KWHout	Actual (measured) exported kWh

READINTS	Number of 15-minute intervals in the meter read period

18.2	Methodology

ERCOT will develop Load Profiles for both non-interval metered loads and Non-Metered Loads.  A Load Profiling Methodology is the fundamental basis on which Load Profiles are created.  The implementation of a Load Profiling Methodology may require statistical Sampling, engineering methods, econometric modeling, or other approaches.

The following Load Profiling methods will be used:

| Type of Load | Load Profiling Methodology |
| --- | --- |
| Non-Interval Metered | Adjusted Static Models |
| Non-Interval Metered with Distributed Renewable Generation | Adjusted Static Models and Engineering Estimates |
| Non-Metered | Engineering Estimates |

Load Profiles will also be developed for Interval Data Recorders (IDRs) for use in settlements when actual IDR data is not available.  All Load Profiles will conform to the ERCOT-defined Settlement Interval length.

Adoption of a new methodology requires approval of TAC, without the necessity of complying with the procedures in Section 21, Process for Protocols Revision.  TAC shall establish the implementation date for approved changes, as TAC deems appropriate, recognizing the magnitude of the impacts on Market Participants.

18.2.2	Load Profiles For Non-Interval Metered Loads Without Distributed Renewable Generation

Load Profiles for non-interval metered loads will be created using statistical models developed from appropriate load research sample data.  These models are referred to as “adjusted static.” These model equations will relate daily Settlement Interval load patterns to relevant weather descriptors such as maximum and minimum dry-bulb temperature and humidity.  Other daily characteristics such as day-of-the-week and sunrise/sunset times will also be employed.

[PRR478: Replace Section 18.2.2 above with the following upon system implementation:] For market open, Load Profiles for non-interval metered loads were created using statistical models developed from appropriate load research sample data. These models are referred to as “adjusted static.” These model equations relate daily Settlement Interval Load patterns to relevant weather descriptors such as maximum and minimum dry-bulb temperature and humidity. Other daily characteristics such as day-of-the-week and sunrise/sunset times are also employed. Following market open, new Load Profile segments may be introduced. After these Load Profile segments receive final approval under the provisions of the Load Profiling Guides, Section 12, Request for Profile Segment Changes, Additions, or Removals, they may be settled by using appropriately sized and representative lagged dynamic samples or adjusted static models. The decision to use a lagged dynamic sample or adjusted static model shall be based on the judgment of ERCOT’s Load Profiling Department, subject to TAC approval.

18.2.3	Load Profiles For Non-Interval Metered Loads With Distributed Renewable Generation

Load Profiles for non-interval metered loads that utilize distributed renewable generation (e.g., photovoltaic or wind) will be created using a hybrid approach.  At least a portion of the load profile will be based on adjusted static models, while engineering estimates and/or generation models may be integrated as well or otherwise utilized.  TDSPs are responsible for providing monthly metered inflow (consumption) and outflow (generation) kWh.

18.2.9	Adjustments and Changes to Load Profile Development

ERCOT and the appropriate ERCOT TAC subcommittee will conduct an ongoing evaluation of the current Load Profiling Methodology.  Together they will determine whether appropriate changes to the methodology should be made or whether another approach or combination of approaches is warranted.  Any Market Participant may request a review of the Load Profiling Methodology.  Adoption of a new Load Profiling Methodology must be approved by TAC, as provided in Section 18.2, Methodology.

Any Market Participant may petition ERCOT for adjustments to the existing Load Profiles and for development of new Load Profiles.  The Market Participant making the request shall submit their proposal in writing to ERCOT.  ERCOT will post to the Market Information System (MIS) the request and respond to such requests within sixty (60) days.  ERCOT shall coordinate with the appropriate ERCOT TAC subcommittee for each change request.  ERCOT shall strive to make the necessary changes within a reasonable period of time.

ERCOT, in coordination with the appropriate ERCOT TAC subcommittee, may make changes to existing Load Profiles and establish additional Load Profiles.  All changes to Load Profiles shall adhere to these Protocols.  When additional Load Profiles are established, ERCOT shall evaluate the impact on existing Load Profiles and associated load research samples.

A Market Participant may submit a request to ERCOT for conditional approval of a new Load Profile segment following the approval process as specified in the Load Profiling Guides, Section 12, Request for Profile Segment Changes, Additions, or Removals.  In conjunction with this request, ERCOT staff shall specify the requirements for additional Load research sampling and shall define specific and objective criteria to be met by the analysis of this Load research data to meet the requirements for final approval.  Provided the request for conditional approval has received the appropriate ERCOT committee approval and ERCOT staff determines the specified criteria are met, the request shall be granted final approval.  If ERCOT staff determines the specified criteria are not met, the request shall be denied.

Section 9.9, Profile Development Cost Recovery Fee for a Non-ERCOT Sponsored Load Profile Segment, describes the process for compensating the originator of a profile segment change request by REPs wishing to subscribe to the profile segment.

ERCOT shall give at least one hundred fifty (150) days notice to all Market Participants prior to market implementation of any change in Load Profile Methodology, existing Load Profiles, or when any additional Load Profiles are developed.  This notice shall include a Load Profile change implementation timeline, which specifies dates on which key events during the Load Profile change process will take place.  Upon any change in Load Profile Types, TDSPs shall send any revised ESI Load Profile assignments required by the change to the registration system within the implementation timeline.  After the new Load Profile(s) becomes available, changes to Load Profile Types will be effective on the next meter read date for each ESI ID.

If one or more Load Profiles require changes to reduce excessive UFE, as determined by the appropriate ERCOT TAC subcommittee, TAC may provide a shorter notice period and implementation date, than otherwise provided herein, for such required changes to Load Profiles.  If the Load Profile Methodology requires changes to reduce excessive UFE, as determined by the appropriate ERCOT TAC subcommittee, TAC may provide an expedited notice period and implementation date.  TAC may require the standard Load Profile revision process follow such expedited revisions for long-term resolution.