| PRR Number | 756 | PRR Title | Distributed Renewable Generation Modifications |
| --- | --- | --- | --- |
| Date |  | March 11, 2008 |  |
| Submitter’s Information |  |  |  |
| Name |  | Lee Starr on behalf of the Commercial Operations Subcommittee (COPS) |  |
| E-mail Address |  | lstarr@btutilities.com |  |
| Company |  | Bryan Texas Utilities (BTU) |  |
| Phone Number |  | (979) 821-5601 |  |
| Cell Number |  | (979) 229-5501 |  |
| Market Segment |  | Municipal |  |

Comments

| Overall Market Benefit | Clarity. |
| --- | --- |
| Overall Market Impact | None. |
| Consumer Impact | None. |

Following discussion at the March 11, 2008 COPS meeting, COPS supports this Protocol Revision Request (PRR).  COPS agreed to request reconsideration of the request to grant Urgent status to this PRR at the March 20, 2008 PRS meeting.

Proposed Protocol Language Revision

2.1	Definitions

PhotoVoltaic

Of or pertaining to a material or device in which electricity is generated as a result of exposure to light.

2.2	Acronyms

PV	PhotoVoltaic

11.4.4.2	Load Reduction for Excess PhotoVoltaic Generation

Adjusted Metered Load shall be reduced for excess generation from ESI IDs with PhotoVoltaic (PV) generation behind the meter where there is a meter that measures excess energy into the grid in a separate register.  Only ESI IDs that have been assigned a PV profile segment as specified in Load Profiling Guide Appendix D, Profile Decision Tree, shall be eligible for this reduction.

Intervals beginning 11:00 A.M. and ending 3:00 P.M. CPT (spanning 16 15-minute intervals) shall be reduced by the following amount.

PV_adjusti  =   kWh_Gen / (read_days * 16)

Where:

PV_adjusti	Reduction for PV excess generation for interval i

kWh_Gen	Actual (measured) kWh flowing into the Distribution System (outflow from the Premise)

read_days	Number of days in meter read period

11.4.4.3	Load Reduction for Excess Non-PhotoVoltaic Renewable Generation

Adjusted Metered Load shall be reduced for excess generation from ESI IDs with non-PV renewable generation behind the meter where there is a meter that measures excess energy into the grid in a separate register.  Only ESI IDs that have been assigned a non-PV renewable distributed generation profile segment as specified in Load Profiling Guide Appendix D, Profile Decision Tree, shall be eligible for this reduction.

All intervals in the meter read period shall be reduced by the following amount.

REn_adjusti  =   kWh_gen / read_ints

Where:

REn_adjusti	Reduction for non-PV excess renewable generation for interval i

kWh_gen	Actual (measured) kWh flowing into the Distribution System (outflow from the Premise)

read_ints	Number of 15-minute intervals in the meter read period

18.2	Methodology

ERCOT will develop Load Profiles for both non-interval metered loads and Non-Metered Loads.  A Load Profiling Methodology is the fundamental basis on which Load Profiles are created.  The implementation of a Load Profiling Methodology may require statistical Sampling, engineering methods, econometric modeling, or other approaches.

The following Load Profiling methods will be used:

| Type of Load | Load Profiling Methodology |
| --- | --- |
| non-interval metered | adjusted static models |
| non-interval metered with distributed renewable generation | adjusted static models and engineering estimates |
| non-metered | engineering estimates |

Load Profiles will also be developed for Interval Data Recorders (IDRs) for use in settlements when actual IDR data is not available.  All Load Profiles will conform to the ERCOT-defined Settlement Interval length.

Adoption of a new methodology requires approval of TAC, without the necessity of complying with the procedures in Section 21, Process for Protocols Revision.  TAC shall establish the implementation date for approved changes, as TAC deems appropriate, recognizing the magnitude of the impacts on Market Participants.

18.2.2	Load Profiles For Non-Interval Metered Loads Without Distributed Renewable Generation

Load Profiles for non-interval metered loads will be created using statistical models developed from appropriate load research sample data.  These models are referred to as “adjusted static.” These model equations will relate daily Settlement Interval load patterns to relevant weather descriptors such as maximum and minimum dry-bulb temperature and humidity.  Other daily characteristics such as day-of-the-week and sunrise/sunset times will also be employed.

[PRR478: Replace Section 18.2.2 above with the following upon system implementation:] For market open, Load Profiles for non-interval metered loads were created using statistical models developed from appropriate load research sample data. These models are referred to as “adjusted static.” These model equations relate daily Settlement Interval Load patterns to relevant weather descriptors such as maximum and minimum dry-bulb temperature and humidity. Other daily characteristics such as day-of-the-week and sunrise/sunset times are also employed. Following market open, new Load Profile segments may be introduced. After these Load Profile segments receive final approval under the provisions of the Load Profiling Guides, Section 12, Request for Profile Segment Changes, Additions, or Removals, they may be settled by using appropriately sized and representative lagged dynamic samples or adjusted static models. The decision to use a lagged dynamic sample or adjusted static model shall be based on the judgment of ERCOT’s Load Profiling Department, subject to TAC approval.

18.2.3	Load Profiles For Non-Interval Metered Loads With Distributed Renewable Generation

Load Profiles for non-interval metered Loads that utilize distributed renewable generation (e.g., PhotoVoltaic or wind) will be created using a hybrid approach.  At least a portion of the Load Profile will be based on adjusted static models, while engineering estimates and/or generation models may be integrated as well or otherwise utilized.

18.2.9	Adjustments and Changes to Load Profile Development

ERCOT and the appropriate ERCOT TAC subcommittee will conduct an ongoing evaluation of the current Load Profiling Methodology.  Together they will determine whether appropriate changes to the methodology should be made or whether another approach or combination of approaches is warranted.  Any Market Participant may request a review of the Load Profiling Methodology.  Adoption of a new Load Profiling Methodology must be approved by TAC, as provided in Section 18.2, Methodology.

Any Market Participant may petition ERCOT for adjustments to the existing Load Profiles and for development of new Load Profiles.  The Market Participant making the request shall submit their proposal in writing to ERCOT.  ERCOT will post to the Market Information System (MIS) the request and respond to such requests within sixty (60) days.  ERCOT shall coordinate with the appropriate ERCOT TAC subcommittee for each change request.  ERCOT shall strive to make the necessary changes within a reasonable period of time.

ERCOT, in coordination with the appropriate ERCOT TAC subcommittee, may make changes to existing Load Profiles and establish additional Load Profiles.  All changes to Load Profiles shall adhere to these Protocols.  When additional Load Profiles are established, ERCOT shall evaluate the impact on existing Load Profiles and associated load research samples.

A Market Participant may submit a request to ERCOT for conditional approval of a new Load Profile segment following the approval process as specified in the Load Profiling Guides, Section 12, Request for Profile Segment Changes, Additions, or Removals.  In conjunction with this request, ERCOT staff shall specify the requirements for additional Load research sampling and shall define specific and objective criteria to be met by the analysis of this Load research data to meet the requirements for final approval.  Provided the request for conditional approval has received the appropriate ERCOT committee approval and ERCOT staff determines the specified criteria are met, the request shall be granted final approval.  If ERCOT staff determines the specified criteria are not met, the request shall be denied.

Section 9.9, Profile Development Cost Recovery Fee for a Non-ERCOT Sponsored Load Profile Segment, describes the process for compensating the originator of a profile segment change request by REPs wishing to subscribe to the profile segment.

ERCOT shall give at least one hundred fifty (150) days notice to all Market Participants prior to market implementation of any change in Load Profile Methodology, existing Load Profiles, or when any additional Load Profiles are developed.  This notice shall include a Load Profile change implementation timeline, which specifies dates on which key events during the Load Profile change process will take place.  Upon any change in Load Profile Types, TDSPs shall send any revised ESI Load Profile assignments required by the change to the registration system within the implementation timeline.  After the new Load Profile(s) becomes available, changes to Load Profile Types will be effective on the next meter read date for each ESI ID.

If one or more Load Profiles require changes to reduce excessive UFE, as determined by the appropriate ERCOT TAC subcommittee, TAC may provide a shorter notice period and implementation date, than otherwise provided herein, for such required changes to Load Profiles.  If the Load Profile Methodology requires changes to reduce excessive UFE, as determined by the appropriate ERCOT TAC subcommittee, TAC may provide an expedited notice period and implementation date.  TAC may require the standard Load Profile revision process follow such expedited revisions for long-term resolution.