EIP 
External Interfaces Specification

- v1.09

March 4, 2008

- Document Revisions

| Date | Version | Description | Author(s) |
| --- | --- | --- | --- |
| 12/19/2006 | 0.10 | First draft | Scott Neumann |
| 12/20/2006 | 0.11 | Revised security section | Nemat Sarnevesht |
| 12/21/2006 | 0.13 | Augmentation to descriptions of bidding interfaces. | Scott Neumann |
| 12/22/2006 | 0.14 | Added sections 2.4-2.10 to discuss other aspects of web services strategy. | Scott Neumann |
| 12/27/2006 | 0.15 | Miscellaneous corrections and clarifications. | Scott Neumann |
| 12/28/2006 | 0.16 | Miscellaneous changes to reflect internal review | Scott Neumann |
| 01/10/2007 | 0.17 | Miscellaneous changes to reflect internal review. Provided additional bid types. | Scott Neumann |
| 1/19/2007 | 0.18 | Changes to reflect additional interfaces | Scott Neumann |
| 1/22/2007 | 0.19 | Correction to XML example to reflect schema changes. | Scott Neumann |
| 2/9/2007 | 0.20 | Updates to reflect initial TPTF comments, addition of Bid IDs, addition of more interfaces and resolution of issues. | Scott Neumann |
| 2/16/2007 | 0.21 | Defined additional interfaces. Clarifications on data types. | Scott Neumann |
| 2/23/2007 | 0.22 | Defined additional interfaces. Changed transactionId to mrid. Added description of required elements for bidding. Revised model interfaces. | Scott Neumann |
| 2/27/2007 | 0.23 | Revisions to award structures. Defined additional interfaces. | Scott Neumann |
| 3/9/2007 | 0.24 | Changed mrid to mRID. Added text to section 2.3.4 to clarify use of transaction IDs. Corrected the BidSet figure in section 3.3. Added clarifications on the use of BidSets to section 3.3. | Scott Neumann |
| 3/16/2007 | 0.25 | Revised to reflect comments from market participants. Added tables to describe required fields for each bid. Changed tradeDate to tradingDate. Added curveStyle to PriceCurve to be used as an indicator of the type of curve. Added section on error handling. | Scott Neumann |
| 3/22/2007 | 0.26 | Added more interface definitions to section 4. Augmented description of CIM usage, particularily with respect to CIM curves and schedules. Eliminated functionally redundant interfaces from section 4. Modified interfaces in section 4 to leverage IrregularIntervalSchedules instead of RegularIntervalSchedules where appropriate. Added XML examples. | Scott Neumann |
| 3/23/2007 | 0.27 | Edits required for public release. | Scott Neumann |
| 3/26/2007 | 0.28 | Correction to notification list in section 5.3.1 | Scott Neumann |
| 3/27/2007 | 0.29 | Minor corrections to some examples. Corrected figure in section 5.3.6. | Scott Neumann |
| 3/28/2007 | 0.90 | Revisions to reflect decisions of TPTF API SubGroup meeting: - Redefined timestamps in section 2.3.2 - Removed units from curves and schedules - Miscellaneous corrections to XML examples - Added state diagram for bids to section 3.1 | Scott Neumann |
| 3/30/2007 | 0.91 | Corrected omission of two curves for incremental/decremental offers | Scott Neumann |
| 4/19/2007 | 0.92 | Revisions to address April 13 comments from market participants. Other corrections and updates: - General: Corrected verbs within document to be consistent with WSDL , specifically ‘change’ is used instead of ‘update’ - 7.2.2: Corrected noun to be OutageSchedule - Added Report interface to section 9, which is a WS wrapper for PR50007 - Extended description of the base64 encoding and compression in section 2.1.3. Added example in new appendix G. | Scott Neumann |
|  | 0.93 | - Added captions for tables and figures. - Added request option to section 4.3.2 - Corrected payload definition for section 7.2.1 - Corrected timestamps that still incorrectly used Zulu time - Clarified use of common header and payload in section 5.3. - Corrected sequence diagram for Outages in section 7. - Corrected valid intervals for OutputSchedule, DCTieSchedules and SelfSchedules - Revised ThreePartOffer to reflect change in MMS definition to allow an hourly FIP/FOP - Clarified data requirements in message ‘sub-structures’ on diagrams in section 3.1. - Changed section 3.3.8 to use HSL, LSL, HEL and LEL | Scott Neumann |
| 5/2/2007 | 0.94 | - Changed some section headings in section 3 to be consistent with the protocol terminology. - Changed AS types to be consistent with protocols (e.g. Reg-Up, Reg-Down, …) - Replaces term ‘bid’ with ‘bid/offer/trade/schedule’ where appropriate - Added notification for Startup/Shutdown Instructions - Added notification for AS Obligations - Defined TmSchedule in section 2.3.1 - Global change to use TmSchedule type for XML definitions - Changes BidPriceCurve to EnergyOfferCurve in 3.3.1 - Added combined cycle to 3.3.8, 3.3.13 and generally anywhere resource is specified - Changed BidPriceCurve to PriceCurve - Changes CapacitySchedule to ASSchedule in 3.3.5 - Made more complex ‘Error’ structure for bids/offers/trades/schedules - Return ERCOT current time in section 8.2.2 - Added notification for rejection of bids/offers/trades - Appendix H extended to identify open issues | Scott Neumann |
| 5/15/2007 | 0.95 | - Updated names based on the protocols - IncDecOffer uses IncrementalCurve or DecrementalCurve, but not both - Clarified that QSE ID should be the DUNS number - Changed examples to use 00:00 instead of 24:00 for timestamps due to XML Schema dateTime inconsistencies. - Changed types for some COP elements from int to float - BidSet queries will not return bids/offers/trades or schedules that were invalid | Scott Neumann |
| 5/21/2007 | 1.00 | Updates to reflect initial approval by TPTF as version 1.0. Added externalId. Note that subsequent versions will address outstanding issues. | Scott Neumann |
| 6/14/2007 | 1.01 | - Updated SelfArrangedAS diagram to reflect optional externalID - Simplified Notification interface and addition of an acknowledgement - Added TestNotification message - Added StatusCheck Notification type for checking notification link status - PriceCurves and BidPriceCurves changed to support 10 points and 9 segments - Corrected typos - expirationTime is no longer used as a key field for any transaction mRID (see section 2.3.4) - deleteOutputSchedules flag added to ThreePartOffer - deleteTPOs flag added to OutputSchedule - Multiple FipFops allowed on IncDecOffer - Status codes of SUBMITTED, ACCEPTED, REJECTED, PENDING and ERRORS defined for status on bids, offers, trades and schedules. Revised bid state diagram. - Revised XSDs in appendices to reflect changes - Added change ActiveNotificationURL interface to section 8. | Scott Neumann |
| 7/19/2007 | 1.02 | - Removed deleteOutputSchedules from ThreePartOffer - Added statement related to ordering guarantees in section 2.6. - Added note regarding the handling of timestamps that are not interval ‘alighed’ in section 2.3.2 - Corrected figure 10 to reflect use of ‘INC’ or ‘DEC’ type in mRID for IncDecOffers - Corrected Figure 9 to not have expiration column - Modified Figure 10 to add column to map protocol terms to XML tags - Modified sections 3.3.1, 3.3.3, 3.3.4, 3.3.5 and 3.3.11 to not use expirationTime as a key value - Corrected references to ‘IrregularIntervalSchedule’ to be ‘TmSchedule’ - Revised section 5.1 and Figure 109 to reflect revised notification scheme - Revised ASOffer structure, uses a variation of PriceCurve that provides price points for each specific AS type. This affects sections 3.3.4, 2.3.1, 2.3.3 and 2.3.4 |  |
| 8/10/2007 | 1.03 | - Reflected restriction on use of 24:00 described by section 1.6 into sections 2.3.1 and 2.3.2 - Extended description of notifications to describe use of port 443 and signing requirements in section 5.3 - BidSet ‘get’ and ‘cancel’ operations must be for the same trade date - Corrected operatingMode tag in 3.3.8 description - Removed tradeId as a key for all types of trades, as this is not supported by MMS - Corrected Figure 13 to not use BidSet for cancel - Clarified the use of ‘short name’ as the QSE ID as opposed to DUNS number for use on header/Source - Corrected CRR - Modify get SystemStatus to return version information - Clarified that only one BidSet can be provided on a submission in section 3.2. - Order of element types in BidSet modified to be consistent with MMS needs - Added externalId to BidSet examples. - Clarified EnergyTrade to specify 15 minute intervals. - Added multiHourBlock indicator to ASOffer - Added externalId to XML examples - Added crrAccountHolderId to section PTP Obligation Bid - Removed expirationTime for ASTrade - Modified examples to use PENDING state - Corrected examples of namespace usage - Inserted new Appendix F to provide representative SOAP examples - Extended description of Error structure in section 3.4 - Clarified that QSE should be identified using the ‘short name’ in the Message/Source, as well as buyer and seller for trades - Revised outage structure, still in draft form | Scott Neumann |
| 9/7/2007 | 1.04 | - Added notification type for confirmed trade - Modified ASOffer, where multiHourBlock flag is now part of ASPriceCurve - Removed extraneous ‘choice’ from PriceCurve definition (this will not affect XML) - Corrected NERCTags on DCTieSchedule to be unbounded. There may be more than one tag for any given hour. - Corrected DCTieSchedule to remove NERC tag from the mRID definition. - Issues added to appendix to identify functional limitations that are a consequence of MMS not being able to support create, get or cancel operations with BidSets of mixed product types. Constraints added to sections 3.5.2 and 3.5.4. - Changed processContents attribute for ‘any’ in Message.xsd to be ‘lax’ - Documented the use of Reply/Timestamp in the response message to identify the time a submission was received by ERCOT - UserID is required for interfaces described in section 3. - Added text to section 2.3.2 to identify how non-hour aligned timestamps are handled. - Generated mRID values will not have hours. - Corrected spelling of CapacitySchedule for PTP Obligation | Scott Neumann |
| 10/11/2007 | 1.05 | - Added explicit note that trading date is based on Texas prevailing time - Added ConfirmedTrade structures to XSDs - Added additional elements and ComplexTypes to better support JAXB implementations - Revised get options in section 3.5.2. - Clarified that verbs ‘changed’ and ‘updated’ are synonyms. - Documented use of ‘partial’ mRID for getting bids/offers/schedules/trades. Limitations of MMS do not permit get for all submissions on a given date - Identified restrictions for TmSchedules in section 2.3.1 - Identified list of supported signature methods in section 2.2.2. - Changed Message.xsd to use ‘skip’ instead of ‘lax’ - Revised outage structure in section 7. - Added text related to error handling in section 3.4 | Scott Neumann |
| 10/31/2007 | 1.06 | - Corrected text and examples for ASOffer tags in section 3.3.4. - Corrected example for cancel reply in section 3.5.4 - Corrected examples in section 5.3.2 and 5.3.3 - Corrected use of mRID for get and cancel - Revised steps 5 and 6 in example of section 3.1 - Added AwardSet element to ErcotInformation.xsd - Clarified the end time for the last TmPoint in a TmSchedule in section 2.3.1 - Message structure corrected to allow multiple ID values in the reply in section 2.1.4 - Changed mRID for PTPObligationBid to NOT include CRR account holder ID. This remains as an optional field reserved for future use - Added StartTime as a request parameter for sections 4.3.10, 4.3.11, 4.3.15, 4.3.16 - Added section for WGRPP notification | Scott Neumann |
| 12/3/2007 | 1.07 | - Minor rearrangement of XSDs to group award elements together in ErcotAwards.xsd, to avoid namespace conflicts. - Changed name of ‘OutageSchedule’ element to ‘OutageSet’ - Moved StartupShutdownInstructions, ASObligations, ConfirmedTrades and UnconfirmedTrades to ErcotAwards.xsd to avoid conflicts. - General updates to Outage Scheduler interfaces to reflect changes and refinements to OS interfaces. |  |
| 1/22/2008 | 1.08 | - Made crrAccountHolderId a required field in PTP Obligation Bid - Added crrAccountHolderId back in the PTP mRID PTP.bidid.crrAHId.source.sink - Section 3.5.2 - Added Querying Bids short mRID specific required tokens for various bid types. - Removed mention of blank token in the mRID writeup - Added the PriceCurve/multiHourBlock flag to Energy Bid and Energy only offer - Added Specific ASType codes for ASOffer, ASTrade, and SelfArrangeAS - Made expirationTime a required Field for ASOffer, Three Part Offer, Inc Dec offer, and Energy Only Offer - Made NOIEPeakLoadForecast a required field for CRR - Made StartTime/EndTime not Required fields for cop/os/inc/dec/3po as we use StartTime/EndTime associated with each individual 3PSO element to send to MMS - Removed NewSchedule + added "Planned" attributes to OutageSchedule Complex type - Added Figure 133 - Outage Creation Elements - Added a table of valid AS Types for all AS transactions to section 2.3.5 | Nemat Sarnevesht |
| 2/07/2008 | 1.09 | - Corrected payload of “Get” Awardset - Added the the “Outage” to the payload of the “Create” OS Reply - Changed the verb from “Update” to “Change” in Outageset to be consistent with the BidSets - Corrected the Short mRID logic to include needed tokens for CRR, EB, EOO, PTP - 5.3.12 Updated Outage notification structure - 5.3.14 Updated the Wind Generation Notification Structure and section - 2.1.1 Updated the diagram for EWS Operations-Verbs-Nouns - Tagged startTime and endTime as not used and deprecated - OutputSchedule / EnergySchedule / startTime - OutputSchedule / EnergySchedule / endTime - CRR / CapacitySchedule/ startTime - CRR / CapacitySchedule/ endTime - PTPObligation / CapacitySchedule / startTime - PTPObligation / CapacitySchedule / endTime - SelfArrangedAS / CapacitySchedule / startTime - SelfArrangedAS / CapacitySchedule / endTime - EnergyTrade / EnergySchedule / startTime - EnergyTrade / EnergySchedule / endTime - CapacityTrade / CapacitySchedule / startTime - CapacityTrade / CapacitySchedule / endTime - ASTrade / ASSchedule / startTime - ASTrade / ASSchedule / endTime - EnergySchedule / EnergySchedule / startTime - EnergySchedule / EnergySchedule / endTime - SelfSchedule / EnergySchedule / startTime - SelfSchedule / EnergySchedule / endTime - For Outage Query added for Request/ID to namevalue pair: OuatgeID or GroupID - Updated all Outage Scheduer XML Schema figures - Added description for Outage Types - Added note to refelect duplication for Market totals (4.3.4) and Total DAM Energy (4.3.23) - Corrected the Noun for LoadProfile to LoadProfiles in 4.3.17 - Removed Mitigated and Proxy Cureves from Market Information Section - Updated all XML schemas in Appendix C | Nemat Sarnevesht |

- Document Approvals

| Date | Approved By | Approval Documented In (select) |
| --- | --- | --- |
|  | Susan Jinright Enterprise Architecture Manager | ___ Approval email on file ___ Signature |

The following table identifies section by section approvals. This recognizes that the document will evolve and need to be approved in phases during the course of the project.  A convention of adding “***” to the section title is used to identify the sections ready for approval.  *** is added at the highest level and not repeated through the subsections.

| Date | Version | Sections Approved | Approved By |
| --- | --- | --- | --- |
| 5/21/2007 | 1.0 | 1, 2, 3.1, 3.2, 3.3.1-3.3.5, 3.3.8, 3.3.10, 3.3.11, 3.3.13, 3.4, 3.5, 4.1, 4.2, 4.3.2, 4.3.3, 4.3.5, 4.3.26, 5.3.2, 5.3.3, 6, 7.1, 7.2.1, 8, 9 | TPTF |
| 7/19/2007 | 1.02 | 3.3.7, 3.3.14, 3.3.15, 4.3.12, 4.3.13, 4.3.20, 4.3.21, 4.3.25, 5.3.1, 5.3.9, 5.3.12 | TPTF |
| 11/6/2007 | 1.06 | 3, 4 | TPTF |
| Jan 2008 | 1.08 | All | TPTF |

- Table of Contents

List of Figures	v

1.	Introduction ***	4

1.1.	Purpose	4

1.2.	Scope	4

1.3.	Definitions, Acronyms, and Abbreviations	5

1.4.	References	7

1.5.	Overview	7

1.6.	Program-level Standards	7

2.	Services Organization ***	9

2.1.	Common Message Structure	9

2.1.1.	Message Header Structure	9

2.1.2.	Request Message Structures	13

2.1.3.	Payload Structures	14

2.1.4.	Response Message Structures	16

2.2.	Common Security Implementation	17

2.2.1.	Secure the Transport layer	17

2.2.2.	Secure SOAP messages	18

2.3.	Modeling and Conventions	19

2.3.1.	Use of the IEC CIM	19

2.3.2.	Representation of Time	24

2.3.3.	Market Products	25

2.3.4.	Management and Use of Transaction IDs (mRIDs)	26

2.3.5.	Other Conventions	28

2.4.	Delivery Approach	29

2.5.	Technical Interoperability	30

2.6.	Service Level Agreements	30

2.7.	Auditing, Monitoring and Management	32

2.8.	Versioning	32

2.9.	Governance	33

2.10.	Web Service Configuration Standards	33

3.	Market Transaction Service ***	34

3.1.	Interfaces Provided	34

3.2.	Interfaces Required	37

3.3.	Message Specifications	38

3.3.1.	Three-Part Supply Offer	41

3.3.2.	Self-Arranged Ancillary Service Quantities	47

3.3.3.	Incremental and Decremental Energy Offer Curves ***	50

3.3.4.	Ancillary Service Offer	55

3.3.5.	Ancillary Service Trade	61

3.3.6.	Capacity Trade	63

3.3.7.	Congestion Revenue Rights (CRR) Offer	65

3.3.8.	Current Operating Plan (COP)	69

3.3.9.	DC Tie Schedule	74

3.3.10.	DAM Energy Bid	78

3.3.11.	DAM Energy-Only Offer	80

3.3.12.	Energy Trade	82

3.3.13.	Output Schedule	85

3.3.14.	PTP Obligation Bid	87

3.3.15.	Self-Schedule	89

3.4.	Error Handling	91

3.5.	Usage and XML Examples	93

3.5.1.	Offer and Bid Set Submission	93

3.5.2.	Querying Bids, Offers, Trades and Schedules	97

3.5.3.	Updating Bids, Offers, Trades and Schedules	99

3.5.4.	Canceling Bids, Offers, Trades and Schedules	100

4.	Market Information ***	102

4.1.	Interfaces Provided	102

4.2.	Interfaces Required	103

4.3.	Message Specifications	103

4.3.1.	Get AwardSet	104

4.3.2.	Forecasted Load	106

4.3.3.	Real-Time System Load	107

4.3.4.	Market Totals   *** Same as Total DAM Energy in section 4.3.23 ***	109

4.3.5.	Market LMPs and SPPs	111

4.3.6.	Market MCPCs	114

4.3.7.	Binding Constraints	116

4.3.8.	Active Contingencies in SCED	118

4.3.9.	Ancillary Service Obligation	120

4.3.10.	Dynamic Ratings	121

4.3.11.	Voltage Profiles	123

4.3.12.	Total Regulation	124

4.3.13.	Load Ratio Share	125

4.3.14.	Competitive Constraints	127

4.3.15.	Load Distribution Factors	129

4.3.16.	Shift Factor	130

4.3.17.	Customer Load Profile	131

4.3.18.	Aggregated Ancillary Service Offer Curves	133

4.3.19.	Ancillary Service Plan	134

4.3.20.	Unit Availability	136

4.3.21.	Startup and Shutdown Instructions	138

4.3.22.	Total Ancillary Service Offers	139

4.3.23.	Total DAM Energy   *** Same as Total Market Totals in section 4.3.23 ***	141

4.3.24.	Derated CRRs	142

5.	Notifications ***	144

5.1.	Interfaces Provided	144

5.2.	Interfaces Required	145

5.3.	Message Specifications	146

5.3.1.	Notices and Alerts ***	146

5.3.2.	Offer and Bid Set Acceptance	148

5.3.3.	Offer and Bid Set Errors	148

5.3.4.	Confirmed and Unconfirmed Trades	149

5.3.5.	Energy Offer Awards	151

5.3.6.	DAM Energy-Only Offer Awards	152

5.3.7.	DAM Energy Bid Award	153

5.3.8.	Ancillary Service Awards	154

5.3.9.	CRR Awards	155

5.3.10.	PTP Obligation Awards	156

5.3.11.	Ancillary Service Obligations	157

5.3.12.	Outage Notifications	158

5.3.13.	Startup/Shutdown Instructions	159

5.3.14.	Wind Generation	160

6.	Acknowledgement of Alerts ***	161

6.1.	Interfaces Provided	161

6.2.	Interfaces Required	161

6.3.	Message Specifications	162

7.	Outage Scheduling Interfaces ***	163

7.1.	Interfaces Provided	163

7.2.	Message Specifications	163

7.2.1.	Outage Creation	163

7.2.2.	Outage Query	179

7.2.3.	Outage Update	180

7.2.4.	Outage Cancellation	181

8.	Utility Interfaces ***	182

8.1.	Interfaces Provided	182

8.2.	Message Specifications	182

8.2.1.	Get mRID	182

8.2.2.	System Status	183

8.2.3.	Request Test Notification	184

8.2.4.	Change Active Notification URL	184

9.	Reports ***	186

9.1.	Interfaces Provided	186

9.2.	Message Specifications	186

9.2.1.	Get Reports	186

Appendix A: WS-Notifications	188

Appendix B: WSDL for Market Requests	191

Appendix C: XML Schemas for Message and Payload Definitions	193

Appendix E: Annotated SOAP Message	238

Appendix F: SOAP Examples	242

Appendix G: XSL Examples	248

Appendix H: Payload Compression Example	252

Appendix I: Issues	253

# List of Figures

Figure 1 - Web Service Interfaces	11

Figure 2 - Message Header Structure	12

Figure 3 - Request Message Structure	13

Figure 4 - Message Request Parameters	14

Figure 5 - Message Payload Container Structure	15

Figure 6 - Response Message Structure	16

Figure 7 - CIM Curves and Schedules	20

Figure 8 - Time Schedule Structure	22

Figure 9 - Required Keys for Market Products	26

Figure 10 - mRID Structures	28

Figure 11 - Example BidSet Sequence Diagram	35

Figure 12 - State Diagram for Bids, Trades, Offers and Schedules	37

Figure 13 -  Message: create BidSet	38

Figure 14 - Message: reply BidSet	38

Figure 15 - BidSet Header Structure	39

Figure 16 – Bid/Offer/Trade/Schedule Common Elements	41

Figure 17 – ThreePartOffer Structure	42

Figure 18 - PriceCurve Structure	43

Figure 19 - Fip/Fop, StartupCost and MinimumEnergy Structures	44

Figure 20 - ThreePartOffer Requirements	46

Figure 21 - SelfArrangedAS Structure	48

Figure 22 - CapacitySchedule Structure	49

Figure 23 - SelfArrangedAS Requirements	50

Figure 24 - IncDecOffer Structure	51

Figure 25 - IncDecOffer Requirements	53

Figure 26 - ASOffer Structure	55

Figure 27 - Price Curves using ASPriceCurve	57

Figure 28 - ASOffer Requirements	59

Figure 29 - ASTrade Structure	62

Figure 30 - ASTrade Requirements	63

Figure 31 - CapacityTrade Structure	64

Figure 32 - CapacityTrade Requirements	65

Figure 33 - CRR Structure	66

Figure 34 - MinimumReservationPrice Structure	67

Figure 35 - CRR Requirements	68

Figure 36 - COP Structure	70

Figure 37 - Limits Structure	71

Figure 38 - ASCapacity Structure	71

Figure 39 - COP Requirements	72

Figure 40 - DCTieSchedule Structure	75

Figure 41 - EnergySchedule Structure	76

Figure 42 - NERCTags Structure	76

Figure 43 - DCTieSchedule Requirements	77

Figure 44 - EnergyBid Structure	78

Figure 45 - EnergyBid Requirements	79

Figure 46 – EnergyOnlyOffer Structure	80

Figure 47 - EnergyOnlyOffer Requirements	81

Figure 48 - EnergyTrade Structure	82

Figure 49 - EnergySchedule Structure	83

Figure 50 - EnergyTrade Requirements	84

Figure 51 - OutputSchedule Structure	85

Figure 52 - OutputSchedule Requirements	86

Figure 53 - PTPObligation Requirements	87

Figure 54 - MaximumPrice Sub-Structure	88

Figure 55 - PTPObligation Requirements	88

Figure 56 - SelfSchedule Structure	90

Figure 57 – SelfSchedule  Requirements	91

Figure 58 - Error Handling	92

Figure 59 - create BidSet Request Parameters	93

Figure 60 - reply BidSet Parameters	94

Figure 61 – Short mRID Key String	97

Figure 62 - get BidSet Parameters	98

Figure 63 - reply BidSet Parameters	98

Figure 64 - change BidSet Parameters	99

Figure 65 - reply BidSet Parameters	99

Figure 66 - cancel BidSet Parameters	100

Figure 67 - reply BidSet Parameters	101

Figure 68 - Market Information Request Sequence Diagram	102

Figure 69 - Parameters for 'get' Requests	103

Figure 70 - Parameters for 'reply' Messages	103

Figure 71 - Get AwardSet Parameters	104

Figure 72 - Get AwardSet Response	104

Figure 73 - AwardSet Container Structure	105

Figure 74 - Generalized Award Structure	105

Figure 75 – Load Forecast Request Parameters	106

Figure 76 – Load Forecast Response Parameters	107

Figure 77 - Load Forecast Structure	107

Figure 78 - SystemLoad Request Parameters	108

Figure 79 - SystemLoad Response Parameters	108

Figure 80 - SystemLoad Structure	109

Figure 81 - Market Totals Request Parameters	110

Figure 82 - MarketTotals Response Parameters	110

Figure 83 - MarketTotals Structure	111

Figure 84 - LMP and SPP Request Parameters	112

Figure 85 - LMP and SPP Response Parameters	112

Figure 86 - LMP Container Structure	113

Figure 87 - SPP Container Structure	114

Figure 88 - MCPC Container Structure	116

Figure 89 - BindingConstraint Container Structure	118

Figure 90 - SCEDViolatedConstraints Container Structure	120

Figure 91 - ASObligations Container Structure	121

Figure 92 - DynamicRatings Container Structure	122

Figure 93 - VoltageProfiles Container Structure	124

Figure 94 - TotalRegulation Structure	125

Figure 95 - LoadRatioShares Container Structure	127

Figure 96 - CompetitiveConstraints Container Structure	128

Figure 97 - LoadDistributionFactors Container Structure	130

Figure 98 - ShiftFactors Container Structure	131

Figure 99 - LoadProfile Container Structure	132

Figure 100 - AggregatedASOfferCurve Schedule	134

Figure 101 - ASSystemPlan Container Structure	135

Figure 102 - ASSchedule Structure	136

Figure 103 - UnitAvailabilities Container Structure	137

Figure 104 - StartupShutdownInstructions Container Structure	139

Figure 105 - ASOffers Structure	140

Figure 106 - TotalEnergy Structure	142

Figure 107 - DeratedCRRs Container Structure	143

Figure 108 - Notification Sequence Diagram	145

Figure 109 - WS-Notifications Message Structure	146

Figure 110 - Notification Structure	147

Figure 111 - Notice and Alert Types	148

Figure 112 - UnconfirmedTrades Container	149

Figure 113 - Confirmed Trades	150

Figure 114 - AwardedEnergyOffer Structure	151

Figure 115 - AwardedEnergyOnlyOffer Structure	152

Figure 116 - AwardedEnergyBid Structure	153

Figure 117 - AwardedAS Structure	154

Figure 118 - AwardedCRR Structure	155

Figure 119 - AwardedPTPObligation Structure	156

Figure 120 – Ancillary Service Obligation Structure	157

Figure 121 - Outage Structure	158

Figure 122 - StartupShutdownInstructions Structure	159

Figure 123 - Wind Generation Resource Production Potential Structure	160

Figure 124 - Alert Acknowledgement Request	161

Figure 125 - Alert Acknowledgement Response	161

Figure 126 - Outage Scheduling Interface Sequence Diagram	163

Figure 127 - Outage Creation Request	164

Figure 128 - Outage Creation Response	164

Figure 129 - OutageSet Structure	164

Figure 130 - High-level Outage Structure	165

Figure 131 - Outage Information	166

Figure 132 - Outage Grouping, Details, Schedule, Contacts, Notes, and Opportunity	171

Figure 133 – Outage Creation Elements	179

Figure 134 - OutageSet Query Request	179

Figure 135 - OutageSet Query Response	180

Figure 136 - Outage Update Request	180

Figure 137 - Outage Update Response	181

Figure 138 - Outage Cancel Request	181

Figure 139 - Outage Cancel Response	181

Figure 140 - Get mRID Request	182

Figure 141 - Get mRID Response	183

Figure 142 - SystemStatus Request	183

Figure 143 - SystemStatus Response	184

Figure 144 - TestNotification Request	184

Figure 145 - TestNotification Response	184

Figure 146 – Change ActiveNotificationURL Request	185

Figure 147 – Change ActiveNotificationURL Response	185

Figure 148 - Reports Request	186

Figure 149 - Reports Response	187

Figure 150 - Reports Container	187

# Introduction ***

This document describes the message structure machine to machine interfaces for Market Participant applications that need to interact with ERCOT Nodal Market systems. The intended audience of this document are developers that will be integrating Market Participant applications to the ERCOT Nodal systems through the use of the interfaces described within this specification.

Where sections 1 and 2 of this document apply to all interfaces, sections 3 and beyond describe specific groupings of interfaces. The appendices provide XML Schemas, WSDLs and additional examples.

The interfaces and related interactions described by this document define the externally-visible (black box view) perspective of the services provided by this project. It is the intent of this specification and interface architecture to shield Market Participants from the details of systems integration internal to ERCOT.

## Purpose

The interfaces described by this document are intended to be used by Market Participants for machine to machine integration. This document is intended to provide all the details of the message structures and technical interoperability standards required for the machine to machine interface.

## Scope

The scope of this document is to describe web services provided for integration by Market Participants from the perspective of external integration. This document has program level scope as related to web services that would be used by Market Participants for machine to machine interaction with nodal applications as detailed in an agreed list of interfaces to be managed by the Nodal project. The intent of this design is to leverage the integration layer (IL) to expose web services needed for external integration by Market Participants.

The following are specifically outside the scope of this document:

- The details of integration from the IL to specific applications (e.g. MMS, EMS) is outside the scope of this specification and will be described in subsequent design documents

- Inter-Control Center Communications Protocol (ICCP) communications

- Interactions with application User Interfaces (UI)

- Access of information through MIS

This document is not intended as a replacement for the current services provided by the ERCOT used to access reports. The ‘get Reports’ interface described in section 9.2.1 provides a wrapper for this interface.

## Definitions, Acronyms, and Abbreviations

| Term/Acronym | Definition |
| --- | --- |
| ADJ | Adjustment |
| AS | Ancillary Services, includes spinning reserve, non-spinning reserve, responsive reserve and regulation |
| award | An award identifies the acceptance of a bid or offer. |
| BidSet | A BidSet is an XML container type for a collection of bids, offers, trades and schedules as defined by the IEC Common Information Model |
| CIM | Common Information Model, an IEC standard |
| CRR | Congestion Revenue Rights, a system implemented at ERCOT by Nexant that is responsible for CRR auctions |
| CSV | A file format that uses values separated by commas |
| DAM | Day Ahead Market |
| DRUC | Day-Ahead Reliability Unit Commitment |
| EMS | Energy Management System, a system implemented at ERCOT by Areva |
| FIP | Fuel Index Price |
| FOP | Fuel Oil Price |
| FTP | File Transfer Protocol |
| GUID | Globally Unique Identifier. GUIDs are usually implemented as random 128 bit numbers that are usually represented as a string of hexadecimal digits. |
| HASL | High ancillary services limit |
| HDL | High dispatch limit |
| HEL | High emergency limit |
| HRUC | Hour-Ahead Reliability Unit Commitment |
| HSL | High sustained limit |
| IEC | International Electrotechnical Commission |
| LASL | Low ancillary services limit |
| LDL | Low dispatch limit |
| LEL | Low emergency limit |
| LMP | Location marginal price |
| LSL | Low sustained limit |
| MCPC | Market clearing price for capacity |
| MIS | Market Information System, an umbrella for the various interfaces provided to Market Participants by ERCOT |
| MMS | Market Management System, a system implemented at ERCOT by ABB |
| MP | Market Participant |
| MW | Megawatt, a measure of power |
| MWh | Megawatt hour, a measure of energy |
| NMMS | Network Model Management System, a system implemented at ERCOT by Siemens |
| NOMCR | Network operating model change request |
| Non-Spin | Non-spinning reserve |
| OASIS | Organization for the Advancement of Structured Information Systems |
| Operating Date | Synonymous with Trade Date |
| PMCR | Planning Model Change Request |
| POC | Proof Of Concept |
| QSE | Qualified Scheduling Entity. This is a type of Market Participant. Each QSE is identified by a certificate. A QSE is identified by a DUNS number, ‘long name’ and ‘short name’ |
| RDF | Resource Descriptor format. An XML format used by NMMS for model exchanges using IEC 61970-501. |
| Reg-Down | Regulation down |
| Reg-Up | Regulation up |
| RRS | Responsive reserve service |
| RTM | Real-Time Market |
| SAMR | Special Action Model Request |
| sink | Sink settlement point |
| SOAP | Simple Object Access Protocol |
| SoSA | System of Systems Architecture |
| source | Within a bid, offer, schedule, trade or award this refers to the source settlement point. Within a message header, this refers to the ID of the market participant. |
| SP | Settlement Point |
| SPP | Settlement Point Price |
| TP | Transmission Provider |
| Trading Date | Synonymous with Operating Date. Represented in XML using the ‘tradingDate’ tag. This is based on Texas prevailing time. |
| UTC | Universal Coordinated Time, an international standard |
| WS | Web Services. There are many web service standards that are commonly prefixed by ‘WS’. |
| WSDL | Web Services Definition Language |
| XML | eXtensible Markup Language |
| XSD | XML Schema, used to define the structure of XML documents |
| XSL | XML Stylesheet Language |
| Z | Zulu, an indicator for the use of GMT or UTC time |

## References

| Artifact | Definition |
| --- | --- |
| External Interfaces Conceptual Design | Conceptual design for external interfaces using web services |
| External Interfaces Security Design Specification | Detailed security design for external interfaces. This is a companion document to the External Interfaces Conceptual design. |
| OASIS WS-Notifications | OASIS Web Services Base Notification standard |
| OASIS WS-Security | OASIS Web Services Security |

- Additional references to related standards are described in section 1.6.

## Overview

This document focuses on the external interface design and related interface definitions from all perspectives except for security, which is described in detail in a companion document. The interfaces are to be provided using web services, where a rationale is provided in subsequent sections. The web services defined by this document will support a wide variety of machine to machine information exchanges.

## Program-level Standards

In general, the design described by this document will leverage web services and related security standards as defined by the World-Wide Web Consortium (W3C) and OASIS. Program-level standards include those related to security, as well as basic web service standards including:

- XML

- XML Schema

- XPath

- XSL

- SOAP

- Web Services

- WSDL

- RDF

These are described in the companion security design document. W3C standards can be freely accessed from http://www.w3.org.

Another key program standard is the IEC Common Information Model (CIM), as defined by IEC 61970-301. This is used to define models used by ERCOT, which are exchanged using IEC 61970-501. It will also be leveraged by this design for the definition of messages used for interfaces. There is also a standard for message structures defined by IEC 61968-1. There standards can be purchased from the IEC web site at http://www.iec.ch. Materials related to IEC standards, including the CIM model it self can be freely obtained from the UCA International Users Group sharepoint at http://sharepoint.ucausers.group.org/CIM.

There are several key Internet Engineering Task Force references. These include:

- Internet Engineering Task Force RFC 2828: Internet Security Glossary

- Internet Engineering Task Force RFC 2119: Key words to indicate RFC requirement levels

- Internet Engineering Task Force RFC 2246: Transport Layer Security (TLS)

- Internet Engineering Task Force RFC 3275: XML Digital Signature and Processing

- IETF documents can be freely obtained from http://www.ietf.org.

An ERCOT variation of the OASIS WS-BaseNotifications standard is used to define the mechanism for issuance of asynchronous notification messages to Market Participants. Given the application of WS-Notifications for Nodal, only a subset of the capabilities of WS-Notifications are required. OASIS standards being leveraged include:

- Web Services Notifications (WS-Notifications)

- Web Services Security (WS-Security)

- Web Services Security X.509 Token Profile

- Web Services Security Username Token Profile

- OASIS standards can be freely obtained from http://www.oasis-open.org.

The definition of timestamps is specified by ISO-8601, with the exception that timestamps of 24:00:00 are not used for compatibility reasons. This is partly a consequence of the XML Schema definition for ‘dateTime’, where hour 24 is not explicitly allowed. There are some implementations of timestamps within software products that do not correctly handle timestamps of 24:00:00.

ISO standards can be purchased online from a variety of sources including ANSI, at http://www.ansi.org. Descriptions of ISO-8601 can be freely obtained from other sources including wikipedia

# Services Organization ***

The services described by this document are defined using a combination of Web Services Definition Language (WSDL) and XML Schema. The WSDLs are organized as follows:

- One or more WSDLs defined by ERCOT, defining operations related to synchronous request/reply web service messages

- WSDL defined by OASIS for WS-Notifications to provide support for asynchronous messaging, using web services

In both of the above cases, one or more XML Schemas (XSD) are used to define the structure of message payloads.

Example WSDL and XSD are provided in the appendices. It is anticipated that these would be key design artifacts for developers. The following diagram provides an overview of the interfaces described by this document.

## Common Message Structure

Unless otherwise specified, all messages use a common message envelope, where a predefined structure is used for requests and another structure is used for responses. This structure is based upon the IEC 61968-1 standard. Messages are constructed with several sections, including:

- Header: required for all messages, using a common structure for all service interfaces

- Request: optional, defining parameters needed to qualify request messages

- Reply: Used for response messages to indicate success, failure and error details

- Payload: optional, used to convey message information as a consequence of the ‘verb’ and ‘noun’ in the message Header. The payload structure provides options for payload compression.

### Message Header Structure

Common to both the request and response messages is a header structure. The header has several required fields that must be populated, these include:

- Verb, to identify a specific action to be taken. There are an enumerated set of valid verbs, where commonly used values include ‘get’, ‘create’, ‘change’, ‘cancel’, ‘close’ and ‘reply’. For notification messages ‘past tense’ verbs are used, which can include ‘created’, ‘changed’, ‘canceled’ and ‘closed’. Implementations should treat verbs ‘update’ and ‘updated’ as synonyms to ‘change’ and ‘changed’. There is also a special case when the noun is ‘BidSet’, where a ‘create’ is equivalent to a ‘change’ due to the nature of the market system.

- Noun: to identify the subject of the action and/or the type of the payload (e.g. BidSet, Notification) if a payload is provided.

- Source: identifying the source of the message, which should be the ID of the Market Participant or ERCOT (typically for reply messages). This should be the ‘short name’ of the QSE. (This will be verified against the DUNS number in the certificate)

- Revision: To indicate the revision of the message definition. This should be ‘1’ by default.

- Nonce: A unique number that would not be repeated by the Market Participant within the period of at least a day. This could be a sequence number, large random number or a GUID. This is defined by WS-Security. A combination of this number and the timestamp make the message unique for a given time period.

- Created: A timestamp to indicate when the message was created. This value and the Nonce are used to protect against replay attacks. This is defined by WS-Security.

- UserId: Should be supplied as it may be required for some interfaces, depending upon underlying implementations.

The following diagram describes the header structure used for request and response messages.

- Figure 1 - Web Service Interfaces

- Figure 2 - Message Header Structure

There are several optional fields that may be populated. In the above diagram, the optional items are represented using dashed borders. If the MessageID is populated on a request, it will be returned on the reply. The Comment field is never used for any processing-related logic. The UserID may be used to indicate the person responsible for initiating a transaction, and will be logged as appropriate, but verification is the responsibility of the Source system.

In order to identify the Market Participant in a uniform manner, the registered ‘short name’ of the Market Participant should be supplied as the value of Header/Source. ERCOT will verify that this value is consistent with the Market Participant as identified by the certificate, noting the the DUNS number is contained within the certificate.

### Request Message Structures

The following diagram describes the structure of a request message that would be used in conjunction with a WSDL operation.

- Figure 3 - Request Message Structure

The RequestMessage can also optionally contain a package with parameters relevant to the request, called Request. It is likely that different or variant Request packages may be defined to be used in conjunction with messages for a specific web service operation. In those cases, the corresponding WSDL and XSD would identify the optional parameters. The description of the interface (in subsequent sections of this document) would identify the usage of those parameters. The following is an example RequestType used in the definition of a Request package that defines some common parameters used for requests, however it is important to note that these are typically application specific. These parameters are most commonly used in conjunction with ‘get’ requests as qualifiers.

- Figure 4 - Message Request Parameters

One key use of the RequestType is to avoid the placement of application specific request parameters in the header or within payload definitions. Also, where a set of requests that were supported by a specific web service operation had significantly different requirements for information in the RequestType, it could justify the use of RequestType variants, where each variant will be used for the definition of messages for the specific web service operation.

### Payload Structures

There are some requests where a Payload must be provided, as would be the case for a message with a verb of ‘create’ or ‘change’. Payloads are typically XML documents that conform to a defined XML schema. However, there are exceptions to this rule. Some XML payloads may not have useful XML schemas, as in the case of RDF files or dynamic query results, as well as non-XML formats such as CSV and PDF. There may also be cases where a large payload must be compressed, in the event that it would become very large and otherwise consume significant network bandwidth.  In order to accommodate a variety of payload format options the following payload structure is used.

- Figure 5 - Message Payload Container Structure

In the previous diagram, any type of XML document may be included, using the XML ‘any’ structure. While this provides options for loose-coupling, specific complex types defined by XML schemas (XSDs) can be used as well. The WSDL in the appendix provides an example of this case.

Payloads can also be supplied as XML encoded strings using the ‘Document’ tag, although this method is less preferred than used of the XML ‘any’.

There are also some cases where a zipped, base64 encoded string is necessary, and would be passed using the ‘Compressed’ tag. The Gnu Zip compression shall be used in order to provide compatibility within both Java and Microsoft .Net implementations. A Java example is provided in Appendix G. Specific examples of the usage of payload compression would be where:

- An XML payload, conforming to a recognized XML schema exceeds a predefined size (e.g. 1MB). This would be very common for large Market Participant sets of bids, offers, trades or schedules

- A payload has a non-XML format, such as PDF, Excel spreadsheet, CSV file or binary image

- A payload is XML, but has no XML schema and exceeds a predefined size, as would be the case of a dynamic query that would return an XML result set

When a payload is compressed and base64 encoded, it is stored within the Payload/Compressed message element as a string.

The format tag can be used to identify specific data formats, such as XML, RDF, PDF, DOC, CSV, etc. This is especially useful if the payload is compressed. The use of this tag is optional, and would typically only be used when the payload is stored using the Payload/Compressed message element.

The above options provide an alternative to the use of SOAP attachments. SOAP attachments are more difficult to secure since the SOAP envelope signature signs the SOAP body but does not sign the attachment. This also requires that the payload is processed separately from the rest of the SOAP message (e.g. the message is parsed to extract the payload, and then the payload is parsed and processed). However, we believe this implementation approach is less complex than using SOAP attachments.

### Response Message Structures

The following diagram describes the structure of a response message that would be used in conjunction with a WSDL operation, as a response to the request message.

- Figure 6 - Response Message Structure

The ReplyCode would be set to OK to indicate that the request was successful, otherwise it would be set to ERROR, and one or more Error elements would be provided to describe the error(s). There may also be more specific error information provided within the payload, as in the case of bids within a BidSet container.

If the MessageID was set in the Header for the RequestMessage, the value will be returned in the Header of the ResponseMessage.

## Common Security Implementation

This section will provide an overview of security from the perspective of implementation requirements for Market Participants. Market Participants MUST take two basic steps in securing their Web Services Interaction with ERCOT:

- Secure the Transport layer

- Secure SOAP messages

### Secure the Transport layer

The transport layer is secured by deploying Secure Socket Layer (SSL) and Transport Layer Security (TLS) following these steps:

- Obtain client side certificate (these Certificates are issued by Verisign under the ERCOT brand).

- Implement mutual authentication (explained below).

- Ensure minimum SSL/TLS security settings

Note that TLS is an enhanced specification based on SSL. References to SSL refer to both SSL and TLS.

SSL is a standard mechanism for Web services that is available on virtually all application servers. This widely used, mature technology, which secures the communication channel between client and server, will satisfy all of ERCOT’s use cases for secure Web Service communications. Since it works at the transport layer, SSL covers all information passed in the channel as part of a message exchange between a client and a server, including attachments. Authentication is an important aspect of establishing an HTTPS connection.  Many platforms support the following authentication mechanisms for Web Services using HTTPS:

- The server authenticates itself to clients with SSL and makes its certificate available.

- The client uses basic authentication over an SSL channel.

- Mutual authentication with SSL, using the server certificate as well as the client certificate, so that both parties can authenticate to each other.

With Web Services, the interaction use case is usually machine to machine; that is, it is an interaction between two application components with no human involvement. Machine-to-machine interactions have a different trust model from typical website interactions. In a machine-to-machine interaction, trust must be established proactively, since there can be no real-time interaction with a user about whether to trust a certificate. Ordinarily, when a user interacts with a website via a browser and the browser does not have the certificate for the site, the user is prompted about whether to trust the certificate. The user can accept or reject the certificate at that moment. With Web Services, the individuals involved in the deployment of the Web Service interaction must distribute and exchange the server certificate, and the client certificate ( for mutual authentication), prior to the interaction occurrence.

The combination of the two settings—CONFIDENTIAL for transport guarantee and CLIENT-CERT for auth-method—enables mutual authentication. When set to these values, the containers for the client and the target service both provide digital certificates sufficient to authenticate each other. (These digital certificates contain client-specific identifying information.)

### Secure SOAP messages

Besides creating a secure communication channel between a client and a Web Service, ERCOT Web Service message exchanges require that security information be embedded within the SOAP message itself. This is often the case when a message needs to be processed by several intermediary nodes before it reaches the target service or when a message must be passed among several services to be processed.

Message-level security is very useful in XML document-centric applications, since different sections of the XML document may have different security requirements or be intended for different users.

SOAP messages’ signing is done through the following:

- Obtain application/system signing certificate.  (These certificates are issued by Verisign under the ERCOT brand).

- Sign all SOAP messages, using Web Services Security Standards and its X.509 Certificate Token Profile (see J2EE and .NET code snippets in Appendix D)

- Message headers MUST include a timestamp and a nonce

- Validate all SOAP messages have:

  - Signature

  - Certificates

  - Revocation status of certificates

  - Use of timestamp and nonce (to prevent replay attacks)

There are a variety of signature method/security tokens that are commonly used. ERCOT will sign outbound messages using SHA-1/RSA and strongly recommends its use by market participants for signing messages. However, the following SOAP signing methods will be supported for incoming messages:

- MD5/RSA

- DSA (which implies SHA-1)

- SHA-256 and SHA-512 with RSA

- ECDSA

Appendix D provides examples for the generation of signatures. Appendix E provides an annotated example of a SOAP message.

## Modeling and Conventions

There are several conventions that are used for definitions, data items and information models.  Note that additional values and conventions will be defined as market requirements are finalized.

### Use of the IEC CIM

Where possible the IEC CIM should be leveraged. The ERCOT Nodal project and related systems use the CIM as a key standard. One example of this is for the submission and management of models through the NMMS system. Examples of leveraging the CIM for external web services include:

- The use of data structures defined by the IEC CIM where appropriate in payload definitions

- CIM naming conventions are used wherever possible, e.g. ClassName, propertyName

- The properties ‘startTime’ and ‘endTime’ are typically used to identify time intervals, as they are also used within many CIM classes. Instead of using combinations of start date, start hour and potentially an interval number (e.g. to represent 15 minute intervals), absolute times should be specified.

- The property ‘mRID’ is used to uniquely identify objects such as bids, offers, trades, awards and schedules

- A trade or operating date is always derived from the startTime of a bid, offer, trade or schedule.

Where interfaces have references to resources, QSEs, settlement points and electrical buses, the  ‘name’ property is used for the reference, as opposed to ‘mrid’. This is to maximize legibility and to provide for consistency with underlying market applications. Consequentially, simplified XML is used, where the use of the ‘name’ property is implied as opposed to explicit use of a <name> tag.

The following class diagram describes the structure of CIM Curves, IrregularIntervalSchedules and RegularIntervalSchedules. These classes are CIM building blocks, where there are many other classes in the CIM that inherit from these three basic types of curves and schedules (e.g. PriceCurve).

- Figure 7 - CIM Curves and Schedules

For use within ERCOT Nodal, customizations and specializations of CIM curves and schedules are defined. One example is that units (e.g. xUnit, y1Unit, value1Unit) are not used.

For Curves:

- A curveStyle can be specified to identify the type of curve (e.g. CURVE, FIXED, VARIABLE) for interpretation of the curve by an application:

- A commonly used curve is a PriceCurve, where the curve is defined by

‘CurveData’ elements where the ‘xvalue’ is used for MW and the ‘y1value’ is used for $/MWh. Not all curve styles are allowed for each product type. The interpretation of curveStyle is as follows:

  - FIXED: only one point is provided, where the intent is that the bid or offer is for all or nothing (i.e. either ‘xvalue’ or 0)

  - VARIABLE: only one point is provided, but the intent is that any value in the range 0 to ‘Xvalue’ can be taken

  - CURVE: Up to 10 points (to define up to 9 segments) can be provided

- Curves are often used where values are NOT plotted against time (e.g. MW vs. $/MWh)

- For PriceCurves for ancillary services, ASCurveData elements are used (instead of CurveData elements) to define points for REGUP, REGDN, RRS and NSRS values.

The following is an example of a PriceCurve, that is derived from a CIM Curve:

<PriceCurve>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>60</xvalue>

<y1value>40.00</y1value>

</CurveData>

<CurveData>

<xvalue>80</xvalue>

<y1value>45.00</y1value>

</CurveData>

<CurveData>

<xvalue>90</xvalue>

<y1value>47.00</y1value>

</CurveData>

</PriceCurve>

In order to better meet the needs of ERCOT Market Participants for interfaces related to bidding, an alternative to an IrregularIntervalSchedule is defined. This is called a ‘TmSchedule’. A TmSchedule has one or more ‘TmPoints’, where one or more values can be specified at each point in time. This is illustrated by the following diagram:

- Figure 8 - Time Schedule Structure

The following is an XML example of an EnergySchedule, which is based upon a TmSchedule:

<EnergySchedule>

- <startTime>2007-10-17T00:00:00-05:00</startTime>

<endTime>2007-10-18T00:00:00-06:00</endTime>

- <TmPoint>

<time>2007-10-17T00:00:00-05:00</time>

- <value1>120</value1>

</TmPoint>

<TmPoint>

<time>2007-10-17T10:00:00-06:00</time>

<value1>130</value1>

</TmPoint>

<TmPoint>

<time>2007-10-17T16:00:00-06:00</time>

<value1>115</value1>

- </TmPoint>

</EnergySchedule>

It is important to note that a TmPoint identifies the start of an interval. The following diagram provides a graphic depiction of the XML example.

The optional ‘ending’ tag can be used to identify the end time for a specific TmPoint. This allows a schedule to have gaps. This is demonstrated by the following example.

<EnergySchedule>

- <startTime>2007-10-17T00:00:00-05:00</startTime>

<endTime>2007-10-18T00:00:00-06:00</endTime>

- <TmPoint>

<time>2007-10-17T00:00:00-05:00</time>

- <ending>2007-10-17T05:00:00-05:00</ending>

<value1>120</value1>

</TmPoint>

<TmPoint>

<time>2007-10-17T10:00:00-06:00</time>

<value1>130</value1>

</TmPoint>

<TmPoint>

<time>2007-10-17T16:00:00-06:00</time>

<value1>115</value1>

- </TmPoint>

</EnergySchedule>

- The following diagram provides a graphic depiction of the previous XML example.

It is important to note a few conditions that may be treated as errors by applications using the data represented in a TmSchedule:

- TmPoints should be provided in chronological order

- TmPoints that result in a zero width time interval may be treated as errors

- The endTime should be less than the startTime of the TmSchedule

- For any TmPoint, if specified, the ending must be greater than the time value

- Each TmPoint must exist within the startTime and endTime of the TmSchedule

Also, the use of ‘ending’ is optional. If no ‘ending’ is supplied, the last TmPoint is assumed to and at the ‘endTime’ for the TmSchedule.

### Representation of Time

The ISO 8601 standard is used to define the representations of time values that are conveyed through interfaces. This avoids issues related to time zones and daylight savings time changes.

- For timestamps in messages published by ERCOT would use Texas prevailing time, using the following format: 2007-03-27T14:00:00-05:00 (as time changes from CDT to CST, the -05:00 would change to -06:00)

- Timestamps in messages sent to ERCOT by market participants could use any ISO 8601 compliant timestamp (with the exception of 24:00, as described in section 1.6)

It is extremely important to note that the use of ISO 8601 timestamps within message definitions for the external interfaces defined by this document in no way constrains other representations of time that may include:

- User interfaces, where local time or market hours may be used as desired

- Reports, where reports would be generated using an appropriate local time

- Internal integration, where an application may internally require some other time structure

In some cases it may be desirable to convert between a timestamp and a local time or a market hour. This can be readily accomplished using software functions, XSL and/or XPath expressions. Examples of time conversion using XSL are provided in Appendix F. Most development environments have a wide variety of APIs that can also be used for necessary time conversions.

In cases where there are a sequence of startTime and endTime intervals provided (as in the cases of ResourceStatus or MinimumEnergy structures as examples), overlapping time intervals are not allowed and will be flagged as an error. Typically in these cases the endTime for one interval should identically match the startTime for the next interval.

In cases where a time that is ‘aligned’ with an interval is expected, but not provided on a request message (e.g. 2008-01-01T01:03:23-06:00), the integration layer may round the time to the nearest minute. This may result in an error being generated by the target application if the minute is not a valid interval boundary. In cases where a time that is aligned with an hour boundary is expected, but not provided on a request message, the integration layer may round the time to the nearest hour. In this case, there may be no generation of an error message.

### Market Products

The market transaction interfaces described in this document use a number of different market products, as well as several types of schedules. The following table identifies each product or schedule type, along with the set of message fields (aside from the market participant ID of the submitting QSE, operating date and operating hour) that are used to distinctly identify a bid, trade, offer or schedule. In effect the primary key for each product type is formed by the QSE ID (short name), operating date, operating hour and items described in the table of Figure 9. The table of Figure 10 provides a mapping of the XML tag names for each product to the approved protocol terms.

| Product or Schedule Type (XML tag) | resource | As Type | sp | bid Id | Crr Id | Offer Id | Crr AH Id | source | sink | buyer | seller | INC/ DEC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASOffer | X | X |  |  |  |  |  |  |  |  |  |  |
| ASTrade |  | X |  |  |  |  |  |  |  | X | X |  |
| Capacity Trade |  |  |  |  |  |  |  |  |  | X | X |  |
| COP | X |  |  |  |  |  |  |  |  |  |  |  |
| CRR |  |  |  |  | X | X | X | X | X |  |  |  |
| DCTieSchedule |  |  | X |  |  |  |  |  |  | X | X |  |
| EnergyBid |  |  | X | X |  |  |  |  |  |  |  |  |
| Energy Only Offer |  |  | X | X |  |  |  |  |  |  |  |  |
| EnergyTrade |  |  | X |  |  |  |  |  |  | X | X |  |
| IncDecOffer | X |  |  |  |  |  |  |  |  |  |  | X |
| OutputSchedule | X |  |  |  |  |  |  |  |  |  |  |  |
| PTPObligation |  |  |  | X |  |  |  | X | X |  |  |  |
| SelfArrangedAS |  | X |  |  |  |  |  |  |  |  |  |  |
| SelfSchedule |  |  |  |  |  |  |  | X | X |  |  |  |
| ThreePartOffer | X |  |  |  |  |  |  |  |  |  |  |  |

- Figure 9 - Required Keys for Market Products

The items marked with an ‘X’ are required key fields needed for submission. The operating date for each bid, trade, offer or schedule is derived from the startTime provided by the bid, offer, trade or schedule.

The XML schema provided to describe product types has all fields optional. This is because the schemas can be used for create, get and cancel operations. A bid, offer, trade or schedule can be referenced for a ‘get’ or ‘cancel’ request by specifying the transaction ID (mRID) of the desired bids, offers, trades or schedules (using the mRID that was returned by the reply to the submission request).

In each section related to market products, a table is provided that describes data requirements. Fields that are used as ‘keys’ to identify a bid/offer/trade/schedule are identified by ‘K’. Required fields are identified by ‘Y’, option fields are identified by ‘N’. There are some exception cases where identified key fields are identified as not being required (e.g. expiration), and are notated as ‘K, N’.

### Management and Use of Transaction IDs (mRIDs)

In order to uniquely identify a bid, offer or trade, a transaction ID is allocated by ERCOT. The name of the tag used for the transaction ID is ‘mRID’, as this conforms with the IEC CIM. This transaction ID is returned immediately in response to the submission of a BidSet, where each bid, offer or trade within the BidSet is given its own transaction ID. If a set of N valid unique bids are submitted using a BidSet, there will be N transaction IDs supplied, each corresponding to the order of the bid/offer/trade/schedule on the input BidSet. This transaction ID (mRID) may be used for the following purposes:

- To query the status of a specific transaction

- To cancel a specific transaction

- To relate an award to a bid

It is important to note that an update or change to a transaction does not use an mRID. To update a transaction it is simply resubmitted, where it will overwrite the previous submission of the bid. The resulting mRID for both the original submission and the revised submission will be the same.

A transaction ID (mRID) is a string that takes the following form:

<QSE ID>.<date>.<key string>

Within this, the following substrings are defined:

- QSE ID: ID of the market participant

- Date in format YYYYMMDD

- Key string, a derived string that includes required key fields for each market product

Examples of transaction ID (mRID) are as follows:

- SOMEQSE.20081112.COP.UnitABC

QSE2.20081112.TPO.UnitXYZ

When referring to a specific hour of a bid, the hour may be appended to the transaction ID, resulting in a string of the following form:

<QSE ID>.<date>.<key string>.<hour>

Examples of transaction ID as would be used for hourly awards are as follows:

- QSE2.20081112.TPO.UnitXYZ.1

QSE2.20081112.TPO.UnitXYZ.2

QSE2.20081112.TPO.UnitXYZ.2R   (example for change to CST from CDT)

QSE2.20081112.TPO.UnitXYZ.3

QSE2.20081112.TPO.UnitXYZ.4

On a day with 25 hours, the mRID the represents the ‘repeated’ hour is suffixed with an ‘R’, e.g. ‘QSE2.20081112.TPO.UnitXYZ.2R’.

Within the definitions of bids, trades, offers and awards, the transaction ID is conveyed using the ‘mRID’ property. This is for consistency with the IEC CIM. The following table describes the initial scheme to be used for each product type.

| Product or Schedule Type (Protocol term) | Section Reference | Product or Schedule Type (XML tag) | Key String Composition (all are prefixed with QSE ID and date to form mRID) |
| --- | --- | --- | --- |
| Ancillary Service Offer | 3.3.4 | ASOffer | ASO.resource.asType |
| Ancillary Service Trade | 3.3.5 | ASTrade | AST.asType.buyer.seller |
| Capacity Trade | 3.3.6 | CapacityTrade | CT.buyer.seller |
| Current Operating Plan | 3.3.8 | COP | COP.resource |
| Congestion Revenue Rights Offer | 3.3.7 | CRR | CRR.crrId.offerid.crrAHId.source.sink |
| DC Tie Schedule | 3.3.9 | DCTieSchedule | DCT.sp.buyer.seller |
| DAM Energy Bid | 3.3.10 | EnergyBid | EB.sp.bidId |
| DAM Energy-Only Offer | 3.3.11 | EnergyOnlyOffer | EOO. sp.bidId |
| Energy Trade | 3.3.12 | EnergyTrade | ET.sp.buyer.seller |
| Incremental and Decremental Energy Offer Curves | 3.3.3 | IncDecOffer | IDO.resource.type |
| Output Schedule | 3.3.13 | OutputSchedule | OS.resource |
| PTP Obligation Bid | 3.3.14 | PTPObligation | PTP.bidid.crrAHId.source.sink |
| Self-Arranged Ancillary Service Quantities | 3.3.2 | SelfArrangedAS | SAA.asType |
| Self-Schedule | 3.3.15 | SelfSchedule | SS.source.sink |
| Three Part Supply Offer | 3.3.1 | ThreePartOffer | TPO.resource |
| Outage Schedule | 7.2.1 | OutageSet | ID assigned by Outage Scheduler |

- Figure 10 - mRID Structures

An important consideration has to do with the use of the interfaces described in this document in conjunction with the submission of bids by a user using a web-based user interface. The web-based user interface for bidding has no notion of the transaction ID itself, just the individual values used to construct the key string.

Another important note is that mRID values should not be used for purposes of non-repudiation. It is the intent of the signature of the SOAP message, as referenced in section 2.2.2 to provide mechanisms that can be leveraged for non-repudiation.

Note: The structure of the transaction ID (mRID) is subject to change in the future. The ‘get mRID’ interface described in section 8.2.1 provides a mechanism to generate an mRID given a set of ‘key’ values.

### Other Conventions

The following are other conventions that must be followed by this specification:

- Within XML definitions, tags should be namespace qualified. For example, a tag of <tag> should be prefixed by a specific namespace reference, e.g. <ns:tag>. This will help to eliminate ambiguity. (Note that many examples in this document are not namespace qualified for brevity and to aid legibility)

- Units for power quantities are in megawatts (MW).

- Units for capacity quantities are in megawatts (MW).

- Units for energy quantities are in megawatt-hours (MWh).

- Units for energy prices are in $/MWh.

- Valid market types include: DAM, RTM, DRUC, HRUC, SCED, SASM, CRR and ADJ. Note that these are not currently required by bidding interfaces

- Trading dates are specified using YYYY-MM-DD, which indicates the operating day

- Valid ancillary service types offered as products include:

| ASType | Supported in |
| --- | --- |
| Off-Non-Spin | ASOffer, |
| Reg-Down | ASOffer,SelfArrangedAS,ASTrade |
| REGUP-RRS-ONNS | ASOffer, |
| Reg-Up | SelfArrangedAS,ASTrade |
| RRS | SelfArrangedAS,ASTrade |
| RRSLD | SelfArrangedAS |
| Non-Spin | SelfArrangedAS,ASTrade |

- Settlement point names are the same as the name of the associated electrical bus, if defined at a bus level. They may also be defined at other levels (e.g. hubs, load zones).

- QSE short names (as opposed to DUNS numbers) are used to identify the QSE as well as ‘buyer’ and ‘seller’ for trades.

## Delivery Approach

In advance of a Nodal go live and in accordance with agreed schedule and dependencies, ERCOT will provide the following:

- Interface specifications for web services

- Design artifacts, including XML schemas and WSDLs

- Source code examples for web service clients

- A sand box environment for testing the interactions between the Market Participants and ERCOT. This environment will eventually be used for qualification of QSEs.

The interface specifications, artifacts and implementation of the sand box environment will be staged. An iterative implementation approach will be used, where feedback from each stage will be used to plan subsequent stages.

## Technical Interoperability

There are several strategies that are being employed in order to achieve technical interoperability of MPs with ERCOT. These include:

  - Use of open standards

  - API subgroup allows input from Market Participants on interoperability issues

  - The early deployment of a sandbox environment that enables ERCOT to work with the Market Participants to insure interoperability issues are addressed prior to market trials

  - Providing sample Java and .Net web service client code

  - Deployment of interfaces via the sandbox environment for early testing by Market Participants, so that feedback can be provided to ERCOT

Open standards are a key part of the strategy to achieve technical interoperability. Standards of particular interest include:

- W3C standards

- OASIS WS-* standards

- IEC Common Information Model and related standards (e.g. IEC 61968-1)

It is very important that the implementation of Web Service interfaces not be dependent upon any proprietary, third party products. Another key requirement is that implementation of web service clients must be possible using both Java and .Net development tools.

More details on technical interoperability will be provided as a consequence of existing web services provided by ERCOT, detailed design and experience with the Sand Box environment.

## Service Level Agreements

Different categories of Web Services will have different service level agreements (SLAs). The SLAs for some Web Services are directly impacted by the variability in the amount of data that can be transferred (e.g. large BidSets).

The response time periods specified for each interface covered by an SLA typically will vary to some degree, based upon factors such as network and system loading. Consequentially, each SLA will be stated in a manner such that each SLA will be honored Z% of the time.

The following are example SLA specifications for categories of Web Services. However, more specific SLAs will be identified for specific interfaces as required.

- Web Services for DAM interactions:

- Must be over X% available for a given trading day

- Validation of bid/trade/offer/schedule sets will be processed within M1 minutes after the BidSet is initially submitted or changed

- Inquiries about a specific BidSet will be processed within S1 seconds, with an additional S2 seconds per bid

- Web Services for RTM interactions:

- Must be over X% available for a given trading day

- Validation of BidSets will be processed within M1 minutes after the BidSet is initially submitted or changed

- BidSet inquiries will be processed within S1 seconds, with an additional S2 seconds per bid

Web services for obtaining real-time market information and providing acknowledgments and confirmations:

  - Must be over X% available

  - Requests involving less than 10KB data should be processed within S3 seconds, unless otherwise specified for a specific interface

- Web services for other than real time transactions and information requests:

  - Must be over Y% available

  - Requests involving less than 10KB data should be processed within S4 seconds, unless otherwise specified for a specific interface

Notification interfaces, where the SLA obligations are also on the Market Participant as well as ERCOT:

  - ERCOT will post notifications to a Market Participant within S5 seconds of the time of internal posting

  - Notification service interface provided by Market Participant should be minimally Y% available. Any ‘downtime’ or periods of inaccessibility will directly impact the timeliness of notifications (e.g. validation of BidSets, alerts, etc.) from ERCOT.

Note: These values and additional SLAs will be finalized once the vendor requirements are complete.

An important note is that there are limits to the ability of ERCOT to process all requests in the same order in which they are received. This is a consequence of a number of factors, including:

- The potential that a Market Participant may choose an implementation approach where they may issue multiple requests concurrently

- The fact that the ERCOT software that accepts requests will be multi-threaded

In cases where ordering is important, the Market Participant should issue requests in the desired order from a single client machine and only issue subsequent requests when the response is received from the current request.

## Auditing, Monitoring and Management

ERCOT will perform auditing, monitoring and management for Web Services described by this specification using common services. Internal auditing by ERCOT will be used to track and insure that SLAs are met by ERCOT. All Web Service requests will be logged by ERCOT in order to permit calculations related to SLAs. The signatures supplied on SOAP messages will be recorded with transactions as a means of non-repudiation of each transaction.

It is important to recognize that ERCOT is not responsible for the monitoring and management of Market Participant software and network connectivity. Therefore ERCOT cannot guarantee that notification interfaces provided by Market Participants are accessible as needed for timely delivery of notifications.

## Versioning

It is important to recognize that new versions of interfaces may be provided over time, largely as a consequence of:

- Staging of initial implementation

- New requirements

- Upgrades to vendor products

Wherever possible, interfaces will be evolved through augmentation, where a newer version of an interface is compatible with a previous version of an interface. However, this will not always be possible. New versions of interfaces will be manifested by:

- Changes to WSDLs

- Changes to XML Schemas

- Changes to software implementations

New versions will be deployed within a Sand Box environment for a testing/trial period. WSDL and XML schemas namespaces will include a date reference. Messages will use the Header/Revision field to identify a specific revision - this will enable ERCOT to process multiple versions of an interface where appropriate.

A detailed versioning strategy will be developed and presented for approval to the API Subgroup and TPTF.

## Governance

The web service interfaces will be critical to the operations of both ERCOT and Market Participants. The Web Services will evolve for many reasons, especially as the needs of the market evolve. Governance policies and processes will need to be defined for the Web Service lifecycle that provide strict guidelines related to:

- Design

- Implementation

- Testing

- Deployment

- Management

A comprehensive governance strategy will need to be developed and implemented by ERCOT with input from the API Subgroup and TPTF.

## Web Service Configuration Standards

ERCOT will configure its web servers with specific parameters for use of Web Services by Market Participants (e.g. for security). Market Participants will also need to set up Web Services to handle notifications from ERCOT. ERCOT will define specific configuration details and parameters to be used by Market Participants.

Detailed web service configuration standards will be provided to the API Subgroup and TPTF and will be refined through detailed design and experience with the Nodal Sand Box environment.

# Market Transaction Service ***

The purpose of the MarketTransaction Service is to support interfaces required for market transactions. This section describes the use of web services by Market Participants as required for bidding processes that involve the submission, change and cancellation of bids, trades, offers and schedules for specific markets.

On a given trading day, each Market Participant provides information (using a BidSet) to ERCOT that will be used at the close of the market to determine awards and obligations and to provide schedules needed for market operations. Up until the close of the market, a Market Participant may create, update or cancel bids, trades, offers and schedules. When submitted, ERCOT will validate the submission, reporting format or submission errors to Market Participants using notification messages.

## Interfaces Provided

The interfaces provide the means to create (i.e. submit), get (i.e. query) and cancel (i.e. withdraw) bids/trades/offers/schedules on a given trading date. A single container class ‘BidSet’ is used to hold a set of bids/trades/offers/schedules within the Payload section of the message, where each of the bids, trades, schedules or offers may be of a different type.

The following diagram shows an example message sequence, using the ‘verb’ and ‘noun’ convention. Where this section focuses on the requests made by Market Participant systems to the ERCOT Nodal Web Services, the sequence diagram also includes notification messages sent from ERCOT to Market Participant Notification services (as described in section 5).

- Figure 11 - Example BidSet Sequence Diagram

The message sequence example shown involves the following steps:

- Market participant sends a RequestMessage for ‘create BidSet’ with an initial BidSet to ERCOT for a specific market

- In response to step 1, ERCOT performs a simple syntax scan and typically sends a ResponseMessage with ReplyCode=OK. In the response payload, each bid/offer/trade/schedule will identify a ‘SUBMITTED’ status and an mRID value. (An alternative example could result in a reply of ‘ERRORS’ if the syntax check failed, in which case steps 3 and 4 would not occur). This reply is synchronous.

- ERCOT validates the bids within the BidSet. This could take several minutes. This processing is done asynchronously.

- A notification message (using verb=changed) is sent to the notification interface provided by the Market Participant. The status of the bids within the BidSet will indicate whether the bid/offer/trade/schedule was PENDING/ACCEPTED or had ERRORS. This message will not include the complete BidSet.

- When multiple bids are submitted in a large payload, the Market Participant will asynchronously receive the notification holding the bids status for the "accepted" and "rejected" (in the event of ERRORS) bids along with the rejection reason.  As always, MPs may make a request to get the current bids within the submitted BidSet using a RequestMessage with verb=get. The request may be for individual bids using the mRID values, or for the entire set of bids within the BidSet using the TradeDate (using short mRIDs).  It will return all successful (accepted) bids and not the rejected bids.

- If appropriate, reply is sent for get BidSet in step 5.

- A Market Participant may choose to cancel one or more bids using a RequestMessage with verb=cancel. The mRID is used for cancellation of a bid. (See section 2.3.4)

- In response to step 7, a ResponseMessage is sent with verb=’reply’. This response is synchronous.

- The Market Participant may resubmit some bids (e.g. to correct errors), to change bids (e.g. change prices) and/or submit new bids, using a RequestMessage with verb=change (note that BidSets are a special case where the verbs ‘create’ and ‘change’ can be used interchangeably).

- In response to step 9, a ResponseMessage is sent with verb=’reply’. This response is synchronous.

- The newly submitted bids within the BidSet (i.e. the set of bids consequential to steps 7 and 9) are validated. This may take minutes, and processing is performed asynchronously.

- As in step 4, a notification message is sent to the Market Participant to indicate whether or not the BidSet was validated and accepted (with either an ACCEPTED or PENDING state). The notification message uses verb=changed.

- After the close of the market, awards and obligations are determined.

- Market participants are notified of specific awards and/or obligations. The notification message uses verb=’created’, noun=’AwardSet’.

The following diagram describes the potential state transitions of each bid/offer/trade/schedule within a BidSet:

- Figure 12 - State Diagram for Bids, Trades, Offers and Schedules

## Interfaces Required

The following table describes the parameters used in the request message (RequestMessage) for market transactions, noting that each transaction has a request and a response message. The verbs create, update, get and cancel are respectively used to submit (create or change), query and cancel BidSets.

| Message Element | Value |
| --- | --- |
| Header/Verb | create/get/change/cancel |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | ID of user |
| Payload | BidSet (a single BidSet may be used for create and update request messages, and may optionally be used for get requests to identify specific bids of interest) |

- Figure 13 -  Message: create BidSet

The corresponding response messages (ResponseMessage) would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error message if the ReplyCode is ERROR |
| Reply/Timestamp | The time the submission was received by ERCOT |
| Payload | BidSet, where the status of each bid/offer/trade/schedule within the BidSet may be SUBMITTED, PENDING, ACCEPTED, UNCONFIRMED, REJECTED or ERRORS. Specific errors may be identified for each bid. |

- Figure 14 - Message: reply BidSet

In the cases of payloads that would otherwise exceed 1 megabyte, the payloads should be zipped, base64 encoded and stored within the ‘Payload/Compressed’ tag.

For the purposes of BidSets, the verbs create and update can be used interchangeably. Note that only one BidSet is permitted for a given message, and all transactions within the BidSet must be for the same trading date.

## Message Specifications

A BidSet is the payload type used for the submission, query and cancellation of bids and offers, and serves as a container for the different types of bids, offers, trades and scheduled that are submitted by a QSE for a given trading day. A BidSet identifies the trading date and Market Participant with a sequence of any number of bids, trades, offers and schedules.

The header of a BidSet is shown in the following diagram:

- Figure 15 - BidSet Header Structure

An important note is that the BidSet uses an XSD sequence, requiring each type of bid, trade, offer or schedule to be provided in a specific order. Please reference the XML Schema for the correct order.

The submission (using ’create’ verb and ‘BidSet’ noun in the message header) of a BidSet by a Market Participant will have the effect of either creating new bids (or offers, schedules, trades, etc.) for a given trading date, or overwriting existing bids that were previously submitted for a given BidSet for a given trading day.

Additionally, bids, offers, trades and schedules may be submitted in partial batches, where more than one BidSet is submitted for a given trading day. When this is done, new bids are aggregated with previously submitted bids and updates to previously submitted bids overwrite the previous bid. The following sequence describes the aggregation of BidSets:

- A first BidSet is submitted by a market participant for a trading day, with bids 1, 2, 3, 4.

- A second BidSet is submitted by the market participant for the day with bids 5, 6, 7, 8.

- The market participant ‘get’s the BidSet for the day, the reply returns the details of bids 1-8.

- The market participant submits a cancel for bid 1.

- A third BidSet is submitted by the market participant for the day, where there are updates to bids 5 and 7, as well as a new bid 9

- The market participant ‘get’s the BidSet for the day, the reply returns the details of bids 2-9.

In order to cancel a bid, a BidSet must be sent using the ‘cancel’ verb, with the specific bids to be cancelled identified within the BidSet. Cancel can only be used to cancel specific bids, not a whole BidSet for a given market.

In order to query a BidSet, A BidSet is sent using the ‘get’ verb, where the desired bids for the specific BidSet are identified. There are then two options:

- If no bids are identified within the BidSet, all bids for the particular trading date will be returned in the BidSet in the response message.

- If specific bids are identified within the BidSet, only the details of the specified bids will be returned in the BidSet in the response message.

When a BidSet is returned by a ‘get’ request for a given trade date, the status value of each in the BidSet is populated. Values could include:

  - SUBMITTED (to indicate submission, but no further processing has been completed)

  - ACCEPTED (to indicate successful validation and acceptance of the transaction)

  - UNCONFIRMED (to indicate a trade that is not yet confirmed, once confirmed it would go to the ACCEPTED state)

  - PENDING (to indicate a transaction that is not yet accepted, as it may be submitted for a day after the next trading date or the second phase of validation within MMS has not yet completed)

  - REJECTED (to indicate a transaction that has been rejected)

  - CANCELED (to indicate a transaction was canceled, this can not be retrieved by a get)

  - ERRORS (to indicate that there are one or more errors for the transaction)

The following sub sections describe the structure of specific bid, offer, trade and schedule types. Typically each bid/offer/trade/schedule will have a set of properties that along with the type of bid/offer/trade/schedule makes it unique (see section 2.3.3). Typically these would include:

- Bid, offer, schedule or trade type

- Resource

- Ancillary Service type

When submitting a bid/offer/schedule/trade using create or update, all properties for the bid must be specified. When performing a get or cancel request, only those parameters that uniquely identify the bid/offer/trade/schedule must be specified through the use of an mRID. The following diagram shows information commonly maintained for each type of bid/offer/trade/schedule. The status for a given bid/offer/trade/schedule may be SUBMITTED, PENDING, ACCEPTED, ERRORS or CANCELED. If the status is ERRORS, there may be one or more error stings identified.

- Figure 16 – Bid/Offer/Trade/Schedule Common Elements

The mRID is not supplied for the initial submission of a bid, but must be supplied for cancellations to previously submitted bids. The marketType is currently optional and is reserved for future use. When an mRID is specified for a ‘get’ or ‘cancel’ request, it is supplied using a message request/ID tag.

The ‘externalId’ may be populated by the QSE with an identifier of their choice. If supplied upon submission, the identifier will then be used in conjunction with notifications of acceptance or rejection due to errors.

### Three-Part Supply Offer

The following diagram defines the structure of a Three-Part Supply Offer that could be included within a BidSet, using the ThreePartOffer tag. This is one of the more complex structures as it involves startup costs, minimum generation costs and price curves. The PriceCurve is defined by CIM directly as a subclass of CurveSchedule.

- Figure 17 – ThreePartOffer Structure

The error tag is used to return one or more errors that may be the consequence of the failure of business or syntax validation rules for each type of bid/offer/trade/schedule. The EnergyOfferCurve element is based upon a PriceCurve type. Within PriceCurve, the values of ‘xvalue’ are used to identify the quantity in MW, and the values of ‘y1value’ are used to define the price in $/MWh. The ‘curveStyle’ serves as the ‘indicator’, to identify whether the data is to be treated as FIXED, VARIABLE or a CURVE. The number of points in each PriceCurve is limited to 10. Where more than one PriceCurve is supplied for a bid/offer/trade, the start and end times must not overlap. The incExcFlag is to indicate whether the capacity of the ThreePartOffer is inclusive of the capacity bid in the AS market. The reason code is typically used for adjustments. The structure of a PriceCurve type is detailed in the following diagram:

- Figure 18 - PriceCurve Structure

The following diagrams describe the structures used for FIP/FOP, startup costs and minimum generation costs within the ThreePartOffer message. Note that if more than one StartupCost or MinimumEnergy is provided, there must not be overlapping start and end times. That is to say that more than one StartupCost or more than one MinimumEnergy can be specified within the day, provided that the time intervals for each instance are distinct.

- Figure 19 - Fip/Fop, StartupCost and MinimumEnergy Structures

On submission, the following items are required for a ThreePartOffer. Fields that are used as ‘keys’ to identify a bid/offer/trade/schedule (see section 2.3.3) are identified by ‘K’. Required fields are identified by ‘Y’, option fields are identified by ‘N’. There are some exception cases where identified key fields are identified as not being required (e.g. expirationTime), and are notated as ‘K, N’.

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | N | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | N | datetime | End time for bid | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| resource | K | string | Resource | Valid resource name |
| combinedCycle | N | string | Combined cycle to which resource is associated | NULL or Valid plant name if unit is part of a combined cycle |
| expirationTime | Y | datetime | Time of offer expiration | Valid time within trade date adjustment period |
| FipFop/startTime | Y | datetime | Start time | Valid hour boundary within the trade date |
| FipFop/endTime | Y | datetime | End time | Valid hour boundary within the trade date |
| FipFop/fipPercent | Y | decimal | Fuel index price percent | >= 0, <= 100 |
| FipFop/fopPercent | Y | decimal | Fuel oil price percent | >= 0, <= 100 |
| StartupCost/startTime | N | datetime | Start time for startup costs | Valid hour boundary within the trade date |
| StartupCost/endTime | N | datetime | End time for startup costs | Valid hour boundary within the trade date |
| StartupCost/hot | N | decimal | Hot startup cost | >= 0 |
| StartupCost/intermediate | N | decimal | Intermediate startup cost | >= 0 |
| StartupCost/cold | N | decimal | Cold startup cost | >= 0 |
| MinimumEnergy/startTime | N | datetime | Start time for minimum energy costs | Valid hour boundary within the trade date |
| MinimumEnergy/endTime | N | datetime | End time for energy costs | Valid hour boundary within the trade date |
| MinimumEnergy/cost | N | decimal | Minimum energy cost | >= 0 |
| EnergyOfferCurve/startTime | N | datetime | Start time for curve | Valid hour boundary within the trade date |
| EnergyOfferCurve/endTime | N | datetime | End time for curve | Valid hour boundary within the trade date |
| EnergyOfferCurve/curveStyle | N | string | Used as an indicator to describe the type of ‘curve’ | CURVE |
| EnergyOfferCurve/CurveData/xvalue | N | float | Megawatts | Quantity in MW |
| EnergyOfferCurve/CurveData/y1value | N | float | $/MWh | Price in $/MWh |
| EnergyOfferCurve/incExcFlag | N | string | Identifies whether or not an AS offer is linked to this offer. | INC or EXC. Default should be INC, where it is linked to an ASOffer |
| EnergyOfferCurve/reason | N | string | Reason for adjustment | Text description |

Figure 20 - ThreePartOffer Requirements

There should minimally be a StartupCost, MinimumEnergy or PriceCurve specified for a Three Part Offer. The following is an XML example of a Three Part Offer:

<ThreePartOffer>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

- <externalId>MyExternalID12345</externalId>

<expirationTime>2007-12-31T04:00:00-06:00</expirationTime>

<resource>AcmeUnit1</resource>

<combinedCycle>AcmeCC</combinedCycle>

<FipFop>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<fipPercent>50</fipPercent>

<fopPercent>50</fopPercent>

</FipFop>

<StartupCost>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<hot>333.33</hot>

<intermediate>444.44</intermediate>

<cold>666.66</cold>

</StartupCost>

<MinimumEnergy>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<cost>1001.11</cost>

- </MinimumEnergy>

<EnergyOfferCurve>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>60</xvalue>

<y1value>40.00</y1value>

</CurveData>

<CurveData>

<xvalue>80</xvalue>

<y1value>45.00</y1value>

</CurveData>

<CurveData>

<xvalue>90</xvalue>

<y1value>47.00</y1value>

</CurveData>

<incExcFlag>INC</incExcFlag>

</EnergyOfferCurve>

<EnergyOfferCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>60</xvalue>

<y1value>42.00</y1value>

</CurveData>

<CurveData>

<xvalue>80</xvalue>

<y1value>55.00</y1value>

</CurveData>

<CurveData>

<xvalue>90</xvalue>

<y1value>57.00</y1value>

</CurveData>

<incExcFlag>INC</incExcFlag>

</EnergyOfferCurve>

</ThreePartOffer>

### Self-Arranged Ancillary Service Quantities

Self-Arranged Ancillary Service Quantities describe a schedule of ancillary services that a QSE has arranged. The following diagram describes the structure of a schedule for self arranged ancillary services.

- Figure 21 - SelfArrangedAS Structure

Within the SelfArrangedAS is a CapacitySchedule. CapacitySchedules are defined using TmSchedules types, as shown in the following diagram. The values of ‘value1’ within the CapacitySchedule are used to identify megawatts.

- Figure 22 - CapacitySchedule Structure

On submission, the following items are required for a SelfArrangedAS:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| asType | K | string | Ancillary service type | Reg-Down, Reg-Up, RRS, RRSLD, Non-Spin |
| CapacitySchedule/startTime | N | datetime | not used | not used |
| CapacitySchedule/endTime | N | datetime | not used | not used |
| CapacitySchedule/ TmPoint/time | Y | datetime | Absolute time for start of interval | Valid time within trading date |
| CapacitySchedule/ TmPoint/ending | N | datetime | Absolute time for end of interval | Valid time within trading date |
| CapacitySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 23 - SelfArrangedAS Requirements

The following XML is an example of a Self-Arranged Ancillary Service Quantities, showing the use of a CapacitySchedule:

<SelfArrangedAS>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID12346</externalId>

<asType>NON-SPIN</asType>

<CapacitySchedule>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>30</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>39</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T04:00:00-06:00</time>

<value1>30</value1>

</TmPoint>

</CapacitySchedule>

</SelfArrangedAS>

### Incremental and Decremental Energy Offer Curves ***

The following diagram describes the structure of  Incremental and Decremental Energy Offer Curves. This shows the details of the FipFop and PriceCurve structures that are contained within.

- Figure 24 - IncDecOffer Structure

Both the IncrementalCurve and DecrementalCurve use the PriceCurve structure as described previously in this document, but only one (either an IncrementalCurve or a DecrementalCurve) can be specified per each IncDecOffer. On submission, the following items are required for an IncDecOffer:

| Element |  | Req? |  | Datatype |  | Description | Values |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| startTime |  | N |  | datetime |  | Start time for bid | Valid start hour boundary for trade date |  |
| endTime |  | N |  | datetime |  | End time for bid | Valid end hour boundary for trade date |  |
| externalId | N |  | string |  | External ID |  |  | QSE supplied |
| resource |  | K |  | string |  | Resource | Valid resource name |  |
| combinedCycle |  | N |  | string |  | Combined cycle to which resource is associated | Valid plant name if unit is part of a combined cycle |  |
| expirationTime |  | N |  | datetime |  | Offer expiration time | Valid time before or during the trade date |  |
| FipFop/fipPercent |  | N |  | decimal |  | Fuel index price percent | >= 0, <= 100 |  |
| FipFop/fopPercent |  | N |  | decimal |  | Fuel oil price percent | >= 0, <= 100 |  |
| IncrementalCurve/startTime |  | If INC offer |  | datetime |  | Start time for curve | Valid hour boundary |  |
| IncrementalCurve/endTime |  | If INC offer |  | datetime |  | End time for curve | Valid hour boundary |  |
| IncrementalCurve/curveStyle |  | If INC offer |  | string |  | Type of curve | CURVE |  |
| IncrementalCurve/CurveData/xvalue |  | If INC offer |  | float |  | Megawatts | Quantity in MW |  |
| IncrementalCurve/CurveData/y1value |  | If INC offer |  | float |  | $/MWh | Incremental price in $/MWh, must be greater than corresponding Decremental curve value |  |
| DecrementalCurve/startTime |  | If DEC offer |  | datetime |  | Start time for curve | Valid hour boundary |  |
| DecrementalCurve/endTime |  | If DEC offer |  | datetime |  | End time for curve | Valid hour boundary |  |
| DecrementalCurve/curveStyle |  | If DEC offer |  | string |  | Type of curve | CURVE |  |
| DecrementalCurve/CurveData/xvalue |  | If DEC offer |  | float |  | Megawatts | Quantity in MW |  |
| DecrementalCurve/CurveData/y1value |  | If DEC offer |  | float |  | $/MWh | Decremental price in $/MWh, must be less than corresponding incremental curve value |  |

Figure 25 - IncDecOffer Requirements

The following is an XML example for an IncDecOffer, where an incremental offer is provided using an IncrementalCurve:

<IncDecOffer>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

- <externalId>MyExternalID12347</externalId>

<expirationTime>2007-12-31T23:00:00-06:00</expirationTime>

<resource>ACMEUnit2</resource>

<combinedCycle>ACMECC</combinedCycle>

<FipFop>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<fipPercent>20</fipPercent>

<fopPercent>70</fopPercent>

</FipFop>

<IncrementalCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>35.50</y1value>

</CurveData>

<CurveData>

<xvalue>40</xvalue>

<y1value>45.50</y1value>

</CurveData>

<CurveData>

<xvalue>60</xvalue>

<y1value>55.50</y1value>

</CurveData>

</IncrementalCurve>

<IncrementalCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>37.50</y1value>

</CurveData>

<CurveData>

<xvalue>40</xvalue>

<y1value>47.50</y1value>

</CurveData>

<CurveData>

<xvalue>60</xvalue>

<y1value>57.50</y1value>

</CurveData>

</IncrementalCurve>

</IncDecOffer>

The following is an example of an decremental offer, using the DecrementalCurve:

<IncDecOffer>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

- <externalId>MyExternalID12348</externalId>

<expirationTime>2007-12-31T23:00:00-06:00</expirationTime>

<resource>ACMEUnit2</resource>

<combinedCycle>ACMECC</combinedCycle>

<FipFop>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<fipPercent>20</fipPercent>

<fopPercent>70</fopPercent>

</FipFop>

<DecrementalCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>25.50</y1value>

</CurveData>

<CurveData>

<xvalue>40</xvalue>

<y1value>25.50</y1value>

</CurveData>

<CurveData>

<xvalue>60</xvalue>

<y1value>25.50</y1value>

</CurveData>

</DecrementalCurve>

<DecrementalCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>CURVE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>27.50</y1value>

</CurveData>

<CurveData>

<xvalue>40</xvalue>

<y1value>27.50</y1value>

</CurveData>

<CurveData>

<xvalue>60</xvalue>

<y1value>27.50</y1value>

</CurveData>

</DecrementalCurve>

</IncDecOffer>

### Ancillary Service Offer

An Ancillary Service Offer is used to offer ancillary services from a specified resource. The following diagram describes the structure of an Ancillary Service Offer (ASOffer):

- Figure 26 - ASOffer Structure

The ASPriceCurve structure when used for ASOffers uses either OnLineReserves (to provide price points for Reg-Up, RRS and/or Online Non-Spin), RegDown elements (to provide price points for Reg-Down) or OffLineNonSpin elements ( for Offline Non-Spin). The choice depends upon the value of asType, where the values provided can be one of REGUP-RRS-ONNS, Reg-Down or Off-Non-Spin. Up to five points can be specified for OnLineReserves, but only one point can be specified for RegDown or OffLineNonSpin. This structure is described in the following figure:

The following diagram provides details of the OnLineReserves, RegDown and OffLineNonSpin structures.

- Figure 27 - Price Curves using ASPriceCurve

On submission, the following items are required for an ASOffer:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| resource | K | string | Resource | Valid resource name |
| asType | K | string | AS key code | REGUP-RRS-ONNS, Reg-Down or Off-Non-Spin |
| combinedCycle | N | string | Combined cycle | Valid combined cycle ID |
| expirationTime | Y | datetime | Time of offer expiration | Valid time before trade date |
| PriceCurve/startTime | Y | datetime | Start time for curve | Valid hour boundary |
| PriceCurve/endTime | Y | datetime | End time for curve | Valid hour boundary |
| ASPriceCurve/OnLineReserves/xvalue | If asType = REGUP-RRS-ONNS | float | Megawatts | Quantity in MW |
| ASPriceCurve/ OnLineReserves /REGUP | N | float | $/MWh | Price in $/MWh |
| ASPriceCurve/ OnLineReserves /RRS | N | float | $/MWh | Price in $/MWh |
| ASPriceCurve/ OnLineReserves /ONNS | N | float | $/MWh | Price in $/MWh |
| ASPriceCurve/ OnLineReserves /block | If asType = REGUP-RRS-ONNS | string | Used as an indicator to describe the type of block | FIXED or VARIABLE |
| ASPriceCurve/RegDown/xvalue or ASPriceCurve/OffLineNonSpin/xvalue | If asType = Reg-Down or Off-Non-Spin | float | Megawatts | Quantity in MW |
| ASPriceCurve/RegDown/REGDN | N | float | $/MWh | Price in $/MWh |
| ASPriceCurve/OffLineNonSpin/OFFNS | N | float | $/MWh | Price in $/MWh |
| ASPriceCurve/RegDown/block or ASPriceCurve/OffLineNonSpin/block | If asType = Reg-Down or Off-Non-Spin | string | Used as an indicator to describe the type of block | FIXED or VARIABLE |
| ASPriceCurve/multiHourBlock | N | boolean | Indicates if offer must be taken as a block for all hours. | Default is false |

Figure 28 - ASOffer Requirements

The following is an XML example for an ASOffer, for the AS types of Reg-Up, RRS and online Non-Spin:

<ASOffer>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

- <externalId>MyExternalID12343</externalId>

<expirationTime>2007-12-31T22:00:00-06:00</expirationTime>

<resource>AcmeUnit1</resource>

<asType>REGUP-RRS-ONNS</asType>

<ASPriceCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T03:00:00-06:00</endTime>

<OnLineReserves>

<xvalue>60</xvalue>

<REGUP>20.00</REGUP>

<RRS>20.00</RRS>

<ONNS>20.00</ONNS>

<block>FIXED</block>

</OnLineReserves>

<multiHourBlock>false</multiHourBlock>

</ASPriceCurve>

<ASPriceCurve>

<startTime>2008-01-01T03:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<OnLineReserves>

<xvalue>80</xvalue>

<REGUP>23.00</REGUP>

<RRS>20.00</RRS>

<ONNS>20.00</ONNS>

<block>FIXED</block>

</OnLineReserves>

<multiHourBlock>false</multiHourBlock>

</ASPriceCurve>

</ASOffer>

<ASOffer>

- <startTime>2007-07-25T00:00:00-06:00</startTime>

<endTime>2007-07-25T00:00:00-06:00</endTime>

<mRID></mRID>

<externalId>String</externalId>

<marketType>RTM</marketType>

<status>SUBMITTED</status>

<expirationTime>2007-07-25T00:00:00-06:00</expirationTime>

<resource>String</resource>

<asType>Reg-Up</asType>

<combinedCycle>String</combinedCycle>

<ASPriceCurve>

<startTime>2007-07-25T00:00:00-06:00</startTime>

<endTime>2007-07-25T00:00:00-06:00</endTime>

<OnLineReserves>

<xvalue>80</xvalue>

<REGUP>3.5</REGUP>

<RRS>20</RRS>

<ONNS>15</ONNS>

<block>FIXED</block>

</OnLineReserves>

<multiHourBlock>false</multiHourBlock>

</ASPriceCurve>

</ASOffer>

The following is another XML example for an ASOffer, where the AS type is only Reg-Down:

<ASOffer>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

- <externalId>MyExternalID12345</externalId>

<expirationTime>2007-12-31T22:00:00-06:00</expirationTime>

<resource>AcmeUnit1</resource>

<asType>Reg-Down</asType>

<ASPriceCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T03:00:00-06:00</endTime>

<RegDown>

<xvalue>60</xvalue>

<REGDN>20.00</REGDN>

<block>FIXED</block>

</RegDown>

<multiHourBlock>false</multiHourBlock>

</ASPriceCurve>

<ASPriceCurve>

<startTime>2008-01-01T03:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<RegDown>

<xvalue>80</xvalue>

<REGDN>23.00</REGDN>

<block>FIXED</block>

</RegDown>

<multiHourBlock>false</multiHourBlock>

</ASPriceCurve>

</ASOffer>

The following is another XML example for an ASOffer, where the AS type is Off-Non-Spin:

<ASOffer>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

- <externalId>MyExternalID12341</externalId>

<expirationTime>2007-12-31T22:00:00-06:00</expirationTime>

<resource>AcmeUnit1</resource>

<asType>Off-Non-Spin</asType>

<ASPriceCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T03:00:00-06:00</endTime>

<OffLineNonSpin>

<xvalue>60</xvalue>

<OFFNS>20.00</OFFNS>

<block>VARIABLE</block>

</ OffLineNonSpin >

- <multiHourBlock>true</multiHourBlock>

</ASPriceCurve>

<ASPriceCurve>

<startTime>2008-01-01T03:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

< OffLineNonSpin >

<xvalue>80</xvalue>

<OFFNS>23.00</OFFNS>

<block>VARIABLE</block>

</ OffLineNonSpin >

- <multiHourBlock>true</multiHourBlock>

</ASPriceCurve>

</ASOffer>

### Ancillary Service Trade

An Ancillary Service Trade is used to describe a scheduled trade of ancillary services between a buyer and a seller. The following diagram describes the structure of an Ancillary Service Trade (ASTrade).

- Figure 29 - ASTrade Structure

The ASSchedule structure is based upon a TmSchedule as described previously in this document. The submission of an ASTrade (or generally any trade) requires a matching trade to be submitted by the counter party before the close of the market. On submission, the following items are required for an ASTrade:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| asType | K | string | Ancillary service type | Reg-Down, Reg-Up, RRS, Non-Spin |
| buyer | K | string | Trade buyer | Valid QSE |
| seller | K | string | Trade seller | Valid QSE |
| ASSchedule/startTime | N | datetime | not used | not used |
| ASSchedule/endTime | N | dateTime | not used | not used |
| ASSchedule/ TmPoint/time | Y | datetime | Absolute time for beginning of interval | Valid time within the trading date |
| ASSchedule/ TmPoint/ending | N | datetime | Absolute time for end of interval | Valid time within the trading date |
| ASSchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 30 - ASTrade Requirements

The following is an XML example for an ASTrade:

<ASTrade>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID12340</externalId>

<buyer>Acme</buyer>

<seller>Cogswell</seller>

<asType>Reg-Up</asType>

<ASSchedule>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>70</value1>

</TmPoint>

</ASSchedule>

</ASTrade>

### Capacity Trade

The following diagram describes the structure of a Capacity Trade:

- Figure 31 - CapacityTrade Structure

The CapacitySchedule structure is identical to the structure described previously in this document. The submission of a trade requires a matching trade to be submitted by the counter party. The following table describes the required fields:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| buyer | K | string | Trade buyer | Valid QSE |
| seller | K | string | Trade seller | Valid QSE |
| CapacitySchedule/startTime | N | datetime | not used | not used |
| CapacitySchedule/endTime | N | dateTime | not used | not used |
| CapacitySchedule/ TmPoint/time | Y | datetime | Absolute time for beginning of interval | Valid time within the trading date |
| CapacitySchedule/ TmPoint/ending | N | datetime | Absolute time for end of interval | Valid time within the trading date |
| CapacitySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 32 - CapacityTrade Requirements

The following is an XML example for a CapacityTrade:

<CapacityTrade>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID12335</externalId>

<buyer>ACME</buyer>

<seller>XYZ</seller>

<CapacitySchedule>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>70</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>90</value1>

</TmPoint>

</CapacitySchedule>

</CapacityTrade>

### Congestion Revenue Rights (CRR) Offer

The following diagram describes the structure of a Congestion Revenue Rights (CRR) Offer:

- Figure 33 - CRR Structure

The following structure is used for minimum reservation prices. Note than if more than on MinimumReservationPrice is provided, the start and end times must not overlap.

- Figure 34 - MinimumReservationPrice Structure

The CapacitySchedule structure is identical to the structure described previously in this document. On submission, the following table describes items are required for a CRR. In addition, the PeakLoadForecast must be specified for NOIEs.

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for offer | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for offer | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| crrId | K | string | CRR ID | ID of CRR |
| offerId | K | string | Offer ID | QSE supplied |
| crrAccountHolderId | K | string | CRR account holder ID | ID of CRR account holder |
| source | K | string | Source settlement point | Valid settlement point name |
| sink | K | string | Sink settlement point | Valid settlement point name |
| CapacitySchedule/startTime | N | datetime | not used | not used |
| CapacitySchedule/endTime | N | dateTime | not used | not used |
| CapacitySchedule/ TmPoint/time | Y | datetime | Absolute start time for interval | Valid time in the trading date |
| CapacitySchedule/ TmPoint/ending | N | datetime | Absolute end time for interval | Valid time in the trading date |
| CapacitySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |
| MnimumReservationPrice/ startTime | Y | datetime | Start time for minimum reservation price | Valid hour boundary |
| MnimumReservationPrice/ endTime | N | datetime | End time for minimum reservation price | Valid hour boundary |
| MnimumReservationPrice/ price | N | decimal | Minimum price in $/MWh | Default (if not specified) is $2000 |
| NOIEPeakLoadForecast | Y | float | Start time for peak load forecast | Peak load forecast in MW |

Figure 35 - CRR Requirements

The following is an XML example for a CRR:

<CRR>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID11345</externalId>

<crrId>345</crrId>

<source>SP3536</source>

<sink>SP3333</sink>

<CapacitySchedule>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>50</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>60</value1>

</TmPoint>

</CapacitySchedule>

<MinimumReservationPrice>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<price>33.00</price>

- </MinimumReservationPrice>

<NOIEPeakLoadForecast>1700</NOIEPeakLoadForecast>

</CRR>

### Current Operating Plan (COP)

The following diagram describes the structure of a Current Operating Plan (COP), where the COP provides a schedule for a resource:

- Figure 36 - COP Structure

The structures for Limits and ASCapacity are shown in the following diagrams. If more than one ResourceStatus, Limits or ASCapacity block is provided, start and end times must not overlap.

- Figure 37 - Limits Structure

- Figure 38 - ASCapacity Structure

COP submissions are never cancelled, as a COP must be submitted. A resubmission of a COP overwrites the previous submission. On submission, the following items are required for a COP:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | N | datetime | Start time for COP | Valid start hour boundary for trade date |
| endTime | N | datetime | End time for COP | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| resource | K | string | Resource | Valid resource name |
| combinedCycle | N | string | Combined cycle | Valid combined cycle ID |
| ResourceStatus/startTime | Y | datetime | Start time for status | Valid hour boundary |
| ResourceStatus /endTime | Y | datetime | End time for status | Valid hour boundary |
| ResourceStatus /operatingMode | Y | string | status | Valid status code |
| Limits/startTime | Y | datetime | Start time for limits | Valid hour boundary |
| Limits/endTime | Y | datetime | End time for limits | Valid hour boundary |
| Limits/hsl | Y | float | High sustained limit in MW | >=0 |
| Limits/lsl | Y | float | Low sustained limit in MW | >=0 |
| Limits/hel | Y | float | High emergency limit in MW | >=0 |
| Limits/lel | Y | float | Low sustained limit in MW | >=0 |
| ASCapacity/startTime | Y | datetime | Start time for AS capacity schedule | Valid hour boundary |
| ASCapacity/endTime | Y | dateTime | End time for AS capacity schedule | Valid hour boundary |
| ASCapacity/regUp | Y | float | Regulation up MW | >= 0 |
| ASCapacity/regDown | Y | float | Regulation down MW | >= 0 |
| ASCapacity/rrs | Y | float | Responsive reserve MW | >= 0 |
| ASCapacity/nonSpin | Y | float | Non-spinning MW | >= 0 |

Figure 39 - COP Requirements

The following is an XML example for a COP:

<COP>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID12345</externalId>

<resource>ACMEUnit2</resource>

<ResourceStatus>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06 :00</endTime>

<operatingMode>ON</operatingMode>

</ResourceStatus>

<ResourceStatus>

<startTime>2008-01-01T10:00:00-06 :00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<operatingMode>OFF</operatingMode>

</ResourceStatus>

<Limits>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<hsl>100</hsl>

<lsl>20</lsl>

<hel>110</hel>

<lel>10</lel>

</Limits>

<Limits>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<hsl>100</hsl>

<lsl>20</lsl>

<hel>110</hel>

<lel>10</lel>

</Limits>

<ASCapacity>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<regUp>20</regUp>

<regDown>20</regDown>

<rrs>40</rrs>

<nonSpin>80</nonSpin>

</ASCapacity>

<ASCapacity>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<regUp>0</regUp>

<regDown>0</regDown>

<rrs>0</rrs>

<nonSpin>0</nonSpin>

</ASCapacity>

</COP>

The terms maxEmergency, maximumEconomic, minEmergency and minimumEcononic are used to in order to be consistent with the IEC CIM.

Currently defined resource operatingModes include:

- ONRUC – On-Line and the hour is a RUC-Committed Interval

- ONREG – On-Line Resource with Energy Offer Curve providing Regulation Service

- ON – On-Line Resource with Energy Offer Curve

- ONDSR – On-Line Dynamically Scheduled Resource

- ONOS – On-Line Resource with Output Schedule

- ONOSREG – On-Line Resource with Output Schedule providing Regulation Service

- ONDSRREG – On-Line Dynamically Scheduled Resource providing Regulation Service

- ONTEST – On-Line Test with Output Schedule

- ONEMR – On-Line EMR (available for commitment or dispatch only for ERCOT-declared Emergency Conditions; the QSE may appropriately set LSL and HSL to reflect operating limits)

- ONRR – On-Line as a synchronous condenser (hydro) providing Responsive Reserve but unavailable for dispatch by SCED and available for commitment by RUC

- OUT – Off-Line and unavailable

- OFFNS – Off -Line but reserved for Non-Spin

- OFF – Off-Line but available for commitment by DAM and RUC

- EMR – Available for commitment only for ERCOT-declared Emergency Condition events; the QSE may appropriately set LSL and HSL to reflect operating limits

- ONRGL – Available for dispatch of Regulation Service

- ONRRCLR – Available for dispatch of Responsive Reserve Service as a Controllable Load Resource

- ONRL – Available for dispatch of Responsive Reserve Service or Non-Spin, excluding Controllable Load Resources

- OUTL – Not available

### DC Tie Schedule

The following diagram describes the structure of a DC Tie Schedule:

- Figure 40 - DCTieSchedule Structure

The following diagrams describe the structure of the EnergySchedule and the NERCTags. The EnergySchedule is based upon a TmSchedule type.

- Figure 41 - EnergySchedule Structure

- Figure 42 - NERCTags Structure

On submission, the following items are required for a DCTieSchedule:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid 15 minute interval boundary for trade date |
| endTime | K | datetime | End time for bid | Valid 15 minute interval boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| buyer | K | string | Trade buyer | Valid QSE |
| seller | K | string | Trade seller | Valid QSE |
| sp | K | string | Settlement point | Valid settlement point name |
| NERCTags/startTime | Y | datetime | Start time for tag | Valid time string |
| NERCTags/endTime | Y | datetime | End time for tag | Valid time string |
| NERCTags/tag | Y | string | NERC tag string | Valid NERC tag string |
| EnergySchedule/startTime | N | datetime | not used | not used |
| EnergySchedule/endTime | N | datetime | not used | not used |
| EnergySchedule/ TmPoint/time | Y | datetime | Absolute time for start of interval | Valid time in the trading date |
| EnergySchedule/ TmPoint/ending | N | datetime | Absolute time for end of interval | Valid time in the trading date |
| EnergySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 43 - DCTieSchedule Requirements

The following is an XML example for a DCTieSchedule:

<DCTieSchedule>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID12345</externalId>

<buyer>ACME</buyer>

<seller>XYZ</seller>

<sp>DC56</sp>

<EnergySchedule>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>500</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T04:00:00-06:00</time>

<value1>400</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T10:00:00-06:00</time>

<value1>520</value1>

</TmPoint>

</EnergySchedule>

<NERCTags>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<tag>NERC Tag String</tag>

</NERCTags>

</DCTieSchedule>

### DAM Energy Bid

The following diagram describes the structure of a DAM Energy Bid:

- Figure 44 - EnergyBid Structure

The PriceCurve structure is identical to the structure described previously in this document. On submission, the following items are required for an EnergyBid:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| sp | K | string | Settlement point | Valid settlement point name |
| bidId | K | string | Bid ID | MP supplied bid ID |
| PriceCurve/startTime | Y | datetime | Start time for curve | Valid hour boundary |
| PriceCurve/endTime | Y | datetime | End time for curve | Valid hour boundary |
| PriceCurve/curveStyle | Y | string | Used as an indicator to describe the type of ‘curve’ | FIXED, VARIABLE or CURVE |
| PriceCurve/CurveData/xvalue | Y | float | Megawatts | Quantity in MW |
| PriceCurve/CurveData/y1value | Y | float | Price in $/MWh | $/MWh |
| PriceCurve/multiHourBlock | N | Boolean | Indicates if offer must be taken as a block for all hours | Default is false |

Figure 45 - EnergyBid Requirements

The following is an XML example for an EnergyBid:

<EnergyBid>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

- <externalId>MyExternalID12345</externalId>

<expirationTime>2007-12-31T23:00:00-06:00</expirationTime>

<sp>ACME4</sp>

<bidID>2</bidID>

<PriceCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>FIXED</curveStyle>

<CurveData>

<xvalue>40</xvalue>

<y1value>43.54</y1value>

<multiHourBlock>0</multiHourBlock>

</CurveData>

</PriceCurve>

<PriceCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>FIXED</curveStyle>

<CurveData>

<xvalue>50</xvalue>

<y1value>63.54</y1value>

<multiHourBlock>0</multiHourBlock>

</CurveData>

</PriceCurve>

</EnergyBid>

### DAM Energy-Only Offer

The following diagram describes the structure of a DAM Energy-Only Offer:

- Figure 46 – EnergyOnlyOffer Structure

The EnergyOfferCurve structure uses the PriceCurve type structure described previously in this document. On submission, the following items are required for an EnergyOnlyOffer:

| Element | Req? | Datatype | Description | Values |  |
| --- | --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |  |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |  |
| externalId | N | string | External ID |  | QSE supplied |
| sp | K | string | Settlement point | Valid settlement point name |  |
| bidId | K | string | Bid ID | QSE supplied bid ID |  |
| expirationTime | Y | datetime | Time of offer expiration | Valid time before trade date |  |
| EnergyOfferCurve/startTime | Y | datetime | Start time for curve | Valid hour boundary |  |
| EnergyOfferCurve/endTime | Y | datetime | End time for curve | Valid hour boundary |  |
| EnergyOfferCurve/curveStyle | Y | string | Used as an indicator to describe the type of ‘curve’ | FIXED, VARIABLE or CURVE |  |
| EnergyOfferCurve/CurveData/xvalue | Y | float | Quantity in megawatts | MW |  |
| EnergyOfferCurve/CurveData/y1value | Y | float | Price in $/MWh | $/MWh |  |
| PriceCurve/multiHourBlock | N | Boolean | Indicates if offer must be taken as a block for all hours | Default is false |  |

Figure 47 - EnergyOnlyOffer Requirements

The following is an XML example for an EnergyOnlyOffer:

<EnergyOnlyOffer>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

- <externalId>MyExternalID12345</externalId>

<expirationTime>2007-12-31T22:00:00-06:00</expirationTime>

<sp>ACME3</sp>

<bidID>1</bidID>

<EnergyOfferCurve>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<curveStyle>VARIABLE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>43.00</y1value>

<multiHourBlock>0</multiHourBlock>

</CurveData>

</EnergyOfferCurve>

<EnergyOfferCurve>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<curveStyle>VARIABLE</curveStyle>

<CurveData>

<xvalue>20</xvalue>

<y1value>53.00</y1value>

<multiHourBlock>0</multiHourBlock>

</CurveData>

</EnergyOfferCurve>

</EnergyOnlyOffer>

### Energy Trade

An EnergyTrade describes a scheduled trade of energy between a buyer and a seller at a specified settlement point. The following diagram describes the structure of an Energy Trade:

- Figure 48 - EnergyTrade Structure

Within an EnergyTrade is an EnergySchedule. EnergySchedules are defined using the TmSchedule type, where intervals may be up to 15 minutes in granularity. This is shown in the following diagram:

- Figure 49 - EnergySchedule Structure

In order for a trade to be accepted, both the buyer and seller must submit matching trades, as identified using the buyer and seller tags. On submission, the following items are required for an EnergyTrade:

- startTime

- endTime

- buyer (QSE ID)

- seller (QSE ID)

- sp (settlement point name)

- EnergySchedule

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| sp | K | string | Settlement point | Valid settlement point name |
| buyer | K | string | Trade buyer | Valid QSE |
| seller | K | string | Trade seller | Valid QSE |
| EnergySchedule/startTime | N | datetime | not used | not used |
| EnergySchedule/endTime | N | datetime | not used | not used |
| EnergySchedule/ TmPoint/time | Y | datetime | Absolute time for start of 15 minute interval | Valid time in the trading date |
| EnergySchedule/ TmPoint/ending | N | datetime | Absolute time for end of 15 minute interval | Valid time in the trading date |
| EnergySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 50 - EnergyTrade Requirements

The following is an XML example for an EnergyTrade:

<EnergyTrade>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID12345</externalId>

<buyer>ACME</buyer>

<seller>Cogswell</seller>

<sp>ACME23</sp>

<EnergySchedule>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>200</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:15:00-06:00</time>

<value1>220</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>260</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:45:00-06:00</time>

<value1>100</value1>

</TmPoint>

</EnergySchedule>

</EnergyTrade>

### Output Schedule

An OutputSchedule describes the scheduled output for a resource at 5 minute intervals. The following diagram describes the structure of an Output Schedule:

- Figure 51 - OutputSchedule Structure

The EnergySchedule structure is identical to the structure described previously in this document. On submission, the following items are required for an OutputSchedule:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | N | datetime | Start time for schedule | Valid 5 minute interval boundary for trade date |
| endTime | N | datetime | End time for schedule | Valid 5 minute interval boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| resource | K | string | Resource | Valid resource name |
| combinedCycle | N | string | Combined cycle | Valid combined cycle ID |
| deleteTPOs | N | boolean | Set to true if three part offers for resource are to be deleted | 1 or 0 (default=0) |
| EnergySchedule/startTime | N | datetime | Not Used | Not Used |
| EnergySchedule/endTime | N | datetime | Not Used | Not Used |
| EnergySchedule/ TmPoint/time | Y | datetime | Absolute time for start of interval | Valid time between startTime and endTime for schedule |
| EnergySchedule/ TmPoint/ending | N | datetime | Absolute time for end of interval | Valid time between startTime and endTime for schedule |
| EnergySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 52 - OutputSchedule Requirements

The following is an XML example for an OutputSchedule:

<OutputSchedule>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID12345</externalId>

<resource>ResourceXYZ</resource>

<EnergySchedule>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>200</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>220</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>260</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>100</value1>

</TmPoint>

</EnergySchedule>

</OutputSchedule>

### PTP Obligation Bid

The following diagram describes the structure of a PTP Obligation bid:

- Figure 53 - PTPObligation Requirements

The CapacitySchedule structure is identical to the structure described previously in this document. The following diagram describes the structure for maximum prices. If more than one MaximumPrice block is provided, start and end times must not overlap.

- Figure 54 - MaximumPrice Sub-Structure

On submission, the following items are required for a PTPObligation:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid start hour boundary for trade date |
| endTime | K | datetime | End time for bid | Valid end hour boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| source | K | string | Source settlement point | Valid settlement point name |
| sink | K | string | Sink settlement point | Valid settlement point name |
| crrAccountHolderId | K | string | CRR account holder ID | Valid CRR account holder |
| bidId | K | string | MP assigned bid ID | Any string |
| MaximumPrice/startTime | Y | datetime | Start time for maximum price | Valid hour boundary |
| MaximumPrice/endTime | Y | datetime | End time for maximum price | Valid hour boundary |
| MaximumPrice/cost | Y | decimal | Maximum price | Any value |
| CapacitySchedule/startTime | N | datetime | not used | not used |
| CapacitySchedule/endTime | N | dateTime | not used | not used |
| CapacitySchedule/ TmPoint/time | Y | datetime | Absolute time for beginning of interval | Valid time between schedule start and end times |
| CapacitySchedule/ TmPoint/ending | N | datetime | Absolute time for end of interval | Valid time between schedule start and end times |
| CapacitySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 55 - PTPObligation Requirements

The following is an XML example for a PTPObligation:

<PTPObligation>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID12345</externalId>

<source>XYZ3</source>

<sink>ACME34</sink>

<bidId>1</bidId>

<CapacitySchedule>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>30</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>35</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T03:00:00-06:00</time>

<value1>20</value1>

</TmPoint>

</CapacitySchedule>

<MaximumPrice>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-01T10:00:00-06:00</endTime>

<price>333.5</price>

</MaximumPrice>

<MaximumPrice>

<startTime>2008-01-01T10:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<price>373.5</price>

</MaximumPrice>

</PTPObligation>

### Self-Schedule

The following diagram describes the structure of a Self-Schedule:

- Figure 56 - SelfSchedule Structure

The EnergySchedule structure is defined using a TmSchedule type. On submission, the following items are required for a SelfSchedule:

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| startTime | K | datetime | Start time for bid | Valid 15 minute boundary for trade date |
| endTime | K | datetime | End time for bid | Valid 15 minute boundary for trade date |
| externalId | N | string | External ID | QSE supplied |
| source | K | string | Source settlement point | Valid settlement point name |
| sink | K | string | Sink settlement point | Valid settlement point name |
| EnergySchedule/startTime | N | datetime | not used | not used |
| EnergySchedule/endTime | N | dateTime | not used | not used |
| EnergySchedule/ TmPoint/time | Y | datetime | Absolute time for start of interval | Valid time within the trade date |
| EnergySchedule/ TmPoint/ending | N | datetime | Absolute time for end of interval | Valid time within the trade date |
| EnergySchedule/ TmPoint/value1 | Y | float | Megawatts | >= 0 |

Figure 57 – SelfSchedule  Requirements

The following is an XML example for a SelfSchedule:

<SelfSchedule>

- <startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>MyExternalID12345</externalId>

<source>SP456</source>

<sink>ACME23</sink>

<EnergySchedule>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>35</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T01:00:00-06:00</time>

<value1>50</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T02:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

</EnergySchedule>

</SelfSchedule>

## Error Handling

The purpose of this section is to describe the handling of errors related to submissions, updates, queries and cancellations. This is especially important, as BidSets can be partially accepted, where some bids, offers, trades or schedules within a BidSet can be accepted, and others rejected.

| Condition | Action | Error Reported |
| --- | --- | --- |
| Bad parameters in message header | Request is rejected | ERROR: INVALID REQUEST |
| Invalid Source or UserId | Request is rejected | ERROR: NOT AUTHORIZED |
| Payload can not be interpreted | Request is rejected | ERROR: BAD PAYLOAD |
| Parameters within BidSet header are invalid, e.g. bad trading date | No bids or updates are accepted | ERROR: BAD BIDSET |
| Request for a get, cancel or update identifies an unknown transaction (i.e. bid, offer, trade, schedule) | The request is processed only for the known transactions. | WARNING: UNKNOWN ID: …… |
| Some transactions within a BidSet are rejected | Valid transactions are accepted as submitted, invalid transactions are rejected and marked with errors. | BidSet on reply identifies errors for those transactions that were rejected on a transaction by transaction basis. |

Figure 58 - Error Handling

It is also important to note that the submission of a BidSet involves multiple steps:

- BidSet is syntax scanned, where only basic validity checks are performed. The reply identifies where basic validity checks failed. Transactions within the BidSet that pass basic validation checks are marked as ‘SUBMITTED’ and a transaction ID (mRID) value is returned.

- A more thorough validity check is performed asynchronously by the Market Management System (MMS)

- After acceptance or rejection by MMS, a notification is issued that identified whether the transaction was ‘ACCEPTED’, is ‘PENDING’ or had an error.

The following structure is used to report errors, where the severity, area of the error or the specific time can be identified.

The error structure is populated in the following manner:

- severity may be ‘error’, ‘warning’ or ‘informational’, as determined by the source system

- area may be used to indicate the tag or parent tag (e.g. EnergySchedule), where an error was identified

- interval may be used to identify the time of the data item of concern, where many data items may be related to a schedule submission. For market transaction interfaces, a market interval (using hour ending and minute ending) is used as opposed to a timestamp

- text as provided by the source system

When possible, the described error structure will be used. However, there are many cases where error5 structure can not be used, as in cases where schema validations fail and normal validation is not possible. These are typically the result of programming errors. Examples would include:

- XML is not well formed

- XML is not schema compliant, from the perspective of basic data structures

- XML is not schema compliant, from the perspective that values provided are invalid (e.g. invalid time strings, nulls where values are required, non-numeric values where numeric values are required, obviously out of range values, invalid enumerations)

## Usage and XML Examples

The purpose of this section is to provide usage details with XML examples for the submission (create), query (get), update and cancellation (cancel) of bids, offers, trades and schedules.

### Offer and Bid Set Submission

When submitting a set of bids, offers, trades and/or schedules, the following structures are used within the message:

| Message Element | Value |
| --- | --- |
| Header/Verb | create |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload | BidSet |

Figure 59 - create BidSet Request Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | BidSet, returning transaction IDs (mRIDs) for individual bids, offers, trades or schedules and or error messages. |

Figure 60 - reply BidSet Parameters

The following is an example message for a request message for the submission of a BidSet by a Market Participant. The following example uses a SelfArrangedAS.

<RequestMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<Header>

<Verb>create</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3264874657467444949</Nonce>

<Created>2006-12-09 15:35:57Z</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>SOMEQSE</Source>

<UserID>Nemat</UserID>

<MessageID>231232466</MessageID>

<Comment>Example message</Comment>

</Header>

<Payload>

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>2007-01-04</tradingDate>

<SelfArrangedAS>

<startTime>2007-12-17T00:00:00-06:00</startTime>

<endTime>2007-12-18T00:00:00-06:00</endTime>

<externalId>myExternalID45473</externalId>

<asType>Non-Spin</asType>

<CapacitySchedule>

<TmPoint>

<time>2007-12-17T00:00:00-06:00</time>

<value1>120</value1>

</TmPoint>

<TmPoint>

<time>2007-12-17T02:00:00-06:00</time>

<value1>130</value1>

</TmPoint>

<TmPoint>

<time>2007-12-17T06:00:00-06:00</time>

<value1>115</value1>

</TmPoint>

</CapacitySchedule>

</SelfArrangedAS>

… additional bids inserted here …

</BidSet>

</Payload>

</RequestMessage>

The following is an example response message, where the submission request was successful:

<ResponseMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

- <Header>

<Verb>reply</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>74646464</Nonce>

<Created>2006-12-09T15:36:03-06:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>ERCOT</Source>

<UserID>MMS</UserID>

<MessageID>231232466</MessageID>

</Header>

<Reply>

<ReplyCode>OK</ReplyCode>

</Reply>

<Payload>

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>2007-01-04</tradingDate>

<SelfArrangedAS>

<mRID>SOMEQSE.20070104.XXXXX</mRID>

<externalId>myExternalID45473</externalId>

<status>SUBMITTED</status>

</SelfArrangedAS>

</BidSet>

</Payload>

</ResponseMessage>

The following is an example response where a submission was unsuccessful, as a consequence of invalid parameters on the BidSet:

<ResponseMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

- <Header>

<Verb>reply</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>74646464</Nonce>

<Created>2006-12-09T15:36:03-06:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>ERCOT</Source>

<UserID>MMS</UserID>

<MessageID>231232466</MessageID>

</Header>

<Reply>

<ReplyCode>ERROR</ReplyCode>

<Error>Bad trading date</Error>

</Reply>

</ResponseMessage>

The following is an example response where a submission was unsuccessful, as a consequence of syntax failures within a specific bid/offer/trade/schedule within the BidSet. It is important to note that only the status and errors (if any exist) are returned for each bid.

<ResponseMessage xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

- <Header>

<Verb>reply</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>74646464</Nonce>

<Created>2006-12-09T15:36:03-06:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>ERCOT</Source>

<UserID>MMS</UserID>

<MessageID>231232466</MessageID>

</Header>

<Reply>

<ReplyCode>ERROR</ReplyCode>

<Error>Bid syntax errors</Error>

</Reply>

<Payload>

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>2007-01-04</tradingDate>

<XYZ>

<status>ERROR</status>

<error><severity>error</severity><text>Unknown bid type XYZ</text></error>

</XYZ>

<ThreePartOffer>

<status>ERROR</status>

<error><severity>error</severity><text>Bad schema</text></error>

</ThreePartOffer>

<COP>

<mRID>SOMEQSE.20070104.YYYYYYYY</mRID>

<externalId>SOMEQSE external ID 123</externalId>

<status>SUBMITTED</status>

</COP>

... sequence of bids with status and  errors (if any)  appears here ...

</BidSet>

</Payload>

</ResponseMessage>

The more detailed validations errors would typically be available after a period of minutes, where a notification would be issued to identify that errors were found.

An important note is that the order of the bids, offers, trades and schedules within the reply BidSet is identical to the order that was provided in the BidSet for the submission. This is important so that the mRID in the response can be associated with the correct bid, offer, trade or schedule from the submission.

### Querying Bids, Offers, Trades and Schedules

A query request uses the ‘get’ verb on the message header is used to query specific bids, offers, trades and schedules. There are two options for querying a set of bids, offers, trades or schedules:

- The transaction ID (mRID) for each bid, offer, trade or schedule of interest is identified using the message Request/ID parameter. These must be for the same trade date.

- In all cases only the trading date and a product type is supplied, where only the bids, offers, schedules and trades for that trading date of the specified product type will be returned. This is done using a partial (shortened) mRID in the form ‘<QSE>.<date>.<product>’, such as ‘SOMEQSE.20081112.COP’.

Table below reflects the required short mRID tokens for every bid type:

| BID Type | Type (XML tag) | Short mRID Key String |
| --- | --- | --- |
| Ancillary Service Offer | ASOffer | <QSE>.<TradingDate>.ASO |
| Ancillary Service Trade | ASTrade | <QSE>.<TradingDate>.AST |
| Capacity Trade | CapacityTrade | <QSE>.<TradingDate>.CT |
| Current Operating Plan | COP | <QSE>.<TradingDate>.COP |
| CRR Offer | CRR | <QSE>.<TradingDate>.CRR.Source.Sink |
| DC Tie Schedule | DCTieSchedule | <QSE>.<TradingDate>.DCT |
| DAM Energy Bid | EnergyBid | <QSE>.<TradingDate>.EB.SP |
| DAM Energy-Only Offer | EnergyOnlyOffer | <QSE>.<TradingDate>.EOO.SP |
| Energy Trade | EnergyTrade | <QSE>.<TradingDate>.ET |
| Inc Dec Energy Offer | IncDecOffer | <QSE>.<TradingDate> |
| Output Schedule | OutputSchedule | <QSE>.<TradingDate>.OS |
| PTP Obligation Bid | PTPObligation | <QSE>.<TradingDate>.PTP.Source.Sink |
| Self-Arranged AS | SelfArrangedAS | <QSE>.<TradingDate>.SAA |
| Self-Schedule | SelfSchedule | <QSE>.<TradingDate>.SS |
| Three Part Supply Offer | ThreePartOffer | <QSE>.<TradingDate>.TPO |

- Figure 61 – Short mRID Key String

In all cases the verb is ‘get’, the noun is ‘BidSet’ and the Request/tradingDate is specified. An important note is that this interface will not return any bids, offers, trades or schedules that were invalid upon submission. The externalId will not be returned, as it is not maintained by MMS.

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | ID of user |
| Request/ID | One or more mRID values, where the products are of the same type and for the same trading date. |
| Request/TradingDate | Trading date of interest |

Figure 62 - get BidSet Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | BidSet, returning details for desired bids, offers, trades and schedules |

Figure 63 - reply BidSet Parameters

The following is an example response payload:

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>2008-01-01</tradingDate>

<ASTrade>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>ACCEPTED</status>

<buyer>Acme</buyer>

<seller>Cogswell</seller>

<asType>Reg-Up</asType>

<ASSchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T20:00:00-06:00</time>

<value1>400</value1>

</TmPoint>

</ASSchedule>

</ASTrade>

</BidSet>

### Updating Bids, Offers, Trades and Schedules

A request to update specific bids, offers, trades and schedules is identically the same as a submission request. If the bid, offer, schedule or trade was previously submitted it will be updated. If it did not previously exist, it will be created.

For the purpose of bidding, the ‘create’ and ‘change’ verbs are used interchangeably.

| Message Element | Value |
| --- | --- |
| Header/Verb | change |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | ID of user |
| Payload | BidSet |

Figure 64 - change BidSet Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | BidSet, returning transaction IDs (mRIDs) for individual bids, offers, trades or schedules and or error messages. |

Figure 65 - reply BidSet Parameters

The following is an example request payload, where a previously submitted ASTrade will be updated. There may be more than one bid, offer, trade or schedule updated at a time. New submissions may also be mixed with updates.

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>2008-01-01</tradingDate>

<ASTrade>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<externalId>My External ID 123454</externalId>

<buyer>Acme</buyer>

<seller>Cogswell</seller>

<asType>Reg-Up</asType>

<ASSchedule>

<startTime>2008-01-01T00:00:00-06:00</startTime>

<endTime>2008-01-02T00:00:00-06:00</endTime>

<TmPoint>

<time>2008-01-01T00:00:00-06:00</time>

<value1>40</value1>

</TmPoint>

<TmPoint>

<time>2008-01-01T20:00:00-06:00</time>

<value1>70</value1>

</TmPoint>

</ASSchedule>

</ASTrade>

</BidSet>

### Canceling Bids, Offers, Trades and Schedules

A cancellation request uses the ‘cancel’ verb on the message header in order to cancel one or more query specific bids, offers, trades and schedules, as identified using an mRID. It is important to note that a COP submission can not be canceled, as it is required and can otherwise only be updated. It is also important to note that the mRID values must identify bids, offers, trades and schedules for the same trade date.

| Message Element | Value |
| --- | --- |
| Header/Verb | cancel |
| Header/Noun | BidSet |
| Header/Source | Market participant ID |
| Header/UserID | ID of user |
| Request/ID | One or more mRID values |

Figure 66 - cancel BidSet Parameters

When canceling a bid, it is also possible to cancel individual hours of a bid. This would be done by suffixing the mRID of the bid to be canceled with the specific hour. If the hour is not provided, all hours will be canceled.

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BidSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload/BidSet | BidSet identifying bids canceled, identifying mRID and status=CANCELED |

Figure 67 - reply BidSet Parameters

Note: Some details of this interface are dependent upon finalization of the MMS design to determine if the cancel will be confirmed synchronously or asynchronously.

The following is an example BidSet that would be provided in the response message:

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>2008-01-01</tradingDate>

<ASOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<status>CANCELED</status>

</ASOffer>

<ASOffer>

<mRID>ACME.20080101.XXXXXXXY</mRID>

<status>CANCELED</status>

</ASOffer>

</BidSet>

# Market Information ***

This service is used to request specific types of market-related information. These are read-only interfaces, as opposed to transactional interfaces.

Note: The interfaces described in this section are subject to change based upon final EMS and/or MMS designs.

## Interfaces Provided

Specific interfaces using specific combinations of verbs and nouns (i.e. payload types) are defined to permit a market participant to programmatically access market information. The verb to be used for requests would in all cases be ‘get’. The noun would identify the type of information being requested. Each request could use a message ‘Request’ package to specify one or more parameters that would qualify the request.

The processing sequence is shown in the following sequence diagram.

- Figure 68 - Market Information Request Sequence Diagram

## Interfaces Required

The messages for market information requests would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | Name of payload type |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/? | Optional: Other request parameters may be specified as needed |
| Payload | Message payload data with type defined by Noun |

Figure 69 - Parameters for 'get' Requests

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Defined payload type name |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | May be any number of error messages |
| Payload | Defined payload type |

Figure 70 - Parameters for 'reply' Messages

In the cases of payloads that would otherwise exceed 1 megabyte, the payloads would be zipped, base64 encoded and stored within the ‘Payload/Compressed’ tag.

## Message Specifications

Specific payload definitions are not currently defined. These will be defined in future revisions.

### Get AwardSet

The Get AwardSet interface provides the means for a market participant to obtain awards. The following parameters are specified in the RequestMessage:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | AwardSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/TradingDate | Trading date |

Figure 71 - Get AwardSet Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | AwardSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/AwardSet | AwardSet |

Figure 72 - Get AwardSet Response

An AwardSet is returned in the payload of the ResponseMessage using the following structure:

- Figure 73 - AwardSet Container Structure

Within an AwardSet, there are a variety of different Award types. The following diagram describes the basic structure for each type of Award:

- Figure 74 - Generalized Award Structure

- The specific structured for each type of Award are described in section 5.3.

### Forecasted Load

The purpose of this interface is to provide the means to obtain forecasted load values from ERCOT. Depending upon the time interval specified, the forecasts may be medium-term (MT) or long term (LT). The input parameters to this request include:

- startTime

- endTime

- zone, which could be a weather zone or a load zone. If either ‘system’ or omitted, the system load forecast is returned.

- Desired forecast type (MT or LT), noting that the MT forecast is only available for seven days ahead.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LoadForecast |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |
| Request/Zone | Desired weather or load zone. Default is for system load. |
| Request/Option | Desired forecast type: MT or LT |

Figure 75 – Load Forecast Request Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LoadForecast |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/LoadForecast | Load forecast |

Figure 76 – Load Forecast Response Parameters

The following diagram describes the structure of a load forecast, which is based upon a TmSchedule. For medium term load forecasts, the values of ‘value1’ indicate the hourly forecasted megawatts. In the case of long-term (LT) forecasts, the value of ‘value2’ will be used to indicate the minimum load for a given day and the value of ‘value1’ will be used to identify the peak load for the day. This will be used to return the data for the specified zone (or system is zone was not specified).

- Figure 77 - Load Forecast Structure

### Real-Time System Load

The purpose of this interface is to provide the means to obtain real-time system load from ERCOT. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | SystemLoad |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Optional: Start time of interest |
| Request/EndTime | Optional: End time of interest |

Figure 78 - SystemLoad Request Parameters

If no startTime is specified, the loads for the current day will be returned. The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | SystemLoad |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/SystemLoad | System loads for period of interest |

Figure 79 - SystemLoad Response Parameters

The structure of SystemLoad is described by the following diagram. This used a TmSchedule.

- Figure 80 - SystemLoad Structure

### Market Totals   *** Same as Total DAM Energy in section 4.3.23 ***

The purpose of this interface is to provide a query for market totals. Market totals supported by this interface include:

- Total energy bought in DAM

- Total energy sold in DAM

- Total ancillary service offers received (for each type of ancillary service)

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | MarketTotals |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/option | One of: - AncillaryServiceOffers - EnergyBoughtInDAM - EnergySoldInDAM |
| Request/asType | Ancillary services type (optional, used for option AncillaryServiceOffers) |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

Figure 81 - Market Totals Request Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | MarketTotals |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/MarketTotals | Market totals of interest |

Figure 82 - MarketTotals Response Parameters

The Totals structure is based upon a CIM RegularIntervalSchedule, where the values of ‘value1’ are total megawatt values.

- Figure 83 - MarketTotals Structure

### Market LMPs and SPPs

The purpose of this interface is to provide a query for market LMPs and SPPs. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LMPs (or SPPs) |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |
| Request/MarketType | DAM or RTM |
| Request/ID | Optional: bus name for LMP request or settlement point name for SPP request |

Figure 84 - LMP and SPP Request Parameters

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LMPs (or SPPs) |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ | LMPs (or SPPs) |

Figure 85 - LMP and SPP Response Parameters

The structure of an LMPs/SPPs payload are shown by the following diagrams, where each LMP/SPP uses a TmSchedule type for a specified bus or SettlementPoint.

- Figure 86 - LMP Container Structure

- Figure 87 - SPP Container Structure

In both cases the data structure is based upon a TmSchedule, where values of ‘value1’ are in $/MWh and values of ‘time’ are seconds from the startTime. The last point on the schedule is valid until the endTime.

### Market MCPCs

The purpose of this interface is to provide a query for MCPCs, for either the DAM or ADJ market.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | MCPCs |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/MarketType | DAM or RTM |
| Request/StartTime | Start time |
| Request/EndTime | End time |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | MCPCs |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/MCPCs | MCPCs |

The following diagram describes the structure of MCPCs:

- Figure 88 - MCPC Container Structure

Within each MCPCSchedule, an asType is specified and the values of ‘value1’ convey individual MCPC values.

### Binding Constraints

This section describes interfaces used to retrieve binding constraints. There are several options for retrieval of binding constraints:

- Binding constraints related to DRUC

- Binding constraints related to HRUC

- DAM/RTM shadow prices for binding constraints

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | BindingConstraints |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/Option | DAM/RTM/DRUC/HRUC |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | BindingConstraints |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/BindingConstraints | BindingConstraints |

The following diagram describes the structures that are used for the retrieval of binding constraints and shadow prices:

- Figure 89 - BindingConstraint Container Structure

The following elements are used to report binding constraints:

- Name of the constraint

- Value that exceeded limit

- Limit defined for constraint

- Maximum shadow price

- Delivery time

- Settlement point names (sp)

- Shift factors

- RUC type (DRUC, HRUC, DAM, RTM)

### Active Contingencies in SCED

This section describes interfaces used to retrieve active contingencies in SCED. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | SCEDViolatedConstraints |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/marketType | RTM or ADJ |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | SCEDViolatedConstraints |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/SCEDViolatedConstraints | SCEDViolatedConstraints |

The structure of SCED violated constraints are described by the following diagram:

- Figure 90 - SCEDViolatedConstraints Container Structure

The following elements are used to report violated constraints:

- Name of the constraint

- Value that exceeded limit

- Defined limit for constraint

- Timestamp of violation

- Settlement point names (sp)

- Shift factors

### Ancillary Service Obligation

The purpose of this interface is to provide ancillary service obligation by service type for the requesting QSE. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | ASObligations |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ASType | Optional: AS type |
| Request/OperatingDate | OperatingDate |

If the AS type is not specified, all AS obligations will be returned. The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | ASObligations |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ASObligations | ASObligations |

The structure of the ASObligations payload is described by the following diagram, using a CIM RegularIntervalSchedule. The values of ‘value1’ would represent the MW obligation and the values of ‘value2’ would represent a positive or negative difference from COP.

- Figure 91 - ASObligations Container Structure

### Dynamic Ratings

This section describes interfaces used to retrieve current thermal ratings for conducting equipment. The time of interest is specified on the request. If the time is not specified on the request, data is returned for the current day. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | DynamicRatings |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | DynamicRatings |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ DynamicRatings | DynamicRatings |

The payload structure is described by the following diagram:

- Figure 92 - DynamicRatings Container Structure

The ‘equipment’ tag identifies the name of the transmission element. The types of ratings that are currently defined include:

- Normal (in MVA)

- 2-hour (in MVA)

- 15-min (in MVA)

### Voltage Profiles

This section describes interfaces used to retrieve voltage profiles for electrical buses. The time of interest is specified on the request. If the time is not specified on the request, data is returned for the current day. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | VoltageProfiles |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | VoltageProfiles |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/VoltageProfiles | VoltageProfiles |

The payload structure is described by the following diagram:

- Figure 93 - VoltageProfiles Container Structure

### Total Regulation

This section describes interfaces used to retrieve the daily total reg up and reg down for a given time period. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | TotalRegulation |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | TotalRegulation |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/TotalRegulation | Schedule of total regulation. Value1 = reg up MW, value2 = reg down MW. |

- The payload structure is described by the following diagram, using a TmSchedule.

- Figure 94 - TotalRegulation Structure

The values of ‘value1’ are the regulation up in megawatts, and the values of ‘value2’ are the regulation down megawatts.

### Load Ratio Share

This section describes interfaces used to retrieve the load ratio share for each QSE. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LoadRatioShares |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Time of interest |
| Request/Zone | Optional: weather zone of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LoadRatioShares |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/LoadRatioShares | LoadRatioShares |

The LoadRatioShares payload structure is described by the following diagram:

- Figure 95 - LoadRatioShares Container Structure

The values of ‘value1’ would be used to identify the proportion assigned to each QSE for each hour.

### Competitive Constraints

This section describes interfaces used to retrieve competitive constraints. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | CompetitiveConstraints |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | Optional: End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | CompetitiveConstraints |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ CompetitiveConstraints | List of competitive constraints |

The payload structure is described by the following diagram:

- Figure 96 - CompetitiveConstraints Container Structure

The following elements are used to report competitive constraints:

- Name of the constraint

- Limit: name of limit

- Value: value that exceeded limit

- Status of constraint

- Timestamp of constraint

- Flag: DAILY, MONTHLY or YEARLY

### Load Distribution Factors

This section describes interfaces used to retrieve load distribution factors. The time of interest is specified on the request. If the time is not specified on the request, data is returned for the current day. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LoadDistributionFactors |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | ‘SYSTEM’ or ID of weather zone or load group for which the factors for ‘children’ are desired |
| Request/StartTime | Time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LoadDistributionFactors |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ LoadDistributionFactors | LoadDistributionFactors |

The payload structure is described by the following diagram:

- Figure 97 - LoadDistributionFactors Container Structure

Each load distribution factor identifies:

- Name of the zone, group or load

- Level, to identify ZONE, LOADGROUP or LOAD

- Factor, as a proportion

### Shift Factor

This section describes interfaces used to retrieve real-time Shift Factor. The time of interest is specified on the request. If the time is not specified on the request, data is returned for the current day. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | ShiftFactors |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | ShiftFactors |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ShiftFactors | ShiftFactors |

The payload structure is described by the following diagram, where there is one shift factor for each resource and branch pair:

- Figure 98 - ShiftFactors Container Structure

For each shift factor the following elements are provided:

- Name of resource

- Name of branch (equipment)

- Shift factor

### Customer Load Profile

This section describes interfaces used to retrieve the load profile for non-IDR metered customers. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | LoadProfiles |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | ID of customer(s) |
| Request/StartTime | Start time |
| Request/EndTime | End time |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | LoadProfiles |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |
| Payload/LoadProfile | LoadProfiles |

The payload structure is described by the following diagram:

- Figure 99 - LoadProfile Container Structure

- The values of ‘value1’ are used to convey the KWh consumption for each time interval.

- TBD: The specific key needed to reference a customer load profile needs to be identified.

### Aggregated Ancillary Service Offer Curves

This section describes interfaces used to retrieve aggregated ancillary service offer curves. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | AggregatedASOfferCurves |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time |
| Request/EndTime | End time |
| Request/asType | Optional: AS type (if not specified, all are returned) |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | AggregatedASOfferCurves |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ AggregatedASOfferCurves | AggregatedASOfferCurves |

The payload structure is described by the following diagrams, where there is one curve for each AS type per hour.

- Figure 100 - AggregatedASOfferCurve Schedule

For each point on the AggregatedASOfferCurve, the xvalue identifies a megawatt level, the y1value indicates a $/hour value.

### Ancillary Service Plan

This section describes interfaces used to retrieve the Ancillary Service Plan. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | ASSystemPlan |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time |
| Request/EndTime | End time |
| Request/asType | Optional: AS type (if not specified, all are returned) |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | ASSystemPlan |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ASSystemPlan | ASSystemPlan |

The payload structure is described by the following diagram:

- Figure 101 - ASSystemPlan Container Structure

- Figure 102 - ASSchedule Structure

- The values of ‘value1’ identify the total scheduled MW capacity for the given AS type.

### Unit Availability

This section describes interfaces used to retrieve the availability for units. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | UnitAvailabilities |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time |
| Request/EndTime | End time |
| Request/option | Optional: RMR or BlackStart, default is all |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | UnitAvailabilities |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ UnitAvailabilities | UnitAvailabilities |

The payload structure is described by the following diagram:

- Figure 103 - UnitAvailabilities Container Structure

For each UnitAvailability element, the following are provided:

- Name of resource

- Contract type: RMR or BlackStart

- Start time

- End time (if known)

- Available (true/false)

- Available megawatts

### Startup and Shutdown Instructions

This section describes interfaces used to retrieve unit startup and shutdown instructions. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | StartupShutdownInstructions |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | StartupShutdownInstructions |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ StartupShutdownInstructions | StartupShutdownInstructions |

The payload structure is described by the following diagram:

- Figure 104 - StartupShutdownInstructions Container Structure

For each startup or shutdown instruction, the following elements are provided:

- RUC type (DRUC, HRUC, DAM)

- Delivery date

- Resource name

- Cleared commit

- Cleared decommit

- Reason code for decommit

### Total Ancillary Service Offers

This section describes interfaces used to retrieve total ancillary services offers. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | TotalASOffers |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time of interest |
| Request/EndTime | End time of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | TotalASOffers |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ TotalASOffers | TotalASOffers |

The payload structure is described by the following diagrams:

- Figure 105 - ASOffers Structure

The values of ‘value1’ identify the total MW offers for the given AS type. There is no consideration given to linked offers.

### Total DAM Energy   *** Same as Total Market Totals in section 4.3.23 ***

This section describes interfaces used to retrieve the total amount of energy bought or sold in the Day Ahead Market. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | TotalEnergy |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Start time |
| Request/EndTime | End time |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | TotalEnergy |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/ TotalEnergy | TmSchedule, where value1=total energy bought and value2=total energy sold |

The payload structure is described by the following diagram:

- Figure 106 - TotalEnergy Structure

The values of ‘value1’ indicate the total energy bought and the values of ‘value2’ indicate the total energy sold.

### Derated CRRs

This section describes interfaces used to retrieve derated CRRs. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | DeratedCRRs |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/OperatingDate | Date of interest |

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | DeratedCRRs |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/DeratedCRRs | DeratedCRRs |

The payload structure is described by the following diagram:

- Figure 107 - DeratedCRRs Container Structure

# Notifications ***

A key aspect of the design for external interfaces for Market Participants is to provide a simplified variant of the OASIS WS-Notifications standard for the definition of an ERCOT Notification interface. Each Market Participant using the external interface would be required to provide a listener interface for the receipt of notification messages, compliant with the interface provided by ERCOT.

## Interfaces Provided

The interfaces provided are based upon the WS-Notifications specification, although for use by ERCOT the interface has been simplified and augmented to fill one gap in the WS-Notification specification. For the purposes of use by Market Participants, there is only one interfaces of interest that is implemented by a Market Participant listener, this is the ‘Notify’ interface.

The ‘Notify’ interface is used as a means to asynchronously receive information by Market Participants from ERCOT. Specific examples of this information include:

- Notices

- Alerts

- BidSet acceptance or error reporting

- Awards

- Obligations

- Startup/Shutdown Instructions

- Outages

The following sequence diagram describes the typical/potential set of information exchanges related to Notifications. The actual flows would be different based on the type of notification.

- Figure 108 - Notification Sequence Diagram

## Interfaces Required

The contents of the any structure would be wrapped using the ResponseMessage structure defined in section 2.1.4, as this would allow necessary security information to be passed in a uniform manner. The following table describes the contents of the Notify message.

| Message Element | Value |
| --- | --- |
| NotificationMessage/Message/any | One or more messages as defined using ERCOT common message structure |

- The following table describes the contents of the Acknowledge message.

| Message Element | Value |
| --- | --- |
| ReplyCode | OK or ERROR |
| TimeStamp | Current time string |

## Message Specifications

The WS-Notifications specification identifies a single message structure for notification messages. The payload that is to be delivered by the notification message is represented by the ‘any’ element in the following message structure, loosely coupling the definition of the payload from the definition of the container message structure defined by WS-Notifications thereby allowing WS-Notifications to be used with any XML data structure. ERCOT uses the SubscriptionReference, Topic and ProducerReference are not used.

- Figure 109 - WS-Notifications Message Structure

It is important to note that the payloads for NotificationMessages are ALWAYS conveyed using the common message structure (e.g. Message, RequestMessage or ResponseMessage) as defined in section 2.1 serving as the ‘any’ sub-structure in the NotificationMessage structure.

ERCOT uses a variant of WS-Notifications, where an ‘Acknowledge’ message is sent to acknowledge the receipt of a notification. The WSDL and XSD provided by ERCOT are also simplified to eliminate definitions for interfaces and structures not used by ERCOT. These are described in the appendix.

ERCOT will allow a Market Participant to specify up to two URLs. ERCOT will send notifications to port 443 of the specified URL using an HTTPS connection. ERCOT will sign the SOAP message, where the signature can be verified using the ERCOT public key. This will verify that the notification was sent by ERCOT and was not altered. The ‘Acknowledge’ message does not need to be signed.

### Notices and Alerts ***

The specific payload for an Alert or Notice is described in the following diagram. This could be used for Notices and Alerts. Where there may be many different types of Notifications, the primary difference between a Notice and an Alert is that there is an expectation that an Alert would be acknowledged.

- Figure 110 - Notification Structure

Specific types of notices and alerts are described by the following table:

| Type | Alert? | Description | Protocol Ref | Dest | Data |
| --- | --- | --- | --- | --- | --- |
| SCEDFailure | N | Notice of SCED failure | 6.5.9.2 | All QSEs | Time interval |
| NoCurve | N | Notice that an RTM energy curve or output schedule was not submitted for resource |  | QSE | Resource, time interval |
| NeedAdditionalAS | N | Notice of need for additional AS resources | 3.16 | All QSEs | Needed MW, AS type, time interval |
| Alert | Y | Generic alert | 6.5.9.C.C |  |  |
| Resettlement | N | Notice of resettlement for DAM/RTM | 9.2.6, 9.5.7 | All MPs | Trade Date |
| RTMTrueUp | N | Notice of True-Up settlement timeline changes for RTM | 9.5.9 | All MPs | Trade date, Time of true-up |
| StatusCheck | N | Used to check status of notification link | NA | All MPs | None |

Figure 111 - Notice and Alert Types

### Offer and Bid Set Acceptance

The purpose of this notification is to indicate the acceptance of a BidSet to a QSE. This is necessary as BidSets are validated some time after the initial submission.

A more detailed description of this process is described in section 3. The following is an example payload where all bids have been set to a PENDING state.

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>2008-01-01</tradingDate>

<ASOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<externalId>XID12345</externalId>

<status>PENDING</status>

</ASOffer>

<ASOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<externalId>XID12346</externalId>

<status>PENDING</status>

</ASOffer>

</BidSet>

### Offer and Bid Set Errors

The purpose of this message is to indicate that one or more bids in a BidSet had errors. Those bids in a BidSet without errors are otherwise accepted. Error may also indicate the rejection of bids or offers for reasons such as the failure to receive credit approval.

Examples of errors are described in section 3.4. The following is an XML example where some bids were not valid and therefore rejected.

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>2008-01-01</tradingDate>

<ASOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<externalId>XID12360</externalId>

<status>PENDING</status>

</ASOffer>

<ASOffer>

<mRID>ACME.20080101.XXXXXXXX</mRID>

<externalId>XID12361</externalId>

<status>ERROR</status>

<error><severity>ERROR</severity><text>Invalid AS Type</text></error>

</ASOffer>

</BidSet>

### Confirmed and Unconfirmed Trades

A notification is issued periodically to a QSE where they have been identified as the counter party in a trade, but have not yet entered a matching energy, capacity or ancillary services trade. If a matching trade is not entered by the close of the market, the trade is rejected.

The following structure is used for conveying unconfirmed trades:

- Figure 112 - UnconfirmedTrades Container

The details of the structures for ASTrades, CapacityTrades and EnergyTrades are described in sections 3.3.5, 3.3.6 and 3.3.12, respectively. The following is an XML of an unconfirmed trades payload:

<UnconfirmedTrades>

<ASTrade>

<mRID>ACME.20080101.YYYYYYYY</mRID>

</ASTrade>

<CapacityTrade>

<mRID>XYZ.20080101.YYYYYYYY</mRID>

</CapacityTrade>

<EnergyTrade>

<mRID>ACME.20080101.YYYYYYYY</mRID>

</EnergyTrade>

</UnconfirmedTrades>

The mRID used in the message is the mRID for the trade that was submitted by the counterparty. Similarly to unconfirmed trades, the following figure describes the container for confirmed trades.

- Figure 113 - Confirmed Trades

The following is an XML of an confirmed trade’s payload:

<ConfirmedTrades>

<ASTrade>

<mRID>ACME.20080101.YYYYYYYY</mRID>

</ASTrade>

<CapacityTrade>

<mRID>XYZ.20080101.YYYYYYYY</mRID>

</CapacityTrade>

<EnergyTrade>

<mRID>ACME.20080101.YYYYYYYY</mRID>

</EnergyTrade>

</ConfirmedTrades>

### Energy Offer Awards

The purpose of this message is to identify Energy Offer Awards to a QSE. The structure of this message is shown by the following diagram.

- Figure 114 - AwardedEnergyOffer Structure

The following is an XML example:

<AwardedEnergyOffer>

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

- <endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String</mRID>

<marketType>DAM</marketType>

<resource>ACMEUnit1</resource>

<dispatchMWh>110</dispatchMWh>

<spp>22.78</spp>

<lmp>23.28</lmp>

</AwardedEnergyOffer>

### DAM Energy-Only Offer Awards

The purpose of this message is to identify DAM Energy-Only Offer Awards to a QSE. The structure of this message is shown by the following diagram.

- Figure 115 - AwardedEnergyOnlyOffer Structure

The following is an XML example:

<AwardedEnergyOnlyOffer>

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

- <endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String</mRID>

<marketType>DAM</marketType>

<sp>SP4</sp>

<awardedMWh>50</awardedMWh>

<spp>23.62</spp>

</AwardedEnergyOnlyOffer>

### DAM Energy Bid Award

The purpose of this message is to identify DAM Energy Bid Awards to a QSE. The structure of this message is shown by the following diagram.

- Figure 116 - AwardedEnergyBid Structure

The following is an XML example:

<AwardedEnergyBid>

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

- <endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String</mRID>

<marketType>DAM</marketType>

<sp>SP3</sp>

<awardedMWh>70</awardedMWh>

<spp>23.65</spp>

</AwardedEnergyBid>

### Ancillary Service Awards

The purpose of this message is to identify Ancillary Services Awards to a QSE. The structure of this message is shown by the following diagram.

- Figure 117 - AwardedAS Structure

The following is an XML example:

<AwardedAS>

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

- <endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String.1</mRID>

<marketType>DAM</marketType>

<resource>ACMEUnit3</resource>

<asType>REG-UP</asType>

<dispatchMW>100</dispatchMW>

<selfSchedMW>80</selfSchedMW>

<mcpc>13.55</mcpc>

</AwardedAS>

### CRR Awards

The purpose of this message is to identify CRR Awards to a QSE. The structure of this message is shown by the following diagram.

- Figure 118 - AwardedCRR Structure

The following is an XML example:

<AwardedCRR>

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

- <endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String.1</mRID>

<awardedMW>100</awardedMW>

<source>ABC</source>

<sink>XYZ</sink>

<price>12.56</price>

<crrId>11243</crrId>

</AwardedCRR>

### PTP Obligation Awards

The purpose of this message is to identify PTP Obligation Awards to a QSE. The structure of this message is shown by the following diagram.

- Figure 119 - AwardedPTPObligation Structure

The following is an XML example:

<AwardedPTPObligation>

<qse>ACME</qse>

<startTime>2008-01-01T00:00:00-06:00</startTime>

- <endTime>2008-01-01T01:00:00-06:00</endTime>

<tradingDate>2008-01-01</tradingDate>

<mRID>MRID-String</mRID>

<marketType>String</marketType>

<awardedMW>0</awardedMW>

<source>SP23</source>

<sink>SP4</sink>

<price>23.76</price>

<bidId>1</bidId>

</AwardedPTPObligation>

### Ancillary Service Obligations

The purpose of this notification message is to identify Ancillary Service Obligations to a QSE. The structure of this message is shown by the following diagram.

- Figure 120 – Ancillary Service Obligation Structure

### Outage Notifications

The purpose of this notification is to notify market participants of created, changed or canceled outages. The following high-level is structure used to convey outage notifications. The detailed structure is defined in section 7.2.1.

- Figure 121 - Outage Structure

### Startup/Shutdown Instructions

The purpose of this notification is to notify market participants of startup and shutdown instructions. The following structure is used to convey these notifications.

- Figure 122 - StartupShutdownInstructions Structure

### Wind Generation

The purpose of this interface is to provide a notification for Wind Generation Resource Production Potential (WGRPP).

- Figure 123 - Wind Generation Resource Production Potential Structure

# Acknowledgement of Alerts ***

The purpose of this service is to provide the means for a Market Participant to acknowledge Alert messages. Alert messages are sent as notification messages, as described in section 5.

## Interfaces Provided

There is a single operation provided to acknowledge Alert messages.

## Interfaces Required

The messages to acknowledge alerts would use the following message fields, where the ‘close’ verb is used to signify acknowledgement of the Alert.:

| Message Element | Value |
| --- | --- |
| Header/Verb | close |
| Header/Noun | Alert |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | ID of notification |
| Payload | No payload required |

Figure 124 - Alert Acknowledgement Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Alert |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload | None |

Figure 125 - Alert Acknowledgement Response

## Message Specifications

There is no payload used for either request or response messages in conjunction with the acknowledgement of alerts. The ID of the Alert is specified in the Request package within the message.

# Outage Scheduling Interfaces ***

The purpose of this section is to describe interfaces related to the creation, update, query and cancellation of outages. These interfaces would be used by QSEs and TPs, as needed to support integration with their system.

## Interfaces Provided

Interfaces are provided for the creation, update, query and cancellation of outages by QSEs and TPs. The interfaces use a basic request/reply interaction pattern. The payload used for these interfaces is an ‘OutageSet’, which is a container that may contain zero, one or more ‘Outage’ objects.

- Figure 126 - Outage Scheduling Interface Sequence Diagram

- Outages can be submitted by TP for transmission elements, or by QSEs for resources.

There are also notifications sent to MPs when outages are created or changed. These are described in section 5.y.on elements, or by QSEs for resources. be returned.ustomers.t participant for a given tr

## Message Specifications

### Outage Creation

The request message for outage creation or update would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | create |
| Header/Noun | OutageSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload/Outage | OutageSet |

- Figure 127 - Outage Creation Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | OutageSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/Outage | OutageSet |

- Figure 128 - Outage Creation Response

The structure of the OutageSet payload is described by the following diagram. This leverages the structure of an Outage defined later in this section.

- Figure 129 - OutageSet Structure

The high-level structure of an Outage record is described by the following diagram:

- Figure 130 - High-level Outage Structure

- Figure 131 - Outage Information

Within the OutageInfo substructure, the outage type, state and nature of work are identified. Valid outage type codes include:

- FR  - Forced Outage

- M1 - Maintenance Level 1

- M2 - Maintenance Level 2

- M3 - Maintenance Level 3

- OP - Planned Outage Non-Reliability with Opportunity

- PL - Planned Outage Reliability

- RS - Forced Outage with Remedial Switching Action

- SM -  Simple Transmission Outage

- UE - Forced Outage with Unavoidable Extension

- FE - Forced Outage with Forced Extension

The valid state codes include:

- ENYS

- Recvd

- Study

- Rejctd

- Accptd

- Active

- FExt

- UExt

- Withdr

- Cancld

- PComp

- ExpOpp

- CompEd

- CompNE

The legal codes that are currently defined  for nature of work as related to transmission equipment outages include:

- LM: line maintenance

- LR: line rebuild/upgrade

- BM: breaker maintenance

- BR: breaker repair/replace

- TM: transformer maintenance

- TR: transformer repair/replace

- TT: transformer testing

- SM: switch maintenance

- SR: switch repair/replace

- DR: repair/replace damaged equipment

- AO: associate equipment outage

- RW: relay work

- RE: replace equipment

- HS: repair hot spot

- UM: unknown

- OT: other

The codes for nature of work as related to resource outages include:

- IP: inlet air problems/repairs

- BS: black start test

- BO: boroscope

- CI: combustion inspection and repair

- EX: exhaust problems/repair

- FW: feedwater problems/repairs

- GW: gas company pipeline work

- FL: fuel limitation/lack of fuel

- LS: lack of steam load availability

- OV: overhaul

- TW: steam turbine limited/unavailable due to CT/GT work

- TL: tube leak

- VR: turbine/generator vibration/repair

- MO: mothballed

- UN: unknown

- OT: other

- Figure 132 - Outage Grouping, Details, Schedule, Contacts, Notes, and Opportunity

As should by the sub-structures, details are provided to reflect a resource outage or an equipment outage. For each resource, a name and resource type must be supplied. The resource is identified by its short name, and the resource type is one of ‘RMR’, ‘Blackstart’ or ‘Non-Reliability’. For each equipment, a name and equipment type must be supplied. The equipment is identified by its short name and the equipment type is one of ‘LN’, ‘BR’, ‘BS’, ‘GN’, ‘TR’.

- Notes provided may be types as ‘General’, ‘Supporting’ or ‘RASPS’.

On submission, the following items are required for an Outage creation::

| Element | Req? | Datatype | Description | Values |
| --- | --- | --- | --- | --- |
| mRID | N | string | Outage ID Assigned by OS | Assigned by OS |
| Command | N | string | Requestor Update | UNGROUP, ACKNOWLEDGE, CHANGE_STATUS |
| OutageInfo/outageId | Y | string | Unique identifier for an outage. This is for all Outages and not specifically for Opportunity Outages. |  |
| OutageInfo/outageType | Y | string | Type of Outage | FR,M1,M2,M3,OP,PL, RS,SM,UE,FE |
| OutageInfo/participant | N | string | Outage Participant |  |
| OutageInfo/versionId | N | string | It is automatically generated by the system, it is used for keeping an audit trail of all changes made to an Outage. |  |
| OutageInfo/state | N | string | The State of the Outage, taken from ENYS, Recvd, Study, Rejct, Accpt, Apprv, Active, FExt, UExt, Withd, Cancl, ExpOp, CompEd, CompNE, PComp | ENSYS,Recvd,Study,Rejctd Accptd,Active,FExt,UExt,Withdr Cancld,PComp,ExpOpp,CompEd CompNE |
| OutageInfo/status | N | string | The Status of the Outage, taken from ENYS, RetE, Study, Rejct, Apprv, Accpt, Withd, Cancl, ExpOpp. | ENYS,RatE,Accpt,Apprv,Rejct, Study,Withd,Cancl,Pendg,Compl |
| OutageInfo/previousState | N | string | The previous value of the Outage State, used by the State Machine. | ENSYS,Recvd,Study,Rejctd Accptd,Active,FExt,UExt,Withdr Cancld,PComp,ExpOpp,CompEd CompNE |
| OutageInfo/previousStatus | N | string | The previous value of the Outage Status, used by the State Machine. | ENYS,RatE,Accpt,Apprv,Rejct, Study,Withd,Cancl,Pendg,Compl |
| OutageInfo/requestedStatus | N | string | The requested value of the Outage State, used by the State Machine. | ENYS,RatE,Accpt,Apprv,Rejct, Study,Withd,Cancl,Pendg,Compl |
| OutageInfo/lastModified | N | dateTime | When Created or last updated | Last Modified gives us a date as to when, Last Modified By tells us who. Both are needed in the OS. The contact information will only tell you who submitted the outage. There will be several users at each company with there own certificate. We need to know who the last person was that modified the outage and when. |
| OutageInfo/lastModifiedBy | N | string | Creator or responsible for last update |  |
| OutageInfo/warningFlag | N | Boolean | Warning Exists | Y, N |
| OutageInfo/warningAcknowledge | N | Boolean | Warning Acknowledge | Y,N |
| OutageInfo/warningMessage | N | string | Warnings are part of the xml response. The message indicates the issue to be resolved, they are fixed messages with integrated variables for customizing the message for the Outage. | The free-form text of the message. |
| OutageInfo/Requestor/name | Y | string | Contact name |  |
| OutageInfo/Requestor/primaryContact | Y | string | Primary Contact phone no. |  |
| OutageInfo/Requestor/SecondaryContact | Y | string | Secondary Contact Phone No. |  |
| OutageInfo/Requestor/tertiaryContact | N | string | Tertiary Contact Phone No. |  |
| OutageInfo/Requestor/role | Y | string | Creator Role |  |
| OutageInfo/requestDate | N | date | Request Date |  |
| Group/groupId | N | string | This is the ability to group outages. A group outage has a Group ID. Each outage is still a separate outage with an outage ID as well. The Requestor can add to a group when creating the outage request. Once the outages have been submitted they can be removed from the group, creating an independent outage. Once the group has been submitted the requestor can’t add to it. The grouping will be used in the EMS-Outage Evaluation tool. Outage grouping name | An Outage may be a member of an Outage Group. An Outage Group must have at least one and can have more than one Outage. |
| Group/name | Y | string |  |  |
| Group/ResourceOutage/operatingCompany | Y | string | Group Resource Outage Operating Company Name |  |
| Group/ResourceOutage/station | Y | string | Group Resource Outage Station Name |  |
| Group/ResourceOutage/Resource/resource | Y | string | This is the name of a Resource that is a Designated Resource for this Opportunity Outage. |  |
| Group/ResourceOutage/Resource/resourceType | Y | string | This is the type of a Resource that is a Designated Resource for this Opportunity Outage |  |
| Group/ResourceOutage/HSL | Y | string | Group Outage Resource HSL |  |
| Group/ResourceOutage/LSL | Y | string | Group Outage Resource LSL |  |
| Group/ResourceOutage/OutageIdent | N | string | Unique identifier for an outage. |  |
| Group/ResourceOutage/natureOfWork | Y | string | This is a description of the nature of the work to be performed during the Outage. The allowable values for this field are selectable from a configurable finite list. |  |
| Group/EquipmentOutage/OperatingCompany | Y | string |  |  |
| Group/EquipmentOutage/Equipment/equipmentName | Y | string |  |  |
| Group/EquipmentOutage/Equipment/equipmentStationName | Y | string |  |  |
| Group/EquipmentOutage/Equipment/equipmentType | Y | string | This is the type of Equipment subject to removal from service as a consequence of the Outage: e.g. CB, LINE, etc. |  |
| Group/EquipmentOutage/fromStation | Y | string | The acronym of the name of the “From” Station in which an Equipment resides for all lines. |  |
| Group/EquipmentOutage/toStation | Y | string | The acronym of the name of the “To” Station in which an Equipment resides for all lines. |  |
| Group/EquipmentOutage/normalState | Y | string | Equipment Normal State |  |
| Group/EquipmentOutage/outageState | Y | string | Equipment Outage State |  |
| Group/EquipmentOutage/voltage | Y | Decimal | Equipment Outage Voltage |  |
| Group/EquipmentOutage/NOMCR | N | string | Equipment Outage NOMCR |  |
| Group/EquipmentOutage/emergencyRestorationTime | Y | string | Eq. Outage Emer Restoration Time |  |
| Group/EquipmentOutage/outageident | N | string | Outage Identity |  |
| Group/EquipmentOutage/natureOfWork | Y | string | Eq. Outage Nature of Work |  |
| ResourceOutage/operatingCompany | Y | string |  |  |
| ResourceOutage/station | Y | string | This is the station name of a Resource that is a Designated Resource for this Outage. |  |
| ResourceOutage/Resource/resource | Y | string | This is the name of a Resource that is a Designated Resource for this Outage. |  |
| ResourceOutage/Resource/resourceType | Y | string | This is the type of a Resource that is a Designated Resource for this Outage. |  |
| ResourceOutage/HSL | Y | string | Resource Outage HSL |  |
| ResourceOutage/LSL | Y | string | Resource Outage LSL |  |
| ResourceOutage/outageIdent | N | string | Outage Identity |  |
| ResourceOutage/natureOfWork | Y | string | Resource Outage Nature of Work |  |
| EquipmentOutage/operatingCompany | Y | string |  |  |
| EquipmentOutage/Equipment/equipmentName | Y | string | The name of the Equipment that can be the subject of an Outage. |  |
| EquipmentOutage/Equipment/equipmentStationName | Y | string | The name of the Equipment Station that can be the subject of an Outage. |  |
| EquipmentOutage/Equipment/equipmentType | Y | string | The Type of the Equipment that can be the subject of an Outage. |  |
| EquipmentOutage/fromStation | Y | string | This is the name of the Station in which an Equipment resides for all equipment types other than lines. For lines this refers to the “From” station |  |
| EquipmentOutage/toStation | Y | string | This is the name of the “To” Station in which an Equipment resides for all lines. |  |
| EquipmentOutage/normalState | Y | string | This field, which is pertinent only for switching devices, contains the normal state of the device: OPEN or CLOSED. This field is the same as the Normal State of the Equipment entity. |  |
| EquipmentOutage/outageState | Y | string | This field, which is pertinent only for switching devices, contains the state of the device when taken out of service: OPEN or CLOSED. This field is the opposite of the Normal State of the Equipment entity. |  |
| EquipmentOutage/voltage | Y | Decimal | The voltage level at which the Equipment resides. For transformers it is the high side voltage level. |  |
| EquipmentOutage/NOMCR | N | String |  |  |
| EquipmentOutage/emergencyRestorationTime | Y | dateTime | This is the time in hours necessary to terminate the Outage and return the Equipment to service. |  |
| EquipmentOutage/natureOfWork | Y | string |  |  |
| EquipmentOutage/outageIdent | N | string | Outage Identity |  |
| Schedule/plannedStart | Y | dateTime | This is the date/time at which the Outage is planned to start. |  |
| Schedule/plannedEnd | Y | dateTime | This is the date/time at which the Outage is planned to end. |  |
| Schedule/earliestStart | N | dateTime | This is the earliest date/time at which the Outage may start. |  |
| Schedule/latestEnd | N | dateTime | This is the latest date/time at which the Outage may end. |  |
| Schedule/actualStart | N | dateTime | This is the actual date/time at which the Outage started. |  |
| Schedule/actualEnd | N | dateTime | This is the actual date/time at which the Outage ended. |  |
| Schedule/new_plannedStart | N | dateTime | This is the proposed date/time at which an Unavoidable Extension may start. |  |
| Schedule/new_plannedEnd | N | dateTime | This is the proposed date/time at which an Unavoidable Extension may end. |  |
| Schedule/new_earliestStart | N | dateTime | This is the proposed earliest date/time at which a Resource Opportunity Outage may start. |  |
| Schedule/new_latestEnd | N | dateTime | This is the proposed latest date/time at which a Resource Opportunity Outage may end. |  |
| Recurrance/datesList/dates/outageident | N | String | Outage Identity |  |
| Recurrance/datesList/dates/plannedStart | Y | Date | Planned start date of outage recurrence |  |
| Recurrance/datesList/dates/plannedEnd | Y | Date | Planned End date of outage recurrence |  |
| Recurrance/datesList/dates/earliestStart | Y | Date | Earliest start date of outage recurrence |  |
| Recurrance/datesList/dates/latestEnd | Y | Date | Latest Planned End date of outage recurrence |  |
| Opportunity/start | Y | dateTime | For Opportunity Outages, the outage Start Time. |  |
| Opportunity/end | Y | dateTime | For Opportunity Outages, the outage End Time. |  |
| Opportunity/equipment/equipmentName | Y | string | This is the Identifier of an Opportunity Outage that has one or more Designated Resources. |  |
| Opportunity/equipment/equipmentStationName | Y | string | This is the Equipment Station Name of an Opportunity Outage hat has one or more Designated Resources. |  |
| Opportunity/equipment/equipmentType | Y | string | This is the Equipment type of an Opportunity Outage hat has one or more Designated Resources. |  |
| Notes/RequestorNotes/Note/lastModified | N | string | There are three sections that the requestor can enter notes, Requestor Notes, Supporting Information and Remedial Action or Special Protection System notes. Notes are required for some outage types but not all. They are not required when the outage is submitted but are required for some outages to be completed. There is another section for reviewer notes. This is for ERCOT to enter notes and the MP can only read them. |  |
| Notes/RequestorNotes/Note/createdBy | N | string |  |  |
| Notes/RequestorNotes/Note/company | N | string |  |  |
| Notes/RequestorNotes/Note/comment | N | string |  |  |
| Notes/ReviewerNotes/Note/lastModified | N | string |  |  |
| Notes/ReviewerNotes/Note/createdBy | N | string |  |  |
| Notes/ReviewerNotes/Note/company | N | string |  |  |
| Notes/ReviewerNotes/Note/comment | N | string |  |  |
| Notes/SupportingNotes/Note/lastModified | N | string |  |  |
| Notes/SupportingNotes/Note/createdBy | N | string |  |  |
| Notes/SupportingNotes/Note/company | N | string |  |  |
| Notes/SupportingNotes/Note/comment | N | string |  |  |
| Notes/RASPSNotes/Note/lastModified | N | string |  |  |
| Notes/RASPSNotes/Note/createdBy | N | string |  |  |
| Notes/RASPSNotes/Note/company | N | string |  |  |
| Notes/RASPSNotes/Note/comment | N | string |  |  |
| Error/severity | N | string | Severity of Error Generated |  |
| Error/area | N | string | Area of generated error |  |
| Error/interval | N | string |  |  |
| Error/text | Y | string | Error Text |  |

- Figure 133 – Outage Creation Elements

### Outage Query

When getting Outages, an OutageSet container class is used to return zero or more outages that match the query criteria. The request message for querying outages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | OutageSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/StartTime | Optional: start time of interest |
| Request/EndTime | Optional: end time of interest |
| Request/ID | Optional: OutageID:outage id of interest GroupID:group id of interest |
| Request/GroupID | Optional: group ID for the outages of interest |

- Figure 134 - OutageSet Query Request

If startTime and endTime are not specified, only active outages will be returned. The mRID can be used to request details of a specific outage. The GroupID can be used to request outages that are part of a group.

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | OutageSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/OutageSet | Set of outages |

- Figure 135 - OutageSet Query Response

### Outage Update

The request message for outage update would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | Change |
| Header/Noun | OutageSet |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload/Outage | OutageSet |

- Figure 136 - Outage Update Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | OutageSet |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |

- Figure 137 - Outage Update Response

### Outage Cancellation

The request message for outage creation would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | cancel |
| Header/Noun | Outage |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/ID | mRID of outage to be canceled |

Figure 138 - Outage Cancel Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Outage |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK |
| Reply/Error | Error message, if error encountered |

- Figure 139 - Outage Cancel Response

# Utility Interfaces ***

The purpose of this section is to describe interfaces that provide utility services that are supplemental to the interfaces described in previous sections.

## Interfaces Provided

The interfaces provided include the following:

- The means to status check the ERCOT web services

- The means to generate an mRID from a set of key values

- Additional utility interfaces may be defined in the future as needed.

## Message Specifications

### Get mRID

The purpose of this interface is to provide the means to obtain an mRID value that corresponds to a given set of key values for a market product. The generated mRID values are structured according to the rules in section 2.3.4. Hours are not included in the generated mRID strings.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | mRID |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload/BidSet | A BidSet that provides the ‘key’ information required for a specific type of market product as needed to construct the desired mRID |

Figure 140 - Get mRID Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | mRID |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Reply/ID | mRID string |

Figure 141 - Get mRID Response

### System Status

The purpose of this interface is to provide the means to verify that a connection can be established with the ERCOT web services through a simple status check. The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | SystemStatus |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |

Figure 142 - SystemStatus Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | SystemStatus |
| Header/Source | ERCOT |
| Header/Revision | Identifies current version of interfaces |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Reply/Timestamp | Identifies current ERCOT local time |

Figure 143 - SystemStatus Response

### Request Test Notification

The purpose of this interface is to provide the means to request that a test notification be sent back to the MP. The payload provided on the request will then be used as the notification payload. The following table describes the request parameters:

| Message Element | Value |
| --- | --- |
| Header/Verb | create |
| Header/Noun | TestNotification |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Payload/any | Any message payload |

Figure 144 - TestNotification Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | TestNotification |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Reply/Timestamp | Identifies current ERCOT local time |

Figure 145 - TestNotification Response

### Change Active Notification URL

The purpose of this interface is to provide the means to request that ERCOT send notifications to the defined PRIMARY or BACKUP URL. This interface does NOT allow a specific URL to be specified. The following table describes the request parameters:

| Message Element | Value |
| --- | --- |
| Header/Verb | change |
| Header/Noun | ActiveNotificationURL |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/Option | PRIMARY or BACKUP |

Figure 146 – Change ActiveNotificationURL Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | ActiveNotificationURL |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Reply/Timestamp | Identifies current ERCOT local time |

Figure 147 – Change ActiveNotificationURL Response

# Reports ***

The purpose of this section is to describe interfaces that provide access to reports. This is a web service repackaging of the MID REST Service (PR 500007) that is currently used by Zonal.

## Interfaces Provided

The interfaces provided include the following:

- The means to request the URLs to a set of reports for a given operating date using web services

The actual download of the documents themselves would continue to be done using the URL that is returned for each report.

## Message Specifications

### Get Reports

The purpose of this interface is to provide the means to obtain a list of URLs for reports related to a specific operating date. The URLs can then be used for downloading the reports.

The request message would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | get |
| Header/Noun | Reports |
| Header/Source | Market participant ID |
| Header/UserID | Optional: ID of user |
| Request/operatingDate | The operating date of interest |

Figure 148 - Reports Request

The corresponding response messages would use the following message fields:

| Message Element | Value |
| --- | --- |
| Header/Verb | reply |
| Header/Noun | Reports |
| Header/Source | ERCOT |
| Reply/ReplyCode | Reply code, success=OK, error=ERROR |
| Reply/Error | Error message, if error encountered |
| Payload/Reports | List of Report structures, where each Report identifies a number of data items including the URL where the actual report may be downloaded |

Figure 149 - Reports Response

The following structure is used within the payload to convey a list of reports:

- Figure 150 - Reports Container

# Appendix A: WS-Notifications

The OASIS WS-Notifications specification can be obtained from http://www.oasis-open.org. The Notification interface to be used by ERCOT involves several modification to the WS-Notification standard. These include:

- The use of an Acknowledge message in order to improve reliability

- Reduction of the interface into a single XSD and a simplified WSDL

The key portions of WS-Notifications that are relevant to this specification are provided here for convenience. The following is the subset of the XML Schema for WS-Notifications that is relevant to this design.

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp2 (http://www.altova.com) by Scott Neumann (UISOL) -->

<!--

- This Schema represents a revision to the OASIS WS-Notitication standard for use by

ERCOT Market participants. This revision is intended to:

1. Reduce complexity by eliminating unused interfaces and references to external XSDs

2. Improve reliability through the addition of a positive acknowledgement message after

receipt of a notification message

- Copyright (C) OASIS Open (2004-2006). All Rights Reserved.

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works. However, this document itself may not be modified in any way, such as by removing the copyright notice or references to OASIS, except as needed for the purpose of developing OASIS specifications, in which case the procedures for copyrights defined in the OASIS Intellectual Property Rights document must be followed, or as required to translate it into languages other than English.

-->

<xsd:schema xmlns:wsnt="http://www.ercot.com/schema/2007-06/nodal/notification" xmlns:xsd="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/notification" elementFormDefault="qualified" attributeFormDefault="unqualified">

<!-- ================== Message Helper Types  ===================== -->

<xsd:complexType name="NotificationMessageHolderType">

<xsd:sequence>

<xsd:element name="Message">

<xsd:complexType>

<xsd:sequence>

<xsd:any namespace="##any"

processContents="lax"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

</xsd:sequence>

</xsd:complexType>

<xsd:element name="NotificationMessage"

type="wsnt:NotificationMessageHolderType"/>

<!-- ========== Message Types for NotificationConsumer  =========== -->

<xsd:element name="Notify">

<xsd:complexType>

<xsd:sequence>

<xsd:element ref="wsnt:NotificationMessage"

maxOccurs="unbounded"/>

<xsd:any namespace="##other" processContents="lax"

minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="Acknowledge">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="ReplyCode" type="xsd:string"

minOccurs="0"/>

<xsd:element name="Timestamp" type="xsd:dateTime"

minOccurs="0"/>

<xsd:any namespace="##other" processContents="lax"

minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="Fault">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="FaultCode" type="xsd:string"

minOccurs="0"/>

<xsd:element name="Timestamp" type="xsd:dateTime"

minOccurs="0"/>

<xsd:any namespace="##other" processContents="lax"

minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

</xsd:schema>

The following WSDL represents the portion of WS-Notifications that is used for the receipt of Notification messages.

<?xml version="1.0" encoding="utf-8"?>

<!--

- This Schema represents a revision to the OASIS WS-Notitication standard for use by

ERCOT Market participants. This revision is intended to:

1. Reduce complexity by eliminating unused interfaces and references to external XSDs

2. Improve reliability through the addition of a positive acknowledgement message after

receipt of a notification message

- Copyright (C) OASIS Open (2004-2006). All Rights Reserved.

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works. However, this document itself may not be modified in any way, such as by removing the copyright notice or references to OASIS, except as needed for the purpose of developing OASIS specifications, in which case the procedures for copyrights defined in the OASIS Intellectual Property Rights document must be followed, or as required to translate it into languages other than English.

-->

<wsdl:definitions name="WS-BaseNotification"

targetNamespace="http://www.ercot.com/wsdl/2007-06/nodal/notification"

xmlns:wsntw="http://www.ercot.com/wsdl/2007-06/nodal/notification"

xmlns:wsnt="http://www.ercot.com/schema/2007-06/nodal/notification"

xmlns:xsd="http://www.w3.org/2001/XMLSchema"

xmlns:wsdl="http://schemas.xmlsoap.org/wsdl/">

<!-- ===================== Types Definitions ====================== -->

<wsdl:types>

<xsd:schema>

<xsd:import

namespace="http://www.ercot.com/schema/2007-06/nodal/notification"

schemaLocation="Notification.xsd"/>

</xsd:schema>

</wsdl:types>

<!-- ================ NotificationConsumer::Notify ================

- Notify(

NotificationMessage

(SubscriptionReference, TopicExpression, ProducerReference,

Message)*

returns: Ack

-->

<wsdl:message name="Notify">

- <wsdl:part name="Notify" element="wsnt:Notify"/>

</wsdl:message>

<wsdl:message name="Acknowledge">

<wsdl:part name="Acknowledge" element="wsnt:Acknowledge"/>

</wsdl:message>

<wsdl:message name="Fault">

<wsdl:part name="Fault" element="wsnt:Fault"/>

</wsdl:message>

<!-- ========= NotificationConsumer PortType Definition =========== -->

- <wsdl:portType name="NotificationConsumer">

<wsdl:operation name="Notify">

<wsdl:input message="wsntw:Notify" />

<wsdl:output message="wsntw:Acknowledge" />

<wsdl:fault message="wsntw:Fault" name="fault1"/>

</wsdl:operation>

</wsdl:portType>

</wsdl:definitions>

# Appendix B: WSDL for Market Requests

This WSDL uses a set of operations for servicing all market requests, related to bidding information requests and alert acknowledgements.

<?xml version = "1.0" encoding = "UTF-8"?>

<wsdl:definitions attributeFormDefault = "unqualified" elementFormDefault = "qualified" targetNamespace = "http://www.ercot.com/wsdl/2007-05/nodal/ewsAbstract" xmlns:ns = "http://www.ercot.com/wsdl/2007-05/nodal/ewsAbstract" xmlns:ns2 = "http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:soap = "http://schemas.xmlsoap.org/wsdl/soap/" xmlns:wsdl = "http://schemas.xmlsoap.org/wsdl/" xmlns:xsd = "http://www.w3.org/2001/XMLSchema">

<wsdl:import location = "Message.xsd" namespace = "http://www.ercot.com/schema/2007-05/nodal/ews"/>

- <wsdl:types>

<xsd:schema targetNamespace = "http://www.ercot.com/wsdl/2007-05/nodal/ewsAbstract">

- <xsd:include schemaLocation = "Message.xsd"/>

</xsd:schema>

</wsdl:types>

<wsdl:message name = "RequestMessage">

<wsdl:part element = "ns2:RequestMessage" name = "Message"/>

</wsdl:message>

<wsdl:message name = "ResponseMessage">

<wsdl:part element = "ns2:ResponseMessage" name = "Message"/>

</wsdl:message>

<wsdl:message name = "FaultMessage">

<wsdl:part element = "ns2:FaultMessage" name = "part1"/>

</wsdl:message>

<wsdl:portType name = "Operations">

<wsdl:operation name = "MarketTransactions">

<wsdl:input message = "ns:RequestMessage"/>

<wsdl:output message = "ns:ResponseMessage"/>

<wsdl:fault message = "ns:FaultMessage" name = "fault1"/>

</wsdl:operation>

<wsdl:operation name = "MarketInfo">

<wsdl:input message = "ns:Request"/>

<wsdl:output message = "ns:Response"/>

<wsdl:fault message = "ns:FaultMessage" name = "fault1"/>

</wsdl:operation>

<wsdl:operation name = "Alerts">

<wsdl:input message = "ns:Request"/>

<wsdl:output message = "ns:Response"/>

<wsdl:fault message = "ns:FaultMessage" name = "fault1"/>

</wsdl:operation>

- </wsdl:portType>

<wsdl:binding name = "NodalSOAP" type = "ns:Operations">

<soap:binding style = "document" transport = "http://schemas.xmlsoap.org/soap/http"/>

- <wsdl:operation name = "MarketTransactions">

<soap:operation soapAction = "http://www.ercot.com/Nodal/MarketTransactions"/>

- <wsdl:input>

<soap:body use = "literal"/>

</wsdl:input>

<wsdl:output>

<soap:body use = "literal"/>

</wsdl:output>

<wsdl:fault name = "fault1">

<soap:body use = "literal"/>

- </wsdl:fault>

</wsdl:operation>

<wsdl:operation name = "MarketInfo">

<soap:operation soapAction = "http://www.ercot.com/Nodal/MarketInfo"/>

<wsdl:input>

<soap:body use = "literal"/>

</wsdl:input>

<wsdl:output>

<soap:body use = "literal"/>

</wsdl:output>

<wsdl:fault name = "fault1">

<soap:body use = "literal"/>

</wsdl:fault>

</wsdl:operation>

<wsdl:operation name = "Alerts">

<soap:operation soapAction = "http://www.ercot.com/Nodal/Alerts"/>

<wsdl:input>

<soap:body use = "literal"/>

</wsdl:input>

<wsdl:output>

<soap:body use = "literal"/>

</wsdl:output>

<wsdl:fault name = "fault1">

<soap:body use = "literal"/>

</wsdl:fault>

- </wsdl:operation>

</wsdl:binding>

<wsdl:service name = "Nodal">

<wsdl:port binding = "ns:NodalSOAP" name = "NodalSOAP">

<soap:address location = "http://www.ercot.com/Nodal/"/>

</wsdl:port>

- </wsdl:service>

</wsdl:definitions>

# Appendix C: XML Schemas for Message and Payload Definitions

The following diagram describes the relationships between the XML Schemas defined by this document.

The following XML schema is used to define Message structures for request and response messages for the WSDL defined in Appendix B. These are provided here for reference purposes, but versions should be downloaded from the ERCOT web site for development use.

<?xml version="1.0" encoding="UTF-8"?>

<!--   Message.xsd                                                                            -->

<!--                                                                                          -->

<!--    Release Date: September 2007                                                          -->

<!--    Ext Int Spec Version No. 1.05                                                         -->

<!--                                                                                          -->

<!--    9/12/2007 Nemat Version 0.2.1 Corrected the namespace: Added the missing /message     -->

<!--    10/3/2007 Scott Version 0.3   Changed from lax to skip                                -->

<!--    11/7/2007 Nemat Version 0.3.1  ReplyType ID maxOccurs="unbounded" qualification   -->

<xsd:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews/message" xmlns:xsd="http://www.w3.org/2001/XMLSchema" xmlns:wsse="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd" xmlns:wsu="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews/message" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.1">

<xsd:import namespace="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd" schemaLocation="WSS200401wssecurity-secext-10.xsd"/>

<xsd:import namespace="http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd" schemaLocation="WSS200401wssecurity-utility-10.xsd"/>

- <xsd:complexType name="RequestType">

<xsd:sequence>

- <xsd:element name="MarketType" minOccurs="0">

<xsd:simpleType>

- <xsd:restriction base="xsd:string">

<xsd:enumeration value="DAM"/>

- <xsd:enumeration value="ADJ"/>

<xsd:enumeration value="RTM"/>

- </xsd:restriction>

</xsd:simpleType>

- </xsd:element>

<xsd:element name="OperatingDate" type="xsd:dateTime" minOccurs="0"/>

- <xsd:element name="TradingDate" type="xsd:date" minOccurs="0"/>

<xsd:element name="StartTime" type="xsd:dateTime" minOccurs="0"/>

- <xsd:element name="EndTime" type="xsd:dateTime" minOccurs="0"/>

<xsd:element name="Zone" type="xsd:string" minOccurs="0"/>

- <xsd:element name="ASType" type="xsd:string" minOccurs="0"/>

<xsd:element name="Option" type="xsd:string" minOccurs="0"/>

- <xsd:element name="SortBy" type="xsd:string" minOccurs="0"/>

<xsd:element name="ID" type="xsd:string" minOccurs="0" maxOccurs="unbounded"/>

- <xsd:any namespace="##other" minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="ReplyType">

<xsd:sequence>

<xsd:element name="ReplyCode" type="xsd:string"/>

<xsd:element name="Error" type="xsd:string" minOccurs="0" maxOccurs="unbounded"/>

<xsd:element name="Timestamp" type="xsd:dateTime" minOccurs="0"/>

<xsd:element name="ID" type="xsd:string" minOccurs="0" maxOccurs="unbounded"/>

<xsd:any namespace="##other" minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="PayloadType">

<xsd:sequence>

<xsd:choice>

<xsd:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/>

<xsd:element name="Document" type="xsd:string" minOccurs="0" maxOccurs="unbounded"/>

<xsd:element name="Compressed" type="xsd:string" minOccurs="0"/>

</xsd:choice>

<xsd:element name="format" type="xsd:string" minOccurs="0"/>

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="ReplayDetectionType">

<xsd:sequence>

<xsd:element name="Nonce" type="wsse:EncodedString"/>

<xsd:element name="Created" type="wsu:AttributedDateTime"/>

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="HeaderType">

<xsd:sequence>

<xsd:element name="Verb" default="get">

<xsd:simpleType>

<xsd:restriction base="xsd:string">

<xsd:enumeration value="cancel"/>

<xsd:enumeration value="canceled"/>

<xsd:enumeration value="change"/>

<xsd:enumeration value="changed"/>

<xsd:enumeration value="create"/>

<xsd:enumeration value="created"/>

<xsd:enumeration value="close"/>

<xsd:enumeration value="closed"/>

<xsd:enumeration value="delete"/>

<xsd:enumeration value="deleted"/>

<xsd:enumeration value="get"/>

<xsd:enumeration value="reply"/>

<xsd:enumeration value="submit"/>

<xsd:enumeration value="update"/>

<xsd:enumeration value="updated"/>

</xsd:restriction>

</xsd:simpleType>

</xsd:element>

<xsd:element name="Noun" type="xsd:string"/>

<xsd:element name="ReplayDetection" type="ReplayDetectionType"/>

<xsd:element name="Revision" type="xsd:string" default="001"/>

<xsd:element name="Source" type="xsd:string"/>

<xsd:element name="UserID" type="xsd:string" minOccurs="0"/>

<xsd:element name="MessageID" type="xsd:string" minOccurs="0"/>

<xsd:element name="Comment" type="xsd:string" minOccurs="0"/>

<xsd:any namespace="##other" minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

<xsd:element name="Message">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="Header" type="HeaderType"/>

<xsd:choice>

<xsd:element name="Request" type="RequestType" minOccurs="0"/>

<xsd:element name="Reply" type="ReplyType" minOccurs="0"/>

</xsd:choice>

<xsd:element name="Payload" type="PayloadType" minOccurs="0"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="RequestMessage">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="Header" type="HeaderType"/>

<xsd:element name="Request" type="RequestType" minOccurs="0"/>

<xsd:element name="Payload" type="PayloadType" minOccurs="0"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="ResponseMessage">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="Header" type="HeaderType"/>

<xsd:element name="Reply" type="ReplyType" minOccurs="0"/>

<xsd:element name="Payload" type="PayloadType" minOccurs="0"/>

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:element name="FaultMessage">

<xsd:complexType>

<xsd:sequence>

<xsd:element name="Reply" type="ReplyType" minOccurs="0"/>

</xsd:sequence>

</xsd:complexType>

- </xsd:element>

</xsd:schema>

The following XML schemas provide the definition of structures for payload definitions. The focus of the examples provided here are for bids, offers and ERCOT notifications.

- ErcotCommonTypes.xsd

<?xml version="1.0" encoding="UTF-8"?>

<!--   ErcotCommonTypes.xsd                                                                                     -->

<!--                                                                                                            -->

<!--    Release Date: September 2007                                                                            -->

<!--    Ext Int Spec Version No. 1.05                                                                           -->

<!--                                                                                                            -->

<!--    10/3/2007 Scott   Corrected enum for operating mode OUTL                                                  -->

<!--    01/22/2008 0.3.0 Nemat Added RRSLD to ASType enumerations.                    -->

<!--    02/29/2009 0.3.1 Nemat Added type=Float to ratingValue , Comment by Austin Energy  -->

<!--                                        Added type=String to name in IdentifiedObject, , Comment by Austin Energy  -->

<!--                                                                                                            -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.1">

- <!--ERCOT Common Types: ErcotCommonTypes.xsd-->

<!--The type definitions in this XSD are potentially used across many interfaces-->

<!---->

<!--Simple Type Definitions-->

<xs:simpleType name="SettlementPoint">

<xs:annotation>

<xs:documentation>Settlement point, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="ElectricalBus">

<xs:annotation>

<xs:documentation>Electrical bus, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="URL">

<xs:annotation>

<xs:documentation>URL</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:anyURI"/>

</xs:simpleType>

<xs:simpleType name="QSE">

<xs:annotation>

<xs:documentation>QSE type, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="ASType">

<xs:annotation>

<xs:documentation>Ancillary services type</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="Reg-Up"/>

<xs:enumeration value="Reg-Down"/>

<xs:enumeration value="Non-Spin"/>

<xs:enumeration value="RRS"/>

<xs:enumeration value="REGUP-RRS-ONNS"/>

<xs:enumeration value="Off-Non-Spin"/>

<xs:enumeration value="On-Non-Spin"/>

<xs:enumeration value="RRSLD"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="BlockType">

<xs:annotation>

<xs:documentation>Block types</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="FIXED"/>

<xs:enumeration value="VARIABLE"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="RUCType">

<xs:annotation>

<xs:documentation>RUC type</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="DRUC"/>

<xs:enumeration value="HRUC"/>

<xs:enumeration value="DAM"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="OperatingMode">

<xs:annotation>

<xs:documentation>Operating mode type</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="ONRUC"/>

<xs:enumeration value="ON"/>

<xs:enumeration value="ONREG"/>

<xs:enumeration value="ONTEST"/>

<xs:enumeration value="ONOS"/>

<xs:enumeration value="ONOSREG"/>

<xs:enumeration value="ONDSRREG"/>

<xs:enumeration value="ONDSR"/>

<xs:enumeration value="OFF"/>

<xs:enumeration value="OFFNS"/>

<xs:enumeration value="ONEMR"/>

<xs:enumeration value="ONRR"/>

<xs:enumeration value="OUT"/>

<xs:enumeration value="EMR"/>

<xs:enumeration value="ONRGL"/>

<xs:enumeration value="ONRRCLR"/>

<xs:enumeration value="ONRL"/>

<xs:enumeration value="OUTL"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Resource">

<xs:annotation>

<xs:documentation>Resource, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="CombinedCycle">

<xs:annotation>

<xs:documentation>Combined cycle configuration</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="Equipment">

<xs:annotation>

<xs:documentation>Equipment, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="Hub">

<xs:annotation>

<xs:documentation>Hub, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="Zone">

<xs:annotation>

<xs:documentation>Zone, identified by name</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="EnergyConsumer">

<xs:annotation>

<xs:documentation>EnergyConsumer</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="DayType">

<xs:annotation>

<xs:documentation>Day type</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:simpleType name="TransactionStatusType">

<xs:annotation>

<xs:documentation>Transaction status type for bids, offers, trades and schedules</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="SUBMITTED"/>

<xs:enumeration value="ACCEPTED"/>

<xs:enumeration value="PENDING"/>

<xs:enumeration value="REJECTED"/>

<xs:enumeration value="ERRORS"/>

<xs:enumeration value="UNCONFIRMED"/>

<xs:enumeration value="CANCELED"/>

</xs:restriction>

</xs:simpleType>

<!--Types for bidding-->

<xs:complexType name="CapacitySchedule">

<xs:annotation>

<xs:documentation>Capacity Schedule: value1Unit is MW</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="PriceSchedule">

<xs:annotation>

<xs:documentation>Price Schedules: value1Unit is $/MW</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="EnergySchedule">

<xs:annotation>

<xs:documentation>MW Schedules: value1Unit is MW</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="Error">

<xs:annotation>

<xs:documentation>Error structure</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="severity" minOccurs="0">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="ERROR"/>

<xs:enumeration value="WARNING"/>

<xs:enumeration value="INFORMATIVE"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="area" type="xs:string" minOccurs="0"/>

<xs:element name="interval" type="xs:string" minOccurs="0"/>

<xs:element name="text" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="BidPriceCurve">

<xs:annotation>

<xs:documentation>Relationship between a price in $/hour (Y-axis) and a MW value (X-axis). </xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime">

<xs:annotation>

<xs:documentation>Start time to identify when curve is used</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="endTime" type="xs:dateTime">

<xs:annotation>

<xs:documentation>End time to indicate when curve is no longer used</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="curveStyle" minOccurs="0">

<xs:annotation>

<xs:documentation>Style of curve: FIXED, VARIABLE, CURVE</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="FIXED"/>

<xs:enumeration value="VARIABLE"/>

<xs:enumeration value="CURVE"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="CurveData" maxOccurs="10">

<xs:complexType>

<xs:annotation>

<xs:documentation>The point data values that define a curve</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="xvalue" type="xs:float">

<xs:annotation>

<xs:documentation>The data value of the X-axis variable,  depending on the X-axis units</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="y1value" type="xs:float">

<xs:annotation>

<xs:documentation>The data value of the  first Y-axis variable, depending on the Y-axis units</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="incExcFlag" minOccurs="0">

<xs:annotation>

<xs:documentation>Inclusive/Exclusive flag for three part offers to identify whether ancillary services are included or excluded</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="INC"/>

<xs:enumeration value="EXC"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="reason" minOccurs="0">

<xs:annotation>

<xs:documentation>Reason for adjustment</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="OUT"/>

<xs:enumeration value="FUEL"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PriceCurve">

<xs:annotation>

<xs:documentation>Relationship between a price in $/hour (Y-axis) and a MW value (X-axis). </xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime">

<xs:annotation>

<xs:documentation>Start time to identify when curve is used</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="endTime" type="xs:dateTime">

<xs:annotation>

<xs:documentation>End time to indicate when curve is no longer used</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="curveStyle" minOccurs="0">

<xs:annotation>

<xs:documentation>Style of curve: FIXED, VARIABLE, CURVE</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="FIXED"/>

<xs:enumeration value="VARIABLE"/>

<xs:enumeration value="CURVE"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="CurveData" maxOccurs="10">

<xs:complexType>

<xs:annotation>

<xs:documentation>The point data values that define a curve</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="xvalue" type="xs:float">

<xs:annotation>

<xs:documentation>The data value of the X-axis variable,  depending on the X-axis units</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="y1value" type="xs:float">

<xs:annotation>

<xs:documentation>The data value of the  first Y-axis variable, depending on the Y-axis units</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="incExcFlag" minOccurs="0">

<xs:annotation>

<xs:documentation>Inclusive/Exclusive flag for three part offers to identify whether ancillary services are included or excluded</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="INC"/>

<xs:enumeration value="EXC"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="reason" minOccurs="0">

<xs:annotation>

<xs:documentation>Reason for adjustment</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="OUT"/>

<xs:enumeration value="FUEL"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ASPriceCurve">

<xs:annotation>

<xs:documentation>Defines curves for AS offers</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime">

<xs:annotation>

<xs:documentation>Start time to identify when curve is used</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="endTime" type="xs:dateTime">

<xs:annotation>

<xs:documentation>End time to indicate when curve is no longer used</xs:documentation>

</xs:annotation>

</xs:element>

<xs:choice>

<xs:element name="OnLineReserves" maxOccurs="5">

<xs:complexType>

<xs:sequence>

<xs:element name="xvalue" type="xs:float">

<xs:annotation>

<xs:documentation>MW quantity</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="REGUP" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>Reg-Up price</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="RRS" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>RRS price</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ONNS" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>Online Non-Spin price</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="block" type="BlockType">

<xs:annotation>

<xs:documentation>block type</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="RegDown" maxOccurs="5">

<xs:complexType>

<xs:sequence>

<xs:element name="xvalue" type="xs:float">

<xs:annotation>

<xs:documentation>MW quantity</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="REGDN" type="xs:float">

<xs:annotation>

<xs:documentation>Reg-Down price</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="block" type="BlockType">

<xs:annotation>

<xs:documentation>block type</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="OffLineNonSpin" maxOccurs="5">

<xs:complexType>

<xs:sequence>

<xs:element name="xvalue" type="xs:float">

<xs:annotation>

<xs:documentation>MW quantity</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="OFFNS" type="xs:float">

<xs:annotation>

<xs:documentation>Offline Non-Spin price</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="block" type="BlockType">

<xs:annotation>

<xs:documentation>block type</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:choice>

<xs:element name="multiHourBlock" type="xs:boolean" default="false" minOccurs="0">

<xs:annotation>

<xs:documentation>Must be taken as a block for all hours</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Rating">

<xs:annotation>

<xs:documentation>Rating</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="ratingType">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="Normal"/>

<xs:enumeration value="2-hour"/>

<xs:enumeration value="15-min"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="ratingValue" type="xs:float"/>

<xs:element name="units" minOccurs="0">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="MVA"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:complexType>

<!--CIM type definitions-->

<xs:complexType name="IdentifiedObject">

<xs:annotation>

<xs:documentation>CIM IdentifiedObject superclass</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="name" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PowerSystemResource">

<xs:annotation>

<xs:documentation>CIM PSR superclass</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="IdentifiedObject"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="ConductingEquipment">

<xs:annotation>

<xs:documentation>CIM Conducting Equipment</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="PowerSystemResource"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="IntervalSchedule">

<xs:annotation>

<xs:documentation>IntervalSchedule is used to schedule values for give time intervals</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="operatingDate" type="xs:date">

<xs:annotation>

<xs:documentation>Operating date</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="IntervalData" type="IntervalData" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="IntervalData">

<xs:annotation>

<xs:documentation>IntervalData defines a set of values for one or more intervals</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startInterval" type="xs:time">

<xs:annotation>

<xs:documentation>Starting interval, identified by end time in format HH:MM</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="endInterval" type="xs:time">

<xs:annotation>

<xs:documentation>Ending interval, identified by end time in format HH:MM</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value1" type="xs:integer">

<xs:annotation>

<xs:documentation>Data value 1</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value2" type="xs:integer" minOccurs="0">

<xs:annotation>

<xs:documentation>Data value 2</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

<xs:attribute name="repeatedHour" type="xs:boolean" default="false">

<xs:annotation>

<xs:documentation>Flag to indicate change from CDT->CST</xs:documentation>

</xs:annotation>

</xs:attribute>

</xs:complexType>

<xs:complexType name="TmSchedule">

<xs:annotation>

<xs:documentation>Time schedule, with a set of values ordered by absolute time within the startTime and endTime</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="BasicIntervalSchedule">

<xs:sequence>

<xs:element name="TmPoint" type="TmPoint" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="TmPoint">

<xs:annotation>

<xs:documentation>Timepoints for TmSchedule</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="time" type="xs:dateTime">

<xs:annotation>

<xs:documentation>Absolute time start time for the data values</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ending" type="xs:dateTime" minOccurs="0">

<xs:annotation>

<xs:documentation>Ending time for values, if not  beginning of next TmPoint or end of TmSchedule</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value1" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>Data value 1</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value2" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>Data value 2</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="value3" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>Data value 3</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="BasicIntervalSchedule">

<xs:annotation>

<xs:documentation>CIM Basic Interval Schedule</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="endTime" type="xs:dateTime" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Curve">

<xs:annotation>

<xs:documentation>CIM Curve</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="IdentifiedObject">

<xs:sequence>

<xs:element name="curveStyle" type="xs:string" minOccurs="0"/>

<xs:element name="CurveData" type="CurveData" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="CurveData">

<xs:annotation>

<xs:documentation>CIm Curve Data</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="xvalue" type="xs:float" minOccurs="0"/>

<xs:element name="y1value" type="xs:float" minOccurs="0"/>

<xs:element name="y2value" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Document">

<xs:annotation>

<xs:documentation>CIM Document superclass</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="mRID" type="xs:string" minOccurs="0"/>

<xs:element name="docType" type="xs:string"/>

<xs:element name="lastModified" type="xs:dateTime"/>

<xs:element name="comments" type="xs:string" minOccurs="0"/>

<xs:element name="docStatus" type="xs:string" minOccurs="0"/>

<xs:element name="docTitle" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PNode">

<xs:annotation>

<xs:documentation>CIM Pricing Node</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="name" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="LoadForecast">

<xs:annotation>

<xs:documentation>Forecasted load: value1Unit is MW</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="RegulatingCondEq">

<xs:annotation>

<xs:documentation>CIM Regulating conducting equipment</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="ConductingEquipment"/>

</xs:complexContent>

- </xs:complexType>

</xs:schema>

- ErcotTransactionTypes.xsd

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp1 (http://www.altova.com) by Scott Neumann (UISOL) -->

<!--   ERCOT Transaction Types: ErcotTransactionTypes.xsd                                         -->

<!--                                                                                              -->

<!--    Release Date: August 2007                                                                 -->

<!--    Ext Int Spec Version No. 1.05                                                             -->

<!--                                                                                              -->

<!--    10/2/07  Nemat                 corrected typo in CapacitySchedule                         -->

<!--    10/3/07  Scott                 added complex types to better support JAXB                 -->

<!--    10/3/07  Scott                 corrected combinedCycle type for CapacitySchedule          -->

<!--    01/2408 Nemat                Corrected combinedCycle to CombinedCycle instead of String  -->

<!--                                                                                              -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.1">

- <!--ERCOT Transaction Types: ErcotTransactionTypes.xsd-->

<!--The types defined in this XSD are used for market transaction interfaces-->

<!---->

<xs:include schemaLocation="ErcotCommonTypes.xsd"/>

<!--Simple Type Definitions-->

<!--Types for bidding-->

<xs:complexType name="MarketRequest">

<xs:annotation>

<xs:documentation>Market common request parameters</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="tradingDate" type="xs:date"/>

<xs:element name="status" type="xs:string" minOccurs="0"/>

<xs:element name="mode" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="BidSet">

<xs:annotation>

<xs:documentation>CIM container class for bids and offers</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="MarketRequest">

<xs:sequence>

<xs:element name="COP" type="COP" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ThreePartOffer" type="ThreePartOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="OutputSchedule" type="OutputSchedule" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="IncDecOffer" type="IncDecOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="CRR" type="CRR" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ASOffer" type="ASOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="EnergyBid" type="EnergyBid" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="EnergyOnlyOffer" type="EnergyOnlyOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="PTPObligation" type="PTPObligation" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="SelfArrangedAS" type="SelfArrangedAS" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="EnergyTrade" type="EnergyTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="CapacityTrade" type="CapacityTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ASTrade" type="ASTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="DCTieSchedule" type="DCTieSchedule" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="SelfSchedule" type="SelfSchedule" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="FipFop">

<xs:annotation>

<xs:documentation>FIP FOP percentages</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="fipPercent" type="xs:float"/>

<xs:element name="fopPercent" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="StartupCost">

<xs:annotation>

<xs:documentation>Resource startup costs</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="hot" type="xs:float" minOccurs="0"/>

<xs:element name="intermediate" type="xs:float" minOccurs="0"/>

<xs:element name="cold" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MinimumEnergy">

<xs:annotation>

<xs:documentation>Resource minimum generation costs</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="cost" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MaximumPrice">

<xs:annotation>

<xs:documentation>Maximum prices</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="price" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MinimumPrice">

<xs:annotation>

<xs:documentation>Minimum prices</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="price" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ResourceStatus">

<xs:annotation>

<xs:documentation>Resource Status </xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="operatingMode" type="OperatingMode"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Limits">

<xs:annotation>

<xs:documentation>Resource Limits</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="hsl" type="xs:float"/>

<xs:element name="lsl" type="xs:float"/>

<xs:element name="hel" type="xs:float"/>

<xs:element name="lel" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ASCapacity">

<xs:annotation>

<xs:documentation>Ancillary Services Capacity</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="regUp" type="xs:float" minOccurs="0"/>

<xs:element name="regDown" type="xs:float" minOccurs="0"/>

<xs:element name="rrs" type="xs:float" minOccurs="0"/>

<xs:element name="nonSpin" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="NERCTags">

<xs:annotation>

<xs:documentation>NERC Tags</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="tag" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<!--Market products-->

<xs:complexType name="ThreePartOffer">

<xs:annotation>

<xs:documentation>Three Part Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Offer">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="combinedCycle" type="CombinedCycle" minOccurs="0"/>

<xs:element name="FipFop" type="FipFop" minOccurs="0" maxOccurs="25"/>

<xs:element name="StartupCost" type="StartupCost" minOccurs="0" maxOccurs="25"/>

<xs:element name="MinimumEnergy" type="MinimumEnergy" minOccurs="0" maxOccurs="25"/>

<xs:element name="EnergyOfferCurve" type="PriceCurve" minOccurs="0" maxOccurs="25"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="COP">

<xs:annotation>

<xs:documentation>Current Operating Plan</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="combinedCycle" type="CombinedCycle" minOccurs="0"/>

<xs:element name="ResourceStatus" type="ResourceStatus" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Limits" type="Limits" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ASCapacity" type="ASCapacity" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="IncDecOffer">

<xs:annotation>

<xs:documentation>Incremental Decremental Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Offer">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="combinedCycle" type="CombinedCycle" minOccurs="0"/>

<xs:element name="FipFop" type="FipFop" minOccurs="0" maxOccurs="unbounded"/>

<xs:choice>

<xs:element name="IncrementalCurve" type="PriceCurve" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="DecrementalCurve" type="PriceCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="EnergyBid">

<xs:annotation>

<xs:documentation>Energy Bid</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="expirationTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="sp" type="SettlementPoint" minOccurs="0"/>

<xs:element name="bidID" type="xs:string" minOccurs="0"/>

<xs:element name="PriceCurve" type="PriceCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="EnergyOnlyOffer">

<xs:annotation>

<xs:documentation>Energy Only Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Offer">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint" minOccurs="0"/>

<xs:element name="bidID" type="xs:string" minOccurs="0"/>

<xs:element name="EnergyOfferCurve" type="PriceCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="EnergyTrade">

<xs:annotation>

<xs:documentation>Energy Trade</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Trade">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint" minOccurs="0"/>

<xs:element name="EnergySchedule" type="EnergySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="CapacityTrade">

<xs:annotation>

<xs:documentation>Capacity Trade</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Trade">

<xs:sequence>

<xs:element name="CapacitySchedule" type="CapacitySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="ASTrade">

<xs:annotation>

<xs:documentation>Ancillary Services Trade</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Trade">

<xs:sequence>

<xs:element name="asType" type="ASType" minOccurs="0"/>

<xs:element name="ASSchedule" type="TmSchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="ASOffer">

<xs:annotation>

<xs:documentation>Ancillary Services Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Offer">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="asType" type="ASType" minOccurs="0"/>

<xs:element name="combinedCycle" type="CombinedCycle" minOccurs="0"/>

<xs:element name="ASPriceCurve" type="ASPriceCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="SelfArrangedAS">

<xs:annotation>

<xs:documentation>Self-Arranged Ancillary Services</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="asType" type="ASType" minOccurs="0"/>

<xs:element name="CapacitySchedule" type="CapacitySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="CRR">

<xs:annotation>

<xs:documentation>Congestion Revenue Rights</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="crrId" minOccurs="0"/>

<xs:element name="offerId" minOccurs="0"/>

<xs:element name="crrAccountHolderId" minOccurs="0"/>

<xs:element name="source" type="SettlementPoint" minOccurs="0"/>

<xs:element name="sink" type="SettlementPoint" minOccurs="0"/>

<xs:element name="CapacitySchedule" type="CapacitySchedule" minOccurs="0"/>

<xs:element name="MinimumReservationPrice" type="MinimumPrice" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="NOIEPeakLoadForecast" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="PTPObligation">

<xs:annotation>

<xs:documentation>PTP Obligation</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="source" type="SettlementPoint" minOccurs="0"/>

<xs:element name="sink" type="SettlementPoint" minOccurs="0"/>

<xs:element name="crrAccountHolderId" type="xs:string" minOccurs="0"/>

<xs:element name="bidId" type="xs:string" minOccurs="0"/>

<xs:element name="CapacitySchedule" type="TmSchedule" minOccurs="0"/>

<xs:element name="MaximumPrice" type="MaximumPrice" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<!--Schedules-->

<xs:complexType name="Schedule">

<xs:annotation>

<xs:documentation>CIM Schedule class</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="endTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="mRID" type="xs:string" minOccurs="0"/>

<xs:element name="externalId" type="xs:string" minOccurs="0"/>

<xs:element name="marketType" type="xs:string" minOccurs="0"/>

<xs:element name="status" type="TransactionStatusType" minOccurs="0"/>

<xs:element name="error" type="Error" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="OutputSchedule">

<xs:annotation>

<xs:documentation>Output Schedule</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="resource" type="Resource" minOccurs="0"/>

<xs:element name="combinedCycle" type="CombinedCycle" minOccurs="0"/>

<xs:element name="deleteTPOs" type="xs:boolean" minOccurs="0"/>

<xs:element name="EnergySchedule" type="EnergySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="DCTieSchedule">

<xs:annotation>

<xs:documentation>DC Tie Schedule</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="buyer" type="xs:string" minOccurs="0"/>

<xs:element name="seller" type="xs:string" minOccurs="0"/>

<xs:element name="sp" type="SettlementPoint" minOccurs="0"/>

<xs:element name="EnergySchedule" type="EnergySchedule" minOccurs="0"/>

<xs:element name="NERCTags" type="NERCTags" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="SelfSchedule">

<xs:annotation>

<xs:documentation>Self schedules</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Schedule">

<xs:sequence>

<xs:element name="source" type="SettlementPoint" minOccurs="0"/>

<xs:element name="sink" type="SettlementPoint" minOccurs="0"/>

<xs:element name="EnergySchedule" type="EnergySchedule" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<!--Parent classes for bidding-->

<xs:complexType name="Bid">

<xs:annotation>

<xs:documentation>CIM Bid class</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="startTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="endTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="mRID" type="xs:string" minOccurs="0"/>

<xs:element name="externalId" type="xs:string" minOccurs="0"/>

<xs:element name="marketType" type="xs:string" minOccurs="0">

<xs:annotation>

<xs:documentation>reserved for future use</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="status" minOccurs="0">

<xs:simpleType>

<xs:restriction base="TransactionStatusType"/>

</xs:simpleType>

</xs:element>

<xs:element name="error" type="Error" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Trade">

<xs:annotation>

<xs:documentation>Abstract super class for trades</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="buyer" type="QSE" minOccurs="0"/>

<xs:element name="seller" type="QSE" minOccurs="0"/>

<xs:element name="tradeID" type="xs:string" minOccurs="0">

<xs:annotation>

<xs:documentation>reserved for future use</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="Offer">

<xs:annotation>

<xs:documentation>Abstrract super class for offers</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Bid">

<xs:sequence>

<xs:element name="expirationTime" type="xs:dateTime" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

- </xs:complexType>

</xs:schema>

- ErcotInformationTypes.xsd

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp2 (http://www.altova.com) by Nemat Sarnevesht (UISol) -->

<!--   ERCOT Information Types: ErcotInformationTypes.xsd                                           -->

<!--                                                                                                -->

<!--    Release Date: December 2007                                                                 -->

<!--    Ext Int Spec Version No. 1.07                                                               -->

<!--                                                                                                -->

<!--    10/3/2007 0.3.0 Scott    Redefined elements to use complex types to better support JAXB           -->

<!--    10/3/2007 0.3.1 Scott    Moved StartupShutdown to Awards.xsd                                      -->

<!--    02/29/08   0.3.2 Nemat  Removed ASSystemPlan Complex Type as it is not used. Comment by Austin Energy      -->

<!--                               Added type=Float to Constraint/Value, Constraint/limit, and Constraint/maxShadowPrice , Comment by Austin Energy  -->

<!--                               Added type=String to name in LoadDistributionFactor,  Comment by Austin Energy -->

<!--                                                                                                -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.2">

- <!--This XSD defines types used for informational interfaces-->

<!---->

<xs:include schemaLocation="ErcotCommonTypes.xsd"/>

<!--Market Information-->

<xs:complexType name="VoltageProfile">

<xs:complexContent>

<xs:extension base="TmSchedule">

<xs:sequence>

<xs:element name="bus" type="ElectricalBus"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="LMP">

<xs:annotation>

<xs:documentation>Locational Marginal Price</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="PriceSchedule">

<xs:sequence>

<xs:element name="bus" type="ElectricalBus"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="SPP">

<xs:annotation>

<xs:documentation>Settlement Point Price</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="PriceSchedule">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="MCPCSchedule">

<xs:complexContent>

<xs:extension base="PriceSchedule">

<xs:sequence>

<xs:element name="asType" type="ASType"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="LoadRatioShare">

<xs:complexContent>

<xs:extension base="TmSchedule">

<xs:sequence>

<xs:element name="qse" type="QSE"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="Constraint">

<xs:complexContent>

<xs:extension base="IdentifiedObject">

<xs:sequence>

<xs:element name="value" type="xs:float">

<xs:annotation>

<xs:documentation>Value that triggered constraint</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="limit" type="xs:float">

<xs:annotation>

<xs:documentation>Limit defined for constraint</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="maxShadowPrice" type="xs:float" minOccurs="0">

<xs:annotation>

<xs:documentation>Maximum shadow price</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="SettlementPointFactor">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint"/>

<xs:element name="shiftFactor" type="xs:decimal"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="SCEDViolatedConstraint">

<xs:complexContent>

<xs:extension base="Constraint">

<xs:sequence>

<xs:element name="timestamp" type="xs:dateTime"/>

<xs:element name="SF" type="SettlementPointFactor" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="BindingConstraint">

<xs:complexContent>

<xs:extension base="Constraint">

<xs:sequence>

<xs:element name="deliveryTime" type="xs:dateTime"/>

<xs:element name="SF" type="SettlementPointFactor" maxOccurs="unbounded"/>

<xs:element name="rucType" type="RUCType" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="CompetitiveConstraint">

<xs:complexContent>

<xs:extension base="IdentifiedObject">

<xs:sequence>

<xs:element name="limit" type="xs:float"/>

<xs:element name="value" type="xs:float"/>

<xs:element name="status" type="xs:string"/>

<xs:element name="timestamp" type="xs:dateTime"/>

<xs:element name="flag">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="DAILY"/>

<xs:enumeration value="MONTHLY"/>

<xs:enumeration value="YEARLY"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="LoadDistributionFactor">

<xs:sequence>

<xs:element name="name" type="xs:string"/>

<xs:element name="level">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="ZONE"/>

<xs:enumeration value="LOADGROUP"/>

<xs:enumeration value="LOAD"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="factor" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ShiftFactor">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="equipment" type="Equipment"/>

<xs:element name="factor" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="AggregatedASOfferCurve">

<xs:sequence>

<xs:element name="asType" type="ASType"/>

<xs:element name="PriceCurve" type="PriceCurve"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ASSchedule">

<xs:sequence>

<xs:element name="asType" type="ASType"/>

<xs:element name="Schedule" type="TmSchedule"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="UnitAvailability">

<xs:annotation>

<xs:documentation>Unit availability schedule</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="contractType" minOccurs="0">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="RMR"/>

<xs:enumeration value="BlackStart"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="available" type="xs:boolean"/>

<xs:element name="availableMW" type="xs:integer" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="DeratedCRR">

<xs:annotation>

<xs:documentation>Derated CRRs</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="crrHolder" type="xs:string"/>

<xs:element name="crrId" type="xs:string"/>

<xs:element name="source" type="SettlementPoint"/>

<xs:element name="sink" type="SettlementPoint"/>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="inputQuantity" type="xs:integer"/>

<xs:element name="deratedQuantity" type="xs:integer"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="DynamicRating">

<xs:annotation>

<xs:documentation>Dynamic rating</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="equipment" type="Equipment"/>

<xs:element name="rating" type="Rating" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="NonConformLoadSchedule">

<xs:annotation>

<xs:documentation>Non conforming load schedule</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule">

<xs:sequence>

<xs:element name="EnergyConsumer" type="EnergyConsumer"/>

<xs:element name="DayType" type="DayType" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="VoltageLimits">

<xs:sequence>

<xs:element name="bus" type="ElectricalBus"/>

<xs:element name="minimumKV" type="xs:float"/>

<xs:element name="maximumKV" type="xs:float"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="TapLimits">

<xs:sequence>

<xs:element name="equipment" type="Equipment"/>

<xs:element name="minimumStep" type="xs:integer"/>

<xs:element name="maximumStep" type="xs:integer"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ProxyCurve">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="PriceCurve" type="PriceCurve"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MitigatedCurve">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="PriceCurve" type="PriceCurve"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="UnitAvailabilities">

<xs:annotation>

<xs:documentation>UnitAvailability container</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="UnitAvailability" type="UnitAvailability" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="SPPs">

<xs:annotation>

<xs:documentation>SPP container</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="SPP" type="SPP" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="LMPs">

<xs:annotation>

<xs:documentation>LMP container</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="LMP" type="LMP" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MCPCs">

<xs:annotation>

<xs:documentation>MCPC container</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="MCPCSchedule" type="MCPCSchedule" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="TotalASOffers">

<xs:annotation>

<xs:documentation>TotalASOffers</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="ASOffers" type="ASSchedule" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ProxyCurves">

<xs:annotation>

<xs:documentation>ProxyCurve container</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="ProxyCurve" type="ProxyCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MitigatedCurves">

<xs:annotation>

<xs:documentation>MitigatedCurve container</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="MitigatedCurve" type="MitigatedCurve" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

- </xs:complexType>

</xs:schema>

- ErcotNotificationTypes.xsd

<?xml version="1.0" encoding="UTF-8"?>

<!--   -ERCOT Notification Types: ErcotNotificationTypes.xsd                                          -->

<!--                                                                                                  -->

<!--    Release Date: December 2007                                                                  -->

<!--    Ext Int Spec Version No. 1.07                                                                 -->

<!--                                                                                                  -->

<!--    10/3/2007  Scott   0.3.0 added ConfirmedTrades                                                      -->

<!--    12/6/2007  Scott   0.3.1 moved ConfirmedTrades and UnconfirmedTrades to Awards.xsd                  -->

<!--                                                                                                  -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.1">

<!--ERCOT Notification Types: ErcotNotificationTypes.xsd-->

<!--This XSD defines types used for notification interfaces-->

<!---->

<xs:include schemaLocation="ErcotCommonTypes.xsd"/>

<xs:include schemaLocation="ErcotTransactionTypes.xsd"/>

<!--Notifications-->

<!--Elements used for payload definitions using ComplexTypes-->

<xs:complexType name="Notification">

<xs:annotation>

<xs:documentation> Notitication class</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="notificationID" type="xs:string"/>

<xs:element name="notificationType" type="xs:string"/>

<xs:element name="priority" type="xs:int"/>

<xs:element name="source" type="xs:string"/>

<xs:element name="issued" type="xs:dateTime"/>

<xs:element name="expiration" type="xs:dateTime" minOccurs="0"/>

<xs:element name="summary" type="xs:string"/>

<xs:element name="details" type="xs:string" minOccurs="0"/>

<xs:element name="objectID" type="xs:string" minOccurs="0"/>

<xs:element name="referenceURL" type="xs:string" minOccurs="0"/>

<xs:any namespace="##other" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

- </xs:complexType>

</xs:schema>

- ErcotAwardTypes.xsd

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp1 (http://www.altova.com) by Scott Neumann (UISOL) -->

<!--   ErcotAwardTypes.xsd                                                                                        -->

<!--                                                                                                              -->

<!--    Release Date: December 2007                                                                               -->

<!--    Ext Int Spec Version No. 1.07                                                                             -->

<!--                                                                                                              -->

<!--    12/6/2007  Scott   0.2   added ConfirmedTrades and UnconfirmedTrades                                            -->

<!--    12/6/2007  Scott   0.3.0  added StartupShutdown                                                                  -->

<!--    02/29/2008  Nemat 0.3.1  Removed Element ASObligation as it is defined in ErcotAwards.xsd, Comment from Austin Energy -->

<!--                                          Added type spp and added lmp with type=lmp to AwardedEnergyOffer, Comment from Austin Energy -->

<!--                                          Added type=Float to mcpcp in AwardedAS   , Comment from Austin Energy -->

<!--                                                                                                              -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.1">

<!--ERCOT Awards Types: ErcotAwardTypes.xsd-->

<!--The types defined in this XSD define structures used for awards-->

<!---->

<xs:include schemaLocation="ErcotCommonTypes.xsd"/>

<xs:include schemaLocation="ErcotTransactionTypes.xsd"/>

<xs:include schemaLocation="ErcotInformationTypes.xsd"/>

<!--Awards-->

<xs:complexType name="AwardSet">

<xs:annotation>

<xs:documentation>Container for awards</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="tradingDate" type="xs:date"/>

<xs:element name="marketType" type="xs:string" minOccurs="0"/>

<xs:sequence>

<xs:element name="AwardedAS" type="AwardedAS" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedCRR" type="AwardedCRR" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedEnergyBid" type="AwardedEnergyBid" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedEnergyOffer" type="AwardedEnergyOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedEnergyOnlyOffer" type="AwardedEnergyOnlyOffer" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="AwardedPTPObligation" type="AwardedPTPObligation" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Award">

<xs:annotation>

<xs:documentation>Award superclass</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="qse" type="QSE"/>

<xs:element name="startTime" type="xs:dateTime"/>

<xs:element name="endTime" type="xs:dateTime"/>

<xs:element name="tradingDate" type="xs:date"/>

<xs:element name="mRID" type="xs:string"/>

<xs:element name="marketType" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="AwardedAS">

<xs:annotation>

<xs:documentation>Ancillary Service Award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="asType" type="ASType"/>

<xs:element name="dispatchMW" type="xs:int" minOccurs="0"/>

<xs:element name="selfSchedMW" type="xs:int" minOccurs="0"/>

<xs:element name="mcpc" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedEnergyOffer">

<xs:annotation>

<xs:documentation>EnergyAward Offer</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="dispatchMWh" type="xs:int"/>

<xs:element name="spp" type="xs:float" minOccurs="0"/>

<xs:element name="lmp" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedCRR">

<xs:annotation>

<xs:documentation>CRR award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="awardedMW" type="xs:int"/>

<xs:element name="source" type="SettlementPoint"/>

<xs:element name="sink" type="SettlementPoint"/>

<xs:element name="price" type="xs:float"/>

<xs:element name="crrId" type="xs:string"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedEnergyBid">

<xs:annotation>

<xs:documentation>Energy Bid Award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint"/>

<xs:element name="awardedMWh" type="xs:int"/>

<xs:element name="spp" type="xs:float" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedPTPObligation">

<xs:annotation>

<xs:documentation>PTP Obligation award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="awardedMW" type="xs:int"/>

<xs:element name="source" type="SettlementPoint"/>

<xs:element name="sink" type="SettlementPoint"/>

<xs:element name="price" type="xs:float"/>

<xs:element name="bidId" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AwardedEnergyOnlyOffer">

<xs:annotation>

<xs:documentation>Energy Only Offer Award</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="Award">

<xs:sequence>

<xs:element name="sp" type="SettlementPoint"/>

<xs:element name="awardedMWh" type="xs:int"/>

<xs:element name="spp" type="SPP" minOccurs="0"/>

<xs:element name="lmp" type="LMP" minOccurs="0"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="ASObligation">

<xs:annotation>

<xs:documentation>Ancillary Service Obligation</xs:documentation>

</xs:annotation>

<xs:complexContent>

<xs:extension base="TmSchedule">

<xs:sequence>

<xs:element name="asType" type="ASType"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="UnconfirmedTrades">

<xs:annotation>

<xs:documentation>Unconfirmed trades</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="ASTrade" type="ASTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="CapacityTrade" type="CapacityTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="EnergyTrade" type="EnergyTrade" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ConfirmedTrades">

<xs:annotation>

<xs:documentation>Confirmed trades</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="ASTrade" type="ASTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="CapacityTrade" type="CapacityTrade" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="EnergyTrade" type="EnergyTrade" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="StartupShutdown">

<xs:annotation>

<xs:documentation>Unit startup and shitdown instructions</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="rucType" type="RUCType"/>

<xs:element name="deliveryDate" type="xs:dateTime"/>

<xs:element name="resource" type="Resource"/>

<xs:element name="clearedCommit" type="xs:boolean"/>

<xs:element name="clearedDecommit" type="xs:boolean"/>

<xs:element name="decommitReasonCode" type="xs:string"/>

</xs:sequence>

- </xs:complexType>

</xs:schema>

- ErcotOutageTypes.xsd

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp2 (http://www.altova.com) by Nemat Sarnevesht (UISol) -->

<!--   ERCOT Outage Types: ErcotOutageTypes.xsd                                                      -->

<!--                                                                                                 -->

<!--    Release Date: September 2007                                                                 -->

<!--    Ext Int Spec Version No. 1.07                                                               -->

<!--                                                                                                 -->

<!--    10/3/2007  Scott    0.3.0 Revised outage types                                -->

<!--    12/6/2007  Scott    0.3.1 Revised to reflect Outageset                -->

<!--    02/02/2008 Nemat  0.3.2 Removed NewSchedule + added "Planned" attributes to   OutageSchedule Complex type -->

<!--    02/15/2008 NS/VK  0.3.3 Added missing types for lastModifiedBy, HSL, LSL , added OutageIdent to ResourceOutage and equipmentOutage   -->

<!--                                         added types to contact elements ,  Added more enums to EquipmentType , added Datelist type and applied it to Recurence   -->

<!--    02/28/2008 VK       0.3.4  updated comment for mRID,created a choice for Recurrance and Schedule,changed type of                     -->

<!--                                          emergencyRestorationTime,changed equipment to equipmentName, added equipmentStationName element under -->

<!--                                          OutageEquipment, changed enumeration for StateCode   -->

<!--                                                                                                 -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.4">

<!--ERCOT Outage Types: ErcotOutageTypes.xsd-->

<!--This XSD defines types used for outage interfaces-->

<!---->

<xs:include schemaLocation="ErcotCommonTypes.xsd"/>

<!--Outage Scheduling-->

<xs:complexType name="Outage">

<xs:annotation>

<xs:documentation>Scheduled Outage</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="mRID" type="xs:string" minOccurs="0">

<xs:annotation>

<xs:documentation>Outage Ident, assigned by OS</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="Command" type="Command" minOccurs="0"/>

<xs:element name="OutageInfo" type="OutageInfo" minOccurs="0">

<xs:annotation>

<xs:documentation>Outgage identification, state and status information</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="Group" type="OutageGrouping" minOccurs="0">

<xs:annotation>

<xs:documentation>Outage groupiing</xs:documentation>

</xs:annotation>

</xs:element>

<xs:choice>

<xs:element name="ResourceOutage" type="ResourceOutage" minOccurs="0">

<xs:annotation>

<xs:documentation>Resource outage details</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="EquipmentOutage" type="EquipmentOutage" minOccurs="0">

<xs:annotation>

<xs:documentation>transmission equipment outage details</xs:documentation>

</xs:annotation>

</xs:element>

</xs:choice>

<xs:choice>

<xs:element name="Schedule" type="OutageSchedule" minOccurs="0">

<xs:annotation>

<xs:documentation>Outage scheduling information</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="Recurrance" type="Recurrance" minOccurs="0"/>

</xs:choice>

<xs:element name="Opportunity" type="Opportunity" minOccurs="0"/>

<xs:element name="Notes" minOccurs="0">

<xs:complexType>

<xs:sequence>

<xs:element name="RequestorNotes" type="Notes" minOccurs="0"/>

<xs:element name="ReviewerNotes" type="Notes" minOccurs="0"/>

<xs:element name="SupportingNotes" type="Notes" minOccurs="0"/>

<xs:element name="RASPSNotes" type="Notes" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="Error" type="Error" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="OutageSchedule">

<xs:sequence>

<xs:element name="plannedStart" type="xs:dateTime"/>

<xs:element name="plannedEnd" type="xs:dateTime"/>

<xs:element name="earliestStart" type="xs:dateTime" minOccurs="0"/>

<xs:element name="latestEnd" type="xs:dateTime" minOccurs="0"/>

<xs:element name="actualStart" type="xs:dateTime" minOccurs="0"/>

<xs:element name="actualEnd" type="xs:dateTime" minOccurs="0"/>

<xs:element name="new_plannedStart" type="xs:dateTime" minOccurs="0"/>

<xs:element name="new_plannedEnd" type="xs:dateTime" minOccurs="0"/>

<xs:element name="new_earliestStart" type="xs:dateTime" minOccurs="0"/>

<xs:element name="new_latestEnd" type="xs:dateTime" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Notes">

<xs:annotation>

<xs:documentation>Set of notes</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="Note" type="Note" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Note">

<xs:annotation>

<xs:documentation>Based on CIM Document</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="lastModified" type="xs:dateTime"/>

<xs:element name="createdBy" type="xs:string"/>

<xs:element name="company" type="xs:string"/>

<xs:element name="comment" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ResourceOutage">

<xs:sequence>

<xs:element name="operatingCompany" type="xs:string"/>

<xs:element name="station" type="xs:string"/>

<xs:element name="Resource" type="OutageResource"/>

<xs:element name="HSL" type="xs:int"/>

<xs:element name="LSL" type="xs:int"/>

<xs:element name="outageIdent" type="xs:string" minOccurs="0"/>

<xs:element name="natureOfWork">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="IP"/>

<xs:enumeration value="BS"/>

<xs:enumeration value="BO"/>

<xs:enumeration value="CI"/>

<xs:enumeration value="EX"/>

<xs:enumeration value="FW"/>

<xs:enumeration value="CW"/>

<xs:enumeration value="FP"/>

<xs:enumeration value="GW"/>

<xs:enumeration value="FL"/>

<xs:enumeration value="LS"/>

<xs:enumeration value="OV"/>

<xs:enumeration value="TW"/>

<xs:enumeration value="TL"/>

<xs:enumeration value="VR"/>

<xs:enumeration value="MO"/>

<xs:enumeration value="UN"/>

<xs:enumeration value="OT"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="EquipmentOutage">

<xs:sequence>

<xs:element name="operatingCompany" type="xs:string"/>

<xs:element name="Equipment" type="OutageEquipment"/>

<xs:element name="fromStation" type="Station"/>

<xs:element name="toStation" type="Station"/>

<xs:element name="normalState" type="xs:string"/>

<xs:element name="outageState" type="xs:string"/>

<xs:element name="voltage" type="xs:float"/>

<xs:element name="NOMCR" type="xs:string" minOccurs="0"/>

<xs:element name="emergencyRestorationTime" type="xs:positiveInteger"/>

<xs:element name="outageIdent" type="xs:string" minOccurs="0"/>

<xs:element name="natureOfWork">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="LM"/>

<xs:enumeration value="LR"/>

<xs:enumeration value="BM"/>

<xs:enumeration value="BR"/>

<xs:enumeration value="TM"/>

<xs:enumeration value="TR"/>

<xs:enumeration value="TT"/>

<xs:enumeration value="SM"/>

<xs:enumeration value="SR"/>

<xs:enumeration value="DR"/>

<xs:enumeration value="AO"/>

<xs:enumeration value="RW"/>

<xs:enumeration value="RE"/>

<xs:enumeration value="HS"/>

<xs:enumeration value="UN"/>

<xs:enumeration value="OT"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="OutageGrouping">

<xs:sequence>

<xs:element name="groupId" type="xs:string" minOccurs="0"/>

<xs:element name="name" type="xs:string"/>

<xs:choice>

<xs:element name="ResourceOutage" type="ResourceOutage" minOccurs="0" maxOccurs="30"/>

<xs:element name="EquipmentOutage" type="EquipmentOutage" minOccurs="0" maxOccurs="30"/>

</xs:choice>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Contact">

<xs:sequence>

<xs:element name="name" type="xs:string">

<xs:annotation>

<xs:documentation>contact name</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="primaryContact" type="xs:string">

<xs:annotation>

<xs:documentation>primary contact phone number</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="secondaryContact" type="xs:string">

<xs:annotation>

<xs:documentation>secondary contact phone number</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="tertiaryContact" type="xs:string" minOccurs="0"/>

<xs:element name="role" type="xs:string"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="EquipmentList">

<xs:sequence>

<xs:element name="Equipment" type="OutageEquipment" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="EquipmentType">

<xs:restriction base="xs:string">

<xs:enumeration value="LN"/>

<xs:enumeration value="XF"/>

<xs:enumeration value="CP"/>

<xs:enumeration value="CB"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="OutageEquipment">

<xs:sequence>

<xs:element name="equipmentName" type="xs:string"/>

<xs:element name="equipmentStationName" type="xs:string"/>

<xs:element name="equipmentType" type="EquipmentType"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="OutageInfo">

<xs:sequence>

<xs:element name="outageId" type="xs:string" minOccurs="0"/>

<xs:element name="outageType">

<xs:annotation>

<xs:documentation>Type of outage</xs:documentation>

</xs:annotation>

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="FR"/>

<xs:enumeration value="M1"/>

<xs:enumeration value="M2"/>

<xs:enumeration value="M3"/>

<xs:enumeration value="OP"/>

<xs:enumeration value="PL"/>

<xs:enumeration value="RS"/>

<xs:enumeration value="SM"/>

<xs:enumeration value="UE"/>

<xs:enumeration value="FE"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="participant" type="xs:string" minOccurs="0">

<xs:annotation>

<xs:documentation>Requesting company</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="versionId" type="xs:unsignedByte" minOccurs="0"/>

<xs:element name="state" type="StateCode" minOccurs="0"/>

<xs:element name="status" type="StatusCode" minOccurs="0"/>

<xs:element name="previousState" type="StateCode" minOccurs="0"/>

<xs:element name="previousStatus" type="StatusCode" minOccurs="0"/>

<xs:element name="requestedStatus" type="StatusCode" minOccurs="0"/>

<xs:element name="lastModified" type="xs:dateTime" minOccurs="0">

<xs:annotation>

<xs:documentation>when created or last updated</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="lastModifiedBy" type="xs:string" minOccurs="0">

<xs:annotation>

<xs:documentation>creator or responsible for last update</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="warningFlag" type="xs:boolean" minOccurs="0">

<xs:annotation>

<xs:documentation>Warning?</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="warningAcknowledge" type="xs:boolean" minOccurs="0">

<xs:annotation>

<xs:documentation>Warning acknowledged?</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="warningMessage" type="xs:string" minOccurs="0"/>

<xs:element name="Requestor" type="Contact" minOccurs="0">

<xs:annotation>

<xs:documentation>Outage contact information</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="requestDate" type="xs:date" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="Station">

<xs:restriction base="xs:string"/>

</xs:simpleType>

<xs:complexType name="OutageResource">

<xs:sequence>

<xs:element name="resource" type="Resource"/>

<xs:element name="resourceType" type="ResourceType"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="ResourceType">

<xs:restriction base="xs:string">

<xs:enumeration value="RMR"/>

<xs:enumeration value="BLK"/>

<xs:enumeration value="NON"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="Opportunity">

<xs:sequence>

<xs:element name="start" type="xs:dateTime"/>

<xs:element name="end" type="xs:dateTime"/>

<xs:element name="equipment" type="OutageEquipment" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="OutageSet">

<xs:annotation>

<xs:documentation>Container class for Outages</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="Outage" type="Outage" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="Day">

<xs:annotation>

<xs:documentation>Days of week</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="Monday"/>

<xs:enumeration value="Tuesday"/>

<xs:enumeration value="Wednesday"/>

<xs:enumeration value="Thursday"/>

<xs:enumeration value="Friday"/>

<xs:enumeration value="Saturday"/>

<xs:enumeration value="Sunday"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="Days">

<xs:annotation>

<xs:documentation>Set of days</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="Day" type="Day" minOccurs="0" maxOccurs="7"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="Periodicity">

<xs:restriction base="xs:string">

<xs:enumeration value="Weekly"/>

<xs:enumeration value="Periodic"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="StatusCode">

<xs:restriction base="xs:string">

<xs:enumeration value="ENYS"/>

<xs:enumeration value="RatE"/>

<xs:enumeration value="Accpt"/>

<xs:enumeration value="Apprv"/>

<xs:enumeration value="Rejct"/>

<xs:enumeration value="Study"/>

<xs:enumeration value="Withd"/>

<xs:enumeration value="Cancl"/>

<xs:enumeration value="Pendg"/>

<xs:enumeration value="Compl"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="StateCode">

<xs:restriction base="xs:string">

<xs:enumeration value="ENYS"/>

<xs:enumeration value="Recvd"/>

<xs:enumeration value="Study"/>

<xs:enumeration value="Rejct"/>

<xs:enumeration value="Accpt"/>

<xs:enumeration value="Apprv"/>

<xs:enumeration value="Active"/>

<xs:enumeration value="AExt"/>

<xs:enumeration value="Withd"/>

<xs:enumeration value="Cancl"/>

<xs:enumeration value="PComp"/>

<xs:enumeration value="ExpOp"/>

<xs:enumeration value="CompEd"/>

<xs:enumeration value="CompNE"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="Recurrance">

<xs:sequence>

<xs:element name="datesList" type="DatesList"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="RecurrancePeriod">

<xs:sequence>

<xs:choice>

<xs:element name="day" type="Day" minOccurs="0" maxOccurs="7"/>

<xs:element name="daysBetween" type="xs:int" minOccurs="0"/>

</xs:choice>

<xs:element name="Periodicity" type="Periodicity"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="Command">

<xs:restriction base="xs:string">

<xs:enumeration value="UNGROUP"/>

<xs:enumeration value="ACKNOWLEDGE"/>

<xs:enumeration value="CHANGE_STATUS"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="DatesList">

<xs:sequence>

<xs:element name="dates" type="Dates" maxOccurs="50"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Dates">

<xs:sequence>

<xs:element name="outageIdent" type="xs:string" minOccurs="0"/>

<xs:element name="plannedStart" type="xs:dateTime"/>

<xs:element name="plannedEnd" type="xs:dateTime"/>

<xs:element name="earliestStart" type="xs:dateTime"/>

<xs:element name="latestEnd" type="xs:dateTime"/>

</xs:sequence>

- </xs:complexType>

</xs:schema>

- ErcotMiscTypes.xsd

<?xml version="1.0" encoding="UTF-8"?>

<!--   ERCOT Miscellaneous Types: ErcotMiscTypes.xsd                                               -->

<!--                                                                                                                            -->

<!--    Release Date: August 2007                                                                                -->

<!--    Ext Int Spec Version No. 1.03                                                                             -->

<!--                                                                                                                            -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.2">

- <!--ERCOT Miscellaneous Types: ErcotMiscTypes.xsd-->

<!--This XSD provides miscellaneous type definitions not covered in other XSDs-->

<!---->

<xs:include schemaLocation="ErcotCommonTypes.xsd"/>

<!--Report Access-->

<xs:complexType name="Report">

<xs:sequence>

<xs:element name="operatingDate" type="xs:date">

<xs:annotation>

<xs:documentation>Operating date for extracts</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="accessGroup">

<xs:annotation>

<xs:documentation>Access group name, e.g. 'Public'</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="reportGroup">

<xs:annotation>

<xs:documentation>Group name for reports, e.g.  'Market Information Extract'</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="fileName">

<xs:annotation>

<xs:documentation>File name of report</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="created" type="xs:dateTime">

<xs:annotation>

<xs:documentation>When report file was created</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="size">

<xs:annotation>

<xs:documentation>Size of file n KB</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="format">

<xs:annotation>

<xs:documentation>File format, e.g. PDF, zip, txt</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="URL" type="URL">

<xs:annotation>

<xs:documentation>URL for file download</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

- </xs:complexType>

</xs:schema>

- ErcotTransactions.xsd

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp2 (http://www.altova.com) by Nemat Sarnevesht (UISol) -->

<!--   ERCOT Transactions: ErcotTransacions.xsd-                                                                  -->

<!--                                                                                                              -->

<!--    Release Date: September 2007                                                                              -->

<!--    Ext Int Spec Version No. 1.05                                                                             -->

<!--                                                                                                              -->

<!--    10/3/2007  Scott   added elements to better support JAXB                                                  -->

<!--                                                                                                              -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3">

- <!--ERCOT Transactions: ErcotTransacions.xsd-->

<!--This XSD defines the BidSet container element used for market transaction interfaces-->

<!---->

<xs:include schemaLocation="ErcotTransactionTypes.xsd"/>

<xs:element name="BidSet" type="BidSet"/>

<xs:element name="ASOffer" type="ASOffer"/>

<xs:element name="ASTrade" type="ASTrade"/>

<xs:element name="CapacityTrade" type="CapacityTrade"/>

<xs:element name="COP" type="COP"/>

<xs:element name="CRR" type="CRR"/>

<xs:element name="DCTieSchedule" type="DCTieSchedule"/>

<xs:element name="EnergyBid" type="EnergyBid"/>

<xs:element name="EnergyOnlyOffer" type="EnergyOnlyOffer"/>

<xs:element name="EnergyTrade" type="EnergyTrade"/>

<xs:element name="IncDecOffer" type="IncDecOffer"/>

<xs:element name="OutputSchedule" type="OutputSchedule"/>

<xs:element name="PTPObligation" type="PTPObligation"/>

<xs:element name="SelfArrangedAS" type="SelfArrangedAS"/>

<xs:element name="SelfSchedule" type="SelfSchedule"/>

<xs:element name="ThreePartOffer" type="ThreePartOffer"/>

<xs:element name="ASCapacity" type="ASCapacity"/>

</xs:schema>

- ErcotInformation.xsd

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp1 (http://www.altova.com) by Scott Neumann (UISOL) -->

<!--  ERCOT Information: ErcotInformation.xsd                                                               -->

<!--                                                                                                        -->

<!--    Release Date: December 2007                                                                        -->

<!--    Ext Int Spec Version No. 1.07                                                                       -->

<!--                                                                                                        -->

<!--    10/3/2007   Scott   added element definition to better support JAXB                                  -->

<!--    10/29/2007  Scott   0.3.1 Added AwardSet element                                                          -->

<!--    12/3/2007   Scott   0.3.2  Restructured to use ErcotAwards.xsd                                                         -->

<!--                                                                                                        -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.2">

<!--ERCOT Information: ErcotInformation.xsd-->

<!--This XSD defines the elements used for informational interfaces-->

<xs:include schemaLocation="ErcotTransactionTypes.xsd"/>

<xs:include schemaLocation="ErcotInformationTypes.xsd"/>

<xs:include schemaLocation="ErcotAwards.xsd"/>

<!---->

<!--Elements used for payload definitions using ComplexTypes-->

<xs:element name="LoadForecast" type="LoadForecast"/>

<xs:element name="SystemLoad" type="TmSchedule"/>

<xs:element name="MarketTotals" type="TmSchedule"/>

<xs:element name="SPPs" type="SPPs">

<xs:annotation>

<xs:documentation>Settlement Point Prices</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="LMPs" type="LMPs">

<xs:annotation>

<xs:documentation>Locational Marginal Prices</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="MCPCs" type="MCPCs"/>

<xs:element name="BindingConstraints">

<xs:complexType>

<xs:sequence>

<xs:element name="BindingConstraint" type="BindingConstraint" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="TotalRegulation" type="TmSchedule"/>

<xs:element name="LoadRatioShares">

<xs:complexType>

<xs:sequence>

<xs:element name="LoadRatioShare" type="LoadRatioShare" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="TotalEnergy" type="TmSchedule"/>

<xs:element name="CompetitiveConstraints">

<xs:complexType>

<xs:sequence>

<xs:element name="CompetitiveConstraint" type="CompetitiveConstraint" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="ShiftFactors">

<xs:complexType>

<xs:sequence>

<xs:element name="ShiftFactor" type="ShiftFactor" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="AggregatedASOfferCurves">

<xs:complexType>

<xs:sequence>

<xs:element name="AggregatedASOfferCurve" type="AggregatedASOfferCurve" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="TotalASOffers" type="TotalASOffers"/>

<xs:element name="ASSystemPlan">

<xs:complexType>

<xs:sequence>

<xs:element name="ASSchedule" type="ASSchedule" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="UnitAvailabilities" type="UnitAvailabilities"/>

<xs:element name="DynamicRatings">

<xs:complexType>

<xs:sequence>

<xs:element name="DynamicRating" type="DynamicRating" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="LoadProfiles">

<xs:complexType>

<xs:sequence>

<xs:element name="LoadProfile" type="NonConformLoadSchedule" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="LoadDistributionFactors">

<xs:complexType>

<xs:sequence>

<xs:element name="LoadDistributionFactor" type="LoadDistributionFactor" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="VoltageProfiles">

<xs:complexType>

<xs:sequence>

<xs:element name="VoltageLimits" type="VoltageLimits" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="TapLimits" type="TapLimits" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="DeratedCRRs">

<xs:complexType>

<xs:sequence>

<xs:element name="DeratedCRR" type="DeratedCRR" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="ProxyCurves" type="ProxyCurves"/>

<xs:element name="MitigatedCurves" type="MitigatedCurves"/>

<xs:element name="SCEDViolatedConstraints">

<xs:complexType>

<xs:sequence>

<xs:element name="SCEDViolatedConstraint" type="SCEDViolatedConstraint" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

- </xs:element>

</xs:schema>

- ErcotAwards.xsd

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp2 (http://www.altova.com) by Nemat Sarnevesht (UISol) -->

<!--   ErcotAwards.xsd                                                                                -->

<!--                                                                                                        -->

<!--    Release Date: December 2007                                                                        -->

<!--    Ext Int Spec Version No. 1.07                                                                       -->

<!--                                                                                                        -->

<!--    12/6/2007  Scott   0.3.0 Initial version                                   -->

<!--                                                                                                        -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.0">

- <!--ERCOT Awards: ErcotAwards.xsd-->

<!--This XSD defines container elements that may be conveyed for awards and related market processing -->

<!---->

<xs:include schemaLocation="ErcotAwardTypes.xsd"/>

<!--Awards-->

<!--Elements used for payload definitions using ComplexTypes-->

<xs:element name="AwardSet" type="AwardSet"/>

<xs:element name="AwardedAS" type="AwardedAS"/>

<xs:element name="AwardedCRR" type="AwardedCRR"/>

<xs:element name="AwardedEnergyBid" type="AwardedEnergyBid"/>

<xs:element name="AwardedEnergyOffer" type="AwardedEnergyOffer"/>

<xs:element name="AwardedEnergyOnlyOffer" type="AwardedEnergyOnlyOffer"/>

- <xs:element name="AwardedPTPObligation" type="AwardedPTPObligation"/>

<!--Other types related to market processing and results -->

<xs:element name="UnconfirmedTrades" type="UnconfirmedTrades"/>

<xs:element name="ConfirmedTrades" type="ConfirmedTrades"/>

<xs:element name="StartupShutdownInstructions">

<xs:complexType>

<xs:sequence>

<xs:element name="StartupShutdown" type="StartupShutdown" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="ASObligations">

<xs:complexType>

<xs:sequence>

<xs:element name="ASObligation" type="ASObligation" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

- </xs:element>

</xs:schema>

- ErcotNotifications.xsd

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp1 (http://www.altova.com) by Scott Neumann (UISOL) -->

<!--   ErcotNotification.xsd                                                                                -->

<!--                                                                                                        -->

<!--    Release Date: December 2007                                                                         -->

<!--    Ext Int Spec Version No. 1.07                                                                       -->

<!--                                                                                                        -->

<!--    10/3/2007  Scott   0.3.0 added element definition to better support JAXB                                  -->

<!--    12/3/2007  Scott   0.3.1 restructured to use ErcotAwards.xsd                                              -->

<!--                                                                                                        -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.3.1">

- <!--ERCOT Notifications: ErcotNotification.xsd-->

<!--This XSD defines container elements that may be conveyed in notification messages-->

<!---->

<xs:include schemaLocation="ErcotNotificationTypes.xsd"/>

<xs:include schemaLocation="ErcotAwards.xsd"/>

<xs:include schemaLocation="ErcotOutages.xsd"/>

<!--Notifications-->

<!--Elements used for payload definitions using ComplexTypes-->

<xs:element name="Notification" type="Notification"/>

</xs:schema>

- ErcotOutages.xsd

<?xml version="1.0" encoding="UTF-8"?>

- <!-- edited with XMLSpy v2007 sp2 (http://www.altova.com) by Nemat Sarnevesht (UISol) -->

<!--   ERCOT Outages: ErcotOutages.xsd-                                                  -->

<!--                                                                                                          -->

<!--    Release Date: August 2007                                                              -->

<!--    Ext Int Spec Version No. 1.07                                                          -->

<!--     08/29/07    Scott  0.2.0  Initial version                                              -->

<!--    12/6/2007  Scott   0.2.1 Rename OutageSchedule to OutageSet         -->

<!--    2/28/2008  Venkat Kaza   0.2.2 Eliminiated element Outage                 -->

<!--                                                                                                          -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.2.2">

<!--ERCOT Outages: ErcotOutages.xsd-->

<!--This XSD defines elements used for outage interfaces-->

<!---->

<xs:include schemaLocation="ErcotOutageTypes.xsd"/>

<!--Outage Scheduling-->

<xs:element name="OutageSet">

<xs:annotation>

<xs:documentation>Container element for outages</xs:documentation>

</xs:annotation>

<xs:complexType>

<xs:sequence>

<xs:element name="Outage" type="Outage" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

- </xs:element>

</xs:schema>

- ErcotUtilities.xsd

<?xml version="1.0" encoding="UTF-8"?>

<!--   ERCOT Utilities: ErcotUtilities.xsd                                                                        -->

<!--                                                                                                                            -->

<!--    Release Date: August 2007                                                                                -->

<!--    Ext Int Spec Version No. 1.03                                                                             -->

<!--                                                                                                                            -->

<xs:schema xmlns="http://www.ercot.com/schema/2007-06/nodal/ews" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.ercot.com/schema/2007-06/nodal/ews" elementFormDefault="qualified" attributeFormDefault="unqualified" version="0.2">

<!--ERCOT Utilities: ErcotUtilities.xsd-->

<!--This XSD defines elements used for utility interfaces-->

<!---->

<xs:include schemaLocation="ErcotMiscTypes.xsd"/>

<!--Report Access-->

<!--Elements used for payload definitions using ComplexTypes-->

<xs:element name="Reports">

<xs:complexType>

<xs:sequence>

<xs:element name="Report" type="Report" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

- </xs:element>

</xs:schema>

Appendix D: Sample Code for Signing a SOAP Message

The purpose of this appendix is to provide examples for signing SOAP messages.

- J2EE

- Java Cryptography Architecture Example

import java.security.*;

...

// generate new RSA keypair

KeyPairGenerator k =

KeyPairGenerator.getInstance(“RSA”);

KeyPair kp = k.generateKeyPair();

// sign data

Signature s =

Signature.getInstance("SHA1withRSA");

s.initSign(kp.getPrivate());

byte[] signature = s.sign(data);

.NET

public static string ClientBase64KeyID = "ODytWwSUPj9/uGbXZTAdEhhzxLE=";

public void CallWebService()

{

PDCRegistrationProxy proxy = new PDCRegistrationProxy();

SoapContext requestContext = proxy.RequestSoapContext;

X509SecurityToken token = GetSigningToken();        //private–keystore

SignupRequest request = new SignupRequest();

request.Name = "Utility Integration Solutions";

request.Address = ERCOT, Taylor, TX";

requestContext.Security.Tokens.Add(token);

requestContext.Security.Elements.Add(new Signature(token));

SignupResponse response = proxy.SignupForPDC(request);

….

# Appendix E: Annotated SOAP Message

An annotated example of a SOAP message follows, which includes the required security elements. The example is divided into sections and marked by a number, e.g. 1.  Following the SOAP message are explanatory notes describing each numbered section. In order to improve readability, some attributes that have URI values have been shortened.

The example does not include any namespace declarations.  The table below lists the namespaces used in this example.

| Prefix | Description | Namespace |
| --- | --- | --- |
| SOAP-ENV | SOAP Envelop | http://schemas.xmlsoap.org/soap/envelope/ |
| ds | XML Digital Signature | http://www.w3.org/2000/09/xmldsig# |
| wsse | Web Service Security Extensions | http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-secext-1.0.xsd |
| wsu | Web Services Security Utility | http://www.docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd |
| xsd | XML Schema | http://www.w3.org/2001/XMLSchema |
| msg | An private namespace for specifying elements and attributes that are specific to ERCOT Web Services | TBD |

1

<SOAP-ENV:Envelope ">

<SOAP-ENV:Header ">

2

- <wsse:Security SOAP-ENV:mustUnderstand="1">

<wsse:BinarySecurityToken

- EncodingType="…#Base64Binary"

- ValueType="…#X509v3"

wsu:Id="CertId-1776694">

MIIDDDCCAfSgAwIBAgIQM6YEf7FVYx/tZyEXgVComTANBgkqhkiG9w0 DAVPQVNJUzEeMBwGA1UEAwwVT0FTSVMgSW50ZXJvcCBUZXN0IENBMB4DTE4MDMxOTIzNTk1OVowQjEOMAwGA1UECgwFT0FTSVMxIDAeBgNVBAsVGVzdCBDZXJ0MQ4wDAYDVQQDDAVBbGljZTCBnzANBgkqhkiG9w0BAQE9By1VYo0aHrkKCNT4DkIgPL/SgahbeKdGhrbu3K2XG7arfD9tqIBIKMyvq+mUnMpNcKnLXLOjkTmMCqDYbbkehJlXPnaWLzve+mW0pJdPxtf3r sZKT8DN5Kyz+EZsCAwEAAaOBkzCBkDAJBgNVHRMEAjAAMDMGA1UdHwQaW50ZXJvcC5iYnRlc3QubmV0L2NybC9jYS5jcmwwDgYDVR0PAQH/BAQ4l0TUHZ1QV3V2QtlLNDm+PoxiDAfBgNVHSMEGDAWgBTAnSj8wes1oR3 JTw==

- </wsse:BinarySecurityToken>

3

- <ds:Signature>

<ds:SignedInfo>

<ds:CanonicalizationMethod

- Algorithm="http://www.w3.org/2001/10/xml-exc-c14n#" />

<ds:SignatureMethod

- Algorithm="http://www.w3.org/2000/09/xmldsig#rsa-sha1" />

4

<ds:Reference URI="#id-1464350">

- <ds:Transforms>

<ds:Transform

- Algorithm="http://www.w3.org/2001/10/xml-exc-c14n#" />

</ds:Transforms>

<ds:DigestMethod

- Algorithm="http://www.w3.org/2000/09/xmldsig#sha1" />

<ds:DigestValue">

- 1JmC1C0FrlPB42xfFKolgaCew5k=

</ds:DigestValue>

</ds:Reference>

5

- <ds:Reference URI="#STRId-13498124">

<ds:Transforms>

<ds:Transform

Algorithm="…#STR-Transform" />

<wsse:TransformationParameters">

<ds:CanonicalizationMethod

- Algorithm="...xml-exc-c14n#"/>

</wsse:TransformationParameters>

</ds:Transform>

- </ds:Transforms>

<ds:DigestMethod

- Algorithm="http://www.w3.org/2000/09/xmldsig#sha1" />

<ds:DigestValue>

sEaCJjrObpDVfM8zuabwQdBGKbY=

- </ds:DigestValue>

</ds:Reference>

6

- </ds:SignedInfo>

<ds:SignatureValue">

H1b7jH2bHpbrzJXkFS0msdUYycDMH4n6m4oTRtbo4Yk35/JzGcuwUYZ3

AwPcnqmcP5ROshjJparaPGuvQhbFR7zCxet2aoawJFWgG8jIeuDZDE8y6n+kbBzxadF2tGN8/nH6IlKg0+onD09i81rPHDAa 2kstCclX2NDet1Rnmfs=

</ds:SignatureValue>

7

- <ds:KeyInfo>

<wsse:SecurityTokenReference wsu:Id="STRId-13498124">

<wsse:Reference URI="#CertId-1776694" ValueType="…#X509v3" />

- </wsse:SecurityTokenReference>

</ds:KeyInfo>

- </ds:Signature>

</wsse:Security>

</SOAP-ENV:Header>

8

- <SOAP-ENV:Body wsu:Id="id-1464350" >

<msg:Message>

<msg:Header>

<msg:Verb>verb</msg:Verb>

<msg:Noun>noun</msg:Noun>

<msg:ReplayDetection>

- <wsu:Created>

2006-11-29T20:05:55.022Z

- </wsu:Created>

<wsse:Nonce EncodingType="…#Base64Binary">

75753793-50c2-455b-a9b3-123cb26474e7

- </wsse:Nonce>

</msg:ReplayDetection>

- <msg:Revision>1</msg:Revision>

<msg:Source>market participant ID</msg:Source>

</msg:Header>

.

.

.

</msg:Message>

- </SOAP-ENV:Body>

</SOAP-ENV:Envelope>

1

- This section starts the SOAP envelope and the SOAP header

2

This section starts the Web Services Security Extensions, which includes security tokens and the digital signature. The first element in this section is the signer’s X.509 certificate, which is encoded in Base 64 binary. Note the identification of this element (CertId-1776694). The ID is later used in section 7 to reference this certificate. Note that there is only one certificate in this message. The message verifier must ensure that the certificate chains to a trusted root.

3

This section starts the digital signature block of the SOAP message. The signature is computed using the SHA-1 hash algorithm with RSA encryption.

4

This section designates the first of two objects that are signed. This one points to the entire message body (#id-1464350), which is specified in section 8 . The hashing algorithm is SHA-1.

5

This section designates the second of two objects that are signed. This one points to the reference to the certificate (#STRId-13498124), which is specified in section 7 .

6

This section specifies the value of the signature. That is, the SHA-1 hash of references to sections 7 and 8  and the encryption of this hash using the signer’s private key.

7

This section designates a reference to the signer’s certificate. In this case, the certificate is embedded in this SOAP message, and is referenced via the ID #CertId-1776694. This ID instructs the message verifier to get the certificate from section 2of this SOAP message.

8

This section starts the SOAP message body. It is designated using ID id-1464350, which is referenced as a signed element in section 2. Note that the message body includes an element called ReplayDetection, which consists of a timestamp indicating when the message was signed and a unique number (the nonce). These two elements help detect and prevent replay attacks. The rest of the message body (i.e., the business transaction) is not shown.

# Appendix F: SOAP Examples

The purpose of this appendix is to provide some complete SOAP examples for reference purposes. Examples are provided for requests, replies and notifications.

- SOAP Request Example

<?xml version="1.0" encoding="UTF-8"?>

- <SOAP-ENV:Envelope xmlns:SOAP-ENV="http://schemas.xmlsoap.org/soap/envelope/">

<SOAP-ENV:Body>

<RequestMessage xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

- <Header>

<Verb>create</Verb>

<Noun>BidSet</Noun>

<ReplayDetection>

<Nonce>3265874657400444949</Nonce>

<Created>2007-06-13T17:14:06.788-05:00</Created>

</ReplayDetection>

<Revision>001</Revision>

<Source>LOWER_COLORA_RIVER_AUTHO_QSE</Source>

<UserID>0</UserID>

<MessageID>MessageID1</MessageID>

<Comment>Sample message template</Comment>

</Header>

<Payload xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

<BidSet xmlns="http://www.ercot.com/schema/2007-05/nodal/ews" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<tradingDate>2007-07-26</tradingDate>

<ThreePartOffer>

<startTime>2007-07-26T00:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<externalId>12345</externalId>

<expirationTime>2007-07-26T00:00:00-05:00</expirationTime>

<resource>GIDEON_GIDEONG1</resource>

<combinedCycle>0</combinedCycle>

<FipFop>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<fipPercent>50</fipPercent>

<fopPercent>50</fopPercent>

</FipFop>

<StartupCost>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<hot>8172.00</hot>

<intermediate>4086.00</intermediate>

<cold>1362.00</cold>

</StartupCost>

<MinimumEnergy>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<cost>8172.00</cost>

</MinimumEnergy>

<EnergyOfferCurve>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<CurveData>

<xvalue>25</xvalue>

<y1value>39.23</y1value>

</CurveData>

<CurveData>

<xvalue>37</xvalue>

<y1value>40.23</y1value>

</CurveData>

<CurveData>

<xvalue>52</xvalue>

<y1value>41.23</y1value>

</CurveData>

<CurveData>

<xvalue>64</xvalue>

<y1value>42.23</y1value>

</CurveData>

<CurveData>

<xvalue>73</xvalue>

<y1value>43.23</y1value>

</CurveData>

<CurveData>

<xvalue>85</xvalue>

<y1value>44.23</y1value>

</CurveData>

<CurveData>

<xvalue>96</xvalue>

<y1value>55.23</y1value>

</CurveData>

<CurveData>

<xvalue>108</xvalue>

<y1value>75.23</y1value>

</CurveData>

<CurveData>

<xvalue>121</xvalue>

<y1value>101.23</y1value>

</CurveData>

<CurveData>

<xvalue>140</xvalue>

<y1value>102.23</y1value>

</CurveData>

</EnergyOfferCurve>

</ThreePartOffer>

<ThreePartOffer>

<startTime>2007-07-26T00:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<externalId>12346</externalId>

<expirationTime>2007-07-26T00:00:00-05:00</expirationTime>

<resource>GIDEON_GIDEONG2</resource>

<combinedCycle>0</combinedCycle>

<FipFop>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<fipPercent>50</fipPercent>

<fopPercent>50</fopPercent>

</FipFop>

<StartupCost>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<hot>8172.00</hot>

<intermediate>4086.00</intermediate>

<cold>1362.00</cold>

</StartupCost>

<MinimumEnergy>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<cost>8172.00</cost>

</MinimumEnergy>

<EnergyOfferCurve>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<CurveData>

<xvalue>25</xvalue>

<y1value>34.14</y1value>

</CurveData>

<CurveData>

<xvalue>39</xvalue>

<y1value>35.14</y1value>

</CurveData>

<CurveData>

<xvalue>50</xvalue>

<y1value>36.14</y1value>

</CurveData>

<CurveData>

<xvalue>63</xvalue>

<y1value>37.14</y1value>

</CurveData>

<CurveData>

<xvalue>72</xvalue>

<y1value>38.14</y1value>

</CurveData>

<CurveData>

<xvalue>86</xvalue>

<y1value>39.14</y1value>

</CurveData>

<CurveData>

<xvalue>96</xvalue>

<y1value>75.14</y1value>

</CurveData>

<CurveData>

<xvalue>106</xvalue>

<y1value>150.14</y1value>

</CurveData>

<CurveData>

<xvalue>116</xvalue>

<y1value>175.14</y1value>

</CurveData>

<CurveData>

<xvalue>138</xvalue>

<y1value>176</y1value>

</CurveData>

</EnergyOfferCurve>

</ThreePartOffer>

<ThreePartOffer>

<startTime>2007-07-26T00:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<externalId>12347</externalId>

<expirationTime>2007-07-26T00:00:00-05:00</expirationTime>

<resource>GIDEON_GIDEONG3</resource>

<combinedCycle>0</combinedCycle>

<FipFop>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<fipPercent>50</fipPercent>

<fopPercent>50</fopPercent>

</FipFop>

<StartupCost>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<hot>8172.00</hot>

<intermediate>4086.00</intermediate>

<cold>1362.00</cold>

</StartupCost>

<MinimumEnergy>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<cost>8172.00</cost>

</MinimumEnergy>

<EnergyOfferCurve>

<startTime>2007-07-26T01:00:00-05:00</startTime>

<endTime>2007-07-27T00:00:00-05:00</endTime>

<CurveData>

<xvalue>56</xvalue>

<y1value>30.60</y1value>

</CurveData>

<CurveData>

<xvalue>85</xvalue>

<y1value>86.68</y1value>

</CurveData>

<CurveData>

<xvalue>112</xvalue>

<y1value>96.93</y1value>

</CurveData>

<CurveData>

<xvalue>145</xvalue>

<y1value>100.75</y1value>

</CurveData>

<CurveData>

<xvalue>176</xvalue>

<y1value>125.64</y1value>

</CurveData>

<CurveData>

<xvalue>207</xvalue>

<y1value>150.43</y1value>

</CurveData>

<CurveData>

<xvalue>233</xvalue>

<y1value>150.83</y1value>

</CurveData>

<CurveData>

<xvalue>265</xvalue>

<y1value>151.28</y1value>

</CurveData>

<CurveData>

<xvalue>299</xvalue>

<y1value>155.63</y1value>

</CurveData>

<CurveData>

<xvalue>331</xvalue>

<y1value>156</y1value>

</CurveData>

</EnergyOfferCurve>

- </ThreePartOffer>

</BidSet>

- </Payload>

</RequestMessage>

</SOAP-ENV:Body>

</SOAP-ENV:Envelope>

- SOAP Reply Example

<?xml version="1.0" encoding="UTF-8"?>

- <SOAP-ENV:Envelope xmlns:SOAP-ENV="http://schemas.xmlsoap.org/soap/envelope/">

<SOAP-ENV:Body>

<ns0:ResponseMessage xmlns:SOAP-ENV = "http://schemas.xmlsoap.org/soap/envelope/" xmlns:ns0 = "http://www.ercot.com/schema/2007-05/nodal/ews/msg" xmlns:wsu = "http://docs.oasis-open.org/wss/2004/01/oasis-200401-wss-wssecurity-utility-1.0.xsd">

<ns0:Header>

<ns0:Verb>reply</ns0:Verb>

<ns0:Noun>BidSet</ns0:Noun>

<ns0:ReplayDetection>

<ns0:Nonce>2:36 PM</ns0:Nonce>

<ns0:Created>2007-08-08T11:54:32.105-05:00</ns0:Created>

</ns0:ReplayDetection>

<ns0:Revision>1.0</ns0:Revision>

<ns0:Source>ERCOT</ns0:Source>

</ns0:Header>

<ns0:Reply>

- <ns0:ReplyCode>OK</ns0:ReplyCode>

<ns0:Timestamp>2007-08-08T11:54:52.214-05:00</ns0:Timestamp>

</ns0:Reply>

<ns0:Payload>

- <ns1:BidSet xmlns:ns1 = "http://www.ercot.com/schema/2007-05/nodal/ews">

<ns1:tradingDate>2007-05-10</ns1:tradingDate>

- <ns1:ThreePartOffer>

<ns1:mRID>111111112.20070517.TPO.SOBAY_7_SY4</ns1:mRID>

- <ns1:externalId>12345</ns1:externalId>

<ns1:status>SUBMITTED</ns1:status>

- </ns1:ThreePartOffer>

</ns1:BidSet>

</ns0:Payload>

</ns0:ResponseMessage>

</SOAP-ENV:Body>

</SOAP-ENV:Envelope>

- SOAP Notification Example

<?xml version="1.0" encoding="UTF-8"?><SOAP-ENV:Envelope xmlns:SOAP-ENV="http://schemas.xmlsoap.org/soap/envelope/">

- <SOAP-ENV:Body>

<ns0:Notify xmlns:ns0="http://www.ercot.com/schema/2007-06/nodal/notification">

<ns0:NotificationMessage>

<ns0:Message>

<ns0:ResponseMessage xmlns:ns0="http://www.ercot.com/schema/2007-05/nodal/ews/msg">

- <ns0:Header>

<ns0:Verb>changed</ns0:Verb>

<ns0:Noun>BidSet</ns0:Noun>

<ns0:ReplayDetection>

<ns0:Nonce>f148e6952a1370b5019b512f71145bd9</ns0:Nonce>

<ns0:Created>2007-08-08T11:55:38.08-05:00</ns0:Created>

</ns0:ReplayDetection>

<ns0:Revision>1.02</ns0:Revision>

<ns0:Source>ERCOT</ns0:Source>

</ns0:Header>

<ns0:Reply>

<ns0:ReplyCode>OK</ns0:ReplyCode>

<ns0:Timestamp>2007-08-08T11:55:38.08-05:00</ns0:Timestamp>

</ns0:Reply>

<ns0:Payload>

<ns1:BidSet xmlns:ns1="http://www.ercot.com/schema/2007-05/nodal/ews">

<ns1:tradingDate>2007-05-10</ns1:tradingDate>

<ns1:ThreePartOffer>

<ns1:mRID>111111112.20070517.TPO.SOBAY_7_SY4</ns1:mRID>

<ns1:externalId>12342</ns1:externalId>

<ns1:status>ACCEPTED</ns1:status>

</ns1:ThreePartOffer>

</ns1:BidSet>

- </ns0:Payload>

</ns0:ResponseMessage>

</ns0:Message>

</ns0:NotificationMessage>

</ns0:Notify>

</SOAP-ENV:Body>

</SOAP-ENV:Envelope>

# Appendix G: XSL Examples

The purpose of this appendix is to provide XSL examples related to the processing and transformation of thre XML described in this specification. One example of XSL usage would be to convert UTC time into separate date and hour values based on the local time zone. For example, given the following XML:

<startTime>2007-12-17T06:00:00Z</startTime>

An XSL template can be used to generate tags that are desired by a consumer of the message such as the following where the hour is an hour ending in local time:

<startTime>2007-12-17T06:00:00Z</startTime>

<startDate>2007-12-17</startDate>

<startHour>1</startHour>

The following XSL example is implemented using custom functions within TIBCO BusinessWorks.

<?xml version='1.0'?>

<xsl:stylesheet version="1.0"

xmlns:xsl="http://www.w3.org/1999/XSL/Transform"

xmlns:msg="http://www.uisol.com"

xmlns:tib="http://www.tibco.com/bw/xslt/custom-functions"

exclude-result-prefixes="msg tib">

<xsl:template match="/">

- <xsl:apply-templates/>

</xsl:template>

<xsl:template match="*">

- <xsl:choose>

<xsl:when test="name() = 'msg:startTime'">

- <xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

<xsl:variable name="vStartTime">

<xsl:value-of select="tib:translate-timezone(text(),

tib:get-timezone-from-dateTime(current-dateTime()))"/>

- </xsl:variable>

<msg:startDate>

- <xsl:value-of select="substring($vStartTime,1,10)"/>

</msg:startDate>

<msg:startTime>

<xsl:choose>

<xsl:when test="substring($vStartTime,12,2) = '00'">24</xsl:when>

<xsl:otherwise><xsl:value-of select="format-number(substring($vStartTime,12,2),'#0')"/></xsl:otherwise>

</xsl:choose>

- </msg:startTime>

</xsl:when>

<xsl:when test="name() = 'msg:endTime'">

<xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

<xsl:variable name="vEndTime">

<xsl:value-of select="tib:translate-timezone(text(),

tib:get-timezone-from-dateTime(current-dateTime()))"/>

</xsl:variable>

<msg:endDate>

<xsl:value-of select="substring($vEndTime,1,10)"/>

</msg:endDate>

<msg:endTime>

<xsl:choose>

<xsl:when test="substring($vEndTime,12,2) = '00'">24</xsl:when>

<xsl:otherwise>

<xsl:value-of select="format-number(substring($vEndTime,12,2),'#0')"/>

</xsl:otherwise>

</xsl:choose>

</msg:endTime>

</xsl:when>

<xsl:otherwise>

<xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

- </xsl:otherwise>

</xsl:choose>

</xsl:template>

</xsl:stylesheet>

The following XSL example is implemented using Xalan:

<?xml version='1.0'?>

<xsl:stylesheet version="1.0"

xmlns:xsl="http://www.w3.org/1999/XSL/Transform"

xmlns:msg="http://www.uisol.com"

xmlns:lxslt="http://xml.apache.org/xslt"

xmlns:mk="ext1"

extension-element-prefixes="mk"

exclude-result-prefixes="msg lxslt">

<lxslt:component prefix="mk" functions="ConvertDate">

- <lxslt:script lang="javascript">

function ConvertDate(inYear,inMonth,inDay,inTime,inType){

var strDate = inMonth + "/" + inDay + "/" + inYear + " " + inTime + " UTC";

var GMTDate = new Date(strDate);

strDate = inType == "Date" ? GMTDate.getFullYear() + "-" + GMTDate.getMonth() + "-" + GMTDate.getDate() : GMTDate.getHours();

return strDate.toString();

}

</lxslt:script>

</lxslt:component>

<xsl:template match="/">

- <xsl:apply-templates/>

</xsl:template>

<xsl:template match="*">

- <xsl:choose>

<xsl:when test="(name() = 'msg:startTime' or name() = 'msg:endTime')”>

- <xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

<xsl:variable name="vYear"><xsl:value-of select="substring(text(),1,4)"/></xsl:variable>

<xsl:variable name="vMonth"><xsl:value-of select="substring(text(),6,2)"/></xsl:variable>

<xsl:variable name="vDay"><xsl:value-of select="substring(text(),9,2)"/></xsl:variable>

<xsl:variable name="vTime"><xsl:value-of select="substring(text(),12,8)"/></xsl:variable>

<xsl:choose>

- <xsl:when test="name() = 'msg:startTime'">

<msg:startDate>

<xsl:value-of select="mk:ConvertDate(string($vYear),string($vMonth),string($vDay),string($vTime),'Date')"/>

</msg:startDate>

<msg:startTime>

<xsl:value-of select="mk:ConvertDate(string($vYear),string($vMonth),string($vDay),string($vTime),'Time')"/>

- </msg:startTime>

</xsl:when>

- <xsl:when test="name() = 'msg:endTime'">

<msg:endDate>

<xsl:value-of select="mk:ConvertDate(string($vYear),string($vMonth),string($vDay),string($vTime),'Date')"/>

</msg:endDate>

<msg:endTime>

<xsl:value-of select="mk:ConvertDate(string($vYear),string($vMonth),string($vDay),string($vTime),'Time')"/>

- </msg:endTime>

</xsl:when>

</xsl:choose>

</xsl:when>

<xsl:otherwise>

<xsl:copy>

<xsl:apply-templates/>

</xsl:copy>

- </xsl:otherwise>

</xsl:choose>

</xsl:template>

</xsl:stylesheet>

# Appendix H: Payload Compression Example

The purpose of this section is to provide an example of the code required to compress and encode a payload, where the payload is then passed as the contents of the Payload/Compressed element. The following is a Java example that leverages commonly used classes for compression and base64 encoding.

package BusinessService.TS.WebServiceClient.CompressionClient;

import java.util.*;

import java.io.*;

import java.io.File;

import java.io.FileInputStream;

import java.io.FileOutputStream;

import java.io.InputStream;

import java.io.IOException;

import java.util.zip.*;

import org.apache.soap.encoding.soapenc.Base64;

public class CompressionClientCompressandEncode{

protected byte[] input = null;

protected String base64GzipInput = "";

public byte[] getinput() {

return input;

}

public void setinput(byte[] val) {

input = val;

}

public String getbase64GzipInput() {

return base64GzipInput;

}

public void setbase64GzipInput(String val) {

base64GzipInput = val;

}

public CompressionClientCompressandEncode() {

}

public void invoke() throws Exception {

In  : byte[] input

Out : String base64GzipInput

ByteArrayOutputStream bas = new ByteArrayOutputStream();

GZIPOutputStream bis = new GZIPOutputStream(bas);

bis.write(input);

bis.close();

base64GzipInput = Base64.encode(bas.toByteArray());

}

}

# Appendix I: Issues

- The purpose of this section is to capture issues and resolutions.

| Issue ID | Date | Issues | Resolution |
| --- | --- | --- | --- |
| 1 | 1/15/2007 | Bid IDs. Need to augment interfaces to provide a unique bid ID that can be used for the update and cancellation of bids as opposed to specifying a set of primary key values that differ for each bid type | Added mRID definition. |
| 2 | 1/15/2007 | Need to finalize scheme for acceptance and rejection of trades | Duplicate trades must be entered. Periodic notification for unmatched trades. |
| 3 | 1/17/2007 | Currently EnergySchedules and CapacitySchedules use CIM IrregularTimeSchedules. Could decide to use a more compact/custom structure | Decided to stay consistent with CIM. This was revisited by item 21. |
| 4 | 1/17/2007 | References to resources (i.e. resource names) could be more CIM-compliant using RegisteredResource.name | Reference is by name only. Use simplest XML. |
| 5 | 1/17/2007 | References to sources, sinks and sp could be more CIM-compliant by using Pnode.name. Related to issue 4. | Reference is by name only. Use simplest XML. |
| 6 | 1/18/2007 | May want to add QSE ID to each award message | Done. |
| 7 | 1/23/2007 | Need to clarify the use of market types for bidding interfaces for consistency with Nodal protocols. | Market types are not used. Retain as an optional field for potential future use. |
| 8 | 1/23/2007 | SOAP faults will be added to the WSDL in the future. | Added to SandBox. |
| 9 | 2/11/2007 | Need to define indicator for ASOffer and mcpc for AwardedAS. Define lmp and spp for AwardedEnergyOffer. | Use curveStyle |
| 10 | 2/11/2007 | Need to define enumerations for AS types and revise section 2.3.5 and add enumerations to XML schema. We would then update examples. | Reg-Up, Reg-Down, Non-Spin, RRS |
| 11 | 2/20/2007 | Need to define resource status enumerations and add to XML Schema | Done. |
| 12 | 3/2/2007 | Need to decide whether or not expiration is used for EnergyOnlyBid or EnergyOnlyOffer | Current view is both. |
| 13 | 3/2/2007 | Need to identify the specific identifier to be used to retrieve load profiles for non-IDR customers. |  |
| 14 | 3/2/2007 | Need to add notifications to MPs for the create, update or cancel of Outages | Section 5.3.11 |
| 15 | 3/14/2007 | Need to define SLAs |  |
| 16 | 3/14/2007 | In section 3.3.4, is indicator needed? | Indicator is now handled by curveStyle |
| 17 | 3/14/2007 | Consider segmentation of Ercot.xsd into more XSDs, e.g. ErcotCommon.xsd, ErcotBid.xsd, ErcotNotification.xsd, ErcotInfo.xsd | done |
| 18 | 3/21/2007 | Need to define convention for resource names, as unit names are not unique unless concatenated with the station name |  |
| 19 | 3/23/2007 | Depending upon usage (i.e. more than one record per bid), it may be more efficient to redefine complex types for StartupCost, MinimumEnergy, MaximumPrice, MinimumReservationPrice, Limits, ResourceStatus or ASCapacity as an IrregularIntervalSchedule. | Decided to leave as is |
| 20 | 3/23/2007 | XML examples should be use correct namespaces and qualifications. | Done in revision 1.03 |
| 21 | 3/27/2007 | After revision 1.0 and finalization of the MMS representation of market intervals, the bidding interfaces will be revised as appropriate to use ‘market time’, where market hours and intervals will be identified. TPTF will be consulted on the final design. | Decided to use TmSchedule structure. Document revised to reflect this. |
| 22 | 3/27/2007 | Need to identify the means to notify market participants when documents in the repository have been updated or created. |  |
| 23 | 3/27/2007 | MPs need to provide input with respect to additional filtering capabilities needed for interfaces. This would apply primarily to those interfaces in section 4. This input would be used to extend the interface specifications. |  |
| 24 | 3/27/2007 | Note that ASOffer may change pending MMS design | Revised definition provided in revision 1.02 |
| 25 | 3/27/2007 | IncDecOffer should be modified to allow two curves. | Defined two curves, need final definition to be consistent with MMS design. See item 38. |
| 26 | 3/30/2007 | Need to add non-spin and RRS deployment. This is dependent upon functionality and associated notifications from MMS. | Non-Spin and RRS AS Types where added to AS Offer and SelfArrangedAS,ASTrade |
| 27 | 4/6/2007 | Corrected verb to be ‘change’ instead of ‘update’ to be consistent with enumerations within WSDL | done |
| 28 | 4/19/2007 | Need to add labels for figures and tables throughout document | done |
| 29 | 4/24/2007 | Need to add notification message for startup and shutdowns | done |
| 30 | 5/2/2007 | Need to revise IncDecOffer interface so that it is consistent with final MMS design | done |
| 31 | 5/2/2007 | Would like to have PENDING status prior to credit approval, after approval it would be ACCEPTED. This is useful when transactions are submitted 7 days in advance. Need to process this within integration layer, as MMS will not handle this status. Issue related to queries. | Done in revisions 1.02, 1.03 |
| 32 | 5/2/2007 | Need to use DUNS numbers for QSE ID and for trades. Need to verify with MMS. | The ‘short name’ of the QSE should be used for ‘Message/Source’ and for ‘buyer’ and ‘seller’ within trades. |
| 33 | 5/2/2007 | Need web service for DC tie flows |  |
| 34 | 5/2/2007 | Need to explore use of QSE supplied external IDs that are managed by MMS | See item 39 |
| 35 | 5/2/2007 | Need notification message for AS Obligations | done |
| 36 | 5/2/2007 | Need notification for bid rejection | done |
| 37 | 5/2/2007 | Upon request by MPs, added combinedCycle wherever resource was identified (e.g. COP). Need to verify this will be supported within MMS | done |
| 38 | 5/14/2007 | Need to revise mRID for IncDecOffers to reflect INC or DEC. | Used ‘choice’ in XML schema |
| 39 | 5/16/2007 | Need to add ‘externalId’ tag to all types of bids, offers, trades and schedules in section 3. The externalId will be managed by Integration Layer, not by MMS. | done |
| 40 | 6/13/2007 | Need to finalize how cancel energy flag will be used by ABB response messages |  |
| 41 | 6/20/2007 | Should make startTime and endTime for TmSchedules optional, as a schedule can be fully defined with TmPoints using time and end elements. | Done in revision 1.02 |
| 42 | 6/27/2007 | ASOffer needs multi-hour block indicator for load resources | Done in revision 1.03 |
| 43 | 6/27/2007 | Need to be sure that curves and their usage are unambiguously described in section 2.3.1 so that it is consistent with usage by ABB | Done in revision 1.03 |
| 44 | 6/28/2007 | In addition to ‘non-controllable’ load deployment, need XML schema and notification interface for generation and controllable loads Non-spin deployment. The required fields are: - QSE_ID, - Resource Name, - Deployed MW - Duration of the deployment ( start, and end time). |  |
| 45 | 7/24/2007 | Reg-Down for ASOffer should allow for 5 points | Done in revision 1.03 |
| 46 | 7/25/2007 | Need to add account holder ID to CRR. Should offer ID also be added? Should it be a key? | Done in revision 1.03 |
| 47 | 7/25/2007 | Peak load forecast for CRR should be a single value | Corrected in version 1.03 |
| 48 | 7/25/2007 | MMS does not support tradeId for differentiating trades. Need to remove tradeId as a required item or key value from specification. | Removed in version 1.03 |
| 49 | 7/26/2007 | Need to add comments to each XSD file to identify the purpose. | Added comment lines at the beginning of each XSD. |
| 50 | 8/9/2007 | Outage structures are still in a draft status. |  |
| 51 | 8/10/2007 | Need to address get mRID issue for IncDecOffer. | Extended mRID for IncDecOffer in version 1.02 |
| 52 | 8/22/2007 | Due to limitations of MMS, get BidSet can only be for common product types on a specific date. | Revised specification to identify this limitation. |
| 53 | 8/23/2007 | Need to add offerId to CRR mrid | Now a key field in version 1.04 |
| 54 | 8/23/2007 | Need to add crrAccountHolderId to PTPObligation mrid | Now a key field in version 1.04 |
| 55 | 8/23/2007 | Need to update error messages in section 3.4 to reflect implementation |  |
| 56 | 8/29/2007 | Due to limitations of MMS, cancel BidSet can only be for common product types on a specific date. | Revised specification to identify this limitation. |
| 57 | 8/29/2007 | Due to limitations of MMS, create BidSet can only be for common product types on a specific date. | This will be supported by functionality within the integration layer to be deployed at a future date. |
| 58 | 10/4/2007 | Some of the type definitions for the revised outage structures may be moved to CommonTypes.xsd in the future |  |
| 59 | 10/4/2007 | Need to revise outage interface to provide outage update capability. |  |
| 60 | 10/17/2007 | Need to correct text, figure 28 and example in section 3.3.4 for ASOffer to match XSD. | Corrected in version 1.06 |
| 61 | 10/17/2007 | Need to add examples for cancels of different hours |  |
| 62 | 10/17/2007 | Need to handle time for sections 4.3.10, 4.3.11, 4.3.15, 4.3.16 | Added in version 1.06 |
| 63 | 10/30/2007 | Need to add notification for WGRPP |  |