| NPRR Number | 112 | NPRR Title | Emergency Base Point Price Revision |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Recommended Action |  | Approval |
| Date of Decision |  | March 20, 2008 |  |  |
| Protocol Section Requiring Revision |  | 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) clarifies the calculation of the Emergency Base Point Price per QSE per Resource by interval (EBPPR) variable. |  |  |
| Overall Market Benefit |  | The proposal establishes consistency in Energy Offer Curve extension methodology between the Market Management System (MMS) and Settlements. It also reduces the Charge for Emergency Power Increases (LAEMREAMT). |  |  |
| Overall Market Impact |  | None. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - NPRR112 was posted on 3/5/08. - On 3/20/08, PRS considered NPRR112. |  |  |
| PRS Decision |  | On 3/20/08, PRS voted to recommend approval of NPRR112 as submitted. The motion passed with three abstentions from the Consumer and the Independent Power Marketer (IPM)(2) Market Segments. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 3/20/08, PRS noted that the Consumers and IPM Market Segments abstained from the Transition Plan Task Force (TPTF) vote. ERCOT Staff explained that this NPRR corrects the gap when an Offer falls outside the emergency base point curve. Consumer participants questioned why an Entity would not submit an energy offer curve up to the cap or High Dispatch Limit (HDL). ERCOT Staff responded this NPRR represents the methodology chosen by the TPTF and is consistent with how the Security-Constrained Economic Dispatch (SCED) mitigates. |  |  |

ERCOT/Market Segment Impacts and Benefits

| Assumptions | 1 | Formulas should accurately the intent of the Protocols and accuracy enhances transparency of the market. |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None |  |

| Original Sponsor |  |
| --- | --- |
| Name | Bill Barnes |
| Company | ERCOT |
| Market Segment | Not applicable |

Proposed Protocol Language Revision

6.6.9.1	Payment for Emergency Power Increase Directed by ERCOT

(1)	If the Emergency Base Point issued to a Generation Resource is higher than the SCED Base Point immediately before the Emergency Condition, then ERCOT shall pay the QSE an additional compensation for the Resource at its Resource Node Settlement Point.  The payment for a given 15-minute Settlement Interval is calculated as follows:

EMREAMT q, r, p	=	(-1) * EMREPR q, r, p * EMREq, r, p

Where:

EMREPR q, r, p	=	Max (0, EBPWAPR q, r, p – RTSPP p)

EBPWAPR q, r, p	=	Σy(EBPPR q, r, p, y * EBP q, r, p, y * TLMP y) /

Σy(EBPq, r, p, y * TLMP y)

EMRE q, r, p	=	Max (0, Min (AEBP q, r, p * ¼ RTMG q,r,  pr) – ¼ * BP q, r, p)

AEBP q, r, p		=	Σy(EBP q, r, p, y * TLMPy / 3600)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EMREAMT q, r, p | $ | Emergency Energy Amount per QSE per Settlement Point per Resource—The payment to QSE q as additional compensation for the additional energy produced by Generation Resource r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| EMREPR q, r, p | $/MWh | Emergency Energy Price per QSE per Settlement Point per Resource—The compensation rate for the additional energy produced by Generation Resource r at Resource Node p represented by QSE q in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| EMRE q, r, p | MWh | Emergency Energy per QSE per Settlement Point per Resource—The additional energy produced by Generation Resource r at Resource Node p represented by QSE q in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| EBPWAPR q, r, p | $/MWh | Emergency Base Point Weighted Average Price per QSE per Settlement Point per Resource—The weighted average of the energy prices corresponding with the Emergency Base Points on the Energy Offer Curve for Resource r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval. |
| BP q, r, p | MW | Base Point per QSE per Settlement Point per Resource—The Base Point of Resource r at Resource Node p represented by QSE q from the SCED prior to the Emergency Condition. |
| AEBP q, r, p | MW | Aggregated Emergency Base Point— The Generation Resource’s aggregated Emergency Base Point, for the 15-minute Settlement Interval. |
| EBP q, r, p, y | MW | Emergency Base Point per QSE per Settlement Point per Resource by interval—The Emergency Base Point of Resource r at Resource Node p represented by QSE q for the Emergency Base Point interval or SCED interval y. If a Base Point instead of an Emergency Base Point is effective during the interval y, its value equals the Base Point. |
| EBPPR q, r, p, y | $/MWh | Emergency Base Point Price per QSE per Settlement Point per Resource by interval – The average incremental energy cost calculated per the Energy Offer Curve for the output levels between the SCED Base Point immediately before the Emergency Condition and the Emergency Base Point of Resource r at Resource Node p represented by QSE q for the Emergency Base Point interval or SCED interval y |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| RTMG q, r, p | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource—The metered generation of Resource r at Resource Node p represented by QSE q in Real-Time for the 15-minute Settlement Interval. |
| TLMP y | second | Duration of Emergency Base Point interval or SCED interval per interval—The duration of the portion of the Emergency Base Point interval or SCED interval y within the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |
| y | none | An Emergency Base Point interval or SCED interval that overlaps the 15-minute Settlement Interval. |

(2)	The extension of the Energy Offer Curve used to calculate the Emergency Base Point Price is illustrated with the pictures below.  If the Emergency Base Point MW value is greater than the largest MW value on the Energy Offer Curve submitted by the QSE for the Resource, then the Energy Offer Curve is extended to the Emergency Base Point MW value with a $/MWh value that is the greater of the highest $/MWh value on the Energy Offer Curve submitted by the QSE or the Mitigated Offer Cap (pursuant to Section 4.4.9.4.1, Mitigated Offer Cap) for the highest MW output on the Energy Offer Curve submitted by the QSE for the Resource.

Q1       Q2         SCED          Q3         EBP        MW

$/

MWh

P 3

P2

P1

The area under the capped Energy Offer Curve equals

(EBPPR * (EBP – SCED BP))

Mitigated Offer Cap

Extended portion of Energy Offer Curve

Q1       Q2        SCED             Q3         EBP        MW

$/

MWh

P 3

P2

P1

The area under the capped Energy Offer Curve equals

(EBPPR * (EBP – SCED BP))

Mitigated Offer Cap

Extended portion of Energy Offer Curve

(3)	The total additional compensation to each QSE for emergency power increases of Generation Resources for the 15-minute Settlement Interval is calculated as follows:

EMREAMTQSETOT q	=	ΣrΣpEMREAMT q, r, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EMREAMTQSETOT q | $ | Emergency Energy Amount QSE Total per QSEThe total of the payments to QSE q as additional compensation for emergency power increases of the non-RMR Generation Resources represented by this QSE for the 15-minute Settlement Interval. |
| EMREAMT q, r, p | $ | Emergency Energy Amount per QSE per Settlement Point per Resource—The payment to QSE q as additional compensation for the additional energy produced by Generation Resource r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| q | None | A QSE. |
| p | None | A Resource Node Settlement Point. |
| r | None | A Generation Resource. |