| NPRR Number | 103 | NPRR Title | Settlement of Power Imported via DC Ties and Block Load Transfer Under a Declared Emergency Condition |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Recommended Action |  | Approval |
| Date of Decision |  | March 20, 2008 |  |  |
| Protocol Sections Requiring Revision |  | 6.6.3.4, Real-Time Energy for DC Tie Import 6.6.3.5, Real-Time Payment for a Block Load Transfer Point |  |  |
| Proposed Effective Date |  | Upon Texas Nodal Market Implementation |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Revision Description |  | During a declared Emergency Condition, ERCOT may order an emergency energy import over the Direct Current Tie (DC Tie) interconnects and/or a Block Load Transfer (BLT) from the ERCOT Control Area to a non-ERCOT Control Area. These proposed revisions add Settlement formulas to pay a Qualified Scheduling Entity (QSE) for emergency energy imported over the DC Ties and for energy supplied to BLT Load moved from the ERCOT Control Area to a non-ERCOT Control Area during a declared Emergency Condition. These payments may only be made for actions taken by the QSE in response to an ERCOT Dispatch Instruction. |  |  |
| Overall Market Benefit |  | Encourage availability of an alternative source of energy during an Emergency Electric Curtailment Plan (EECP) and thus increase grid and system reliability. |  |  |
| Overall Market Impact |  | Costs of emergency power would be uplifted. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts |  | ERCOT credit staff and the Credit Work Group (Credit WG) have reviewed PRR103 and do not believe that it requires changes to credit monitoring activity or the calculation of liability.. |  |  |
| Procedural History |  | - NPRR103 was posted on 2/13/08. - On 2/19/08, ERCOT Staff comments were posted. - On 2/21/08, PRS considered NPRR103. - On 3/13/08, the Impact Analysis (IA) was posted. - On 3/19/08, ERCOT Staff comments were posted. - On 3/20/08, PRS again considered NPRR103, the IA and this Recommendation Report. |  |  |
| PRS Decision |  | On 2/21/08, PRS unanimously voted to recommend approval of NPRR103 as revised by ERCOT comments and PRS. All Market Segments were present for the vote. On 3/20/08, PRS unanimously voted to recommend approval of NPRR103 as revised by ERCOT Staff comments dated 3/19/08. Then PRS voted to forward the IA and this Recommendation Report to TAC. All Market Segments were present for both votes. |  |  |
| Summary of PRS Discussion |  | On 2/21/08, PRS discussed eliminating the term “Emergency” in language related to the BLT, because a BLT only occurs during Emergency Conditions, and this additional terminology would, therefore, be duplicative. There was no discussion preceding the 3/20/08 votes. |  |  |

ERCOT/Market Segment Impacts and Benefits

| Assumptions | 1 | None |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | NA |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Grid Reliability |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | None |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 | None |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT Staff 021908 | Offered revised language for the purpose of style, clarification and internal consistency |
| ERCOT Staff 031908 | Proposed corrections. |

| Original Sponsor |  |
| --- | --- |
| Name | Trip Doggett, on behalf of the Transition Plan Task Force (TPTF) |
| Company | ERCOT |
| Market Segment | N/A |

Proposed Protocol Language Revision

6.6.3.4	Real-Time Energy Payment for DC Tie Import

(1)	The payment to each QSE for energy imported into the ERCOT System through each DC Tie is calculated based on the Real-Time Settlement Point Price at the DC Tie Settlement Point.  The payment for a given 15-minute Settlement Interval is calculated as follows:

RTDCIMPAMT q, p	=	(-1) * RTSPP p * (RTDCIMP q, p * ¼)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTDCIMPAMT q, p | $ | Real-Time DC Import Amount per QSE per Settlement Point—The payment to QSE q for DC Tie import through DC Tie p, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| RTDCIMP q, p | MW | Real-Time DC Import per QSE per Settlement Point—The aggregated DC Tie Schedule submitted by QSE q as an importer into the ERCOT System through DC Tie p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A DC Tie Settlement Point. |

(2)	ERCOT shall pay each QSE for energy imported into the ERCOT System during a declared Emergency Condition through each DC Tie in response to an ERCOT Dispatch Instruction.  The payment for a given 15-minute Settlement Interval is calculated as follows:

RTEDCIMPAMT q, p	=	(-1) * Max {RTSPP p, ( VCOSTEMGENERGYq *CA)}* (RTEDCIMP q, p * ¼)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTEDCIMPAMT q, p | $ | Real-Time Emergency DC Import Amount per QSE per Settlement Point—The payment to QSE q for emergency DC Tie import through DC Tie p, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| FIP | $/MMBtu | Fuel Index Price – As defined in Section 2, Definitions and Acronyms. |
| RTEDCIMP q, p | MW | Real-Time emergency DC Import per QSE per Settlement Point—The aggregated DC Tie Schedule for emergency energy imported by QSE q into the ERCOT System during emergency conditions through DC Tie p, for the 15-minute Settlement Interval. |
| VCOSTEMGENERGYq | $/MWh | Verified Cost for Emergency Energy — The ERCOT verified cost for the energy imported by QSE q into the ERCOT System during declared Emergency Condition through a DC Tie p as instructed by an ERCOT dispatch instruction. |
| CA | # | Cost Adder-- A multiplier of 1.10 |
| q | none | A QSE. |
| p | none | A DC Tie Settlement Point. |

(3)	The total of the payments to each QSE for all energy imported into the ERCOT System through DC Ties for the 15-minute Settlement Interval is calculated as follows:

RTDCIMPAMTQSETOT q	=	p(RTDCIMPAMT q, p + RTEDCIMPAMT q, p)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTDCIMPAMTQSETOT q | $ | Real-Time DC Import Amount QSE Total per QSEThe total of the payments to QSE q for energy imported into the ERCOT System through DC Ties for the 15-minute Settlement Interval. |
| RTDCIMPAMT q, p | $ | Real-Time DC Import Amount per QSE per Settlement Point—The payment to QSE q for DC Tie import through DC Tie p, for the 15-minute Settlement Interval. |
| RTEDCIMPAMT q, p | $ | Real-Time Emergency DC Import Amount per QSE per Settlement Point—The payment to QSE q for emergency DC Tie import through DC Tie p, for the 15-minute Settlement Interval. |
| q | None | A QSE. |
| p | None | A DC Tie Settlement Point. |

6.6.3.5	Real-Time Payment for a Block Load Transfer Point

(1)	ERCOT shall pay each QSE for the energy delivered to an ERCOT Load through a BLT Point that is moved in response to an ERCOT Verbal Dispatch Instruction during a declared Emergency Condition, from the ERCOT Control Area to a Non-ERCOT Control Area.  The payment for a given 15-minute Settlement Interval is calculated as follows:

BLTRAMTq, bltp,p = (-1) * MAX {RTSPPp ,  ( VCOSTEMGENERGYq,bltp)*CA} * BLTR q, bltp,p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| BLTRAMT q, bltp , p | $ | Block Load Transfer Resource Amount per QSE per Settlement Point per BLT Point—The payment to QSE q for the BLT Resource that delivers energy to Load Zone p through BLT Point bltp, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| VCOSTEMGENERGYq,bltp | $/MWh | Verified Cost for Emergency Energy — The ERCOT verified cost for the energy delivered to an ERCOT Load through BLT Point bltp during a declared Emergency Condition in ERCOT as determined by an ERCOT Verbal Dispatch Instruction. |
| CA | # | Cost Adder-- A multiplier of 1.10 |
| BLTR q, p, bltp | MWh | Block Load Transfer Resource per QSE per Settlement Point per BLT Point—The energy delivered to an ERCOT Load in Load Zone p through BLT Point bltp represented by QSE q, for the 15-minute Settlement Interval. |
| q | None | A QSE. |
| p | None | A Load Zone Settlement Point. |
| bltp | None | A BLT Point. |

(3)	The total of the payments to each QSE for all energy delivered to ERCOT Loads through BLT Points for the 15-minute Settlement Interval is calculated as follows:

BLTRAMTQSETOT q	=	ΣpΣbltp(BLTRAMT q, bltp, p)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| BLTRAMTQSETOT q | $ | Block Load Transfer Resource Amount QSE Total per QSEThe total of the payments to QSE q for energy delivered into the ERCOT System through BLT Points for the 15-minute Settlement Interval. |
| BLTRAMT q, bltp, p | $ | Block Load Transfer Resource Amount per QSE per Settlement Point per BLT Point—The payment to QSE q for the BLT Resource at BLT Point bltp, which delivers energy to Load Zone p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Load Zone Settlement Point. |
| bltp | none | A BLT Point. |